3GPP TSG-RAN WG2 Meeting #129bis	R2-2502860

Wuhan, China, Apr 7th – 11th, 2024

Agenda item:		8.7.5

Source:		Samsung

Title:			RLC Enhancements for XR

Document for:	Discussion & Decision

# 1 Introduction

In RAN2#129, following agreements were made on RLC enhancements:

Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

Also, RAN2 reached following agreements for PDCP SN gap reporting for mobility scenario, considering the Rel-19 AM RLC enhancements:

PDCP SN gap reporting in context of R19 RLC enhancements - From RAN2 point of view, for Rel-18 alternative 3 should not be supported. RAN2 doesn’t see a big problem to solve for Rel-18. - RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem to solve due to RLC enhancements. If it is a problem further discussions are needed on the alternatives. - The session chair indicates that it can be treated in RLC enhancement AI

In this paper we discuss the following:

- Relevant Stage-3 issues for R19 RLC enhancements

- PDCP SN gap reporting during mobility in R19

- Impact of R19 RLC enhancement on PDCP SN gap report triggering

# 2 Discussion

## 2.1 Successful delivery indication in case of fake ACK

##### 5.2.3.1.1 General … When receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity shall: - send an indication to the upper layers of successful delivery of the RLC SDU; - set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet.

When a discard indication is received from upper layer for an RLC SDU, the transmitting side of an AM RLC entity considers the RLC SDU as obsolete and stops transmission or retransmission for the RLC SDU. Further, upon receiving fake ACK for the obsolete RLC SDU, the transmitting side of an AM RLC entity moves its transmission window and thereby, obsolete SDU is discarded. However, the question is whether an indication of successful delivery should be  sent or not to the upper layer in this case? The purpose of this indication at upper layer (PDCP) is to determine whether retransmission of the PDCP SDU is to be performed. However, as the PDCP SDU corresponding to obsolete RLC SDU has already been discarded by PDCP, no such purpose exists. Therefore, there is no need to provide an indication of successful delivery to the upper layers for an obsolete RLC SDU.

Proposal 1: Upon receiving ACK for an obsolete RLC SDU, the transmitting side of an AM RLC entity does not send an indication of successful delivery of the RLC SDU to the upper layers. (Adopt TP 1)

## 2.2 Timing for status reporting upon t-RxDiscard expiry

### 5.3.4 Status reporting … When constructing a STATUS PDU, the AM RLC entity shall: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: - for an RLC SDU for which no byte segments have been received yet: - include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU. - for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet: - include in the STATUS PDU a set of NACK_SN, SOstart and SOend. - for a continuous sequence of RLC SDUs that have not been received yet: - include in the STATUS PDU a set of NACK_SN and NACK range; - include in the STATUS PDU, if required, a pair of SOstart and SOend. - set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing in the resulting STATUS PDU.

During CR review, it is marked as FFS about what time the status report is sent after the expiry of t-RxDiscard e.g., immediately or later? It can be noted that when t-RxDiscard expires, the yellow highlighted condition (i.e., RX_Next <= SN < RX_Highest_Status) may not be TRUE, as possibly RX_Next and RX_Highest_Status may point to same SN and RX_Highest_Status could be updated only after t-Reassembly timer expiry following the t-RxDiscard expiry. Therefore, it would not be possible to convey any NACK_SN, SOstart and SOend fields in the status report, even when RX_NEXT may have at least one byte segment missing. However, ACK_SN can still be set to RX_Next and thereby, indicates that all abandoned RLC SDUs are positively acknowledged. We think this is a quite meaningful and timely status information for the transmitter. Therefore, it is reasonable that the status report should be immediately sent after the expiry of t-RxDiscard.

Proposal 2: Expiry of t-RxDiscard triggers a status report and status report is immediately sent to peer RLC entity.

## PDCP SN gap reporting for AM DRBs during Mobility

Previosuly RAN2 discussed an issue raised by RAN3 LS. Issue concerns to the PDCP SN gap report during mobility. In this scenario, UE may discard some PDCP SDUs before handover and informs the source gNB through a PDCP SN gap report. Now, the source gNB during the handover operation forwards the buffered packets along with the SNs for the PDCP packets that are not received in the SN STATUS TRANSFER message to the target gNB. However, target gNB can not recover those PDCP SDUs through retransmissions of not received packets which are already discarded by the UE. Also target gNB is not aware about the discard of those PDCP SDUs and this will lead to excesive delay due t-reordering operations.

In RAN2#129, it was agreed to discuss the issue and potentially address in the context of R19 AM RLC enhancements. Some companies think that the issue can be solved by allowing a re-transmisison or a re-submission of the PDCP SN gap report after handover to the target gNB that was earlier sent to the source gNB during mobility. However, such approach breaks the basic tenet of radio protocols wherein a Control PDU is neither stored nor retransmitted.

Instead, we think it is possible to consider a new trigger for PDCP SN gap report after handover that allows the reporting of the discard information including the previous set of discarded PDCP SDUs. More precisely, for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, when a PDCP SN gap report was previously transmitted during mobility (e.g. prior to PDCP entity re-establishment) but its successful delivery has not been confirmed by lower layers, the transmitting PDCP entity should trigger a PDCP SN gap report that includes the same set of previously discarded PDCP SDUs, apart from newly discarded PDCP SDUs, if any. Further, a separate configuration “sn-GapReportMobility” can be used for R19 AM DRBs for this purpose, to distinguish from the legacy configuration.

Proposal 3: RAN2 agree to the following approach in order to address PDCP SN gap reporting for AM DRBs during mobility (Adopt TP 2):

- For AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, when a PDCP SN gap report was previously transmitted during mobility but its successful delivery has not been confirmed by lower layers, the transmitting PDCP entity triggers a PDCP SN gap report that includes the same set of previously discarded PDCP SDUs, apart from newly discarded PDCP SDUs, if any.

- A separate configuration “sn-GapReportMobility” is used for R19 AM DRBs for this purpose to distinguish from the legacy configuration.

## 2.4  Impact of R19 RLC enhancements on PDCP SN gap report triggering

TS 38.323 For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. ….

In legacy R18, upon receiving a discard indication for a RLC SDU from PDCP, a RLC SDU is either fully submitted to lower layer, or fully discarded at RLC (when no segment is submitted to lower layer).

With R19 AM RLC enhancements (i.e. stopReTxObsoleteSDU), we can have a new scenario where RLC SDU may be incompletely submitted to lower layer. The obsolete SDUs (incompletely received SDUs, if any) are discarded at the receiving side of AM RLC entity after t-RxDiscard expiry. That is, the receiving PDCP entity may not receive the obsolete SDUs. However, the receiving PDCP entity also does not get to know about the discard signalling for such incompletely submitted SDUs in PDCP SN gap report. This causes the reoderering delays to creep in.

Therefore, with R19 AM RLC enhancements in place, we have an issue about the highlighted legacy phrase “the discarded PDCP SDU(s)” in the third condition as it does not address the new scenario. More precisely, for R19 specifications, the third condition should be enhanced to “at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers” while triggering a PDCP SN gap report.

The benefit is that the PDCP SN gap report can early and accurately notify more discard information to the peer receiving PDCP entity and it will prevent unnecessary re-ordering delay.

Proposal 4: RAN2 to discuss if R18 condition for triggering PDCP SN gap report “the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers” needs an update in context of R19 AM RLC enhancements (i.e. stopReTxObsoleteSDU).

Proposal 5: Based on P4, RAN2 to agree if in context of R19 AM RLC enhancements (i.e. stopReTxObsoleteSDU), condition for triggering PDCP SN gap report should be updated as: (Adopt TP3)

- at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers

# 3 Conclusion

Based on the discussion above, request RAN2 to consider the following proposals:

Proposal 1: Upon receiving ACK for an obsolete RLC SDU, the transmitting side of an AM RLC entity does not send an indication of successful delivery of the RLC SDU to the upper layers. (Adopt TP 1)

Proposal 2: Expiry of t-RxDiscard triggers a status report and status report is immediately sent to peer RLC entity.

Proposal 3: RAN2 agree to the following approach in order to address PDCP SN gap reporting for AM DRBs during mobility (Adopt TP 2):

- For AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, when a PDCP SN gap report was previously transmitted during mobility but its successful delivery has not been confirmed by lower layers, the transmitting PDCP entity triggers a PDCP SN gap report that includes the same set of previously discarded PDCP SDUs, apart from newly discarded PDCP SDUs, if any.

- A separate configuration “sn-GapReportMobility” is used for R19 AM DRBs for this purpose to distinguish from the legacy configuration.

Proposal 4: RAN2 to discuss if R18 condition for triggering PDCP SN gap report “the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers” needs an update in context of R19 AM RLC enhancements (i.e. stopReTxObsoleteSDU).

Proposal 5: Based on P4, RAN2 to agree if in context of R19 AM RLC enhancements (i.e. stopReTxObsoleteSDU), condition for triggering PDCP SN gap report should be updated as: (Adopt TP3)

- at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers

# 4 Annex: Text Proposals

TP 1

TS 38.322 ##### 5.2.3.1.1 General … When receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity shall: - send an indication to the upper layers of successful delivery of the RLC SDU (excluding RLC SDU for which the transmission and retransmission are stopped); - set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet. …

TP 2

TS 38.323 ### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. For AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReportMobility in TS 38.331 [3]), when a PDCP SN gap report was previously transmitted during mobility but its succesful delivery has not been confirmed by lower layers, the transmitting PDCP entity shall trigger a PDCP SN gap report that includes the same set of previously discarded PDCP SDUs, apart from newly discarded PDCP SDUs, if any. If a PDCP SN gap report is triggered for these discarded PDCP SDU(s), the transmitting PDCP entity shall: - compile a PDCP SN gap report as indicated below by: - setting the FDC field to the smallest COUNT value among the COUNT values associated with these discarded PDCP SDU(s); - if more than one PDCP SDU is discarded: - allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first of these discarded PDCP SDU(s) up to and including the last of these discarded PDCP SDU(s), rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first; - setting in the discard bitmap field as '1' for these discarded PDCP SDU(s); - setting in the discard bitmap field as '0' for all other PDCP SDU(s). - submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface. NOTE: It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

TP 3

TS 38.323 For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - if stopReTxObsoleteSDU is set to enabled for AM DRB and at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers; otherwise, the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. ….