3GPP TSG-RAN WG2 Meeting #129b	R2-2502863

Wuhan, China, April. 7th-11th, 2025

Agenda Item:	8.8.2

Source: 	CSCN

Title: 	Discussion on Downlink Coverage Enhancements

Document for: 	Discussion and Decision

Introduction

Regarding downlink coverage enhancements in NR NTN, some agreements and observations achieved, in RAN2#129 [1][2] shown as follows:

Agreements: - RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer - RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame - RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) - The number of SMTC/gaps a UE needs to consider at any time will not be increased further

From RAN2 perspective, we support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC. No LS is sent to RAN1. RAN2 observes that if the cells active simultaneously are in clusters, existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may be sufficient (FFS if any solution identified for option b applies to option a as well). RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

Furthermore, the discussion on cell barring cloud be continued in this meeting.

- Cell barring Proposal 6: It is up to NW implementation to set cellBarredNTN-r17 to barred to bar UEs not supporting R19 extended SSB periodicity. A new field cellBarredNTN-r19 (value is set to barred or notBarred) is introduced for future use. UEs supporting extended SSB periodicity ignore cellBarredNTN-r17 if the new cellBarredNTN-r19 is present, and follow cellBarredNTN-r17 if the new cellBarredNTN-r19 is absent. Continue in the next meeting

In this contribution, we discuss system level downlink coverage enhancements in NR NTN based on the agreements[1] and observations [2] of RAN2#129 meeting .

Discussion

2.1 SMTC impacts in RRC_IDLE and RRC_Inactive

In the discussion during the last meeting, it was observed from the RAN2 perspective that both option a and option b illumination mode received support [1]. From our point of view, the actual situation may encompass more than these two options, which depends on the implementation scope of network deployment. Certainly, we can still consider the scattered beam of option b as typical scenario for further analysis to address the issue of insufficient SMTC configuration.

According to the RAN plenary's conclusion, the maximum SSB periodicity in NTN is 160 ms. Additionally, based on the RAN2#129 chairnotes in DL-CE, it has consensus that there is beam sweeping within a cell and beam hopping between cells. Taking SET 1-2 as an example, which comprises 1058 footprints and features 16 simultaneously active beams. Each cell comprises 4 SSB beams, identified by the SSB index #0, #1, #2, and #3, as shown below.

Figure 1 SMTC configuration for different SSB timing offsets

In the case of scattered beam, it is obvious that , as RAN2’s offline report’s proposal, UEs in the serving cell need to monitor SSB bursts may occur in the different timing offsets (more than 4) within 160ms. So current SMTC need to be enhanced for UE measurement under the condition of intra-frequency.

Considering the conclusion of RAN2, the number of SMTC/gaps a UE needs to consider at any time will not be increased further. According to the current spec, the SMTC configuration in NTN is limited to a maximum of 4 (considering both UE capabilities and network configurations for SMTC1 and SMTC4). This means that the network can provide some assistant information to allow the UE to monitor the accurate SSB burst as much as possible.

Several potential solutions are presented as options 1-4 in the report for information [2].

Possible solutions (for information only): 1. Configure multiple SMTCs (more than 4) and SMTC selection (e.g. location-based SMTC selection, UE autonomous selection) 2. SSB-specific SMTC 3. UE assistance information reporting 4. Configure one SMTC with small periodicity, in each period (SMTC occasion) UE is indicated which PCI to use

As shown in figure 1, when cell level SMTCs configuration without SMTC selection, the number of SMTCs a UE needs to consider may be more than 4. We prefer utilizing SSB index associated SMTC configuration to reduce the number of SMTC, which including option A: SSB index associated SMTC configuration at the cell level and option B: SSB index associated SMTC configuration at the beam level (i.e. SSB-specific SMTC configuration).

Option A is SSB index associated SMTC configuration at the cell level. In addition to the SSB burst timing of the serving cell, the gNB also needs to configure the SSB burst timing of the neighboring cells corresponding to the SSB index of serving cell. Then UE can autonomously select SMTCs for cell reselection based on the detected SSB index. The benefit of Option A is that it can minimize the introduction of additional assistant information, as the UE will detect the SSB and calculate the SSB index anyway, which is a procedure that will definitely take place to the UE.

Proposal 1: SSB index associated SMTC configuration at the cell level can be introduced for SMTC enhancement.

Additionally, Option B is SSB index associated SMTC configuration at the beam level (i.e., SSB-specific SMTC mechanism).

Further analysis indicates that, broadcasting all the cell-level SSB associated SMTC configurations is actually a waste of resource, especially under the condition of limited satellite air interface resources and link budgets. Fundamentally, a gNB onboard is capable of acquiring global information regarding the entire cell. In fact, the network can roughly distinguish coverage areas by SSB index especially in the case of DL-CE cells with beam hopping, even in RRC_IDLE/INACTIVE mode. Furthermore, because of varying areas corresponding to different SSB beams, the NW can indicate separate SMTC configurations associated with each specific SSB index of serving cell (e.g. SSB 1 associated SMTC with ServingCell #0, NeighborCell #2, #6 and SSB 2 associated SMTC for ServingCell #0, NeighborCell #4, #5, #6). Considering the case of satellite beam hopping, the NW can transmit different SMTC configurations on separate beams. Therefore, in Option B, the network can further assist the UE to reduce the number of SMTC configurations, which is beneficial for the UE and NW as it can minimize the measurement process with less air interface resource.

Proposal 2: SSB-specific SMTC configuration can be introduced to beneficial both UE and NW for measurement.

Either UE movement or satellite movement may lead to changes in the SSB index detected by the UE. One possible scenario is that UE movement when UE is at the edge of serving SSB beam (e.g. footprint), or satellite beam sweeping over the cell (e.g. in EMC mode). Therefore, UE should reacquire the configuration to ensure the accuracy of UE measurement when UE detects the changes of SSB index.

Proposal 3: If SSB-specific SMTC applied, UE should reacquire the SMTC configuration when SSB index changes.

2.2 Gap impacts in RRC_CONNECTED

As described above, several companies proposed increasing the number of configured SMTCs to accommodate scenarios where different DL-CE cells adopt different SSB offsets. Similarly, the configuration of measurement gaps is also impacted as shown in Fig.2, taking two-frame offset between the SSBs of different cells as an example, the GapConfig IE (e.g., mgrp, mgl, gapOffset) cannot sufficiently characterize the properties of such DL-CE cells.

In the current spec, a maximum of two GapConfigs can be configured per measurement object (requiring UE capability parallelMeasurementGap). However, this is insufficient fully to configure the DL-CE cells. As shown in Fig.2, since UE A is currently operating in the cell edge of overlapping coverage areas from multiple cells served by Target gNB-2, the Source gNB-1 needs to configure measurement gap for three neighboring cells (Cell #2, #4, #5) to UE A. Nevertheless, ‌as the SSBs of these neighboring cells belong to three different SSB bursts‌, the MeasGapConfig cannot be accurately configured in this scenario.

There may exist two particular configurations as outlined below.

‌Configuration 1‌: If the MeasObjectNR(with ssbFrequency 2) of Target gNB-2 is associated with two MeasGapIds linked to two GapConfigs (both with MGRP = 160 ms), one target neighboring cell would be missing (shown in red block). If the UE moves towards this missing cell, it will result in a call drop.

‌Configuration 2‌: If the MeasObjectNR(with ssbFrequency 2) of Target gNB-2 is associated with a single GapConfig (MGRP = 20 ms), five invalid gaps would be configured unnecessarily, causing unexpected interruption on serving cell (shown in red block).

Figure 2 Legacy MeasCapConfig for DL-CE cell

Observation 1: Due to the SSB periodicity extension and beam hopping, gap configuration may be insufficient for DL-CE cells.

The implementation of gap not only disrupts data transmission continuity in the serving cell but also introduces scheduling complexity at the gNB. Furthermore, SSB beams misalignment with the UE offers no benefits for mobility.

For example, under optimal conditions, the gNB should configure dedicated measurement gaps individually for UE A, B, and C as illustrated in Fig. 3. However, improving measurement gap accuracy presents particular challenges for DL-CE cells due to SSB timing offsets between different cells and the gNB’s limited awareness of UE-detectable neighboring cells at the UE’s current location. Under the precondition that the gNB cannot obtain the UE location, inaccurate configuration of gap may occur. On the contrary, frequent UE location updates and reporting would introduce significant signalling overhead and potential privacy risks.

Figure 3 different UE with different detectable SSB

Therefore, a large proportion of configured gaps being ineffective should be avoided in practical applications. To effectively minimize and accurately configure the gap while avoiding invalid configurations, the gNB shall implement different configurations for UEs in different areas.

Proposal 4: RAN2 is recommended to discuss the method for accurate gap configuration for the DL-CE cells.

Proposal 5: The gNB shall implement different configurations for UEs in different areas to avoid invalid configurations.

2.3 Access restriction

In RAN plenary #106 meeting, the maximum of the additional SSB periodicity default value (apart from the existing 20 ms value) in NTN is 160 ms. From RAN2 perspective, it has not been determined yet whether to continue using the existing barring bit mechanism. We hold the option that, taking into account other potential enhancements in DL-CE such as PDCCH repetition for SIB1 and SIB1/Msg4 PDSCH repetition at link level and SMTC enhancement at system level, it is necessary to implement selectively bar UE capable DL-CE.

We think one bit with the value {barred, notBarred} is sufficient to achieve this. More specifically, for UEs supporting DL-CE, UE shall follow the legacy mechanism if the new barring bit is not present, UE shall consider the cell is not allowed for access if the new barring bit is set to "barred" and allowed for access if the new barring bit is set to "not Barred".

Proposal 6: A new NTN bar bit with the value {barred, notBarred} should be introduced to achieve selectively bar UEs supporting DL-CE.

2.4 Cell/beam level DTX/DRX

Considering that satellite service area may consist of more than one hundred satellite beams (e.g.106 satellite beams in Set 1-1), only a portion of these beams can be activated at any given time due to limited power and the nominal EIRP density per satellite beam. In order to enhance downlink coverage, beam hopping is an efficient technology that allows dynamic and flexible power sharing between satellite beams. Since it is possible that some satellite beams within a cell are active while others remain inactive at the same time based on UE traffic requirements and network implementation, beam level DTX should also be supported to enable more flexible power sharing between satellite beams.

During the beam inactive period, there is no Tx signalling for this beam. In order to save UE's power, the UE should not do any operations in this state, which is applied to RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs. More specifically, for UEs in RRC_IDLE/RRC_INACTIVE.UEs should not initial RRC connection establishment/RRC connection resume procedure, monitor paging, or perform serving cell measurements during the beam inactive period For UEs in RRC_CONNECTED, UEs should not perform serving cell measurements or detect radio link failure during such period.

Proposal 7: RAN2 may discuss on supporting beam level DTX and the UE behaviors in different period.

Conclusions

Based on above analyses,observations and proposals are summarized as follows.

Proposal 1: SSB index associated SMTC configuration at the cell level can be introduced for SMTC enhancement.

Proposal 2: SSB-specific SMTC configuration can be introduced to beneficial both UE and NW for measurement.

Proposal 3: If SSB-specific SMTC applied, UE should reacquire the SMTC configuration when SSB index changes.

Observation 1: Due to the SSB periodicity extension and beam hopping, gap configuration may be insufficient for DL-CE cells.

Proposal 4: RAN2 is recommended to discuss the method for accurate gap configuration for DL-CE cells.

Proposal 5: The gNB shall implement different configurations for UEs in different areas to avoid invalid configurations.

Proposal 6: A new NTN bar bit with the value {barred, notBarred} should be introduced to achieve selectively bar UEs supporting DL-CE.

Proposal 7: RAN2 may discuss on supporting beam level DTX and the UE behaviors in different period.

Reference

- Report of 3GPP TSG RAN WG2 meeting #129

- R2-2501415, Report of [AT129][304][R19 NR NTN] LS to RAN1 on DL coverage enhancements