3GPP TSG-RAN WG2 Meeting #129bis	                                 R2-2502866

Wuhan, China, Apr. 7th – 11th , 2025

Source:        ZTE Corporation

Title:           Consideration on the Remaining Issues of PL Offset MAC CE

Agenda item:   8.12.2

Document for:  Discussion and Decision

- Introduction

In the post email discussion [Post129][206][MIMO_Ph5] Running CR for TS 38.321 (Samsung), the MAC CE for the Path loss offset has been well designed based on the following agreements:

One PL offset value is indicated for each TCI state included in the new MAC CE. The new MAC CE contains one serving cell ID and one BWP ID TCI state ID is used to indicate a TCI state in the new MAC CE (i.e., no bitmap for TCI states is needed) The new MAC CE can include flexible number of PL offset values.

Meanwhile a working assumption and a FFS issue were also left:

- Working assumption: UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found. - Agreement RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

In this paper we would like to share our considerations on the remaining issues on this topic.

- Discussion

In this chapter, we first discuss more detail issues on the PL offset MAC CE, then focus the 2TA impact on RAN2 spec, at last we share our views on the FFS issue of the PHR trigger condition

2.1 PL Offset MAC CE

In this chapter, we’d like to discuss some details issues for the PL offset update MAC CE.

Issue 1: How to capture the working assumption on the Relationship between the RRC parameter and the MAC CE

In the last meeting, there is a working assumption that “UE applies the latest PL offset value received in RRC or MAC CE.” However this has not been captured in the running CRs, the question is whether it should be captured in the RAN1 spec or the RAN2 spec. This parameter would be used at the Physical layer, thus it’s better to indicate this working assumption to the RAN1, and confirm with RAN1 whether this would be captured in RAN1 spec.

Proposal 1: RAN2 to discuss how to capture the working assumption that the “UE applies the latest PL offset value received in RRC or MAC CE.”

Proposal 1a: RAN2 to indicate the working assumption that the “UE applies the latest PL offset value received in RRC or MAC CE.” to RAN1 and to confirm with RAN1 whether this would be captured in RAN1 spec.

Issue 2: Applied Channel Types

This issue is about the applied channel type, besides the PUCCH/PUSCH/SRS, it has also been agreed in RAN1 that the PL offset would be used for the PDCCH-order PRACH. Thus it’s easy to get the proposal 4.

R1#119 Agreement Support to apply PL offset on PDCCH-order PRACH in FR2 - The design for applying PL offset on PDCCH-order PRACH in FR1 is fully reused here. Note: there is no extra enhancement for Tx beam determination for PDCCH-order PRACH in FR2 in Rel-19

Proposal 2: The TCI states in the MAC CE are applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

Issue 3: Whether the PL offset update MAC CE is applied to a single serving cell or multiple serving cells

In the R17/18, the serving cell set mechanism has been supported with simultaneousSpatial-UpdatedList1 or simultaneousSpatial-UpdatedList2. Thus this mechanism can also be kept for the PL offset update MAC CE. However if it requires to introduce new simultaneousSpatial-UpdatedList, it would cause some specification complexity. Furthermore, the path loss would be quite different from different cells, thus no need to introduce this mechanism to the Path loss offset MAC CE.

Proposal 3: The PL offset update MAC CE is applied to a single serving cell instead of the serving cell set.

2.2 About 2 TA

On 2 TA, as described in the RAN1 LS [1], many agreements have been achieved.

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped. “RAN2 could check whether this RAN1 agreement would cause any MAC impact”

In RAN2 spec, the restriction on coresetPoolIndex for the 2TA feature is mainly included in the RRC spec,e.g.

tag2 This field to indicate the second TAG information for the serving cell, it is optionally configured in a serving cell if and only if the serving cell is configured with more than one value for the coresetPoolIndex.

Considering that it would not cause too much specification work and this topic is quite separate from the other topics, this can be done in the RAN2 running CR review stage.

Proposal 4: On 2TA with single DCI case, RAN2 would remove the restriction that coresetPoolIndex needs to be configured in the RRC running CR.

2.3 About PHR Trigger Condition

In the last meeting, a FFS was also left as following:

- Agreement RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

According to the latest RAN1 running CR [1], it has been clearly clarified that the pl-Offset would be used for the uplink power control related descriptions except the Type3 PHR (in the clause 7.7.3) for that there is no consensus on whether the pl-Offset should be used for the Type3 PHR.

7.1 Uplink Power control In the remaining of this clause, except for clause 7.7.3, if in a PUSCH, PUCCH, or SRS transmission occasion i a UE applies a TCI-State or a TCI-UL-State that includes pl-Offset with value PLb,f,c,offset in dB [11, TS 38.321], the UE sets PLb,f,cqd=PLb,f,cqd-PLb,f,c,offset.

Thus, there is no need to add further clarification in RAN2 spec.

Proposal 5: No need to capture the PL offset impact on the PHR trigger condition in RAN2 spec.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: RAN2 to discuss how to capture the working assumption that the “UE applies the latest PL offset value received in RRC or MAC CE.”

Proposal 1a: RAN2 to indicate the working assumption that the “UE applies the latest PL offset value received in RRC or MAC CE.” to RAN1 and to confirm with RAN1 whether this would be captured in RAN1 spec.

Proposal 2: The TCI states in the MAC CE are applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

Proposal 3: The PL offset update MAC CE is applied to a single serving cell instead of the serving cell set.

Proposal 4: On 2TA with single DCI case, RAN2 would remove the restriction that coresetPoolIndex needs to be configured in the RRC running CR.

Proposal 5: No need to capture the PL offset impact on the PHR trigger condition in RAN2 spec.

- Reference

- R1-2501661  Introduction of NR MIMO Phase 5  38.213   Samsung