3GPP TSG-RAN WG2 Meeting #129bis 	   R2-2502872

# Wuhan, China, 7th – 11th April, 2025

Source:	TCL

Title:	Remaining issues on timely RLC retransmission

Agenda item:	8.7.5

Document for:	Discussion and decision

- Introduction

In RAN2 #129 meeting, some agreements on RLC timely retransmissions has been reached as follows:

Agreements on RLC timely retransmissions Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC.

In this contribution we will further discuss the FFS and other remaining issues for timely RLC retransmission.

- Discussion

  - New RLC timer or reusing discardTimer

As mentioned in last meeting by many companies, since there would be too many cross-layer indication from PDCP layer to RLC layer, we think introducing a timer in the RLC transmission entity is simpler and clearer to specify the triggering operation for autonomous retransmission and/or polling in RLC specification.

Proposal 1: It is suggested RAN2 to introduce a new timer in the RLC transmission entity for each SDU to trigger autonomous retransmission and enhanced polling.

  - Avoiding unnecessary polling

For the enhanced polling mechanism, it can be triggered based on the remaining time of an SDU, but its effect is not limited to that SDU, as the corresponding status report will reflect the acknowledgment status of other SDUs. Therefore, it is not necessary for every SDU to trigger enhanced polling. For example, if the remaining times of two SDUs are very close, and the first SDU has already triggered enhanced polling, there is no need for the second SDU to trigger enhanced polling; otherwise, it may lead to excessively frequent polling.

Moreover, after introducing the enhanced polling mechanism, there will be two types of polling mechanisms in the system, legacy polling and enhanced polling. Considering that the functionalities of these two polling mechanisms are very similar, or even identical, it is also necessary to avoid triggering unnecessary polling operations between legacy polling and enhanced polling.

To avoid unnecessary polling, one possible solution is to implement a common polling suppression mechanism for both legacy polling and enhanced polling. Specifically, once a polling operation is triggered, a polling suppression time period will be set. During this time period, even if the triggering conditions are met, polling will not be activated, including both legacy polling and enhanced polling.

Proposal 2: It is suggested RAN2 to introduce a polling suppression mechanism for both legacy and enhanced polling to avoid unnecessary polling.

- Conclusion

In this contribution, we discussed the FFS and other remaining issues for timely RLC retransmission and provided our proposals as follows:

Proposal 1: It is suggested RAN2 to introduce a new timer in the RLC transmission entity for each SDU to trigger autonomous retransmission and enhanced polling.

Proposal 2: It is suggested RAN2 to introduce a polling suppression mechanism for both legacy and enhanced polling to avoid unnecessary polling.