3GPP TSG RAN WG2 #129bis	                                           R2-2502874

Wuhan, China, April 07th – 11th , 2025

Source:	CEWiT

Title:	Discussion on Ambient IoT Paging

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

In RAN2#129 [1] meeting, the following agreements have been made on paging aspects related to A-IoT.

- Agreements - Parallel service requests by the same reader is not supported. - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed). - The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it. - The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2. - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer. - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field - The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

In this contribution, we discuss the contents, monitoring and other aspect for paging message for A-IoT.

- Discussion

2.1 Contents of A-IoT paging message

Upon receiving a request, the reader initiates communication by sending an A-IoT paging message that signals the device(s) required to respond. This paging message includes the identifiers of the targeted devices, which are crucial for ensuring that only the intended A-IoT devices are alerted and can respond accordingly. In addition to the device identifiers, the paging message can carry supplementary information to enhance the communication process. This may include the resource configurations for transmission of MSG1/MSG3, Furthermore, the message may offer guidance on generating a random number (Q) or a random device identifier, which is essential for avoiding collisions during the RACH process and establishing unique identifiers for communication.

This structured approach ensures that the A-IoT device can efficiently handle service requests while minimizing communication delays and optimizing resource usage.

Proposal 1: Support the additional following contents for paging message :

- Information related to random Number (Q) or random device ID Generation

- The number of time domain resources and the number of the frequency domain resources for MSG1 transmission

When the paging message includes the IDs of multiple devices, it must also specify the transmission resources for MSG1 for each device. Once a device decodes the paging message, it needs to identify which specific resource to use among those provided. Therefore, a clear association between each device ID and its corresponding resource must be included in the message.

Proposal 2: RAN 2 to support including mapping association indication between D2R resources and device IDs in the paging message.

The paging message includes a single identifier, as agreed upon in the RAN2#129 [1] meeting. This identifier is used to represent both the device ID and the group ID. Since the group ID and device ID can have variable lengths, the A-IoT device must be able to differentiate between them within the given identifier.

Proposal 3: RAN2 to support to discuss on how to distinguish between group ID and device ID.

2.2 Repeated paging request

It has been established that an A-IoT device is expected to perform only one procedure at a time. It would be beneficial to clarify that the device can have only one active transaction ID at any given time. This ensures that the device does not need to store multiple transaction IDs in its memory, simplifying its operation.

Proposal 4: Support to maintain the only single active transaction ID during one procedure.

2.3 Transaction ID

The Transaction ID serves as a unique identifier that helps the device associate an incoming paging message with a specific service request originating from the Core Network (CN) or Application Function (AF). This ensures that the device can correctly process the request without confusion.

During the RAN2#129[1] meeting, it was agreed that an A-IoT device is designed to handle only one procedure at a time to maintain simplicity and efficiency. A 1-bit indicator, similar to a New Data Indicator (NDI)-type mechanism, is sufficient to distinguish between an ongoing procedure and the start of a new one.

With this approach, when a new paging message arrives, the 1-bit indicator helps determine whether the message belongs to an existing transaction or if it initiates a fresh procedure. This eliminates the need for additional storage and processing complexity, keeping the device operation streamlined and cost-effective.

Proposal 5: Support 1-bit indicator in transaction ID to distinguish between the existing or the new procedure.

2.4 Determination of resources for random access

RAN2 has agreed that slotted-ALOHA is the baseline for A-IoT random access procedure. In this process, each A-IoT device randomly selects one of the time-frequency resources to transmit its A-IoT MSG1. In the RAN2 #129 [1] meeting, it is agreed that paging message can include the total number of resources for MSG1. For A-IoT random access, to further select the resources for the MSG1 transmission a separate R2D trigger message can be used after the paging message. This ensures that the A-IoT device is aware of the resources available for transmitting its initial message. As shown in fig. 1, a predefined time window, referred to as the RACH time window can be used for A-IoT device to select the time resources randomly from this window. This RACH window allows multiple devices to backscatter their responses. The RACH window represents the duration allocated for Msg1 transmissions, during which multiple devices can transmit at different time instances. In the RACH window different occasions for MSG1 transmission are available. The R2D trigger message can be used to indicate the start of the RACH window. The time offset between the reception of R2D trigger message and the start of RACH window is predefined and called as TR2D_min. Similar to RFID, the A-IoT device can use similar approach i.e. random number based on Q value approach to further chose one time resource form the RACH window for transmitting the MSG1. In this case, the A-IoT device generates a random number based on Q value, loads it to a timer, start decrementing the timer and transmits the MSG1 at the expiry of the timer. The Q value can be configured through the paging message.

Fig.1 TDMA of Msg1 Transmission

Proposal 6: For TDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access:

- Based on the slotted aloha in a predefined RACH time window.

Proposal 7: R2D trigger used to determine the start of the RACH time window for transmission of MSG1 is supported.

- The time duration between the reception of R2D trigger and the start of RACH time window is indicated by the TR2D_min.

Proposal 8: Support to generate the random number based on Q value configured in paging message by the A-IoT device for transmission of MSG1 in the predefine RACH time window.

- Conclusion

In this contribution, we provide our views on aspects of paging message for Ambient IoT. The proposals are summarized as follows.

Proposal 1: Support the additional following contents for paging message :

- Information related to random Number (Q) or random device ID Generation

- The number of time domain resources and the number of the frequency domain resources for MSG1 transmission

Proposal 2: RAN 2 to support including mapping association indication between D2R resources and device IDs in the paging message.

Proposal 3: RAN2 to support to discuss on how to distinguish between group ID and device ID.

Proposal 4: Support to maintain the only single active transaction ID during one procedure.

Proposal 5: Support 1-bit indicator in transaction ID to distinguish between the existing or the new procedure.

Proposal 6: For TDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access:

- Based on the slotted aloha in a predefined RACH time window.

Proposal 7: R2D trigger used to determine the start of the RACH time window for transmission of MSG1 is supported.

- The time duration between the reception of R2D trigger and the start of RACH time window is indicated by the TR2D_min.

Proposal 8: Support to generate the random number based on Q value configured in paging message by the A-IoT device for transmission of MSG1 in the predefine RACH time window.

# References

[1]   RAN2#129 Chair Notes.