3GPP TSG RAN WG2 #129-bis                                	R2-2502876

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.5.3

Source: 	Sharp

Title: 	Discussion on remaining issues of on-demand SIB1

Document for:	Discussion

Introduction

The Rel-19 WID on enhancements to NES for NR was approved in [1], which includes the following objectives:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

At RAN2#129 meeting [6], it was agreed that UEs in RRC_CONNECTED state could acquire OD-SIB1 when T311 is running:

Agreements on OD-SIB1 1. There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1. 2. Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running: - When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid. - The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition. - The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs. 3. The UE follows the legacy validity principle of stored SIB. 4. For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected. 5. SIB-x can be cell specific or area specific, as legacy. 6. Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

In this contribution, we will provide our views on the remaining details of on-demand SIB1 (OD-SIB1) procedure after a radio link failure event.

Discussions

OD-SIB1 request procedure after radio link failure

Based on R-19 NES meeting progress and current 3GPP specification [8], a UE may firstly receive and store a UL-WUS configuration from the source cell. The source cell is also the cell A of the UE while the UE receives the UL-WUS configuration of neighbor cells from the source cell. So, the UE would store a valid UL-WUS configuration while the UE is staying in RRC connected state. Then, the UE detects a radio link failure condition is met at the UE side. Based on [8], the UE would start the timer T311 when the UE initiates a cell selection procedure and the UE would stop the timer T311 upon the UE selects a suitable cell for RRCReestablishmentRequest message transmission via a random access procedure. However, in order to support OD-SIB1 request, now the UE may select an OD-SIB1 cell while T311 is running and so additional conditions should be considered in Release-19.

Fig. 1 Signaling flow of UL-WUS configuration, OD-SIB1 request procedure, and RRC re-establishment procedure

Firstly, after a RLF condition is considered in the UE side, the UE would start a timer T311 and initiate a cell selection procedure. Also based on [8], the running T311 would be stopped after the UE selects a suitable cell for which the UE has valid and up-to-date essential system information.

5.3.7.3 Actions following cell selection while T311 is running Upon selecting a suitable NR cell, the UE shall: 1> ensure having valid and up to date essential system information as specified in clause 5.2.2.2; 1> stop timer T311;

However, OD-SIB1 request may inevitably need to support RAN sharing scenario. In addition, the network identity list supported by the selected cell, which is also part of the essential system information, is included in its SIB1. Therefore, it may not be possible for the UE to decide the selected OD-SIB1 cell is suitable cell or not if the UE does not know the network(s) supported by the target cell. If OD-SIB1 request procedure is supported during RAN sharing scnario, it is suggested that the UL-WUS configuration of one OD-SIB1 cell can further include the information of the network(s) supported by the OD-SIB1 cell. By doing so, RAN2 WG can guarantte the OD-SIB1 request procedure would not increase the failure rate of RRC re-establishment procedure.

- OD-SIB1 request may support RAN sharing scenario.

UL-WUS configuration should include the network identities supported by OD-SIB1 cells if OD-SIB1 request supports RAN sharing.

Furthermore, based on the RAN2#129 agreement, when T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid. In this meeting, RAN2 WG is suggested to decide when should the UE stop the running timer T311 after the UE initiating a OD-SIB1 request procedure towards a target cell.

RAN2 WG is suggested to decide the stop timing of timer T311 after a OD-SIB1 request procedure is triggered by the UE:

- Option#1: T311 is stopped upon the UE receives the RAR message from the target cell.

- Option#2: T311 is stopped upon the UE receives the requested SIB1 from the target cell.

In addition, RAN2 WG is suggested to decide the OD-SIB1 failure condition upon T311 expiry. Two options

are provided:

Option#1: UE moves to RRC_IDLE state if the UE fails to obtain the RAR message from the target cell

upon T311 expiry.

Option#2: UE moves to RRC_IDLE state if the UE fails to obtain the SIB1 from the target cell upon T311

expiry.

RAN2 WG is suggested to decide the OD-SIB1 request failure condition while the timer T311 expires.

- a) Option#1: UE moves to RRC_IDLE state if the UE fails to obtain the RAR message from the target cell upon T311 expiry.

- b) Option#2: UE moves to RRC_IDLE state if the UE fails to obtain the SIB1 from the target cell upon T311 expiry.

Since the stop condition of timer T311 during a triggered OD-SIB1 request procedure is not clear, it is possible the timer T311 may still count before the UE having valid and up to date essential system information from a selected cell as specified in TS 38.331. Due to the wide range of among {1000ms, 3000ms, 5000ms, 10000ms, 15000ms, 20000ms, 30000ms}, a UE may have the chance to initiate a second cell selection & OD-SIB1 request procedure towards a target cell#2 after  a fail first cell selection & OD-SIB1 request procedure with a cell#1 (e.g., after MAC indicates max number of preamble transmission for the OD-SIB1 request with the cell#1 or the UE fails to receive SIB1 after MSG2 (ACK) is received from the cell#1).

RLF-TimersAndConstants information element

-- ASN1START

-- TAG-RLF-TIMERSANDCONSTANTS-START

RLF-TimersAndConstants ::=          SEQUENCE {

t310                                ENUMERATED {ms0, ms50, ms100, ms200, ms500, ms1000, ms2000, ms4000, ms6000},

n310                                ENUMERATED {n1, n2, n3, n4, n6, n8, n10, n20},

n311                                ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10},

...,

[[

t311                                ENUMERATED {ms1000, ms3000, ms5000, ms10000, ms15000, ms20000, ms30000}

]]

}

-- TAG-RLF-TIMERSANDCONSTANTS-STOP

-- ASN1STOP

- UE may be able to initiate one or more OD-SIB1 request procedures with different       cells while T311 is counting.

Based on this observation, we suggest RAN2 to confirm the Proposal 3:

RAN2 WG is suggested to confirm whether one UE is enabled to initiate another cell selection and OD-SIB1 request procedure towards a second cell after the OD-SIB1 request procedure towards a first cell has failed and the timer T311 is counting.

Backhaul support

During the RRC re-establishment procedure, UE context may to be moved between different radio nodes if the source cell and the selected cell are configured by different nodes, to maintain the RRC connection between the UE and the serving RAN. However, since the UE needs to obtain the SIB1 of the target cell before the UE initiates a RRC re-establishment request procedure, additional latency caused by the OD-SIB1 request procedure should be included.

- Processing latency in air interface and backhaul may not be ignorable for UE  context retrieval if OD-SIB1 request is implemented before the RRC re-establishment procedure.

Therefore, to a radio access network, it is possible that the OD-SIB1 request may be supported between the serving radio node of the UE (which supports the source cell) and a subset of neighbor radio nodes (which support some neighbor cells around the UE) but not supported between the serving radio node and other neighbor radio nodes. RAN2 WG is suggested to confirm whether OD-SIB1 request can be supported between every radio node to maintain an OD-SIB1 capable UE’s RRC connection. Since this issue belongs the scope of RAN3 WG, a liaison to RAN3 WG may need to be considered.

RAN2 WG is suggested to confirm whether all of the radio nodes in RAN should support OD-SIB1 request procedure after RLF. A liaison to RAN3 WG may be needed.

If not all of the radio nodes in RAN would support OD-SIB1 request procdure after RLF, it is possible that only a subset OD-SIB1 cells in a UL-WUS configuration support OD-SIB1 request after RLF. If so, UL-WUS configuration should include additional flag/indicator associated with OD-SIB1 cell(s) to indicate whether OD-SIB1 request after RLF is supported or not.

Indicator/flag is suggested to be configured with OD-SIB1 cell(s) in UL-WUS configuration to indicate whether OD-SIB1 request after RLF is supported by the OD-SIB1 cell(s).

Conclusions

Based on the discussion, we have the following observations and proposals:

Firstly, by considering OD-SIB1 request may support RAN sharing scenario, we propose UL-WUS configuration should include the network identities supported by OD-SIB1 cells.

- OD-SIB1 request may support RAN sharing scenario.

- UL-WUS configuration should include the network identities supported by OD-SIB1 cells if OD-SIB1 request supports RAN sharing.

Secondly, RAN2 WG is suggest to discuss the stop condition of T311 while OD-SIB1 reqeust after RLF is triggered.

- RAN2 WG is suggested to decide the stop timing of timer T311 after a OD-SIB1 request procedure is triggered by the UE:

- a) Option#1: T311 is stopped upon the UE receives the RAR message from the target cell.

- b) Option#2: T311 is stopped upon the UE receives the requested SIB1 from the target cell.

In addition, RAN2 WG is suggested to decide the OD-SIB1 failure condition upon T311 expiry.

- RAN2 WG is suggested to decide the OD-SIB1 request failure condition while the timer T311 expires.

- a) Option#1: UE moves to RRC_IDLE state if the UE fails to obtain the RAR message from the target cell upon T311 expiry.

- b) Option#2: UE moves to RRC_IDLE state if the UE fails to obtain the SIB1 from the target cell upon T311 expiry.

Due to the stop condition of T311 for OD-SIB1 request after RLF is not clear, UE may be able to initiate one or more OD-SIB1 request procedures with different  cells while T311 is counting.

- UE may be able to initiate one or more OD-SIB1 request procedures with different         cells while T311 is counting.

- RAN2 WG is suggested to confirm whether one UE is enabled to initiate another cell selection and OD-SIB1 request procedure towards a second cell after the OD-SIB1 request procedure towards a first cell has failed and the timer T311 is counting.

Finally, RAN2 WG is suggested to confirm that whether all of the OD-SIB1 cells configured in one UL-WUS configuration would support OD-SIB1 request after RLF. If not, indicator/flag to indicate whether OD-SIB1 request after RLF is supported to the OD-SIB1 cells should be included in UL-WUS configuration.

- Processing latency in air interface and backhaul may not be ignorable for UE     context retrieval if OD-SIB1 request is implemented before the RRC re-establishment procedure.

- RAN2 WG is suggested to confirm whether all of the radio nodes in RAN should support OD-SIB1 request procedure after RLF. A liaison to RAN3 WG may be needed.

- Indicator/flag is suggested to be configured with OD-SIB1 cell(s) in UL-WUS configuration to indicate whether OD-SIB1 request after RLF is supported by the OD-SIB1 cell(s).

References

RP-242354, Revised WID: Enhancements of network energy savings for NR.

Chair’s notes RAN2#126, May 20th – May 24th, 2024

Chair’s notes RAN2#127, August 19th – August 23rd, 2024.

Chair’s notes RAN2#127bis, October 14th – October 18th, 2024.

Chair’s notes RAN2#128, November 14th – November 18th, 2024.

Chair’s notes RAN2#129, February, 17th – February 21th, 2024.

RP-243297, “Way forward on NES”

3GPP TS 38.331 V18.4.0 (2024-12-20)