3GPP TSG-RAN WG2 Meeting #129bis	R2-2502878

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.6.4

Source:	Nokia

Title:	On Conditional LTM

WID/SID:	NR_Mob_Ph4-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#129 the following was agreed for Conditional LTM:

Agreements on C-LTM:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

The primary motivation for CLTM is to introduce robustness to the LTM procedure where the UE may not be able to receive the cell-switch command at right radio condition which may lead to radio link failure. Compared to CHO, the conditional LTM is expected to bring the benefits of robustness along with the interruption time reduction benefits of LTM. In this contribution, we discuss the remaining issues related to early synchronization procedures, cell switch execution and co-existence with CHO. .

# 2	Discussion

We identify the following key aspects to be discussed in RAN2.

- Early synchronization

- Conditional cell switch execution

- Co-existence with CHO

## Early synchronization

### RACH-based early TA acquisition

In the last meeting, it was agreed to provide the TA information to the UE from the source using a new MAC CE:

RAN2#128 agreements: - The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. - The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM

The content of the MAC CE is not agreed yet. Before discussing the content of the MAC CE, a few considerations on the maintenance of the TA value it transfers should be made. After a TA value has been obtained, it must be ensured that at the time of the conditional cell switch, the TA value is valid. Regarding maintenance of the TA provided to the UE with the new MAC CE, in RAN2#128, the following agreement was made:

RAN2#128 agreement: Candidate cell TA is maintained by a new timer.

From the above agreements, it is evident that the UE may have to store and maintain multiple TAs for supporting RACH-less CLTM. It is still not clear how the timer should be handled after the CLTM cell switch. RAN2 shall discuss how UE shall handle CLTM TA timer(s) during cell switch execution. At least the following options can be considered:

- CLTM TAT is stopped immediately, and UE autonomously starts legacy PTAG TAT based on new serving cell’s configuration

- CLTM TAT is stopped immediately, and UE autonomously starts legacy PTAG TAT for amount of time remaining to CLTM TAT expiry

- CLTM TAT is left running either until its expiration or until UE receives PTAG TAC from new serving cell

Observation 1: The following options are possible for handling CLTM TA timer(s) during cell switch execution.

- CLTM TAT is stopped immediately, and UE autonomously starts legacy PTAG TAT based on new serving cell’s configuration

- CLTM TAT is stopped immediately, and UE autonomously starts legacy PTAG TAT for amount of time remaining to CLTM TAT expiry

- CLTM TAT is left running either until its expiration or until UE receives PTAG TAC from new serving cell

From our perspective, start of legacy PTAG TAT without being triggered by new serving cell’s TAC is not acceptable since it artificially enhances TA validity. From UEs perspective, second option is equivalent, in our view, to the third one since UE, in fact, leaves CLTM running after cell switch. The only difference is such that, starting from the cell switch, PTAG TAT is running.

Observation 2: first of above-listed options allows UE to assume that new serving cell TA can be valid for longer than it was configured by the NW (CLTM TAT value). From UE perspective, the remaining two options are equivalent in the sense that UE assumes CLTM TA value is valid for the same amount of time.

Proposal 1: if CLTM TAT is running, UE shall assume that TA is still valid. Once it expires, UE shall assume that TA is not valid anymore. RAN2 to discuss the best WF for handling the TA value after CLTM cell switch execution.

Proposal 2: If TA is valid, UE shall be allowed to use CG resources. When TA becomes invalid, UE shall be forbidden to use (or to start using) CG resources

Proposal 3: When TA becomes invalid before successful cell switch completion, UE shall fall back to RACH-based cell switch.

### UE-based TA estimation

An alternative way for the UE to obtain the TA required for RACH-less cell switch is UE-based TA estimation. In Rel-18 LTM, if the UE is configured with Rel. 18 LTM and the TA in MAC CE is given as FFF, the UE may execute RACH-less LTM with its own measured TA. In RAN2#127bis, it was agreed to support UE-based TA measurement also for CLTM:

RAN2#127bis agreement: UE based TA measurement mechanism is supported for conditional intra-CU LTM.

As discussed in Section 2.3.1, in RAN2#128, it was agreed that the NW can provide the TA information of a candidate cell to the UE via a new MAC CE. In case the UE is configured with UE based TA; the network may trigger the RACH-based early UL synchronization procedure based on the PDCCH order for given candidate cell(s). This TA acquired from the candidate cell is to be shared to the UE by the network over the proposed new MAC CE as agreed above.

Proposal 4: In case CLTM is configured along with UE based TA estimation for given candidate cell(s), RAN2 to confirm that the TA information, which is obtained over the new MAC CE for the same candidate cell(s), if valid will get precedence over UE estimated TA.

For the maintenance of a TA associated with a serving cell, a TA validity timer (TAT) is configured over RRC and maintained at the UE. As discussed in Sec. 2.3.1, in RAN2#128 it was agreed that candidate cell TA will also by maintained by a new timer. Upon a conditional cell switch being triggered to a candidate, if the TA maintenance timer of the NW-given TA associated to that candidate has expired and the UE is not configured with UE-based TA estimation for that candidate, the UE will have no choice but to fall back to RACH-based CLTM cell switch execution. However, if the UE is configured with UE-based TA estimation, in case the TA value received via the new MAC CE has expired or it was set as ‘FFF’, or no TA-carrying MAC CE has been received, the UE can use the UE-estimated TA to perform RACH-less cell switch execution. Similarly, the TA value received at the UE over the new MAC CE is set as “FFF” or in case if no new MAC CE is received at the UE, the UE can use the UE estimated TA.

Proposal 5: In case of CLTM, if the TA provided by the network over the new MAC CE or any network entity becomes invalid, or set as “FFF”, or no new MAC CE received, if UE is configured with UE based TA estimation and UE has successfully measured the TA, UE can perform RACH-less cell switch.

Another aspect that requires further discussion in RAN2 is the inter-working between CLTM and NW-triggered LTM. While the conditions for CLTM cell switch execution are being evaluated at the UE or before CLTM cell switch is initiated, the network may decide to trigger cell switch to a given LTM prepared candidate cell and send a MAC CE cell switch command (CSC MAC CE). In that case, UE will execute the CSC MAC CE obtained from the network and trigger the LTM cell switch. However, if the TA value in the CSC MAC CE is set as “FFF”, there can be two possible scenarios. Firstly, UE might already acquire the early TA over CLTM MAC CE whose TAT is still running (early TA is valid). Secondly, UE could have been configured with UE based TA during the preparation phase and it estimated the TA which is which is valid. In this situation, UE must prioritize the network provided TA it received over the new CLTM MAC CE to perform a RACH-less cell switch execution.

Proposal 6: Upon reception of CSC with TA = FFF, UE checks if a valid TA obtained through CLTM TAC MAC CE is available. If no TA is available and the UE is configured with UE-measured TA, UE can use its own TA estimate for LTM execution.

## Conditional cell switch execution

Allowing the UE to trigger a cell switch execution has significant impact on the actions that need to be taken before and after a conditional cell switch. An important consequence is the network’s uncertainty about the timing of the cell switch, which makes the preparation of the network for the UE’s access to the target cell more complicated. A possible way to overcome this challenge and simplify the procedures related to UE’s first access to the target is the transmission of a message from the UE to the source cell, informing the source cell that a conditional cell switch condition has been met. Upon receiving such a notification, the source cell can trigger procedures for UE’s access to the target cell as in Rel. 18 LTM, e.g., send a cell switch notification, or it can even decide whether a cell switch is allowed, in case more network control on the cell switch is desired, and inform the UE about it.

Observation 3: Autonomous UE’s decision on cell switch causes uncertainty at NW side with regards to timing of the cell switch and consequent preparation for UE admission.

Observation 4: Lack of any cell switch indication to the NW leads to inflexibilities in resource management at NW side.

Nevertheless, such a message may never reach the source cell as the UE-source cell link quality may already be too low soon after the conditional cell switch condition is met. In addition, while the UE will be waiting for a response to the message, the link may further deteriorate potentially resulting in radio link failure. Therefore, we should further study if and how this message should be transmitted. Alternatively, the UE could directly inform the target about its access, e.g. by sending a scheduling request to the target.

Observation 5: Early signalling with the source cell prior to cell switching can allow to mimic NW behaviour from Rel-18 LTM with regards to how candidate target cell is informed about cell switch.

Observation 6: Early signalling with the source cell prior to cell switching would be risky as it would need to be performed in potentially very bad radio conditions.

Observation 7: Early indication to the target cell before switching can allow to save PUSCH (CG) resources of target cell to be wasted or kept reserved for too long.

In both cases, it shall be decided how UE shall inform the NW about cell switch. From our perspective, cell switch indication to current serving cell could be realized either over PUSCH (e.g., new MAC CE) or PUCCH (e.g., new type of CSI). The drawback of using PUSCH is such that UE needs a grant for that. In our opinion, it increases the risk of delivery problems. Using PUCCH, on the other hand, will most likely require assigning separate PUCCH resources for such transmission, but it could help to shorten the time needed for UE to indicate current serving cell about the switch. The approach to cell switch indication sent to the target cell shall be different because in this case UE is not able to inform target cell about anything. Such transmission shall allow target cell to recognize that UE tries to connect without wasting CG resources. For that purpose, in our view, the most suitable physical channels will be PRACH or PUCCH. Using PRACH will be safe from TA perspective, but to enable that UE will need to be configured with one another dedicated preamble to allow target cell not only to recognize the UE but also to recognize that this PRACH transmission is not related either to early UL synchronization or RACH-based admission. Additional problem is such that the difference between this approach and RACH-based admission will be minimal. Using PUCCH is not as safe from TA perspective as PRACH, but it allows to not reserving more preamble resources which are already extensively used by LTM. It also allows to use much less resources of target cell comparing to CG. Especially if SRI sent using certain PUCCH format could be considered.

Proposal 7: RAN2 to decide whether UE shall inform the serving cell or the target cell about the switch before CLTM execution. For example, the following could be considered:

- if it is decided that UE informs serving cell about cell switch, it could use:

  - either PUSCH-based communication e.g., new MAC CE

  - or PUCCH-based communication e.g., new type of CSI

- if it is decided that UE informs target cell about cell switch, it could use:

  - either PRACH-based communication

  - or PUCCH-based communication e.g., SRI

For RACH-less access to the target cell, apart from the TA, the UE must be provided with an UL grant. In Rel. 18 LTM, both CG and DG were supported as they have their own benefits. In RAN2#127bis, support of CG was agreed:

RAN2#127bis agreement: For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

Providing a CG may incur a large resource reservation overhead, especially when multiple CGs for multiple UEs are provided by the target cell, with the resources being reserved from RRC configuration to cell switch execution. Nevertheless, for non-conditional LTM, thanks to the network triggering of the cell switch and the transmission of cell switch notification, optimizations of the resource management on the network side are possible, as well as monitoring of the CG only after a cell switch has been decided. The lack of network awareness of when the cell switch in CLTM is triggered requires for alternative solutions to avoid monitoring of the CGs starting from RRC configuration and to enable more flexible resource management on the network side. To this end, as the RAR transmission may constitute the last signaling exchange between the UE and the source cell before a potential conditional cell switch, the RAR transmission and its preceding steps (RA preamble transmission, TA information transfer) may be used for the activation of a CG. Alternatively, a message from the UE to the NW can be used for the activation of a CG.

Proposal 8: RAN2 to discuss CG activation based on RA-driven early TA acquisition procedure and/or message from the UE to the NW informing about cell switch triggering.

Furthermore, it is likely that conditional cell switch execution conditions for multiple cells are met. In the current RRC CR we have captured the following procedure:

1> if this procedure is triggered due to fulfilment of LTM cell switch conditions: 2> if more than one LTM candidate configuration has triggered this procedure: 3> select one of the LTM candidate configurations as the selected cell for the LTM cell switch execution; Editor’s Note: FFS how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution.

Observation 8: Considering the procedural text in the current RRC CR, in case more than one LTM candidate configuration has triggered the LTM procedure, UE would select one of the LTM candidate configurations and execute CLTM towards it.

The actions that the UE has performed to prepare a cell switch to a candidate cell prior to the triggering of the condition may be different for each of the triggered candidates, e.g., the UE may have performed DL and UL synchronization with one of the triggered candidates, but only DL synchronization with another. To reduce cell switch interruption time, it is reasonable that the UE prioritizes execution to the former.

Proposal 9: When conditional cell switch execution conditions for multiple candidate cells have been triggered, the UE should trigger the cell switch to a candidate cell for which Early synchronisation steps are completed.

## Co-existence with CHO

In Rel-18 and Rel-19 it has been decided that CHO can co-exist with regular LTM. It means that LTM candidate target cell can be also configured as CHO candidate target cell. We cannot see any reason to block this possibility in the context of CLTM.

Proposal 10: Follow similar principle as Rel. 18 LTM, CLTM and CHO can be configured independently for different candidate cells.

On the other hand, configuring CLTM candidate target cell to be also CHO candidate target cell for the same UE requires further discussion, since both features aim to provide robustness gains.

Observation 9: configuring CLTM candidate target cell to be also CHO candidate target cell for the same UE requires further discussion, since both features aim to provide robustness gains.

Proposal 11: RAN2 to discuss if co-existence of CHO and CLTM for the same candidate target cell is supported.

# 3	Conclusion

This document has made the following observations:

Observation 1: The following options are possible for handling CLTM TA timer(s) during cell switch execution.

- CLTM TAT is stopped immediately, and UE autonomously starts legacy PTAG TAT based on new serving cell’s configuration

- CLTM TAT is stopped immediately, and UE autonomously starts legacy PTAG TAT for amount of time remaining to CLTM TAT expiry

- CLTM TAT is left running either until its expiration or until UE receives PTAG TAC from new serving cell

Observation 2: first of above-listed options allows UE to assume that new serving cell TA can be valid for longer than it was configured by the NW (CLTM TAT value). From UE perspective, the remaining two options are equivalent in the sense that UE assumes CLTM TA value is valid for the same amount of time.

Observation 3: Autonomous UE’s decision on cell switch causes uncertainty at NW side with regards to timing of the cell switch and consequent preparation for UE admission.

Observation 4: Lack of any cell switch indication to the NW leads to inflexibilities in resource management at NW side.

Observation 5: Early signalling with the source cell prior to cell switching can allow to mimic NW behaviour from Rel-18 LTM with regards to how candidate target cell is informed about cell switch.

Observation 6: Early signalling with the source cell prior to cell switching would be risky as it would need to be performed in potentially very bad radio conditions.

Observation 7: Early indication to the target cell before switching can allow to save PUSCH (CG) resources of target cell to be wasted or kept reserved for too long.

Observation 8: Considering the procedural text in the current RRC CR, in case more than one LTM candidate configuration has triggered the LTM procedure, UE would select one of the LTM candidate configurations and execute CLTM towards it.

Observation 9: configuring CLTM candidate target cell to be also CHO candidate target cell for the same UE requires further discussion, since both features aim to provide robustness gains.

And proposed the following:

Proposal 1: if CLTM TAT is running, UE shall assume that TA is still valid. Once it expires, UE shall assume that TA is not valid anymore. RAN2 to discuss the best WF for handling the TA value after CLTM cell switch execution.

Proposal 2: If TA is valid, UE shall be allowed to use CG resources. When TA becomes invalid, UE shall be forbidden to use (or to start using) CG resources

Proposal 3: When TA becomes invalid before successful cell switch completion, UE shall fall back to RACH-based cell switch.

Proposal 4: In case CLTM is configured along with UE based TA estimation for given candidate cell(s), RAN2 to confirm that the TA information, which is obtained over the new MAC CE for the same candidate cell(s), if valid will get precedence over UE estimated TA.

Proposal 5: In case of CLTM, if the TA provided by the network over the new MAC CE or any network entity becomes invalid, or set as “FFF”, or no new MAC CE received, if UE is configured with UE based TA estimation and UE has successfully measured the TA, UE can perform RACH-less cell switch.

Proposal 6: Upon reception of CSC with TA = FFF, UE checks if a valid TA obtained through CLTM TAC MAC CE is available. If no TA is available and the UE is configured with UE-measured TA, UE can use its own TA estimate for LTM execution.

Proposal 7: RAN2 to decide whether UE shall inform the serving cell or the target cell about the switch before CLTM execution. For example, the following could be considered:

- if it is decided that UE informs serving cell about cell switch, it could use:

  - either PUSCH-based communication e.g., new MAC CE

  - or PUCCH-based communication e.g., new type of CSI

- if it is decided that UE informs target cell about cell switch, it could use:

  - either PRACH-based communication

  - or PUCCH-based communication e.g., SRI

Proposal 8: RAN2 to discuss CG activation based on RA-driven early TA acquisition procedure and/or message from the UE to the NW informing about cell switch triggering.

Proposal 9: When conditional cell switch execution conditions for multiple candidate cells have been triggered, the UE should trigger the cell switch to a candidate cell for which Early synchronisation steps are completed.

Proposal 10: Follow similar principle as Rel. 18 LTM, CLTM and CHO can be configured independently for different candidate cells.

Proposal 11: RAN2 to discuss if co-existence of CHO and CLTM for the same candidate target cell is supported.

# 4	References

[1]	[POST128][107][MOB] RRC CR for LTM and CLTM (Ericsson), RAN2#128, Orlando, USA, November 18-22, 2024.