3GPP TSG-RAN WG2 Meeting #129bis	R2-2502880

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.3.3

Source:	Nokia

Title:	On the measurement event prediction

WID/SID:	FS_NR_AIML_Mob - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Following options for actions based on the predicted measurement events have been captured in TR 38.744 [1]:

If measurement event is predicted based on intra-frequency temporal domain case A, there are two options w.r.t. how to decide on the time point to transmit handover command: Option 1: Relying on legacy measurement event Option 2: Relying on predicted measurement event Figure 5.5.1-1: Handover model option 1 Option 1 is illustrated in Figure 5.5.1-1. At current time i.e. t0 measurement event e.g. A3 event is predicted at some point of time in future. Network will not transmit handover command until a real measurement event is reported for the same neighbouring cell. In this way, the main benefit of this option is to save handover preparation time. Figure 5.5.1-2: Handover model option 2 Option 2 is illustrated in Figure 5.5.1-2. At current time i.e. t0 measurement event e.g. A3 event is predicted @ future time t1. Network transmits handover command when entering condition of the predicted measurement event based on actual measurement result @ t2 unless t2 is earlier than handover preparation phase. In later case, network transmits handover command immediately after handover preparation phase. In this way, not only handover preparation could be saved but also handover can be executed earlier.

In email discussion [2], the following proposals were agreed:

Proposal 1: For evaluation purpose, it is assumed PW length = TTT length =320ms Proposal 2: For evaluation purpose, Option 2 is agreed i.e. each counter can be updated only one time for related predicted and/or ground truth measurement event. Proposal 3: To introduce “Total number of HOF per UE per second” as SLS metric

In the sequel we present our results for the measurement event prediction use-case. We consider both Case A and Case B indirect predictions, as well as direct event predictions.

# 2	Measurement Event Prediction

## 2.1	Time Domain Case A and Direct Measurement Event Prediction (FR2)

In this sub-use-case, the objective is to predict an A3 measurement event before it occurs and use the prediction for mobility optimization.

### 2.1.1	Discussion on the Evaluation Assumptions for Time Domain Case A

In this section we discuss a clarification of the evaluated scenario and the evaluation assumption for the time domain case A prediction of future measurement events for mobility optimization. The measurement event prediction inference may be either periodic or event-based:

- Periodic measurement event prediction: Whenever configured with an A3, the A3 prediction is inferred after each measurement sampling period.

- Event-based measurement event prediction: The A3 measurement event prediction is inferred when specified prediction condition is met. For example, when an A3 entry condition is fulfilled, predict if the condition will hold for TTT.

In [1], the three counters n1, n2 and n3 are defined:

For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following: Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD]

In [2], the following was discussed and agreed:

For the same predicted measurement event and/or ground truth measurement event, the same counter including n1, n2 and n3 is counted only once i.e.: - n3 is counted only one time for related predicted measurement event and ground truth measurement event per agreed counter definition - n1 is counted only one time for related predicted measurement event per agreed counter definition - n2 is counted only one time for related ground truth measurement event per agreed counter definition

However, for the periodic measurement event prediction, further clarifications are required. First, for a measurement event prediction to be useful in mobility optimization (time domain case A), sufficient early warning may be required. Therefore, for n3 counter (true event prediction) evaluation, minimum event prediction distance should be considered, i.e., how much before the ground truth measurement event the prediction should be made to increase the n3.

Observation 1: For the periodic time domain case A measurement event prediction evaluation, a minimum event prediction distance should be considered for the n3 counter (true event prediction), i.e., how much before the ground truth measurement event the prediction should be made to increase the n3 counter.

Furthermore, it should be further clarified, which false event detections are related to the same related predicted measurement event, so that they are counted only once.

Observation 2: Further clarification is required in periodic measurement event prediction evaluation, which false event detections are related so that they are counted only once.

Additionally, if there is more than one target cell candidate, it should be clarified if the prediction accuracy counters are to be evaluated for each target candidate cell individually, i.e., each target candidate cell is its own data sample in the evaluation, or if predicting the right target candidate cell is included in the metrics calculation. In the latter case, RAN2 should discuss, how the metrics are calculated.

Observation 3: If there is more than one target cell candidate, it should be clarified if the prediction accuracy counters are to be evaluated for each target candidate cell individually, or if predicting the right target candidate cell is included in the metrics calculation.

Event-based measurement event prediction simplifies and clarifies the Observations 1-3, particularly if we take the A3 entry condition being fulfilled as the trigger condition for inferring a measurement event prediction. In this case, the A3 entry condition determines:

- What is one data sample to be evaluated for counters n1, n2 and n3.

- Target candidate cell

- The exact time within the PW, where the measurement report is sent, if the measurement event condition is fulfilled, i.e., after the TTT elapses.

Observation 4: Event-based measurement event prediction simplifies and clarifies the Observations 1-3 if we take the A3 entry condition being fulfilled as the trigger condition for inferring a measurement event prediction.

The measurement event prediction model predicts if the A3 condition will hold for the TTT, i.e., if the A3 leave condition is fulfilled or not within TTT, which is compared to the ground truth to compute the binary classification counters n1, n2 and n3. Every sample updates only one of the counters once. We may also choose to delay the inference a specified time or number of measurement periods after the A3 entry. Figures 2.1.1-1 and 2.1.1-2 show examples of data samples for periodic measurement event prediction with PW > TTT and for A3-entry-triggered measurement event prediction, respectively.

Figure 2.1.1-1 Example: Periodic measurement event prediction data sample, PW > TTT

Figure 2.1.1-2 Example: A3-entry-triggered measurement event prediction data sample

In our view, to make the accuracy evaluation results comparable between companies, A3-entry-triggered measurement event prediction should be prioritized in future measurement event prediction evaluations.

Proposal 1: To make the accuracy evaluation results comparable between companies, A3-entry-triggered measurement event prediction should be prioritized in future measurement event prediction evaluations.

### 2.1.2	Discussion on the Actions and Simulation Assumptions

Two options for handover execution decision have been documented in TR 38.744 [1]:

- Relying on legacy measurement event

- Relying on predicted measurement event

In both options, the decision for an early handover preparation may be done based on the predicted measurement event.

Figure 2.1.2-1 Sequence diagram of Option 1

Figure 2.1.2-1 shows a sequence diagram of evaluation Option 1:

- Network configures UE with a measurement configuration including an A3 measurement event. Network additionally chooses to configure UE to report a prediction for the A3 event.

- The UE measures the serving and the target candidate cells as specified in the measurement configuration.

- When the UE evaluates that the A3 prediction condition is fulfilled for one of the candidate cells, it infers an A3 prediction for that cell.

- If the A3 prediction reporting condition is fulfilled and an A3 is predicted, the UE reports the prediction to the network as configured.

- The network may or may not decide to act based on the reported A3 prediction. In case of Option 1, the network may decide on an early handover preparation.

- The UE continues to measure the serving cell and the target candidate cells as configured.

- If the configured A3 event condition is met,

- The UE sends a measurement report as configured.

- The network decides if handover execution should be triggered based on the measurement repot

- If the network decides to execute the handover, it sends the HO command to the UE.

In the context of Option 2, the assumption is that the network decides on both the handover preparation and execution based on the reported predicted A3 event and sends the handover command based on it. However, the network decisions based on the reported predicted A3 event are network implementation. Moreover, in the SID [3], we have the following text:

The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively.

Observation 5: Considering the SID, whether NW makes HO decision based on predicted or legacy reported event is a matter of NW implementation.

In option 1, the early preparation can potentially improve handover performance by reducing the handover execution time after the handover decision. Early preparation can be also implemented without measurement event prediction by configuring the UE to report an additional A3 with a lower A3 offset.

In option 2, both the handover preparation and execution are triggered based on the prediction. The potential advantage of option 2 over option 1 is that if the A3 has been configured with a too high offset or too long TTT, causing the handover execution to trigger too late, triggering the handover execution earlier can reduce too late handovers that may cause RLFs.

However, if the A3 has been configured with optimal configuration or with a configuration causing too early handover execution, triggering the handover earlier based on a predicted measurement event can cause too-early handovers leading to handover failures and mobility performance degradation.

Observation 6: Triggering HO earlier than needed can also cause degradation of system-level KPIs due to too-early handovers leading to potential HOF.

Observation 7: Triggering handover execution before the measurement event based on the predicted measurement event improves the mobility performance only if the A3 measurement event is configured to inform the NW about a potential handover too late. Such situation can anyway be mitigated through NW configuration.

The following parameters have been agreed for the measurement prediction use case evaluation.

Table 2.1.2-1 Assumptions for measurement event prediction based on time domain case A agreed in TR 38.744 Table 5.3.1-1

| Parameters | baseline value | Note |
| --- | --- | --- |
| A3 event offset (dB) | 2 | Open for 3dB |
| TTT (ms) | 320 | Open for one shorter value |
| UE speed (km/h) | 90 | Open for 60 and 120km/h |
| OW length (ms, *) | N/A | Up to implementation |
| PW length (ms,**) | 400 | Open for more values |
| Max ETD (ms, *) | 80 | Open for more values |

To evaluate the AI/ML gain, the baseline scenario should have an optimized A3 configuration, (i.e. in the sense that the A3 parameters are typically optimized during network design to improve HO KPIs). Especially it should be studied, if any experienced failures due to too-late handovers may be reduced by optimizing the A3 offset. Companies are encouraged to evaluate a sweep of A3 event offset configurations to ensure a meaningful baseline for the evaluation.

Observation 8: To meaningfully evaluate the AI/ML gain, the baseline scenario should have optimized A3 configuration. Especially it should be studied, if any experienced failures due to too-late handovers may be reduced by optimizing the A3 offset.

Proposal 2: Companies are encouraged to evaluate a sweep of different A3 event offset configurations to ensure a meaningful baseline for the evaluation.

### 2.1.3	Evaluation Results

In this evaluation, we assess the system level performance of a direct measurement event prediction model and its impact on mobility KPIs. We use the same simulation scenario of our previous contribution [4, 5]. To recap, we use the agreed-on 7-site FR2 simulation scenario, but with an additional UE trajectory option in which the UEs can move through a straight-line trajectory representing a street of width 12 meters. The UEs begin their trajectory at a random point along the width of the street at its beginning and move along the street till they reach its end. Moreover, the large-scale parameters of the propagation model are held constant across the simulation drops, while the small-scale parameters are randomized across the drops. This gives a propagation environment that is stationary enough for the ML model to learn patterns in the RSRP time series but also stochastic enough to challenge the model to learn general patterns. Table 2.1.3-1 summarizes the simulation assumptions.

Table 2.1.3-1: Simulation assumptions for measurement event prediction

| Reported simulation assumptions | UE trajectory option | See above |
| --- | --- | --- |
|  | UE trajectory boundary processing option | See above |
|  | UE speed | 90 km/h |
|  | SoftLOS | Yes |
|  | Spatial consistency option | Option B |
|  | Number of TX beams | 8 |
|  | Number of RX beams | 4 |
|  | Measurement reduction rate | N/A |
|  | Sub-use-case | Case 2 (cell to cell) |
|  | Observation window (OW) | 2000 ms |
|  | Prediction window (PW) | 320 ms |
| Data Size (Number of Samples) | Training | 4K |
|  | Testing | 2K |
| AI/ML model input/output | Model input | L3 filtered RSRP measurements |
|  | Model output | A3 event prediction |
| AI/ML model description | Model type | Random Forest |

The model takes as an input a time series of RSRP measurements of the source and target cells (for an observation window of 2000 ms) right before the A3 event entry and then predicts whether the A3 event will remain fulfilled for the duration of the TTT (and thereafter result in a measurement report) or not. After the model is trained, it is then tested in a system level simulation in the following manner: each time an A3 event is entered for a UE (and the TTT timer starts), the model predicts whether it will remain fulfilled for the duration of the TTT and result in a measurement report or not. If it is predicted as such, the target cell will perform early handover preparation, effectively eliminating the handover preparation time.

Table 2.1.3-2 shows the mobility KPIs for this SLS. We sweep over three different values for the A3 event threshold (1, 2, and 3 dB). For each value, we calculate the mobility KPIs (the number of handover success, handover failure, and radio link failure per UE per second) for each of the following three cases:

- Baseline

  - Scenario: Handover preparation and execution are always based on measurements reported based on the A3 measurement event

  - Simulation assumption: The handover preparation duration after A3 is 40ms, and no ML model is deployed.

- Early

  - Scenario: We assume an early handover preparation is always done, for example based on measurements reported based on another earlier measurement event. More specifically, in this case we use the same A3 configuration, but without TTT to report the A3 entry.

  - Simulation assumption: The handover is already prepared, when the A3 event occurs, so preparation duration after A3 is set to 0ms. No ML model is deployed.

- AI/ML

  - Scenario: Early handover preparation is only done, when the ML model predicts an A3 event at the A3 entry, as shown in Figure 2.1.2-1.

  - Simulation assumption: If no A3 was predicted, HO preparation duration after A3 is set to 40ms, if A3 was predicted HO preparation duration after A3 is set to 0ms, and the ML model described above is deployed.

Table 2.1.3-2: Mobility KPIs (per UE per second) for the measurement event prediction SLS

| A3 Threshold | 1 dB |  |  | 2 dB |  |  | 3 dB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Method | Baseline | Early | AI/ML | Baseline | Early | AI/ML | Baseline | Early | AI/ML |
| HO Success | 0.33144 | 0.33639 | 0.33398 | 0.31125 | 0.31384 | 0.31264 | 0.29838 | 0.30097 | 0.30005 |
| HO Failure | 0.02079 | 0.01782 | 0.01898 | 0.01884 | 0.01694 | 0.01773 | 0.01639 | 0.01472 | 0.01528 |
| RLF | 0.00236 | 0.00181 | 0.00194 | 0.00269 | 0.00250 | 0.00255 | 0.00315 | 0.00296 | 0.00301 |
| Total Failures | 0.02315 | 0.01963 | 0.02092 | 0.02153 | 0.01944 | 0.02028 | 0.01954 | 0.01768 | 0.01829 |

The “total failures” is the sum of handover failures and radio link failures. We can see that using a measurement event prediction model to prepare handovers ahead of time leads to a decrease in handover failures and radio link failures, with an increase in handover success. This holds true for all three values of the A3 threshold, although the effect becomes less pronounced as the threshold gets larger. As the A3 threshold increases, though, the total number of handover success and handover failures decreases, but at the cost of increasing radio link failures.

Table 2.1.3-3 Confusion matrix of direct measurement event prediction model

|  |  | Predicted Class |  |
| --- | --- | --- | --- |
|  |  | 0 | 1 |
| True Class | 0 | 0.874 | 0.126 |
|  | 1 | 0.163 | 0.837 |

Table 2.1.3-4 Percentage of unnecessary early preparations

| Baseline | 0% |
| --- | --- |
| Early | 50% |
| AI/ML | 13.1% |

The trade-off of early handover preparation is the additional resource usage, which is why unnecessary early preparations should be minimized. Table 2.1.3-3 shows the confusion matrix of the measurement event prediction model and Table 2.1.3-4 the percentage of unnecessary early preparations. Always performing early preparation leads to 50% of preparations being unnecessary. With ML, only 13% of the early preparations are unnecessary. Cost of early preparation can be further studied to better evaluate the advantage of the method.

Observation 9: Using a measurement event prediction model to prepare handovers ahead of time leads to a decrease in handover failures and radio link failures, with an increase in handover success.

Observation 10: Using AI/ML measurement event prediction can reduce the number of unnecessary early preparations, while still reducing the number of failures.

## 2.2	Time Domain Case B (FR1)

### 2.2.1	Discussion

Figure 2.2.1-1 Example 1 of intra-frequency temporal domain case B

Figure 2.2.1-2 Example 2 of intra-frequency temporal domain case B

In time domain case B RRM measurement prediction, the number of required downlink radio measurements is reduced by the UE skipping specified measurements periods and predicting the skipped measurements from the past measurements using a ML model. Two examples for skipping patterns are shown in Figures 2.2.1-1 and 2.2.1-2.

It has been agreed to study the measurement event prediction also with time domain case B RRM measurement prediction in FR1. Following observations can be made:

Observation 11: In time domain case B measurement event prediction, the PW is typically short, and predictions are not made for every measurement sampling period.

Observation 12: If time domain case B measurement event is configured, mobility decisions may use measurement-event-triggered reporting based on measurements and predictions.

### 2.2.2	Evaluation Assumptions

The following metric has been agreed in TR 38.744:

The performance metric F1 score is defined as following: F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3) For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following: Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD] The ETD i.e. timing difference between ground-truth event and predicted event is illustrated in Figure 5.3.1-1: Figure 5.3.1-1: illustration of ETD As illustrated in Figure 5.3.1-1, only if the ETD between a predicted event and a ground-truth event e.g. ground-truth event 2 is less than or equal to maximum ETD, the ETD can still be tolerated. Otherwise, both false event and missed event are detected.

The configurations agreed in TR 38.744 [1] Table 5.3.1-2 are given in Table 2.2.2-1.

Table 2.2.2-1 Assumptions for measurement event prediction based on time domain case B

| Parameters | baseline value | Note |
| --- | --- | --- |
| A3 event offset (dB) | 2 | Open for 3dB |
| TTT (ms) | 320 | Open for one shorter value |
| UE speed (km/h) | 30 | Open for 60 and 90km/h |
| OW length (ms,*) | N/A | Up to implementation |
| PW length (ms,**) | 200 (non-sliding) 40 (sliding) | Open for more values |
| Max ETD (ms,*) | 80 | Open for more values |
| MRRT | 50% | Open for more values |

Furthermore, measurement sampling period in FR1 has been agreed to be 40ms.

### 2.2.3	Evaluation Results

In the following, we present the simulation results for indirect measurement event prediction based on RRM measurement prediction in temporal domain case B. In accordance with the RAN2 agreement, we display the F1 score metric across different MRRTs. The baseline method employs sample and hold, while the AIML methods include linear regression and random forest. Additionally, we present the results for various UE speeds and observation window lengths.

Figure 2.2.3-1 Simulation results of F1 score vs MRRTs for different AI/ML models and observation windows, UE speed 30km/h

Figure 2.2.3-2 Simulation results of F1 score vs MRRTs for different AI/ML models and observation windows, UE speed 90km/h

The above simulation results analysis on indirect measurement event prediction based on temporal domain case B led to the following observations and proposals.

Observation 13: As MRRTs increase, the event prediction F1 scores exhibit a decreasing trend.

Observation 14: AI/ML models outperform the baseline sample and hold method, particularly at higher MRRTs. Notably, linear regression demonstrates the best performance in terms of F1 score.

# 3	Conclusion

This document has made the following observations:

Observation 1: For the periodic time domain case A measurement event prediction evaluation, a minimum event prediction distance should be considered for the n3 counter (true event prediction), i.e., how much before the ground truth measurement event the prediction should be made to increase the n3 counter.

Observation 2: Further clarification is required in periodic measurement event prediction evaluation, which false event detections are related so that they are counted only once.

Observation 3: If there is more than one target cell candidate, it should be clarified if the prediction accuracy counters are to be evaluated for each target candidate cell individually, or if predicting the right target candidate cell is included in the metrics calculation.

Observation 4: Event-based measurement event prediction simplifies and clarifies the Observations 1-3 if we take the A3 entry condition being fulfilled as the trigger condition for inferring a measurement event prediction.

Observation 5: Considering the SID, whether NW makes HO decision based on predicted or legacy reported event is a matter of NW implementation.

Observation 6: Triggering HO earlier than needed can also cause degradation of system-level KPIs due to too-early handovers leading to potential HOF.

Observation 7: Triggering handover execution before the measurement event based on the predicted measurement event improves the mobility performance only if the A3 measurement event is configured to inform the NW about a potential handover too late. Such situation can anyway be mitigated through NW configuration.

Observation 8: To meaningfully evaluate the AI/ML gain, the baseline scenario should have optimized A3 configuration. Especially it should be studied, if any experienced failures due to too-late handovers may be reduced by optimizing the A3 offset.

Observation 9: Using a measurement event prediction model to prepare handovers ahead of time leads to a decrease in handover failures and radio link failures, with an increase in handover success.

Observation 10: Using AI/ML measurement event prediction can reduce the number of unnecessary early preparations, while still reducing the number of failures.

Observation 11: In time domain case B measurement event prediction, the PW is typically short, and predictions are not made for every measurement sampling period.

Observation 12: If time domain case B measurement event is configured, mobility decisions may use measurement-event-triggered reporting based on measurements and predictions.

Observation 13: As MRRTs increase, the event prediction F1 scores exhibit a decreasing trend.

Observation 14: AI/ML models outperform the baseline sample and hold method, particularly at higher MRRTs. Notably, linear regression demonstrates the best performance in terms of F1 score.

And proposed the following:

Proposal 1: To make the accuracy evaluation results comparable between companies, A3-entry-triggered measurement event prediction should be prioritized in future measurement event prediction evaluations.

Proposal 2: Companies are encouraged to evaluate a sweep of different A3 event offset configurations to ensure a meaningful baseline for the evaluation.

# 4	References

[1] TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR (Release 19)

[2] R2-2501555, Summary of [AT129][027][AI Mob] Simulation methodology (OPPO), RAN2#129, Feb. 2025, Athens, Greece.

[3] RP-234055 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, TSG RAN Meeting 102, Dec. 2023, Edinburgh, Great Britain.

[4] R2-2500948 Generalization of ML mobility use-cases, Nokia, Nokia Shanghai Bell, 3GPP RAN2#129, Athens, Greece, February 17 – 21, 2025.

[5] R2-2500947 On the measurement event prediction, Nokia, Nokia Shanghai Bell, 3GPP RAN2#129, Athens, Greece, February 17 – 21, 2025.