3GPP TSG-RAN WG2 #129bis	R2-2502902

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.1.2.2

Source:	Ericsson

Title:	LCM for UE-side models for beam management

Document for:	Discussion

# 1	Introduction

In this contribution we share our views on FFSs and further aspects on the following LCM topics for UE-sided models:

- Applicability reporting and inference configuration

- AI/ML functionality activation/deactivation

- Triggering of data collection for training and resource configuration for training.

# 2	Discussion

## 2.1 Open issues from RRC running CR

The email discussion “[POST129][025][AI PHY] RRC running CR (Ericsson)” focused on the first draft of the RRC running CR for AI/ML for PHY. In our view, the following main issues are still open:

- Procedures for option B (sets of inference related parameters), namely what information and in which message is sent from the network, and what information and in which message is sent for applicability reporting from the UE;

- Content of the configuration for UE request for data collection and content of the UE request to start/stop data collection.

RAN2 to discuss the following open issues based on the RRC running CR:

- How to support option B;

- Content of the configuration for UE request for data collection and content of the UE request to start/stop data collection.

We discuss the open issues above, as well as other FFSs and issues in the following sections.

## 2.2	Applicability reporting and inference configuration

### 2.2.1 Option A (full inference configuration)

RAN2 has agreed to support option A (full inference configuration), consistent with RAN1 agreements, where the UE sends the applicability report in RRCReconfigurationComplete and the updated applicability in UAI. Let us discuss whether the applicability report from the UE can be sent before or after security activation. We note that this a new type of report from the UE, so there is no specific legacy rule for it. In our view, a malicious applicability report can result in a severe performance degradation. For example, wrongly reporting an inference configuration as applicable may result in the NW activating that inference configuration, which is in fact inapplicable at the UE. Consequently, the UE will report wrong predictions or may have an unexpected behavior, causing a severe performance degradation. Thus, the applicability report should be sent only after security activation. This can be solved from the NW side, by sending the RRCReconfiguration with the inference configuration only after security activation, which ensures that the UE replies with the applicability report only after security activation.

The RRCReconfiguration with inference configuration is sent by the NW after security activation, so the applicability report is sent by the UE also after security activation.

### 2.2.2 Option B (partial inference configuration / sets of inference related parameters)

Based on previous RAN2 agreements and input from RAN1, the details of how/what to signal for option B (partial inference configuration / sets of inference related parameters) are still open. Option B was discussed during last RAN2#129 meeting [2]. From the discussion, it turned out that is not clear what option B really is and how that is configured:

Option B-(a): According to some companies, option B contains a subset of the parameters included in option A, i.e. full inference configuration. In this case the partial inference configuration would be provisioned via a first CSI-ReportConfig configuration or a new report configuration, e.g. CSI-ReportConfigForPrediction (added in CSI-MeasConfig). In case the UE replies that the configuration is applicable, the gNB provides the full inference configuration in a second CSI-ReportConfig in a follow-up RRCReconfiguration message.

Option B-(b): According to some other companies, option B contains very minimal information provided by the gNB (e.g. a list of configurable associated IDs, the number of SetA/B, etc.) to make the UE able to determine the applicability. As described below, this minimal information can be included in the otherConfig, and in the UAI the UE can provide information to the gNB on how to make that minimal information applicable, and hence to allow the gNB to provide an applicable full inference configuration.

Analysis of Option B-(a)

In the following, we focus on option B-(a) above and analyze whether it offers advantages over option A (full inference configuration). An alleged advantage of option B-(a) vs option A is that by first providing a partial inference configuration, there is a signaling overhead gain. Hence, we first analyze the overhead of option B-(a) versus option A, where we assume the following:

- For option A, we assume that a full inference configuration in the RRCReconfiguration message contains the parameters shown in Table 1, comprising of new parameters needed for the inference configuration and legacy parameters that are mandatory in CSI-ReportConfig. NW sends N full configurations in one RRCReconfiguration message.

- For option B-(a), if the partial configuration is sent in the legacy CSI-ReportConfig, the mandatory legacy fields need to be transmitted and thus option B-(a) would have the same size as option A and would thus not reduce the signaling size. Thus, we assume that the configuration for option B-(a) is sent in a new report e.g. CSI-ReportConfigForPrediction. Since from RAN1 it is not yet clear which inference related parameters are included in the set of inference related parameters or are needed to determine applicability, we assume the sub-cases of option B-(a)1, B-(a)2, and B-(a)3, each with the set of parameters shown in Table 1. Importantly, NW sends N partial configurations in the first RRCReconfiguration message. In a follow-up RRCReconfiguration message, NW sends a full configuration as in option A, assuming the UE reported at least one partial configuration as applicable.

- The number of occupied bits for each of the options above is listed in Table 1. Additionally, the header overhead for sending one RRCReconfiguration message is 80 bits including PDCP overhead (48 bits), RLC overhead (16 bits), and MAC overhead (16 bits).

- Additionally, the resources for measurements and predictions need to be configured. For all options A and B-(a), the resources need to be configured already in the first RRCReconfiguration message, since the inference configurations contain references to Set A and Set B. The estimated number of bits needed for resource configuration (Set A/B, or legacy if a non-AI/ML configuration is needed) is 3440 bits.

Table 1 Parameters included in a full inference configuration (option A) and different variants of partial inference configurations in option B-(a).

| Parameters in one configuration |  | Option A (same as option B(a) if sent in legacy CSI-ReportConfig) | Option B-(a)1 | Option B-(a)2 | Option B-(a)3 |
| --- | --- | --- | --- | --- | --- |
| Parameter | Number of bits |  |  |  |  |
| AssociatedID#1 (new) | 7 | ü | ü | ü | ü |
| AssociatedID#2 (new) | 7 | ü | ü | ü | ü |
| Configuration ID - reportConfigId (mandatory in CSI-ReportConfig) for option A - a new configuration ID for options B-(a)1, B(a)-2, and B(a)-3 | 6 | ü | ü | ü | ü |
| resourcesForChannelMeasurement (mandatory in CSI-ReportConfig, e.g. can represet Set A) | 7 | ü | ü | ü | ü |
| reference to resources in Set B (new) | 7 | ü | ü | ü | ü |
| reportConfigType (mandatory in CSI-ReportConfig) | 17 | ü | - | ü | ü |
| reportQuantity (mandatory in CSI-ReportConfig) | 3 | ü | - | - | ü |
| reportFreqConfiguration (mandatory in CSI-ReportConfig) | 10 | ü | - | - | ü |
| timeRestrictionForChannelMeasurements (mandatory in CSI-ReportConfig) | 1 | ü | - | - | - |
| timeRestrictionForInterferenceMeasurements (mandatory in CSI-ReportConfig) | 1 | ü | - | - | - |
| groupBasedBeamReporting (mandatory in CSI-ReportConfig) | 4 | ü | - | - | - |
| CSI-ReportPeriodicityAndOffset (mandatory in CSI-ReportConfig) | 13 | ü | - | - | - |
| Total number of bits in one configuration [bits] |  | 83 | 34 | 51 | 64 |

The signaling reduction of each variant of option B-(a) with respect to option A as a function of the number of configurations in the first message is shown in Figure 1. Overall, the signaling reduction versus option A is not large (at most 8% for option B-(a)1) for a reasonable number of configurations, i.e. up to 10. In practice, the signaling reduction may even be lower, if retransmissions, etc. are taken into account. Furthermore, for low to moderate numbers of configurations (e.g. below 8 for option B-(a)3), option B-(a) introduces additional overhead (negative signaling reduction) versus option A. This is due to the need to send a full inference configuration in a follow-up RRCReconfiguration message, after sending the initial message with partial configurations.

Option B-(a) reduces the downlink signaling versus option A only marginally (at most 8%) and can even increase the overhead for a low number of partial configurations from the NW.

Consequently, option B-(a) is not justified in many scenarios and option A should be sufficient.

Figure 1 Signalling reduction of variants of option B (partial configuration) with respect to option A (full configuration), for different numbers of configurations N in the first RRCReconfiguration message.

Analysis of Option B-(b)

For option B-(b) we assume:

- The partial configuration contains only the parameters: AssociatedID#1, AssociatedID#2 and Configuration ID, with a total size of 20 bits.

- Since this partial configuration does not contain references to the resources, configuring the resources is only needed in the follow-up RRCReconfiguration. These resources in the follow-up RRCReconfiguration are either Set A/B, if at least one of the partial configurations was applicable, or resources needed for a legacy CSI report, if all partial configurations were non-applicable and the NW configures the UE with a non-AI/ML configuration. Thus, the resources contribute to the total size of the messages in option B-(b) (as in option A or B-(a), regardless of whether they are configured in the first or follow-up RRCReconfiguration).

The results for option B-(b) are also shown in Figure 1. Some signaling reduction (4–11%) is achieved by option B-(b), if more than 5 partial inference configurations are included in the initial RRCReconfiguration.

Some signaling reduction (4–11%) can be achieved by Option B-(b) (including a minimum number of parameters, i.e. associated IDs), if the NW sends at least 5 partial inference configurations.

Therefore, if supported at all, option B should contain only a minimum set of inference related parameters, as in the option B-(b) outlined above. However, based on this minimum set of parameters (e.g. associated IDs as in option B-(b)), the UE cannot be expected to reliably report the applicability with a simple “applicable/non-applicable” reply as for option A. Thus, the UE should be allowed to reply with a preferred set of inference related parameters that complement those parameters received from the network and together provide enough information for the NW to configure an applicable full inference configuration.

Hence, option B-(b) enables the UE to send its own preferred set of inference related parameters, upon configuration from the NW. For example, the UE may have stored from earlier training phases (e.g. earlier times when the UE was present in that cell) some parameter sets based on which it is has trained the model. Hence by receiving this minimum set of parameters, the UE may already have enough information to determine how to make an inference configuration applicable, so that it can inform the gNB about the applicability and also about the recommended configuration for the full inference configuration, e.g. the UE may indicate the SetA/SetB that would be applicable.

From the signaling message perspective, it is natural to include the small set of parameters for option B within otherConfig, so that the UE can reply with its preference in the UAI, based on the existing framework to report UE preference or assistance information. Another advantage of sending the partial configuration in otherConfig is that the NW can check the applicability at the UE for a non-serving cell, before configuring that cell for the UE. This is not possible with CSI-ReportConfig and is important since it can help the gNB to decide whether to configure CA or not. Namely, before configuring CA, the gNB considers the increase in the energy consumption, among other aspects. If the gNB knows that the UE can apply an inference configuration and thus needs to monitor only a reduced set of RSs in a CC, then the gNB has an incentive to configure CA, given the low energy cost. Similarly, there is an incentive also for the UE to operate with CA and inference configuration(s), due to the lower energy cost compared with conventional non-AIML based L1 measurements. Another use case for this is the handover. If the source gNB knows that the UE has a partial inference configuration applicable for a target cell, this information can be sent to the target (by forwarding the UAI to the target with existing procedures), and the target does not need to transmit the associated reference signals on which the UE can perform predictions.

Option B is supported only if the NW can send a minimum set of inference related parameters (e.g. associated IDs, or number of beams in Set A/B, or an empty configuration) and the UE can reply in the applicability report with preferred sets of parameters (e.g. the specific Set(s) A/B that would be applicable) that complement those from the NW. Based on this received preferred sets of parameters, the gNB can then configure the full inference configuration.

The one or more set(s) of inference related parameters for applicability report (option B) are sent to the UE in otherConfig and the UE responds by sending the applicability report via UAI.

## 2.3	Functionality activation/deactivation

RAN2#127bis agreed that:

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

Further, RAN2#128 agreed that:

2. When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

Given the two agreements above, it is critical for the network to be informed about functionalities/configurations that become non-applicable as soon as possible, so that the network can deactivate those functionalities/configurations. Thus, it should be mandated (and not optional) that the UE reports when an applicable functionality/configuration becomes non-applicable. This aspect was also discussed in RAN2#128 and it was suggested that we revise the agreement from RAN2#127bis as follows: “UE shall report to the network when an applicable AI functionality becomes non-applicable.”

Revise agreement 3 from RAN2#127bis as: “UE shall report to the network when an applicable AI functionality becomes non-applicable.”

Further, RAN2#129 agreed that:

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

It was thus clarified how the UE should behave when the periodic CSI report configuration is initially determined to be applicable. The agreement also implies that a periodic CSI report configuration that is initially reported as non-applicable will not be activated by the UE. This case (i.e. the UE being configured with a periodic CSI report configuration that is not active) is new compared to legacy procedures and it should be clarified how the UE should behave further. Different solutions are possible where the UE either releases the configuration or keeps the configuration and waits for further NW signaling. In our view, the simplest and fastest way to resolve this ambiguity is to allow the UE to release this configuration after reporting it as non-applicable to the network.

If the UE reports a periodic CSI report configuration (i.e. full inference configuration in CSI-ReportConfig) as non-applicable in the initial applicability report, the UE releases the configuration.

## 2.4	UE-side Data Collection for Training

RAN2#129 agreed the following regarding UE-side data collection for training:

- Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

Thus, it is still FFS what is the content of the UE’s indication related to its preference to be configured with radio resources for data collection and also how to indicate its preference to “start” and “stop” the data collection. For the content of the “start” indication, we envision the following cases:

- If the UE was already provided with an inference configuration (in CSI-Report configuration) or a set of inference related parameters (e.g. included in the otherConfig IE that configures the UE to transmit the data collection preference) and wants to collect data for training according to that configuration/set of parameters, then the UE can include in the data collection request an indication pointing to an ID of that configuration/set of parameters (i.e. CSI-ReportConfigId of the inference configuration or to an identification of the set of inference related parameters included in the OtherConfig).

- If the UE wants to perform data collection and it does not have a reference inference configuration/set of inference related parameters, the UE can include in the request a set of preferred parameters for the data collection configuration, including associated ID(s), Set A, Set B, etc., based on RAN1 discussions on which parameters are needed to decide the applicability of an inference configuration/set of inference related parameters.

The configuration to enable the UE to transmit the data collection preference is transmitted via the OtherConfig IE, and it can contain the set of parameters (e.g. associated IDs, SetA/SetB) from which the UE is allowed to transmit its preference to perform the data collection.

The UE can initiate the transmission of the UAI to inform the gNB about its preference to “start” or “stop” the data collection.

The UAI can contain references to the inference configuration(s) configured by the gNB or to the set of parameters included in the OtherConfig for which the UE wants to perform the data collection.

Another important aspect is that the radio resources for training need a configuration, which should coexist with the legacy radio measurement configuration (used for conventional “real time” measurement reports). For example, the gNB may need to provide a specific measurement configuration (CSI-RS/SSBs) for the UE to perform data collection for training and for the related reporting, including the Set A/B, and the associated NW-side additional conditions. Drawing inspiration from the RAN1 agreements on the inference and monitoring configurations, it seems natural to assume that the configuration of radio measurement resources for the UE-side data collection should be included in the CSI-reporting configuration. In this way, all the three fundamental configurations (inference, monitoring, data collection) needed to enable LCM can be provided within the same signalling which simplifies the signalling and procedures implementation.

The configuration of the radio measurement resources (including Set A/B, associated IDs, etc.) in which the UE should perform radio measurements for the purpose of UE-side data collection are provide via the CSI report configuration (as the inference and monitoring configuration as per RAN1 agreements).

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Option B-(a) reduces the downlink signaling versus option A only marginally (at most 8%) and can even increase the overhead for a low number of partial configurations from the NW.

Observation 2	Some signaling reduction (4–11%) can be achieved by Option B-(b) (including a minimum number of parameters, i.e. associated IDs), if the NW sends at least 5 partial inference configurations.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss the following open issues based on the RRC running CR:

	How to support option B;

	Content of the configuration for UE request for data collection and content of the UE request to start/stop data collection.

Proposal 2	The RRCReconfiguration with inference configuration is sent by the NW after security activation, so the applicability report is sent by the UE also after security activation.

Proposal 3	Option B is supported only if the NW can send a minimum set of inference related parameters (e.g. associated IDs, or number of beams in Set A/B, or an empty configuration) and the UE can reply in the applicability report with preferred sets of parameters (e.g. the specific Set(s) A/B that would be applicable) that complement those from the NW. Based on this received preferred sets of parameters, the gNB can then configure the full inference configuration.

Proposal 4	The one or more set(s) of inference related parameters for applicability report (option B) are sent to the UE in otherConfig and the UE responds by sending the applicability report via UAI.

Proposal 5	Revise agreement 3 from RAN2#127bis as: “UE shall report to the network when an applicable AI functionality becomes non-applicable.”

Proposal 6	If the UE reports a periodic CSI report configuration (i.e. full inference configuration in CSI-ReportConfig) as non-applicable in the initial applicability report, the UE releases the configuration.

Proposal 7	The configuration to enable the UE to transmit the data collection preference is transmitted via the OtherConfig IE, and it can contain the set of parameters (e.g. associated IDs, SetA/SetB) from which the UE is allowed to transmit its preference to perform the data collection.

Proposal 8	The UE can initiate the transmission of the UAI to inform the gNB about its preference to “start” or “stop” the data collection.

Proposal 9	The UAI can contain references to the inference configuration(s) configured by the gNB or to the set of parameters included in the OtherConfig for which the UE wants to perform the data collection.

Proposal 10	The configuration of the radio measurement resources (including Set A/B, associated IDs, etc.) in which the UE should perform radio measurements for the purpose of UE-side data collection are provide via the CSI report configuration (as the inference and monitoring configuration as per RAN1 agreements).

# References

R1-2410898 / R2-2500011, “Reply LS on applicable functionality reporting for beam management UE-sided model”, 3GPP TSG RAN WG1#119, Orlando, US, November 18th – 22nd, 2024.

R2-2501517, “Summary of offline discussion [AT129][026][AI Phy] Signaling for BM”