3GPP TSG-RAN2 Meeting #129-bis	R2-2502909

Wuhan, China, 7 - 11 April 2025

Agenda Item:	6.1.3.3 Other

Source: 	Ericsson

Title:  	Emergency callback and paging

Document for:	Discussion and Decision

# Introduction

In this contribution emergency callback and paging are discussed further.

# Background

Emergency call

The UE indicates an emergency call with establishment/resume cause emergency. And the CN uses a special ARP value for the QoS flow(s) of the emergency call (see TS 23.501 clause 5.16.4):

When the (R)AN resources for Emergency Services are established, the ARP value for Emergency Services indicates the usage for Emergency Services to the 5G-AN.

…

Dynamic PCC is used for UEs establishing emergency service and shall be used to manage IMS emergency sessions when an operator allows IMS emergency sessions. When establishing Emergency Services with a SMF, the PCF provides the SMF with the QoS parameters, including an ARP value reserved for the Emergency Services to prioritize the QoS Flows when performing admission control, as defined in TS 23.503 [45].

…

The ARP reserved for emergency service shall only be assigned to QoS Flows associated with an emergency PDU Session. If the UE is Emergency Registered over a given access, it shall not request a PDU Session to any other DNN over this access.

Emergency callback

While the UE has an emergency PDU session, the UE and NW shall not use eDRX to prevent that an emergency callback (aka Public Safety Answering Point (PSAP) Callback) is delayed because the UE is in eDRX (see TS 23.501 clause 5.31.7 and TS 24.501 clause 5.3.16):

When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has PDU Session associated with emergency services. When the PDU Session associated with emergency services is released, the UE and AMF shall reuse the negotiated extended idle mode DRX parameters in the last Registration Update procedure.

If the UE did not receive the Negotiated extended DRX parameters IE, or if the UE has an emergency PDU session, the UE shall use the stored UE specific DRX parameter, if available.

If the network did not accept the request to use eDRX, or if the UE has an emergency PDU session, the network shall use the stored UE specific DRX parameter, if available.

An emergency callback is a normal call, i.e. the special ARP value for emergency is not used for emergency call back and the emergency callback is not prioritized.

Normal DRX in RRC_IDLE and RRC_INACTIVE

In RRC_IDLE the UE monitors CN paging, and in RRC_INACTIVE the UE monitors both CN and RAN paging.

In RRC_IDLE the UE uses the shortest of the default paging cycle in SIB1 (320 ms up to 2,56 sec) and the UE specific DRX if configured via NAS signalling.

In RRC_INACTIVE the UE uses the shortest of the default paging cycle in SIB1, UE specific DRX (if configured via NAS signalling) and the configured in ran-PagingCycle (320 ms up to 2,56 sec) in suspendConfig in RRCRelease message.

eDRX in RRC_IDLE (Rel-17)

The UE can request eDRX during registration using NAS protocol, i.e. the UE can be configured in upper layers with eDRX (TeDRX, CN) from 2,56 sec up to 10485.76 sec (2.91 h) to monitor CN paging. eDRX longer than 10,24 sec uses Hyper SFN (10,24 sec duration) and Paging Transmission Window (PTW).

When the UE is configured with eDRX, then this eDRX configuration (including TeDRX, CN and PTW) is conveyed in the PAGING message from CN to RAN, see TS 38.413:

9.2.4.1	PAGING message

| IE/Group Name | Presence | IE type and reference |
| --- | --- | --- |
| Paging DRX | O | 9.3.1.90 |
| Paging Priority | O | 9.3.1.78 |
| UE Radio Capability for Paging | O | 9.3.1.68 |
| NB-IoT Paging eDRX Information | O | 9.3.1.138 |
| NB-IoT Paging DRX | O | 9.3.1.139 |
| WUS Assistance Information | O | 9.3.1.143 |
| E-UTRA Paging eDRX Information | O | 9.3.1.154 |
| NR Paging eDRX Information | O | 9.3.1.227 |
| PEIPS Assistance Information | O | 9.3.1.232 |

9.3.1.227	NR Paging eDRX Information

This IE indicates the NR Paging eDRX parameters as defined in TS 38.304 [12].

| IE/Group Name | Presence | IE type and reference | Semantics description |
| --- | --- | --- | --- |
| NR Paging eDRX Cycle | M | ENUMERATED (hfquarter, hfhalf, hf1, hf2, hf4, …, hf1024) | TeDRX CN defined in TS 38.304 [12]. Unit: [number of hyperframes]. |
| NR Paging Time Window | O | ENUMERATED (s1, …, s32) | PTW defined in TS 38.304 [12]. Unit: [1.28 seconds] |

If eDRX information is present in the PAGING message then then gNB uses eDRX to page the UE in RRC_IDLE (CM-IDLE) using 5G-S-TMSI as the UE identity, otherwise the gNB uses normal DRX.

eDRX in RRC_INACTIVE (Rel-17 and Rel-18)

In the MAC related UE capabilities the UE can indicate to support “short” eDRX up to 10,24 sec (extendedDRX-CycleInactive-r17) or “long” eDRX longer than 10,24 sec up to 2.91 hours (extendedDRX-CycleInactive-r18) in RRC_INACTIVE:

MAC-ParametersCommon ::=    SEQUENCE {

…

extendedDRX-CycleInactive-r17            ENUMERATED {supported}     OPTIONAL,

…

extendedDRX-CycleInactive-r18            ENUMERATED {supported}     OPTIONAL,

When the UE is configured with eDRX via NAS protocol, then the gNB can configure the UE with “short” or “long” eDRX in RRC_INACTIVE, provided that the eDRX in RRC_IDLE (negotiated via NAS) is equal or longer. The eDRX information is included in Core Network Assistance Information for RRC INACTIVE IE in INITIAL CONTEXT SETUP REQUEST message:

SuspendConfig ::=                   SEQUENCE {

…

ran-PagingCycle                     PagingCycle,

…

ran-ExtendedPagingCycle-r17         ExtendedPagingCycle-r17       OPTIONAL  -- Cond RANPaging

…

ran-ExtendedPagingCycleConfig-r18   ExtendedPagingCycleConfig-r18 OPTIONAL, -- Cond RANPaging

…

PagingCycle ::=                     ENUMERATED {rf32, rf64, rf128, rf256}

…

ExtendedPagingCycle-r17 ::=         ENUMERATED {rf256, rf512, rf1024, spare1}

…

ExtendedPagingCycleConfig-r18 ::=  SEQUENCE {

extendedPagingCycle-r18            ENUMERATED {hf2, hf4, hf8, …, hf1024},

pagingPTWLength-r18                ENUMERATED {ms1280, ms2560, …, ms40960}

…

As with normal DRX any incoming DL data/signalling is stored in the gNB until the UE is reached with eDRX in RRC_INACTIVE up to and including 10,24 sec. The “long” eDRX cycle ranges from 20,24 sec (2 hyper-frames) up to 2,91 hours (1024 hyper-frames). During “long” eDRX the incoming DL data/signalling is stored in CN while the UE is in RRC_INACTIVE, i.e. CM-CONNECTED. For this reason the RAN has to request the use of long eDRX cycle from CN (MT Communication Handling procedure):

The requested “long” eDRX cycle, i.e. the duration for which the CN may have to store DL data/signalling, is included in the NR Paging Long eDRX Information for RRC INACTIVE IE in MT COMMUNICATION HANDLING REQUEST message. The RAN can also request to deactivate the eDRX handling in CN.

In Core Network Assistance Information for RRC INACTIVE IE in INITIAL CONTEXT SETUP REQUEST message the UE capabilities are included and the CN indicates whether it supports MT Communication Handling procedure.

When there is DL data/signalling the AMF will request the gNB to start RAN paging just before the PTW is about to start:

The gNB triggers RAN paging when there is DL data/signalling in the buffer.

Thus the gNB can configure “long” eDRX in RRC_INACTIVE when:

- UE supports “long” eDRX in RRC_INACTIVE

- TeDRX, RAN ≤ TeDRX, CN, and

- AMF confirmed to store DL signalling/data while the UE is in eDRX with TeDRX, RAN in RRC_INACTIVE

The UE only uses eDRX in the cell when allowed in SIB1 (eDRX-AllowedIdle-r17, eDRX-AllowedInactive-r17, eDRX-AllowedInactive-r18).

# Discussion

After long/short eDRX in RRC_INACTIVE was introduced, RAN2 discussed whether it should be clarified in 38.304 that when the UE has an emergency PDU session then eDRX is not used [1]:

In RRC_INACTIVE state, if the UE has a PDU Session for emergency services, the UE shall not operate in eDRX, instead the UE shall operate in DRX.

However, the gNB is not aware whether the UE has an emergency PDU session after the UE has been released to RRC_INACTIVE and the radio resources and UE context are released in RAN.

It was discussed whether explicit signalling should be introduced to inform the gNB when the UE has an emergency PDU session or that this could be implicitly signalled (e.g. by omitting the eDRX configuration in the PAGING message for CM-IDLE paging) [2].

RAN2 decided to ask SA2 [3]:

- Is the gNB informed explicitly informed when the UE has an emergency PDU session (e.g. using special ARP value)?

- Or there are other means that UE and gNB do not use eDRX when the UE has an emergency PDU session?

The intention is to avoid delays with emergency callback when the UE is in eDRX. An emergency callback is significantly delayed when the UE with an emergency PDU session uses eDRX in Idle or Inactive.

Two solutions based on NW implementation have been discussed in SA2:

- The UE is kept in RRC_CONNECTED while the UE has an emergency PDU session

- The eDRX information is omitted, while the UE has an emergency PDU session, which cause the gNB not to configure eDRX in RRC_INACTIVE when the UE is released to RRC_INACTIVE.

UE is kept in RRC_CONNECTED

When the gNB has an emergency call, indicated by the special ARP value, then the gNB implementation can use a longer RRC inactivity timer than it would normally use to trigger RRC connection release:

Based on the special ARP value for emergency services the gNB can delay the release of the emergency call to facilitate emergency callback.

When the AMF sends UE CONTEXT RELEASE COMMAND message (e.g. because the emergency PDU session inactivity timer has expired) the gNB releases the RRC connection as normal, also when the RRC inactivity timer is still running. That is, the gNB can use a (very) long RRC inactivity timer and wait for the AMF to release the connection.

When the UE roams into bad or out of coverage, then Radio Link Failure (RLF) may happen. RLF is detected when Radio Link Monitoring (RLM) indicates too many consecutive out-of-sync indications that are not compensated by in-sync indications. When RLM is detected the UE will (temporarily) transit to RRC_IDLE and try to select a suitable cell. If a suitable cell is found, i.e. the UE did not roam out of coverage permanently, then the UE will initiate RRC re-establishment. When the RRC re-establishment is successful then the UE returns to RRC_CONNECTED. When the RRC re-establishment has failed, the UE has roamed out of coverage, i.e. subsequent paging for emergency callback would also fail (independent whether eDRX is used or not). UE behavior for out of coverage is left to UE implementation, i.e. when and how often to retry:

When RLF is detected and out of coverage is temporarily then RRC re-establishment will successfully return the UE to RRC_CONNECTED. When RLF is detected and the UE remains out of coverage then the UE enters RRC_IDLE. When the UE is out of coverage the UE is not reachable for paging.

For how long and how often the UE tries to find a suitable cell and attempt RRC re-establishment is up to UE implementation. But it can be assumed that the UE initially tries this for some time continuously, and when unsuccessful continues (indefinitely) with longer intervals.

eDRX info is omitted

Before the emergency voice call is released the AMF can use the UE CONTEXT MODIFICATION REQUEST message and omit the NR Paging eDRX Information in Core Network Assistance Information for RRC INACTIVE IE:

9.3.1.15	Core Network Assistance Information for RRC INACTIVE

This IE provides assistance information for RRC_INACTIVE configuration.

| UE Specific DRX | 9.3.1.90 |
| --- | --- |
| E-UTRA Paging eDRX Information | 9.3.1.154 |
| Extended UE Identity Index Value | 9.3.3.52 |
| UE Radio Capability for Paging | 9.3.1.68 |
| NR Paging eDRX Information | 9.3.1.227 |
| PEIPS Assistance Information | 9.3.1.232 |
| Hashed UE Identity Index Value | 9.3.3.62 |
| CN MT Communication Handling | ENUMERATED (supported, …) |

9.3.1.227	NR Paging eDRX Information

This IE indicates the NR Paging eDRX parameters as defined in TS 38.304 [12].

| IE/Group Name | Presence | IE type and reference | Semantics description |
| --- | --- | --- | --- |
| NR Paging eDRX Cycle | M | ENUMERATED (hfquarter, hfhalf, hf1, hf2, hf4, …, hf1024) | TeDRX CN defined in TS 38.304 [12]. Unit: [number of hyperframes]. |
| NR Paging Time Window | O | ENUMERATED (s1, …, s32) | PTW defined in TS 38.304 [12]. Unit: [1.28 seconds] |

In case the eDRX info is removed in the UE context modification, then the gNB will not configure the UE in RRC_INACTIVE with neither short nor long eDRX, even when the UE supports eDRX in RRC_INACTIVE.

Furthermore the AMF can omit the NR Paging eDRX Information in PAGING message for CN paging. When this eDRX is missing the gNB will use normal DRX for paging:

9.2.4.1	PAGING message

| IE/Group Name | Presence | IE type and reference |
| --- | --- | --- |
| Paging DRX | O | 9.3.1.90 |
| Paging Priority | O | 9.3.1.78 |
| UE Radio Capability for Paging | O | 9.3.1.68 |
| NB-IoT Paging eDRX Information | O | 9.3.1.138 |
| NB-IoT Paging DRX | O | 9.3.1.139 |
| WUS Assistance Information | O | 9.3.1.143 |
| E-UTRA Paging eDRX Information | O | 9.3.1.154 |
| NR Paging eDRX Information | O | 9.3.1.227 |
| PEIPS Assistance Information | O | 9.3.1.232 |

When the eDRX info is missing the gNB will not configure eDRX:

When NR Paging eDRX Information IE is omitted in the UE context, then the gNB will not configure eDRX when the UE is suspended to RRC_INACTIVE. When NR Paging eDRX Information IE is omitted in PAGING message then the gNB will not use eDRX to trigger CN paging for UEs in RRC_IDLE/RRC_INACTIVE.

From RAN2 perspective both methods are feasible. But when the UE is kept in RRC_CONNECTED:

When the UE is kept in RRC_CONNECTED then there is no latency due to paging for emergency callback nor missed paging and paging escalation is avoided.

How to handle UE-ID based subgrouping during an emergency PDU session is FFS in RAN2:

- FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In SA2 the use of LP-WUS during an emergency PDU session is also FFS (see clause 5.4.12a.2 in TS 23.501 V19.2.1):

Editor's note:	The use of LP-WUSPS for emergency sessions and emergency call back is FFS.

Furthermore SA2 agreed to remove the requirement that the UE shall not send 5GMM capability LP-WUSPS assistance information support indication (LP-WUSPSAI-SI) when the UE has an emergency PDU session [6]. If the UE does not support LP-WUSPSAI-SI then the UE also does not include paging probability information in AMF LP-WUSPS Assistance Information in the registration request. When the LP-WUS 5GMM capability is missing the AMF will not allocate a CN-assigned LP-WUS subgroup ID in the registration accept, see [7-12] for the SA2 correction to TS 23.501 and CT1 LP-WUS corrections to TS 24.501:

When the UE is kept in RRC_CONNECTED then no specification changes are required to avoid further emergency callback latency when LP-WUS is configured.

Based on the discussion above it is proposed:

RAN2 captures in a NOTE in 38.300 that using the ARP value reserved for emergency service the gNB can delay connection release to reduce the latency of a potential emergency callback.

RAN2 informs SA2 and RAN3 about the RAN2 agreement.

A TP for TS 38.300 and a draft LS are provided in the Annex for discussion and agreement.

# Summary

RAN2 is kindly asked to discuss emergency callback and paging:

Observation 1	Based on the special ARP value for emergency services the gNB can delay the release of the emergency call to facilitate emergency callback.

Observation 2	When RLF is detected and out of coverage is temporarily then RRC re-establishment will successfully return the UE to RRC_CONNECTED. When RLF is detected and the UE remains out of coverage then the UE enters RRC_IDLE. When the UE is out of coverage the UE is not reachable for paging.

Observation 3	When NR Paging eDRX Information IE is omitted in the UE context, then the gNB will not configure eDRX when the UE is suspended to RRC_INACTIVE. When NR Paging eDRX Information IE is omitted in PAGING message then the gNB will not use eDRX to trigger CN paging for UEs in RRC_IDLE/RRC_INACTIVE.

Observation 4	When the UE is kept in RRC_CONNECTED then there is no latency due to paging for emergency callback nor missed paging and paging escalation is avoided.

Observation 5	When the UE is kept in RRC_CONNECTED then no specification changes are required to avoid further emergency callback latency when LP-WUS is configured.

Proposal 1	RAN2 captures in a NOTE in 38.300 that using the ARP value reserved for emergency service the gNB can delay connection release to reduce the latency of a potential emergency callback.

Proposal 2	RAN2 informs SA2 and RAN3 about the RAN2 agreement.

# References

- R2-2410557, Correction on emergency call back when eDRX is configured in RRC_INACTIVE, Nokia, CR 38.304, Rel-17, RAN2#128

- R2-2410772, On eDRX Operation with Emergency Services, ZTE, DISC, RAN2#128

- R2-2411041, LS on emergency services and eDRX, LS out, ZTE, Rel-17, To: SA2, CT1, Cc: RAN3, RAN2#128

- S2-2502427, Reply LS on emergency call back and paging, LS out, SA2, Rel-17, To: RAN2, Cc: CT1, RAN3, SA2#167

- R2-2501093, LP-WUS in Idle and Inactive, Ericsson, DISC; RAN2#129

- S2-2502428, Removing unnecessary restrictions for emergency PDU sessions, Qualcomm, CR 23.501, SA2#167

- S2-2501979, Clarification on NAS LP-WUS Subgroup Support Indication meaning, HW, CR 23.501, SA2#167

- C1-250797, Support of LP-WUSPS assistance, general aspects, HW, CR 24.501, CT1#153

- C1-250799, Abnormal case in Generic UE configuration update procedure for handling Paging subgroup ID for LP-WUSPS, Apple, CR 24.501, CT1#153

- C1-250800, Abnormal case in Registration procedure for handling Paging subgroup ID for LP-WUSPS, Apple, CR 24.501, CT1#153

- C1-251159, Support of LP-WUSPS assistance, vivo, CR 24.501, CT1#153

- C1-251160, Low Power Wake UP Signal with Paging Subgrouping - registration, Ericsson, CR 24.501, CT1#153

# Annex

## TP for TS 38.300

## 16.5	Emergency Services

### 16.5.1	Overview

NG-RAN provides support for Emergency Services either directly or through fallback mechanisms towards E-UTRA. The support of Emergency Services is broadcast in system information (see TS 38.331 [12]).

NOTE:	Using the ARP value reserved for emergency service, see clause 5.16.4 of TS 23.501 [3], the gNB can delay connection release to reduce the latency of a potential emergency callback.

### 16.5.2	IMS Emergency call

An IMS Emergency call support indication is provided to inform the UE that emergency bearer services are supported. In normal service state the UE is informed if the PLMN supports emergency services through an Emergency Service Support indicator in the Attach and TAU procedures (see TS 23.501 [3]). In limited service state and for emergency services other than eCall over IMS, a UE is informed about if a cell supports emergency services over NG-RAN from a broadcast indication (ims-EmergencySupport). The broadcast indicator is set to "support" if any AMF in a non-shared environment or at least one of the PLMN's in a shared environment supports IMS emergency bearer services.

### 16.5.3	eCall over IMS

NG-RAN broadcast an indication to indicate support of eCall over IMS (eCallOverIMS-Support). UEs that are in limited service state need to consider both eCallOverIMS-Support and ims-EmergencySupport to determine if eCall over IMS is possible. UEs that are not in limited service state need to only consider eCallOverIMS-Support to determine if eCall over IMS is possible. The broadcast indicator is set to "support" if the PLMN in a non-shared environment, or all PLMNs in a shared environment, supports eCall over IMS.

### 16.5.4	Fallback

RAT fallback towards E-UTRA connected to 5GC is performed when NR does not support Emergency Services and System fallback towards E-UTRA connected to EPS is performed when 5GC does not support Emergency Services. Depending on factors such as CN interface availability, network configuration and radio conditions, the fallback procedure results in either CONNECTED state mobility (handover procedure) or IDLE state mobility (redirection) - see TS 23.501 [3] and TS 38.331 [12].

### 16.5.5	Barring exemption for emergency calls

The network may allow (e)RedCap UEs and 2Rx XR UEs to consider the cell as an acceptable cell to perform emergency calls in the cell, even if the cell would otherwise be considered as barred due to the cell barring indication in SIB1 for (e)RedCap UEs or 2Rx XR UEs. This is enabled via the barringExemptEmergencyCall by the network (see TS 38.304 [10] and TS 38.331 [12]).

## Draft reply LS

3GPP TSG-RAN2 Meeting #129-bis	R2-25xxxxx

Wuhan, China, 7 - 11 April 2025

Title:	DRAFT Reply LS on emergency call back and paging

Response to:	LS (S2-2502427/R2-2501752) Reply LS on emergency call back and paging

Release:	Rel-17

Work Item:	NR_newRAT-Core, NR_redcap-Core

Source:	Ericsson [TSG RAN WG2]

To:	SA2, CT1, RAN3

Cc:

Contact Person:

#### Name:	Tuomas Tirronen

- E-mail Address:	tuomas.tirronen@ericsson.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 thanks SA2 for their reply LS on emergency call back and paging in S2-2502427/R2-2501752.

RAN2 captured in TS 38.300:

NOTE:	Using the ARP value reserved for emergency service, see clause 5.16.4 of TS 23.501 [3], the gNB can delay connection release to reduce the latency of a potential emergency callback.

2. Actions:

To SA2, CT1, RAN3:

ACTION: 	RAN2 kindly asks SA2, CT1 and RAN3 to take the RAN2 agreements into account.

3. Date of Next TSG-RAN2 Meetings:

TSG RAN2 Meeting #130		19 – 23 May 2025		Malta

TSG RAN2 Meeting #131		19 – 23 May 2025		Bengaluru