3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502939

Wuhan, PRC, 07 – 11 April 2025

Agenda item:	8.13.3

Source:	Samsung

Title:	SRAP design for R19 multi-hop SL relaying

Document for:	Discussion & Decision

# Introduction

At the RAN2#128 meeting (November 2024), an initial discussion was held on the mechanics of SRAP operation for R19, and the following was agreed:

Agreements:

Maintain the principle that the SRAP header enables mapping from the e2e bearer ID to the egress PC5 RLC channel.  This does not preclude changes/rewrites/addition of the SRAP header by the intermediate UEs.

Remote UE ID and BEARER ID are included in Rel-19 SRAP header for multi-hop L2 U2N relay.

At the subsequent, most recent RAN2 meeting (RAN2#129, February 2025), the following was further agreed:

Agreements:

In multi-hop L2 U2N relay, besides the agreed remote UE ID and BEARER ID, in addition, at least include D/C field in the SRAP PDU header.

In multi-hop L2 U2N relay, for SRAP header, at least support the legacy single-hop U2N SRAP header, i.e., the agreed UE ID field is used for the remote UE local ID (including the case of an intermediate relay UE acting as a remote UE). FFS on whether to have one additional format to include remote UE L2 ID and what use case it is applied for.

Agreement:

Each Layer-2 Intermediate Relay UE has a single PC5 SRAP entity.

In this paper, we make several high-level proposals on the SRAP design for R19 MH, and how it may be captured in RAN2 specifications.

# Discussion

In R19, the legacy U2N Remote UE now becomes (i.e. needs to incorporate functions of) an intermediate/first U2N Relay UE – in other words, when performing functions of an intermediate/first Relay UE it does not just terminate or generate relaying data, but also passes data on further down the chain. Intermediate relays essentially perform a relaying function similar to the U2U Relay UEs, but due to the agreements above on having only one local ID in the SRAP header, it would appear that the design of intermediate relays should be based instead on the legacy U2N Remote UE design. Therefore our first proposal follows this line of reasoning and is as follows:

Proposal 1. Operation of R19 MH “intermediate” and “first” relays is to be based on procedures for R17/18 (legacy) U2N Remote UEs.

Proposal 1 essentially means that R19 relays should be an extension of U2N Remote UE operation; some of the differences are covered in the proposals that follow.

Focusing on downlink/downstream first, in the legacy case the U2N Remote UE terminates the packets it receives from the U2N Relay UE. It does not forward them on. The intermediate Relay UE in R19 MH on the other hand needs to perform both functions, and the forwarding function needs to include a choice of next-hop relay. The downstream aspects are covered in the following three proposals:

Proposal 2. To cover the operation of a R19 intermediate Relay UE, the receiving SRAP operation of legacy U2N Remote UE in the downstream needs to be modified so that – based on the UE ID field of the received SRAP packet – the UE either delivers the packet to higher layers, or passes it on to the transmitting part of the SRAP entity.

Proposal 3. To cover the operation of a R19 intermediate Relay UE, the transmitting SRAP operation of legacy U2N Remote UE in the downstream needs to be introduced so that it can receive an SRAP packet from the receiving part of the SRAP entity, and so that it can – based on the next hop configured for this Relay UE per destination UE – determine the next Relay UE (or destination Remote UE, if the Relay in question is a first Relay UE) to deliver the packet to.

Proposal 4. The egress link on PC5 interface towards the next hop is determined based on the L2 ID of the next-hop UE (e.g. sl-L2IdentityNextHop) configured for the concerned destination UE ID.

Turning our attention to the uplink/upstream, currently (legacy case) the U2N Remote UE does not receive packets from other UEs but only handles packets generated by higher layers. Therefore we propose:

Proposal 5. To cover the operation of a R19 intermediate Relay UE, the receiving SRAP operation of legacy U2N Remote UE in the upstream needs to be introduced so that the UE can receive and process SRAP packets from other UEs.

One function specific to the last Relay UE is the special SRB0 handling. Special SRB0 handling may not be needed for packets from/for Remote UEs not attaching directly to the Relay UE as this could be the function of the first Relay UE in R19 MH. In line with this reasoning, we propose the following:

Proposal 6. To cover the operation of the R19 last Relay UE, the legacy U2N Relay UE handling of SRB0 can be modified to apply the special SRB0 handling only for SL UEs attaching directly to this last Relay UE.

With regards to the spec impact, in our view we should first discuss the potential enhancements for each type of UE (as indicated in previous proposals), based on the basic legacy U2N Remote UE / U2N Relay UE design, and only then discuss how to develop the SRAP spec. e.g., reuse the existing sections for U2N Relay UE and U2N Remote UE, or create new sections for intermediate Relay UEs.

Proposal 7. RAN2 to first discuss enhancements needed for each type of R19 MH SL relaying UEs (Remote UE, fist/intermediate Relay UE, last Relay UE), and then discuss whether these enhancements will be captured via changes to the existing sections, or by creating new sections e.g. for the intermediate Relay UE.

# Conclusion

RAN2 is kindly asked to discuss and agree the following proposals:

Proposal 1. Operation of R19 MH “intermediate” and “first” relays is to be based on procedures for R17/18 (legacy) U2N Remote UEs.

Proposal 2. To cover the operation of a R19 intermediate Relay UE, the receiving SRAP operation of legacy U2N Remote UE in the downstream needs to be modified so that – based on the UE ID field of the received SRAP packet – the UE either delivers the packet to higher layers, or passes it on to the transmitting part of the SRAP entity.

Proposal 3. To cover the operation of a R19 intermediate Relay UE, the transmitting SRAP operation of legacy U2N Remote UE in the downstream needs to be introduced so that it can receive an SRAP packet from the receiving part of the SRAP entity, and so that it can – based on the next hop configured for this Relay UE per destination UE – determine the next Relay UE (or destination Remote UE, if the Relay in question is a first Relay UE) to deliver the packet to.

Proposal 4. The egress link on PC5 interface towards the next hop is determined based on the L2 ID of the next-hop UE (e.g. sl-L2IdentityNextHop) configured for the concerned destination UE ID.

Proposal 5. To cover the operation of a R19 intermediate Relay UE, the receiving SRAP operation of legacy U2N Remote UE in the upstream needs to be introduced so that the UE can receive and process SRAP packets from other UEs.

Proposal 6. To cover the operation of the R19 last Relay UE, the legacy U2N Relay UE handling of SRB0 can be modified to apply the special SRB0 handling only for SL UEs attaching directly to this last Relay UE.

Proposal 7. RAN2 to first discuss enhancements needed for each type of R19 MH SL relaying UEs (Remote UE, fist/intermediate Relay UE, last Relay UE), and then discuss whether these enhancements will be captured via changes to the existing sections, or by creating new sections e.g. for the intermediate Relay UE.