3GPP TSG-RAN WG2 Meeting #129bis	R2-2502946

Wuhan, China, 7 - 11 April 2025

Title: 	Discussion on MBS broadcast over NTN

Source: 	Huawei, HiSilicon, China Southern Power Grid, Turkcell

Agenda Item:	8.8.4

Document for:	Discussion and decision

# Introduction

One of the objectives for Rel-19 NTN WI [1] is the support of broadcast service over NTN:

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

During RAN2 #129 meeting, the following agreements in [2] were made:

Agreements:

- In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list

- It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH)

- Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates”

In this contribution, we provide our views on the remaining issues.

# Discussion

The last RAN2 meeting has introduced a new SIB to convey the representation format of intended geographical areas. The previous RAN2 meetings have narrowed down the methods to provide the intended service areas to the UEs and considered the following possibilities: SIB20, SIB21, or the MBSBroadcastConfiguration message. Each possibility has different implications, since SIB20, SIB21 and MBSBroadcastConfiguration take effect at different stages of the attempt to receive MBS service

- SIB20 is used to convey the MCCH/MTCH configuration information (the frequency resource for MCCH and MTCH, the time resource for MCCH). If the serving cell sends SIB20, the UE will receive the MCCH of this cell.

- SIB21 provides information related to service continuity for MBS reception, i.e., the mapping of MBS FSAI and the current/ neighbour frequencies. If the MBS capable UE is receiving or interested in receiving an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider the MBS frequency to be the highest priority during the cell selection procedure.

the MBSBroadcastConfiguration message (provided via MCCH, also called MCCH information) is used to convey the control information for MBS (including the MRB configuration, PDSCH configuration for MTCH per MBS session, and neighbour cells which are also broadcasting this MBS session). The UE will apply the corresponding configuration of an MBS session, e.g., the scheduling information and the MRB configuration per MBS session. Besides, the UE also determines whether the MBS session of interest is provided in any neighbour cell or not. The MCCH change notification message is used to indicate the UE to perform MCCH information re-acquisition, i.e., acquire the updated MBSBroadcastConfiguration message. When the UE receives an MCCH change notification, it acquires the updated MCCH in the same MCCH modification period where the change notification is sent.For UE power saving, the possible enhancements to allow UE to skip unnecessary MCCH acquisition when UE is not within intended service area of any interested broadcast service has been raised in RAN2#127bis meeting. However, no progress has been achieved yet. From our point of view, there could be some candidate solutions to achieve the goal of power saving as follows:

Alt.1: The UE decides whether it is within the ISAs belonging to the serving cell, according to the new SIB or the SIB20, upon entering a new cell. If not, the UE does not perform MCCH acquisition. Besides, if there is updated new SIB or the updated SIB20 update showing that the newly added areas cover the UE’s location, the UE needs to perform MCCH acquisition.

Alt.2: The UE reads the MCCH when entering a new Cell. The UE decides to perform MCCH re-acquisition only when the UE has received 1 bit contained in the MCCH change notification used to indicate that the mapping between the MBS session and ISA has changed.

Per current specification, the MCCH change notification only contains 2 bits, one bit indicated for the start of new MBS services, the other indicated for the MCCH config modification other than the change caused by the start of new MBS services. The 3rd bit should be introduced to indicate the mapping change, which could be caused by that new MBS session(s) and its ISAs are added or the existing MBS session has add/mod/delete its associated ISAs. In this case, the mapping change may impact the UE to receive the interested MBS session. That is why the UE needs to perform MCCH re-acquisition to check the detailed change for each MBS session upon receiving this bit in MCCH change notification.

Alt.3: The UE reads the MCCH when entering a new Cell. The UE outside the ISAs of interested MBS session monitor the MCCH change notification every new time interval introduced in SIBx, instead of every MCCH modification period. This way, it enables the UE to monitor the MCCH change notification not so frequently when the UE is outside the ISAs.

Proposal 1: RAN2 discuss the following enhancements to allow UE to skip unnecessary MCCH acquisition:

Alt.1: The UE decides whether it is within the ISAs belonging to the serving cell, according to the new SIB or the SIB20, upon entering a new cell. If not, the UE does not perform MCCH acquisition. Besides, if there is new SIB update or the SIB20 update showing that the new add-in areas cover the UE’s location, the UE needs to perform MCCH acquisition.

Alt.2: The UE reads the MCCH when entering a new Cell. The UE decides to perform MCCH re-acquisition only when the MCCH change notification has 1 bit used to indicate that the mapping between the MBS session and ISA has changed.

Alt.3: The UE reads the MCCH when entering a new Cell. The UE outside the ISAs of interested MBS session monitor the MCCH change notification every new time interval introduced in SIBx, instead of every MCCH modification period.

The last RAN2 meeting agreed to provide a list of service area IDs associated with one MBS session in the MCCH. The MCCH, i.e., MBSBroadcastConfiguration, provides the support status of MBS sessions not only for the serving cell, but also for neighbour cells. That means, in addition to the MBS session ID list supported by the serving cell, it also indicates the MBS session ID list provided by the neighbour cells. If MBSBroadcastConfiguration also provides the intended service area IDs for neighbour cells if the ISA could vary from cell to cell for one MBS session. One benefit is, upon mobility to a neighbour cell, the UE that has previously obtained the ISA Ids for the service sees the definitions of ISAs from the new SIB and immediately knows whether it is currently in an ISA of the service, and if not, it may not read MCCH.

Observation 1: If the MCCH also provides ISA IDs for neighbour cells, the UE can avoid monitoring MCCH when moving to a neighbour cell but out of the ISAs of the service.

Besides, based on neighbour cell information provided in the MBSBroadcast configuration, the UE can request unicast reception of the service of interest before moving to a cell not providing the MBS session. If the UE is in RRC_IDLE, this involves establishing an RRC connection. If the UE is in RRC_CONNECTED, the UE may request to switch to unicast mode for MBS session data reception as it comes closer to a target cell.

However, when moving to the new cell, if the UE is not in any of the intended service area, there is no use for the UE to start performing unicast reception of the service. If the MCCH also provides ISA IDs for neighbour cells and the ISA IDs are aligned between the serving and the neighbour cell, the UE can avoid switching to unicast reception unnecessarily.

Observation 2: If the MCCH also provides ISA IDs for neighbour cells, the UE can avoid unnecessarily switching to unicast reception before moving to a neighbour cell but out of the ISAs of the service.

Proposal 2:  Both current serving cell and neighbour cells MBS session specific intended service area ID(s) are indicated in the MCCH.

Proposal 3: To determine the ISA of an ISA ID indicated in MCCH of the serving cell for an MBS service in a neighbour cell, the UE can use the new SIB of the serving or of the neighbour cell.

SIB21 is used to guarantee the service continuity of MBS session when the UE in RRC_IDLE initiates the cell reselection procedure or when the UE is in RRC_CONNECTED. The UE in RRC_IDLE can prioritize an MBS frequency on which the MBS session of interest is provided when the UE performs cell reselection and the UE in RRC_CONNECTED can indicate the frequency of the MBMS service of interest, so that the network keeps the UE on that frequency. If an MBS service is very popular, this may increase the network load in the frequency where it is provided, possibly requiring to restrict other services for the UE, so it is preferrable that the UEs only prioritize or indicate a frequency when they are in the ISA of the service.

The UE may already know the ISAs of the service from reading MCCH in neighbour cells, but it may not be the case. Therefore, it is useful for the cell to indicate ISAs not only for MBS services that may be provided in the current frequency, but also for MBS services that are provided in neighbour frequencies, so the new SIB could include both.

In addition, the set of services provided in one frequency may not use the same set of ISAs as services provided in another frequency. If SIB21 can indicate which ISAs are used in which frequencies, the UE that is currently not located in any ISA where the MBS service of interest is provided can avoid prioritizing or indicating the frequency of the MBS service of interest, in order to preserve other services.

Proposal 4: SIB21 can indicate the subset of ISAs used for each frequency and the UE needs not prioritize (RRC_IDLE) or indicate (RRC_CONNECTED) the frequency of an MBS service of interest if the UE is not in any of the ISAs used in the frequency that provides the service.

# Conclusion

In this document, we discussed the support of broadcast service via NR NTN, and proposed the following:

Proposal 1: RAN2 discuss the following enhancements to allow UE to skip unnecessary MCCH acquisition:

Alt.1: The UE decides whether it is within the ISAs belonging to the serving cell, according to the new SIB or the SIB20, upon entering a new cell. If not, the UE does not perform MCCH acquisition. Besides, if there is new SIB update or the SIB20 update showing that the new add-in areas cover the UE’s location, the UE needs to perform MCCH acquisition.

Alt.2: The UE reads the MCCH when entering a new Cell. The UE decides to perform MCCH re-acquisition only when the MCCH change notification has 1 bit used to indicate that the mapping between the MBS session and ISA has changed.

Alt.3: The UE reads the MCCH when entering a new Cell. The UE outside the ISAs of interested MBS session monitor the MCCH change notification every new time interval introduced in SIBx, instead of every MCCH modification period.

Proposal 2:  Both current serving cell and neighbour cells MBS session specific intended service area ID(s) are indicated in the MCCH.

Proposal 3: To determine the ISA of an ISA ID indicated in MCCH of the serving cell for an MBS service in a neighbour cell, the UE can use the new SIB of the serving or of the neighbour cell.

Proposal 4: SIB21 can indicate the subset of ISAs used for each frequency and the UE needs not prioritize (RRC_IDLE) or indicate (RRC_CONNECTED) the frequency of an MBS service of interest if the UE is not in any of the ISAs used in the frequency that provides the service.

# References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

- R2-2501333, Report from Break-out session on NR-NTN and IoT-NTN, Session chair (ZTE)