3GPP TSG-RAN WG2 Meeting #129bis	 R2-2502949

Wuhan, China, 17th - 21st February 2025

Agenda Item:	8.3.5

Source:	KDDI Corporation

Title:	Discussion on specification impact for event prediction

SID:	FS_NR_AIML_Mob – Release 19

Document for: 	Discussion and Decision

- Introduction

In this contribution we provide our view on specification impact for event prediction. While RAN2 decided that RLF and HOF prediction will not be studied in Rel-19 study item, their specification impact should be similar to that of measurement event prediction because of the similarity of the prediction process. Chair notes for RAN2_129bis provided following guidance on LCM :

Contributions should focus on the LCM framework/signaling for RRM measurement prediction and measurement event prediction, and the gap from the currently agreed LCM framework for the beam management use case under the AI/ML for NR air interface work item. Contributions can address required signaling and protocol to enable reporting of inference outcome for RRM measurement prediction and measurement event prediction for a UE sided model, or the required signaling and protocol aspects to enable reporting of UE assistance for inference operation of a network sided model for RRM measurement prediction.

- Discussion

2.1 Additional signalling for inference results exchange and prediction-based handover for UE sided model

Especially in future state prediction (temporal domain case A), inference results exchange and prediction-based handover command is clearly not covered by the AI/ML for NR air interface. It is considered possible to utilize the prediction results as assist information just in implementation, and the handover process operated within a conventional framework based on actual measurements. However, we think there is a gain for the network if having an explicit protocol for prediction results and prediction-based command, especially in temporal domain case A. The studies on measurement prediction have provided some assumptions for prediction-based handover command, which is useful for discussion of necessity of additional signalling consider with additional overhead.

RAN2 to discuss whether additional signalling is necessary for inference result exchange and prediction-based handover command.

2.2 Data collection

2.2.1 Event data collection

We think one of the gaps from the AI/ML for NR air interface is that the AI/ML for mobility needs data of the “event”, not only measurement value. Even the measurement event prediction is realized by indirect prediction, we think event occurrences also need to be collected, such as log data, for every functionality of AI/ML for mobility.

For model training, monitoring, RAN2 to discuss protocol aspects of event data collection and logging.

2.2.1 Data collection for monitoring KPI

In order to avoid event count duplication and overestimation/underestimation as shown in Figure 1, following proposal is discussed in RAN2 #129 meeting.

Figure 1. Event duplication issue.

- n3 is counted only one time for related predicted measurement event and ground truth measurement event per agreed counter definition - n1 is counted only one time for related predicted measurement event per agreed counter definition - n2 is counted only one time for related ground truth measurement event per agreed counter definition where Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD]

The accuracy metric provided by above rule seems to be beneficial for show the AI/ML performance, but we think it contains some ambiguity and AI/ML should be evaluated by HO performance eventually. However, we also think that it is not appropriate and unrealistic to use HO performance as main KPI for model monitoring and updating, therefore the above counter rule should be considered as main KPI for monitoring. In addition, because the above rule does not move the counter every time an inference is made, but rather the decision is made after multiple inferences are completed, the specification impact related to accuracy should be further discussed (Of course, whether this is implementation issue or not.)

For model monitoring KPI, RAN2 to further discuss on event count rule.

2.3 Model transfer for model ID based LCM

We think AI/ML for mobility may be more scenario/configuration/site specific than Air Interface. The appropriate parameters in model transfer for model ID based LCM should be discussed, such as location information, UE speed.

RAN2 to discuss mobility specified parameters in model transfer for model ID based LCM.

- Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1  RAN2 to discuss whether additional signalling is necessary for inference result exchange and prediction-based handover command.

Proposal 2  For model training, monitoring, RAN2 to discuss protocol aspects of event data collection and logging.

Proposal 3  For model monitoring KPI, RAN2 to further discuss on event count rule.

Proposal 4  RAN2 to discuss mobility specified parameters in model transfer for model ID based LCM.