3GPP TSG-RAN WG2 Meeting #129bis	R2-2502964

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:		8.2.3

Source:				Samsung

Title:						Discussion on A-IoT random access

Document for:		Discussion

1 Introduction

This contribution discusses the remaining issues on A-IoT random access.

2 Discussion

2.1 Msg1 format

In previous RAN2 meeting it was left FFS whether a MAC header is required for Msg1

Agreements on MAC PDU format design - Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

Random access procedure for Ambient IoT is always triggered by the reader. Since the reader allocates Msg1 access occasions, there is no ambiguity whether the received D2R message is for Msg1 or not. Therefore, it is sufficient to specify MAC PDU including 16 bits random number without MAC header (i.e. message type field, length for SDU and variable part(s)).

Proposal 1: Msg1 is defined as one MAC PDU including 16 bits random number without MAC header.

2.2 Msg2 format

Msg2 is used for contention resolution by echoing the received 16 bits random number in Msg1. In order to reduce signalling overhead, RAN2 agreed to support multiplexing of information for multiple devices in R2D message for msg2. For multiplexing case, multiple devices may generate the same random number ID and transmit Msg1 from different access occasions. But its probability is expected to be sufficiently low, which could be handled by reader implementation. So we think that each echoed random ID in Msg2 is enough to address contention resolution even for multiplexing case.

Proposal 2: For multiple devices in Msg2, each echoed random ID indicates the specific device for contention resolution.

In case of failure reception of Msg3 from some specific device(s) after the initial transmission of Msg2, the reader can retransmit Msg2 to trigger them to resend Msg3. Since the reader can include only failed devices’ information of echoed random ID(s) through retransmitting Msg2, separate Msg2 format is not necessary for initial transmission of Msg2 and retransmission of Msg2.

Proposal 3: Same Msg2 format is used for initial transmission and retransmission of Msg2.

2.3 NACK-based feedback of Msg3

In previous meeting RAN2 has discussed whether NACK-based feeback is needed for Msg3 to trigger reaccess and made the following agreement.

NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback

Similar to multiplexing case in Msg2, it could be signalling efficient to support multiplexing of information for multiple devices in one R2D message for NACK based feedback to Msg3.

Proposal 4: For NACK-based feedback to trigger re-access, new R2D message is defined as one R2D MAC PDU including one or more AS ID information about failure of Msg3 reception from device(s).

Although a certain targeted device receives NACK-based feedback including its AS ID, we think that subsequent paging needs to be received to initiate random access procedure. The reason is that paging message includes a number of msg1 resources. One may consider that NACK-based feedback can include a number of msg1 resources but it does not bring much benefit.

Proposal 5: For NACK-based feedback to trigger re-access, no scheduling information (i.e., a number of msg 1 resource) is included.

We understand that RAN2 will discuss how to define the duration for Msg2 monitoring i.e., based on timer value. Therefore, we think that similar behavior needs to be defined for NACK-based feedback monitoring. For example, the device considers the transmission of Msg3 as successful if either NACK-based feedback or command message is not received within a certain time duration.

Proposal 6: RAN2 assumes that the device considers the transmission of Msg3 as successful if no R2D message (NACK-based feeback or command) is received within a certain time duration.

3 Conclusion

Based on above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: Msg1 is defined as one MAC PDU including 16 bits random number without MAC header.

Proposal 2: For multiple devices in Msg2, each echoed random ID indicates the specific device for contention resolution.

Proposal 3: Same Msg2 format is used for initial transmission and retransmission of Msg2.

Proposal 4: For NACK-based feedback to trigger re-access, new R2D message is defined as one R2D MAC PDU including one or more AS ID information about failure of Msg3 reception from device(s).

Proposal 5: For NACK-based feedback to trigger re-access, no scheduling information (i.e., a number of msg 1 resource) is included.

Proposal 6: RAN2 assumes that the device considers the transmission of Msg3 as successful if no R2D message (NACK-based feeback or command) is received within a certain time duration.

4 Reference