3GPP TSG RAN WG2 Meeting #129bis            	                      R2-2502975

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.10.4

Source:	Xiaomi

Title:	Discussion on SON/MDT for NTN mobility

Document for:	Discussion and Decision

Introduction

In the post RAN2 #129 meeting, some progresses are made on SON for NTN, but no agreements are made on MDT for NTN [1].

- Existing IEs for CHO are used to report the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”.

RAN3#126 meeting made some progress on SON/MDT for NTN and sent and LS to RAN2 in [2]. The main issues that involved with RAN2 are listed as follows:

- Regarding SON/MDT for NTN: On MRO for NTN, RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not. - On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP.

This document starts the discussion on MDT for NTN based on RAN3’s LS and RAN2#129 progresses.

Discussion

MDT for NTN

Considering the large coverage of NTN cells, it is inefficient to log all MDT results across the cell when the UE is in the IDLE/INACTIVE state. Thus, a new definition that enables MDT logging within a smaller and more flexible area scope is needed, such as a geographical area (a reference location plus a radius).

Additionally, the geographical area-based approach leverages existing well-defined IEs in specifications, such as the circular area defined in SIB25 for NTN UEs to skip neighbor measurements of TN cells, which can be reused. Regarding the mapped cell ID, while it serves as a logical identifier between the CN and NTN RAN to maintain a non-satellite-moving cell concept, this is transparent to UEs. Merely making the mapped cell ID known to UEs is insufficient, as UEs still need to determine the corresponding geographical area behind it to fully utilize the information.

Proposal 1: The geographical area (a reference location plus a radius) can be reused for area scope of logged MDT over Uu for NTN.

Conclusion

In this contribution, we propose:

Proposal 1: The geographical area (a reference location plus a radius) can be reused for area scope of logged MDT over Uu for NTN.

Reference

[1] R2_129_Maint_SONMDT_Mattias_20250221_Final

[2] R3-247908, LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT.