3GPP TSG-RAN WG2 #129bis	R2-2502976

Wuhan, CN, 7 – 11 Apr. 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on the LP-WUS handling for Emergency call back

Document for:	Discussion and decision

Agenda Item:	8.4.2

- Introduction

RAN2#128 agreed made the following agreement on subgrouping [1]

FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In addition, RAN2 sent an LS [2] to confirm SA2's understanding of the operation of emergency call during eDRX, and SA2 sent reply LS [3]. This LS exchange concerns eDRX, which allows the NW to indicate whether to use eDRX to the UE via RRC. It is assumed to align the solution for LP-WUS with eDRX

- Discussion

Regarding to the Reply LS on emergency call back and paging state, SA2 provide the view as below:

SA2 analysed the identified issue above and would like to provide the following feedback:

- Existing specifications have specified that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to RAN node. See related text in TS 23.501 clause 5.16.4.3, 5.16.4.7, 5.16.4.9 and TS 23.502 clause 6.1.3.10. Therefore the RAN node can be aware of an emergency PDU session based on the reserved ARP value when the PDU session context exist in the RAN node.

- In addition there is inactivity monitoring for the presence of user plane resources for the emergency PDU session in SMF as defined in TS 23.502 clause 4.4.2.2 and TS 23.503 clause 6.1.3.10.

Based on the above, some companies in SA2 believe that the RAN node is capable through configuration to not release the emergency PDU session user plane resources based on existing implementation, and to not release the UE to RRC_INACTIVE state/RRC_IDLE state when the UE has an emergency PDU session. Therefore the issue highlighted in RAN2 LS may not exist. However there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated, or whether to introduce new signalling to NG-RAN.

We believe it is up to gNB implementation on the base e.g. where the network controls the UE to prevent it from transitioning to the Idle state for a specific period after an emergency call. However, considering the importance of emergency call back communication, leaving this process to implementation may not satisfy the operator’s requirement. SA2 also clarifies that PSAP callback is delivered as normal call in IMS PDU session.in [4] where UE and network cannot disable LP-WUS functionality during or after the PDU session activated. So only gNB should have the solution to solve this issue.

Observation 1: Depending on the operator’s policy, gNB should be implemented to deactivate LP-WUS per UE

Our view is that the gNB should control for the UE with dedicated signal not to activate the UE LP-WUS i.e. to activate/deactivate the LP-WUS function of the UE from the NW (RAN or CN) explicitly and let the NW make it sure that UE is not in low power state. The SIB broadcasts whether LP-WUS can be used in the cell, but since this problem is UE dedicated, it is necessary to introduce a new UE dedicated signaling.

Proposal 1: gNB control the UE to deactivate UE LP-WUS capability with dedicated signal

- Summary and proposal

Observation 1: Depending on the operator’s policy, gNB should be implemented to deactivate LP-WUS per UE

Proposal 1: gNB control the UE to deactivate UE LP-WUS capability with dedicated signal

- References

[1]	Draft_RAN2_128_Meeting_Report_v1

[2]	R2-2411041 LS on emergency services and eDRX

[3] 	R2-2501753/ S2-2502427 Reply LS on emergency call back and paging

[4] 	S2-2502428 CR for 23.501 Removing unnecessary restrictions for emergency PDU sessions