3GPP TSG-RAN WG2 Meeting #129bis	R2-2502982

Wuhan, China, Apr. 7th – 11th, 2025

Source: 	RAN2 Vice Chairman (CATT)

Title:	Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others

## Organizational email discussion

[AT129bis][200] Organizational – Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others (RAN2 VC)

Scope:

a) Share plans for online/offline discussions during the meeting, and

b) Share draft session notes and agreements for review

#### 7.0.2.13	NR MIMO evolution

(NR_MIMO_evo_DL_UL-Core; leading WG: RAN1; REL-18; WID: RP-233028)

R2-2501712	LS on RRC parameter for PRACH transmission in 2TA (R1-2501540; contact: CATT)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL-Core	To:RAN2

Noted

R2-2501719	Reply LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP (R1- 2501611; contact: CATT)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL-Core	To:RAN2

Noted

R2-2501723	Reply LS on UL 8Tx (R1-2501636; contact: Samsung)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL-Core	To:RAN2

Noted

8Tx

R2-2502810	Discussion on supporting 8Tx	ASUSTeK	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Noted

Confirm the assumption that for UL 8Tx 2TB transmission scheduled by one DCI, MAC entity considers two uplink grants for two TBs in two HARQ processes with one HARQ process ID and two HARQ buffers.

R2-2502811	Correction on supporting 8Tx in MAC specification	ASUSTeK, Samsung, ZTE	CR	Rel-18	38.321	18.5.0	2068	-	F	NR_MIMO_evo_DL_UL-Core

=> Withdrawn

R2-2502812	Stage-2 Correction on UL 8Tx	ASUSTeK	CR	Rel-18	38.300	18.5.0	0987	-	F	NR_MIMO_evo_DL_UL-Core

DISCUSSION

-	ZTE do not think stage 2 CR is needed, even though the texts are correct. Nokia, LG E, Ericsson agree.

The CR is not pursued.

R2-2502835	Correction on supporting 8Tx in MAC specification	ASUSTeK, Samsung, ZTE	CR	Rel-18	38.321	18.5.0	1990	2	F	NR_MIMO_evo_DL_UL-Core	R2-2500410

The CR is revised to R2-2503034, with impact to NW added in the cover page.

R2-2503034 is agreed in principle.

Other changes

R2-2502105	Corrections on simultaneousU-TCI-UpdateListx and RACH-ConfigTwoTA	CATT	CR	Rel-18	38.331	18.5.1	5291	-	F	NR_MIMO_evo_DL_UL-Core

The content of the CR is agreeable, will update with proper style and submitted to the next R2 meeting.

R2-2502855	Correction to 306 on PMI subband R value	Ericsson	CR	Rel-18	38.306	18.5.0	1267	-	F	NR_MIMO_evo_DL_UL-Core

DISCUSSION

-	Xiaomi think this is editorial change and think this can be taken into account in rapp’s CR. Nokia also OK with the intention.

Intention is agreeable, will be included in the next version of UE capability Rapp CR.

R2-2502987	Correction for UE capability on DMRS port	Huawei, HiSilicon	discussion	Rel-18	NR_MIMO_evo_DL_UL

DISCUSSION

-	ZTE think the intention is OK, since it is just to align with R1. But ZTE think it can be included in the Rapp’s CR.

-	HW think it is not editorial.

-	Xiaomi think it is OK to have a separate CR.

-	Xiaomi suggest companies to carefully check the mega CR.

Delete "at least" before "one port" in the description of ‘mappingTypeA-1SymbolFL-DMRS-Addition2Symbol-r18’. The changes will be included in the next version of UE capability Rapp CR.

## 8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

(NR_LPWUS-Core; leading WG: RAN1; REL-19; WID RP-241824)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.4.1	Organizational

LS, Rapporteur input, including workplan, Running CRs, etc.

LS

R2-2501743	LS on LP-WUS UE RF (R4-2503003; contact: vivo)	RAN4	LS in	Rel-19	NR_LPWUS-Core	To:RAN1	Cc:RAN2

Noted

Running CRs

R2-2501954	Running 37.340 CR for LP-WUS	ZTE Corporation, Sanechips	draftCR	Rel-19	37.340	18.5.0	NR_LPWUS-Core

Noted, will update/endorse in post meeting email discussion

[Post129bis][206][LPWUS] Running CR for 37.340 (ZTE)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

R2-2502913	Introduction of Low-Power Wake-Up Signal and Receiver for NR	Ericsson	draftCR	Rel-19	38.300	18.5.0	NR_LPWUS-Core

Noted, will update/endorse in post meeting email discussion

[Post129bis][207][LPWUS] Running CR for 38.300 (Ericsson)

Intended outcome: Updated running CR based on new agreements for endorsement

Deadline:  Long

R2-2502141	38.304 Running CR for LP-WUS (CATT)	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted, will update/endorse in post meeting email discussion

[Post129bis][208][LPWUS] Running CR for 38.304 (CATT)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

R2-2502153	RRC Running CR for LP-WUS WUR	vivo (Rapporteur)	draftCR	Rel-19	38.331	18.5.1	B	NR_LPWUS-Core

Noted, will update/endorse in post meeting email discussion

[Post129bis][209][LPWUS] Running CR for 38.331 (vivo)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

R2-2502307	Running MAC CR for LP-WUS	Apple	draftCR	Rel-19	38.321	18.5.0	B	NR_LPWUS-Core

Noted, will update/endorse in post meeting email discussion

[Post129bis][210][LPWUS] Running CR for 38.321 (Apple)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

Others

R2-2502098	Discussion on UE capability for LP-WUS	Huawei, HiSilicon	discussion	Rel-19

Noted

Subgrouping capability

Proposal 1: Support of LP-WUS in IDLE/INACTIVE (either OOK-based or OFDM-based) indicates support of UE ID-based subgrouping by default.

Proposal 2: The capability of LP-WUS in IDLE/INACTIVE (coupled with UE ID-based subgrouping capability) is included in UE-RadioPagingInfo-r17 in RRC UECapabilityInformation message. The granularity (such as per band) and other details for capability of LP-WUS in IDLE/INACTIVE depends on RAN1 features list.

RRM measurement capability

Proposal 3: RAN2 capability defines RRM measurement fully offloading and RRM measurement relaxation for LP-WUS.

Proposal 4: Define RRM measurement capability as “Conditionally mandatory features without UE radio access capability parameters”. It is mandatory to support RRM measurement fully offloading and RRM measurement relaxation in RRC_IDLE/RRC_INACTIVE for UEs which support LP-WUS.

DISCUSSION

P1

-	Xiaomi ok with this.

-	Ericsson agree, think we do not need a spereate UE capability only for UE-ID based subgrouping.

-	QC wonders what is the difference for OOK-based or OFDM-based. QC think the wording ‘support of LP-WUS’ is not that clear.

P3, P4

-	Xiaomi think they are discussed in R1, think we can just wait. Vivo agree.

-	vivo think we can discuss whether we need separate capabilities for offloading and relaxation.

-	QC want to separate the capabilities for LPWUS reception and offloading/relaxation.

-	Ericsson think there is no much power saving gain if we only support LPWUS monitoring but not relaxation.

[Post129bis][218][LPWUS] UE capabilities (Huawei)

Intended outcome: Summary with proposals on RAN2 UE capabilities for LP-WUS

Deadline:  Long

R2-2502142	Summary of [Post129][212][LPWUS] Running CR for TS 38.304 (CATT)	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2502154	Whether/How to reduce the number of thresholds for LP-WUS monitoring and RRM relaxation/offloading 	vivo	discussion	Rel-19	NR_LPWUS-Core

=> The above two contributions are moved to 8.4.3

R2-2501955	Summary of [Post129][213] LP-WUS in MR-DC	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

=> Moved to 8.4.4

### 8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping, and entry/exit condition for LP-WUS monitoring

Sub-grouping related aspects

R2-2502324	Discussion on LP-WUS procedure and configuration	OPPO	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 5	Regarding the UE_ID based subgrouping formula for LP-WUS, revise description of Np:

Np is the number of subgroupNumForUEID for PEI, if configured and UE supports could monitor PEI in this cell; otherwise, Np is 1.

Proposal 6	Support LP-WUS and subgrouping for both DRX and eDRX UEs.

Proposal 7	For UE-ID based subgrouping for LP-WUS, UE_ID is defined as 5G-S-TMSI mod X, where X value is given as below.

<table>

Proposal 8	UE-ID based subgrouping for LP-WUS is not used when the UE has an active emergency PDU session.

R2-2502910	LP-WUS in idle and inactive	Ericsson	discussion

Noted

Proposal 1	LP-WUS is supported with eDRX without additional changes. I.e. the UE monitors LP-WUS during PTW instead of monitoring normal DRX during PTW as in regular eDRX.

Proposal 2	Ask RAN3 to provide 18 bits of the Hashed UE Identity Index Value IE for UE-ID based LP-WUS subgrouping in Rel-19.

Proposal 3	When the UE has an emergency PDU session LP-WUS is not used. Send an LS to CT1, RAN3 and SA2 to inform them.

R2-2502143	LP-WUS in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: LP-WUS is supported to be operated with eDRX simultaneously.

Proposal 2: For UE_ID based subgrouping for LP-WUS, UE_ID = 5G-S-TMSI mod X, X needs to be 2^18 for DRX case and 2^20 for eDRX case if supported.

Proposal 3: Send LS to RAN3 (CC SA2/CT1) to ask their concern on the agreed formula of UE_ID based subgrouping for LP-WUS.

Proposal 4: Confirm the principle that determines CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping.

DISCUSSIONS

On P5 from OPPO paper

-	Xiaomi think it requires more checking from UE and think the previous agreement is fine. Ericsson agree with Xiaomi. HW think this proposed change will create mis-match btw NW and UE. QC share the view from OPPO P5.

On emergency PDU session related issue

-	OPPO think there is some related discussion in another BO session, better to align.

-	QC think SA2 already remove any restriction so from R2 point of view we do no need to discuss. Vivo agree.

LP-WUS is supported with eDRX, FFS on exact impact if any

[AT129bis][201][LPWUS] Proposals for UE ID based sub-grouping (CATT)

Scope: Discuss remaining details for the formula of UE ID based sub-grouping, and also cover potential issue for emergency PDU session

Intended outcome: Summary with proposals in R2-2503031.

Deadline: Before Thursday CB

R2-2503031	[AT129bis][201][LPWUS] Proposals for UE ID based sub-grouping	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

DISCUSSION

P1, P2

-	NEC think either 31 or 32 can work, and think 32 matches better with UE ID design. NEC suggest to decide.

-	Ericsson think there can be a special group which cover all the UEs, so think 32 is better.

Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID=5G-S-TMSI mod X.

X is based on 32 subgrouping number. Details can be discussed in the running CR.

P3

-	Tejas prefer Option 1. QC want to further check.

-	CATT suggest to put in into the open issue list.

P4

-	HW think P4 depends on P3, so we need to wait until we conclude on P3. ZTE share this view.

-	Ericsson think we need to ask R3 whether it is feasible to have 5 additional bits for UE ID in the formula.  OPPO share this view.

-	ZTE agree with P4 to send a LS. NEC agree. OPPO ok to send LS.

-	vivo suggest we only inform conclusion. Ericsson think it is not sufficient to inform the LS.

Send LS to RAN3 (CC SA2/SA3) to inform our agreements on UE ID based subgrouping.

Correct the typo as following for the previous agreed formula of UE_ID based subgrouping for LP-WUS:

        - Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1.

Confirm the principle for determining CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping. Details will be discussed in the running CR.

[Post129bis][217][LPWUS] LS to RAN3 (CC SA2/SA3) to inform agreements on UE ID based subgrouping (NEC)

Intended outcome: Approve the LS

Deadline:  Short

LPWUS configuration

R2-2502308	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 9: The LP-WUS subgrouping configuration is provided in SIB1.

R2-2502097	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19

Noted

Proposal 2: Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

DISCUSSIONS

-	Apple explains that their P9 is already included in the current running CR. NEC think P9 can be updated to cover all LP-WUS related configuration except for measurement configurations. Ericsson not sure about putting all configuration in SIB1.

-	Xiaomi agree all the configuration is from SIB, and no dedicated configuration.

-	Samsung, IDT, CATT support HW P2.

-	Samsung ok to have the modified P9 as working assumption. Lenovo suggest to leave measurement configurations FFS.

All the LP-WUS related configurations except for measurement configurations are provided in SIB1. FFS the details on measurement configurations.

Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

Entry/exit condition of LPWUS monitoring

R2-2502447	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Noted

Proposal 7	Use existing Srxlev/Squal for MR measurement based entry/exit condition evaluation.

Proposal 8	Introduce Srxlev_lp/Squal_lp for LR measurement based entry/exit condition evaluation. The formula to derive Srxlev/Squal from measured MR value can be  used for deriving Srxlev_lp/Squal_lp from measured LR value.

R2-2502143	LP-WUS in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 5: Cell selection value, i.e., existing Srxlev/ Squal in TS 38.304, is used for serving cell quality by MR for conditions of LP-WUS monitoring, measurement relaxation and serving cell measurement offloading for LP-WUS UEs.

Proposal 6: Measured value is used for serving cell quality by LR for conditions of LP-WUS monitoring, measurement relaxation and serving cell measurement offloading for LP-WUS UEs.

DISCUSSION

P7 in QC paper, P5 in CATT paper

-	Ericsson think it is not crystal clear. QC clarify the intention is to reuse what we have in legacy. VDF from QC P7 is clear and good.

-	OPPO ask whether it is for serving cell meas offloading or for relaxation. CATT think it is for both.

-	Xiaomi, Samsung, OPPO understand these two proposals have the same intention. Xiaomi think CATT proposal is more clear.  Samsung Ok with these two proposals.

Use existing Srxlev/Squal for all MR measurement based entry/exit condition evaluation.

P8 in QC paper, P6 in CATT paper

-	Ericsson think we do not need offset values other than Qrxlevmin/ Qqualmin. QC, VDF agree this is useful. Ericsson think we should use ‘dB’ also for LR. NEC agree.

-	Vivo think there is not need to introduce additional. Samsung agree and support CATT proposal. Xiaomi, Apple agree.

-	CATT explains after some offline this seems not so critical issue.

Use measured value for all LR measurement based entry/exit condition evaluation.

Other aspects (e.g., enabling/disabling, activation/deactivation, etc.)

R2-2502659	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 2: Support dedicated LP-WUS monitoring (de-)activation per UE via RRC release message.

R2-2502097	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19

Noted

Proposal 3: The CN indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality by NAS signalling; the absence of indication means UE is allowed to use LP-WUS functionality

DISCUSSION

P3 from HW paper

-	Ericsson agree with P3 from HW paper, since network can notice some issue and should be able to disable LPWUS for those UEs. VDF also like HW P3.

-	Xiaomi think NW can assign UEs to different sub-groups so the issue is not that severe.

-	QC, Samsung think this is up to SA2. HW think it is just similar mechanism as PEI and think we can decide and inform SA2.

-	LG E think this is also related to potential handling of emergency PDU session, so think it is useful.

-	vivo think using NAS singaling is better.

P2 from IDT paper

-	VDF do not see the benefit of IDT P2 since NW may not know the future status/activities of UE. Xiaomi, QC share this view.

-	DCM support IDT from P2. OPPO ok to have this functionality. OPPO think if we ever have some mechanism then it is better to use RRC release. NEC share this view.

-	HW think if gNB do so then UE cannot use LPWUS function until after going back to connected.

-	Samsung think we already agree not to use dedicated signalling.

Agreements for LP-WUS in IDLE/INACTIVE

LP-WUS is supported with eDRX, FFS on exact impact if any Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID = 5G-S-TMSI mod X. X is based on 32 subgrouping number. Details can be discussed in the running CR. Send LS to RAN3 (CC SA2/SA3) to inform our agreements on UE ID based subgrouping. Correct the typo as following for the previous agreed formula of UE_ID based subgrouping for LP-WUS: - Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1. Confirm the principle for determining CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping. Details will be discussed in the running CR. All the LP-WUS related configurations except for measurement configurations are provided in SIB1. FFS the details on measurement configurations. Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes. Use existing Srxlev/Squal for all MR measurement based entry/exit condition evaluation. Use measured value for all LR measurement based entry/exit condition evaluation.

R2-2501831	LP-WUS in RRC_IDLE/INACTIVE	HONOR	discussion	Rel-19	NR_LPWUS-Core

R2-2501893	Discussion on LP-WUS in RRC_IDLE INACTIVE 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2501960	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2501997	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

R2-2502005	Remaining issues on LP-WUS paging monitoring	Xiaomi Communications	discussion

R2-2502014	IDLE/INACTIVE mode procedures for supporting LP-WUS	Tejas Network Limited	discussion	Rel-19

R2-2502097	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19

R2-2502143	LP-WUS in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2502155	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2502212	LP-WUS in IDLE and INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2502227	Remaining issues on LP-WUS in RRC IDLE or INACTIVE	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2502308	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2502324	Discussion on LP-WUS procedure and configuration	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2502447	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

R2-2502486	RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode	Sony	discussion	Rel-19	NR_LPWUS-Core

R2-2502597	Procedure and Configuration of LP-WUS in RRC IDLE/INACTIVE	Lenovo	discussion	Rel-19	NR_LPWUS-Core

R2-2502659	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2502743	Discussion on LP-WUS in RRC_IDLE and RRC_INACTIVE	Sharp	discussion	Rel-19

R2-2502901	Further Consideration on LP-WUS operation in IDLE/INACTIVE	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2502910	LP-WUS in idle and inactive	Ericsson	discussion

R2-2502976	Discussion on the LP-WUS handling for Emergency call back	NTT DOCOMO INC..	discussion	Rel-19	NR_LPWUS-Core

R2-2502977	Discussion on the LP-WUS capability issue within non-homogeneous deployment	NTT DOCOMO INC..	discussion	Rel-19	NR_LPWUS-Core

### 8.4.3	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

RRM measurement relaxation

R2-2502142	Summary of [Post129][212][LPWUS] Running CR for TS 38.304 (CATT)	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: General description and criteria of RRM relaxation and offloading for LP-WUS are captured in TS 38.304. Other details of RRM relaxation and offloading for LP-WUS are captured in RAN4.

Proposal 2: Send an LS to inform RAN4.

R2-2501998	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

Noted

Proposal 3. For the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation,

-	Separate thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

R2-2502156	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 9: The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ.

DISCUSSION

-	On CATT P1, ZTE think it is straightforward.

-	Nokia, vivo, Apple support Samsung P3. On Samsung P3, ZTE thik ‘if..’ should be removed. HW think R4 is also discussing this and think we should wait for R4 progress, Nokia think R2 can discuss threshold and R4 discuss requirements.

-	ZTE support P9 from vivo paper.

R2-2501894	Discussion on LP-WUS RRM 	NEC	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal-1: introduce separate entry/exit condition, i.e., the LP-WUS UE performs relaxed MR serving cell measurement when entry condition is fulfilled and stops performing relaxed MR serving cell measurement when exit condition is fulfilled.

Entry condition: based on MR measurement results and optional LR measurement results;

Exit condition: based on LR measurement results.

Proposal-2: same criterion should be applied to both MR serving cell and neighbour cell measurement relaxation.

Proposal-3: once the criteria for MR serving cell measurement offloading is met, then both MR serving cell and neighbour cell measurements should be stopped.

R2-2502351	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo	discussion	Rel-19

Noted

Proposal 4: RAN2 to consider ‘not fulfilling the entry condition’ as the exit condition for serving cell measurement relaxation.

Proposal 5: The entry condition for serving cell RRM measurement relaxation is configured independently with the neighboring cell RRM measurement relaxation.

R2-2502154	Whether/How to reduce the number of thresholds for LP-WUS monitoring and RRM relaxation/offloading 	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: RAN2 to discuss whether/how to reduce the number of thresholds for LP-WUS/WUR WI considering the following one or more directions:

•	Direction 1: Remove the LR based thresholds for all entry conditions in the above 4 RRM relaxation/offloading and LP-WUS monitoring procedures

•	Direction 2: Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Neighboring Cell RRM measurement relaxation

•	Direction 3: Merge the entry/exit conditions for Serving Cell RRM measurement offloading or Serving Cell RRM measurement relaxation or Neighboring Cell RRM measurement relaxation and LP-WUS monitoring

•	Direction 4: Merge the entry/exit conditions for Serving Cell RRM measurement offloading and the current threshold for stopping neighboring cell measurement

DISCUSSION

On vivo P1

-	IDT think with LPWUS we have separate receivers so it is unsafe to combine with legacy thresholds. CATT ok with high level principle and think we should check in details how to reduce. Ericsson think we should discuss how to reduce, Apple, HW, Samsung agree. Apple fine with all directions listed. HW not ready for direction 4, since it is too restrictive. ZTE think it is too early to down-select, Samsung agree. QC do not agree with direction 1, for the others we can check later.

-	Ericsson has concern on serving cell relaxation, and think it should not be configured together with serving cell measurement offloading. Apple think it is up to NW implementation.

[AT129bis][202][LPWUS] Proposals for RRM measurement relaxation (vivo)

Scope: Discuss remaining aspects for RRM measurement relaxation, including definition of the thresholds/conditions, can also discuss how to reduce the number of thresholds

Intended outcome: Summary with proposals in R2-2503032.

Deadline: Before Thursday CB.

R2-2503032	Discussion report on [AT129bis][202][LPWUS]	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

DISCUSSION

-	HW think the first one need to update to reflect R4 terminology of different types of LP WURs. Vivo think it is fine to update. Ericsson think we should follow R1/R4 when it comes to type of WUR.

-	ZTE ask if we consider low mobility condition when we merge the conditions for serving and neighbour cell measurement relx. Vivo explains that by now we haven’t agreed low mobility based condition, and think it is FFS.

-	Nokia has strong concern on the last agreement point.

RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any).

RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR mesasuring SSB threhold or both.

It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell.

The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ.

How to define LP-RSRP and LP-RSRQ is up to RAN1.

The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided, details up to running CR rapporteur and companies’ review.

Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

Chair observation: only one company have strong concern on the following discussion point

- Can further discuss Direction 3:

      - Direction 3a. merge LPWUS monitoring and serving cell offloading

      - Direction 3b. merge LPWUS monitoring and Rel-19 RRM relaxation

Other agreements on RRM measurement relaxation and offloading

RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any). RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR mesasuring SSB threhold or both. It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell. The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ. How to define LP-RSRP and LP-RSRQ is up to RAN1. The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided, details up to running CR rapporteur and companies’ review. Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

R2-2501894	Discussion on LP-WUS RRM 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2501967	Further discussion on the criteria for RRM measurement relaxation and offloading	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2501998	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

R2-2502006	Discussion on RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

R2-2502144	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2502156	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2502213	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2502228	Remaining issues on measurement offloading and relaxation	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2502309	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2502325	Discussion on RRM measurement in RRC IDLE and INACTIVE	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2502351	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo	discussion	Rel-19

R2-2502449	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

=> Revised in R2-2502995

R2-2502995	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

R2-2502660	Discussion on RRM measurement relaxation and offloading	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2502722	Discussion on RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2502744	Discussion on RRM measurement relaxation and offloading	Sharp	discussion	Rel-19

R2-2502757	Discussion on neighboring cell measurement with LR	InterDigital, Ericsson, Nokia, Sony, Vodafone, KT	discussion	Rel-19	NR_LPWUS-Core

R2-2502760	Discussion on RRM measurement relaxation and offloading for RRC_IDLE and INACTIVE	China Telecom	discussion

R2-2502911	LP-WUS and RRM measurements	Ericsson	discussion

R2-2502931	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core	R2-2501090

### 8.4.4	Procedures for LP-WUS in RRC_CONNECTED

Procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring.

On short DRX cycle

R2-2502477	LP-WUS in CONNECTED mode	InterDigital	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 6: For Option 1-1, when Short DRX cycle is used, the UE does not monitor LP-WUS outside the Active Time.

Proposal 7: For Option 1-2, the UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used.

R2-2502882	LP-WUS in RRC_CONNECTED	Nokia, Nokia Shanghai Bell	discussion	NR_LPWUS-Core

Noted

Proposal 1: For option 1-1, it is up to network configuring short DRX cycle with LP-WUS. Send an LS to RAN1 asking if LP-WUS is applicable to short DRX cycle as well as long DRX cycle for option 1-1.

Proposal 2: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. Send an LS to RAN1 asking if there are any technical concerns regarding configuring of short DRX cycle in option 1-2.

R2-2502448	LP-WUS operation in CONNECTED state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Noted

Proposal 1	For Option 1-1, short DRX can be configured, and UE will stop LP-WUS monitor when short DRX is started.

Proposal 2	For Option 1-2, short DRX is not configured.

DISCUSSION

Short DRX for Option 1-1

-	Xiaomi support IDT proposal 6. NEC, Tejas agree.

-	OPPO fine to follow DCP mechanism but think QC P1 is more clear. Lenovo agree.

Short DRX for Option 1-2

-	Xiaomi support IDT P7. Samsung, OPPO, CATT agree.

-	Nokia think R1 is discussing different options regarding the relationship btw LPWUS monitoring and DRX cycle, so we should ask R1.

-	CATT think we can have working assumption in R2 and inform R1.

For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used.

Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used.

Send an LS to RAN1 to inform the above conclusions, can revisit if needed based on RAN1 feedback.

Other aspects related to the procedure (e.g., collision handling, UAI, etc.)

R2-2502157	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 3: For LP-WUS Option 1-2, during the timer triggered by LP-WUS is running, periodic CSI/L1-RSRP report and SP CSI report behaviour are the same as legacy drx-onDurationTimer duration without DCP configuration, i.e. always report.

Proposal 4: The preferred value of time offset between LP-WUS and PDCCH reported via UAI is expected to be longer than the value for the minimum time gap between LP-WUS and MR to start PDCCH monitoring corresponding to the SCS reported via UE capability.

Proposal 5: For LP-WUS Option 1-1 and Option 1-2, RAN2 should consider the UE behaviour when all LP-WUS occasion collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR.

Proposal 6: For LP-WUS Option 1-1 and Option 1-2, in case all the LP-WUS occasion of an LP-WUS collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR, UE starts the corresponding drx-onDurationTimer for LP-WUS Option 1-1 and the corresponding new timer for LP-WUS Option 1-2.

Proposal 7: For LP-WUS Option 1-1, in case UE cannot detect all LP-WUS occasion of a LP-WUS, RAN2 assumes it is up to the network configuration for UE to determine whether to start the corresponding drx-onDurationTimer, e.g. lpwus-Wakeup.

R2-2502912	LP-WUS in connected	Ericsson	discussion

Noted

Proposal 4	In Option 1-1, when the UE is not able to monitor the LP-WUS occasion the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

Proposal 6	If configured, the UE can signal a preferred time offset via UAI signalling.

DISCUSSION

On collision cases

-	Xiaomi not sure if these collisions cases are valid for LPWUS because it is different receiver than MR. HW think we cannot assume UE is able to use MR and LR at the same time. OPPO think Xiaomi question is valid and think we in RAN2 cannot decide. CATT agree and think we should send LS to R1. WI rapp think the design depends on R1 but it can be discussed in R2. Vivo also OK to send LS to R1. IDT think we can first agree Ericsson P4 and then inform/ask R1.

-	HW support P4 from Ericsson. HW think LPWUS is a sequence then it can be detected by UE when it is transmitted by NW.

-	Apple wonders whether such collision handling impact MAC spec or not.

Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

Send LS to inform this working assumption, can also ask a) what are the cases when UE cannot monitor LP-WUS, b) whether UE can monitor LR and MR simultaneously.

On UAI

-	Xiaomi not sure whether UAI is preferred value or it is a rather static UE capability.

-	Vivo think R1 already agree UAI and UE capability.

-	Apple think it is still FFS in R1 what the UAI is providing (whether it is minimum value or maximum). Apple suggest to wait. Nokia think R2 can discuss UAI.

-	IDT, vivo ok to have Ericsson P6.

-	vivo suggest to ask R1 whether the UAI value is smaller or greater than UE capability. Nokia think it is sufficient to ask R1 what should be reported in UAI.

If configured, the UE can signal a preferred time offset via UAI signalling.

Ask RAN1 for further information regarding their conclusions.

[Post129bis][203][LPWUS] LS to R1 LP-WUS in RRC_CONNECTED (InterDigital)

Intended outcome: Approve the LS.

Deadline: Short

Dual DRX group

R2-2501769	Discussing on LP-WUS monitoring in Connected mode	Xiaomi	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 5	In CA with dual DRX groups, LP-WUS can be configured per DRX group or for all serving cells.

Proposal 6	LP-WUS configured in a DRX group is used to trigger PDCCH monitoring in all serving cells in the same DRX group.

Proposal 7	Whether UE can monitor LP-WUS for two DRX groups is up to UE capability.

R2-2501992	Remainng issues on LP-WUS in RRC_CONNECTED	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 2. RAN2 asks RAN1 whether LP-WUS and the secondary DRX group can be configured simultaneously, different from DCP.

DISCUSSION

-	Ericsson think this was not agreed for DCP and think it is good to have LPWUS and dual DRX together, e.g., for the case of FR1 + FR2 (latency improvement for busty data). LG E, OPPO, ZTE see the benefit.

-	Lenovo have a different view than Ericsson and think there would be impact, e.g.,  to CSI report. Lenovo think we do not support this for at least option 1-1, and also OK ask R1 first.

-	CATT think R1 discussed this during the week and there is no consensus, so think we can wait for R1 progress.

-	Nokia do not think R1 can discuss further since they only have 1 more meeting for R19 WID. Nokia think if there is no consensus then we should conclude like that.

-	OPPO think we can conclude in R2 and support LPWUS + dual DRX.

-	ZTE think we can agree in R2 and we ask R1 to see if they have concern.

-	Apple ask what is the intentioned behaviour, i.e., whether LPWUS is waking up to two cell groups simultaneously. Vivo think the two cell groups are waking up by LPWUS at the same time.

-	WI rapp suggest to conclude in R2 and ask R1. Nokia think the LS is not beneficial for now.

-	HW not ready send LS.

FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group

MRDC

R2-2501955	Summary of [Post129][213] LP-WUS in MR-DC	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells.

Proposal 2: For NR-DC, the LP-WUS in MCG and SCG can be configured independently.

Proposal 3: Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

DISCUSSION

-	Xiaomi generally ok with the proposals, for think for P2 it is open whether MCG and SCG can be configured together. ZTE think such configuration is possible and do not see showstopper. HW agree with ZTE.

-	Apple think such configurations depend on UE capability. QC agree.

For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells.

For NR-DC, the LP-WUS in MCG and SCG can be configured independently.

Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

Agreements on LP-WUS in RRC_CONNECTED

For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used. Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used. Send an LS to RAN1 to inform the above conclusions, can revisit if needed based on RAN1 feedback. Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2. Send LS to inform this working assumption, can also ask a) what are the cases when UE cannot monitor LP-WUS, b) whether UE can monitor LR and MR simultaneously. If configured, the UE can signal a preferred time offset via UAI signalling. Ask RAN1 for further information regarding their conclusions. FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells. For NR-DC, the LP-WUS in MCG and SCG can be configured independently. Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

R2-2501769	Discussing on LP-WUS monitoring in Connected mode	Xiaomi	discussion	Rel-19	NR_LPWUS-Core

R2-2501832	Procedures for LP-WUS in RRC_CONNECTED	HONOR	discussion	Rel-19	NR_LPWUS-Core

R2-2501895	Discussion on LP-WUS in RRC_CONNECTED 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2501961	Procedures for LP-WUS in RRC_CONNECTED	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2501992	Remainng issues on LP-WUS in RRC_CONNECTED	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2501999	Procedures for LP-WUS in RRC Connected Mode	Samsung	discussion	Rel-19

R2-2502016	LP-WUS operation in Connected mode	Tejas Network Limited	discussion	Rel-19

R2-2502145	Analysis on LP-WUS for RRC_CONNECTED	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2502157	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2502310	Procedures for LP-WUS in RRC_CONNECTED	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2502326	Discussion on LP-WUS in RRC_CONNECTED	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2502448	LP-WUS operation in CONNECTED state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

R2-2502471	Further discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2502477	LP-WUS in CONNECTED mode	InterDigital	discussion	Rel-19	NR_LPWUS-Core

R2-2502598	LP-WUS in RRC Connected Mode	Lenovo	discussion	Rel-19	NR_LPWUS-Core

R2-2502723	Discussion on LP-WUS operation in CONNECTED mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2502882	LP-WUS in RRC_CONNECTED	Nokia, Nokia Shanghai Bell	discussion	NR_LPWUS-Core

R2-2502912	LP-WUS in connected	Ericsson	discussion

## 8.11	Evolution of NR duplex operation: Sub-band full duplex (SBFD)

(NR_duplex_evo-Core; leading WG: RAN1; REL-19; WID: RP‑241614)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.11.1	Organizational

Incoming LS, Rapporteur input, including workplan, running CRs, etc..

Output of email discussion [Post129][217].

LS

R2-2501713	Reply LS on CSI-RS measurement with SBFD operation (R1-2501560; contact: MediaTek)	RAN1	LS in	Rel-19	NR_duplex_evo-Core	To:RAN4	Cc:RAN2

Noted

R2-2501731	SBFD information exchange among gNBs for CLI mitigation (R3-250888; contact: Huawei)	RAN3	LS in	Rel-19	NR_duplex_evo-Core	To:RAN2	Cc:RAN1

DISCUSSION

-	HW explains that R3 request us to introduce the configuration/parameter but currently we still need some input from R1.

-	OPPO think we can wait and think no need to reply. LG E agree.

Noted

R2-2501738	LS on L1 CLI measurement (R4-2502632; contact: Huawei)	RAN4	LS in	Rel-19	NR_duplex_evo-Core	To:RAN1	Cc:RAN2

Noted

Running CRs

R2-2501851	TS 38300 Running CR for SBFD	CATT	draftCR	Rel-19	38.300	18.5.0	NR_duplex_evo-Core

Noted, will update/endorse in post meeting email discussion

[Post129bis][211][SBFD] Running CR for 38.300 (CATT)

Intended outcome: Updated running CR based on new agreements for endorsement

Deadline:  Long

R2-2502549	RRC running CR for Evolution of NR duplex operation (SBFD)	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.5.1	B	NR_duplex_evo-Core

=> Revised in R2-2502978

R2-2502978	RRC running CR for Evolution of NR duplex operation (SBFD)	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.5.1	B	NR_duplex_evo-Core

Noted, will update/endorse in post meeting email discussion

[Post129bis][212][SBFD] Running CR for 38.331 (Huawei)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

R2-2502279	TS38.304 impacts on supporting Rel-19 SBFD	NEC	discussion	Rel-19	NR_duplex_evo-Core

DISCUSSION

-	NEC explains there is no 304 impact so no need for running CR.

Noted

No running CR is needed for TS38.304, unless RAN1/RAN4 indicate that it is needed.

R2-2502591	MAC running CR for Evolution of NR duplex operation: SBFD	Samsung	draftCR	Rel-19	38.321	18.5.0	B	NR_duplex_evo-Core

DISCUSSION

-	Apple has concern on CSI-RS based SBFD, think R1 and R4 didn’t agree. Apple think if we do so we should first ask R1 and R4. Samsung point out this is taken into account with the EN ‘Editor’s Note: Will reflect agreement(s), if any, on CSI-RS based CFRA.’

Noted, will update/endorse in post meeting email discussion

[Post129bis][213][SBFD] Running CR for 38.321 (Samsung)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

R2-2502567	Introduction of SBFD UE capabilities (Running CR)	Ericsson	draftCR	Rel-19	38.306	18.5.0	B	NR_duplex_evo-Core

R2-2502568	Introduction of SBFD UE capabilities (Running CR)	Ericsson	draftCR	Rel-19	38.331	18.5.1	B	NR_duplex_evo-Core

DISCUSSION

-	Xiaomi think the CR only contain R1/R4 capabilities, so we do it in mega CR. Samsung share this view.

-	OPPO think for some cases, e.g., CA, we may need some discussions in R2.

Noted, with the understanding that running CR is needed only when we introduce new capabilities form R2 point of view.

Output of email discussion [Post129][217]

R2-2502210	Summary of [Post129][217][SBFD] List of open issues of RRC impact	Huawei, HiSilicon (Rapporteur)	discussion	Rel-19	NR_duplex_evo-Core

Noted

[Proposal 1] On RO type signaling for CFRA, discuss/conclude on:

1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig.

2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated.

3. Within the corresponding RACH configuration for the additional ROs, one bit is used to indicate the RO type for CFRA.

4. For CFRA triggered by PDCCH order including UL early sync, RAN2 can wait/follow RAN1 agreement.

[Proposal 2] Discuss option issue on the UE behaviour when the signalling on the RO type and RSRP threshold are absent. Consider solution options 1) either NT indication on RO type or on RSRP threshold is madatorily present. 2) When both NT indication on RO type and RSRP threshold are absent, the SBFD-aware UE should prioritize the additional RO over the legacy RO.

[Proposal 3]  Choose one option below on whether confirm WA "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. " and discuss related RRC impact:

Option 1: Confirm the WA "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. ".

Option 2: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for Position SI request and SI request.

[Proposal 4] RAN2 waits for conclusion from RAN3 and RAN1 on NZP CSI-RS resources configuration and SBFD frequency configuration information.

DISCUSSION

P1

-	LG E ok with P1 in general. LG E not sure whether LTM is supported with SBFD, but if they are used together then point 3 in the proposal need to be further checked. HW think we do not need to discuss the combination of the features in this stage. ZTE think we can disucss.

-	QC ok with point 1, 2 and 4 in P1.

-	Ericsson ok with point 1 and 2, for point 4 not sure whether LTM and SBFD can be configured together.

-	ZTE point 4 is already agreement in R1, so no need to agree again in R2. OPPO think R1 agreement is only for PDCCH order.

-	CATT think for point 4 we should also discuss the RA type indicating by RRC parameter. HW think if R1 has such configuration in the parameter list we can add.

-	Apple understand UE need to know which option (1 or 2) is used in the cell for SBFD configuration, so UE need to combine SIB configuration with other configurations as included in point 1 and 2.

-	Nokia think point 1 and 2 already clear.

On RO type signaling for CFRA

- 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig.

- 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated.

P2

-	LG E ok with either option 1 or 2, but option 2 is better.

-	vivo agree with option 1 and think option 2 is not needed. IDT, CMCC, CATT, Ericsson prefer option 1. QC do not agree.

-	QC, ZTE prefer option 2.

-	Nokia, Samsung, Sony think there is one more option that UE select first available RO. HW explain there is limited support in email discussion so not listed here.

-	Ericsson think if we do not make it mandatory then it should be up to UE implementation.

When both NW indication on RO type and RSRP threshold are absent, it is up to UE implementation to select the RO type.

P3

-	HW think there is benefit for cell edge UE even for SI request case. CATT, vivo think there is benefit for positioning SI to get A-GNSS assist info.

-	LG E think detailed approach is needed for SI request, e.g., to handle CLI, and think time is limited to finalize this in the WI. Apple do not see the benefit in SI request case, and there is great spec impact. Xiaomi, Samsung share this view.

Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

P4

-	LG E think we can wait, no need for an agreement.

Agreements on SBFD RACH aspects

On RO type signaling for CFRA - 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig. - 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated. When both NW indication on RO type and RSRP threshold are absent, it is up to UE implementation to select the RO type. Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

### 8.11.2	Random access in SBFD

RAN2 impacts to support SBFD operation to support random access in SBFD symbols by UEs in RRC _CONNECTED mode and RRC_IDLE/INACTIVE mode.

On RSRP threshold for RO type selection (e.g., configuration, related procedure, etc.)

R2-2502316	Random Access Operation of SBFD	Nokia Corporation	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: For Option 2, in case the legacy ROs and the additional ROs have the same value for one of the RACH configuration mandatory parameters, only the legacy RACH configuration will be configured with this parameter.  For Option 2, in case the SBFD aware UEs did not receive a dedicated mandatory parameter in the additional RACH configuration, the SBFD-aware UEs determine this dedicated parameter from the legacy RACH configuration.

Proposal 2: Since the RACH configuration Option 1 is focused on reducing the latency of RA for SBFD UEs in good coverage area, we propose SBFD UEs selects the additional RO for initial access when the measured RSRP is above the RSRP threshold.

Proposal 3:  For RACH configuration Option 2, in case the additional prachConfigurationIndex indicates long PRACH preamble formats, the UE selects the additional ROs for initial access when the measured RSRP is below the RSRP threshold.

R2-2501860	Random Access for SBFD Operation	NEC	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1:	The RO type (legacy RO or additional RO) indication is provided by SIB1 for the case of CBRA. FFS whether UE specific dedicated RRC signalling is needed.

Proposal 2:	Selection of the additional RO type below and above the configured SSB RSRP are both supported.

Proposal 3: The network configures when the additional RO to apply, the SSB measurement result is either below or above the configured SSB RSRP threshold.

R2-2502000	Random access in SBFD	Samsung	discussion	Rel-19

Noted

Proposal 2. NW may indicate whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold.

DISCUSSION

P2 from Samsung

-	Apple think it is reasonable. Ericsson understand P2 means we use a choice structure to indicate, and agree with the proposal.

-	ZTE think the main case is for cell center UEs, no need for duplicated functionality for cell edge. IDT, Lenovo share this view. IDT agree with Nokia proposal.

-	Nokia explains that in their proposal it goes one step further to provide in which resource configuration we use ‘above’ or ‘below’ the threshold as the condition. QC understand the difference btw Nokia proposal and Samsung proposal is that the latter requires explicit signalling. QC prefer Nokia proposal. Samsung think UE may not support all the options.  Ericsson think Nokia proposal require more complexity in UE.

-	HW do not think it is good to use implicit signalling. LG E share this view and do not want to tie the threshold configuration with other configurations. Apple agree.

NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold.

RO type selection before or after RA type selection

R2-2501849	Random Access in SBFD symbols	CATT	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

Proposal 2: For SBFD-aware UE, whether msg1/3 repetition is applied is determined based on the selected RO type.

R2-2501878	Impacts on the random access by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 7: Support to select RO type after the RA type selection.

DISCUSSION

-	LG E has concern on HW P7. Ericsson, IDT, Samsung share this concern. Apple think there is no big issue from UE processing point of view.

-	Charter, Apple, ZTE, OPPO support HW proposal, since R1 did not agree 2step RA.

-	QC slightly prefer CATT proposal considering the complexity of the procedure.

-	MAC rapp think CATT proposal is simpler from spec impact point of view. Sharp agree.

-	OPPO think we should consider both spec impact and UE implementation.

-	CMCC think this agreement relate to fallback mechanism.

-	HW suggest to have post meeting email discussion on this topic. Samsung do not think this is needed, as pros and cons have been discussed sufficiently.

-	Samsung not sure how to implement the HW proposal in MAC. CATT agree.

Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

On RO type fallback

R2-2501945	Discussion on Random Access in SBFD	Sharp	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1	RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold.

R2-2502967	Random Access in SBFD	Lenovo	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 6: Switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is not supported.

DISCUSSION

-	Nokia, Ericsson, IDT, Samsung support P1 from Sharp.

-	LG E, CATT support Lenovo proposal.

-	Charter wonders why additional RO can succeed after fail on legacy RO.

-	Lenovo ask what is the procedure if UE still fails after fallback to SBFD RO.

-	CATT ask what if UE fallback from 2step to 4step, can UE again fallback to additional RO after 4step. ZTE think this is valid.

RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

RA-RNTI collision

R2-2502495	Random access for SBFD Operation	Sony	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 6: Simply introduce a configurable offset for the symbol index (s_id) in the RNTI equation, and the offset is signalled in SIB1 as a part of PRACH resource configuration for SBFD-aware UEs.

R2-2502565	SBFD RA aspects	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 10	No need to update the RA-RNTI formula for SBFD RA configuration.

DISCUSSION

-	Sony think we need to handle the collision. Xiaomi, Nokia agree.

-	Ericsson think in R1 there is understanding that it is up to NW implementation. Ericsson think if we enhance there would be more issues. OPPO agree.

R2-2502510	Remaining issues for RACH in SBFD	Apple	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 7: RAN2 should consult with RAN1 and RAN4 whether to support CSI-RS based CFRA on SBFD symbols.

DISCUSSIOS

-	OPPO think R1 already agreed CSI-RS meas on SBFD symbol

-	LG E think this issue is valid, and this requires more discussions in R1/R4 regarding measurement and reporting.

-	Ericsson think we do not need to discuss this, and it is up to UE implementation which is quite simple.

-	ZTE think this concern is valid but think we can FFS and do not think a LS is needed. Xiaomi agree.

-	Apple think current running CR already covers CSI-RS based RACH so this is not right. Apple encourages companies to do checking internally and can discuss in the future.

-	HW has sympathy on Apple comments and think MAC spec editor can remove this related part if that is controversial.

Additional agreements on SBFD RACH aspects

NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold. Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources. RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

R2-2501797	Discussion on RACH in SBFD	Xiaomi	discussion	Rel-19

R2-2501849	Random Access in SBFD symbols	CATT	discussion	Rel-19	NR_duplex_evo-Core

R2-2501860	Random Access for SBFD Operation	NEC	discussion	Rel-19	NR_duplex_evo-Core

R2-2501878	Impacts on the random access by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

R2-2501945	Discussion on Random Access in SBFD	Sharp	discussion	Rel-19	NR_duplex_evo-Core

R2-2502000	Random access in SBFD	Samsung	discussion	Rel-19

R2-2502082	Discussion on random access procedure in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2502316	Random Access Operation of SBFD	Nokia Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2502387	Discussion on random access procedure in SBFD	vivo	discussion	Rel-19	NR_duplex_evo-Core

R2-2502394	Remaining issues of SBFD RACH procedure	OPPO	discussion	Rel-19	NR_duplex_evo-Core

R2-2502495	Random access for SBFD Operation	Sony	discussion	Rel-19	NR_duplex_evo-Core

R2-2502510	Remaining issues for RACH in SBFD	Apple	discussion	Rel-19	NR_duplex_evo-Core

R2-2502565	SBFD RA aspects	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

R2-2502566	CSI-RS measurements and SBFD operation in CA and DC	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

R2-2502588	Views on random access for SBFD	Qualcomm Incorporated	discussion	NR_duplex_evo-Core

R2-2502642	Discussion on Random Access operation in SBFD	InterDigital, Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2502706	Discussion on random access in SBFD	CMCC	discussion	Rel-19	NR_duplex_evo-Core

R2-2502850	Discussion on Random Access procedure for SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2502967	Random Access in SBFD	Lenovo	discussion	Rel-19	NR_duplex_evo-Core

### 8.11.3	Other aspects

Other RAN2 impacts with SBFD if not covered by the previous agenda items.

SBFD in DC

R2-2502083	Discussion on multiple carrier and measurements in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: Support SBFD cell(s) to be configured in DC, including NR-DC, (NG)EN-DC and NE-DC.

Proposal 2: RAN2 supports the following:

	In EN-DC and NGEN-DC, only the SN can configure SBFD;

	In NE-DC, only the MN can configure SBFD;

	In NR-DC, both the MN and the SN can configure SBFD.

R2-2502318	Other impacts by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: Send an LS to RAN1 on whether SBFD can work with DC case and if yes, whether any special handling is needed.

DISCUSSION

-	LG E, CATT share the view from HW, and think we can check with R1.

-	IDT do not support DC since there is R3 signaling design, think DC is not so critical.

-	Ericsson think either DC or CA critical and think R4 conclusion is not clear yet. Ericsson think there are more issue from UE point of view. Ericsson want to ask both CA and DC if we send LS, QC share this view. CATT think we only ask DC is sufficient, because R1 already discussed CA.

-	CATT think we need to finalize to complete MAC spec.

-	QC also agree with HW proposal.

-	Samsung do not think DC is within WID scope and think RP involvement is needed. Vivo share this view.

-	HW also think R1 is currently working on CA, so we do not need to ask CA. ZTE agree.

-	Ericsson think it is clear NW can configured SBFD in MCG.

Send an LS to RAN1 (CC RAN3 and RAN4) on whether SBFD and DC can be configured simultaneously, and whether there is any issue for such configuration.

[Post129bis][219][SBFD] LS to RAN1 (CC RAN3/RAN4) on simultaneous configuration of SBFD and DC (Huawei)

Intended outcome: Approve the LS

Deadline:  Short

MAC CE design SP CLI Measurement Resource Set Activation/Deactivation

R2-2502801         Other Aspects of SBFD      Samsung              discussion            Rel-19              NR_duplex_evo-Core

Noted

Proposal 2: RAN2 introduce the new MAC CE for semi-persistent CLI resource activation/deactivation which include at least following information.

-        A/D field: indicates the activate or deactivate of the indicated CLI resources

-        Source cell/BWP ID which the CLI resources configuration is provided

-        CLI measurement resource type (either SRS-RSRP or CSI-RSSI)

-        CLI measurement resource set ID

-        CLI measurement resource(s) ID

Proposal 9: The configured SP CLI measurement resource sets are initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync.

Proposal 10: RAN2 consider the above MAC CE format for SP CLI Measurement Resource Set Activation/Deactivation as a baseline.

R2-2501883	Other aspects of SBFD	Xiaomi	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 3: SP CLI measurement resource set activation/deactivation MAC CE includes following fields: A/D, Serving Cell ID, BWP ID, CLI measurement resource set ID (for CLI-RSSI or SRS-RSRP measurement), TCI State IDs.

R2-2502589	Other aspects of SBFD	Qualcomm Incorporated	discussion	NR_duplex_evo-Core

Noted

Proposal 3: In the new SP CLI measurement resource set activation/deactivation MAC CE, the following fields are included, i.e., A/D field, serving cell/BWP ID field, CLI-RSSI or SRS-RSRP resource set ID field and the corresponding TCI state ID fields.

R2-2501850         Discussion on other aspects of SBFD            CATT     discussion              Rel-19    NR_duplex_evo-Core

Noted

Proposal 2: RAN2 to discuss which option to support for the new SP CLI MAC CE:

-              Option 1: Only one type of resource (L1 CLI-RSSI or L1 CLI-SRS-RSRP) can be included in one MAC CE,

-              Option 2:  Both types of SP L1 CLI measurement resources (L1 CLI-RSSI and L1 CLI-SRS-RSRP) can be included in one MAC CE.

DISCUSSION

P3 from Xiaomi paper

-	MAC editor suggest that Xiaomi proposal is quite simple.

-	Samsung want to confirm it as agreement. QC share this view.

P2 from CATT paper

ZTE ask for CATT P2 how do we introduce TCI state in the MAC CE, and only want to indicate one type of resource. LG E think Option 1 is sufficient but there is R1 dependency.

-	LG E suggest we try to make a WA based on option 1.

-	Samsung not sure how Option 2 can work from signalling design point of view.

-	OPPO think if RAN1 agree with simultaneous configuration of two types of resources we can design the MAC CE to serve that purpose.

Working assumption: The configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync.

SP CLI measurement resource set activation/deactivation MAC CE includes following fields: A/D, Serving Cell ID, BWP ID, CLI measurement resource set ID (for CLI-RSSI or SRS-RSRP measurement), TCI State IDs.

Agreements on other SBFD aspects

Send an LS to RAN1 (CC RAN3 and RAN4) on whether SBFD and DC can be configured simultaneously, and whether there is any issue for such configuration. Working assumption: The configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync. SP CLI measurement resource set activation/deactivation MAC CE includes following fields: A/D, Serving Cell ID, BWP ID, CLI measurement resource set ID (for CLI-RSSI or SRS-RSRP measurement), TCI State IDs.

R2-2501850	Discussion on other aspects of SBFD	CATT	discussion	Rel-19	NR_duplex_evo-Core

R2-2501883	Other aspects of SBFD	Xiaomi	discussion	Rel-19	NR_duplex_evo-Core

R2-2502083	Discussion on multiple carrier and measurements in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2502318	Other impacts by the evolution of duplex operation	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

R2-2502388	SBFD other aspects	vivo	discussion	Rel-19	NR_duplex_evo-Core

R2-2502395	Discussion on the SBFD related measurement and BFR	OPPO	discussion	Rel-19	NR_duplex_evo-Core

R2-2502644	Discussion on resource configuration in SBFD	InterDigital, Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2502801	Other Aspects of SBFD	Samsung	discussion	Rel-19	NR_duplex_evo-Core

R2-2502851	Other aspects on SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2502918	Other aspects of SBFD	Nokia	discussion	Rel-19	NR_duplex_evo-Core

## 8.12	NR MIMO Phase 5

(NR_MIMO_Ph5-Core; leading WG: RAN1; REL-19; WID: RP-242394)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.12.1	Organizational

LSs and rapporteur input, including workplan, running CRs, etc.

Output of email discussion [Post129][208].

LS

R2-2501705	LS to RAN2 on MAC impacts for Rel-19 NR MIMO Ph5 (R1-2500846; contact: Samsung)	RAN1	LS in	Rel-19	NR_MIMO_Ph5	To:RAN2

Noted

Work plan

R2-2502715	Work Plan for Rel-19 on NR MIMO Phase 5	CMCC,Samsung,MediaTek	Work Plan	Rel-19	NR_MIMO_Ph5-Core

Noted

Running CRs

Chair: Plan is to directly note these draft CRs, and update/endorse with post meeting email discussions. Also, we will discuss and prepare open issue lists (e.g., per spec) after this meeting.

R2-2502545	Running CR for MIMO Phase 5	Ericsson	CR	Rel-19	38.331	18.5.1	5306	-	B	NR_MIMO_Ph5-Core

Noted, will be updated after the meeting

[Post129bis][214][ MIMO_Ph5] Running CR for 38.331 (Ericsson)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

R2-2502664	Introduction of Rel-19 MIMO	Samsung	draftCR	Rel-19	38.321	18.5.0	B	NR_MIMO_Ph5-Core

Noted, will be updated after the meeting

[Post129bis][215][ MIMO_Ph5] Running CR for 38.321 (Samsung)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

R2-2502716	Draft Running 38300 CR for Rel-19 MIMO Phase 5	CMCC,Samsung,MediaTek	draftCR	Rel-19	38.300	18.5.0	B	NR_MIMO_Ph5-Core

=> Revised in R2-2502988

R2-2502988	Draft Running 38300 CR for Rel-19 MIMO Phase 5	CMCC	draftCR	Rel-19	38.300	18.5.0	B	NR_MIMO_Ph5-Core

Noted, will be updated after the meeting

[Post129bis][216][ MIMO_Ph5] Running CR for 38.300 (CMCC)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

Output of email discussion [Post129][208]

R2-2502546	Report of [Post129][208][ MIMO_Ph5]	Ericsson	discussion

=> Moved to 8.12.3

### 8.12.2	Asymmetric DL sTRP/UL mTRP

RRC/MAC aspects related to asymmetric DL sTRP/UL mTRP

Remaining aspects on MAC CE design

Number of TCI states

R2-2503033	Discussion on Asymmetric DL sTRP/UL mTRP	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1：We think there is no need to add a maximum number restriction of the TCI states indicated by the PL offset MAC CE.

R2-2502834	Discussion on remaining issues on Asymmetric DL sTRP/UL mTRP	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: Select one from the following two options for the new MAC CE format.

- Option1: Use 1-bit reserved field in the new MAC CE to indicate whether the next TCI State/PL offset exists, as shown in Figure 3.

- Option2: Use a new field in the new MAC CE to indicate the exact number of PL offsets, as shown in Figure 4.Proposal 2: The new MAC CE for PL offset update includes the impacted TCI State ID(s).

DISCUSSONS

-	Apple support P1 from CMCC. CATT agree.

-	Nokia do not support P1 from HW paper, since we can rely on length field. HW think the length filed is not typically used for that purpose. LG E disagree with HW comments.

-	LG E points out the maximum number depends on the number of configured TCI states.

No need to add a maximum number restriction of the TCI states indicated by the PL offset MAC CE.

Applicable channels

R2-2502146	Discussion on Asymmetric DL sTRP UL mTRP	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: The PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

DISCUSSONS

-	Apple think we should make it clear it is for PDCCH-ordered CFRA.

-	Ericsson think this is just to align the understanding and there is no impact to R2 spec. Samsung agree.

RAN2 understand the PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order CFRA.

Whether a note is need in the MAC spec

R2-2502392	Discussion on the remaining issues of the pathloss offset update	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: To clarify the NOTE 1 in 5.4.6 of TS 38.321 that the pathloss used for triggering PHR includes both the measurement result of the pathloss reference and the pathloss offset.

R2-2501993	Consideration on Asymmetric DL sTRP/UL mTRP	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1. RAN2 consider to revert the following agreement in the last meeting.

	RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

DISCUSSIONS

-	LG E ok now to have clarifications in note. Samsung think we need further checking regarding the exact wording of the note.

-	Xiaomi think we should clarify the PL offset is only for triggering PHR type 1. Ericsson share this view.

-	QC think there is the case where PL offset is not configured at all.

We will capture in a note to reflect the previous understanding ‘RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset.’. FFS on exact wording.

WA of PL update

R2-2502665	Discussion on Asymmetric DL sTRP/UL mTRP	Samsung	discussion	Rel-19	NR_MIMO_Ph5

Noted

Proposal 2: RAN2 assumes that UE applies the latest PL offset value received in RRC or MAC CE

R2-2502866	Consideration on the Remaining Issues of PL Offset MAC CE	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: RAN2 to discuss how to capture the working assumption that the “UE applies the latest PL offset value received in RRC or MAC CE.”

Proposal 1a: RAN2 to indicate the working assumption that the “UE applies the latest PL offset value received in RRC or MAC CE.” to RAN1 and to confirm with RAN1 whether this would be captured in RAN1 spec.

DISCUSSIONS

-	Samsung explain that with P2 in their contribution there is no need for further spec change. OPPO agree.

-	OPPO think the current MAC CR (deliver the updated value to lower layer) is not sufficient, and think UE also need to update the stored value in RRC. Apple do not agree.

-	HW think there is issue in HO case when target gNB do not know whether UE already updates the value based on MAC CE. Nokia think the case off RRC resume should also be considered.

-	Ericsson ok to confirm the WA, and think other details can perhaps be handled by need code or field description. CMCC agree.

From RAN2 point of view, UE applies the latest PL offset value received in RRC or MAC CE.

On 2TA

R2-2502665	Discussion on Asymmetric DL sTRP/UL mTRP	Samsung	discussion	Rel-19	NR_MIMO_Ph5

Noted

Proposal 4: For 2TA in asymmetric DL sTRP/UL mTRP scenario, Rel-18 2TA operation is applied with the following RRC changes:

•	remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value;

•	a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

DISCUSSIONS

-	Samsung think with these agreements, the impact is in RRC.

-	Samsung explains that after offline the concern was removed, and suggested to add that this is with pathloss offset configured.

For 2TA in asymmetric DL sTRP/UL mTRP scenario with pathloss offset configured Rel-18 2TA operation is applied with the following RRC changes:

- •	remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value;

- •	a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario.

On RACH

R2-2502834	Discussion on remaining issues on Asymmetric DL sTRP/UL mTRP	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 6: For PRACH transmission, PL offset is applied only to PDCCH-order CFRA, and no further MAC impact is needed.

R2-2501943	Discussion on Asymmetric DL sTRP/UL mTRP	Xiaomi	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 2: When PDCCH-order indicates the pathloss offset, RSRP used for the selection of RACH procedure combines both RSRP measured for pathloss reference and pathloss offset.

R2-2502317	RAN2 Aspects of Asymmetric DL sTRP/UL mTRP	Nokia Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 3: Discuss  the impact of PL offsets on UL transmissions of RACH procedures, such as PRACH, Msg3 (for the 4-step RACH procedure) and MsgA (for the 2-step RACH procedure).

Proposal 4: If RAN2 determines that PL offset impacts the RACH procedures, then RAN2 should examine whether the PL offset affects the RSRP comparison in RA resource selection.

DISCUSSIONS

-	Samsung think HW P6 already confirmed by RAN1, and think with that we need to further check the MAC spec impact.

-	On Xiaomi P2, Samsung agree we need to discuss the RSRP in the RACH procedure. Ericsson agree.

-	Apple think there is no issue for PDCCH order RACH. HW, QC, LG E share Apple’s view.

-	QC point out the case for UE initiated RACH is quite difficult. ZTE think we can first decide whether this is only applicable to PDCCH-order CFRA, Nokia agree.

-	LG E wonders how do we know the association btw RACH resource and the TCI state (thus PL offset).

For PRACH transmission, PL offset is applicable only to PDCCH-order CFRA.

Agreements on Asymmetric DL sTRP/UL mTRP

No need to add a maximum number restriction of the TCI states indicated by the PL offset MAC CE. RAN2 understand the PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order CFRA. We will capture in a note to reflect the previous understanding ‘RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset.’. FFS on exact wording. From RAN2 point of view, UE applies the latest PL offset value received in RRC or MAC CE. For 2TA in asymmetric DL sTRP/UL mTRP scenario with pathloss offset configured Rel-18 2TA operation is applied with the following RRC changes: - • remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value; - • a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario. For PRACH transmission, PL offset is applicable only to PDCCH-order CFRA.

R2-2501943	Discussion on Asymmetric DL sTRP/UL mTRP	Xiaomi	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2501993	Consideration on Asymmetric DL sTRP/UL mTRP	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502063	Further discussion on asymmetric DL sTRP and UL mTRP	SHARP Corporation	discussion	NR_MIMO_Ph5-Core

R2-2502146	Discussion on Asymmetric DL sTRP UL mTRP	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502167	Discussion on MAC CE impact for asymmetric DL sTRP/UL mTRP scenarios	vivo	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502304	Asymmetric DL/UL mTRP user plane impacts	Ericsson	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502317	RAN2 Aspects of Asymmetric DL sTRP/UL mTRP	Nokia Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502373	Discussion on PL offset	Lenovo	discussion	Rel-19

R2-2502392	Discussion on the remaining issues of the pathloss offset update	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502496	Enhancement for Asymmetric DL sTRP/UL mTRP	Sony	discussion	Rel-19	NR_MIMO_Ph5

R2-2502536	Discussion on asymmetric DL sTRP/UL mTRP	China Telecommunications Corp.	discussion

R2-2502543	Discussion on UL only mTRP 	Qualcomm Incorporated	discussion

R2-2502665	Discussion on Asymmetric DL sTRP/UL mTRP	Samsung	discussion	Rel-19	NR_MIMO_Ph5

R2-2502713	Discussion on Asymmetric DL sTRP/UL mTRP	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

=> Revised in R2-2503033

R2-2502827	Discussion on PL offset value for Asymmetric DL sTRP/UL mTRP	ASUSTeK	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502834	Discussion on remaining issues on Asymmetric DL sTRP/UL mTRP	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502866	Consideration on the Remaining Issues of PL Offset MAC CE	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2503033	Discussion on Asymmetric DL sTRP/UL mTRP	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

### 8.12.3	Others

Other issues if not covered by the previous agenda items.

UE initiated beam report

DRX

R2-2501986	Enhancements for UE-initiated/event-driven beam management	Ofinno, LLC	discussion

Noted

Proposal 1: In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time when a new UCI for UE-initiated beam reporting is sent on first PUCCH.

Proposal 2: To discuss whether UE transmits UE-initiated CSI report outside of DRX active time.

R2-2502314	Discussion on UE-initiated Beam Reporting	Apple	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 3: There may be some impact on CDRX operation if the event triggered BR is triggered outside DRX active time.

DISCUSSION

-	HW agree with P1 from Ofinno. HW wonders whether UE can transmit the new UCI if it is NOT in active time. Also HW ask whether the active time ends after UE sends the PUSCH.

-	Apple think UE can transmit the new UCI outside DRX active time, but it should go back to active time to receive the UL grant for mode A. QC, Nokia share this view.

-	LG E think new UCI is similar to SR in legacy so similar behaviour should apply.

-	Samsung wonders whether we need to further check mode B.

In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time after a new UCI for UE-initiated beam reporting is sent on first PUCCH.

UL Skipping

R2-2502393	Clarification on the uplink grant used for the UE initiated beam report	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal: RAN2 is kindly requested to confirm the understandings that the pre-configured CG PUSCH for the UE initiated beam report in Mode-B does not carry MAC PDU.

R2-2502666	Discussion on UE-initiated Beam Reporting and CSI enhancement	Samsung	discussion	Rel-19	NR_MIMO_Ph5

Noted

Proposal 5: The current UL skipping procedure already exclude UEI BR since it is transmitted in UCI, no additional MAC impact.

Proposal 6: The type-1 CG allocated for the mode-B UEI BR is transparent to MAC and excluded from MAC UL-SCH transmission procedure.

FFS if any other MAC impact for UL skipping

R2-2503036	Way forward on the uplink grant used for UEIBR	OPPO, Huawei, Ofinno, Samsung, ZTE, Qualcomm, LG, Nokia, ASUSTek	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

DISCUSSION

-	Ericsson ask whether we send to R1. Nokia and OPPO think this is common understanding in R2 and no need to send LS.

Confirm the following RAN2 understandings:

      - The CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH).

      - The DG PUSCH carrying the beam report of Mode-A carries MAC PDU (i.e. UL-SCH) as legacy.

BWP

R2-2502833	Enhancements for UE-initiated/event-driven beam management	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 7: The UE continues to perform CSI measurements for the UEIBM procedure when the active BWP is the dormant BWP.

Proposal 8: If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP.

DISCUSSION

-	LG E agree and think this is same with legacy behaviour. Samsung agree.

-	Apple wonders if any spec is needed with P7 and P8. Samsung think we need to add something.

The UE continues to perform CSI measurements for the UEIBM procedure when the active BWP is the dormant BWP.

If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP.

R2-2502828	Discussion on MAC impact regarding UEI reporting	ASUSTeK	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal:	For UE-initiated/event-driven beam reporting Mode-B, the MAC entity shall start or restart bwp-InactivityTimer associated with the Cell where the CG-PUSCH resource of a CSI report configuration is configured if the MAC entity performs the PUCCH transmission of the CSI report configuration.

DISCUSSION

-	LG E think UEI BR is different case and think UE does not re-start, since it is not for data transmission. Apple share this view.

-	OPPO think this is enhancement and think if UE switch BWP then the old results are less useful.

-	ASUSTeK think today the timer is restarted for CG grant, so it is not that different, and think without this proposal what is the UE behaviour.

Coordination btw R1&R2 work

R2-2501994	Discussion on UEI beam reporting impact	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1. RAN2 does not discuss to capture the event evaluation procedure in MAC specification.

R2-2502544	Discussion on UE Initiated Beam Report 	Qualcomm Incorporated	discussion

Noted

Proposal 1. RAN2 confirms the following working assumption:

“RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision.”

Proposal 2. If event evaluation is handled by MAC layer, it contains the following steps:

PHY send indication per new beam at each evaluation occasion

MAC decides whether the event is triggered

If event is triggered, MAC indicates PHY to send the UCI and a CSI report on PUSCH.

DISCUSSION

-	LG E point out that R1 already agreed to include in their spec.

-	WI rapp suggest to capture a clear conclusion to reflect our understanding.

RAN2 understand the event evaluation and report triggering for UE-initiated beam report is captured by RAN1 spec.

RRC impact

R2-2502546	Report of [Post129][208][ MIMO_Ph5]	Ericsson	discussion

Noted

enabledCurrentBeamReport-r19 is added as an optional need-R field.

Reuse resourcesForChannelMeasurement in CSI-ReportConfig. Clarify in the field description that for UEI BM, the new beam to be measured is either CSI-RS (nzp-CSI-RS-ResourceSetList) or SSB (csi-SSB-ResourceSetList).

ng-n1-n2-r19, cri-typeI-SinglePanel-ri-restriction-r19/cri-typeII-ri-restriction-r19 and cri-typeI-SinglePanel-CBSR-r19/cri-typeII-CBSR-r19 in the same way as corresponding legacy fields i.e.:

        - ng-n1-n2-r19 is defined in the same way as ng-n1-n2 in R15 typeI-multiPanel

        - cri-typeI-SinglePanel-ri-restriction-r19/cri-typeII-ri-restriction-r19 are defined in the same way as legacy RI restrictions

        - cri-typeI-SinglePanel-CBSR-r19/cri-typeII-CBSR-r19 are defined in the same way as n1-n2-codebookSubsetRestriction-r18.

mrSelectedResources is defined as a SEQUENCE structure containing two fields with integer values from one to eight.

delayOffsetCompensation can be located under CSI-AperiodicTriggerState and outside of CSI-AssociatedReportConfigInfo and that the parameter triggeringScheme is not needed.

Define numberofSubbandsPO as a list (with size up to the number of subbands) where each element is an integer value within the maximum size of a BWP.

DISCUSSION

-	Samsung think P1 can be postponed and we should wait for R1 further input.

-	Ericsson point out that option 3 as it is does not work, so suggest to have option. 2.

The following is used the signaling of typeI-CBSR and typeII-CBSR and typeI-softScalingRank:

- (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2;

R2-2502547	Impacts from other NR MIMO Phase 5 objectives	Ericsson	discussion

Noted

Proposal 2	CSI-typeI/II group based CBSR is defined as option 2 i.e. (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2.

Proposal 3	The same structure defined for CBSR CSI-typeI/II is also used for typeI-softScalingRank.

Agreements on other aspects

MAC impact In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time after a new UCI for UE-initiated beam reporting is sent on first PUCCH. Confirm the following RAN2 understandings: - The CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH). - The DG PUSCH carrying the beam report of Mode-A carries MAC PDU (i.e. UL-SCH) as legacy. FFS if any other MAC impact for UL skipping The UE continues to perform CSI measurements for the UEIBM procedure when the active BWP is the dormant BWP. If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP. RAN2 understand the event evaluation and report triggering for UE-initiated beam report is captured by RAN1 spec. RRC impact enabledCurrentBeamReport-r19 is added as an optional need-R field. Reuse resourcesForChannelMeasurement in CSI-ReportConfig. Clarify in the field description that for UEI BM, the new beam to be measured is either CSI-RS (nzp-CSI-RS-ResourceSetList) or SSB (csi-SSB-ResourceSetList). ng-n1-n2-r19, cri-typeI-SinglePanel-ri-restriction-r19/cri-typeII-ri-restriction-r19 and cri-typeI-SinglePanel-CBSR-r19/cri-typeII-CBSR-r19 in the same way as corresponding legacy fields i.e.: - ng-n1-n2-r19 is defined in the same way as ng-n1-n2 in R15 typeI-multiPanel - cri-typeI-SinglePanel-ri-restriction-r19/cri-typeII-ri-restriction-r19 are defined in the same way as legacy RI restrictions - cri-typeI-SinglePanel-CBSR-r19/cri-typeII-CBSR-r19 are defined in the same way as n1-n2-codebookSubsetRestriction-r18. mrSelectedResources is defined as a SEQUENCE structure containing two fields with integer values from one to eight. delayOffsetCompensation can be located under CSI-AperiodicTriggerState and outside of CSI-AssociatedReportConfigInfo and that the parameter triggeringScheme is not needed. Define numberofSubbandsPO as a list (with size up to the number of subbands) where each element is an integer value within the maximum size of a BWP. The following is used the signaling of typeI-CBSR and typeII-CBSR and typeI-softScalingRank: - (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2;

R2-2501944	Discussion on open issues of RRC spec impact	Xiaomi	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2501986	Enhancements for UE-initiated/event-driven beam management	Ofinno, LLC	discussion

R2-2501994	Discussion on UEI beam reporting impact	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502065	Further discussion on UE-initiated/event-driven beam management	SHARP Corporation	discussion	NR_MIMO_Ph5-Core

R2-2502147	Discussion on UE-initiated Beam Reporting and CSI Enhancement	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502168	Discussion on UE-initiated/event-driven beam management	vivo	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502267	Discussion on UE initiated beam reporting	China Telecommunications Corp.	discussion	NR_MIMO_Ph5

R2-2502314	Discussion on UE-initiated Beam Reporting	Apple	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502319	RAN2 Aspects of the NR MIMO	Nokia Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502374	Discussion on UEIBR	Lenovo	discussion	Rel-19

R2-2502393	Clarification on the uplink grant used for the UE initiated beam report	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502544	Discussion on UE Initiated Beam Report 	Qualcomm Incorporated	discussion

R2-2502547	Impacts from other NR MIMO Phase 5 objectives	Ericsson	discussion

R2-2502666	Discussion on UE-initiated Beam Reporting and CSI enhancement	Samsung	discussion	Rel-19	NR_MIMO_Ph5

R2-2502714	Discussion on UE-initiated/event-driven beam management	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502828	Discussion on MAC impact regarding UEI reporting	ASUSTeK	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502833	Enhancements for UE-initiated/event-driven beam management	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2502867	Consideration on the UEIBM and Other Issues 	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

## 8.19	NR Others

Tdoc limit: 2

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-19 specific WIs/SIs that has no RAN WI.

Additional tdocs on top of limit can be allowed for co-sourced contribution with 3 or more companies

Number of UEs in RRC_INACTIVE with data transmission

R2-2501758	Reply to RAN2 LS on Number of UEs in RRC_INACTIVE state with data transmission (S5-250827; contact: China Telecom)	SA5	LS in	Rel-19	PM_KPI_5G_Ph4	To:RAN2	Cc:RAN3

DISCUSSION

-	Nokia think the request is not very clear, e.g., in SA spec there is some related measurements which is DU level, and R2 proposals are per cell.

-	CT think we can discuss based on company proposals.

-	CATT think SA5 request is clear and we can discuss in R2.

Noted

R2-2501910	Discussion on the number of SDT UEs (LS S5-250827)	CATT	discussion	Rel-19	PM_KPI_5G_Ph4

Noted

Proposal 1: According to SA5 reply LS, the measurement metrics “Mean/max number of RRC_INACTIVE UEs with ongoing SDT per cell” are introduced in TS 38.314.

Proposal 2: RAN2 to discuss whether the following measurement metrics are also introduced:

1.	Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the DL per DRB per cell

2.	Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the UL per DRB per cell

3.	Mean/max number of RRC_INACTIVE UEs with ongoing SDT per DRB per cell

4.	Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the DL per SRB per cell

5.	Mean/max number of RRC_INACTIVE UEs with ongoing SDT in the UL per SRB per cell

6.	Mean/max number of RRC_INACTIVE UEs with ongoing SDT per SRB per cell

Proposal 3: As “active UEs” and “inactive UEs” are defined in different levels, R16-R18 spec change in TS 38.314 is needed to clarify in clause 4.2.1.4 inactive UEs refer to UEs in RRC_INACTIVE.

DISCUSSION

P1

-	Nokia think there is no CR proposal to change according to P1. Nokia ask whether we can use RRC values instead.

-	CATT clarifies that there is a co-coursed CR.

P2

-	Nokia think those are not requested by SA5. Nokia think the existing measurments can already reflect connected and inactive combined, so we should not change what we have now.

P3

-	Nokia think inactive refers to buffer status not RRC state so we should not touch that.

R2-2502248	Introduction of number of UEs in RRC_INACTIVE state with data transmission [KPI_SDT_SA5]	China Telecom, Huawei, HiSilicon, ZTE Corporation, Sanechips, CATT	CR	Rel-19	38.314	18.0.0	0034	-	B	TEI19

=> Revised in R2-2503035

DISCUSSION

-	Nokia think in cover page we should remove ME impact.

-	Nokia think the CR should be correction, not for R19. Nokia think it is not good to refer to the RRC_INACTIVE in 38.331. Xiaomi not sure about this, and ask what it the definition in RRC.

-	Nokia point out the definition of active is clear in the talbe i.e., ‘Mean number of Active UEs in the DL per DRB per cell.’

-	HW think we can try to remove the reference to 331 but the intention is clear and we all know how SDT in inactive state works.

-	Ericsson understand this is requested by S5 and want to make sure this is from Rel-19.

-	HW understand with the proposal we reuse existing measurement as much as possible, and the wording ‘separately’ reflect that.

[AT129bis][204][NR_Others] CR for Introduction of number of UEs in RRC_INACTIVE state with data transmission (China Telecom)

Intended outcome: agreeable CR in R2-2503035

Deadline: before Friday CB

R2-2503035  Introduction of number of UEs in RRC_INACTIVE state with data transmission	China Telecom, Huawei, HiSilicon, ZTE Corporation, Sanechips, CATT, Ericsson 	CR	Rel-19	38.314	18.0.0	0034	1	B	PM_KPI_5G_Ph4

DISCUSSION

-	CT explains in offline the content is stable, but there is one company think the cat should not be B but it should be F.

-	Nokia still have concern on the CR, thinking this is not prvoding what SA5 wants, and also think this should be backward compatible. Nokia think it is not that clear when the SDT procedure start from CU UP point of view. Nokia think change to the section title is not normative.

-	HW think we already discuss and replied to SA5 that gNB knows the # of SDT UEs in RRC INACTIVE. HW think we can agree in principle and send LS back to SA5.

-	ZTE agree with HW. ZTE think it should be cat B. CATT agree.

-	CATT think some Nokia concern is only internal gNB functions so no need to reflect in the spec.

-	Ericsson think this has been discussed couple of meetings, and wonders if adding a new chapter will solve the concern.

-	Nokia think we can have two CRs, one is to change the title, and the other is to use new capture to capture what SA5 wants additionally.

-	CT and HW think the concerns are already resolved by the previous discussions and interactions with SA5 and think the current updated CR is good.

The CR is in principle agreed. Can check further whether the cat of the CR needs to be changed.

Send LS to SA5 to check if this CR solves their requirements. We request SA5 to reply it ASAP so that we can finalize the issue before end of May meeting.

[Post129bis][220][NR_Others] LS to SA5 on number of UEs in RRC_INACTIVE state with data transmission  (China Telecom)

Intended outcome: Approve the LS

Deadline:  Short

Support of 7Mhz channel bandwidth

R2-2501744	LS on Signalling for 7 MHz Channel Bandwidth (R4-2503017; contact: T-Mobile)	RAN4	LS in	Rel-19	NR_FR1_7MHz_BW-Core	To:RAN2	Cc:RAN3

Noted

R2-2501911	Discussion on signalling for 7 MHz Channel Bandwidth (LS R4-2503017)	CATT	discussion	Rel-19	NR_FR1_7MHz_BW-Core

Noted

R2-2502635	Discussion on 7MHz bandwidth capabilities	Nokia	discussion	Rel-19	NR_FR1_7MHz_BW-Core

Noted

R2-2502809	RAN2 impacts for 7 MHz Channel Bandwidth	Huawei, HiSilicon	discussion	Rel-19	NR_FR1_7MHz_BW-Core

Noted

R2-2502869	Consideration on Supporting 7M Channel Bandwidth	ZTE Corporation	discussion	Rel-19	NR_FR1_7MHz_BW-Core

Noted

DISCUSSION

Whether we wait for more R4 input

-	QC think we can wait since we in R2 are not sure whether 7M can be min or max. Xiaomi agree regarding capability of min and max bandwidth.

-	Ericsson think we can do the CR now and for now we can make the signaling as generic as possible. T-Mobile share this view.

-	HW think from the LS the other case than  a band with a minimum channel bandwidth of 3MHz or 5MHz.’ we need to wait for R4. Vivo share this view. OPPO also fine to wait.

-	QC think the WID is still ongoing in R4 so no hurry for R2. Xiaomi agree.

-	QC not sure whether this also impact R1?

-	Ericsson think if we wait then we need to have plan, e.g., whether we check with R4 on the issues?

-	T-mobile think we can send LS to R4 to ask about min BW. Xiaomi fine with this.

UE capability signalling

-	Xiaomi think we need two capabilities: the supported BW is clear and can start from R15.

-	HW think R4 indicate they do not think it critical to have 7M as min and max BW.

Which release to start from

-	QC think R19 is too late, but no strong view which earlier release is best, can think further.

-	vivo think we can do similar as 35/45M so start from Rel-19.

R2-2502569	Introduction of 7MHz channel bandwidth	Ericsson, T-Mobile	CR	Rel-17	38.306	17.12.0	1257	-	B	NR_FR1_7MHz_BW-Core, TEI17

R2-2502570	Introduction of 7MHz channel bandwidth	Ericsson, T-Mobile	CR	Rel-18	38.306	18.5.0	1258	-	A	NR_FR1_7MHz_BW-Core, TEI18

R2-2502571	Introduction of 7MHz channel bandwidth	Ericsson, T-Mobile	CR	Rel-17	38.331	17.12.0	5307	-	B	NR_FR1_7MHz_BW-Core, TEI17

R2-2502572	Introduction of 7MHz channel bandwidth	Ericsson, T-Mobile	CR	Rel-18	38.331	18.5.1	5308	-	A	NR_FR1_7MHz_BW-Core, TEI18

The CRs are postponed.

[Post129bis][205][NR_Others] Reply LS to RAN4 on 7MHz channel bandwidth (T-mobile)

Scope: Discuss questions to RAN4 (e.g., the support of 7Mhz as the minimum or maximum channel bandwidth)

Intended outcome: Approve the LS.

Deadline:  Short

On CSSF optimization

R2-2501739	LS on CSSF optimization for NR RRM Phase 5 (R4-2502662; contact: Apple)	RAN4	LS in	Rel-19	NR_RRM_Ph5-Core	To:RAN2

Noted

R2-2501972	Discussion on CSSF optimization for NR RRM Phase 5	vivo	discussion	Rel-19	NR_RRM_Ph5-Core

Noted

R2-2502313	RAN2 impact on CSSF optimization	Apple, Ericsson, CATT	discussion	Rel-19	NR_RRM_Ph5-Core

Noted

DISCUSSION

-	Apple think vivo P1 is not needed.

-	ZTE suggest to send LS to R4 asking the benefit of this whole feature. ZTE think currently if NW configure multiple MOs in the same band then UE will not measure any, so not sure why NW configure so. QC share this view.

-	CATT think R2 should follow the LS and discuss signaling. CATT think with this feature there is more flexible for NW to enable configuration in the band. HW share this view.

-	HW think if the feature itself should be discussed in RAN4.

-	OPPO think we should discuss both options, a) go with R4 requst, and b) try to use existing MO configuration to achieve the same goal. OPPO think we can either wait or ask R4.

-	Ericsson think R4 is still discussing so we just wait.

-	QC think we can explain to R4 what is already possible with R2 spec, and ask R4 whether we need more.

-	Apple suggest we wait until after the next meeting, so based on new agreement from R4 we can discuss the LS to them. Apple suggest we do not challenge the motivation as the WID obj is defined by RAN.

Chair: we can discuss in the next meeting based on further progress in RAN4

Others

R2-2501742	LS on switching periods for low-low band switching (R4-2502877; contact: AT&T)	RAN4	LS in	Rel-19	NR_LBCA_Sw-Core	To:RAN1	Cc:RAN2

Noted

R2-2501760	LS on Vendor Specific Trace Record (S5-251089; contact: Ericsson)	SA5	LS in	Rel-19	TraceQoE_OAM	To:RAN3	Cc:RAN2

Noted

The following documents will be treated in other breakout sessions.

R2-2501753	Reply LS on Satellite IDs for store-and-forward operation (S2-2502450; contact: CICT Mobile, CATT)	SA2	LS in	Rel-19	5GSAT_Ph3-ARC	To:RAN2	Cc:RAN3

R2-2501718	Reply LS on Ku band numerology (R1-2501609; contact: Eutelsat)	RAN1	LS in	Rel-19	NR_NTN_Ku_bands-Core	To:RAN4	Cc:RAN, RAN2

R2-2501759	Reply LS on AI/ML UE sided data collection (S5-250828; contact: Intel, NEC, Huawei)	SA5	LS in	Rel-19	AIML_MGT_Ph2	To:RAN, RAN2	Cc:SA, SA2, SA3

## List of post meeting email discussions

Short

[Post129bis][203][LPWUS] LS to R1 LP-WUS in RRC_CONNECTED (InterDigital)

Intended outcome: Approve the LS.

Deadline: Short

[Post129bis][205][NR_Others] Reply LS to RAN4 on 7MHz channel bandwidth (T-Mobile)

Scope: Discuss questions to RAN4 (e.g., the support of 7Mhz as the minimum or maximum channel bandwidth)

Intended outcome: Approve the LS.

Deadline:  Short

[Post129bis][217][LPWUS] LS to RAN3 (CC SA2/SA3) to inform agreements on UE ID based subgrouping (NEC)

Intended outcome: Approve the LS

Deadline:  Short

[Post129bis][219][SBFD] LS to RAN1 (CC RAN3/RAN4) on simultaneous configuration of SBFD and DC (Huawei)

Intended outcome: Approve the LS

Deadline:  Short

[Post129bis][220][NR_Others] LS to SA5 on number of UEs in RRC_INACTIVE state with data transmission  (China Telecom)

Intended outcome: Approve the LS

Deadline:  Short

Long

[Post129bis][206][LPWUS] Running CR for 37.340 (ZTE)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

[Post129bis][207][LPWUS] Running CR for 38.300 (Ericsson)

Intended outcome: Updated running CR based on new agreements for endorsement

Deadline:  Long

[Post129bis][208][LPWUS] Running CR for 38.304 (CATT)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

[Post129bis][209][LPWUS] Running CR for 38.331 (vivo)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

[Post129bis][210][LPWUS] Running CR for 38.321 (Apple)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

[Post129bis][218][LPWUS] UE capabilities (Huawei)

Intended outcome: Summary with proposals on RAN2 UE capabilities for LP-WUS

Deadline:  Long

[Post129bis][211][SBFD] Running CR for 38.300 (CATT)

Intended outcome: Updated running CR based on new agreements for endorsement

Deadline:  Long

[Post129bis][212][SBFD] Running CR for 38.331 (Huawei)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

[Post129bis][213][SBFD] Running CR for 38.321 (Samsung)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

[Post129bis][214][ MIMO_Ph5] Running CR for 38.331 (Ericsson)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

[Post129bis][215][ MIMO_Ph5] Running CR for 38.321 (Samsung)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long

[Post129bis][216][ MIMO_Ph5] Running CR for 38.300 (CMCC)

Intended outcome: Updated running CR based on new agreements for endorsement, open issue list (if needed)

Deadline:  Long