3GPP TSG-RAN WG2 #129bis 	R2-2502989

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.2.2

Source:	Ericsson

Title:	Discussion on DL messages for Ambient IoT UEs

Document for:	Discussion/Decision

# 1	Introduction

In RAN#107, a new WI has been approved for Ambient IoT devices [1]. The following RAN2 related objectives are capture in the WID:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

In this contribution we continue to discuss the open issues on downlink messages for A-IoT devices based on the scope highlighted above considering the agreements so far.

# 2	Discussion

## 2.1 Simultaneous Paging Procedures

In RAN2#129, the following agreements were made:

      - Parallel service requests by the same reader is not supported.

      - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed).

Multiple readers may be deployed to serve an area, and their coverage can overlap. A reader may start an inventory procedure which is interleaved by another inventory procedure started by another reader assuming that there is no coordination between readers in time and/or frequency domains. A-IoT device should ideally participate only in one of the paging rounds and this principle has been agreed in RAN2#129.

Now the question is how an A-IoT device should behave if it receives a service request while it has been performing a procedure associated with a service request which was received earlier. One option is that the device abandons performing the ongoing procedure associated with the service request earlier and starts performing the procedure associated with the new service request. Assuming that it is not possible for the device to know whether the new service request is provided by the same/different reader, this is regardless of whether the new service request is provided by same/different reader and/or marked with same/different transaction ID. Another option is that the device ignores any other service request received if it is performing a procedure associated with a service request received earlier.

For the first option, radio and energy resources may be wasted since the A-IoT device would switch procedures before completion assuming that the readers are not coordinated. It is also possible that overall performance is degraded as successful completion may be prolonged when communication from multiple readers is interleaved. Note that it is not possible for a reader to know whether a device has abandoned the procedure so it would continue to perform the procedure until it assumes failure.

If the device abandons the ongoing procedure, radio and energy resources may be wasted since the A-IoT device would switch procedures before completion assuming that readers are not coordinated.

Overall performance may be degraded as successful completion may be prolonged if ongoing procedure is abandoned when communication from multiple readers is interleaved.

In the second option the device can proceed with an ongoing procedure that it has started to perform. In general, this would be relatively more efficient from radio and energy resources and overall network performance standpoint. However, for this option, it should be clear for the device whether and when an ongoing procedure is completed or pending. This is so that the device can determine whether to respond to another service request based on whether, e.g., it has responded successfully to a service request with the same transaction ID or it is a service request with a new transaction ID.

For example, a device may know whether an ongoing procedure is completed if the reader transmits a NACK message indicating an unsuccessful procedure or if the reader transmits a QueryRep-like message, i.e., an R2D transmission which determines the Msg1 resource(s) as agreed in RAN2#129, to indicate the next accession occasion slot (which would implicitly indicate ACK).

It is also possible that the reader continues with a command in Msg4. Such command always triggers a response from the device, regardless of whether it is “read” or “write”. Then the question is whether the reader provides an ACK or NACK, i.e., scheduling retransmission and this has not been discussed and concluded in RAN2 yet. It is also possible that a device may miss a message and thus considers a procedure ongoing even though this is not the case. Then the device can rely on the following R2D transmission which determines the Msg1 resources, i.e., QueryRep-like message, or the requirements agreed in RAN1 regarding when a device may receive a response from the reader at the latest.

If the device proceeds with the ongoing procedure it would be more efficient from radio and energy resources and overall network performance standpoint.

It should be clear for the device whether and when an ongoing procedure is completed or pending.

Considering the following agreement in RAN2; “NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback”, we assume that RAN2 intends to make it clear for the device to know whether and when an ongoing procedure is completed or pending Therefore, we propose the following:

Device ignores any other paging service request received if it is performing a procedure associated with a paging service request received earlier.

Now, we need to determine the device behaviour when it receives any R2D message while performing a procedure associated with a service request. Assuming that it is not possible for the device to know whether an R2D message is associated with the ongoing service request procedure, unless, for example, reader ID is provided along with the R2D message, it should ignore a message based on expectancy. For example, if the device has transmitted Msg1, it would ignore if it received Msg4 etc. However, there can still be cases where it is not possible for the device to distinguish between messages from different readers. To keep it simple, RAN2 can ignore those cases with the assumption that they would eventually be resolved due to collisions or upper layer handling, but we may need to address the case for Query-like messages that determine Msg1 resources for efficient slotted ALOHA operation. This is discussed in the next section.

RAN2 should discuss whether it should be possible for a device to distinguish Query-like messages that determine Msg1 resources.

## 2.2 How to determine Msg1 resources

In RAN2#129 the following agreement was made:

From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signalling later:

Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or

Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

In general, we prefer a unified format, where possible, for all R2D messages. Therefore, we think it would be beneficial if we use the common format describe in our contribution on data transmission aspects [4] where a separate type is introduced for R2D message which determines Msg1 resources. The message should include the following information:

- time and frequency resources to be used when transmitting Msg1

- indication for the current access occasion, i.e., the slot number

There may also be a need for end indication and information to distinguish these messages coming from different readers to address the multi-reader scenario. But first we discuss whether there is a need to provide the time and frequency resources in every message in a paging round or it would be sufficient to configure once in the service request message, i.e., Msg0, and update if needed. It is not clear to us whether such flexibility, i.e., configuring time/frequency resources differently per access occasion within a round, is needed from reader perspective. In that case one should also address whether the latest configuration prevails in the remaining access occasions, or the device should assume the default configuration. Considering that these devices may run out of energy and come back after harvesting, approaches that require state tracking should not be considered. Yet another criterion is simplicity, so we propose the following:

Msg1 resources are determined using a common format for R2D messages, i.e., an R2D message with a separate type.

In multi-reader scenario, as discussed in the section above, it would be beneficial for the device know whether and when an ongoing procedure is completed or pending. One implicit indication for the device is to receive a R2D message that configures the Msg1 resources for the next access occasion, i.e., the device can assume that the procedure is completed successfully unless it received a NACK. To indicate that the last access occasion can be considered completed there may be a need for “end” indication.

There may be a need for “end” indication for the last access occasion in a paging round.

In multi-reader scenario, assuming that it is not possible for the device to know from which reader an R2D message is received, a device may use unintended time/frequency resources configured by a reader which is not associated with its ongoing procedure. This would not only mean a collision and therefore a failure for multiple devices, but also advancing the access occasion, i.e., the slot number, mistakenly. This can be addressed by introducing an identifier in the message. One possibility is reader ID, however as discussed previously for other messages/cases, this will introduce complexity as it requires coordination between readers. A similar approach that is used in Msg1, can be utilized here, i.e., a reader generated random number per paging round large enough in size can be provided in the message to distinguish readers.

A device may use unintended time/frequency resources configured by a reader which is not associated with its ongoing procedure.

A device may advance the access occasion, i.e., slot number, mistakenly.

Based on the discussion above, we propose the following: message that configures Msg1 resources should include the following:

R2D message that configures Msg1 resources should include the following:

- - time and frequency resources to be used when transmitting Msg1

- - indication for the current access occasion, i.e., the slot number

- - “end” indication for the last access occasion in a paging round

- - a reader generated random number per paging round, i.e., FFS size

## 2.3 How to generate the transaction ID

In RAN2#127, it was agreed to support a mechanism for the reader to trigger multiple/subsequent A-IoT paging messages that are associated with the same service request from the CN. In RAN2#127bis, it was agreed that A-IoT paging message can include information to avoid duplicate response from the device to a reader. In RAN2#129, it was agreed that the “transaction ID” can be generated by reader based on CN correlation ID.  But it is FFS how reader will generate “transaction ID” and the size of transaction ID”.

In this section we discuss how reader should generate transaction ID based on CN correlation ID. We think that there should be one to one mapping between transaction ID and correlation ID, otherwise one may observe the following:

- Same CN correlation ID maps to different transaction IDs, i.e., subsequent A-IoT paging messages associated with the same service request will have different transaction IDs. This may lead to that the device send duplicate responses to subsequent paging unnecessarily.

- Different CN correlation IDs map to the same transaction ID. In this case the device will regard the later received A-IoT paging message as subsequent paging message and ignore if it has successfully responded earlier.

RAN2 assumes there is one to one mapping between CN correlation ID and transaction ID.

We do not think there is a need for an additional reader ID or a need for using reader specific aspects when generating the transaction ID since this would require coordination in the network which would increase complexity. We prefer to use the CN correlation ID as the transaction ID assuming that SA2 would end up designing a ID with a “short” size. If it turns out that this is not the case, i.e., CN correlation ID has a size long enough not acceptable from RAN2 standpoint, we can consider reducing the ID in size by masking or introducing a function that maps the ID to a transaction ID following the principle that there is one to one mapping between CN correlation ID and the transaction ID as discussed above, i.e., a CN correlation ID should always be mapped to the same transaction ID regardless of reader. It should be possible to map the CN correlation ID to an ID shorter in size assuming that for indoor scenarios service areas are rather limited, i.e., no need to differentiate many readers.

RAN2 assumes that transaction ID is the CN correlation ID (or a truncated CN correlation ID) associated with the service request from the CN. This can be revisited based on the outcome of the discussion in SA2.

Send an LS to SA2/RAN3 regarding the size of the CN correlation ID and inform that RAN2 prefers to minimize the size.

## 2.4 Paging identifier and visibility in the MAC layer

In RAN2#129, the following agreements have been made:

The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/” filtering criteria”, while the exact format of latter is supposed to be designed by SA2.”

The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer”

We think that RAN2 should clarify what is intended by “visible” here. If the discussion is on whether it should be possible for the reader to, for example, mask/regenerate the identifier provided by the CN, we prefer that this should not be possible. In that sense, we do not support MAC layer sub-group paging solutions. We assume that temporary ID is intended for paging a single device, hence there is no need for sub-grouping. Note that it is not clear yet whether group paging via masking permanent ID would be possible since paging identifier may be encrypted.

The paging identifier should be transparent to the reader, i.e., it should not be possible for the reader to, e.g., mask/regenerate the identifier provided by the CN.

## 2.5 Indication of random-access type

In RAN2#127bis it was agreed that “From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. If multiple device IDs in single paging is supported and A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA.”

A related discussion is if the reader should indicate explicitly whether a device responds to a paging message with a contention free or contention based random access. In our view there is no need to indicate such explicitly. For contention based random access, the reader only needs to indicate information that is required for a device to determine which slot to respond randomly, yet for contention free random access, the reader needs to indicate the dedicated resources for the targeted device(s). This means that the content of such paging messages will be different for contention free, and contention based random access.

From RAN2 perspective it is feasible to support paging multiple device IDs depending on TB size.

When A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA.

Whether a device should respond to the paging message with contention free or contention-based RA is not indicated explicitly.

# 3	Conclusion

In this contribution we discussed downlink/paging messages for A-IoT devices. Based on the discussion in the previous sections we made the following observations:

Observation 1	If the device abandons the ongoing procedure, radio and energy resources may be wasted since the A-IoT device would switch procedures before completion assuming that readers are not coordinated.

Observation 2	Overall performance may be degraded as successful completion may be prolonged if ongoing procedure is abandoned when communication from multiple readers is interleaved.

Observation 3	If the device proceeds with the ongoing procedure it would be more efficient from radio and energy resources and overall network performance standpoint.

Observation 4	It should be clear for the device whether and when an ongoing procedure is completed or pending.

Observation 5	RAN2 should discuss whether it should be possible for a device to distinguish Query-like messages that determine Msg1 resources.

Observation 6	There may be a need for “end” indication for the last access occasion in a paging round.

Observation 7	A device may use unintended time/frequency resources configured by a reader which is not associated with its ongoing procedure.

Observation 8	A device may advance the access occasion, i.e., slot number, mistakenly.

Observation 9	From RAN2 perspective it is feasible to support paging multiple device IDs depending on TB size.

Observation 10	When A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA.

Based on the discussion in the previous section, we propose the following:

Proposal 1	Device ignores any other paging service request received if it is performing a procedure associated with a paging service request received earlier.

Proposal 2	Msg1 resources are determined using a common format for R2D messages, i.e., an R2D message with a separate type.

Proposal 3	R2D message that configures Msg1 resources should include the following:

- time and frequency resources to be used when transmitting Msg1

- indication for the current access occasion, i.e., the slot number

- “end” indication for the last access occasion in a paging round

- a reader generated random number per paging round, i.e., FFS size

Proposal 4	RAN2 assumes there is one to one mapping between CN correlation ID and transaction ID.

Proposal 5	RAN2 assumes that transaction ID is the CN correlation ID (or a truncated CN correlation ID) associated with the service request from the CN. This can be revisited based on the outcome of the discussion in SA2.

Proposal 6	Send an LS to SA2/RAN3 regarding the size of the CN correlation ID and inform that RAN2 prefers to minimize the size.

Proposal 7	The paging identifier should be transparent to the reader, i.e., it should not be possible for the reader to, e.g., mask/regenerate the identifier provided by the CN.

Proposal 8	Whether a device should respond to the paging message with contention free or contention-based RA is not indicated explicitly.

# 4      References

RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, 3GPP TSG RAN Meeting #107, Incheon, South Korea, March, 2025.

TR 22.840, V0.4.0, “Study on Ambient power-enabled Internet of Things (Release 19)”

TR 23.700-13, V1.2.1, “Study on Architecture support of Ambient power-enabled Internet of Things (release 19)”.

R2-2502303, “A-IoT Data Transmission and Other Aspects”, Ericsson, RAN2#129bis, Wuhan, China, 7th – 11th April, 2025.