3GPP TSG-RAN WG2 #129bis	R2-2503001

# Wuhan, China, 7th – 11th April 2025

Agenda Item:	8.6.1

Source:	Ericsson

Title:	Summary of offlines [AT129b][102][MOB] and [AT129b][110][MOB]

Document for:	Discussion

# 1	Introduction

This document is to discuss the following offline discussions:

[AT129b][102][MOB] (Ericsson) – max 45 mins

Scope: Discuss the identified RRC FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502535 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503001.

Deadline: Comeback in CB session.

And

[AT129b][110][MOB] (Ericsson) => max 45 mins

Scope: Clearly clarify how option2 works and check if there is a real technical blocking issue. If it works w/o real technical blocking issue, then we can go to option2. If there is a real technical blocking issue for option2, we’ll go to option1.

Intended outcome: Discussion summary in R2-2503013.

Deadline: Comeback in CB session

# 2	Discussion

After the offline discussion the following proposals related to the remaining FFS have been discussed and seems agreeable.

### 5.3.5.18.6

Editor’s Note: How to understand which sk-counter the UE shall select is FFS (e.g., link a list of sk-counter to a Rel-19 ID or LTM candidate ID).

A list of sk-counters is linked to a Rel-19 ID configured by the SN.

### 5.3.7.2

At RLF and reconfiguration with sync failure:

  - If RLF:

    - If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

  - if reconfiguration failure (inter-CU LTM)

    - If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

### 6.3.2

Editor’s Note: FFS whether only one indication is needed to indicate LTM configuration as complete configuration.

How to indicate whether an LTM candidate configuration is a complete configuration is the same as Rel-18 (no different indication for MCG and SCG part of the candidate configuration).

### 11.2.2

Editor’s Note: FFS whether the SN can signal to the MN that has released inter-CU SN LTM candidate cells

Editor’s Note: FFS whether the indication whether the SN is allowed to configure an inter-CU LTM configuration is provided in the Xn signalling.

The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message. We include this agreement in the LS to RAN3.

### Others

Editor’s Note: LTM Cell Switch Command MAC CE supported for CLTM candidate

Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

# 3	Conclusion

According to what has what has been discussed in section 2:

- A list of sk-counters is linked to a Rel-19 ID configured by the SN.

At RLF and reconfiguration with sync failure:

  - If RLF:

    - If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

  - if reconfiguration failure (inter-CU LTM)

    - If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast RLF recovery. FFS if fast RLF recovery with different Rel-19 IDs is allowed.

How to indicate whether an LTM candidate configuration is a complete configuration is the same as Rel-18 (no different indication for MCG and SCG part of the candidate configuration).

The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message. We include this agreement in the LS to RAN3.

Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

# References

R2-2502534, Running RRC CR for inter-CU and conditional LTM, Ericsson, RAN2#129, February 2025.