3GPP TSG-RAN WG2 Meeting #129bis	R2-2503086

Wuhan, China, 7th – 11th April 2025

Agenda Item:	8.13.1

Source:	LG

Title:	[AT129bis][407][Relay] Rel-19 relay CR to 38.300 (LG)

Document for: Discussion and Decision

1	Introduction

This contribution gives the discussion summary of following email discussion.

[AT129bis][407][Relay] Rel-19 relay CR to 38.300 (LG)

Scope: Collect any comments on the CR in R2-2502695.  Address the terminology question from R2-2502278 (to affect all specifications) and whether to describe “intermediate relay UE acting as a remote UE” (to affect at least 38.331).

Intended outcome: Endorsable CR in R2-2503076

Deadline: Wednesday 2025-04-09 1900 CST

2	Discussion

The following may be related to the spec rapporteur. However, anyone who is interested in this terminology issue can answer the questions and write down your comments:

To implement specs for the multi-hop relay, terminology alignment between specs needs to be discussed. In the multi-hop relay, we used three terms, such as last Relay UE, intermediate Relay UE, and first Relay UE, due to the convenience of discussion during the meeting. However, we need to think about how we really need to use the terminologies at the spec implementation.

It may be complicated works to differentiate “U2N last Relay UE” from the legacy Rel-17 “U2N Relay UE”. Because even though the functionality is the same, the spec should add the description, for example, “U2N Relay UE and U2N last Relay UE”. It may cause a spec change burden and also generate our mistakes missing the correction. So,

1-1) How about using the “U2N Relay UE” in the multi-hop instead of using “U2N last Relay UE”? If you think we need to differentiate between the legacy Rel-17 “U2N Relay UE” and Rel-19 “U2N last Relay UE”, can you show us an example of the reason it is useful? (In this case, we can use last “U2N Relay UE”, only when it is specially needed.

Do you agree to commonly use the legacy “U2N Relay UE” instead of using “U2N last Relay UE”?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | We think the principle for doing differentiation is that the normative text of UE behaviour for first relay UE and the other relay UEs are significant different. The general functional part of multi-hop U2N last relay UE and single hop relay UE are same, e.g. discovery function/notification function/SRAP data forwarding function/local ID request in SUI message function. Although detailed behaviour or signaling are slightly different, we do no see too much benefits to do such differentiation. |
| CATT | See comments | We are fine not to futher distinguish the U2N last Relay UE with legacy R17 U2N Relay UE. Our concern is the relationship between the U2N Relay UE with the new introduced first relay UE/intermediate UE/last relay UE. Option1, in R19, U2N Relay UE can only cover last relay UE? Option2, in R19, U2N Relay UE covers first/intermediate relay UE? Option3, in R19, U2N Relay UE covers first/intermedite/last relay UE? [LG] Our original intention was option 3. All U2N Relay UEs(first/intermediate/last) can perform the behavior of (original) U2N Relay. But in some cases, if the U2N Relay UE works as an intermediate Relay UE, the intermediate Relay UE performs some more behavior or doesn’t perform some other behavior comparing the (original) U2N Relay UE. We thought only these cases were described additionally as intermediate U2N Relay UE. We thought this approach might reduce the spec change. But it was just our initial opinion. We are open and after listing other companies’ opinions, we can make compromised results. |
| OPPO | Yes |  |
| Ericsson | No (for the time being) | We think we need to keep using the last U2N relay UE for the time being, since this term is also used in other specs. Specially, there is still ongoing discussions on latency reduction, e.g., fast forwarding. We are not sure if the legacy U2N Rel-17 Relay UE behavior can be fully reused for the last relay UE without any change. We can keep using this term in the current running CR, and make change (by removing this term) if RAN2 has concluded all remaining issues. Since anyway, such change is easy to conduct. |
| MediaTek Inc. | Yes |  |
| vivo | Yes |  |
| LG Electronics | Yes | We think that U2N last/ intermediate/first Relay UE can be considered as U2N Relay UEs in our specifications. If a specific text in the specification is only applied to U2N last Relay UE, U2N last relay UE can be used in the specification. This approach will reduce the repeated addition of new R19 terms to the existing texts in our CRs especially when R19 relay UEs copies legacy operation. |
| Huawei, HiSilicon | See Comments | We think U2N Relay UE can cover U2N last Relay UE only. We also think it might be clearer to introduce new entities for the Multihop Relay in the specs for example to describe the multihop Service continuity Scenarios in Stage 2 and use these terms in the Stage 3 Specs also. Other point is if we look at protocol stacks of the U2N Intermediate or U2N First Relay UE and compare it with Last Relay UE they are quite diferrent. Hence, we think introducing the new entities might be cleaner although they have larger spec impacts. |

Actually, the “U2N first Relay UE” is a kind of intermediate U2N Relay UE. The only difference of the “U2N first Relay UE” from the “U2N intermediate Relay UE” is that it has connections with Remote UEs. However, considering intermediate Relay UE can always act as a Remote UE (at least approach 1), we can say that all the intermediate Relay UE can be a first Relay UE at some point. In addition, for the description of the multi-hop relay, sometimes the indication of direction seems to be needed. Considering all the cases, we can use the wording of parent U2N Relay UE or child U2N Relay UE instead of “U2N first Relay UE”. So,

1-2) how about to use just “U2N intermediate Relay UE” instead of using “U2N first Relay UE”? If we have to specially indicate the “U2N first Relay UE”, we can use parent “U2N Relay UE” or serving “U2N Relay UE” something like that.

Do you agree to use “U2N intermediate Relay UE” instead of “U2N first Relay UE”?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | We think the principle for doing differentiation is that the normative text of UE behaviour for first relay UE and the other relay UEs are significant different. We believe the main difference between first relay UE and the other intermediate relay UE is SRAP header operation: Only first relay UE need to construct/remove the SRAP header for remote UE’s SRB0 message. The other intermediate relay UEs just forward the message directly. This can be done by identifying the SRAP header is present or not OR first relay UE will deliver the remote UE’s SRB0 message to dedicated uplink PC5 RLC channel based on network configured for baseline procedure. |
| CATT | See comments | Prefer to keep the first relay UE, and further clarify the relationship between the first relay UE and intermediate relay UE is enough(i.e., the first relay UE is a kind of intermediate relay UE). |
| OPPO | Yes | We can use serving U2N Relay |
| Ericsson | No | We also prefer to keep using the first relay UE for the time being, use this term would be helpful to draft the spec. while removing this term, may have a risk of complexing the spec texts, and causing UE behaviors unclear. We can wait until RAN2 has concluded all remaining issues, to see if such term can be removed. |
| MediaTek Inc. | No | It might cause some between U2N intermediate Relay UE and U2N first Relay UE, the main difference is U2N first Relay UE has PC5 connection with U2N Remote UE while U2N intermediate Relay UE (except U2N first Relay UE) has no PC5 connection with U2N Remote UE. |
| vivo | No | We share the view from MediaTek, the current ‘first relay UE’ is helpful to distinguish different UE roles. |
| LG Electronics | Yes | We think that U2N intermediate /first Relay UE can be considered as U2N intermediate Relay UEs in our specifications. If a specific text in the specification is only applied to U2N first Relay UE, U2N first relay UE can be used in the specification. This approach will reduce the repeated addition of new R19 terms to the existing texts in our CRs especially when R19 relay UEs copies legacy operation. |
| Huawei, HiSilicon | No | We also prefer to keep using the first relay UE for the time being, use this term would be helpful to draft the spec. However we need to discuss and agree what we will call the relays entities where we have the following multihop chains for example - gNB-> Last Relay UE -> Multi hop Remote UE or gNB-> First Relay UE -> Multi hop Remote UE - gNB-> Last Relay UE -> Intermediate Relay UE -> Multi hop Remote UE or gNB-> Last Relay UE -> First Relay UE -> Multi hop Remote UE |

In the case of U2N intermediate Relay UE, it has two roles such as a U2N Relay UE and a U2N Remote UE. When it acts as a U2N Relay UE, the intermediate Relay UE forwards UL/DL data via sidelink. When the U2N Relay UE acts as a U2N Remote UE, the operation of the U2N intermediate Relay UE is similar to the U2N Remote UE.

In the current 38.300 CR, all the relay UEs regardless of “U2N last Relay UE”, “U2N intermediate Relay UE” or “U2N first Relay UE” are described as “U2N Relay UE”. Then when describing the role as a Remote UE, the phrase is added like “the U2N Relay UE can act as a Remote UE”. However, looking at the other CRs, companies are describing clearly “U2N intermediate Relay UE”.

1-3) (a) Do you think we can use “U2N Relay UE” instead of using “U2N intermediate Relay UE”? In this case, we can use intermediate “U2N Relay UE” only when it needs to clarify. Or, (b) do you think we should use clearly the terminology of “U2N intermediate Relay UE” to differentiate from “U2N Relay UE”?

| Company | Your preference (a)/(b) | Comments |
| --- | --- | --- |
| ZTE | B | RAN2 has agreement to say that “RAN2 intend to align with SA2 terminology when it is stable”.In SA2’s specification, intermediate relay UE and last relay UE have different definitions. And for parent link, intermediate relay UE and last relay UE use different RATs, i.e. sidelink and Uu RAT respectively. So, it would be good to differentiate the intermediate relay UE from last relay UE. |
| CATT | (a) |  |
| OPPO | a | Our understanding is (a) doesn’t against the previous agreement to align with SA2 terminology since intermediate relay UE will be defined but we will use “U2N Relay” if no additional clarification is needed. |
| Ericsson | b | We agree with ZTE, also see our comments above. Stick to the current terms, would be helpful to simplify the spec texts. We can wait until RAN2 has concluded the issues, and decide if current term can be removed. |
| MediaTek Inc. | (b) | Agree with ZTE and Ericsson. |
| vivo | B |  |
| LG Electronics | A | We think that U2N last/intermediate/first Relay UE can be considered as U2N Relay UEs in our specifications. If a specific text in the specification is only applied to U2N intermediate Relay UE, U2N intermediate relay UE can be used in the specification. This approach will reduce the repeated addition of new R19 terms to the existing texts in our CRs especially when R19 relay UEs copies legacy operation. |
| Huawei, HiSilicon | B | Since the protocol stacks of intermediate and the last relay UE are different it would be better to use the different terms for these entities |

Even though the intermediate Relay UE doesn’t have its own data to transmit and receive, it can act as a Remote UE in terms of functionality (in the case of approach 1).

1-4) Do you think the differentiate is needed between when the intermediate Relay UE has its own data to transmit and receive as a remote UE, and when just acts as a remote/relay UE in terms of the functionality (in this case the intermediate Relay UE doesn’t have its own data)? For example, when the U2N intermediate Relay UE has its own data, how to describe it. It can be described as a Remote UE? Or intermediate Relay UE?

If you think the differentiation is needed, feel free to describe how to differentiate.

| Company | Differentiation is needed? (yes/no) | Comments |
| --- | --- | --- |
| ZTE | Not clear about the meaning of“Yes/No”, Regarding the example, it can be described as a remote UE, no need to describe the intermediate relay UE has its own data explicitly in stage3 specification | 3GPP specification only describe the logical function, not physical function. If “the intermediate relay UE acting as a remote UE” inherit the function/behaviour from “Remote UE”, no need to duplicate the corresponding description in stage3. |
| CATT | No | RAN2 clarifies the wording that intermediate relay UE takes the role of the remote UE in the procedure is not equivalent to the intermediate relay UE has U2N relay data transmission requirement. From our point of view, this issue had been discussed during the scenario. There is no limitation that the intermediate relay UE has U2N relay data transmission requirement. But once we talk about the intermediate relay UE acting as a remote UE, we also don’t limit that there must have U2N relay data transmission requirement for the intermediate relay UE. For approach1, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED. Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED. The connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE, currently, there is no requirement that the intermediate relay UE has U2N relay data transmission requirement at all. |
| OPPO | No | Not see the motivation for the differitiation since we understand in R17 U2N Relay, we didn’t diffentiate the remote UE with data or remote UE without data from terminology perspective. |
| Ericsson | No | If intermediate relay UE operates as remote UE to transmit its own data, this is already captured/reflected in the legacy spec, there is no additional spec impact foreseen. |
| MediaTek Inc. | No |  |
| vivo | No |  |
| LG Electronics | No |  |
| Huawei, HiSilicon | No | This is not dependent whether it has data to transmit or not. It is mainly about the dual roles Intermediate and the First Relay UE plays. The main intention to describe the Intermediate Relay UE/ First Relay UE acting as a Remote UE is that Intermediate Relay UE/First Relay UE connects to the network via the U2N Last Relay UE acting as a Multihop U2N Remote UE and then acts as an intermediate/First Relay UE |

1-5) If there are any further issues for the terminology we need to discuss, feel free to write down.

| Company | Comments |
| --- | --- |
| CATT | Whether we need to introduce one new terminology to distinguish the remote UE for single hop and multi hop? The below two achievements are used as example: - Option1(From 331): Multihop U2N Remote UE: A UE that is involved in the multi hop relay communication and communicates with the network via a U2N Last Relay UE, a U2N Intermediate Relay UE and a U2N First Relay UE if these are configured on the multihop path. U2N Remote UE: A UE that communicates with the network via a U2N Relay UE in single hop relay communication. - Option2(From 304): U2N Remote UE: a UE that communicates with the network via a U2N Relay UE, or via a U2N Last Relay UE and one or multiple U2N Intermediate Relay UE(s). |
| Ericsson | we used option 2 in 38.323, but we have no strong view. |
| Huawei HiSilicon | So far we have used the entities that we agreed as the RAN 2 agreement to draft the CR. However we need to discuss and agree what we will call the relays entities using the new entities agreed in RAN2 when we have the following multihop chains - A) gNB-> Last Relay UE -> Multi hop Remote UE or B) gNB-> First Relay UE -> Multi hop Remote UE - A) gNB-> Last Relay UE -> Intermediate Relay UE -> Multi hop Remote UE or B) gNB-> Last Relay UE -> First Relay UE -> Multi hop Remote UE |

# 3	Conclusion

This contribution makes the following proposals:

Proposal 1: We can use “U2N Relay UE” instead of “U2N last Relay UE” for spec implementation.

Proposal 2: We can discuss whether “U2N intermediate Relay UE” can be used instead of “U2N first Relay UE”.

- If we use “U2N intermediate Relay UE” instead of “U2N first Relay UE”, we can use parent “U2N Relay UE” or serving “U2N Relay UE” to refer to the "U2N first Relay UE".

Proposal 3: We can discuss whether “U2N Relay UE” can be used commonly instead of “U2N last Relay UE”, “U2N intermediate Relay UE” or “U2N first Relay UE”.

- If we have to describe some specific functionality of the "U2N last Relay UE”, "U2N intermediate Relay UE" or "U2N first Relay UE", we can use the last/intermediate/first “U2N Relay UE”.

Proposal 4: It is not needed to differentiate between when an intermediate U2N Relay UE has its data to transmit or receive, and when an intermediate U2N Relay UE acts as only Remote UE without its own data.

Proposal 5: We can discuss whether to differentiate between the U2N Remote UE in single-hop and U2N Remote UE in multi-hop.