3GPP TSG RAN WG2#129bis	R2-2501711

Wuhan, China, 7th - 11th April 2025

3GPP TSG-RAN WG1 Meeting #120	R1- 2501525

Athens, Greece, February  17th –21st, 2025

Title: 	Reply LS on LMF-based AI/ML Positioning for Case 3b

Response to:	R1-2500389(S2-2501341)

Release:	Rel-19

Work Item:	AIML_CN

Source:	RAN1

To:	SA2

Cc:	RAN2, RAN3

Contact person

Name:	Florent Munier

E-mail Address:	florent.munier @ ericsson.com

# Attachments:	None

# 1	Overall description

RAN1 thanks SA2 for the LS on LMF-based AI/ML Positioning for Case 3b.

RAN1 would like to provide the following agreements and working assumptions reached up to RAN1#119 related to Case 3b as response to SA2 on data types, with reference to RAN1 chairman notes for each.

Agreement [R1-2401937] For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model. Agreement [R1-2401937] For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement [R1-2401937] For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement [R1-2401937] In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. Agreement [R1-2401937] For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc. Agreement [R1-2403821] For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT. Agreement [R1-2403821] For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement [R1-2403821] For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement [R1-2405782] For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed. Working Assumption [R1-2405782] For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement [R1-2407587] For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications. Agreement [R1-2407587] For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement [R1-2409341] For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information Agreement [R1-2409341] From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement. Agreement [R1-2405782] Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement [R1-2405782] Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). Agreement [R1-2407587] For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples, - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples. - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement [R1-2407587] For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Agreement [R1-2500003] For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows. - starting time = first detected path rounded down with timing granularity T. Note: UE-side measurement is a separate discussion. Agreement [R1-2500003] For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

Further, RAN1 would like to provide the following agreements  and conclusion reached at  RAN1#120 related to Case 3b:

Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting: - k = {0...5} Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt' values” in RAN1#119 agreement is resolved by supporting: - Nt' = {8, 16, 24} Agreement For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B): (A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b). - Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly. - The descriptive info of the channel measurement includes at least: - measurement type (A) or type (B) and - measurement parameters. - Regarding measurement parameters, - For (A) path-based measurement, the measurement parameters include: number of additional paths, k. - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k. - FFS: Nt - Note: Type (A) and Type (B) are used for discuss purpose only. Conclusion For Rel-19 AI/ML based positioning Case 3b, - If the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that: the gNB cannot provide measurement with the requested measurement type. - The gNB does not respond to LMF with a measurement report where the measurement type is different from requested. - It is up to RAN3 to decide the signaling details. - Note: It is RAN1 understanding that this is the same behavior as in legacy when the NG-RAN node cannot support the requested measurement. Agreement - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above.

Note1: the working assumptions and agreements above are based on RAN1 understanding for RAN work item (NR_AIML_air).

Note2: "AI/ML based positioning" by itself refers to all positioning cases, including Case 3b.

RAN1 would continue work and may provide additional input in the future to SA2 for Case 3b if any new agreement reached in RAN1.

# 2	Actions

To SA2

ACTION: 	RAN1 respectfully asks SA2 to take the above reply into account for their future work.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #120-bis	7th April– 11th April, 2025	China

TSG RAN WG1 Meeting #121	19th May– 23rd May, 2025	Malta

# 4	Reference

- R1-2401937, Report of RAN1#116 meeting, Athens, Greece, Feb. 2024.

- R1-2403821, Report of RAN1#116bis meeting, Changsha, China, April 2024.

- R1-2405782, Report of RAN1#117 meeting, Fukuoka, Japan, May 2024.

- R1-2407587, Report of RAN1#118 meeting, Maastricht, Netherlands, Aug. 2024.

- R1-2409341, Report of RAN1#118bis meeting, Hefei, China, Oct. 2024.

- R1-250003, Report of RAN1#119 meeting, Orlando, USA, Nov. 2024.

- R1-2501523, Reply LS on LMF-based AI/ML Positioning for Case 2b, RAN1, Athens, Feb. 2025