# 3GPP TSG-RAN2#64						Tdoc R2-086828

# Prague, The Czech Republic, 10th – 14th November 2008

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | 0061 | rev | - | Current version: | 8.3.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to PDCP STATUS REPORT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia Corporation, Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE |  |  |  |  |  |  | Date: |  | 10/11/2008 |
| Category: | F |  |  |  |  |  |  | Release: |  |  |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change: |  | - In the current specfication, Bitmap field is not inserted when there is only one missing SDU. If a SDU is missing and number of SDUs are correctly received after the SDU(e.g. SDU 10 is missing, and SDU 11 ~ 20 are correctly received), those correctly received SDUs will not be reported in the PDCP STATUS REPORT, which will in turn result in unnecessary retransmission of those SDUs. - Bit position corresponding to the padding bit (inserted due to byte alignment) is set to ‘1’. If a SDU corresponding to the padding bit has actually been transmitted but not received by the receiver, the SDU will be wrongly taken as successfully received by the transmitter. For example, SDU 10 is missing, and SDU 11, 12, 13 have been correctly received. FMS is set to 10, and Bitmap is set to ‘1111 1111’. If SDU 14 has been transmitted but unsuccessfully, the 4th bit in the Bitmap wrongly indicates that SDU 14 has been correctly received. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Bit map is inserted when there is any out-of-sequence PDCP SDUs stored in the reordering buffer - Bit position corresponding to the padding bit is set to ‘0’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary retransmission and data loss could occur |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 5.3.1	Transmit operation

When upper layers indicate that a handover has occurred, for radio bearers that are mapped on RLC AM, the UE shall:

-	if the radio bearer is configured by upper layers to send a PDCP status report in the uplink, and if the Flush_Timer is running, compile a status report as indicated below after processing the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers as specified in the subclause 5.2.2.1, and submit it to lower layers as the first PDCP PDU for the transmission, by:

-	setting the FMS field to the PDCP SN of the first missing PDCP SDU;

-	if there areout-of-sequence PDCP SDUs stored, allocating a Bitmap field of length in bits equal to the number of PDCP SNs from and not including the first missing PDCP SDU up to and including the last out-of-sequence PDCP SDUs, rounded up to the next multiple of 8;

-	setting as ‘0’ in the corresponding position in the bitmap field for all PDCP SDUs that have not been received as indicated by lower layers, and optionally PDCP SDUs for which decompression have failed;

-	setting as ‘1’ in the corresponding position in the bitmap field for all PDCP SDUs that have been correctly received and not have been set to ‘0’ above;

-	setting as ‘0’ in all other positions.