3GPP TSG-RAN WG2 Meeting #79bis	R2-124421

Bratislava, Slovakia, 8 - 12 October 2012

Agenda item:		6

Source:	Nokia Siemens Networks, Nokia Corporation

Title:	Inactivity Timer Expiry and Short DRX Cycle

Document for:		Discussion and Decision

# 1	Introduction

Because there are currently no test cases defined for short DRX [36.523], we would like to check what the common understanding is regarding the starting point of the short DRX cycle upon expiry of the inactivity timer.

# 2	Specified Behaviour

The DRX subclause of 36.321 mixes steps that are targeted to control the PDCCH monitoring activity of the current subframe with steps to control the activity of the next. As a result, some steps should be run just before a subframe starts while some others cannot be run before knowing if a PDCCH has been received or even after decoding a MAC CE. Interpretation of the DRX subclause can therefore be confusing. A few discussions papers and CRs on the DRX subclause have been discussed and agreed in RAN2 in the past:

-	At RAN2#62bis, R2-083308 first highlighted possible issues with processing of the PDCCH.

-	At RAN2#63, R2-084105 also highlighted possible issues with processing time of the MAC CE.

-	At RAN2#65bis, it was confirmed with R2-092068 and R2-092069 that the drx-InactivityTimer extends PDCCH monitoring activity by the same number of subframes as configured by RRC. For instance, a drx-InactivityTimer set to psf1 will extend PDCCH monitoring by one subframe after receiving a PDCCH for a new transmission.

-	At RAN2#69bis, a clarification on DRX operation was proposed in R2-102096 but not agreed, without confirming that the proposal was correct or not. Also a timing ambiguity w.r.t. when the MAC CE command is executed by the UE was acknowledged with R2-102394.

-	At RAN2#69bis, an email discussion report discussing two possible interpretations for the short → long and long → short transitions was provided in R2-102440. It was later confirmed at RAN2#70 that the order of the steps matter and that to ensure that the correct cycle is always followed, the start of the onDurationTimer by the SFN condition must be executed last i.e. once the correct cycle to use for this subframe is known (corresponding CR provided in R2-102721).

-	At RAN2#70, with the approval of R2-102721, it was also confirmed that when the drx-InactivityTimer expires in subframe n, the first subframe following the short cycle is that same subframe n and if the SFN of that subframe fulfils the modulo condition, onDurationTimer is also started and PDDCH monitoring starts that subframe n.

Based on those discussions, an interpretation of how the DRX subclause should be read is provided in Annex A. It distinguishes the steps that can be run slightly before the start of a subframe to get PDCCH monitoring ready on time from the steps that requires PDCCH or even MAC CE decoding (and therefore affect the behaviour of the following subframes). W.r.t timers, it is noted that in order to control PDCCH monitoring, whether timers are known to be running matters when a subframe is about to start.

# 3	Expiry of drx-InactivityTimer

When agreeing that the drx-InactivityTimer extends PDCCH monitoring activity by the same number of subframes as configured by RRC, it should be clear that if the last PDCCH for a first transmission is received in subframe n, with a drx-InactivityTimer set to psfx, PDCCH monitoring is then extended up to and including subframe n+psfx. What may not be entirely clear however is in which subframe does the drx-InactivityTimer expire for such a behaviour to be followed: is it subframe n+psfx  or n+psfx +1?

In Rel-8, DRX relies on the expiry of three timers: drx-InactivityTimer, drxShortCycleTimer and HARQ RTT Timer. Since the intended behaviour for the HARQ RTT timer is well known, let us use this as basis to analyse timer expiry. Figure 1 below shows how the HARQ RTT timer behaves:

-	in subframe 1 a PDCCH for new transmission is received → the HARQ timer is started;

-	8 subframes later, PDCCH monitoring should also be turned on → the HARQ timer has to expire for the condition “expire in this subframe” to be validated.

Figure 1 : HARQ RTT Timer

With a value of 8 for the HARQ RTT timer fixed for FDD in 36.321, we can see that the above interpretation is correct: PDCCH monitoring is activated for the subframe where the HARQ RTT timer expires. Because timer expiry is predicable, it can be checked by the UE slightly before the start of the actual subframe to make sure PDCCH monitoring is activated on time (simialry as for the on-duration): the drx-TetransmissionTimer is started for subframe 9 and PDCCH monitoring follows for that subframe (green steps in the Annex).

Observation: PDCCH monitoring is activated by the expiry of the HARQ RTT Timer in the same subframe where the timer exipres.

If we now assume a similar behaviour for the drx-InactivityTimer, in order to extend PDCCH monitoring by psfx subframes, the timer must be started in the subframe where PDCCH is received (which is already obvious from 36.321) and must be running untill subframe n+ psfx where it has to expire. Taking for instance a drx-InactivityTimer set to psf8 (8 subframes), a first PDCCH received in subframe 1, PDCCH monitoring continues untill subframe 9 (see Figure 2 below). Just before starting subframe 9, the UE validates the PDCCH monitoring since at this point in time the timer still is running. In parrallel though, the timer is also known to expire in subframe 9, which also trigger the usage of the short DRX cycle and the start of the drxShortCycleTimer.

Observation: after receiving a PDCCH for new transmission in subframe n, a drx-InactivityTimer set to psfx expires in subframe n+ psfx and that subframe is the first one following the short cycle (drxShortCycleTimer starts as well as onDurationTimer if the SFN condition is fulfilled)

Figure 1 : Inactivity Timer

In this last subframe (#9 in the example), on one hand the drx-InactivityTimer is considered as running to control the active time and PDCCH monitoring when the subframe starts, and on the other hand the steps related to the expiry of the drx-InactivityTimer for the same subframe are also followed. While this matches what has been specified in 36.321 and what has been clarified in the past at RAN2#70, from a pure modelling perspective, this might seem odd. A more logical behaviour would be for the drx-InactivityTimer to first expire and only in the following subframe to follow the actions related to that expiry: stop monitoring PDCCH and start following the short DRX cycle if such is configured.

Observation: from a pure modelling perspective, after receiving a PDCCH for new transmission in subframe n, a drx-InactivityTimer set to psfx should expire in subframe n+ psfx + 1 and in that subframe PDCCH monitoring stops while that subframe is the first one following the short cycle (drxShortCycleTimer starts as well as onDurationTimer if the SFN condition is fulfilled).

# 4	Conclusion

This contribution has analysed the DRX subclause and past RAN2 agreements to conclude the following:

Observation: after receiving a PDCCH for new transmission in subframe n, a drx-InactivityTimer set to psfx expires in subframe n+ psfx and that subframe is the first one following the short cycle (drxShortCycleTimer starts as well as onDurationTimer if the SFN condition is fulfilled).

But also the following observation can be made

Observation: from a pure modelling perspective, after receiving a PDCCH for new transmission in subframe n, a drx-InactivityTimer set to psfx should expire in subframe n+ psfx + 1 and in that subframe PDCCH monitoring stops while that subframe is the first one following the short cycle (drxShortCycleTimer starts as well as onDurationTimer if the SFN condition is fulfilled).

Given the contradiction above, RAN2 is requested to clarify in which subframe should a short cycle start after receiving a PDCCH for new transmission in subframe n for a drx-InactivityTimer set to psfx : is it in subframe n+ psfx  or n+ psfx + 1 ?

# A1	DRX Subclause

A copy of the DRX subclause in Rel-8 is given below:

-	in green: the steps that can be executed shortly before the start of the subframe to guarantee that the PDCCH monitoring activity is ready on time i.e. the steps that govern the current subframe.

-	in yellow: the steps requiring PDCCH decoding i.e. the steps that govern the next subframe

-	in red: the steps that require processing of the MAC CE, and for which a timing ambiguity is known i.e. additional steps that govern the next subframes.

DRX Subclause Start

The UE may be configured by RRC with a DRX functionality that controls the UE’s PDCCH monitoring activity for the UE’s C-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI and Semi-Persistent Scheduling C-RNTI (if configured). When in RRC_CONNECTED, if DRX is configured, the UE is allowed to monitor the PDCCH discontinuously using the DRX operation specified in this subclause; otherwise the UE monitors the PDCCH continuously. When using DRX operation, the UE shall also monitor PDCCH according to requirements found in other subclauses of this specification. RRC controls DRX operation by configuring the timers onDurationTimer, drx-InactivityTimer, drx-RetransmissionTimer (one per DL HARQ process except for the broadcast process), the longDRX-Cycle, the value of the drxStartOffset and optionally the drxShortCycleTimer and shortDRX-Cycle. A HARQ RTT timer per DL HARQ process (except for the broadcast process) is also defined (see subclause 7.7).

When a DRX cycle is configured, the Active Time includes the time while:

-	onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimer or mac-ContentionResolutionTimer (as described in subclause 5.1.5) is running; or

-	a Scheduling Request sent on PUCCH is pending (as described in subclause 5.4.4); or

-	an uplink grant for a pending HARQ retransmission can occur and there is data in the corresponding HARQ buffer; or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the UE has not been received after successful reception of a Random Access Response for the preamble not selected by the UE (as described in subclause 5.1.4).

When DRX is configured, the UE shall for each subframe:

-	if a HARQ RTT Timer expires in this subframe and the data in the soft buffer of the corresponding HARQ process was not successfully decoded:

-	start the drx-RetransmissionTimer for the corresponding HARQ process.

-	if a DRX Command MAC control element is received:

-	stop onDurationTimer;

-	stop drx-InactivityTimer.

-	if drx-InactivityTimer expires or a DRX Command MAC control element is received in this subframe:

-	if the Short DRX cycle is configured:

-	start or restart drxShortCycleTimer;

-	use the Short DRX Cycle.

-	else:

-	use the Long DRX cycle.

-	if drxShortCycleTimer expires in this subframe:

-	use the Long DRX cycle.

-	If the Short DRX Cycle is used and [(SFN * 10) + subframe number] modulo (shortDRX-Cycle) = (drxStartOffset) modulo (shortDRX-Cycle); or

-	if the Long DRX Cycle is used and [(SFN * 10) + subframe number] modulo (longDRX-Cycle) = drxStartOffset:

-	start onDurationTimer.

-	during the Active Time, for a PDCCH-subframe, if the subframe is not required for uplink transmission for half-duplex FDD UE operation and if the subframe is not part of a configured measurement gap:

-	monitor the PDCCH;

-	if the PDCCH indicates a DL transmission or if a DL assignment has been configured for this subframe:

-	start the HARQ RTT Timer for the corresponding HARQ process;

-	stop the drx-RetransmissionTimer for the corresponding HARQ process.

-	if the PDCCH indicates a new transmission (DL or UL):

-	start or restart drx-InactivityTimer.

-	when not in Active Time, CQI/PMI/RI on PUCCH and SRS shall not be reported.

Regardless of whether the UE is monitoring PDCCH or not the UE receives and transmits HARQ feedback when such is expected.

NOTE:	A UE may optionally choose to not send CQI/PMI/RI reports on PUCCH and/or SRS transmissions for up to 4 subframes following a PDCCH indicating a new transmission (UL or DL) received in subframe n-i, where n is the last subframe of Active Time and i is an integer value from 0 to 3. After Active Time is stopped due to the reception of a PDCCH or a MAC control element a UE may optionally choose to continue sending CQI/PMI/RI reports on PUCCH and/or SRS transmissions for up to 4 subframes. The choice not to send CQI/PMI/RI reports on PUCCH and/or SRS transmissions is not applicable for subframes where onDurationTimer is running and is not applicable for subframes n-i to n.

DRX Subclause End