# 3GPP TSG RAN WG2 Meeting #83                                                         	R2-132440

# Barcelona, Spain, Aug. 19 – 23, 2013

# Title: 	Mobility Performance of Inter-frequency Small Cells with Clustered and Random Deployments

# Source: 	BlackBerry, UK Limited

# Agenda Item:	7.2.1

# Document for: 	Discussion

# Introduction

During RAN2#81, three deployment scenarios were identified for small cell enhancements:

Scenario #1: Macro and small cells on the same carrier frequency (intra frequency) connected via non-ideal backhaul

Scenario #2: Macro and small cells on different carrier frequencies (inter frequency) connected via non-ideal backhaul

Scenario #3: Only small cells on one or more carrier frequencies connected via non-ideal backhaul typically low and medium UE mobility

During RAN2#81b and #82, multiple contributions showed the mobility performance of Scenario #2 [1]-[4]. In this contribution, we investigate the mobility performance of Scenario #2 for different small cell deployment scenarios, i.e., clustered small cells and randomly placed small cells. We also compare the performance of Scenario #2 and Scenario #1.

# Simulation Assumptions

Mobility performance has been evaluated using the large area HetNet simulations. There are 57 macro cells in the system with an ISD of 500m. For both small cell deployment scenarios, we assume 10 small cells per macro cell.

- In case of clustered small cells, we use the small cell cluster model agreed in RAN1#72 [5]. We assume one small cell cluster per macro cell as shown in Figure 1. In each cluster, 10 small cells are randomly dropped within a radius of 50m. The minimum distance between two small cells is 20m. The small cell cluster is placed at the boresight 1/2 ISD (i.e., the small cell cluster is deployed at the macro cell boundary).

- In case of randomly placed small cells, 10 small cells are randomly placed in a macro cell. The minimum distance between two small cells is 20m. The minimum distance between a small cell and a macro cell is 75m [7].

In case of Scenario #2, we assume that the macro cell and the small cell are on 2GHz and 3.5GHz, respectively. The pathloss in the small cell at 3.5GHz is derived based on the ITU UMi (Urban Micro) NLOS pathloss model:

PL=36.7log10d+ 22.7+26log10fc=36.8+36.7log10d, d in m, fc in GHz

For Scenario #2 of inter-frequency small cells, the A3 event is configured for both intra-frequency and inter-frequency. For intra-frequency cells the A3 event is triggered based on RSRP and for inter-frequency cells the trigger is based on RSRQ.

The remaining simulation assumptions follow from TR36.839 and TR36.814. Table 1 summarizes our simulation assumptions.

Figure 1 small cell cluster deployment

Table 1 Simulation assumptions

| Items | Descriptions |
| --- | --- |
| Macro cell ISD | 500m |
| Carrier frequency | Scenario #1 of intra-freq small cells: 2GHz for both macro and small cells Scenario #2 of inter-freq small cells: 2GHz for macro cell, 3.5GHz for small cell |
| Small cell placement | Cluster deployment: One small cell cluster per macro cell. In each cluster, 10 small cells are randomly dropped within a radius of 50m. The cluster center is at boresight 1/2 ISD. The minimum distance between two small cells is 20m. Random placement: 10 small cells are randomly placed in one macro cell. The minimum distance between two small cells is 20m. The minimum distance between a small cell and a macro cell is 75m. |
| Target cell selection | Based on RSRP for intra-frequency cells; based on RSRQ for inter-frequency cells |
| Intra-freq RSRP measurement | measurement period 200ms; L3 filter K=1 |
| Inter-freq RSRQ measurement | measurement period 480ms; L3 filter K=1; |
| A3 offset | 2dB for both intra-frequency A3 event (RSRP based) and inter-frequency A3 event (RSRQ based) |
| TTT | 160ms |
| UE speed | 3km/h, 30km/h, and 60km/h |
| Pathloss | Macro cell: PL=15.3+37.6log10d, d in m Small cell at 2GHz: PL=30.5+36.7log10d, d in m Small cell at 3.5GHz: PL=36.8+36.7log10d, d in m |
| UE receiver | 1x2 MRC |
| Fast fading | On |
| Cell loading | 100% |

# Simulation Results

Figure 2 and Figure 3 show the RLF and HO performance of clustered small cells and randomly placed small cells. For comparison we show the performance of both Scenario #1 (intra-frequency small cell) and Scenario #2 (inter-frequency small cell). The complete set of the results are shown in Appendix. Here are the observations:

- We observe that the macro only deployment has a better HOF/RLF performance than the intra-frequency and inter-frequency small cell deployments.

- For the same small cell placement (i.e. clustered or random placement), inter-frequency small cells give a better mobility performance than intra-frequency small cells. This is due to the less interference if the small and macro cells are deployed on different carriers. Although in case of inter-frequency small cells the long measurement period for inter-frequency cells may delay the handover and degrade the mobility performance, it seems that the gain from the reduced interference is more than the degradation from the long measurement period.

- For inter-frequency small cells, we observe that the randomly placed small cells give a worse HOF/RLF performance than the clustered small cells. In case of randomly placed small cells, as small cells are separated far apart, there are more macro-to-pico and pico-to-macro handovers which are inter-frequency handovers. The long measurement period of inter-frequency measurements could delay the handovers and degrade the performance.

- We observe that in some scenarios the mobility performance of inter-frequency small cells could be comparable to or even worse than that of intra-frequency small cells. For example, for UEs at 30km/h, the HOF/RLF performance of randomly placed inter-frequency small cells is similar to or even worse than that of clustered intra-frequency small cells.

- For intra-frequency small cells, we also observe that the randomly placed small cell deployment has a worse HOF/RLF performance than the clustered small cell deployment. In case of randomly placed small cells, as small cells are separated far apart, there are more pico-to-macro and macro-to-pico handovers. As we know from Rel-11 HetNet mobility SI high HOF rates could occur to pico-to-macro HOs [6] due to quickly attenuated signal strength from the pico cell.

Figure 2 RLF performance

Figure 3 HO performance

Figure 4 shows the HOF rates of different HO types for inter-frequency small cells with UE speed of 30km/h. Here are the observations.

- For both clustered and randomly placed small cells, due to the quickly attenuated signal strength from the pico, pico-to-pico handover shows the worst performance and pico-to-macro handover shows the second worst performance.

- The clustered small cells give a much lower pico-to-pico HOF rate than the randomly placed small cells. In the clustered small cell deployment, as the coverage areas of small cells could be heavily overlapped, the intra-frequency pico-to-pico handover could be triggered early based on RSRP when the UE is not far from the serving small cell eNB. The strong signal strength from the serving small cell leads to a low HOF rate. Although the interference in the small cell layer could be strong in the clustered environment, MRC receiver at the UE could suppress the interference to certain extent and provide a sufficient post-procesing SINR to decode HO command successfully.

- We also observe that the clustered small cells give a much lower pico-to-macro HOF rate than the randomly placed small cells. In the clustered small cell deployment, the strong interference in the small cell layer could lead to a low RSRQ and could trigger the inter-frequency pico-to-macro handover early based on RSRQ when the UE is not far from the serving small cell eNB. The strong signal strength from the serving small cell leads to a low HOF rate.

Figure 4 HOF rates of different HO types for inter-frequency small cells (UE speed 30km/h)

# Conclusion

In this paper, we discussed the mobility performance of Scenario #1 and Scenario #2 for both clustered small cells and randomly placed small cells. For the cases we simulated, we have the following observations:

Observation 1: The macro only deployment has a better HOF/RLF performance than the intra-frequency and inter-frequency small cell deployments.

Observation 2: For the same small cell placement (i.e., clustered or randomly placed small cells), the inter-frequency small cell deployment has a better HOF/RLF performance than the intra-frequency deployment.

Observation 3:  The clustered small cells give a better HOF/RLF performance than the randomly placed small cells for both inter-frequency and intra-frequency small cell deployments.

Observation 4: Depending on the small cell placement, in some scenarios the mobility performance of inter-frequency small cells could be comparable to or even worse than that of intra-frequency small cells.

Observation 5: For inter-frequency small cell deployment, pico-to-pico and pico-to-macro handovers show high HOF rates.

# Reference

R2-131087, Mobility performance for small cell deployment scenario 2, Samsung

R2-131455, Mobility performance of inter-frequency small cells, Research in Motion

R2-132072, Mobility performance for small cell deployment scenario 2, Samsung

R2-131928, Mobility robustness in inter-frequency scenario, Renesas

R1-130856, Evaluation assumptions for small cell enhancements – physical layer

TR36.839, Mobility enhancements in heterogeneous networks

TR36.814, Further advancements for E-UTRA physical layer aspects

# Appendix

Table 2 Handover performance

|  |  |  |  | Handover performance in HetNets |  |  |  |  | macro only system |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handover state | Handover metrics | macro-pico | pico-macro | macro-macro | pico-pico | Overall | macro-macro |
| 3km/h, interfreq, cluster |  | 2 | HOFs/UE/s | 0.000000 | 0.000040 | 0.000250 | 0.000250 | 0.000540 | 0.000280 |
|  |  |  | HO failure rate [%] | 0.000000 | 0.500835 | 5.654762 | 2.831595 | 1.812555 | 2.169625 |
|  |  | 3 | HOFs/UE/s | 0.000013 | 0.000000 | 0.000000 | 0.000000 | 0.000013 | 0.000000 |
|  |  |  | HO failure rate [%] | 0.152439 | 0.000000 | 0.000000 | 0.000000 | 0.044209 | 0.000000 |
|  |  | Overall | Successful HOs/UE/s | 0.008633 | 0.007856 | 0.004178 | 0.008594 | 0.029261 | 0.012638 |
|  |  |  | HOFs/UE/s | 0.000013 | 0.000040 | 0.000250 | 0.000250 | 0.000554 | 0.000280 |
|  |  |  | HO failure rate [%] | 0.152439 | 0.500835 | 5.654762 | 2.831595 | 1.856764 | 2.169625 |
| 30km/h, interfreq, cluster |  | 2 | HOFs/UE/s | 0.000079 | 0.001736 | 0.001525 | 0.007390 | 0.010730 | 0.003478 |
|  |  |  | HO failure rate [%] | 0.222222 | 5.607477 | 4.461538 | 12.096427 | 6.634146 | 4.567117 |
|  |  | 3 | HOFs/UE/s | 0.000131 | 0.000053 | 0.000000 | 0.000105 | 0.000289 | 0.000015 |
|  |  |  | HO failure rate [%] | 0.370370 | 0.169924 | 0.000000 | 0.172191 | 0.178862 | 0.019857 |
|  |  | Total | Successful HOs/UE/s | 0.035292 | 0.029165 | 0.032662 | 0.053596 | 0.150715 | 0.072656 |
|  |  |  | HOFs/UE/s | 0.000210 | 0.001788 | 0.001525 | 0.007495 | 0.011019 | 0.003493 |
|  |  |  | HO failure rate [%] | 0.592593 | 5.777400 | 4.461538 | 12.268618 | 6.813008 | 4.586974 |
| 60km/h, interfreq, cluster |  | 2 | HOFs/UE/s | 0.000235 | 0.003994 | 0.003211 | 0.033049 | 0.040488 | 0.008287 |
|  |  |  | HO failure rate [%] | 0.423729 | 11.383929 | 5.000000 | 34.197731 | 16.105919 | 6.467877 |
|  |  | 3 | HOFs/UE/s | 0.000940 | 0.000078 | 0.000000 | 0.000078 | 0.001096 | 0.000018 |
|  |  |  | HO failure rate [%] | 1.694915 | 0.223214 | 0.000000 | 0.081037 | 0.436137 | 0.014405 |
|  |  | Total | Successful HOs/UE/s | 0.054272 | 0.031012 | 0.061007 | 0.063513 | 0.209804 | 0.119826 |
|  |  |  | HOFs/UE/s | 0.001175 | 0.004072 | 0.003211 | 0.033127 | 0.041585 | 0.008306 |
|  |  |  | HO failure rate [%] | 2.118644 | 11.607143 | 5.000000 | 34.278768 | 16.542056 | 6.482282 |
| 3km/h, interfreq, random |  | 2 | HOFs/UE/s | 0.000000 | 0.000373 | 0.000172 | 0.000918 | 0.001463 | 0.000280 |
|  |  |  | HO failure rate [%] | 0.000000 | 3.389831 | 5.555556 | 16.842105 | 4.661792 | 2.169625 |
|  |  | 3 | HOFs/UE/s | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 |
|  |  |  | HO failure rate [%] | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 |
|  |  | Overall | Successful HOs/UE/s | 0.011831 | 0.010626 | 0.002925 | 0.004531 | 0.029913 | 0.012638 |
|  |  |  | HOFs/UE/s | 0.000000 | 0.000373 | 0.000172 | 0.000918 | 0.001463 | 0.000280 |
|  |  |  | HO failure rate [%] | 0.000000 | 3.389831 | 5.555556 | 16.842105 | 4.661792 | 2.169625 |
| 30km/h, interfreq, random |  | 2 | HOFs/UE/s | 0.000000 | 0.006215 | 0.001442 | 0.012927 | 0.020583 | 0.003478 |
|  |  |  | HO failure rate [%] | 0.000000 | 12.315271 | 5.942623 | 25.490196 | 11.330049 | 4.567117 |
|  |  | 3 | HOFs/UE/s | 0.000149 | 0.000099 | 0.000000 | 0.000149 | 0.000398 | 0.000015 |
|  |  |  | HO failure rate [%] | 0.265252 | 0.197044 | 0.000000 | 0.294118 | 0.218938 | 0.019857 |
|  |  | Total | Successful HOs/UE/s | 0.056082 | 0.044149 | 0.022820 | 0.037636 | 0.160688 | 0.072656 |
|  |  |  | HOFs/UE/s | 0.000149 | 0.006314 | 0.001442 | 0.013076 | 0.020981 | 0.003493 |
|  |  |  | HO failure rate [%] | 0.265252 | 12.512315 | 5.942623 | 25.784314 | 11.548987 | 4.586974 |
| 60km/h, interfreq, random |  | 2 | HOFs/UE/s | 0.000246 | 0.012900 | 0.004054 | 0.036488 | 0.053688 | 0.008287 |
|  |  |  | HO failure rate [%] | 0.275862 | 18.197574 | 9.322034 | 35.273159 | 17.493995 | 6.467877 |
|  |  | 3 | HOFs/UE/s | 0.002089 | 0.000000 | 0.000000 | 0.000369 | 0.002457 | 0.000018 |
|  |  |  | HO failure rate [%] | 2.344828 | 0.000000 | 0.000000 | 0.356295 | 0.800641 | 0.014405 |
|  |  | Total | Successful HOs/UE/s | 0.086736 | 0.057988 | 0.039437 | 0.066588 | 0.250749 | 0.119826 |
|  |  |  | HOFs/UE/s | 0.002334 | 0.012900 | 0.004054 | 0.036857 | 0.056145 | 0.008306 |
|  |  |  | HO failure rate [%] | 2.620690 | 18.197574 | 9.322034 | 35.629454 | 18.294636 | 6.482282 |
| 3km/h, intrafreq, cluster |  | 2 | HOFs/UE/s | 0.000278 | 0.000404 | 0.000292 | 0.000390 | 0.001364 | 0.000280 |
|  |  |  | HO failure rate [%] | 4.166667 | 6.041667 | 6.402439 | 3.971631 | 4.917210 | 2.169625 |
|  |  | 3 | HOFs/UE/s | 0.000000 | 0.000000 | 0.000000 | 0.000014 | 0.000014 | 0.000000 |
|  |  |  | HO failure rate [%] | 0.000000 | 0.000000 | 0.000000 | 0.141844 | 0.050176 | 0.000000 |
|  |  | Overall | Successful HOs/UE/s | 0.006401 | 0.006276 | 0.004272 | 0.009407 | 0.026357 | 0.012638 |
|  |  |  | HOFs/UE/s | 0.000278 | 0.000404 | 0.000292 | 0.000404 | 0.001378 | 0.000280 |
|  |  |  | HO failure rate [%] | 4.166667 | 6.041667 | 6.402439 | 4.113475 | 4.967386 | 2.169625 |
| 30km/h, intrafreq, cluster |  | 2 | HOFs/UE/s | 0.003308 | 0.005867 | 0.004279 | 0.008646 | 0.022100 | 0.003478 |
|  |  |  | HO failure rate [%] | 9.047045 | 18.370166 | 13.125846 | 15.288612 | 14.017907 | 4.567117 |
|  |  | 3 | HOFs/UE/s | 0.000044 | 0.000044 | 0.000000 | 0.000088 | 0.000176 | 0.000015 |
|  |  |  | HO failure rate [%] | 0.120627 | 0.138122 | 0.000000 | 0.156006 | 0.111919 | 0.019857 |
|  |  | Total | Successful HOs/UE/s | 0.033217 | 0.026027 | 0.028320 | 0.047818 | 0.135382 | 0.072656 |
|  |  |  | HOFs/UE/s | 0.003353 | 0.005911 | 0.004279 | 0.008734 | 0.022277 | 0.003493 |
|  |  |  | HO failure rate [%] | 9.167672 | 18.508287 | 13.125846 | 15.444618 | 14.129827 | 4.586974 |
| 60km/h, intrafreq, cluster |  | 2 | HOFs/UE/s | 0.013005 | 0.015150 | 0.009519 | 0.032580 | 0.070255 | 0.008287 |
|  |  |  | HO failure rate [%] | 19.556452 | 32.753623 | 17.108434 | 37.732919 | 27.578947 | 6.467877 |
|  |  | 3 | HOFs/UE/s | 0.000268 | 0.000000 | 0.000134 | 0.000402 | 0.000804 | 0.000018 |
|  |  |  | HO failure rate [%] | 0.403226 | 0.000000 | 0.240964 | 0.465839 | 0.315789 | 0.014405 |
|  |  | Total | Successful HOs/UE/s | 0.053228 | 0.031105 | 0.045988 | 0.053362 | 0.183683 | 0.119826 |
|  |  |  | HOFs/UE/s | 0.013273 | 0.015150 | 0.009653 | 0.032982 | 0.071060 | 0.008306 |
|  |  |  | HO failure rate [%] | 19.959677 | 32.753623 | 17.349398 | 38.198758 | 27.894737 | 6.482282 |
| 3km/h, intrafreq, random |  | 2 | HOFs/UE/s | 0.000345 | 0.000960 | 0.000165 | 0.000495 | 0.001965 | 0.000280 |
|  |  |  | HO failure rate [%] | 3.225806 | 9.129815 | 4.564315 | 8.461538 | 6.405868 | 2.169625 |
|  |  | 3 | HOFs/UE/s | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 |
|  |  |  | HO failure rate [%] | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 | 0.000000 |
|  |  | Overall | Successful HOs/UE/s | 0.010350 | 0.009555 | 0.003450 | 0.005355 | 0.028711 | 0.012638 |
|  |  |  | HOFs/UE/s | 0.000345 | 0.000960 | 0.000165 | 0.000495 | 0.001965 | 0.000280 |
|  |  |  | HO failure rate [%] | 3.225806 | 9.129815 | 4.564315 | 8.461538 | 6.405868 | 2.169625 |
| 30km/h, intrafreq, random |  | 2 | HOFs/UE/s | 0.005120 | 0.013481 | 0.002268 | 0.008749 | 0.029618 | 0.003478 |
|  |  |  | HO failure rate [%] | 7.868526 | 21.780105 | 9.020619 | 24.953789 | 15.824100 | 4.567117 |
|  |  | 3 | HOFs/UE/s | 0.000000 | 0.000130 | 0.000000 | 0.000000 | 0.000130 | 0.000015 |
|  |  |  | HO failure rate [%] | 0.000000 | 0.209424 | 0.000000 | 0.000000 | 0.069252 | 0.019857 |
|  |  | Total | Successful HOs/UE/s | 0.059950 | 0.048284 | 0.022878 | 0.026313 | 0.157424 | 0.072656 |
|  |  |  | HOFs/UE/s | 0.005120 | 0.013610 | 0.002268 | 0.008749 | 0.029748 | 0.003493 |
|  |  |  | HO failure rate [%] | 7.868526 | 21.989529 | 9.020619 | 24.953789 | 15.893352 | 4.586974 |
| 60km/h, intrafreq, random |  | 2 | HOFs/UE/s | 0.019509 | 0.034181 | 0.007023 | 0.021538 | 0.082252 | 0.008287 |
|  |  |  | HO failure rate [%] | 17.755682 | 34.706815 | 14.106583 | 41.566265 | 26.535750 | 6.467877 |
|  |  | 3 | HOFs/UE/s | 0.000312 | 0.000000 | 0.000156 | 0.000312 | 0.000780 | 0.000018 |
|  |  |  | HO failure rate [%] | 0.284091 | 0.000000 | 0.313480 | 0.602410 | 0.251762 | 0.014405 |
|  |  | Total | Successful HOs/UE/s | 0.090056 | 0.064303 | 0.042609 | 0.029967 | 0.226934 | 0.119826 |
|  |  |  | HOFs/UE/s | 0.019822 | 0.034181 | 0.007179 | 0.021851 | 0.083032 | 0.008306 |
|  |  |  | HO failure rate [%] | 18.039773 | 34.706815 | 14.420063 | 42.168675 | 26.787513 | 6.482282 |

Table 3 RLF performance

|  | Average number of RLFs/UE/second |  |  |  |
| --- | --- | --- | --- | --- |
|  | State 1 | State 2_Normal | State 2_HOF | Overall |
| 3km/h, interfreq, cluster | 0.000000 | 0.000000 | 0.000474 | 0.000474 |
| 30km/h, interfreq, cluster | 0.000000 | 0.000000 | 0.008231 | 0.008231 |
| 60km/h, interfreq, cluster | 0.000000 | 0.000000 | 0.030229 | 0.030229 |
| 3km/h, interfreq, random | 0.000000 | 0.000000 | 0.001405 | 0.001405 |
| 30km/h, interfreq, random | 0.000000 | 0.000000 | 0.018992 | 0.018992 |
| 60km/h, interfreq, random | 0.000000 | 0.000000 | 0.046685 | 0.046685 |
| 3km/h, intrafreq, cluster | 0.000000 | 0.000000 | 0.001002 | 0.001002 |
| 30km/h, intrafreq, cluster | 0.000000 | 0.000000 | 0.016807 | 0.016807 |
| 60km/h, intrafreq, cluster | 0.000000 | 0.000000 | 0.052826 | 0.052826 |
| 3km/h, intrafreq, random | 0.000000 | 0.000000 | 0.001800 | 0.001800 |
| 30km/h, intrafreq, random | 0.000000 | 0.000000 | 0.025665 | 0.025665 |
| 60km/h, intrafreq, random | 0.000000 | 0.000000 | 0.061650 | 0.061650 |
| 3km/h, macro only | 0.000000 | 0.000000 | 0.000229 | 0.000229 |
| 30km/h, macro only | 0.000000 | 0.000000 | 0.002011 | 0.002011 |
| 60km/h, macro only | 0.000000 | 0.000000 | 0.004134 | 0.004134 |