3GPP TSG-RAN WG2 Meeting #87 	R2-143163

# Dresden, Germany, 18 Aug – 22 Aug 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.331 | CR |  | rev | - | Current version: | 12.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE capabilities for 3GPP WLAN interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UTRA_LTE_WLAN_interw-Core |  |  |  |  |  |  | Date: |  | 2014-08_07 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Introduction of UE capabilities for 3GPP WLAN interworking for UMTS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capability bits are introduced for indicating if the UE has support for 3GPP/WLAN interworking: - New capability for WLAN support. - New capability to indicate support for RAN assistance parameters and whether ANDSF or RAN rules are supported. - New capability for E-UTRAN/UTRAN serving cell - New capability for WLAN radio metrics support (RSNI, RCPI) - New capability for WLAN channel utilization and backhaul support |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A rel-12 UE and onwards will not be able to report it´s 3GPP/WLAN capabilities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.3.41, 11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 25.304, 36.304, 36.331 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First modified section

#### 10.3.3.41	UE multi-mode/multi-RAT capability

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Multi-RAT capability |  |  |  |  |  |
| Support of GSM | MP |  | Boolean |  |  |
| Support of multi-carrier | MP |  | Boolean |  |  |
| Multi-mode capability | MP |  | Enumerated (TDD, FDD, FDD/TDD) |  |  |
| Support of UTRAN to GERAN NACC | CV-not_iRAT_HoInfo |  | Boolean |  | REL-5 |
| Support of Handover to GAN | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportHandoverToGAN) | Absence of this IE means that the UE does not support Handover to GAN. | REL-6 |
| Support of Inter-RAT PS handover | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportInter-RAT-PS-Handover) | Absence of this IE means that the UE does not support Inter-RAT PS Handover to GERAN | REL-6 |
| Support of PS Handover to GAN | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportPSHandoverToGAN) | Absence of this IE means that the UE does not support PS Handover to GAN. | REL-7 |
| Support of E-UTRA FDD | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportEUTRAFDD) | Absence of this IE means that the UE does not support E-UTRA FDD | REL-8 |
| Support of Inter-RAT PS Handover to E-UTRA FDD | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportInter-RATHOToEUTRAFDD) | Absence of this IE means that the UE does not support Inter-RAT PS Handover to E-UTRA FDD | REL-8 |
| Support of E-UTRA TDD | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportEUTRATDD) | Absence of this IE means that the UE does not support E-UTRA TDD | REL-8 |
| Support of Inter-RAT PS Handover to E-UTRA TDD | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportInter-RATHOToEUTRATDD) | Absence of this IE means that the UE does not support Inter-RAT PS Handover to E-UTRA TDD | REL-8 |
| EUTRA Feature Group Indicators | CV-not_iRAT_HoInfo2 |  | Bit string (4) | The definitions of the bits are described in Annex E | REL-8 |
| Support of E-UTRA Multiple Frequency Band Indicators | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | Absence of this IE means that the UE does not support the signalling requirements of multiple radio frequency bands in E-UTRA, as defined in [67], and the UE does not understand the EARFCN signalling for all bands, that overlap with the band(s) supported by the UE. | REL-10 |
| Support of E-UTRA FDD measurements and reporting in CELL_FACH | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportEUTRAmeasurementsandreportinginCELLFACH) | Absence of this IE means that the UE does not support E-UTRA measurement for CELL_FACH for E-UTRA FDD | REL-11 |
| Support of E-UTRA TDD measurements and reporting in CELL_FACH | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportEUTRAmeasurementsandreportinginCELLFACH) | Absence of this IE means that the UE does not support E-UTRA measurement for CELL_FACH for E-UTRA TDD | REL-11 |
| Support of WLAN offloading |  |  | Enumerated (TRUE) | Absence of this IE means that the UE does not support WLAN offloading | REL-12 |
| Support of WLAN offloading (RAN assistance parameters) |  |  | Enumerated (ANDSF, RAN, none) | Absence of this IE means that the UE does not support WLAN offloading with assistance parameters | REL-12 |
| Support of WLAN offloading (RAN assistance parameters for UTRAN serving cell) |  |  | Enumerated (TRUE) | Absence of this IE means that the UE does not support the UTRAN parameters for WLAN offloading | REL-12 |
| Support of WLAN offloading (RAN assistance parameters for WLAN radio metrics) |  |  | Enumerated (TRUE) | Absence of this IE means that the UE does not support the WLAN parameters for WLAN offloading | REL-12 |
| Support of WLAN offloading (RAN assistance parameters for WLAN channel utilization and backhaul) |  |  | Enumerated (TRUE) | Absence of this IE means that the UE does not support the WLAN channel utilization and backhaual parameters for WLAN offloading | REL-12 |

| Condition | Explanation |
| --- | --- |
| not_iRAT_HoInfo | The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is mandatory present. |
| not_iRAT_HoInfo2 | The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is optional. |

NOTE:	The UE sets the capability of Inter-RAT PS Handover (by means of the IE "Support of Inter-RAT PS handover") to the same value as the corresponding GERAN capability in [5].

Next modified section

## 11.3	Information element definitions

/////////////---------- skip unmodified definitions ----------------//////////////////

MultiModeRAT-Capability-v590ext ::=	SEQUENCE {

supportOfUTRAN-ToGERAN-NACC			BOOLEAN

}

MultiModeRAT-Capability-v680ext ::=	SEQUENCE {

supportOfHandoverToGAN				ENUMERATED { doesSupportHandoverToGAN }		OPTIONAL

}

MultiModeRAT-Capability-v770ext ::=	SEQUENCE {

supportOfPSHandoverToGAN			ENUMERATED { doesSupportPSHandoverToGAN }	OPTIONAL

}

MultiModeRAT-Capability-v860ext ::=	SEQUENCE {

supportOfEUTRAFDD					ENUMERATED { doesSupportEUTRAFDD }				OPTIONAL,

supportOfInterRATHOToEUTRAFDD		ENUMERATED { doesSupportInterRATHOToEUTRAFDD }	OPTIONAL,

supportOfEUTRATDD					ENUMERATED { doesSupportEUTRATDD }				OPTIONAL,

supportOfInterRATHOToEUTRATDD		ENUMERATED { doesSupportInterRATHOToEUTRATDD }	OPTIONAL,

eutraFeatureGroupIndicators			BIT STRING (SIZE (4))							OPTIONAL

}

MultiModeRAT-Capability-vae0ext ::=			SEQUENCE {

eutraSupportOfMFBI						ENUMERATED { true }							OPTIONAL

}

MultiModeRAT-Capability-vb50ext ::=	SEQUENCE {

supportOfEUTRAFDDMeasurementReportingInCELLFACH		ENUMERATED { 																doesSupportOfEUTRAFDDMeasurementReportingInCELLFACH }	OPTIONAL,

supportOfEUTRATDDMeasurementReportingInCELLFACH		ENUMERATED { 																doesSupportOfEUTRATDDMeasurementReportingInCELLFACH }	OPTIONAL

}

MultiModeRAT-Capability-vcxyext ::=	SEQUENCE {

supportOfWLANOffloading							ENUMERATED { true } OPTIONAL,

supportOfWLANOffloadingRANassist				ENUMERATED { andsf, ran, none }	OPTIONAL,

supportOfWLANOffloadingRANassistUTRAN			ENUMERATED { true } OPTIONAL,

supportOfWLANOffloadingRANassistWLANradio		ENUMERATED { true } OPTIONAL,

supportOfWLANOffloadingRANassistWLANbackhaul	ENUMERATED { true }	OPTIONAL

}

N-300 ::=							INTEGER (0..7)

N-301 ::=							INTEGER (0..7)

N-302 ::=							INTEGER (0..7)

N-304 ::=							INTEGER (0..7)

N-308 ::=							INTEGER (1..8)

N-310 ::=							INTEGER (0..7)

/////////////---------- skip unmodified definitions ----------------//////////////////

UE-RadioAccessCapability-vcxyext-IEs ::= SEQUENCE {

--the capability is for 1.28Mcps TDD only

supportofNon-rectResourceAllocation				ENUMERATED { true }				OPTIONAL,

--the capability is for FDD only

measurementCapability							MeasurementCapability-vcxyext	OPTIONAL,

supportofEnhancedServingCellChangeForEvent1C	ENUMERATED { true }				OPTIONAL,

supportofCellReselectionIndicationReporting		ENUMERATED { true }				OPTIONAL,

multiModeRAT-Capability							MultiModeRAT-Capability-vcxyext

}