3GPP TSG-RAN WG 2 Meeting #87	R2-143164

Dresden, Germany, 18th – 22th August 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.306 | CR | 0xxx | rev | - | Current version: | 12.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE capabilities for 3GPP WLAN interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UTRA_LTE_WLAN_interw-Core |  |  |  |  |  |  | Date: |  | 2014-08-07 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | In order for the UE to indicate its 3GPP/WLAN interworking capabilities |  |  |  |  |  |  |  |  |
| Summary of change: |  | New optional capabilities to indicates support for 3GPP/WLAN interworking. - New capability for WLAN support. - New capability to indicate support for RAN assistance parameters and whether ANDSF or RAN rules are supported. - New capability for E-UTRAN/UTRAN serving cell - New capability for WLAN radio metrics support (RSNI, RCPI) - New capability for WLAN channel utilization and backhaul support |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A rel-12 UE and onwards will not be able to report it´s 3GPP/WLAN capabilities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7, 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS/TR 36.304 CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR 36.331 CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR 25.331 CR ... TS/TR 25.304 CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First modified section

## 4.7	Multi-RAT related parameters

Support of GSM

Defines whether GSM is supported or not. There is a separate parameter for each GSM frequency band.

Support of multi-carrier

Defines whether multi-carrier is supported or not.

Support of UTRAN to GERAN NACC

Defines whether UTRAN to GERAN NACC is supported or not.

Support of Handover to GAN

Defines whether CS Handover to GAN is supported or not.

Support of Inter-RAT PS Handover

Defines whether Inter-RAT PS Handover to GERAN is supported or not.

Support of PS Handover to GAN

Defines whether PS Handover to GAN is supported or not.

Support of E-UTRA FDD

Defines whether E-UTRA FDD is supported or not. There is a separate parameter for each E-UTRA frequency band. If the UE supports E-UTRA FDD, it shall also support absolute priority based cell re-selection to GERAN if GERAN is supported by the UE.

Support of Inter-RAT PS Handover to E-UTRA FDD

Defines whether Inter-RAT PS Handover to E-UTRA FDD is supported or not.

Support of E-UTRA FDD measurements and reporting in CELL_FACH

Defines whether E-UTRA measurement for CELL_FACH for E-UTRA FDD is supported or not.

Support of E-UTRA TDD

Defines whether E-UTRA TDD is supported or not. There is a separate parameter for each E-UTRA frequency band. If the UE supports E-UTRA TDD, it shall also support absolute priority based cell re-selection to GERAN if GERAN is supported by the UE.

Support of Inter-RAT PS Handover to E-UTRA TDD

Defines whether Inter-RAT PS Handover to E-UTRA TDD is supported or not.

Support of E-UTRA TDD measurements and reporting in CELL_FACH

Defines whether E-UTRA measurement for CELL_FACH for E-UTRA TDD is supported or not.

Support of E-UTRA Multiple Frequency Band Indicators

This parameter is only applicable for a UE supporting E-UTRA. It indicates if the UE supports the signalling requirements of multiple radio frequency bands in a cell, as defined in [17],and if the UE understands the EARFCN signalling for all bands, that overlap with the band(s) supported by the UE, and that are defined in the earliest version of [18] that includes all UE supported bands.

Suppport of WLAN offloading

This parameter defines whether the UE supports 3GPP/WLAN offloading.

Support of WLAN offloading (RAN assistance parameters)

This parameter defines whether the UE supports 3GPP/WLAN offloading with assistance of RAN parameters, and whether this are supported in ANDSF and/or in RAN rules.FFS (Provided in system information or Dedicated message XXX)

Support of WLAN offloading (RAN assistance parameters for E-UTRAN serving cell)

This parameter defines whether the UE supports the UTRAN serving cell RAN assistance parameters.

#### Support of WLAN offloading (RAN assistance parameters for WLAN radio metrics)

This parameter defines whether the UE supports WLAN Radio metrics (RSNI, RCPI) RAN assistance parameters.

#### Support of WLAN offloading (RAN assistance parameters for WLAN channel utilization and backhaul)

This parameter defines whether the UE supports WLAN channel utilization and backhaul RAN assistance parameters

Second modified section

# 5	Possible UE radio access capability parameter settings

## 5.1	Value ranges

# Table 5.1: UE radio access capability parameter value ranges

|  |  | UE radio access capability parameter | Value range |
| --- | --- | --- | --- |
| PDCP parameters |  | Support for RFC 2507 | Yes/No |
|  |  | Support for RFC 3095 | Yes/No |
|  |  | Support for RFC 3095 context relocation | Yes/No |
|  |  | Support for loss-less SRNS relocation | Yes/No |
|  |  | Support for loss-less DL RLC PDU size change | Yes/No |
|  |  | Maximum header compression context space | 1024, 2048, 4096, 8192, 16384, 32768, 65536, 131072 bytes |
|  |  | Maximum number of ROHC context sessions | 2, 4, 8, 12, 16, 24, 32, 48, 64, 128, 256, 512, 1024, 16384 |
|  |  | Support for Reverse Decompression | Not supported, 1..65535 |
|  |  | Support for CS voice over HSPA | Yes/No |
| RLC, MAC-hs, MAC-ehs and MAC-i/is parameters |  | Total RLC AM, MAC-hs and MAC-ehs buffer size | 2, 10, 50, 100, 150, 200, 300, 400, 500, 750, 1000, 1150, 1250, 1800, 2000, 2300, 2550, 3400, 3500, 4400, 4500, 5000 kBytes |
|  |  | Maximum number of AM entities | 3, 4, 5, 6, 8, 16, 30 |
|  |  | Maximum RLC AM window size | 2047, 4095 |
|  |  | Support for MAC-ehs | Yes/No |
|  |  | Support for two logical channels | Yes/No |
|  |  | Support of MAC-i/is | Yes/No |
|  |  | Support of MAC-ehs window size extension | Yes/No |
|  |  | Support of UM RLC re-establishment via reconfiguration | Yes/No |
| PHY parameters | Transport channel parameters in downlink | Maximum sum of number of bits of all transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840, 204640 |
|  |  | Maximum sum of number of bits of all convolutionally coded transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840, 204640 |
|  |  | Maximum sum of number of bits of all turbo coded transport blocks being received at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840, 204640 |
|  |  | Maximum number of simultaneous transport channels | 4, 8, 16, 32 |
|  |  | Maximum number of simultaneous CCTrCH | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum total number of transport blocks received within TTIs that end within the same 10 ms interval | 4, 8, 16, 32, 48, 64, 96, 128, 256, 512 |
|  |  | Maximum number of TFC | 16, 32, 48, 64, 96, 128, 256, 512, 1024 |
|  |  | Maximum number of TF | 32, 64, 128, 256, 512, 1024 |
|  |  | Support for turbo decoding | Yes/No |
|  | Transport channel parameters in uplink | Maximum sum of number of bits of all transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum sum of number of bits of all convolutionally coded transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum sum of number of bits of all turbo coded transport blocks being transmitted at an arbitrary time instant | 640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840 |
|  |  | Maximum number of simultaneous transport channels | 2, 4, 8, 16, 32 |
|  |  | Maximum number of simultaneous CCTrCH of DCH type (TDD only) | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum total number of transport blocks transmitted within TTIs that start at the same time | 2, 4, 8, 16, 32, 48, 64, 96, 128, 256, 512 |
|  |  | Maximum number of TFC | 4, 8, 16, 32, 48, 64, 96, 128, 256, 512, 1024 |
|  |  | Maximum number of TF | 32, 64, 128, 256, 512, 1024 |
|  |  | Support for turbo encoding | Yes/No |
|  | FDD Physical channel parameters in downlink | Maximum number of DPCHcodes to be simultaneously received | 1, 2, 3, 4, 5, 6, 7, 8 |
|  |  | Maximum number of physical channel bits received in any 10 ms interval (DPCH, S-CCPCH) | 600, 1200, 2400, 3600, 4800, 7200, 9600, 14400, 19200, 28800, 38400, 48000, 57600, 67200, 76800 |
|  |  | Support for SF 512 and 80 ms TTI for DPCH | Yes/No |
|  |  | Support of HS-PDSCH in CELL_DCH | Yes/No |
|  |  | Support of HS-SCCHless HS-DSCH | Yes/No |
|  |  | Support of HS-PDSCH in CELL_FACH | Yes/No |
|  |  | Support of HS-PDSCH in CELL_PCH and URA_PCH | Yes/No |
|  |  | Support of Enhanced F-DPCH | Yes/No |
|  |  | Support of Target Cell Pre-Configuration | Yes/No |
|  |  | Support of HS-DSCH DRX operation | Yes/No |
|  |  | Support of Node B triggered HS-DPCCH transmission | Yes/No |
|  |  | Support of HS-DSCH DRX operation with second DRX cycle | Yes/No |
|  |  | Support for Two DRX schemes in URA_PCH and CELL_PCH | Yes/No |
|  |  | Support of TX Diversity on DL Control Channels by MIMO Capable UE when MIMO operation is active | Yes/No |
|  |  | Support for cell-specific Tx diversity configuration for dual-cell operation | Yes/No |
|  |  | Support of MIMO only with single-stream restriction | Yes/No |
|  |  | Support for dual cell with MIMO operation in different bands | Yes/No |
|  |  | Support of MIMO mode with four transmit antennas operation only with dual-stream restriction | Yes/No |
|  |  | Support of dual band operation (Radio Access Capability Band Combination List) | 1..16 |
|  |  | >Band Combination | 1..256 |
|  |  | >Supported Carrier Combination List |  |
|  |  | >>Carrier Combination (1,2) | Yes/No |
|  |  | >>Carrier Combination (2,1) | Yes/No |
|  |  | >>Carrier Combination (1,3) | Yes/No |
|  |  | >>Carrier Combination (3,1) | Yes/No |
|  |  | >>Carrier Combination (2,2) | Yes/No |
|  |  | >>Carrier Combination (1,4) | Yes/No |
|  |  | >>Carrier Combination (4,1) | Yes/No |
|  |  | >>Carrier Combination (1,5) | Yes/No |
|  |  | >>Carrier Combination (5,1) | Yes/No |
|  |  | >>Carrier Combination (1,6) | Yes/No |
|  |  | >>Carrier Combination (6,1) | Yes/No |
|  |  | >>Carrier Combination (1,7) | Yes/No |
|  |  | >>Carrier Combination (7,1) | Yes/No |
|  |  | >>Carrier Combination (2,3) | Yes/No |
|  |  | >>Carrier Combination (3,2) | Yes/No |
|  |  | >>Carrier Combination (2,4) | Yes/No |
|  |  | >>Carrier Combination (4,2) | Yes/No |
|  |  | >>Carrier Combination (2,5) | Yes/No |
|  |  | >>Carrier Combination (5,2) | Yes/No |
|  |  | >>Carrier Combination (2,6) | Yes/No |
|  |  | >>Carrier Combination (6,2) | Yes/No |
|  |  | >>Carrier Combination (3,3) | Yes/No |
|  |  | >>Carrier Combination (3,4) | Yes/No |
|  |  | >>Carrier Combination (4,3) | Yes/No |
|  |  | >>Carrier Combination (4,4) | Yes/No |
|  |  | >>Carrier Combination (3,5) | Yes/No |
|  |  | >>Carrier Combination (5,3) | Yes/No |
|  |  | Support for Multiflow | Yes/No (per frequency band) |
|  |  | >Support for Multiflow and MIMO | No/Single stream/Dual stream NOTE: If a UE supports single-stream MIMO transmission with Multiflow, then single-stream MIMO is supported in all the frequency bands where Multiflow operation is supported. The dual-stream MIMO transmission with Multiflow support is per frequency band. |
|  |  | >Support for Multiflow with non-contiguous carrier allocation | 5, 10, any gap size (per frequency band) |
|  |  | Support for Multiflow in different bands | Yes/No (per frequency band combination) |
|  |  | Support for Multiflow with MIMO operation in different bands | No/Single stream/Dual stream NOTE1: If a UE does not support "Multiflow with MIMO operation in different bands", then no MIMO is supported in any of the band combinations where Multiflow is supported. NOTE2: If a UE supports single-stream "Multiflow with MIMO operation in different bands", then single-stream MIMO is supported in all the band combinations where Multiflow is supported. NOTE3: If a UE supports dual-stream "Multiflow with MIMO operation in different bands", then dual-stream MIMO transmission is supported in all the band combinations, in which the UE supports Multiflow and dual-stream MIMO in both bands of the corresponding band combination. Otherwise the UE supports single-stream MIMO in that band combination. |
|  |  | Support of MIMO mode with four transmit antennas operation | Yes/No (per frequency band) |
|  |  | Non-contiguous multi-cell | 1..3 |
|  |  | >Aggregated cells | 2, 3, 4 |
|  |  | >Gap size | 5, 10, any gap size |
|  |  | >Non-contiguous multi-cell Combination (2,2) | Yes/No |
|  |  | >Non-contiguous multi-cell Combination (3,1) (1,3) | Yes/No |
|  |  | Support of HS-DPCCH power offset extension | Yes/No |
|  |  | Support of STTD on DL Control Channels when Multiflow operation is active | Yes/No |
|  |  | Non-contiguous multi-cell with MIMO | Yes/No |
|  | FDD Physical channel parameters in uplink | Maximum number of DPDCH bits transmitted per 10 ms | 600, 1200, 2400, 4800, 9600, 19200, 28800, 38400, 48000, 57600 |
|  |  | Support of E-DPDCH | Yes/No |
|  |  | Support of Discontinuous Transmission in CELL_DCH | Yes/No |
|  |  | Support of Slot Format #4 | Yes/No |
|  |  | Support for E-DPCCH power interpolation formula | Yes/No |
|  |  | Support for E-DPCCH power boosting | Yes/No |
|  |  | Support of common E-DCH | Yes/No |
|  |  | Support of Common E-RGCH based interference control | Yes/No |
|  |  | Support of Fallback to R99 PRACH | Yes/No |
|  |  | Support of Concurrent deployment | Yes/No |
|  |  | Support of TTI alignment and Per HARQ process | Yes/No |
|  |  | Support of uplink open loop transmit diversity | Yes/No (per frequency band) |
|  |  | Support of uplink closed loop transmit diversity | Yes/No (per frequency band) |
|  |  | Support of Uplink MIMO | Yes/No (per frequency band) |
|  | TDD 3.84 Mcps physical channel parameters in downlink | Maximum number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per frame | 1, 2, 3..224 |
|  |  | Minimum SF | 16, 1 |
|  |  | Support of PDSCH | Yes/No |
|  |  | Support of HS-PDSCH | Yes/No |
|  |  | Maximum number of physical channels per timeslot | 1..16 |
|  | TDD 3.84 Mcps physical channel parameters in uplink | Maximum Number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per timeslot | 1, 2 |
|  |  | Minimum SF | 16, 8, 4, 2, 1 |
|  |  | Support of PUSCH | Yes/No |
|  |  | Support of E-PUCH | Yes/No |
|  | TDD 7.68 Mcps physical channel parameters in downlink TDD 7.68 Mcps physical channel parameters in downlink | Maximum number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per frame | 1, 2, 3..448 |
|  |  | Minimum SF | 32, 1 |
|  |  | Support of PDSCH | Yes/No |
|  |  | Support of HS-PDSCH | Yes/No |
|  |  | Maximum number of physical channels per timeslot | 1..32 |
|  | TDD 7.68 Mcps physical channel parameters in uplink | Maximum Number of timeslots per frame | 1..14 |
|  |  | Maximum number of physical channels per timeslot | 1, 2 |
|  |  | Minimum SF | 32, 16, 8, 4, 2, 1 |
|  |  | Support of PUSCH | Yes/No |
|  |  | Support of E-PUCH | Yes/No |
|  | TDD 1.28 Mcps physical channel parameters in downlink | Maximum number of timeslots per subframe | 1..6 |
|  |  | Maximum number of physical channels per subframe | 1, 2, 3, …, 96 |
|  |  | Minimum SF | 16, 1 |
|  |  | Support of PDSCH | Yes/No |
|  |  | Support of HS-PDSCH | Yes/No |
|  |  | Maximum number of physical channels per timeslot | 1..16 |
|  |  | Support 8PSK | Yes/No |
|  |  | UE specific capability Information LCR TDD | Enumerated (NF, TriRxUniTx, TriRxTriTx, HexRxUniTx, HexRxTriTx, HexRxHexTx, TwoRxUniTxDiscontiguous, TwoRxTwoTxDiscontiguous, TwoRxUniTxContiguous, TwoRxTwoTxContiguous) NOTE: If three frequencies are supported, the three frequencies shall be configured within 5 MHz; if six frequencies are supported, the six frequencies shall be configured within 10MHz; TwoRxUniTxDiscontiguous and TwoRxTwoTxDiscontiguous mean that the UE is capable of supporting two non-adjacent carriers; TwoRxUniTxContiguous and TwoRxTwoTxContiguous mean that the UE is only capable of supporting two adjacent carriers; if two non-adjacent carriers are supported, the two carriers can be in the same band or in two different bands. |
|  |  | Support of HS-PDSCH in CELL_FACH | Yes/No |
|  |  | Support of SPS | Yes/No |
|  |  | Support of HS-SCCH/E-AGCH Discontinuous Reception | Yes/No |
|  |  | Support of SF Mode For HS-PDSCH dual stream | Yes/No |
|  |  | Support of Enhanced TS0 | Yes/No |
|  |  | Support of Non-rectangular Resource Allocation | Yes/No |
|  | TDD 1.28 Mcps physical channel parameters in uplink | Maximum number of timeslots per subframe | 1..6 |
|  |  | Maximum number of physical channels per timeslot | 1, 2 |
|  |  | Minimum SF | 16, 8, 4, 2, 1 |
|  |  | Support of 8PSK | Yes/No |
|  |  | Support of PUSCH | Yes/No |
|  |  | Support of E-PUCH | Yes/No |
|  |  | Support of E-DCH in CELL_FACH | Yes/No |
| RF parameters | FDD RF parameters | UE power class | 3, 4 NOTE: Only power classes 3 and 4 are part of this release of the specification |
|  |  | Radio frequency bands | The radio frequency bands defined in [4]. UEs that support band XIX shall also support band VI |
|  |  | Tx/Rx frequency separation | Defined in [4] for the respective supported radio frequency band |
|  |  | Support of Multiple Frequency Band Indicators | Yes/No |
| RF parameters | TDD 3.84 Mcps RF parameters | UE power class | 2, 3 NOTE: Only power classes 2 and 3 are part of this release of the specification |
|  |  | Radio frequency bands | The radio frequency bands defined in [5] |
|  | TDD 1.28 Mcps RF parameters | UE power class | 2, 3 |
|  |  | Radio frequency bands | The radio frequency bands defined in [5] |
| Multi-mode related parameters |  | Support of UTRA FDD | Yes/No |
|  |  | Support of UTRA TDD 3.84 Mcps | Yes/No |
|  |  | Support of UTRA TDD 1.28 Mcps | Yes/No |
| Multi-RAT related parameters |  | Support of GSM | Yes/No (per GSM frequency band) |
|  |  | Support of multi-carrier | Yes/No |
|  |  | Support of UTRAN to GERAN Network Assisted Cell Change | Yes/No |
|  |  | Support of Handover to GAN | Yes/No |
|  |  | Support of Inter-RAT PS Handover | Yes/No |
|  |  | Support of PS Handover to GAN | Yes/No |
|  |  | Support of E-UTRA FDD | Yes/No (per E-UTRA frequency band) |
|  |  | Support of Inter-RAT PS Handover to E-UTRA FDD | Yes/No |
|  |  | Support of E-UTRA FDD measurements and reporting in CELL_FACH | Yes/No |
|  |  | Support of E-UTRA TDD | Yes/No (per E-UTRA frequency band) |
|  |  | Support of Inter-RAT PS Handover to E-UTRA TDD | Yes/No |
|  |  | Support of E-UTRA Multiple Frequency Band Indicators | Yes/No |
|  |  | Support of E-UTRA TDD measurements and reporting in CELL_FACH | Yes/No |
|  |  | Suppport of WLAN offloading | Yes/No |
|  |  | Support of WLAN offloading (RAN assistance parameters) | ANDSF / RAN rules / Both/ None |
|  |  | Support of WLAN offloading (RAN assistance parameters for E-UTRAN serving cell) | Yes/No |
|  |  | Support of WLAN offloading (RAN assistance parameters for WLAN radio metrics) | Yes/No |
|  |  | Support of WLAN offloading (RAN assistance parameters for WLAN channel utilization and backhaul) | Yes/No |
|  |  | Support of ciphering algorithm UEA0 | Yes |
| Security parameters |  | Support of ciphering algorithm UEA1 | Yes |
|  |  | Support of ciphering algorithm UEA2 | Yes |
|  |  | Support of integrity protection algorithm UIA1 | Yes |
|  |  | Support of integrity protection algorithm UIA2 | Yes |
|  |  | Standalone location method(s) supported | Yes/No |
| UE positioning related parameters |  | UE based OTDOA supported | Yes/No |
|  |  | Network assisted GPS support | Network based / UE based / Both/ None |
|  |  | Network Assisted GANSS support List: | per GANSS |
|  |  | >GANSS ID | Galileo / SBAS / Modernized GPS / QZSS / GLONASS/ BDS |
|  |  | >SBAS IDs | WAAS / EGNOS / MSAS / GAGAN |
|  |  | >GANSS mode | Network based / UE based / Both/ None |
|  |  | >GANSS Signal ID | 0..7 |
|  |  | >GANSS Signal IDs | Yes/No (per GANSS signal) |
|  |  | >Support for GANSS timing of cell frames measurement | Yes/No |
|  |  | >Support for GANSS Carrier-Phase Measurement | Yes/No |
|  |  | >Support for non-native assistance choices | Yes/No |
|  |  | Support for GPS timing of cell frames measurement | Yes/No |
|  |  | Support for IPDL | Yes/No |
|  |  | Support for Rx-Tx time difference type 2 measurement | Yes/No |
|  |  | Support for UE Positioning assisted GPS measurement validity in CELL_PCH and URA_PCH RRC states | Yes |
|  |  | Support for SFN-SFN observed time difference type 2 measurement | Yes/No |
|  |  | Need for downlink compressed mode | Yes/No (per frequency band, UTRA mode and RAT) |
| Measurement related capabilities |  | Need for uplink compressed mode | Yes/No (per frequency band, UTRA mode and RAT) |
|  |  | Support for absolute priority based cell re-selection in UTRAN | Yes/No |
|  |  | Support for System Information Block type 11bis | Yes |
|  |  | Adjacent Frequency measurements without compressed mode | Yes/No |
|  |  | Inter-band Frequency measurements without compressed mode | Yes/No |
|  |  | Enhanced inter-frequency measurements without compressed mode | Yes/No |
|  |  | Extended measurements ID support | Yes/No |
|  |  | Frequency specific compressed mode | Yes/No |
|  |  | Frequency specific compressed mode for non-contiguous operation | Yes/No |
|  |  | Inter-frequency detected set measurements | Yes/No |
|  |  | Inter-frequency measurements on configured carriers without compressed mode | Yes/No |
|  |  | Cells excluded from detected set measurements | Yes/No |
|  |  | Wideband RSRQ FDD measurements | Yes/No |
|  |  | Wideband RSRQ TDD measurements | Yes/No |
|  |  | Access Stratum release indicator | R99, REL-4, REL-5, REL-6, REL-7, REL-8, REL-9, REL-10, REL-11 |
| General capabilities |  | Device type | Benefits from NW-based battery consumption optimisation / Does not benefit from NW-based battery consumption optimisation |
|  |  | DL capability with simultaneous HS-DSCH configuration | 32 kbps, 64 kbps, 128 kbps, 384 kbps |
| DL capabilities with simultaneous HS-DSCH |  | UL capabilities with simultaneous E-DCH | 64 kbps |
| UL capabilities with simultaneous E-DCH |  | Support of logged measurements in Idle mode and PCH States | Yes/No |
| UE based network performance measurements parameters |  | Support of UTRAN ANR | Yes/No |
|  |  | CSG Proximity Indication capabilities | Support of intra-frequency proximity indication |
| Home Node B Inbound Mobility Related Parameters | CSG Proximity Indication capabilities Neighbour Cell SI Acquisition capabilities | Support of inter-frequency proximity indication | Yes/No |
|  |  | Support of E-UTRA proximity indication | Yes/No |
|  |  | Support of intra-frequency SI acquisition for HO | Yes/No |
|  | Neighbour Cell SI Acquisition capabilities Voice over UTRA PS HS Support | Support of inter-frequency SI acquisition for HO | Yes/No |
|  |  | Support of E-UTRA SI acquisition for HO | Yes/No |
|  |  | Yes/No | Yes/No |
| IMS Voice Parameters |  | SRVCC Support from UTRA to UTRA | Yes/No |
|  |  | SRVCC Support from UTRA to GERAN | Yes/No |
|  |  | rSRVCC support from UTRA CS to E-UTRAN FDD | Yes/No |
|  |  | rSRVCC support from UTRA CS to E-UTRAN TDD | Yes/No |