3GPP TSG-RAN WG2 Meeting #91	R2-153160

# Beijing, China, August 24 – August 28, 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | CRNum | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDCP SDU discard in split bearers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_dualC_enh-Core |  |  |  |  |  |  | Date: |  | 2015-08-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The current specification mandates that the UE shall discard the PDCP SDU when the corresponding discardTimer expires. However, if the PDCP SDU is already associated with a PDCP SN, the discard of the PDCP SDU will create a SN gap. For split bearers, the SN gap would cause increased reordering delay because all the following PDCP SDUs are stored in the receiver until the reordering timer expiry. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A NOTE is added to explain that SN gap would cause reordering delay in the receiving PDCP entity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reordering delay would be increased due to discard of PDCP SDU already associated with a PDCP SN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

5.4        PDCP discard

When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the UE shall discard the PDCP SDU along with the corresponding PDCP PDU. If the corresponding PDCP PDU has already been submitted to lower layers the discard is indicated to lower layers.

NOTE:	For split bearers, discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP PDUs, which increases PDCP reordering delay in the receiving PDCP entity. How to prevent SN gap in the transmitted PDCP PDUs after PDCP SDU discard is left up to UE implementation.