3GPP TSG RAN WG2 Meeting #91	R2-153682

Beijing, China, Aug 24th - 28th, 2015

Source:	CATT

Title:	Considerations on Layer-2 ID Collision

Agenda Item:	7.5.1.4

Document for:	Discussion and Decision

# Introduction

In [1], the description of one-to-one communication is as below:

ProSe direct communication one-to-one is realised by establishing a secure layer-2 link over PC5 between two UEs. Each UE has a Layer-2 ID for unicast communication that is included in the Source Layer-2 ID field of every frame that it sends on the layer-2 link and in the Destination Layer-2 ID of every frame that it receives on the layer-2 link. NOTE: Conflicts between Destination Layer-2 ID for unicast and one-to-many communication will be resolved by RAN2 WG. The UE needs to ensure that the Layer-2 ID for unicast communication is at least locally unique. So the UE should be prepared to handle Layer-2 ID conflicts with adjacent UEs using unspecified mechanisms (e.g. self-assign a new Layer-2 ID for unicast communication when a conflict is detected). The layer-2 link for ProSe direct communication one-to-one is identified by the combination of the Layer-2 IDs of the two UEs. This means that the UE can engage in multiple layer-2 links for ProSe direct communication one-to-one using the same Layer-2 ID.

It is clear that SA2 want to reuse ProSe UE ID as Source Layer-2 ID and Destination Layer-2 ID to realize one-to-one communication. SA2 hopes Layer-2 ID collision to be resolved by RAN2. In this contribution, we discuss Layer-2 ID collision cases and give some proposals.

# Discussion

## Scenarios and impacts of Layer-2 ID collision

Scenarios:

In PC5 interface, the receiver identifies the RLC UM entity and PDCP entity by the combination of Source Layer-2 ID, Destination Layer-2 ID and LCID. If UE receives two one-to-one communication messages with the same Source Layer-2 ID, it can be regarded as Layer-2 ID collision. The layer-2 ID collision scenarios can be analyzed from the following two stages of ProSe one-to-one communication:

Stage 1: before one-to-one communication establishment complete.

Stage 2: after one-to-one communication establishment complete.

In stage 1, UE does not know which UE wants to perform one-to-one communication with it, hence UE should receive all the packets with its Destination Layer-2 ID. If an UE receives two one-to-one communication messages with the same Source Layer-2 ID, Layer-2 ID collision appears. For example, as shown in Figure 1, UE2 and UE3 may use the same Source Layer-2 ID to send the one-to-one communication establishment messages (include one-to-one communication request, setup, reject and complete messages) to UE1, hence Layer-2 ID collision happens and UE1 can’t distinguish UE2 from UE3 because they uses the same  the Source Layer-2 ID.

Figure 1: Layer-2 ID collision scenario in stage 1

In stage 2, UE has established one-to-one communication with other UE, it only needs to receive the packets which have the certain Source Layer-2 ID and Destination Layer-2 ID. Hence, a pair of UEs which have established one to one communication can be considered as a unit to distinguish. Only the scenario that more than two pairs of UEs with the same Source Layer-2 ID and Destination Layer-2 ID is considered to be Layer-2 ID collision. For example, as shown in Figure  2, UE1 is performing one to one communication with UE2, UE3 is performing one to one communication with UE4, if the Source Layer-2 ID of UE1 is same as UE4, the Source Layer-2 ID of UE2 is same as UE3, Layer-2 ID collision happens and hence UE1 and UE4 can’t distinguish the received packet is from UE2 or UE3.

Figure 2: Layer-2 ID collision scenario in stage 2

Impacts:

If Layer-2 ID collision happens, UE may receive packets from different UEs  with the same Source Layer-2 ID, it will combine the packets from these different UEs, some packets may be discarded in RLC entity, and the reassemble of RLC SDU may be confused.

## Probability of Layer-2 ID collision

Because the probabilities of Layer-2 ID collision for two stages may be different, thus the probability of Layer-2 ID collision will be analyzed individually for stage 1 and stage 2.

For stage 1, probability of Layer-2 ID collision is computed by

(1)

Where,

-	Prob is probability of Layer-2 ID collision.

-	N is total number of Layer-2 ID. N=224=16777216.

-	k is the number of UEs.

From the formula (1), we can get the probability of Layer-2 ID collision for stage 1 is:

Figure 3: Layer-2 ID collision probability for stage 1

Some reference value is shown in table 1:

Table 1:  typical values for Layer-2 ID collision probability for stage 1

| Number of UE | Probability of Layer-2 ID collision |
| --- | --- |
| 581 | 1% |
| 1312 | 5% |
| 1881 | 10% |
| 2737 | 20% |
| 3460 | 30% |
| 4140 | 40% |
| 4823 | 50% |
| 5545 | 60% |
| 6356 | 70% |
| 7349 | 80% |
| 8790 | 90% |

For stage 2, probability of Layer-2 ID collision is computed by

(2)

Where,

-	Prob is probability of Layer-2 ID collision,

-	N is total number of Layer-2 ID. N=224=16777216.

-	j is the number of UE pairs.

From the formula (2), we can get the probability of Layer-2 ID collision for stage 2 is:

Figure 4: Layer-2 ID collision probability for stage 2

According to the above figure, when j≤100000, Prob<1.8 x 10-5.

From the results above, we can conclude that probability of Layer-2 ID collision for stage 2 is low enough to ignore. However, the probability of Layer-2 ID collision for stage 1 can’t be ignored.

Observation: The Layer-2 ID collision for stage 2 can be ignored, but it cannot be ignored for stage 1.

To avoid the packets in different stages impact each other, the packet for one-to-one communication establishment message should be different from the other packets in MAC layer.

One simple solution is using a special LCID value, e.g. LCID=’00000’, to indicate the message contained in the MAC PDU is an one-to-one communication establishment message, just like Uu.

Proposal 1: Using a special LCID value to indicate the packet is a one-to-one communication message.

Proposal 2: Only the Layer-2 ID collision in one-to-one communication establishment procedure needs to be resolved.

## Solutions for Layer-2 ID collision in one-to-one communication stage 1

In ProSe one to one communication establishment procedure, L2 ID collision may be happened either between request UEs or response UEs.

Layer-2 ID collision between two one-to-one communication request UEs.

To resolve the Layer-2 ID collision between request UEs, a request UE ID, e.g. IMSI, IMEI, even random value, should be introduced in one-to-one communication request, setup and reject messages. For example, UE2 and UE3 send one-to-one communication request message to UE1 with the same source Layer-2 ID. UE1 knows that Layer-2 ID collision between UE2 and UE3 by the request UE ID in request message, it can reject one-to-one communication with one or both of UE2 and UE3, and indicate it to reselect a Layer-2 ID by set the request UE ID field in reject message to the same value contained in the request message.

Proposal 3: Introduce a request UE Identity in the content of ProSe one-to-one communication request, setup and reject messages to indentify the UE who sends the message.

Layer-2 ID collision between two one-to-one communication response UEs.

To resolve the Layer-2 ID collision between response UEs, a response UE ID, e.g. IMSI, IMEI, even random value, should be introduced in one-to-one communication setup and reject messages. For example, UE1 sends a one-to-one communication request message with the Destination Layer-2 ID is Layer-2 ID1 which is used by both UE2 and UE3. Both UE2 and UE3 send one-to-one communication setup messages to UE1. UE1 knows that Layer-2 ID collision between UE2 and UE3 by the response UE ID in setup message, it can reject one-to-one communication with one or both of UE2 and UE3, and indicate it to reselect a Layer-2 ID by set the response UE ID field in reject message to the same value contained in the setup message.

Proposal 4: Introduce a response UE Identity in the content of ProSe one-to-one communication setup and reject messages to indentify the response UE.

To avoid an UE establishes one to one communication with multi UEs which use the same Layer-2 ID, it should be permit to reject one to one communication establishment by both request UE and response UE.

Proposal 5: The request UE can also send ProSe one-to-one communication reject message.

# Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Using a special LCID value to indicate the packet is a one-to-one communication message.

Proposal 2: Only the Layer-2 ID collision in one-to-one communication establishment procedure needs to be resolved.

Proposal 3: Introduce a request UE Identity in the content of ProSe one-to-one communication request, setup and reject messages to indentify the UE who sends the message.

Proposal 4: Introduce a response UE Identity in the content of ProSe one-to-one communication setup and reject messages to indentify the response UE.

Proposal 5: The request UE can also send ProSe one-to-one communication reject message.

# References

  - 3GPP TR 23.713 V1.4.0