3GPP TSG-RAN WG2#91								R2-153745

24--28 August 2015, Beijing, China.

Agenda Item:	7.1.3

Source: 	Motorola Mobility

Title:  	Understanding DRX operation with LAA

Document for:	Discussion and Decision

# Introduction

DRX in LTE is designed to optimize power consumption at the UE while still allowing substantial flexibility in scheduling transmissions to the UE. In particular, the UE’s active time is “elastic” and extends based on the amount of data that the eNB schedules for the UE. If the eNB has very little or no data to transmit to the UE, the UE can revert to DRX sleep immediately after the on-duration. With this mechanism, the UE can be configured with a fairly short on-duration while still allowing for delivery of a large amount of data within the DRX cycle. This ensures that the active time is short most of the time and longer only when there is a substantial amount of buffered data for the UE at the eNB.

In this contribution we analyse the impact of LAA operation on LTE DRX. We provide results of a simulation study that compares power consumption and delay characteristics of LAA carrier operation with different levels of Wi-Fi traffic.

# Discussion

Before transmitting on an LAA carrier, an LAA-eNB has to perform an LBT procedure. Transmission by the LAA-eNB can proceed only if the LBT procedure is successful, i.e., there are no other transmissions on-going on the carrier. The LBT procedure can potentially delay transmissions. Here we analyse how the LBT procedure impacts packet delivery delays and DRX active times (and UE power consumption).

When compared to a licensed carrier, LBT operation for unlicensed carrier results in longer active times for the same amount of data. The following are the potential effects on DRX of LAA operation:

- UE active time can get extended compared to licensed carrier operation.

  - Due to maximum channel occupancy time requirements on the unlicensed carrier, even though the eNB has data waiting for the UE in its buffer, after transmission in a few subframes, it has to release the channel and re-contend for access, this results in the UE monitoring many non-scheduled subframes till the next TX burst from the eNB arrives or until expiry of the inactivity timer.

- Packet delivery delay can increase compared to licensed carrier operation.

  - Due to channel unavailability, packets on the LAA carriers can be delayed. In some DRX cycles, the on-duration timer may expire before the LAA eNB has a chance to schedule the UE. The LAA eNB has to then wait until the start of the next DRX cycle to try to schedule the UE. Such a case is shown in Figure 1. Of course, allowing longer active times reduces the delays by giving the eNB more opportunities to transmit via the LAA SCell, but it also increases UE power consumption as mentioned above.

  - Delayed packets can result in failures at upper layers such as TCP and significant performance problems. In order to avoid such failures, the network may need to deliver packets that have experienced significant delay (e.g., delay greater than some delay bound) on the LAA carrier, through a licensed carrier. For real time services such as VoIP, the network may choose to drop packets that have been delayed more than the delay bound. However, as the number of dropped packets increases, the quality of service and user experience degrades rapidly.

Figure 1

UE Power Consumption Considerations

Assuming a single DRX configuration across all configured carriers, an extension of active time on an LAA carrier implies an extension of active time on all carriers. This then can have a multiplicative effect on the power consumption. That is, the delays on the LAA SCell, can cause not only the LAA receiver but also the PCell receiver to be kept on for longer periods.

eNB Considerations

An eNB operating on a licensed carrier can configure DRX cycles that are staggered for different UEs. This enables the eNB to manage resources efficiently. For example, suppose that there are 2 UEs in a cell and the eNB wants to use a 20 ms DRX cycle for each. The DRX cycle of the second UE can be offset by 10 ms compared to the first UE. The eNB can then deliver up to n packets to each UE within its DRX cycle, while keeping the active time for each of the two UEs quite small. Offsetting the DRX cycles allows for the partitioning of activity of different devices into non-overlapping periods.

On an LAA carrier, suppose that we have one UE attached to the LAA eNB and one Wi-Fi device attached to a Wi-Fi AP. The device attached to Wi-Fi obviously cannot be placed on a DRX schedule. Consequently, there can be Wi-Fi transmissions when the LAA eNB wants to transmit to the UE in its DRX active time. This leads to an inefficient use of the carrier; for instance, the Wi-Fi transmissions may occupy the channel till the end of the active time for the UE and the UE may have entered DRX. To be able to effectively utilize the carrier, the UE would need to have long DRX active times. Thus, in general, the approach of partitioning activity of different devices into non-overlapping periods is not feasible for devices that use LAA and share the channel with Wi-Fi devices on an unlicensed carrier.

# Simulation Results

A simulation study was performed to understand the impact of LAA operation on the active times at the UE in the presence of different levels of Wi-Fi activity. An LAA eNB is assumed to schedule VoIP packets arriving periodically with an inter-arrival time of 20 ms. A DRX cycle of 20 ms is assumed. In order to capture the real-time characteristics of VoIP, a delay bound of 50 ms is assumed for the VoIP traffic. That is, if an LAA VoIP packet is waiting in the queue for 50 ms or longer, it is dropped. Explicit air interface transmissions not modelled for simplicity, i.e., every subframe is assumed to be received by the UE successfully.

LAA VoIP packets are 1 ms long. In order to access the carrier, the LAA eNB uses ETSI option B (for LBE) as described in ETSI EN301.893 with maximum size of extended CCA check of q=10.

Wi-Fi traffic (when present) is taken to be a FTP model 3 traffic pattern (for a packet size of 2 MB, each packet requiring 400 ms transmission time). The packets are segmented into 4 ms sub-packets for delivery. Wi-Fi APs contend with the LAA eNB for channel access using Wi-Fi channel access rules.

The first analysis shows the active times to meet the LAA VoIP delay bound for 98% of the packets for different Wi-Fi arrival rates. Wi-Fi arrival rates varying from 0 packets/sec (no Wi-Fi traffic) to 2 packets/sec are considered. Note that the no Wi-Fi traffic case is equivalent to operation on a licensed carrier. For each Wi-Fi arrival rate, the lowest required active time to meet the delay bound is shown (selected from active times obtained for different settings of on-duration and inactivity timer). As can be seen from these results, even for low Wi-Fi activity the active time increases significantly compared to the case where there is no Wi-Fi traffic. For example, for a Wi-Fi arrival rate of 0.2 packets/sec (~ 3.2 Mbps), the UE is active 25% of the time compared to only 10% of the time when there is no Wi-Fi traffic. For a Wi-Fi arrival rate of 2 packets/sec (~ 32 Mbps), which is still quite low, the UE is active 55% of the time.

Figure 2: Active Times for Different Wi-Fi traffic levels to achieve < 2% delayed LAA packets

The second analysis shows the percentage of LAA packets that are dropped (i.e., cannot be transmitted within the delay bound) as a function of the active time, for two selected Wi-Fi arrival rates. As can be seen, even modest increases in Wi-Fi traffic can show a significant increase in the percentage of dropped LAA packets. For example, suppose the Wi-Fi arrival rate is 0.2 packets/sec and the UE is configured with a DRX configuration that results in a mean active time of 25% of the DRX cycle (resulting in a percentage of dropped packets close to 2%). Then, if the Wi-Fi arrival rate increases to 1 packet/sec (~16 Mbps), the percentage of dropped LAA packets jumps to 11.6%.

Figure 3: Percentage of packets exceeding delay bound for different active times

# Conclusions

As expected, the LAA operation incurs penalties both in terms of active time (and UE power consumption as a consequence) and packet delivery delays, when compared to LTE operation on licensed carriers. The required active times to maintain an acceptable level of dropped LAA packets increases significantly as activity on the carrier increases. In particular, when compared to licensed carrier operation, the increases in active time can be quite large. Given that DRX active times are common across all carriers, such increases in active times due to LAA operation can result in significant power consumption impact for the UE. So, while periodic traffic such as VoIP can be operated with relatively low power consumption on a licensed carrier, the same type of traffic can require much higher power consumption if LAA is used.

The LAA packet delivery delays also manifest themselves as increased percentages of dropped packets. This can affect both real time traffic (such as VoIP) and non-real time traffic. As Wi-Fi packet arrival rates increase, the percentage of dropped packets can increase by large amounts for a given DRX configuration. Wi-Fi packet arrival rates are outside the control of the eNB and are unpredictable. This implies that, in order to consistently keep the dropped packet rates within acceptable limits, the network has to choose a DRX configuration that results in long active times. This again implies significantly higher power consumption at the UE.

For LAA, independent DRX configurations are viewed unfavourably in RAN2, due to the associated complexities. Given the results shown above we feel that it is necessary to investigate modifications to the DRX functionality.

Observation 1:  LAA operation on an unlicensed carrier can result in:

- Significantly higher power consumption at the UE when compared to licensed carrier operation, and

- Significantly more dropped packets when compared to licensed carrier operation.

Observation 2: The approach of partitioning activity of different devices into non-overlapping periods is not feasible for devices that use LAA and share the channel with Wi-Fi devices on an unlicensed carrier.

# References

- RP-141817, “Revised SID: Study on Licensed-Assisted Access to Unlicensed Spectrum”

- ETSI EN 301 893, “5 GHz high performance RLAN; Harmonized EN covering the essential requirements of article 3.2 of the R&TTE Directive”

# Annex – Simulation Assumptions

| Wifi_TXop | 4 ms |
| --- | --- |
| Wifi_CW_min | 15 |
| q (as described in [2]for LBE option B) | 10 |
| Max channel occupancy time for LAA before eCCA/Idle period | 4 ms |
| DRX cycle | 20 ms |
| Packet_arrival rate for Wi-Fi (FTP model 3) | Varying from 0 packets/sec to 2 packets/sec |
| Packet arrival rate for LAA (VoIP) | 50 packets/second periodic |
| Simulation-time per run | 800 sec |

There are some differences between licensed carrier operation and unlicensed carrier operation even in the absence of any Wi-Fi traffic, due to the LBT operation; however, the impact of these differences on active times and delays are assumed to be negligible.