3GPP TSG RAN WG2 Meeting #91	DRAFT	R2-153922

Beijing, China, 24th - 28th August 2015

Agenda Item:	5.2

Source: 		Sony

Title:			Text proposals for IMU sensor for indoor positioning

Document for:	Discussion and Decision

# Introduction

The Rel-13 Study Item on “Study on Indoor Positioning Enhancements for UTRA and LTE” has objective to study potential 3GPP positioning enhancements in indoor and other challenging environments [1].

RAT-independent positioning techniques have also been discussed. In RAN1 #80 meeting in Athens, Barometric sensor information is relevant for this study item but will not impact RAN1 specifications [2]. In RAN1 #80b meeting in Belgrade, existing WiFi and/or Bluetooth can be used for positioning and should be introduced in the TR as one of the potential enhancement techniques for RAT-independent indoor positioning [3]. At the RAN1#81 meeting in Fukuoka a first part for text proposal related to Inertial measurement unit (IMU) sensor based positioning was endorsed for [8], in [7].

Inertial measurement unit (IMU) sensor can be used as potential indoor positioning enhancement techniques. An IMU can measure and report velocity, orientation, and gravitational forces of a device, using a combination of accelerometers and gyroscopes, sometimes also magnetometers. IMU can also be found in many existing devices/UEs in the market.

In this contribution, we provide our view on IMU sensor for indoor positioning to the RAN2 group, and further text proposal that is currently FFS in [8]

# Discussion

The latest FCC requirements mandate barometer-equipped handsets or user equipment (UE) should be available within three years [4]. WiFi and Bluetooth are already available in almost all type of handsets/UEs. Moreover, IMU sensor can also be found in many handsets. It has been reported that those sensors can be used to assist indoor positioning [5].

Observations 1: Most of the existing UE in the market are already equipped with WiFi, Bluetooth, and IMU sensor (e.g. accelerometers, gyroscopes, magnetometers).

Each of these items can be used for UE positioning purposes. However, it will bring more advantages if all or part of those items can jointly work together in order to obtain accurate UE positioning measurement and precise UE positioning estimation.

Proposal 1: Consider IMU sensor (e.g. accelerometers, gyroscopes, magnetometers) in a UE for positioning purposes. This will not impact RAN1 specifications. The detailed aspects that may affect RAN2 specification are given in the proposed text proposal below.

# Conclusion

In this contribution, we discussed our views on UE based RAT-independent positioning techniques. The observations and proposals based on the discussion are given as follow:

Observations 1: Most of the existing UE in the market are already equipped with WiFi, Bluetooth, and sensors (e.g. accelerometers, gyroscopes, magnetometers).

Proposal 1: Consider IMU sensor data (accelerometers, gyroscopes, magnetometers) in a UE for positioning purposes. This will not impact RAN1 specifications. The detailed aspects that may affect RAN2 specification are given in the proposed text proposal below.

# References

RP-141102, “Revised SID: Study on Indoor Positioning Enhancements for UTRA and LTE,”

Chairman’s Notes 3GPP RAN1 #80 Meeting

Chairman’s Notes 3GPP RAN1 #80bis Meeting

FCC Fourth report and order, https://apps.fcc.gov/edocs_public/attachmatch/FCC-15-9A1.pdf, Feb 2015.

Lyu-Han Chen, et.al, "Intelligent Fusion of Wi-Fi and Inertial Sensor-Based Positioning Systems for Indoor Pedestrian Navigation," Sensors Journal, IEEE , vol.14, no.11, pp.4034,4042, Nov. 2014

R1-153090, UE based positioning techniques.

R1-153584 Text proposals for IMU sensor for indoor positioning	SONY

TR 37.837 0.4.0, Study on Indoor Positioning Enhancements for UTRA and LTE

# Text Proposal for TR 37.857

<Start of first changed section>

## 2 References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- RP-141102, “Revised SID: Study on Indoor Positioning Enhancements for UTRA and LTE”

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- 3GPP TS 22.071: “Location Services (LCS); Service description; Stage 1”.

- 3GPP TR 36.873: “Study on 3D channel model for LTE”.

- 3GPP TR 36.819: “Coordinated multi-point operation for LTE physical layer aspects”.

- 3GPP TR 36.872: “Small cell enhancements for E-UTRA and E-UTRAN - Physical layer aspects”.

- 3GPP TR 36.814: “Evolved Universal Terrestrial Radio Access (E-UTRA); Further advancements for E-UTRA physical layer aspects”.

- 3GPP TS 36.133: “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”.

- 3GPP TS 36.214: “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer; Measurements”.

- R1-103410: “Chairman’s note on simulation assumptions for UTDOA”.

- R1-152516, “Updated Baseline Evaluation Results of OTDOA,” Ericsson.

- R1-152517, “Revised Simulation Results on Impact from Imperfect Network synchronization,” Ericsson.

- R1-153218, “Further Evaluation for OTDOA and hybrid for indoor positioning without synchronization error,” Huawei, HiSilicon.

- R1-153219, “Further Evaluation for OTDOA and hybrid for indoor positioning with synchronization error,” Huawei, HiSilicon.

- R1-152634, “Summary of System Level Analysis for Indoor User Positioning,” Intel Corporation.

- R1-153018, “Baseline evaluation results for indoor positioning,” LG Electronics.

- R1-152799, “Indoor Positioning Simulation Results,” Qualcomm.

- R1-152800, “Impact of Network Synchronization Error on OTDOA Baseline Performance,” Qualcomm.

- 3GPP TS 36.355, "LTE Positioning Protocol (LPP)".

- 3GPP TS 36.455, "LTE Positioning Protocol A (LPPa)".

- R1-153423, “Initial TBS Simulation Results,” Qualcomm.

- R1-153381, “MBS Initial Simulation Results,” NextNav.

- 3GPP TS 36.211, "Physical channels and modulation".

- R1-153075 “MBS description and configuration parameters,” NextNav.

- 3GPP TS 36.133 “Requirements for support of radio resource management”.

- 3GPP TS 23.271 “Functional stage 2 description of Location Services (LCS)”.

- 3GPP TS 36.305 “Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN”.

- 3GPP TS 25.331, "Radio Resource Control (RRC); Protocol Specification".

- 3GPP TS 25.453, "UTRAN Iupc interface Positioning Calculation Application Part (PCAP) signalling".

- 3GPP TS 25.413, “UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling”

- 3GPP TS 25.305, “Stage 2 functional specification of User Equipment (UE) positioning in UTRAN”.

- OMA-TS-LPPe-V1_0-20150414-C, "OMA LPP Extensions (LPPe) 1.0", 14 April 2015.

- IEEE P802.11-REVmc/D3.3, October 2014.

- 3GPP TS 36.331, "Radio Resource Control (RRC); Protocol specification".

- Lyu-Han Chen, et.al, "Intelligent Fusion of Wi-Fi and Inertial Sensor-Based Positioning Systems for Indoor Pedestrian Navigation," Sensors Journal, IEEE , vol.14, no.11, pp.4034,4042, Nov. 2014

<End of changed section>

<Start of next changed section>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [2] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [2].

A-GNSS	Assisted-Global Navigation Satellite System

CDF	Cumulative Distribution Function

CRS	Cell-specific Reference Signal

D2D	Device-to-Device

E-CID	Enhanced Cell Identification

FDD	Frequency Division Duplex

IE	Information Element

IMU	Intertial Measurements Unit

LCS		LoCation Services

LCS-AP	LCS Application Part

LPP	LTE Positioning Protocol

OTDOA	Observed Time Difference of Arrival

PCAP	Positioning Calculation Application Part

PCI	Physical Cell Identity

PDSCH	Physical Downlink Shared Channel

PRS	Positioning Reference Signals

RANAP	Radio Access Network Application Part

RFPM	Radio Frequency Pattern Matching

RRC	Radio Resource Control

RSTD	Reference Signal Time Difference

SCE	Small Cell Enhancements

SRS	Sounding Reference Signal

UTDOA	Uplink Time Difference of Arrival

TBS	Terrestrial Beacon System

UL-RTOA	Uplink Relative Time of Arrival

<End of changed section>

<Start of next changed section>

7.2.4 IMU sensor based positioning

Inertial measurement unit (IMU) sensor can be used as potential indoor positioning enhancement techniques. An IMU can measure and report the velocity, orientation, and gravitational forces of a device, using a combination of accelerometers and gyroscopes, sometimes also magnetometers.

IMU can be found in many existing devices/UEs in the market. It has been widely reported that IMU sensor information can be used for positioning purposes, in particular for indoor positioning.

IMU sensor information is relevant for this study item, but will not impact RAN1 specifications.

RAN2 identified the following Inertial measurement unit (IMU) based positioning options which may operate in the following mode:

- Standalone/autonomous:
The UE performs IMU related sensor measurements and computation without network assistance.

The only protocol impact would be for the UE in the measurement/location report message to indicate that IMU as method has been used to enhance the positioning calculation.

For specification impact, the following specifications would be affected, for E-UTRA, 36.355 and 36.305, and for UTRA, 25.331, 25.305, 25.453 and 25.413. Main chapters affected would be, references, abbreviations, and Information Element (IE) chapters related to positioning methods to be used in measurement/location report message, and optionally in capability signalling.

<End of last changed section>