# 10	Message and information element functional definition and content

## 10.1	General

The function of each Radio Resource Control message together with message contents in the form of a list of information elements is defined in subclause 10.2.

Functional definitions of the information elements are then described in subclause 10.3.

Information elements are marked as either MP - Mandatory present, MD - Mandatory with default value, OP - Optional, CV - Conditional on value or CH - Conditional on history (see Table 10.1 with information extracted from [14]).

Table 10.1: Meaning of abbreviations used in RRC messages and information elements

| Abbreviation | Meaning |
| --- | --- |
| MP | Mandatory present A value for that information is always needed, and no information is provided about a particular default value. If ever the transfer syntax allows absence (e.g., due to extension), then absence leads to an error diagnosis. |
| MD | Mandatory with default value A value for that information is always needed, and a particular default value is mentioned (in the 'Semantical information' column). This opens the possibility for the transfer syntax to use absence or a special pattern to encode the default value. |
| CV | Conditional on value The need for a value for that information depends on the value of some other IE or IEs, and/or on the message flow (e.g., channel, SAP). The need is specified by means of a condition, the result of which may be that the information is mandatory present, mandatory with default value, not needed or optional. If one of the results of the condition is that the information is mandatory present, the transfer syntax must allow for the presence of the information. If in this case the information is absent an error is diagnosed. If one of the results of the condition is that the information is mandatory with default value, and a particular default value is mentioned (in the 'Semantical information' column), the transfer syntax may use absence or a special pattern to encode the default value. If one of the results of the condition is that the information is not needed, the transfer syntax must allow encoding the absence. If in this case the information is present, it will be ignored. In specific cases however, an error may be diagnosed instead. If one of the results of the condition is that the information is optional, the transfer syntax must allow for the presence of the information. In this case, neither absence nor presence of the information leads to an error diagnosis. |
| CH | Conditional on history The need for a value for that information depends on information obtained in the past (e.g., from messages received in the past from the peer). The need is specified by means of a condition, the result of which may be that the information is mandatory present, mandatory with default value, not needed or optional. The handling of the conditions is the same as described for CV. |
| OP | Optional The presence or absence is significant and modifies the behaviour of the receiver. However whether the information is present or not does not lead to an error diagnosis. |

### 10.1.1	Protocol extensions

RRC messages may be extended in future versions of this protocol, either by adding values for choices, enumerated and size constrained types or by adding information elements. An important aspect concerns the behaviour of a UE, conforming to this revision of the standard, upon receiving a not comprehended future extension. The details of this error handling behaviour are provided in clause 9.

NOTE 1:	By avoiding the need for partial decoding (skipping uncomprehended IEs to continue decoding the remainder of the message), the RRC protocol extension mechanism also avoids the overhead of length determinants for extensions. "Variable length extension containers" (i.e. non critical extension containers that have their abstract syntax defined using the ASN.1 type "BIT STRING") have been defined to support the introduction of extensions to a release after the subsequent release is frozen (and UEs based on that subsequent release may appear). For this container a length determinant is used, which facilitates partial decoding of the container as well as the decoding of the extensions included after the container.

Two kinds of protocol extensions are distinguished: non-critical and critical extensions. In general, a receiver shall process a message including not comprehended non-critical extensions as if the extensions were absent. However, a receiver shall entirely reject a message including not comprehended critical extensions (there is no partial rejection) and notify the sender, as specified in clause 9.

The general mechanism for adding critical extensions is by defining a new version of the message, which is indicated at the beginning of the message.

The UE shall always comprehend the complete transfer syntax specified for the protocol version it supports; if the UE comprehends the transfer syntax defined within protocol version A for message 1, it shall also comprehend the transfer syntax defined within protocol version A for message 2.

The following table shows for which messages only non-critical extensions may be added, for which messages both critical and non-critical extensions may be added and for which messages neither critical nor non-critical extensions may be added.

NOTE 2:	Critical extensions can only be added to certain downlink messages.

| Extensions | Message |
| --- | --- |
| Critical and non-critical extensions | ACTIVE SET UPDATE 10.2.1 ASSISTANCE DATA DELIVERY 10.2.4 CELL CHANGE ORDER FROM UTRAN 10.2.5 CELL UPDATE CONFIRM 10.2.8 COUNTER CHECK 10.2.9 DOWNLINK DIRECT TRANSFER 10.2.11 HANDOVER TO UTRAN COMMAND 10.2.16a HANDOVER FROM UTRAN COMMAND 10.2.15 LOGGING MEASUREMENT CONFIGURATION 10.2.16da MEASUREMENT CONTROL 10.2.17 PHYSICAL CHANNEL RECONFIGURATION 10.2.22 PHYSICAL SHARED CHANNEL ALLOCATION 10.2.25 RADIO BEARER RECONFIGURATION 10.2.27 RADIO BEARER RELEASE 10.2.30 RADIO BEARER SETUP 10.2.33 RRC CONNECTION REJECT 10.2.36 RRC CONNECTION RELEASE 10.2.37 RRC CONNECTION SETUP 10.2.40 SECURITY MODE COMMAND 10.2.43 SIGNALLING CONNECTION RELEASE 10.2.46 TRANSPORT CHANNEL RECONFIGURATION 10.2.50 UE CAPABILITY ENQUIRY 10.2.55 UE CAPABILITY INFORMATION CONFIRM 10.2.57 UE INFORMATION REQUEST 10.2.57a UPLINK PHYSICAL CHANNEL CONTROL 10.2.59 URA UPDATE CONFIRM 10.2.61 UTRAN MOBILITY INFORMATION 10.2.62 |
| Non-critical extensions only | ACTIVE SET UPDATE COMPLETE 10.2.2 ACTIVE SET UPDATE FAILURE 10.2.3 CELL CHANGE ORDER FROM UTRAN FAILURE 10.2.6 CELL UPDATE 10.2.7 COUNTER CHECK RESPONSE 10.2.10 ETWS PRIMARY NOTIFICATION WITH SECURITY 10.2.12a HANDOVER TO UTRAN COMPLETE 10.2.16b INITIAL DIRECT TRANSFER 10.2.16c HANDOVER FROM UTRAN FAILURE 10.2.16 MBMS Access Information 10.2.16e MBMS Common p-t-m rb Information 10.2.16f MBMS Current Cell p-t-m rb Information 10.2.16g MBMS General Information 10.2.16h MBMS Modification request 10.2.16i MBMS Modified services Information 10.2.16j MBMS Neighbouring Cell p-t-m rb Information 10.2.16k MBMS Scheduling Information 10.2.16L MBMS Unmodified services Information 10.2.16m MEASUREMENT CONTROL FAILURE 10.2.18 MEASUREMENT REPORT 10.2.19 PAGING TYPE 1 10.2.20 PAGING TYPE 2 10.2.21 PHYSICAL CHANNEL RECONFIGURATION COMPLETE 10.2.23 PHYSICAL CHANNEL RECONFIGURATION FAILURE 10.2.24 PUSCH CAPACITY REQUEST 10.2.26 RADIO BEARER RECONFIGURATION COMPLETE 10.2.28 RADIO BEARER RECONFIGURATION FAILURE 10.2.29 RADIO BEARER RELEASE COMPLETE 10.2.31 RADIO BEARER RELEASE FAILURE 10.2.32 RADIO BEARER SETUP COMPLETE 10.2.34 RADIO BEARER SETUP FAILURE 10.2.35 RRC CONNECTION RELEASE COMPLETE 10.2.38 RRC CONNECTION REQUEST 10.2.39 RRC CONNECTION SETUP COMPLETE 10.2.41 RRC STATUS 10.2.42 SECURITY MODE COMPLETE 10.2.44 SECURITY MODE FAILURE 10.2.45 SIGNALLING CONNECTION RELEASE INDICATION 10.2.47 Master Information Block 10.2.48.8.1 System Information Block type 1 to System Information Block type 25 10.2.48.8.4 to 10.2.48.8.28 SYSTEM INFORMATION CHANGE INDICATION 10.2.49 TRANSPORT CHANNEL RECONFIGURATION COMPLETE 10.2.51 TRANSPORT CHANNEL RECONFIGURATION FAILURE 10.2.52 TRANSPORT FORMAT COMBINATION CONTROL 10.2.53 TRANSPORT FORMAT COMBINATION CONTROL FAILURE 10.2.54 UE CAPABILITY INFORMATION 10.2.56 UE INFORMATION RESPONSE 10.2.57b UPLINK DIRECT TRANSFER 10.2.58 URA UPDATE 10.2.60 UTRAN MOBILITY INFORMATION CONFIRM 10.2.63 UTRAN MOBILITY INFORMATION FAILURE 10.2.64 |
| No extensions | CELL UPDATE FDD 10.2.7a SYSTEM INFORMATION 10.2.48 SYSTEM INFORMATION 2 10.2.48b First Segment 10.2.48.1 First Segment 2 10.2.48.1a Subsequent or last Segment 10.2.48.3 Subsequent Segment 2 10.2.48.3a or last Segment 2 10.2.48.4a Complete SIB 10.2.48.6 Complete SIB 2 10.2.48.6a |

NOTE 3:	For the SYSTEM INFORMATION message protocol extensions are only possible at the level of system information blocks.

#### 10.1.1.1	Non-critical extensions

##### 10.1.1.1.1	Extension of an information element with additional values or choices

In future versions of this protocol, non-critical values may be added to choices, enumerated and size constrained types.

For choices, enumerated and size constrained types it is possible to indicate how many non-critical spare values need to be reserved for future extension. In this case, the tabular format should indicate the number of spare values that are needed. The value range defined in ASN.1 for the extensible IE should include the number of spares that are needed, since a value outside the range defined for this IE will result in a general ASN.1 violation error.

For downlink messages, spare values may be defined for non-critical information elements for which the need is specified to be MD or OP (or CV case leading to MD or OP). In this case, a receiver not comprehending the received spare value shall consider the information element to have the default value or consider it to be absent respectively.

For uplink messages spare values may be defined for all information elements, including those for which the need is specified to be MP (or CV case leading to MP).

In all cases at most one spare should be defined for choices. In this case, information elements applicable to the spare choices shall be added to the end of the message.

##### 10.1.1.1.2	Extension of a message with additional information elements

In future versions of this protocol, non-critical information elements may be added to RRC messages. These additional information elements shall be normally appended at the end of the message; the transfer syntax specified in this revision of the standard facilitates this. A receiver conformant to this revision of the standard shall accept such extension, and proceed as if it was not included.
A transmitter conformant to this version of the standard shall not include an extension reserved for introducing non critical extensions in later versions of the standard; i.e. the corresponding parameter defined in the ASN.1 shall be absent.

NOTE:	If an extension, reserved for future non-critical extensions, is included (even if it is empty), this may result in transfer syntax errors when received by an implementation conforming to a later version of the standard.

Extensions to a release that are introduced after the subsequent release is frozen may however be inserted prior to the end of the message. To facilitate this, "variable length extension containers" have been introduced in most messages.

#### 10.1.1.2	Critical extensions

##### 10.1.1.2.1	Extension of an information element with additional values or choices

In versions of this protocol, choices, enumerated and size constrained types may be extended with critical values. For extension with critical values the general critical extension mechanism is used, i.e. for this no spare values are reserved since backward compatibility is not required.

##### 10.1.1.2.2	Extension of a message with additional information elements

In future versions of this protocol, RRC messages may be extended with new information elements. Since messages including critical extensions are rejected by receivers not comprehending them, these messages may be modified completely, e.g. IEs may be inserted at any place and IEs may be removed or redefined.

## 10.2	Radio Resource Control messages

### 10.2.1	ACTIVE SET UPDATE

NOTE:	Only for FDD.

This message is used by UTRAN to add, replace or delete radio links in the active set of the UE.

RLC-SAP: AM or UM

Logical channel: DCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Activation time | MD |  | Activation time 10.3.3.1 | Default value is "now". |  |
| Dynamic activation time | OP |  | Dynamic activation time 10.3.3.1a |  | REL-13 |
| New U-RNTI | OP |  | U-RNTI 10.3.3.47 |  |  |
| New H-RNTI | OP |  | H-RNTI 10.3.3.14a |  | REL-6 |
| New Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-6 |
| New Secondary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-6 |
| CN information elements |  |  |  |  |  |
| CN Information info | OP |  | CN Information info 10.3.1.3 |  |  |
| Phy CH information elements |  |  |  |  |  |
| DTX-DRX timing information | OP |  | DTX-DRX timing information 10.3.6.34b |  | REL-7 |
| DTX-DRX Information | OP |  | DTX-DRX Information 10.3.6.34a |  | REL-7 |
| HS-SCCH less Information | OP |  | HS-SCCH less Information 10.3.6.36ab |  | REL-7 |
| MIMO parameters | OP |  | MIMO parameters 10.3.6.41a |  | REL-7 |
| MIMO mode with four transmit antennas parameters | OP |  | MIMO mode with four transmit antennas parameters 10.3.6.142 |  | REL-11 |
| DCH Enhancements info FDD | OP |  | DCH Enhancements info FDD 10.3.6.149 |  | REL-12 |
| Uplink radio resources |  |  |  |  |  |
| Maximum allowed UL TX power | MD |  | Maximum allowed UL TX power 10.3.6.39 | Default value is the existing "maximum UL TX power. |  |
| Uplink secondary cell info FDD | OP |  | Uplink secondary cell info FDD 10.3.6.115 | FDD only | REL-9 |
| E-DCH reconfiguration information on secondary UL frequency | OP |  | E-DCH reconfiguration information on secondary UL frequency 10.3.6.121 | FDD only | REL-9 |
| Uplink CLTD info FDD | OP |  | Uplink CLTD info FDD 10.3.6.125 | FDD only | REL-11 |
| F-TPICH reconfiguration info | OP |  | F-TPICH reconfiguration info 10.3.6.128 | FDD only | REL-11 |  |
| Uplink OLTD info FDD | OP |  | Uplink OLTD info FDD 10.3.6.126 | FDD only | REL-11 |
| Downlink radio resources |  |  |  |  |  |
| Radio link addition information | OP | 1 to <maxRL-1> |  | Radio link addition information required for each RL to add |  |
| >Radio link addition information | MP |  | Radio link addition information 10.3.6.68 |  |  |
| Radio link addition information on secondary UL frequency | OP |  | Radio link addition information on secondary UL frequency 10.3.6.119 | FDD only | REL-9 |
| Serving Cell Change Parameters | CV-TargetCellPreconf |  | Serving Cell Change Parameters 10.3.6.74b |  | REL-8 |
| Radio link removal information | OP | 1 to <maxRL> |  | Radio link removal information required for each RL to remove |  |
| >Radio link removal information | MP |  | Radio link removal information 10.3.6.69 |  |  |
| Radio link removal information on secondary UL frequency | OP |  | Radio link removal information on secondary UL frequency 10.3.6.120 | FDD only | REL-9 |
| TX Diversity Mode | MD |  | TX Diversity Mode 10.3.6.86 | Default value is the TX diversity mode currently used in all or part of the active set. |  |
| DPC Mode | OP |  | Enumerated (Single TPC, TPC triplet in soft) | "Single TPC" is DPC_Mode=0 and "TPC triplet in soft" is DPC_mode=1 in [29]. | REL-5 |
| Serving HS-DSCH cell information | OP |  | Serving HS-DSCH cell information 10.3.6.74a |  | REL-6 |
| E-DCH reconfiguration information | OP |  | E-DCH reconfiguration information 10.3.6.69a |  | REL-6 |
| UL 16QAM configuration | OP |  | UL 16QAM configuration 10.3.6.86o |  | REL-7 |
| UL 64QAM configuration | OP |  | UL 64QAM configuration 10.3.6.86b |  | REL-11 |
| Uplink MIMO info FDD | OP |  | Uplink MIMO info FDD 10.3.6.145 | FDD only | REL-11 |
| E-DCH reconfiguration information same serving cell | OP |  | E-DCH reconfiguration information same serving cell 10.3.6.69b | This IE is not present if the serving E-DCH cell is changed with this message. | REL-7 |
| E-TFC Boost Info | OP |  | E-TFC Boost Info 10.3.6.106 | Absence of this IE means that boosting of E-DPCCH is disabled | REL-7 |
| E-DPDCH power interpolation | OP |  | Boolean | TRUE means E-DPDCH power interpolation formula is used, FALSE means E-DPDCH power extrapolation formula is used for the computation of the gain factor ed according to [29]. Absence of this IE means that E-DPDCH power extrapolation formula is used for the computation of the gain factor ed according to [29]. | REL-7 |
| Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-8 |
| Additional downlink secondary cell info list FDD | OP | 2 |  |  | REL-10 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-10 |
| Additional downlink secondary cell info list FDD 2 | OP | 4 |  |  | REL-11 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-11 |
| DPCCH2 info FDD | OP |  | DPCCH2 info FDD 10.3.6.148 | FDD only | REL-12 |

| Condition | Explanation |
| --- | --- |
| TargetCellPreconf | This IE is mandatory present if Radio Links are added and at least one added Radio Link contains the IE Target cell preconfiguration. |

### 10.2.2	ACTIVE SET UPDATE COMPLETE

NOTE:	For FDD only.

This message is sent by UE when active set update has been completed.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |

### 10.2.3	ACTIVE SET UPDATE FAILURE

NOTE:	Only for FDD.

This message is sent by UE if the update of the active set has failed, e.g. because the radio link is not a part of the active set.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| Failure cause | MP |  | Failure cause and error information 10.3.3.14 |  |

### 10.2.4	ASSISTANCE DATA DELIVERY

This message is sent by UTRAN to convey UE positioning assistance data to the UE.

RLC-SAP: AM

Logical channel: DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Measurement Information elements |  |  |  |  |  |
| UE positioning OTDOA assistance data for UE-based | OP |  | UE positioning OTDOA assistance data for UE-based 10.3.7.103a |  |  |
| UE positioning GPS assistance data | OP |  | UE positioning GPS assistance data 10.3.7.90 |  |  |
| UE positioning GANSS assistance data | OP |  | UE positioning GANSS assistance data 10.3.7.90b |  | REL-7 |

### 10.2.5	CELL CHANGE ORDER FROM UTRAN

This message is used to order a cell change from UTRA to another radio access technology, e.g., GSM.

RLC-SAP: AM

Logical channel: DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Activation time | MD |  | Activation time 10.3.3.1 | Default value is "now" |  |
| RB Information elements |  |  |  |  |  |
| RAB information list | OP | 1 to <maxRABsetup> |  | This IE should not be included in this version of the protocol. |  |
| >RAB info | MP |  | RAB info 10.3.4.8 |  |  |
| Other information elements |  |  |  |  |  |
| Target cell description | MP |  |  |  |  |
| >CHOICE Radio Access Technology | MP |  |  | Two spare values are needed. |  |
| >>GSM |  |  |  |  |  |
| >>>BSIC | MP |  | BSIC 10.3.8.2 |  |  |
| >>>Band Indicator | MP |  | Enumerated (DCS 1800 band used, PCS 1900 band used) | Indicates how to interpret the BCCH ARFCN |  |
| >>>BCCH ARFCN | MP |  | Integer (0..1023) | [45] |  |
| >>>NC mode | OP |  | Bit string(3) | Includes bits b1-b3 of the NC mode IE specified in [43]. b1 is the least significant bit. NOTE: The Bit string should be extended to 4 bits in a later version of the message. |  |
| >>>CHOICE GERAN System Info type | OP |  |  |  | REL-5 |
| >>>>SI |  |  | GERAN system information 10.3.8.4f | See [44] | REL-5 |
| >>>>PSI |  |  | GERAN system information 10.3.8.4f | See [44] | REL-5 |
| >>IS-2000 |  |  |  |  |  |

### 10.2.6	CELL CHANGE ORDER FROM UTRAN FAILURE

This message is sent on the RRC connection used before the Cell change order from UTRAN was executed. The message indicates that the UE has failed to seize the new channel in the other radio access technology.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| Other information elements |  |  |  |  |
| Inter-RAT change failure | MP |  | Inter-RAT change failure 10.3.8.5 |  |

### 10.2.7	CELL UPDATE

This message is used by the UE to initiate a cell update procedure.

RLC-SAP: TM

Logical channel: CCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| U-RNTI | MP |  | U-RNTI 10.3.3.47 |  |  |
| RRC transaction identifier | CV-Failure |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| START list | MP | 1 to <maxCNdomains> |  | START [40] values for all CN domains. |  |
| >CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >START | MP |  | START 10.3.3.38 | START value to be used in this CN domain. |  |
| AM_RLC error indication(RB2, RB3 or RB4) | MP |  | Boolean | TRUE indicates AM_RLC unrecoverable error [16] occurred on RB2, RB3 or RB4 in the UE |  |
| AM_RLC error indication(RB>4) | MP |  | Boolean | TRUE indicates AM_RLC unrecoverable error [16] occurred on RB>4 in the UE |  |
| Cell update cause | MP |  | Cell update cause 10.3.3.3 |  |  |
| Traffic volume indicator | OP |  | Enumerated (TRUE) | This IE shall be set to TRUE when the criteria for event based traffic volume measurement reporting is fulfilled. Absence of this element means not fulfilled. | REL-6 |
| Failure cause | OP |  | Failure cause and error information 10.3.3.14 |  |  |
| RB timer indicator | MP |  | RB timer indicator 10.3.3.28 |  |  |
| Establishment cause | OP |  | Establishment cause 10.3.3.11 |  | REL-5 |
| CS Call Type | CV-ConversationalCS |  | Enumerated (speech, video, other) | One spare value is needed | REL-7 |
| HS-PDSCH in CELL_FACH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-PDSCH reception in CELL_FACH state. Note 1. | REL-7 |
| Support of common E-DCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support E-DCH enhanced random access in CELL_FACH state and Idle mode. Note 1. | REL-8 |
| Support of HS-DSCH DRX operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-DSCH DRX operation in CELL_FACH state. | REL-8 |
| Support of MAC-i/is | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support MAC-i/is operation. | REL-8 |
| Support of SPS operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support SPS operation. | REL-8 |
| Support of control channel DRX operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support control channel DRX operation. | REL-8 |
| UE Mobility State Indicator | MD |  | Enumerated (High-mobilityDetected) | Absence of this IE implies that, according to [4] either the High mobility state is not applicable or it has not been detected by the UE. | REL-7 |
| Capability change indicator | OP |  | Enumerated (TRUE) | TRUE indicates that the UE capability has changed compared to the value stored in the variable UE_CAPABILITY_TRANSFERRED. | REL-7 |
| Reconfiguration Status Indicator | OP |  | Enumerated (TRUE) | TRUE indicates a reconfiguration procedure is ongoing within the UE or a response message has been submitted to RLC and the UE is waiting for the layer 2 ACK. | REL-6 |
| MBMS Selected Services | OP |  | MBMS Selected Services Short 10.3.9a.7d |  | REL-6 |
| Support for Two DRX schemes in URA_PCH and CELL_PCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Two DRX schemes in URA_PCH and CELL_PCH | REL-7 |
| Security Revert Status Indicator | OP |  | Enumerated (Reverted Back, Normal Operation) | Reverted Back indicates this UE has reverted back to the old security configuration by aborting the ongoing security mode command. Normal Operation indicates this UE has not reverted back to the old security configuration by aborting an ongoing security mode command. The absence of this IE indicates that the UE does not support reporting of security revert status. | REL-10 |
| Measurement information elements |  |  |  |  |  |
| Measured results on RACH | OP |  | Measured results on RACH 10.3.7.45 |  |  |
| Other Information elements |  |  |  |  | REL-10 |
| Logged Meas Available | OP |  | Enumerated (TRUE) | Indicates the UE has logged measurements to report to the network | REL-10 |
| ANR Logging Results Available | OP |  | Enumerated (TRUE) | True indicates the UE has ANR logging results to report to the Network. | REL-10 |

NOTE 1:	For 1.28 Mcps TDD, UE supporting HS-PDSCH in CELL_FACH always supports E-DCH enhanced random access in CELL_FACH state and Idle mode, and vice versa. And UE supporting HS-PDSCH reception in CELL_FACH state always supports HS-DSCH reception in CELL_PCH and URA_PCH state and CELL_FACH HS-DSCH DRX operation.

| Condition | Explanation |
| --- | --- |
| Failure | This IE is mandatory present if the IE "Failure cause" is present and not needed otherwise. |
| ConversationalCS | This IE is mandatory present if the IE "Establishment cause" has the value "Originating Conversational Call" or "Emergency Call" and a CS call is being initiated. Otherwise it is not needed. |

### 10.2.7a	CELL UPDATE FDD

NOTE:	Only for FDD.

This message is used by the UE to initiate a cell update procedure.

RLC-SAP: TM

Logical channel: CCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| U-RNTI | MP |  | U-RNTI 10.3.3.47 |  |  |
| START list | MP | 1 to <maxCNdomains> |  | START [40] values for all CN domains. |  |
| >CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >START | MP |  | START 10.3.3.38 | START value to be used in this CN domain. |  |
| AM_RLC error indication(RB2, RB3 or RB4) | MP |  | Boolean | TRUE indicates AM_RLC unrecoverable error [16] occurred on RB2, RB3 or RB4 in the UE |  |
| AM_RLC error indication(RB>4) | MP |  | Boolean | TRUE indicates AM_RLC unrecoverable error [16] occurred on RB>4 in the UE |  |
| Cell update cause | MP |  | Cell update cause 10.3.3.3 |  |  |
| Failure cause | OP |  | Failure cause and error information 10.3.3.14 |  |  |
| RRC transaction identifier | CV-Failure |  | RRC transaction identifier 10.3.3.36 |  |  |
| RB timer indicator | MP |  | RB timer indicator 10.3.3.28 |  |  |
| Establishment cause | OP |  | Establishment cause 10.3.3.11 |  |  |
| Traffic volume indicator | OP |  | Enumerated (TRUE) | This IE shall be set to TRUE when the criteria for event based traffic volume measurement reporting is fulfilled. Absence of this element means not fulfilled. |  |
| Reconfiguration Status Indicator | OP |  | Enumerated (TRUE) | TRUE indicates a reconfiguration procedure is ongoing within the UE or a response message has been submitted to RLC and the UE is waiting for the layer 2 ACK. |  |
| CS Call Type | CV-ConversationalCS |  | Enumerated (speech, video, other) | One spare value is needed |  |
| HS-PDSCH in CELL_FACH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-PDSCH reception in CELL_FACH state. |  |
| UE Mobility State Indicator | MD |  | Enumerated (High-mobilityDetected) | Absence of this IE implies that, according to [4] either the High mobility state is not applicable or it has not been detected by the UE. |  |
| Capability change indicator | OP |  | Enumerated (TRUE) | TRUE indicates that the UE capability has changed compared to the value stored in the variable UE_CAPABILITY_TRANSFERRED. |  |
| Support of common E-DCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support E-DCH enhanced random access in CELL_FACH state and Idle mode. |  |
| Support of HS-DSCH DRX operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-DSCH DRX operation in CELL_FACH state. |  |
| Support of MAC-i/is | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support MAC-i/is operation. |  |
| Security Revert Status Indicator | OP |  | Enumerated (Reverted Back, Normal Operation) | Reverted Back indicates this UE has reverted back to the old security configuration by aborting the ongoing security mode command. Normal Operation indicates this UE has not reverted back to the old security configuration by aborting an ongoing security mode command. The absence of this IE indicates that the UE does not support reporting of security revert status. |  |
| Measurement information elements |  |  |  |  |  |
| Measured results on RACH | OP |  | Measured results on RACH FDD 10.3.7.45a |  |  |
| Other Information elements |  |  |  |  |  |
| Logged Meas Available | OP |  | Enumerated (TRUE) | Indicates the UE has logged measurements to report to the network |  |
| ANR Logging Results Available | OP |  | Enumerated (TRUE) | True indicates the UE has ANR logging results to report to the Network. |  |

| Condition | Explanation |
| --- | --- |
| Failure | This IE is mandatory present if the IE "Failure cause" is present and not needed otherwise. |
| ConversationalCS | This IE is mandatory present if the IE "Establishment cause" has the value "Originating Conversational Call" or "EmergencyCall" and a CS call is being initiated. Otherwise it is not needed. |

### 10.2.8	CELL UPDATE CONFIRM

This message confirms the cell update procedure and can be used to reallocate new RNTI information for the UE valid in the new cell.

RLC-SAP: UM

Logical channel: CCCH or DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE Information Elements |  |  |  |  |  |
| U-RNTI | CV-CCCH |  | U-RNTI 10.3.3.47 |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Integrity protection mode info | OP |  | Integrity protection mode info 10.3.3.19 | The UTRAN should not include this IE unless it is performing an SRNS relocation or a cell reselection from GERAN Iu mode |  |
| Ciphering mode info | OP |  | Ciphering mode info 10.3.3.5 | The UTRAN should not include this IE unless it is performing either an SRNS relocation or a cell reselection from GERAN Iu mode, and a change in ciphering algorithm. |  |
| Activation time | MD |  | Activation time 10.3.3.1 | Default value is "now" |  |
| New U-RNTI | OP |  | U-RNTI 10.3.3.47 |  |  |
| New C-RNTI | OP |  | C-RNTI 10.3.3.8 |  |  |
| New DSCH-RNTI | OP |  | DSCH-RNTI 10.3.3.9a | Should not be set in FDD. If received, the UE behaviour is unspecified. |  |
| New H-RNTI | OP |  | H-RNTI 10.3.3.14a |  | REL-5 |
| New Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-6 |
| New Secondary E-RNTI | OP |  | E-RNTI 10.3.3.10a | FDD only | REL-6 |
| URA_PCH with seamless transition and RNTI handling mode | CV-URA_PCH |  | Enumerated (TRUE) | The presence of this IE indicates that the UE shall clear the variable H_RNTI, C_RNTI and E_RNTI upon cell re-selection when operating in URA_PCH with seamless transition. The absence indicates that the UE shall not clear the variable H_RNTI, C_RNTI and E_RNTI upon cell re-selection when operating in URA_PCH with seamless transition. | REL-13 |
| RRC State Indicator | MP |  | RRC State Indicator 10.3.3.35a |  |  |
| UTRAN DRX cycle length coefficient | OP |  | UTRAN DRX cycle length coefficient 10.3.3.49 |  |  |
| Wait time | OP |  | Wait time 10.3.3.50 |  |  |
| RLC re-establish indicator (RB2, RB3 and RB4) | MP |  | RLC re-establish indicator 10.3.3.35 | Should not be set to TRUE if IE "Downlink counter synchronisation info" is included in message. |  |
| RLC re-establish indicator (RB5 and upwards) | MP |  | RLC re-establish indicator 10.3.3.35 | Should not be set to TRUE if IE "Downlink counter synchronisation info" is included in message. |  |
| CN Information Elements |  |  |  |  |  |
| CN Information info | OP |  | CN Information info 10.3.1.3 |  |  |
| UTRAN Information Elements |  |  |  |  |  |
| URA identity | OP |  | URA identity 10.3.2.6 |  |  |
| RNC support for change of UE capability | OP |  | Boolean | Should be included if the message is used to perform an SRNS relocation | REL-7 |
| RB information elements |  |  |  |  |  |
| RAB information for setup | CV-CS |  | RAB information for setup 10.3.4.10 | RAB identity shall be identical to the one currently configured and UE behaviour is unspecified otherwise. |  |
| RB information to release list | OP | 1 to <maxRB> |  |  |  |
| >RB information to release | MP |  | RB information to release 10.3.4.19 |  |  |
| RB information to reconfigure list | OP | 1 to <maxRB> |  |  |  |
| >RB information to reconfigure | MP |  | RB information to reconfigure 10.3.4.18 |  |  |
| RB information to be affected list | OP | 1 to <maxRB> |  |  |  |
| >RB information to be affected | MP |  | RB information to be affected 10.3.4.17 |  |  |
| Downlink counter synchronisation info | OP |  |  |  |  |
| >RB with PDCP information list | OP | 1 to <maxRBallRABs> |  |  |  |
| >>RB with PDCP information | MP |  | RB with PDCP information 10.3.4.22 | This IE is needed for each RB having PDCP in the case of lossless SRNS relocation |  |
|  | OP |  |  |  | REL-5 |
| >>PDCP context relocation info | OP |  | PDCP context relocation info 10.3.4.1a | This IE is needed for each RB having PDCP and performing PDCP context relocation | REL-5 |
| PDCP ROHC target mode | OP |  | PDCP ROHC target mode 10.3.4.2a |  | REL-5 |
| Retrievable configuration info | OP |  | Retrievable configuration info 10.3.8.25 |  | REL-13 |
| Other state configuration info | OP |  | Other state configuration info 10.3.8.27 |  | REL-13 |
| TrCH Information Elements |  |  |  |  |  |
| Uplink transport channels |  |  |  |  |  |
| UL Transport channel information common for all transport channels | OP |  | UL Transport channel information common for all transport channels 10.3.5.24 |  |  |
| Deleted TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >Deleted UL TrCH information | MP |  | Deleted UL TrCH information 10.3.5.5 |  |  |
| Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >Added or Reconfigured UL TrCH information | MP |  | Added or Reconfigured UL TrCH information 10.3.5.2 |  |  |
| Downlink transport channels |  |  |  |  |  |
| DL Transport channel information common for all transport channels | OP |  | DL Transport channel information common for all transport channels 10.3.5.6 |  |  |
| Deleted TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >Deleted DL TrCH information | MP |  | Deleted DL TrCH information 10.3.5.4 |  |  |
| Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >Added or Reconfigured DL TrCH information | MP |  | Added or Reconfigured DL TrCH information 10.3.5.1 |  |  |
| PhyCH information elements |  |  |  |  |  |
| Frequency info | OP |  | Frequency info 10.3.6.36 |  |  |
| Multi-frequency Info | OP |  | Multi-frequency Info 10.3.6.39a | This IE is used for 1.28 Mcps TDD only | REL-7 |
| DTX-DRX timing information | OP |  | DTX-DRX timing information 10.3.6.34b |  | REL-7 |
| DTX-DRX Information | OP |  | DTX-DRX Information 10.3.6.34a |  | REL-7 |
| HS-SCCH less Information | OP |  | HS-SCCH less Information 10.3.6.36ab |  | REL-7 |
| MIMO parameters | OP |  | MIMO parameters 10.3.6.41a |  | REL-7 |
| MIMO mode with four transmit antennas parameters | OP |  | MIMO mode with four transmit antennas parameters 10.3.6.142 |  | REL-11 |
| DCH Enhancements info FDD | OP |  | DCH Enhancements info FDD 10.3.6.149 |  | REL-12 |
| Control Channel DRX information | OP |  | Control Channel DRX information 1.28 Mcps TDD 10.3.6.107 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| SPS Information | OP |  | SPS information 1.28 Mcps TDD 10.3.6.110 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| MU-MIMO info | OP |  | MU-MIMO info 1.28 Mcps TDD 10.3.6.122 | This IE is used for 1.28 Mcps TDD only | REL-10 |
| Non-rectangular resource allocation indicator | OP |  | Enumerated(TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the non-rectangular resource allocation is not used. | REL-12 |
| Non-rectangular resource specific timeslots set | CV-Non-rectangularResourceAllocation |  | Bit string (7) | For 1.28 Mcps TDD only. The value 1 of a bit indicates the corresponding timeslot in which the channelization codes are assigned by physical control channel. The value 0 of a bit indicates the corresponding timeslot in which all channelization codes are assigned when the timeslot is scheduled to the UE. The first/leftmost bit of the bit string is for TS0. | REL-12 |
| Uplink radio resources |  |  |  |  |  |
| Maximum allowed UL TX power | MD |  | Maximum allowed UL TX power 10.3.6.39 | Default value is the existing maximum UL TX power |  |
| Uplink DPCH info | OP |  | Uplink DPCH info 10.3.6.88 |  |  |
| E-DCH Info | OP |  | E-DCH Info 10.3.6.97 |  | REL-6 |
| Uplink secondary cell info FDD | OP |  | Uplink secondary cell info FDD 10.3.6.115 | FDD only | REL-9 |
| Multi-carrier E-DCH Info for LCR TDD | OP |  | Multi-carrier E-DCH Info for LCR TDD 10.3.6.97a | 1.28Mcps TDD only | REL-10 |
| Uplink CLTD info FDD | OP |  | Uplink CLTD info FDD 10.3.6.125 | FDD only | REL-11 |
| Uplink OLTD info FDD | OP |  | Uplink OLTD info FDD 10.3.6.126 | FDD only | REL-11 |
| Other TTI E-DCH Configuration Information | OP |  | Other TTI E-DCH Configuration Information 10.3.6.152 | FDD only | REL-12 |
| Filtered UE power headroom reporting information | OP |  | Filtered UE power headroom reporting information 10.3.7.140 | FDD only | REL-12 |
| Downlink radio resources |  |  |  |  |  |
| Downlink HS-PDSCH Information | OP |  | Downlink HS_PDSCH Information 10.3.6.23a |  | REL-5 |
| Downlink information common for all radio links | OP |  | Downlink information common for all radio links 10.3.6.24 |  |  |
| Downlink information per radio link list | OP | 1 to <maxRL> |  | Send downlink information for each radio link to be set-up |  |
| >Downlink information for each radio link | MP |  | Downlink information for each radio link 10.3.6.27 |  |  |
| PhyCH information elements (Complete specification) |  |  |  |  |  |
| >>DCH Enhancements info FDD | OP |  | DCH Enhancements info FDD 10.3.6.149 |  | REL-12 |
| Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-8 |
| Additional downlink secondary cell info list FDD | OP | 2 |  |  | REL-10 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-10 |
| Additional downlink secondary cell info list FDD 2 | OP | 4 |  |  | REL-11 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-11 |
| Common E-RGCH info FDD | OP |  | Common E-RGCH info FDD 10.3.6.138 | FDD only | REL-11 |
| MBMS PL Service Restriction Information | OP |  | Enumerated (TRUE) |  | REL-6 |
| CELL_DCH measurement occasion info LCR | OP |  | CELL_DCH measurement occasion info LCR 10.3.7.126 |  | REL-9 |
| WLAN Information Elements |  |  |  |  |  |
| Dedicated WLAN Offload Information | OP |  | Dedicated WLAN Offload Information 10.3.2.10 |  | REL-12 |

| Condition | Explanation |
| --- | --- |
| CCCH | This IE is mandatory present when CCCH is used and ciphering is not required and not needed otherwise. |
| CS | This IE is optionally present only if CS domain RAB mapping is reconfigured between DCH and E-DCH/HS-DSCH and shall not be present otherwise. |
| Non-rectangularResourceAllocation | This IE is optionally present if non-rectangular resource allocation is configured. Otherwise, it is not needed. |
| URA_PCH | This IE is optional present when IE "RRC State Indicator" is set to URA_PCH. Otherwise it is not needed. |

### 10.2.9	COUNTER CHECK

This message is used by the UTRAN to indicate the current COUNT-C MSB values associated to each radio bearer utilising UM or AM RLC mode and to request the UE to compare these to its COUNT-C MSB values and to report the comparison results to UTRAN.

RLC-SAP: AM

Logical channel: DCCH

Direction: UTRANUE

| Information Element/Group name | Presence | Multi | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  |  |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| RB information elements |  |  |  |  |
| RB COUNT-C MSB information | MP | 1 to < maxRBallRABs > |  | For each RB (excluding signalling radio bearers) using UM or AM RLC. |
| >RB COUNT-C MSB information | MP |  | RB COUNT-C MSB information 10.3.4.14 |  |

### 10.2.10	COUNTER CHECK RESPONSE

This message is used by the UE to respond to a COUNTER CHECK message.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Presence | Multi | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  |  |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| RB information elements |  |  |  |  |
| RB COUNT-C information | OP | 1 to < maxRBallRABs > |  |  |
| >RB COUNT-C information | MP |  | RB COUNT-C information 10.3.4.15 |  |

### 10.2.11	DOWNLINK DIRECT TRANSFER

This message is sent by UTRAN to transfer higher layer messages.

RLC-SAP: AM

Logical channel: DCCH

Direction: UTRAN -> UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| CN information elements |  |  |  |  |
| CN Domain Identity | MP |  | Core Network Domain Identity 10.3.1.1 |  |
| NAS message | MP |  | NAS message 10.3.1.8 |  |

### 10.2.12	Void

### 10.2.12a	ETWS PRIMARY NOTIFICATION WITH SECURITY

This message is used by UTRAN to send ETWS information to the UE.

RLC-SAP: AM or UM

Logical channel: DCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  | REL-8 |
| Other Information elements |  |  |  |  | REL-8 |
| ETWS information | MP |  | ETWS information 10.3.8.4ea |  | REL-8 |

### 10.2.13	Void

### 10.2.14	Void

### 10.2.15	HANDOVER FROM UTRAN COMMAND

This message is used for handover from UMTS to another system e.g. GSM. One or several messages from the other system can be included in the Inter-RAT message information element in this message. These messages are structured and coded according to that systems specification.

RLC-SAP: AM

Logical channel: DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| SR-VCC Info | CV-SRVCC |  | SR-VCC info 10.3.4.24a |  | REL-8 |
| rSR-VCC info | CV-rSRVCC |  | rSR-VCC Info 10.3.3.36a |  | REL-11 |
| Activation time | MD |  | Activation time 10.3.3.1 | Default value is "now" |  |
| RB information elements |  |  |  |  |  |
| RAB information list | OP | 1 to <maxRABsetup> |  | For each RAB to be handed over. In handover to GERAN Iu mode the RAB information is included in the GERAN Iu message below. |  |
| >RAB info | MP |  | RAB info 10.3.4.8 |  |  |
| RAB info to replace | CV-SRVCC |  | RAB info to replace 10.3.4.11a |  | REL-8 |
| Other information elements |  |  |  |  |  |
| CHOICE System type | MP |  |  | This IE indicates which specification to apply, to decode the transported messages |  |
| >GSM |  |  |  |  |  |
| >>Frequency band | MP |  | Enumerated (GSM/DCS 1800 band used), GSM/PCS 1900 band used) |  |  |
| >>CHOICE GSM message |  |  |  |  |  |
| >>>Single GSM message | MP |  | Bit string (no explicit size constraint) | Formatted and coded according to GSM specifications The first/leftmost/most significant bit of the bit string contains bit 8 of the first octet of the GSM message. |  |
| >>>GSM message List | MP | 1.to.<maxInterSysMessages> | Bit string (1..512) | Formatted and coded according to GSM specifications. The first/leftmost/most significant bit of the bit string contains bit 8 of the first octet of the GSM message. |  |
| >>CHOICE GERAN System Info type | OP |  |  |  | REL-6 |
| >>>SI | MP |  | GERAN system information 10.3.8.4f | See [44] | REL-6 |
| >>>PSI | MP |  | GERAN system information 10.3.8.4f | See [44] | REL-6 |
| >GERAN Iu |  |  |  |  | REL-5 |
| >>Frequency band | MP |  | Enumerated (GSM/DCS 1800 band used), GSM/PCS 1900 band used) |  | REL-5 |
| >>CHOICE GERAN Iu message |  |  |  |  | REL-5 |
| >>>Single GERAN Iu message | MP |  | Bit string (no explicit size constraint) | Formatted and coded according to [53]. The first/leftmost/most significant bit of the bit string contains bit 8 of the first octet of the message. | REL-5 |
| >>>GERAN Iu message List | MP | 1 to <maxInterSysMessages> | Bit string (1..32768) | Formatted and coded according to [53]. The first/leftmost/most significant bit of the bit string contains bit 8 of the first octet of the message. | REL-5 |
| >cdma2000 |  |  |  |  |  |
| >>cdma2000MessageList | MP | 1.to.<maxInterSysMessages> |  |  |  |
| >>>MSG_TYPE(s) | MP |  | Bit string (8) | Formatted and coded according to cdma2000 specifications. The MSG_TYPE bits are numbered b0 to b7. The first/leftmost/most significant bit of the bit string contains bit 7 of the MSG_TYPE. |  |
| >>>cdma2000Messagepayload(s) | MP |  | Bit string (1..512) | Formatted and coded according to cdma2000 specifications. The first/leftmost/most significant bit of the bit string contains the bit 7 of the first octet of the cdma2000 message. |  |
| >E-UTRA |  |  |  |  | REL-8 |
| >>E-UTRA message | MP |  | Octet string | Formatted and coded according to E-UTRA specifications The first/leftmost/most significant bit of the octet string contains bit 8 of the first octet of the E-UTRA message. | REL-8 |

| Condition | Explanation |
| --- | --- |
| SRVCC | This IE is mandatory present when an SR-VCC procedure is initiated and not needed otherwise. |
| rSRVCC | This IE is mandatory present when an rSR-VCC procedure is initiated and not needed otherwise. |

### 10.2.16	HANDOVER FROM UTRAN FAILURE

This message is sent on the RRC connection used before the Inter-RAT Handover was executed. The message indicates that the UE has failed to seize the new channel in the other system.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Other information elements |  |  |  |  |  |
| Inter-RAT handover failure | OP |  | Inter-RAT handover failure 10.3.8.6 |  |  |
| CHOICE System type | OP |  |  | This IE indicates which specification to apply to decode the transported messages |  |
| >GSM |  |  |  |  |  |
| >>GSM message List | MP | 1.to.<maxInterSysMessages> | Bit string (1..512) | Formatted and coded according to GSM specifications. The first/leftmost/most significant bit of the bit string contains bit 8 of the first octet of the GSM message. |  |
| >GERAN Iu |  |  |  |  | REL-5 |
| >>GERAN Iu message List | MP | 1 to <maxInterSysMessages> | Bit string (1..32768) | Formatted and coded according to [53]. The first/leftmost/most significant bit of the bit string contains bit 8 of the first octet of the message. | REL-5 |
| >cdma2000 |  |  |  |  |  |
| >>cdma2000MessageList | MP | 1.to.<maxInterSysMessages> |  |  |  |
| >>>MSG_TYPE(s) | MP |  | Bit string (8) | Formatted and coded according to cdma2000 specifications. The MSG_TYPE bits are numbered b0 to b7. The first/leftmost/most significant bit of the bit string contains bit 7 of the MSG_TYPE. |  |
| >>>cdma2000Messagepayload(s) | MP |  | Bit string (1..512) | Formatted and coded according to cdma2000 specifications. The first/leftmost/most significant bit of the bit string contains bit 7 of the first octet of the cdma2000 message. |  |
| >E-UTRA |  |  |  |  | REL-8 |
| >>E-UTRA message | OP |  | Octet string | Formatted and coded according to E-UTRA specifications. The first/leftmost/most significant bit of the octet string contains bit 8 of the first octet of the E-UTRA message. | REL-8 |

### 10.2.16a	HANDOVER TO UTRAN COMMAND

This message is sent to the UE via other system to make a handover to UTRAN.

RLC-SAP: N/A (Sent through a different RAT)

Logical channel: N/A (Sent through a different RAT)

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| New U-RNTI | MP |  | U-RNTI Short 10.3.3.48 |  |  |
| Ciphering algorithm | OP |  | Ciphering algorithm 10.3.3.4 |  |  |
| RNC support for change of UE capability | MP |  | Boolean |  | REL-7 |
| New H-RNTI | OP |  | H-RNTI 10.3.3.14a |  | REL-6 |
| New Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-6 |
| New Secondary E-RNTI | OP |  | E-RNTI 10.3.3.10a | FDD only | REL-6 |
| Specification mode information elements |  |  |  |  | REL-8 |
| Default configuration for CELL_FACH | OP |  | Default configuration for CELL_FACH 10.3.4.0a |  | REL-8 |
| CHOICE specification mode | MP |  |  |  |  |
| >Complete specification |  |  |  |  |  |
| RB information elements |  |  |  |  |  |
| >>Signalling RB information to setup list | MP | 1 to <maxSRBsetup> |  | For each signalling radio bearer established |  |
| >>>Signalling RB information to setup | MP |  | Signalling RB information to setup 10.3.4.24 |  |  |
| >>RAB information to setup list | OP | 1 to <maxRABsetup> |  | For each RAB established |  |
| >>>RAB information for setup | MP |  | RAB information for setup 10.3.4.10 |  |  |
| Uplink transport channels |  |  |  |  |  |
| >>UL Transport channel information common for all transport channels | MP |  | UL Transport channel information common for all transport channels 10.3.5.24 |  |  |
| >>Added or Reconfigured TrCH information | MP | 1 to <maxTrCH> |  |  |  |
| >>>Added or Reconfigured UL TrCH information | MP |  | Added or Reconfigured UL TrCH information 10.3.5.2 |  |  |
| Downlink transport channels |  |  |  |  |  |
| >>DL Transport channel information common for all transport channels | MP |  | DL Transport channel information common for all transport channels 10.3.5.6 |  |  |
| >>Added or Reconfigured TrCH information | MP | 1 to <maxTrCH> |  |  |  |
| >>>Added or Reconfigured DL TrCH information | MP |  | Added or Reconfigured DL TrCH information 10.3.5.1 |  |  |
| Uplink radio resources |  |  |  |  |  |
| >>Uplink DPCH info | MP |  | Uplink DPCH info 10.3.6.88 |  |  |
| >>E-DCH Info | OP |  | E-DCH Info 10.3.6.97 |  | REL-6 |
| >>Uplink secondary cell info FDD | OP |  | Uplink secondary cell info FDD 10.3.6.115 | FDD only | REL-11 |  |
| Downlink radio resources (Complete specification) |  |  |  |  |  |
| >>Downlink HS-PDSCH Information | OP |  | Downlink HS-PDSCH Information 10.3.6.23a |  | REL-6 |
| >>Downlink information common for all radio links | MP |  | Downlink information common for all radio links 10.3.6.24 |  |  |
| >>Downlink information per radio link | MP | 1 to <maxRL> |  |  |  |
| >>>Downlink information for each radio link | MP |  | Downlink information for each radio link 10.3.6.27 |  |  |
| >>Downlink secondary cell info FDD |  | OP |  | Downlink secondary cell info FDD for Handover to UTRAN 10.3.6.31b | FDD only | REL-11 |  |
| >>Additional downlink secondary cell info list FDD |  | OP | 2 |  |  | REL-11 |  |
| >>>Downlink secondary cell info FDD |  | OP |  | Downlink secondary cell info FDD for Handover to UTRAN 10.3.6.31b | FDD only | REL-11 |  |
| PhyCH information elements (Complete specification) |  |  |  |  |  |
| >>DCH Enhancements info FDD | OP |  | DCH Enhancements info FDD 10.3.6.149 |  | REL-12 |
| >Preconfiguration |  |  |  |  |  |
| >>CHOICE Preconfiguration mode | MP |  |  |  |  |
| >>>Predefined configuration | MP |  | Predefined configuration identity 10.3.4.5 | NOTE 1 |  |
| >>>Default configuration |  |  |  |  |  |
| >>>>Default configuration mode | MP |  | Enumerated (FDD, TDD) | Indicates whether the FDD or TDD version of the default configuration shall be used |  |
| >>>>Default configuration identity | MP |  | Default configuration identity 10.3.4.0 |  |  |
| >>RAB info | OP |  | RAB info Post 10.3.4.9 | One RAB is established |  |
| >>CHOICE mode | MP |  |  |  |  |
| >>>FDD |  |  |  |  |  |
| >>>>Uplink DPCH info | MP |  | Uplink DPCH info Post 10.3.6.89 |  |  |
| >>>CHOICE tdd |  |  |  |  |  |
| >>>>1.28 Mcps TDD and 7.64 Mcps TDD |  |  |  |  |  |
| >>>>> Uplink DPCH info | MP |  | Uplink DPCH info 10.3.6.88 |  |  |
| Downlink radio resources (Preconfiguration) |  |  |  |  |  |
| >>>>3.84 Mcps TDD |  |  |  |  |  |
| >>>>>Uplink DPCH info |  | MP |  | Uplink DPCH info Post 10.3.6.89 |  |  |
| >>Downlink information common for all radio links |  | MP |  | Downlink information common for all radio links Post 10.3.6.25 |  |  |
| >>Downlink information per radio link |  | MP | 1 to <maxRL> |  | Send downlink information for each radio link to be set-up. In TDD MaxRL is 1. |  |
| >>>Downlink information for each radio link |  | MP |  | Downlink information for each radio link Post 10.3.6.28 |  |  |
| >>CHOICE mode |  | MP |  |  |  |  |
| >>>FDD |  |  |  |  | (no data) |  |
| >>>TDD |  |  |  |  |  |  |
| >>>>Primary CCPCH Tx Power |  | MP |  | Primary CCPCH Tx Power 10.3.6.59 |  |  |
| Frequency info |  | MP |  | Frequency info 10.3.6.36 |  |  |
| Multi-frequency Info |  | OP |  | Multi-frequency Info 10.3.6.39a | This IE is used for 1.28 Mcps TDD only | REL-7 |
| Maximum allowed UL TX power |  | MP |  | Maximum allowed UL TX power 10.3.6.39 |  |  |

NOTE 1:	Predefined configurations are not used in case of handover from E-UTRAN.

### 10.2.16b	HANDOVER TO UTRAN COMPLETE

This message is sent by the UE when a handover to UTRAN has been completed.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE  UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE Information elements |  |  |  |  |  |
| START list | CH | 1 to <maxCNdomains> |  | START [40] values for all CN domains. |  |
| >CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >START | MP |  | START 10.3.3.38 |  |  |
| RB Information elements |  |  |  |  |  |
| COUNT-C activation time | OP |  | Activation time 10.3.3.1 | Used for radio bearers mapped on RLC-TM. |  |
| Other Information elements |  |  |  |  | REL-10 |
| Logged Meas Available | OP |  | Enumerated (TRUE) | Indicates the UE has logged measurements to report to the network | REL-10 |
| Connection Establishment Failure Info Available | OP |  | Enumerated (TRUE) | True indicates the UE has logged measurements from a RRC connection establishment failure to report to the network | REL-11 |

### 10.2.16c	INITIAL DIRECT TRANSFER

This message is used to initiate a signalling connection based on indication from the upper layers, and to transfer a NAS message.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE -> UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| PLMN identity | OP |  | PLMN identity 10.3.1.11 | This IE indicates the PLMN to which the UE requests the signalling connection to be established. | REL-6 |
| Support of CSG | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support access control based on CSG | REL-8 |
| CN information elements |  |  |  |  |  |
| CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| Intra Domain NAS Node Selector | MP |  | Intra Domain NAS Node Selector 10.3.1.6 |  |  |
| NAS message | MP |  | NAS message 10.3.1.8 |  |  |
| START | OP |  | START 10.3.3.38 | START value to be used in the CN domain as indicated in the IE "CN domain identity". This IE shall always be present in this version of the protocol. |  |
| Establishment cause | OP |  | Establishment cause 10.3.3.11 |  | Rel-5 |
| CS Call Type | CV-ConversationalCS |  | Enumerated (speech, video, other) | One spare value is needed | REL-7 |
| Measurement information elements |  |  |  |  |  |
| Measured results on RACH | OP |  | Measured results on RACH 10.3.7.45 |  |  |
| MBMS joined information | OP |  |  |  | REL-6 |
| >P-TMSI | OP |  | P-TMSI (GSM-MAP) 10.3.1.13 |  | REL-6 |

| Condition | Explanation |
| --- | --- |
| ConversationalCS | This IE is mandatory present if the IE "Establishment cause" has the value "Originating Conversational Call" or "Emergency Call", the IE "CN domain identity" has the value "CS domain" and the UE is in CELL_FACH state. Otherwise it is not needed. |

### 10.2.16d	INTER RAT HANDOVER INFO

This message is sent by the UE via another radio access technology to provide information to the target RNC when preparing for a handover to UTRAN.

RLC-SAP: N/A (Sent through a different RAT)

Logical channel: N/A (Sent through a different RAT)

Direction: UE  UTRAN

| Information Element/Group Name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Radio Bearer IEs |  |  |  |  |  |
| Predefined configuration status information | OP |  | Predefined configuration status information 10.3.4.5a |  |  |
| Predefined configuration status information compressed | OP |  | Predefined configuration status information compressed 10.3.4.5b |  | REL-5 |
| UE Information elements |  |  |  |  |  |
| UE security information | OP |  | UE security information 10.3.3.42b |  |  |
| UE security information2 | OP |  | UE security information2 10.3.3.42c |  | REL-6 |
| UE Specific Behaviour Information 1 interRAT | OP |  | UE Specific Behaviour Information 1 interRAT 10.3.3.52 | This IE shall not be included in this version of the protocol |  |
| UE capability container | OP |  |  |  |  |
| >UE radio access capability | MP |  | UE radio access capability 10.3.3.42 |  |  |
| >UE radio access capability extension | MP |  | UE radio access capability extension 10.3.3.42a | Although this IE is not always required, the need has been set to MP to align with the ASN.1 |  |
| UE radio access capability compressed | OP |  | UE radio access capability compressed 10.3.3.42o |  | REL-5 |
| UE radio access capability comp 2 | CV-Fdd |  | UE radio access capability comp 2 10.3.3.42oa |  | REL-6 |
| UE radio access capability comp for 1.28 Mcps | CV-128Tdd |  | UE radio access capability comp for 1.28 Mcps TDD 10.3.3.42ob |  | REL-8 |
| Support for F-DPCH | CV-SupportForHS |  | Enumerated (TRUE) | Presence of this information element indicates that F-DPCH is supported by the UE. Absence of this information element indicates that F-DPCH is not supported | REL-12 |

| Condition | Explanation |
| --- | --- |
| Fdd | This IE is mandatory present for FDD, otherwise it is not needed. |
| 128Tdd | This IE is mandatory present for 1.28 Mcps TDD, otherwise it is not needed. |
| SupportForHS | This IE is mandatory present if the UE supports HS-PDSCH, otherwise it is not needed. |

### 10.2.16da	LOGGING MEASUREMENT CONFIGURATION

This message is used to configure logged measurements

RLC-SAP: AM

Logical channel: DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  | REL-10 |
| UE information elements |  |  |  |  | REL-10 |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  | REL-10 |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  | REL-10 |
| Other Information Elements |  |  |  |  | REL-10 |
| Logged Measurements Configuration Info | OP |  | Logged Measurements Configuration Info 10.3.7.132 |  | REL-10 |
| Logged ANR configuration Info | OP |  | Logged ANR Configuration Info 10.3.7.42a |  | REL-10 |

### 10.2.16e	MBMS Access Information

This message is transmitted periodically by UTRAN to inform UEs that have joined or selected a particular MBMS service about the need to establish an RRC connection or to perform a cell update. While the message contents may change within a modification period, all occurances of the information within a modification period concern the same MBMS service(s). If the cell on which this message is sent is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this message would not have been received.

RLC-SAP: UM

Logical channel: MCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message type | MP |  | Message Type |  | REL-6 |
| Service list | MP | 1 to <maxMBMSservCount> |  |  | REL-6 |
| >MBMS short transmission ID | MP |  | MBMS Short transmission identity 10.3.9a.10 | Reference/ index to a transmission listed in the MBMS Modified Services Information or MBMS Unmodified Services Information | REL-6 |
| >Access probability factor - Idle | MP |  | Integer (0 to 960 by step of 32, 1000) | Access probability factor for UEs in idle mode. The actual Access Probability (AP) is a function of the Access Probability Factor (APF): AP (APF) = 2-(APF/100) | REL-6 |
| >Access probability factor – connected | MD |  | Integer (0 to 960 by step of 32, 1000) | Access probability factor for UEs in connected mode. The actual Access Probability (AP) is a function of the Access Probability Factor (APF): AP (APF) = 2-(APF/100) Default value is the value included in IE "Access probability factor - Idle" | REL-6 |
| >Connected mode counting scope | MP |  |  |  | REL-6 |
| >>URA_PCH | MP |  | BOOLEAN | TRUE means that UEs in URA_PCH state shall participate in counting | REL-6 |
| >>CELL_PCH | MP |  | BOOLEAN | TRUE means that UEs in CELL_PCH state shall participate in counting | REL-6 |
| >> CELL_FACH | MP |  | BOOLEAN | TRUE means that UEs in CELL_FACH state shall participate in counting | REL-6 |

### 10.2.16f	MBMS Common p-t-m rb Information

This message is transmitted periodically by UTRAN to inform UEs about the p-t-m RB configuration information that may be common between different services, applicable in the current and/ or in neighbouring cells. The message contents does not change within a modification period.

RLC-SAP: UM

Logical channel: MCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message type | MP |  | Message Type |  | REL-6 |
| RB information list | MP | 1 to <maxMBMS-CommonRB> |  |  | REL-6 |
| >RB identity | MP |  | MBMS Common RB identity 10.3.9a.3 |  | REL-6 |
| >PDCP info | MP |  | PDCP info 10.3.4.2 |  | REL-6 |
| >RLC info | MP |  | RLC info MBMS 10.3.4.23a |  | REL-6 |
| TrCh information for each TrCh | MP | 1 to <maxMBMS-CommonTrCh> |  |  | REL-6 |
| >Transport channel identity | MP |  | MBMS Common TrCh identity 10.3.9a.4 |  | REL-6 |
| >TFS | MP |  | Transport format set 10.3.5.23 |  | REL-6 |
| TrCh information for each CCTrCh | OP | 1 to <maxMBMS-CommonCCTrCh> |  | The list needs not include the CCTrCh for which the default TFCS for MBMS applies, as specified in subclause 14.10.1. | REL-6 |
| >CCTrCH identity | MP |  | MBMS Common CCTrCh identity 10.3.9a.1 |  | REL-6 |
| >TFCS | MP |  | Transport format combination set 10.3.5.20 |  | REL-6 |
| PhyCh information | MP | 1 to <maxMBMS-CommonPhyCh> |  |  | REL-6 |
| >PhyCh identity | MP |  | MBMS Common PhyCh identity 10.3.9a.2 |  | REL-6 |
| >CHOICE mode | MP |  |  |  | REL-8 |
| >>FDD or TDD |  |  |  |  | REL-8 |
| >>>Secondary CCPCH info MBMS | MP |  | Secondary CCPCH info MBMS 10.3.6.71a |  | REL-6 |
| >>3.84 Mcps TDD IMB |  |  |  | 3.84 Mcps TDD IMB is a subset of 3.84 Mcps TDD. | REL-8 |
| >>>Secondary CCPCH frame type 2 info | MP |  | Secondary CCPCH frame type 2 info 10.3.6.114 |  | REL-8 |
| LCR TDD MBSFN information | OP |  | LCR TDD MBSFN Information 10.3.6.78c | 1.28 Mcps TDD only: included only if some timeslots of the secondary frequency of a multi-frequency cell are designated to MBSFN. | REL-7 |

### 10.2.16g	MBMS Current Cell p-t-m rb Information

This message is transmitted periodically by UTRAN to inform UEs about the PTM RB configuration used to in a cell, in case one or more MBMS service is provided using p-t-m radio bearers. The message contents does not change within a modification period.

RLC-SAP: UM

Logical channel: MCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message type | MP |  | Message Type | Current cell PTM RB info | REL-6 |
| S-CCPCH list | OP | 1 to <maxSCCPCH> |  | Absent in case MTCH are only mapped to the S-CCPCH(s) included in SIB type 5 or 5bis | REL-6 |
| >S-CCPCH identity | OP |  | MBMS Current cell S-CCPCH identity 10.3.9a.5 | When L1- or L2 combining applies, this identity is used to refer to this S-CCPCH within the NEIGHBOURING CELL P-T-M RB INFORMATION message If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. When Chip combining (1.28Mcps TDD only) applies, this identity is used to refer to the S-CCPCH within the NEIGHBOURING CELL P-T-M RB INFORMATION message | REL-6 |
| >Secondary CCPCH info | MP |  | MBMS Common PhyCh identity 10.3.9a.2 | Refers to a configuration in the common RB info | REL-6 |
| >MBMS Soft Combining Timing Offset | CV-Soft-FDD |  | MBMS Soft Combining Timing Offset 10.3.9a.10a | Timing offset applied in the CFN calculation in sub-clause 8.5.15.5. The default value is 0 ms. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. | REL-6 |
| >TrCh information common for all TrCh | MD |  | MBMS Common CCTrCh identity 10.3.9a.1 | Refers to a (TFCS) configuration in the common RB info The default value of the TFCS is specified in subclause 14.10.1 | REL-6 |
| >TrCH information list | MP | 1 to <maxFACHPCH> |  | List of FACH transport channels carrying one or more MTCH and optionally one MSCH | REL-6 |
| >>TrCh information | MP |  | MBMS Common TrCh identity 10.3.9a.4 | Refers to a (TFS) configuration in the common RB info | REL-6 |
| >>RB information list | OP | 1 to <maxRBperTrCh> |  | The IE is absent if temporarily no RBs are mapped to this TrCh or if the TrCH only carries MSCH | REL-6 |
| >>>RB information | MP |  | MBMS p-t-m RB information 10.3.9a.7a |  | REL-6 |
| >>MSCH configuration information | OP |  | MSCH configuration information 10.3.9a.16 |  | REL-6 |
| S-CCPCH in SIB type 5 | OP | 1 to <maxSCCPCH> |  | Every S-CCPCH’s included in SIB type 5 or 5bis may carry MTCH If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. | REL-6 |
| >S-CCPCH identity |  |  | Integer (1..maxSCCPCH) | Index of the S-CCPCH within the list included in SIB type 5 or 5bis | REL-6 |
| >TrCH information list | MP | 1 to <maxFACHPCH> |  | List of FACH transport channels carrying one or more MTCH and optionally one MSCH | REL-6 |
| >>TrCh identity | MP |  | Integer (1..maxFACHPCH) | Index of the FACH within the list of TrChs defined for that S-CCPCH as included in SIB type 5 or 5bis | REL-6 |
| >>RB information list | OP | 1 to <maxRBperTrCh> |  | The IE is absent if this TrCh only carries MSCH | REL-6 |
| >>>RB information | MP |  | MBMS p-t-m RB information 10.3.9a.7a |  | REL-6 |
| >>MSCH configuration information | OP |  | MSCH configuration information 10.3.9a.16 | Included if the TrCH carries MSCH | REL-6 |
| MBSFN TDM Info List | CV-Mbsfn |  | MBSFN TDM Information List 10.3.9a.12b |  | REL-7 |

| Condition | Explanation |
| --- | --- |
| Soft-FDD | This IE is used only for FDD. It is mandatory default for FDD if the IE "L1 combining" is included in MBMS NEIGHBOURING CELL P-T-M RB INFORMATION. Otherwise it is not needed. |
| Mbsfn | The IE is optionally present if the cell supports MBSFN, and not needed otherwise. |

### 10.2.16h	MBMS General Information

This message is transmitted periodically by UTRAN to inform UEs about the general MBMS (configuration) information. The message contents does not change within a modification period.

RLC-SAP: UM

Logical channel: MCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message type | MP |  | Message Type |  | REL-6 |
| MBMS preferred frequency information | OP |  | MBMS preferred frequency information 10.3.7.43a | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. | REL-6 |
| MBMS timers and counters | MP |  | MBMS specific timers and counters 10.3.9a.11 |  | REL-6 |
| MICH configuration information | MP |  | MICH configuration information 10.3.9a.14 |  | REL-6 |
| Cell group identity | MP |  | Bit string (12) | Identifies the group of cells for which the same common RLC and PDCP entity is used as the current cell | REL-6 |
| Default MSCH configuration information | OP |  | MSCH configuration information 10.3.9a.16 | The default MSCH configuration | REL-6 |
| Indicate changes in MBMS Selected Services | MP |  | Boolean | TRUE means the UE indicates changes in MBMS Selected Services while in URA_PCH, CELL_PCH or in CELL_FACH Default value is FALSE. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE has the value FALSE. NOTE 1. | REL-6 |
| MBSFN inter frequency neighbour list | OP |  | MBSFN inter frequency neighbour list 10.3.7.43b | May be included for FDD and 3.84/7.68 Mcps TDD if the cell is operating in MBSFN mode | Rel-7 |
| Network Standard Time Information | OP |  | Network standard time information 10.3.9a.17 | Only for 1.28 Mcps TDD | Rel-8 |

NOTE 1:	The default value is used when the non-critical extension v6b0NonCriticalExtensions is not included.

### 10.2.16i	MBMS Modification request

The UE transmits this message to request UTRAN to take certain actions to improve the UE's ability to receive its (prioritised) activated MBMS services and/ or sessions.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE  UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message type | MP |  | Message Type |  | REL-6 |
| UE information elements |  |  |  |  | REL-6 |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  | REL-6 |
| MBMS information elements |  |  |  |  | REL-6 |
| MBMS preferred frequency request | OP |  | MBMS service identity 10.3.9a.8 | The MBMS preferred frequency the UE would like to be moved to. The MBMS preferred frequency is identified by the identity of the MBMS service the UE would like to receive. | REL-6 |
| MBMS RB list requested to be released | OP | 1 to <maxRB> |  | RBs of lower priority MBMS services inhibiting reception of a higher priority service | REL-6 |
| >RB information to release | MP |  | RB information to release 10.3.4.19 |  | REL-6 |
| MBMS Selected Service Info | MP |  | MBMS Selected Service Info 10.3.9a.7b | NOTE 1. | REL-6 |
| Support of MBMS service change for a ptp RB | OP |  | Enumerated (TRUE) | In this version of the specification, this IE has no meaning but it shall be included upon the conditions specified in subclause 8.7.6.2a. | REL-6 |

NOTE 1:	The default value is used when the non-critical extension v6b0NonCriticalExtensions is not included.

### 10.2.16j	MBMS Modified services Information

This information is transmitted periodically by UTRAN to inform UEs about a change applicable for one or more MBMS services available in the current cell and possibly in neighbouring cells.

RLC-SAP: AM (DCCH only) or UM

Logical channel: MCCH, DCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message type | MP |  | Message Type |  | REL-6 |
| UE information elements |  |  |  |  | REL-6 |
| Integrity check info | CV-DCCHCH |  | Integrity check info 10.3.3.16 | Integrity check info is included if integrity protection is applied | REL-6 |
| MBMS information elements |  |  |  |  | REL-6 |
| Modified service list | OP | 1.. <maxMBMSservModif> |  |  | REL-6 |
| >MBMS Transmission identity | MP |  | MBMS Transmission identity 10.3.9a.12 |  | REL-6 |
| >MBMS required UE action | MP |  | Enumerated (None, Acquire counting info, Acquire counting info – PTM RBs unmodified, Acquire PTM RB info, RequestPTP RB, Release PTM RB) | Indicates required UE action upon receiving the message. When sent on the DCCH, only the following values apply: None, Acquire PTM RB info, request PTP RB). If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 all values except “Acquire PTM RB info” and "Release PTM RB" are handled as if “None” was received. | REL-6 |
| >MBMS PTM RB Release Cause | CV-PTM_RB_RELEASE_LCR_TDD |  | Enumerated (NormalRelease, Out of MBMS Service Coverage in RAN, Network Abnormal Release) | Indicates the PTM RB Release cause. For 1.28 Mcps TDD only. | REL-7 |
| >MBMS preferred frequency | OP |  |  | Indicates the frequency that UEs shall consider as the preferred frequency. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. | REL-6 |
| >>PFL index | CV-MCCH |  | Integer (1.. <maxMBMS-Freq>) | Index pointing to an entry in the list included in MBMS GENERAL INFORMATION. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. | REL-6 |
| >>PFL info | CV- DCCH |  | Frequency info 10.3.6.36 |  | REL-6 |
| >Continue MCCH reading | MP |  | BOOLEAN | MCCH in- band notification. Indicates whether or not the UE should continue reading MCCH in the next modification period. Not applicable when sent on the DCCH | REL-6 |
| >MBSFN cluster frequency | CV-MBSFN |  | Integer (1..<maxMBSFNclusters> | For FDD and 3.84/7.68 Mcps TDD index pointing to a frequency indicated in the IE "MBSFN inter frequency neighbour list " in MBMS General Information. Default: the current MBSFN cluster. For 1.28 Mcps TDD index pointing to a frequency indicated in the SIB 11. Default: the current MBSFN cluster. | REL-7 |
| MBMS re- acquire MCCH | CV-MCCHOP |  | Enumerated (TRUE) |  | REL-6 |
| MBMS dynamic persistence level | CV-MCCHOP |  | Dynamic persistence level 10.3.6.35 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified | REL-6 |
| End of modified MCCH information | CV-MCCHOP |  | Integer (1..16) | If present: the UE may assume that, in each repetition period, all the MCCH information preceding the MBMS UNMODIFIED SERVICES INFORMATION message is transmitted within the indicated number of TTIs. | REL-6 |
| MBMS number of neighbour cells | MP |  | Integer (0..32) | Indicates the number of MBMS NEIGHBOURING CELL P-T-M RB INFORMATION messages that are contained within the MCCH transmission. Not applicable when sent on the DCCH. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if the value 0 was received. | REL-6 |
| MBMS all unmodified p-t-m services | CV-MCCHOP |  | Enumerated (TRUE) | TRUE means that the UE should re-acquire the PtM information for all services listed in the message MBMS Unmodified services Information with the IE “MBMS required UE action” set to “Acquire PTM RB info” | REL-6 |
| MBMS p-t-m activation time | CV-MCCHOP |  | MBMS p-t-m activation time 10.3.9a.7o |  | REL-6 |
| MIB Value tag | OP |  | MIB Value tag 10.3.8.9 | Indicates the matching System Information. | REL-7 |

| Condition | Explanation |
| --- | --- |
| MCCH | This IE is mandatory present if the message is sent via MCCH and not needed otherwise. |
| DCCH | This IE is mandatory present if the message is sent via DCCH and not needed otherwise. |
| MCCHOP | This IE is optionally present if the message is sent via MCCH and not needed otherwise. |
| DCCHCH | This IE is present conditionnaly on history if the message is sent via DCCH and not present otherwise. |
| MBSFN | This IE is mandatory default when the cell on which this IE is sent is operating in MBSFN mode according to subclause 8.1.1.6.3. Otherwise the IE is not needed. |
| PTM_RB_RELEASE_LCR_TDD | This IE is optionally present if the IE “MBMS required UE action” has the value “Release PTM RB”. Otherwise it is not needed. For 1.28 Mcps TDD only. |

### 10.2.16k	MBMS Neighbouring Cell p-t-m rb Information

This message is transmitted periodically by UTRAN to inform UEs about the p-t-m RB configuration used to in neighbouring cells, indicating the UE may perform selection and/ or soft combining. The message content does not change within a modification period. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this message is unspecified.

RLC-SAP: UM

Logical channel: MCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message type | MP |  | Message Type |  | REL-6 |
| Neighbouring cell identity | MP |  | Integer (0..<maxCellMeas1>) | The intra-frequency cell id of the cell obtained from the IE 'Intra-frequency Cell Info list' in SIB 11/SIB 11bis. In case the intra-frequency cell id is omitted in the IE ‘Intra-frequency Cell Info list’ in SIB 11/SIB11bis, it refers to the index (starting at zero) in the CELL_INFO_LIST as if the CELL_INFO_LIST was constructed from SIB11/SIB11bis only, see 8.6.7.3. | REL-6 |
| Neighbouring cell’s S-CCPCH list | MP | 1 to <maxSCCPCH> |  |  | REL-6 |
| >CHOICE PhyCh | MP |  |  |  | REL-7 |
| >>By reference |  |  |  |  | REL-7 |
| >>>Secondary CCPCH info | MP |  | MBMS Common PhyCh identity 10.3.9a.2 | S-CCPCH configuration used in neighbouring cell. Refers to a configuration in the common RB info of the current cell | REL-6 |
| >>Extended |  |  |  | FDD only | REL-7 |
| >>>Secondary CCPCH info | OP |  | MBMS Common PhyCh identity 10.3.9a.2 | Referenced S-CCPCH configuration used in neighbouring cell. Refers to a configuration in the common RB info of the current cell | REL-7 |
| >>>Secondary CCPCH info MBMS Diff | MP |  | Secondary CCPCH info MBMS Diff 10.3.6.71b | Differential S-CCPCH configuration. Physical configuration parameters in this IE replace the corresponding parameters in the referenced S-CCPCH configuration. If the referenced S-CCPCH configuration is absent, the full set of parameters is needed. | REL-7 |
| >Secondary CCPCH Power Offset Difference | MD |  | Integer (-6, -3, 3, 6) | Difference (Pn Of) between the S-CCPCH power offset (Pn) of the neighboring cell S-CCPCH and the S-CCPCH power offset (Of) of the serving cell that is going to be combined to this neighbour cell S-CCPCH, in dB. Default value is 0. Note 3 and 4. | REL-6 |
| >L1 combining | OP |  |  | L2- combining applies if the IE is absent | REL-6 |
| >>CHOICE mode | MP |  |  |  | REL-6 |
| >>>FDD |  |  |  |  | REL-6 |
| >>>>MBMS Soft Combining Timing Offset | MP |  | MBMS Soft Combining Timing Offset 10.3.9a.10a | Timing offset applied in the CFN calculation in subclause 8.5.15.5 | REL-6 |
| >>>>MBMS transmission time difference | MP |  | Integer (0..3) | Indicates the time difference between the TTIs on the current and the neighbouring cell’s SCCPCH that can be L1- combined | REL-6 |
| >>>>MBMS L1 combining schedule | OP |  | MBMS L1 combining schedule 10.3.9a.7 | If included partial layer 1 combining applies, in which case this IE indicates when L1- combining applies. If the IE is absent, L1 combining applies continuously | REL-6 |
| >>>TDD |  |  |  | (no data) | REL-6 |
|  |  |  |  | If L1 combining is configured, for 1.28Mcps TDD UE, MBMS data reception shall be implemented by means of chip combine receiver [55]. | REL-7 |
| >CHOICE L23 configuration | MP |  |  |  | REL-6 |
| >>SameAs Current cell |  |  |  | Apart from the physical channel configuration and the MSCH configuration information, the same configuration as for the indicated S-CCPCH used in the current cell applies. The MSCH is mapped on the same transport channel as in the current cell. | REL-6 |
| >>>Current cell's S-CCPCH | MP |  | MBMS Current cell S-CCPCH identity 10.3.9a.5 | Reference to the S-CCPCH in the current cell which uses exaclty the same configuration (excluding MSCH configuration). | REL-6 |
| >>>MSCH configuration information | OP |  | MSCH configuration information 10.3.9a.16 |  | REL-6 |
| >>Different |  |  |  |  | REL-6 |
| >>>TrCh information for common for all TrCh | MD |  | MBMS Common CCTrCh identity 10.3.9a.1 | Refers to a (TFCS) configuration in the common RB info The default value of the TFCS is specified in subclause 14.10.1 | REL-6 |
| >>>TrCH information list | MP | 1 to <maxFACHPCH> |  | List of FACH transport channels carrying one or more MTCH and optionally one MSCH | REL-6 |
| >>>>TrCh information | MP |  | MBMS Common TrCh identity 10.3.9a.4 | Refers to a (TFS) configuration in the common RB info | REL-6 |
| >>>>TrCh combining status | MP |  | BOOLEAN | Value TRUE means that TrCh combining is used for this transport channel (TDD only). Note 2. The IE shall be ignored in FDD mode. | REL-6 |
| >>>>RB information list | OP | 1 to <maxRBperTrCh> |  | The IE is only present for the radio bearers for which selection (FDD) or transport channel (TDD) combining applies. | REL-6 |
| >>>>>RB information | MP |  | MBMS p-t-m RB information 10.3.9a.7a |  | REL-6 |
| >>>>MSCH configuration information | OP |  | MSCH configuration information 10.3.9a.16 | Included if the TrCH carries MSCH | REL-6 |

NOTE 1:	The signalling supports the option that UTRAN maps one service to L1 combining slots for some neighbours and to the L2 combining slots for other neighbours ie. the use of different combining schemes for different neighbours

NOTE 2:	Transport combining can only be indicated when the complete L2 configuration is provided for the neighbouring cell (i.e. using L2 configuration choice “different”). Fortunately, a scenario in which the neighbouring cell configuration is different from the current cell is regarded as the typical scenario for using transport combining.

NOTE 3:	For FDD, an S-CCPCH power offset is defined as the offset between the transmitted power of the data part of one S-CCPCH and the transmitted power of the P-CPICH of a given cell (Ps-ccpch – Pp-cpich). For TDD, an S-CCPCH power offset is defined as the offset between one S-CCPCH and the P-CCPCH of a given cell (Ps-ccpch – Pp-ccpch).

NOTE 4:	The Secondary CCPCH Power Offset Difference IE gives the UE an indication of the S-CCPCH power on the neighbouring cells that may be used to complete the neighbouring cell ranking based on P-CPICH power for FDD or P-CCPCH for TDD.

### 10.2.16L	MBMS Scheduling Information

This message is transmitted periodically by UTRAN to inform UEs when the MBMS services, provided on the same S-CCPCH as the message is sent, are scheduled to be transmitted. The UE may use the scheduling information to discontinue receiving the concerned S-CCPCH. The message is transmitted in accordance with the MSCH configuration applicable for the corresponding S-CCPCH.

RLC-SAP: UM

Logical channel: MSCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message type | MP |  | Message Type |  | REL-6 |
| Service scheduling info list | MP | 1 to < maxMBMSservSched> |  |  | REL-6 |
| >MBMS Transmission identity | MP |  | MBMS Transmission identity 10.3.9a.12 |  | REL-6 |
| >MBMS Service transmissions info list | OP | 1 to < maxMBMSTransmis> |  | One or more sets of scheduling information comprising of the beginning and duration of an MBMS service transmission for one scheduling period | REL-6 |
| >>Start | MP |  | Integer (0..1020) by step of 4 | Indicates the start of the transmission relative to the start of the TTI in which the MBMS SCHEDULING INFORMATION message was received. In number of radio frames (i.e. the value 0 would correspond to the start of the MBMS transmission being in the same TTI as the MBMS Scheduling Information message.) See note 1. | REL-6 |
| >>Duration | MP |  | Integer (4..1024) by step of 4 | In number of radio frames | REL-6 |
| >Next scheduling period | MP |  | Integer (0..31) | Number of scheduling periods (see 10.3.9a.16), after the current scheduling period, in which no data will be transmitted for the concerned service. If the Next scheduling period is set to 0, data may be transmitted for the concerned service in the scheduling period immediately following the current scheduling period | REL-6 |

NOTE: 	If UTRAN sets the value of the IE "Start" to a value less than or equal to 16 then the UE may not successfully receive the start of the MTCH transmission.

### 10.2.16m	MBMS Unmodified services Information

This message is transmitted periodically by UTRAN to inform UEs about the MBMS services, available in the current cell and possibly in neighbouring cells, that have not changed if the IE "MBMS all unmodified p-t-m services" is not included in the MBMS MODIFIED SERVICES INFORMATION message in this modification period. The message is repeated every repetition period while its contents does not change within a modification period.

RLC-SAP: UM

Logical channel: MCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message type | MP |  | Message Type |  | REL-6 |
| Unmodified services list | OP | 1 to <maxMBMSservUnmodif> |  | If the IE “MBMS all unmodified p-t-m services” is included in the MBMS MODIFIED SERVICES INFORMATION message in this modification period, the services with the IE “MBMS required UE action” set to “Acquire PTM RB info” in the message MBMS Unmodified services Information should be considered as modified. | REL-6 |
| >MBMS Transmission identity | MP |  | MBMS Transmission identity 10.3.9a.12 |  | REL-6 |
| >MBMS required UE action | MP |  | Enumerated (None, Acquire PTM RB info, Request PTP RB ) | Indication of the UE action required to receive the service. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 all values except “Acquire PTM RB info” are handled as if “None” was received. | REL-6 |
| >MBMS preferred frequency | OP |  | Integer (1.. <maxMBMS-Freq>) | Information about the frequency that UEs shall consider as the preferred frequency layer for cell re-selection during a session for an activated MBMS service, as specified in [25.304]. Index pointing to an entry in the list included in MBMS GENERAL INFORMATION. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. | REL-6 |
| >MBSFN cluster frequency | CV-MBSFN |  | Integer (1..<maxMBSFNclusters> | For FDD and 3.84/7.68 Mcps TDD index pointing to a frequency indicated in the IE "MBSFN inter frequency neighbour list " in MBSFN General Information. Default: the current MBSFN cluster. For 1.28 Mcps TDD index pointing to a frequency indicated in the SIB 11. Default: the current MBSFN cluster. | REL-7 |

| Condition | Explanation |
| --- | --- |
| MBSFN | This IE is mandatory present when the cell on which this IE is sent is operating in MBSFN mode according to subclause 8.1.1.6.3. Otherwise the IE is not needed. |

### 10.2.17	MEASUREMENT CONTROL

This message is sent by UTRAN to setup, modify or release a measurement in the UE.

RLC-SAP: AM

Logical channel: DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Measurement Information elements |  |  |  |  |  |
| Measurement Identity | CV-command2 |  | Measurement Identity 10.3.7.48 |  |  |
| Measurement Command | MP |  | Measurement Command 10.3.7.46 |  |  |
| Release Some Measurements | CV-command3 |  | Release some measurements 10.3.7.46b |  | REL-12 |
| Measurement Reporting Mode | OP |  | Measurement Reporting Mode 10.3.7.49 |  |  |
| Additional measurements list | OP |  | Additional measurements list 10.3.7.1 |  |  |
| CHOICE Measurement type | CV-command |  |  |  |  |
| >Intra-frequency measurement |  |  | Intra-frequency measurement 10.3.7.36 |  |  |
| >Inter-frequency measurement |  |  | Inter-frequency measurement 10.3.7.16 |  |  |
| >Inter-RAT measurement |  |  | Inter-RAT measurement 10.3.7.27 |  |  |
| >UE positioning measurement |  |  | UE positioning measurement 10.3.7.100 |  |  |
| >Traffic Volume measurement |  |  | Traffic Volume measurement 10.3.7.68 |  |  |
| >Quality measurement |  |  | Quality measurement 10.3.7.56 |  |  |
| >UE internal measurement |  |  | UE internal measurement 10.3.7.77 |  |  |
| >CSG Proximity detection |  |  | CSG Proximity detection 10.3.7.123 |  | REL-9 |
| >E-UTRA measurement for CELL_FACH |  |  | E-UTRA measurement for CELL_FACH 10.3.7.137 |  | REL-11 |
| CELL_DCH measurement occasion info LCR | OP |  | CELL_DCH measurement occasion info LCR 10.3.7.126 |  | REL-9 |
| Measurement Scaling Factor | CV_Measurement |  | Enumerated (SF_UTRA_CF1, SF_UTRA_CF2) | The Measurement Scaling Factor is used to scale the measurement performance for UTRA/E-UTRA frequencies configured in CELL_DCH as specified in [19]. Even if no UTRA or E-UTRA frequency is configured for reduced measurement performance, the network may configure a scaling factor and configure any of the scaling factor values [19]. | REL-12 |
| Physical channel information elements |  |  |  |  |  |
| DPCH compressed mode status info | OP |  | DPCH compressed mode status info 10.3.6.34 |  |  |

| Condition | Explanation |
| --- | --- |
| Command2 | The IE is mandatory present if the IE "Measurement Command" is set to "Setup", "Modify" or "Release", otherwise the IE is not needed. |
| Command3 | The IE is mandatory present if the IE "Measurement Command" is set to "ReleaseSome", otherwise the IE is not needed. |
| Command | The IE is mandatory present if the IE "Measurement command" is set to "Setup", optional if the IE "Measurement command" is set to "modify", otherwise the IE is not needed. |
| Measurement | The IE is optionally present if the IE "Inter-frequency measurement" and/or IE "Inter-RAT measurement" is present, otherwise the IE is not needed. |

### 10.2.18	MEASUREMENT CONTROL FAILURE

This message is sent by UE, if it cannot initiate a measurement as instructed by UTRAN.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Failure cause | MP |  | Failure cause and error information 10.3.3.14 |  |  |
| Measurement identity | OP |  | Measurement identity 10.3.7.48 |  | REL-12 |

### 10.2.19	MEASUREMENT REPORT

This message is used by UE to transfer measurement results to the UTRAN.

RLC-SAP: AM or UM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Activation time | CV-PreConf |  | Activation time 10.3.3.1 |  | REL-8 |
| Measurement Information Elements |  |  |  |  |  |
| Measurement identity | MP |  | Measurement identity 10.3.7.48 |  |  |
| Measured Results | OP |  | Measured Results 10.3.7.44 |  |  |
| Measured Results on secondary UL frequency | OP |  | Measured Results on secondary UL frequency 10.3.7.118 |  | REL-9 |
| Measured Results on RACH | OP |  | Measured Results on RACH 10.3.7.45 |  |  |
| Additional Measured results | OP | 1 to <maxAdditionalMeas> |  |  |  |
| >Measured Results | MP |  | Measured Results 10.3.7.44 |  |  |
| Additional Measured results on secondary UL frequency | OP | 1 to <maxAdditionalMeas> |  |  | REL-9 |
| >Measured Results | MP |  | Measured Results on secondary UL frequency10.3.7.118 |  |  |
| Event results | OP |  | Event results 10.3.7.7 |  |  |
| Event results on secondary UL frequency | OP |  | Cell measurement event results on secondary UL frequency 10.3.7.117 |  | REL-9 |
| Inter-RAT cell info indication | CV-IRAT |  | Integer (0..3) |  | REL-5 |
| E-UTRA Measured Results | OP |  | E-UTRA Measured Results 10.3.7.6c |  | REL-8 |
| E-UTRA Event Results | OP |  | E-UTRA Event Results 10.3.7.6a |  | REL-8 |
| E-UTRA results for CELL_FACH | OP |  | E-UTRA results for CELL_FACH 10.3.7.138 |  | REL-11 |
| CSG Proximity Indication | OP |  | 10.3.7.122 |  | REL-9 |
| Other Information elements |  |  |  |  | Rel-10 |
| Logged Meas Available | OP |  | Enumerated (TRUE) | Indicates the UE has logged measurements to report to the network | Rel-10 |
| ANR Logging Results Available | OP |  | Enumerated (TRUE) | True indicates the UE has ANR logging results to report to the Network. | REL-10 |

| Condition | Explanation |
| --- | --- |
| IRAT | The IE is optionally present if at least one of the IE "Inter-RAT measured results list" and the IE "Inter-RAT measurement event results" is included in the message. Otherwise, the IE is not needed. |
| PreConf | The IE is mandatory if the table “Target cell preconfigurations” in the variable includes the cell that triggered the event and the IE “Activation time offset” is different from 0. Otherwise, the IE is not needed. |

### 10.2.20	PAGING TYPE 1

This message is used to send information on the paging channel. One or several UEs, in idle or connected mode, can be paged in one message, which also can contain other information.

RLC-SAP: TM

Logical channel: PCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE Information elements |  |  |  |  |  |
| Paging record list | OP | 1 to <maxPage1> |  |  |  |
| >Paging record | MP |  | Paging record 10.3.3.23 |  |  |
| Other information elements |  |  |  |  |  |
| BCCH modification info | OP |  | BCCH modification info 10.3.8.1 |  |  |
| ETWS information | OP |  | ETWS information 10.3.8.4ea |  | REL-8 |

If the encoded message does not fill a transport block, the RRC layer shall add padding according to subclause 12.1.

### 10.2.21	PAGING TYPE 2

This message is used to page a UE in connected mode, when using the DCCH for CN originated paging.

RLC-SAP: AM

Logical channel: DCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| Paging cause | MP |  | Paging cause 10.3.3.22 |  |
| CN Information elements |  |  |  |  |
| CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |
| Paging Record Type Identifier | MP |  | Paging Record Type Identifier 10.3.1.10 |  |

### 10.2.22	PHYSICAL CHANNEL RECONFIGURATION

This message is used by UTRAN to assign, replace or release a set of physical channels used by a UE.

RLC-SAP: AM or UM

Logical channel: DCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE Information Elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Integrity protection mode info | OP |  | Integrity protection mode info 10.3.3.19 | The UTRAN should not include this IE unless it is performing an SRNS relocation |  |
| Ciphering mode info | OP |  | Ciphering mode info 10.3.3.5 | The UTRAN should not include this IE unless it is performing an SRNS relocation and a change in ciphering algorithm |  |
| Activation time | MD |  | Activation time 10.3.3.1 | Default value is "now" |  |
| Dynamic activation time | OP |  | Dynamic activation time 10.3.3.1a |  | REL-13 |
| Delay restriction flag | OP |  | Enumerated (TRUE) | This IE is always set to TRUE and included if the activation time is restricted according to subclause 8.6.3.1 | REL-6 |
| New U-RNTI | OP |  | U-RNTI 10.3.3.47 |  |  |
| New C-RNTI | OP |  | C-RNTI 10.3.3.8 |  |  |
| New DSCH-RNTI | OP |  | DSCH-RNTI 10.3.3.9a | Should not be set in FDD. If received the UE behaviour is unspecified. |  |
| New H-RNTI | OP |  | H-RNTI 10.3.3.14a |  | REL-5 |
| New Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-6 |
| New Secondary E-RNTI | OP |  | E-RNTI 10.3.3.10a | FDD only | REL-6 |
| RRC State Indicator | MP |  | RRC State Indicator 10.3.3.35a |  |  |
| UE Mobility State Indicator | CV-FACH_PCH |  | Enumerated (High-mobilityDetected) | Absence of this IE implies that, according to [4], the UE shall consider itself being in the mobility state the UE has maintained in CELL_DCH state or being not in high mobility state after the state transition, if applicable. | REL-7 |
| UTRAN DRX cycle length coefficient | OP |  | UTRAN DRX cycle length coefficient 10.3.3.49 |  |  |
| CN Information Elements |  |  |  |  |  |
| CN Information info | OP |  | CN Information info 10.3.1.3 |  |  |
| UTRAN mobility information elements |  |  |  |  |  |
| URA identity | OP |  | URA identity 10.3.2.6 |  |  |
| RNC support for change of UE capability | OP |  | Boolean | Should be included if the message is used to perform an SRNS relocation | REL-7 |
| Reconfiguration in response to requested change of UE capability | OP |  | Enumerated (TRUE) |  | REL-7 |
| RB information elements |  |  |  |  |  |
| Downlink counter synchronisation info | OP |  |  |  |  |
| >RB with PDCP information list | OP | 1 to <maxRBallRABs> |  |  |  |
| >>RB with PDCP information | MP |  | RB with PDCP information 10.3.4.22 | This IE is needed for each RB having PDCP in the case of lossless SRNS relocation |  |
|  | OP |  |  |  | REL-5 |
| >>PDCP context relocation info | OP |  | PDCP context relocation info 10.3.4.1a | This IE is needed for each RB having PDCP and performing PDCP context relocation | REL-5 |
| Retrievable configuration info | OP |  | Retrievable configuration info 10.3.8.25 |  | REL-13 |
| Downlink transport channels |  |  |  |  | REL-8 |
| HARQ Info | OP |  | HARQ info 10.3.5.7a |  | REL-7 |
| PhyCH information elements |  |  |  |  |  |
| Frequency info | OP |  | Frequency info 10.3.6.36 |  |  |
| Multi-frequency Info | OP |  | Multi-frequency Info 10.3.6.39a | This IE is used for 1.28 Mcps TDD only | REL-7 |
| DTX-DRX timing information | OP |  | DTX-DRX timing information 10.3.6.34b |  | REL-7 |
| DTX-DRX Information | OP |  | DTX-DRX Information 10.3.6.34a |  | REL-7 |
| HS-SCCH less Information | OP |  | HS-SCCH less Information 10.3.6.36ab |  | REL-7 |
| MIMO parameters | OP |  | MIMO parameters 10.3.6.41a |  | REL-7 |
| MIMO mode with four transmit antennas parameters | OP |  | MIMO mode with four transmit antennas parameters 10.3.6.142 |  | REL-11 |
| DCH Enhancements info FDD | OP |  | DCH Enhancements info FDD 10.3.6.149 |  | REL-12 |
| Control Channel DRX information | OP |  | Control Channel DRX information 1.28 Mcps TDD 10.3.6.107 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| SPS Information | OP |  | SPS information 1.28 Mcps TDD 10.3.6.110 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| MU-MIMO info | OP |  | MU-MIMO info 1.28 Mcps TDD 10.3.6.122 | This IE is used for 1.28 Mcps TDD only | REL-10 |
| Non-rectangular resource allocation indicator | OP |  | Enumerated(TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the non-rectangular resource allocation is not used. | REL-12 |
| Non-rectangular resource specific timeslots set | CV-Non-rectangularResourceAllocation |  | Bit string (7) | For 1.28 Mcps TDD only. The value 1 of a bit indicates the corresponding timeslot in which the channelization codes are assigned physical control channel. The value 0 of a bit indicates the corresponding timeslot in which all channelization codes are assigned when the timeslot is scheduled to the UE. The first/leftmost bit of the bit string is for TS0. | REL-12 |
| Uplink radio resources |  |  |  |  |  |
| Maximum allowed UL TX power | MD |  | Maximum allowed UL TX power 10.3.6.39 | Default value is the existing value of the maximum allowed UL TX power |  |
| Uplink DPCH info | OP |  | Uplink DPCH info 10.3.6.88 |  |  |
| E-DCH Info | OP |  | E-DCH Info 10.3.6.97 |  | REL-6 |
| Uplink secondary cell info FDD | OP |  | Uplink secondary cell info FDD 10.3.6.115 | FDD only | REL-9 |
| Multi-carrier E-DCH Info for LCR TDD | OP |  | Multi-carrier E-DCH Info for LCR TDD 10.3.6.97a | 1.28Mcps TDD only | REL-10 |
| Uplink CLTD info FDD | OP |  | Uplink CLTD info FDD 10.3.6.125 | FDD only | REL-11 |
| Uplink OLTD info FDD | OP |  | Uplink OLTD info FDD 10.3.6.126 | FDD only | REL-11 |
| Downlink radio resources |  |  |  |  |  |
| Downlink HS-PDSCH Information | OP |  | Downlink HS_PDSCH Information 10.3.6.23a |  | REL-5 |
| Downlink information common for all radio links | OP |  | Downlink information common for all radio links 10.3.6.24 |  |  |
| Downlink information per radio link list | OP | 1 to <maxRL> |  | Send downlink information for each radio link |  |
| >Downlink information for each radio link | MP |  | Downlink information for each radio link 10.3.6.27 |  |  |
| Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-8 |
| Additional downlink secondary cell info list FDD | OP | 2 |  |  | REL-10 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-10 |
| Additional downlink secondary cell info list FDD 2 | OP | 4 |  |  | REL-11 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-11 |
| Common E-RGCH info FDD | OP |  | Common E-RGCH info FDD 10.3.6.138 | FDD only | REL-11 |
| DPCCH2 info FDD | OP |  | DPCCH2 info FDD 10.3.6.148 | FDD only | REL-12 |
| MBMS PL Service Restriction Information | OP |  | Enumerated (TRUE) |  | REL-6 |
| CELL_DCH measurement occasion info LCR | OP |  | CELL_DCH measurement occasion info LCR 10.3.7.126 |  | REL-9 |

| Condition | Explanation |
| --- | --- |
| FACH_PCH | This IE is mandatory default when a transition from CELL_DCH to CELL_FACH, URA_PCH or CELL_PCH is requested by the message and is not needed otherwise. |
| Non-rectangularResourceAllocation | This IE is optionally present if non-rectangular resource allocation is configured. Otherwise, it is not needed. |

### 10.2.23	PHYSICAL CHANNEL RECONFIGURATION COMPLETE

This message is sent from the UE when a physical channel reconfiguration has been done.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE  UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Uplink integrity protection activation info | OP |  | Integrity protection activation info 10.3.3.17 |  |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  | (no data) |  |
| >TDD |  |  |  |  |  |
| >>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>Uplink Timing Advance | OP |  | Uplink Timing Advance 10.3.6.95 |  |  |
| >>>>Extended Uplink Timing Advance | OP |  | Extended Uplink Timing Advance 10.3.6.95a |  | REL-7 |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Extended Uplink Timing Advance | OP |  | Extended Uplink Timing Advance 10.3.6.95a |  | REL-7 |
| >>>1.28 Mcps TDD |  |  |  | (no data) | REL-4 |
| Other Information elements |  |  |  |  | REL-7 |
| Deferred measurement control reading | OP |  | Enumerated (TRUE) | Indicates the UE has not read SIB11, SIB11bis, SIB11ter (if supported) and SIB12, if available. | REL-7 |
| RB Information elements |  |  |  |  |  |
| COUNT-C activation time | OP |  | Activation time 10.3.3.1 | Used for radio bearers mapped on RLC-TM. |  |
| Uplink counter synchronisation info | OP |  |  |  |  |
| >RB with PDCP information list | OP | 1 to <maxRBallRABs> |  |  |  |
| >>RB with PDCP information | MP |  | RB with PDCP information 10.3.4.22 |  |  |
| >START list | MP | 1 to <maxCNdomains> |  | START [40] values for all CN domains. |  |
| >>CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >>START | MP |  | START 10.3.3.38 | START value to be used in this CN domain. |  |

### 10.2.24	PHYSICAL CHANNEL RECONFIGURATION FAILURE

This message is sent by UE if the configuration given by UTRAN is unacceptable or if the UE failed to assign, replace or release a set of physical channel(s).

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message type | MP |  | Message type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | OP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| Failure cause | MP |  | Failure cause and error information 10.3.3.14 |  |

### 10.2.25	PHYSICAL SHARED CHANNEL ALLOCATION

NOTE:	Only for TDD.

This message is used by UTRAN to assign physical resources to USCH/DSCH transport channels in TDD, for temporary usage by the UE.

RLC-SAP: UM on SHCCH, UM on DCCH

Logical channel: SHCCH or DCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message type |  |
| DSCH-RNTI | OP |  | DSCH-RNTI 10.3.3.9a |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Uplink timing advance Control | MD |  | Uplink Timing Advance Control 10.3.6.96 | Default value is the existing value for uplink timing advance |
| PUSCH capacity allocation info | OP |  | PUSCH Capacity Allocation info 10.3.6.64 |  |
| PDSCH capacity allocation info | OP |  | PDSCH Capacity Allocation info 10.3.6.42 |  |
| Confirm request | MD |  | Enumerated(No Confirm, Confirm PDSCH, Confirm PUSCH) | Default value is No Confirm |
| Traffic volume report request | OP |  | Integer (0 .. 255) | Indicates the number of frames between start of the allocation period and sending measurement report. The value should be less than the value for Allocation Duration. |
| ISCP Timeslot list | OP | 1 to maxTS |  |  |
| >Timeslot number | MP |  | Timeslot number 10.3.6.84 | Timeslot numbers, for which the UE shall report the timeslot ISCP in PUSCH CAPACITY REQUEST message. |
| Request P-CCPCH RSCP | MP |  | Boolean | TRUE indicates that a Primary CCPCH RSCP measurement shall be reported by the UE in PUSCH CAPACITY REQUEST message. |

### 10.2.26	PUSCH CAPACITY REQUEST

NOTE:	Only for TDD.

This message is used by the UE for request of PUSCH resources to the UTRAN.

RLC-SAP: TM

Logical channel: SHCCH

Direction: UE  UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| DSCH-RNTI | OP |  | DSCH-RNTI 10.3.3.9a |  |
| RRC transaction identifier | CV-ProtErr |  | RRC transaction identifier 10.3.3.36 |  |
| Traffic Volume | OP |  | Traffic Volume, measured results list 10.3.7.67 |  |
| Timeslot list | OP | 1 to maxTS |  |  |
| >Timeslot number | MP |  | Timeslot number 10.3.6.84 |  |
| >Timeslot ISCP | MP |  | Timeslot ISCP info 10.3.7.65 |  |
| Primary CCPCH RSCP | OP |  | Primary CCPCH RSCP info 10.3.7.54 |  |
| CHOICE Allocation confirmation | OP |  |  |  |
| >PDSCH Confirmation |  |  | Integer(1..hiPDSCHidentities) |  |
| >PUSCH Confirmation |  |  | Integer(1..hiPUSCHidentities) |  |
| Protocol error indicator | MD |  | Protocol error indicator 10.3.3.27 | Default value is FALSE |
| Protocol error information | CV-ProtErr |  | Protocol error information 10.3.8.12 |  |

| Condition | Explanation |
| --- | --- |
| ProtErr | This IE is mandatory present if the IE "Protocol error indicator" has the value TRUE. Otherwise it is not needed. |

### 10.2.27	RADIO BEARER RECONFIGURATION

This message is sent from UTRAN to reconfigure parameters related to a change of QoS or to release and setup a radio bearer used for ptp transmission of MBMS services of the broadcast type. This procedure can also change the multiplexing of MAC, reconfigure transport channels and physical channels. This message is also used to perform a handover from GERAN Iu mode to UTRAN.

RLC-SAP: AM or UM or sent through GERAN Iu mode

Logical channel: DCCH or sent through GERAN Iu mode

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE Information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Integrity protection mode info | OP |  | Integrity protection mode info 10.3.3.19 | The UTRAN should not include this IE unless it is performing an SRNS relocation or a handover from GERAN Iu mode |  |
| Ciphering mode info | OP |  | Ciphering mode info 10.3.3.5 | The UTRAN should not include this IE unless it is performing either an SRNS relocation or a handover from GERAN Iu mode and a change in ciphering algorithm |  |
| Activation time | MD |  | Activation time 10.3.3.1 | Default value is "now" |  |
| Dynamic activation time | OP |  | Dynamic activation time 10.3.3.1a |  | REL-13 |
| Delay restriction flag | OP |  | Enumerated (TRUE) | This IE is always set to TRUE and included if the activation time is restricted according to subclause 8.6.3.1 | REL-6 |
| New U-RNTI | OP |  | U-RNTI 10.3.3.47 |  |  |
| New C-RNTI | OP |  | C-RNTI 10.3.3.8 |  |  |
| New DSCH-RNTI | OP |  | DSCH-RNTI 10.3.3.9a | Should not be set in FDD. If received The UE behaviour is unspecified. |  |
| New H-RNTI | OP |  | H-RNTI 10.3.3.14a |  | REL-5 |
| New Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-6 |
| New Secondary E-RNTI | OP |  | E-RNTI 10.3.3.10a | FDD only | REL-6 |
| URA_PCH with seamless transition and RNTI handling mode | CV-URA_PCH |  | Enumerated (TRUE) | The presence of this IE indicates that the UE shall clear the variable H_RNTI, C_RNTI and E_RNTI upon cell re-selection when operating in URA_PCH with seamless transition. The absence indicates that the UE shall not clear the variable H_RNTI, C_RNTI and E_RNTI upon cell re-selection when operating in URA_PCH with seamless transition. | REL-13 |
| RRC State Indicator | MP |  | RRC State Indicator 10.3.3.35a |  |  |
| UE Mobility State Indicator | CV-FACH_PCH |  | Enumerated (High-mobilityDetected) | Absence of this IE implies that, according to [4], the UE shall consider itself being in the mobility state the UE has maintained in CELL_DCH state or being not in high mobility state after the state transition, if applicable. | REL-7 |
| UTRAN DRX cycle length coefficient | OP |  | UTRAN DRX cycle length coefficient 10.3.3.49 |  |  |
| Access Group identity | OP |  | Integer (0..15) |  | REL-12 |
| CN information elements |  |  |  |  |  |
| CN Information info | OP |  | CN Information info 10.3.1.3 |  |  |
| UTRAN mobility information elements |  |  |  |  |  |
| RNC support for change of UE capability | OP |  | Boolean | Should be included if the message is used to perform an SRNS relocation | REL-7 |
| Reconfiguration in response to requested change of UE capability | OP |  | Enumerated (TRUE) |  | REL-7 |
| URA identity | OP |  | URA identity 10.3.2.6 |  |  |
| Specification mode information elements |  |  |  |  | REL-8 |
| Default configuration for CELL_FACH | CV-SRNS_RELOCATION |  | Default configuration for CELL_FACH 10.3.4.0a |  | REL-8 |
| CHOICE specification mode | MP |  |  |  | REL-5 |
| >Complete specification |  |  |  |  |  |
| RB information elements |  |  |  |  |  |
| >>RAB information to reconfigure list | OP | 1 to < maxRABsetup > |  |  |  |
| >>>RAB information to reconfigure | MP |  | RAB information to reconfigure 10.3.4.11 |  |  |
| >>RAB information for MBMS ptp bearer list | OP | 1 to < maxMBMSservSelect > |  |  | REL-6 |
| >>>RAB information for MBMS ptp bearer | MP |  | RAB information for MBMS ptp bearer 10.3.4.9a |  | REL-6 |
| >>RB information to reconfigure list | MP | 1to <maxRB> |  | Although this IE is not always required, need is MP to align with ASN.1 |  |
|  | OP |  |  |  | REL-4 |
| >>>RB information to reconfigure | MP |  | RB information to reconfigure 10.3.4.18 |  |  |
| >>RB information to be affected list | OP | 1 to <maxRB> |  |  |  |
| >>>RB information to be affected | MP |  | RB information to be affected 10.3.4.17 |  |  |
| >>RB with PDCP context relocation info list | OP | 1 to <maxRBallRABs> |  | This IE is needed for each RB having PDCP and performing PDCP context relocation | REL-5 |
| >>>PDCP context relocation info | MP |  | PDCP context relocation info 10.3.4.1a |  | REL-5 |
| >>PDCP ROHC target mode | OP |  | PDCP ROHC target mode 10.3.4.2a |  | REL-5 |
| >>UM RLC re-establishment RB List | OP | 1 to <maxRB> |  |  | REL-10 |
| >>>RB bearer to be re-established | MP |  | RB identity, 10.3.4.16 | UE behaviour is unspecified if RB identity in this IE indicates a value, which is less than or equal to 4 or indicates a radio bearer, which does not use UM RLC or is not associated with PS domain. | REL-10 |
| >>Other state configuration info | OP |  | Other state configuration info 10.3.8.27 |  | REL-13 |
| TrCH Information Elements |  |  |  |  |  |
| Uplink transport channels |  |  |  |  |  |
| >>UL Transport channel information common for all transport channels | OP |  | UL Transport channel information common for all transport channels 10.3.5.24 |  |  |
| >>Deleted TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >>>Deleted UL TrCH information | MP |  | Deleted UL TrCH information 10.3.5.5 |  |  |
| >>Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >>>Added or Reconfigured UL TrCH information | MP |  | Added or Reconfigured UL TrCH information 10.3.5.2 |  |  |
| Downlink transport channels |  |  |  |  |  |
| >>DL Transport channel information common for all transport channels | OP |  | DL Transport channel information common for all transport channels 10.3.5.6 |  |  |
| >>Deleted TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >>>Deleted DL TrCH information | MP |  | Deleted DL TrCH information 10.3.5.4 |  |  |
| >>Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >>>Added or Reconfigured DL TrCH information | MP |  | Added or Reconfigured DL TrCH information 10.3.5.1 |  |  |
| Retrievable configuration |  |  |  |  |  |
| >>Retrievable configuration info | OP |  | Retrievable configuration info 10.3.8.25 |  | REL-13 |
| >Preconfiguration |  |  |  |  | REL-5 |
| >>CHOICE Preconfiguration mode | MP |  |  | This value only applies in case the message is sent through GERAN Iu mode |  |
| >>>Predefined configuration identity | MP |  | Predefined configuration identity 10.3.4.5 |  |  |
| >>>Default configuration |  |  |  |  |  |
| >>>>Default configuration mode | MP |  | Enumerated (FDD, TDD) | Indicates whether the FDD or TDD version of the default configuration shall be used |  |
| >>>>Default configuration identity | MP |  | Default configuration identity 10.3.4.0 |  |  |
| PhyCH information elements |  |  |  |  |  |
| Frequency info | OP |  | Frequency info 10.3.6.36 |  |  |
| Multi-frequency Info | OP |  | Multi-frequency Info 10.3.6.39a | This IE is used for 1.28 Mcps TDD only | REL-7 |
| DTX-DRX timing information | OP |  | DTX-DRX timing information 10.3.6.34b |  | REL-7 |
| DTX-DRX Information | OP |  | DTX-DRX Information 10.3.6.34a |  | REL-7 |
| HS-SCCH less Information | OP |  | HS-SCCH less Information 10.3.6.36ab |  | REL-7 |
| MIMO parameters | OP |  | MIMO parameters 10.3.6.41a |  | REL-7 |
| MIMO mode with four transmit antennas parameters | OP |  | MIMO mode with four transmit antennas parameters 10.3.6.142 |  | REL-11 |
| DCH Enhancements info FDD | OP |  | DCH Enhancements info FDD 10.3.6.149 |  | REL-12 |
| Control Channel DRX information | OP |  | Control Channel DRX information 1.28 Mcps TDD 10.3.6.107 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| SPS Information | OP |  | SPS information 1.28 Mcps TDD 10.3.6.110 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| MU-MIMO info | OP |  | MU-MIMO info 1.28 Mcps TDD 10.3.6.122 | This IE is used for 1.28 Mcps TDD only | REL-10 |
| Non-rectangular resource allocation indicator | OP |  | Enumerated(TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the non-rectangular resource allocation is not used. | REL-12 |
| Non-rectangular resource specific timeslots set | CV-Non-rectangularResourceAllocation |  | Bit string (7) | For 1.28 Mcps TDD only. The value 1 of a bit indicates the corresponding timeslot in which the channelization codes are assigned by physical control channel. The value 0 of a bit indicates the corresponding timeslot in which all channelization codes are assigned when the timeslot is scheduled to the UE. The first/leftmost bit of the bit string is for TS0. | REL-12 |
| Uplink radio resources |  |  |  |  |  |
| Maximum allowed UL TX power | MD |  | Maximum allowed UL TX power 10.3.6.39 | Default value is the existing maximum UL TX power |  |
| Uplink DPCH info | OP |  | Uplink DPCH info 10.3.6.88 |  |  |
| E-DCH Info | OP |  | E-DCH Info 10.3.6.97 |  | REL-6 |
| Uplink secondary cell info FDD | OP |  | Uplink secondary cell info FDD 10.3.6.115 | FDD only | REL-9 |
| Multi-carrier E-DCH Info for LCR TDD | OP |  | Multi-carrier E-DCH Info for LCR TDD 10.3.6.97a | 1.28Mcps TDD only | REL-10 |
| Uplink CLTD info FDD | OP |  | Uplink CLTD info FDD 10.3.6.125 | FDD only | REL-11 |
| Uplink OLTD info FDD | OP |  | Uplink OLTD info FDD 10.3.6.126 | FDD only | REL-11 |
| Other TTI E-DCH Configuration Information | OP |  | Other TTI E-DCH Configuration Information 10.3.6.152 | FDD only | REL-12 |
| Filtered UE power headroom reporting information | OP |  | Filtered UE power headroom reporting information 10.3.7.140 | FDD only | REL-12 |
| Downlink radio resources |  |  |  |  |  |
| Downlink HS-PDSCH Information | OP |  | Downlink HS-PDSCH Information 10.3.6.23a |  | REL-5 |
| Downlink information common for all radio links | OP |  | Downlink information common for all radio links 10.3.6.24 |  |  |
| Downlink information per radio link list | MP | 1 to <maxRL> |  | Although this IE is not always required, need is MP to align with ASN.1 |  |
|  | OP |  |  |  | REL-4 |
| >Downlink information for each radio link | MP |  | Downlink information for each radio link 10.3.6.27 |  |  |
| Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-8 |
| Additional downlink secondary cell info list FDD | OP | 2 |  |  | REL-10 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-10 |
| Additional downlink secondary cell info list FDD 2 | OP | 4 |  |  | REL-11 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-11 |
| Common E-RGCH info FDD | OP |  | Common E-RGCH info FDD 10.3.6.138 | FDD only | REL-11 |
| DPCCH2 info FDD | OP |  | DPCCH2 info FDD 10.3.6.148 | FDD only | REL-12 |
| MBMS PL Service Restriction Information | OP |  | Enumerated (TRUE) |  | REL-6 |
| CELL_DCH measurement occasion info LCR | OP |  | CELL_DCH measurement occasion info LCR 10.3.7.126 |  | REL-9 |

| Condition | Explanation |
| --- | --- |
| FACH_PCH | This IE is mandatory default when a transition from CELL_DCH to CELL_FACH, URA_PCH or CELL_PCH is requested by the message and is not needed otherwise. |
| SRNS_RELOCATION | This IE is optional if the message is used to perform an SRNS relocation and is not needed otherwise. |
| Non-rectangularResourceAllocation | This IE is optionally present if non-rectangular resource allocation is configured. Otherwise, it is not needed. |
| URA_PCH | This IE is optional present when IE "RRC State Indicator" is set to URA_PCH. Otherwise it is not needed. |

### 10.2.28	RADIO BEARER RECONFIGURATION COMPLETE

This message is sent from the UE when a RB and signalling link reconfiguration has been done.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE  UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Uplink integrity protection activation info | OP |  | Integrity protection activation info 10.3.3.17 |  |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  | (no data) |  |
| >TDD |  |  |  |  |  |
| >>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>Uplink Timing Advance | OP |  | Uplink Timing Advance 10.3.6.95 |  |  |
| >>>>Extended Uplink Timing Advance | OP |  | Extended Uplink Timing Advance 10.3.6.95a |  | REL-7 |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Extended Uplink Timing Advance | OP |  | Extended Uplink Timing Advance 10.3.6.95a |  | REL-7 |
| >>>1.28 Mcps TDD |  |  |  | (no data) | REL-4 |
| Other Information elements |  |  |  |  | REL-7 |
| Deferred measurement control reading | OP |  | Enumerated (TRUE) | Indicates the UE has not read SIB11, SIB11bis, SIB11ter (if supported) and SIB12, if available. | REL-7 |
| RB Information elements |  |  |  |  |  |
| COUNT-C activation time | OP |  | Activation time 10.3.3.1 | Used for radio bearers mapped on RLC-TM. |  |
| Uplink counter synchronisation info | OP |  |  |  |  |
| >RB with PDCP information list | OP | 1 to <maxRBallRABs> |  |  |  |
| >>RB with PDCP information | MP |  | RB with PDCP information 10.3.4.22 |  |  |
| >START list | MP | 1 to <maxCNdomains> |  | START [40] values for all CN domains. |  |
| >>CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >>START | MP |  | START 10.3.3.38 | START value to be used in this CN domain. |  |

### 10.2.29	RADIO BEARER RECONFIGURATION FAILURE

This message is sent by UE if the configuration given by UTRAN is unacceptable or if the UE failed to establish the physical channel(s).

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| Failure cause | MP |  | Failure cause and error information 10.3.3.14 |  |
| RB information elements |  |  |  |  |
| Radio bearers for which reconfiguration would have succeeded List | OP | 1 to <maxRB> |  |  |
| >Radio bearer for which reconfiguration would have succeeded | MP |  | RB identity, 10.3.4.16 |  |

### 10.2.30	RADIO BEARER RELEASE

This message is used by UTRAN to release a radio bearer. It can also include modifications to the configurations of transport channels and/or physical channels. It can simultaneously indicate release of a signalling connection when UE is connected to more than one CN domain.

RLC-SAP: AM or UM

Logical channel: DCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE Information Elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Integrity protection mode info | OP |  | Integrity protection mode info 10.3.3.19 | The UTRAN should not include this IE unless it is performing an SRNS relocation. |  |
| Ciphering mode info | OP |  | Ciphering mode info 10.3.3.5 | The UTRAN should not include this IE unless it is performing an SRNS relocation and a change in ciphering algorithm. |  |
| Activation time | MD |  | Activation time 10.3.3.1 | Default value is "now" |  |
| Dynamic activation time | OP |  | Dynamic activation time 10.3.3.1a |  | REL-13 |
| New U-RNTI | OP |  | U-RNTI 10.3.3.47 |  |  |
| New C-RNTI | OP |  | C-RNTI 10.3.3.8 |  |  |
| New DSCH-RNTI | OP |  | DSCH-RNTI 10.3.3.9a | Should not be set in FDD. If received the UE behaviour is unspecified. |  |
| New H-RNTI | OP |  | H-RNTI 10.3.3.14a |  | REL-5 |
| New Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-6 |
| New Secondary E-RNTI | OP |  | E-RNTI 10.3.3.10a | FDD only | REL-6 |
| RRC State Indicator | MP |  | RRC State Indicator 10.3.3.35a |  |  |
| UE Mobility State Indicator | CV-FACH_PCH |  | Enumerated (High-mobilityDetected | Absence of this IE implies that, according to [4], the UE shall consider itself being in the mobility state the UE has maintained in CELL_DCH state or being not in high mobility state after the state transition, if applicable. | REL-7 |
| UTRAN DRX cycle length coefficient | OP |  | UTRAN DRX cycle length coefficient 10.3.3.49 |  |  |
| Access Group identity | OP |  | Integer (0..15) |  | REL-12 |
| CN Information Elements |  |  |  |  |  |
| CN Information info | OP |  | CN Information info 10.3.1.3 |  |  |
| Signalling Connection release indication | OP |  | CN domain identity 10.3.1.1 |  |  |
| UTRAN mobility information elements |  |  |  |  |  |
| URA identity | OP |  | URA identity 10.3.2.6 |  |  |
| RNC support for change of UE capability | OP |  | Boolean | Should be included if the message is used to perform an SRNS relocation | REL-7 |
| RB Information Elements |  |  |  |  |  |
| RAB information to reconfigure list | OP | 1 to < maxRABsetup > |  |  |  |
| >RAB information to reconfigure | MP |  | RAB information to reconfigure 10.3.4.11 |  |  |
| RB information to release list | MP | 1 to <maxRB> |  |  |  |
| >RB information to release | MP |  | RB information to release 10.3.4.19 |  |  |
| RB information to reconfigure list | OP | 1to <maxRB> |  |  | REL-6 |
| >RB information to reconfigure | MP |  | RB information to reconfigure 10.3.4.18 |  | REL-6 |
| RB information to be affected list | OP | 1 to <maxRB> |  |  |  |
| >RB information to be affected | MP |  | RB information to be affected 10.3.4.17 |  |  |
| Downlink counter synchronisation info | OP |  |  |  |  |
| >RB with PDCP information list | OP | 1 to <maxRBallRABs> |  |  |  |
| >>RB with PDCP information | MP |  | RB with PDCP information 10.3.4.22 | This IE is needed for each RB having PDCP in the case of lossless SRNS relocation |  |
|  | OP |  |  |  | REL-5 |
| >RB with PDCP context relocation info list | OP | 1 to <maxRBallRABs> |  |  | REL-5 |
| >>PDCP context relocation info | MP |  | PDCP context relocation info 10.3.4.1a | This IE is needed for each RB having PDCP and performing PDCP context relocation | REL-5 |
| Retrievable configuration info | OP |  | Retrievable configuration info 10.3.8.25 |  | REL-13 |
| TrCH Information Elements |  |  |  |  |  |
| Uplink transport channels |  |  |  |  |  |
| UL Transport channel information common for all transport channels | OP |  | UL Transport channel information common for all transport channels 10.3.5.24 |  |  |
| Deleted TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >Deleted UL TrCH information | MP |  | Deleted UL TrCH information 10.3.5.5 |  |  |
| Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >Added or Reconfigured UL TrCH information | MP |  | Added or Reconfigured UL TrCH information 10.3.5.2 |  |  |
| Downlink transport channels |  |  |  |  |  |
| DL Transport channel information common for all transport channels | OP |  | DL Transport channel information common for all transport channels 10.3.5.6 |  |  |
| Deleted TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >Deleted DL TrCH information | MP |  | Deleted DL TrCH information 10.3.5.4 |  |  |
| Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >Added or Reconfigured DL TrCH information | MP |  | Added or Reconfigured DL TrCH information 10.3.5.1 |  |  |
| PhyCH information elements |  |  |  |  |  |
| Frequency info | OP |  | Frequency info 10.3.6.36 |  |  |
| Multi-frequency Info | OP |  | Multi-frequency Info 10.3.6.39a | This IE is used for 1.28 Mcps TDD only | REL-7 |
| DTX-DRX timing information | OP |  | DTX-DRX timing information 10.3.6.34b |  | REL-7 |
| DTX-DRX Information | OP |  | DTX-DRX Information 10.3.6.34a |  | REL-7 |
| HS-SCCH less Information | OP |  | HS-SCCH less Information 10.3.6.36ab |  | REL-7 |
| MIMO parameters | OP |  | MIMO parameters 10.3.6.41a |  | REL-7 |
| MIMO mode with four transmit antennas parameters | OP |  | MIMO mode with four transmit antennas parameters 10.3.6.142 |  | REL-11 |
| DCH Enhancements info FDD | OP |  | DCH Enhancements info FDD 10.3.6.149 |  | REL-12 |
| Control Channel DRX information | OP |  | Control Channel DRX information 1.28 Mcps TDD 10.3.6.107 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| SPS Information | OP |  | SPS information 1.28 Mcps TDD 10.3.6.110 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| MU-MIMO info | OP |  | MU-MIMO info 1.28 Mcps TDD 10.3.6.122 | This IE is used for 1.28 Mcps TDD only | REL-10 |
| Non-rectangular resource allocation indicator | OP |  | Enumerated(TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the non-rectangular resource allocation is not used. | REL-12 |
| Non-rectangular resource specific timeslots set | CV-Non-rectangularResourceAllocation |  | Bit string (7) | For 1.28 Mcps TDD only. The value 1 of a bit indicates the corresponding timeslot in which the channelization codes are assigned by physical control channel. The value 0 of a bit indicates the corresponding timeslot in which all channelization codes are assigned when the timeslot is scheduled to the UE. The first/leftmost bit of the bit string is for TS0. | REL-12 |
| Uplink radio resources |  |  |  |  |  |
| Maximum allowed UL TX power | MD |  | Maximum allowed UL TX power 10.3.6.39 | Default value is the existing maximum UL TX power |  |
| Uplink DPCH info | OP |  | Uplink DPCH info 10.3.6.88 |  |  |
| E-DCH Info | OP |  | E-DCH Info 10.3.6.97 |  | REL-6 |
| Uplink secondary cell info FDD | OP |  | Uplink secondary cell info FDD 10.3.6.115 | FDD only | REL-9 |
| Multi-carrier E-DCH Info for LCR TDD | OP |  | Multi-carrier E-DCH Info for LCR TDD 10.3.6.97a | 1.28Mcps TDD only | REL-10 |
| Uplink CLTD info FDD | OP |  | Uplink CLTD info FDD 10.3.6.125 | FDD only | REL-11 |
| Uplink OLTD info FDD | OP |  | Uplink OLTD info FDD 10.3.6.126 | FDD only | REL-11 |
| Other TTI E-DCH Configuration Information | OP |  | Other TTI E-DCH Configuration Information 10.3.6.152 | FDD only | REL-12 |
| Filtered UE power headroom reporting information | OP |  | Filtered UE power headroom reporting information 10.3.7.140 | FDD only | REL-12 |
| Downlink radio resources |  |  |  |  |  |
| Downlink HS-PDSCH Information | OP |  | Downlink HS-PDSCH Information 10.3.6.23a |  | REL-5 |
| Downlink information common for all radio links | OP |  | Downlink information common for all radio links 10.3.6.24 |  |  |
| Downlink information per radio link list | OP | 1 to <maxRL> |  | Send downlink information for each radio link to be set-up |  |
| >Downlink information for each radio link | MP |  | Downlink information for each radio link 10.3.6.27 |  |  |
| Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-8 |
| Additional downlink secondary cell info list FDD | OP | 2 |  |  | REL-10 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-10 |
| Additional downlink secondary cell info list FDD 2 | OP | 4 |  |  | REL-11 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-11 |
| Common E-RGCH info FDD | OP |  | Common E-RGCH info FDD 10.3.6.138 | FDD only | REL-11 |
| MBMS PL Service Restriction Information | OP |  | Enumerated (TRUE) |  | REL-6 |
| MBMS RB list released to change transfer mode | OP | 1 to <maxRB> |  |  | REL-6 |
| >RB information to release | MP |  | RB information to release 10.3.4.19 |  | REL-6 |
| CELL_DCH measurement occasion info LCR | OP |  | CELL_DCH measurement occasion info LCR 10.3.7.126 |  | REL-9 |

| Condition | Explanation |
| --- | --- |
| FACH_PCH | This IE is mandatory default when a transition from CELL_DCH to CELL_FACH, URA_PCH or CELL_PCH is requested by the message and is not needed otherwise. |
| Non-rectangularResourceAllocation | This IE is optionally present if non-rectangular resource allocation is configured. Otherwise, it is not needed. |

### 10.2.31	RADIO BEARER RELEASE COMPLETE

This message is sent from the UE when radio bearer release has been completed.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE  UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 | Integrity check info is included if integrity protection is applied |  |
| Uplink integrity protection activation info | OP |  | Integrity protection activation info 10.3.3.17 |  |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  | (no data) |  |
| >TDD |  |  |  |  |  |
| >>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>Uplink Timing Advance | OP |  | Uplink Timing Advance 10.3.6.95 | This information element shall be present in case of handover procedure if timing advance is enabled. Calculated timing advance value for the new cell after handover in a synchronous TDD network |  |
| >>>>Extended Uplink Timing Advance | OP |  | Extended Uplink Timing Advance 10.3.6.95a |  | REL-7 |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Extended Uplink Timing Advance | OP |  | Extended Uplink Timing Advance 10.3.6.95a | This information element shall be present in case of handover procedure if timing advance is enabled. Calculated timing advance value for the new cell after handover in a synchronous TDD network | REL-7 |
| >>>1.28 Mcps TDD |  |  |  | (no data) | REL-4 |
| Other Information elements |  |  |  |  | REL-7 |
| Deferred measurement control reading | OP |  | Enumerated (TRUE) | Indicates the UE has not read SIB11, SIB11bis, SIB11ter (if supported) and SIB12, if available. | REL-7 |
| RB Information elements |  |  |  |  |  |
| COUNT-C activation time | OP |  | Activation time 10.3.3.1 | Used for radio bearers mapped on RLC-TM. |  |
| Uplink counter synchronisation info | OP |  |  |  |  |
| >RB with PDCP information list | OP | 1 to <maxRBallRABs> |  | This IE is needed for each RB having PDCP in the case of lossless SRNS relocation |  |
| >>RB with PDCP information | MP |  | RB with PDCP information 10.3.4.22 |  |  |
| >START list | MP | 1 to <maxCNdomains> |  | START [40] values for all CN domains. |  |
| >>CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >>START | MP |  | START 10.3.3.38 | START value to be used in this CN domain. |  |

### 10.2.32	RADIO BEARER RELEASE FAILURE

This message is sent by UE if the configuration given by UTRAN is unacceptable or if radio bearer cannot be released.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| Failure cause | MP |  | Failure cause and error information 10.3.3.14 |  |
| RB information elements |  |  |  |  |
| Radio bearers for which reconfiguration would have succeeded | OP | 1 to <maxRB> |  |  |
| >Radio bearer for which reconfiguration would have been succeeded | MP |  | RB identity, 10.3.4.16 |  |

### 10.2.33	RADIO BEARER SETUP

This message is sent by UTRAN to the UE to establish new radio bearer(s). It can also include modifications to the configurations of transport channels and/or physical channels.

RLC-SAP: AM or UM

Logical channel: DCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE Information Elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Integrity protection mode info | OP |  | Integrity protection mode info 10.3.3.19 | The UTRAN should not include this IE unless it is performing an SRNS relocation or an SR-VCC |  |
| Ciphering mode info | OP |  | Ciphering mode info 10.3.3.5 | The UTRAN should not include this IE unless it is performing an SRNS relocation and a change in ciphering algorithm or it is performing an SR-VCC |  |
| SR-VCC info | CV-SRVCC |  | SR-VCC info 10.3.4.24a |  | REL-8 |
| Activation time | MD |  | Activation time 10.3.3.1 | Default value is "now" |  |
| Dynamic activation time | OP |  | Dynamic activation time 10.3.3.1a |  | REL-13 |
| New U-RNTI | OP |  | U-RNTI 10.3.3.47 |  |  |
| New C-RNTI | OP |  | C-RNTI 10.3.3.8 |  |  |
| New DSCH-RNTI | OP |  | DSCH-RNTI 10.3.3.9a | Should not be set in FDD. If received the UE behaviour is unspecified. |  |
| New H-RNTI | OP |  | H-RNTI 10.3.3.14a |  | REL-5 |
| New Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-6 |
| New Secondary E-RNTI | OP |  | E-RNTI 10.3.3.10a | FDD only | REL-6 |
| RRC State Indicator | MP |  | RRC State Indicator 10.3.3.35a |  |  |
| UTRAN DRX cycle length coefficient | OP |  | UTRAN DRX cycle length coefficient 10.3.3.49 |  |  |
| Access Group identity | OP |  | Integer (0..15) |  | REL-12 |
| CN Information Elements |  |  |  |  |  |
| CN Information info | OP |  | CN Information info 10.3.1.3 |  |  |
| UTRAN mobility information elements |  |  |  |  |  |
| URA identity | OP |  | URA identity 10.3.2.6 |  |  |
| RNC support for change of UE capability | OP |  | Boolean | Should be included if the message is used to perform an SRNS relocation | REL-7 |
| RB Information Elements |  |  |  |  |  |
| CHOICE specification mode | MP |  |  |  | REL-6 |
| >Complete specification |  |  |  | In this version of the specification, only this value is specified | REL-6 |
| >>Signalling RB information to setup list | OP | 1 to <maxSRBsetup> |  | For each signalling radio bearer established |  |
| >>>Signalling RB information to setup | MP |  | Signalling RB information to setup 10.3.4.24 |  |  |
| >>RAB information to setup list | OP | 1 to <maxRABsetup> |  | For each RAB established |  |
| >>>RAB information for setup | MP |  | RAB information for setup 10.3.4.10 |  |  |
| >>RAB information to reconfigure list | OP | 1 to < maxRABsetup > |  |  | REL-6 |
| >>>RAB information to reconfigure | MP |  | RAB information to reconfigure 10.3.4.11 |  | REL-6 |
| >>RB information to reconfigure list | OP | 1to <maxRB> |  |  | REL-6 |
| >>>RB information to reconfigure | MP |  | RB information to reconfigure 10.3.4.18 |  | REL-6 |
| >>RB information to be affected list | OP | 1 to <maxRB> |  |  |  |
| >>>RB information to be affected | MP |  | RB information to be affected 10.3.4.17 |  |  |
| >>Downlink counter synchronisation info | OP |  |  |  |  |
| >>>RB with PDCP information list | OP | 1 to <maxRBallRABs> |  |  |  |
| >>>>RB with PDCP information | MP |  | RB with PDCP information 10.3.4.22 | This IE is needed for each RB having PDCP in the case of lossless SRNS relocation |  |
|  | OP |  |  |  | REL-5 |
| >>>>PDCP context relocation info | OP |  | PDCP context relocation info 10.3.4.1a | This IE is needed for each RB having PDCP and performing PDCP context relocation | REL-5 |
| >>PDCP ROHC target mode | OP |  | PDCP ROHC target mode 10.3.4.2a |  | REL-5 |
| TrCH Information Elements |  |  |  |  |  |
| Uplink transport channels |  |  |  |  |  |
| >>UL Transport channel information common for all transport channels | OP |  | UL Transport channel information common for all transport channels 10.3.5.24 |  |  |
| >>Deleted TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >>>Deleted UL TrCH information | MP |  | Deleted UL TrCH information 10.3.5.5 |  |  |
| >>Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >>>Added or Reconfigured UL TrCH information | MP |  | Added or Reconfigured UL TrCH information 10.3.5.2 |  |  |
| Downlink transport channels |  |  |  |  |  |
| >>DL Transport channel information common for all transport channels | OP |  | DL Transport channel information common for all transport channels 10.3.5.6 |  |  |
| >>Deleted TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >>>Deleted DL TrCH information | MP |  | Deleted DL TrCH information 10.3.5.4 |  |  |
| >>Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >>>Added or Reconfigured DL TrCH information | MP |  | Added or Reconfigured DL TrCH information 10.3.5.1 |  |  |
| Retrievable configuration |  |  |  |  |  |
| >>Retrievable configuration info | OP |  | Retrievable configuration info 10.3.8.25 |  | REL-13 |
| PhyCH information elements |  |  |  |  |  |
| Frequency info | OP |  | Frequency info 10.3.6.36 |  |  |
| Multi-frequency Info | OP |  | Multi-frequency Info 10.3.6.39a | This IE is used for 1.28 Mcps TDD only | REL-7 |
| DTX-DRX timing information | OP |  | DTX-DRX timing information 10.3.6.34b |  | REL-7 |
| DTX-DRX Information | OP |  | DTX-DRX Information 10.3.6.34a |  | REL-7 |
| HS-SCCH less Information | OP |  | HS-SCCH less Information 10.3.6.36ab |  | REL-7 |
| MIMO parameters | OP |  | MIMO parameters 10.3.6.41a |  | REL-7 |
| MIMO mode with four transmit antennas parameters | OP |  | MIMO mode with four transmit antennas parameters 10.3.6.142 |  | REL-11 |
| DCH Enhancements info FDD | OP |  | DCH Enhancements info FDD 10.3.6.149 |  | REL-12 |
| Control Channel DRX information | OP |  | Control Channel DRX information 1.28 Mcps TDD 10.3.6.107 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| SPS Information | OP |  | SPS information 1.28 Mcps TDD 10.3.6.110 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| MU-MIMO info | OP |  | MU-MIMO info 1.28 Mcps TDD 10.3.6.122 | This IE is used for 1.28 Mcps TDD only | REL-10 |
| Non-rectangular resource allocation indicator | OP |  | Enumerated(TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the non-rectangular resource allocation is not used. | REL-12 |
| Non-rectangular resource specific timeslots set | CV-Non-rectangularResourceAllocation |  | Bit string (7) | For 1.28 Mcps TDD only. The value 1 of a bit indicates the corresponding timeslot in which the channelization codes are assigned by physical control channel. The value 0 of a bit indicates the corresponding timeslot in which all channelization codes are assigned when the timeslot is scheduled to the UE. The first/leftmost bit of the bit string is for TS0. | REL-12 |
| Uplink radio resources |  |  |  |  |  |
| Maximum allowed UL TX power | MD |  | Maximum allowed UL TX power 10.3.6.39 | Default value is the existing maximum UL TX power |  |
| Uplink DPCH info | OP |  | Uplink DPCH info 10.3.6.88 |  |  |
| E-DCH Info | OP |  | E-DCH Info 10.3.6.97 |  | REL-6 |
| Uplink secondary cell info FDD | OP |  | Uplink secondary cell info FDD 10.3.6.115 | FDD only | REL-9 |
| Multi-carrier E-DCH Info for LCR TDD | OP |  | Multi-carrier E-DCH Info for LCR TDD 10.3.6.97a | 1.28Mcps TDD only | REL-10 |
| Uplink CLTD info FDD | OP |  | Uplink CLTD info FDD 10.3.6.125 | FDD only | REL-11 |
| Uplink OLTD info FDD | OP |  | Uplink OLTD info FDD 10.3.6.126 | FDD only | REL-11 |
| Other TTI E-DCH Configuration Information | OP |  | Other TTI E-DCH Configuration Information 10.3.6.152 | FDD only | REL-12 |
| Filtered UE power headroom reporting information | OP |  | Filtered UE power headroom reporting information 10.3.7.140 | FDD only | REL-12 |
| Downlink radio resources |  |  |  |  |  |
| Downlink HS-PDSCH Information | OP |  | Downlink HS-PDSCH Information 10.3.6.23a |  | REL-5 |
| Downlink information common for all radio links | OP |  | Downlink information common for all radio links 10.3.6.24 |  |  |
| Downlink information per radio link list | OP | 1 to <maxRL> |  | Send downlink information for each radio link |  |
| >Downlink information for each radio link | MP |  | Downlink information for each radio link 10.3.6.27 |  |  |
| Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-8 |
| Additional downlink secondary cell info list FDD | OP | 2 |  |  | REL-10 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-10 |
| Additional downlink secondary cell info list FDD 2 | OP | 4 |  |  | REL-11 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-11 |
| Common E-RGCH info FDD | OP |  | Common E-RGCH info FDD 10.3.6.138 | FDD only | REL-11 |
| MBMS PL Service Restriction Information | OP |  | Enumerated (TRUE) |  | REL-6 |
| CELL_DCH measurement occasion info LCR | OP |  | CELL_DCH measurement occasion info LCR 10.3.7.126 |  | REL-9 |

| Condition | Explanation |
| --- | --- |
| SRVCC | This IE is mandatory if an SR-VCC procedure is initiated and not needed otherwise. |
| Non-rectangularResourceAllocation | This IE is optionally present if non-rectangular resource allocation is configured. Otherwise, it is not needed. |

### 10.2.34	RADIO BEARER SETUP COMPLETE

This message is sent by the UE to confirm the establishment of the radio bearer.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE  UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Uplink integrity protection activation info | OP |  | Integrity protection activation info 10.3.3.17 |  |  |
| CHOICE mode | OP |  |  |  |  |
| >FDD |  |  |  | (no data) |  |
| >TDD |  |  |  |  |  |
| >>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>Uplink Timing Advance | OP |  | Uplink Timing Advance 10.3.6.95 | This information element shall be present in case of handover procedure if timing advance is enabled. Calculated timing advance value for the new cell after handover in a synchronous TDD network |  |
| >>>>Extended Uplink Timing Advance | OP |  | Extended Uplink Timing Advance 10.3.6.95a |  | REL-7 |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Extended Uplink Timing Advance | OP |  | Extended Uplink Timing Advance 10.3.6.95a | This information element shall be present in case of handover procedure if timing advance is enabled. Calculated timing advance value for the new cell after handover in a synchronous TDD network | REL-7 |
| >>>1.28 Mcps TDD |  |  |  | (No data) | REL-4 |
| START | OP |  | START 10.3.3.38 | This information element is not needed for transparent mode RBs if prior to this procedure there exists one RB using RLC-TM. |  |
| Other information elements |  |  |  |  | REL-7 |
| Deferred measurement control reading | OP |  | Enumerated (TRUE) | Indicates the UE has not read SIB11, SIB11bis, SIB11ter (if supported) and SIB12, if available. | REL-7 |
| RB Information elements |  |  |  |  |  |
| COUNT-C activation time | OP |  | Activation time 10.3.3.1 | Used for radio bearers mapped on RLC-TM. |  |
| Uplink counter synchronisation info | OP |  |  |  |  |
| >RB with PDCP information list | OP | 1 to <maxRBallRABs> |  | This IE is needed for each RB having PDCP in the case of lossless SRNS relocation |  |
| >>RB with PDCP information | MP |  | RB with PDCP information 10.3.4.22 |  |  |
| >START list | MP | 1 to <maxCNdomains> |  | START [40] values for all CN domains. |  |
| >>CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >>START | MP |  | START 10.3.3.38 | START value to be used in this CN domain. |  |

### 10.2.35	RADIO BEARER SETUP FAILURE

This message is sent by UE, if it does not support the configuration given by UTRAN.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| Failure cause | MP |  | Failure cause and error information 10.3.3.14 |  |
| RB information elements |  |  |  |  |
| Radio bearers for which reconfiguration would have succeeded | OP | 1 to <maxRB> |  |  |
| >Radio bearer for which reconfiguration would have succeeded | MP |  | RB identity, 10.3.4.16 |  |

### 10.2.36	RRC CONNECTION REJECT

The network transmits this message when the requested RRC connection cannot be accepted.

RLC-SAP: UM

Logical channel: CCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Initial UE identity | MP |  | Initial UE identity 10.3.3.15 |  |  |
| Rejection cause | MP |  | Rejection cause 10.3.3.31 |  |  |
| Wait time | MP |  | Wait time 10.3.3.50 |  |  |
| Redirection info | OP |  | Redirection info 10.3.3.29 | The UE shall ignore any unsupported frequencies included in this IE. |  |
| Counting completion | OP |  | Enumerated (TRUE) | This field may be present if the Rejection Cause is set to “unspecified” otherwise it shall be ignored. | REL-6 |
| Extended Wait Time | OP |  | Extended Wait Time 10.3.3.12a |  | REL-10 |

### 10.2.37	RRC CONNECTION RELEASE

This message is sent by UTRAN to release the RRC connection. The message also releases the signalling connection and all radio bearers between the UE and UTRAN.

RLC-SAP: UM

Logical channel: CCCH or DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| CHOICE identity type | CV-CCCH |  |  |  | REL-5 |
| >U-RNTI |  |  | U-RNTI 10.3.3.47 |  |  |
| > Group identity |  | 1 to <maxURNTIgroup> |  |  | REL-5 |
| >>Group release information | MP |  | Group release information 10.3.3.14o |  | REL-5 |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CV-DCCH |  | Integrity check info 10.3.3.16 | Integrity check info is included if integrity protection is applied |  |
| N308 | CH-Cell_DCH |  | Integer(1..8) |  |  |
| Release cause | MP |  | Release cause 10.3.3.32 |  |  |
| UE Mobility State Indicator | CV-DCCH_MD |  | Enumerated (High-mobilityDetected) | Absence of this IE implies that, according to [4] the UE shall consider itself being in the mobility state the UE has maintained in CELL_DCH state or being not in high-mobility state when entering in Idle Mode, if applicable. | REL-7 |
| Extended Wait Time | OP |  | Extended Wait Time 10.3.3.12a |  | REL-10 |
| Other information elements |  |  |  |  |  |
| Rplmn information | OP |  | Rplmn information 10.3.8.15 |  |  |
| Redirection info | OP |  | Redirection info 10.3.3.29 |  | REL-6 |

| Condition | Explanation |
| --- | --- |
| CCCH | This IE is mandatory present when CCCH is used and not needed otherwise. |
| DCCH | This IE is mandatory present when DCCH is used and not needed otherwise. |
| DCCH_MD | This IE is mandatory default when DCCH is used and not needed otherwise. |
| Cell_DCH | This IE is mandatory present when UE is in CELL_DCH state and not needed otherwise. |

### 10.2.38	RRC CONNECTION RELEASE COMPLETE

This message is sent by UE to confirm that the RRC connection has been released.

RLC-SAP: AM or UM

Logical channel: DCCH

Direction: UE  UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| Error indication | OP |  | Failure cause and error information 10.3.3.14 |  |

### 10.2.39	RRC CONNECTION REQUEST

RRC Connection Request is the first message transmitted by the UE when setting up an RRC Connection to the network.

RLC-SAP: TM

Logical channel: CCCH

Direction: UE  UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| Radio Bearer IEs |  |  |  |  |  |
| Predefined configuration status information | MP |  | Boolean | TRUE indicates the UE has all pre- configurations stored with the same value tag as broadcast in the cell in which the RRC connection establishment is initiated | REL-5 |
| UE information elements |  |  |  |  |  |
| Initial UE identity | MP |  | Initial UE identity 10.3.3.15 |  |  |
| Establishment cause | MP |  | Establishment cause 10.3.3.11 |  |  |
| Protocol error indicator | MD |  | Protocol error indicator 10.3.3.27 | Default value is FALSE |  |
| >UE Specific Behaviour Information 1 idle | OP |  | UE Specific Behaviour Information 1 idle 10.3.3.51 | This IE shall not be included in this version of the protocol |  |
| Domain indicator | MP |  | CN domain identity 10.3.1.1 |  | REL-6 |
| CS Call type | CV-CS-Domain |  | Enumerated (speech, video, other) | One spare value is needed | REL-6 |
| UE capability indication | OP |  | Enumerated (HS-DSCH, HS-DSCH+E-DCH) | Absence of this IE implies that neither HS-DSCH nor E-DCH are supported by the UE | REL-6 |
| MBMS Selected Services | OP |  | MBMS Selected Services Short 10.3.9a.7d |  | REL-6 |
| UE Mobility State Indicator | MD |  | Enumerated (High-mobilityDetected) | Absence of this IE implies that, according to [4] either the High mobility state is not applicable or it has not been detected by the UE. | REL-7 |
| Support for F-DPCH | CV-SupportForHS |  | Enumerated (TRUE) | Presence of this information element indicates that F-DPCH is supported by the UE. Absence of this information element indicates that F-DPCH is not supported | REL-6 |
| Support for Enhanced F-DPCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support enhanced F-DPCH | REL-7 |
| HS-PDSCH in CELL_FACH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-PDSCH reception in CELL_FACH state. Note 1 | REL-7 |
| MAC-ehs support | OP |  | Enumerated (TRUE) | The presence of this IE indicates that UE supports MAC-ehs, octet aligned transport block table, the use of special value of HE field to indicate end of an SDU for RLC AM and different HS-SCCHs in contiguous TTIs. The absence of this IE indicates that the UE does not support either MAC-ehs, octet aligned transport block table or the use of special value of HE field to indicate end of an SDU for RLC AM or different HS-SCCHs in contiguous TTIs. | REL-7 |
| DPCCH Discontinuous Transmission support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support DPCCH Discontinuous Transmission | REL-7 |
| Support of common E-DCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support E-DCH enhanced random access in CELL_FACH state and Idle mode. Note 1 | REL-8 |
| Multi cell support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support dual cell operations on adjacent frequencies | REL-8 |
| Dual cell MIMO support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support dual cell with MIMO operation on adjacent frequencies. | REL-9 |
| More than two cell or MIMO mode with four transmit antennas support | OP |  | Enumerated (higher rate, lower rate) | For FDD, the absence of this IE indicates that the UE does not support multi-cell operation on more than two cells. For 1.28 Mcps TDD, the absence of this IE indicates that the UE does not support MU-MIMO. If present, higher rate indicates uplink and downlink MU-MIMO, lower rate indicates uplink MU-MIMO | REL-10 |
|  |  |  |  | For FDD, the absence of this IE indicates that the UE does not support multi-cell operation on more than two cells or MIMO mode with four transmit antennas. For 1.28 Mcps TDD, the absence of this IE indicates that the UE does not support MU-MIMO. If present, higher rate indicates uplink and downlink MU-MIMO, lower rate indicates uplink MU-MIMO | REL-11 |
| Pre-redirection info | OP |  | Pre-redirection info 10.3.3.25a | The presence of this IE indicates the UE support of radio access technologies that the UE could be directed to, and that the UE was not redirected from EUTRA. | REL-8 |
| Support of MAC-i/is | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support MAC-i/is operation. | REL-8 |
| Support of SPS operation | OP |  | Enumerated (TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the UE does not support SPS operation. | REL-8 |
| Support for CS Voice over HSPA | OP |  | Enumerated (TRUE) | The IE indicates the UE’s support for CS Voice over HSPA, if set. | REL-8 |
| System Information Container Stored Indicator | OP |  | Enumerated (TRUE) | The presence of this IE indicates that the UE is accessing the cell using System Information stored in the variable SYSTEM_INFORMATION_CONTAINER | REL-9 |
| Support of the first Frequency Band | OP |  | Enumerated (TRUE) | The presence of this IE indicates the UE supports the 1st band broadcased in SIB5/5bis. For 1.28 Mcps TDD, the IE also indicates support band e if no band is broadcasted in SIB5/5bis. | REL-10 |
| Support of the second Frequency Band | OP |  | Enumerated (TRUE) | The presence of this IE indicates the UE supports the 2nd band broadcased in SIB5/5bis. For 1.28 Mcps TDD, the IE also indicates support band f if no band is broadcasted in SIB5/5bis. | REL-10 |
| CSFB Indication | OP |  | Enumerated (TRUE) | The presence of this IE indicates that the RRC Connection Request is due to CSFB call initiated in E-UTRA. |  |
| Measurement information elements |  |  |  |  |  |
| Measured results on RACH | OP |  | Measured results on RACH 10.3.7.45 |  |  |
| Access stratum release indicator | MP |  | Enumerated(REL-4, | Absence of the IE implies R99. The IE also indicates the release of the RRC transfer syntax supported by the UE. 6 spare values are needed. | REL-4 |
|  |  |  | REL-5, |  | REL-5 |
|  |  |  | REL-6, |  | REL-6 |
|  |  |  | REL-7, |  | REL-7 |
|  |  |  | REL-8, |  | REL-8 |
|  |  |  | REL-9, |  | REL-9 |
|  |  |  | REL-10, |  | REL-10 |
|  |  |  | REL-11, |  | REL-11 |
|  |  |  | REL-12, |  | REL-12 |
|  |  |  | REL-13) |  | REL-13 |

NOTE 1:	For 1.28 Mcps TDD, UE supporting HS-PDSCH in CELL_FACH always supports E-DCH enhanced random access in CELL_FACH state and Idle mode, and vice versa.

| Condition | Explanation |
| --- | --- |
| CS-Domain | This IE is mandatory present if the IE "Domain indicator" has the value "CS domain". Otherwise it is not needed. |
| SupportForHS | This IE is mandatory present if the UE supports HS-PDSCH, otherwise it is not needed. |

If the encoded message does not fill a transport block, the RRC layer shall insert padding according to subclause 12.1.

### 10.2.40	RRC CONNECTION SETUP

This message is used by the network to accept the establishment of an RRC connection for a UE, including assignment of signalling link information, transport channel information and optionally physical channel information.

RLC-SAP: UM

Logical channel: CCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE Information Elements |  |  |  |  |  |
| Initial UE identity | MP |  | Initial UE identity 10.3.3.15 |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Activation time | MD |  | Activation time 10.3.3.1 | Default value is "now" |  |
| New U-RNTI | MP |  | U-RNTI 10.3.3.47 |  |  |
| New C-RNTI | OP |  | C-RNTI 10.3.3.8 |  |  |
| New H-RNTI | OP |  | H-RNTI 10.3.3.14a |  | REL-6 |
| New Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-6 |
| New Secondary E-RNTI | OP |  | E-RNTI 10.3.3.10a | FDD only | REL-6 |
| RRC State Indicator | MP |  | RRC State Indicator 10.3.3.35a |  |  |
| UTRAN DRX cycle length coefficient | MP |  | UTRAN DRX cycle length coefficient 10.3.3.49 |  |  |
| Capability update requirement | MD |  | Capability update requirement 10.3.3.2 | Default value is defined in subclause 10.3.3.2 |  |
| RNC support for change of UE capability | MP |  | Boolean |  | REL-7 |
| Specification mode information elements |  |  |  |  | REL-8 |
| Default configuration for CELL_FACH | OP |  | Default configuration for CELL_FACH 10.3.4.0a |  | REL-8 |
| CHOICE specification mode | MP |  |  |  | REL-5 |
| >Complete specification |  |  |  |  |  |
| RB Information Elements |  |  |  |  |  |
| >>Signalling RB information to setup list | MP | 3 to 4 |  |  |  |
| >>>Signalling RB information to setup | MP |  | Signalling RB information to setup 10.3.4.24 |  |  |
| TrCH Information Elements |  |  |  |  |  |
| Uplink transport channels |  |  |  |  |  |
| >>UL Transport channel information common for all transport channels | OP |  | UL Transport channel information common for all transport channels 10.3.5.24 |  |  |
| >>Added or Reconfigured TrCH information list | MP | 1 to <maxTrCH> |  | Although this IE is not required when the IE "RRC state indicator" is set to "CELL_FACH", need is MP to align with ASN.1 |  |
|  | OP |  |  |  | REL-4 |
| >>>Added or Reconfigured UL TrCH information | MP |  | Added or Reconfigured UL TrCH information 10.3.5.2 |  |  |
| Downlink transport channels |  |  |  |  |  |
| >>DL Transport channel information common for all transport channels | OP |  | DL Transport channel information common for all transport channels 10.3.5.6 |  |  |
| >>Added or Reconfigured TrCH information list | MP | 1 to <maxTrCH> |  | Although this IE is not required when the IE "RRC state indicator" is set to "CELL_FACH", need is MP to align with ASN.1 |  |
|  | OP |  |  |  | REL-4 |
| >>>Added or Reconfigured DL TrCH information | MP |  | Added or Reconfigured DL TrCH information 10.3.5.1 |  |  |
| >Preconfiguration |  |  |  |  | REL-5 |
| >>CHOICE Preconfiguration mode | MP |  |  |  | REL-5 |
| >>>Predefined configuration identity | MP |  | Predefined configuration identity 10.3.4.5 |  | REL-5 |
| >>>Default configuration |  |  |  |  | REL-5 |
| >>>>Default configuration mode | MP |  | Enumerated (FDD, TDD) | Indicates whether the FDD or TDD version of the default configuration shall be used | REL-5 |
| >>>>Default configuration identity | MP |  | Default configuration identity 10.3.4.0 |  | REL-5 |
| PhyCH information elements |  |  |  |  |  |
| Frequency info | OP |  | Frequency info 10.3.6.36 |  |  |
| Multi-frequency Info | OP |  | Multi-frequency Info 10.3.6.39a | This IE is used for 1.28 Mcps TDD only | REL-7 |
| DTX-DRX timing information | OP |  | DTX-DRX timing information 10.3.6.34b |  | REL-7 |
| DTX-DRX Information | OP |  | DTX-DRX Information 10.3.6.34a |  | REL-7 |
| HS-SCCH less Information | OP |  | HS-SCCH less Information 10.3.6.36ab |  | REL-7 |
| SPS Information | OP |  | SPS information 1.28 Mcps TDD 10.3.6.110 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| MU-MIMO info | OP |  | MU-MIMO info 1.28 Mcps TDD 10.3.6.122 | This IE is used for 1.28 Mcps TDD only | REL-10 |
| Non-rectangular resource allocation indicator | OP |  | Enumerated(TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the non-rectangular resource allocation is not used. | REL-12 |
| Non-rectangular resource specific timeslots set | CV-Non-rectangularResourceAllocation |  | Bit string (7) | For 1.28 Mcps TDD only. The value 1 of a bit indicates the corresponding timeslot in which the channelization codes are assigned by physical control channel. The value 0 of a bit indicates the corresponding timeslot in which all channelization codes are assigned when the timeslot is scheduled to the UE. The first/leftmost bit of the bit string is for TS0. | REL-12 |
| Uplink radio resources |  |  |  |  |  |
| Maximum allowed UL TX power | MD |  | Maximum allowed UL TX power 10.3.6.39 | Default value is the existing maximum UL TX power |  |
| Uplink DPCH info | OP |  | Uplink DPCH info 10.3.6.88 |  |  |
| E-DCH Info | OP |  | E-DCH Info 10.3.6.97 |  | REL-6 |
| Downlink radio resources |  |  |  |  |  |
| Downlink HS-PDSCH Information | OP |  | Downlink HS-PDSCH information 10.3.6.23a |  | REL-6 |
| Downlink information common for all radio links | OP |  | Downlink information common for all radio links 10.3.6.24 |  |  |
| Downlink information per radio link list | OP | 1 to <maxRL> |  | Send downlink information for each radio link to be set-up |  |
| >Downlink information for each radio link | MP |  | Downlink information for each radio link 10.3.6.27 |  |  |
| Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-8 |
| Additional downlink secondary cell info list FDD | OP | 2 |  |  | REL-10 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-10 |
| Common E-RGCH info FDD | OP |  | Common E-RGCH info FDD 10.3.6.138 | FDD only | REL-11 |

| Condition | Explanation |
| --- | --- |
| Non-rectangularResourceAllocation | This IE is optionally present if non-rectangular resource allocation is configured. Otherwise, it is not needed. |

### 10.2.41	RRC CONNECTION SETUP COMPLETE

This message confirms the establishment of the RRC Connection by the UE.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE  UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE Information Elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| START list | MP | 1 to <maxCNdomains> |  | START [40] values for all CN domains. |  |
| >CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >START | MP |  | START 10.3.3.38 | START value to be used in this CN domain. |  |
| UE radio access capability | OP |  | UE radio access capability 10.3.3.42 |  |  |
| UE radio access capability extension | OP |  | UE radio access capability extension 10.3.3.42a |  |  |
| Other information elements |  |  |  |  |  |
| UE system specific capability | OP | 1 to <maxInterSysMessages> |  |  |  |
| >Inter-RAT UE radio access capability | MP |  | Inter-RAT UE radio access capability 10.3.8.7 |  |  |
| Deferred measurement control reading | OP |  | Enumerated (TRUE) | Indicates the UE has not read SIB11, SIB11bis, SIB11ter (if supported) and SIB12, if available. | REL-7 |
| Logged Meas Available | OP |  | Enumerated (TRUE) | Indicates the UE has logged measurements to report to the network | REL-10 |
| ANR Logging Results Available | OP |  | Enumerated (TRUE) | True indicates the UE has ANR logging results to report to the Network. | REL-10 |
| Connection Establishment Failure Info Available | OP |  | Enumerated (TRUE) | True indicates the UE has logged measurements from a RRC connection establishment failure to report to the network. | REL-11 |

### 10.2.41a	RRC FAILURE INFO

This message is sent by the UE via another radio access technology to provide information about the cause for failure to perform the requested operation.

RLC-SAP: N/A (Sent through a different RAT)

Logical channel: N/A (Sent through a different RAT)

Direction: UE  UTRAN

| Information Element/Group Name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other Information elements |  |  |  |  |
| Failure cause | MP |  | Failure cause 10.3.3.13 |  |
| Protocol error information | CV-ProtErr |  | Protocol error information 10.3.8.12 |  |

| Condition | Explanation |
| --- | --- |
| ProtErr | Presence is mandatory if the IE "Failure cause" has the value "Protocol error"; otherwise the element is not needed in the message. |

### 10.2.42	RRC STATUS

This message is sent to indicate a protocol error.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 | Integrity check info is included if integrity protection is applied |
| Identification of received message | CV-Message identified |  |  |  |
| >Received message type | MP |  | Message Type |  |
| >RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Other information elements |  |  |  |  |
| Protocol error information | MP |  | Protocol error information 10.3.8.12 |  |

| Condition | Explanation |
| --- | --- |
| Message identified | This IE is mandatory present if the IE "Protocol error cause" in the IE "Protocol error information" has any other value than "ASN.1 violation or encoding error" or "Message type non-existent or not implemented" and not needed otherwise. |

### 10.2.43	SECURITY MODE COMMAND

This message is sent by UTRAN to start or reconfigure ciphering and/or integrity protection parameters.

RLC-SAP: AM

Logical channel: DCCH

Direction: UTRAN to UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | MP |  | Integrity check info 10.3.3.16 |  |
| Security capability | MP |  | Security capability 10.3.3.37 |  |
| Ciphering mode info | OP |  | Ciphering mode info 10.3.3.5 | Only present if ciphering shall be controlled |
| Integrity protection mode info | OP |  | Integrity protection mode info 10.3.3.19 | Only present if integrity protection shall be controlled |
| CN Information elements |  |  |  |  |
| CN domain identity | MP |  | CN domain identity 10.3.1.1 | Indicates which ciphering and integrity protection keys are applicable |
| Other information elements |  |  |  |  |
| UE system specific security capability | CH | 1 to <maxInterSysMessages> |  | This IE is included if the IE "Inter-RAT UE radio access capability" was included in RRC CONNECTION SETUP COMPLETE message |
| >Inter-RAT UE security capability | MP |  | Inter-RAT UE security capability 10.3.8.8a |  |

### 10.2.44	SECURITY MODE COMPLETE

This message is sent by UE to confirm the reconfiguration of ciphering and/or integrity protection.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | MP |  | Integrity check info 10.3.3.16 |  |
| Uplink integrity protection activation info | OP |  | Integrity protection activation info 10.3.3.17 |  |
| RB Information elements |  |  |  |  |
| Radio bearer uplink ciphering activation time info | OP |  | RB activation time info 10.3.4.13 |  |

### 10.2.45	SECURITY MODE FAILURE

This message is sent to indicate a failure to act on a received SECURITY MODE COMMAND message.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| Failure cause | MP |  | Failure cause and error information 10.3.3.14 |  |

### 10.2.46	SIGNALLING CONNECTION RELEASE

This message is used to notify the UE that its ongoing signalling connection to a CN domain has been released.

RLC-SAP: AM

Logical channel: DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 | Integrity check info is included if integrity protection is applied |
| Extended Wait Time | OP |  | Extended Wait Time 10.3.3.12a |  |
| CN information elements |  |  |  |  |
| CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |

### 10.2.47	SIGNALLING CONNECTION RELEASE INDICATION

This message is used by the UE to indicate to UTRAN the release of an existing signalling connection.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message type |  |  |
| UE Information Elements |  |  |  |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| CN information elements |  |  |  |  |  |
| CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| Signalling Connection Release Indication Cause | MP |  | Signalling Connection Release Indication Cause 10.3.3.37a | NOTE1 | REL-8 |

NOTE1:	For the sake of backward compatibility, when the cause value is set to a value other than "UE Requested PS Data session end" according to 8.1.14.2: in this release of the specification, the UE may exclude this IE by not including the ASN.1 non-critical extension where it is referenced, alternatively, if the UE includes this IE it shall set it to "any Other Cause".

### 10.2.48	SYSTEM INFORMATION

This message is used by the UTRAN to convey system information blocks to the UE.

RLC-SAP: TM

Logical channel: BCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SFNprime | CV-channel2 |  | Integer(0..4094 by step of 2) | SFN=SFNprime (for first 10ms frame of 20ms TTI), SFN=SFNprime+1 (for last 10ms frame of 20ms TTI) |
| CHOICE Segment combination | MP |  |  | Five spares are needed |
| >Combination 1 |  |  |  | (no data) |
| >Combination 2 |  |  |  |  |
| >>First Segment | MP |  | First Segment, 10.2.48.1 |  |
| >Combination 3 |  |  |  |  |
| >>Subsequent Segment | MP |  | Subsequent Segment, 10.2.48.3 |  |
| >Combination 4 |  |  |  |  |
| >>Last segment | MP |  | Last segment (short),10.2.48.5 |  |
| >Combination 5 |  |  |  | NOTE 2 |
| >>Last segment | MP |  | Last Segment (short)10.2.48.5 |  |
| >>First Segment | MP |  | First Segment (short), 10.2.48.2 |  |
| >Combination 6 |  |  |  | NOTE 2 |
| >>Last Segment | MP |  | Last Segment (short), 10.2.48.5 |  |
| >>Complete list | MP | 1 to maxSIBperMsg |  | NOTE 1 |
| >>>Complete | MP |  | Complete SIB (short),10.2.48.7 |  |
| >Combination 7 |  |  |  | NOTE 2 |
| >>Last Segment | MP |  | Last Segment (short), 10.2.48.5 |  |
| >>Complete list | MP | 1..< maxSIBperMsg> |  | NOTE 1 |
| >>>Complete | MP |  | Complete SIB (short),10.2.48.7 |  |
| >>First Segment | MP |  | First Segment (short), 10.2.48.2 |  |
| >Combination 8 |  |  |  | NOTE 2 |
| >>Complete list | MP | 1 to maxSIBperMsg |  | NOTE 1 |
| >>>Complete | MP |  | Complete SIB (short),10.2.48.7 |  |
| >Combination 9 |  |  |  | NOTE 2 |
| >>Complete list | MP | 1..MaxSIBperMsg |  | NOTE 1 |
| >>>Complete | MP |  | Complete SIB (short),10.2.48.7 |  |
| >>First Segment | MP |  | First Segment (short), 10.2.48.2 |  |
| >Combination 10 |  |  |  |  |
| >>>Complete SIB of size 215 to 226 | MP |  | Complete SIB,10.2.48.6 |  |
| >Combination 11 |  |  |  |  |
| >>Last segment of size 215 to 222 | MP |  | Last segment,10.2.48.4 |  |
| NOTE 1: If Combination 6 - 9 contains a Master information block, the Master information block shall be located as the first IE in the list. |  |  |  |  |
| NOTE 2: If one of the combinations 5 to 9 is used, the IE "SIB type" = "Extension Type" should not occur more than once in that message, otherwise the UE behaviour is unspecified. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| channel2 | This IE is mandatory present if the channel is BCH, otherwise it is not needed. |

If the encoded message does not fill a transport block, the RRC layer shall insert padding according to subclause 12.1. Padding is needed e.g. if the remaining space is insufficient to start a new First Segment (which requires several bits for SIB type, SEG_COUNT and SIB data).

### 10.2.48a	System Information Container

This message is sent via another radio access technology to provide information to the UE.

-	RLC-SAP: N/A (Sent through a different RAT)

-	Logical channel: N/A (Sent through a different RAT)

-	Direction: A different RAT  UE

| Information Element/Group Name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MIB | MP |  | Master Information Block 10.2.48.8.1 |  | REL-9 |
| SysInfoTypeSB1 | OP |  | Scheduling Block 1 10.2.48.8.2 |  | REL-9 |
| SysInfoTypeSB2 | OP |  | Scheduling Block 2 10.2.48.8.3 |  | REL-9 |
| SysInfoType1 | MP |  | System Information Block type 1 10.2.48.8.4 |  | REL-9 |
| SysInfoType3 | MP |  | System Information Block type 3 10.2.48.8.6 |  | REL-9 |
| SysInfoType5 | MP |  | System Information Block type 5 or 5bis 10.2.48.8.8 |  | REL-9 |
| SysInfoType7 | MP |  | System Information Block type 7 10.2.48.8.10 |  | REL-9 |
| SysInfoType11 | OP |  | System Information Block type 11 10.2.48.8.14 |  | REL-9 |
| SysInfoType11bis | OP |  | System Information Block type 11bis 10.2.48.8.14a |  | REL-9 |
| SysInfoType11ter | OP |  | System Information Block type 11ter 10.2.48.8.14b |  | REL-12 |
| SysInfoType12 | OP |  | System Information Block type 12 10.2.48.8.15 |  | REL-9 |
| SysInfoType22 | OP |  | System Information Block type 22 10.2.48.8.25 |  | REL-11 |

### 10.2.48b	SYSTEM INFORMATION 2

This message is used by the UTRAN to convey system information blocks to the UE. This message is used on the second system information broadcast channel (BCH on SCCPCH).

RLC-SAP: TM

Logical channel: BCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Segment combination | MP |  |  | Six spares are needed |  |
| >Combination 2 |  |  |  |  |  |
| >>First Segment 2 | MP |  | First Segment 2 10.2.48.1a |  | REL-12 |
| >Combination 3 |  |  |  |  |  |
| >>Subsequent Segment 2 | MP |  | Subsequent Segment 2 10.2.48.3a |  | REL-12 |
| >Combination 4 |  |  |  |  |  |
| >>Last segment 2 | MP |  | Last Segment (short) 2 10.2.48.5a |  | REL-12 |
| >Combination 5 |  |  |  |  |  |
| >>Last segment 2 | MP |  | Last Segment (short) 2 10.2.48.5a |  | REL-12 |
| >>First Segment 2 | MP |  | First Segment (short) 2 10.2.48.2a |  | REL-12 |
| >Combination 6 |  |  |  |  |  |
| >>Last Segment 2 | MP |  | Last Segment (short) 2 10.2.48.5a |  | REL-12 |
| >>Complete list 2 | MP | 1 to maxSIBperMsg |  | NOTE 1 |  |
| >>>Complete 2 | MP |  | Complete SIB (short) 2 10.2.48.7a |  | REL-12 |
| >Combination 7 |  |  |  |  |  |
| >>Last Segment 2 | MP |  | Last Segment (short) 2 10.2.48.5a |  | REL-12 |
| >>Complete list 2 | MP | 1..< maxSIBperMsg> |  | NOTE 1 |  |
| >>>Complete 2 | MP |  | Complete SIB (short) 2 10.2.48.7a |  | REL-12 |
| >>First Segment 2 | MP |  | First Segment (short) 2 10.2.48.2a |  | REL-12 |
| >Combination 8 |  |  |  |  |  |
| >>Complete list 2 | MP | 1 to maxSIBperMsg |  | NOTE 1 |  |
| >>>Complete 2 | MP |  | Complete SIB (short) 2 10.2.48.7a |  | REL-12 |
| >Combination 9 |  |  |  |  |  |
| >>Complete list 2 | MP | 1..MaxSIBperMsg |  | NOTE 1 |  |
| >>>Complete 2 | MP |  | Complete SIB (short) 2 10.2.48.7a |  | REL-12 |
| >>First Segment 2 | MP |  | First Segment (short) 2 10.2.48.2a |  | REL-12 |
| >Combination 10 |  |  |  |  |  |
| >>Complete SIB of size 225 to 236 | MP |  | Complete SIB 2 10.2.48.6a |  | REL-12 |
| >Combination 11 |  |  |  |  |  |
| >>Last segment of size 225 to 232 | MP |  | Last Segment 2 10.2.48.4a |  | REL-12 |
| NOTE 1: If Combination 6 - 9 contains aScheduling Block 3, the Scheduling Block 3 shall be located as the first IE in the list. |  |  |  |  |  |

If the encoded message does not fill a transport block, the RRC layer shall insert padding according to subclause 12.1. Padding is needed e.g. if the remaining space is insufficient to start a new First Segment (which requires several bits for SIB type, SEG_COUNT and SIB data).

#### 10.2.48.1	First Segment

This segment type is used to transfer the first segment of a segmented system information block. The IE is used when the first segment fills the entire transport block (Combination 2).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| SIB type | MP |  | SIB Type, 10.3.8.21 |  |
| SEG_COUNT | MP |  | SEG COUNT, 10.3.8.17 |  |
| SIB data fixed | MP |  | SIB data fixed, 10.3.8.19 |  |

#### 10.2.48.1a	First Segment 2

This segment type is used to transfer the first segment of a segmented system information block. The IE is used when the first segment fills the entire transport block (Combination 2). This segment type is used on the second system information broadcast channel (BCH on SCCPCH).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| SIB type | MP |  | SIB and SB type 2, 10.3.8.22ab |  |
| SEG_COUNT | MP |  | SEG COUNT, 10.3.8.17 |  |
| SIB data fixed | MP |  | SIB data fixed 2, 10.3.8.19a |  |

#### 10.2.48.2	First Segment (short)

This segment type is used to transfer the first segment of a segmented system information block. The IE is used when the first segment is concatenated after other segments in a transport block (Combination 5, 7 and 9).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| SIB type | MP |  | SIB Type, 10.3.8.21 |  |
| SEG_COUNT | MP |  | SEG COUNT, 10.3.8.17 |  |
| SIB data variable | MP |  | SIB data variable, 10.3.8.20 |  |

#### 10.2.48.2a	First Segment (short) 2

This segment type is used to transfer the first segment of a segmented system information block. The IE is used when the first segment is concatenated after other segments in a transport block (Combination 5, 7 and 9). This segment type is used on the second system information broadcast channel (BCH on SCCPCH).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| SIB type | MP |  | SIB and SB type 2, 10.3.8.22ab |  |
| SEG_COUNT | MP |  | SEG COUNT, 10.3.8.17 |  |
| SIB data variable | MP |  | SIB data variable 2, 10.3.8.20o |  |

#### 10.2.48.3	Subsequent Segment

This segment type is used to transfer a subsequent segment of a segmented system information block.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| SIB type | MP |  | SIB Type, 10.3.8.21 |  |
| Segment index | MP |  | Segment Index, 10.3.8.18 |  |
| SIB data fixed | MP |  | SIB data fixed, 10.3.8.19 |  |

#### 10.2.48.3a	Subsequent Segment 2

This segment type is used to transfer a subsequent segment of a segmented system information block. This segment type is used on the second system information broadcast channel (BCH on SCCPCH).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| SIB type | MP |  | SIB and SB type 2, 10.3.8.22ab |  |
| Segment index | MP |  | Segment Index, 10.3.8.18 |  |
| SIB data fixed | MP |  | SIB data fixed 2, 10.3.8.19a |  |

#### 10.2.48.4	Last Segment

This segment type is used to transfer the last segment of a segmented system information block. The IE is used when the last segment has a length, excluding length denominator, from 215 through 222 (Combination 11).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| SIB type | MP |  | SIB Type, 10.3.8.21 |  |
| Segment index | MP |  | Segment Index, 10.3.8.18 |  |
| SIB data fixed | MP |  | SIB data fixed, 10.3.8.19 | In case the SIB data is less than 222 bits, padding shall be used. The same padding bits shall be used as defined in clause 12.1 |

#### 10.2.48.4a	Last Segment 2

This segment type is used to transfer the last segment of a segmented system information block. The IE is used when the last segment has a length, excluding length denominator, from 225 through 232 (Combination 11). This segment type is used on the second system information broadcast channel (BCH on SCCPCH).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| SIB type | MP |  | SIB and SB type 2, 10.3.8.22ab |  |
| Segment index | MP |  | Segment Index, 10.3.8.18 |  |
| SIB data fixed | MP |  | SIB data fixed 2, 10.3.8.19a | In case the SIB data is less than 232 bits, padding shall be used. The same padding bits shall be used as defined in clause 12.1 |

#### 10.2.48.5	Last Segment (short)

This segment type is used to transfer the last segment of a segmented system information block. The IE is used when the last segment has a length, excluding length denominator, of upto 214 bits (Combination 4, 5, 6 and 7).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| SIB type | MP |  | SIB Type, 10.3.8.21 |  |
| Segment index | MP |  | Segment Index, 10.3.8.18 |  |
| SIB data variable | MP |  | SIB data variable, 10.3.8.20 |  |

#### 10.2.48.5a	Last Segment (short) 2

This segment type is used to transfer the last segment of a segmented system information block. The IE is used when the last segment has a length, excluding length denominator, of upto 224 bits (Combination 4, 5, 6 and 7). This segment type is used on the second system information broadcast channel (BCH on SCCPCH).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| SIB type | MP |  | SIB and SB type 2, 10.3.8.22ab |  |
| Segment index | MP |  | Segment Index, 10.3.8.18 |  |
| SIB data variable | MP |  | SIB data variable 2, 10.3.8.20o |  |

#### 10.2.48.6	Complete SIB

This segment type is used to transfer a non-segmented system information block. The IE is used when the complete SIB has a length, excluding length denominator, from 215 through 226 (Combination 10).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| SIB type | MP |  | SIB Type, 10.3.8.21 |  |
| SIB data fixed | MP |  | Bit string (226) | The first/leftmost/most significant bit of the bit string contains the first bit of the segment. In case the SIB data is less than 226 bits, padding shall be used. The same padding bits shall be used as defined in clause 12.1 |

#### 10.2.48.6a	Complete SIB 2

This segment type is used to transfer a non-segmented system information block. The IE is used when the complete SIB has a length, excluding length denominator, from 225 through 236 (Combination 10). This segment type is used on the second system information broadcast channel (BCH on SCCPCH).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| SIB type | MP |  | SIB and SB type 2, 10.3.8.22ab |  |
| SIB data fixed | MP |  | Bit string (236) | The first/leftmost/most significant bit of the bit string contains the first bit of the segment. In case the SIB data is less than 236 bits, padding shall be used. The same padding bits shall be used as defined in clause 12.1 |

#### 10.2.48.7	Complete SIB (short)

This segment type is used to transfer a non-segmented system information block. The IE is used when the complete SIB has a length, excluding length denominator, of upto 214 bits (Combination 6, 7, 8 and 9).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| SIB type | MP |  | SIB Type, 10.3.8.21 |  |
| SIB data variable | MP |  | SIB data variable, 10.3.8.20 |  |

#### 10.2.48.7a	Complete SIB (short) 2

This segment type is used to transfer a non-segmented system information block. The IE is used when the complete SIB has a length, excluding length denominator, of upto 224 bits (Combination 6, 7, 8 and 9). This segment type is used on the second system information broadcast channel (BCH on SCCPCH).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| SIB type | MP |  | SIB and SB type 2, 10.3.8.22ab |  |
| SIB data variable | MP |  | SIB data variable 2, 10.3.8.20o |  |

#### 10.2.48.8	System Information Blocks

The IE "SIB data" within the IEs, "First Segment", "Subsequent or last Segment" and "Complete SIB" contains either complete system information block or a segment of a system information block. The actual system information blocks are defined in the following clauses.

##### 10.2.48.8.1	Master Information Block

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |  |
| MIB Value tag | MP |  | MIB Value tag 10.3.8.9 |  |  |
| CN information elements |  |  |  |  |  |
| Supported PLMN types | MP |  | PLMN Type 10.3.1.12 |  |  |
| PLMN Identity | CV-GSM |  | PLMN Identity 10.3.1.11 |  |  |
| Multiple PLMN List | OP |  | Multiple PLMN List 10.3.1.7a | If present, this IE specifies the PLMNs of the cell. If absent, the IE “PLMN Identity” specifies the PLMN of the cell. | REL-6 |
| ANSI-41 information elements |  |  |  |  |  |
| ANSI-41 Core Network Information | CV-ANSI-41 |  | ANSI-41 Core Network Information 10.3.9.1 |  |  |
| References to other system information blocks and scheduling blocks | MP |  | References to other system information blocks and scheduling blocks 10.3.8.14 |  |  |
| CSG Indicator | OP |  | Enumerated (TRUE) | If present, the cell is a CSG cell (see [4]). If absent, the cell is not a CSG cell. | REL-8 |
| SB3 information | OP |  | SB3 information 10.3.8.14a |  | REL-12 |

| Condition | Explanation |
| --- | --- |
| GSM | The IE is mandatory present if the IE "Supported PLMN Types" is set to 'GSM-MAP' or 'GSM-MAP AND ANSI-41', and not needed otherwise |
| ANSI-41 | The IE is mandatory present if the IE "Supported PLMN Types" is set to 'ANSI-41' or 'GSM-MAP AND ANSI-41', and not needed otherwise |

##### 10.2.48.8.2	Scheduling Block 1

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| References to other system information blocks | MP |  | References to other system information blocks 10.3.8.13 |  |

##### 10.2.48.8.3	Scheduling Block 2

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| References to other system information blocks | MP |  | References to other system information blocks 10.3.8.13 |  |

##### 10.2.48.8.3a	Scheduling Block 3

This segment type is used on the second system information broadcast channel (BCH on SCCPCH).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SB3 value tag | MP |  | Cell Value tag 2 10.3.8.4o |  | REL-12 |
| References to other system information blocks | MP |  | References to other system information blocks 2 10.3.8.13a |  | REL-12 |

##### 10.2.48.8.4	System Information Block type 1

The system information block type 1 contains NAS system information as well as UE timers and counters to be used in idle mode and in connected mode.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN information elements |  |  |  |  |
| CN common GSM-MAP NAS system information | MP |  | NAS system information (GSM-MAP) 10.3.1.9 |  |
| CN domain system information list | MP | 1 to <maxCNdomains> |  | Send CN information for each CN domain. |
| >CN domain system information | MP |  | CN domain system information 10.3.1.2 |  |
| UE information |  |  |  |  |
| UE Timers and constants in idle mode | MD |  | UE Timers and constants in idle mode 10.3.3.44 | The UE behaviour is unspecified if this IE is absent. |
| UE Timers and constants in connected mode | MD |  | UE Timers and constants in connected mode 10.3.3.43 | Default value means that for all timers and constants - For parameters with need MD, the defaults specified in 10.3.3.43 apply and - For parameters with need OP, the parameters are absent |

##### 10.2.48.8.5	System Information Block type 2

The system information block type 2 contains the URA identity.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UTRAN mobility information elements |  |  |  |  |
| URA identity list | MP | 1 ..<maxURA> |  |  |
| >URA identity | MP |  | URA identity 10.3.2.6 |  |

##### 10.2.48.8.6	System Information Block type 3

The system information block type 3 contains parameters for cell selection and re-selection.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SIB4 Indicator | MP |  | Boolean | TRUE indicates that SIB4 is broadcast in the cell. When the UE receives SIB3 in the System Information Container message, this IE is interpreted as FALSE. |  |
| UTRAN mobility information elements |  |  |  |  |  |
| Cell identity | MP |  | Cell identity 10.3.2.2 |  |  |
| Cell selection and re-selection info | MP |  | Cell selection and re-selection info for SIB3/4 10.3.2.3 |  |  |
| Cell Access Restriction | MP |  | Cell Access Restriction 10.3.2.1 |  |  |
| Domain Specific Access Restriction Parameters For PLMN Of MIB | OP |  | Domain Specific Access Restriction Parameters 10.3.1.3c | This IE specifies the Domain Specific Access Restriction Parameters for UEs which has chosen the PLMN in the IE “PLMN identity” of the Master Information Block. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. | REL-6 |
| Domain Specific Access Restriction For Shared Network | OP |  |  | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. | REL-6 |
| >CHOICE barring representation | MP |  |  |  | REL-6 |
| >>Domain Specific Access Restriction List |  |  |  |  | REL-6 |
| >>>Domain Specific Access Restriction Parameters For Operator1 | OP |  | Domain Specific Access Restriction Parameters 10.3.1.3c | This IE specifies the Domain Specific Access Restriction Parameters for UEs which has chosen the first PLMN in the IE “multiplePLMNs” in the IE “Multiple PLMN List” of the Master Information Block. | REL-6 |
| >>>Domain Specific Access Restriction Parameters For Operator2 | OP |  | Domain Specific Access Restriction Parameters 10.3.1.3c | This IE specifies the Domain Specific Access Restriction Parameters for UEs which has chosen the second PLMN in the IE “multiplePLMNs” in the IE “Multiple PLMN List” of the Master Information Block. | REL-6 |
| >>>Domain Specific Access Restriction Parameters For Operator3 | OP |  | Domain Specific Access Restriction Parameters 10.3.1.3c | This IE specifies the Domain Specific Access Restriction Parameters for UEs which has chosen the third PLMN in the IE “multiplePLMNs” in the IE “Multiple PLMN List” of the Master Information Block. | REL-6 |
| >>>Domain Specific Access Restriction Parameters For Operator4 | OP |  | Domain Specific Access Restriction Parameters 10.3.1.3c | This IE specifies the Domain Specific Access Restriction Parameters for UEs which has chosen the fourth PLMN in the IE “multiplePLMNs” in the IE “Multiple PLMN List” of the Master Information Block. | REL-6 |
| >>>Domain Specific Access Restriction Parameters For Operator5 | OP |  | Domain Specific Access Restriction Parameters 10.3.1.3c | This IE specifies the Domain Specific Access Restriction Parameters for UEs which has chosen the fifth PLMN in the IE “multiplePLMNs” in the IE “Multiple PLMN List” of the Master Information Block. | REL-6 |
| >>Domain Specific Access Restriction Parameters For All |  |  |  |  | REL-6 |
| >>>Domain Specific Access Restriction Parameters |  |  | Domain Specific Access Restriction Parameters 10.3.1.3c | This IE specifies the common Domain Specific Access Restriction Parameters applied to all PLMNs in the IE “multiplePLMNs” in the IE “Multiple PLMN List” of the Master Information Block. | REL-6 |
| Deferred measurement control reading support | OP |  |  | If present, the UE may apply deferred reading of SIB11, SIB11bis, SIB11ter (if supported), SIB12, SIB18 and SIB19. If not present, deferred reading may not be applied. | REL-7 |
| >CHOICE mode | OP |  |  | If absent, the default reporting quantities are: "CPICH RSCP" (FDD) and "Primary CCPCH RSCP" (TDD). | REL-7 |
| >>FDD |  |  |  |  | REL-7 |
| >>>Intra-frequency reporting quantity SIB3 | MP |  | Enumerated(CPICH Ec/N0, CPICH RSCP) |  | REL-7 |
| >>TDD |  |  |  |  | REL-7 |
| >>>Reporting quantity list | MP | 1 to 2 |  |  | REL-7 |
| >>>>Intra-frequency reporting quantity SIB3 | MP |  | Enumerated (Primary CCPCH RSCP, Timeslot ISCP) |  | REL-7 |
| MBSFN only service | OP |  | Enumerated (TRUE) | Indicates if the cell provides only MBMS services in MBSFN mode | REL-7 |
| Paging Permission with Access Control Parameters For PLMN Of MIB | OP |  | Paging Permission with Access Control Parameters 10.3.1.10a | This IE specifies the Paging Permission With Access Control Parameters for UEs which has chosen the PLMN in the IE “PLMN identity” of the Master Information Block. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. | REL-8 |
| Paging Permission with Access Control For Shared Network | OP |  |  | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. | REL-8 |
| >CHOICE barring representation | MP |  |  |  | REL-8 |
| >>Paging Permission with Access Control List |  |  |  |  | REL-8 |
| >>>Paging Permission with Access Control Parameters For Operator1 | OP |  | Paging Permission with Access Control Parameters 10.3.1.10a | This IE specifies the Paging Permission with Access Control Parameters for UEs which has chosen the first PLMN in the IE “multiplePLMNs” in the IE “Multiple PLMN List” of the Master Information Block. | REL-8 |
| >>>Paging Permission with Access Control Parameters For Operator2 | OP |  | Paging Permission with Access Control Parameters 10.3.1.10a | This IE specifies the Paging Permission with Access Control Parameters for UEs which has chosen the second PLMN in the IE “multiplePLMNs” in the IE “Multiple PLMN List” of the Master Information Block. | REL-8 |
| >>>Paging Permission with Access Control Parameters For Operator3 | OP |  | Paging Permission with Access Control Parameters 10.3.1.10a | This IE specifies the Paging Permission with Access Control Parameters for UEs which has chosen the third PLMN in the IE “multiplePLMNs” in the IE “Multiple PLMN List” of the Master Information Block. | REL-8 |
| >>>Paging Permission with Access Control Parameters For Operator4 | OP |  | Paging Permission with Access Control Parameters 10.3.1.10a | This IE specifies the Paging Permission with Access Control Parameters for UEs which has chosen the fourth PLMN in the IE “multiplePLMNs” in the IE “Multiple PLMN List” of the Master Information Block. | REL-8 |
| >>>Paging Permission with Access Control Parameters For Operator5 | OP |  | Paging Permission with Access Control Parameters 10.3.1.10a | This IE specifies the Paging Permission with Access Control Parameters for UEs which has chosen the fifth PLMN in the IE “multiplePLMNs” in the IE “Multiple PLMN List” of the Master Information Block. | REL-8 |
| >>Paging Permission with Access Control Parameters For All |  |  |  |  | REL-8 |
| >>>Paging Permission with Access Control Parameters | MP |  | Paging Permission with Access Control Parameters 10.3.1.10a | This IE specifies the common Paging Permission with Access Control Parameters applied to all PLMNs in the IE “multiplePLMNs” in the IE “Multiple PLMN List” of the Master Information Block. | REL-8 |
| CSG Identity | OP |  | CSG Identity 10.3.2.8 |  | REL-8 |
| CSG PSC Split Information | OP |  | CSG PSC Split Information 10.3.2.9 | This IE specifies the Primary Scrambling Code reservation information for CSG Cells. | REL-8 |
| IMS Emergency Support Indicator | OP |  | Enumerated (supported) | This IE specifies the support of IMS emergency call in the cell for limited service mode UE | REL-9 |

##### 10.2.48.8.7	System Information Block type 4

The system information block type 4 contains parameters for cell selection and re-selection to be used in connected mode.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UTRAN mobility information elements |  |  |  |  |
| Cell identity | MP |  | Cell identity 10.3.2.2 |  |
| Cell selection and re-selection info | MP |  | Cell selection and re-selection info for SIB3/4 10.3.2.3 |  |
| Cell Access Restriction | MP |  | Cell Access Restriction 10.3.2.1 |  |

##### 10.2.48.8.8	System Information Block type 5 and 5bis

The system information block type 5 contains parameters for the configuration of the common physical channels in the cell. System information block type 5bis uses the same structure as System information block type 5. System information block type 5bis is sent instead of system information block type 5 in cells that use Band IV or Band IX or Band X if it is broadcasted.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SIB6 Indicator | MP |  | Boolean | TRUE indicates that SIB6 is broadcast in the cell. When the UE receives SIB5 in the System Information Container message, this IE is interpreted as FALSE |  |
| SIB22 Indicator | MP |  | Boolean | TRUE indicates that SIB22 is broadcast in the cell. | REL-11 |
| PhyCH information elements |  |  |  |  |  |
| PICH Power offset | MP |  | PICH Power offset 10.3.6.50 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>AICH Power offset | MP |  | AICH Power offset 10.3.6.3 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. |  |
| >TDD |  |  |  |  |  |
| >>PUSCH system information | OP |  | PUSCH system information 10.3.6.66 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |  |
| >>PUSCH system information VHCR | OP |  | PUSCH system information VHCR 10.3.6.66a | Only for 7.68 Mcps TDD If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. | REL-7 |
| >>PDSCH system information | OP |  | PDSCH system information 10.3.6.46 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |  |
| >>TDD open loop power control | MP |  | TDD open loop power control 10.3.6.79 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. |  |
| Primary CCPCH info | OP |  | Primary CCPCH info 10.3.6.57 | Note 1 |  |
| PRACH system information list | MP |  | PRACH system information list 10.3.6.55 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. |  |
| Common E-DCH system info | OP |  | Common E-DCH system info 10.3.6.9a |  | REL-8 |
| Secondary CCPCH system information | MP |  | Secondary CCPCH system information 10.3.6.72 | Note 2 If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. |  |
| CBS DRX Level 1 information | CV-CTCH |  | CBS DRX Level 1 information 10.3.8.3 |  |  |
| Frequency band indicator | OP |  | Frequency band indicator 10.3.6.35b |  | REL-6 |
| Frequency band indicator 2 | OP |  | Frequency band indicator 2 10.3.6.35c |  | REL-6 |
| Frequency band indicator 3 | OP |  | Frequency band indicator 3 10.3.6.35ca |  | REL-10 |
| Frequency Bands Indicator Support | OP |  |  |  | REL-10 |
| >CHOICE mode |  |  |  |  |  |
| >>FDD |  |  |  |  |  |
| >>>Frequency bands indicator for redirection | MP | 1 to< maxFreqBandsIndicatorSupport> |  | The presence of this IE indicates one or two bands for redirection. | REL-10 |
| >>>>CHOICE frequency bands indicator |  |  |  |  | REL-10 |
| >>>>>Frequency bands indicator 1 for redirection |  |  | Frequency band indicator 10.3.6.35b |  | REL-10 |
| >>>>>Frequency bands indicator 2 for redirection |  |  | Frequency band indicator 2 10.3.6.35c |  | REL-10 |
| >>>>>Frequency bands indicator 3 for redirection |  |  | Frequency band indicator 3 10.3.6.35ca |  | REL-10 |
| >>TDD |  |  |  |  |  |
| >>>Frequency bands indicator for 1.28Mcps TDD | MP | 1 to< maxFreqBandsIndicatorSupport> |  | The presence of this IE indicates one or two bands for RF capability. | REL-10 |
| >>>>Frequency bands indicator for TDD | MP |  | Frequency band indicator for TDD 10.3.6.35d |  | REL-10 |
| Multiple Frequency Band indicator list | OP | 1 to< maxMultipleFrequencyBandsFDD > |  | A list of additional frequency bands which the cell belong to. The order of appearance of the additional supported frequency bands in the list indicates their priority i.e from higher to lower. If the UE supports the frequency band indicated explicitly or implicitly in accordance with subclause 8.1.1.6.5, it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the IE "Multiple Frequency Band indicator list". | REL-10 |
| >CHOICE frequency bands indicator | MP |  |  |  | REL-10 |
| >>Frequency band indicator |  |  | Frequency band indicator 10.3.6.35b |  | REL-10 |
| >>Frequency band indicator 2 |  |  | Frequency band indicator 2 10.3.6.35c |  | REL-10 |
| >>Frequency band indicator 3 |  |  | Frequency band indicator 3 10.3.6.35ca |  | REL-10 |
| HSDPA cell Indicator | MD |  | Enumerated (HSDPA Capable Cell) | Default is ’HSDPA capability not indicated’. ’HSDPA Capable Cell’ means that the UE may consider this cell as part of the HSDPA coverage area for display indication only. This indication shall not be used for any other purpose. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. | REL-6 |
| E-DCH cell Indicator | MD |  | Enumerated (E-DCH Capable Cell) | Default is ’E-DCH capability not indicated’. ’E-DCH Capable Cell’ means that the UE may consider this cell as part of the E-DCH coverage area for display indication only. This indication shall not be used for any other purpose. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. | REL-6 |
| Secondary CCPCH system information MBMS | OP |  | Secondary CCPCH system information MBMS 10.3.6.72a | Included if MCCH is on an S-CCPCH used only for MBMS. Note 2 | REL-6 |
| CHOICE mode | OP |  |  |  | REL-7 |
| >FDD |  |  |  |  | REL-7 |
| >>HS-DSCH common system information | MP |  | HS-DSCH common system information 10.3.6.36c | Included if cell supports HS-DSCH reception in CELL_FACH and during IDLE to RRC Connection state transition. | REL-7 |
| >>HS-DSCH paging system information | OP |  | HS-DSCH paging system information 10.3.6.36d | Included if cell supports for UEs in RRC Connected state paging message reception on HS-DSCH. | REL-7 |
| >TDD |  |  |  |  | REL-7 |
| >>HS-DSCH common system information | MP |  | HS-DSCH common system information 1.28Mcps TDD 10.3.6.36ca | Included if cell supports HS-DSCH reception in CELL_FACH and during IDLE to RRC Connection state transition. | REL-8 |
| >>HS-DSCH paging system information | OP |  | HS-DSCH paging system information 1.28Mcps TDD 10.3.6.36da | Included if cell supports for UEs in RRC Connected state paging message reception on HS-DSCH. | REL-8 |
| TDD MBSFN information | OP |  | TDD MBSFN Information 10.3.6.78b | TDD only: included only if some timeslots are designated to MBSFN. | REL-7 |
| HS-DSCH DRX in CELL_FACH Information | OP |  | HS-DSCH DRX in CELL_FACH Information 10.3.6.36g |  | REL-8 |
| HS-DSCH DRX in CELL_FACH Information 1.28 Mcps TDD | OP |  | HS-DSCH DRX in CELL_FACH Information 1.28 Mcps TDD 10.3.6.36h |  | REL-8 |
| Second Frequency info | OP |  | Integer (0 .. 16383) | Note 3 | REL-8 |
| Treset Usage Indicator | OP |  | Enumerated (TRUE) | Only for 1.28 Mcps TDD. The presence of this IE means the timer Treset is not valid when the dedicated H-RNTI is configured in CELL_FACH and CELL_PCH. | REL-8 |
| UpPCH Position Info | CV-Frequency |  | Integer (0 .. 127) | Only for 1.28 Mcps TDD. | REL-8 |
| Cell Update message with optimised encoding | OP |  | Enumerated (TRUE) | Only for FDD. The presence of this IE means that UE shall use the Rel-11 asn1 message type for Cell Update, if the UE supports this message type. | REL-11 |

NOTE 1:	DL scrambling code of the Primary CCPCH is the same as the one for Primary CPICH (FDD only).

NOTE 2:	There is only one MCCH in a cell, which may either be mapped on to an S-CCPCH also used for non- MBMS purposes or to an S-CCPCH dedicated to MBMS. In the first case the MCCH configuration is specified within the IE "Secondary CCPCH system information", in the latter case the MCCH configuration is provided within the IE "Secondary CCPCH system information MBMS".

NOTE 3:	This IE is used in 1.28 Mcps TDD multi-frequency cell to indicate the secondary frequency at which enhanced E-DCH transmission and HS-PDSCH reception for 1.28 Mcps TDD is supported and to indicate that corresponding IEs: "Common E-DCH system info", "HS-DSCH common system information" (TDD) and "HS-DSCH DRX in CELL_FACH Information 1.28 Mcps TDD" can apply for this frequency. The absense of "Frequency info" means that enhanced E-DCH access transmission and HS-PDSCH reception and above IEs apply for primary frequency. For 1.28 Mcps TDD only.

| Condition | Explanation |
| --- | --- |
| CTCH | The IE is mandatory present if the IE "CTCH indicator" is equal to TRUE for at least one FACH, otherwise the IE is not needed in the message |
| Frequency | The IE is optional if the IE "Frequency info" is present, otherwise the IE is not needed. |

##### 10.2.48.8.9	System Information Block type 6

The system information block type 6 contains parameters for the configuration of the common and shared physical channels to be used in connected mode.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PhyCH information elements |  |  |  |  |  |
| PICH Power offset | MP |  | PICH Power offset 10.3.6.50 |  |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>AICH Power offset | MP |  | AICH Power offset 10.3.6.3 |  |  |
| >TDD |  |  |  |  |  |
| >>PUSCH system information | OP |  | PUSCH system information 10.3.6.66 |  |  |
| >>PUSCH system information VHCR | OP |  | PUSCH system information VHCR 10.3.6.66a | Only for 7.68 Mcps TDD | REL-7 |
| >>PDSCH system information | OP |  | PDSCH system information 10.3.6.46 |  |  |
| >>TDD open loop power control | MP |  | TDD open loop power control 10.3.6.79 |  |  |
| Primary CCPCH info | OP |  | Primary CCPCH info 10.3.6.57 | Note 1 |  |
| PRACH system information list | OP |  | PRACH system information list 10.3.6.55 |  |  |
| Secondary CCPCH system information | OP |  | Secondary CCPCH system information 10.3.6.72 |  |  |
| CBS DRX Level 1 information | CV-CTCH |  | CBS DRX Level 1 information 10.3.8.3 |  |  |
| Frequency band indicator | OP |  | Frequency band indicator 10.3.6.35b |  | REL-6 |
| Frequency band indicator 2 | OP |  | Frequency band indicator 2 10.3.6.35c |  | REL-6 |
| Frequency band indicator 3 | OP |  | Frequency band indicator 3 10.3.6.35ca |  | REL-10 |
| Multiple Frequency Band indicator list | OP | 1 to< maxMultipleFrequencyBandsFDD > |  | A list of additional frequency bands which the cell belongs to. The order of appearance of the additional supported frequency bands in the list indicates their priority i.e from higher to lower. If the UE supports the frequency band indicated explicitly or implicitly in accordance with subclause 8.1.1.6.6, it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the IE "Multiple Frequency Band indicator list". | REL-10 |
| >CHOICE frequency bands indicator | MP |  |  |  | REL-10 |
| >>Frequency band indicator |  |  | Frequency band indicator 10.3.6.35b |  | REL-10 |
| >>Frequency band indicator 2 |  |  | Frequency band indicator 2 10.3.6.35c |  | REL-10 |
| >>Frequency band indicator 3 |  |  | Frequency band indicator 3 10.3.6.35ca |  | REL-10 |

NOTE 1:	DL scrambling code of the Primary CCPCH is the same as the one for Primary CPICH (FDD only).

| Condition | Explanation |
| --- | --- |
| CTCH | The IE is mandatory present if the IE "CTCH indicator" is equal to TRUE for at least one FACH, otherwise the IE is not needed |

##### 10.2.48.8.10	System Information Block type 7

The system information block type 7 contains the fast changing parameters UL interference and Dynamic persistence level.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |
| >FDD |  |  |  |  |
| >>UL interference | MP |  | UL interference 10.3.6.87 |  |
| >TDD |  |  |  | (no data) |
| PhyCH information elements |  |  |  |  |
| PRACHs listed in system information block type 5 | MP | 1 to<maxPRACH> |  | The order of the PRACHs is the same as in system information block type 5 or 5bis. |
| >Dynamic persistence level | MP |  | Dynamic persistence level 10.3.6.35 |  |
| PRACHs listed in system information block type 6 | OP | 1 to <maxPRACH> |  | The order of the PRACHs is the same as in system information block type 6. |
| >Dynamic persistence level | MP |  | Dynamic persistence level 10.3.6.35 |  |
| Expiration Time Factor | MD |  | Expiration Time Factor 10.3.3.12 | Default is 1. |

##### 10.2.48.8.11	Void

##### 10.2.48.8.12	Void

##### 10.2.48.8.13	Void

##### 10.2.48.8.14	System Information Block type 11

The system information block type 11 contains measurement control information to be used in the cell.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SIB12 Indicator | MP |  | Boolean | TRUE indicates that SIB12 is broadcast in the cell. |  |
| Measurement information elements |  |  |  |  |  |
| FACH measurement occasion info | OP |  | FACH measurement occasion info 10.3.7.8 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |  |
| Measurement control system information | MP |  | Measurement control system information 10.3.7.47 | For 1.28 Mcps TDD if the cell is operating in MBSFN only mode the UE behaviour upon reception of this IE is unspecified. |  |
| Measurement Scaling Factor | OP |  | Enumerated( SF_UTRA_CF1, SF_UTRA_CF2) | The Measurement Scaling Factor is used to scale the measurement performance for UTRA/E-UTRA frequencies configured in CELL_FACH as specified in [19]. Even if no UTRA or E-UTRA frequency is configured for reduced measurement performance, the network may configure a scaling factor and configure any of the scaling factor values [19]. | REL-12 |
| MBSFN frequency list | OP |  | MBSFN frequency list 10.3.9a.12a | If present contains all neighbouring frequencies of MBSFN clusters operating in MBSFN mode as indicated in subclause 8.1.1.6.3. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |  |

##### 10.2.48.8.14a	System Information Block type 11bis

The system information block type 11bis contains measurement control information to be used in the cell in addition to System Information Block type 11 and optionally UTRAN mobility information for CSG cells.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Measurement information elements |  |  |  |  | REL-6 |
| Measurement control system information extension | OP |  | Measurement control system information extension 10.3.7.47a |  | REL-6 |
| UTRAN mobility information elements |  |  |  |  | REL-8 |
| CSG PSC Split Information | OP |  | CSG PSC Split Information 10.3.2.9 | This IE specifies the Primary Scrambling Code reservation information for CSG Cells. | REL-8 |
| Dedicated CSG frequency list | OP | 1 to <maxDedicatedCSGFreq> |  | This IE specifies the frequencies dedicated for CSG cells only. | REL-8 |
| >Dedicated CSG frequency | MP |  | Frequency Info 10.3.6.36 |  | REL-8 |

##### 10.2.48.8.14b	System Information Block type 11ter

The system information block type 11ter contains measurement control information to be used in the cell in addition to System Information Block type 11 and System Information Block type 11bis.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Measurement information elements |  |  |  |  | REL-12 |
| Measurement control system information inter-frequency extension | OP |  | Inter-frequency cell info list extension 10.3.7.47b |  | REL-12 |

##### 10.2.48.8.15	System Information Block type 12

The system information block type 12 contains measurement control information to be used in connected mode.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Measurement information elements |  |  |  |  |
| FACH measurement occasion info | OP |  | FACH measurement occasion info 10.3.7.8 |  |
| Measurement control system information | MP |  | Measurement control system information 10.3.7.47 |  |

##### 10.2.48.8.16	System Information Block type 13

The system information block type 13 contains ANSI-41 system information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Other information elements |  |  |  |  |
| CN Information Elements |  |  |  |  |
| CN Domain system information list | MP | 1 to <maxCNdomains> |  | Send CN information for each CN domain. |
| >CN Domain system information | MP |  | CN Domain system information 10.3.1.2 |  |
| UE Information |  |  |  |  |
| UE timers and constants in idle mode | MD |  | UE timers and constants in idle mode 10.3.3.44 | The UE behaviour is unspecified if this IE is absent. |
| Capability update requirement | MD |  | Capability update requirement 10.3.3.2 | Default value is defined in subclause 10.3.3.2 |

###### 10.2.48.8.16.1	System Information Block type 13.1

The system information block type 13.1 contains the ANSI-41 RAND information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ANSI-41 information elements |  |  |  |  |
| ANSI-41 RAND information | MP |  | ANSI-41 RAND information 10.3.9.6 |  |

###### 10.2.48.8.16.2	System Information Block type 13.2

The system information block type 13.2 contains the ANSI-41 User Zone Identification information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ANSI-41 information elements |  |  |  |  |
| ANSI-41 User Zone Identification information | MP |  | ANSI-41 User Zone Identification information 10.3.9.7 |  |

###### 10.2.48.8.16.3	System Information Block type 13.3

The system information block type 13.3 contains the ANSI-41 Private Neighbour List information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ANSI-41 information elements |  |  |  |  |
| ANSI-41 Private Neighbour List information | MP |  | ANSI-41 Private Neighbour List information 10.3.9.5 |  |

###### 10.2.48.8.16.4	System Information Block type 13.4

The system information block type 13.4 contains the ANSI-41 Global Service Redirection information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ANSI-41 information elements |  |  |  |  |
| ANSI-41 Global Service Redirection information | MP |  | ANSI-41 Global Service Redirection information 10.3.9.2 |  |

##### 10.2.48.8.17	System Information Block type 14

NOTE:	Only for 3.84 Mcps TDD and 7.68 Mcps TDD.

The system information block type 14 contains parameters for common and dedicated physical channel uplink outer loop power control information to be used in both idle and connected mode.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PhyCH information elements |  |  |  |  |
| Individual Timeslot interference list | MP | 1 to <maxTS> |  |  |
| >Individual Timeslot interference | MP |  | Individual Timeslot interference 10.3.6.38 |  |
| Expiration Time Factor | MD |  | Expiration Time Factor 10.3.3.12 | Default is 1. |

##### 10.2.48.8.18	System Information Block type 15

The system information block type 15 contains information useful for UE-based or UE-assisted positioning methods.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| GPS Data ciphering info | OP |  | UE positioning Ciphering info 10.3.7.86 | If this IE is present then the SIB types 15.1, 15.2 & 15.3 are ciphered in accordance with the Data Assistance Ciphering Algorithm specified in [18] |
| Reference position | MP |  | Ellipsoid point with altitude and uncertainty ellipsoid 10.3.8.4c | approximate position where the UE is located |
| GPS reference time | MP |  | UE positioning GPS reference time 10.3.7.96 |  |
| Satellite information | OP | 1 to <maxSat> |  | This IE is present whenever bad (failed/failing) satellites are detected by UTRAN [18]. |
| >BadSatID | MP |  | Enumerated(0..63) |  |

###### 10.2.48.8.18.0	System Information Block type 15bis

The system information block type 15bis contains information useful for UE-based or UE-assisted positioning methods. The content of this SIB is common to all GANSS.

| Information Element/Group name | Need | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- |
| Reference position | MP | Ellipsoid point with altitude and uncertainty ellipsoid 10.3.8.4c | approximate position where the UE is located | REL-7 |
| GANSS reference time | MP | UE positioning GANSS reference time 10.3.7.96o |  | REL-7 |
| GANSS ionospheric model | OP | UE positioning GANSS ionospheric model 10.3.7.92a |  | REL-7 |
| GANSS additional ionospheric model | OP | UE positioning GANSS additional ionospheric model 10.3.7.92b |  | REL-8 |
| GANSS Earth orientation paramaters | OP | UE positioning GANSS Earth orientation parameters 10.3.7.92c |  | REL-8 |

###### 10.2.48.8.18.1	System Information Block type 15.1

The system information block type 15.1 contains information useful for UE positioning DGPS Corrections. The DGPS Corrections message contents are based on a Type-1 message of DGPS specified in [13].

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| DGPS corrections | MP |  | UE positioning GPS DGPS corrections 10.3.7.91 |  |

###### 10.2.48.8.18.1a	System Information Block type 15.1bis

The system information block type 15.1bis contains information useful for UE positioning DGANSS Corrections. The DGANSS Corrections message contents are based on a Type-1 message of DGANSS specified in [13]. The content of this SIB is GNSS specific.

| Information Element/Group name | Need | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- |
| DGANSS corrections | MP | UE positioning DGANSS corrections 10.3.7.91b |  | REL-7 |

###### 10.2.48.8.18.1b	System Information Block type 15.1ter

The system information block type 15.1ter contains information useful for UE positioning DBDS corrections and UE positioning BDS Ionospheric Grid Model. The content of DBDS Corrections and BDS Ionospheric Grid Model are based on the NAV message specified in [86]. The content of this SIB is for BDS only.

| Information Element/Group name | Need | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- |
| DBDS corrections | OP | UE positioning DBDS corrections 10.3.7.92e |  | REL-12 |
| BDS Ionospheric Grid Model | OP | UE positioning BDS Ionospheric Grid Model 10.3.7.92d |  | REL-12 |

###### 10.2.48.8.18.2	System Information Block type 15.2

The system information block type 15.2 contains information useful for GPS Navigation Model. These IE fields are based on information extracted from the subframes 1 to 3 of the GPS navigation message [12].

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transmission TOW | MP |  | Integer (0..604799) | The approximate GPS time-of-week when the message is broadcast. in seconds |
| SatID | MP |  | Integer (0..63) | Satellite ID |
| GPS Ephemeris and Clock Correction Parameters | MP |  | UE positioning GPS Ephemeris and Clock Correction parameters 10.3.7.91a |  |

###### 10.2.48.8.18.2a	System Information Block type 15.2bis

The system information block type 15.2bis contains information useful for GANSS Navigation Model. The content of this SIB is GNSS specific.

| Information Element/Group name | Need | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- |
| GANSS Navigation Model | MP | UE positioning GANSS navigation model 10.3.7.94a |  | REL-7 |

###### 10.2.48.8.18.2b	System Information Block type 15.2ter

The system information block type 15.2ter contains information useful for GANSS Navigation Model. The content of this SIB is GNSS specific.

| Information Element/Group name | Need | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- |
| GANSS additional navigation models | MP | UE positioning GANSS additional navigation models 10.3.7.94b |  | REL-8 |

###### 10.2.48.8.18.3	System Information Block type 15.3

The system information block type 15.3 contains information useful for ionospheric delay, UTC offset, and Almanac. These IEs contain information extracted from the subframes 4 and 5 of the GPS navigation message, [12].

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transmission TOW | MP |  | Integer (0..604799) | The approximate GPS time-of-week when the message is broadcast. in seconds |
| GPS Almanac and Satellite Health | OP |  | UE positioning GPS almanac 10.3.7.89 |  |
| GPS ionospheric model | OP |  | UE positioning GPS ionospheric model 10.3.7.92 |  |
| GPS UTC model | OP |  | UE positioning GPS UTC model 10.3.7.97 |  |
| SatMask | CV-Almanac |  | Bit string(1..32) | indicates the satellites that contain the pages being broadcast in this data set |
| LSB TOW | CV-Almanac |  | Bit string(8) |  |

| Condition | Explanation |
| --- | --- |
| Almanac | This IE is mandatory present if the IE "GPS Almanac and Satellite Health" is present |

###### 10.2.48.8.18.3a	System Information Block type 15.3bis

The system information block type 15.3bis contains information useful for GANSS time model, UTC offset and Almanac, as well as auxiliary information. The content of this SIB is GNSS specific.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GANSS Almanac | OP |  | UE positioning GANSS almanac 10.3.7.89a |  | REL-7 |
| GANSS time models | OP | 1 to <maxGANSS-1> |  |  | REL-7 |
| >GANSS time model | OP |  | UE positioning GANSS time model 10.3.7.97a |  | REL-7 |
| GANSS UTC model | OP |  | UE positioning GANSS UTC model 10.3.7.97c |  | REL-7 |
| GANSS additional UTC models | OP |  | UE positioning GANSS additional UTC models 10.3.7.97d |  | REL-8 |
| GANSS auxiliary information | OP |  | UE positioning GANSS auxiliary information 10.3.7.97f |  | REL-8 |

###### 10.2.48.8.18.4	System Information Block type 15.4

The system information block type 15.4 contains ciphering information for System Information Block type 15.5 and information useful for OTDOA UE-assisted Positioning method.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| OTDOA Data ciphering info | OP |  | UE positioning Ciphering info 10.3.7.86 | If this IE is present then the for UE-based the System Information Block type 15.5 is ciphered in accordance with the Data Assistance Ciphering Algorithm specified in [18] |
| OTDOA assistance data for UE-assisted | MP |  | UE positioning OTDOA assistance data for UE-assisted 10.3.7.103 |  |

###### 10.2.48.8.18.4a	System Information Block type 15.5

The system information block type 15.5 contains information useful for OTDOA UE-based Positioning method.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| OTDOA assistance data for UE-based | MP |  | UE positioning OTDOA assistance data for UE-based 10.3.7.103a |  |

###### 10.2.48.8.18.5	System Information Block type 15.6

The system information block type 15.6 contains information useful for acquisition of GANSS signals. The content of this SIB is GNSS specific.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GANSS TOD | MP |  | Integer (0..86399) | GANSS Time of Day in seconds where GANSS reference measurement information is valid. | REL-7 |
| GANSS reference measurement information | MP |  | UE positioning GANSS reference measurement information 10.3.7.88b |  | REL-7 |

###### 10.2.48.8.18.6	System Information Block type 15.7

The system information block type 15.7 contains data bits which can be used for data wipe-off. The content of this SIB is GNSS specific.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GANSS data bit assistance | MP |  | UE positioning GANSS data bit assistance 10.3.7.97b |  | REL-7 |

###### 10.2.48.8.18.7	System Information Block type 15.8

The system information block type 15.8 contains ciphering information and real-time integrity information. The content of this SIB is GNSS specific.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GANSS Data ciphering info | OP |  | UE positioning Ciphering info 10.3.7.86 | If this IE is present then the SIB types 15.1bis, 15.1ter, 15.2bis, 15.2ter, 15.3bis, 15.6 and 15.7 are ciphered in accordance with the Data Assistance Ciphering Algorithm specified in [18] | REL-7 |
| GANSS real-time integrity | OP |  | UE positioning GANSS real-time integrity 10.3.7.95b |  | REL-7 |

##### 10.2.48.8.19	System Information Block type 16

The system information block type 16 contains radio bearer, transport channel and physical channel parameters to be stored by UE in idle and connected mode for use during handover to UTRAN.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| RB information elements |  |  |  |  |
| Predefined RB configuration | MP |  | Predefined RB configuration 10.3.4.7 |  |
| TrCH Information Elements |  |  |  |  |
| Predefined TrCH configuration | MP |  | Predefined TrCH configuration 10.3.5.9 |  |
| PhyCH Information Elements |  |  |  |  |
| Predefined PhyCH configuration | MP |  | Predefined PhyCH configuration 10.3.6.56 |  |

##### 10.2.48.8.20	System Information Block type 17

NOTE:	Only for TDD.

The system information block type 17 contains fast changing parameters for the configuration of the shared physical channels to be used in connected mode.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PhyCH information elements |  |  |  |  |  |
| PUSCH system information | OP |  | PUSCH system information 10.3.6.66 |  |  |
| PUSCH system information VHCR | OP |  | PUSCH system information VHCR 10.3.6.66a | Only for 7.68 Mcps TDD | REL-7 |
| PDSCH system information | OP |  | PDSCH system information 10.3.6.46 |  |  |

##### 10.2.48.8.21	System Information Block type 18

The System Information Block type 18 contains PLMN identities of neighbouring cells to be considered in idle mode as well as in connected mode.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Idle mode PLMN identities | OP |  | PLMN identities of neighbour cells 10.3.7.53a |  |  |
| Connected mode PLMN identities | OP |  | PLMN identities of neighbour cells 10.3.7.53a |  |  |
| Idle mode PLMN identities for SIB 11bis | OP |  | PLMN identities of neighbour cells 10.3.7.53a |  | REL-6 |
| Connected mode PLMN identities for SIB 11bis | OP |  | PLMN identities of neighbour cells 10.3.7.53a |  | REL-6 |
| Idle mode PLMN identities for SIB 11ter | OP |  | PLMN identities of neighbour cells for SIB 11ter 10.3.7.53b |  | REL-12 |
| Connected mode PLMN identities for SIB 11ter | OP |  | PLMN identities of neighbour cells for SIB 11ter 10.3.7.53b |  | REL-12 |

##### 10.2.48.8.22	System Information Block type 19

The system information block type 19 contains Inter-RAT frequency and priority information to be used in the cell.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UTRA priority info list | MP |  | UTRA priority info list 10.3.7.113 |  | REL-8 |
| GSM priority info list | OP |  | GSM priority info list 10.3.7.114 |  | REL-8 |
| GSM ScalingFactor for Treselection | OP |  | Real (0.25..4 by step of 0.25) | If present, it is used by the UE as scaling factor for Treselections or Treselections,PCH or Treselections,FACH for inter-RAT absolute priority based cell reselection evaluation to GSM [4]. It replaces Inter-RAT ScalingFactor for Treselection in SIB3/4, if available. | REL-11 |
| E-UTRA frequency and priority info list | OP |  | E-UTRA frequency and priority info list 10.3.7.115 |  | REL-8 |
| E-UTRA ScalingFactor for Treselection | OP |  | Real (0.25..4 by step of 0.25) | If present, it is used by the UE as scaling factor for Treselections or Treselections,PCH or Treselections,FACH for inter-RAT absolute priority based cell reselection evaluation to E-UTRA [4]. It replaces Inter-RAT ScalingFactor for Treselection in SIB3/4, if available. | REL-11 |
| E-UTRA frequency RACH reporting information | OP |  | E-UTRA frequency RACH reporting information 10.3.7.139 |  | REL-11 |
| CELL_FACH Absolute Priority Measurement Indicator | OP |  | Enumerated (High Priority Layers, All Layers) | High Priority Layers means that absolute priority measurements are only required in CELL_FACH state on higher priority layers when Srxlev and Squal of the serving cell are above Sprioritysearch1 and Sprioritysearch2. All Layers means that absolute priority measurements are always required in CELL_FACH state according to the rules in [4] and requirements in [19]. A UE that indicates FGI3 but not FGI4 shall behave as if High priority Layers is indicated. | REL-11 |

##### 10.2.48.8.23	System Information Block type 20

The system information block type20 contains HNBName.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| HNB Name | OP |  | HNB Name 10.3.8.4m |  | REL-8 |

##### 10.2.48.8.24	System Information Block type 21

The system information block type 21 contains parameters for EAB.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| EAB Parameters | OP |  |  | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3, the UE behaviour upon reception of this IE is unspecified. | REL-11 |
| >CHOICE barring representation | MP |  |  |  | REL-11 |
| >>EAB Parameters Per PLMN List |  | 1 to 6 |  | The number of instances of this IE is the number of instances of the IE “Multiple PLMNs” in the IE “Multiple PLMN List” of the MIB plus one. The first instance of this IE specifies the EAB Parameters for UEs which have chosen the PLMN in the IE “PLMN identity” of the MIB, the second instance specifies the EAB Parameters for UEs which have chosen the first PLMN in the IE “Multiple PLMNs” in the IE “Multiple PLMN List” of the MIB, the third instance specifies the EAB Parameters for UEs which have chosen the second PLMN in the IE “Multiple PLMNs” in the IE “Multiple PLMN List” of the MIB, and so on. | REL-11 |
| >>>Domain Specific EAB Parameters | OP |  | Domain Specific EAB Parameters 10.3.1.3d | Absence of this IE means that no extended access barring applies for the corresponding PLMN | REL-11 |
| >>EAB Parameters For All |  |  | Domain Specific EAB Parameters 10.3.1.3d | This IE specifies the common EAB Parameters applicable for all PLMNs, including the PLMN in the IE “PLMN identity” of the MIB and the PLMNs in the IE “Multiple PLMNs” in the IE “Multiple PLMN List” of the MIB. | REL-11 |

##### 10.2.48.8.25	System Information Block type 22

NOTE:	Only for FDD.

The system information block type 22 contains information for Concurrent deployment of 2ms and 10ms TTI in a cell, NodeB triggered HS-DPCCH transmission, Fallback to R99 PRACH, TTI alignment, Per HARQ process activation and de-activation and HS-DSCH DRX operation with second DRX cycle in CELL_FACH. Further, it contains control parameters related to the PRACH access preamble.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PRACH preamble control parameters extension list Type 1 (for Enhanced Uplink) | OP |  | PRACH preamble control parameters extension list for Type 1 (for Enhanced Uplink) 10.3.6.135 | Control parameters of the physical signal. This and/or parameters in IE "PRACH preamble control parameters (for Enhanced Uplink)" is used for access by UE’s supporting concurrent deployment when the outcome of the common E-DCH TTI selection is 10ms. Also used for access if "E-DCH Transmission Time Interval" is equal to 10ms when concurrent deployment is not used, by UEs that support either NodeB triggered HS-DPCCH transmission or HS-DSCH DRX operation with second DRX cycle or Fallback to R99 PRACH. See subclause 8.5.45, 8.5.73 and 8.5.74. | REL-11 |
| PRACH preamble control parameters extension list Type 2 (for Enhanced Uplink) | OP |  | PRACH preamble control parameters extension list (for Enhanced Uplink) 10.3.6.134 | Control parameters of the physical signal. Used for access by UE’s supporting only concurrent deployment, and by UEs supporting TTI alignment and Per HARQ process if the IE "Common E-DCH Resource Configuration Information List Extension" is not present, when the outcome of the common E-DCH TTI selection is 2ms. See subclause 8.5.45 and 8.5.73. | REL-11 |
| PRACH preamble control parameters extension list Type 3 (for Enhanced Uplink) | OP |  | PRACH preamble control parameters extension list (for Enhanced Uplink) 10.3.6.134 | Control parameters of the physical signal. Used for access by UE’s supporting TTI alignment and Per HARQ process if the IE "Common E-DCH Resource Configuration Information List Extension" is present, when the outcome of the common E-DCH TTI selection is 2ms. See subclause 8.5.45, and 8.5.73. | REL-11 |
| Concurrent Deployment of 2ms and 10ms TTI | OP |  | Concurrent Deployment of 2ms and 10ms TTI 10.3.5.25 |  | REL-11 |
| NodeB triggered HS-DPCCH Transmission | OP |  | NodeB triggered HS-DPCCH Transmission 10.3.6.130 |  | REL-11 |
| Fallback R99 PRACH info | OP |  | Fallback R99 PRACH info 10.3.6.141 |  | REL-11 |
| Common E-DCH Resource Configuration Information List Extension | OP |  | Common E-DCH Resource Configuration Information List Extension 10.3.6.140 |  | REL-11 |
| HS-DSCH DRX in CELL_FACH with second DRX cycle Information | OP |  | HS-DSCH DRX in CELL_FACH with second DRX cycle Information 10.3.6.139 |  | Rel-11 |
| Cell reselection indication reporting | OP |  | Enumerated (TRUE) |  | REL-12 |

##### 10.2.48.8.26	System Information Block type 23

The system information block type 23 contains information for RAN-assisted WLAN interworking.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE WLAN representation | MP |  |  |  | REL-12 |
| >WLAN Offload Information Per PLMN List |  | 1 to 6 |  | The number of instances of this IE is the number of instances of the IE "Multiple PLMNs" in the IE "Multiple PLMN List" of the MIB plus one. The first instance of this IE specifies the WLAN Offload Information for UEs which have chosen the PLMN in the IE "PLMN identity" of the MIB, the second instance specifies the WLAN Offload Information for UEs which have chosen the first PLMN in the IE "Multiple PLMNs" in the IE "Multiple PLMN List" of the MIB, the third instance specifies the WLAN Offload Information for UEs which have chosen the second PLMN in the IE "Multiple PLMNs" in the IE "Multiple PLMN List" of the MIB, and so on. | REL-12 |
| >>WLAN Offload Information | OP |  | WLAN Offload Information 10.3.9b.3 | Absence of this IE means that no WLAN Offload Information applies for the corresponding PLMN. | REL-12 |
| >WLAN Offload Information common for all PLMN |  |  |  | This IE specifies the common WLAN Offload Information configuration applicable for all PLMNs, including the PLMN in the IE "PLMN identity" of the MIB and the PLMNs in the IE "Multiple PLMNs" in the IE "Multiple PLMN List" of the MIB. | REL-12 |
| >>WLAN Offload Information | OP |  | WLAN Offload Information 10.3.9b.3 | Absence of this IE means that no WLAN Offload Information applies for all PLMNs. | REL-12 |

##### 10.2.48.8.27	System Information Block type 24

NOTE:	Only for FDD.

The system information block type 24 contains information for controlling access in CELL_FACH state and CELL_PCH state (with seamless transition to CELL_FACH state).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| DTCH transmission blocking | OP |  | Bit string(maxNumAccessGroups) | Bit 0 is the first/leftmost bit of the bit string. Bit n corresponds to the n-th defined Access Group. Value '1' for a bit means the DTCH transmissions for UEs in that Access Group are blocked when UE is in CELL_FACH state and CELL_PCH state with seamless transition to CELL_FACH state. | REL-12 |
| Expiration Time Factor 2 | MD |  | Expiration Time Factor 2 10.3.3.12b | Default is 1. | REL-12 |

##### 10.2.48.8.28	System Information Block type 25

NOTE:	Only for FDD.

The system information block type 25 contains parameters for ACDC barring information in Idle mode.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| ACDC Parameters | MP |  |  | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3, the UE behaviour upon reception of this IE is unspecified. | REL-13 |
| >CHOICE barring representation | MP |  |  |  | REL-13 |
| >>ACDC Parameters Per PLMN List |  | 1 to 6 |  | The number of instances of this IE is the number of instances of the IE "Multiple PLMNs" in the IE "Multiple PLMN List" of the MIB plus one. The first instance of this IE specifies the ACDC Parameters for UEs which have chosen the PLMN in the IE “PLMN identity” of the MIB, the second instance specifies the ACDC Parameters for UEs which have chosen the first PLMN in the IE "Multiple PLMNs" in the IE "Multiple PLMN List" of the MIB, the third instance specifies the ACDC Parameters for UEs which have chosen the second PLMN in the IE "Multiple PLMNs" in the IE "Multiple PLMN List" of the MIB, and so on. | REL-13 |
| >>>PLMN specific ACDC Parameters | OP |  | Specific ACDC Barring Information 10.3.9b.5 | Absence of this IE means that no extended access barring applies for the corresponding PLMN | REL-13 |
| >>>ACDC applicable for Roamer | OP |  | Enumerated (TRUE) | Absence of this IE means that no ACDC barring applies to roaming UE for the corresponding PLMN | REL-13 |
| >>ACDC Parameters For All |  |  |  |  | REL-13 |
| >>>ACDC Information | MP |  | Specific ACDC Barring Information 10.3.9b.5 | This IE specifies the common ACDC Parameters applicable for all PLMNs, including the PLMN in the IE "PLMN identity" of the MIB and the PLMNs in the IE "Multiple PLMNs" in the IE "Multiple PLMN List" of the MIB. | REL-13 |
| >>>ACDC applicable for Roamer | OP |  | Enumerated (TRUE) | Absence of this IE means that no ACDC barring applies to roaming UE for all the PLMN. | REL-13 |

### 10.2.49	SYSTEM INFORMATION CHANGE INDICATION

This message is used to send information on FACH or HS-DSCH (FDD and 1.28 Mcps TDD only) to the UEs in state CELL_FACH, CELL_PCH (FDD and 1.28 Mcps TDD only), or CELL_DCH(TDD only) about coming modification of the system information.

RLC-SAP: TM

Logical channel: BCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| Other information elements |  |  |  |  |  |
| BCCH modification info | MP |  | BCCH modification info 10.3.8.1 |  |  |
| ETWS information | OP |  | ETWS information 10.3.8.4ea |  | REL-8 |

If the encoded message does not fill a transport block, the RRC layer shall insert padding according to subclause 12.1.

### 10.2.50	TRANSPORT CHANNEL RECONFIGURATION

This message is used by UTRAN to configure the transport channel of a UE. This also includes a possible reconfiguration of physical channels. The message can also be used to assign a TFC subset and reconfigure physical channel.

RLC-SAP: AM or UM

Logical channel: DCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE Information Elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Integrity protection mode info | OP |  | Integrity protection mode info 10.3.3.19 | The UTRAN should not include this IE unless it is performing an SRNS relocation |  |
| Ciphering mode info | OP |  | Ciphering mode info 10.3.3.5 | The UTRAN should not include this IE unless it is performing an SRNS relocation and a change in ciphering algorithm |  |
| Activation time | MD |  | Activation time 10.3.3.1 | Default value is "now" |  |
| Dynamic activation time | OP |  | Dynamic activation time 10.3.3.1a |  | REL-13 |
| Delay restriction flag | OP |  | Enumerated (TRUE) | This IE is always set to TRUE and included if the activation time is restricted according to subclause 8.6.3.1 | REL-6 |
| New U-RNTI | OP |  | U-RNTI 10.3.3.47 |  |  |
| New C-RNTI | OP |  | C-RNTI 10.3.3.8 |  |  |
| New DSCH-RNTI | OP |  | DSCH-RNTI 10.3.3.9a | Should not be set in FDD. If received the UE behaviour is unspecified. |  |
| New H-RNTI | OP |  | H-RNTI 10.3.3.14a |  | REL-5 |
| New Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-6 |
| New Secondary E-RNTI | OP |  | E-RNTI 10.3.3.10a | FDD only | REL-6 |
| RRC State Indicator | MP |  | RRC State Indicator 10.3.3.35a |  |  |
| UE Mobility State Indicator | CV-FACH_PCH |  | Enumerated (High-mobilityDetected) | Absence of this IE implies that, according to [4], the UE shall consider itself being in the mobility state the UE has maintained in CELL_DCH state or being not in high mobility state after the state transition, if applicable. | REL-7 |
| UTRAN DRX cycle length coefficient | OP |  | UTRAN DRX cycle length coefficient 10.3.3.49 |  |  |
| CN Information Elements |  |  |  |  |  |
| CN Information info | OP |  | CN Information info 10.3.1.3 |  |  |
| UTRAN mobility information elements |  |  |  |  |  |
| URA identity | OP |  | URA identity 10.3.2.6 |  |  |
| RNC support for change of UE capability | OP |  | Boolean | Should be included if the message is used to perform an SRNS relocation | REL-7 |
| Reconfiguration in response to requested change of UE capability | OP |  | Enumerated (TRUE) |  | REL-7 |
| RB information elements |  |  |  |  |  |
| Downlink counter synchronisation info | OP |  |  |  |  |
| >RB with PDCP information list | OP | 1 to <maxRBallRABs> |  |  |  |
| >>RB with PDCP information | MP |  | RB with PDCP information 10.3.4.22 | This IE is needed for each RB having PDCP in the case of lossless SRNS relocation |  |
|  | OP |  |  |  | REL-5 |
| >>PDCP context relocation info | OP |  | PDCP context relocation info 10.3.4.1a | This IE is needed for each RB having PDCP and performing PDCP context relocation | REL-5 |
| Retrievable configuration info | OP |  | Retrievable configuration info 10.3.8.25 |  | REL-13 |
| TrCH Information Elements |  |  |  |  |  |
| Uplink transport channels |  |  |  |  |  |
| UL Transport channel information common for all transport channels | OP |  | UL Transport channel information common for all transport channels 10.3.5.24 |  |  |
| Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >Added or Reconfigured UL TrCH information | MP |  | Added or Reconfigured UL TrCH information 10.3.5.2 |  |  |
| Downlink transport channels |  |  |  |  |  |
| DL Transport channel information common for all transport channels | OP |  | DL Transport channel information common for all transport channels 10.3.5.6 |  |  |
| Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >Added or Reconfigured DL TrCH information | MP |  | Added or Reconfigured DL TrCH information 10.3.5.1 |  |  |
| PhyCH information elements |  |  |  |  |  |
| Frequency info | OP |  | Frequency info 10.3.6.36 |  |  |
| Multi-frequency Info | OP |  | Multi-frequency Info 10.3.6.39a | This IE is used for 1.28 Mcps TDD only | REL-7 |
| DTX-DRX timing information | OP |  | DTX-DRX timing information 10.3.6.34b |  | REL-7 |
| DTX-DRX Information | OP |  | DTX-DRX Information 10.3.6.34a |  | REL-7 |
| HS-SCCH less Information | OP |  | HS-SCCH less Information 10.3.6.36ab |  | REL-7 |
| MIMO parameters | OP |  | MIMO parameters 10.3.6.41a |  | REL-7 |
| MIMO mode with four transmit antennas parameters | OP |  | MIMO mode with four transmit antennas parameters 10.3.6.142 |  | REL-11 |
| DCH Enhancements info FDD | OP |  | DCH Enhancements info FDD 10.3.6.149 |  | REL-12 |
| Control Channel DRX information | OP |  | Control Channel DRX information 1.28 Mcps TDD 10.3.6.107 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| SPS Information | OP |  | SPS information 1.28 Mcps TDD 10.3.6.110 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| MU-MIMO info | OP |  | MU-MIMO info 1.28 Mcps TDD 10.3.6.122 | This IE is used for 1.28 Mcps TDD only | REL-10 |
| Non-rectangular resource allocation indicator | OP |  | Enumerated(TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the non-rectangular resource allocation is not used. | REL-12 |
| Non-rectangular resource specific timeslots set | CV-Non-rectangularResourceAllocation |  | Bit string (7) | For 1.28 Mcps TDD only. The value 1 of a bit indicates the corresponding timeslot in which the channelization codes are assigned by physical control channel. The value 0 of a bit indicates the corresponding timeslot in which all channelization codes are assigned when the timeslot is scheduled to the UE. The first/leftmost bit of the bit string is for TS0. | REL-12 |
| Uplink radio resources |  |  |  |  |  |
| Maximum allowed UL TX power | MD |  | Maximum allowed UL TX power 10.3.6.39 | Default value is the existing maximum UL TX power |  |
| Uplink DPCH info | OP |  | Uplink DPCH info 10.3.6.88 |  |  |
| E-DCH Info | OP |  | E-DCH Info 10.3.6.97 |  | REL-6 |
| Uplink secondary cell info FDD | OP |  | Uplink secondary cell info FDD 10.3.6.115 | FDD only | REL-9 |
| Multi-carrier E-DCH Info for LCR TDD | OP |  | Multi-carrier E-DCH Info for LCR TDD 10.3.6.97a | 1.28Mcps TDD only | REL-10 |
| Uplink CLTD info FDD | OP |  | Uplink CLTD info FDD 10.3.6.125 | FDD only | REL-11 |
| Uplink OLTD info FDD | OP |  | Uplink OLTD info FDD 10.3.6.126 | FDD only | REL-11 |
| Other TTI E-DCH Configuration Information | OP |  | Other TTI E-DCH Configuration Information 10.3.6.152 | FDD only | REL-12 |
| Filtered UE power headroom reporting information | OP |  | Filtered UE power headroom reporting information 10.3.7.140 | FDD only | REL-12 |
| Downlink radio resources |  |  |  |  |  |
| Downlink HS-PDSCH Information | OP |  | Downlink HS-PDSCH Information 10.3.6.23a |  | REL-5 |
| Downlink information common for all radio links | OP |  | Downlink information common for all radio links 10.3.6.24 |  |  |
| Downlink information per radio link list | OP | 1 to <maxRL> |  | Send downlink information for each radio link |  |
| >Downlink information for each radio link | MP |  | Downlink information for each radio link 10.3.6.27 |  |  |
| Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-8 |
| Additional downlink secondary cell info list FDD | OP | 2 |  |  | REL-10 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-10 |
| Additional downlink secondary cell info list FDD 2 | OP | 4 |  |  | REL-11 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-11 |
| Common E-RGCH info FDD | OP |  | Common E-RGCH info FDD 10.3.6.138 | FDD only | REL-11 |
| DPCCH2 info FDD | OP |  | DPCCH2 info FDD 10.3.6.148 | FDD only | REL-12 |
| MBMS PL Service Restriction Information | OP |  | Enumerated (TRUE) |  | REL-6 |
| CELL_DCH measurement occasion info LCR | OP |  | CELL_DCH measurement occasion info LCR 10.3.7.126 |  | REL-9 |

| Condition | Explanation |
| --- | --- |
| FACH_PCH | This IE is mandatory default when a transition from CELL_DCH to CELL_FACH, URA_PCH or CELL_PCH is requested by the message and is not needed otherwise. |
| Non-rectangularResourceAllocation | This IE is optionally present if non-rectangular resource allocation is configured. Otherwise, it is not needed. |

### 10.2.51	TRANSPORT CHANNEL RECONFIGURATION COMPLETE

This message is sent from the UE when a transport channel reconfiguration has been done.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE  UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Uplink integrity protection activation info | OP |  | Integrity protection activation info 10.3.3.17 |  |  |
| CHOICE mode | OP |  |  |  |  |
| >FDD |  |  |  | (no data) |  |
| >TDD |  |  |  |  |  |
| >>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>Uplink Timing Advance | OP |  | Uplink Timing Advance 10.3.6.95 |  |  |
| >>>>Extended Uplink Timing Advance | OP |  | Extended Uplink Timing Advance 10.3.6.95a |  | REL-7 |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Extended Uplink Timing Advance | OP |  | Extended Uplink Timing Advance 10.3.6.95a |  | REL-7 |
| >>>1.28 Mcps TDD |  |  |  | (no data) | REL-4 |
| Other information elements |  |  |  |  | REL-7 |
| Deferred measurement control reading | OP |  | Enumerated (TRUE) | Indicates the UE has not read SIB11, SIB11bis, SIB11ter (if supported) and SIB12, if available. | REL-7 |
| RB Information elements |  |  |  |  |  |
| COUNT-C activation time | OP |  | Activation time 10.3.3.1 | Used for radio bearers mapped on RLC-TM. Only applicable if the UE is moving to CELL_DCH state due to this procedure |  |
| Uplink counter synchronisation info | OP |  |  |  |  |
| >RB with PDCP information list | OP | 1 to <maxRBallRABs> |  |  |  |
| >>RB with PDCP information | MP |  | RB with PDCP information 10.3.4.22 |  |  |
| >START list | MP | 1 to <maxCNdomains> |  | START [40] values for all CN domains. |  |
| >>CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >>START | MP |  | START 10.3.3.38 | START value to be used in this CN domain. |  |

### 10.2.52	TRANSPORT CHANNEL RECONFIGURATION FAILURE

This message is sent by UE if the configuration given by UTRAN is unacceptable or if the UE failed to establish the physical channel(s).

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| Failure cause | MP |  | Failure cause and error information 10.3.3.14 |  |

### 10.2.53	TRANSPORT FORMAT COMBINATION CONTROL

This message is sent by UTRAN to control the uplink transport format combination within the allowed transport format combination set. This message has different structures depending if the message is sent on transparent (TM) or non-transparent mode (AM or UM).

RLC-SAP: AM or UM

Logical channel: DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| TrCH information elements |  |  |  |  |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  | (no data) |  |
| >TDD |  |  |  |  |  |
| >>TFCS Id | OP |  | Transport Format Combination Set Identity 10.3.5.21 |  |  |
| DPCH/PUSCH TFCS in uplink | MP |  | Transport Format Combination subset 10.3.5.22 | NOTE 1 |  |
| Activation time for TFC subset | MD |  | Activation time 10.3.3.1 | Default value is "now" |  |
| TFC Control duration | OP |  | TFC Control duration 10.3.6.80 |  |  |
| UL AMR rate | OP |  | Enumerated(t0, t1, t2, t3, t4, t5, t6, t7, t8) | Indicates the bit rate as defined in [62] and [63]. | REL-8 |
| NOTE 1: If the IE “UL AMR rate” is included, no DCH is configured on the uplink and the IE “DPCH/PUSCH TFCS in uplink” is not set to “Full transport format combination set” the UE behaviour is unspecified. |  |  |  |  |  |

In case of transparent mode signalling the following message structure shall be used:

RLC-SAP: TM

Logical channel: DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TFC subset identity | MP |  | INTEGER (0..7) |  |

The encoding of this message is specified in subclause 12.4.1.1.

### 10.2.54	TRANSPORT FORMAT COMBINATION CONTROL FAILURE

This message is sent to indicate that a received TRANSPORT FORMAT COMBINATION CONTROL message could not be handled by the UE.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| Failure cause | MP |  | Failure cause and error information 10.3.3.14 |  |

### 10.2.55	UE CAPABILITY ENQUIRY

The UE CAPABILITY ENQUIRY is used by the UTRAN to enquire inter-RAT classmarks from the UE.

RLC-SAP: AM or UM

Logical channel: DCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 | Integrity check info is included if integrity protection is applied |
| Capability update requirement | MP |  | Capability update requirement 10.3.3.2 |  |

### 10.2.56	UE CAPABILITY INFORMATION

This message is sent by UE to convey UE specific capability information to the UTRAN.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE  UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | OP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 | Integrity check info is included if integrity protection is applied |
| UE radio access capability | OP |  | UE radio access capability 10.3.3.42 |  |
| UE radio access capability extension | OP |  | UE radio access capability extension 10.3.3.42a |  |
| Other information elements |  |  |  |  |
| UE system specific capability | OP | 1 to <maxInterSysMessages> |  |  |
| >Inter-RAT UE radio access capability | MP |  | Inter-RAT UE radio access capability 10.3.8.7 |  |

### 10.2.57	UE CAPABILITY INFORMATION CONFIRM

This message is sent by UTRAN to confirm that UE capability information has been received.

RLC-SAP: AM or UM

Logical channel: DCCH

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 | Integrity check info is included if integrity protection is applied |
| Acceptance of requested change of capability | OP |  | Enumerated (Refused, Accepted, Accepted with reconfiguration to follow) | Included if the message was sent in response to a UE request for a capability change in connected mode |

### 10.2.57a	UE INFORMATION REQUEST

This message is used by UTRAN to request data such as Logged Measurements from the UE.

RLC-SAP: AM

Logical channel: DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  | REL-10 |
| UE information elements |  |  |  |  | REL-10 |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  | REL-10 |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  | REL-10 |
| Other Information Elements |  |  |  |  | REL-10 |
| Logged Measurements Report Request | OP |  | Enumerated (TRUE) | This IE indicates that the UE shall report logged measurements to the network if available. | REL-10 |
| Logged ANR Report Request | OP |  | Enumerated (TRUE) |  | REL-10 |
| Connection Establishment Failure Request | OP |  | Enumerated (TRUE) |  | REL-11 |

### 10.2.57b	UE INFORMATION RESPONSE

This message is used by the UE to transfer data such as Logged Measurements to the UTRAN.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  | REL-10 |
| UE information elements |  |  |  |  | REL-10 |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  | REL-10 |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  | REL-10 |
| Other Information Elements |  |  |  |  | REL-10 |
| Logged Meas Report | OP |  | Logged Meas Report 10.3.7.131 |  | REL-10 |
| Logged ANR Report Info | OP |  | Logged ANR Report Info 10.3.7.42b |  | REL-10 |
| Connection Establishment Failure Report | OP |  | Connection Establishment Failure Report 10.3.7.132a |  | REL-11 |

### 10.2.58	UPLINK DIRECT TRANSFER

This message is used to transfer NAS messages for an existing signalling connection.

RLC-SAP: AM

Logical channel: DCCH

Direction: UE ->UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 | Integrity check info is included if integrity protection is applied |
| CN information elements |  |  |  |  |
| CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |
| NAS message | MP |  | NAS message 10.3.1.8 |  |
| Measurement information elements |  |  |  |  |
| Measured results on RACH | OP |  | Measured results on RACH 10.3.7.45 |  |

### 10.2.59	UPLINK PHYSICAL CHANNEL CONTROL

NOTE:	Only for TDD.

This message is used to transfer uplink physical channel parameters to the UE.

RLC-SAP: AM or UM

Logical channel: DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | OP |  | Integrity check info 10.3.3.16 |  |  |
| PhyCH information elements |  |  |  |  |  |
| CCTrCH power control info | OP |  | CCTrCH power control info 10.3.6.8 | Power control information for one CCTrCH |  |
| Special Burst Scheduling | OP |  | Special Burst Scheduling 10.3.6.75a | UL Special Burst generation period in radio frames |  |
| CHOICE TDD option | MP |  |  |  | REL-4 |
| >3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>Alpha | OP |  | Alpha 10.3.6.5 |  |  |
| >>Timing Advance Control | OP |  | UL Timing Advance Control 10.3.6.96 |  |  |
| >>PRACH Constant Value | OP |  | Constant value TDD 10.3.6.11a | Operator controlled PRACH Margin |  |
| >>PUSCH Constant Value | OP |  | Constant value TDD 10.3.6.11a | Operator controlled PUSCH Margin |  |
| >>UE positioning related parameters | CV-IPDLs |  |  |  | REL-4 |
| >>>IPDL-Alpha | MP |  | Alpha 10.3.6.5 |  | REL-4 |
| >>>Max power increase | MP |  | Integer (0..3) | In dB | REL-4 |
| >> HS-SICH power control info | OP |  | HS-SICH Power Control Info 10.3.6.36b | Only applies to TDD 3.84 Mcps | REL-5 |
| >7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>Alpha | OP |  | Alpha 10.3.6.5 |  | REL-7 |
| >>Timing Advance Control | OP |  | UL Timing Advance Control 10.3.6.96 |  | REL-7 |
| >>PRACH Constant Value | OP |  | Constant value TDD 10.3.6.11a | Operator controlled PRACH Margin | REL-7 |
| >>PUSCH Constant Value | OP |  | Constant value TDD 10.3.6.11a | Operator controlled PUSCH Margin | REL-7 |
| >>UE positioning related parameters | CV-IPDLs |  |  |  | REL-4 |
| >>>IPDL-Alpha | MP |  | Alpha 10.3.6.5 |  | REL-4 |
| >>>Max power increase | MP |  | Integer (0..3) | In dB | REL-4 |
| >> HS-SICH power control info | OP |  | HS-SICH Power Control Info 10.3.6.36b | Only applies to TDD 7.68 Mcps | REL-5 |
| >1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>Uplink synchronisation parameters | MD |  |  | Default: Uplink synchronisation step size 1. Uplink synchronisation frequency 1. | REL-4 |
| >>>Uplink synchronisation step size | MP |  | Integer(1..8) | This parameter specifies the step size to be used for the adjustment of the uplink transmission timing | REL-4 |
| >>>Uplink synchronisation frequency | MP |  | Integer(1..8) | This parameter specifies the frequency of the adjustment of the uplink transmission timing | REL-4 |
| >>PRXHS-SICH | OP |  | Integer (-120..-58 by step of 1) | In dBm. Desired power level for HS-SICH. | REL-6 |
| >>TPC step size | OP |  | Integer (1, 2, 3) | In dB. For HS-SICH | REL-6 |

| Condition | Explanation |
| --- | --- |
| IPDLs | This IE is present only if idle periods are applied |

### 10.2.60	URA UPDATE

This message is used by the UE to initiate a URA update procedure.

RLC-SAP: TM

Logical channel: CCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| U-RNTI | MP |  | U-RNTI 10.3.3.47 |  |  |
| RRC transaction identifier | CV- ProtErr |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| URA update cause | MP |  | URA update cause 10.3.3.46 |  |  |
| Protocol error indicator | MD |  | Protocol error indicator 10.3.3.27 | Default value is FALSE |  |
| HS-PDSCH in CELL_PCH and URA_PCH | OP |  | Enumerated (TRUE) | This IE indicates whether the UE supports HS-PDSCH reception in CELL_PCH and URA_PCH states. The absence of this IE indicates that the UE does not support HS-PDSCH reception in CELL_PCH or URA_PCH states. | REL-7 |
| HS-PDSCH in CELL_FACH | OP |  | Enumerated (TRUE) | This IE indicates whether the UE supports HS-PDSCH reception in CELL_FACH state. The absence of this IE indicates that the UE does not support HS-PDSCH reception in CELL_FACH state. | REL-7 |
| Support of HS-DSCH DRX operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-DSCH DRX operation in CELL_FACH state. | REL-8 |
| Support of common E-DCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support E-DCH enhanced random access in CELL_FACH state and Idle mode. | REL-8 |
| Support of MAC-i/is | OP |  | Enumerated (TRUE) | This IE indicates whether the UE supports MAC-i/is operation. | REL-8 |
| Support for Two DRX schemes in URA_PCH and CELL_PCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Two DRX schemes in URA_PCH and CELL_PCH | REL-7 |
| Other information elements |  |  |  |  |  |
| Protocol error information | CV-ProtErr |  | Protocol error information 10.3.8.12 |  |  |
| Other Information elements |  |  |  |  | Rel-10 |
| Logged Meas Available | OP |  | Enumerated (TRUE) | Indicates the UE has logged measurements to report to the network | Rel-10 |
| ANR Logging Results Available | OP |  | Enumerated (TRUE) | True indicates the UE has ANR logging results to report to the Network. | REL-10 |

| Condition | Explanation |
| --- | --- |
| ProtErr | The IE is mandatory present if the IE "Protocol error indicator" has the value TRUE and not needed otherwise. |

### 10.2.61	URA UPDATE CONFIRM

This message confirms the URA update procedure and can be used to reallocate new RNTI information for the UE valid after the URA update.

RLC-SAP: UM

Logical channel: CCCH or DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| U-RNTI | CV-CCCH |  | U-RNTI 10.3.3.47 |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 | Integrity check info is included if integrity protection is applied |  |
| Integrity protection mode info | OP |  | Integrity protection mode info 10.3.3.19 | The UTRAN should not include this IE unless it is performing an SRNS relocation |  |
| Ciphering mode info | OP |  | Ciphering mode info 10.3.3.5 | The UTRAN should not include this IE unless it is performing an SRNS relocation and a change in ciphering algorithm |  |
| New U-RNTI | OP |  | U-RNTI 10.3.3.47 |  |  |
| New C-RNTI | OP |  | C-RNTI 10.3.3.8 |  |  |
| New H-RNTI | OP |  | H-RNTI 10.3.3.14a |  | REL-8 |
| New Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-8 |
| RRC State Indicator | MP |  | RRC State Indicator 10.3.3.35a |  |  |
| UTRAN DRX cycle length coefficient | OP |  | UTRAN DRX cycle length coefficient 10.3.3.49 |  |  |
| CN Information Elements |  |  |  |  |  |
| CN Information info | OP |  | CN Information info 10.3.1.3 |  |  |
| UTRAN mobility information elements |  |  |  |  |  |
| URA identity | OP |  | URA identity 10.3.2.6 |  |  |
| RNC support for change of UE capability | OP |  | Boolean | Should be included if the message is used to perform an SRNS relocation | REL-7 |
| RB information elements |  |  |  |  |  |
| Downlink counter synchronisation info | OP |  |  |  |  |
| >RB with PDCP information list | OP | 1 to <maxRBallRABs> |  |  |  |
| >>RB with PDCP information | MP |  | RB with PDCP information 10.3.4.22 | This IE is needed for each RB having PDCP in the case of lossless SRNS relocation |  |
|  | OP |  |  |  | REL-5 |
| >>PDCP context relocation info | OP |  | PDCP context relocation info 10.3.4.1a | This IE is needed for each RB having PDCP and performing PDCP context relocation | REL-5 |

| Condition | Explanation |
| --- | --- |
| CCCH | This IE is mandatory present when CCCH is used and not needed otherwise. |

### 10.2.62	UTRAN MOBILITY INFORMATION

This message is used by UTRAN to allocate a new RNTI and to convey other UTRAN mobility related information to a UE.

RLC-SAP: AM or UM

Logical channel: DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE Information Elements |  |  |  |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity protection mode info | OP |  | Integrity protection mode info 10.3.3.19 | The UTRAN should not include this IE unless it is performing an SRNS relocation |  |
| Ciphering mode info | OP |  | Ciphering mode info 10.3.3.5 | The UTRAN should not include this IE unless it is performing an SRNS relocation and a change in ciphering algorithm |  |
| New U-RNTI | OP |  | U-RNTI 10.3.3.47 |  |  |
| New C-RNTI | OP |  | C-RNTI 10.3.3.8 |  |  |
| New H-RNTI | OP |  | H-RNTI 10.3.3.14a |  | REL-7 |
| New Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-8 |
| UE Timers and constants in connected mode | OP |  | UE Timers and constants in connected mode 10.3.3.43 |  |  |
| Dedicated priority information | OP |  | Dedicated priority information 10.3.2.7 |  | REL-8 |
| Measurement Release Enhancement | OP |  | Measurement Release Enhancement 10.3.7.46a |  | REL-12 |
| CN Information Elements |  |  |  |  |  |
| CN Information info | OP |  | CN Information info full 10.3.1.3a |  |  |
| Domain Specific Access Restriction Parameters Update | CV-Cell_DCH |  | Domain Specific Access Restriction Parameters Update10.3.1.3c |  | REL-12 |
| Paging Permission with Access Control Parameters Update | CV-Cell_DCH |  | Paging Permission with Access Control Parameters Update 10.3.1.10a |  | REL-12 |
| UTRAN Information Elements |  |  |  |  |  |
| URA identity | OP |  | URA identity 10.3.2.6 |  |  |
| RNC support for change of UE capability | OP |  | Boolean | Should be included if the message is used to perform an SRNS relocation | REL-7 |
| RB Information elements |  |  |  |  |  |
| Downlink counter synchronisation info | OP |  |  |  |  |
| >RB with PDCP information list | OP | 1 to <maxRBallRABs> |  |  |  |
| >>RB with PDCP information | MP |  | RB with PDCP information 10.3.4.22 | This IE is needed for each RB having PDCP in the case of lossless SRNS relocation |  |
|  | OP |  |  |  | REL-5 |
| >>PDCP context relocation info | OP |  | PDCP context relocation info 10.3.4.1a | This IE is needed for each RB having PDCP and performing PDCP context relocation | REL-5 |
| WLAN Information Elements |  |  |  |  |  |
| Dedicated WLAN Offload Information | OP |  | Dedicated WLAN Offload Information 10.3.2.10 |  | REL-12 |

| Condition | Explanation |
| --- | --- |
| Cell_DCH | This IE is optionally present when UE is in CELL_DCH state and not needed otherwise. |

### 10.2.63	UTRAN MOBILITY INFORMATION CONFIRM

This message is used to confirm the new UTRAN mobility information for the UE.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Uplink integrity protection activation info | OP |  | Integrity protection activation info 10.3.3.17 |  |  |
| Other Information Elements |  |  |  |  | REL-7 |
| Deferred measurement control reading | OP |  | Enumerated (TRUE) | Indicates the UE has not read SIB11, SIB11bis, SIB11ter (if supported) and SIB12. | REL-7 |
| Logged Meas Available | OP |  | Enumerated (TRUE) | Indicates the UE has logged measurements to report to the network | REL-10 |
| RB Information elements |  |  |  |  |  |
| COUNT-C activation time | OP |  | Activation time 10.3.3.1 | Used for radio bearers mapped on RLC-TM. Only applicable if the UE is moving to CELL_DCH state due to this procedure |  |
| Uplink counter synchronisation info | OP |  |  |  |  |
| >RB with PDCP information list | OP | 1 to <maxRBallRABs> |  | This IE is needed for each RB having PDCP in the case of lossless SRNS relocation |  |
| >>RB with PDCP information | MP |  | RB with PDCP information 10.3.4.22 |  |  |
| >START list | MP | 1 to <maxCNdomains> |  | START [40] values for all CN domains. |  |
| >>CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >>START | MP |  | START 10.3.3.38 | START value to be used in this CN domain. |  |

### 10.2.64	UTRAN MOBILITY INFORMATION FAILURE

This message is sent to indicate a failure to act on a received UTRAN MOBILITY INFORMATION message.

RLC-SAP: AM

Logical channel: DCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |
| UE information elements |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |
| Failure cause | MP |  | Failure cause and error information 10.3.3.14 |  |

## 10.3	Information element functional definitions

### 10.3.1	CN Information elements

#### 10.3.1.1	CN domain identity

Identifies the type of core network domain.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN domain identity | MP |  | Enumerated (CS domain, PS domain) |  |

#### 10.3.1.2	CN Domain System Information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |
| CHOICE CN Type | MP |  |  |  |
| >GSM-MAP |  |  |  |  |
| >>CN domain specific NAS system information | MP |  | NAS system information (GSM-MAP) 10.3.1.9 |  |
| >ANSI-41 |  |  |  |  |
| >>CN domain specific NAS system information | MP |  | ANSI-41 NAS system information, 10.3.9.4 |  |
| CN domain specific DRX cycle length coefficient | MP |  | CN domain specific DRX cycle length coefficient, 10.3.3.6 |  |

#### 10.3.1.3	CN Information info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PLMN identity | OP |  | PLMN identity 10.3.1.11 |  |  |
| CN common GSM-MAP NAS system information | OP |  | NAS system information (GSM-MAP) 10.3.1.9 |  |  |
| CN domain related information | OP | 1 to <maxCNdomains> |  |  |  |
| >CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >CN domain specific GSM-MAP NAS system info | MP |  | NAS system information (GSM-MAP) 10.3.1.9 |  |  |
| Primary PLMN identity | OP |  | PLMN identity 10.3.1.11 |  | REL-6 |

#### 10.3.1.3a	CN Information info full

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PLMN identity | OP |  | PLMN identity 10.3.1.11 |  |  |
| CN common GSM-MAP NAS system information | OP |  | NAS system information (GSM-MAP) 10.3.1.9 |  |  |
| CN domain related information | OP | 1 to <maxCNdomains> |  |  |  |
| >CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >CN domain specific GSM-MAP NAS system info | MP |  | NAS system information (GSM-MAP) 10.3.1.9 |  |  |
| >CN domain specific DRX cycle length coefficient | MP |  | CN domain specific DRX cycle length coefficient, 10.3.3.6 |  |  |
| Primary PLMN identity | OP |  | PLMN identity 10.3.1.11 |  | REL-6 |

#### 10.3.1.3b	Domain Specific Access Restriction

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE restriction status | MP |  |  |  | REL-6 |
| >no restriction |  |  |  | (no data) This means ‘no Access Class is barred’ | REL-6 |
| >restriction |  |  |  |  | REL-6 |
| >>Domain Specific Access Class Barred List | MD | <MaxAC> |  | The first instance of the parameter corresponds to Access Class 0, the second to Access Class 1 and so on up to Access Class 15. UE reads this IE of its access class stored in SIM. The default value is the IE "Access Class Barred list" contained in the IE "Cell Access Restriction" of the System Information Block Type 3. | REL-6 |
| >>>Access Class Barred | MP |  | Enumerated(barred, not barred) |  | REL-6 |

#### 10.3.1.3c	Domain Specific Access Restriction Parameters

This IE specifies domain specific access class restriction parameters for CS and PS domain.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CS Domain Specific Access Restriction | MP |  | Domain Specific Access Restriction 10.3.1.3b | This IE contains Domain Specific Access Restriction Parameters for CS domain. | REL-6 |
| PS Domain Specific Access Restriction | MP |  | Domain Specific Access Restriction 10.3.1.3b | This IE contains Domain Specific Access Restriction Parameters for PS domain. | REL-6 |

#### 10.3.1.3d	Domain Specific EAB Parameters

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CS Domain Specific EAB Parameters | OP |  | EAB Configuration 10.3.1.3e | This IE contains Domain Specific EAB Parameters for CS domain. | REL-11 |
| PS Domain Specific EAB Parameters | OP |  | EAB Configuration 10.3.1.3e | This IE contains Domain Specific EAB Parameters for PS domain. | REL-11 |
| Common EAB Parameters | CV-Common |  | EAB Configuration 10.3.1.3e | This IE contains Domain Specific EAB Parameters for both CS domain and PS domain. | REL-11 |

| Condition | Explanation |
| --- | --- |
| Common | The IE is mandatory present if neither the IE "CS Domain Specific EAB Parameters" nor the IE "PS Domain Specific EAB Parameters" is present. Otherwise, the IE is not needed. |

#### 10.3.1.3e	EAB Configuration

| Information Element/ Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| EAB Category | MP |  | Enumerated(a, b, c) | This IE identifies the category of UEs for which EAB applies. Value ‘a’ corresponds to all UEs, value ‘b’ corresponds to the UEs that are neither in their HPLMN nor in a PLMN that is equivalent to it, and value ‘c’ corresponds to the UEs that are neither in the PLMN listed as most preferred PLMN of the country where the UEs are roaming in the operator-defined PLMN selector list on the SIM, nor in their HPLMN nor in a PLMN that is equivalent to their HPLMN, see [75]. | REL-11 |
| EAB Access Class Barred List | MP | 10 |  | The first instance of this IE corresponds to Access Class 0, the second instance corresponds to Access Class 1, and so on up to Access Class 9. UE reads its Access Class stored in the SIM. | REL-11 |
| >Access Class Barred | MP |  | Enumerated(barred, not barred) |  | REL-11 |

#### 10.3.1.4	IMEI

This IE contains an International Mobile Equipment Identity. Setting specified in [11].

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IMEI | MP | 15 |  | The first element contains the first IMEI digit, the second element the second IMEI digit and so on. |
| >IMEI digit | MP |  | INTEGER(0..15) |  |

#### 10.3.1.5	IMSI (GSM-MAP)

This IE contains an International Mobile Subscriber Identity, used towards a GSM-MAP type of PLMN. Setting specified in [11].

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IMSI | MP | 6 to 21 |  | The first element contains the first IMSI digit, the second element the second IMSI digit and so on. Although normally upto 15 digits are used for this IE, a bigger length is used to support future extension. |
| >IMSI digit | MP |  | INTEGER(0..9) |  |

#### 10.3.1.6	Intra Domain NAS Node Selector

This IE carries information to be used to route the establishment of a signalling connection to a CN node within a CN domain.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE version | MP |  |  |  |
| >R99 |  |  |  | This choice shall also be used by mobiles that are compliant to this version of the protocol |
| >>CHOICE CN type | MP |  |  |  |
| >>>GSM-MAP |  |  |  |  |
| >>>>CHOICE Routing basis | MP |  |  |  |
| >>>>>local (P)TMSI |  |  |  | TMSI allocated in the current LA or PTMSI allocated in the current RA |
| >>>>>>Routing parameter | MP |  | Bit string (10) | The TMSI/ PTMSI consists of 4 octets (32bits). This can be represented by a string of bits numbered from b0 to b31, with bit b0 being the least significant The "Routing parameter" bit string consists of bits b14 through b23 of the TMSI/ PTMSI. The first/leftmost/most significant bit of the bit string contains bit b23 of the TMSI/PTMSI. |
| >>>>>(P)TMSI of same PLMN, different (RA)LA; or, PTMSI mapped from a valid GUTI allocated by same PLMN |  |  |  | TMSI allocated in another LA of this PLMN or PTMSI allocated in another RA this PLMN; or, PTMSI mapped from a valid GUTI [11] (GUTI allocated in same PLMN) |
| >>>>>>Routing parameter | MP |  | Bit string (10) | The TMSI/ PTMSI consists of 4 octets (32bits). This can be represented by a string of bits numbered from b0 to b31, with bit b0 being the least significant The "Routing parameter" bit string consists of bits b14 through b23 of the TMSI/ PTMSI. The first/leftmost/most significant bit of the bit string contains bit b23 of the TMSI/ PTMSI. |
| >>>>>(P)TMSI of different PLMN; or, PTMSI mapped from a valid GUTI allocated by a different PLMN |  |  |  | TMSI or a PTMSI allocated in another PLMN; or, PTMSI mapped from a valid GUTI [11] (GUTI allocated in another PLMN) |
| >>>>>>Routing parameter | MP |  | Bit string (10) | The TMSI/ PTMSI consists of 4 octets (32bits). This can be represented by a string of bits numbered from b0 to b31, with bit b0 being the least significant. The "Routing parameter" bit string consists of bits b14 through b23 of the TMSI/ PTMSI. The first/leftmost/most significant bit of the bit string contains bit b23 of the TMSI/ PTMSI. |
| >>>>>IMSI(response to IMSI paging) |  |  |  | NAS identity is IMSI |
| >>>>>>Routing parameter | MP |  | Bit string (10) | The "Routing parameter" bit string consists of DecimalToBinary [(IMSI div 10) mod 1000]. The first/leftmost bit of the bit string contains the most significant bit of the result. |
| >>>>>IMSI(cause UE initiated event) |  |  |  | NAS identity is IMSI |
| >>>>>>Routing parameter | MP |  | Bit string (10) | The "Routing parameter" bit string consists of DecimalToBinary [(IMSI div 10) mod 1000]. The first/leftmost bit of the bit string contains the most significant bit of the result. |
| >>>>>IMEI |  |  |  | NAS parameter is IMEI |
| >>>>>>Routing parameter | MP |  | Bit string (10) | The "Routing parameter" bit string consists of DecimalToBinary [(IMEI div 10) mod 1000]. The first/leftmost bit of the bit string contains the most significant bit of the result. |
| >>>>>Spare 1 |  |  | Bit string (10) | This choice shall not be used in this version |
| >>>>>Spare 2 |  |  | Bit string (10) | This choice shall not be used in this version |
| >>>ANSI-41 |  |  | Bit string (14) | All bits shall be set to 0 |
| >Later |  |  | Bit string(15) | This bit string shall not be sent by mobiles that are compliant to this version of the protocol. |

#### 10.3.1.7	Location Area Identification

Identifies uniquely a location area for a GSM-MAP type of PLMN. Setting specified in [5].

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN identity | MP |  | PLMN identity 10.3.1.11 |  |
| LAC | MP |  | Bit string(16) | The first/leftmost bit of the bit string contains the most significant bit of the LAC.. |

#### 10.3.1.7oa	Location/Registration Parameters

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE restriction status | MP |  |  |  | REL-8 |
| >no restriction |  |  |  | (no data) This means ‘no Access Class is barred’ | REL-8 |
| >restriction |  |  |  |  | REL-8 |
| >>Location/Registration Access Class Barred List | MP | 15 |  | The first ten instance of the parameter corresponds to Access Class 0 to 9, and eleventh to fifteenth instance of the parameter corresponds to Access Class 11 to 15, respectively. UE reads this IE of its access class stored in SIM (except for Access Class 10). | REL-8 |
| >>>Access Class Barred | MP |  | Enumerated(barred, not barred) |  | REL-8 |

#### 10.3.1.7a	Multiple PLMN List

This information element identifies the multiple Public Land Mobile Networks (for a GSM-MAP type of PLMN) of a cell in a shared network.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MIB PLMN Identity | MP |  | Boolean | The PLMN identity IE 10.3.1.11, broadcasted in the MIB, shall be included in the multiple PLMN list if and only if this IE is TRUE. | REL-6 |
| Multiple PLMNs | MP | 1 to 5 |  |  | REL-6 |
| >PLMN identity with Optional MCC |  |  | PLMN identity with Optional MCC 10.3.1.11a |  | REL-6 |

#### 10.3.1.8	NAS message

A non-access stratum message to be transferred transparently through UTRAN.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NAS message | MP |  | Octet string (1..4095) | The first octet contains octet 1 [17] of the NAS message, the second octet contains octet 2 of the NAS message and so on. |

#### 10.3.1.9	NAS system information (GSM-MAP)

This information element contains system information that belongs to the non-access stratum for a GSM-MAP type of PLMN. This information is transparent to RRC. It may contain either information specific to one CN domain (CS or PS) or information common for both CN domains.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| GSM-MAP NAS system information | MP |  | Octet string(1..8 ) | The first octet contains octet 1 [5] of the NAS system information element, the second octet contains octet 2 of the NAS system information element and so on. |

#### 10.3.1.10	Paging record type identifier

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging record type identifier | MP |  | Enumerated (IMSI (GSM-MAP), TMSI (GSM-MAP)/ P-TMSI, IMSI (DS-41), TMSI (DS-41)) |  |

#### 10.3.1.10a	Paging Permission with Access Control Parameters

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Paging Response Restriction Indication | MP |  | Enumerated (All, CS, PS, None) |  | REL-8 |
| Location/Registration Restriction Indicator | MP |  | Enumerated (All, CS, PS) |  | REL-8 |
| Location/Registration | MP |  | Location /Registration Parameters 10.3.1.7oa |  | REL-8 |

#### 10.3.1.11	PLMN identity

This information element identifies a Public Land Mobile Network for a GSM-MAP type of PLMN. Setting of digits is defined in [11].

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MCC | MP | 3 |  | The first element contains the first MCC digit, the second element the second MCC digit and so on. |
| >MCC digit | MP |  | INTEGER(0..9) |  |
| MNC | MP | 2 to 3 |  | The first element contains the first MNC digit, the second element the second MNC digit and so on. |
| >MNC digit | MP |  | INTEGER(0..9) |  |

#### 10.3.1.11a	PLMN identity with Optional MCC

This information element is a PLMN identity in subclause 10.3.1.11 where MCC is optional. It is used in a shared network and inserted in the Multiple PLMN List in subclause 10.3.1.7a.

| Information Element/ Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MCC | MD | 3 |  | The first element contains the first MCC digit, the second element the second MCC digit and so on. | REL-6 |
| >MCC digit | MP |  | Integer (0..9) |  | REL-6 |
| MNC | MP | 2 to 3 |  | The first element contains the first MNC digit, the second element the second MNC digit and so on. | REL-6 |
| >MNC digit | MP |  | Integer (0..9) |  | REL-6 |

#### 10.3.1.12	PLMN Type

Identifies the type of Public Land Mobile Network (PLMN). This IE shall be used to control the interpretation of network dependent messages and information elements in the RRC protocol.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Type | MP |  | Enumerated (GSM-MAP, ANSI-41, GSM-MAP and ANSI-41) | One spare value is needed. |

#### 10.3.1.13	P-TMSI (GSM-MAP)

This IE contains a Packet Temporary Mobile Subscriber Identity, used towards a GSM-MAP type of PLMN.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| P-TMSI | MP |  | Bit string (32) | Setting specified in [11]. The first/leftmost bit of the bit string contains the most significant bit of the P-TMSI. |

#### 10.3.1.14	RAB identity

This information element uniquely identifies a radio access bearer within a CN domain.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE RAB identity type | MP |  |  |  |
| >RAB identity (GSM-MAP) |  |  | Bit string (8) | Formatted according to [5]. The first/leftmost bit of the bit string contains the most significant bit of the RAB identity. In case of a radio bearer setup for an MBMS selected service, the RAB identity is set to the value reserved for ‘MBMS Broadcast mode’, see [5]. |
| >RAB identity (ANSI-41) |  |  | Bit string (8) | The first/leftmost bit of the bit string contains the most significant bit of the RAB identity. |

| CHOICE NAS binding info type | Condition under which the given RAB identity type is chosen |
| --- | --- |
| RAB identity (GSM-MAP) | PLMN is of type GSM-MAP |
| RAB identity (ANSI-41) | PLMN is of type ANSI-41 |

#### 10.3.1.15	Routing Area Code

Identifies a routing area within a location area for a GSM-MAP type of PLMN.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Routing Area Code | MP |  | Bit string(8) | Setting specified in [11]. The first/leftmost bit of the bit string contains the most significant bit of the Routing Area Code. |

#### 10.3.1.16	Routing Area Identification

Identifies uniquely a routing area for a GSM-MAP type of PLMN. Setting specified in [11].

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LAI | MP |  | Location area identification 10.3.1.7 |  |
| RAC | MP |  | Routing area code 10.3.1.15 |  |

#### 10.3.1.17	TMSI (GSM-MAP)

This IE contains a Temporary Mobile Subscriber Identity, used towards a GSM-MAP type of PLMN.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TMSI (GSM-MAP) | MP |  | Bit string (32) | Setting specified in [11]. The first/leftmost bit of the bit string contains the most significant bit of the TMSI. |

### 10.3.2	UTRAN mobility Information elements

#### 10.3.2.1	Cell Access Restriction

Indicates the restrictions to cell access.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Cell Barred | MP |  | Enumerated(not barred, barred) | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 and the value in this IE is different from “barred” the UE behaviour is unspecified. |  |
| Intra-frequency cell re-selection indicator | CV-Barred |  | Enumerated(not allowed, allowed) |  |  |
| Tbarred | CV-Barred |  | Integer (10,20,40,80,160,320,640,1280) | [4] [s] |  |
| Cell Reserved for operator use | MP |  | Enumerated(reserved, not reserved) |  |  |
| Cell Reservation Extension | MP |  | Enumerated(reserved, not reserved) |  |  |
| Cell Reserved for CSG | CV-CSG |  | Enumerated(TRUE) |  | REL-8 |
| Access Class Barred list | CV-SIB3-MD | maxAC |  | Default is no access class barred is applied. The first instance of the parameter corresponds to Access Class 0, the second to Access Class 1 and so on up to Access Class 15. UE reads this IE of its access class stored in SIM. |  |
| >Access Class Barred | MP |  | Enumerated (barred, not barred) |  |  |

| Condition | Explanation |
| --- | --- |
| Barred | The IE is mandatory present if the IE "Cell Barred" has the value "Barred"; otherwise the element is not needed in the message. |
| SIB3-MD | The IE is mandatory and has a default value if the IE "Cell Access Restriction" is included in SIB 3. Otherwise the IE is not needed. |
| CSG | The IE is optional present if the cell is a CSG cell and the IE “Cell Reservation Extension” has the value "reserved". Otherwise the IE is not needed. The IE is not included in System Information Block type 4. |

#### 10.3.2.2	Cell identity

This information element identifies a cell unambiguously within a PLMN.

NOTE:	This information element may carry any implementation dependent identity that unambiguously identifies a cell within a PLMN. For GAN to UTRAN HO the Cell id is encoded as required in [9].

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell identity | MP |  | bit string(28) |  |

#### 10.3.2.3	Cell selection and re-selection info for SIB3/4

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Mapping Info | OP |  | Mapping info 10.3.2.5 | For FDD, 3.84 Mcps TDD and 7.68 Mcps TDD, this IE should not be sent. |  |
| Cell selection and reselection quality measure | MP |  | Enumerated (CPICH Ec/N0, CPICH RSCP) | Choice of measurement (CPICH Ec/N0 or CPICH RSCP) to use as quality measure Q for FDD cells. This IE is also sent to the UE in SIB11/12. Both occurrences of the IE should be set to the same value. |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Sintrasearch | OP |  | Integer (-32..20 by step of 2) | If a negative value is received the UE shall consider the value to be 0. [4] [dB] |  |
| >>Sintersearch | OP |  | Integer (-32..20 by step of 2) | If a negative value is received the UE shall consider the value to be 0. [4] [dB] |  |
| >>SsearchHCS | OP |  | Integer (-105..91 by step of 2) | If a negative value is received the UE shall consider the value to be 0. [4] [dB] |  |
| >>RAT List | OP | 1 to <maxOtherRAT> |  |  |  |
| >>>RAT identifier | MP |  | Enumerated (GSM, cdma2000) |  |  |
| >>>Ssearch,RAT | MP |  | Integer (-32..20 by step of 2) | In case the value 20 is received the UE shall consider this IE as if it was absent according to [4] If a negative value is received the UE shall consider the value to be 0. [dB] |  |
| >>>SHCS,RAT | OP |  | Integer (-105..91 by step of 2) | If a negative value is received the UE shall consider the value to be 0. [4] [dB] |  |
| >>>Slimit,SearchRAT | MP |  | Integer (-32..20 by step of 2) | If a negative value is received the UE shall consider the value to be 0. [4] [dB] |  |
| >>Qqualmin | MP |  | Integer (-24..0) | Ec/N0, [dB] |  |
| >>Qrxlevmin | MP |  | Integer (-115..-25 by step of 2) | RSCP, [dBm] |  |
| >> DeltaQrxlevmin | CV-Delta |  | Integer(-4..-2 by step of 2) | If present, the actual value of Qrxlevmin = Qrxlevmin + DeltaQrxlevmin | REL-5 |
| >>Qqualmin-offset | CV-SIB3 |  | Integer (1..16) | Ec/N0, [dB] The default value is 0 | REL-7 |
| >>Qrxlevmin-offset | CV-SIB3 |  | Integer (2..16 by step of 2) | RSCP, [dB] The default value is 0 | REL-7 |
| >>Tx Fail Params | CV-SIB3op |  |  |  | REL-12 |
| >>>ConnEst Fail Count | MP |  | Enumerated (1, 2, 3, 4) | Number of times that the UE detects RRC Connection Establishment failure on the same cell before applying ConnEst Fail Qoffset,temp. [4] | REL-12 |
| >>>Offset Validity | MP |  | Enumerated (30, 60, 120, 240, 300, 420, 600, 900) | Length of time UE uses the ConnEst Fail Qoffset,temp before the offset is removed from the cell. [4] [s] | REL-12 |
| >>>ConnEst Fail Qoffset,temp | OP |  | Integer (0..15) | Temporary cell selection/reselection offset used for this cell if RRC Connection Establishment fails. Absence of the IE means that the UE shall apply the value of infinity. [4] [dB] | REL-12 |
| >TDD |  |  |  |  |  |
| >>Sintrasearch | OP |  | Integer (-105..91 by step of 2) | If a negative value is received the UE shall consider the value to be 0. [4] [dB] |  |
| >>Sintersearch | OP |  | Integer (-105..91 by step of 2) | If a negative value is received the UE shall consider the value to be 0. [4] [dB] |  |
| >>SsearchHCS | OP |  | Integer (-105..91 by step of 2) | If a negative value is received the UE shall consider the value to be 0. [4] [dB] |  |
| >>RAT List | OP | 1 to <maxOtherRAT> |  |  |  |
| >>>RAT identifier | MP |  | Enumerated (GSM, cdma2000) |  |  |
| >>>Ssearch,RAT | MP |  | Integer (-105..91 by step of 2) | In case the value 91 is received the UE shall consider this IE as if it was absent according to [4] If a negative value is received the UE shall consider the value to be 0. [dB] |  |
| >>>SHCS,RAT | OP |  | Integer (-105..91 by step of 2) | If a negative value is received the UE shall consider the value to be 0. [4] [dB] |  |
| >>>Slimit,SearchRAT | MP |  | Integer (-105..91 by step of 2) | If a negative value is received the UE shall consider the value to be 0. [4] [dB] |  |
| >>Qrxlevmin | MP |  | Integer (-115..-25 by step of 2) | RSCP, [dBm] |  |
| >>DeltaQrxlevmin | CV-Delta |  | Integer(-4..-2 by step of 2) | If present, the actual value of Qrxlevmin = Qrxlevmin + DeltaQrxlevmin | REL-5 |
| >>Qrxlevmin-offset | CV-SIB3 |  | Integer (2..16 by step of 2) | RSCP, [dB] The default value is 0 | REL-7 |
| Qhyst1s | MP |  | Integer (0..40 by step of 2) | [4] [dB] |  |
| Qhyst1s,PCH | CV-SIB4 |  | Integer (0..40) | If present, it is used as Qhyst1s for UE in CELL_PCH or URA_PCH state [4] [dB] | REL-5 |
| Qhyst1s,FACH | CV-SIB4 |  | Integer (0..40) | If present, it is used as Qhyst1s for UE in CELL_FACH state [4] [dB] | REL-5 |
| Qhyst2s | CV-FDD-Quality-Measure |  | Integer (0..40 by step of 2) | Default value is Qhyst1s [4] [dB] |  |
| Qhyst2s,PCH | CV-SIB4-FDD-Quality-Measure |  | Integer (0..40) | If present, it is used as Qhyst2s for UE using CPICH Ec/No quality measure in CELL_PCH or URA_PCH state. Default value is Qhyst1s,PCH [4] [dB] | REL-5 |
| Qhyst2s,FACH | CV-SIB4-FDD-Quality-Measure |  | Integer (0..40) | If present, it is used as Qhyst2s for UE using CPICH Ec/No quality measure in CELL_FACH state. Default value is Qhyst1s,FACH [4] [dB] | REL-5 |
| Treselections | MP |  | Integer (0..31) | [s] |  |
| Treselections,PCH | CV-SIB4 |  | Integer (0..31) | If present, it is used as Treselections for UE in CELL_PCH or URA_PCH state [4] [s] | REL-5 |
| Treselections,FACH | CV-SIB4 |  | Real (0..6.2 by step of 0.2) | If present, it is used as Treselections for UE in CELL_FACH state [4] [s] | REL-5 |
| Speed dependent ScalingFactor for Treselection | OP |  | Real (0..1 by step of 0.1) | This IE is used by the UE in high mobility state as scaling factor for Treselections or Treselections,PCH or Treselections,FACH [4]. If present and HCS is used, the UE behaviour is unspecified. | REL-5 |
| Inter-frequency ScalingFactor for Treselection | OP |  | Real (1..4.75 by step of 0.25) | If present, it is used by the UE as scaling factor for Treselections or Treselections,PCH or Treselections,FACH for inter-frequency cell reselection evaluation [4]. If present and HCS is used, the UE behaviour is unspecified. | REL-5 |
| Inter-RAT ScalingFactor for Treselection | OP |  | Real (1..4.75 by step of 0.25) | If present, it is used by the UE as scaling factor for Treselections or Treselections,PCH or Treselections,FACH for inter-RAT cell reselection evaluation [4]. If present and HCS is used, the UE behaviour is unspecified. | REL-5 |
| Non-HCS_TCRmax | MD |  | Enumerated (not used, 30, 60, 120, 180, 240) | [s] Default value is ‘not used’. | REL-5 |
| Non-HCS_NCR | CV-UE speed detector_MD |  | Integer (1..16) | Default value = 8 | REL-5 |
| Non-HCS_TCRmaxHyst | CV-UE speed detector_MP |  | Enumerated (not used, 10, 20, 30, 40, 50, 60, 70) | [s] | REL-5 |
| HCS Serving cell Information | OP |  | HCS Serving cell information 10.3.7.12 |  |  |
| Maximum allowed UL TX power | MP |  | Maximum allowed UL TX power 10.3.6.39 | [dBm] UE_TXPWR_MAX_RACH in [4]. |  |

| Condition | Explanation |
| --- | --- |
| FDD-Quality-Measure | The IE is not needed if the IE "Cell selection and reselection quality measure" has the value CPICH RSCP, otherwise the IE is mandatory and has a default value. |
| Delta | This IE is optional if the value of Qrxlevmin is below –115dBm. It is not needed otherwise. |
| SIB3 | This IE is mandatory default if the IE “Cell selection and re-selection info for SIB3/4” is included in SIB type 3. It is not needed otherwise. |
| SIB3op | This IE is optional if the IE "Cell selection and re-selection info for SIB3/4" is included in SIB type 3. It is not needed otherwise. |
| SIB4 | This IE is optional if the IE “Cell selection and re-selection info for SIB3/4” is included in SIB type 4. It is not needed otherwise. |
| SIB4-FDD-Quality-Measure | This IE is optional if the IE “Cell selection and re-selection info for SIB3/4” is included in SIB type 4, and the IE "Cell selection and reselection quality measure" has the value CPICH Ec/N0. It is not needed otherwise. |
| UE Speed detector_MD (non-HCS) | This IE is not needed if non-HCS_TCRmax equals 'not used', else it is mandatory default. |
| UE Speed detector_MP (non-HCS) | This IE is not needed if non-HCS_TCRmax equals 'not used', else it is mandatory present. |

#### 10.3.2.4	Cell selection and re-selection info for SIB11/12

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |  |
| --- | --- | --- | --- | --- | --- | --- |
| Qoffset1s,n | MD |  | Integer(-50..50) | Default value is 0. [dB] |  |  |
| Qoffset2s,n | CV-FDD-Quality-Measure |  | Integer(-50..50) | Default value is 0. [dB] |  |  |
| Maximum allowed UL TX power | MD |  | Maximum allowed UL TX power 10.3.6.39 | According to UE_TXPWR_MAX_RACH in [4], [dBm]. If applied to FDD or TDD cells, the default is the Maximum allowed UL TX power for the serving cell. If applied to a GSM cell, the default is the UE maximum output power applicable for this GSM cell, according to the UE's radio access capability. |  |  |
| HCS neighbouring cell information | OP |  | HCS Neighbouring cell information 10.3.7.11 |  |  |  |
| CHOICE mode | MP |  |  |  |  |  |
| >FDD |  |  |  |  |  |  |
| >>Qqualmin | CV-FDD-Serving-Cell |  | Integer (-24..0) | Ec/N0, [dB] Default value is Qqualmin for the serving cell |  |  |
| >>Qrxlevmin | MD |  | Integer (-115..-25 by step of 2) | RSCP, [dBm] Default value is Qrxlevmin for the serving cell |  |  |
| >>DeltaQrxlevmin | CV-Delta |  | Integer(-4..-2 by step of 2) | If present, the actual value of Qrxlevmin = Qrxlevmin + DeltaQrxlevmin | REL-5 |  |
| >TDD |  |  |  |  |  |  |
| >>Qrxlevmin | MD |  | Integer (-115..-25 by step of 2) | RSCP, [dBm] Default value is Qrxlevmin for the serving cell |  |  |
| >> DeltaQrxlevmin | CV-Delta |  | Integer(-4..-2 by step of 2) | If present, the actual value of Qrxlevmin = Qrxlevmin + DeltaQrxlevmin | REL-5 |  |
| >GSM |  |  |  |  |  |  |
| >>Qrxlevmin | MD |  | Integer (-115..-25 by step of 2) | GSM RSSI, [dBm] Default value is Qrxlevmin for the serving cell |  |  |

| Condition | Explanation |
| --- | --- |
| FDD-Quality-Measure | This IE is mandatory and has a default value for Intra/Inter Frequency Cells if the IE "Cell selection and reselection quality measure" has the value CPICH Ec/No. Otherwise the IE is absent. |
| FDD-Serving-Cell | This IE is mandatory and has a default value if the serving cell is an FDD cell. Otherwise the IE is mandatory present. |
| Delta | This IE is optional if Qrxlevmin is present and the value of Qrxlevmin is below –115dBm. It is not needed otherwise. |

#### 10.3.2.5		Mapping Info

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Mapping List | MP | 1 to <MaxRAT> |  |  |  |
| >RAT | MP |  | Enumerated (UTRA FDD, UTRA TDD 3.84 Mcps, UTRA TDD 1.28 Mcps, GSM, cdma2000) |  | UTRA TDD 1.28 Mcps is included for REL-4. |
| >Mapping Function Parameter List | MP | 1 to <maxMeasIntervals> |  |  |  |
| >>Function type | MP |  | Enumerated (linear, function type 2, function type 3, function type 4) | Type of the function within the interval. |  |
|  |  |  |  | For 1.28Mcps TDD only. If IE “RAT” is set to “UTRA TDD 3.84 Mcps”, the IE should be interpreted as: Enumerated (frequency scope1, frequency scope2, frequency scope3, RSCP threshold). | REL-11 |
| >>Map_parameter_1 | MD |  | Integer (0..99) | Parameter describing the mapping function between the quality measurement and the representing quality value, see [4]. Default value is zero for the first interval or otherwise the value of Map_parameter_2 of the interval before. |  |
|  |  |  |  | For 1.28Mcps TDD only. See Note1/Note2 | REL-11 |
|  |  |  | Integer (0..127) | For 1.28 Mcps TDD, the parameter is used to indicate the uplink access location of the serving cell. | REL-7 |
| >>Map_parameter_2 | MP |  | Integer (0..99) | Parameter describing the mapping function between the quality measurement and the representing quality value, see [4]. |  |
|  |  |  |  | For 1.28Mcps TDD only. See Note1 | REL-11 |
| >>Upper_limit | CV-MaxInt |  | Integer (1..MaxMeas) | Upper limit of interval for which the Map_parameter_1 and Map_parameter_2 are valid. MaxMeas = 25 if RAT = UTRA FDD / CPICH Ec/N0, MaxMeas = 91 if RAT = UTRA TDD 3.84 Mcps or if RAT = UTRA TDD 1.28 Mcps or if RAT = UTRA FDD/ CPICH RSCP, MaxMeas = 63 if RAT = GSM. | UTRA TDD 1.28 Mcps is included for REL-4. |
|  |  |  |  | For 1.28Mcps TDD only. See Note1 | REL-11 |

| Condition | Explanation |
| --- | --- |
| MaxInt | This IE is mandatory present if Mapping Function Parameter List has not reached maxMeasIntervals and is not needed otherwise. |
| Note1: If IE “function type” is set to “frequency scope1”, for “Map_parameter_1” and “Map_parameter_2”, this value is interpreted as the frequency which applys to the mapping relationship: (0, 99)-> (9400, 9499); for “Upper_limit”, this value is interpreted as the frequency which applys to the mapping relationship: (1, 91)-> (9400, 9490). If IE “function type” is set to “frequency scope2”, for “Map_parameter_1” and “Map_parameter_2”, this value is interpreted as the frequency which applys to the mapping relationship: (0, 99)-> (9500, 9599); for “Upper_limit”, this value is interpreted as the frequency which applys to the mapping relationship: (1, 91)-> (9500, 9590). If IE “function type” is set to “frequency scope3”, for “Map_parameter_1” and “Map_parameter_2”, only the value among (0, 75) is valid and interpreted as frequency which applys to the mapping relationship: (0, 75)-> (10050, 10125); for “Upper_limit”, only the value among (1, 75) is valid and interpreted as frequency which applys to the mapping relationship: (1, 75)-> (10050, 10124). |  |
| Note2: If IE “function type” is set to “RSCP threshold”, only the value among (0, 90) is valid and interpreted as P-CCPCH RSCP value which applys to the mapping relationship: (0, 90)-> ( -115, -25); |  |

#### 10.3.2.6	URA identity

Gives the identity of the UTRAN Registration Area. It can be used to indicate to the UE which URA it shall use in case of overlapping URAs.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| URA identity | MP |  | bit string(16) |  |

#### 10.3.2.7	Dedicated priority Information

This IE indicates priority information for GERAN, UTRAN and E-UTRAN for reselections.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Action | MP |  |  |  | REL-8 |
| >Clear dedicated priorities |  |  | NULL |  | REL-8 |
| >Configure dedicated priorities |  |  |  |  | REL-8 |
| >>T322 | OP |  | Integer (5, 10, 20, 30, 60, 120, 180) | Time in minutes for which the configured dedicated priorities are valid. When the timer expires the UE should revert to using the priorities signalled in system information. Absence of this IE means that the configured dedicated priorities are valid until the next update. One spare value needed. | REL-8 |
| >>Priority Level List | OP | 1 to <maxPrio> |  |  | REL-8 |
|  |  | 1 to <maxPrio-ext> |  |  | REL-13 |
| >>>priority | OP |  | Integer (0.. <maxPrio–1>) | If this IE is absent then the UE behaviour is unspecified. 0 is the lowest priority and maxPrio-1 is the highest. | REL-8 |
| >>>subpriority | CV-E-UTRA-priority |  | Enumerated(oDot2, oDot4, oDot6, oDot8) | Fractional priority value. If present, this value is added to the value of IE "priority". | REL-13 |
| >>>CHOICE Radio Access Technology | MP |  |  |  | REL-8 |
| >>>>UTRA FDD |  |  |  |  | REL-8 |
| >>>>>Frequency List | MP | 1 to < maxNumFDDFreqs> |  |  | REL-8 |
| >>>>>>UARFCN | MP |  | Integer(0 .. 16383) | UARFCN of the downlink carrier frequency [25.101] | REL-8 |
| >>>>UTRA TDD |  |  |  |  | REL-8 |
| >>>>>Frequency List | MP | 1 to < maxNumTDDFreqs> |  |  | REL-8 |
| >>>>>>UARFCN | MP |  | Integer(0 .. 16383) | UARFCN of the downlink carrier frequency [25.101] | REL-8 |
| >>>>E-UTRA |  |  |  |  | REL-8 |
| >>>>>Frequency List |  | 1 to <maxNumEUTRAFreqs> |  |  | REL-8 |
| >>>>>>EARFCN | MP |  | Integer(0 .. 65535) | EARFCN of the downlink carrier frequency [64]. If the IE indicates a value of 65535, then the EARFCN for this instance should be read from the corresponding instance of IE "EARFCN extension". | REL-8 |
| >>>>>>EARFCN extension | OP |  | Integer(65536 …262143) | EARFCN of the downlink carrier frequency [64]. | REL-11 |
| >>>>GSM |  |  |  |  | REL-8 |
| >>>>>GSM cell group | MP |  | GSM cell group 10.3.7.9a |  | REL-8 |
| >>E-UTRA detection | MP |  | Boolean | ‘TRUE’ means that the UE may detect the presence of a E-UTRA cell and report to NAS | REL-8 |

NOTE:	It is always ensured by the UTRAN that priorities for different Radio Access Technologies are always different (e.g. a GERAN group of cells cannot have the same priority as a UTRA or E-UTRA frequency).

NOTE:	It is always ensured by the UTRAN that priorities for which both Threshx,high2 and Threshx,low2 are provided are always different from the priorities for which both Threshx,high2 and Threshx,low2 are not provided.

| Condition | Explanation |
| --- | --- |
| E-UTRA-priority | This IE is optionally present if IE "Radio Access Technology" is set to E-UTRA and if IE "priority" is present. Otherwise it is not needed. |

#### 10.3.2.8	CSG Identity

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CSG Identity | MP |  | Bit string(27) | As defined in [76] | REL-8 |

#### 10.3.2.9	CSG PSC Split Information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Start PSC | MP |  | Integer (0..504 by step of 8) | The value of this IE specifies the start PSC of the first PSC range (NOTE 1). | REL-8 |
| Number of PSCs | MP |  | Enumerated (5, 10, 15, 20, 30, 40, 50, 64, 80, 120, 160, 256, alltheRest) | This IE specifies the number of PSCs reserved for CSG cells in each PSC range. “alltheRest” indicates all values from Start PSC to 511. Three spare values are needed. | REL-8 |
| PSC Range 2 Offset | CV-alltheRest |  | Integer (8..504 by step of 8) | If this IE is included, the UE shall calculate the second PSC range (NOTE 2). If this IE is not included, the UE shall consider the second PSC range to be not present. | REL-8 |
| NOTE 1: Let the IE "Start PSC" = s. and "Number of PSCs" = n. The complete set of (n) PSC values in range 1 is defined as: {s, ((s + 1) mod 512), ((s + 2) mod 512) ... ((s + n-1) mod 512)}. |  |  |  |  |  |
| NOTE 2: Let the IEs "Start PSC" + "Number of PSCs" – 1 + "PSC Range 2 Offset" = s. and "Number of PSCs" = n. The complete set of (n) PSC values in range 2 is defined as: {(s mod 512), ((s + 1) mod 512), ((s + 2) mod 512) ... ((s + n-1) mod 512)}. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| alltheRest | This IE is optionally present if the value of IE “Number of PSCs” is not set to “alltheRest”. Otherwise, it is not needed. |

#### 10.3.2.10	Dedicated WLAN Offload Information

This IE indicates dedicated information for RAN-assisted WLAN interworking.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Configuration info | MP |  |  |  | REL-12 |
| >Continue |  |  | (no data) |  | REL-12 |
| >New configuration |  |  |  |  | REL-12 |
| >>T330 | OP |  | Integer (5, 10, 20, 30, 60, 120, 180) | Time in minutes. One spare value is needed. | REL-12 |
| >>WLAN Offload Information | MP |  | WLAN Offload Information 10.3.9b.3 |  | REL-12 |

### 10.3.3	UE Information elements

#### 10.3.3.1	Activation time

Activation Time defines the frame number/time at which the operation/changes caused by the related message shall take effect. Values between 0 and 255 indicate the absolute value of CFN (Connection Frame Number) of that frame number/time.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Activation time | MP |  | Integer(0.. 255) | CFN [10] |

#### 10.3.3.1a	Dynamic activation time

Activation offset is used to calculate the frame number/time at which the operation/changes caused by the related message shall take effect. The result is a value between 0 and 255 indicating the absolute value of CFN (Connection Frame Number) of that frame number/time.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Activation offset | MD |  | Integer (0..15) | In radio frames. Default value is 0. |

#### 10.3.3.2	Capability Update Requirement

This IE indicates to the UE which specific capabilities to transfer to the network.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UE radio access FDD capability update requirement | MP |  | Boolean | TRUE indicates update required |  |
| UE radio access 3.84 Mcps TDD capability update requirement | MP |  | Boolean | TRUE indicates update required | Name changed in REL-4 |
| UE radio access 7.68 Mcps TDD capability update requirement | MP |  | Boolean | TRUE indicates update required | REL-7 |
| UE radio access 1.28 Mcps TDD capability update requirement | MP |  | Boolean | TRUE indicates update required | REL-4 |
| System specific capability update requirement list | OP | 1 to <maxSystemCapability> |  | In this version, a maximum size of 4 of the list shall be applied and any items after the 4th item in the list shall be ignored. |  |
| >System specific capability update requirement | MP |  | Enumerated (GSM | Five spare values needed. |  |
|  |  |  | , GERAN Iu |  | REL-5 |
|  |  |  | , E-UTRA) |  | REL-8 |
| Requested E-UTRA Frequency Band list | CV-BCHopt | 1 to 16 |  |  | REL-10 |
| >E-UTRA Frequency band | MP |  | Integer (1..256) | As defined in [64]. The value of 64 is reserved and shall not be used | REL-10 |

| Condition | Explanation |
| --- | --- |
| BCHopt | This IE is not needed when sent in SYSTEM INFORMATION. Otherwise, the IE is optional |

Default value is:

"UE radio capability FDD update requirement" = FALSE

"UE radio capability 3.84 Mcps TDD update requirement" = FALSE

"UE radio capability 7.68 Mcps TDD update requirement" = FALSE

"UE radio capability 1.28 Mcps TDD update requirement" = FALSE

"System specific capability update requirement" not present

"Requested E-UTRA Frequency Band list" not present.

#### 10.3.3.3	Cell update cause

Indicates the cause for cell update.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Cell update cause | MP |  | Enumerated (cell reselection, periodical cell update, uplink data transmission, paging response, re-entered service area, radio link failure, RLC unrecoverable error, | One spare value is needed. |  |
|  |  |  | MBMS reception, MBMS ptp RB request) |  | REL-6 |

#### 10.3.3.4	Ciphering Algorithm

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Ciphering algorithm | MP |  | Enumerated (UEA0, UEA1 |  |  |
|  |  |  | , UEA2) |  | REL-7 |

#### 10.3.3.5	Ciphering mode info

This information element contains the ciphering specific security mode control information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Ciphering mode command | MP |  | Enumerated (start/restart) |  |
| Ciphering algorithm | MP |  | Ciphering algorithm 10.3.3.4 |  |
| Ciphering activation time for DPCH | OP |  | Activation time 10.3.3.1 | Used for radio bearers mapped on RLC-TM. Only applicable if the UE is already in CELL_DCH state |
| Radio bearer downlink ciphering activation time info | OP |  | RB activation time info, 10.3.4.13 | Used for radio bearers mapped on RLC-AM or RLC-UM The UTRAN should not include this IE in a message other than a SECURITY MODE COMMAND |

#### 10.3.3.6	CN domain specific DRX cycle length coefficient

A coefficient in the formula to count the paging occasions to be used by a specific UE (specified in [4]) .

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN domain specific DRX cycle length coefficient | MP |  | Integer(6...9) | Refers to 'k' in the formula as specified in [4], Discontinuous reception |

#### 10.3.3.7	Void

#### 10.3.3.7a	Common E-RNTI info

NOTE:	For 1.28 Mcps TDD only.

This IE defines the common E-RNTI used in enhanced CELL_FACH and Idle mode. Each instance in the IE "Common E-RNTI information" is related to an E-RUCCH which is defined in the PRACH system information in System Information Block 5. The first instance in IE "Common E-RNTI information" is related to the first instance of E-RUCCH in the PRACH system information in System Information Block 5 in the order of their appearances, the second instance in IE "Common E-RNTI information" is related to the second instance of E-RUCCH in the PRACH system information in System Information Block 5 and so on.

From each intance in the IE "Common E-RNTI information", we can compile a common E-RNTI list which is related to a certain E-RUCCH. For each E-RUCCH, there are a number of common E-RNTI groups related to it; up to 2 common E-RNTIs allocated in each group. The common E-RNTI list related to a certain E-RUCCH is derived in incremental manner with start code indicated by IE "Starting E-RNTI". The "Starting E-RNTI" corresponds to the first E-RNTI in the first group; if the "Number of E-RNTI per group" is bigger than one, then the sencond E-RNTI in the first group is derived by the first E-RNTI plus 1; the first E-RNTI in the second group is derived by the last of E-RNTI of the first group plus 1… the whole common E-RNTI list related to the E-RUCCH is compiled in this manner.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Common E-RNTI information | MP | 1 to <maxERUCCH> |  |  | REL-8 |
| >Starting E-RNTI | MP |  | E-RNTI 10.3.3.10a | Indicates the starting code of E-RNTI related to a certain E-RUCCH. | REL-8 |
| >Number of group | MP |  | Integer(1.. maxERNTIgroup) | Indicates the number of common E-RNTI groups related to the E-RUCCH. | REL-8 |
| >Number of E-RNTI per group | MP |  | Integer (1.. maxERNTIperGroup) | Indicates the number of common E-RNTIs in the group. | REL-8 |

#### 10.3.3.8	C-RNTI

The cell RNTI (C-RNTI) identifies a UE having a RRC connection within a cell.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| C-RNTI | MP |  | bit string(16) |  |

#### 10.3.3.8a	CSG proximity indication capability

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Support of intra-frequency proximity indication | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support intra-frequency proximity indication | REL-9 |
| Support of inter-frequency proximity indication | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support inter-frequency proximity indication | REL-9 |
| Support of E-UTRA proximity indication | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support E-UTRA proximity indication | REL-9 |

#### 10.3.3.9	Void

#### 10.3.3.9a	DSCH-RNTI

In TDD, the DSCH-RNTI identifies a UE in CELL_DCH or CELL_FACH using a DSCH or USCH within the cell.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DSCH-RNTI | MP |  | bit string(16) |  |

#### 10.3.3.10	Void

#### 10.3.3.10a	E-RNTI

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-RNTI | MP |  | bit string(16) |  | REL-6 |

#### 10.3.3.11	Establishment cause

Cause for an RRC connection establishment request.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Establishment cause | MP |  | Enumerated( Originating Conversational Call, Originating Streaming Call, Originating Interactive Call, Originating Background Call, Originating Subscribed traffic Call, Terminating Conversational Call, Terminating Streaming Call, Terminating Interactive Call, Terminating Background Call, Emergency Call, Inter-RAT cell re-selection, Inter-RAT cell change order, Registration, Detach, Originating High Priority Signalling, Originating Low Priority Signalling, Call re-establishment, Terminating High Priority Signalling, Terminating Low Priority Signalling, Terminating – cause unknown, MBMS reception, MBMS ptp RB request, | Nine spare values are needed. |  |
|  |  |  | Delay Tolerant Access) |  | REL-10 |

#### 10.3.3.12	Expiration Time Factor

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Expiration Time Factor | MP |  | Enumerated( 2times, 4times, 8times, 16times, 32times, 64times, 128times, 256times) |  |

#### 10.3.3.12a	Extended Wait Time

The field defines the wait time for Delay Tolerant access request, to be passed to the UE upper layers.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended Wait time | MP |  | Integer(1.. 1800) | Value in seconds |

#### 10.3.3.12b	Expiration Time Factor 2

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Expiration Time Factor 2 | MP |  | Integer (0..15) |  | REL-12 |

#### 10.3.3.13	Failure cause

Cause for failure to perform the requested procedure.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Failure cause | MP |  | Enumerated (configuration unsupported, physical channel failure, incompatible simultaneous reconfiguration, protocol error, compressed mode runtime error, cell update occurred, invalid configuration, configuration incomplete, unsupported measurement, MBMS session already received correctly, lower priority MBMS service) | Five spare values are needed. |

#### 10.3.3.14	Failure cause and error information

Cause for failure to perform the requested procedure.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Failure cause | MP |  | Failure cause 10.3.3.13 |  |
| Protocol error information | CV-ProtErr |  | Protocol error information 10.3.8.12 |  |
| Deleted TGPSI | CV-CompModeErr |  | TGPSI 10.3.6.82 |  |

| Condition | Explanation |
| --- | --- |
| ProtErr | The IE is mandatory present if the IE "Failure cause" has the value "Protocol error"; otherwise it is not needed in the message. |
| CompModeErr | The IE is mandatory present if the IE "Failure cause" has the value " Compressed mode runtime error"; otherwise it is not needed in the message |

#### 10.3.3.14o	Group release information

Contains addressing information to perform a release of a group of RRC connections.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| U-RNTI group | MP |  | U-RNTI group 10.3.3.47a |  | REL-5 |

#### 10.3.3.14a	H-RNTI

The H-RNTI identifies an UE having a HS-PDSCH assignment within a cell.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| H-RNTI | MP |  | bit string(16) |  | REL-5 |

#### 10.3.3.14b	IMS Voice capability

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Voice over UTRA PS HS Support | OP |  | Enumerated (TRUE) | Absence of this IE means that the UE does not support IMS voice | REL-9 |
| SRVCC Support from UTRA to UTRA | OP |  | Enumerated (TRUE) | Absence of this IE means that the UE does not support SRVCC from UTRA PS HS to UTRA CS | REL-9 |
| SRVCC Support from UTRA to GERAN | OP |  | Enumerated (TRUE) | Absence of this IE means that the UE does not support SRVCC from UTRA PS HS to GERAN CS | REL-9 |
| rSRVCC support from UTRA CS to E-UTRAN FDD | OP |  | Enumerated (TRUE) | Absence of this IE means that the UE does not support rSRVCC from UTRA CS to E-UTRAN FDD | REL-11 |
| rSRVCC support from UTRA CS to E-UTRAN TDD | OP |  | Enumerated (TRUE) | Absence of this IE means that the UE does not support rSRVCC from UTRA CS to E-UTRAN TDD | REL-11 |

#### 10.3.3.15	Initial UE identity

This information element identifies the UE at a request of an RRC connection.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE UE id type | MP |  |  |  |
| >IMSI (GSM-MAP) |  |  | IMSI (GSM-MAP) 10.3.1.5 |  |
| >TMSI and LAI (GSM-MAP) |  |  |  |  |
| >>TMSI (GSM-MAP) | MP |  | TMSI (GSM-MAP) 10.3.1.17 |  |
| >>LAI (GSM-MAP) | MP |  | Location Area Identification 10.3.1.7 |  |
| >P-TMSI and RAI (GSM-MAP) |  |  |  |  |
| >>P-TMSI (GSM-MAP) | MP |  | P-TMSI (GSM-MAP) 10.3.1.13 |  |
| >>RAI (GSM-MAP) | MP |  | Routing Area Identification 10.3.1.16 |  |
| >IMEI |  |  | IMEI 10.3.1.4 |  |
| >ESN (DS-41) |  |  | Bit string (SIZE (32)) | TIA/EIA/IS-2000-4 |
| >IMSI (DS-41) |  |  | Octet string (SIZE (5..7)) | TIA/EIA/IS-2000-4 |
| >IMSI and ESN (DS-41) |  |  |  | TIA/EIA/IS-2000-4 |
| >>IMSI (DS-41) | MP |  | Octet string (SIZE (5..7)) | TIA/EIA/IS-2000-4 |
| >>ESN (DS-41) | MP |  | Bit string (SIZE (32)) | TIA/EIA/IS-2000-4 |
| >TMSI (DS-41) |  |  | Octet string (SIZE (2..17)) | TIA/EIA/IS-2000-4 Although normally upto 12 digits are used for this IE, a bigger length is used to support future extension. |

#### 10.3.3.16	Integrity check info

The Integrity check info contains the RRC message sequence number needed in the calculation of XMAC-I [40] and the calculated MAC-I.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message authentication code | MP |  | bit string(32) | MAC-I [40]. The first/leftmost bit of the bit string contains the most significant bit of the MAC-I. The 27 MSB of the IE shall be set to zero and the 5 LSB of the IE shall be set to the value of the IE "RB identity" for the used signalling radio bearer when the encoded RRC message is used as the MESSAGE parameter in the integrity protection algorithm. |
| RRC Message sequence number | MP |  | Integer (0..15) | The local RRC hyper frame number (RRC HFN) is concatenated with the RRC message sequence number to form the input parameter COUNT-I for the integrity protection algorithm. The IE value shall be set to zero when the encoded RRC message is used as the MESSAGE parameter in the integrity protection algorithm. |

#### 10.3.3.17	Integrity protection activation info

This IE contains the time, in terms of RRC sequence numbers, when a new integrity protection configuration shall be activated for the signalling radio bearers.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RRC message sequence number list | MP | 4 to 5 |  | The RRC sequence number when a new integrity protection configuration shall be applied, for signalling radio bearers in the order RB0, RB1, RB2, RB3, RB4. The value for RB1 shall be ignored if this IE was included in a RRC message sent on RB1. The value for RB2 shall be ignored if this IE was included in a RRC message sent on RB2. |
| >RRC message sequence number | MP |  | Integer (0.. 15) |  |

#### 10.3.3.18	Integrity protection Algorithm

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Integrity protection algorithm | MP |  | Enumerated (UIA1 |  |  |
|  |  |  | , UIA2) |  | REL-7 |

#### 10.3.3.19	Integrity protection mode info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Integrity protection mode command | MP |  | Enumerated(start, modify) |  |
| Downlink integrity protection activation info | CV-modify |  | Integrity protection activation info 10.3.3.17 |  |
| Integrity protection algorithm | OP |  | Integrity protection algorithm 10.3.3.18 |  |
| Integrity protection initialisation number | CV-start |  | Bit string(32) | FRESH [40]. The first/leftmost bit of the bit string contains the most significant bit of the FRESH. |

| Condition | Explanation |
| --- | --- |
| Start | The IE is mandatory present if the IE "Integrity protection mode command" has the value "start ", otherwise it is not needed in the message. |
| Modify | The IE is mandatory present if the IE "Integrity protection mode command" has the value "modify" and not needed otherwise. |

#### 10.3.3.19a	Void

#### 10.3.3.20	Void

#### 10.3.3.21	Measurement capability

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Inter-frequency detected set measurements | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE is able to measure inter-frequency detected cells. | REL-10 |
| Cells excluded from detected set measurements | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE is able to exclude cells from detected set measurements. | REL-11 |
| Wideband RSRQ FDD measurements | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE is able to perform wideband RSRQ FDD measurements. | REL-11 |
| Wideband RSRQ TDD measurements | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE is able to perform wideband RSRQ TDD measurements. | REL-11 |
| Event 2g reporting on a configured secondary downlink frequency | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE is able to perform event 2g reporting on a configured secondary downlink frequency. | REL-12 |
| Enhanced UPH reporting | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE is able to perform reporting of filtered UPH measurement. | REL-12 |
| Increased UE carrier monitoring UTRA | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE supports increased number of UTRA carrier monitoring in connected and idle mode as defined in [19]. | REL-12 |
| Increased UE carrier monitoring E-UTRA | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE supports increased number of E-UTRA carrier monitoring in connected and idle mode as defined in [74]. | REL-12 |
| Extended RSRQ lower value range | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE supports the extended RSRQ lower value range from -34dB to -19.5dB. | REL-12 |
| RSRQ measurement on all symbols | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE supports the RSRQ measurement on all OFDM symbols and the extended upper RSRQ value range from -3dB to 2.5dB. | REL-12 |
| Need for downlink compressed mode |  |  |  |  |  |
| FDD measurements | MP |  | Boolean | TRUE means that the UE requires DL compressed mode in order to perform measurements on FDD |  |
| 3.84 Mcps TDD measurements | CV-3.84_Mcps_tdd_sup |  | Boolean | TRUE means that the UE requires DL compressed mode in order to perform measurements on 3.84 Mcps TDD | Name changed in REL-4 |
| 7.68 Mcps TDD measurements | CV-7.68_Mcps_tdd_sup |  | Boolean | TRUE means that the UE requires DL compressed mode in order to perform measurements on 7.68 Mcps TDD | REL-7 |
| 1.28 Mcps TDD measurements | CV-1.28_Mcps_tdd_sup |  | Boolean | TRUE means that the UE requires DL compressed mode in order to perform measurements on 1.28 Mcps TDD | REL-4 |
| GSM measurements | CV-gsm_sup |  |  |  |  |
| >GSM 900 | MP |  | Boolean | TRUE means that the UE requires DL compressed mode in order to perform measurements on GSM 900 |  |
| >DCS 1800 | MP |  | Boolean | TRUE means that the UE requires DL compressed mode in order to perform measurements on DCS 1800 |  |
| >GSM 1900 | MP |  | Boolean | TRUE means that the UE requires DL compressed mode in order to perform measurements on GSM 1900 |  |
| Multi-carrier measurement | CV-mc_sup |  | Boolean | TRUE means that the UE requires DL compressed mode in order to perform measurements on multi-carrier |  |
| Adjacent Frequency measurements without compressed mode | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE does not require compressed mode for adjacent frequency measurements. | REL-8 |
| Inter-band Frequency measurements without compressed mode | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE does not require compressed mode for Inter-band measurements for the band combinations reported in the IE “Radio Access Capability Band Combination List”. | REL-9 |
| Enhanced inter-frequency measurements without compressed mode | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE does not require compressed mode for measurements on two additional frequencies | REL-10 |
| Frequency specific compressed mode | CV-not_iRAT_HoInf |  | Enumerated (TRUE) | TRUE means that for the dual band operation the UE can apply compressed mode only to the configured frequencies, which belong to the frequency band other than the serving HS-DSCH cell. | REL-10 |
| Inter-frequency measurements on configured carriers without compressed mode | CV-not_iRAT_HoInf |  | Enumerated (TRUE) | TRUE means that the UE does not require compressed mode for measurements on the frequencies which are configured for HS-DSCH operation and associated with the secondary serving HS-DSCH cells. | REL-11 |
| Need for uplink compressed mode |  |  |  |  |  |
| FDD measurements | MP |  | Boolean | TRUE means that the UE requires UL compressed mode in order to perform measurements on FDD |  |
| 3.84 Mcps TDD measurements | CV-3.84_Mcps_tdd_sup |  | Boolean | TRUE means that the UE requires UL compressed mode in order to perform measurements on 3.84 Mcps TDD | Name changed in REL-4 |
| 7.68 Mcps TDD measurements | CV-7.68_Mcps_tdd_sup |  | Boolean | TRUE means that the UE requires UL compressed mode in order to perform measurements on 7.68 Mcps TDD | REL-7 |
| 1.28 Mcps TDD measurements | CV-1.28_Mcps_tdd_sup |  | Boolean | TRUE means that the UE requires DL compressed mode in order to perform measurements on 1.28 Mcps TDD | REL-4 |
| GSM measurements | CV-gsm_sup |  |  |  |  |
| >GSM 900 | MP |  | Boolean | TRUE means that the UE requires UL compressed mode in order to perform measurements on GSM 900 |  |
| >DCS 1800 | MP |  | Boolean | TRUE means that the UE requires UL compressed mode in order to perform measurements on DCS 1800 |  |
| >GSM 1900 | MP |  | Boolean | TRUE means that the UE requires UL compressed mode in order to perform measurements on GSM 1900 |  |
| Multi-carrier measurement | CV-mc_sup |  | Boolean | TRUE means that the UE requires UL compressed mode in order to perform measurements on multi-carrier |  |

| Condition | Explanation |
| --- | --- |
| 3.84_Mcps_tdd_sup | The IE is mandatory present if an IE "TDD RF capability" is present with the IE "Chip rate capability" set to "3.84 Mcps". Otherwise this field is not needed in the message. |
| 7.68_Mcps_tdd_sup | The IE is mandatory present if an IE “TDD RF capability” is present with the IE “Chip rate capability” set to “7.68 Mcps”. Otherwise this field is not needed in the message. |
| 1.28_Mcps_tdd_sup | The IE is mandatory present if an IE "TDD RF capability" is present with the IE "Chip rate capability" set to "1.28 Mcps". Otherwise this field is not needed in the message. |
| gsm_sup | The IE is mandatory present if the IE "Inter-RAT UE radio access capability" indicates support for GSM900, GSM1800 and/or GSM1900. Otherwise this field is not needed in the message. |
| mc_sup | The IE is mandatory present if the IE "Support of multi-carrier" has the value TRUE. Otherwise this field is not needed in the message. |
| not_iRAT_HoInfo | The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is optionally present. |

#### 10.3.3.21a	Measurement capability extension

This IE may be used to replace the measurement capability information provided within IE "Measurement capability".

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| FDD measurements | MP | 1 to <maxFreqBandsFDD> |  | The network should ignore the entry that includes the ‘extension indicator’. |  |
|  |  | 1 to <maxFreqBandsFDD2> |  |  | REL-6 |
|  |  | 1 to <maxFreqBandsFDD3> |  |  | REL-10 |
| >FDD Frequency band | CV-fdd2 |  | Enumerated(Band I, Band II, | The default value is the same as indicated in the IE "Frequency band" included in the IE " UE radio access capability extension". Band numbering is defined in [21]. |  |
|  |  |  | Band III, |  | REL-5 |
|  |  |  | Band VI, Band IV, Band V, Band VII, extension indicator) | The default value is the same as R99, if the IE “FDD Frequency band 2” below is not included. The setting of the value ‘extension indicator’ by the UE is not specified in the specification. | REL-6 |
| >FDD Frequency band 2 | CV-fdd1 |  | Enumerated(Band VIII, Band IX, Band X, Band XI, Band XII, Band XIII, Band XIV, Band XIX, Band XXI, Band XX, Band XXII, extension Indicator) | The default value is the same as indicated in the IE "Frequency band 2" included in the IE " UE radio access capability extension", if the IE “FDD Frequency band” above is not included. The setting of the value ‘extension indicator’ by the UE is not specified in the specification. Four spare values are needed | REL-6 |
| >FDD Frequency band 3 | OP |  | Enumerated (Band XXV, Band XXVI, Band XXXII, extension indicator) | If the IE is not present the value is the same as indicated in the IE "Frequency band 3" if included in the IE " UE radio access capability extension". The setting of the value ‘extension indicator’ by the UE is not specified in the specification. Sixty spare values are needed | REL-10 |
| >Need for DL compressed mode | MP |  | Boolean | TRUE means that the UE requires DL compressed mode in order to perform measurements on the FDD frequency band indicated by the IE "FDD Frequency band" |  |
| >Need for UL compressed mode | MP |  | Boolean | TRUE means that the UE requires UL compressed mode in order to perform measurements on the FDD frequency band indicated by the IE "FDD Frequency band" |  |
| TDD measurements | CV-tdd_sup | 1 to <maxFreqBandsTDD> |  |  |  |
| >TDD Frequency band | MP |  | Enumerated(a, b, c) |  |  |
|  |  |  | Enumerated(a, b, c, d) |  | REL-7 |
| >Need for DL compressed mode | MP |  | Boolean | TRUE means that the UE requires DL compressed mode in order to perform measurements on TDD frequency band indicated by the IE "TDD Frequency band" |  |
| >Need for UL compressed mode | MP |  | Boolean | TRUE means that the UE requires UL compressed mode in order to perform measurements on TDD frequency band indicated by the IE "TDD Frequency band" |  |
| GSM measurements | CV-gsm_sup | 1 to <maxFreqBandsGSM> |  |  |  |
| >GSM Frequency band | MP |  | Enumerated(GSM450, GSM480, GSM850, GSM900P, GSM900E, GSM1800, GSM1900) | as defined in [45]. Nine spare values are needed. |  |
| >Need for DL compressed mode | MP |  | Boolean | TRUE means that the UE requires DL compressed mode in order to perform measurements on GSM frequency band indicated by the IE "GSM Frequency band" |  |
| >Need for UL compressed mode | MP |  | Boolean | TRUE means that the UE requires UL compressed mode in order to perform measurements on GSM frequency band indicated by the IE "GSM Frequency band" |  |
| Multi-carrier measurement | CV-mc_sup |  |  |  |  |
| >Need for DL compressed mode | MP |  | Boolean | TRUE means that the UE requires DL compressed mode in order to perform measurements on multi-carrier |  |
| >Need for UL compressed mode | MP |  | Boolean | TRUE means that the UE requires UL compressed mode in order to perform measurements on multi-carrier |  |
| E-UTRA measurements | CV-eutra_sup | 1 to <maxFreqBandsEUTRA> |  | Note 1 | REL-8 |
| >E-UTRA Frequency band | MP |  | Integer (1..64) | As defined in [64]. If the IE indicates a value of 64, then the E-UTRA Frequency band for this instance should be read from the corresponding instance of IE "E-UTRA Frequency band extension" in the E-UTRA measurements extension. | REL-8 |
| >Need for compressed mode | MP |  | Boolean | TRUE means that the UE requires DL and UL compressed mode in order to perform measurements on E-UTRA frequency band indicated by the IE "E-UTRA Frequency band" | REL-8 |
| E-UTRA measurements extension | CV-extended_eutra_sup | 1 to <maxFreqBandsEUTRA> |  | Note 1 | REL-11 |
| >E-UTRA Frequency band extension | MP |  | Integer (65..256) | As defined in [64]. Note 2 | REL-11 |
| E-UTRA measurements extension 1 | CV- extended_meas_eutra_sup | 1 to <maxFreqBandsEUTRA-ext> |  | Note 1 | REL-11 |
| >E-UTRA Frequency band extension 1 | MP |  | Integer (1..256) | As defined in [64]. The value of 64 is reserved and shall not be used. | REL-11 |
| >Need for compressed mode | MP |  | Boolean | TRUE means that the UE requires DL and UL compressed mode in order to perform measurements on E-UTRA frequency band indicated by the IE "E-UTRA Frequency band extension 1" | REL-11 |
| Note 1: Indicates E-UTRA bands supported and the need for compressed mode, E-UTRAN measurement support may be separately indicated as specified in Annex E. |  |  |  |  |  |
| Note 2: If the corresponding instance of IE "E-UTRA Frequency band" does not indicate the reserved value of 64, then the UE can signal any valid value of IE "E-UTRA Frequency band extension". |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| tdd_sup | The IE is mandatory present if the IE "Multi-mode capability" has the value "TDD" or "FDD/TDD". Otherwise this field is not needed in the message. |
| gsm_sup | The IE is mandatory present if the IE "Support of GSM" has the value TRUE. Otherwise this field is not needed in the message. |
| mc_sup | The IE is mandatory present if the IE "Support of multi-carrier" has the value TRUE. Otherwise this field is not needed in the message. |
| eutra_sup | At least one of these IEs is mandatory present if the IE "Support of E-UTRA" has the value TRUE. Otherwise these fields are not needed in the message. |
| extended_eutra_sup | At least one of these IEs is mandatory present if the IE "Support of E-UTRA" has the value TRUE and the UE supports a E-UTRA frequency band greater than 64. Otherwise these fields are not needed in the message. |
| extended_meas_eutra_sup | At least one of these IEs is mandatory present if the IE "Support of E-UTRA" has the value TRUE and the UE needs to report E-UTRA measurement capability for more than 16 E-UTRA bands. Otherwise these fields are not needed in the message. |
| fdd1 | The IE is mandatory default if the IE "FDD Frequency band" is not included in the message. Otherwise this field is not needed in the message. |
| fdd2 | The IE is mandatory default if the IE "FDD Frequency band 2" is not included in the message. Otherwise this field is not needed in the message. |

#### 10.3.3.21b	Measurement capability TDD

This IE is only used for TDD if the UE is a multi-RAT capable which also supports E-UTRA.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-UTRA measurements | CV-eutra_sup | 1 to <maxFreqBandsEUTRA> |  | Note 1 | REL-8 |
| >E-UTRA Frequency band | MP |  | Integer (1..64) | As defined in [64]. If the IE indicates a value of 64, then the E-UTRA Frequency band for this instance should be read from the corresponding instance of IE "E-UTRA Frequency band extension" in the E-UTRA measurements extension. | REL-8 |
| >Need for Idle Interval | MP |  | Boolean | TRUE means that the UE requires idle interval in order to perform measurements on E-UTRA frequency band indicated by the IE "E-UTRA Frequency band" | REL-8 |
| E-UTRA measurements extension | CV-extended_eutra_sup | 1 to <maxFreqBandsEUTRA> |  | Note 1 | REL-11 |
| >E-UTRA Frequency band extension | MP |  | Integer (65..256) | As defined in [64]. Note 2 | REL-11 |
| Note 1: Indicates E-UTRA bands supported and the need for idle interval, E-UTRAN measurement support may be separately indicated as specified in Annex E. |  |  |  |  |  |
| Note 2: If the corresponding instance of IE "E-UTRA Frequency band" does not indicate the reserved value of 64, then the UE can signal any valid value of IE "E-UTRA Frequency band extension". |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| eutra_sup | At least one of these IEs is mandatory present if the IE "Support of E-UTRA" has the value TRUE. Otherwise these fields are not needed in the message. |
| extended_eutra_sup | At least one of these IEs is mandatory present if the IE "Support of E-UTRA" has the value TRUE and the UE supports a E-UTRA frequency band greater than 64. Otherwise these fields are not needed in the message. |

#### 10.3.3.21ba	Multiflow capability

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Support for Multiflow with MIMO operation in different bands | OP |  | Enumerated(single-stream, dual-stream) | Absence of this IE means that a UE does not support MIMO with Multiflow in different bands. Otherwise, it indicates whether a UE support single-stream or dual-stream MIMO. | REL-11 |
| Longer HARQ processing time | OP |  | Enumerated(TRUE) | Presence of this IE indicates that a UE needs more time to generate the HARQ feedback with Multiflow and MIMO as described in [26], sub-clause 7.7.2. | REL-11 |

#### 10.3.3.21bb	Multiflow per band capability

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Number of cells | MP |  | Integer(2..8) | Total number of cells that can be configured to a UE as a part of Multiflow configuration. In this version of the specification, the maximum value is 4. | REL-11 |
| Number of frequencies | MP |  | Integer(1..4) | Total number of frequencies, across which the Multiflow can be configured. In this version of the specification, the maximum value is 2. | REL-11 |
|  |  |  |  | In this version of the specification, the maximum value is 3. | REL-13 |
| MIMO support | OP |  | Enumerated(single-stream, dual-stream) | Absence of this IE means that a UE does not support MIMO with Multiflow. Otherwise, it indicates whether a UE support single-stream or dual-stream MIMO. | REL-11 |
| Non-contiguous multi-cell | OP |  |  | Presence of this IE indicates that a UE supports Multiflow operation with non-contiguous cells. | REL-11 |
| >Gap size | MP |  | Enumerated(fiveMHz, tenMHz, anyGapSize) | This IE indicates the maximum gap size between the aggregated cells. "fiveMHz" indicates that UE supports 5 MHz gap size. "tenMHz" indicates that UE supports 10 MHz gap size and 5 MHz gap size. "anyGapSize" indicates that UE supports any multiple of 5 MHz gap size. 5 spare values are required. | REL-11 |

#### 10.3.3.21c	Neighbour Cell SI acquisition capability

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Support of intra-frequency SI acquisition for HO | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support intra-frequency SI acquisition for HO | REL-9 |
| Support of inter-frequency SI acquisition for HO | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support inter-frequency SI acquisition for HO | REL-9 |
| Support of E-UTRA SI acquisition for HO | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support E-UTRA SI acquisition for HO | REL-9 |

#### 10.3.3.22	Paging cause

Cause for a CN originated page.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging cause | MP |  | Enumerated( Terminating Conversational Call, Terminating Streaming Call, Terminating Interactive Call, Terminating Background Call, Terminating High Priority Signalling, Terminating Low Priority Signalling, Terminating – cause unknown ) | One spare value is needed. |

#### 10.3.3.23	Paging record

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Used paging identity | MP |  |  |  |  |
| >CN identity |  |  |  |  |  |
| >>Paging cause | MP |  | Paging cause 10.3.3.22 |  |  |
| >>CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >>CHOICE UE Identity | MP |  |  | Three spare values are needed. |  |
| >>>IMSI (GSM-MAP) |  |  | IMSI (GSM-MAP) 10.3.1.5 |  |  |
| >>>TMSI (GSM-MAP) |  |  | TMSI (GSM-MAP) 10.3.1.17 |  |  |
| >>>P-TMSI (GSM-MAP) |  |  | P-TMSI (GSM-MAP) 10.3.1.13 |  |  |
| >>>IMSI (DS-41) |  |  | Octet string (SIZE (5..7)) | TIA/EIA/IS-2000-4 |  |
| >>>TMSI (DS-41) |  |  | Octet string (SIZE (2..17)) | TIA/EIA/IS-2000-4 |  |
| >UTRAN single UE identity |  |  |  |  |  |
| >>U-RNTI | MP |  | U-RNTI 10.3.3.47 |  |  |
| >>CN originated page to connected mode UE | OP |  |  |  |  |
| >>>Paging cause | MP |  | Paging cause 10.3.3.22 |  |  |
| >>>CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >>>Paging record type identifier | MP |  | Paging record type identifier 10.3.1.10 |  |  |
| >>RRC connection release information | MP |  | RRC connection release information 10.3.3.35o |  | REL-5 |
| >UTRAN group identity |  | 1 to <maxURNTIgroup> |  |  | REL-5 |
| >>RRC connection release information | MP |  | RRC connection release information 10.3.3.35o |  | REL-5 |
| >>Group release information | MP |  | Group release information 10.3.3.14o |  | REL-5 |

| Condition | Explanation |
| --- | --- |
| CHOICE Used paging identity | Condition under which the given used paging identity is chosen |
| CN identity | For CN originating pages (for idle mode UEs) |
| UTRAN single UE identity | For UTRAN originating pages (for connected mode UEs), addressing a single UE |
| UTRAN group identity | For UTRAN originating pages (for connected mode UEs), addressing a group of UEs |

#### 10.3.3.24	PDCP capability

Indicates which algorithms and which value range of their parameters are supported by the UE.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Support for lossless SRNS relocation | MP |  | Boolean | TRUE means supported |  |
| Support for lossless DL RLC PDU size change | CV-not_iRAT_HoInfo2 |  | Boolean | TRUE means supported Default value is FALSE | REL-5 |
| Support for RFC2507 | MP |  | Boolean | TRUE means supported |  |
| >Max HC context space | MP |  | Integer(1024, 2048, 4096, 8192, |  |  |
|  |  |  | 16384, 32768, 65536, 131072) | Note 1 | REL-5 |
| Support for RFC 3095 | CV-not_iRAT_HoInfo |  | Boolean | TRUE means header compression according to IETF ROHC standard [83], [84] is supported | REL-4 |
| >Maximum number of ROHC context sessions | MD |  | Integer( 2, 4, 8, 12, 16, 24, 32, 48, 64, 128, 256, 512, 1024, 16384) | Default value is 16. | REL-4 |
| >Reverse decompression depth | MD |  | Integer (0..65535) | Default value is 0 (reverse decompression is not supported). | REL-4 |
| >Support for RFC 3095 context relocation | MP |  | Boolean | TRUE means supported | REL-5 |
| Support for CS Voice over HSPA | CV-not_iRAT_HoInfo3 |  | Enumerated (TRUE) | The IE indicates the UE’s support for CS Voice over HSPA, if set. | REL-8 |
| Note 1: The IE "Max HC context space" values 16384, 32768, 65536 and 131072 are not used in the INTER RAT HANDOVER INFO message. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| not_iRAT_HoInfo | The IE is optionally present in the INTER RAT HANDOVER INFO message. Otherwise, it is mandatory present. |
| not_iRAT_HoInfo2 | The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is mandatory default. |
| not_iRAT_HoInfo3 | The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is optionally present. |

#### 10.3.3.25	Physical channel capability

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Downlink physical channel capability information elements |  |  |  |  |  |
| FDD downlink physical channel capability | CH-fdd_req_sup |  |  |  |  |
| >Max no DPCH codes | MP |  | Integer (1..8) | Maximum number of DPCH codes to be simultaneously received |  |
| >Max no physical channel bits received | MP |  | Integer (1200, 2400, 3600, 4800, 7200, 9600, 14400, 19200, 28800, 38400, 48000, 57600, 67200, 76800) | Maximum number of physical channel bits received in any 10 ms interval (DPCH, S-CCPCH) |  |
| >Support for SF 512 and 80 ms TTI for DPCH | MP |  | Boolean | TRUE means supported |  |
| >Support for DCH Enhancements | CV-not_iRAT_HoInfo2 |  | Enumerated (Basic, Full) | The absence of this IE indicates that the UE does not support DCH Enhancements | REL-12 |
| >Simultaneous support for DCH Enhancements and Compressed Mode operation | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support simultaneous operation of DCH Enhancements and Compressed Mode. | REL-12 |
| >Simultaneous support for DCH Enhancements and DPCCH Discontinuous Transmission | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support simultaneous operation of DCH Enhancements and DPCCH Discontinuous Transmission. | REL-12 |
| >Support of Enhanced Serving Cell Change for Event 1c | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Enhanced Serving Cell Change for Event 1c operation. | REL-12 |
| >Support of DPCCH2 | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support DPCCH2 transmission. | REL-12 |
| >Support for power control algorithm 3 | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support power control algorithm 3. | REL-13 |
| >CHOICE Support of HS-PDSCH | CV-not_iRAT_HoInfo |  |  |  | REL-5 |
| >>Supported |  |  |  |  | REL-5 |
| >>>HS-DSCH physical layer category | MP |  | Integer (1..64) | As defined in [35]. Values 13...64 are spares. See Note 5. | REL-5 |
| >>>HS-DSCH physical layer category extension | OP |  | Integer (1..20) | As defined in [35]. See Note 6. | REL-7 |
| >>>HS-DSCH physical layer category extension 2 | OP |  | Integer (21..24) | As defined in [35]. See Note 12. Absence of this IE means that dual cell operation is not supported. | REL-8 |
| >>>HS-DSCH physical layer category extension 3 | OP |  | Integer (25..28) | As defined in [35]. See Note 13. Absence of this IE means that dual cell operation is not supported with MIMO. | REL-9 |
| >>>HS-DSCH physical layer category extension 4 | OP |  | Integer (29,30) | As defined in [35]. See Note 16. Absence of this IE means the multi-cell operation on three cells is not supported. | REL-10 |
| >>>HS-DSCH physical layer category extension 5 | OP |  | Integer (31,32) | As defined in [35]. See Note 17. Absence of this IE means the multi-cell operation on four cells is not supported. | REL-10 |
| >>>HS-DSCH physical layer category extension 6 | OP |  | Integer (33,34) | As defined in [35]. See Note 18. Absence of this IE means the multi-cell operation on five and /or six cells is not supported. | REL-11 |
| >>>HS-DSCH physical layer category extension 7 | OP |  | Integer (35,36) | As defined in [35]. See Note 19. Absence of this IE means the multi-cell operation on seven and/or eight cells is not supported. | REL-11 |
| >>>HS-DSCH physical layer category extension 8 | OP |  | Integer (37,38) | As defined in [35]. See Note 20. Absence of this IE means the MIMO mode with four transmit antennas operation is not supported. | REL-11 |
| >>>HS-SCCHless HS-DSCH operation support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-SCCHless HS-DSCH operation. | REL-7 |
| >>>Enhanced F-DPCH support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support enhanced F-DPCH | REL-7 |
| >>>HS-PDSCH in CELL_FACH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-PDSCH reception in CELL_FACH | REL-7 |
| >>>HS-PDSCH in CELL_PCH and URA_PCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-PDSCH in neither CELL_PCH nor URA_PCH states. | REL-7 |
| >>>Target Cell Pre-Configuration | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-SCCH reception in target cell | REL-8 |
| >>>Support of HS-DSCH DRX operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-DSCH DRX operation | REL-8 |
| >>>Support of MIMO only with single stream restriction | OP |  | Enumerated (TRUE) | The presence of this IE means that the UE supports MIMO only with single stream restriction. | REL-9 |
| >>> Non-contiguous multi-cell with MIMO | OP |  | Enumerated (TRUE) | The presence of this IE means that the UE supports non-contiguous multi-cell with MIMO in the frequency bands where it supports non-contiguous multi-cell operation. | REL-11 |
| >>>Support of MIMO mode with four transmit antennas only with dual stream restriction | OP |  | Enumerated (TRUE) | The presence of this IE means that the UE supports MIMO mode with four transmit antennas operation only with dual stream restriction. | REL-11 |
| >>>Support of NodeB triggered HS-DPCCH transmission | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support NodeB triggered HS-DPCCH transmission | REL-11 |
| >>>Support of HS-DSCH DRX operation with second DRX cycle | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-DSCH DRX operation with second DRX cycle in CELL_FACH state. | REL-11 |
| >>Unsupported |  |  |  | (no data) | REL-5 |
| >DRX enhancements | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support DRX enhancements. | REL-12 |
| >HS-DPCCH overhead reduction | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-DPCCH overhead reduction for multi-RAB with DCH. | REL-12 |
| 3.84 Mcps TDD downlink physical channel capability | CH-3.84_Mcps_tdd_req_sup |  |  |  | Name changed in REL-4 |
| >Maximum number of timeslots per frame | MP |  | Integer (1..14) |  |  |
| >Maximum number of physical channels per frame | MP |  | Integer (1..224) |  |  |
| >Minimum SF | MP |  | Integer (1, 16) |  |  |
| >Support of PDSCH | MP |  | Boolean | TRUE means supported |  |
| >CHOICE Support of HS-PDSCH | CV-not_iRAT_HoInfo |  |  |  | REL-5 |
| >>Supported |  |  |  |  | REL-5 |
| >>>HS-DSCH physical layer category | MP |  | Integer (1..64) | As defined in [35] | REL-5 |
| >>Unsupported |  |  |  | (no data) | REL-5 |
| >Maximum number of physical channels per timeslot | MP |  | Integer (5..16) |  |  |
| 7.68 Mcps TDD downlink physical channel capability | CH-7.68_Mcps_tdd_req_sup |  |  |  | REL-7 |
| >Maximum number of timeslots per frame | MP |  | Integer (1..14) |  | REL-7 |
| >Maximum number of physical channels per frame | MP |  | Integer (1..448) |  | REL-7 |
| >Minimum SF | MP |  | Integer (1, 32) |  | REL-7 |
| >Support of PDSCH | MP |  | Boolean | TRUE means supported | REL-7 |
| >CHOICE Support of HS-PDSCH | CV-not_iRAT_HoInfo |  |  |  | REL-7 |
| >>Supported |  |  |  |  | REL-7 |
| >>>HS-DSCH physical layer category | MP |  | Integer (1..64) | As defined in [35] | REL-7 |
| >>Unsupported |  |  |  | (no data) | REL-7 |
| >Maximum number of physical channels per timeslot | MP |  | Integer (1..32) |  | REL-7 |
| 1.28 Mcps TDD downlink physical channel capability | CH-1.28_Mcps_tdd_req_sup |  |  |  | REL-4 |
| >Maximum number of timeslots per subframe | MP |  | Integer (1..6) |  | REL-4 |
| >Maximum number of physical channels per subframe | MP |  | Integer (1..96) |  | REL-4 |
| >Minimum SF | MP |  | Integer (1, 16) |  | REL-4 |
| >Support of PDSCH | MP |  | Boolean | TRUE means supported | REL-4 |
| >CHOICE Support of HS-PDSCH | CV-not_iRAT_HoInfo |  |  |  | REL-5 |
| >>Supported |  |  |  |  | REL-5 |
| >>>HS-DSCH physical layer category | MP |  | Integer (1..64) | As defined in [35]. Values 16...64 are spares. See Note 8. | REL-5 |
| >>>HS-DSCH physical layer category extension | OP |  | Integer (1..64) | As defined in [35]. See Note 6. | REL-8 |
| >>>Multi-carrier HS-DSCH physical layer category | OP |  | Integer (1..64) | As defined in [35]. Absent if downlink multiple carrier is not supported. Values 45...64 are spares. See Note 10. | REL-7 |
| >>>Multi-carrier HS-DSCH physical layer category extension | OP |  | Integer (1..36) | As defined in [35]. Absent if downlink multiple carrier is not supported. See Note 11. | REL-8 |
| >>>Multi-carrier HS-DSCH physical layer category extension2 | OP |  | Integer (37..64) | This IE shall be used if the Multi-carrier HS-DSCH physical layer category is larger than 36. Values 45...64 are spares. See Note 11. | REL-10 |
| >>>Support of SF Mode For HS-PDSCH dual stream | OP |  | Enumerated (SF1, SF1/SF16) | For,1.28 Mcps TDD only The absence of this IE indicates that the UE does not support MIMO. | REL-8 |
| >>Unsupported |  |  |  | (no data) | REL-5 |
| >Maximum number of physical channels per timeslot | MP |  | Integer (1..16) |  | REL-4 |
| >Support of 8PSK | MP |  | Boolean | TRUE means supported | REL-4 |
| >Support of non-rectangular resource allocation | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | For 1.28 Mcps TDD only.The absence of this IE indicates that the UE does not support the non-rectangular resource allocation. | REL-12 |
| Uplink physical channel capability information elements |  |  |  |  |  |
| FDD uplink physical channel capability | CH-fdd_req_sup |  |  |  |  |
| >Maximum number of DPDCH bits transmitted per 10 ms | MP |  | Integer (600, 1200, 2400, 4800. 9600, 19200. 28800, 38400, 48000, 57600) |  |  |
| >Support of cell reselection indication reporting | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support the report of Cell Reselection Indication during common E-DCH transmission. | REL-12 |
| >Support of Serving E-DCH cell decoupling | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Serving E-DCH cell decoupling operation. | REL-12 |
| >Support of Radio Links without DPCH/F-DPCH | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Radio Links without DPCH/F-DPCH. | REL-12 |
| >CHOICE Support of E-DCH | CV-not_iRAT_HoInfo |  |  |  | REL-6 |
| >>Supported |  |  |  |  | REL-6 |
| >>>E-DCH physical layer category | MP |  | Integer (1..16) | As defined in [35] in Rel-6 Values 7 to 16 are spares. See Note 4. | REL-6 |
| >>>E-DCH physical layer category extension | OP |  | Integer (7) | As defined in [35]. | REL-7 |
| >>>E-DCH physical layer category extension 2 | OP |  | Integer (8..9) | As defined in [35]. See Note 14. Absence of this IE means that Dual Cell E-DCH operation is not supported. | REL-9 |
| >>>E-DCH physical layer category extension 3 | OP |  | Integer (10..12) | As defined in [35]. See Note 21. Absence of this IE means that neither 64 QAM in the uplink nor UL MIMO operation is supported. | REL-11 |
| >>>DPCCH Discontinuous Transmission support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support DPCCH Discontinuous Transmission | REL-7 |
| >>>Slot Format #4 support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Slot Format #4 | REL-7 |
| >>Unsupported |  |  |  | (no data) | REL-6 |
| >Access Groups based access control | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support control of DTCH transmissions in CELL_FACH and CELL_PCH. | REL-12 |
| >Enhanced TTI switching | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Enhanced TTI switching. | REL-12 |
| >Implicit Grant handling | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Implicit Grants on the Secondary Uplink frequency. | REL-12 |
| >DTX enhancements | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support DTX enhancements. | REL-12 |
| >Support of dual cell E-DCH transmission with DPDCH | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support dual cell E-DCH transmission with DPDCH. | REL-13 |
| 3.84 Mcps TDD uplink physical channel capability | CH-3.84_Mcps_tdd_req_sup |  |  |  | Name changed in REL-4 |
| >Maximum Number of timeslots per frame | MP |  | Integer (1..14) |  |  |
| >Maximum number of physical channels per timeslot | MP |  | Integer (1, 2) |  |  |
| >Minimum SF | MP |  | Integer (1, 2, 4, 8) |  |  |
| >Support of PUSCH | MP |  | Boolean | TRUE means supported |  |
| >CHOICE Support of E-DCH | CV-not_iRAT_HoInfo |  |  |  | REL-7 |
| >>Supported |  |  |  |  | REL-7 |
| >>>E-DCH physical layer category | MP |  | Integer (1..16) | As defined in [35] | REL-7 |
| >>Unsupported |  |  |  | (no data) | REL-7 |
| 7.68 Mcps TDD uplink physical channel capability | CH-7.68_Mcps_tdd_req_sup |  |  |  | REL-7 |
| >Maximum Number of timeslots per frame | MP |  | Integer (1..14) |  | REL-7 |
| >Maximum number of physical channels per timeslot | MP |  | Integer (1, 2) |  | REL-7 |
| >Minimum SF | MP |  | Integer (1, 2, 4, 8) |  | REL-7 |
| >Support of PUSCH | MP |  | Boolean | TRUE means supported | REL-7 |
| >CHOICE Support of E-DCH | CV-not_iRAT_HoInfo |  |  |  | REL-7 |
| >>Supported |  |  |  |  | REL-7 |
| >>>E-DCH physical layer category | MP |  | Integer (1..16) | As defined in [35] | REL-7 |
| >>Unsupported |  |  |  | (no data) | REL-7 |
| 1.28 Mcps TDD uplink physical channel capability | CH-1.28_Mcps_tdd_req_sup |  |  |  | REL-4 |
| >Maximum Number of timeslots per subframe | MP |  | Integer (1..6) |  | REL-4 |
| >Maximum number of physical channels per timeslot | MP |  | Integer (1, 2, |  | REL-4 |
|  |  |  | 3, 4) |  | REL-7 |
| >Minimum SF | MP |  | Integer (1, 2, 4, 8, 16) |  | REL-4 |
| >Support of PUSCH | MP |  | Boolean | TRUE means supported | REL-4 |
| >Support of 8PSK | MP |  | Boolean | TRUE means supported | REL-4 |
| >CHOICE Support of E-DCH | CV-not_iRAT_HoInfo |  |  |  | REL-7 |
| >>Supported |  |  |  |  | REL-7 |
| >>>E-DCH physical layer category | MP |  | Integer (1..6) | As defined in [35] In case of multi-carrier E-DCH, this IE indicates the capability for each single carrier. | REL-7 |
| >>>Multi-carrier E-DCH physical layer category | OP |  | Integer (1..8) | As defined in [35] Absent if multiple carrier E-DCH is not supported. See Note 15. | REL-10 |
| >>>Maximum number of Carriers for Multi-carrier E-DCH | OP |  | Enumerated (nf-2, nf-3, nf-6) | One spare value is needed. | REL-10 |
| >>Unsupported |  |  |  | (no data) | REL-7 |
| NOTE 1: Void. |  |  |  |  |  |
| NOTE 2: Void. |  |  |  |  |  |
| NOTE 3: Void. |  |  |  |  |  |
| NOTE 4: All UEs supporting E-DCH should signal a category between 1 and 6 for this IE even if the UE physical capability category is above 6. |  |  |  |  |  |
| NOTE 5: All UEs supporting HS-DSCH should signal a category between 1 and 12 for this IE even if the UE physical capability category is above 12. This IE corresponds to the HS-DSCH category supported by the UE when MAC-ehs is not configured. |  |  |  |  |  |
| NOTE 6: This IE corresponds to the HS-DSCH category supported by the UE when MAC-ehs is configured. |  |  |  |  |  |
| NOTE 7: Void |  |  |  |  |  |
| NOTE 8: All UEs supporting HS-DSCH should signal a category between 1 and 15 for this IE even if the UE physical capability category is above 15. This IE corresponds to the HS-DSCH category supported by the UE when MAC-ehs is not configured. |  |  |  |  |  |
| NOTE 9: Void. |  |  |  |  |  |
| NOTE 10: All UEs supporting multi-carrier HS-DSCH should signal a category between 1 and 18 or 39, 40, 43, 44 for this IE. This IE corresponds to the multi-carrier HS-DSCH category supported by the UE when MAC-ehs is not configured. |  |  |  |  |  |
| NOTE 11: This IE corresponds to the multi-carrier HS-DSCH category supported by the UE when MAC-ehs is configured. |  |  |  |  |  |
| NOTE 12: This IE corresponds to the HS-DSCH category supported by the UE when dual cell operation is configured. |  |  |  |  |  |
| NOTE 13: This IE corresponds to the HS-DSCH category supported by the UE when dual cell operation is configured with MIMO. |  |  |  |  |  |
| NOTE 14: This IE corresponds to the E-DCH category supported by the UE when Dual Cell E-DCH operation is configured. |  |  |  |  |  |
| NOTE 15: This IE corresponds to the E-DCH category supported by the UE when Multi-carrier E-DCH operation is configured. |  |  |  |  |  |
| NOTE 16: This IE corresponds to the HS-DSCH category supported by the UE when it is configured with multi-cell operation on three cells. |  |  |  |  |  |
| NOTE 17: This IE corresponds to the HS-DSCH category supported by the UE when it is configured with multi-cell operation on four cells. |  |  |  |  |  |
| NOTE 18: This IE corresponds to the HS-DSCH category supported by the UE when it is configured with multi-cell operation on five and/or six cells. |  |  |  |  |  |
| NOTE 19: This IE corresponds to the HS-DSCH category supported by the UE when it is configured with multi-cell operation on seven and/or eight cells. |  |  |  |  |  |
| NOTE 20: This IE corresponds to the HS-DSCH category supported by the UE when MIMO mode with four transmit antennas operation is configured. |  |  |  |  |  |
| NOTE 21: This IE corresponds to the E-DCH category supported by the UE when MIMO mode and/or 64QAM is configured. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| 3.84_Mcps_tdd_req_sup | The IE is mandatory present if the IE "TDD RF capability" is present with the IE "Chip rate capability" set to "3.84 Mcps" and a 3.84 Mcps TDD capability update has been requested in a previous message. Otherwise this field is not needed in the message. |
| 7.68_Mcps_tdd_req_sup | The IE is mandatory present if the IE "TDD RF capability" is present with the IE "Chip rate capability" set to "7.68 Mcps" and a 7.68 Mcps TDD capability update has been requested in a previous message. Otherwise this field is not needed in the message. |
| 1.28_Mcps_tdd_req_sup | The IE is mandatory present if the IE "TDD RF capability" is present with the IE "Chip rate capability" set to "1.28 Mcps" and a 1.28 Mcps TDD capability update has been requested in a previous message. Otherwise this field is not needed in the message. |
| fdd_req_sup | The IE is mandatory present if the IE "Multi-mode capability" has the value "FDD" or "FDD/TDD" and a FDD capability update has been requested in a previous message. Otherwise this field is not needed in the message. |
| not_iRAT_HoInfo | The CHOICE Support of HS-PDSCH is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is mandatory present. |
| not_iRAT_HoInfo | The CHOICE Support of E-DCH is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is mandatory present. |
| not_iRAT_HoInfo2 | The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is optional. |

#### 10.3.3.25a	Pre-redirection info

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Support of E-UTRA FDD | MP |  | Boolean |  | REL-8 |
| Support of E-UTRA TDD | MP |  | Boolean |  | REL-8 |

#### 10.3.3.26	Protocol error cause

This IE indicates the cause for a message or information that was not comprehended.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Protocol error cause | MP |  | Enumerated (ASN.1 violation or encoding error, Message type non-existent or not implemented, Message not compatible with receiver state, Information element value not comprehended, Information element missing, Message extension not comprehended) | Two spare values are needed. |

#### 10.3.3.27	Protocol error indicator

This IE indicates whether a message was transmitted due to a protocol error or not.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Protocol error indicator | MP |  | Boolean | TRUE means a protocol error occurred. FALSE means a protocol error did not occur. |

#### 10.3.3.28	RB timer indicator

This IE is used to indicate to UTRAN if the timers T314 or T315 has expired in the UE.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| T314 expired | MP |  | Boolean | TRUE means that the timer has expired or the stored value is zero. FALSE means that the timer has not expired. |
| T315 expired | MP |  | Boolean | TRUE means that the timer has expired or the stored value is zero. FALSE means that the timer has not expired. |

#### 10.3.3.29	Redirection info

This IE is used to redirect the UE to another frequency or other system. With the Release 6 version a list of cells may be provided to the UE, where cell selection shall be started.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Redirection Information | MP |  |  |  |  |
| >Frequency info |  |  | Frequency info 10.3.6.36 |  |  |
| >Inter-RAT info |  |  | Inter-RAT info 10.3.7.25 |  |  |

#### 10.3.3.30	Re-establishment timer

This information element indicates which timer to associate with RAB.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Re-establishment timer | MP |  | Enumerated(useT314, useT315) |  |

#### 10.3.3.31	Rejection cause

Cause for rejection of RRC connection establishment request.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Rejection cause | MP |  | Enumerated(congestion, unspecified) |  |

#### 10.3.3.32	Release cause

Cause for release of RRC connection.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Release cause | MP |  | Enumerated (normal event, unspecified, pre-emptive release, congestion, re-establishment reject, directed signalling connection re-establishment, user inactivity) | One spare value is needed. |

#### 10.3.3.32a	RF Capability Compressed

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE FDD | MP |  |  |  | REL-5 |
| >Supported |  |  |  |  | REL-5 |
| >>RF capability band FDD list Compressed | MP | 1..<maxfreqbandsFDD> |  |  | REL-5 |
|  |  | 1..<maxfreqbandsFDD2> |  |  | REL-6 |
|  |  | 1..<maxfreqbandsFDD3> |  |  | REL-10 |
| >>>RF Capability Band FDD Compressed | MP |  | Enumerated (not supported, default TxRx separation) | TX/RX frequency separation capability for the supported frequency band(s). Default is the TX/RX frequency separation defined in [21] for each frequency band. Two spare values are needed. | REL-5 |
| >Not supported |  |  | NULL |  | REL-5 |
| CHOICE TDD-3.84Mcps | MP |  |  |  | REL-5 |
| >Supported |  |  |  |  | REL-5 |
| >>Radio Frequency Band TDD List | MP |  | Enumerated (a, b, c, a+b, a+c, b+c, a+b+c) | As defined in [22]. One spare value needed | REL-5 |
|  |  |  | Enumerated (a, b, c, d, a+b, a+c, a+d, b+c, b+d, c+d, a+b+c, a+b+d, a+c+d, b+c+d, a+b+c+d) |  | REL-7 |
| >Not supported |  |  | NULL |  | REL-5 |
| CHOICE TDD-7.68Mcps | MP |  |  |  | REL-7 |
| >Supported |  |  |  |  | REL-7 |
| >>Radio Frequency Band TDD List | MP |  | Enumerated (a, b, c, d, a+b, a+c, a+d, b+c, a+b+c, a+b+d, a+c+d, b+c+d, a+b+c+d) | As defined in [22]. One spare value needed | REL-7 |
| >Not supported |  |  | NULL |  | REL-7 |
| CHOICE TDD-1.28Mcps | MP |  |  |  | REL-5 |
| >Supported |  |  |  |  | REL-5 |
| >>Radio Frequency Band TDD List | MP |  | Enumerated (a, b, c, a+b, a+c, b+c, a+b+c) | As defined in [22]. One spare value needed | REL-5 |
|  |  |  | Enumerated (a, b, c, d, a+b, a+c, a+d, b+c, b+d, c+d, a+b+c, a+b+d, a+c+d, b+c+d, a+b+c+d) |  | REL-7 |
| >Not supported |  |  | NULL |  | REL-5 |

#### 10.3.3.33	RF capability FDD

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UE power class | MP |  | Enumerated(1..4) | As defined in [21] |  |
| Tx/Rx frequency separation | MP |  | Enumerated(default TxRx separation) | Default is the TX/RX frequency separation defined in [21] for each frequency band. Two spare values are needed. |  |
| Support of Multiple Frequency Band Indicators | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | FDD only. Absence of this IE means that the UE does not support the signalling requirements of multiple radio frequency bands, as defined in [85], and the UE does not understand the UARFCN signalling for all bands, that overlap with the band(s) supported by the UE. | REL-10 |

| Condition | Explanation |
| --- | --- |
| not_iRAT_HoInfo | The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is optional. |

#### 10.3.3.33a	RF capability FDD extension

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE power class extension | MP |  | Enumerated(1..4) | As defined in [21]. A UE with UE power class 3bis signals the value 3. Four spare values are needed |
| Tx/Rx frequency separation | MP |  | Enumerated(default TxRx separation) | Default is the TX/RX frequency separation defined in [21] for each frequency band. Two spare values are needed. |

#### 10.3.3.33b	RF capability TDD

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UE power class | MP |  | Enumerated (1..4) | as defined in [22] |  |
| Radio frequency bands | MP |  | Enumerated(a, b, c, a+b, a+c, b+c, a+b+c) | as defined in [22]. One spare value needed. |  |
|  |  |  | Enumerated (a, b, c, d, a+b, a+c, a+d, b+c, b+d, c+d, a+b+c, a+b+d, a+c+d, b+c+d, a+b+c+d, |  | REL-7 |
|  |  |  | Notabcd) | as defined in [22]. The value “Notabcd” indicate that the UE does not support neither one of the bands a, b, c nor d. | REL-8 |
| Radio frequency bands extension list | OP | 1 to <maxFreqBandsTDD-ext> |  | as defined in [22]. | REL-8 |
| >Radio frequency bands ext | MP |  | Enumerated (e, f, g, h, i, j, k, l, m, n, o, p) | as defined in [22]. Band g to band p are reserved for future use. | REL-8 |
| Chip rate capability | MP |  | Enumerated (3.84Mcps, 1.28Mcps, | as defined in [22] |  |
|  |  |  | 7.68 Mcps) |  | REL-7 |

#### 10.3.3.33c	RF capability TDD 1.28 Mcps

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Radio frequency bands | MP |  | Enumerated(a, b, c, a+b, a+c, b+c, a+b+c) | as defined in [22]. One spare value needed. |  |
|  |  |  | Enumerated (a, b, c, d, a+b, a+c, a+d, b+c, b+d, c+d, a+b+c, a+b+d, a+c+d, b+c+d, a+b+c+d) |  | REL-7 |

#### 10.3.3.34	RLC capability

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Total RLC AM buffer size | MP |  | Integer (10, 50, 100, 150, 500, 1000, | Total receiving and transmitting RLC AM buffer and MAC-hs/ehs reordering buffer capability in kBytes. Note 1. Note 2. |  |
|  |  |  | 200, 300, 400, 750, |  | REL-5 |
|  |  |  | 1150, 1250, |  | REL-9 |
|  |  |  | 1500, 1800, 2300, 2550, | Total receiving and transmitting RLC AM buffer and MAC-hs/ehs reordering buffer capability in kBytes. Note 2. Four Spare values are needed. | Rel-10 |
|  |  |  | 3400, 3500, 4400, 4500, 5000) |  | REL-11 |
| Maximum RLC AM Window Size | MP |  | Integer(2047,4095) | Maximum supported RLC TX and RX window in UE |  |
| Maximum number of AM entities | MP |  | Integer (4,5,6,8,16,30) |  |  |
| Support for Two logical channel Configuration | CV-not_iRAT_HoInfo |  | Boolean | TRUE means supported | REL-7 |
| Note 1: The IE "Total RLC AM buffer size" values 200, 300, 400 and 750 are not used in the INTER RAT HANDOVER INFO message. |  |  |  |  |  |
| Note 2: The IE "Total RLC AM buffer size" values 1150, 1250, 1500, 1800, 2300, 2550, 3400, 3500, 4400, 4500 and 5000 are not used in the INTER RAT HANDOVER INFO message. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| not_iRAT_HoInfo | The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is mandatory present. |

#### 10.3.3.35	RLC re-establish indicator

This IE is used to re-configure AM RLC on c-plane and u-plane.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RLC re-establish indicator | MP |  | Boolean | TRUE means re-establish required FALSE means re-establish not required |

#### 10.3.3.35a	RRC State Indicator

Indicates to a UE the RRC state to be entered.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RRC State indicator | MP |  | Enumerated(CELL_DCH, CELL_FACH, CELL_PCH, URA_PCH) |  |

#### 10.3.3.35o	RRC connection release information

Indicates whether the UE shall perform a release of the RRC connection.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Release indicator | MD |  |  | Default value is “No release” | REL-5 |
| >No release |  |  |  |  | REL-5 |
| >Release |  |  |  |  | REL-5 |
| >>Release cause | MP |  | Release cause 10.3.3.32 |  | REL-5 |

#### 10.3.3.36	RRC transaction identifier

This IE contains an identification of the RRC procedure transaction local for the type of the message this IE was included within.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE RRC Tr Id type | MP |  |  |  | REL-5 |
| >Normal |  |  |  |  |  |
| >>RRC transaction identifier | MP |  | Integer (0..3) |  |  |
| >Extended |  |  |  |  | REL-5 |
| >>RRC transaction identifier | MP |  | Integer (0..15) |  | REL-5 |

| CHOICE RRC Tr Id type | Condition under which the given RRC Tr Id type is chosen |
| --- | --- |
| Normal | All cases where the RRC Tr Id type: Extended is not chosen. |
| Extended | Optional in the MEASUREMENT CONTROL message. Mandatory in the MEASUREMENT CONTROL FAILURE message. |

#### 10.3.3.36a	rSR-VCC Info

This IE contains information that allows the calculation of the CK and IK for the PS domain and the uplink traffic transmission information, due to an rSR-VCC procedure.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| NONCE | OP |  | Bit string (128) |  | REL-11 |
| IMS information | MP |  | Octet String(SIZE(1..32)) | Contains ATGW transfer details as defined in [24.237] | REL-11 |

#### 10.3.3.37	Security capability

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Ciphering algorithm capability | MP |  |  |  |  |
| >UEA0 | MP |  | Boolean |  |  |
| >UEA1 | MP |  | Boolean |  |  |
| >UEA2 | MP |  | Boolean | The value TRUE means that UEA2 is fully interoperability tested. If fully interoperability tested, also "Security capability indication" of "UE radio access capability compressed" shall be set to TRUE. | REL-7 |
| >Spare | MP | 13 | Boolean | Shall be set to FALSE by UEs complying with this version of the protocol. |  |
| Integrity protection algorithm capability | MP |  |  |  |  |
| >UIA1 | MP |  | Boolean | The value TRUE means that UIA1, Kasumi, is supported |  |
| >UIA2 | MP |  | Boolean | The value TRUE means that UIA2 is fully interoperability tested. If fully interoperability tested, also "Security capability indication" of "UE radio access capability compressed" shall be set to TRUE. | REL-7 |
| >Spare | MP | 14 | Boolean | Shall be set to FALSE by UEs complying with this version of the protocol. |  |

#### 10.3.3.37a	Signalling Connection Release Indication Cause

This IE is used to indicate to the UTRAN that there is no more PS data for a prolonged period.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Signallng Connection Release Indication Cause | MP |  | Enumerated (UE Requested PS Data session end, any other cause) |  | REL-8 |

#### 10.3.3.38	START

There is a START value per CN domain. The START is used to initialise the 20 MSBs of all hyper frame numbers (MAC-d HFN, RLC UM HFN, RLC AM HFN, RRC HFN) for a CN domain.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| START | MP |  | Bit string (20) | START [40]. The first/leftmost bit of the bit string contains the most significant bit of the START. |

#### 10.3.3.39	Void

#### 10.3.3.40	Transport channel capability

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Downlink transport channel capability information elements |  |  |  |  |
| Max no of bits received | MP |  | Integer(640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840) | Maximum sum of number of bits of all transport blocks received at an arbitrary time instant |
| Max convolutionally coded bits received | MP |  | Integer(640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840) | Maximum sum of number of bits of all convolutionally coded transport blocks received at an arbitrary time instant |
| Max turbo coded bits received | CV-turbo_dec_sup |  | Integer(640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840) | Maximum sum of number of bits of all turbo coded transport blocks received at an arbitrary time instant |
| Maximum number of simultaneous transport channels | MP |  | Integer(4, 8, 16, 32) |  |
| Maximum number of simultaneous CCTrCH | MP |  | Integer (1..8) |  |
| Max no of received transport blocks | MP |  | Integer(4, 8, 16, 32, 48, 64, 96, 128, 256, 512) | Maximum total number of transport blocks received within TTIs that end at within the same 10ms interval |
| Maximum number of TFC | MP |  | Integer(16, 32, 48, 64, 96, 128, 256, 512, 1024) |  |
| Maximum number of TF | MP |  | Integer(32, 64, 128, 256, 512, 1024) |  |
| Support for turbo decoding | MP |  | Boolean | TRUE means supported |
| Uplink transport channel capability information elements |  |  |  |  |
| Max no of bits transmitted | MP |  | Integer(640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840) | Maximum sum of number of bits of all transport blocks transmitted at an arbitrary time instant |
| Max convolutionally coded bits transmitted | MP |  | Integer(640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840) | Maximum sum of number of bits of all convolutionally coded transport blocks transmitted at an arbitrary time instant |
| Max turbo coded bits transmitted | CV-turbo_enc_sup |  | Integer(640, 1280, 2560, 3840, 5120, 6400, 7680, 8960, 10240, 20480, 40960, 81920, 163840) | Maximum sum of number of bits of all turbo coded transport blocks transmitted at an arbitrary time instant |
| Maximum number of simultaneous transport channels | MP |  | Integer(4, 8, 16, 32) |  |
| Maximum number of simultaneous CCTrCH of DCH type | CH-tdd_req_sup |  | Integer (1..8) |  |
| Max no of transmitted transport blocks | MP |  | Integer(4, 8, 16, 32, 48, 64, 96, 128, 256, 512) | Maximum total number of transport blocks transmitted within TTIs that start at the same time |
| Maximum number of TFC | MP |  | Integer(16, 32, 48, 64, 96, 128, 256, 512, 1024) |  |
| Maximum number of TF | MP |  | Integer(32, 64, 128, 256, 512, 1024) |  |
| Support for turbo encoding | MP |  | Boolean | TRUE means supported |

| Condition | Explanation |
| --- | --- |
| turbo_dec_sup | The IE is mandatory present if the IE "Support of turbo decoding" = TRUE. Otherwise this field is not needed in the message. |
| turbo_enc_sup | The IE is mandatory present if the IE "Support of turbo encoding" = TRUE. Otherwise this field is not needed in the message. |
| tdd_req_sup | The IE is mandatory present if the IE "Multi-mode capability" has the value "TDD" or "FDD/TDD" and a TDD capability update has been requested in a previous message. Otherwise this field is not needed in the message. |

#### 10.3.3.41	UE multi-mode/multi-RAT capability

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Multi-RAT capability |  |  |  |  |  |
| Support of GSM | MP |  | Boolean |  |  |
| Support of multi-carrier | MP |  | Boolean |  |  |
| Multi-mode capability | MP |  | Enumerated (TDD, FDD, FDD/TDD) |  |  |
| Support of UTRAN to GERAN NACC | CV-not_iRAT_HoInfo |  | Boolean |  | REL-5 |
| Support of Handover to GAN | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportHandoverToGAN) | Absence of this IE means that the UE does not support Handover to GAN. | REL-6 |
| Support of Inter-RAT PS handover | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportInter-RAT-PS-Handover) | Absence of this IE means that the UE does not support Inter-RAT PS Handover to GERAN | REL-6 |
| Support of PS Handover to GAN | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportPSHandoverToGAN) | Absence of this IE means that the UE does not support PS Handover to GAN. | REL-7 |
| Support of E-UTRA FDD | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportEUTRAFDD) | Absence of this IE means that the UE does not support E-UTRA FDD | REL-8 |
| Support of Inter-RAT PS Handover to E-UTRA FDD | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportInter-RATHOToEUTRAFDD) | Absence of this IE means that the UE does not support Inter-RAT PS Handover to E-UTRA FDD | REL-8 |
| Support of E-UTRA TDD | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportEUTRATDD) | Absence of this IE means that the UE does not support E-UTRA TDD | REL-8 |
| Support of Inter-RAT PS Handover to E-UTRA TDD | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportInter-RATHOToEUTRATDD) | Absence of this IE means that the UE does not support Inter-RAT PS Handover to E-UTRA TDD | REL-8 |
| EUTRA Feature Group Indicators | CV-not_iRAT_HoInfo2 |  | Bit string (4) | The definitions of the bits are described in Annex E | REL-8 |
| Support of E-UTRA Multiple Frequency Band Indicators | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | Absence of this IE means that the UE does not support the signalling requirements of multiple radio frequency bands in E-UTRA, as defined in [67], and the UE does not understand the EARFCN signalling for all bands, that overlap with the band(s) supported by the UE. | REL-10 |
| Support of E-UTRA FDD measurements and reporting in CELL_FACH | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportEUTRAmeasurementsandreportinginCELLFACH) | Absence of this IE means that the UE does not support E-UTRA measurement for CELL_FACH for E-UTRA FDD | REL-11 |
| Support of E-UTRA TDD measurements and reporting in CELL_FACH | CV-not_iRAT_HoInfo2 |  | Enumerated (DoesSupportEUTRAmeasurementsandreportinginCELLFACH) | Absence of this IE means that the UE does not support E-UTRA measurement for CELL_FACH for E-UTRA TDD | REL-11 |
| Support of RAN-assisted WLAN interworking based on RAN rules | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | Absence of this IE means that the UE does not support RAN-assisted WLAN interworking based on access network selection and traffic steering rules. | REL-12 |
| Support of RAN-assisted WLAN interworking based on ANDSF policies | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | Absence of this IE means that the UE does not support RAN-assisted WLAN interworking based on ANDSF policies. | REL-12 |

| Condition | Explanation |
| --- | --- |
| not_iRAT_HoInfo | The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is mandatory present. |
| not_iRAT_HoInfo2 | The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is optional. |

NOTE:	The UE sets the capability of Inter-RAT PS Handover (by means of the IE "Support of Inter-RAT PS handover") to the same value as the corresponding GERAN capability in [5].

#### 10.3.3.42	UE radio access capability

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Access stratum release indicator | MP |  | Enumerated(R99) | Indicates the release of the UE according to [35]. The IE also indicates the release of the RRC transfer syntax supported by the UE. |  |
|  | CV-not_rrc_connectionSetupComplete |  | Enumerated(REL-4, | 6 spare values are needed. | REL-4 |
|  |  |  | REL-5, |  | REL-5 |
|  |  |  | REL-6, |  | REL-6 |
|  |  |  | REL-7, |  | REL-7 |
|  |  |  | REL-8, |  | REL-8 |
|  |  |  | REL-9, |  | REL-9 |
|  |  |  | REL-10, |  | REL-10 |
|  |  |  | REL-11, |  | REL-11 |
|  |  |  | REL-12, |  | REL-12 |
|  |  |  | REL-13) |  | REL-13 |
| DL capability with simultaneous HS-DSCH configuration | CV-not_iRAT_HoInfo |  | Enumerated(32kbps, 64kbps, 128kbps, 384kbps) |  | REL-5 |
| PDCP capability | MP |  | PDCP capability 10.3.3.24 |  |  |
| RLC capability | MP |  | RLC capability 10.3.3.34 |  |  |
| Transport channel capability | MP |  | Transport channel capability 10.3.3.40 |  |  |
| RF capability FDD | OP |  | RF capability FDD 10.3.3.33 |  |  |
| RF capability TDD | OP |  | RF capability TDD 10.3.3.33b | One "TDD RF capability" entity shall be included for every Chip rate capability supported. |  |
|  |  | 1 to 2 |  | Note 1 | REL-4 |
| RF capability TDD 1.28 Mcps | CV-iRAT_HoInfo |  | RF capability TDD 1.28 Mcps 10.3.3.33c | Note 1 | REL-4 |
| Physical channel capability | MP |  | Physical channel capability 10.3.3.25 |  |  |
| UE multi-mode/multi-RAT capability | MP |  | UE multi-mode/multi-RAT capability 10.3.3.41 |  |  |
| Security capability | MP |  | Security capability 10.3.3.37 |  |  |
| UE positioning capability | MP |  | UE positioning capability 10.3.3.45 |  |  |
| Measurement capability | OP |  | Measurement capability 10.3.3.21 |  |  |
| Measurement capability TDD | OP |  | Measurement capability TDD 10.3.3.21b |  | REL-8 |
| Device type | MD |  | Enumerated (DoesNotBenefitFromBatteryConsumptionOptimisation) | Absence of this value means that the device does benefit from NW-based battery consumption optimisation. UE may set the value to DoesNotBenefitFromBatteryConsumptionOptimisation when it does not foresee to particularly benefit from NW-based The IE is not needed in the INTER RAT HANDOVER INFO message. | REL-6 |
| Support for System Information Block type 11bis | OP |  | Enumerated (TRUE) |  | REL-6 |
|  |  |  |  | The IE shall be present and set to TRUE | REL-7 |
| Support for F-DPCH | CV-SupportForHS |  | Enumerated (TRUE) | Presence of this information element indicates that F-DPCH is supported by the UE. Absence of this information element indicates that F-DPCH is not supported | REL-6 |
| MAC-ehs support | OP |  | Enumerated (TRUE) | The presence of this IE indicates that UE supports MAC-ehs, the use of special value of HE field to indicate end of an SDU for RLC AM, octet aligned transport block table and different HS-SCCHs in contiguous TTIs. The absence of this IE indicates that the UE does not support either MAC-ehs, octet aligned transport block table or the use of special value of HE field to indicate end of an SDU for RLC AM or different HS-SCCHs in contiguous TTIs. | REL-7 |
| UE specific capability Information LCR TDD | OP |  | Enumerated (NF, TriRxUniTx, TriRxTriTx, HexRxUniTx, HexRxTriTx, HexRxHexT, | For 1.28 Mcps TDD only | REL-7 |
|  |  |  | TwoRxUniTxDiscontiguous, TwoRxTwoTxDiscontiguous, TwoRxUniTxContiguous, TwoRxTwoTxContiguous) | TwoRxUniTxDiscontiguous and TwoRxTwoTxDiscontiguous mean that the UE is capable of supporting two non-adjacent carriers. TwoRxUniTxContiguous and TwoRxTwoTxContiguous mean that the UE is only capable of supporting two adjacent carriers. 6 spare values needed. Note 3. | REL-10 |
| Support for E-DPCCH Power Boosting | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the E-DPCCH Power Boosting is not supported. The presence of this IE indicates that the E-DPCCH Power Boosting is supported by the UE. | REL-7 |
| Support of common E-DCH | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support E-DCH enhanced random access in CELL_FACH state and Idle mode. | REL-8 |
| Support of MAC-i/is | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support MAC-i/is operation. | REL-8 |
| Support of Common E-RGCH based interference control | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Common E-RGCH based interference control in CELL_FACH state. | REL-11 |
| Support of Fallback to R99 PRACH | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Fallback to R99 PRACH in CELL_FACH state and IDLE mode. | REL-11 |
| Support of Concurrent deployment | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Concurrent deployment of 2ms and 10ms TTI in a cell in CELL_FACH state and IDLE mode. | REL-11 |
| Support of TTI alignment and Per HARQ process | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support TTI alignment and Per HARQ process activation and de-activation in CELL_FACH state and IDLE mode. | REL-11 |
| Support of SPS operation | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the UE does not support SPS operation. | REL-8 |
| Support of Control Channel DRX operation | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the UE does not support Control Channel DRX operation | REL-8 |
| Support of CSG | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support access control based on CSG | REL-8 |
| Support for Two DRX schemes in URA_PCH and CELL_PCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Two DRX schemes in URA_PCH and CELL_PCH | REL-7 |
| Support for E-DPDCH power interpolation formula | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support E-DPDCH power interpolation formula when 16QAM is not configured | REL-7 |
| Support for absolute priority based cell re-selection in UTRAN | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support absolute priority based cell re-selection to UTRA inter-frequency | REL-8 |
| Support of MU-MIMO | OP |  | Enumerated (Uplink, Uplink and Downlink) | The absence of this IE indicates that the UE does not support MU-MIMO. This IE is used for 1.28 Mcps TDD only | REL-10 |
| Radio Access Capability Band Combination List | OP | 1 to 16 |  | The absence of this IE indicates that the UE does not support Dual Band Operation. | REL-9 |
| >Band Combination | MP |  | Integer(1..256) | The integer value n indicates that the nth DB-DC Configuration (A,B) in table 5.0AA in [21] is supported by the UE. | REL-9 |
| >Support for Dual Band Dual Cell E-DCH operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Dual Band Dual Cell E-DCH operation on the corresponding band combination indicated by the IE "Band Combination". | REL-13 |
| >Supported Carrier Combination | OP |  |  | Absence of this IE means that the UE only supports the carrier combination (1,1) for this band combination (A,B). If carrier combination (X,Y) is supported, then carrier combination (M,N) is supported, where 1<=M<=X and 1<=N<=Y, so the corresponding value for that carrier combination (M,N) should be set to TRUE. Carrier combination (X,Y) for a band combination (A,B) refers to a block of X contiguous carriers in Band A and a block of Y contiguous carriers in Band B. | REL-10 |
| >>Carrier Combination (1,2) | MP |  | Boolean | The value TRUE means that carrier combination (1,2) is supported for this band combination (A,B). | REL-10 |
| >>Carrier Combination (2,1) | MP |  | Boolean | The value TRUE means that carrier combination (2,1) is supported for this band combination (A,B). | REL-10 |
| >>Carrier Combination (1,3) | MP |  | Boolean | The value TRUE means that carrier combination (1,3) is supported for this band combination (A,B). | REL-10 |
| >>Carrier Combination (3,1) | MP |  | Boolean | The value TRUE means that carrier combination (3,1) is supported for this band combination (A,B). | REL10 |
| >>Carrier Combination (2,2) | MP |  | Boolean | The value TRUE means that carrier combination (2,2) is supported for this band combination (A,B). | REL-10 |
| >>Carrier Combination (1,4) | MP |  | Boolean | The value TRUE means that carrier combination (1,4) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (4,1) | MP |  | Boolean | The value TRUE means that carrier combination (4,1) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (1,5) | MP |  | Boolean | The value TRUE means that carrier combination (1,5) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (5,1) | MP |  | Boolean | The value TRUE means that carrier combination (5,1) is supported for this band combination (A,B). | REL11 |
| >>Carrier Combination (1,6) | MP |  | Boolean | The value TRUE means that carrier combination (1,6) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (6,1) | MP |  | Boolean | The value TRUE means that carrier combination (6,1) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (1,7) | MP |  | Boolean | The value TRUE means that carrier combination (1,7) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (7,1) | MP |  | Boolean | The value TRUE means that carrier combination (7,1) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (2,3) | MP |  | Boolean | The value TRUE means that carrier combination (2,3) is supported for this band combination (A,B). | REL11 |
| >>Carrier Combination (3,2) | MP |  | Boolean | The value TRUE means that carrier combination (3,2) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (2,4) | MP |  | Boolean | The value TRUE means that carrier combination (2,4) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (4,2) | MP |  | Boolean | The value TRUE means that carrier combination (4,2) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (2,5) | MP |  | Boolean | The value TRUE means that carrier combination (2,5) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (5,2) | MP |  | Boolean | The value TRUE means that carrier combination (5,2) is supported for this band combination (A,B). | REL11 |
| >>Carrier Combination (2,6) | MP |  | Boolean | The value TRUE means that carrier combination (2,6) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (6,2) | MP |  | Boolean | The value TRUE means that carrier combination (6,2) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (3,3) | MP |  | Boolean | The value TRUE means that carrier combination (3,3) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (3,4) | MP |  | Boolean | The value TRUE means that carrier combination (3,4) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (4,3) | MP |  | Boolean | The value TRUE means that carrier combination (4,3) is supported for this band combination (A,B). | REL11 |
| >>Carrier Combination (4,4) | MP |  | Boolean | The value TRUE means that carrier combination (4,4) is supported for this band combination (A,B). | REL11 |
| >>Carrier Combination (3,5) | MP |  | Boolean | The value TRUE means that carrier combination (3,5) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (5,3) | MP |  | Boolean | The value TRUE means that carrier combination (5,3) is supported for this band combination (A,B). | REL-11 |
| >Supported Multiflow Combination | OP |  |  | Presence of this IE indicates that UE supports Multiflow for this band combination. Note 5. | REL-11 |
|  |  |  |  | Note 6 | REL-13 |
| >>Number of cells in band A | MP |  | Integer(1..6) | Total number of cells in band A that can be configured to a UE as a part of Multiflow configuration. In this version of the specification, the maximum value is 2. | REL-11 |
|  |  |  |  | In this version of the specification, the maximum value is 3. | REL-13 |
| >>Number of frequencies in band A | MP |  | Integer(1..3) | Total number of frequencies in band A, across which the Multiflow can be configured. In this version of the specification, the maximum value is 1. | REL-11 |
|  |  |  |  | In this version of the specification, the maximum value is 2. | REL-13 |
| >>Number of cells in band B | MP |  | Integer(1..6) | Total number of cells in band B that can be configured to a UE as a part of Multiflow configuration. In this version of the specification, the maximum value is 2. | REL-11 |
| >>Number of frequencies in band B | MP |  | Integer(1..3) | Total number of frequencies in band B, across which the Multiflow can be configured. In this version of the specification, the maximum value is 1. | REL-11 |
| Support of multi-cell configuration in inter-RAT handover | CV-iRAT_HoInfo |  | Enumerated (TRUE) |  | REL-11 |
| Support of TX Diversity on DL Control Channels by MIMO Capable UE when MIMO operation is active | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the MIMO capable UE does not support TX diversity on DL Control Channels (HS-SCCH, F-DPCH, E-AGCH, E-HICH, E-RGCH) when MIMO is active, P-CPICH is configured on antenna 1, and S-CPICH on antenna 2. | REL-7 |
|  |  |  |  | The absence of this IE indicates that the MIMO capable UE does not support TX diversity on DL Control Channels (HS-SCCH, F-DPCH, E-AGCH, E-HICH, E-RGCH, F-TPICH) when MIMO is active, P-CPICH is configured on antenna 1, and S-CPICH on antenna 2. | REL-11 |
| Support of enhanced TS0 | OP |  | Enumerated (TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the UE does not support the enhanced TS0 | REL-9 |
| Support for cell-specific Tx diversity configuration for dual-cell operation | CV- Dual_Cell_HSDPA_supported |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support cell-specific Tx diversity configuration when configured for dual-cell operation. | REL-8 |
| CSG proximity indication capability | CV-not_iRAT_HoInfo |  | CSG proximity indication capability 10.3.3.8a |  | REL-9 |
| Neighbour Cell SI acquisition capability | CV-not_iRAT_HoInfo |  | Neighbour Cell SI acquisition capability 10.3.3.21c |  | REL-9 |
| Extended measurements Support | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE supports the extended measurement identity range 17 to 32. | REL-9 |
| Support for dual cell with MIMO operation in different bands | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support dual cell with MIMO operation in different bands | REL-10 |
| UE based network performance measurements parameters | CV-not_iRAT_HoInfo |  | UE based network performance measurements parameters 10.3.3.53 |  | REL-10 |
| Support of UTRAN ANR | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support UTRAN ANR. | REL-10 |
| IMS Voice capability | CV-not_iRAT_HoInfo |  | IMS Voice capability 10.3.3.14b |  | REL-9 |
| Multiflow capability | CV-not_iRAT_HoInfo |  | UE Multiflow capability 10.3.3.21ba |  | REL-11 |
| Support of MAC-ehs window size extension | OP |  | Enumerated (TRUE) | For 1.28 Mcps TDD only, the absence of this IE indicates that the UE does not support IE "MAC-ehs window size extension". Note 4. | REL-9 |
| Support of UM RLC re-establishment via reconfiguration | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support UM RLC re-establishment via reconfiguration. | REL-10 |
| Support of HS-DPCCH power offset extension | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support the HS-DPCCH power offset extension. | REL-11 |
| Support of STTD on DL Control Channels when Multiflow operation is active | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that a Multiflow capable UE does not support STTD on DL Control Channels (HS-SCCH, F-DPCH, E-AGCH, E-HICH, E-RGCH, F-TPICH) when Multiflow operation is active. | REL-11 |
| Support of F-TPICH feedback from the Multiflow assisting cell | CV-not_iRAT_HoInfo |  | Enumerated(TRUE) | The absence of this IE indicates that the UE does not support reception of the F-TPICH feedback from the Multiflow assisting cell. | REL-12 |
| Support of DSAC and PPAC update in CELL_DCH | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support DSAC and PPAC update in CELL_DCH. | REL-12 |
| Note 1: The second entity of the "RF capability TDD" is not needed in the INTER RAT HANDOVER INFO message: if both TDD 3.84/7.68 Mcps and TDD 1.28 Mcps are supported, the "RF capability TDD 1.28 Mcps" entity shall be used for TDD 1.28 Mcps; the "UE power class" in the "RF capability TDD" entity shall apply for both chip rates. |  |  |  |  |  |
| Note 2: For 1.28 Mcps TDD, UE supporting E-DCH enhanced random access in CELL_FACH state and Idle mode always supports HS-PDSCH reception in CELL_FACH, CELL_PCH and URA_PCH state and CELL_FACH HS-DSCH DRX operation. |  |  |  |  |  |
| Note 3: Two non-adjacent carriers can be in the same band or in two different bands. |  |  |  |  |  |
| Note 4: This IE should always be set to “TRUE” in this version of the specification. |  |  |  |  |  |
| Note 5: If the Multiflow scenario dual-band three cells on two frequencies is supported in band combination A&B (2 cells in band A and 1 cell in band B), then it is also supported in band combination B&A (2 cells in band B and 1 cell in band A). |  |  |  |  |  |
| Note 6: If the Multiflow scenario dual-band four cells on three frequencies is supported in band combination A&B (with 3 cells in band A and 1 cell in band B), then it is also supported in band combination B&A (with 3 cells in band B and 1 cell in band A), and in band combination A&B (with 2 cells in band A and 2 cells in band B), and in band combination B&A (with 2 cells in band B and 2 cells in band A). |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| not_rrc_connectionSetupComplete | The IE is not needed in the RRC CONNECTION SETUP COMPLETE message. Otherwise the IE is mandatory present. |
| not_iRAT_HoInfo | The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is optional. |
| iRAT_HoInfo | The IE is optional in the INTER RAT HANDOVER INFO message. Otherwise, the IE is not needed. |
| Dual_Cell_HSDPA_supported | This IE is optionally present if Dual-Cell HSDPA is supported. Otherwise it is not needed. The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is optional. |
| SupportForHS | This IE is mandatory present if the UE supports HS-PDSCH, otherwise it is not needed. The IE is not needed in the INTER RAT HANDOVER INFO message. |

#### 10.3.3.42o	UE radio access capability compressed

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Access stratum release indicator | MP |  | Enumerated(R99, REL-4, REL-5, | 6 spare values are needed. | REL-5 |
|  |  |  | REL-6, |  | REL-6 |
|  |  |  | REL-7, |  | REL-7 |
|  |  |  | REL-8, |  | REL-8 |
|  |  |  | REL-9, |  | REL-9 |
|  |  |  | REL-10, |  | REL-10 |
|  |  |  | REL-11, |  | REL-11 |
|  |  |  | REL-12, |  | REL-12 |
|  |  |  | REL-13) |  | REL-13 |
| Total AM RLC buffer size exceeds 10 kByte | MP |  | BOOLEAN |  | REL-5 |
| RF capability compressed | MP |  | RF capability compressed 10.3.3.32a |  | REL-5 |
| Support for System Information Block type 11bis | OP |  | Enumerated (TRUE) |  | REL-6 |
|  |  |  |  | The IE shall be present and set to TRUE | REL-7 |
| MAC-ehs support | OP |  | Enumerated (TRUE) | The presence of this IE indicates that UE supports MAC-ehs, the use of special value of HE field to indicate end of an SDU for RLC AM, octet aligned transport block table and different HS-SCCHs in contiguous TTIs. Absent if either MAC-ehs, octet aligned transport block table or the use of special value of HE field to indicate end of an SDU for RLC AM or different HS-SCCHs in contiguous TTIs are not supported | REL-7 |
| Security capability indication | OP |  | Enumerated (TRUE) | TRUE indicates UE security capabilities beyond R99 requirements, see 10.3.3.37 | REL-7 |
| GANSS support indication | OP |  | Enumerated (TRUE) | TRUE indicates that the UE supports GANSS | REL-7 |

#### 10.3.3.42oa	UE radio access capability comp 2

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| FDD physical channel capability for HS-PDSCH/E-DCH | OP |  |  | Absent if HS-PDSCH and E-DCH are not supported | REL-6 |
| >DL capability with simultaneous HS-DSCH configuration | OP |  | Enumerated (32, 64, 128, 384) | Unit: kbps. This IE is always required, but the need is set to OP to align with ASN.1 | REL-6 |
| >HS-DSCH physical layer category | MP |  | Integer (1..64) | As defined in [35] Values 13...64 are spare. | REL-6 |
| >HS-DSCH physical layer category extension | OP |  | Integer (1..20) | As defined in [35]. See Note 1. | REL-7 |
| >HS-DSCH physical layer category extension 2 | OP |  | Integer (21..24) | As defined in [35]. See Note 2. | REL-8 |
| >HS-DSCH physical layer category extension 3 | OP |  | Integer (25..28) | As defined in [35]. See Note 3. | REL-9 |
| >HS-DSCH physical layer category extension 4 | OP |  | Integer (29,30) | As defined in [35]. See Note 4. | REL-10 |
| >HS-DSCH physical layer category extension 5 | OP |  | Integer (31,32) | As defined in [35]. See Note 5. | REL-10 |
| >HS-DSCH physical layer category extension 6 | OP |  | Integer (33,34) | As defined in [35]. See Note 6. | REL-11 |
| >HS-DSCH physical layer category extension 7 | OP |  | Integer (35,36) | As defined in [35]. See Note 7. | REL-11 |
| >HS-DSCH physical layer category extension 8 | OP |  | Integer (37,38) | As defined in [35]. See Note 8. | REL-11 |
| >CHOICE Support of E-DCH | MP |  |  |  | REL-6 |
| >>Supported |  |  |  |  | REL-6 |
| >>>E-DCH physical layer category | MP |  | Integer (1..16) | As defined in [35] | REL-6 |
| >>>E-DCH physical layer category extension | OP |  | Integer (7) | As defined in [35]. | REL-7 |
| >>>E-DCH physical layer category extension 2 | OP |  | Integer (8..9) | As defined in [35]. | REL-9 |
| >>>E-DCH physical layer category extension 3 | OP |  | Integer(10..12) | As defined in [35]. | REL-11 |
| >>Unsupported |  |  |  | (no data) | REL-6 |
| Support for F-DPCH | CV-SupportForHS |  | Enumerated (TRUE) |  | REL-6 |
|  |  |  |  | Presence of this information element indicates that F-DPCH is supported by the UE. Absence of this information element indicates that F-DPCH is not supported | REL-7 |
| HS-SCCHless HS-DSCH operation support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-SCCHless HS-DSCH operation. | REL-7 |
| Enhanced F-DPCH support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support enhanced F-DPCH | REL-7 |
| HS-PDSCH in CELL_FACH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-PDSCH reception in CELL_FACH | REL-7 |
| HS-PDSCH in CELL_PCH and URA_PCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-PDSCH in neither CELL_PCH nor URA_PCH states. | REL-7 |
| DPCCH Discontinuous Transmission support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support DPCCH Discontinuous Transmission | REL-7 |
| Slot Format #4 support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Slot Format #4 | REL-7 |
| Support for CS Voice over HSPA | OP |  | Enumerated (TRUE) | The IE indicates the UE’s support for CS Voice over HSPA, if set. | REL-8 |
| Support for E-DPCCH Power Boosting | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the E-DPCCH Power Boosting is not supported. The presence of this IE indicates that the E-DPCCH Power Boosting is supported by the UE. | REL-7 |
| Support of MAC-i/is | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support MAC-i/is operation. | REL-8 |
| Support of TX Diversity on DL Control Channels by MIMO Capable UE when MIMO operation is active | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the MIMO capable UE does not support TX diversity on DL Control Channels (HS-SCCH, F-DPCH, E-AGCH, E-HICH, E-RGCH) when MIMO is active, P-CPICH is configured on antenna 1, and S-CPICH on antenna 2. | REL-7 |
|  |  |  |  | The absence of this IE indicates that the MIMO capable UE does not support TX diversity on DL Control Channels (HS-SCCH, F-DPCH, E-AGCH, E-HICH, E-RGCH, F-TPICH) when MIMO is active, P-CPICH is configured on antenna 1, and S-CPICH on antenna 2. | REL-11 |
| Support of HS-DPCCH power offset extension | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support the HS-DPCCH power offset extension. | REL-11 |
| Support of DSAC and PPAC update in CELL_DCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support DSAC and PPAC update in CELL_DCH. | REL-12 |
| Support for DCH Enhancements | OP |  | Enumerated (Basic, Full) | The absence of this IE indicates that the UE does not support DCH Enhancements | REL-12 |
| Simultaneous support for DCH Enhancements and Compressed Mode operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support simultaneous operation of DCH Enhancements and Compressed Mode. | REL-12 |
| Simultaneous support for DCH Enhancements and DPCCH Discontinuous Transmission | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support simultaneous operation of DCH Enhancements and DPCCH Discontinuous Transmission. | REL-12 |
| Support of dual cell E-DCH transmission with DPDCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support dual cell E-DCH transmission with DPDCH. | REL-13 |

NOTE 1:	This IE corresponds to the HS-DSCH category supported by the UE when MAC-ehs is configured.

NOTE 2:	This IE corresponds to the HS-DSCH category supported by the UE when dual cell operation is configured.

NOTE 3:	This IE corresponds to the HS-DSCH category supported by the UE when dual cell operation with MIMO on adjacent frequencies or in different bands is configured.

NOTE 4:	This IE corresponds to the HS-DSCH category supported by the UE when it is configured with multi-cell operation on three cells.

NOTE 5:	This IE corresponds to the HS-DSCH category supported by the UE when it is configured with multi-cell operation on four cells.

NOTE 6:	This IE corresponds to the HS-DSCH category supported by the UE when it is configured with multi-cell operation on five or six cells.

NOTE 7:	This IE corresponds to the HS-DSCH category supported by the UE when it is configured with multi-cell operation on seven or eight cells.

NOTE 8:	This IE corresponds to the HS-DSCH category supported by the UE when MIMO mode with four transmit antennas operation is configured.

| Condition | Explanation |
| --- | --- |
| not_iRAT_HoInfo | The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is optional. |
| SupportForHS | This IE is mandatory present if the UE supports HS-PDSCH, otherwise it is not needed. The IE is not needed in the INTER RAT HANDOVER INFO message. |

#### 10.3.3.42ob	UE radio access capability comp for 1.28 Mcps TDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1.28 Mcps TDD physical channel capability for HS-PDSCH/E-DCH | OP |  |  | Absent if HS-PDSCH and E-DCH are not supported | REL-8 |  |
| >Support of SF Mode For HS-PDSCH dual stream | OP |  | Enumerated (SF1, SF1/SF16) | For,1.28 Mcps TDD only The absence of this IE indicates that the UE does not support MIMO. | REL-8 |  |
| Support of common E-DCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support E-DCH enhanced random access in CELL_FACH state and Idle mode. | REL-8 |  |
| Support of MAC-i/is | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support MAC-i/is operation. | REL-8 |  |
| Support of Control Channel DRX operation | OP |  | Enumerated (TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the UE does not support Control Channel DRX operation | REL-8 |  |
| Support of SPS operation | OP |  | Enumerated (TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the UE does not support SPS operation. | REL-8 |  |
| Radio frequency bands extension list | OP | 1 to <maxFreqBandsTDD-ext> |  | as defined in [22]. | REL-8 |  |
| >Radio frequency bands ext | MP |  | Enumerated (e, f, g, h, i, j, k, l, m, n, o, p) | as defined in [22]. Band g to band p are reserved for future use. | REL-8 |  |
| NOTE 1: All UEs supporting HS-DSCH should signal a category between 1 and 15 for this IE even if the UE physical capability category is above 15. This IE corresponds to the HS-DSCH category supported by the UE when MAC-ehs is not configured. |  |  |  |  |  |
| NOTE 2: This IE corresponds to the HS-DSCH category supported by the UE when MAC-ehs is configured. |  |  |  |  |  |
| NOTE 3: All UEs supporting multi-carrier HS-DSCH should signal a category between 1 and 18 for this IE even if the UE physical capability category is above 18. This IE corresponds to the multi-carrier HS-DSCH category supported by the UE when MAC-ehs is not configured. |  |  |  |  |  |
| NOTE 4: This IE corresponds to the multi-carrier HS-DSCH category supported by the UE when MAC-ehs is configured. |  |  |  |  |  |

#### 10.3.3.42a	UE radio access capability extension

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Frequency band specific capability list | MP | 1 to <maxFreqbandsFDD> |  | The network should ignore the entry that includes the ‘extension indicator’. |  |
|  |  | 1 to <maxFreqbandsFDD2> |  |  | REL-6 |
|  |  | 1 to <maxFreqbandsFDD3> |  |  | REL-10 |
| >Frequency band | MP |  | Enumerated(Band I, Band II, |  |  |
|  |  |  | Band III, |  | REL-5 |
|  |  |  | Band VI, Band IV, Band V, Band VII, extension indicator) | The setting of the value ‘extension indicator’ by the UE is not specified in the specification. | REL-6 |
| >Frequency band 2 | OP |  | Enumerated(Band VIII, Band IX, Band X, Band XI, Band XII, Band XIII, Band XIV, Band XIX, Band XXI, Band XX, Band XXII, extension Indicator) | This IE indicates the supported frequency bands Band VIII and beyond. The setting of the value ‘extension indicator’ by the UE is not specified in the specification. Four spare values are needed | REL-6 |
| >Frequency band 3 | OP |  | Enumerated (Band XXV, Band XXVI, extension Indicator) | This IE indicates the supported frequency bands Band XXIII and beyond. The setting of the value ‘extension indicator’ by the UE is not specified in the specification. Sixty One spare values are needed | REL-10 |
| >RF capability FDD extension | MD |  | RF capability FDD extension 10.3.3.33a | the default values are the same values as in the immediately preceding IE "RF capability FDD extension"; the first occurrence is MP |  |
| >Measurement capability extension | MP |  | Measurement capability extension 10.3.3.21a |  |  |
| >Additional Secondary Cells | OP |  | Enumerated(a1, a2) | The number of additional secondary serving cells supported by the UE. The absence of this IE means that the UE does not support multi-cell operation on three cells or multi-cell operation on four cells. | REL-10 |
| >Additional Secondary Cells 2 | OP |  | Enumerated( a3, a4, a5, a6) | The number of additional secondary serving cells supported by the UE. The absence of this IE means that the UE does not support multi-cell operation on five cells or multi-cell operation on six cells or multi-cell operation on seven cells or multi-cell operation on eight cells. | REL-11 |
| >Non-contiguous multi-cell | OP | 1 to <maxNonContiguousMultiCellCombinations> |  | The presence of this IE indicates that the UE supports the non-contiguous multi-cell HSDPA operation on two, three or four cells. | REL-10 |
| >>Aggregated cells | MP |  | Enumerated(nc-2c, nc-3c, nc-4c) | This IE indicates the maximum number of cells supported in non-contiguous multi-cell operation. "nc-2c" indicates that UE supports 2 cells. "nc-3c" indicates that UE supports 2 and 3 cells. "nc-4c" indicates that UE supports 2, 3, and 4 cells. | REL-10 |
| >>Gap size | MP |  | Enumerated(fiveMHz, tenMHz, anyGapSize) | This IE indicates the maximum gap size between the aggregated cells. "fiveMHz" indicates that UE supports 5 MHz gap size. "tenMHz" indicates that UE supports 10 MHz gap size and 5 MHz gap size. "anyGapSize" indicates that UE supports any multiple of 5 MHz gap size. 5 spare values are required. | REL-10 |
| >>Non-contiguous multi-cell Combination (2,2) | CV-NC-4C |  | Enumerated(TRUE) | The presence of this IE indicates that the UE supports an equal number of contiguous cells on each side of the gap. The absence of this IE indicates that the carrier combination (2,2) is not supported. | REL-10 |
| >>Non-contiguous multi-cell Combination (3,1) (1,3) | CV-NC-4C |  | Enumerated(TRUE) | The presence of this IE indicates that UE supports a different number of contiguous cells on each side of the gap. The absence of this IE indicates that neither the carrier combination (3,1) nor the carrier combination (1,3) are supported. | REL-10 |
| >Support of uplink closed loop transmit diversity | OP |  | Enumerated(TRUE) | Absence of this IE indicates that uplink CLTD is not supported on this frequency band. | REL-11 |
| >Support of uplink open loop transmit diversity | OP |  | Enumerated(TRUE) | Absence of this IE indicates that uplink OLTD is not supported. | REL-11 |
| >Multiflow per band capability | OP |  | Multiflow per band capabilty 10.3.3.21bb |  | REL-11 |
| >Support of Uplink MIMO | OP |  | Enumerated(TRUE) | Absence of this IE indicates that UL MIMO is not supported in this frequency band. | REL-11 |
| >MIMO mode with four transmit antennas per band capability | OP |  | Enumerated(TRUE) | Absence of this IE indicates that MIMO mode with four transmit antennas is not supported on this frequency band. | REL-11 |
| >Frequency specific compressed mode for non-contiguous operation | OP |  | Enumerated(TRUE) | Absence of this IE indicates that the frequency specific compressed mode for non- contiguous operation is not supported in this band. | REL-11 |

| Condition | Explanation |
| --- | --- |
| NC-4C | This IE is optional when the IE "aggregated cells" is set to "nc-4c". Otherwise it is not needed. When the IE "aggregated cells" is set to "nc-4c", at least one of the optional IE shall be included. |

#### 10.3.3.42b	UE security information

Upon receiving a UE information request from another system, the UE shall indicate the requested security information. The UE security information includes the following RRC information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE information elements |  |  |  |  |
| START-CS | MP |  | START 10.3.3.38 | START values to be used in this CN domain. |

#### 10.3.3.42c	UE security information2

Upon receiving a UE information request from another system, the UE shall indicate the requested security information. The UE security information includes the following RRC information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| START-PS | MP |  | START 10.3.3.38 | START values to be used in this CN domain. | Rel-6 |

#### 10.3.3.43	UE Timers and Constants in connected mode

This information element specifies timer- and constants values used by the UE in connected mode.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| T301 | MD |  | Integer(100, 200 .. 2000 by step of 200, 3000, 4000, 6000, 8000) | Value in milliseconds. Default value is 2000. This IE should not be used by the UE in this release of the protocol. One spare value is needed. |  |
| N301 | MD |  | Integer(0..7) | Default value is 2. This IE should not be used by the UE in this release of the protocol. |  |
| T302 | MD |  | Integer(100, 200... 2000 by step of 200, 3000, 4000, 6000, 8000) | Value in milliseconds. Default value is 4000. One spare value is needed. |  |
| N302 | MD |  | Integer(0..7) | Default value is 3. |  |
| T304 | MD |  | Integer(100, 200, 400, 1000, 2000) | Value in milliseconds. Default value is 2000. Three spare values are needed. |  |
| N304 | MD |  | Integer(0..7) | Default value is 2.. |  |
| T305 | MD |  | Integer(5, 10, 30, 60, 120, 360, 720, infinity) | Value in minutes. Default value is 30. Infinity means no update |  |
| T307 | MD |  | Integer(5, 10, 15, 20, 30, 40, 50) | Value in seconds. Default value is 30. One spare value is needed. |  |
| T308 | MD |  | Integer(40, 80, 160, 320) | Value in milliseconds. Default value is 160. |  |
| T309 | MD |  | Integer(1…8) | Value in seconds. Default value is 5. |  |
| T310 | MD |  | Integer(40 .. 320 by step of 40) | Value in milliseconds. Default value is 160. |  |
| N310 | MD |  | Integer(0 .. 7) | Default value is 4. |  |
| T311 | MD |  | Integer(250 .. 2000 by step of 250) | Value in milliseconds. Default value is 2000. |  |
| T312 | MD |  | Integer (0..15) | Value in seconds. Default value is 1. The value 0 is not used in this version of the specification. |  |
| N312 | MD |  | Integer (1, 2, 4, 10, 20, 50, 100, 200, 400, 600, 800, 1000) | Default value is 1. |  |
| T313 | MD |  | Integer (0..15) | Value in seconds. Default value is 3. |  |
| N313 | MD |  | Integer (1, 2, 4, 10, 20, 50, 100, 200) | Default value is 20. |  |
| T314 | MD |  | Integer(0, 2, 4, 6, 8, 12, 16, 20) | Value in seconds. Default value is 12. |  |
| T315 | MD |  | Integer (0,10, 30, 60, 180, 600, 1200, 1800) | Value in seconds. Default value is 180. |  |
| N315 | MD |  | Integer (1, 2, 4, 10, 20, 50, 100, 200, 400, 600, 800, 1000) | Default value is 1. |  |
| T316 | MD |  | Integer(0, 10, 20, 30, 40, 50, infinity) | Value in seconds. Default value is 30. One spare value is needed. |  |
| T317 | MD |  |  | Default value is infinity. |  |
|  |  |  | Enumerated (infinity, infinity, infinity, infinity, infinity, infinity, infinity, infinity) | All the values are changed to "infinity" in the Rel-5. | REL-5 |
| T323 | OP |  | Enumerated (0, 5, 10, 20, 30, 60, 90, 120) | Value in seconds. The use of 0secs indicates no need to apply the inhibit timer. See Note 1 | REL-8 |

NOTE 1:	If T323 is configured or cleared via non-critical extensions of UTRAN MOBILITY INFORMATION message, UTRAN should explicitly signal the other timers and constants in the same message. Otherwise the T323 configuration change is not applied to UE.

#### 10.3.3.44	UE Timers and Constants in idle mode

This information element specifies timer- and constant values used by the UE in idle mode.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| T300 | MP |  | Integer(100, 200... 2000 by step of 200, 3000, 4000, 6000, 8000) | Value in milliseconds. Default value is 1000. Use of Default is described in 10.2.48.8.4 and in 10.2.48.8.16 |
| N300 | MP |  | Integer(0..7) | Default value is 3. Use of Default is described in 10.2.48.8.4 and in 10.2.48.8.16 |
| T312 | MP |  | Integer(0 .. 15) | Value in seconds. Default value is 1. Use of Default is described in 10.2.48.8.4 and in 10.2.48.8.16 The value 0 is not used in this version of the specification. |
| N312 | MP |  | Integer (1, 2, 4, 10, 20, 50, 100, 200, 400, 600, 800, 1000) | Default value is 1. Use of Default is described in 10.2.48.8.4 and in 10.2.48.8.16 |

#### 10.3.3.45	UE positioning capability

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Standalone location method(s) supported | MP |  | Boolean | Defines if a UE can measure its location by some means unrelated to UTRAN TRUE means supported |  |
| UE based OTDOA supported | MP |  | Boolean | TRUE means supported |  |
| Network Assisted GPS support | MP |  | Enumerated ('Network based', 'UE based', 'Both', 'None') | Defines if the UE supports network based or UE based GPS methods. |  |
| Network Assisted GANSS support List | CV-not_iRAT_HoInfo | 1 to <maxGANSS> |  | Absent if GANSS is not supported | REL-7 |
| >GANSS ID | OP |  | Enumerated( | Absence of this IE means Galileo. Eight spare values needed. | REL-7 |
|  |  |  | SBAS, Modernized_GPS, QZSS, GLONASS, | Absence of this IE means Galileo. Four spare values needed. | REL-8 |
|  |  |  | BDS) | Absence of this IE means Galileo. Three spare values needed. | REL-12 |
| >SBAS IDs | CV-GANSS-ID-SBAS |  | Bit String(8) | Defines the specific SBAS(s) supported. This is represented using a bit string with one bit per SBAS as defined in NOTE 1 where a one value indicates support and a zero value no support. | REL-8 |
| >GANSS mode | MP |  | Enumerated ('Network based', 'UE based', 'Both', 'None') | Defines if the UE supports network based or UE based GANSS methods | REL-7 |
| >GANSS Signal ID | OP |  | 10.3.3.45a | Absence of this field means the default value as defined in 10.3.3.45a for the GANSS identified by GANSS ID. | REL-7 |
| >GANSS Signal IDs | OP |  | Bit String(8) | Defines if a UE has the capability to perform measurements on more than one GANSS signal and which signals are supported. This is represented using a bit string with one bit per signal as defined in NOTE 2 where a one value indicates support and a zero value no support. | REL-8 |
| >Support for GANSS timing of cell frames measurement | MP |  | Boolean | Defines if a UE has the capability to perform the UE GANSS timing of cell frames measurement. TRUE means capable | REL-7 |
| >Support for GANSS Carrier-Phase Measurement | OP |  | Boolean | Defines if a UE has the capability to perform the UE GANSS Carrier-Phase Measurement. TRUE means capable | REL-7 |
| >Support for non-native assistance choices | OP |  | Enumerated (TRUE) | Absence of this element means not supported and presence means the UE supports assistance data choices in formats not defined in the ICD of a particular GANSS. Multiple choices exist for assistance data elements defined in 10.3.7.89a, 10.3.7.94a/b, 10.3.7.97d | REL-8 |
| GANSS support indication | CV-iRAT_HoInfo |  | Enumerated (TRUE) | TRUE indicates that the UE supports GANSS | REL-7 |
| Support for GPS timing of cell frames measurement | MP |  | Boolean | Defines if a UE has the capability to perform the UE GPS timing of cell frames measurement [7]. TRUE means capable |  |
| Support for IPDL | MP |  | Boolean | Defines if a UE has the capability to use IPDL to enhance its 'SFN-SFN observed time difference –type 2' measurement. TRUE means supported |  |
| Support for Rx-Tx time difference type2 measurement | MP |  | Boolean | TRUE means supported |  |
| Support for UP assisted GPS measurement validity in CELL_PCH and URA_PCH states | CV-GPSsupported |  | Enumerated (TRUE) |  |  |
| Support for SFN-SFN observed time difference type 2 measurement | OP |  | Enumerated (TRUE) | Absence of this element means not supported and presence means supported. |  |
| AddPos support List | CV-not_iRAT_HoInfo | 1 to <maxAddPos> |  | Absent if Additional Positioning methods are not supported | REL-13 |
| >AddPos ID | MP |  | Enumerated (‘Barometric Pressure’, ‘WLAN’, ‘Bluetooth’, ‘MBS’) | Four spare values are needed | REL-13 |
| >AddPos mode | MP |  | Enumerated ('standalone', 'UE assisted', 'Both') | Defines if the UE supports network based or UE based Additional Positioning methods. One spare value is needed. | REL-13 |

| Condition | Explanation |
| --- | --- |
| GPSsupported | This IE is mandatory present if the IE “Network Assisted GPS support” is set to 'Network based', 'UE based' or 'Both'. Otherwise, it is not needed. |
| not_iRAT_HoInfo | These IEs are not needed in the INTER RAT HANDOVER INFO message. Otherwise, they are optionally present. |
| iRAT_HoInfo | This IE is optionally present in the INTER RAT HANDOVER INFO message. Otherwise, the IE is not needed. |
| GANSS-ID-SBAS | This IE is mandatory present if the IE “GANSS ID” is “SBAS” and not needed otherwise. |

NOTE 1:	Coding of SBAS IDs:

| SBAS IDs Bit String(8) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 (LSB) |
| WAAS | EGNOS | MSAS | GAGAN | - | - | - | - |

NOTE 2:	Coding of GANSS Signal IDs:

| GANSS | GANSS Signal IDs Bit String(8) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 (LSB) |
| Galileo | E1 | E5a | E5b | E6 | E5a+E5b | - | - | - |
| Modernized GPS | L1C | L2C | L5 | - | - | - | - | - |
| SBAS | L1 | - | - | - | - | - | - | - |
| QZSS | QZS-L1 | QZS-L1C | QZS-L2C | QZS-L5 | - | - | - | - |
| GLONASS | G1 | G2 | G3 | - | - | - | - | - |
| BDS | B1I | - | - | - | - | - | - | - |

#### 10.3.3.45a	GANSS Signal Id

The GANSS Signal ID encodes the identification of the signal for each GANSS. It depends on the GANSS Id.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GANSS Signal Identity per Signal Id | MD |  | INTEGER (0..7) | See NOTE 1 | REL-7 |

NOTE 1:

| GANSS Id | Value | Explanation | Version |
| --- | --- | --- | --- |
| Default: Galileo | Default Value | Galileo E1 | REL-7 |
|  | 0 | Galileo E5A |  |
|  | 1 | Galileo E5B |  |
|  | 2 | Galileo E6 |  |
|  | 3 | Galileo E5A + E5B |  |
|  | 4-7 | Reserved |  |
| Modernized GPS | Default Value | GPS L1C | REL-8 |
|  | 0 | GPS L2C |  |
|  | 1 | GPS L5 |  |
|  | 2-7 | Reserved |  |
| SBAS | Default Value | L1 |  |
|  | 0-7 | Reserved |  |
| QZSS | Default Value | QZS-L1 |  |
|  | 0 | QZS-L1C |  |
|  | 1 | QZS-L2C |  |
|  | 2 | QZS-L5 |  |
|  | 3-7 | Reserved |  |
| GLONASS | Default Value | GLONASS G1 |  |
|  | 0 | GLONASS G2 |  |
|  | 1 | GLONASS G3 |  |
|  | 2-7 | Reserved |  |
| BDS | Default Value | B1I | REL-12 |
|  | 0-7 | Reserved |  |

#### 10.3.3.46	URA update cause

Indicates the cause for s URA update.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| URA update cause | MP |  | Enumerated(change of URA, periodic URA update) | One spare value is needed. |

#### 10.3.3.47	U-RNTI

The U-RNTI (UTRAN Radio Network Temporary Identity) is allocated to a UE having a RRC connection and identifies the UE within UTRAN.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRNC identity | MP |  | bit string(12) | The SRNC ID unique in PLMN is encoded to SRNC identity IE in descending order from b31 . The b31 is the most significant bit. If the SRNC ID is more than 12 bits as defined in [57], the remaining bits are included as most significant bits of S-RNTI IE. If the UE ID unique in SRNS encoded in S-RNTI IE is more than 20 bits as defined in [57], the most significant bits are included as the least significant bits of SRNC ID. |
| S-RNTI | MP |  | bit string(20) | The UE ID unique in SRNS is encoded to the S-RNTI IE in descending order. If the S-RNTI IE is more than 20 bits as defined in [57], the remaining bits are included as the least significant bits of SRNC ID. If the SRNC ID is more than 12 bits as defined in [57], S-RNTI include both part of the SRNC ID and the UE-ID. The least significant bits encode the UE-ID to the S-RNTI IE in descending order. The b0 is the least significant bit. |

#### 10.3.3.47a	U-RNTI group

The U-RNTI group is used to identify a group of UEs having an RRC connection.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE group discrimimator | MP |  |  |  | REL-5 |
| >All |  |  |  | (no data) | REL-5 |
| >U-RNTI mask |  |  |  |  | REL-5 |
| >>U-RNTI | MP |  | U-RNTI 10.3.3.47 | The bits that are less significant than the bit position indicated by the U-RNTI bit mask index shall be ignored. | REL-5 |
| >>U-RNTI bit mask index | MP |  | Enumerated(b1, b2,..b31) | Values b1 to b31 indicate bit positions in the S-RNTI and in the SRNC identity. | REL-5 |

#### 10.3.3.48	U-RNTI Short

The U-RNTI (UTRAN Radio Network Temporary Identity) is allocated to a UE having a RRC connection and identifies the UE within UTRAN.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRNC identity | MP |  | bit string(12) | The SRNC identity bits are numbered b20 to b31, where b20 is the least significant bit. If the SRNC ID is more than 12 bits as defined in [57], the remaining bits are included as most significant bits of S-RNTI 2 IE. |
| S-RNTI 2 | MP |  | bit string(10) | The UE ID is encoded in S-RNTI 2 in descending order. If the SRNC ID is more than 12 bits as defined in [57], S-RNTI include both part of the SRNC ID and the UE-ID. The least significant bits encode the UE-ID to the S-RNTI IE in descending order. The b0 is the least significant bit. |

#### 10.3.3.49	UTRAN DRX cycle length coefficient

A coefficient in the formula to count the paging occasions to be used by a specific UE (specified in [4]).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| DRX cycle length coefficient | MP |  | Integer(3...9) | Refers to 'k' in the formula as specified in [4], Discontinuous reception |  |
| DRX cycle length coefficient 2 | MD |  | Integer(3..9) | Refers to 'k' in the formula as specified in [4], Discontinuous reception. The default value is “same as DRX cycle length coefficient”. | REL-7 |
| Time for DRX cycle 2 | MD |  | Integer(80, 160, 320, 640, 1280, 2560, 5120) | Values in milliseconds. The default value is 0 ms. | REL-7 |

#### 10.3.3.50	Wait time

Wait time defines the time period the UE has to wait before repeating the rejected procedure.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Wait time | MP |  | Integer(0.. 15) | Wait time in seconds The value 0 indicates that repetition is not allowed. Wait time should be set to zero if the IE "Extended Wait Time" is present in the RRC Connection Reject message. |

#### 10.3.3.51	UE Specific Behaviour Information 1 idle

This IE indicates the UE conformance typically for RRC connection establishment from idle mode.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Specific Behaviour Information 1 idle | MP |  | bit string(4) |  |

#### 10.3.3.52	UE Specific Behaviour Information 1 interRAT

This IE indicates the UE conformance typically for RRC connection establishment from another RAT.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Specific Behaviour Information 1 interRAT | MP |  | bit string(8) |  |

#### 10.3.3.53	UE based network performance measurements parameters

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Support of Logged Measurements Idle PCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support logged measurements in Idle mode, URA_PCH or CELL_PCH states. | REL-10 |

### 10.3.4	Radio Bearer Information elements

#### 10.3.4.oa	Common RB mapping info

NOTE:	For FDD and 1.28 Mcps TDD only.

A multiplexing option between a logical channel and a MAC-ehs queue.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Logical channel identity | MP |  | Integer(1..15) | Note | REL-7 |
| MAC-ehs queue identity | MP |  | MAC-ehs Queue Id 10.3.5.7f |  | REL-7 |

NOTE:	the IE "Logical channel identity" conveys the value to be used in the "LCH-ID" field of the MAC-ehs header [15] associated with the MAC-ehs reordering queue identified by the IE "MAC-ehs queue identity".

#### 10.3.4.0	Default configuration identity

This information element identifies a default radio parameter configuration. The corresponding default configurations are specified in subclause 13.7.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Default configuration identity | MP |  | Integer (0..10, | See NOTE 1 |  |
|  |  |  | 11, |  | REL-4 |
|  |  |  | 13, | Default configuration ID 2 is not used. | REL-5 |
|  |  |  | 12, 14, 15, 16 |  | REL-6 |
|  |  |  | 17, | Used only in RRC Connection Setup message. | REL-6 |
|  |  |  | 18..22, | See NOTE 1 | REL-6 |
|  |  |  | 23, | Default configuration 23 shall not be used in this version of the specification. | REL-9 |
|  |  |  | 24, |  | REL-10 |
|  |  |  | 25..31) | Reserved for future extension | REL-10 |
| NOTE 1: For TDD, default configuration identities 0, 1, 5, 6, 7 and 22 are only valid from REL-10. |  |  |  |  |  |

#### 10.3.4.0a	Default configuration for CELL_FACH

This information element identifies a default radio parameter configuration for CELL_FACH. The corresponding default configurations are specified in subclause 13.8.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Default configuration mode | MP |  | Enumerated (FDD, TDD) | Indicates whether the FDD or TDD version of the default configuration shall be used | REL-8 |
| Default configuration identity for CELL_FACH | MP |  | Integer (0, |  | REL-8 |
|  |  |  | 1..15) | Reserved for future extension | REL-8 |

#### 10.3.4.1	Downlink RLC STATUS info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Timer_Status_Prohibit | OP |  | Integer(10..550 by step of 10, 550..1000 by step of 50) | Minimum time in ms between STATUS reports |  |
| Missing PDU Indicator | MP |  | Boolean | Value TRUE indicates that UE should send a STATUS report for each missing PDU that is detected |  |
| Timer_STATUS_periodic | OP |  | Integer(100,200, 300, 400, 500, 750, 1000, 2000) | Time in milliseconds |  |
| Timer_Reordering | CV-Multiflow |  | Integer(50,100,150,200,400,600,800,1000) | Time in milliseconds for which a UE should wait upon encountering a gap in the RLC sequence numbers. NOTE1. | REL-11 |
| NOTE1: If this IE is included for SRB0 or SRB1, then the UE behaviour is unspecified. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Multiflow | The IE is optional when the Multiflow operation is configured, otherwise it is not needed. |

#### 10.3.4.1a	PDCP context relocation info

This information element indicates that the header compression context relocation is to be performed during SRNS relocation for the given radio bearer.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| RB identity | MP |  | RB identity 10.3.4.16 |  | REL-5 |
| Downlink RFC 3095 context relocation indication | MP |  | Boolean | TRUE means RFC 3095 context relocation is performed in downlink | REL-5 |
| Uplink RFC 3095 context relocation indication | MP |  | Boolean | TRUE means RFC 3095 context relocation is performed in uplink | REL-5 |

#### 10.3.4.2	PDCP info

The purpose of the PDCP info IE is to indicate which algorithms shall be established and to configure the parameters of each of the algorithms.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Support for lossless SRNS relocation or for lossless DL RLC PDU size change | CV-LosslessCriteria |  | Boolean | TRUE means support |  |
| Max PDCP SN window size | CV-Lossless |  | Enumerated(sn255, sn65535) | Maximum PDCP sequence number window size. The handling of sequence number when the Max PDCP SN window size is 255 is specified in [23]. |  |
| PDCP PDU header | MP |  | Enumerated (present, absent) | Whether a PDCP PDU header is existent or not. |  |
| Header compression information | OP | 1 to <maxPDCPAlgoType> |  |  |  |
| >CHOICE algorithm type | MP |  |  | Note 1 |  |
| >>RFC 2507 |  |  |  | Header compression according to IETF standard RFC 2507 |  |
| >>>F_MAX_PERIOD | MD |  | Integer (1..65535) | Largest number of compressed non-TCP headers that may be sent without sending a full header. Default value is 256. |  |
| >>>F_MAX_TIME | MD |  | Integer (1..255) | Compressed headers may not be sent more than F_MAX_TIME seconds after sending last full header. Default value is 5. |  |
| >>>MAX_HEADER | MD |  | Integer (60..65535) | The largest header size in octets that may be compressed. Default value is 168. |  |
| >>>TCP_SPACE | MD |  | Integer (3..255) | Maximum CID value for TCP connections. Default value is 15. |  |
| >>>NON_TCP_SPACE | MD |  | Integer (3..65535) | Maximum CID value for non-TCP connections. Default value is 15. |  |
| >>>EXPECT_REORDERING | MD |  | Enumerated (reordering not expected, reordering expected) | Whether the algorithm shall reorder PDCP SDUs or not. Default value is "reordering not expected". |  |
| >>RFC 3095 |  |  |  | Header compression according to IETF ROHC standard [83], [84]. | REL-4 |
| >>>Profiles | MP | 1 to <maxROHC- Profiles> |  | Profiles supported by both compressor and decompressor in both UE and UTRAN. Profile 0 shall always be supported. | REL-4 |
| >>>>Profile instance | MP |  | Integer(1.. 3) | 1 = 0x0001, 2 = 0x0002, 3 = 0x0003 (see [52]) | REL-4 |
| >>>Uplink | OP |  |  | Indicates the necessary information elements for Uplink. | REL-4 |
| >>>>Max_CID | MD |  | Integer (1.. 16383) | Highest context ID number to be used by the UE compressor. Default value is 15. | REL-4 |
| >>>Downlink | OP |  |  | Indicates the necessary information elements for Downlink. | REL-4 |
| >>>>Max_CID | MD |  | Integer (1.. 16383) | Highest context ID number to be used by the UE decompressor. Default value is 15. | REL-4 |
| >>>>Reverse_Decompression_Depth | MD |  | Integer (0..65535) | Determines whether reverse decompression should be used or not and the maximum number of packets that can be reverse decompressed by the UE decompressor. Default value is 0 (reverse decompression shall not be used). | REL-4 |
| Note 1: If several occurrences of the same algorithm type are included in the same IE “header compression information”, the UE behaviour is unspecified. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| LosslessCriteria | This IE is mandatory present if the IE "RLC mode" is "Acknowledged", the IE "In-sequence delivery " is TRUE and the IE "SDU Discard Mode" is "No discard" and not needed otherwise. |
| Lossless | This IE is mandatory present if the IE "Support for lossless SRNS relocation or for lossless RLC PDU size change" Is TRUE, otherwise it is not needed. |

#### 10.3.4.2a	PDCP ROHC target mode

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Target Mode | MP |  | Enumerated (O-mode, R-mode) | The UE shall only transit to the signalled mode for operation of ROHC as decribed in [36]. | REL-5 |

#### 10.3.4.3	PDCP SN info

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Receive PDCP sequence number | MP |  | Integer(0..65535) | The PDCP sequence number, which the sender of the message is expecting next to be received. |

#### 10.3.4.4	Polling info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Timer_poll_prohibit | OP |  | Integer(10..550 by step of 10, 600..1000 by step of 50) | Minimum time between polls in ms |
| Timer_poll | OP |  | Integer(10..550 by step of 10, 600..1000 by step of 50) | Time in ms. |
| Poll_PDU | OP |  | Integer(1,2,4,8,16,32,64,128) | Number of PDUs, interval between pollings |
| Poll_SDU | OP |  | Integer(1,4,16,64) | Number of SDUs, interval between pollings |
| Last transmission PDU poll | MP |  | Boolean | TRUE indicates that poll is made at last PDU in transmission buffer |
| Last retransmission PDU poll | MP |  | Boolean | TRUE indicates that poll is made at last PDU in retransmission buffer |
| Poll_Window | OP |  | Integer(50,60,70,80,85,90,95,99) | Percentage of transmission window, threshold for polling |
| Timer_poll_periodic | OP |  | Integer(100,200, 300, 400, 500, 750, 1000, 2000) | Time in milliseconds Timer for periodic polling. |

#### 10.3.4.5	Predefined configuration identity

This information element identifies a pre- defined radio parameter configuration.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predefined radio configuration identity | MP |  | Integer (0..15) |  |

#### 10.3.4.5a	Predefined configuration status information

Another system may provide the UE with one or more predefined UTRAN configurations, comprising of radio bearer, transport channel and physical channel parameters. If requested, the UE shall indicate the configurations it has stored. The predefined configuration status information should include the following RRC information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RB information elements |  |  |  |  |
| Predefined configurations |  | maxPredefConfigCount |  | The list is in order of preconfiguration identity |
| >Predefined configuration value tag | OP |  | Predefined configuration value tag 10.3.4.6 | The UE shall include the value tag if it has stored the concerned configuration |

| Multi Bound | Explanation |
| --- | --- |
| MaxPredefConfigCount | Maximum number of predefined configurations |

#### 10.3.4.5b	Predefined configuration status information compressed

Another system may provide the UE with one or more predefined UTRAN configurations, comprising of radio bearer, transport channel and physical channel parameters. If requested, the UE shall indicate the configurations it has stored. The compressed predefined configuration status information should include the following RRC information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Sets with different value tags | MP |  |  |  | REL-5 |
| >Pre-defined configuration set with different value tags | MP | 1..2 |  |  | REL-5 |
| >>Start position | MD |  | INTEGER (0..10) | Default value is 0, corresponding with the first pre- defined configuration. The pre-defined configuration where the consecutive sequence of pre-defined configurations begins. | REL-5 |
| >>Pre-defined configuration value tag list | MP | 6..<maxPredefConfig> | Pre-defined configuration value tag 10.3.4.6 | Value Tags for each pre-defined configuration starting from the lowest. | REL-5 |
| Other Entries | OP |  |  |  | REL-5 |
| >Pre-defined configuration list with variable size | MP | 1..<maxPredefConfig> |  | List of other pre-defined configurations not included within the Sets with different value tags, in consecutive order starting with the lowest. If there are stored pre-defined configurations positioned after a pre-defined configuration that is not stored, the UE shall indicate the not-stored pre-defined configuration by explicitly indicating it to be absent. If there are no stored pre-defined configurations positioned after a pre-defined configuration that is not stored, then the UE may totally omit these pre-defined configurations from the IE, i.e. reduce the size of the list to correspond to the last position that contained a stored pre-defined configuration. | REL-5 |
| >>Predefined configuration value tag | OP |  | Predefined configuration value tag 10.3.4.6 | The UE shall include the value tag if it has stored the concerned configuration | REL-5 |

#### 10.3.4.6	Predefined configuration value tag

This information element is used to identify different versions of a radio bearer configuration as may be used within one PLMN e.g. to support different UTRAN implementations.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predefined configuration value tag | MP |  | Integer(0..15) |  |

#### 10.3.4.7	Predefined RB configuration

This information element concerns a pre- defined configuration of radio bearer parameters

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE information elements |  |  |  |  |
| Re-establishment timer | MP |  | Re-establishment timer 10.3.3.30 | Only one RAB supported |
| Signalling radio bearer information |  |  |  |  |
| Signalling RB information to setup List | MP | 1 to <maxSRBsetup> |  | For each signalling radio bearer |
| >Signalling RB information to setup | MP |  | Signalling RB information to setup 10.3.4.24 |  |
| RB information |  |  |  | Only one RAB supported |
| RB information to setup list | MP | 1 to <maxRBperRAB> |  |  |
| >RB information to setup | MP |  | RB information to setup 10.3.4.20 |  |

#### 10.3.4.8	RAB info

This IE contains information used to uniquely identify a radio access bearer.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| RAB identity | MP |  | RAB identity 10.3.1.14 |  |  |
| MBMS Service Identity | CV-Message |  | MBMS service ID 10.3.9a.8a | Included when establishing a RB for a MBMS service of the broadcast type; NOTE 1 | REL-6 |
| MBMS Session identity | CV-Message |  | MBMS Session identity 10.3.9a.9 |  | REL-6 |
| CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| NAS Synchronization Indicator | OP |  | NAS Synchronization indicator 10.3.4.12 |  |  |
| Re-establishment timer | MP |  | Re-establishment timer 10.3.3.30 |  |  |
| NOTE 1: Only the "MBMS Service ID" part of the MBMS service identity is signalled in this IE. The UE should consider that the "PLMN identity" part of the MBMS service identity equals the PLMN identity of an MBMS service with the same "MBMS Service ID" stored in the variable ACTIVATED_MBMS_SERVICES. If there is no such MBMS service or more than one such MBMS service, the UE behaviour is unspecified. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Message | This IE is optionally present in the RADIO BEARER SETUP and the SRNS RELOCATION INFO messages and not needed otherwise. |

#### 10.3.4.9	RAB info Post

This IE contains information used to uniquely identify a radio access bearer.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAB identity | MP |  | RAB identity 10.3.1.14 |  |
| CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |
| NAS Synchronization Indicator | OP |  | NAS Synchronization indicator 10.3.4.12 |  |

#### 10.3.4.9a	RAB information for MBMS ptp bearers

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| RB identity | MP |  | RB identity 10.3.4.16 |  | REL-7 |
| MBMS Service Identity | MP |  | MBMS Service ID 10.3.9a.8a | Indicates that this radio bearer is used for a different MBMS service. | REL-7 |
| MBMS Session Identity | OP |  | MBMS Session identity 10.3.9a.9 | Indicates that this radio bearer is used for a different session of the MBMS service. | REL-7 |

#### 10.3.4.10	RAB information for setup

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| RAB info | MP |  | RAB info 10.3.4.8 |  |  |
| CS-HSPA information | CV-CS |  |  |  | REL-8 |
| >UL AMR rate | MD |  | Enumerated (t0, t1, t2, t3, t4, t5, t6, t7, t8) | Indicates the bit rate as defined in [62] and [63]. In case NAS Synchronization Indicator indicates AMR, default value is “t7”. In case NAS Synchronization Indicator indicates AMR-WB, default value is “t8“. 7 spare values are needed. | REL-8 |
| >Max CS delay | MP |  | Integer (20..200 by step of 10) | Indicates the maximum possible delay for CS voice frames Unit is ms | REL-8 |
| RAB info to replace | CV-SRVCC |  | RAB info to replace 10.3.4.11a |  | REL-8 |
| RB information to setup list | MP | 1 to <maxRBperRAB> |  |  |  |
| >RB information to setup | MP |  | RB information to setup 10.3.4.20 |  |  |

| Condition | Explanation |
| --- | --- |
| CS | This IE is mandatory if the IE “CN domain identity” has the value “CS domain” and the RAB is mapped on HS-DSCH/E-DCH. It is not needed otherwise. |
| SRVCC | This IE is mandatory if the RAB is established as part of an SR-VCC procedure and the message is not HANDOVER TO UTRAN COMMAND message and not needed otherwise. |

#### 10.3.4.11	RAB information to reconfigure

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| RAB identity | MP |  | RAB Identity 10.3.1.14 |  |  |
| CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| NAS synchronization indicator | MP |  | NAS Synchronization info 10.3.4.12 | Note 1 |  |
| CS-HSPA information | CV-CS |  |  |  | REL-8 |
| >UL AMR rate | MD |  | Enumerated(t0, t1, t2, t3, t4, t5, t6, t7, t8) | Indicates the bit rate as defined in [62] and [63]. In case NAS Synchronization Indicator indicates AMR, default value is “t7”. In case NAS Synchronization Indicator indicates AMR-WB, default value is “t8“. 7 spare values are needed. | REL-8 |
| >Max CS delay | MP |  | Integer(20..200 by step of 10) | Indicates the maximum possible delay for CS voice frames Unit is ms | REL-8 |

| Condition | Explanation |
| --- | --- |
| CS | This IE is mandatory if the IE “CN domain identity” has the value “CS domain” and the RAB is mapped on HS-DSCH/E-DCH. It is not needed otherwise. |

NOTE 1:	This IE is only relevant for the CS domain.

#### 10.3.4.11a	RAB info to replace

This IE contains information to identify a radio access bearer to be replaced with a new radio access bearer as part of SR-VCC procedures. This IE is not included in case SR-VCC from LTE to UMTS.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| RAB identity | MP |  | RAB identity 10.3.1.14 |  | REL-8 |
| CN domain identity | MP |  | CN domain identity 10.3.1.1 |  | REL-8 |

#### 10.3.4.12	NAS Synchronization indicator

A container for non-access stratum information to be transferred transparently through UTRAN.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NAS Synchronization indicator | MP |  | Bit string(4) | The first/leftmost bit of the bit string contains the most significant bit of the NAS Synchronization indicator. |

#### 10.3.4.13	RB activation time info

This IE contains the time, in terms of RLC sequence numbers, when a certain configuration shall be activated, for a number of radio bearers.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Radio bearer activation time | MP | 1 to <maxRB> |  |  |
| >RB identity | MP |  | RB identity 10.3.4.16 |  |
| >RLC sequence number | MP |  | Integer (0.. 4095) | RLC SN [16] . Used for radio bearers mapped on RLC AM and UM |

#### 10.3.4.14	RB COUNT-C MSB information

The MSB of the COUNT-C values of the radio bearer.

| Information Element/Group name | Needed | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RB identity | MP |  | RB identity 10.3.4.16 |  |
| COUNT-C-MSB-uplink | MP |  | Integer (0.. 2^25-1) | 25 MSBs from COUNT-C associated to this RB |
| COUNT-C-MSB-downlink | MP |  | Integer (0.. 2^25-1) | 25 MSBs from COUNT-C associated to this RB |

#### 10.3.4.15	RB COUNT-C information

The COUNT-C values of the radio bearer.

| Information Element/Group name | Needed | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RB identity | MP |  | RB identity 10.3.4.16 |  |
| COUNT-C-uplink | MP |  | Integer (0.. 2^32-1) |  |
| COUNT-C-downlink | MP |  | Integer (0.. 2^32-1) |  |

#### 10.3.4.16	RB identity

An identification number for the radio bearer affected by a certain message.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RB identity | MP |  | Integer(1..32) | Values 1-4 shall only be used for signalling radio bearers. The IE value minus one shall be used as BEARER in the ciphering algorithm. |

#### 10.3.4.17	RB information to be affected

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RB identity | MP |  | RB identity 10.3.4.16 |  |
| RB mapping info | MP |  | RB mapping info 10.3.4.21 |  |

#### 10.3.4.18	RB information to reconfigure

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RB identity | MP |  | RB identity 10.3.4.16 |  |
| PDCP info | OP |  | PDCP info 10.3.4.2 |  |
| PDCP SN info | OP |  | PDCP SN info 10.3.4.3 | PDCP sequence number info from the network. Present only in case of lossless SRNS relocation. |
| RLC info | OP |  | RLC info 10.3.4.23 |  |
| RB mapping info | OP |  | RB mapping info 10.3.4.21 |  |
| RB stop/continue | OP |  | Enumerated(stop, continue) |  |

#### 10.3.4.19	RB information to release

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RB identity | MP |  | RB identity 10.3.4.16 |  |

#### 10.3.4.20	RB information to setup

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RB identity | MP |  | RB identity 10.3.4.16 |  |
| PDCP info | OP |  | PDCP info 10.3.4.2 |  |
| CHOICE RLC info type | MP |  |  |  |
| >RLC info |  |  | RLC info 10.3.4.23 |  |
| >Same as RB |  |  | RB identity 10.3.4.16 | Identity of RB with exactly the same RLC info IE values |
| RB mapping info | MP |  | RB mapping info 10.3.4.21 |  |

NOTE:		This information element is included within IE "Predefined RB configuration".

#### 10.3.4.21	RB mapping info

A multiplexing option for each possible transport channel MAC-d flow or E-DCH MAC-d flow this RB can be multiplexed on.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information for each multiplexing option | MP | 1 to <maxRBMuxOptions> |  |  |  |  |
| >RLC logical channel mapping indicator | CV-UL-RLCLogicalChannels |  | Boolean | TRUE indicates that the first logical channel shall be used for data PDUs and the second logical channel shall be used for control PDUs. FALSE indicates that control and data PDUs can be sent on either of the two logical channels. This parameter is not used in this release and shall be set to TRUE. |  |  |
| >Number of uplink RLC logical channels | CV-UL-RLC info | 1 to MaxLoCHperRLC |  | 1 or 2 logical channels per RLC entity or radio bearer RLC [16] |  |  |
| >>CHOICE Uplink transport channel type |  |  |  |  | REL-6 |  |
| >>>DCH, RACH, USCH |  |  |  |  | REL-6 |  |
| >>>>Uplink transport channel type | MP |  | Enumerated(DCH,RACH,USCH | USCH is TDD only |  |  |
| >>>>ULTransport channel identity | CV-UL-DCH/USCH |  | Transport channel identity 10.3.5.18 | This is the ID of a DCH or USCH (TDD only) that this RB could be mapped onto. |  |  |
| >>>>Logical channel identity | OP |  | Integer(1..15) | This parameter is used to distinguish logical channels multiplexed by MAC on a transport channel. |  |  |
| >>>>CHOICE RLC size list | MP |  |  | The RLC sizes that are allowed for this logical channel. |  |  |
| >>>>>All |  |  | Null | All RLC sizes listed in the Transport Format Set. 10.3.5.23 |  |  |
| >>>>>Configured |  |  | Null | The RLC sizes configured for this logical channel in the Transport Format Set. 10.3.5.23 if present in this message or in the previously stored configuration otherwise |  |  |
| >>>>>Explicit List |  | 1 to <maxTF> |  | Lists the RLC sizes that are valid for the logical channel. |  |  |
| >>>>>>RLC size index | MP |  | Integer(1..maxTF) | The integer number is a reference to the RLC size which arrived at that position in the Transport Format Set 10.3.5.23 |  |  |
| >>>E-DCH |  |  |  | Note 4 | REL-6 |  |
| >>>>Logical channel identity | MP |  | Integer(1..15) | This parameter is used to distinguish logical channels multiplexed by MAC on a transport channel | REL-6 |  |
| >>>>E-DCH MAC-d flow identity | MP |  | E-DCH MAC-d flow identity 10.3.5.7e |  | REL-6 |  |
| >>>>CHOICE RLC PDU size | MP |  |  |  | REL-8 |  |
| >>>>>Fixed size |  |  |  |  | REL-8 |  |
| >>>>>>DDI | MP |  | Integer (0..62) | If more than 1 UL RLC PDU size is configured for this RB, the different sizes will use subsequent DDI values starting from this DDI value. Value “0x3F” is reserved Note 6 | REL-6 |  |
| >>>>>>RLC PDU size list | MP | 1 to <maxRLCPDUsizePerLogChan> |  |  | REL-6 |  |
| >>>>>>>RLC PDU size | MP |  | Integer(16..5000 by step of 8) | Unit is bits | REL-6 |  |
| >>>>>Flexible size |  |  |  |  | REL-8 |  |
| >>>>>>Length indicator size | CV-UL-RLC AM mode |  | Enumerated (7-bit, 15-bit) |  | REL-8 |  |
| >>>>>>Minimum UL RLC PDU size | MP |  | Integer (16..12040 by step of 8) | Unit is bits | REL-8 |  |
| >>>>>>Largest UL RLC PDU size | MP |  | Integer (16..12040 by step of 8) | Unit is bits | REL-8 |  |
| >>>>Include in Scheduling Info | MP |  | Boolean | Indicates whether or not this logical channel is to be considered when performing scheduling info reporting, as per [15] | REL-6 |  |
| >>MAC logical channel priority | MP |  | Integer(1..8) | This is priority between a user's different RBs (or logical channels). [15] |  |  |
| >Downlink RLC logical channel info | CV-DL-RLC info |  |  |  |  |  |
| >>Number of downlink RLC logical channels | MD | 1 to MaxLoCHperRLC |  | 1 or 2 logical channels per RLC entity or radio bearer RLC [16] Default value is that parameter values for DL are exactly the same as for corresponding UL logical channel. In case two multiplexing options are specified for the UL, the first options shall be used as default for the DL. As regards to the IE "Channel type", rule is specified in 8.6.4.8. |  |  |
| >>>Downlink transport channel type | MP |  | Enumerated(DCH,FACH,DSCH,DCH+DSCH | Note 3 |  |  |
|  |  |  | , HS-DSCH, DCH + HS-DSCH) | Note 4 | REL-5 |  |
| >>>DL DCH Transport channel identity | CV-DL-DCH |  | Transport channel identity 10.3.5.18 |  |  |  |
| >>>DL DSCH Transport channel identity | CV-DL-DSCH |  | Transport channel identity 10.3.5.18 |  |  |  |
| >>>CHOICE DL MAC header type | CV-DL-HS-DSCH |  |  | Depending on the MAC-hs/ehs type of header selected for HS-DSCH | REL-7 |  |
| >>>>MAC-hs |  |  |  |  | REL-7 |  |
| >>>>>DL HS-DSCH MAC-d flow identity | MP |  | MAC-d flow identity 10.3.5.7c |  |  | REL-5 |
| >>>>MAC-ehs |  |  |  |  |  | REL-7 |
| >>>>>DL HS-DSCH MAC-ehs Queue Id | MP |  | MAC-ehs Queue Id 10.3.5.7f | Note 5 |  | REL-7 |
| >>>Logical channel identity | OP |  | Integer(1..15) | 16 is reserved Note 5 |  |  |
| Note 3: The IE "Downlink transport channel type" values " DSCH” and “DCH+DSCH " should not be used for FDD. If received the UE behaviour is unspecified. |  |  |  |  |  |  |
| Note 4: If included in System Information Block Type 16, the values ‘HS-DSCH’ and ‘DCH + HS-DSCH’ do not apply for the IE "Downlink transport channel type". Furthermore, if included in System Information Block Type 16, the value "E-DCH” for the IE "Uplink transport channel type" does not apply. |  |  |  |  |  |  |
| Note 5: If the IE "Downlink transport channel type" is "HS-DSCH" or "DCH + HS-DSCH" and the DL MAC header type is "MAC-ehs", the IE "Logical channel identity" conveys the value to be used in the "LCH-ID" field of the MAC-ehs header [15], associating the logical channel with the MAC-ehs reordering queue identified by the IE "DL HS-DSCH MAC-ehs Queue Id". |  |  |  |  |  |  |
| Note 6: If the radio bearer has a multiplexing option configured with the UL MAC header type of "MAC-i/is" then this value is ignored. |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| UL-RLC info | If "CHOICE Uplink RLC mode" in the IE "RLC info" that applies for that RB (i.e. either the one stored or received in the same message for the RB for which the "RB mapping info" was received, or the one stored or received in the same message for the RB pointed at in the IE "Same as RB" in the IE "RB information to setup" stored or received in the same message) is present this IE is mandatory present. Otherwise the IE is not needed. |
| DL-RLC info | If "CHOICE Downlink RLC mode" in the IE "RLC info" that applies for that RB (i.e. either the one stored or received in the same message for the RB for which the "RB mapping info" was received, or the one stored or received in the same message for the RB pointed at in the IE "Same as RB" in the IE "RB information to setup" stored or received in the same message) is present this IE is mandatory present. Otherwise the IE is not needed. |
| UL-RLCLogicalChannels | If "Number of uplink RLC logical channels" in IE "RB mapping info" is 2, then this IE is mandatory present. Otherwise this IE is not needed. |
| UL-DCH/USCH | If IE "Uplink transport channel type" is equal to "DCH" or "USCH" (TDD only) this IE is mandatory present. Otherwise the IE is not needed. |
| DL-DCH | If IE "Downlink transport channel type" is equal to "DCH", "DCH+DSCH" or “DCH + HS-DSCH” this IE is mandatory present. Otherwise the IE is not needed. |
| DL-DSCH | If IE "Downlink transport channel type" is equal to "DSCH" or "DCH+DSCH" this IE is mandatory present. Otherwise the IE is not needed. |
| DL-HS-DSCH | If IE "Downlink transport channel type" is equal to "HS-DSCH" or “DCH + HS-DSCH” this IE is mandatory present. Otherwise the IE is not needed. |
| UL-RLC AM mode | If "CHOICE Uplink RLC mode" in the IE "RLC info" that applies for that RB (i.e. either the one stored or received in the same message for the RB for which the "RB mapping info" was received, or the one stored or received in the same message for the RB pointed at in the IE "Same as RB" in the IE "RB information to setup" stored or received in the same message) is “AM RLC “ this IE is mandatory present. Otherwise the IE is not needed. |

#### 10.3.4.22	RB with PDCP information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| RB identity | MP |  | RB identity 10.3.4.16 |  |  |
| PDCP SN info | MP |  | PDCP SN info 10.3.4.3 | PDCP sequence number info from the sender of the message for lossless SRNS relocation. |  |
|  |  |  |  | PDCP sequence number info from the sender of the message for lossless SRNS relocation or for lossless DL RLC PDU size change. | REL-5 |

#### 10.3.4.23	RLC info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Uplink RLC mode | OP |  |  | Indicates if Acknowledged, Unacknowledged or Transparent mode RLC shall be used. |  |
| >AM RLC |  |  |  |  |  |
| >>Transmission RLC discard | MP |  | Transmission RLC discard 10.3.4.25 |  |  |
| >>Transmission window size | MP |  | Integer(1,8,16,32,64,128,256,512,768,1024,1536,2047,2560,3072,3584,4095) | Maximum number of RLC PUs sent without getting them acknowledged. This parameter is needed if acknowledged mode is used. UE shall also assume that the UTRAN receiver window is equal to this value. |  |
| >>Timer_RST | MP |  | Integer(50, 100, 150, 200, 250, 300, 350, 400, 450, 500, 550, 600, 700, 800, 900, 1000) | Elapsed time in milliseconds. It is used to trigger the retransmission of RESET PDU. |  |
| >>Max_RST | MP |  | Integer(1, 4, 6, 8, 12 16, 24, 32) | Defined in [16]. If other values than 1 are used for SRB2 the UE behaviour is unspecified. |  |
| >>Polling info | OP |  | Polling info 10.3.4.4 |  |  |
| >UM RLC |  |  |  |  |  |
| >>Transmission RLC discard | CV-CS-HSPA |  | Transmission RLC discard 10.3.4.25 | NOTE 3 |  |
| >TM RLC |  |  |  |  |  |
| >>Transmission RLC discard | OP |  | Transmission RLC discard 10.3.4.25 |  |  |
| >>Segmentation indication | MP |  | Boolean | TRUE indicates that segmentation is performed. |  |
| CHOICE Downlink RLC mode | OP |  |  | Indicates if Acknowledged, Unacknowledged or Transparent mode RLC shall be used |  |
| >AM RLC |  |  |  |  |  |
| >>CHOICE DL RLC PDU size | MP |  |  |  | REL-7 |
| >>>Fixed size |  |  |  |  | REL-7 |
| >>>>DL RLC PDU size | MP |  | Integer(16..5000 by step of 8) | Unit is bits | REL-5 |
| >>>Flexible size |  |  |  | NOTE 1 | REL-7 |
| >>>>Length indicator size | MP |  | Enumerated (7-bit, 15-bit) |  | REL-7 |
| >>In-sequence delivery | MP |  | Boolean | TRUE indicates that RLC shall preserve the order of higher layer PDUs when these are delivered. FALSE indicates that receiving RLC entity could allow SDUs to be delivered to the higher layer in different order than submitted to RLC sublayer at the transmitting side. |  |
| >>Receiving window size | MP |  | Integer(1,8,16,32,64,128,256,512,768,1024,1536,2047,2560,3072,3584,4095) | Maximum number of RLC PUs allowed to be received. This parameter is needed if acknowledged mode is used. UE shall also assume that the UTRAN transmitter window is equal to this value |  |
| >>Downlink RLC status Info | MP |  | Downlink RLC status info 10.3.4.1 |  |  |
| >UM RLC |  |  |  |  |  |
| >>DL UM RLC LI size | MP |  | Integer(7, 15) | Size in bits to use for the downlink RLC UM LI. | REL-5 |
| >>DL Reception Window Size | CV-Not-SIB16o |  | Integer(32, 48, 64, 80, 96, 112) |  | REL-6 |
| >TM RLC |  |  |  |  |  |
| >>Segmentation indication | MP |  | Boolean | TRUE indicates that segmentation is performed. |  |
| One sided RLC re-establishment | MP |  | Boolean | TRUE indicates that only one side of the AM RLC entity is re-established. | REL-5 |
| Alternative E-bit interpretation | CV-Not-SIB16o |  | Enumerated (TRUE) | The absence of this IE implies: "normal E-bit interpretation". | REL-6 |
| Use special value of HE field | CV-Not-SIB16o |  | Enumerated (TRUE) | The absence of this IE implies: “do not use special value of the HE field in the uplink RLC entity”. | REL-7 |
| NOTE 1: If included in SIB type 16, the "Flexible size" does not apply for downlink RLC PDU size (AM RLC). |  |  |  |  |  |
| NOTE 2: Void. |  |  |  |  |  |
| NOTE 3: If this IE is absent and the radio bearer is connected to a CS domain radio access bearer, the UE behaviour is unspecified. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Not-SIB16o | If included in System Information Block Type 16, this IE is not needed. Otherwise the IE is optional |
| CS-HSPA | This IE is mandatory present If the radio bearer is connected to a CS domain radio access bearer, Otherwise the IE is optional. |

NOTE:		This information element is included within IE "Predefined RB configuration".

#### 10.3.4.23a	RLC info MBMS

The IE RLC info MBMS is used for point-to-multipoint radio bearers, featuring only the downlink RLC UMD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| DL UM RLC LI size | MP |  | Integer (7, 15) | Size in bits to use for the downlink RLC UM LI. | REL-6 |
| DL Duplication Avoidance and Reordering info | CV-MTCH |  | UM Duplication Avoidance and Reordering info 10.3.4.26 |  | REL-6 |
| DL Out of sequence delivery info | CV-MCCH |  | UM Out of sequence delivery info 10.3.4.27 |  | REL-6 |

| Condition | Explanation |
| --- | --- |
| MTCH | If the IE concerns MTCH (see Note 1), this IE is optional. Otherwise the IE is not needed. |
| MCCH | If the IE concerns MCCH, this IE is optional. Otherwise the IE is not needed. |

NOTE 1:	The UE behaviour is unspecified if this IE is received with regard to an MTCH configured for MBSFN operation. A network should not send this IE with regard to an MTCH configured for MBSFN operation.

#### 10.3.4.24	Signalling RB information to setup

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RB identity | MD |  | RB identity 10.3.4.16 | Default value is specified in subclause 8.6.4.1 |
| CHOICE RLC info type | MP |  |  |  |
| >RLC info |  |  | RLC info 10.3.4.23 |  |
| >Same as RB |  |  | RB identity 10.3.4.16 | Identity of RB with exactly the same RLC info IE values |
| RB mapping info | MP |  | RB mapping info 10.3.4.21 |  |

NOTE:		This information element is included within IE "Predefined RB configuration".

#### 10.3.4.24a	SR-VCC Info

This IE contains information that allows the calculation of the CK and IK for the CS domain, due to a SR-VCC procedure.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| NONCE | OP |  | Bit string (128) |  | REL-8 |

#### 10.3.4.25	Transmission RLC Discard

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SDU Discard Mode | MP |  |  | Different modes for discharge the RLC buffer on the transmitter side; "Timer based with explicit signalling", "Timer based without explicit signalling", "Discard after Max_DAT retransmissions" or "No_discard". For unacknowledged mode and transparent mode, only Timer based without explicit signalling is applicable. If "No_discard" is used, reset procedure shall be done after Max_DAT retransmissions |
| >Timer based explicit |  |  |  |  |
| >>Timer_MRW | MP |  | Integer(50,60, 70, 80, 90, 100, 120, 140, 160, 180, 200, 300, 400, 500, 700, 900) | Elapsed time in milliseconds. It is used to trigger the retransmission of a STATUS PDU containing an MRW SUFI field |
| >>Timer_discard | MP |  | Integer(100, 250, 500, 750, 1000, 1250, 1500, 1750, 2000, 2500, 3000, 3500, 4000, 4500, 5000, 7500) | Elapsed time in milliseconds before a SDU is discarded. |
| >>MaxMRW | MP |  | Integer(1, 4, 6, 8, 12 16, 24, 32) | Defined in [16] |
| >Timer based no explicit |  |  |  |  |
| >>Timer_discard | MP |  | Integer(10,20,30,40,50,60,70,80,90,100) | Elapsed time in milliseconds before a SDU is discarded. |
| >Max DAT retransmissions |  |  |  |  |
| >>Max_DAT | MP |  | Integer(1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 15, 20, 25, 30, 35, 40) | Defined in [16] |
| >>Timer_MRW | MP |  | Integer(50, 60, 70, 80, 90, 100, 120, 140, 160, 180, 200, 300, 400, 500, 700, 900) | Elapsed time in milliseconds. It is used to trigger the retransmission of a STATUS PDU containing an MRW SUFI field |
| >>MaxMRW | MP |  | Integer(1, 4, 6, 8, 12 16, 24, 32) | Defined in [16] |
| >No discard |  |  |  |  |
| >>Max_DAT | MP |  | Integer(1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 15, 20, 25, 30, 35, 40) | Defined in [16] |

| CHOICE SDU Discard Mode | Condition under which the given SDU Discard Mode is chosen |
| --- | --- |
| Timer based explicit | If the modes for discharge of the RLC buffer on the transmitter side is "Timer based with explicit signalling" |
| Timer based no explicit | If the modes for discharge of the RLC buffer on the transmitter side is "Timer based without explicit signalling" For unacknowledged mode, only Timer based without explicit signalling is applicable. |
| Max DAT retransmissions | If the modes for discharge of the RLC buffer on the transmitter side is "Discard after Max_DAT retransmissions" |
| No discard | If the modes for discharge the of RLC buffer on the transmitter side is "Reset procedure shall be done after Max_DAT retransmissions" |

#### 10.3.4.26	UM Duplication Avoidance and Reordering info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Timer_DAR | MP |  | Integer(40, 80, 120, 160, 240, 320, 480, 640, 960, 1280, 1920, 2560, 3840, 5120) | Timer (in milliseconds) when PDUs are released to the upper layers even though there are outstanding PDUs with lower RLC SN values. | REL-6 |
| Window size DAR | MP |  | Integer(4, 8, 16, 32, 40, 48, 56, 64) |  | REL-6 |

#### 10.3.4.27	UM Out of sequence delivery info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Timer_OSD | CV- notMCCH |  | Integer (40, 80, 120, 160, 240, 320, 480, 640, 960, 1280, 1920, 2560, 3840, 5120) |  | REL-6 |
| Window size OSD | MP |  | Integer(8, 16, 32,40, 48, 56, 64) |  | REL-6 |

NOTE:		This timer used to flush the buffer is configured at RRC level and indicated via a local primitive.

| Condition | Explanation |
| --- | --- |
| notMCCH | If this concerns a logical channel other than MCCH then this IE is mandatory otherwise it is not needed. In the latter case Timer_OSD takes the value of the IE Modification Period as indicated within the IE MCCH configuration information |

### 10.3.5	Transport CH Information elements

#### 10.3.5.1	Added or Reconfigured DL TrCH information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Downlink transport channel type | MP |  | Enumerated(DCH,DSCH | Note 2 |  |
|  |  |  | ,HS-DSCH) | Note 3 | REL-5 |
| DL Transport channel identity | MP |  | Transport channel identity 10.3.5.18 |  |  |
|  | CV-not HS-DSCH |  |  |  | REL-5 |
| CHOICE DL parameters |  |  |  |  |  |
| >Explicit |  |  |  |  |  |
| >>TFS | MP |  | Transport Format Set 10.3.5.23 |  |  |
| >SameAsUL |  |  |  |  |  |
| >>Uplink transport channel type | MP |  | Enumerated(DCH,USCH) | USCH is TDD only |  |
| >>UL TrCH identity | MP |  | Transport channel identity 10.3.5.18 | Same TFS applies as specified for indicated UL TrCH |  |
| >HS-DSCH |  |  |  |  | REL-5 |
| >>HARQ Info | OP |  | HARQ info 10.3.5.7a | For 1.28 Mcps TDD, if HARQ Information is included in the IE DL MultiCarrier Information, this IE shall not be present. | REL-5 |
| >>CHOICE DL MAC header type | OP |  |  |  | REL-7 |
| >>>MAC-hs |  |  |  |  | REL-7 |
| >>>>Added or reconfigured MAC-d flow | MP |  | Added or reconfigured MAC-d flow 10.3.5.1a |  | REL-5 |
| >>>MAC-ehs |  |  |  |  | REL-7 |
| >>>>Added or reconfigured MAC-ehs reordering queue | MP |  | Added or reconfigured MAC-ehs reordering queue 10.3.5.1c |  | REL-7 |
| DCH quality target | OP |  | Quality target 10.3.5.10 |  |  |
| Note 1: Void |  |  |  |  |  |
| Note 2: The IE "Downlink transport channel type" value "DSCH" should not be used for FDD. If received the UE behaviour is unspecified. |  |  |  |  |  |
| Note 3: If included in SIB type 16, the value ‘HS-DSCH’ does not apply for the IE "Downlink transport channel type". |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| NotHS-DSCH | If the downlink transport channel type is DCH or DSCH then this IE is mandatory otherwise it is not needed. |

#### 10.3.5.1a	Added or reconfigured MAC-d flow

This IE is used in relation to the MAC-d flows mapped to the HS-DSCH transport channel.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MAC-hs queue to add or reconfigure list | OP | <1 to maxQueueID> |  |  | REL-5 |
| >MAC-hs queue Id | MP |  | Integer(0..7) | The MAC-hs queue ID is unique across all MAC-d flows. | REL-5 |
| >MAC-d Flow Identity | MP |  | MAC-d Flow Identity 10.3.5.7c |  | REL-5 |
| >T1 | MP |  | Integer(10, 20, 30, 40, 50, 60, 70, 80, 90, 100, 120, 140, 160, 200, 300, 400) | Timer (in milliseconds) when PDUs are released to the upper layers even though there are outstanding PDUs with lower TSN values. | REL-5 |
| >MAC-hs window size | MP |  | Integer(4, 6, 8, 12, 16, 24, 32) |  | REL-5 |
|  |  |  | Integer(32, 64, 96, 128, 160, 192, 256) | The set of values is used for Multi-carrier case in 1.28 Mcps TDD mode for 9bits TSN option. | REL-7 |
| >MAC-d PDU size Info | OP | <1 to max MACdPDUsizes> |  | Mapping of the different MAC-d PDU sizes configured for the HS-DSCH to the MAC-d PDU size index in the MAC-hs header. | REL-5 |
| >>MAC-d PDU size | MP |  | Integer (1..5000) |  | REL-5 |
| >>MAC-d PDU size index | MP |  | Integer(0..7) |  | REL-5 |
| MAC-hs queue to delete list | OP | <1 to maxQueueID> |  |  | REL-5 |
| >MAC-hs queue Id | MP |  | Integer(0..7) | The MAC-hs queue ID is unique across all MAC-d flows. | REL-5 |

#### 10.3.5.1b	Added or reconfigured E-DCH MAC-d flow

This IE is used in relation to MAC-d flows mapped to the E-DCH transport channel.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-DCH MAC-d flow identity | MP |  | E-DCH MAC-d flow identity 10.3.5.7e |  | REL-6 |
| E-DCH MAC-d flow power offset | OP |  | Integer(0..6) | Only allowed to be absent when already defined for this E-DCH MAC-d flow, unit is dB | REL-6 |
| E-DCH MAC-d flow maximum number of retransmissions | OP |  | Integer (0..15) | Only allowed to be absent when already defined for this E-DCH MAC-d flow | REL-6 |
| E-DCH MAC-d flow retransmission timer | CV-Tdd128 |  | Enumerated (10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 65, 70, 75, 80, 85, 90, 95, 100, 110, 120, 140, 160, 200, 240, 280, 320, 400, 480, 560) | Unit: ms | REL-7 |
| E-DCH MAC-d flow multiplexing list | OP |  | Bitstring (maxE-DCHMACdFlow) | Indicates, if this is the first MAC-d flow for which PDUs are placed in the MAC-e or MAC-i PDU, the other MAC-d flows from which MAC-d PDUs are allowed to be included in the same MAC-e or MAC-i PDU. Bit 0 is for MAC-d flow 0, Bit 1 is for MAC-d flow 1, … Value '1' for a bit means multiplexing is allowed. Bit 0 is the first/leftmost bit of the bit string. NOTE: The bit that corresponds to the MAC-d flow itself is ignored. | REL-6 |
| CHOICE transmission grant type | OP |  |  | Only allowed to be absent when already defined for this E-DCH MAC-d flow | REL-6 |
| >Non-scheduled transmission grant info |  |  |  |  | REL-6 |
| >>CHOICE mode | MP |  |  |  | REL-7 |
| >>>FDD |  |  |  |  | REL-7 |
| >>>>Max MAC-e PDU contents size | MP |  | Integer (1..19982) |  | REL-6 |
| >>>>2ms non-scheduled transmission grant HARQ process allocation | MD |  | Bitstring (8) | MAC-d PDUs for this MAC-d flow are only allowed to be transmitted in those processes for which the bit is set to “1”. Bit 0 corresponds to HARQ process 0, bit 1 corresponds to HARQ process 1,… Default value is: transmission in all HARQ processes is allowed. Bit 0 is the first/leftmost bit of the bit string. | REL-6 |
| >>>TDD |  |  | NULL | No data | REL-7 |
| >Scheduled transmission grant info |  |  | NULL |  | REL-6 |

| Condition | Explanation |
| --- | --- |
| Tdd128 | This IE is mandatory present for 1.28Mcps TDD, otherwise it is not needed. |

#### 10.3.5.1c	Added or reconfigured MAC-ehs reordering queue

This IE is used in relation to the MAC-ehs reordering queues mapped to the HS-DSCH transport channel.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MAC-ehs queue to add or reconfigure list | OP | <1 to maxQueueID> |  |  | REL-7 |
| >MAC-ehs queue Id | MP |  | MAC-ehs Queue Id 10.3.5.7f |  | REL-7 |
| >T1 | MP |  | Integer (10, 20, 30, 40, 50, 60, 70, 80, 90, 100, 120, 140, 160, 200, 300, 400) | Timer (in milliseconds) when PDUs are released to the upper layers even though there are outstanding PDUs with lower TSN values. | REL-7 |
| >Treset | MD |  | Integer (1, 2, 3, 4) | Timer in multiples of T1 values (milliseconds). Used when MAC-ehs reordering queue is reset in CELL_FACH and CELL_PCH (FDD only). Default value is 2 | REL-7 |
| >MAC-ehs window size | MP |  | Enumerated (4, 6, 8, 12, 16, 24, 32, | Six spare values are needed. | REL-7 |
|  |  |  | 64, 128, |  | REL-9 |
|  |  |  | 256) |  | REL-11 |
| >MAC-ehs window size extension | OP |  | Enumerated(96,160,196,256) | For 1,28Mcps TDD only, The set of values is used for Multi-carrier case for 9bits TSN option. If this IE is present, UE should ignore the IE "MAC-ehs window size" | REL-9 |

#### 10.3.5.2	Added or Reconfigured UL TrCH information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Uplink transport channel type | MP |  | Enumerated(DCH,USCH | USCH is TDD only |  |
|  |  |  | ,E-DCH) | Note 1 | REL-6 |
| UL Transport channel identity | MP |  | Transport channel identity 10.3.5.18 |  |  |
|  | CV-NotE-DCH |  |  |  | REL-6 |
| CHOICE UL parameters |  |  |  |  | REL-6 |
| >DCH,USCH |  |  |  |  | REL-6 |
| >>TFS | MP |  | Transport Format Set 10.3.5.23 |  |  |
| >E-DCH |  |  |  |  | REL-6 |
| >>UL MAC header type | OP |  | Enumerated (MAC-i/is) | If this IE is present, MAC-i/is header type [15] is used, else MAC-e/es header type [15] is used. | REL-8 |
| >>CHOICE mode | MP |  |  |  | REL-7 |
| >>>FDD |  |  |  |  | REL-7 |
| >>>>E-DCH Transmission Time Interval | MP |  | Enumerated(2,10) | Unit is ms. | REL-6 |
| >>>TDD |  |  |  | No data | REL-7 |
| >>HARQ info for E-DCH | MP |  | HARQ Info for E-DCH 10.3.5.7d |  | REL-6 |
| >>Added or reconfigured E-DCH MAC-d flow list | OP | <1 to maxE-DCHMACdFlow> |  |  | REL-6 |
| >>>Added or reconfigured E-DCH MAC-d flow | MP |  | Added or reconfigured E-DCH MAC-d flow 10.3.5.1b |  | REL-6 |
| Note 1: If included in System Information Block Type 16, the values ‘E-DCH’ does not apply for the IE "Uplink transport channel type". |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| NotE-DCH | If the uplink transport channel type is DCH or USCH then this IE is mandatory otherwise it is not needed. |

NOTE:		This information element is included within IE "Predefined RB configuration".

#### 10.3.5.2a	Additional Dynamic Transport Format Information for CCCH

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| RLC Size | MP |  | Integer (48..4968) | Unit is bits For FDD, values are restricted to: Integer (48..296 by step of 8, 312..1320 by step of 16, 1384..4968 by step of 64) | Rel-6 |
| Number of Transport blocks | MP |  | Integer (1) | NOTE 1 | Rel-6 |

NOTE 1:	The "Integer (1)" type does not result in bits in the transfer syntax and is not needed in the ASN.1.

#### 10.3.5.2b	Additional RACH TFCS for CCCH

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Power offset Information | MP |  | Power Offset Information 10.3.5.8 | The actual TFCS is specified in 8.6.5.12a | Rel-6 |

#### 10.3.5.3	Void

#### 10.3.5.3a	Common MAC-ehs reordering queue list

NOTE:	For FDD and 1.28 Mcps TDD only.

This IE is defines common MAC-ehs priority queue parameters, which can be used for CCCH and SRB1 reception.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MAC-ehs queue to configure list | MP | <1 to maxCommonQueueID> |  |  | REL-7 |
| >MAC-ehs queue Id | MP |  | MAC-ehs Queue Id 10.3.5.7f |  | REL-7 |
| >T1 | MP |  | Integer (10, 20, 30, 40, 50, 60, 70, 80, 90, 100, 120, 140, 160, 200, 300, 400) | Timer (in milliseconds) when PDUs are released to the upper layers even though there are outstanding PDUs with lower TSN values. | REL-7 |
| >Treset | MD |  | Integer (1, 2, 3, 4) | Timer in multiples of T1 values (milliseconds). Used when MAC-ehs reordering queue is reset in CELL_FACH and CELL_PCH (FDD and 1.28 Mcps TDD only). Default value is 2 | REL-7 |
| >MAC-ehs window size | MP |  | Integer (4, 6, 8, 12, 16, 24, 32) |  | REL-7 |

#### 10.3.5.3b	Common E-DCH MAC-d flows

This IE is used in relation to MAC-d and MAC-c (for FDD) flows mapped to the E-DCH transport channels.

NOTE:	Only for FDD and 1.28 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Configured E-DCH MAC-d flow list | MP | 1 to <maxE-DCHMACdFlow> |  |  | REL-8 |
| >E-DCH MAC-d flow identity | MP |  | E-DCH MAC-d Flow Identity 10.3.5.7e | NOTE 1 | REL-8 |
| >E-DCH MAC-d flow power offset | MP |  | Integer(0..6) |  | REL-8 |
| >E-DCH MAC-d flow maximum number of retransmissions | MP |  | Integer (0..15) |  | REL-8 |
| >E-DCH MAC-d flow multiplexing list | CV-NotCCCHFDD |  | Bitstring (maxE-DCHMACdFlow) | Indicates, if this is the first MAC-d flow for which PDUs are placed in the MAC-i PDU, the other MAC-d flows from which MAC-d PDUs are allowed to be included in the same MAC-i PDU. Bit 0 is for MAC-d flow 0, Bit 1 is for MAC-d flow 1, … Value '1' for a bit means multiplexing is allowed. Bit 0 is the first/leftmost bit of the bit string. NOTE: The bit that corresponds to the MAC-d flow itself is ignored. | REL-8 |
| >E-DCH MAC-d flow retransmission timer | CV-Tdd128 |  | Enumerated (10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 65, 70, 75, 80, 85, 90, 95, 100, 110, 120, 140, 160, 200, 240, 280, 320, 400, 480, 560) | Unit: ms | REL-8 |

NOTE 1:	For FDD, E-DCH MAC-d flow identity 7 is reserved for CCCH transmission.

| Condition | Explanation |
| --- | --- |
| Tdd128 | This IE is mandatory present for 1.28 Mcps TDD, otherwise it is not needed. |
| NotCCCHFDD | This IE is not needed for the E-DCH MAC-d flow reserved for CCCH transmission for FDD, otherwise it is optional. |

#### 10.3.5.4	Deleted DL TrCH information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Downlink transport channel type | MP |  | Enumerated (DCH,DSCH | Note 1 |  |
|  |  |  | , HS-DSCH) |  | REL-5 |
| DL Transport channel identity | MP |  | Transport channel identity 10.3.5.18 |  |  |
|  | CV-notHS-DSCH |  |  |  | REL-5 |
| CHOICE DL MAC header type | CV-HS-DSCH |  |  |  | REL-7 |
| >MAC-hs |  |  |  |  | REL-7 |
| >>DL HS-DSCH MAC-d flow identity | MP |  | MAC-d flow identity 10.3.5.7c |  | REL-5 |
| >MAC-ehs |  |  |  |  | REL-7 |
| >>DL HS-DSCH MAC-ehs reordering queue | MP |  | MAC-ehs Queue Id 10.3.5.7f |  | REL-7 |
| Note 1: The IE "Downlink transport channel type" value " DSCH " should not be used for FDD. If received the UE behaviour is unspecified. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| NotHS-DSCH | If the downlink transport channel type is DCH or DSCH then this IE is mandatory otherwise it is not needed. |
| HS-DSCH | If the downlink transport channel type is HS-DSCH then this IE is mandatory otherwise it is not needed. |

#### 10.3.5.5	Deleted UL TrCH information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Uplink transport channel type | MP |  | Enumerated(DCH,USCH | USCH is TDD only |  |
|  |  |  | ,E-DCH) |  | REL-6 |
| UL Transport channel identity | MP |  | Transport channel identity 10.3.5.18 |  |  |
|  | CV-NotE-DCH |  |  |  | REL-6 |
| E-DCH MAC-d flow identity | CV-E-DCH |  | E-DCH MAC-d flow identity 10.3.5.7e |  | REL-6 |

| Condition | Explanation |
| --- | --- |
| NotE-DCH | If the uplink transport channel type is DCH or USCH then this IE is mandatory otherwise it is not needed. |
| E-DCH | If the uplink transport channel type is E-DCH then this IE is mandatory otherwise it is not needed. |

#### 10.3.5.6	DL Transport channel information common for all transport channels

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SCCPCH TFCS | OP |  | Transport Format Combination Set 10.3.5.20 | This IE should not be included in this version of the protocol. |  |
| CHOICE mode | MP |  |  | Although this IE is not always required, need is MP to align with ASN.1 |  |
|  | OP |  |  |  | REL-4 |
| >FDD |  |  |  |  |  |
| >>CHOICE DL parameters | OP |  |  |  |  |
| >>>Explicit |  |  |  |  |  |
| >>>>DL DCH TFCS | MP |  | Transport Format Combination Set 10.3.5.20 | Although this IE is not always required, need is MP to align with ASN.1 |  |
|  | OP |  |  |  | REL-4 |
| >>>SameAsUL |  |  |  | (no data) See note 2 |  |
| >TDD |  |  |  |  |  |
| >>Individual DL CCTrCH information | OP | 1 to <maxCCTrCH> |  |  |  |
| >>>DL TFCS Identity | MP |  | Transport format combination set identity 10.3.5.21 | Identifies a special CCTrCH for shared or dedicated channels. |  |
| >>>CHOICE DL parameters | MP |  |  |  |  |
| >>>>Independent |  |  |  |  |  |
| >>>>>DL TFCS | MP |  | Transport format combination set 10.3.5.20 |  |  |
| >>>>SameAsUL |  |  |  | See note 2 |  |
| >>>>>UL DCH TFCS Identity | MP |  | Transport format combination set identity 10.3.5.21 | Same TFCS applies as specified for the indicated UL DCH TFCS identity except for information applicable for UL only |  |

NOTE 1:		This information element is included within IE "Predefined TrCh configuration".

NOTE 2:	The UTRAN should only use the choice "SameAsUL" if the transport channel numbering, the number of TFs in the TFS of each transport channel, and the TFCS are identical in the uplink and downlink.

#### 10.3.5.7	Void

#### 10.3.5.7a	HARQ Info

This IE is used in relation to the HS-DSCH transport channel.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Number of Processes | MP |  | Integer (1..8, |  | REL-5 |
|  |  |  | 12, 14, 16, | NOTE 1, NOTE 2 | REL-7 |
|  |  |  | 24,28,32) |  | REL-11 |
| CHOICE Memory Partitioning | MP |  |  |  | REL-5 |
| >Implicit |  |  |  | UE shall apply memory partitioning of equal size across all HARQ processes | REL-5 |
| >Explicit |  |  |  |  | REL-5 |
| >>Memory size | MP | <1 to MaxHProcesses> |  |  | REL-5 |
| >>>Process Memory size | MP |  | Integer(800 .. 16000 by step of 800, 17600 .. 32000 by step of 1600, 36000 .. 80000 by step of 4000, 88000 .. 160000 by step of 8000, 176000 .. 304000 by step of 16000) | Maximum number of soft channel bits available in the virtual IR buffer [27] | REL-5 |
| >>Additional memory sizes for MIMO | OP | <1 to MaxHProcesses> |  | NOTE 2 | REL-7 |
| >>>Process Memory size | MP |  | Integer (800 .. 16000 by step of 800, 17600 .. 32000 by step of 1600, 36000 .. 80000 by step of 4000, 88000 .. 160000 by step of 8000, 176000 .. 304000 by step of 16000) | Maximum number of soft channel bits available in the virtual IR buffer [27] | REL-7 |
| >>Additional memory sizes for 3rd MIMO stream | OP | <1 to MaxHProcesses> |  | NOTE 2 | REL-11 |
| >>>Process Memory size | MP |  | Integer (800 .. 16000 by step of 800, 17600 .. 32000 by step of 1600, 36000 .. 80000 by step of 4000, 88000 .. 160000 by step of 8000, 176000 .. 304000 by step of 16000) | Maximum number of soft channel bits available in the virtual IR buffer [27] | REL-11 |
| >>Additional memory sizes for 4th MIMO stream | OP | <1 to MaxHProcesses> |  | NOTE 2 | REL-11 |
| >>>Process Memory size | MP |  | Integer (800 .. 16000 by step of 800, 17600 .. 32000 by step of 1600, 36000 .. 80000 by step of 4000, 88000 .. 160000 by step of 8000, 176000 .. 304000 by step of 16000) | Maximum number of soft channel bits available in the virtual IR buffer [27] | REL-11 |
| Blind HARQ retransmissions for HSDPA | OP |  | Enumerated(TRUE) | FDD only. TRUE means that blind HARQ retransmissions for HSDPA is enabled. | REL-13 |
| NOTE 1: If the IE “Number of processes" is not set to one of the values 12, 14 or 16 and either the IE “MIMO parameters” (10.3.6.41a) is included and the IE "MIMO operation" is set to either “start” or “continue”, the UE behaviour is unspecified. If the IE "Number of processes" is set to one of the values 12, 14, or 16 and the IE "MIMO parameters" (10.3.6.41a) is not included, the UE behaviour is unspecified. |  |  |  |  |  |
| NOTE 2: The values of the IE “Number of processes" greater than 8 and the IE "Additional memory sizes for MIMO" are not needed in SIB type 5 and SIB type 5bis. |  |  |  |  |  |

#### 10.3.5.7b	Void

#### 10.3.5.7c	MAC-d Flow Identity

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MAC-d flow identity | MP |  | Integer (0..7) |  | REL-5 |

#### 10.3.5.7d	HARQ Info for E-DCH

This IE is used in relation to the E-DCH transport channel.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  | REL-7 |
| >FDD |  |  |  |  | REL-7 |
| >>HARQ RV Configuration | MP |  | Enumerated (rv0, rvtable) | If “rv0” is indicated, the UE shall only use E_DCH RV index 0. If “rvtable” is indicated, the UE shall use an RSN based RV index as specified in [27] | REL-6 |
| >TDD |  |  |  |  | REL-7 |
| >>HARQ RV Configuration | MP |  | Enumerated (rv0, rvtable) | If “rv0” is indicated, the UE shall only use E_DCH RV index 0. If “rvtable” is indicated, the UE shall use an RSN based RV index as specified in [31] | REL-7 |

#### 10.3.5.7e	E-DCH MAC-d Flow Identity

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-DCH MAC-d flow identity | MP |  | Integer (0..maxE-DCHMACdFlow-1) |  | REL-6 |

#### 10.3.5.7f	MAC-ehs Queue Id

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MAC-ehs queue id | MP |  | Integer (0..7) |  | REL-7 |

#### 10.3.5.8	Power Offset Information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Gain Factors | MP |  |  |  |
| >Signalled Gain Factors |  |  |  |  |
| >>CHOICE mode |  |  |  |  |
| >>>FDD |  |  |  |  |
| >>>>Gain Factor c | MP |  | Integer (0.. 15) | For UL DPCCH or control part of PRACH |
| >>>TDD |  |  |  | (no data) |
| >>Gain Factor d | MP |  | Integer (0..15) | For UL DPDCH or data part of PRACH and all uplink channels in TDD |
| >>Reference TFC ID | OP |  | Integer (0..3) | If this TFC is a reference TFC, indicates the reference ID. |
| >Computed Gain Factors |  |  |  |  |
| >>Reference TFC ID | MP |  | Integer (0.. 3) | Indicates the reference TFC Id of the TFC to be used to calculate the gain factors for this TFC. In case of using computed gain factors, at least one signalled gain factor is necessary for reference. |
| CHOICE mode | MP |  |  |  |
| >FDD |  |  |  |  |
| >>Power offset P p-m | OP |  | Integer(-5..10) | In dB. Power offset between the last transmitted preamble and the control part of the message (added to the preamble power to receive the power of the message control part ) Needed only for PRACH |
| >TDD |  |  |  | (no data) |

| CHOICE Gain Factors | Condition under which the way to signal the Gain Factors is chosen |
| --- | --- |
| Signalled Gain Factors | The values for gain factors c (only in FDD mode) and d are signalled directly for a TFC. |
| Computed Gain Factors | The gain factors c (only in FDD mode) and d are computed for a TFC, based on the signalled settings for the associated reference TFC. |

#### 10.3.5.9	Predefined TrCH configuration

This information element concerns a pre- defined configuration of transport channel parameters.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| UL Transport channel information common for all transport channels | MP |  | UL Transport channel information common for all transport channels 10.3.5.24 |  |
| Added or Reconfigured TrCH information |  |  |  |  |
| Added or Reconfigured UL TrCH information | MP | 1 to <maxTrCHpreconf> |  |  |
| >Added or Reconfigured UL TrCH information | MP |  | Added or Reconfigured UL TrCH information 10.3.5.2 |  |
| DL Transport channel information common for all transport channels | MP |  | DL Transport channel information common for all transport channels 10.3.5.6 |  |
| Downlink transport channels |  |  |  |  |
| Added or Reconfigured DL TrCH information | MP | 1 to <maxTrCHpreconf> |  |  |
| >Added or Reconfigured DL TrCH information | MP |  | Added or Reconfigured DL TrCH information 10.3.5.1 |  |

#### 10.3.5.10	Quality Target

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BLER Quality value | MP |  | Real(-6.3 ..0 by step of 0.1) | Signalled value is Log10(Transport channel BLER quality target) |

#### 10.3.5.11	Semi-static Transport Format Information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Transmission time interval | MP |  | Integer(10, 20, 40, 80, dynamic | In ms. The value dynamic is only used in TDD mode. For FDD DCH, the value “80” is applicable only when SF=512. |  |
|  |  |  | 5) | 5 is only applicable for the RACH in 1.28 Mcps TDD | REL-4 |
| Type of channel coding | MP |  | Enumerated(No coding, Convolutional, Turbo) | The option "No coding" is only valid for TDD. |  |
| Coding Rate | CV-Coding |  | Enumerated(1/2, 1/3) |  |  |
| Rate matching attribute | MP |  | Integer(1..hiRM) |  |  |
| CRC size | MP |  | Integer(0, 8, 12, 16, 24) | in bits |  |

| Condition | Explanation |
| --- | --- |
| Coding | This IE is mandatory present if IE "Type of channel coding" is "Convolutional" and not needed otherwise. |

#### 10.3.5.12	Void

#### 10.3.5.13	TFCS Explicit Configuration

| Information Element/Group name | Need | Multi | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE TFCS representation | MP |  |  |  |
| >Complete reconfiguration |  |  |  |  |
| >>TFCS complete reconfiguration information | MP |  | TFCS Reconfiguration/Addition information 10.3.5.15 |  |
| >Addition |  |  |  |  |
| >>TFCS addition information | MP |  | TFCS Reconfiguration/Addition information 10.3.5.15 |  |
| >Removal |  |  |  |  |
| >>TFCS removal information | MP |  | TFCS Removal Information 10.3.5.16 |  |
| >Replace |  |  |  |  |
| >>TFCS removal information | MP |  | TFCS Removal Information 10.3.5.16 |  |
| >>TFCS addition information | MP |  | TFCS Reconfiguration/Addition information 10.3.5.15 |  |

#### 10.3.5.14	Void

#### 10.3.5.15	TFCS Reconfiguration/Addition Information

| Information Element/Group name | Need | Multi | IE type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE CTFC Size | MP |  |  |  |  |
| >2 bit CTFC |  |  |  |  |  |
| >>CTFC information | MP | 1 to <maxTFC> |  |  |  |
| >>>2bit CTFC | MP |  | Integer(0..3) |  |  |
| >>>Power offset Information | OP |  | Power Offset Information 10.3.5.8 | Needed only for uplink physical channels. |  |
| >>>Power offset Information 10ms Mode | CV-Message |  | Power offset Information 10ms Mode 10.3.5.27 | FDD only. Needed only for uplink physical channels. If not included, the corresponding TFC shall not be considered for 10ms Mode. | REL-12 |
| >4 bit CTFC |  |  |  |  |  |
| >>CTFC information | MP | 1 to <maxTFC> |  |  |  |
| >>>4bit CTFC | MP |  | Integer(0..15) |  |  |
| >>>Power offset Information | OP |  | Power Offset Information 10.3.5.8 | Needed only for uplink physical channels. |  |
| >>>Power offset Information 10ms Mode | CV-Message |  | Power offset Information 10ms Mode 10.3.5.27 | FDD only. Needed only for uplink physical channels. If not included, the corresponding TFC shall not be considered for 10ms Mode. | REL-12 |
| >6 bit CTFC |  |  |  |  |  |
| >>CTFC information | MP | 1 to <maxTFC> |  |  |  |
| >>>6 bit CTFC | MP |  | Integer(0..63) |  |  |
| >>>Power offset Information | OP |  | Power Offset Information 10.3.5.8 | Needed only for uplink physical channels. |  |
| >>>Power offset Information 10ms Mode | CV-Message |  | Power offset Information 10ms Mode 10.3.5.27 | FDD only. Needed only for uplink physical channels. If not included, the corresponding TFC shall not be considered for 10ms Mode. | REL-12 |
| >8 bit CTFC |  |  |  |  |  |
| >>CTFC information | MP | 1 to <MaxTFC> |  |  |  |
| >>>8 bit CTFC | MP |  | Integer(0..255) |  |  |
| >>>Power offset Information | OP |  | Power Offset Information 10.3.5.8 | Needed only for uplink physical channels. |  |
| >>>Power offset Information 10ms Mode | CV-Message |  | Power offset Information 10ms Mode 10.3.5.27 | FDD only. Needed only for uplink physical channels. If not included, the corresponding TFC shall not be considered for 10ms Mode. | REL-12 |
| >12 bit CTFC |  |  |  |  |  |
| >>CTFC information | MP | 1 to <maxTFC> |  |  |  |
| >>>12 bit CTFC | MP |  | Integer(0..4095) |  |  |
| >>>Power offset Information | OP |  | Power Offset Information 10.3.5.8 | Needed only for uplink physical channels. |  |
| >>>Power offset Information 10ms Mode | CV-Message |  | Power offset Information 10ms Mode 10.3.5.27 | FDD only. Needed only for uplink physical channels. If not included, the corresponding TFC shall not be considered for 10ms Mode. | REL-12 |
| >16 bit CTFC |  |  |  |  |  |
| >>CTFC information | MP | 1 to <maxTFC> |  |  |  |
| >>>16 bit CTFC | MP |  | Integer(0..65535) |  |  |
| >>>Power offset Information | OP |  | Power Offset Information 10.3.5.8 | Needed only for uplink physical channels. |  |
| >>>Power offset Information 10ms Mode | CV-Message |  | Power offset Information 10ms Mode 10.3.5.27 | FDD only. Needed only for uplink physical channels. If not included, the corresponding TFC shall not be considered for 10ms Mode. | REL-12 |
| >24 bit CTFC |  |  |  |  |  |
| >>CTFC information | MP | 1 to <MaxTFC> |  |  |  |
| >>>24 bit CTFC | MP |  | Integer(0..16777215) |  |  |
| >>>Power offset Information | OP |  | Power Offset Information 10.3.5.8 | Needed only for uplink physical channels. |  |
| >>>Power offset Information 10ms Mode | CV-Message |  | Power offset Information 10ms Mode 10.3.5.27 | FDD only. Needed only for uplink physical channels. If not included, the corresponding TFC shall not be considered for 10ms Mode. | REL-12 |

| Condition | Explanation |
| --- | --- |
| Message | This IE is not needed if the IE "Uplink Transport channel information common for all transport channels" is included in RRC CONNECTION SETUP or SYSTEM INFORMATION BLOCK TYPE 16 messages. Otherwise it is optional. |

#### 10.3.5.16	TFCS Removal Information

| Information Element/Group name | Need | Multi | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Removal TFCI information | MP | 1 to <maxTFC> |  |  |
| >TFCI | MP |  | Transport Format Combination (TFC) 10.3.5.19 | In TDD 0 is a reserved value |

#### 10.3.5.17	Void

#### 10.3.5.18	Transport channel identity

This information element is used to distinguish transport channels. Transport channels of different type (RACH, USCH, FACH/PCH, DSCH or DCH) have separate series of identities. This also holds for uplink and downlink transport channel identities (i.e. for DCH). Depending on in which context a transport channel identity n that is sent, it will have different meaning

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport channel identity | MP |  | Integer(1..32) |  |

#### 10.3.5.19	Transport Format Combination (TFC)

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport format combination | MP |  | Integer (0.. 1023) |  |

#### 10.3.5.20	Transport Format Combination Set

Indicates the allowed combinations of already defined Transport formats and the mapping between these allowed TFCs and the corresponding TFCI values.

For TDD, different coded composite transport channels have independent transport format combination sets and thus independent TFCI values.

| Information Element/Group name | Need | Multi | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TFCI Field 1 Information | MP |  | TFCS explicit Configuration 10.3.5.13 |  |

#### 10.3.5.21	Transport Format Combination Set Identity

NOTE:	Only for TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TFCS ID | MD |  | Integer (1...8) | Indicates the identity of every TFCS within a UE. Default value is 1. |
| Shared Channel Indicator | MP |  | Boolean | TRUE indicates the use of shared channels. Default is FALSE. |

#### 10.3.5.22	Transport Format Combination Subset

Indicates which Transport format combinations in the already defined Transport format combination set are allowed.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Subset representation | MP |  |  |  |
| >Minimum allowed Transport format combination index |  |  | Transport format combination 10.3.5.19 |  |
| >Allowed transport format combination list |  | 1 to <maxTFC> |  |  |
| >>Allowed transport format combination | MP |  | Transport format combination 10.3.5.19 |  |
| >Non-allowed transport format combination list |  | 1 to <maxTFC> |  |  |
| >>Non-allowed transport format combination | MP |  | Transport format combination 10.3.5.19 |  |
| >Restricted TrCH information |  | 1 to <maxTrCH> |  |  |
| >>Uplink transport channel type | MP |  | Enumerated(DCH, USCH) | USCH is TDD only |
| >>Restricted UL TrCH identity | MP |  | Transport channel identity 10.3.5.18 |  |
| >>Allowed TFIs | OP | 1 to <maxTF> |  |  |
| >>>Allowed TFI | MP |  | Integer(0..31) |  |
| >Full transport format combination set |  |  |  | (No data) |

#### 10.3.5.23	Transport Format Set

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Transport channel type | MP |  |  |  |  |
| >Dedicated transport channels |  |  |  | The transport channel that is configured with this TFS is of type DCH |  |
| >>Dynamic Transport Format Information | MP | 1 to <maxTF> |  |  |  |
| >>>RLC Size | MP |  | Integer(16..5000 by step of 8) | Unit is bits |  |
| >>>Number of TBs and TTI List | MP | 1 to <maxTF> |  | Present for every valid number of TB's (and TTI) for this RLC Size. |  |
| >>>>Transmission Time Interval | CV-dynamicTTI |  | Integer(10,20,40,80) | Unit is ms. |  |
| >>>>Number of Transport blocks | MP |  | Integer(0..512) |  |  |
| >>>CHOICE Logical Channel List | MP |  |  | The logical channels that are allowed to use this RLC Size |  |
| >>>>ALL |  |  | Null | All logical channels mapped to this transport channel. |  |
| >>>>Configured |  |  | Null | The logical channels configured to use this RLC size in the RB mapping info. 10.3.4.21 if present in this message or in the previously stored configuration otherwise |  |
| >>>>Explicit List |  | 1 to 15 |  | Lists the logical channels that are allowed to use this RLC size. |  |
| >>>>>RB Identity | MP |  | RB identity 10.3.4.16 |  |  |
| >>>>>LogicalChannel | CH-UL-RLCLogicalChannels |  | Integer(0..1) | Indicates the relevant UL logical channel for this RB. "0" corresponds to the first, "1" corresponds to the second UL logical channel configured for this RB in the IE "RB mapping info". |  |
| >>Semi-static Transport Format Information | MP |  | Semi-static Transport Format Information 10.3.5.11 |  |  |
| >Common transport channels |  |  |  | The transport channel that is configured with this TFS is of a type not equal to DCH |  |
| >>Dynamic Transport Format Information | MP | 1 to <maxTF> |  | Note |  |
| >>>RLC Size | MP |  | Integer (48..4968) | Unit is bits. For FDD, values are restricted to: Integer (48..296 by step of 8, 312..1320 by step of 16, 1384..4968 by step of 64). |  |
| >>>Number of TBs and TTI List | MP | 1 to <maxTF> |  | Present for every valid number of TB's (and TTI) for this RLC Size. |  |
| >>>>Number of Transport blocks | MP |  | Integer(0..512) |  |  |
| >>>>CHOICE mode | MP |  |  |  |  |
| >>>>>FDD |  |  |  | (no data) |  |
| >>>>>TDD |  |  |  |  |  |
| >>>>>>Transmission Time Interval | CV-dynamicTTI |  | Integer(10,20,40,80) | Unit is ms. |  |
| >>>CHOICE Logical Channel List | MP |  |  | The logical channels that are allowed to use this RLC Size. |  |
| >>>>ALL |  |  | Null | All logical channels mapped to this transport channel. |  |
| >>>>Configured |  |  | Null | The logical channels configured to use this RLC size in the RB mapping info. 10.3.4.21 if present in this message or in the previously stored configuration otherwise |  |
| >>>>Explicit List |  | 1 to 15 |  | Lists the logical channels that are allowed to use this RLC size. |  |
| >>>>>RB Identity | MP |  | RB identity 10.3.4.16 |  |  |
| >>>>>LogicalChannel | CV-UL-RLCLogicalChannels |  | Integer(0..1) | Indicates the relevant UL logical channel for this RB. "0" corresponds to the first, "1" corresponds to the second UL logical channel configured for this RB in the IE "RB mapping info". |  |
| >>Semi-static Transport Format Information | MP |  | Semi-static Transport Format Information 10.3.5.11 |  |  |

| Condition | Explanation |
| --- | --- |
| dynamicTTI | This IE is mandatory present if dynamic TTI usage is indicated in IE Transmission Time Interval in Semi-static Transport Format Information. Otherwise it is not needed. |
| UL-RLCLogicalChannels | If "Number of uplink RLC logical channels" in IE "RB mapping info" in this message is 2 or the IE "RB mapping info" is not present in this message and 2 UL logical channels are configured for this RB, then this IE is mandatory present. Otherwise this IE is not needed. |

#### 10.3.5.24	UL Transport channel information common for all transport channels

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PRACH TFCS | OP |  | Transport format combination set 10.3.5.20 | This IE should not be included in this version of the protocol. |  |
| CHOICE mode | OP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>TFC subset | MD |  | Transport Format Combination Subset 10.3.5.22 | Default value is the complete existing set of transport format combinations |  |
| >>UL DCH TFCS | MP |  | Transport format combination set 10.3.5.20 |  |  |
| >TDD |  |  |  |  |  |
| >>Individual UL CCTrCH information | OP | 1 to <maxCCTrCH> |  |  |  |
| >>>UL TFCS Identity | MP |  | Transport format combination set identity 10.3.5.21 | Identifies a special CCTrCH for shared or dedicated channels. |  |
| >>>UL TFCS | MP |  | Transport format combination set 10.3.5.20 |  |  |
| >>>TFC subset | MD |  | Transport Format Combination Subset 10.3.5.22 | Default value is the complete existing set of transport format combinations |  |
| TFC subset list | OP | 1 to <maxTFCsub> |  | The maximum number of elements in the TFC subset list is 8. | REL-4 |
| >CHOICE mode | MP |  |  |  | REL-4 |
| >>FDD |  |  |  | (no data) | REL-4 |
| >>TDD |  |  |  |  | REL-4 |
| >>>TFCS Id | OP |  | Transport Format Combination Set Identity 10.3.5.21 |  | REL-4 |
| >TFC subset | MD |  | Transport Format Combination Subset 10.3.5.22 |  | REL-4 |

NOTE:		This information element is included within IE "Predefined TrCh configuration".

#### 10.3.5.25	Concurrent Deployment of 2ms and 10ms TTI

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE TTI selection | MP |  |  | This IE indicates whether common E-DCH TTI selection is fixed or a threshold based common E-DCH selection criteria is used. | REL-11 |
| >Fixed |  |  |  |  | REL-11 |
| >>Fixed common E-DCH TTI selection | MP |  | Enumerated (2ms,10ms) |  | REL-11 |
| >Threshold based |  |  |  |  | REL-11 |
| >>Common E-DCH TTI selection threshold | MP |  | Integer (0..15) | In terms of dB. | REL-11 |
| Concurrent TTI partition index | MP |  | Integer (0.. maxEDCHs-1) |  | REL-11 |
| Common E-DCH system info parameters for Concurrent TTI | OP |  | Common E-DCH system info parameters for Concurrent TTI 10.3.6.131 | Common E-DCH parameters used to replace those listed in the IE "Common E-DCH system info" of the System Information Block Type 5/5bis, if the TTI applied on E-DPDCH (as per subclause 8.5.45) is different from the TTI signalled in the IE "E-DCH Transmission Time Interval". | REL-11 |

#### 10.3.5.26	Common E-DCH MAC-d flow info for Concurrent TTI

This IE is used in relation to MAC-d and MAC-c flows mapped to the E-DCH transport channels.

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Configured E-DCH MAC-d flow list | MP | 1 to <maxE-DCHMACdFlow> |  | NOTE 1 NOTE 4 | REL-11 |
| >E-DCH MAC-d flow identity | MP |  | E-DCH MAC-d Flow Identity 10.3.5.7e |  | REL-11 |
| >E-DCH MAC-d flow power offset | MD |  | Integer(0..6) | NOTE 2 | REL-11 |
| >E-DCH MAC-d flow maximum number of retransmissions | MD |  | Integer (0..15) | NOTE 2 | REL-11 |
| NOTE 1: E-DCH MAC-d flow identities that are present in "Common E-DCH MAC-d flows" of the System Information Block Type 5/5bis but do not appear in this list shall use value included in "Common E-DCH MAC-d flows". NOTE 2: Default value is the value for the flow with the same "E-DCH MAC-d flow identity" included in the IE "Common E-DCH MAC-d flows" of the System Information Block Type 5/5bis. NOTE 4: For E-DCH MAC-d flows that are present in this list, the IE "E-DCH MAC-d flow multiplexing list" shall use the corresponding value for the flow with the same "E-DCH MAC-d flow identity" included in the IE "Common E-DCH MAC-d flows" of the System Information Block Type 5/5bis. |  |  |  |  |  |

NOTE 3:	For FDD, E-DCH MAC-d flow identity 7 is reserved for CCCH transmission.

#### 10.3.5.27	Power Offset Information 10ms Mode

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Gain Factors | MP |  |  |  | REL-12 |
| >Signalled Gain Factors |  |  |  |  |  |
| >>Gain Factor c | MP |  | Integer (0.. 15) | For UL DPCCH | REL-12 |
| >>Gain Factor d | MP |  | Integer (0..15) | For UL DPDCH | REL-12 |
| >>Reference TFC ID | OP |  | Integer (0..3) | If this TFC is a reference TFC, indicates the reference ID. | REL-12 |
| >Computed Gain Factors |  |  |  |  |  |
| >>Reference TFC ID | MP |  | Integer (0.. 3) | Indicates the reference TFC Id of the TFC to be used to calculate the gain factors for this TFC. In case of using computed gain factors, at least one signalled gain factor is necessary for reference. | REL-12 |

| CHOICE Gain Factors | Condition under which the way to signal the Gain Factors is chosen |
| --- | --- |
| Signalled Gain Factors | The values for gain factors c and d are signalled directly for a TFC. |
| Computed Gain Factors | The gain factors c and d are computed for a TFC, based on the signalled settings for the associated reference TFC. |

### 10.3.6	Physical CH Information elements

#### 10.3.6.1	AC-to-ASC mapping

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AC-to-ASC mapping table | MP | maxASCmap |  |  |
| >AC-to-ASC mapping | MP |  | Integer(0…7) | Mapping of Access Classes to Access Service Classes (see subclause 8.5.13.) |

#### 10.3.6.2	AICH Info

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Channelisation code | MP |  | Integer(0..255) | SF is fixed and equal to 256 |
| STTD indicator | MP |  | STTD Indicator 10.3.6.78 |  |
| AICH transmission timing | MP |  | Enumerated (0, 1) | See parameter AICH_Transmission_Timing in [26] |

#### 10.3.6.3	AICH Power offset

NOTE:	Only for FDD.

This parameter is used to indicate the power level of AICH channel. This is the power per transmitted Acquisition Indicator, Extended Acquisition Indicator  minus power of the Primary CPICH.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AICH Power offset | MP |  | Integer(-22..+5) | Offset in dB |

#### 10.3.6.4	Allocation period info

NOTE:	Only for TDD.

Parameters used by UE to determine period of shared channel allocation.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Allocation Activation Time | MP |  | Integer (0..255) | Start the allocation period at the given CFN. |
| Allocation Duration | MP |  | Integer (1..256) | Total number of frames for the allocation period. |

#### 10.3.6.5	Alpha

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Alpha Value | MP |  | Enumerated(0, 1/8, 2/8, 3/8, 4/8, 5/8, 6/8, 7/8, 1) |  |

#### 10.3.6.6	ASC setting

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Available signature Start Index | MP |  | Integer(0..15) |  |  |
| >>Available signature End Index | MP |  | Integer(0..15) |  |  |
| >>Assigned Sub-Channel Number | MP |  | Bit string(4) | This IE defines the subchannel assignment as specified in 8.6.6.29. The first/leftmost bit of the bit string contains the most significant bit of the Assigned Sub-Channel Number.. |  |
| >TDD |  |  |  |  |  |
| >>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>Available Channelisation codes indices | MD |  | Bit string(8) | Each bit indicates availability of a channelisation code index, where the channelisation code indices are numbered "channelisation code index 0" to "channelisation code index 7". The value 1 of a bit indicates that the channelisation code index is available for the ASC this IE is associated to. The value 0 of a bit indicates that the channelisation code index is not available for the ASC this IE is associated to. Default is that all channelisation codes defined in PRACH Info are available. |  |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Available Channelisation codes indices | MD |  | Bit string(16) | Each bit indicates availability of a channelisation code index, where the channelisation code indices are numbered "channelisation code index 0" to "channelisation code index 15". The value 1 of a bit indicates that the channelisation code index is available for the ASC this IE is associated to. The value 0 of a bit indicates that the channelisation code index is not available for the ASC this IE is associated to. Default is that all channelisation codes defined in PRACH Info are available. | REL-7 |
| >>>1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>>>Available SYNC_UL codes indices | MD |  | Bit string(8) | Each bit indicates availability of a SYNC_UL code index, where the SYNC_UL code indices are numbered "SYNC_UL code index 0" to "SYNC_UL code index 7". The value 1 of a bit indicates that the SYNC_UL code index is available for the ASC this IE is associated to. The value 0 of a bit indicates that the SYNC_UL code index is not available for the ASC this IE is associated to. Default is that all SYNC_UL codes defined in SYNC_UL Info are available. | REL-4 |
| >>CHOICE subchannel size | MP |  |  |  |  |
| >>>Size1 |  |  |  |  |  |
| >>>>Available Subchannels | MP |  | null | Indicates that all Subchannels are available |  |
| >>>Size2 |  |  |  |  |  |
| >>>>Available Subchannels | MD |  | Bit string (2) | NOTE |  |
| >>>Size4 |  |  |  |  |  |
| >>>>Available Subchannels | MD |  | Bit string (4) | NOTE |  |
| >>>Size8 |  |  |  |  |  |
| >>>>Available Subchannels | MD |  | Bit string (8) | NOTE |  |
| >>>Size16 |  |  |  |  |  |
| >>>>Available Subchannels | MD |  | Bit string (16) | NOTE |  |

NOTE:	Each bit indicates availability of a subchannel, where the subchannels are numbered subchannel 0, subchannel 1 etc. The value 1 of a bit indicates that the subchannel is available for the ASC this IE is associated with. The value 0 of a bit indicates that the subchannel is not available for the ASC this IE is associated with. Default value of the IE is that all subchannels within the size are available for the ASC this IE is associated with.

#### 10.3.6.7	Void

#### 10.3.6.8	CCTrCH power control info

Parameters used by UE to set the SIR target value for uplink open loop power control in TDD.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| TFCS Identity | OP |  | Transport Format Combination Set Identity 10.3.5.21 | TFCS Identity of this CCTrCH. Default value is 1. |
| Uplink DPCH power control info | MP |  | Uplink DPCH power control info 10.3.6.91 |  |

#### 10.3.6.8a	Cell and Channel Identity info

NOTE:	Only for TDD.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Burst type | MP |  | Enumerated (Type1, Type2) | Identifies the channel in combination with the Midamble shift and slot number. It is not used in 1.28 Mcps TDD and may be set to either value. This IE should be ignored by the receiver. |
| Midamble Shift | MP |  | Integer (0…15) |  |
| Time Slot | MP |  | Timeslot number 10.3.6.84 | This IE is present only if no IPDL scheme is configured in the reference cell. Otherwise the slot is defined by the IPDL configuration. |
| Cell parameters ID | MP |  | Cell parameters ID 10.3.6.9 | Identifies the cell |

#### 10.3.6.9	Cell parameters Id

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell parameter Id | MP |  | Integer(0..127) |  |

#### 10.3.6.9a	Common E-DCH system info

NOTE:	Only for FDD and 1.28 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UL interference for common E-DCH | OP |  | UL interference 10.3.6.87 |  | REL-8 |
| Common E-DCH MAC-d flows | MP |  | Common E-DCH MAC-d flows 10.3.5.3b |  | REL-8 |
| CHOICE mode | MP |  |  |  | REL-8 |
| >FDD |  |  |  |  | REL-8 |
| >>PRACH preamble control parameters (for Enhanced Uplink) | MP |  | PRACH preamble control parameters (for Enhanced Uplink) 10.3.6.54a | Control parameters of the physical signal. | REL-8 |
| >>Initial Serving grant value | MP |  | Integer (0..37) | (0..37) indicates E-DCH serving grant index as defined in [15]. | REL-8 |
| >>E-DCH Transmission Time Interval | MP |  | Enumerated (2,10) | Unit is ms. | REL-8 |
| >>E-AGCH Info | MP |  | E-AGCH Info 10.3.6.100 |  | REL-8 |
| >>HARQ info for E-DCH | MP |  | HARQ info for E-DCH 10.3.5.7d |  | REL-8 |
| >>Uplink DPCH power control info | MP |  | Uplink DPCH power control info for Common E-DCH 10.3.6.91a |  | REL-8 |
| >>E-DPCCH info | MP |  | E-DPCCH Info 10.3.6.98 |  | REL-8 |
| >>E-DPDCH info | MP |  | E-DPDCH info 10.3.6.99 |  | REL-8 |
| >>F-DPCH TPC command error rate target | MP |  | Real (0.01..0.1 by step of 0.01). | Downlink F-DPCH information. The actual value of dl-FDPCH-TPCcommandErrorRate = IE value * 0.01 | REL-8 |
| >>Additional E-DCH transmission back off | MP |  | Integer (0..15) | In terms of TTIs. | REL-8 |
| >>Maximum E-DCH resource allocation for CCCH | MP |  | Enumerated (8, 12, 16, 20, 24, 32, 40, 80) | In terms of TTIs. | REL-8 |
| >>Maximum period for collision resolution phase | MP |  | Integer (8..24) | In terms of TTIs. | REL-8 |
| >>E-DCH transmission continuation back off | MP |  | Enumerated (0, 4, 8, 16, 24, 40, 80, infinity) | In terms of TTIs. If set to "infinity", implicit common E-DCH resource release is disabled. | REL-8 |
| >>ACK/NACK support on HS-DPCCH | MP |  | Boolean | TRUE indicates that HS-DPCCH shall be used when a common E-DCH resource is allocated to the UE for DTCH/DCCH transmission after collision resolution. FALSE indicates that HS-DPCCH shall not be used | REL-8 |
| >>Measurement Feedback Info | OP |  | Measurement Feedback Info 10.3.6.40a |  | REL-8 |
| >>Common E-DCH Resource Configuration information list | MP | 1 to <maxEDCHs> |  |  | REL-8 |
| >>>Soffset | MD |  | Integer(0..9) | (0..9) indicates symbol offset as defined in [26] NOTE3 | REL-8 |
| >>>F-DPCH Code number | CV-Short |  | Integer (0..255) | If this IE is not present, the value is equal to the last occurrence of this IE. | REL-8 |
| >>>E-RGCH Information | OP |  | E-RGCH Info for Common E-DCH 10.3.6.123 |  | REL-8 |
| >>>E-HICH info | MP |  | E-HICH info for Common E-DCH 10.3.6.124 |  | REL-8 |
| >>>Uplink DPCH code info | MP |  | Uplink DPCH code info for Common E-DCH 10.3.6.87b |  | REL-8 |
| >TDD |  |  |  |  | REL-8 |
| >>CHOICE TDD option | MP |  |  |  | REL-8 |
| >>>7.68 Mcps TDD |  |  | NULL |  | REL-8 |
| >>>3.84 Mcps TDD |  |  | NULL |  | REL-8 |
| >>>1.28 Mcps TDD |  |  |  |  | REL-8 |
| >>>>E-RUCCH Info | MP |  | E-RUCCH Info 1.28 Mcps TDD 10.3.6.103a |  | REL-8 |
| >>>>E-PUCH info | MP |  | E-PUCH Info 1.28 Mcps TDD 10.3.6.104a | Note1 | REL-8 |
| >>>>E-HICH info | MP |  | E-HICH info 1.28 Mcps TDD 10.3.6.101a | Note1 | REL-8 |
| >>>>E-AGCH Info | MP |  | E-AGCH Info 1.28 Mcps TDD 10.3.6.100a | Note1 | REL-8 |
| >>>>HARQ info for E-DCH | MP |  | HARQ info for E-DCH 10.3.5.7d |  | REL-8 |
| >>>>CCCH transmission info | MP |  |  | Note2 | REL-8 |
| >>>>>Common E-RNTI info | MP |  | Common E-RNTI info 10.3.3.7a |  | REL-8 |
| >>>>>HARQ maximum number of retransmissions | MP |  | Integer (0..7) |  | REL-8 |
| >>>>>HARQ retransmission timer | MP |  | Enumerated (10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 65, 70, 75, 80, 85, 90, 95, 100, 110, 120, 140, 160) | Unit: ms | REL-8 |
| >>>>>HARQ power offset | MP |  | Integer(0..6) |  | REL-8 |
| NOTE3: The default value is defined by: Soffset = (offset+common E-DCH resource list position) mod 10 where "offset" is the value of the last instance of the IE "Soffset". If the IE "Soffset" was not present in the first occurrence, "offset" is equal to zero. "common E-DCH resource list position" indicates the Common E-DCH resource Configuration Information number by order of appearance. The value of the "common E-DCH resource list position" is zero for the first occurrence. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Short | This IE is mandatory present for the first occurrence. Otherwise, this IE is optional. |

NOTE1:	These IEs correspond to the HS-DSCH configuration for CELL_FACH, CELL_PCH and URA_PCH state.

NOTE2:	These IEs correspond to CCCH transmission configuration for CELL_FACH state on the primary frequency and secondary frequency.

#### 10.3.6.10	Common timeslot info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 2nd interleaving mode | MD |  | Enumerated(Frame, Timeslot) | Frame timeslot related interleaving. Default value is "Frame" |
| TFCI coding | MD |  | Integer(4,8,16,32) | Describes the amount of bits for the TFCI bits code word as described in [31]. Defaults is no TFCI bit: In case of 8 PSK in 1.28Mcps TDD: 4 corresponds to 6 TFCI code word bits. 8 corresponds to 12 TFCI code word bits. 16 corresponds to 24 TFCI code word bits. 32 corresponds to 48 TFCI code word bits. |
| Puncturing limit | MP |  | Real(0.40..1.0 by step of 0.04) |  |
| Repetition period | MD |  | Integer(1, 2,4,8,16,32,64) | Default is continuous allocation. Value 1 indicate continuous |
| Repetition length | MP |  | Integer(1.. Repetition period –1 ) | NOTE: This is empty if repetition period is set to 1. |

#### 10.3.6.10a	Common timeslot info MBMS

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 2nd interleaving mode | MD |  | Enumerated(Frame, Timeslot) | Frame timeslot related interleaving. Default value is "Frame" |
| TFCI coding | MD |  | Integer(4,8,16,32) | Describes the amount of bits for the TFCI bits code word as described in [31]. Defaults is no TFCI bit: In case of 8 PSK in 1.28Mcps TDD: 4 corresponds to 6 TFCI code word bits. 8 corresponds to 12 TFCI code word bits. 16 corresponds to 24 TFCI code word bits. 32 corresponds to 48 TFCI code word bits. |
| Puncturing limit | MP |  | Real(0.40..1.0 by step of 0.04) |  |

#### 10.3.6.11	Constant value

NOTE:	Only for FDD.

This constant value is used by the UE to calculate the initial output power on PRACH according to the Open loop power control procedure.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Constant value | MP |  | Integer (-35..-10) | In dB |

#### 10.3.6.11a	Constant value TDD

NOTE:	Only for 3.84 Mcps TDD and 7.68 Mcps TDD.

3.84 Mcps TDD and 7.68 Mcps TDD constant values are used for open loop power control of PRACH, USCH, HS-SICH and UL DPCH as defined in subclause 8.5.7.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TDD Constant value | MP |  | Integer (-35..+10) | In dB |

#### 10.3.6.12	Void

#### 10.3.6.13	Void

#### 10.3.6.14	Void

#### 10.3.6.15	Void

#### 10.3.6.16	Default DPCH Offset Value

Indicates the default offset value within interleaving size at a resolution of 512chip (1/5 slot) in FDD and a resolution of one frame in TDD to offset CFN in the UE. This is used to distribute discontinuous transmission periods in time and also to distribute NodeB-RNC transmission traffics in time. Even though the CFN is offset by DOFF, the start timing of the interleaving will be the timing that "CFN mod (interleaving size)"=0 (e.g. interleaving size: 2,4,8) in both UE and SRNC.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE mode |  |  |  |  |
| >FDD |  |  |  |  |
| >>Default DPCH Offset Value (DOFF) | MP |  | Integer (0..306688 by step of 512) | Number of chips=. 0 to 599 time 512 chips, see [10]. |
| >TDD |  |  |  |  |
| >>Default DPCH Offset Value (DOFF) | MP |  | Integer(0..7) | Number of frames; See [10] |

#### 10.3.6.17	Downlink channelisation codes

NOTE:	Only for 1.28 Mcps TDD and 3.84 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE codes representation | MP |  |  |  |
| >Consecutive codes |  |  |  |  |
| >>First channelisation code | MP |  | Enumerated ( (16/1)...(16/16)) | If a TFCI exists in this timeslot, it is mapped to the channelisation code as defined in [30]. |
| >>Last channelisation code | MP |  | Enumerated ( (16/1)...(16/16)) | If this is the same as First channelisation code, only one code is used by the physical layer. |
| >Bitmap |  |  |  |  |
| >>Channelisation codes bitmap | MP |  | Bit string( chCode16-SF16, chCode15-SF16, chCode14-SF16, chCode13-SF16, chCode12-SF16, chCode11-SF16, chCode10-SF16, chCode9-SF16, chCode8-SF16, chCode7-SF16, chCode6-SF16, chCode5-SF16, chCode4-SF16, chCode3-SF16, chCode2-SF16, chCode1-SF16 ) SIZE (16) | Each bit indicates the availability of a channelisation code for SF16, where the channelisation codes are numbered as channelisation code 1 (SF16) to channelisation code 16 (SF16). (For SF 16, a 1 in the bitmap means that the corresponding code is used, a 0 means that the corresponding code is not used.) If all bits are set to zero, SF 1 shall be used. For dedicated MBSFN frequency timeslot, if the first half in the bit pattern is set to 1010101000000000, the first channelisation code with SF 2 shall be used; if the second half of bit pattern is set to 0000000010101010, the second channelisation code with SF 2 shall be used; if all bits are set to 1010101010101010, both the first and second channelisation codes with SF2 shall be used. |

#### 10.3.6.17a	Downlink channelisation codes VHCR

NOTE:	Only for 7.68 Mcps TDD VHCR

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE codes representation | MP |  |  |  | REL-7 |
| >Consecutive codes |  |  |  |  | REL-7 |
| >>First channelisation code | MP |  | Enumerated ( (32/1)...(32/32)) | If a TFCI exists in this timeslot, it is mapped to the channelisation code as defined in [30]. | REL-7 |
| >>Last channelisation code | MP |  | Enumerated ( (32/1)...(32/32)) | If this is the same as First channelisation code, only one code is used by the physical layer. | REL-7 |
| >Bitmap |  |  |  |  | REL-7 |
| >>Channelisation codes bitmap | MP |  | Bit string(32) | Each bit indicates the availability of a channelisation code for SF32, where the channelisation codes are numbered as channelisation code 1 (SF32) to channelisation code 32 (SF32). (For SF 32, a 1 in the bitmap means that the corresponding code is used, a 0 means that the corresponding code is not used.) If all bits are set to zero, SF 1 shall be used. | REL-7 |

#### 10.3.6.18	Downlink DPCH info common for all RL

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Timing Indication | MP |  | Enumerated(Initialise, Maintain) | NOTE |  |
| Timing maintained Synchronization indicator | CV-Synch |  | Enumerated (FALSE) | FALSE indicates that no synchronisation procedure shall be performed for timing maintained inter-frequency hard handover [29]. Absence of this element means that the synchronization procedure A shall be used. | REL-6 |
| Downlink DPCH power control information | OP |  | Downlink DPCH power control information 10.3.6.23 |  |  |
| MAC-d HFN initial value | CV-Message |  | Bit string(24) |  | REL-4 |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Power offset P Pilot-DPDCH | MP |  | Integer(0..24) | Power offset equals PPilot - PDPDCH, range 0..6 dB, in steps of 0.25 dB |  |
| >>Downlink rate matching restriction information | OP |  | Downlink rate matching restriction information 10.3.6.31 | If this IE is set to "absent", no Transport CH is restricted in TFI. |  |
| >>Spreading factor | MP |  | Integer(4, 8, 16, 32, 64, 128, 256, 512) |  |  |
| >>Fixed or Flexible Position | MP |  | Enumerated (Fixed, Flexible) |  |  |
| >>TFCI existence | MP |  | Boolean | TRUE indicates that TFCI is used. When spreading factor is less than or equal to 64, FALSE indicates that TFCI is not used and therefore DTX is used in the TFCI field. |  |
| >>CHOICE SF | MP |  |  |  |  |
| >>>SF = 256 |  |  |  |  |  |
| >>>>Number of bits for Pilot bits | MP |  | Integer (2,4,8, | In bits |  |
|  |  |  | 0) | In bits. For FDD only. NOTE 2. | REL-12 |
| >>>SF = 128 |  |  |  |  |  |
| >>>>Number of bits for Pilot bits | MP |  | Integer(4,8, | In bits |  |
|  |  |  | 0) | In bits. For FDD only. | REL-12 |
| >>>Otherwise |  |  |  | (no data). In ASN.1 choice "Otherwise" is not explicitly available as all values are available, it is implied by the use of any value other than 128 or 256. |  |
| >TDD |  |  |  | (no data) |  |

| CHOICE SF | Condition under which the given SF is chosen |
| --- | --- |
| SF=128 | "Spreading factor" is set to 128 |
| SF=256 | "Spreading factor" is set to 256 |
| Otherwise | "Spreading factor" is set to a value distinct from 128 and 256 |

| Condition | Explanation |
| --- | --- |
| Message | This IE is not needed if the IE "Downlink DPCH info common for all RL" is included in RRC CONNECTION SETUP or HANDOVER TO UTRAN COMMAND messages. Otherwise it is optional. |
| Synch | The IE is not needed in the CELL UPDATE CONFIRM, HANDOVER TO UTRAN COMMAND and the RRC CONNECTION SETUP messages or if the IE "Timing Indication" is set to "Initialise" or if the IE “Frequency Info” is not included. Otherwise, it is optional. |

NOTE:	Within the handover to UTRAN command message, only value "initialise" is applicable.

NOTE 2:	Within the RRC CONNECTION SETUP message, the value 0 is not applicable.

#### 10.3.6.19	Downlink DPCH info common for all RL Post

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Downlink DPCH power control information | OP |  | Downlink DPCH power control information 10.3.6.23 |  |

#### 10.3.6.20	Downlink DPCH info common for all RL Pre

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |
| >FDD |  |  |  |  |
| >>Spreading factor | MP |  | Integer(4, 8, 16, 32, 64, 128, 256, 512) | Defined in CHOICE SF512-Andpilot with "number of its for pilot bits" in ASN.1 |
| >>Fixed or Flexible Position | MP |  | Enumerated (Fixed, Flexible) |  |
| >>TFCI existence | MP |  | Boolean | TRUE indicates that TFCI is used. When spreading factor is less than or equal to 64, FALSE indicates that TFCI is not used and therefore DTX is used in the TFCI field. |
| >>CHOICE SF | MP |  |  |  |
| >>>SF = 256 |  |  |  |  |
| >>>>Number of bits for Pilot bits | MP |  | Integer (2,4,8) | In bits |
| >>>SF = 128 |  |  |  |  |
| >>>>Number of bits for Pilot bits | MP |  | Integer(4,8) | In bits |
| >>>Otherwise |  |  |  | (no data) |
| >TDD |  |  |  |  |
| >>Common timeslot info | MP |  | Common Timeslot Info 10.3.6.10 |  |

| CHOICE SF | Condition under which the given SF is chosen |
| --- | --- |
| SF=128 | "Spreading factor" is set to 128 |
| SF=256 | "Spreading factor" is set to 256 |
| Otherwise | "Spreading factor" is set to a value distinct from 128 and 256 |

#### 10.3.6.21	Downlink DPCH info for each RL

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Primary CPICH usage for channel estimation | MP |  | Primary CPICH usage for channel estimation 10.3.6.62 |  |  |
| >>DPCH frame offset | MP |  | Integer(0..38144 by step of 256) | Offset (in number of chips) between the beginning of the P-CCPCH frame and the beginning of the DPCH frame This is called DPCH,n in [26] |  |
| >>Secondary CPICH info | OP |  | Secondary CPICH info 10.3.6.73 |  |  |
| >>DL channelisation code | MP | 1 to <maxDPCH-DLchan> |  | For the purpose of physical channel mapping [27] the DPCHs are numbered, starting from DPCH number 1, according to the order that they are contained in this IE. |  |
| >>>Secondary scrambling code | MD |  | Secondary scrambling code 10.3.6.74 | Default is the same scrambling code as for the Primary CPICH |  |
| >>>Spreading factor | MP |  | Integer(4, 8, 16, 32, 64, 128, 256, 512) | Defined in CHOICE SF512-AndCodenumber with "code number" in ASN.1. |  |
| >>>Code number | MP |  | Integer(0..Spreading factor - 1) |  |  |
| >>>Scrambling code change | CH-SF/2 |  | Enumerated (code change, no code change) | Indicates whether the alternative scrambling code is used for compressed mode method 'SF/2'. |  |
| >>TPC combination index | MP |  | TPC combination index 10.3.6.85 |  |  |
| >>Power offset PTPC-DPDCH | OP |  | Integer (0..24) | Power offset equals PTPC- PDPDCH, range 0..6 dB, in steps of 0.25 dB | REL-5 |
| >>Power offset POSRB | OP |  | Integer (0..24) | Power offset to be applied under conditions defined in [29]. Range 0..6dB in steps of 0.25dB. | REL-12 |
| >>Closed loop timing adjustment mode | CH-TxDiversity Mode |  | Integer(1, 2) | It is present if Tx Diversity is used in the radio link. |  |
| >1.28 Mcps TDD or 3.84 Mcps TDD |  |  |  |  |  |
| >>DL CCTrCh List | OP | 1..<maxCCTrCH> |  | DL physical channels to establish or reconfigure list. |  |
| >>>TFCS ID | MD |  | Integer(1..8) | Identity of this CCTrCh. Default value is 1 |  |
| >>>Time info | MP |  | Time Info 10.3.6.83 |  |  |
| >>>Common timeslot info | MD |  | Common Timeslot Info 10.3.6.10 | Default is the current Common timeslot info |  |
| >>>Downlink DPCH timeslots and codes | MD |  | Downlink Timeslots and Codes 10.3.6.32 | Default is to use the old timeslots and codes. |  |
| >>>UL CCTrCH TPC List | MD | 0..<maxCCTrCH> |  | UL CCTrCH identities for TPC commands associated with this DL CCTrCH. Default is previous list or all defined UL CCTrCHs. This list is not required for 1.28 Mcps TDD and is to be ignored by the UE. |  |
| >>>>UL TPC TFCS Identity | MP |  | Transport Format Combination Set Identity 10.3.5.21 |  |  |
| >>DL CCTrCH List to Remove | OP | 1..<maxCCTrCH> |  | DL physical channels to remove list. |  |
| >>>TFCS ID | MP |  | Integer(1..8) |  |  |
| >7.68 Mcps TDD | CV-Message |  |  |  | REL-7 |
| >>DL CCTrCh List | OP | 1..<maxCCTrCH> |  | DL physical channels to establish or reconfigure list. | REL-7 |
| >>>TFCS ID | MD |  | Integer(1..8) | Identity of this CCTrCh. Default value is 1 | REL-7 |
| >>>Time info | MP |  | Time Info 10.3.6.83 |  | REL-7 |
| >>>Common timeslot info | MD |  | Common Timeslot Info 10.3.6.10 | Default is the current Common timeslot info | REL-7 |
| >>>Downlink DPCH timeslots and codes VHCR | MD |  | Downlink Timeslots and Codes VHCR 10.3.6.32a | Default is to use the old timeslots and codes. | REL-7 |
| >>>UL CCTrCH TPC List | MD | 0..<maxCCTrCH> |  | UL CCTrCH identities for TPC commands associated with this DL CCTrCH. Default is previous list or all defined UL CCTrCHs. This list is not required for 1.28 Mcps TDD and is to be ignored by the UE. | REL-7 |
| >>>>UL TPC TFCS Identity | MP |  | Transport Format Combination Set Identity 10.3.5.21 |  | REL-7 |
| >>DL CCTrCH List to Remove | OP | 1..<maxCCTrCH> |  | DL physical channels to remove list. | REL-7 |
| >>>TFCS ID | MP |  | Integer(1..8) |  | REL-7 |

| Condition | Explanation |
| --- | --- |
| SF/2 | The information element is mandatory present if the UE has a compressed mode pattern sequence configured in variable TGPS_IDENTITY or included in the message including IE "Downlink DPCH info for each RL", which is using compressed mode method "SF/2". Otherwise the IE is not needed. |
| TxDiversity Mode | This IE is mandatory present if any TX Diversity Mode is used on the radio link, i.e. if STTD or "closed loop mode 1" is used on the radio link. Otherwise the IE is not needed. |
| Message | This CHOICE is not needed in the ACTIVE SET UPDATE message, otherwise it is optional. |

#### 10.3.6.22	Downlink DPCH info for each RL Post

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |
| >FDD |  |  |  |  |
| >>Primary CPICH usage for channel estimation | MP |  | Primary CPICH usage for channel estimation 10.3.6.62 |  |
| >>Secondary scrambling code | MD |  | Secondary scrambling code 10.3.6.74 | Default is the same scrambling code as for the Primary CPICH |
| >>CHOICE Spreading factor | MP |  | Integer(4, 8, 16, 32, 64, 128, 256, 512) | Defined in CHOICE SF512-AndCodenumber with "code number" in ASN.1. |
| >>Code number | MP |  | Integer(0.. Spreading factor - 1) |  |
| >>Scrambling code change | CH-SF/2 |  | Enumerated (code change, no code change) | Indicates whether the alternative scrambling code is used for compressed mode method 'SF/2'. |
| >>TPC combination index | MP |  | TPC combination index 10.3.6.85 |  |
| >1.28 Mcps TDD or 3.84 Mcps TDD |  |  |  |  |
| >>Downlink DPCH timeslots and codes | MP |  | Downlink Timeslots and Codes 10.3.6.32 |  |
| >7.68 Mcps TDD |  |  |  | REL-7 |
| >>Downlink DPCH timeslots and codes VHCR | MP |  | Downlink Timeslots and Codes VHCR 10.3.6.32a | REL-7 |

| Condition | Explanation |
| --- | --- |
| SF/2 | The information element is mandatory present if the UE has a compressed mode pattern sequence configured in variable TGPS_IDENTITY or included in the message including IE "Downlink DPCH info for each RL Post", which is using compressed mode method "SF/2". Otherwise the IE is not needed. |

#### 10.3.6.23	Downlink DPCH power control information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |
| >FDD |  |  |  |  |
| >>DPC Mode | MP |  | Enumerated (Single TPC, TPC triplet in soft) | "Single TPC" is DPC_Mode=0 and "TPC triplet in soft" is DPC_mode=1 in [29]. |
| >TDD |  |  |  |  |
| >>TPC Step Size | OP |  | Integer (1, 2, 3) | In dB |

#### 10.3.6.23oa	Downlink F-DPCH info common for all RL

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Timing Indication | MP |  | Enumerated (Initialise, Maintain) |  | REL-6 |
| Timing maintained Synchronization indicator | CV-Synch |  | Enumerated (FALSE) | FALSE indicates that no synchronisation procedure shall be performed for timing maintained inter-frequency hard handover [29]. Absence of this element means that the synchronization procedure A shall be used. | REL-6 |
| Downlink F-DPCH power control information | OP |  | Downlink DPCH power control information 10.3.6.23 |  | REL-6 |
| TPC command error rate target | MP |  | Real (0.01..0.1 by step of 0.01) |  | REL-6 |

| Condition | Explanation |
| --- | --- |
| Synch | The IE is not needed in the CELL UPDATE CONFIRM, HANDOVER TO UTRAN COMMAND and the RRC CONNECTION SETUP messages or if the IE "Timing Indication" is set to "Initialise" or if the IE “Frequency Info” is not included. Otherwise, it is optional. |

#### 10.3.6.23ob	Downlink F-DPCH info for each RL

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Primary CPICH usage for channel estimation | MP |  | Primary CPICH usage for channel estimation 10.3.6.62 |  | REL-6 |
| F-DPCH frame offset | MP |  | Integer (0..38144 by step of 256) | Offset (in number of chips) between the beginning of the P-CCPCH frame and the beginning of the F-DPCH frame This is called F-DPCH,n in [26] | REL-6 |
| F-DPCH slot format | OP |  | Integer (0..9 by steps of 1) | Slot format used by F-DPCH in [26]. Absence of this element means that slot format 0 is applied. | REL-7 |
| Secondary CPICH info | OP |  | Secondary CPICH info 10.3.6.73 |  | REL-6 |
| Secondary scrambling code | MD |  | Secondary scrambling code 10.3.6.74 | Default is the same scrambling code as for the Primary CPICH | REL-6 |
| Code number | MP |  | Integer (0..255) |  | REL-6 |
| TPC combination index | MP |  | TPC combination index 10.3.6.85 |  | REL-6 |
| STTD indication | CV-NoHOtoUTRAN |  | Enumerated (TRUE) | This IE shall be set to TRUE when STTD is used. Absence of this element means STTD is not used. | REL-6 |
| Power Control Algorithm 3 | CV-Message |  | Power Control Algorithm 3 10.3.6.153 |  | REL-13 |

| Condition | Explanation |
| --- | --- |
| NoHOtoUTRAN | This IE is not needed in the HANDOVER TO UTRAN COMMAND and it is optional in all the other messages in which the IE "Downlink F-DPCH info for each RL" can be included. |
| Messsage | This IE is not needed in the HANDOVER TO UTRAN COMMAND and RRC CONNECTION SETUP messages, and it is optional in all the other messages in which the IE "Downlink F-DPCH info for each RL" can be included. |

#### 10.3.6.23a	Downlink HS-PDSCH Information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| HS-SCCH Info | OP |  | HS-SCCH Info 10.3.6.36a | For 1.28 Mcps TDD, if IE DL Multi-Carrier Information is present, ignore this IE. | REL-5 |
| Measurement Feedback Info | OP |  | Measurement Feedback Info 10.3.6.40a |  | REL-5 |
| CHOICE mode | MP |  |  |  | REL-5 |
| >TDD |  |  |  |  |  |
| >>CHOICE TDD option | MP |  |  |  | REL-5 |
| >>>3.84 Mcps |  |  |  |  |  |
| >>>>HS-PDSCH Timeslot Configuration | OP |  | HS-PDSCH Timeslot Configuration 10.3.6.36o |  | REL-5 |
| >>>7.68 Mcps |  |  |  |  | REL-7 |
| >>>>HS-PDSCH Timeslot Configuration VHCR | OP |  | HS-PDSCH Timeslot Configuration VHCR 10.3.6.36oa |  | REL-7 |
| >>> 1.28 Mcps |  |  |  |  |  |
| >>>> HS-PDSCH Midamble Configuration | OP |  | HS-PDSCH Midamble Configuration 10.3.6.36oo | For 1.28 Mcps TDD, if IE DL Multi-Carrier Information is present, ignore this IE. | REL-5 |
| >>>> DL Multi-Carrier Information | OP |  | DL Multi-Carrier Information 10.3.6.28a |  | REL-7 |
| >>>> Out of Sync window | MD |  | Enumerated (40, 80, 160, 320, 640) | Value in milliseconds. Default value is 160. Three spare values needed. | REL-8 |
| >>>>TS0 Indicator | OP |  | Enumerated (TRUE) | Absence of this IE means that the enhanced TS0 is not used. The presence of this IE means that the first bit of timeslot information on HS-SCCH is used to indicate TS0. | REL-9 |
| >>>>TS0 channelization codes | CV-TS0Non-rectangularResourceAllocationSpecificTimeslots |  | Downlink channelisation codes 10.3.6.17 | For 1.28Mcps TDD only. This IE indicates the channelization codes in TS0 that are configured for HS-PDSCH channel. | REL-12 |
| >FDD |  |  |  |  |  |
| >>Downlink 64QAM configured | OP |  | Enumerated (TRUE) | Absence of this IE means that the HS-SCCH does not use the 64QAM format. The presence of this IE means the UE uses the octet aligned table [15]. | REL-7 |
| >>HS-DSCH TB size table | CV-Not64QAM |  | Enumerated (octet aligned) | If this IE is present, octet aligned table [15] is used, else bit aligned table [15] is used. NOTE 1 | REL-7 |
| NOTE 1: This rule applies only if the IE "Downlink HS-PDSCH information" is present in the received message. The UE behaviour is unspecified if the IE "HS-DSCH TB size table" is present in the corresponding ASN.1 non-critical extension when the IE "Downlink HS-PDSCH information" is absent in the received message. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Not64QAM | This IE is optionally present if 64QAM is not configured and MAC-ehs is configured. Otherwise it is not needed. |
| TS0Non-rectangularResourceAllocationSpecificTimeslots | In messages other than HANDOVER TO UTRAN COMMAND, this IE is mandatory present if both of non-rectangular resource allocation indicator and TS0 indicator are configured and non-rectangular resource specific timeslots set is not configured. Otherwise, it is not needed. |

#### 10.3.6.24	Downlink information common for all radio links

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE DPCH info | OP |  |  |  | REL-6 |
| >Downlink DPCH info common for all RL | MP |  | Downlink DPCH info common for all RL 10.3.6.18 |  |  |
| >Downlink F-DPCH info common for all RL | MP |  | Downlink F-DPCH info common for all RL 10.3.6.23oa |  | REL-6 |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>DPCH compressed mode info | OP |  | DPCH compressed mode info 10.3.6.33 |  |  |
| >>TX Diversity Mode | MD |  | TX Diversity Mode 10.3.6.86 | Default value is the existing value of TX Diversity mode |  |
| >TDD |  |  |  | (no data) |  |
| >>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>3.84 Mcps TDD |  |  |  | (no data) | REL-4 |
| >>>7.68 Mcps TDD |  |  |  | (no data) | REL-7 |
| >>>1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>>>TSTD indicator | MP |  | TSTD indicator 10.3.6.85a |  | REL-4 |
| Default DPCH Offset Value | OP |  | Default DPCH Offset Value, 10.3.6.16 |  |  |
| MAC-hs reset indicator | CV-messageType |  | Enumerated (TRUE) | TRUE Indicates the MAC-hs/ehs entity needs to be reset. | REL-5 |
| Post-verification period | OP |  | Enumerated (TRUE) | TRUE indicates that a post-verification period shall be used [29]. Absence of this element means that a post-verification period shall not be used. | REL-6 |
| MAC-hs reset indicator for Multiflow assisting cells | CV-messageType |  | Enumerated (TRUE) | TRUE indicates that the MAC-hs/ehs entity related to the assisting cells needs to be reset. | REL-11 |

| Condition | Explanation |
| --- | --- |
| MessageType | The IE is not needed in the HANDOVER TO UTRAN COMMAND and the RRC CONNECTION SETUP messages. Otherwise, it is optional. |

#### 10.3.6.25	Downlink information common for all radio links Post

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Downlink DPCH info common for all RL | MP |  | Downlink DPCH info common for all RL Post 10.3.6.19 |  |

#### 10.3.6.26	Downlink information common for all radio links Pre

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Downlink DPCH info common for all RL | MP |  | Downlink DPCH info common for all RL Pre 10.3.6.20 |  |

#### 10.3.6.27	Downlink information for each radio link

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  |  |
| >>Serving HS-DSCH radio link indicator | MP |  | Boolean | The value TRUE indicates that this radio link is the serving HS-DSCH radio link | REL-5 |
| >>Serving E-DCH radio link indicator | MP |  | Boolean | The value TRUE indicates that this radio link is the serving E-DCH radio link | REL-6 |
| >TDD |  |  |  |  |  |
| >>Primary CCPCH info | MP |  | Primary CCPCH info 10.3.6.57 |  |  |
| Cell ID | OP |  | Cell ID 10.3.2.2 |  | REL-4 |
| CHOICE DPCH info | OP |  |  |  | REL-6 |
| >Downlink DPCH info for each RL |  |  | Downlink DPCH info for each RL 10.3.6.21 |  |  |
| >Downlink F-DPCH info for each RL |  |  | Downlink F-DPCH info for each RL 10.3.6.23ob |  | REL-6 |
| >Radio Links without DPCH/F-DPCH info |  |  | Radio Links without DPCH/F-DPCH info 10.3.6.147 |  | REL-12 |
| E-AGCH Info | OP |  | E-AGCH Info 10.3.6.100 |  | REL-6 |
| E-ROCH Info | OP |  | E-ROCH Info FDD 10.3.6.146 |  | REL-11 |
| CHOICE mode |  |  |  |  | REL-7 |
| >FDD |  |  |  |  | REL-7 |
| >>CHOICE E-HICH Information | OP |  |  |  | REL-6 |
| >>>E-HICH Information | MP |  | E-HICH Info 10.3.6.101 |  | REL-6 |
| >>>E-HICH release indicator |  |  |  | (no data) | REL-6 |
| >>>Secondary TB E-HICH release indicator |  |  |  | (no data) | REL-11 |
| >>CHOICE E-RGCH Information | OP |  |  |  | REL-6 |
| >>>E-RGCH Information | MP |  | E-RGCH Info 10.3.6.102 |  | REL-6 |
| >>>E-RGCH release indicator |  |  |  | (no data) | REL-6 |
| >>CHOICE F-TPICH Info | OP |  |  |  | REL-11 |
| >>>F-TPICH Info |  |  | F-TPICH Info 10.3.6.127 |  | REL-11 |
| >>>F-TPICH release indicator |  |  |  | (no data) | REL-11 |
| >TDD |  |  |  | (no data) | REL-7 |
| >>E-HICH Information | OP |  | E-HICH Info 10.3.6.101 |  | REL-7 |

#### 10.3.6.28	Downlink information for each radio link Post

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Choice mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  |  |
| >TDD |  |  |  |  |  |
| >>Primary CCPCH info | MP |  | Primary CCPCH info post 10.3.6.58 |  |  |
| Downlink DPCH info for each RL | MP |  | Downlink DPCH info for each RL Post 10.3.6.22 |  |  |

#### 10.3.6.28a	DL Multi-Carrier Information (1.28 Mcps TDD only)

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| TSN-Length | OP |  | Enumerated (tsn-6bits, tsn-9bits) |  | REL-7 |
| Multi-Carrier number | OP |  | Integer(1..maxTDD128Carrier) |  | REL-7 |
| Dl-HSPDSCH-MultiCarrier-Information | OP | 1 to <maxTDD128Carrier> |  | The number of HS-SCCH Sets shall not be larger than UE Multi-Carrier capability | REL-7 |
| >Carrier-Uarfcn | MP |  | Integer(0..16383) |  | REL-7 |
| >HARQ Info | OP |  | HARQ info 10.3.5.7a |  | REL-7 |
| >HS-PDSCH Midamble Configuration | OP |  | HS-PDSCH Midamble Configuration 10.3.6.36oo |  | REL-7 |
| >HS-SCCH Set Configuration | OP | 1 to <maxHS-SCCHs> |  |  | REL-7 |
| >>HS-SCCH-Receive-Uarfcn | MP |  | Integer(0..16383) |  | REL-7 |
| >>Timeslot number | MP |  | Integer (0..6) |  | REL-7 |
| >>First Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-7 |
| >>Second Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-7 |
| >>Midamble Allocation mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) |  | REL-7 |
| >>Midamble Shift | CV-UE |  | Integer (0..15) |  | REL-7 |
| >>Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) |  | REL-7 |
| >>BLER target | MP |  | Real (-3.15..0 by step of 0.05) | Signalled value is Log10(HS-SCCH BLER quality target). The UE shall use the BLER target signalled in the first occurrence of the HS-SCCH Set Configuration. | REL-7 |
| >>HS-SICH configuration |  |  |  |  | REL-7 |
| >>>Timeslot number | MP |  | Integer (0..6) |  | REL-7 |
| >>>Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-7 |
| >>>Midamble Allocation mode | MP |  | Enumerated (Default midamble, UE specific midamble) |  | REL-7 |
| >>>Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) |  | REL-7 |
| >>>Midamble Shift | CV-UE |  | Integer (0..15) |  | REL-7 |
| >>>Ack-Nack Power Offset | MP |  | Integer (-7..8 by step of 1) | dB. | REL-7 |
| >>>PRXHS-SICH | MP |  | Integer (-120..-58 by step of 1) | dBm. Desired power level for HS-SICH. | REL-7 |
| >>>TPC step size | MP |  | Integer (1, 2, 3) | dB. | REL-7 |

| Condition | Explanation |
| --- | --- |
| UE | This IE is mandatory present when the value of the IE "Midamble Allocation Mode" is "UE specific midamble" and not needed otherwise. |

#### 10.3.6.29	Void

#### 10.3.6.30	Void

#### 10.3.6.31	Downlink rate matching restriction information

This IE indicates which TrCH is restricted in TFI.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Restricted TrCH information | OP | 1 to <maxTrCH> |  |  |
| >Downlink transport channel type | MP |  | Enumerated(DCH) |  |
| >Restricted DL TrCH identity | MP |  | Transport channel identity 10.3.5.18 |  |
| >Allowed TFIs | MP | 1 to <maxTF> |  |  |
| >>Allowed TFI | MP |  | Integer(0..31) |  |

#### 10.3.6.31a	Downlink secondary cell info FDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Configuration info | MP |  |  |  | REL-8 |
| >Continue |  |  | (no data) | Used in reconfigurations without interruption of dual cell operation. | REL-8 |
| >New configuration |  |  |  |  | REL-8 |
| >>New H-RNTI | MP |  | H-RNTI 10.3.3.14a |  | REL-8 |
| >>Downlink 64QAM configured | OP |  | Enumerated (TRUE) | Absence of this IE means that the secondary cell HS-SCCH does not use the 64QAM format. The presence of this IE means the UE uses the octet aligned table [15]. | REL-8 |
| >>HS-DSCH TB size table | CV-Not64QAM |  | Enumerated (octet aligned) | If this IE is present, octet aligned table [15] is used, else bit aligned table [15] is used. | REL-8 |
| >>Primary CPICH info | MP |  | Primary CPICH Info 10.3.6.60 |  | REL-8 |
| >>DL Scrambling Code | MD |  | Secondary scrambling code 10.3.6.74 | DL Scrambling code to be applied for HS-DSCH and HS-SCCH. Default is same scrambling code as for the primary CPICH. | REL-8 |
| >>HS-SCCH Channelisation Code Information | MP | 1 to <maxHSSCCHs > |  | Note 1 | REL-8 |
| >>>HS-SCCH Channelisation Code | MP |  | Integer (0..127) |  | REL-8 |
| >>Measurement Power Offset | MP |  | Real (-6 .. 13 by step of 0.5) | The measurement power offset, , in dB, as described in [29]. | REL-8 |
| >>Measurement Power Offset_1 | OP |  | Real (-6 .. 13 by step of 0.5) | The measurement power offset, 1, in dB, as described in [29]. | REL-11 |
| >>UARFCN downlink (Nd) | MP |  | Integer(0 .. 16383) | [21] | REL-8 |
| >>Different Tx diversity mode configuration from serving HS-DSCH cell | OP |  | Enumerated (Different) | The presence of this IE means that the Tx diversity Mode is different to the Diversity Mode indicated in the serving HS-DSCH cell. Absence of this IE means that the same Tx diversity mode is used as that used for the serving HS-DSCH cell. Note 2 | REL-8 |
| >>Multiflow configuration | OP |  | 10.3.6.129 Multiflow configuration info | If present, provides the Multiflow specific configuration information for this cell. | REL-11 |
| Secondary cell MIMO parameters | OP |  | 10.3.6.72b Secondary cell MIMO parameters | FDD only | REL-9 |
| Secondary cell MIMO mode with four transmit antennas parameters | OP |  | Secondary cell MIMO mode with four transmit antennas parameters 10.3.6.144 | FDD only | REL-11 |
| Note 1: The list of HS-SCCH(s) is assumed to be indexed starting from one. |  |  |  |  |  |
| Note 2: The UE behaviour is unspecified if the IE "Different Tx diversity mode configuration from serving HS-DSCH cell" is present and the IE "CHOICE Configuration info" is not set to "New configuration" in the received message. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Not64QAM | This IE is optionally present if 64QAM is not configured and MAC-ehs is configured. Otherwise it is not needed. |

#### 10.3.6.31b	Downlink secondary cell info FDD for Handover to UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Configuration info | MP |  |  |  | REL-11 |
| >Continue |  |  | (no data) | Note 1. | REL-11 |
| >New configuration |  |  |  |  | REL-11 |
| >>New H-RNTI | MP |  | H-RNTI 10.3.3.14a |  | REL-11 |
| >>Primary CPICH info | MP |  | Primary CPICH Info 10.3.6.60 |  | REL-11 |
| >>DL Scrambling Code | MD |  | Secondary scrambling code 10.3.6.74 | DL Scrambling code to be applied for HS-DSCH and HS-SCCH. Default is same scrambling code as for the primary CPICH. | REL-11 |
| >>HS-SCCH Channelisation Code Information | MP | 1 to <maxHSSCCHs > |  | Note 2 | REL-11 |
| >>>HS-SCCH Channelisation Code | MP |  | Integer (0..127) |  | REL-11 |
| >>Measurement Power Offset | MP |  | Real (-6 .. 13 by step of 0.5) | The measurement power offset, , in dB, as described in [29]. | REL-11 |
| >>UARFCN downlink (Nd) | MP |  | Integer(0 .. 16383) | [21] | REL-11 |
| >>Different Tx diversity mode configuration from serving HS-DSCH cell | OP |  | Enumerated (Different) | The presence of this IE means that the Tx diversity Mode is different to the Diversity Mode indicated in the serving HS-DSCH cell. Absence of this IE means that the same Tx diversity mode is used as that used for the serving HS-DSCH cell. Note 3 | REL-11 |
| Note 1: The value "Continue" should not be set. |  |  |  |  |  |
| Note 2: The list of HS-SCCH(s) is assumed to be indexed starting from one. |  |  |  |  |  |
| Note 3: The UE behaviour is unspecified if the IE "Different Tx diversity mode configuration from serving HS-DSCH cell" is present and the IE "CHOICE Configuration info" is not set to "New configuration" in the received message. |  |  |  |  |  |

#### 10.3.6.32	Downlink Timeslots and Codes

NOTE:	Only for 1.28 Mcps TDD and 3.84 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| First Individual timeslot info | MP |  | Individual timeslot info 10.3.6.37 | Individual timeslot info for the first timeslot used by the physical layer. |
| First timeslot channelisation codes | MP |  | Downlink channelisation codes 10.3.6.17 | These codes shall be used by the physical layer in the timeslot given in First Individual timeslot info. |
| CHOICE more timeslots | MP |  |  |  |
| >No more timeslots |  |  |  | (no data) |
| >Consecutive timeslots |  |  |  |  |
| >>Number of additional timeslots | MP |  | Integer(1..maxTS-1) | The timeslots used by the physical layer shall be timeslots: N mod maxTS (N+1) mod maxTS ... (N+k) mod maxTS in that order, where N is the timeslot number in the First individual timeslot info and k the Number of additional timeslots. The additional timeslots shall use the same parameters (e.g. channelisation codes, midamble shifts etc.) as the first timeslot. |
| >Timeslot list |  |  |  |  |
| >>Additional timeslot list | MP | 1 to <maxTS-1> |  | The first instance of this parameter corresponds to the timeslot that shall be used second by the physical layer, the second to the timeslot that shall be used third and so on. |
| >>>CHOICE parameters | MP |  |  |  |
| >>>>Same as last |  |  |  |  |
| >>>>>Timeslot number | MP |  | Timeslot Number 10.3.6.84 | The physical layer shall use the same parameters (e.g. channelisation codes, midamble shifts etc.) for this timeslot as for the last one. |
| >>>>New parameters |  |  |  |  |
| >>>>>Individual timeslot info | MP |  | Individual timeslot info 10.3.6.37 |  |
| >>>>>Channelisation codes | MP |  | Downlink channelisation codes 10.3.6.17 |  |

#### 10.3.6.32a	Downlink Timeslots and Codes VHCR

NOTE:	Only for 7.68 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| First Individual timeslot info | MP |  | Individual timeslot info 10.3.6.37 | Individual timeslot info for the first timeslot used by the physical layer. | REL-7 |
| First timeslot channelisation codes VHCR | MP |  | Downlink channelisation codes VHCR 10.3.6.17a | These codes shall be used by the physical layer in the timeslot given in First Individual timeslot info. | REL-7 |
| CHOICE more timeslots | MP |  |  |  | REL-7 |
| >No more timeslots |  |  |  | (no data) | REL-7 |
| >Consecutive timeslots |  |  |  |  | REL-7 |
| >>Number of additional timeslots | MP |  | Integer(1..maxTS-1) | The timeslots used by the physical layer shall be timeslots: N mod maxTS (N+1) mod maxTS ... (N+k) mod maxTS in that order, where N is the timeslot number in the First individual timeslot info and k the Number of additional timeslots. The additional timeslots shall use the same parameters (e.g. channelisation codes, midamble shifts etc.) as the first timeslot. | REL-7 |
| >Timeslot list |  |  |  |  | REL-7 |
| >>Additional timeslot list | MP | 1 to <maxTS-1> |  | The first instance of this parameter corresponds to the timeslot that shall be used second by the physical layer, the second to the timeslot that shall be used third and so on. | REL-7 |
| >>>CHOICE parameters | MP |  |  |  | REL-7 |
| >>>>Same as last |  |  |  |  | REL-7 |
| >>>>>Timeslot number | MP |  | Timeslot Number 10.3.6.84 | The physical layer shall use the same parameters (e.g. channelisation codes, midamble shifts etc.) for this timeslot as for the last one. | REL-7 |
| >>>>New parameters |  |  |  |  | REL-7 |
| >>>>>Individual timeslot info | MP |  | Individual timeslot info 10.3.6.37 |  | REL-7 |
| >>>>>Channelisation codes VHCR | MP |  | Downlink channelisation codes VHCR 10.3.6.17a |  | REL-7 |

#### 10.3.6.33	DPCH compressed mode info

NOTE:	Only for FDD.

This information element indicates the parameters of the compressed mode to be used by the UE in order to perform inter-frequency and inter-RAT measurements.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Transmission gap pattern sequence | MP | 1 to <maxTGPS> |  |  |  |
| >TGPSI | MP |  | TGPSI 10.3.6.82 |  |  |
| >TGPS Status Flag | MP |  | Enumerated(activate, deactivate) | This flag indicates whether the Transmission Gap Pattern Sequence shall be activated or deactivated. |  |
| >TGCFN | CV-Active |  | Integer (0..255) | Connection Frame Number of the first frame of the first pattern within the Transmission Gap Pattern Sequence. |  |
| >Transmission gap pattern sequence configuration parameters | OP |  |  |  |  |
| >>TGMP | MP |  | Enumerated(TDD measurement, FDD measurement, GSM carrier RSSI measurement, GSM Initial BSIC identification, GSM BSIC re-confirmation, Multi-carrier measurement, | Transmission Gap pattern sequence Measurement Purpose. |  |
|  |  |  | E-UTRA measurement) | One spare value is needed | REL-8 |
| >>Frequency specific compressed mode | CV-Active-FDD |  | Enumerated (TRUE) | TRUE means that the frequency specific compressed mode is configured. | REL-10 |
| >>TGPRC | MP |  | Integer (1..511, Infinity) | The number of transmission gap patterns within the Transmission Gap Pattern Sequence. |  |
| >>TGSN | MP |  | Integer (0..14) | Transmission Gap Starting Slot Number The slot number of the first transmission gap slot within the TGCFN. |  |
| >>TGL1 | MP |  | Integer(1..14) | The length of the first Transmission Gap within the transmission gap pattern expressed in number of slots |  |
| >>TGL2 | MD |  | Integer (1..14) | The length of the second Transmission Gap within the transmission gap pattern. If omitted, then TGL2=TGL1. The value of TGL2 shall be ignored if TGD is set to "undefined" |  |
| >>TGD | MP |  | Integer(15..269, undefined) | Transmission gap distance indicates the number of slots between starting slots of two consecutive transmission gaps within a transmission gap pattern. If there is only one transmission gap in the transmission gap pattern, this parameter shall be set to undefined. |  |
| >>TGPL1 | MP |  | Integer (1..144) | The duration of transmission gap pattern 1. |  |
| >>RPP | MP |  | Enumerated (mode 0, mode 1). | Recovery Period Power control mode during the frame after the transmission gap within the compressed frame. Indicates whether normal PC mode or compressed PC mode is applied |  |
| >>ITP | MP |  | Enumerated (mode 0, mode 1). | Initial Transmit Power is the uplink power control method to be used to compute the initial transmit power after the compressed mode gap. |  |
| >>CHOICE UL/DL mode | MP |  |  |  |  |
| >>>DL only |  |  |  | Compressed mode used in DL only |  |
| >>>>Downlink compressed mode method | MP |  | Enumerated (SF/2, higher layer scheduling) | Method for generating downlink compressed mode gap. If F-DPCH is configured this IE shall not be used by the UE |  |
| >>>UL only |  |  |  | Compressed mode used in UL only |  |
| >>>>Uplink compressed mode method | MP |  | Enumerated (SF/2, higher layer scheduling) | Method for generating uplink compressed mode gap |  |
| >>>UL and DL |  |  |  | Compressed mode used in UL and DL |  |
| >>>>Downlink compressed mode method | MP |  | Enumerated (SF/2, higher layer scheduling) | Method for generating downlink compressed mode gap. If F-DPCH is configured this IE shall not be used by the UE |  |
| >>>>Uplink compressed mode method | MP |  | Enumerated (SF/2, higher layer scheduling) | Method for generating uplink compressed mode gap |  |
| >>Downlink frame type | MP |  | Enumerated (A, B) | If F-DPCH is configured this IE shall not be used by the UE |  |
| >>DeltaSIR1 | MP |  | Real(0..3 by step of 0.1) | Delta in DL SIR target value to be set in the UE during the frame containing the start of the first transmission gap in the transmission gap pattern (without including the effect of the bit-rate increase). If F-DPCH is configured this IE shall not be used by the UE |  |
| >>DeltaSIRafter1 | MP |  | Real(0..3 by step of 0.1) | Delta in DL SIR target value to be set in the UE one frame after the frame containing the start of the first transmission gap in the transmission gap pattern. If F-DPCH is configured this IE shall not be used by the UE |  |
| >>DeltaSIR2 | OP |  | Real(0..3 by step of 0.1) | Delta in DL SIR target value to be set in the UE during the frame containing the start of the second transmission gap in the transmission gap pattern (without including the effect of the bit-rate increase) When omitted, DeltaSIR2 = DeltaSIR1. If F-DPCH is configured this IE shall not be used by the UE |  |
| >>DeltaSIRafter2 | OP |  | Real(0..3 by step of 0.1) | Delta in DL SIR target value to be set in the UE one frame after the frame containing the start of the second transmission gap in the transmission gap pattern. When omitted, DeltaSIRafter2 = DeltaSIRafter1. If F-DPCH is configured this IE shall not be used by the UE |  |
| >>N Identify abort | CV-Initial BSIC |  | Integer(1..128) | Indicates the maximum number of repeats of patterns that the UE shall use to attempt to decode the unknown BSIC of the GSM cell in the initial BSIC identification procedure |  |
| >>T Reconfirm abort | CV-Re-confirm BSIC |  | Real(0.5..10.0 by step of 0.5) | Indicates the maximum time allowed for the re-confirmation of the BSIC of one GSM cell in the BSIC re-confirmation procedure. The time is given in steps of 0.5 seconds. |  |

| Condition | Explanation |
| --- | --- |
| Active | This IE is mandatory present when the value of the IE "TGPS Status Flag" is "Activate" and not needed otherwise. |
| Active-FDD | This IE can be included only for FDD measurements. It is optional present when the value of the IE "TGPS Status Flag" is "Activate" and not needed otherwise. |
| Initial BSIC | This IE is mandatory present when the value of the IE "TGMP" is set to "GSM Initial BSIC identification" and not needed otherwise. |
| Re-confirm BSIC | This IE is mandatory present when the value of the IE "TGMP" is set to "GSM BSIC re-confirmation" and not needed otherwise. |

#### 10.3.6.34	DPCH Compressed Mode Status Info

This information element indicates status information of the compressed mode used by the UE in order to perform inter-frequency and inter-RAT measurements.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TGPS reconfiguration CFN | MP |  | Integer (0..255) |  |
| Transmission gap pattern sequence | MP | 1 to <maxTGPS> |  |  |
| >TGPSI | MP |  | TGPSI 10.3.6.82 | Transmission Gap Pattern Sequence Identifier |
| >TGPS Status Flag | MP |  | Enumerated(activate, deactivate) | This flag indicates whether the Transmission Gap Pattern Sequence it shall be activated or deactivated. |
| >TGCFN | CV-Active |  | Integer (0..255) | Connection Frame Number of the first frame of the first pattern within the Transmission Gap Pattern Sequence. |
| >Frequency specific compressed mode | CV-Active-OP |  | Enumerated (TRUE) | TRUE means that the frequency specific compressed mode is configured. |

| Condition | Explanation |
| --- | --- |
| Active | This IE is mandatory present when the value of the IE "TGPS Status Flag" is "Activate" and not needed otherwise. |
| Active-OP | This IE is optional present when the value of the IE "TGPS Status Flag" is "Activate" and not needed otherwise. |

#### 10.3.6.34a	DTX-DRX information

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| DTX Information | OP |  |  |  | REL-7 |
| >CHOICE E-DCH TTI length | MP |  |  |  | REL-7 |
| >>10 ms |  |  |  |  | REL-7 |
| >>>UE DTX cycle 1 | MP |  | Enumerated (1, 5, 10, 20) | Units of subframes. | REL-7 |
| >>>UE DTX cycle 2 | MP |  | Enumerated (5, 10, 20, 40, 80, 160) | Units of subframes. Two spare values are needed. | REL-7 |
| >>>MAC DTX cycle | MP |  | Enumerated (5, 10, 20) | Units of subframes. One spare value is needed. | REL-7 |
| >>2 ms |  |  |  |  | REL-7 |
| >>>UE DTX cycle 1 | MP |  | Enumerated (1, 4, 5, 8, 10, 16, 20) | Units of subframes. One spare value is needed. | REL-7 |
| >>>UE DTX cycle 2 | MP |  | Enumerated (4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160) | Units of subframes. Four spare values are needed. | REL-7 |
| >>>MAC DTX cycle | MP |  | Enumerated (1, 4, 5, 8, 10, 16, 20) | Units of subframes. One spare value is needed. | REL-7 |
| >Inactivity Threshold for UE DTX cycle 2 | MP |  | Enumerated (1, 4, 8, 16, 32, 64, 128, 256) | Units of E-DCH TTIs. Eight spare values are needed. | REL-7 |
| >Default SG in DTX Cycle 2 | OP |  | Integer (0..37,38) | Serving Grant value to be used at the transition in DTX-Cycle-2. (0..37) indicates E-DCH serving grant index as defined in [15]; index 38 means zero grant. | REL-7 |
| >UE DTX long preamble length | MD |  | Enumerated (4, 15) | Units of slots Default value is 2 slots | REL-7 |
| >MAC Inactivity Threshold | MP |  | Enumerated (1, 2, 4, 8, 16, 32, 64, 128, 256, 512, Infinity) | Units of E-DCH TTIs. Five spare values are needed. | REL-7 |
| >CQI DTX Timer | MP |  | Enumerated (0, 1, 2, 4, 8, 16, 32, 64, 128, 256, 512, Infinity) | Units of subframes. Four spare values are needed. | REL-7 |
| >UE DPCCH burst_1 | MP |  | Enumerated (1, 2, 5) | Units of sub-frames. One spare value is needed. | REL-7 |
| >UE DPCCH burst_2 | MP |  | Enumerated (1, 2, 5) | Units of sub-frames. One spare value is needed. | REL-7 |
| DTX Information for secondary uplink frequency | OP |  |  |  | REL-12 |
| >CHOICE E-DCH TTI length | MP |  |  |  | REL-12 |
| >>10 ms |  |  | NULL |  | REL-12 |
| >>2 ms |  |  |  |  | REL-12 |
| >>>UE DTX cycle 1 secondary | MD |  | Enumerated (1, 4, 5, 8, 10, 16, 20) | Units of subframes. Default value is the value included in the IE "UE DTX cycle 1". One spare value is needed. | REL-12 |
| >>>UE DTX cycle 2 secondary | MP |  | Enumerated (4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280) | Units of subframes. Fourteen spare values are needed. | REL-12 |
| >Inactivity Threshold for UE DTX cycle 2 secondary | MD |  | Enumerated (1, 4, 8, 16, 32, 64, 128, 256) | Units of E-DCH TTIs. Default value is the value included in the IE "Inactivity Threshold for UE DTX cycle 2". Eight spare values are needed. | REL-12 |
| >UE DTX long preamble length secondary | MD |  | Enumerated (4, 15) | Units of slots. Default value is the value included in the IE "UE DTX long preamble length". | REL-12 |
| >UE DPCCH burst 1 secondary | MD |  | Enumerated (1, 2, 5) | Units of sub-frames. Default value is the value included in the IE "UE DPCCH burst_1". One spare value is needed. | REL-12 |
| >UE DPCCH burst 2 secondary | MD |  | Enumerated (1, 2, 5) | Units of sub-frames. Default value is the value included in the IE "UE DPCCH burst_2". One spare value is needed. | REL-12 |
| DRX Information | OP |  |  |  | REL-7 |
| >UE DRX cycle | MP |  | Enumerated (4, 5, 8, 10, 16, 20) | Units of subframes. Two spare values are needed. | REL-7 |
| >UE DRX cycle 2 | OP |  | Enumerated (4, 5, 8, 10, 16, 20, 32, 40, 64, 80) | Units of subframes. Six spare values are needed. | REL-12 |
| >Inactivity Threshold for UE DRX cycle | MP |  | Enumerated (0, 1, 2, 4, 8, 16, 32, 64, 128, 256, 512) | Units of subframes. Five spare values are needed. | REL-7 |
| >Inactivity Threshold for UE DRX cycle 2 | OP |  | Enumerated (0, 1, 2, 4, 8, 16, 32, 64, 128, 256, 512) | Units of subframes. Five spare values are needed. | REL-12 |
| >Inactivity Threshold for UE Grant Monitoring | MP |  | Enumerated (0, 1, 2, 4, 8, 16, 32, 64, 128, 256 ) | Units of E-DCH TTIs. Six spare values are needed. | REL-7 |
| >UE DRX Grant Monitoring | MP |  | Boolean |  | REL-7 |
| Uplink DPCCH slot format information | MP |  | Enumerated (1, 4) | Slot format # to be used on UL DPCCH [26]. One spare value is needed. | REL-7 |

#### 10.3.6.34b	DTX-DRX timing information

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE timing | MP |  |  |  | REL-7 |
| >Continue |  |  | (no data) | Used in reconfigurations without interruption of DTX-DRX and HS-SCCH less. | REL-7 |
| >New timing |  |  |  |  | REL-7 |
| >>Enabling Delay | MP |  | Enumerated (0, 1, 2, 4, 8, 16, 32, 64, 128) | In radio frames. Seven spare values are needed. | REL-7 |
| >>UE DTX DRX Offset | MP |  | Integer (0.. 159) | Units of subframes. Offset of the DTX and DRX cycles at the given TTI. | REL-7 |
| Note: Only the values of the IE “UE DTX DRX Offset” that fulfill the equation UE DTX DRX Offset mod 5=0 can be configured with 10ms E-DCH TTI. The UE behaviour is not specified if any other values are used with 10ms TTI. |  |  |  |  |  |

#### 10.3.6.35	Dynamic persistence level

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Dynamic persistence level | MP |  | Integer(1..8) | Level shall be mapped to a dynamic persistence value in the range 0 .. 1. The mapping is described in subclause 8.5.12. |

#### 10.3.6.35a	FPACH info

NOTE:	Only for 1.28 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Timeslot number | MP |  | Integer(0..6) |  | REL-4 |
| Channelisation code | MP |  | Enumerated((16/1)..(16/16) |  | REL-4 |
| Midamble Shift and burst type | MP |  | Midamble shift and burst type 10.3.6.41 |  | REL-4 |
| WT | MP |  | Integer(1..4) | The number of sub-frames, following the sub-frame in which the SYNC UL is transmitted, in which the FPACH can be transmitted. | REL-4 |

#### 10.3.6.35b	Frequency band indicator

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Frequency band indicator | MP |  | Enumerated(Band I, Band II, Band III, Band IV, Band V, Band VI, Band VII, extension indicator) |  | REL-6 |

#### 10.3.6.35c	Frequency band indicator 2

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Frequency band indicator 2 | MP |  | Enumerated(Band VIII, Band IX, Band X, Band XI, Band XII, Band XIII, Band XIV, Band XV, Band XVI, Band XVII, Band XVIII, Band XIX, Band XX, Band XXI, Band XXII, extension indicator) | Band XV – Band XVIII and Band XXII are yet to be defined in this version of the specification | REL-6 |

#### 10.3.6.35ca 	Frequency band indicator 3

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Frequency band indicator 3 | MP |  | Enumerated (Band XXV, Band XXVI, extension indicator) | Sixty One spare values are needed. | REL-10 |

#### 10.3.6.35d	Frequency band indicator for TDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Frequency band indicator | MP |  | Enumerated(Band a, Band b, Band c, Band d, Band e, Band f, reserved, reserved) |  | REL-10 |

#### 10.3.6.36	Frequency info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |
| >FDD |  |  |  |  |
| >>UARFCN uplink (Nu) | OP |  | Integer(0..16383) | If this IE is not present, the default duplex distance defined for the operating frequency band shall be used [21] |
| >>UARFCN downlink (Nd) | MP |  | Integer(0 .. 16383) | [21] |
| >TDD |  |  |  |  |
| >>UARFCN (Nt) | MP |  | Integer(0 .. 16383) | [22] |

#### 10.3.6.36oo	HS-PDSCH Midamble Configuration

NOTE:	Only for TDD 1.28 Mcps.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Midamble Allocation Mode | MP |  | Enumerated(Default midamble, Common midamble, UE specific midamble) | This midamble allocation mode applies to all HS-PDSCH resources assigned to the UE. | REL-5 |
| Midamble Configuration | MP |  | Integer(2, 4, 6, 8, 10, 12, 14, 16) | This configuration applies to all HS-PDSCH resources assigned to the UE. | REL-5 |
| Midamble Shift | CV-UE |  | Integer(0..15) | This shift, when present, applies to all HS-PDSCH resources assigned to the UE. | REL-5 |

| Condition | Explanation |
| --- | --- |
| UE | This IE is mandatory present when the value of the IE "Midamble Allocation Mode" is "UE specific midamble" and not needed otherwise. |

#### 10.3.6.36o	HS-PDSCH Timeslot Configuration

NOTE:	Only for TDD 3.84 Mcps.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| HS-PDSCH Timeslot Configuration List | MP | 1 to <maxTS-1> |  |  | REL-5 |
| >Timeslot Number | MP |  | Integer (0..14) |  | REL-5 |
| >CHOICE Burst Type | MP |  |  |  | REL-5 |
| >>Type 1 |  |  |  |  | REL-5 |
| >>>Midamble Allocation Mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) |  | REL-5 |
| >>>Midamble configuration burst type 1 and 3 | MP |  | Integer (4, 8, 16) | As defined in [30] | REL-5 |
| >>>Midamble Shift | CV-UE |  | Integer (0..15) |  | REL-5 |
| >>Type 2 |  |  |  |  | REL-5 |
| >>>Midamble Allocation Mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) |  | REL-5 |
| >>>Midamble configuration burst type 2 | MP |  | Integer (3, 6) | As defined in [30] | REL-5 |
| >>>Midamble Shift | CV-UE |  | Integer (0..5) |  | REL-5 |

| Condition | Explanation |
| --- | --- |
| UE | This IE is mandatory present when the value of the IE "Midamble Allocation Mode" is "UE specific midamble" and not needed otherwise. |

#### 10.3.6.36oa	HS-PDSCH Timeslot Configuration VHCR

NOTE:	Only for TDD 7.68 Mcps.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| HS-PDSCH Timeslot Configuration List | MP | 1 to <maxTS-1> |  |  | REL-7 |
| >Timeslot Number | MP |  | Integer (0..14) |  | REL-7 |
| >CHOICE Burst Type | MP |  |  |  | REL-7 |
| >>Type 1 |  |  |  |  | REL-7 |
| >>>Midamble Allocation Mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) |  | REL-7 |
| >>>Midamble configuration burst type 1 and 3 | MP |  | Integer (4, 8, 16) | As defined in [30] | REL-7 |
| >>>Midamble Shift | CV-UE |  | Integer (0..15) |  | REL-7 |
| >>Type 2 |  |  |  |  | REL-7 |
| >>>Midamble Allocation Mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) |  | REL-7 |
| >>>Midamble configuration burst type 2 | MP |  | Integer (4, 8) | As defined in [30] | REL-7 |
| >>>Midamble Shift | CV-UE |  | Integer (0..7) |  | REL-7 |

| Condition | Explanation |
| --- | --- |
| UE | This IE is mandatory present when the value of the IE "Midamble Allocation Mode" is "UE specific midamble" and not needed otherwise. |

#### 10.3.6.36a	HS-SCCH Info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  | REL-5 |
| >FDD |  |  |  |  | REL-5 |
| >>DL Scrambling Code | MD |  | Secondary scrambling code 10.3.6.74 | DL Scrambling code to be applied for HS-DSCH and HS-SCCH. Default is same scrambling code as for the primary CPICH. | REL-5 |
| >>HS-SCCH Channelisation Code Information | MP | 1 to <maxHSSCCHs > |  | Note 2 | REL-5 |
| >>>HS-SCCH Channelisation Code | MP |  | Integer (0..127) |  | REL-5 |
| >TDD |  |  |  |  | REL-5 |
| >>CHOICE TDD option | MP |  |  |  | REL-5 |
| >>>3.84 Mcps |  |  |  |  | REL-5 |
| >>>> Ack-Nack Power Offset | MP |  | Integer (-7..8 by step of 1) | dB Note 1 | REL-5 |
| >>>> HS-SICH Power Control Info | MP |  | HS-SICH Power Control Info 10.3.6.36b |  | REL-5 |
| >>>> BLER target | MP |  | Real (-3.15..0 by step of 0.05) | Signalled value is Log10(HS-SCCH BLER quality target).This IE is Not Present in REL-5. | REL-6 |
| >>>> Dhs-sync | OP |  | Integer (-20..+10) | Value in dB set to indicate the dB difference between the maximum allowed HS-SCCH physical channel transmit power [33] and the beacon reference power (it is one means of controlling the area of HS-DSCH operation within the cell). | REL-6 |
| >>>>HS-SCCH Set Configuration | MP | 1 to <maxHSSCCHs> |  |  | REL-5 |
| >>>>>Timeslot number | MP |  | Integer (0..14) |  | REL-5 |
| >>>>>Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-5 |
| >>>>>Midamble Allocation mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) | HS-SCCH always uses burst type 1. | REL-5 |
| >>>>>Midamble configuration | MP |  | Integer (4, 8, 16) |  | REL-5 |
| >>>>>Midamble Shift | CV-UE |  | Integer(0..15) |  | REL-5 |
| >>>>>BLER target | MP |  | Real (-3.15..0 by step of 0.05) | Signalled value is Log10(HS-SCCH BLER quality target). This IE is not present in REL-6 and beyond. | REL-5 |
| >>>>>HS-SICH configuration |  |  |  |  | REL-5 |
| >>>>>>Timeslot number | MP |  | Integer (0..14) |  | REL-5 |
| >>>>>>Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-5 |
| >>>>>>Midamble Allocation mode | MP |  | Enumerated (Default midamble, UE specific midamble) | HS-SICH always uses burst type 1. | REL-5 |
| >>>>>>Midamble configuration | MP |  | Integer (4, 8, 16) |  | REL-5 |
| >>>>>>Midamble Shift | CV-UE |  | Integer (0..15) |  | REL-5 |
| >>>7.68 Mcps |  |  |  |  | REL-7 |
| >>>>Ack-Nack Power Offset | MP |  | Integer (-7..8 by step of 1) | dB | REL-7 |
| >>>>HS-SICH Power Control Info | MP |  | HS-SICH Power Control Info 10.3.6.36b |  | REL-7 |
| >>>>BLER target | MP |  | Real (-3.15..0 by step of 0.05) | Signalled value is Log10(HS-SCCH BLER quality target). | REL-7 |
| >>>>Dhs-sync | OP |  | Integer (-20..+10) | Value in dB set to indicate the dB difference between the maximum allowed HS-SCCH physical channel transmit power [33] and the beacon reference power (it is one means of controlling the area of HS-DSCH operation within the cell). | REL-7 |
| >>>>Dhs-sync | OP |  | Integer (-20..+10) | Value in dB set to indicate the dB difference between the maximum allowed HS-SCCH physical channel transmit power [33] and the beacon reference power (it is one means of controlling the area of HS-DSCH operation within the cell). | REL-7 |
| >>>>HS-SCCH Set Configuration | MP | 1 to <maxHS-SCCHs> |  |  | REL-7 |
| >>>>>Timeslot number | MP |  | Integer (0..14) |  | REL-7 |
| >>>>>Channelisation code | MP |  | Enumerated ((32/1) ..(32/32)) |  | REL-7 |
| >>>>>Midamble Allocation mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) | HS-SCCH always uses burst type 1. | REL-7 |
| >>>>>Midamble configuration | MP |  | Integer (4, 8, 16) |  | REL-7 |
| >>>>>Midamble Shift | CV-UE |  | Integer(0..15) |  | REL-7 |
| >>>>>HS-SICH configuration |  |  |  |  | REL-5 |
| >>>>>>Timeslot number | MP |  | Integer (0..14) |  | REL-5 |
| >>>>>>Channelisation code | MP |  | Enumerated ((32/1) ..(32/32)) |  | REL-5 |
| >>>>>>Midamble Allocation mode | MP |  | Enumerated (Default midamble, UE specific midamble) | HS-SICH always uses burst type 1. | REL-5 |
| >>>>>>Midamble configuration | MP |  | Integer (4, 8, 16) |  | REL-5 |
| >>>>>>Midamble Shift | CV-UE |  | Integer (0..15) |  | REL-5 |
| >>>1.28 Mcps |  |  |  |  | REL-5 |
| >>>>Ack-Nack Power Offset | MP |  | Integer (-7..8 by step of 1) | dB. This IE is Not Present in REL-5. Note | REL-6 |
| >>>>PRXHS-SICH | MP |  | Integer (-120..-58 by step of 1) | dBm. Desired power level for HS-SICH. This IE is Not Present in REL-5 | REL-6 |
| >>>>TPC step size | MP |  | Enumerated (1, 2, 3) | dB. TPC step size for HS-SICH. One spare value. This IE is Not Present in REL-5. | REL-6 |
| >>>>BLER target | MP |  | Real (-3.15..0 by step of 0.05) | Signalled value is Log10(HS-SCCH BLER quality target). This IE is Not Present in REL-5. | REL-6 |
| >>>>HS-SCCH TPC step size | CV- NoDPCH_EAGCH |  | Enumerated (1, 2, 3) | dB. One spare value. | REL-8 |
| >>>>Power Control GAP | MD |  | Integer (1…255) | Unit: Number of subframes Default value is 1. | REL-7 |
| >>>>Pathloss compensation switch | MD |  | Boolean | TRUE: UE shall perform the pathloss compensation for HS-SICH power control when HS-SICH transmission gap is less than “Power Control GAP”. FALSE: UE shall not consider the pathloss compensation for HS-SICH power control. Default value is FALSE. | REL-7 |
| >>>HS-SCCH Set Configuration | MP | 1 to <maxHSSCCHs> |  |  | REL-5 |
| >>>>>Timeslot number | MP |  | Integer (0..6) |  | REL-5 |
| >>>>>First Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-5 |
| >>>>>Second Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-5 |
| >>>>>Midamble Allocation mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) |  | REL-5 |
| >>>>> Midamble Shift | CV-UE |  | Integer (0..15) |  | REL-5 |
| >>>>>Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) |  | REL-5 |
| >>>>>BLER target | MP |  | Real (-3.15..0 by step of 0.05) | Signalled value is Log10(HS-SCCH BLER quality target). This IE is not present in REL-6 and beyond. | REL-5 |
| >>>>>HS-SICH configuration |  |  |  |  | REL-5 |
| >>>>>>Timeslot number | MP |  | Integer (0..6) |  | REL-5 |
| >>>>>>Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-5 |
| >>>>>>Midamble Allocation mode | MP |  | Enumerated (Default midamble, UE specific midamble) |  | REL-5 |
| >>>>>>Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) |  | REL-5 |
| >>>>>>Midamble Shift | CV-UE |  | Integer (0..15) |  | REL-5 |
| >>>>>>Ack-Nack Power Offset | MP |  | Integer (-7..8 by step of 1) | dB. This IE is not present in REL-6 and beyond. Note | REL-5 |
| >>>>>>PRXHS-SICH | MP |  | Integer (-120..-58 by step of 1) | dBm. Desired power level for HS-SICH. This IE is not present in REL-6 and beyond. | REL-5 |
| >>>>>>TPC step size | MP |  | Enumerated (1, 2, 3) | dB. One spare value. This IE is not present in REL-6 and beyond. | REL-5 |
| Note 1: Ack-Nack Power Offset is the difference in the desired RX power between HS-SICH transmissions conveying an acknowledgement and transmissions conveying a negative acknowledgement signalled to the UE in IE "HS-SCCH Info". Note 2: The list of HS-SCCH(s) is assumed to be indexed starting from one. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| UE | This IE is mandatory present when the value of the IE "Midamble Allocation Mode" is "UE specific midamble" and not needed otherwise. |
| NoDPCH_EAGCH | This IE is mandatory present when neither DPCH nor E-AGCH is configured and not needed otherwise. |

#### 10.3.6.36ab	HS-SCCH less information

NOTE:	For FDD only

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE HS-SCCH less operation | MP |  |  |  | REL-7 |
| >Continue HS-SCCH less operation |  |  | (no data) |  | REL-7 |
| >New HS-SCCH less operation |  |  |  |  | REL-7 |
| >>HS-PDSCH Code Index | MP |  | Integer (1..15) | Index of first HS-PDSCH code | REL-7 |
| >>Transport Block Size List |  | 1..< maxHS-SCCHLessTrBlk > |  |  | REL-7 |
| >>>Transport Block Size Index | MP |  | Integer (1..90) | Index of the MAC-hs or MAC-ehs transport block size as described in appendix A of [15] | REL-7 |
| >>>HS-PDSCH Second Code Support | MP |  | Boolean | Indicates whether the second HS-PDSCH code is used for this TB size. If TRUE, the HS-PDSCH second code index value is the value of IE “HS-PDSCH Code Index” incremented by 1. | REL-7 |

#### 10.3.6.36b	HS-SICH Power Control Info

This IE is used to transfer HS-SICH power control info to the UE and only applies to TDD 3.84 Mcps and 7.68 Mcps.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UL target SIR | MP |  | Real (-11..20 by step of 0.5 ) | dB | REL-5 |
| HS-SICH Constant value | MP |  | Constant value 10.3.6.11 | NOTE 1 | REL-5 |

NOTE 1:	Alignment to ASN.1: the IE "Constant value TDD" (10.3.6.11a) should have been used to provide the correct value range. The IE "Constant value" (10.3.6.11) is used to keep compatibility with REL-5.

#### 10.3.6.36c	HS-DSCH common system information

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CCCH mapping info | MP |  | Common RB mapping info 10.3.4.oa |  | REL-7 |
| SRB1 mapping info | MD |  | Common RB mapping info 10.3.4.oa | Note | REL-7 |
| Common MAC-ehs reordering queue list | MP |  | Common MAC-ehs reordering queue list 10.3.5.3a |  | REL-7 |
| HS-SCCH system info | MP |  | HS-SCCH system info 10.3.6.36e |  | REL-7 |
| HARQ system Info | MP |  | HARQ Info 10.3.5.7a |  | REL-7 |
| Common H-RNTI Information | MP | 1 to <maxCommonHRNTI> |  |  | REL-7 |
| >Common H-RNTI | MP |  | H-RNTI 10.3.3.14a |  | REL-7 |
| BCCH specific H-RNTI | MP |  | H-RNTI 10.3.3.14a |  | REL-7 |

NOTE:	The default values for the "SRB1 mapping info" are 1 for the Logical channel identity and 0 for the MAC-ehs queue identity.

#### 10.3.6.36ca	HS-DSCH common system information 1.28Mcps TDD

NOTE:	For 1.28 Mcps TDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CCCH mapping info | MP |  | Common RB mapping info 10.3.4.oa |  | REL-8 |
| SRB1 mapping info | MD |  | Common RB mapping info 10.3.4.oa | Note 1 | REL-8 |
| Common MAC-ehs reordering queue list | MP |  | Common MAC-ehs reordering queue list 10.3.5.3a |  | REL-8 |
| HS-SCCH system info | MP |  | HS-SCCH system info 1.28Mcps TDD 10.3.6.36ea | Note 2 | REL-8 |
| HARQ system Info | MP |  | HARQ Info 10.3.5.7a |  | REL-8 |
| HS-PDSCH Midamble Configuration | MP |  | HS-PDSCH Midamble Configuration 10.3.6.36oo |  | REL-8 |
| Common H-RNTI Information | MP | 1 to <maxCommonHRNTI> |  |  | REL-8 |
| >Common H-RNTI | MP |  | H-RNTI 10.3.3.14a |  | REL-8 |
| BCCH specific H-RNTI | MP |  | H-RNTI 10.3.3.14a |  | REL-8 |

NOTE 1:	The default values for the "SRB1 mapping info" are 1 for the Logical channel identity and 0 for the MAC-ehs queue identity.

NOTE 2:	These IEs correspond to the HS-DSCH configuration for CELL_FACH, CELL_PCH state.

#### 10.3.6.36d	HS-DSCH paging system information

These parameters enable the UE in CELL_PCH and URA_PCH to receive transmissions on HSDPA cell resources.

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| DL Scrambling Code | MD |  | Secondary scrambling code 10.3.6.74 | DL Scrambling code to be applied for HS-DSCH and HS-SCCH. Default is same scrambling code as for the primary CPICH. | REL-7 |
| PICH for HSDPA supported paging list | MP | 1 to <maxSCCPCH> |  |  | REL-7 |
| >HSDPA associated PICH info | MP |  | PICH info 10.3.6.49 |  | REL-7 |
| >HS-PDSCH Channelisation Code | MP |  | Integer (0..15) | HS-PDSCH channel, associated with the PICH for HS-SCCH less PAGING TYPE 1 message transmission. | REL-7 |
| Number of PCCH transmissions | MP |  | Integer (1..5) | number of subframes used to transmit the PAGING TYPE 1. | REL-7 |
| Transport Block Size List | MP | 1 to 2 |  |  | REL-7 |
| >Transport Block Size Index | MP |  | Integer (1..32) | Index of value range 1 to 32 of the MAC-ehs transport block size as described in appendix A of [15] | REL-7 |

#### 10.3.6.36da	HS-DSCH paging system information 1.28Mcps TDD

These parameters enable the UE in CELL_PCH and URA_PCH to receive PCCH, DCCH/DTCH transmissions on HSDPA cell resources.

NOTE:	Only for 1.28 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PICH for HS-DSCH list | MP | 1 to <maxSCCPCH> |  |  | REL-8 |
| >CHOICE Configuration Mode | MP |  |  |  | REL-8 |
| >>Implicit |  |  |  | The choice Indicates the PICH is shared with PCH. | REL-8 |
| >>>Occurrence sequence number in S-CCPCH systme Info | MD |  | Integer (1.. maxSCCPCH) | Indicates the occurrence sequecnce number of "PICH Info" in Secondary CCPCH System Information. Default value implys the first occurrence or the only occurrence of "PICH Info" in Secondary CCPCH System Information. | REL-8 |
| >>Explicit |  |  |  |  | REL-8 |
| >>>HSDPA associated PICH info | MP |  | PICH info 10.3.6.49 |  | REL-8 |
| DTCH/DCCH Reception window size | MP |  | Integer (1..16) | Number of subframe for UE to detect the HS-SCCH | REL-8 |
| PCCH related information | OP |  |  | See Note 1 | REL-8 |
| >Paging associated HS-PDSCH info | MP | 1 to <maxSCCPCH> |  | See Note 3 | REL-8 |
| >>HS-PDSCH Midamble Configuration | MP |  | HS-PDSCH Midamble Configuration 10.3.6.36oo |  | REL-8 |
| >>Timeslot Resource Related InformationI | MP |  | Bit string (6) | Each bit indicates availability of a timeslot, where the bit 0 corresponds to TS0, the bit 1 is TS2, the bit 2 is TS3… bit 6 corresponds to TS6. The value 1 of a bit indicates that the corresponding timeslot is available. Bit 0 is the first/leftmost bit of the bit string. | REL-8 |
| >>Code Resource Information | MP |  |  | Note 2 | REL-8 |
| >>>Start code | MP |  | Enumerated((16/1)..(16/16)) |  | REL-8 |
| >>>Stop code | MP |  | Enumerated((16/1)..(16/16)) |  | REL-8 |
| >Paging Sub-Channel Size | MP |  | Integer (1..3) | number of frames for a Paging sub-channel | REL-8 |
| >Transport Block Size List | MP | 1 to 2 |  |  | REL-8 |
| >>Transport Block Size Index | MP |  | Integer (1..32) | Index of value range 1 to 32 of the MAC-ehs transport block size as described in Table 9.2.3.3.8 [15] | REL-8 |

NOTE 1:	The IE is not signalled when the "PICH info" is present in "Secondary CCPCH system information" in SIB5 and SIB6.

NOTE 2:	HS-PDSCH channelisation codes are allocated contiguously from a signalled start code to a signalled stop code, and the allocation includes both the start and stop code. If a value of Start code = 16 and Stop code = 1 is signalled, a spreading factor of SF=1 shall be used for the HS-PDSCH resources.

NOTE 3:	The timing between PICH and the paging associated HS-PDSCH is specified in [30]. UE should receives the HS-PDSCH based on the timing specification.

#### 10.3.6.36e	HS-SCCH system info

These parameters enable the UE to receive transmission on HSDPA cell resources, when the UE is not in CELL_DCH.

NOTE:	for FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| DL Scrambling Code | MD |  | Secondary scrambling code 10.3.6.74 | DL Scrambling code to be applied for HS-DSCH and HS-SCCH. Default is same scrambling code as for the primary CPICH. | REL-7 |
| HS-SCCH Channelisation Code Information | MP | 1 to <maxHSSCCHs> |  |  | REL-7 |
| >HS-SCCH Channelisation Code | MP |  | Integer (0..127) (NOTE) |  | REL-7 |

NOTE:	UTRAN should use the first indexed HS-SCCH Channelisation code for the BCCH specific H-RNTI to indicate system information change information. Otherwise UE behaviour is unspecified.

#### 10.3.6.36ea	HS-SCCH system info 1.28Mcps TDD

These parameters enable the UE to receive transmission on HSDPA cell resources on primary frequency or secondary frequency, when the UE is not in CELL_DCH.

NOTE:	for 1.28 Mcps TDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| HS-SCCH Set Configuration | MP | 1 to <maxHSSCCHs> |  |  | REL-8 |
| >Timeslot number | MP |  | Integer (0..6) |  | REL-8 |
| >First Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-8 |
| >Second Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-8 |
| >Midamble Allocation mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) |  | REL-8 |
| >Midamble Shift | CV-UE |  | Integer (0..15) |  | REL-8 |
| >Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) |  | REL-8 |
| >HS-SICH configuration | MP |  |  |  | REL-8 |
| >>Timeslot number | MP |  | Integer (0..6) |  | REL-8 |
| >>Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-8 |
| >>Midamble Allocation mode | MP |  | Enumerated (Default midamble, UE specific midamble) |  | REL-8 |
| >>Midamble Shift | CV-UE |  | Integer (0..15) |  | REL-8 |
| >>Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) |  | REL-8 |
| PRXHS-SICH | MP |  | Integer (-120..-58 by step of 1) | dBm. Desired power level for HS-SICH. | REL-8 |
| Ack-Nack Power Offset | MP |  | Integer (-7..8 by step of 1) | dB. | REL-8 |
| TPC step size | MP |  | Enumerated (1, 2, 3) | dB. TPC step size for HS-SICH. One spare value. | REL-8 |
| BLER target | MP |  | Real (-3.15..0 by step of 0.05) | Signalled value is Log10(HS-SCCH BLER quality target). | REL-8 |
| Power Control GAP | MD |  | Integer (1…255) | Unit: Number of subframes Default value is 1. | REL-8 |
| Pathloss compensation switch | MD |  | Boolean | TRUE: UE shall perform the pathloss compensation for HS-SICH power control when HS-SICH transmission gap is less than “Power Control GAP”. FALSE: UE shall not consider the pathloss compensation for HS-SICH power control. Default value is FALSE. | REL-8 |

| Condition | Explanation |
| --- | --- |
| UE | This IE is mandatory present when the value of the IE "Midamble Allocation Mode" is "UE specific midamble" and not needed otherwise. |

NOTE:	UTRAN should use the first indexed HS-SCCH Channelisation code for the BCCH specific H-RNTI to indicate system information change information on primary frequency or secondary frequency. Otherwise UE behaviour is unspecified.

#### 10.3.6.36f	Void

#### 10.3.6.36g	HS-DSCH DRX in CELL_FACH information

These parameters configure the UE in CELL_FACH state to discontinuously receive HS-DSCH.

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| T321 | MP |  | Enumerated (100, 200, 400, 800) | Determines the time the UE waits until initiating DRX operation, in ms. | REL-8 |
| HS-DSCH DRX cycleFACH | MP |  | Enumerated (4, 8, 16, 32) | Determines the length of the DRX Cycle during DRX operation, in frames | REL-8 |
| HS-DSCH Rx burstFACH | MP |  | Enumerated (1, 2, 4, 8, 16) | Determines the period within the DRX Cycle that the UE continuously receives HS-DSCH, in frames. Three spare values are needed | REL-8 |
| DRX Interruption by HS-DSCH data | MP |  | Boolean | TRUE means that the DRX operation can be interrupted by HS-DSCH data. FALSE means that the DRX operation cannot be interrupted by HS-DSCH data | REL-8 |

#### 10.3.6.36h	HS-DSCH DRX in CELL_FACH information 1.28Mcps TDD

These parameters configure the UE in CELL_FACH state to discontinuously receive HS-DSCH.

NOTE:	Only for 1.28 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| T321 | MP |  | Enumerated (100, 200, 400, 800) | Determines the time the UE waits until initiating DRX operation, in ms. | REL-8 |
| DRX cycleFACH | MP |  | Enumerated (4, 8, 16, 32) | Determines the length of the DRX Cycle during DRX operation, in frames | REL-8 |
| Rx burstFACH | MP |  | Enumerated (1, 2, 4, 8, 16) | Determines the period within the DRX Cycle that the UE continuously receives HS-DSCH, in frames. Three spare values are needed | REL-8 |

#### 10.3.6.37	Individual timeslot info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Timeslot number | MP |  | Timeslot number 10.3.6.84 | Timeslot within a frame |  |
| TFCI existence | MP |  | Boolean | TRUE indicates that the TFCI exists. It shall be coded in the physical channel defined in [30] of this timeslot. |  |
| Midamble Shift and burst type | MP |  | Midamble shift and burst type 10.3.6.41 | For 1.28 Mcps TDD, if the cell is operating in MBSFN mode, the UE shall ignore the contents of this IE. |  |
| CHOICE TDD option | MP |  |  |  | REL-4 |
| >3.84 Mcps TDD |  |  |  | (no data) | REL-4 |
| >7.68 Mcps TDD |  |  |  | (no data) | REL-7 |
| >1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>Modulation | MP |  | Enumerated(QPSK, 8PSK) |  | REL-4 |
| >>SS-TPC Symbols | MP |  | Enumerated(0, 1, 16/SF) | Denotes amount of SS and TPC bits send in this timeslot | REL-4 |
| >>Additional TPC-SS Symbols | OP |  | Integer(1..15) | Specifies the number of additional codes in this timeslot that carry TPC and SS symbols as specified in [33] | REL-4 |

#### 10.3.6.38	Individual Timeslot interference

Parameters used by the UE for uplink open loop power control in TDD.

| Information element | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Timeslot number | MP |  | Timeslot number 10.3.6.84 |  |
| UL Timeslot Interference | MP |  | UL Interference TDD 10.3.6.87a |  |

#### 10.3.6.39	Maximum allowed UL TX power

This information element indicates the maximum allowed uplink transmit power.

| Information Element | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum allowed UL TX power | MP |  | Integer(-50..33) | In dBm |

#### 10.3.6.39a	Multi-frequency Info (1.28 Mcps TDD only)

| IE/Group Name | Presence | Multi | IE Type and Reference | Semantics Description | Version |
| --- | --- | --- | --- | --- | --- |
| Second Frequency Info | OP |  | Integer (0 .. 16383) | 1.28 Mcps TDD only | REL-7 |
| FPACH Frequency Info | OP |  | Integer (0 .. 16383) | 1.28 Mcps TDD only | REL-7 |
| UpPCH Position Info | OP |  | Integer (0 .. 127) | Only for 1.28 Mcps TDD, if UpPCH occupies UpPTS, the parameter is not needed. Or the parameter is caculated for the uplink access position of a cell | REL-7 |

#### 10.3.6.40	Void

#### 10.3.6.40a	Measurement Feedback Info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  | REL-5 |
| >FDD |  |  |  |  | REL-5 |
| >>Measurement Power Offset | MP |  | Real (-6 .. 13 by step of 0.5) | The measurement power offset, , in dB, as described in [29]. | REL-5 |
| >>CQI Feedback cycle, k | MP |  | Integer (0, 2, 4, 8, 10, 20, 40, 80, 160, | In milliseconds. | REL-5 |
|  |  |  | 16, 32, 64) | In milliseconds. | REL-7 |
| >>HS-DPCCH reduction configuration | CV-Message |  |  |  | REL-12 |
| >>>HS-DPCCH reduction type | OP |  | Enumerated (Type1, Type2) | The Type of HS-DPCCH reduction the UE shall apply. Absense of this IE means that the UE shall apply both HS-DPCCH reduction Type1 and HS-DPCCH reduction Type2. | REL-12 |
| >>>CQI Feedback cycle2, k | MP |  | Enumerated (0, 8, 10, 16, 20, 32, 40, 64, 80, 160) | In milliseconds. Six spare values are needed. | REL-12 |
| >>>CQI cycle switch timer | MP |  | Enumerated (4, 8, 16, 32, 64, 128, 256, 512, Infinity) | Units of subframes. Seven spare values are needed. | REL-12 |
| >>CQI repetition factor | MP |  | Integer (1..4) |  | REL-5 |
| >>CQI repetition factor for Multiflow assisting cells | CV-Multiflow |  | Integer (1..4) | If present, this IE indicates that all the assisting cells use the CQI repetition factor indicated by this IE. | REL-11 |
| >>CQI | MP |  | Integer (0..8 | Refer to quantization of the power offset in [28] | REL-5 |
|  |  |  | 9..10) |  | REL-11 |
| >TDD |  |  |  | (no data) | REL-5 |

| Condition | Explanation |
| --- | --- |
| Message | This IE is not needed in the HANDOVER TO UTRAN COMMAND message. Otherwise it is optional. |
| Multiflow | The IE is optional when the Multiflow operation is configured, otherwise it is not needed. |

#### 10.3.6.41	Midamble shift and burst type

NOTE:	Only for TDD.

This information element indicates burst type and midamble allocation. Three different midamble allocation schemes exist:

-	Default midamble: the midamble shift is selected by layer 1 depending on the associated channelisation code (DL and UL)

-	Common midamble: the midamble shift is chosen by layer 1 depending on the number of channelisation codes (possible in DL only)

-	UE specific midamble: a UE specific midamble is explicitly assigned (DL and UL).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE TDD option | MP |  |  |  | REL-4 |
| >3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>CHOICE Burst Type | MP |  |  |  |  |
| >>>Type 1 |  |  |  |  |  |
| >>>>Midamble Allocation Mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) |  |  |
| >>>>Midamble configuration burst type 1 and 3 | MP |  | Integer(4, 8, 16) | As defined in [30] |  |
| >>>>Midamble Shift | CV-UE |  | Integer(0..15) |  |  |
| >>>Type 2 |  |  |  |  |  |
| >>>>Midamble Allocation Mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) |  |  |
| >>>>Midamble configuration burst type 2 | MP |  | Integer(3, 6) | As defined in [30] |  |
| >>>>Midamble Shift | CV-UE |  | Integer(0..5) |  |  |
| >>>Type 3 |  |  |  |  |  |
| >>>>Midamble Allocation Mode | MP |  | Enumerated (Default midamble, UE specific midamble) |  |  |
| >>>>Midamble configuration burst type 1 and 3 | MP |  | Integer(4, 8, 16) | As defined in [30] |  |
| >>>>Midamble Shift | CV-UE |  | Integer (0..15) | NOTE: Burst Type 3 is only used in uplink. |  |
| >>> MBSFN Burst Type |  |  |  | (no data) DL only | REL-7 |
| >7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>CHOICE Burst Type | MP |  |  |  | REL-7 |
| >>>Type 1 |  |  |  |  | REL-7 |
| >>>>Midamble Allocation Mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) |  | REL-7 |
| >>>>Midamble configuration burst type 1 and 3 | MP |  | Integer(4, 8, 16) | As defined in [30] | REL-7 |
| >>>>Midamble Shift | CV-UE |  | Integer(0..15) |  | REL-7 |
| >>>Type 2 |  |  |  |  | REL-7 |
| >>>>Midamble Allocation Mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) |  | REL-7 |
| >>>>Midamble configuration burst type 2 | MP |  | Integer(4, 8) | As defined in [30] | REL-7 |
| >>>>Midamble Shift | CV-UE |  | Integer(0..7) |  | REL-7 |
| >>>Type 3 |  |  |  |  | REL-7 |
| >>>>Midamble Allocation Mode | MP |  | Enumerated (Default midamble, UE specific midamble) |  | REL-7 |
| >>>>Midamble configuration burst type 1 and 3 | MP |  | Integer(4, 8, 16) | As defined in [30] | REL-7 |
| >>>>Midamble Shift | CV-UE |  | Integer (0..15) | NOTE: Burst Type 3 is only used in uplink. | REL-7 |
| >>> MBSFN Burst Type |  |  |  | (no data) DL only | REL-7 |
| >1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>Midamble Allocation Mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) |  | REL-4 |
| >>Midamble configuration | MP |  | Integer(2, 4, 6, 8, 10, 12, 14, 16) | As defined in [30] | REL-4 |
| >>Midamble Shift | CV-UE |  | Integer (0..15) |  | REL-4 |

| Condition | Explanation |
| --- | --- |
| UE | This IE is mandatory present when the value of the IE "Midamble Allocation Mode" is "UE-specific midamble" and not needed otherwise. |

#### 10.3.6.41a	MIMO parameters

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MIMO operation | MP |  | Enumerated (start, continue) |  | REL-7 |
| CHOICE mode | MP |  |  |  | REL-8 |
| >FDD |  |  |  |  | REL-8 |
| >>MIMO N_cqi_typeA/M_cqi ratio | OP |  | Enumerated(1/2, 2/3, 3/4, 4/5, 5/6, 6/7, 7/8, 8/9, 9/10, 1/1) |  | REL-7 |
| >>MIMO pilot configuration | OP |  | MIMO pilot configuration 10.3.6.41b |  | REL-7 |
| >>Precoding weight set restriction | OP |  | Enumerated (TRUE) | If present and MIMO operation is set to start, the UE starts using precoding weight set restriction. | REL-7 |
| >TDD |  |  |  |  | REL-8 |
| >>CHOICE TDD option | MP |  |  |  | REL-8 |
| >>>1.28 Mcps TDD |  |  |  |  | REL-8 |
| >>>>MIMO SF Mode for HS-PDSCH dual stream | MP |  | Enumerated (SF1, SF1/SF16) |  | REL-8 |
| >>>>HS-SICH Reference Signal Info | OP | <1 to maxHSSCCHs > |  | The order of the list corresponds to the order of HS-SCCHs in HS-SCCH info | REL-8 |
| >>>>> Reference Signal Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) | Midamble Allocation mode is UE specific midamble allocation | REL-8 |
| >>>>>Reference Signal Midamble Shift | MP |  | Integer (0..15) | The allocated midamble shift. | REL-8 |
| >>>>>Reference Signal Timeslot number | MP |  | Integer (1..5) |  | REL-8 |
| >>>3.84 Mcps TDD or 7.68 Mcps TDD |  |  |  | (no data) | REL-8 |

#### 10.3.6.41b	MIMO pilot configuration

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Second CPICH pattern | MP |  |  |  | REL-7 |
| >Antenna2 P-CPICH |  |  |  | (no data) | REL-7 |
| >Antenna1 S-CPICH |  |  |  |  | REL-7 |
| >>Channelisation code | MP |  | Integer (0..255) |  | REL-7 |
| >> Power Offset for S-CPICH for MIMO | OP |  | Integer(-6 .. 0) | Power offset relative to the P-CPICH Tx power in dB | REL-7 |

#### 10.3.6.41c	Non-scheduled transmission grant info (TDD only)

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE TDD Option |  |  |  |  | REL-7 |
| >3.84/7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>Timeslot Resource Related InformationI | MP |  | Bit string (13) | Bitmap indicating which of the timeslots configured for E-DCH are allocated for non-scheduled transmissions | REL-7 |
| >>Power Resource Related Information | MP |  | Integer (1..32) | Specifies the maximum allowed E-PUCH resource that the UE may use [15]. | REL-7 |
| >>Activation Time | MP |  | Activation time 10.3.3.1 |  | REL-7 |
| >>Repetition period and length | MD |  |  | Default is "Repetition period" = 1 (continuous) | REL-7 |
| >>>Repetition period | MP |  | Integer (1, 2, 4, 8, 16, 32, 64) | Periodicity in terms of TTIs for which resource is allocated. 1= continuous dallocation, 2 indicates every other TTI, 4 = every 4th, 8 every 8th etc. | REL-7 |
| >>>Repetition Length | MP |  | Integer (1..Repetition period-1) | TTI in which resource is assigned. If Repetition period = 1 then value is ignored | REL-7 |
| >>Code Resource Information | MP |  | Enumerated ((1/1),(2/1),(2/2),(4/1)..(4/4),(8/1)..(8/8),(16/1)..(16/16)) |  | REL-7 |
| >1.28 Mcps TDD |  |  |  |  | REL-7 |
| >>NE-UCCH | MD |  | Integer (1..8) | Number of E-UCCH and TPC instances within an E-DCH TTI. Default = 1. | REL-7 |
| >>NE-HICH | CV-Scheduled E-PUCH |  | Integer(4..15) | Minimum number of slots between start last active slot of E-DCH TTI and start of ACK/NACK on E-HICH. | REL-7 |
| >>Timeslot Resource Related InformationI | MP |  | Bit string (5) | Bitmap indicating which of the timeslots configured for E-DCH are allocated for non-scheduled transmissions | REL-7 |
| >>Power Resource Related Information | MP |  | Integer (1..32) | Indicates 0-31 PRRI index defined in [31], which specifies the maximum allowed E-PUCH resource that the UE may use [15]. | REL-7 |
| >>Activation Time | MP |  | Activation time 10.3.3.1 | Specifies the E-PUCH Offset in Radio Frame level. | REL-7 |
| >>Subframe number | MP |  | Integer (0..1) | Specifies the E-PUCH Offset in subframe level. | REL-7 |
| >>Repetition period and length | MD |  |  | Default is "Repetition period" = 1 (continuous) | REL-7 |
| >>>Repetition period | MP |  | Integer (1, 2, 4, 8, 16, 32, 64) | Periodicity in terms of TTIs for which resource is allocated. 1= continuous allocation, 2 indicates every other TTI, 4 = every 4th, 8 every 8th etc. | REL-7 |
| >>>Repetition Length | MP |  | Integer (1..Repetition period-1) | TTI in which resource is assigned. If Repetition period = 1 then value is ignored | REL-7 |
| >>Code Resource Information | MP |  | Enumerated ((1/1), (2/1),(2/2),(4/1)..(4/4),(8/1)..(8/8),(16/1)..(16/16)) |  | REL-7 |
| >>E-HICH Information | MP |  |  |  | REL-7 |
| >>>Timeslot number | MP |  | Integer (0..6) |  | REL-7 |
| >>>Channelisation code | MP |  | Enumerated ((16/1)..(16/16)) |  | REL-7 |
| >>>Midamble Allocation mode | MP |  | Enumerated (Default midamble, UE specific midamble) |  | REL-7 |
| >>>Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) |  | REL-7 |
| >>>Midamble Shift | CV-UE specific |  | Integer (0..15) |  | REL-7 |
| >>>Signature Sequence Group Index | MP |  | Integer (0..19) |  | REL-7 |
| >>T-SI-NST | CV-E-RUCCH info |  | Enumerated (everyEDCHTTI, 20,40,60,80,160,200) | Values in ms. If T-SI-NST expires then UE should include scheduling information in the next new non-scheduled MAC-e PDU or MAC-i PDU. | REL-7 |

| Condition | Explanation |
| --- | --- |
| Scheduled E-PUCH | If NE-HICH is given in scheduled E-PUCH information, this IE is not needed, otherwise it is mandatory |
| UE specific | If UE specific midamble allocation mode is configured, this IE is mandatory, otherwise it is not needed. |
| E-RUCCH info | This IE is optionally present if the IE "E-RUCCH Info" is not included, otherwise the IE is not needed. |

#### 10.3.6.42	PDSCH Capacity Allocation info

NOTE:	Only for TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDSCH allocation period info | MP |  | Allocation Period Info 10.3.6.4 |  |
| CHOICE Configuration | MP |  |  |  |
| >Old configuration |  |  |  |  |
| >>TFCS ID | MD |  | Integer(1..8) | Default is 1. |
| >>PDSCH Identity | MP |  | Integer(1..hiPDSCHidentities) |  |
| >New configuration |  |  |  |  |
| >>PDSCH Info | MP |  | PDSCH Info 10.3.6.44 |  |
| >>PDSCH Identity | OP |  | Integer(1..hiPDSCHidentities) |  |
| >>PDSCH power control info | OP |  | PDSCH power control info 10.3.6.45 |  |

#### 10.3.6.43	Void

#### 10.3.6.44	PDSCH info

NOTE:	Only for TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| TFCS ID | MD |  | Integer(1..8) | TFCS to be used. Default value is 1. |  |
| Common timeslot info | OP |  | Common timeslot info 10.3.6.10 |  |  |
| PDSCH timeslots and codes for1.28 Mcps TDD or 3.84 Mcps TDD | OP |  | Downlink Timeslots and Codes 10.3.6.32 | Default is to use the old timeslots and codes. |  |
| PDSCH timeslots and codes for 7.68 Mcps TDD | OP |  | Downlink Timeslots and Codes VHR 10.3.6.32a | Default is to use the old timeslots and codes. | REL-7 |

#### 10.3.6.45	PDSCH Power Control info

NOTE:	Only for TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TPC Step Size | OP |  | Integer (1, 2, 3) | In dB |
| UL CCTrCH TPC List | OP | 1..<maxCCTrCH> |  | UL CCTrCH identities for TPC commands associated with this DL CCTrCH. This list is not used in 1.28 Mcps TDD. |
| >UL TPC TFCS Identity | MP |  | Transport Format Combination Set Identity 10.3.5.21 |  |

#### 10.3.6.46	PDSCH system information

NOTE:	Only for TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PDSCH information | MP | 1 to <maxPDSCH> |  |  |  |
| >PDSCH Identity | MP |  | Integer(1..hiPDSCHidentities) |  |  |
| >PDSCH info | MP |  | PDSCH info 10.3.6.44 |  |  |
| >SFN Time Info | CH-Block17 |  | SFN Time Info 10.3.6.75 |  |  |
| >DSCH TFS | OP |  | Transport format set 10.3.5.23 |  |  |
| >DSCH Transport Channels | OP | 1 to <maxTrCH> |  | If PDSCH is configured for 3.84Mcps TDD in Rel-5 or for 7.68 Mcps in Rel-7 this IE may be included. | REL-5 |
| >> DSCH Transport channel identity | MP |  | Transport channel identity 10.3.5.18 |  | REL-5 |
| >>DSCH TFS | MP |  | Transport format set 10.3.5.23 |  | REL-5 |
| >DSCH TFCS | OP |  | Transport Format Combination Set 10.3.5.20 |  |  |

| Condition | Explanation |
| --- | --- |
| Block17 | This IE is not needed in System Information Block 17. Otherwise it is optional. |

#### 10.3.6.47	Void

#### 10.3.6.48	Persistence scaling factors

This IE defines scaling factors associated with ASC 2 – ASC 7 to be applied to the dynamic persistence value.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Access Service Class | MP | 1 to maxASCpersist |  | multiplicity corresponds to the number of PRACH partitions minus 2 |
| >Persistence scaling factor | MP |  | Real(0.9..0.2, by step of 0.1) | Scaling factors in the range 0,…,1 |

#### 10.3.6.49	PICH Info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Channelisation code | MP |  | Integer(0..255) | SF is fixed and equal to 256 |  |
| >>Number of PI per frame | MP |  | Integer (18, 36, 72, 144) |  |  |
| >>STTD indicator | MP |  | STTD Indicator 10.3.6.78 |  |  |
| >TDD |  |  |  |  |  |
| >>Timeslot number | MD |  | Timeslot number 10.3.6.84 | Default value is the timeslot used by the SCCPCH carrying the associated PCH. |  |
| >>Midamble shift and burst type | MP |  | Midamble shift and burst type 10.3.6.41 |  |  |
| >>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>Channelisation code | MD |  | Enumerated ( (16/1)...(16/16)) | Default value is the channelisation code used by the SCCPCH carrying the associated PCH. |  |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Channelisation code | MD |  | Enumerated ( (32/1)...(32/32)) | Default value is the channelisation code used by the SCCPCH carrying the associated PCH. | REL-7 |
| >>>1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>>>Codes list | MP | 1..2 |  | The IE shall always be set to 2 to align with [30] | REL-4 |
| >>>>>Channelisation code | MP |  | Enumerated ( (16/1)...(16/16)) |  | REL-4 |
| >>Repetition period/length | MD |  | Enumerated((4/2),(8/2), (8/4),(16/2), (16/4), (32/2),(32/4),(64/2),(64/4)) | Default value is "(64/2)". |  |
| >>Offset | MP |  | Integer (0...Repetition period -1) | SFN mod Repetitionperiod = Offset. |  |
| >>Paging indicator length | MD |  | Integer (4, 8, 16) | Indicates the length of one paging indicator in Bits. Default value is 4. |  |
| >>NGAP | MD |  | Integer(2, 4, 8) | Number of frames between the last frame carrying PICH for this Paging Occasion and the first frame carrying paging messages for this Paging Occasion. Default value is 4. |  |
| >>NPCH | MD |  | Integer(1 .. 8) | Number of paging groups. Default value is 2. |  |

#### 10.3.6.50	PICH Power offset

This is the power transmitted on the PICH minus power of the Primary CPICH in FDD and Primary CCPCH Tx Power in TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PICH Power offset | MP |  | Integer(-10 .. +5) | Offset in dB |

#### 10.3.6.50a	PLCCH Info

NOTE:	Only for 1.28 Mcps TDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLCCH sequence number | M |  | Integer(1..14) |  |
| Timeslot number | M |  | Integer(0..6) |  |
| Channelisation Code | M |  | Enumerated((16/1)..(16/16)) |  |
| TPC command target rate | M |  | Real(0.01..0.1 by steps of 0.01) | Transmit Power Control command target rate |

#### 10.3.6.51	PRACH Channelisation Code List

NOTE:	Only for 3.84 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE SF | MP |  |  |  |  |
| >SF16 |  |  |  |  |  |
| >>Channelisation Code List | MP | 1 to 8 |  |  |  |
| >>>Channelisation code | MP |  | Enumerated ((16/1)...(16/16)) | There is a 1:1 mapping between spreading code and midamble shift defined in [30] for channelisation codes (16/1) to (16/8). NOTE: channelisation codes (16/9) to (16/16) are not to be used. |  |
| >SF8 |  |  |  |  |  |
| >>Channelisation Code List | MP | 1 to 8 |  |  |  |
| >>>Channelisation Code | MP |  | Enumerated((8/1)..(8/8)) |  |  |

#### 10.3.6.51a	PRACH Channelisation Code 1.28 Mcps TDD

NOTE:	Only for 1.28 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Channelisation Code List | MP | 1 to 4 |  |  | REL-4 |
| >Channelisation Code | MP |  | Enumerated( (4/1)..(4/4),(8/1)..(8/8),(16/1)..(16/16)) |  | REL-4 |

#### 10.3.6.51b	PRACH Channelisation Code List VHCR

NOTE:	Only for 7.68 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE SF | MP |  |  |  | REL-7 |
| >SF32 |  |  |  |  | REL-7 |
| >>Channelisation Code List | MP | 1 to 16 |  |  | REL-7 |
| >>>Channelisation code | MP |  | Enumerated ((32/1)...(32/32)) | There is a 1:1 mapping between spreading code and midamble shift defined in [30] for channelisation codes (32/1) to (32/16). NOTE: channelisation codes (32/17) to (32/32) are not to be used. | REL-7 |
| >SF16 |  |  |  |  | REL-7 |
| >>Channelisation Code List | MP | 1 to 16 |  |  | REL-7 |
| >>>Channelisation code | MP |  | Enumerated ((16/1)...(16/16)) | There is a 1:1 mapping between spreading code and midamble shift defined in [30] for channelisation codes (16/1) to (16/16). | REL-7 |

#### 10.3.6.52	PRACH info (for RACH)

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Available Signature | MP |  | Bit string(16) | Each bit indicates availability for a signature, where the signatures are numbered "signature 0" up to "signature 15". The value 1 of a bit indicates that the corresponding signature is available and the value 0 that it is not available. |  |
| >>Available SF | MP |  | Integer (32,64,128,256) | In chips per symbol Defines the minimum allowed SF (i.e. the maximum rate) |  |
| >>Preamble scrambling code number | MP |  | Integer (0 .. 15) | Identification of scrambling code see [28] |  |
| >>Puncturing Limit | MP |  | Real(0.40..1.00 by step of 0.04) |  |  |
| >>Available Sub Channel Number | MP |  | Bit string(12) | Each bit indicates availability for a subchannel, where the subchannels are numbered "subchannel 0" to "subchannel 11". The value 1 of a bit indicates that the corresponding subchannel is available and the value 0 indicates that it is not available. |  |
| >TDD |  |  |  |  |  |
| >>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>Timeslot number | MP |  | Timeslot number 10.3.6.84 |  |  |
| >>>>PRACH Channelisation Code List | MP |  | PRACH Channelisation Code List 10.3.6.51 |  |  |
| >>>>PRACH Midamble | MP |  | Enumerated (Direct, Direct/Inverted) | Direct or direct and inverted midamble are used for PRACH |  |
| >>>>PNBSCH allocation | OP |  | PNBSCH allocation 10.3.8.10a | Identifies frames used for cell synchronisation purposes | REL-4 |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Timeslot number | MP |  | Timeslot number 10.3.6.84 |  | REL-7 |
| >>>>PRACH Channelisation Code List VHCR | MP |  | PRACH Channelisation Code List VHCR 10.3.6.51b |  | REL-7 |
| >>>>PRACH Midamble | MP |  | Enumerated (Direct, Direct/Inverted) | Direct or direct and inverted midamble are used for PRACH | REL-7 |
| >>>>PNBSCH allocation | OP |  | PNBSCH allocation 10.3.8.10a | Identifies frames used for cell synchronisation purposes | REL-7 |
| >>>1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>>>SYNC_UL info | MP |  | SYNC_UL info 10.3.6.78a |  | REL-4 |
| >>>>PRACH Definition | MP | 1..<maxPRACH_FPACH> |  |  | REL-4 |
| >>>>>Timeslot number | MP |  | Timeslot number 10.3.6.84 |  | REL-4 |
| >>>>>PRACH Channelisation Code | MP |  | PRACH Channelisation Code 1.28 Mcps TDD 10.3.6.51a |  | REL-4 |
| >>>>>Midamble Shift and burst type | MP |  | Midamble shift and burst type 10.3.6.41 |  | REL-4 |
| >>>>>FPACH info | MP |  | FPACH info 10.3.6.35a |  | REL-4 |

#### 10.3.6.53	PRACH partitioning

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Access Service class | MP | 1 to maxASC |  | If only "NumASC+1" (with, NumASC+1 < maxASC) ASCs are listed, the remaining (NumASC+2 through maxASC) ASCs are unspecified. |
| >ASC Setting | MD |  | ASC setting 10.3.6.6 | The default values are same as the previous ASC. If the "default" is used for the first ASC, the default values are all available signatures and "all available sub-channels" for FDD and "all available channelisation codes" and "all available subchannels" with "subchannel size=Size 1" in TDD. |

#### 10.3.6.54	PRACH power offset

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Power Ramp Step | MP |  | Integer (1..8) | Power step when no acquisition indicator is received in dB |
| Preamble Retrans Max | MP |  | Integer (1..64) | Maximum number of preambles in one preamble ramping cycle |

#### 10.3.6.54a		PRACH preamble control parameters (for Enhanced Uplink)

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Available Signature | MD |  | Bit string(16) | The default value is the inverse of the bitstring indicated in the IE "Available Signature" in the IE "PRACH Info (for RACH)" in the first occurrence in list "PRACH system information list" in SIB5 or SIB5bis. Each bit indicates availability for a signature. Each available signature on the AICH is associated with one Common E-DCH Resource Configuration in the “Common E-DCH resource configuration information list”. | REL-8 |
| E-AI Indication | MP |  | BOOLEAN | TRUE: E-AIs are in use on the AICH. FALSE: E-AIs are not in use on the AICH. | REL-8 |
| Preamble scrambling code number | MD |  | Integer (0 .. 15) | The default value is the value indicated in the IE "Preamble scrambling code number" in the IE "PRACH Info (for RACH)" in the first occurrence in list "PRACH system information list" in SIB5 or SIB5bis. Identification of scrambling code, see [28] | REL-8 |
| Available Sub Channel Number | MD |  | Bit string(12) | The default value is the same value indicated in the IE "Available Sub Channel Number" in the IE "PRACH system information list" in the first occurrence in list "PRACH system information list" in SIB5 or SIB5bis. Each bit indicates availability for a subchannel, where the subchannels are numbered "subchannel 0" to "subchannel 11". The value 1 of a bit indicates that the corresponding subchannel is available and the value 0 indicates that it is not available. | REL-8 |
| PRACH partitioning | MD |  | PRACH partitioning 10.3.6.53 | Default value is the value of "PRACH partitioning" in the first occurrence in list “PRACH system information list” in SIB5 or SIB5bis. | REL-8 |
| Persistence scaling factors | OP |  | Persistence scaling factors 10.3.6.48 | This IE shall not be present if only ASC 0 and ASC 1 are defined. If this IE is absent, default value is the value of "Persistence scaling factors" in the first occurrence in list “PRACH system information list” in SIB5 or SIB5bis, if value exists | REL-8 |
| AC-to-ASC mapping | MD |  | AC-to-ASC mapping 10.3.6.1 | Default value is the value of "AC-to-ASC mapping" in the first occurrence in list “PRACH system information list” in SIB5 or SIB5bis. | REL-8 |
| Primary CPICH TX power | MD |  | Primary CPICH TX power 10.3.6.61 | Default value is the value of "Primary CPICH TX power" in the first occurrence in list “PRACH system information list” in SIB5 or SIB5bis. | REL-8 |
| Constant value | MD |  | Constant value 10.3.6.11 | Default value is the value of "Constant value" in the first occurrence in list “PRACH system information list” in SIB5 or SIB5bis | REL-8 |
| PRACH power offset | MD |  | PRACH power offset 10.3.6.54 | Default value is the value of "PRACH power offset" in the first occurrence in list “PRACH system information list” in SIB5 or SIB5bis | REL-8 |
| PRACH transmission parameters | MD |  | RACH transmission parameters 10.3.6.67 | Default value is the value of "RACH transmission parameters" in the first occurrence in list “PRACH system information list” in SIB5 or SIB5bis. | REL-8 |
| AICH info | MD |  | AICH info 10.3.6.2 | Default value is the value of "AICH info" in the first occurrence in list “PRACH system information list” in SIB5 or SIB5bis. | REL-8 |
| Power offset Pp-e | MP |  | Integer(-5..10) | In dB. Power offset between the last transmitted preamble and the initial DPCCH transmission as defined in [26] | REL-8 |

#### 10.3.6.55	PRACH system information list

| Information element | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PRACH system information | MP | 1 .. <maxPRACH> |  |  |  |
| >PRACH info | MP |  | PRACH info (for RACH) 10.3.6.52 |  |  |
| >Transport channel identity | MP |  | Transport channel identity 10.3.5.18 |  |  |
| >RACH TFS | MD |  | Transport format set 10.3.5.23 | Default value is the value of "RACH TFS" for the previous PRACH in the list NOTE: The first occurrence is then MP). NOTE: For TDD in this release there is a single TF within the RACH TFS. |  |
| >Additional RACH TFS for CCCH | OP |  | Additional Dynamic Transport Format Information for CCCH 10.3.5.2a | FDD only | REL-6 |
| >RACH TFCS | MD |  | Transport Format Combination Set 10.3.5.20 | Default value is the value of "RACH TFCS" for the previous PRACH in the list. NOTE: The first occurrence is then MP). NOTE: For TDD in this release there is no TFCS required. |  |
| >Additional RACH TFCS for CCCH | CV-Add-RACH-TFS |  | Additional RACH TFCS for CCCH 10.3.5.2b |  | REL-6 |
| >PRACH partitioning | MD |  | PRACH partitioning 10.3.6.53 | Default value is the value of "PRACH partitioning" for the previous PRACH in the list (note : the first occurrence is then MP) |  |
| >Persistence scaling factors | OP |  | Persistence scaling factors 10.3.6.48 | This IE shall not be present if only ASC 0 and ASC 1 are defined. If this IE is absent, value is the value of "Persistence scaling factors" for the previous PRACH in the list if value exists |  |
| >AC-to-ASC mapping | CV-SIB5-MD |  | AC-to-ASC mapping 10.3.6.1 | Only present in SIB 5 and in SIB 5bis. Default value is the value of "AC-to-ASC mapping" for the previous PRACH in the list. NOTE: The first occurrence is then MP in SIB5 and in SIB5bis. |  |
| >CHOICE mode | MP |  |  |  |  |
| >>FDD |  |  |  |  |  |
| >>>Primary CPICH TX power | MD |  | Primary CPICH TX power 10.3.6.61 | Default value is the value of "Primary CPICH TX power" for the previous PRACH in the list. NOTE: The first occurrence is then MP. |  |
| >>>Constant value | MD |  | Constant value 10.3.6.11 | Default value is the value of "Constant value" for the previous PRACH in the list. NOTE: The first occurrence is then MP. |  |
| >>>PRACH power offset | MD |  | PRACH power offset 10.3.6.54 | Default value is the value of "PRACH power offset" for the previous PRACH in the list. NOTE: The first occurrence is then MP. |  |
| >>>RACH transmission parameters | MD |  | RACH transmission parameters 10.3.6.67 | Default value is the value of "RACH transmission parameters" for the previous PRACH in the list. NOTE: The first occurrence is then MP. |  |
| >>>AICH info | MD |  | AICH info 10.3.6.2 | Default value is the value of "AICH info" for the previous PRACH in the list. NOTE: The first occurrence is then MP. |  |
| >>TDD |  |  |  | (no data) |  |

| Condition | Explanation |
| --- | --- |
| SIB5-MD | The information element is present only in SIB 5 and in SIB5bis. In SIB 5 and in SIB 5bis it is mandatory with default. |
| Add-RACH-TFS | The information element is MP if the IE "Additional RACH TFS for CCCH" is included. Otherwise the information element is not needed. |

NOTE:	If the setting of the PRACH information results in that a combination of a signature, preamble scrambling code and subchannel corresponds to a RACH with different TFS and/or TFCS, then for that combination only the TFS/TFCS of the PRACH listed first is valid, where PRACHs listed in System Information Block type 5 or 5bis shall be counted first.

#### 10.3.6.56	Predefined PhyCH configuration

This information element concerns a pre- defined configuration of physical channel parameters.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Uplink radio resources |  |  |  |  |
| Uplink DPCH info | MP |  | Uplink DPCH info Pre 10.3.6.90 |  |
| Downlink radio resources |  |  |  |  |
| Downlink information common for all radio links | OP |  | Downlink information common for all radio links Pre 10.3.6.26 |  |

#### 10.3.6.57	Primary CCPCH info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>TX Diversity indicator | MP |  | Boolean | TRUE indicates that transmit diversity is used. |  |
| >TDD |  |  |  |  |  |
| >>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>3.84 and 7.68 Mcps TDD |  |  |  |  | REL-4 |
| >>>>CHOICE SyncCase | OP |  |  |  |  |
| >>>>>Sync Case 1 |  |  |  |  |  |
| >>>>>>Timeslot | MP |  | Integer (0...14) | PCCPCH timeslot |  |
| >>>>>Sync Case 2 |  |  |  |  |  |
| >>>>>>Timeslot | MP |  | Integer(0..6) |  |  |
| >>>1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>>>TSTD indicator | MP |  | TSTD indicator 10.3.6.85a |  | REL-4 |
| >>Cell parameters ID | OP |  | Cell parameters Id 10.3.6.9 | The Cell parameters ID is described in [32]. |  |
| >>SCTD indicator | MP |  | SCTD indicator 10.3.6.70a |  |  |

#### 10.3.6.58	Primary CCPCH info post

NOTE:	Only for TDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE TDD option | MP |  |  |  | REL-4 |
| >3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>CHOICE SyncCase | MP |  |  |  |  |
| >>>Sync Case 1 |  |  |  |  |  |
| >>>>Timeslot | MP |  | Integer (0...14) | PCCPCH timeslot |  |
| >>>Sync Case 2 |  |  |  |  |  |
| >>>>Timeslot | MP |  | Integer(0..6) |  |  |
| >7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>CHOICE SyncCase | MP |  |  |  | REL-7 |
| >>>Sync Case 1 |  |  |  |  | REL-7 |
| >>>>Timeslot | MP |  | Integer (0…14) | PCCPCH timeslot | REL-7 |
| >>>Sync Case 2 |  |  |  |  | REL-7 |
| >>>>Timeslot | MP |  | Integer(0..6) |  | REL-7 |
| >1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>TSTD indicator | MP |  | TSTD indicator 10.3.6.85a |  | REL-4 |
| Cell parameters ID | MP |  | Cell parameters Id 10.3.6.9 | The Cell parameters ID is described in [32]. |  |
| SCTD indicator | MP |  | SCTD indicator 10.3.6.70a |  |  |

#### 10.3.6.59	Primary CCPCH TX Power

NOTE:	Only for TDD.

| Information Element/group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Primary CCPCH Tx Power | MP |  | Integer(6..43) | In dBm |

#### 10.3.6.60	Primary CPICH info

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Primary scrambling code | MP |  | Integer(0..511) |  |

#### 10.3.6.61	Primary CPICH Tx power

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Primary CPICH Tx Power | MP |  | Integer(-10..50) | Power in dBm. |

#### 10.3.6.62	Primary CPICH usage for channel estimation

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Primary CPICH usage for channel estimation | MP |  | Enumerated(Primary CPICH may be used, Primary CPICH shall not be used) |  |

#### 10.3.6.63	PUSCH info

NOTE:	Only for 1.28 Mcps and 3.84 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TFCS ID | MD |  | Integer(1..8) | Default value is 1 |
| Common timeslot info | OP |  | Common timeslot info 10.3.6.10 |  |
| PUSCH timeslots and codes | OP |  | Uplink Timeslots and Codes 10.3.6.94 |  |

#### 10.3.6.63a	PUSCH info VHCR

NOTE:	Only 7.68 Mcps TDD (REL-7).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| TFCS ID | MD |  | Integer(1..8) | Default value is 1 | REL-7 |
| Common timeslot info | OP |  | Common timeslot info 10.3.6.10 |  | REL-7 |
| PUSCH timeslots and codes VHCR | OP |  | Uplink Timeslots and Codes VHCR 10.3.6.94b |  | REL-7 |

#### 10.3.6.64	PUSCH Capacity Allocation info

NOTE:	Only for TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE PUSCH allocation | MP |  |  |  |  |
| >PUSCH allocation pending |  |  |  | (no data) |  |
| >PUSCH allocation assignment |  |  |  |  |  |
| >>PUSCH allocation period info | MP |  | Allocation Period Info 10.3.6.4 |  |  |
| >>PUSCH power control info | OP |  | PUSCH power control info 10.3.6.65 |  |  |
| >>CHOICE Configuration | MP |  |  |  |  |
| >>>Old configuration |  |  |  |  |  |
| >>>>TFCS ID | MD |  | Integer(1..8) | Default is 1. |  |
| >>>>PUSCH Identity | MP |  | Integer(1..hiPUSCHidentities) |  |  |
| >>>New 1.28 Mcps TDD or 3.84 Mcps TDD configuration |  |  |  |  |  |
| >>>>PUSCH info | MP |  | PUSCH info 10.3.6.63 |  |  |
| >>>>PUSCH Identity | OP |  | Integer(1.. hiPUSCHidentities) |  |  |
| >>>New 7.68 Mcps TDD configuration |  |  |  |  | REL-7 |
| >>>>PUSCH info VHCR | MP |  | PUSCH info VHCR 10.3.6.63a |  | REL-7 |
| >>>>PUSCH Identity | OP |  | Integer(1.. hiPUSCHidentities) |  | REL-7 |

#### 10.3.6.65	PUSCH power control info

NOTE:	Only for TDD.

Interference level measured for a frequency at the UTRAN access point used by UE to set PUSCH output power.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UL target SIR | MP |  | Real (-11 .. 20 by step of 0.5 dB) |  |  |
|  |  |  |  | For 1.28 Mcps TDD this parameter represents PRXPUSCHdes with range Integer(-120…-58 by step of 1) dBm | REL-4 |
| CHOICE TDD option | MP |  |  |  | REL-4 |
| >3.84 Mcps TDD |  |  |  | (no data) | REL-4 |
| >7.68 Mcps TDD |  |  |  | (no data) | REL-7 |
| >1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>TPC Step Size | OP |  | Integer (1, 2, 3) | In dB | REL-4 |
| >>Beacon PL Est. | OP |  | Enumerated (TRUE) | The presence of this IE indicates that the UE may take into account path loss estimated from beacon function physical channels. The absence indicates that UE may not take into account path loss estimated from beacon function physical channels | REL-6 |

#### 10.3.6.66	PUSCH system information

NOTE:	Only for 1.28 Mcps or 3.84 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PUSCH information | MP | 1 to <maxPUSCH> |  |  |  |
| >PUSCH Identity | MP |  | Integer(1..hiPUSCHidentities) |  |  |
| >PUSCH info | MP |  | PUSCH info 10.3.6.63 |  |  |
| >SFN Time Info | CH-Block17 |  | SFN Time Info 10.3.6.75 |  |  |
| >USCH TFS | OP |  | Transport format set 10.3.5.23 |  |  |
| >USCH Transport Channels | OP | 1 to <maxTrCH> |  | If PUSCH is configured for 3.84 Mcps TDD in Rel-5 this IE may be included. | REL-5 |
| >> USCH Transport channel identity | MP |  | Transport channel identity 10.3.5.18 |  | REL-5 |
| >>USCH TFS | MP |  | Transport format set 10.3.5.23 |  | REL-5 |
| >USCH TFCS | OP |  | Transport Format Combination Set 10.3.5.20 |  |  |

| Condition | Explanation |
| --- | --- |
| Block17 | This IE is not needed in System Information Block 17. Otherwise it is optional. |

#### 10.3.6.66a	PUSCH system information VHCR

NOTE:	Only for 7.68 Mcps TDD (REL-7).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PUSCH information | MP | 1 to <maxPUSCH> |  |  | REL-7 |
| >PUSCH Identity | MP |  | Integer(1..hiPUSCHidentities) |  | REL-7 |
| >PUSCH info VHCR | MP |  | PUSCH info VHCR 10.3.6.63a |  | REL-7 |
| >SFN Time Info | CH-Block17 |  | SFN Time Info 10.3.6.75 |  | REL-7 |
| >USCH TFS | OP |  | Transport format set 10.3.5.23 |  | REL-7 |
| >USCH Transport Channels | OP | 1 to <maxTrCH> |  | If PUSCH is configured for 7.68 Mcps TDD in Rel-7 this IE may be included. | REL-7 |
| >> USCH Transport channel identity | MP |  | Transport channel identity 10.3.5.18 |  | REL-7 |
| >>USCH TFS | MP |  | Transport format set 10.3.5.23 |  | REL-7 |
| >USCH TFCS | OP |  | Transport Format Combination Set 10.3.5.20 |  | REL-7 |

| Condition | Explanation |
| --- | --- |
| Block17 | This IE is not needed in System Information Block 17. Otherwise it is optional. |

#### 10.3.6.67	RACH transmission parameters

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Mmax | MP |  | Integer(1..32) | Maximum number of preamble cycles |
| NB01min | MP |  | Integer(0..50) | Sets lower bound for random back-off |
| NB01max | MP |  | Integer(0..50) | Sets upper bound for random back-off |

#### 10.3.6.68	Radio link addition information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  |  |
| Cell ID | OP |  | Cell ID 10.3.2.2 |  | REL-4 |
| CHOICE DPCH info | MP |  |  |  | REL-6 |
| >Downlink DPCH info for each RL |  |  | Downlink DPCH info for each RL 10.3.6.21 |  |  |
| >Downlink F-DPCH info for each RL |  |  | Downlink F-F-DPCH info for each RL 10.3.6.23ob |  | REL-6 |
| >Radio Links without DPCH/F-DPCH info |  |  | Radio Links without DPCH/F-DPCH info 10.3.6.147 |  | REL-12 |
| E-HICH Information | OP |  | E-HICH Info 10.3.6.101 |  | REL-6 |
| E-RGCH Information | OP |  | E-RGCH Info 10.3.6.102 |  | REL-6 |
| F-TPICH info | OP |  | F-TPICH Info 10.3.6.127 |  | REL-11 |
| Target cell preconfiguration information | OP |  | Target cell preconfiguration information 10.3.6.79a |  | REL-8 |

#### 10.3.6.69	Radio link removal information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  |

#### 10.3.6.69a	E-DCH reconfiguration information

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-DCH RL Info new serving cell | OP |  |  |  | REL-6 |
| >Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 | Indicates scheduling E-DCH cell from the active set cells. | REL-6 |
| >E-AGCH Info | MP |  | E-AGCH Info 10.3.6.100 |  | REL-6 |
| >E-ROCH Info | OP |  | E-ROCH Info FDD 10.3.6.146 |  | REL-11 |
| >Serving Grant | OP |  |  |  | REL-6 |
| >>Serving Grant value | OP |  | Integer (0..37,38) | (0..37) indicates E-DCH serving grant index as defined in [15]; index 38 means zero grant. | REL-6 |
| >>Primary/Secondary Grant Selector | MP |  | Enumerated (primary, secondary) | Indicates whether the Serving Grant is received with a Primary E-RNTI or Secondary E-RNTI. | REL-6 |
| >E-DPCCH/DPCCH power offset | OP |  | Integer (0..8) | Refer to quantization of the power offset in [28]. | REL-6 |
| >Reference E-TFCIs | OP | 1 to 8 |  | See [29]. | REL-6 |
| >>Reference E-TFCI | MP |  | Integer (0..127) |  | REL-6 |
| >>Reference E-TFCI PO | MP |  | Integer (0..29, | Refer to quantization of the power offset in [28]. | REL-6 |
|  |  |  | 30, 31) | Values 30 and 31 are only used for E-TFCI > ETFCI Boost | REL-7 |
| >Power Offset for Scheduling Info | OP |  | Integer (0..6) | Only used when no MACd PDU’s are included in the same MAC-e or MAC-i PDU. Unit is in dB. | REL-6 |
| >3-Index-Step Threshold | OP |  | Integer (0..37) | Refers to an index in the “SG-Table” (see [15]). NOTE 1 | REL-6 |
| >2-Index-Step Threshold | OP |  | Integer (0..37) | Refers to an index in the “SG-Table” (see [15]). NOTE 1 | REL-6 |
| >E-HICH Information | OP |  | E-HICH Info 10.3.6.101 | This IE is not present if the serving E-DCH cell is added to the active set with this message. | REL-6 |
| >CHOICE E-RGCH Information | OP |  |  | This IE is not present if the serving E-DCH cell is added to the active set with this message | REL-6 |
| >>E-RGCH Information | MP |  | E-RGCH Info 10.3.6.102 |  | REL-6 |
| >>E-RGCH release indicator |  |  |  | (no data) | REL-6 |
| E-DCH RL Info other cells | OP | 1 to <maxEDCHRL> |  | This IE is not allowed to include information on a RL added by this message, except in the case of an update to target cell preconfiguration information. | REL-6 |
| >Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-6 |
| >CHOICE E-HICH Information | OP |  |  |  | REL-6 |
| >>E-HICH Information | MP |  | E-HICH Info 10.3.6.101 |  | REL-6 |
| >>E-HICH release indicator |  |  |  | (no data) | REL-6 |
| >>Secondary TB E-HICH release indicator |  |  |  | (no data) | REL-11 |
| >CHOICE E-RGCH Information | OP |  |  |  | REL-6 |
| >>E-RGCH Information | MP |  | E-RGCH Info 10.3.6.102 |  | REL-6 |
| >>E-RGCH release indicator |  |  |  | (no data) | REL-6 |
| NOTE 1: If the 3-index-Step Threshold value is greater than 2-index-Step Threshold, the UE behaviour is unspecified. |  |  |  |  |  |

#### 10.3.6.69b	E-DCH reconfiguration information same serving cell

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-DPCCH/DPCCH power offset | OP |  | Integer (0..8) | This IE is not present if the serving E-DCH cell is changed with this message. Refer to quantization of the power offset in [28]. | REL-7 |
| Reference E-TFCIs | OP | 1 to 8 |  | This IE is not present if the serving E-DCH cell is changed with this message. See [29]. | REL-7 |
| >Reference E-TFCI | MP |  | Integer (0..127) |  | REL-7 |
| >Reference E-TFCI PO | MP |  | Integer (0..29, | Refer to quantization of the power offset in [28]. | REL-7 |
|  |  |  | 30, 31) | Values 30 and 31 are only used for E-TFCI > ETFCI Boost | REL-7 |

#### 10.3.6.70	Void

#### 10.3.6.70a	SCTD indicator

NOTE:	Only for TDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SCTD indicator | MP |  | Boolean | TRUE indicates that SCTD is used |

#### 10.3.6.71	Secondary CCPCH info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Secondary scrambling code | OP |  | Secondary scrambling code 10.3.6.74 | May only be sent for SCCPCH channels not carrying the PCH. |  |
| >>STTD indicator | MP |  | STTD Indicator 10.3.6.78 |  |  |
| >>Spreading factor | MP |  | Integer(4, 8, 16, 32, 64, 128, 256) |  |  |
| >>Code number | MP |  | Integer(0..Spreading factor - 1) |  |  |
| >>Pilot symbol existence | MP |  | Boolean | TRUE means the existence. |  |
| >>TFCI existence | MP |  | Boolean | TRUE indicates that TFCI is used. When spreading factor is less than or equal to 64, FALSE indicates that TFCI is not used and therefore DTX is used in the TFCI field. |  |
| >>Fixed or Flexible Position | MP |  | Enumerated (Fixed, Flexible) |  |  |
| >>Timing Offset | MD |  | Integer(0..38144 by step of 256) | Chip Delay of the Secondary CCPCH relative to the Primary CCPCH. Default value is 0. |  |
| >1.28 Mcps TDD or 3.84 Mcps TDD |  |  |  |  |  |
| >>Offset | MP |  | Integer (0...Repetition Period -1) | SFN modulo Repetition period = offset. Repetition period is the one indicated in the accompanying Common timeslot info IE |  |
| >>Common timeslot info | MP |  | Common timeslot info 10.3.6.10 |  |  |
| >>Individual timeslot info | MP |  | Individual timeslot info 10.3.6.37 |  |  |
| >>Code List | MP | 1 to 16 |  |  |  |
| >>>Channelisation Code | MP |  | Enumerated((16/1)..(16/16)) |  |  |
| >7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>Offset | MP |  | Integer (0...Repetition Period -1) | SFN modulo Repetition period = offset. Repetition period is the one indicated in the accompanying Common timeslot info IE | REL-7 |
| >>Common timeslot info | MP |  | Common timeslot info 10.3.6.10 |  | REL-7 |
| >>Individual timeslot info | MP |  | Individual timeslot info 10.3.6.37 |  | REL-7 |
| >>Code List | MP | 1 to 32 |  |  | REL-7 |
| >>>Channelisation Code | MP |  | Enumerated((32/1)..(32/32)) |  | REL-7 |

#### 10.3.6.71a	Secondary CCPCH info MBMS

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  | REL-6 |
| >FDD |  |  |  |  | REL-6 |
| >>Secondary scrambling code | OP |  | Secondary scrambling code 10.3.6.74 | May only be sent for SCCPCH channels not carrying the PCH. | REL-6 |
| >>STTD indicator | MP |  | STTD Indicator 10.3.6.78 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3, the UE behaviour upon reception of this IE is unspecified. | REL-6 |
| >>Spreading factor | MP |  | Integer(4, 8, 16, 32, 64, 128, 256) |  | REL-6 |
| >>Code number | MP |  | Integer(0..Spreading factor – 1) |  | REL-6 |
| >>Timing Offset | MD |  | Integer(0..38144 by step of 256) | Chip Delay of the Secondary CCPCH relative to the Primary CCPCH. Default value is 0. | REL-6 |
| >>CHOICE Modulation | CV-MBSFN |  |  |  | REL-7 |
| >>>QPSK | MP |  |  | (no data) | REL-7 |
| >>>16QAM | MP |  |  |  | REL-7 |
| >>>>CPICH secondary CCPCH power offset | MP |  | Integer(-11..4) |  | REL-7 |
| >1.28/3.84 Mcps TDD |  |  |  |  | REL-6 |
| >> Common timeslot info MBMS | MP |  | Common timeslot info MBMS 10.3.6.10a | Interleaving mode, TFCI coding and puncturing limit for Secondary CCPCH info MBMS | REL-6 |
| >>Downlink Timeslots and Codes | MP |  | Downlink timeslots and codes 10.3.6.32 | One or more timeslots and codes for S-CCPCH supporting MBMS MTCH | REL-6 |
| >> MBSFN Special Time Slot | OP |  | Time Slot LCR Extension 10.3.6.83a | For 1.28 Mcps TDD MBSFN only mode, this IE indicates the SCCPCH is deployed on the MBSFN Special Time Slot [30]. The IE ‘Timeslot number’ in ‘Downlink Timeslots and Codes’ shall be ignored if this IE appears. | REL-7 |
| >>Modulation | MP |  | Enumerated (QPSK, 16QAM) |  | REL-7 |
| >7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>Common timeslot info MBMS | MP |  | Common timeslot info MBMS 10.3.6.10a | Interleaving mode, TFCI coding and puncturing limit for Secondary CCPCH info MBMS | REL-7 |
| >>Downlink Timeslots and Codes VHCR | MP |  | Downlink timeslots and codes VHCR 10.3.6.32a | One or more timeslots and codes for S-CCPCH supporting MBMS MTCH | REL-7 |
| >>Modulation | MP |  | Enumerated (QPSK, 16QAM) |  | REL-7 |
| >3.84 Mcps TDD MBSFN IMB |  |  |  |  | REL-8 |
| >>Secondary scrambling code | OP |  | Secondary scrambling code 10.3.6.74 | Not applicable: if present, the UE behaviour is unspecified | REL-8 |
| >>STTD indicator | MP |  | STTD Indicator 10.3.6.78 | Not applicable: the UE behaviour is unspecified | REL-8 |
| >>Spreading factor | MP |  | Integer (4, 8, 16, 32, 64, 128, 256) | SF = 256; other values not applicable | REL-8 |
| >>Code number | MP |  | Integer (0..Spreading factor – 1) | Only code numbers 2 to 15 are applicable | REL-8 |
| >>Timing Offset | MD |  | Integer (0..38144 by step of 256) | Not applicable: if present, the UE behaviour is unspecified | REL-8 |
| >>CHOICE Modulation | CV-MBSFN |  |  |  | REL-8 |
| >>>QPSK | MP |  |  | (no data) | REL-8 |
| >>>16QAM | MP |  |  | Not applicable: the UE behaviour is unspecified | REL-8 |
| >>>>CPICH secondary CCPCH power offset | MP |  | Integer (-11..4) |  | REL-8 |

| Condition | Explanation |
| --- | --- |
| MBSFN | This IE is mandatory present when the cell on which this IE is sent is operating in MBSFN mode according to subclause 8.1.1.6.3. Otherwise the IE is not needed. |

NOTE:	For FDD, the S-CCPCH signalled in this IE is always configured with flexible position, TFCI is always present and pilot is always absent.

#### 10.3.6.71b	Secondary CCPCH info MBMS Diff

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Secondary scrambling code | OP |  | Secondary scrambling code 10.3.6.74 | May only be sent for SCCPCH channels not carrying the PCH. | REL-7 |
| STTD indicator | MP |  | STTD Indicator 10.3.6.78 |  | REL-7 |
| Spreading factor | OP |  | Integer(4, 8, 16, 32, 64, 128, 256) |  | REL-7 |
| Code number | OP |  | Integer(0..Spreading factor – 1) |  | REL-7 |
| Timing Offset | OP |  | Integer(0..38144 by step of 256) | Chip Delay of the Secondary CCPCH relative to the Primary CCPCH. | REL-7 |

NOTE:	For FDD, the S-CCPCH signalled in this IE is always configured with flexible position, TFCI is always present and pilot is always absent.

#### 10.3.6.72	Secondary CCPCH system information

| Information element | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Secondary CCPCH system information list | MP | 1 to <maxSCCPCH> |  |  |  |
| >Secondary CCPCH info | MP |  | Secondary CCPCH info 10.3.6.71 | Note 1 |  |
| >TFCS | MD |  | Transport format combination set 10.3.5.20 | For FACHs and PCH Default value is the value of "TFCS" for the previous SCCPCH in the list. NOTE: The first occurrence is then MP. |  |
| >FACH/PCH information list | MD | 1 to <maxFACHPCH> |  | Default value is the value of "FACH/PCH" for the previous SCCPCH in the list. NOTE: The first occurrence is then MP. |  |
| >>TFS | MP |  | Transport format set 10.3.5.23 | For each FACH and PCH Note 2 |  |
| >>Transport channel identity | MP |  | Transport channel identity 10.3.5.18 |  |  |
| >>CTCH indicator | MP |  | Boolean | The value TRUE indicates that a CTCH is mapped on the FACH, and FALSE that no CTCH is mapped. |  |
| >PICH info | OP |  | PICH info 10.3.6.49 | PICH info is present only when PCH is multiplexed on Secondary CCPCH |  |
| MCCH configuration information | CV-SIB type 5 |  | MCCH configuration information 10.3.9a.13 | Included if MCCH is on an S-CCPCH used also for Non- MBMS Note 3 | REL-6 |

NOTE 1:	The secondary CCPCHs carrying a PCH shall be listed first.

NOTE 2:	TFS for PCH shall be the first "FACH/PCH information" in the list if a PCH exists for the respective secondary CCPCH.

NOTE 3:	If the IE "MCCH configuration information is included", it applies to the last S-CCPCH in the IE "Secondary CCPCH system information list" and to the last FACH included in the IE "FACH/PCH information list".

| Condition | Explanation |
| --- | --- |
| SIB type 5 | The IE is optional if the IE " Secondary CCPCH system information" is included in the SIB type 5 or 5bis, otherwise the IE is not needed in the message |

#### 10.3.6.72a	Secondary CCPCH system information MBMS

| Information element | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Secondary CCPCH system information | MP |  |  | An S-CCPCH carrying MCCH and possibly also MTCH and MSCH | REL-6 |
| >Secondary CCPCH info MBMS | MP |  | Secondary CCPCH info MBMS 10.3.6.71a |  | REL-6 |
| >TFCS | MP |  | Transport format combination set 10.3.5.20 |  | REL-6 |
| >FACH carrying MCCH | MP |  |  |  | REL-6 |
| >>TFS | MP |  | Transport format set 10.3.5.23 |  | REL-6 |
| >>MCCH configuration information | MP |  | MCCH configuration information 10.3.9a.13 |  | REL-6 |
| >FACH carrying MTCH list | OP | 1 to <maxFACHPCH> |  |  | REL-6 |
| >>TFS | MP |  | Transport format set 10.3.5.23 |  | REL-6 |
| >Scheduling information | OP |  |  |  | REL-6 |
| >>FACH carrying MSCH | MP |  | Transport format set 10.3.5.23 | Transport channel carrying MSCH | REL-6 |

#### 10.3.6.72b	Secondary cell MIMO parameters

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Configuration info | MP |  |  |  | REL-9 |
| >Continue |  |  | (no data) | (no data)Used in reconfigurations without interruption of dual cell operation with MIMO. | REL-9 |
| >New configuration |  |  |  |  | REL-9 |
| >> MIMO N_cqi_typeA/M_cqi ratio | OP |  | Enumerated(1/2, 2/3, 3/4, 4/5, 5/6, 6/7, 7/8, 8/9, 9/10, 1/1) |  | REL-9 |
| >>MIMO pilot configuration | OP |  | MIMO pilot configuration 10.3.6.41b |  | REL-9 |
| >>Precoding weight set restriction | OP |  | Enumerated (TRUE) | If present, the UE applies precoding weight set restriction to this secondary cell. | REL-9 |

#### 10.3.6.73	Secondary CPICH info

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Secondary scrambling code | MD |  | Secondary scrambling code 10.3.6.74 | Default is the same scrambling code as for the Primary CPICH |
| Channelisation code | MP |  | Integer(0..255) | SF=256 |

#### 10.3.6.74	Secondary scrambling code

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Secondary scrambling code | MP |  | Integer(1..15) |  |

#### 10.3.6.74a	Serving HS-DSCH cell information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| ACK | OP |  | Integer(0..8 | Refer to quantization of the power offset in [28] | REL-6 |
|  |  |  | 9..10) |  | REL-11 |
| NACK | OP |  | Integer(0..8 | Refer to quantization of the power offset in [28] | REL-6 |
|  |  |  | 9..10) |  | REL-11 |
| HARQ_preamble_mode | MP |  | Integer(0, 1) | 1 indicates that preamble and postamble are used on the HS-DPCCH – see [29] | REL-6 |
| Primary CPICH info | OP |  | Primary CPICH info 10.3.6.60 | Indicate the new HS-DSCH serving cell | REL-6 |
| Downlink HS-PDSCH Information | OP |  | Downlink HS_PDSCH Information 10.3.6.23a |  | REL-6 |
| HARQ Info | OP |  | HARQ info 10.3.5.7a |  | REL-6 |
| MAC-hs reset indicator | OP |  | Enumerated (TRUE) | TRUE Indicates the MAC-hs/ehs entity needs to be reset. | REL-6 |
| MAC-hs reset indicator for Multiflow assisting cells | OP |  | Enumerated (TRUE) | TRUE indicates that the MAC-hs/ehs entity related to the assisting cells needs to be reset. | REL-11 |

#### 10.3.6.74b	Serving Cell Change Parameters

The IE "Serving Cell Change Parameters" is used prepare the UE for enhanced serving cell change.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Serving Cell Change MAC reset | MP |  | Boolean | TRUE means the MAC-hs/ehs entity needs to be reset, FALSE means TPC combination Index is used to determine if MAC-hs/ehs reset is needed, 8.2.2.3 | REL-8 |
| Serving Cell Change Message Type | MP |  | Enumerated (RadioBearerSetup, RadioBearerReconfiguration, TransportChannelReconfiguration, PhysicalChannelReconfiguration) | The reconfiguration procedure configuration used for Target cell preconfiguration, 8.2.2.4 | REL-8 |
| Serving Cell Change Transaction Id | MP |  | Integer (0..3) | Transaction Id that shall be used as the RRC Transaction Identifier with the corresponding reponse message configured by the IE “Serving Cell Change Message Type” | REL-8 |
| Enhanced Serving Cell Change for Event 1c Support Indicator | OP |  | Enumerated (TRUE) | TRUE means that Enhanced Serving Cell Change for Event 1c is enabled | REL-12 |

#### 10.3.6.75	SFN Time info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Activation time SFN | MP |  | Integer (0..4095) | System frame number start of the physical channel existence. |
| Duration | MP |  | Integer(1..4096) | Total number of frames the physical channel will exist. |

#### 10.3.6.75a	Special Burst Scheduling

NOTE:	Only for TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Special Burst Generation Period | MP |  | Integer (0..7) | Value represents number of radio frames 0 = 2 frames, 1 = 4 frames, 2 = 8 frames, 3 = 16 frames, 4 = 32 frames, 5 = 64 frames, 6 = 128 frames, 7 =256 frames |

#### 10.3.6.76	Void

#### 10.3.6.77	Void

#### 10.3.6.78	STTD indicator

NOTE:	Only for FDD

Indicates whether STTD is used or not.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| STTD Indicator | MP |  | Boolean | TRUE means that STTD is used |

#### 10.3.6.78o	SYNC_UL codes bitmap

NOTE:	Only for 1.28 Mcps TDD.

| Information Element/ Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SYNC_UL codes bitmap | MP |  | Bitstring(8) | Each bit indicates availability of a SYNC_UL code, where the SYNC_UL codes are numbered "code 0" to "code 7". The value 1 of a bit indicates that the corresponding SYNC_UL code can be used. The value 0 of a bit indicates that the corresponding SYNC_UL code can not be used. | REL-7 |

#### 10.3.6.78a	SYNC_UL info

NOTE:	Only for 1.28 Mcps TDD.

| Information Element/ Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SYNC_UL codes bitmap | MP |  | SYNC_UL codes bitmap 10.3.6.78o |  | REL-4 |
| PRXUpPCHdes | MP |  | Integer(-120…-58 by step of 1) | In dBm | REL-4 |
| Power Ramp Step | MP |  | Integer(0,1,2,3) | In dB | REL-4 |
| Max SYNC_UL Transmissions | MP |  | Integer(1,2,4,8) | Maximum numbers of SYNC_UL transmissions in a power ramping sequence. | REL-4 |
| Mmax | MP |  | Integer(1..32) | Maximum number of synchronisation attempts. | REL-4 |
| E-RUCCH SYNC_UL codes bitmap | MP |  | SYNC_UL codes bitmap 10.3.6.78o |  | REL-7 |

#### 10.3.6.78b		TDD MBSFN Information

NOTE:	Only for TDD

This information indicates which timeslots are assigned to TDD MBSFN and the scrambling codes assigned to each timeslot.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Time slot list | MP | 1.. <maxTS> |  |  | REL-7 |
| >Timeslot Number | MP |  | Timeslot Number 10.3.6.84 |  | REL-7 |
| >Cell parameters ID | MP |  | Cell parameters Id 10.3.6.9 |  | REL-7 |

#### 10.3.6.78c		LCR TDD MBSFN Information

NOTE:	Only for 1.28Mcps TDD

This information indicates which timeslots of the secondary frequency of a multi-frequency cell are assigned to 1.28Mcps TDD MBSFN and the scrambling codes related to each timeslot.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Time slot info list | MP | 1 to <maxMBSFNclusters> |  |  | REL-7 |
| >Frequency index | MP |  | Integer (1..<maxMBSFNclusters>) | For 1.28 Mcps TDD index pointing to a frequency indicated in the SIB 11. | REL-7 |
| >Time slot list | MP | 1 to <maxTS> |  |  | REL-7 |
| >>Timeslot Number | MP |  | Timeslot Number 10.3.6.84 |  | REL-7 |
| >>Cell parameters ID | MP |  | Cell parameters Id 10.3.6.9 |  | REL-7 |

#### 10.3.6.78d	SYNC_UL info for E-RUCCH

NOTE:	Only for 1.28 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PRXUpPCHdes | CV-Synchronisation parameters |  | Integer(-120…-58 by step of 1) | In dBm | REL-7 |
| Power Ramp Step | CV-Synchronisation parameters |  | Integer(0,1,2,3) | In dB | REL-7 |
| Max SYNC_UL Transmissions | CV-Synchronisation parameters |  | Integer(1,2,4,8) | Maximum numbers of SYNC_UL transmissions in a power ramping sequence. | REL-7 |
| Mmax | MP |  | Integer(1..32) | Maximum number of synchronisation attempts. | REL-7 |
| E-RUCCH SYNC_UL codes bitmap | MP |  | SYNC_UL codes bitmap 10.3.6.78o |  | REL-7 |

| Condition | Explanation |
| --- | --- |
| Synchronisation parameters | This IE is mandatory present if the IE “Synchronisation parameters” does not exist, Otherwise it is not needed. |

#### 10.3.6.79	TDD open loop power control

This information element contains parameters for open loop power control setting for TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Primary CCPCH Tx Power | MP |  | Primary CCPCH Tx Power 10.3.6.59 | For path loss calculation |  |
| CHOICE TDD option | MP |  |  |  | REL-4 |
| >3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>Alpha | OP |  | Alpha 10.3.6.5 |  |  |
| >>PRACH Constant Value | MP |  | Constant Value TDD 10.3.6.11a | Operator controlled PRACH Margin |  |
| >>DPCH Constant Value | MP |  | Constant Value TDD 10.3.6.11a | Operator controlled UL DPCH Margin |  |
| >>PUSCH Constant Value | OP |  | Constant Value TDD 10.3.6.11a | Operator controlled PUSCH Margin |  |
| >>UE positioning related parameters | CV-IPDLs |  |  |  | REL-4 |
| >>>IPDL-Alpha | MP |  | Alpha 10.3.6.5 |  | REL-4 |
| >>>Max power increase | MP |  | Integer (0..3) | In db | REL-4 |
| >7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>Alpha | OP |  | Alpha 10.3.6.5 |  | REL-7 |
| >>PRACH Constant Value | MP |  | Constant Value TDD 10.3.6.11a | Operator controlled PRACH Margin | REL-7 |
| >>DPCH Constant Value | MP |  | Constant Value TDD 10.3.6.11a | Operator controlled UL DPCH Margin | REL-7 |
| >>PUSCH Constant Value | OP |  | Constant Value TDD 10.3.6.11a | Operator controlled PUSCH Margin | REL-7 |
| >>UE positioning related parameters | CV-IPDLs |  |  |  | REL-7 |
| >>>IPDL-Alpha | MP |  | Alpha 10.3.6.5 |  |  |
| >>>Max power increase | MP |  | Integer (0..3) | In db | REL-7 |
| >1.28 Mcps TDD |  |  |  | (no data) | REL-4 |

| Condition | Explanation |
| --- | --- |
| IPDLs | This IE is present only if idle periods are applied |

#### 10.3.6.79a	Target cell preconfiguration information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |  |
| --- | --- | --- | --- | --- | --- | --- |
| Activation time offset | OP |  | Interger (0..255) | If present, the “Activation time offset” IE is used by UE to calculate Activation time for inclusion in measurement report for event 1d or 1c. | REL-8 |  |
| New H-RNTI | MP |  | H-RNTI 10.3.3.14a |  | REL-8 |  |
| New Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-8 |  |
| New Secondary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-8 |  |
| Serving HS-DSCH cell information | MP |  | Serving HS-DSCH cell information 10.3.6.74a |  | REL-8 |  |
| E-DCH reconfiguration information | MP |  | E-DCH reconfiguration information 10.3.6.69a |  | REL-8 |  |
| DTX-DRX timing information | OP |  | DTX-DRX timing information 10.3.6.34b |  |  | REL-8 |
| DTX-DRX Information | OP |  | DTX-DRX Information 10.3.6.34a |  |  | REL-8 |
| HS-SCCH less Information | OP |  | HS-SCCH less Information 10.3.6.36ab |  |  | REL-8 |
| MIMO parameters | OP |  | MIMO parameters 10.3.6.41a |  | REL-8 |  |
| MIMO mode with four transmit antennas parameters | OP |  | MIMO mode with four transmit antennas parameters 10.3.6.142 |  | REL-11 |  |
| Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a |  | REL-8 |  |
| Additional downlink secondary cell info list FDD | OP | 2 |  |  | REL-10 |  |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-10 |  |
| Additional downlink secondary cell info list FDD 2 | OP | 4 |  |  | REL-11 |  |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-11 |  |
| Uplink secondary cell info FDD | OP |  | Uplink secondary cell info FDD 10.3.6.115 | FDD only | REL-9 |  |
| E-DCH reconfiguration information on secondary UL frequency | OP |  | E-DCH reconfiguration information on secondary UL frequency 10.3.6.121 | FDD only | REL-9 |  |
| Uplink CLTD info FDD | OP |  | Uplink CLTD info FDD 10.3.6.125 | FDD only | REL-11 |  |
| F-TPICH reconfiguration info | OP |  | F-TPICH reconfiguration info 10.3.6.128 |  | REL-11 |  |
| Uplink OLTD info FDD | OP |  | Uplink OLTD info FDD 10.3.6.126 | FDD only | REL-11 |  |
| UL 16QAM configuration | OP |  | UL 16QAM configuration 10.3.6.86o |  | REL-11 |  |
| UL 64QAM configuration | OP |  | UL 64QAM configuration 10.3.6.86b |  | REL-11 |  |
| Uplink MIMO info FDD | OP |  | Uplink MIMO info FDD 10.3.6.145 | FDD only | REL-11 |  |
| DPCCH2 info FDD | OP |  | DPCCH2 info FDD 10.3.6.148 | FDD only | REL-12 |  |

#### 10.3.6.80	TFC Control duration

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TFC Control duration | MP |  | Integer (1, 2, 4, 8, 16, 24, 32, 48, 64, 128, 192, 256, 512) | Defines the period in multiples of 10 ms frames for which the defined TFC sub-set is to be applied. |

#### 10.3.6.81	Void

#### 10.3.6.82	TGPSI

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TGPSI | MP |  | Integer(1..MaxTGPS) | Transmission Gap Pattern Sequence Identifier Establish a reference to the compressed mode pattern sequence. Up to <MaxTGPS> simultaneous compressed mode pattern sequences can be used. |

#### 10.3.6.83	Time info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Activation time | MD |  | Activation time 10.3.3.1 | Frame number start of the physical channel existence. Default value is "Now" |
| Duration | MD |  | Integer(1..4096, infinite) | Total number of frames the physical channel will exist. Default value is "infinite". |

#### 10.3.6.83a	Time Slot LCR Extension

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Time Slot LCR Extension |  |  | ENUMERATED (TS7, …) | TS7 indicates the MBSFN Special Timeslot for 1.28 Mcps TDD MBSFN Dedicated Carrier. Three spare values are reserved. |

#### 10.3.6.84	Timeslot number

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE TDD option | MP |  |  |  | REL-4 |
| >3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>Timeslot number | MP |  | Integer(0..14) | Timeslot within a frame |  |
| >7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>Timeslot number | MP |  | Integer(0..14) | Timeslot within a frame | REL-7 |
| >1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>Timeslot number | MP |  | Integer(0..6) | Timeslot within a subframe | REL-4 |

#### 10.3.6.85	TPC combination index

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| TPC combination index | MP |  | Integer(0..5) | Radio links with the same index have TPC bits, which for the UE are known to be the same. |  |
|  |  |  |  | Radio links that are part of the E-DCH active set and with the same index have acknowledgement feedback signals on the E-HICH which for the UE are known to be the same. | REL-6 |
|  |  |  |  | Radio links with the same index have TPI bits, which for the UE are known to be the same. | REL-11 |

#### 10.3.6.85a	TSTD indicator

NOTE:	Only for 1.28 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| TSTD indicator | MD |  | Boolean | Default value is TRUE | REL-4 |

#### 10.3.6.86	TX Diversity Mode

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Tx diversity Mode | MP |  | Enumerated (none, STTD, closed loop mode1) |  |

#### 10.3.6.86o	UL 16QAM configuration

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UL 16QAM settings | CV-2msTTI |  | UL 16QAM settings 10.3.6.86a | Presence of this IE indicates that the UE should operate in 16QAM mode; absence indicates that the UE is not to operate in 16QAM mode. See Note 2. | REL-7 |
| MAC-es/e reset indicator | OP |  | Enumerated (TRUE) | TRUE Indicates the MAC-es/e or MAC-i/is entity needs to be reset. | REL-7 |
| E-TFCI table index | OP |  | Integer (0..1) | Indicates which standardised E-TFCI TB size table shall be used. See Note 1. | REL-7 |
| NOTE 1: If the UE is operating in 16QAM mode, the value of “E-TFCI table index” is increased by 2, and indices in the SG-Table refer to Scheduling Grant Table 2 in [15]. |  |  |  |  |  |
| NOTE 2: If this IE is not present, the indices signaled on the E-AGCH refer to the Mapping of Absolute Grant Value Table 16B in [27]. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2msTTI | This IE is present only if 2ms TTI is configured, otherwise it is not needed. |

#### 10.3.6.86a	UL 16QAM settings

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| BetaEd gain E-AGCH table selection | MP |  | Integer (0..1) | The value 0 indicates that the indices signaled on the E-AGCH refer to the Mapping of Absolute Grant Value Table 16B in [27], and the value 1 indicates Mapping using Absolute Grant Value Table 16B.1 in [27]. | REL-7 |

#### 10.3.6.86b	UL 64QAM configuration

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UL 64QAM settings | OP |  | UL 64QAM settings 10.3.6.86c | Presence of this IE indicates that the UE should operate in 64QAM mode; absence indicates that the UE is not to operate in 64QAM mode. | REL-11 |
| MAC-is/i reset indicator | OP |  | Enumerated (TRUE) | TRUE Indicates that MAC-i/is entity needs to be reset. | REL-11 |

#### 10.3.6.86c	UL 64QAM settings

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| BetaEd gain E-AGCH table selection | MP |  | Integer (0..1) | The value 0 indicates that the indices signaled on the E-AGCH refer to the Mapping of Absolute Grant Value Table 16B in [27], and the value 1 indicates Mapping using Absolute Grant Value Table 16B.1 in [27]. | REL-11 |

#### 10.3.6.87	UL interference

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UL interference | MP |  | Integer (-110..-70) | In dBm |

NOTE:	In TDD, this IE is a timeslot specific value.

#### 10.3.6.87a	UL interference TDD

NOTE:	Only for TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TDD UL interference | MP |  | Integer (-110..-52) | In dBm |

NOTE:	This IE is a timeslot specific value.

#### 10.3.6.87b	Uplink DPCH code info for Common E-DCH

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Scrambling code type | MP |  | Enumerated(short, long) |  | REL-8 |
| Scrambling code number | CV-Short |  | Integer(0..16777215) | NOTE1 | REL-8 |
| NOTE1: If this IE is not included, the value is given by: Scrambling code number = (offset + common E-DCH resource list position) mod 16777216 where "offset" is the value of the last occurrence of the IE "scrambling code number". "common E-DCH resource list position" indicates the Common E-DCH resource Configuration Information number by order of appearance. The value of the "common E-DCH resource list position" is zero for the first occurrence. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Short | This IE is mandatory present for the first occurrence. Otherwise, this IE is optional. |

#### 10.3.6.88	Uplink DPCH info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Uplink DPCH power control info | OP |  | Uplink DPCH power control info 10.3.6.91 |  |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Scrambling code type | MP |  | Enumerated(short, long) |  |  |
| >>Scrambling code number | MP |  | Integer(0..16777215) |  |  |
| >>Number of DPDCH | MD |  | Integer(1..maxDPDCH-UL, | Default value is 1. Number of DPDCH is 1 in HANDOVER TO UTRAN COMMAND. |  |
|  |  |  | 0) | Value 0 may only be used if target configuration has no uplink DPDCH. | REL-6 |
| >>Spreading factor | CV-DPDCHpresent |  | Integer(4, 8, 16, 32, 64, 128, 256) | Minimum allowed SF of the channelisation code for data part |  |
| >>TFCI existence | MP |  | Boolean | TRUE means existence. |  |
| >>Number of FBI bits | OP |  | Integer (1, 2) | In bits. |  |
| >>Number of TPC bits | CV- Neither TFCI nor FBI present |  | Integer (4) | In bits | REL-7 |
| >>Puncturing Limit | CV-DPDCHpresent |  | Real(0.40 ..1 by step of 0.04) |  |  |
| >TDD |  |  |  |  |  |
| >>Uplink Timing Advance Control | OP |  | Uplink Timing Advance Control 10.3.6.96 |  |  |
| >>UL CCTrCH List | OP | 1 to <maxCCTrCH> |  | UL physical channels to establish or reconfigure list. |  |
| >>>TFCS ID | MD |  | Integer(1..8) | Default value is 1. |  |
| >>>UL target SIR | MP |  | Real (-11 .. 20 by step of 0.5dB) | In dB |  |
|  |  |  |  | For 1.28 Mcps TDD this parameter represents PRXDPCHdes with range Integer(-120…-58 by step of 1) dBm | REL-4 |
| >>>Time info | MP |  | Time info 10.3.6.83 |  |  |
| >>>Common timeslot info | MD |  | Common timeslot info 10.3.6.10 | Default is the current Common timeslot info |  |
| >>> CHOICE TDD option |  |  |  |  | REL-7 |
| >>>> 3.84 Mcps TDD |  |  |  |  | REL-7 |
| >>>>>Uplink DPCH timeslots and codes | MD |  | Uplink Timeslots and Codes 10.3.6.94 | Default is to use the old timeslots and codes. |  |
| >>>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>>Uplink DPCH timeslots and codes VHCR | MD |  | Uplink Timeslots and Codes VHCR 10.3.6.94b | Default is to use the old timeslots and codes | REL-7 |
| >>>> 1.28 Mcps TDD |  |  |  |  | REL-7 |
| >>>>>Uplink DPCH timeslots and codes LCR | MD |  | Uplink Timeslots and Codes LCR 10.3.6.94a | Default is to use the old timeslots and codes. | REL-7 |
| >>UL CCTrCH List to Remove | OP | 1..<maxCCTrCH> |  | UL physical channels to remove list |  |
| >>>TFCS ID | MP |  | Integer(1..8) |  |  |

| Condition | Explanation |
| --- | --- |
| DPDCHpresent | This IE is mandatory if the target configuration includes a DPDCH, otherwise it is not needed. |
| Neither TFCI nor FBI present | This IE is optional if the IE “TFCI existence” is set to FALSE and the IE “Number of FBI bits” is not present, otherwise it is not needed. |

#### 10.3.6.89	Uplink DPCH info Post

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Uplink DPCH power control info | MP |  | Uplink DPCH power control info Post 10.3.6.92 |  |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Scrambling code type | MP |  | Enumerated(short, long) |  |  |
| >>Reduced scrambling code number | MP |  | Integer(0..8191) | Sub-range of values for initial use upon handover to UTRAN. |  |
| >>Spreading factor | MP |  | Integer(4, 8, 16, 32, 64, 128, 256) | SF of the channelisation code for data part There is only one DPDCH for this case |  |
| >1.28 Mcps TDD and 3.84 Mcps TDD |  |  |  |  |  |
| >>Uplink Timing Advance Control | OP |  | Uplink Timing Advance Control 10.3.6.96 |  |  |
| >>Uplink DPCH timeslots and codes | MP |  | Uplink Timeslots and Codes 10.3.6.94 |  |  |
| >7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>Uplink Timing Advance Control | OP |  | Uplink Timing Advance Control 10.3.6.96 |  | REL-7 |
| >>Uplink DPCH timeslots and codes | MP |  | Uplink Timeslots and Codes VHCR 10.3.6.94b |  | REL-7 |

#### 10.3.6.90	Uplink DPCH info Pre

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Uplink DPCH power control info | OP |  | Uplink DPCH power control info Pre 10.3.6.93 |  |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>TFCI existence | MP |  | Boolean | TRUE means existence. Default value is TRUE See NOTE 1. |  |
| >>Number of TPC bits | CV-No TFCI |  | Integer (4) | In bits | REL-7 |
| >>Puncturing Limit | MP |  | Real(0.40 ..1 by step of 0.04) |  |  |
| >TDD |  |  |  |  |  |
| >>Common timeslot info | MP |  | Common Timeslot Info 10.3.6.10 |  |  |
| NOTE 1: Number of FBI bits is not signalled but 0 is implied as transmit diversity cannot be setup here. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Single | This IE is mandatory present if the IE "Number of DPDCH" is "1" and not needed otherwise. |
| No TFCI | This IE is optional if the IE “TFCI existence” is set to FALSE, otherwise it is not needed. |

#### 10.3.6.91	Uplink DPCH power control info

Parameters used by UE to set DPCH initial output power and to use for closed-loop power control in FDD and 1.28 Mcps TDD and parameters for uplink open loop power control in 3.84 Mcps TDD and 7.68 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>DPCCH Power offset | MP |  | Integer(-164,..-6 by step of 2) | In dB |  |
| >>PC Preamble | MP |  | Integer (0..7) | In number of frames |  |
| >>SRB delay | MP |  | Integer(0..7) | In number of frames |  |
| >>Power Control Algorithm | MP |  | Enumerated (algorithm 1, algorithm 2) | Specifies algorithm to be used by UE to interpret TPC commands |  |
| >>TPC step size | CV-algo |  | Integer (1, 2) | In dB |  |
| >>ACK | OP |  | Integer (0..8 | Refer to quantization of the power offset in [28] | REL-5 |
|  |  |  | 9..10) |  | REL-11 |
| >>NACK | OP |  | Integer (0..8 | refer to quantization of the power offset in [28] | REL-5 |
|  |  |  | 9..10) |  | REL-11 |
| >>Ack-Nack repetition factor | OP |  | Integer(1..4) |  | REL-5 |
| >>Ack-Nack repetition factor for Multiflow assisting cells | CV-Multiflow |  | Integer(1..4) | If present, this IE indicates that all the assisting cells use the Ack-Nack repetition factor indicated by this IE. Absence of this IE indicates that there is no Ack-Nack repetition for multiflow assisting cell(s). | REL-11 |
| >>HARQ_preamble_mode | MP |  | Integer (0, 1) | 1 indicates that preamble and postamble are used on the HS-DPCCH – see [29] | REL-6 |
| >TDD |  |  |  |  |  |
| >>UL target SIR | OP |  | Real (-11 .. 20 by step of 0.5dB) | In dB |  |
|  |  |  |  | For 1.28 Mcps TDD this parameter represents PRXDPCHdes with range Integer(-120…-58 by step of 1) dBm | REL-4 |
| >>CHOICE UL OL PC info | MP |  |  |  |  |
| >>>Broadcast UL OL PC info |  |  | Null | No data |  |
| >>>Individually Signalled | OP |  |  |  |  |
| >>>>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>>>Individual timeslot interference info | MP | 1 to <maxTS> |  |  |  |
| >>>>>>>Individual timeslot interference | MP |  | Individual timeslot interference 10.3.6.38 |  |  |
| >>>>>>DPCH Constant Value | MP |  | Constant Value TDD 10.3.6.11a | Quality Margin |  |
| >>>>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>>>Individual timeslot interference info | MP | 1 to <maxTS> |  |  | REL-7 |
| >>>>>>>Individual timeslot interference | MP |  | Individual timeslot interference 10.3.6.38 |  | REL-7 |
| >>>>>>DPCH Constant Value | MP |  | Constant Value TDD 10.3.6.11a | Quality Margin | REL-7 |
| >>>>>1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>>>>>Beacon PL Est. | CV-houtran |  | Enumerated (TRUE) | The presence of this IE indicates that the UE may take into account path loss estimated from beacon function physical channels. The absence indicates that UE may not take into account path loss estimated from beacon function physical channels | REL-6 |
| >>>>>>TPC step size | MP |  | Integer(1,2,3) |  | REL-4 |
| >>>>Primary CCPCH Tx Power | OP |  | Primary CCPCH Tx Power 10.3.6.59 | For Pathloss Calculation |  |

| Condition | Explanation |
| --- | --- |
| algo | The IE is mandatory present if the IE "Power Control Algorithm" is set to "algorithm 1", otherwise the IE is not needed |
| houtran | This IE is optional in Cell Update Confirm, Physical Channel Reconfiguration, Radio Bearer Reconfiguration, Radio Bearer Release, Radio Bearer Setup, Transport Channel Reconfiguration and Uplink Physical Channel Control. The IE is not included in the Handover To UTRAN Command |
| Multiflow | The IE is optional when the Multiflow operation is configured, otherwise it is not needed. |

#### 10.3.6.91a	Uplink DPCH power control info for Common E-DCH

Parameters used by UE to set DPCH initial output power and to use for closed-loop power control in FDD when Enhanced Uplink in CELL_FACH state and Idle mode is applied.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Power Control Algorithm | MP |  | Enumerated (algorithm 1, algorithm 2) | Specifies algorithm to be used by UE to interpret TPC commands | REL-8 |
| TPC step size | CV-algo |  | Integer (1, 2) | In dB | REL-8 |
| ACK | OP |  | Integer (0..8) | Refer to quantization of the power offset in [28] | REL-8 |
| NACK | OP |  | Integer (0..8) | refer to quantization of the power offset in [28] | REL-8 |
| Ack-Nack repetition factor | OP |  | Integer(1..4) |  | REL-8 |

| Condition | Explanation |
| --- | --- |
| algo | The IE is mandatory present if the IE "Power Control Algorithm" is set to "algorithm 1", otherwise the IE is not needed |

#### 10.3.6.92	Uplink DPCH power control info Post

Parameters used by UE to set DPCH initial output power and to use for closed-loop power control.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>DPCCH Power offset | MP |  | Integer(-110..-50 by step of 4) | In dB |  |
| >>PC Preamble | MP |  | Integer (0..7) | in number of frames |  |
| >>SRB delay | MP |  | Integer (0..7) | In number of frames |  |
| >TDD |  |  |  |  |  |
| >>UL target SIR | MP |  | Real (-11 .. 20 by step of 0.5dB) | In dB |  |
|  |  |  |  | For 1.28 Mcps TDD this parameter represents PRXDPCHdes with range Integer(-120…-58 by step of 1) dBm | REL-4 |
| >>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>UL Timeslot Interference | MP |  | UL Interference TDD 10.3.6.87a |  |  |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>UL Timeslot Interference | MP |  | UL Interference TDD 10.3.6.87a |  | REL-7 |
| >>>1.28 Mcps TDD |  |  |  | (no data) | REL-4 |

| Condition | Explanation |
| --- | --- |
| algo | The IE is mandatory present if the IE "Power Control Algorithm" is set to "algorithm 1", otherwise the IE is not needed |

#### 10.3.6.93	Uplink DPCH power control info Pre

Parameters used by UE to set DPCH initial output power and to use for closed-loop power control in FDD and parameters for uplink open loop power control in 3.84 Mcps TDD and 7.68 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Power Control Algorithm | MP |  | Enumerated (algorithm 1, algorithm 2) | Specifies algorithm to be used by UE to interpret TPC commands |  |
| >>TPC step size | CV-algo |  | Integer (1, 2) | In dB |  |
| >TDD |  |  |  | (No data) |  |
| >>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>DPCH Constant Value | MP |  | Constant Value TDD 10.3.6.11a | Quality Margin |  |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>DPCH Constant Value | MP |  | Constant Value TDD 10.3.6.11a | Quality Margin | REL-7 |
| >>>1.28 Mcps TDD |  |  |  | (no data) | REL-4 |

| Condition | Explanation |
| --- | --- |
| algo | The IE is mandatory present if the IE "Power Control Algorithm" is set to "algorithm 1", otherwise the IE is not needed |

#### 10.3.6.94	Uplink Timeslots and Codes

NOTE:	Only for 1.28 Mcps TDD and 3.84 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Dynamic SF usage | MP |  | Boolean |  |
| First Individual timeslot info | MP |  | Individual timeslot info 10.3.6.37 | Individual timeslot info for the first timeslot used by the physical layer. |
| First timeslot Code List | MP | 1..2 |  | Code list used in the timeslot. given in First individual timeslot info. |
| >Channelisation Code | MP |  | Enumerated((1/1),)(2/1),(2/2),(4/1)..(4/4),(8/1)..(8/8),(16/1)..(16/16)) |  |
| CHOICE more timeslots | MP |  |  |  |
| >No more timeslots |  |  |  | (no data) |
| >Consecutive timeslots |  |  |  |  |
| >>Number of additional timeslots | MP |  | Integer(1..maxTS-1) | The timeslots used by the physical layer shall be timeslots: N mod maxTS (N+1) mod maxTS ... (N+k) mod maxTS in that order, where N is the timeslot number in the First individual timeslot info and k the Number of additional timeslots. The additional timeslots shall use the same parameters (e.g. channelisation codes, midamble shifts etc.) as the first timeslot. |
| >Timeslot list |  |  |  |  |
| >>Additional timeslot list | MP | 1 to <maxTS-1> |  | The first instance of this parameter corresponds to the timeslot that shall be used second by the physical layer, the second to the timeslot that shall be used third and so on. |
| >>>CHOICE parameters | MP |  |  |  |
| >>>>Same as last |  |  |  |  |
| >>>>>Timeslot number | MP |  | Timeslot Number 10.3.6.84 | This physical layer shall use the same parameters (e.g. channelisation codes, midamble shifts etc.) for this timeslot as for the last one. |
| >>>>New parameters |  |  |  |  |
| >>>>>Individual timeslot info | MP |  | Individual timeslot info 10.3.6.37 |  |
| >>>>>Code List | MP | 1..2 |  |  |
| >>>>>>Channelisation Code | MP |  | Enumerated((1/1),)(2/1),(2/2),(4/1)..(4/4),(8/1)..(8/8),(16/1)..(16/16)) |  |

#### 10.3.6.94a	Uplink Timeslots and Codes LCR

NOTE:	Only for 1.28 Mcps TDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Dynamic SF usage | MP |  | Boolean |  |
| First Individual timeslot info | MP |  | Individual timeslot info 10.3.6.37 | Individual timeslot info for the first timeslot used by the physical layer. |
| First timeslot Code List | MP | 1..2 |  | Code list used in the timeslot. given in First individual timeslot info. |
| >Channelisation Code | MP |  | Enumerated((1/1),)(2/1),(2/2),(4/1)..(4/4),(8/1)..(8/8),(16/1)..(16/16)) |  |
| >PLCCH Info | CV-no_DL_DPCH |  | PLCCH Info 10.3.6.50a |  |
| CHOICE more timeslots | MP |  |  |  |
| >No more timeslots |  |  |  | (no data) |
| >Consecutive timeslots |  |  |  |  |
| >>Number of additional timeslots | MP |  | Integer(1..maxTS-1) | The timeslots used by the physical layer shall be timeslots: N mod maxTS (N+1) mod maxTS … (N+k) mod maxTS in that order, where N is the timeslot number in the First individual timeslot info and k the Number of additional timeslots. The additional timeslots shall use the same parameters (e.g. channelisation codes, midamble shifts etc.) as the first timeslot. |
| >Timeslot list |  |  |  |  |
| >>Additional timeslot list | MP | 1 to <maxTS-1> |  | The first instance of this parameter corresponds to the timeslot that shall be used second by the physical layer, the second to the timeslot that shall be used third and so on. |
| >>>CHOICE parameters | MP |  |  |  |
| >>>>Same as last |  |  |  |  |
| >>>>>Timeslot number | MP |  | Timeslot Number 10.3.6.84 | This physical layer shall use the same parameters (e.g. channelisation codes, midamble shifts etc.) for this timeslot as for the last one. |
| >>>>New parameters |  |  |  |  |
| >>>>>Individual timeslot info | MP |  | Individual timeslot info 10.3.6.37 |  |
| >>>>>Code List | MP | 1..2 |  |  |
| >>>>>>Channelisation Code | MP |  | Enumerated((1/1),)(2/1),(2/2),(4/1)..(4/4),(8/1)..(8/8),(16/1)..(16/16)) |  |
| >>>>>> PLCCH Info | CV-no_DL_DPCH |  | PLCCH Info 10.3.6.50a | Default is same as set of parameter values as assigned to last timeslot |

| Condition | Explanation |
| --- | --- |
| No_DL_DPCH | This IE is mandatory present for 1.28 Mcps TDD only in the case that Uplink Timeslots and Codes are assigned for UL DPCH and no DL DPCH is allocated to the UE. |

#### 10.3.6.94b	Uplink Timeslots and Codes VHCR

NOTE:	Only for 7.68 Mcps TDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Dynamic SF usage | MP |  | Boolean |  | REL-7 |
| First Individual timeslot info | MP |  | Individual timeslot info 10.3.6.37 | Individual timeslot info for the first timeslot used by the physical layer. | REL-7 |
| First timeslot Code List | MP | 1..2 |  | Code list used in the timeslot. given in First individual timeslot info. | REL-7 |
| >Channelisation Code | MP |  | Enumerated((1/1),)(2/1),(2/2),(4/1)..(4/4),(8/1)..(8/8.16/1) .. (16/16).(32/1)..(32/32)) |  | REL-7 |
| CHOICE more timeslots | MP |  |  |  | REL-7 |
| >No more timeslots |  |  |  | (no data) | REL-7 |
| >Consecutive timeslots |  |  |  |  | REL-7 |
| >>Number of additional timeslots | MP |  | Integer(1..maxTS-1) | The timeslots used by the physical layer shall be timeslots: N mod maxTS (N+1) mod maxTS ... (N+k) mod maxTS in that order, where N is the timeslot number in the First individual timeslot info and k the Number of additional timeslots. The additional timeslots shall use the same parameters (e.g. channelisation codes, midamble shifts etc.) as the first timeslot. | REL-7 |
| >Timeslot list |  |  |  |  | REL-7 |
| >>Additional timeslot list | MP | 1 to <maxTS-1> |  | The first instance of this parameter corresponds to the timeslot that shall be used second by the physical layer, the second to the timeslot that shall be used third and so on. | REL-7 |
| >>>CHOICE parameters | MP |  |  |  | REL-7 |
| >>>>Same as last |  |  |  |  | REL-7 |
| >>>>>Timeslot number | MP |  | Timeslot Number 10.3.6.84 | This physical layer shall use the same parameters (e.g. channelisation codes, midamble shifts etc.) for this timeslot as for the last one. | REL-7 |
| >>>>New parameters |  |  |  |  | REL-7 |
| >>>>>Individual timeslot info | MP |  | Individual timeslot info 10.3.6.37 |  | REL-7 |
| >>>>>Code List | MP | 1..2 |  |  | REL-7 |
| >>>>>>Channelisation Code | MP |  | Enumerated((1/1),)(2/1),(2/2),(4/1)..(4/4),(8/1)..(8/8).(16/1).. (16/16).(32/1)..(32/32)) |  | REL-7 |

#### 10.3.6.95	Uplink Timing Advance

NOTE:	Only for 3.84 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UL Timing Advance | MP |  | Integer (0..63) | Absolute timing advance value to be used to avoid large delay spread at the NodeB |  |

#### 10.3.6.95a	Extended Uplink Timing Advance

NOTE:	Only for 3.84 and 7.68 Mcps TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE TDD mode |  |  |  |  | REL-7 |
| >3.84 Mcps TDD |  |  |  |  | REL-7 |
| >>Extended UL Timing Advance | MP |  | Integer (0..255) | Absolute timing advance value to be used to avoid large delay spread at the NodeB | REL-7 |
| >7.68.Mcps TDD |  |  |  |  | REL-7 |
| >>Extended UL Timing Advance | MP |  | Integer (0..511) | Absolute timing advance value to be used to avoid large delay spread at the NodeB | REL-7 |

#### 10.3.6.96	Uplink Timing Advance Control

NOTE:	Only for TDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Timing Advance | MP |  |  |  |  |
| >Disabled |  |  | Null | Indicates that no timing advance is applied |  |
| >Enabled |  |  |  |  |  |
| >>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>UL Timing Advance | MD |  | Uplink Timing Advance 10.3.6.95 | Absolute timing advance value to be used to avoid large delay spread at the NodeB. Default value is the existing value for uplink timing advance. |  |
| >>Extended Timing Advance | OP |  | Extended UL Timing Advance Control 10.3.6.95a |  | REL-7 |
| >>>>Activation Time | OP |  | Activation Time 10.3.3.1 | Frame number timing advance is to be applied. This IE is required when a new UL Timing Advance adjustment is specified and Activation Time is not otherwise specified in the RRC message. |  |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Extended UL Timing Advance | MD |  | Extended Uplink Timing Advance 10.3.6.95a | Absolute timing advance value to be used to avoid large delay spread at the NodeB. Default value is the existing value for uplink timing advance. | REL-7 |
| >>>>Activation Time | OP |  | Activation Time 10.3.3.1 | Frame number timing advance is to be applied. This IE is required when a new UL Timing Advance adjustment is specified and Activation Time is not otherwise specified in the RRC message. | REL-7 |
| >>1.28 Mcps TDD |  |  |  | (no data) | REL-4 |
| >>>Uplink synchronisation parameters | MD |  |  | Default: Uplink synchronisation step size is 1. Uplink synchronisation frequency is 1. | REL-4 |
| >>>>Uplink synchronisation step size | MP |  | Integer(1..8) | This parameter specifies the step size to be used for the adjustment of the uplink transmission timing | REL-4 |
| >>>>Uplink synchronisation frequency | MP |  | Integer(1..8) | This parameter specifies the frequency of the adjustment of the uplink transmission timing | REL-4 |
| >>>Synchronisation parameters | CV-HandoverType |  |  |  |  |
| >>>>SYNC_UL codes bitmap | MP |  | SYNC_UL codes bitmap 10.3.6.78o |  | REL-4 |
| >>>>FPACH info | MP |  | FPACH info 10.3.6.35a |  | REL-4 |
| >>>>PRXUpPCHdes | MP |  | Integer(-120…-58 by step of 1) | In dBm | REL-4 |
| >>>>SYNC_UL procedure | MD |  |  | Default is: Max SYNC_UL Transmission is 2. Power Ramp Step is 2. | REL-4 |
| >>>>>Max SYNC_UL Transmissions | MP |  | Integer(1,2,4,8) | Maximum numbers of SYNC_UL transmissions in a power ramping sequence. | REL-4 |
| >>>>>Power Ramp Step | MP |  | Integer(0,1,2,3) | In dB | REL-4 |

| Condition | Explanation |
| --- | --- |
| HandoverType | This IE is mandatory present if it is inter-RAT handover, otherwise it is optional. |

#### 10.3.6.97	E-DCH Info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MAC-es/e reset indicator | OP |  | Enumerated (TRUE) | TRUE Indicates the MAC-es/e or MAC-i/is entity needs to be reset. | REL-6 |
| CHOICE mode | MP |  |  |  | REL-7 |
| >FDD |  |  |  |  | REL-7 |
| >>E-DPCCH info | OP |  | E-DPCCH Info 10.3.6.98 |  | REL-6 |
| >>E-DPDCH info | OP |  | E-DPDCH info 10.3.6.99 |  | REL-6 |
| >>Uplink MIMO info | OP |  | Uplink MIMO Info FDD 10.3.6.145 |  | REL-11 |
| >>Scheduled Transmission configuration | OP |  |  |  | REL-6 |
| >>>2ms scheduled transmission grant HARQ process allocation | MD |  | Bitstring (8) | MAC-d PDUs belonging to MAC-d flows not configured with a “Max MAC-e PDU contents size” are only allowed to be transmitted in those processes for which the bit is set to “1”. Bit 0 corresponds to HARQ process 0, bit 1 corresponds to HARQ process 1,… Default value is: transmission in all HARQ processes is allowed. Bit 0 is the first/leftmost bit of the bit string. | REL-6 |
| >>>Serving Grant | OP |  |  |  | REL-6 |
| >>>>Serving Grant value | MP |  | Integer (0..37,38) | (0..37) indicates E-DCH serving grant index as defined in [15]; index 38 means zero grant. | REL-6 |
| >>>>Primary/Secondary Grant Selector | MP |  | Enumerated (“primary”, “secondary”) | Indicates whether the Serving Grant is received with a Primary E-RNTI or Secondary E-RNTI | REL-6 |
| >>UL 16QAM settings | OP |  | UL 16QAM settings 10.3.6.86a | Presence of this IE indicates that the UE should operate in 16QAM mode; absence indicates that the UE is not to operate in 16QAM mode. See Note 1. | REL-7 |
| >>UL 64QAM settings | OP |  | UL 64QAM settings 10.3.6.86c | Presence of this IE indicates that the UE should operate in 64QAM mode; absence indicates that the UE is not to operate in 64QAM mode. | REL-11 |
| >TDD |  |  |  |  | REL-7 |
| >>E-RUCCH info | OP |  | E-RUCCH Info 10.3.6.103 |  | REL-7 |
| >>E-PUCH info | OP |  | E-PUCH Info 10.3.6.104 |  | REL-7 |
| >>Non-scheduled transmission grant info | OP |  | Non-scheduled transmission grant info 10.3.6.41c |  | REL-7 |
| NOTE 1: If this IE is not present, the indices signaled on the E-AGCH refer to the Mapping of Absolute Grant Value Table 16B in [27]. |  |  |  |  |  |

#### 10.3.6.97a	Multi-carrier E-DCH Info for LCR TDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Configuration info | MP |  |  |  | REL-10 |
| >Continue |  |  |  | (no data) | REL-10 |
| >New configuration |  |  |  |  | REL-10 |
| >>TSN-Length | OP |  | Enumerated (tsn-14bits) | The presence of this IE indicates the length of TSN shall be 14 bits, otherwise, the length of TSN shall be 6 bits. | REL-10 |
| >>Uplink Multi-carrier E-DCH Information List | OP | 1 to < maxTDD128Carrier-1> |  |  | REL-10 |
| >>>UARFCN | MP |  | Integer(0..16383) |  | REL-10 |
| >>>PRXdes_base | MP |  | Integer (-112..-50 by step of 1) | dBm. Reference desired power level for E-PUCH | REL-10 |
| >>>E-PUCH info | MD |  | E-PUCH Info for multi-carrier E-DCH 1.28Mcps TDD 10.3.6.104b | Default value is the value of the previous " E-PUCH info " in the list. Default value of first occurrence is the value of “E-PUCH info” in the IE “E-DCH info”. | REL-10 |
| >>>E-AGCH Info | MD |  | E-AGCH Info 1.28Mcps TDD 10.3.6.100a | Default value is the value of the previous " E-AGCH Info " in the list. Default value of first occurrence is the value of “E-AGCH Info” in the IE “Downlink information for each radio link”. | REL-10 |
| >>>E-HICH Info | MD |  | E-HICH Info 1.28Mcps TDD 10.3.6.101a | Default value is the value of the previous " E-HICH Information " in the list. Default value of first occurrence is the value of “E-HICH Information” in the IE “Downlink information for each radio link”. | REL-10 |
| >>Uplink Multi-carrier E-DCH Delete List | CV-serving_cell_change | 1 to < maxTDD128Carrier-1> |  |  | REL-10 |
| >>>UARFCN | MP |  | Integer(0..16383) |  | REL-10 |

| Condition | Explanation |
| --- | --- |
| serving_cell_change | This IE is not needed in the serving cell change. Otherwise, the IE is Optional |

#### 10.3.6.98	E-DPCCH Info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-DPCCH/DPCCH power offset | MP |  | Integer (0..8) | Refer to quantization of the power offset in [28] | REL-6 |
| Happy bit delay condition | MP |  | Enumerated (2ms, 10ms,20ms,50ms, 100ms, 200ms, 500ms, 1000ms) | To be used when determining the setting of the happy bit (see [15]) | REL-6 |
| E-TFC Boost Info | OP |  | E-TFC Boost Info 10.3.6.106 | Absence of this IE means that boosting of E-DPCCH is disabled | REL-7 |
| E-DPDCH power interpolation | OP |  | Boolean | TRUE means E-DPDCH power interpolation formula is used, FALSE means E-DPDCH power extrapolation formula is used for the computation of the gain factor ed according to [29]. Absence of this IE means that E-DPDCH power extrapolation formula is used for the computation of the gain factor ed according to [29]. | REL-7 |

#### 10.3.6.99	E-DPDCH Info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-TFCI table index | MP |  | Integer (0..1) | Indicates which standardised E-TFCI TB size table shall be used. See Note 3. | REL-6 |
| E-DCH minimum set E-TFCI | MD |  | Integer (0..127) | See [15]; Absence means no E-DCH minimum set | REL-6 |
| Reference E-TFCIs | MP | 1 to 8 |  | See [29] | REL-6 |
| >Reference E-TFCI | MP |  | Integer (0..127) |  | REL-6 |
| >Reference E-TFCI PO | MP |  | Integer (0..29, | Refer to quantization of the power offset in [28] | REL-6 |
|  |  |  | 30, 31) | Values 30 and 31 are only used for E-TFCI > ETFCI Boost | REL-7 |
| Minimum reduced E-DPDCH gain factor | OP |  | Enumerated (8/15, 11/15, 15/15, 21/15, 30/15, 42/15, 60/15, 84/15) | βed,k,reduced,min [29]; the default value is 8/15. If the configurable reduced E-DPDCH gain factor is not supported by the UE, the default value is used. | REL-8 |
| Maximum channelisation codes | MP |  | Enumerated (sf256, sf128, sf64, sf32, sf16, sf8, sf4, 2sf4, 2sf2, 2sf2and2sf4) |  | REL-6 |
| PLnon-max | MP |  | Real (0.44 ..1.0 by step of 0.04) | As defined in [27] | REL-6 |
| Scheduling Information Configuration | MP |  |  |  | REL-6 |
| >Periodicity for Scheduling Info – no grant | MD |  | Enumerated (everyEDCHTTI,4,10,20,50,100,200,500,1000) | Values in ms. Default value is “no report” NOTE 1. | REL-6 |
| >Periodicity for Scheduling Info – grant | MD |  | Enumerated (everyEDCHTTI,4,10,20,50,100,200,500,1000) | Values in ms. Default value is “no report” NOTE 1. | REL-6 |
| >Power Offset for Scheduling Info | MP |  | Integer (0..6) | Only used when no MACd PDUs are included in the same MAC-e or MAC-i PDU. Unit is in dB. | REL-6 |
| 3-Index-Step Threshold | MD |  | Integer (0..37) | Refers to an index in the “SG-Table” (see [15]). Default value is 0. NOTES 2, 3. | REL-6 |
| 2-Index-Step Threshold | MD |  | Integer (0..37) | Refers to an index in the “SG-Table” (see [15]). Default value is 0. NOTES 2, 3. | REL-6 |
| NOTE 1: If the Periodicity is set to 4ms and the E-DCH TTI is set to 10ms, the UE shall interpret the periodicity value as 10ms. NOTE 2: If the 3-index-Step Threshold value is greater than 2-index-Step Threshold, the UE behaviour is unspecified. NOTE 3: If the UE is operating in 16QAM mode, the value of “E-TFCI table index” is increased by 2, and indices in the SG-Table refer to Scheduling Grant Table 2 in [15]. If the UE is operating in 64QAM mode, the UE shall use E-TFCI table 4 as specified in [15] irrespective of the value of “E-TFCI table index, and indices in the SG-Table refer to Scheduling Grant Table 2 in [15]. |  |  |  |  |  |

#### 10.3.6.100	E-AGCH Info

Includes the configuration for the E-DCH related Absolute Grant Channel.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  | REL-7 |
| >FDD |  |  |  |  | REL-7 |
| >>E-AGCH Channelisation Code | MP |  | Integer (0..255) |  | REL-6 |
| >TDD |  |  |  |  | REL-7 |
| >>CHOICE TDD Option | MP |  |  |  | REL-7 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Long Term Grant Indicator | MD |  | Boolean | FALSE = Short Term TRUE = Long Term Default = FALSE (Short Term) | REL-7 |
| >>>>Length of TTRI field | MP |  | Integer(1..12) | Indicated length of the bitmap used to indicate which of the timeslots configured for E-DCH are allocated | REL-7 |
| >>>>E-AGCH set configuration | MP | <1 to maxNumE-AGCH> |  |  | REL-7 |
| >>>>>TS number | MP |  | Integer(0..14) |  | REL-7 |
| >>>>>Channelisation code | MP |  | Enumerated(16/1, 16/2,....16/16) |  | REL-7 |
| >>>>>CHOICE Burst Type |  |  |  |  | REL-7 |
| >>>>>>Type 1 |  |  |  |  | REL-7 |
| >>>>>>>Midamble allocation mode | MP |  | Enumerated(Default, Common, UE specific) |  | REL-7 |
| >>>>>>>Midamble configuration burst type 1 | MP |  | Enumerated(4, 8, 16) |  | REL-7 |
| >>>>>>>Midamble shift | CV-UE specific |  | Integer(0..15) |  | REL-7 |
| >>>>>>Type 2 |  |  |  |  |  |
| >>>>>>>Midamble allocation mode | MP |  | Enumerated(Default, Common, UE specific) |  | REL-7 |
| >>>>>>>Midamble configuration burst type 2 | MP |  | Enumerated(3, 6) |  | REL-7 |
| >>>>>>>Midamble shift | CV-UE specific |  | Integer(0..5) |  | REL-7 |
| >>>>E-AGCH BLER target | MP |  | Real(-3.15 to 0 step 0.05) |  | REL-7 |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Long Term Grant Indicator | MD |  | Boolean | FALSE = Short Term TRUE = Long Term Default = FALSE (Short Term) | REL-7 |
| >>>>Length of TTRI field | MP |  | Integer(1..12) | Indicated length of the bitmap used to indicate which of the timeslots configured for E-DCH are allocated | REL-7 |
| >>>>E-AGCH set configuration | MP | <1 to maxNumE-AGCH> |  |  | REL-7 |
| >>>>>TS number | MP |  | Integer(0..14) |  | REL-7 |
| >>>>>Channelisation code | MP |  | Enumerated(32/1, 32/2,....32/32) |  | REL-7 |
| >>>>>CHOICE Burst Type |  |  |  |  | REL-7 |
| >>>>>>Type 1 |  |  |  |  | REL-7 |
| >>>>>>>Midamble allocation mode | MP |  | Enumerated(Default, Common, UE specific) |  | REL-7 |
| >>>>>>>Midamble configuration burst type 1 | MP |  | Enumerated(4, 8, 16) |  | REL-7 |
| >>>>>>>Midamble shift | CV-UE specific |  | Integer(0..15) |  | REL-7 |
| >>>>>>Type 2 |  |  |  |  |  |
| >>>>>>>Midamble allocation mode | MP |  | Enumerated(Default, Common, UE specific) |  | REL-7 |
| >>>>>>>Midamble configuration burst type 2 | MP |  | Enumerated(3, 6) |  | REL-7 |
| >>>>>>>Midamble shift | CV-UE specific |  | Integer(0..5) |  | REL-7 |
| >>>>E-AGCH BLER target | MP |  | Real(-3.15 to 0 step 0.05) |  | REL-7 |
| >>>1.28 Mcps TDD |  |  |  |  | REL-7 |
| >>>>RDI Indicator | MP |  | Boolean | TRUE indicates a RDI field is present on E-AGCH type 1, | REL-7 |
| >>>>TPC step size | MP |  | Integer (1, 2, 3) | dB. | REL-7 |
| >>>>E-AGCH set configuration | MP | <1 to maxNumE-AGCH> |  |  | REL-7 |
| >>>>>Timeslot number | MP |  | Integer (0..6) |  | REL-7 |
| >>>>>First Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-7 |
| >>>>>Second Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-7 |
| >>>>>Midamble Allocation mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) |  | REL-7 |
| >>>>>Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) |  | REL-7 |
| >>>>> Midamble Shift | CV-UE specific |  | Integer (0..15) |  | REL-7 |
| >>>>E-AGCH BLER target | MP |  | Real(-3.15 to 0 step 0.05) |  | REL-7 |
| >>>>E-AGCH Inactivity Monitor Threshold | MD |  | Enumerated (0, 1, 2, 4, 8, 16, 32, 64, 128, 256, 512, infinity) | Units of subframes. Default value is "8". Four spare values are needed. | REL-8 |

#### 10.3.6.100a	E-AGCH Info 1.28Mcps TDD

Includes the configuration for the E-DCH related Absolute Grant Channel.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| RDI Indicator | MP |  | Boolean | TRUE indicates a RDI field is present on E-AGCH type 1, | REL-8 |
| TPC step size | MP |  | Integer (1, 2, 3) | dB. | REL-8 |
| E-AGCH set configuration | MP | <1 to maxNumE-AGCH> |  |  | REL-8 |
| >Timeslot number | MP |  | Integer (0..6) |  | REL-8 |
| >First Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-8 |
| >Second Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-8 |
| >Midamble Allocation mode | MP |  | Enumerated (Default midamble, Common midamble, UE specific midamble) |  | REL-8 |
| >Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) |  | REL-8 |
| >Midamble Shift | CV-UE specific |  | Integer (0..15) |  | REL-8 |
| E-AGCH BLER target | MP |  | Real(-3.15 to 0 step 0.05) |  | REL-8 |

| Condition | Explanation |
| --- | --- |
| UE specific | This IE is mandatory present when the value of the IE "Midamble Allocation Mode" is "UE specific midamble" and not needed otherwise. |

#### 10.3.6.101	E-HICH Info

Includes the configuration for the E-DCH related HARQ Acknowledgement Indicator Channel.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  | REL-7 |
| >FDD |  |  |  |  | REL-7 |
| >>Channelisation Code | MP |  | Integer (0..127) |  | REL-6 |
| >>Signature Sequence | MP |  | Integer (0..39) |  | REL-6 |
| >>Secondary TB Signature Sequence | OP |  | Integer (0..39) | The second E-HICH signature used to acknowledge the UL MIMO secondary stream transport block. | REL-11 |
| >TDD |  |  |  |  | REL-7 |
| >>CHOICE TDD Option | MP |  |  |  | REL-7 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-7 |
| >>>>NE-HICH | MP |  | Integer(4..44) | Minimum number of slots between start last active slot of E-DCH TTI and start of ACK/NACK on E-HICH (see[?}) | REL-7 |
| >>>>TS number | MP |  | Integer(0..14) |  | REL-7 |
| >>>>Channelisation Code | MP |  | Enumerated(16/1, 16/2,..16/16) |  | REL-7 |
| >>>>Burst type | MP |  | Enumerated(Type1, Type2) |  | REL-7 |
| >>>>Midambe allocation mode | MP |  | Enumerated(Default, Common) |  | REL-7 |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>NE-HICH | MP |  | Integer(4..44) | Minimum number of slots between start last active slot of E-DCH TTI and start of ACK/NACK on E-HICH (see[?}) | REL-7 |
| >>>>TS number | MP |  | Integer(0..14) |  | REL-7 |
| >>>>Channelisation Code | MP |  | Enumerated(32/1, 32/2,..32/32) |  | REL-7 |
| >>>>Burst type | MP |  | Enumerated(Type1, Type2) |  | REL-7 |
| >>>>Midambe allocation mode | MP |  | Enumerated(Default, Common) |  | REL-7 |
| >>>1.28 Mcps TDD |  |  |  |  | REL-7 |
| >>>> NE-HICH | MP |  | Integer (4..15) | Minimum number of slots between start last active slot of E-DCH TTI and start of ACK/NACK on E-HICH | REL-7 |
| >>>>E-HICH set configuration |  | <1 to maxNumE-HICH> |  |  | REL-7 |
| >>>>>EI | MP |  | Integer (0..3) |  | REL-7 |
| >>>>>Timeslot number | MP |  | Integer (0..6) |  | REL-7 |
| >>>>>Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-7 |
| >>>>>Midamble Allocation mode | MP |  | Enumerated (Default midamble, UE specific midamble) |  | REL-7 |
| >>>>>Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) |  | REL-7 |
| >>>>>Midamble Shift | CV-UE specific |  | Integer (0..15) |  | REL-7 |

#### 10.3.6.101a	E-HICH Info 1.28Mcps TDD

Includes the configuration for the E-DCH related HARQ Acknowledgement Indicator Channel.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| NE-HICH | MP |  | Integer (4..15) | Minimum number of slots between start last active slot of E-DCH TTI and start of ACK/NACK on E-HICH | REL-8 |
| E-HICH set configuration | MP | <1 to maxNumE-HICH> |  |  | REL-8 |
| >EI | MP |  | Integer (0..3) |  | REL-8 |
| >Timeslot number | MP |  | Integer (0..6) |  | REL-8 |
| >Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-8 |
| >Midamble Allocation mode | MP |  | Enumerated (Default midamble, UE specific midamble) |  | REL-8 |
| >Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) |  | REL-8 |
| >Midamble Shift | CV-UE specific |  | Integer (0..15) |  | REL-8 |

| Condition | Explanation |
| --- | --- |
| UE specific | This IE is mandatory present when the value of the IE "Midamble Allocation Mode" is "UE specific midamble" and not needed otherwise. |

#### 10.3.6.102	E-RGCH Info (FDD only)

Includes the configuration for the E-DCH related Relative Grant Channel. The E-RGCH is using the same channelisation code as configured for the E-HICH channel (see subclause 10.3.6.101).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Signature Sequence | MP |  | Integer(0..39) |  | REL-6 |
| RG combination index | MP |  | Integer(0..5) | Cells with an index equal to the index of the Serving E-DCH cell belong to the Serving E-DCH RLS. The E-RGCH from these cells have RG commands which for the UE are known to be the same as the RG commands from the Serving E-DCH cell. | REL-6 |

#### 10.3.6.103	E-RUCCH Info (TDD only)

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE TDD mode |  |  |  |  | REL-7 |
| >3.84/7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>E-RUCCH constant value | MP |  | Integer (-35..10) | Power to be used | REL-7 |
| >>E-RUCCH persistence scaling factor | MP |  | Real(0.9..0.2 by step of 0.1) |  | REL-7 |
| >>T-RUCCH | MD |  | Integer(40..320 by step of 40) | Default =200ms If T-RUCH expires before Grant is received then uE should send scheduling information again on E-RUCH | REL-7 |
| >>E-RUCCH timeslot number | MD |  | Timeslot number 10.3.6.84 | Default is same as PRACH timeslot number | REL-7 |
| >>E-RUCCH midamble | MD |  | Enumerated(Direct, Direct/Inverted) | Default is as PRACH midamble | REL-7 |
| >>T-Adv | MD |  | Enumerated (1, 2, 4, 8, 16, 32, 128, infinity) | Default is 1 second | REL-7 |
| >>T-SCHED | MD |  | Enumerated (0, 40ms, 80ms, 160ms, 25oms, 1sec, 2sec, 4sec) | Default is 0 second (include Scheduling Information in every MAC-e PDU) | REL-7 |
| >>CHOICE TDD option |  |  |  |  | REL-7 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-7 |
| >>>>CHOICE SF | MP |  |  |  | REL-7 |
| >>>>>SF16 |  |  |  |  | REL-7 |
| >>>>>>Channelisation Code List | MP | 1 to 8 |  |  | REL-7 |
| >>>>>>>Channelisation code | MP |  | Enumerated ((16/1)...(16/8)) | There is a 1:1 mapping between spreading code and midamble shift defined in [30] for channelisation codes (16/1) to (16/8). NOTE: channelisation codes (16/9) to (16/16) are not to be used. | REL-7 |
| >>>>>SF8 |  |  |  |  | REL-7 |
| >>>>>>Channelisation Code List | MP | 1 to 8 |  |  | REL-7 |
| >>>>>>>Channelisation Code | MP |  | Enumerated((8/1)..(8/8)) |  | REL-7 |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>CHOICE SF | MP |  |  |  | REL-7 |
| >>>>>SF32 |  |  |  |  | REL-7 |
| >>>>>>Channelisation Code List | MP | 1 to 16 |  |  | REL-7 |
| >>>>>>>Channelisation code | MP |  | Enumerated ((32/1)...(32/16)) | There is a 1:1 mapping between spreading code and midamble shift defined in [30] for channelisation codes (32/1) to (32/16). NOTE: channelisation codes (32/17) to (32/32) are not to be used. | REL-7 |
| >>>>>SF16 |  |  |  |  | REL-7 |
| >>>>>>Channelisation Code List | MP | 1 to 16 |  |  | REL-7 |
| >>>>>>>Channelisation Code | MP |  | Enumerated((16/1)..(16/16)) |  | REL-7 |
| >1.28 Mcps TDD |  |  |  |  | REL-7 |
| >>T-RUCCH | MD |  | Enumerated (20, 40, 60, 80, 120, 160, 200, 240, 280, 320, 400, 500, 600, 800, 1000, 2000) | Default = 120 ms If T-RUCCH expires before Grant is received then UE should send scheduling information again on E-RUCCH | REL-7 |
| >>N-RUCCH | MD |  | Integer (0…7) | Default = 3 Maximum number of retransmissions of scheduling information on E-RUCCH | REL-7 |
| >>T-WAIT | MD |  | Enumerated (everyEDCHTTI, 40, 80, 160, 320, 640, 1000, 2000) | Values in ms. Default = 40ms If T-WAIT expires then UE should send scheduling information on E-RUCCH | REL-7 |
| >>T-SI | MD |  | Enumerated (everyEDCHTTI, 20, 40, 60, 80, 160, 200) | Values in ms. Default = 40ms If periodic T-SI expires then UE should include scheduling information in a new MAC-e PDU | REL-7 |
| >>Extended Estimation Window | OP |  | Integer (2..5) | Values in 5ms TTI. Indicates how many next consecutive 5ms TTIs the UE should estimate whether there is a Grant valid before sending Scheduling Infomation via E-RUCCH | REL-7 |
| >>E-RUCCH Access Service class | OP | 1 to <maxASC> |  | If only "NumASC+1" (with, NumASC+1 < maxASC) ASCs are listed, the remaining (NumASC+2 through maxASC) ASCs are unspecified. | REL-7 |
| >>>E-RUCCH ASC Setting | MD |  | ASC setting 10.3.6.6 | The default values are same as the PRACH ASC. If the "default" is used for the first ASC, the default values are all available signatures and "all available sub-channels" for FDD and "all available channelisation codes" and "all available subchannels" with "subchannel size=Size 1" in TDD. | REL-7 |
| >>E-RUCCH persistence scaling factor list | OP | 1 to <maxASCpersist> |  | multiplicity corresponds to the number of E-RUCCH ASCs minus 2 | REL-7 |
| >>>Persistence scaling factor | MP |  | Real(0.9..0.2 by step of 0.1) |  | REL-7 |
| >>SYNC_UL info | OP |  | SYNC_UL info for E-RUCCH 10.3.6.78d |  | REL-7 |
| >>PRACH Information | OP | 1..<maxPRACH_FPACH> |  |  | REL-7 |
| >>>Timeslot number | MP |  | Timeslot number 10.3.6.84 |  | REL-7 |
| >>> Channelisation Code List | MP | 1 to 2 |  |  | REL-7 |
| >>>> Channelisation Code | MP |  | Enumerated ((4/1)..(4/4),(8/1)..(8/8),(16/1)..(16/16)) |  | REL-7 |
| >>>Midamble Shift and burst type | MP |  | Midamble shift and burst type 10.3.6.41 |  | REL-7 |
| >>>FPACH info | OP |  | FPACH info 10.3.6.35a |  | REL-7 |

#### 10.3.6.103a	E-RUCCH Info 1.28Mcps TDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| T-RUCCH | MD |  | Enumerated (20, 40, 60, 80, 120, 160, 200, 240, 280, 320, 400, 500, 600, 800, 1000, 2000) | Default = 120 ms If T-RUCCH expires before Grant is received then UE should send scheduling information again on E-RUCCH | REL-8 |
| N-RUCCH | MD |  | Integer (0…7) | Default = 3 Maximum number of retransmissions of scheduling information on E-RUCCH | REL-8 |
| T-WAIT | MD |  | Enumerated (everyEDCHTTI, 40, 80, 160, 320, 640, 1000, 2000) | Values in ms. Default = 40ms If T-WAIT expires then UE should send scheduling information on E-RUCCH | REL-8 |
| T-SI | MD |  | Enumerated (everyEDCHTTI, 20, 40, 60, 80, 160, 200) | Values in ms. Default = 40ms If periodic T-SI expires then UE should include scheduling information in a new MAC-e PDU | REL-8 |
| Extended Estimation Window | OP |  | Integer (2..5) | Values in 5ms TTI. Indicates how many next consecutive 5ms TTIs the UE should estimate whether there is a Grant valid before sending Scheduling Infomation via E-RUCCH | REL-8 |
| E-RUCCH Access Service class | OP | 1 to <maxASC> |  | If only "NumASC+1" (with, NumASC+1 < maxASC) ASCs are listed, the remaining (NumASC+2 through maxASC) ASCs are unspecified. The IE is only included when E-DCH is configured on secondary frequency for a specific UE | REL-8 |
| >E-RUCCH ASC Setting | MD |  | ASC setting 10.3.6.6 | The default values are same as the PRACH ASC. If the "default" is used for the first ASC, the default values are all available signatures and "all available sub-channels" for FDD and "all available channelisation codes" and "all available subchannels" with "subchannel size=Size 1" in TDD. | REL-8 |
| E-RUCCH persistence scaling factor list | OP | 1 to <maxASCpersist> |  | multiplicity corresponds to the number of E-RUCCH ASCs minus 2 | REL-8 |
| >Persistence scaling factor | MP |  | Real(0.9..0.2 by step of 0.1) |  | REL-8 |
| SYNC_UL info | OP |  | SYNC_UL info for E-RUCCH 10.3.6.78d |  | REL-8 |
| PRACH Information | OP | 1..<maxPRACH_FPACH> |  |  | REL-8 |
| >Timeslot number | MP |  | Timeslot number 10.3.6.84 |  | REL-8 |
| > Channelisation Code List | MP | 1 to 2 |  |  | REL-8 |
| >>Channelisation Code | MP |  | Enumerated ((4/1)..(4/4),(8/1)..(8/8),(16/1)..(16/16)) |  | REL-8 |
| >Midamble Shift and burst type | MP |  | Midamble shift and burst type 10.3.6.41 |  | REL-8 |
| >FPACH info | OP |  | FPACH info 10.3.6.35a |  | REL-8 |

#### 10.3.6.104		E-PUCH Info (TDD only)

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-TFCS information | MP |  | E-TFCS info 10.3.6.105 |  | REL-7 |
| CHOICE TDD mode |  |  |  |  | REL-7 |
| >3.84/7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>NE-UCCH | MD |  | Integer(1..12) | No of slots that are required to carry TPC and TFCI (consecutively allocated slots beginning with the first). Default = 1 (first allocated slot). | REL-7 |
| >>E-PUCH constant value | MP |  | Integer (-35..10) | Power to be used | REL-7 |
| >>E-PUCH TS configuration list | MP | <1 to maxTS-2> |  |  | REL-7 |
| >>>TS number | MP |  | Integer(0..14) |  | REL-7 |
| >>>CHOICE Burst Type | MP |  |  |  | REL-7 |
| >>>>Type 1 |  |  |  |  | REL-7 |
| >>>>>Midamble allocation mode | MP |  | Enumerated(Default, UE specific) |  | REL-7 |
| >>>>>Midamble configuration burst type 1 | MP |  | Enumerated(4, 8, 16) |  | REL-7 |
| >>>>>Midambe shift | CV-UE specific |  | Integer (0..15) |  | REL-7 |
| >>>>Type 2 |  |  |  |  | REL-7 |
| >>>>>Midamble allocation mode | MP |  | Enumerated(Default, UE specific) |  | REL-7 |
| >>>>>Midamble configuration burst type 2 | MP |  | Enumerated(3, 6) |  | REL-7 |
| >>>>>Midambe shift | CV-UE specific |  | Integer (0..5) |  | REL-7 |
| >>E-PUCH code hopping | MP |  | Boolean |  | REL-7 |
| >>E-PUCH TPC step size | MP |  | Integer (1,2,3) |  | REL-7 |
| >>Minimum allowed code rate | MP |  | Integer (0..63) | Maps 0.055 to 1.0 in steps of 0.015 | REL-7 |
| >>Maximum allowed code rate | MP |  | Integer (0..63) | Maps 0.055 to 1.0 in steps of 0.015 | REL-7 |
| >1.28 Mcps TDD |  |  |  |  | REL-7 |
| >>SNPL Reporting Type | OP |  | Enumerated (type1, type2) |  | REL-7 |
| >>PRXdes_base | MP |  | Integer (-112..-50 by step of 1) | dBm. Reference desired power level for E-PUCH | REL-7 |
| >>Beacon PL Est. | MD |  | Boolean | TRUE indicates that the UE may take into account path loss estimated from beacon function physical channels. Default value is FALSE | REL-7 |
| >>TPC step size | MP |  | Integer (1, 2, 3) | dB. | REL-7 |
| >>Pebase power control gap | MD |  | Integer (1…255) | Unit: Number of subframes, Default value is 1. Value 255 represents infinite in which case closed loop power control shall always be used. | REL-7 |
| >>Uplink synchronisation parameters | MD |  |  | Default: Uplink synchronisation step size 1. Uplink synchronisation frequency 1. | REL-7 |
| >>>Uplink synchronisation step size | MP |  | Integer (1..8) | This parameter specifies the step size to be used for the adjustment of the uplink transmission timing | REL-7 |
| >>>Uplink synchronisation frequency | MP |  | Integer (1..8) | This parameter specifies the frequency of the adjustment of the uplink transmission timing | REL-7 |
| >>E-PUCH TS configuration list | MP | <1 to maxTS-LCR-1> |  |  | REL-7 |
| >>>TS number | MP |  | Integer (1..5) |  | REL-7 |
| >>>Midamble shift and burst type | MP |  |  |  | REL-7 |
| >>>>Midamble Allocation Mode | MP |  | Enumerated (Default midamble, UE specific midamble) |  | REL-7 |
| >>>>Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) | As defined in [30] | REL-7 |
| >>>>Midamble Shift | CV-UE specific |  | Integer (0..15) |  | REL-7 |
| >>Minimum allowed code rate | MP |  | Integer (0..63) | Maps 0.055 to 1.0 in steps of 0.015 | REL-7 |
| >>Maximum allowed code rate | MP |  | Integer (0..63) | Maps 0.055 to 1.0 in steps of 0.015 | REL-7 |
| >>Maximum number of retransmissions for Scheduling Info | MP |  | Integer (0..15) | Only used when no MAC-d PDUs are included in the same MAC-e PDU or MAC-i PDU. | REL-7 |
| >>Retransmission Timer for Scheduling Info | MP |  | Enumerated (10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 65, 70, 75, 80, 85, 90, 95, 100, 110, 120, 140, 160, 200, 240, 280, 320, 400, 480, 560) | Only used when no MAC-d PDUs are included in the same MAC-e PDU or MAC-i PDU. Unit is ms. | REL-7 |
| Power Offset for Scheduling Info | OP |  | Integer (0..6) | If present, this IE should be ignored for LCR TDD. Only used when no MACd PDUs are included in the same MACe PDU. Unit is in dB. | REL-7 |

| Condition | Explanation |
| --- | --- |
| UE specific | This IE is mandatory present when the value of the IE "Midamble Allocation Mode" is "UE specific midamble" and not needed otherwise. |

#### 10.3.6.104a		E-PUCH Info 1.28Mcps TDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-TFCS information | MP |  | E-TFCS info 10.3.6.105 |  | REL-8 |
| SNPL Reporting Type | OP |  | Enumerated (type1, type2) |  | REL-8 |
| PRXdes_base | MP |  | Integer (-112..-50 by step of 1) | dBm. Reference desired power level for E-PUCH | REL-8 |
| Beacon PL Est. | MD |  | Boolean | TRUE indicates that the UE may take into account path loss estimated from beacon function physical channels. Default value is FALSE | REL-8 |
| TPC step size | MP |  | Integer (1, 2, 3) | dB. | REL-8 |
| Pebase power control gap | MD |  | Integer (1…255) | Unit: Number of subframes, Default value is 1. Value 255 represents infinite in which case closed loop power control shall always be used. | REL-8 |
| Uplink synchronisation parameters | MD |  |  | Default: Uplink synchronisation step size 1. Uplink synchronisation frequency 1. | REL-8 |
| >Uplink synchronisation step size | MP |  | Integer (1..8) | This parameter specifies the step size to be used for the adjustment of the uplink transmission timing | REL-8 |
| >Uplink synchronisation frequency | MP |  | Integer (1..8) | This parameter specifies the frequency of the adjustment of the uplink transmission timing | REL-8 |
| E-PUCH TS configuration list | MP | <1 to maxTS-LCR-1> |  |  | REL-8 |
| >TS number | MP |  | Integer (1..5) |  | REL-8 |
| >Midamble shift and burst type | MP |  |  |  | REL-8 |
| >>Midamble Allocation Mode | MP |  | Enumerated (Default midamble, UE specific midamble) |  | REL-8 |
| >>Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) | As defined in [30] | REL-8 |
| >>Midamble Shift | CV-UE specific |  | Integer (0..15) |  | REL-8 |
| Minimum allowed code rate | MP |  | Integer (0..63) | Maps 0.055 to 1.0 in steps of 0.015 | REL-8 |
| Maximum allowed code rate | MP |  | Integer (0..63) | Maps 0.055 to 1.0 in steps of 0.015 | REL-8 |
| Maximum number of retransmissions for Scheduling Info | MP |  | Integer (0..15) | Only used when no MAC-d PDUs are included in the same MAC-i PDU. | REL-8 |
| Retransmission Timer for Scheduling Info | MP |  | Enumerated (10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 65, 70, 75, 80, 85, 90, 95, 100, 110, 120, 140, 160, 200, 240, 280, 320, 400, 480, 560) | Only used when no MAC-d PDUs are included in the same MAC-i PDU. Unit is ms. | REL-8 |

#### 10.3.6.104b		E-PUCH Info for multi-carrier E-DCH 1.28Mcps TDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-PUCH TS configuration list | MP | <1 to maxTS-LCR-1> |  |  | REL-10 |
| >TS number | MP |  | Integer (1..5) |  | REL-10 |
| >Midamble shift and burst type | MP |  |  |  | REL-10 |
| >>Midamble Allocation Mode | MP |  | Enumerated (Default midamble, UE specific midamble) |  | REL-10 |
| >>Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) | As defined in [30] | REL-10 |
| >>Midamble Shift | CV-UE specific |  | Integer (0..15) |  | REL-10 |

| Condition | Explanation |
| --- | --- |
| UE specific | If UE specific midamble allocation mode is configured, this IE is mandatory, otherwise it is not needed. |

#### 10.3.6.105	E-TFCS info (TDD only)

| IE/Group Name | Presence | Multi | IE Type and Reference | Semantics Description | Version |
| --- | --- | --- | --- | --- | --- |
| Reference Beta Information QPSK list | MP | <1 to 8> |  |  | REL-7 |
| >Reference Code Rate | MP |  | Integer (0..10) | Unit: - Range: 0..1 Step: 0.1 | REL-7 |
| >Reference Beta | MP |  | Integer (-15..16) | Unit range -15db to +16db | REL-7 |
| Reference Beta Information 16QAM list | MP | <1 to 8> |  |  | REL-7 |
| >Reference Code Rate | MP |  | Integer (0..10) | Unit: - Range: 0..1 Step: 0.1 | REL-7 |
| >Reference Beta | MP |  | Integer (-15..16) | Unit range -15db to +16db | REL-7 |

#### 10.3.6.106	E-TFC Boost Info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-TFCI boost | MP |  | Integer (0..127) | E-TFCI threshold beyond which boosting of E-DPCCH is enabled | REL-7 |
| Delta T2TP | CV- E-TFCI boost127 |  | Integer (0..6) |  | REL-7 |

| Condition | Explanation |
| --- | --- |
| E-TFCI boost127 | If E-TFCI-Boost is set to 127 this IE is not needed, otherwise it is mandatory. |

#### 10.3.6.107	Control Channel DRX information 1.28Mcps TDD

NOTE:	For 1.28 Mcps TDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Control Channel DRX operation | MP |  |  |  | REL-8 |
| >Continue Control Channel DRX operation |  |  |  |  | REL-8 |
| >>Enabling Delay | OP |  | Enumerated (0, 1, 2, 4, 8, 16, 32, 64, 128, infinity) | In radio frames. Six spare values are needed. | REL-8 |
| >New Control Channel DRX operation |  |  |  |  | REL-8 |
| >>HS-SCCH DRX information | MP |  | HS-SCCH DRX information 1.28 Mcps TDD 10.3.6.108 |  | REL-8 |
| >>E-AGCH DRX Information | OP |  | E-AGCH DRX information 1.28 Mcps TDD 10.3.6.109 |  | REL-8 |
| >>Enabling Delay | MP |  | Enumerated (0, 1, 2, 4, 8, 16, 32, 64, 128, infinity) | In radio frames. Six spare values are needed. | REL-8 |

#### 10.3.6.108	HS-SCCH DRX information 1.28 Mcps TDD

NOTE:	For 1.28 Mcps TDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| HS-SCCH DRX cycle | MP |  | Enumerated (1, 2, 4, 8, 16, 32, 64) | Units of subframes. One spare value is needed. | REL-8 |
| Inactivity Threshold for HS-SCCH DRX cycle | OP |  | Enumerated (1, 2, 4, 8, 16, 32, 64, 128, 256, 512, infinity) | Units of subframes. Five spare values are needed. | REL-8 |
| HS-SCCH DRX Offset | MP |  | Integer (0..63) | Units of subframes. Offset of the HS-SCCH DRX cycles. | REL-8 |

#### 10.3.6.109	E-AGCH DRX information 1.28 Mcps TDD

NOTE:	For 1.28 Mcps TDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE E-AGCH DRX information type | MP |  |  |  | REL-8 |
| >Same as HS-SCCH |  |  |  | (no data) Indicate the E-AGCH DRX Cycle and Offset are the same as the HS-SCCH DRX Cycle and Offset, and the E-AGCH Inactivity Monitor Threshold is absent | REL-8 |
| >E-AGCH DRX parameters |  |  |  |  | REL-8 |
| >>E-AGCH DRX cycle | MP |  | Enumerated (1, 2, 4, 8, 16, 32, 64) | Units of subframes. One spare value is needed. | REL-8 |
| >>E-AGCH Inactivity Monitor Threshold | OP |  | Enumerated (0, 1, 2, 4, 8, 16, 32, 64, 128, 256, 512, infinity) | Units of subframes. Four spare values are needed. If it is absent, the same IE in the IE “E-AGCH Info” is used as E-AGCH inactivity monitor threshold. | REL-8 |
| >>E-AGCH DRX Offset | MP |  | Integer (0..63) | Units of subframes. Offset of the E-AGCH DRX cycles. | REL-8 |

#### 10.3.6.110	SPS information 1.28 Mcps TDD

NOTE:	For 1.28 Mcps TDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-DCH SPS information | OP |  | E-DCH SPS information 1.28 Mcps TDD 10.3.6.111 |  | REL-8 |
| HS-DSCH SPS information | OP |  | HS-DSCH SPS information 1.28 Mcps TDD 10.3.6.112 |  | REL-8 |

#### 10.3.6.111	E-DCH SPS information 1.28 Mcps TDD

NOTE:	For 1.28 Mcps TDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE E-DCH SPS operation | MP |  |  |  | REL-8 |
| >Continue E-DCH SPS operation |  |  |  | (no data) | REL-8 |
| >New E-DCH SPS operation |  |  |  |  | REL-8 |
| >>E-HICH Information | MP |  |  |  | REL-8 |
| >>>CHOICE Configuration Mode | MP |  |  |  | REL-8 |
| >>>>Implicit |  |  |  |  | REL-8 |
| >>>>>EI | OP |  | Integer (0..3) | If the IE does not exist, UE shall use the same configuration as the E-HICH for non-schedule transmission | REL-8 |
| >>>>Explicit |  |  |  |  | REL-8 |
| >>>>>Timeslot number | MP |  | Integer (0..6) |  | REL-8 |
| >>>>>Channelisation code | MP |  | Enumerated ((16/1)..(16/16)) |  | REL-8 |
| >>>>>Midamble Allocation mode | MP |  | Enumerated (Default midamble, UE specific midamble) |  | REL-8 |
| >>>>>Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) |  | REL-8 |
| >>>>>Midamble Shift | CV-UE specific |  | Integer (0..15) |  | REL-8 |
| >>>Signature Sequence Group Index | MP |  | Integer (0..19) |  | REL-8 |
| >>Transmission Pattern List | OP | 1..< maxEDCHTxPattern-TDD128 > |  |  | REL-8 |
| >>>Repetition period | MP |  | Integer (1, 2, 4, 8, 16, 32) | Value 1 indicate continuous | REL-8 |
| >>>Repetition length | MP |  | Integer (1.. Repetition period – 1) | NOTE: This is empty if repetition period is set to 1. | REL-8 |
| >>Initial SPS info for E-DCH | OP |  |  |  | REL-8 |
| >>>NE-UCCH | MD |  | Integer (1..8) | Number of E-UCCH and TPC instances within an E-DCH TTI. Default = 1. | REL-8 |
| >>>Code Resource Information | MP |  | Enumerated ((1/1), (2/1),(2/2),(4/1)..(4/4),(8/1)..(8/8),(16/1)..(16/16)) | As defined in [31], indicating which of the Channel Codes configured for E-DCH are allocated for SPS resource | REL-8 |
| >>>Timeslot Resource Related InformationI | MP |  | Bit string (5) | As defined in [31], indicating which of the timeslots configured for E-DCH are allocated for SPS resource | REL-8 |
| >>>Power Resource Related Information | MP |  | Integer (1..32) | Indicates 0-31 PRRI index defined in [31], which specifies the maximum allowed E-PUCH resource that the UE may use [15]. | REL-8 |
| >>>Activation Time | MP |  | Activation time 10.3.3.1 | Specifies the E-PUCH Offset in Radio Frame level. | REL-8 |
| >>>Subframe number | MP |  | Integer (0..1) | Specifies the E-PUCH Offset in subframe level. | REL-8 |
| >>>Initial Tx pattern Index | MP |  | Integer (0.. maxEDCHTxPattern-TDD128-1) |  | REL-8 |

| Condition | Explanation |
| --- | --- |
| UE specific | If UE specific midamble allocation mode is configured, this IE is mandatory, otherwise it is not needed. |

#### 10.3.6.112	HS-DSCH SPS information 1.28 Mcps TDD

NOTE:	For 1.28 Mcps TDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE HS-DSCH SPS operation | MP |  |  |  | REL-8 |
| >Continue HS-DSCH SPS operation |  |  |  | (no data) | REL-8 |
| >New HS-DSCH SPS operation |  |  |  |  | REL-8 |
| >>Transport Block Size List | OP | 1..<maxTbsForHSDSCH-TDD128> |  |  | REL-8 |
| >>>Transport Block Size Index | MP |  | Integer (1..63) | Index of the MAC-hs transport block size as described in [15] | REL-8 |
| >>Receive Pattern List | OP | 1..<maxRxPatternForHSDSCH-TDD128> |  |  | REL-8 |
| >>>Repetition period | MP |  | Integer (1, 2, 4, 8, 16, 32) | Value 1 indicate continuous | REL-8 |
| >>>Repetition length | MP |  | Integer (1.. Repetition period – 1) | NOTE: This is empty if repetition period is set to 1. | REL-8 |
| >>HARQ Info for Semi-Persistent Scheduling | OP |  |  |  | REL-8 |
| >>>Number of Processes | MP |  | Integer (1..8) |  | REL-8 |
| >>>Process Memory size | MP |  | Integer (800 .. 16000 by step of 800, 17600 .. 32000 by step of 1600, 36000 .. 80000 by step of 4000, 88000 .. 160000 by step of 8000, 176000 .. 304000 by step of 16000) | Maximum number of soft channel bits available in the virtual IR buffer [27] | REL-8 |
| >>HS-SICH List | MP | 1..<maxHSSICH-TDD128> |  |  | REL-8 |
| >>>CHOICE Configuration Mode | MP |  |  |  | REL-8 |
| >>>>Implicit |  |  |  |  | REL-8 |
| >>>>>HS-SCCH Index | MP |  | Integer (1.. maxHSSCCHs) |  | REL-8 |
| >>>>Explicit |  |  |  |  | REL-8 |
| >>>>>Timeslot number | MP |  | Integer (0..6) |  | REL-8 |
| >>>>>Channelisation code | MP |  | Enumerated ((16/1) ..(16/16)) |  | REL-8 |
| >>>>>Midamble Allocation mode | MP |  | Enumerated (Default midamble, UE specific midamble) |  | REL-8 |
| >>>>>Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) |  | REL-8 |
| >>>>>Midamble Shift | CV-UE |  | Integer (0..15) |  | REL-8 |
| >>Initial SPS info for HS-DSCH | OP |  |  |  | REL-8 |
| >>>Timeslot Information | MP |  | Bit string (5) | As defined in [31], indicating which of the timeslots configured for HS-PDSCH are allocated for SPS resource | REL-8 |
| >>>Code Resource Information | MP |  |  | Note 1 | REL-8 |
| >>>>Start code | MP |  | Enumerated ((16/1)..(16/16)) |  | REL-8 |
| >>>>Stop code | MP |  | Enumerated ((16/1)..(16/16)) |  | REL-8 |
| >>>Activation Time | MP |  | Activation time 10.3.3.1 | Specifies the HS-PDSCH Offset in Radio Frame level. | REL-8 |
| >>>Subframe number | MP |  | Integer (0..1) | Specifies the HS-PDSCH Offset in subframe level. | REL-8 |
| >>>Initial Transport Block Size Index | MP |  | Integer (0.. maxTbsForHSDSCH-TDD128-1) |  | REL-8 |
| >>>Initial Rx pattern Index | MP |  | Integer (0.. maxRxPatternForHSDSCH-TDD128-1) |  | REL-8 |
| >>>HS-SICH Index | MP |  | Integer (0.. maxHSSICH-TDD128-1) |  | REL-8 |
| >>>Modulation | MP |  | Enumerated (QPSK, 16QAM) |  | REL-8 |

| Condition | Explanation |
| --- | --- |
| UE specific | If UE specific midamble allocation mode is configured, this IE is mandatory, otherwise it is not needed. |

NOTE 1:	HS-PDSCH channelisation codes are allocated contiguously from a signalled start code to a signalled stop code, and the allocation includes both the start and stop code. If a value of Start code = 16 and Stop code = 1 is signalled, a spreading factor of SF=1 shall be used for the HS-PDSCH resources.

#### 10.3.6.113	Downlink channelisation codes MBSFN IMB

NOTE:	3.84 Mcps TDD MBSFN IMB only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| First channelisation code | MP |  | Integer (1..15) | Channelisation code (F) for SF16 | REL-8 |
| Last channelisation code | OP |  | Integer (1..15) | Channelisation code (L) for SF16; If present: consecutive codes from F to L (modulo 16, excluding code 0) are used; If absent: only one code (F) is used | REL-8 |

#### 10.3.6.114	Secondary CCPCH frame type 2 info

NOTE:	3.84 Mcps TDD MBSFN IMB only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Sub-frame number | MP |  | Integer (0..4) |  | REL-8 |
| Downlink channelisation codes | MP |  | Downlink channelisation codes MBSFN IMB 10.3.6.113 |  | REL-8 |
| CHOICE modulation | MP |  |  |  | REL-8 |
| >QPSK |  |  |  | (no data) | REL-8 |
| >16QAM |  |  |  |  | REL-8 |
| >>CPICH secondary CCPCH power offset | MP |  | Integer (−11..4) | In dB | REL-8 |

#### 10.3.6.115	Uplink secondary cell info FDD

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Configuration info | MP |  |  |  | REL-9 |
| >Continue |  |  | (no data) | Uplink secondary cell info parameters remain unchanged. | REL-9 |
| >New configuration |  |  |  |  | REL-9 |
| >>Secondary serving E-DCH cell info | OP |  | Secondary serving E-DCH cell info 10.3.6.116 |  | REL-9 |
| >>Secondary E-DCH info common | OP |  | Secondary E-DCH info common 10.3.6.117 |  | REL-9 |
| >>Downlink information per radio link list on secondary UL frequency | OP |  | Downlink information per radio link list on secondary UL frequency 10.3.6.118 |  | REL-9 |

#### 10.3.6.116	Secondary serving E-DCH cell info

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-9 |
| Secondary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-9 |

#### 10.3.6.117	Secondary E-DCH info common

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Frequency info | MP |  | Frequency info 10.3.6.36 |  | REL-9 |
| Scrambling code type | MP |  | Enumerated(short, long) |  | REL-9 |
| Scrambling code number | MP |  | Integer(0..16777215) |  | REL-9 |
| 2ms scheduled transmission grant HARQ process allocation | MD |  | Bitstring (8) | MAC-d PDUs are only allowed to be transmitted in those processes for which the bit is set to “1”. Bit 0 corresponds to HARQ process 0, bit 1 corresponds to HARQ process 1,… Default value is: transmission in all HARQ processes is allowed. Bit 0 is the first/leftmost bit of the bit string. | REL-9 |
| Serving Grant | OP |  |  |  | REL-9 |
| >Primary/Secondary Grant Selector | MP |  | Enumerated (“primary”, “secondary”) | Indicates whether the Serving Grant is received with a Primary E-RNTI or Secondary E-RNTI | REL-9 |
| Minimum reduced E-DPDCH gain factor | OP |  | Enumerated (8/15, 11/15, 15/15, 21/15, 30/15, 42/15, 60/15, 84/15) | βed,k,reduced,min [29]; the default value is 8/15. If the configurable reduced E-DPDCH gain factor is not supported by the UE, the default value is used. | REL-9 |
| E-DCH minimum set E-TFCI | OP |  | Integer (0..127) | See [15]; Absence means no E-DCH minimum set on secondary uplink frequency | REL-9 |
| Minimum TEBS threshold | CV-Implicit |  | Enumerated(2, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2K, 4K, 8K, 16K, 32K, 64K, 128K, 256K, 512K, 1024K) | In bytes And N Kbytes = N*1024 bytes. Twelve spare values are needed. | REL-12 |
| Implicit Grant handling | CV-Message |  | Enumerated (TRUE) | Absense of this information element means that the UE is not configured for Implicit Grant handling on the secondary uplink frequency. | REL-12 |
| DPCCH Power offset for secondary UL frequency | MP |  | Integer (0..7 by step of 1) | In dB The power offset between the initial DPCCH power level on secondary UL frequency when it is activated and the current DPCCH power level on primary UL frequency. | REL-9 |
| PC Preamble | MP |  | Integer (0..7) | In number of frames | REL-9 |
| DPCCH Power Reset after DTX on secondary uplink frequency | CV-Message |  | DPCCH Power Reset after DTX on secondary uplink frequency 10.3.6.151 |  | REL-12 |

| Condition | Explanation |
| --- | --- |
| Message | This IE is not needed if the IE "Uplink secondary cell info FDD" is included in HANDOVER TO UTRAN COMMAND message. Otherwise it is optional. |
| Implicit | This IE is optional if the IE "Implicit Grant handling" is present in the message. Otherwise it is not needed. |

#### 10.3.6.118	Downlink information per radio link list on secondary UL frequency

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Downlink information for each radio link on secondary UL frequency |  | 1 to <maxEDCHRL> |  |  | REL-9 |
| >Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-9 |
| >Cell ID | OP |  | Cell ID 10.3.2.2 |  | REL-9 |
| >Downlink F-DPCH info for each RL on secondary UL frequency | MP |  | Downlink F-DPCH info for each RL 10.3.6.23ob |  | REL-9 |
| >E-AGCH Info | OP |  | E-AGCH Info 10.3.6.100 |  | REL-9 |
| >E-HICH Information | OP |  | E-HICH Info 10.3.6.101 |  | REL-9 |
| >CHOICE E-RGCH Information | OP |  |  |  | REL-9 |
| >>E-RGCH Information |  |  | E-RGCH Info 10.3.6.102 |  | REL-9 |
| >>E-RGCH release indicator |  |  |  | (no data) | REL-9 |
| >Secondary serving E-DCH radio link indicator | CV-Message |  | Enumerated (TRUE) | TRUE indicates that this radio link is the secondary serving E-DCH radio link | REL-12 |
| >Radio Links without DPCH/F-DPCH indicator | CV-Message2 |  |  | (no data) | REL-12 |

| Condition | Explanation |
| --- | --- |
| Message | This IE is not needed if the IE "Uplink secondary cell info FDD" is included in HANDOVER TO UTRAN COMMAND message. Otherwise it is optional. |
| Message2 | This IE is not needed if the IE "Uplink secondary cell info FDD" is included in HANDOVER TO UTRAN COMMAND message, or if the IE "Uplink secondary cell info FDD" is included as part of Target cell preconfiguration information in the ACTIVE SET UPDATE message. Otherwise it is optional. |

#### 10.3.6.119	Radio link addition information on secondary UL frequency

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Radio link addition information list on secondary UL frequency |  | 1 to <maxEDCHRL-1> |  |  | REL-9 |
| >Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-9 |
| >Cell ID | OP |  | Cell ID 10.3.2.2 |  | REL-9 |
| >Downlink F-DPCH info for each RL on secondary UL frequency | MP |  | Downlink F-DPCH info for each RL 10.3.6.23ob |  | REL-9 |
| >E-HICH Information | MP |  | E-HICH Info 10.3.6.101 |  | REL-9 |
| >E-RGCH Information | OP |  | E-RGCH Info 10.3.6.102 |  | REL-9 |
| >Radio Links without DPCH/F-DPCH indicator | OP |  |  | (no data) | REL-12 |

#### 10.3.6.120	Radio link removal information on secondary UL frequency

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Radio link removal information list on secondary UL frequency |  | 1 to <maxEDCHRL> |  |  | REL-9 |
| >Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-9 |

#### 10.3.6.121	E-DCH reconfiguration information on secondary UL frequency

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-DCH RL Info for new secondary serving E-DCH cell | OP |  |  |  | REL-9 |
| >E-AGCH Info | MP |  | E-AGCH Info 10.3.6.100 |  | REL-9 |
| >E-HICH Information | OP |  | E-HICH Info 10.3.6.101 | This IE is not present if the secondary serving E-DCH cell is added to the secondary E-DCH active set with this message. | REL-9 |
| >CHOICE E-RGCH Information | OP |  |  | This IE is not present if the secondary serving E-DCH cell is added to the secondary E-DCH active set with this message. | REL-9 |
| >>E-RGCH Information | MP |  | E-RGCH Info 10.3.6.102 |  | REL-9 |
| >>E-RGCH release indicator |  |  |  | (no data) | REL-9 |
| E-DCH RL Info for other cells | OP | 1 to <maxEDCHRL> |  | This IE is not allowed to include information on a RL added by this message, except in the case of an update to target cell preconfiguration information. | REL-9 |
| >Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-9 |
| >E-HICH Information | OP |  | E-HICH Info 10.3.6.101 |  | REL-9 |
| >CHOICE E-RGCH Information | OP |  |  |  | REL-9 |
| >>E-RGCH Information | MP |  | E-RGCH Info 10.3.6.102 |  | REL-9 |
| >>E-RGCH release indicator |  |  |  | (no data) | REL-9 |

#### 10.3.6.122	MU-MIMO info 1.28 Mcps TDD

For 1.28 Mcps TDD only

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE MU-MIMO info | MP |  |  |  | REL-10 |
| >Continue |  |  | (no data) |  | REL-10 |
| >New configuration |  |  |  |  | REL-10 |
| >>MU-MIMO operation | MP |  | Enumerated (Uplink, Downlink, Uplink and Downlink) | One spare value is needed. | REL-10 |
| >>Standalone midamble info | OP |  |  |  | REL-10 |
| >>>Midamble configuration | MP |  | Integer (2, 4, 6, 8, 10, 12, 14, 16) | Midamble Allocation mode is UE specific midamble allocation | REL-10 |
| >>>Midamble shift | MP |  | Integer (0..15) | The allocated midamble shift. | REL-10 |
| >>>Timeslot information | MP |  | Bit string (5) | Each bit indicates availability of a timeslot, where the timeslot is numbered "Timeslot 1" to "Timeslot 5". The value 1 of a bit indicates that the corresponding timeslot is available. "Timeslot 1" is the first/leftmost bit of the bit string. | REL-10 |
| >>>Activation time | MP |  | Activation time 10.3.3.1 | Specifies the standalone midamble channel Offset in Radio Frame level. | REL-10 |
| >>>Subframe number | MP |  | Integer (0..1) | Specifies the standalone midamble channel Offset in subframe level. | REL-10 |
| >>>Repetition period | MP |  | Enumerated (1, 2, 4, 8, 16, 32, 64) | Value 1 indicates continuous, in subframe level. One spare value is needed. | REL-10 |
| >>>Reference beta | CV-UL_E-DCH |  | Integer (-15.. 16) | In dB. | REL-10 |

| Condition | Explanation |
| --- | --- |
| UL_E-DCH | This IE is mandatory present if IE "uplink transport channel type" is equal to "E-DCH". Otherwise it is not needed. |

#### 10.3.6.123	E-RGCH Info for Common E-DCH

Includes the configuration for the E-DCH related Relative Grant Channel to be used with common E-DCH in CELL_FACH and Idle mode. The E-RGCH is using the same channelisation code as configured for the E-HICH channel (see subclause 10.3.6.124).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Signature Sequence | MD |  | Integer(0..39) | NOTE1 | REL-8 |
| NOTE1: The default value is defined by: Signature Sequence = (offset + common E-DCH resource list position) mod 40 where "offset" is the value of the last occurrence of the IE "Signature Sequence". If "Signature Sequence" is not present in the first occurrence, "offset" is equal to zero. "common E-DCH resource list position" indicates the Common E-DCH resource Configuration Information number by order of appearance. The value of the "common E-DCH resource list position" is zero for the first occurrence. |  |  |  |  |  |

#### 10.3.6.124	E-HICH Info for Common E-DCH

Includes the configuration for the E-DCH related HARQ Acknowledgement Indicator Channel.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Channelisation Code | CV-Short |  | Integer (0..127) | If this IE is not present, the value is equal to the last occurrence of this IE. | REL-8 |
| Signature Sequence | MD |  | Integer (0..39) | NOTE1 | REL-8 |
| NOTE1: The default value is defined by: Signature Sequence = (offset + common E-DCH resource list position) mod 40 where "offset" is the value of the last occurrence of the IE "Signature Sequence". If "Signature Sequence" is not present in the first occurrence, "offset" is equal to zero. "common E-DCH resource list position" indicates the Common E-DCH resource Configuration Information number by order of appearance. The value of the "common E-DCH resource list position" is zero for the first occurrence. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Short | This IE is mandatory present for the first occurrence. Otherwise, this IE is optional. |

#### 10.3.6.125	Uplink CLTD info FDD

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Configuration info | MP |  |  |  | REL-11 |
| >Continue |  |  | (no data) | Used in reconfigurations without interruption of uplink CLTD operation. | REL-11 |
| >New configuration |  |  |  |  |  |
| >>S-DPCCH info | MP |  |  |  | REL-11 |
| >>>S-DPCCH/DPCCH power offset | MP |  | Integer (0..6) | Refer to quantization of the power offset in [28]. | REL-11 |
| >>Initial CLTD activation state | CV-InitConfig |  | Enumerated (first state, second state) | Indicates the CLTD activation state as described in [27]. | REL-11 |
| >>Primary CPICH info | CV-DCHorMultiflow |  | Primary CPICH Info 10.3.6.60 | Indicates the cell that determines the precoding weights amongst the active set cells. | REL-11 |

| Condition | Explanation |
| --- | --- |
| DCHorMultiflow | If Uplink CLTD operation is configured, this IE is mandatory present if HS-DSCH is not configured and only DCH is configured. If the Multiflow operation and Uplink CLTD are simultaneously configured, then this IE is optional and indicates the Multiflow assisting serving HS-DSCH cell. |
| InitConfig | This IE is mandatory present in the initial uplink CLTD configuration. Otherwise it is optional. |

#### 10.3.6.126	Uplink OLTD info FDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Uplink OLTD activation | MP |  | Enumerated (TRUE) |  | REL-11 |

#### 10.3.6.127	F-TPICH info

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| F-TPICH slot format | MP |  | Integer (0..9) | Slot format used by F-TPICH in [26]. | REL-11 |
| F-TPICH code number | MP |  | Integer (0..255) |  | REL-11 |
| F-TPICH frame offset | MP |  | Integer (0..38144 by step of 256) | Offset (in number of chips) between the beginning of the P-CCPCH frame and the beginning of the F-TPICH frame. This is called F-TPICH,m in [26]. | REL-11 |

NOTE:	The UTRAN should configure the F-TPICH info either only for the RL transmitted from the cell which determines the precoding weights, or for all the RL(s) within the RLS that the cell which determines the precoding weights belongs to, otherwise the UE behaviour is not specified.

#### 10.3.6.128	F-TPICH reconfiguration info

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| F-TPICH info new cell that determines the precoding weights | OP |  |  | This IE is not present if the cell is added to the active set with this message. | REL-11 |
| >F-TPICH info | MP |  | F-TPICH Info 10.3.6.127 |  | REL-11 |
| F-TPICH info other cells | OP | 1 to <maxRL> |  | This IE is not allowed to include information on a RL added by this message, except in the case of an update to target cell preconfiguration information. | REL-11 |
| >Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-11 |
| >CHOICE F-TPICH Info | MP |  |  |  | REL-11 |
| >>F-TPICH Info |  |  | F-TPICH Info 10.3.6.127 |  | REL-11 |
| >>F-TPICH release indicator |  |  |  | (no data) | REL-11 |

#### 10.3.6.129	Multiflow configuration info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Multiflow cell | MP |  | Enumerated(intra-NodeB,inter-NodeB) | Presence of this IE indicates that Multiflow is configured. "intra-NodeB" means that this cell belongs to the same Node B as the serving HS-DSCH cell and should be associated to the same MAC-ehs entity. "inter-NodeB" indicates that this is the inter-Node B cell and should be associated with the second MAC-ehs entity. | REL-11 |
| Multiflow time reference cell | OP |  | Enumerated(TRUE) | Presence of this IE indicates that this cell is the time reference for the UL HS-DPCCH channel. | REL-11 |

#### 10.3.6.130	NodeB triggered HS-DPCCH Transmission

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| HS-DPCCH transmission continuation back off | OP |  | Enumerated (10, 20, 30, 40, 80, 160, 320, 800) | In terms of ms. If not present, implicit common E-DCH resource release based on HS-DPCCH transmission continuation back off is disabled. | REL-11 |

#### 10.3.6.131	Common E-DCH system info parameters for Concurrent TTI

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Common E-DCH MAC-d flow info for Concurrent TTI | OP |  | Common E-DCH MAC-d flow info for Concurrent TTI 10.3.5.26 |  | REL-11 |
| Initial Serving grant value | MD |  | Integer (0..37) | (0..37) indicates E-DCH serving grant index as defined in [15]. NOTE 1 | REL-11 |
| E-AGCH Info | MD |  | E-AGCH Info 10.3.6.100 | NOTE 1 | REL-11 |
| Uplink DPCH power control info for Concurrent TTI | OP |  | Uplink DPCH power control info for Concurrent TTI 10.3.6.132 |  | REL-11 |
| E-DPCCH/DPCCH power offset | MD |  | Integer (0..8) | Refer to quantization of the power offset in [28]. Default value is the value included in the IE "E-DPCCH Info" included in the IE "Common E-DCH System Info" of the System Information Block Type 5/5bis | REL-11 |
| E-DPDCH info | MP |  | E-DPDCH info 10.3.6.99 |  | REL-11 |
| Additional E-DCH transmission back off | MD |  | Integer (0..15) | In terms of TTIs. NOTE 1 | REL-11 |
| Maximum E-DCH resource allocation for CCCH | MD |  | Enumerated (8, 12, 16, 20, 24, 32, 40, 80) | In terms of TTIs. NOTE 1 | REL-11 |
| Maximum period for collision resolution phase | MD |  | Integer (8..24) | In terms of TTIs. NOTE 1 | REL-11 |
| E-DCH transmission continuation back off | MD |  | Enumerated (0, 4, 8, 16, 24, 40, 80, infinity) | In terms of TTIs. If set to "infinity", implicit common E-DCH resource release is disabled. NOTE 1 | REL-11 |
| Measurement Feedback Info for Concurrent TTI | OP |  | Measurement Feedback Info for Concurrent TTI 10.3.6.133 |  | REL-11 |
| NOTE 1: Default value is the value included in the IE "Common E-DCH system info" of the System Information Block Type 5/5bis. |  |  |  |  |  |

#### 10.3.6.132	Uplink DPCH power control info for Concurrent TTI

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| ACK | OP |  | Integer (0..8) | Refer to quantization of the power offset in [28] | REL-11 |
| NACK | OP |  | Integer (0..8) | refer to quantization of the power offset in [28] | REL-11 |
| Ack-Nack repetition factor | OP |  | Integer(1..4) |  | REL-11 |
| NOTE: The IE’s "Power Control Algorithm" and "TPC step size" shall use the corresponding values included in the IE "Uplink DPCH power control info" of the System Information Block Type 5/5bis. |  |  |  |  |  |

#### 10.3.6.133	Measurement Feedback Info for Concurrent TTI

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CQI Feedback cycle, k | OP |  | Integer (0, 2, 4, 8, 10, 16, 20, 32, 40, 64, 80, 160) | In milliseconds. | REL-11 |
| CQI repetition factor | OP |  | Integer (1..4) |  | REL-11 |
| CQI | OP |  | Integer (0..8) | Refer to quantization of the power offset in [28] | REL-11 |
| NOTE: The IE "Measurement Power Offset" shall use the value included in the IE "Measurement Feedback Info" of the System Information Block Type 5/5bis. |  |  |  |  |  |

#### 10.3.6.134		PRACH preamble control parameters extension list (for Enhanced Uplink)

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PRACH preamble control parameters list (for Enhanced Uplink) | MP | 1 to <maxPRACH_EUL> |  |  | REL-11 |
| >PRACH preamble control parameters (for Enhanced Uplink) | MP |  | PRACH preamble control parameters extension (for Enhanced Uplink) 10.3.6.136 | Control parameters of the physical signal. | REL-11 |
| >Weight | OP |  | Real (0.2..1 by step of 0.2) | Probability of selecting this occurrence of PRACH preamble control parameters. | REL-11 |
| NOTE 1: A weight given by: 1 – (sum of the values of the IE "Weight" across all occurrences), shall be used as value of the Lth occurrence of the IE "Weight”, where L is as defined in sub-clause 8.5.73. |  |  |  |  |  |

NOTE 2:	The sum of the values of the IE "Weight" across all occurrences in this IE should be less than 1.

#### 10.3.6.135		PRACH preamble control parameters extension list for Type 1 (for Enhanced Uplink)

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PRACH preamble control parameters list (for Enhanced Uplink) | MP | 1 to <maxPRACH_EUL> |  |  | REL-11 |
| >PRACH preamble control parameters (for Enhanced Uplink) | MP |  | PRACH preamble control parameters extension (for Enhanced Uplink) 10.3.6.136 | Control parameters of the physical signal. | REL-11 |
| >Weight | OP |  | Real (0.2..1 by step of 0.2) | Probability of selecting this occurrence of PRACH preamble control parameters. | REL-11 |
| NOTE 1: A weight given by: 1 – (sum of the values of the IE "Weight" across all occurrences), shall be used as value of the Kth occurrence of the IE "Weight”, where K is as defined in sub-clause 8.5.73 and 8.5.74. |  |  |  |  |  |

NOTE 2:	The sum of the values of the IE "Weight" across all occurrences in this IE should be less than or equal to 1.

#### 10.3.6.136		PRACH preamble control parameters extension (for Enhanced Uplink)

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Available Signature | MP |  | Bit string(16) | Each bit indicates availability for a signature. Each available signature on the AICH is associated with one Common E-DCH Resource Configuration in the “Common E-DCH resource configuration information list”. | REL-11 |
| Preamble scrambling code number | MP |  | Integer (0 .. 15) | Identification of scrambling code, see [28] | REL-11 |
| AICH info | MD |  | AICH Info compressed 10.3.6.137 | Default value is the value of "AICH info" in the first occurrence in list “PRACH system information list” in SIB5 or SIB5bis. | REL-11 |
| NOTE: The value of the IE’s "E-AI indication", "Available Sub Channel Number", "PRACH partitioning", "Persistence scaling factors", "AC-to-ASC mapping", "Primary CPICH TX power", "Constant value", "PRACH power offset", "PRACH transmission parameters", "Power offset Pp-e" are used from the IE "PRACH preamble control parameters (for Enhanced Uplink)". |  |  |  |  |  |

#### 10.3.6.137	AICH Info compressed

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Channelisation code | MP |  | Integer(0..255) | SF is fixed and equal to 256 |
| NOTE: The value of the IE’s "STTD indicator" and "AICH transmission timing" are used from the IE "AICH info" in the first occurrence in the list “PRACH system information list” in SIB5 or SIB5bis. |  |  |  |  |

#### 10.3.6.138	Common E-RGCH info FDD

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Configuration info | MP |  |  |  | REL-11 |
| >Continue |  |  | (no data) | Used in reconfigurations without change to the Common E-RGCH configuration. | REL-11 |
| >New configuration |  |  |  |  |  |
| >>E-RGCH Neighbour Cell List | MP |  | Bitstring (maxCellMeas) | Indicates whether the UE should evaluate a cell for performing E-RGCH reception. Bit 0 is for the first cell in "New intra-frequency cells" in the "Intra-frequency cell info list" included in system information, Bit 1 is for the second cell in "New intra-frequency cells" in the "Intra-frequency cell info list" included in system information, … Value '1' for a bit means UE should evaluate the cell. Bit 0 is the first/leftmost bit of the bit string. NOTE: The bit that corresponds to the serving cell itself is ignored. | REL-11 |
| >>Common E-RGCH channel configuration list | MP | 1 to < maxCellMeas > |  | List of E-RGCH information of cells that a UE should evaluate for performing E-RGCH reception. The first entry in the list corresponds to thefirst/leftmost bit of value ‘1’ in E-RGCH Neighbour Cell List. The second entry in the list corresponds to the second bit of value ‘1’ in the E-RGCH Neighbour Cell List. … | REL-11 |
| >>>Channelisation Code | CV-Common |  | Integer (0..127) | If this IE is not present, the value is equal to the last occurrence of this IE. | REL-11 |
| >>>Signature Sequence | CV-Common |  | Integer (0..39) | If this IE is not present, the value is equal to the last occurrence of this IE. | REL-11 |
| >>Minimum Serving Grant | MD |  | Integer (0..37) | (0..37) indicates E-DCH serving grant index as defined in [15]. Default value is Initial Serving grant value broadcasted in the Common E-DCH system info. | REL-11 |
| >>Reporting Range Constant | MP |  | Real(0..14.5 by step of 0.5) | In dB. | REL-11 |
| >>Filter coefficient | MP |  | Filter coefficient 10.3.7.9 | Filter coefficient | REL-11 |

| Condition | Explanation |
| --- | --- |
| Common | This IE is mandatory present for the first occurrence. Otherwise, this IE is optional. |

#### 10.3.6.139	HS-DSCH DRX in CELL_FACH with second DRX cycle information

These parameters configure the UE in CELL_FACH state to discontinuously receive HS-DSCH with second DRX cycle.

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| HS-DSCH second DRX cycleFACH | MP |  | Enumerated (4, 8, 16, 32, 64, 128, 256, 512) | Determines the length of the DRX Cycle during second DRX operation, in frames. | REL-11 |
| CHOICE DRX level | MP |  |  | This IE indicates whether both the 1st and the 2nd DRX cycle are used (2-level DRX) or only the 2nd DRX cycle is used (1-level DRX) | REL-11 |
| >1-level DRX |  |  |  |  | REL-11 |
| >>HS-DSCH second Rx burstFACH | MD |  | Enumerated (1, 2) | Determines the period within the second DRX Cycle that the UE continuously receives HS-DSCH, in frames. Default value is the value included in the IE "HS-DSCH Rx burstFACH" of the System Information Block Type 5/5bis. | REL-11 |
| >>T329 | MD |  | Enumerated (0.5, 1, 2, 4) | Determines the time the UE waits until initiating second DRX operation, in seconds. The default value is T321 (ms) contained in the IE "HS-DSCH DRX in CELL_FACH information" of the System Information Block Type 5/5bis. | REL-11 |
| >2-level DRX |  |  |  |  | REL-11 |
| >>T328 | MD |  | Enumerated (20, 40, 60, 80) | Determines the time the UE waits until initiating first DRX operation, in ms. The default value is T321 (ms) contained in the IE "HS-DSCH DRX in CELL_FACH information" of the System Information Block Type 5/5bis. | REL-11 |
| >>HS-DSCH first Rx burstFACH | MD |  | Enumerated (0.4, 0.8) | Determines the period within the first DRX Cycle that the UE continuously receives HS-DSCH, in frames. Default value is the value included in the IE "HS-DSCH Rx burstFACH" of the System Information Block Type 5/5bis. | REL-11 |
| >>HS-DSCH first DRX cycleFACH | MD |  | Enumerated (2, 4, 8, 16, 32, 64) | Determines the length of the DRX Cycle during first DRX operation, in frames. Default value is the value included in the IE "HS-DSCH DRX cycleFACH" of the System Information Block Type 5/5bis. | REL-11 |
| >>HS-DSCH second Rx burstFACH | MD |  | Enumerated (1, 2) | Determines the period within the second DRX Cycle that the UE continuously receives HS-DSCH, in frames. Default value is the value included in the IE "HS-DSCH Rx burstFACH" of the System Information Block Type 5/5bis. | REL-11 |
| >>T329 | MD |  | Enumerated (0.5, 1, 2, 4) | Determines the time the UE waits until initiating second DRX operation, in seconds. The default value is T321 (ms) contained in the IE "HS-DSCH DRX in CELL_FACH information" of the System Information Block Type 5/5bis. | REL-11 |

#### 10.3.6.140	Common E-DCH Resource Configuration Information List Extension

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Scheduled Transmission configuration | OP |  |  |  | REL-11 |
| >Configuration for 2 ms TTI common E-DCH resources | MP | 1 to <maxEDCHs> |  |  | REL-11 |
| >>2ms HARQ process allocation | MD |  | Bitstring (8) | Bit 0 is the first/leftmost bit of the bit string. MAC-c or MAC-d PDUs for this common E-DCH MAC-d flow are only allowed to be transmitted in those processes for which the bit is set to “1”. Bit 0 corresponds to HARQ process 0, bit 1 corresponds to HARQ process 1,… NOTE1 | REL-11 |
| Coffset | OP |  | Integer(0..29) | (0..29) indicates cell offset as defined in [26] | REL-11 |
| NOTE1: The default value is equal to the same value of the last occurrence. The default value of the first occurrence is that transmission in the first HARQ processes is allowed. |  |  |  |  |  |

#### 10.3.6.141	Fallback R99 PRACH info

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CCCH Fallback | MP |  | BOOLEAN |  | REL-11 |
| DCCH Fallback | MP |  | BOOLEAN |  | REL-11 |

#### 10.3.6.142	MIMO mode with four transmit antennas parameters

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Configuration info | MP |  |  |  | REL-11 |
| >Continue |  |  | (no data) | Used in reconfigurations without interruption of MIMO mode with four transmit antennas. | REL-11 |
| >New configuration |  |  |  |  | REL-11 |
| >>MIMO mode with four transmit antennas N_cqi_typeA/M_cqi ratio | OP |  | Enumerated(1/2, 2/3, 3/4, 4/5, 5/6, 6/7, 7/8, 8/9, 9/10, 1/1) |  | REL-11 |
| >>MIMO mode with four transmit antennas pilot configuration | OP |  | MIMO mode with four transmit antennas pilot configuration 10.3.6.143 |  | REL-11 |
| >>Precoding weight set restriction | OP |  | Bit string (64) | The first/leftmost bit contains the most significant bit, where a bit value of zero indicates that the precoding indices reporting is not allowed, as defined in [29]. | REL-11 |

#### 10.3.6.143	MIMO mode with four transmit antennas pilot configuration

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Antenna2 S-CPICH | OP |  |  |  | REL-11 |
| >Channelisation code | MP |  | Integer (0..255) |  | REL-11 |
| >Power Offset for S-CPICH for MIMO mode with four transmit antennas on Antenna2 | OP |  | Integer(-6 .. 0) | Power offset relative to the P-CPICH Tx power in dB | REL-11 |
| Antenna3and4 | OP |  |  |  | REL-11 |
| >Antenna3and4 S-CPICH | OP |  |  |  | REL-11 |
| >>Antenna3 S-CPICH | OP |  |  |  | REL-11 |
| >>>Channelisation code | MP |  | Integer (0..255) |  | REL-11 |
| >>Antenna4 S-CPICH | OP |  |  |  | REL-11 |
| >>>Channelisation code | MP |  | Integer (0..255) |  | REL-11 |
| >>Common Power Offset for S-CPICH for MIMO mode with four transmit antennas on Antenna3 and 4 | OP |  | Integer(-12 .. 0) | Power offset relative to the P-CPICH Tx power in dB | REL-11 |
| >Antenna3and4 D-CPICH | OP |  |  |  | REL-11 |
| >>Antenna3 D-CPICH | OP |  |  |  | REL-11 |
| >>>Channelisation code | MP |  | Integer (0..255) |  | REL-11 |
| >>Antenna4 D-CPICH | OP |  |  |  | REL-11 |
| >>>Channelisation code | MP |  | Integer (0..255) |  | REL-11 |
| >>Common Power Offset for D-CPICH for MIMO mode with four transmit antennas on Antenna3 and 4 | OP |  | Integer(-12 .. 0) | Power offset relative to the P-CPICH Tx power in dB | REL-11 |
| >>Initial status of D-CPICH | OP |  | Enumerated(activated) | The absence of this IE indicates that the initial status of D-CPICH is deactivated. | REL-11 |

#### 10.3.6.144	Secondary cell MIMO mode with four transmit antennas parameters

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Configuration info | MP |  |  |  | REL-11 |
| >Continue |  |  | (no data) | Used in reconfigurations without interruption of MIMO mode with four transmit antennas operation. | REL-11 |
| >New configuration |  |  |  |  | REL-11 |
| >> MIMO mode with four transmit antennas N_cqi_typeA/M_cqi ratio | OP |  | Enumerated(1/2, 2/3, 3/4, 4/5, 5/6, 6/7, 7/8, 8/9, 9/10, 1/1) |  | REL-11 |
| >>MIMO mode with four transmit antennas pilot configuration | OP |  | MIMO mode with four transmit antennas pilot configuration 10.3.6.143 |  | REL-11 |
| >>Precoding weight set restriction | OP |  | Bit string (64) | The first/leftmost bit contains the most significant bit, where a bit value of zero indicates that the precoding indices reporting is not allowed, as defined in [29]. | REL-11 |

#### 10.3.6.145	Uplink MIMO info FDD

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Configuration Info |  |  |  |  | REL-11 |
| >Continue |  |  | (no data) | Used in reconfigurations without interruption of uplink MIMO operation. | REL-11 |
| >New configuration |  |  |  |  |  |
| >>S-E-DPCCH Power Offset | MP |  | Integer (0..17) | This IE provides an index for the S-E-DPCCH power offset mapping, as specified in [9] subclause 4.2.1.5. | REL-11 |
| >>Minimum E-TFCI for Rank2 Transmission | MP |  | Integer (0..127) |  | REL-11 |
| >>Inter-stream Interference Compensation Index | OP |  | Integer (0..15) | If present, this IE provides an index for the inter-stream interference compensation factor ISI, as specified in [28] subclause 4.2.1.3. | REL-11 |

#### 10.3.6.146	E-ROCH info FDD

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Channelization Code | MP |  | Integer (0..255) |  | REL-11 |
| E-ROCH E-RNTI | MP |  | E-RNTI 10.3.3.10a |  | REL-11 |

#### 10.3.6.147	Radio Links without DPCH/F-DPCH info

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Frame offset | MP |  | Integer (0..38144 by step of 256) | Offset (in number of chips) between the beginning of the P-CCPCH frame and the beginning of the DPCH/F-DPCH frame This is called DPCH,n or F-DPCH,n in [26] | REL-12 |

#### 10.3.6.148	DPCCH2 info FDD

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Configuration Info | MP |  |  |  | REL-12 |
| >Continue |  |  | (no data) | Used in reconfigurations without change to the DPCCH2 configuration. | REL-12 |
| >New configuration |  |  |  |  | REL-12 |
| >>DPCCH2 info | OP |  |  |  | REL-12 |
| >>>DPCCH2 Tx Power offset | MP |  | Integer (0..20) | In dB | REL-12 |
| >>F-DPCH info | OP |  |  |  | REL-12 |
| >>>F-DPCH slot format | OP |  | Integer (0..9) | Slot format used by F-DPCH for DPCCH2 in [26]. | REL-12 |
| >>>Code number | OP |  | Integer (0..255) |  | REL-12 |
| >>>TPC command error rate target | OP |  | Real (0.01..0.1 by step of 0.01) |  | REL-12 |
| >>Extended E-DPCCH/DPCCH power offset | OP |  | Integer (9..15) | Refer to quantization of the power offset in [28] | REL-12 |
| >>Designated non-serving HS-DSCH cell info | OP |  |  |  | REL-12 |
| >>>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-12 |

#### 10.3.6.149	DCH Enhancements info FDD

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Configuration info | MP |  |  |  | REL-12 |
| >Continue |  |  | (no data) | Used in reconfigurations without interruption of DCH Enhancements operation. Note 1. | REL-12 |
| >New configuration |  |  |  |  | REL-12 |
| >>CHOICE UL Transmission Mode | MP |  |  |  | REL-12 |
| >>>10ms only |  |  | (no data) | All TFC’s are always transmitted in 10ms Mode | REL-12 |
| >>>20ms only |  |  | (no data) | All TFC’s are always transmitted in 20ms Mode | REL-12 |
| >>>10ms or 20ms |  |  |  | TFC’s can be transmitted in 10ms Mode or 20ms Mode as per Uplink Transmission Mode Swtiching procedure described in [15]. | REL-12 |
| >>>>Uplink Transmission Mode switching parameters | MP |  | Uplink Transmission Mode Switching Parameters 10.3.6.150 | K, L, M parameters used for Trasmission Mode Switching procedure described in [15]. | REL-12 |
| >>CHOICE DL FET Mode | MP |  |  |  | REL-12 |
| >>>Basic |  |  | (no data) | In this mode, the ACK/NACK field in UL DPCCH does not indicate whether DL DPCH has been succefully decoded. | REL-12 |
| >>>Full |  |  |  | In this mode, the ACK/NACK field in UL DPCCH is used to indicate whether DL DPCH has been succefully decoded. | REL-12 |
| >>>>Early DCH quality target | MP |  | Quality target 10.3.5.10 |  | REL-12 |
| >>>>Early DCH quality target slot | MP |  | Integer (11.. 28) |  | REL-12 |
| >>>>TrCh Concatenation Info | MP | 1 to <maxNrOfConcatTrCH> |  | Provides the list of DL Transport Channels of type DCH that are subject to concatenation in the physical layer. | REL-12 |
| >>>>>DCH ID | MP |  | Transport channel identity 10.3.5.18 | DL transport channel type = DCH | REL-12 |
| Note 1: The value "Continue" should not be set when the "DCH Enhancements info FDD" IE is included in the HANDOVER TO UTRAN COMMAND message. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCM_opt | This IE is OPTIONAL when sent in SYSTEM INFORMATION. Otherwise, the IE is not needed. |

#### 10.3.6.150	Uplink Transmission Mode Switching Parameters

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| K | MP |  | Integer (1..32) | See [19]. | REL-12 |
| L | MP |  | Integer (1..32) | See [19]. | REL-12 |
| M | MP |  | Integer (1..32) | See [19]. | REL-12 |

#### 10.3.6.151	DPCCH Power Reset after DTX on secondary uplink frequency

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Filter coefficient | MP |  | Integer (0..5) |  | REL-12 |
| Secondary Power Offset After Gap | MP |  | Integer (-8..20) by steps of 2 | Unit is in dB. An offset which is applied to a filtered DPCCH transmit power taken from the primary uplink frequency. [29] | REL-12 |
| Inactivity Threshold for Reset DPCCH Power | OP |  | Enumerated (10, 20, 30, 40, 50, 60, 80, 120) | Value in ms. The inactivity gap allowed on DPCCH after which the power for secondary uplink frequency is determined as an offset of the transmit power on primary uplink frequency. [29] Absense of this IE means that the UE shall apply a threshold of infinity for the inactivity gap on the DPCCH | REL-12 |

#### 10.3.6.152	Other TTI E-DCH Configuration Information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Configuration info | MP |  |  |  | REL-12 |
| >Continue |  |  | (no data) |  | REL-12 |
| >New configuration |  |  |  |  | REL-12 |
| >>E-DCH Info | MP |  | E-DCH Info 10.3.6.97 |  | REL-12 |
| >>Added or Reconfigured UL TrCH information | MP |  | Added or Reconfigured UL TrCH information 10.3.5.2 |  | REL-12 |
| >>DTX-DRX Information | OP |  | DTX-DRX Information 10.3.6.34a |  | REL-12 |
| >>DTX-DRX timing information | OP |  | DTX-DRX timing information 10.3.6.34b |  | REL-12 |
| Activation delay | MD |  | Enumerated (0, 1, 2, 3, 4, 5) | In radio frames. Default value is 0. Two spare values are needed. | REL-12 |

#### 10.3.6.153	Power Control Algorithm 3

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| TPC slot position | OP |  | Integer (0..4) | Specifies TPC commands slot position for Algorithm 3 to be used by UE to interpret TPC commands for the radio link | REL-13 |
| TPC step size | OP |  | Integer (1, 2) | In dB | REL-13 |
| Decimation factor | OP |  | Enumerated (3 slots, 5 slots) |  | REL-13 |

### 10.3.7	Measurement Information elements

#### 10.3.7.1	Additional measurements list

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Additional measurements | MP | 1 to <MaxAdditionalMeas> |  |  |
| >Additional measurement identity | MP |  | Measurement identity 10.3.7.48 |  |

#### 10.3.7.2	Cell info

Includes non-frequency related cell info used in the IE "inter-frequency cell info list" and "intra frequency cell info list".

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Cell individual offset | MD |  | Real(-10..10 by step of 0.5) | In dB Default value is 0 dB Used to offset measured quantity value If the cell from which this IE has been received is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. |  |
| Reference time difference to cell | OP |  | Reference time difference to cell 10.3.7.60 | In chips. This IE is absent for serving cell. If the cell from which this IE has been received is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |  |
| Read SFN indicator | MP |  | Boolean | TRUE indicates that read of SFN is requested for the target cell If the cell from which this IE has been received is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Primary CPICH info | OP |  | Primary CPICH info 10.3.6.60 | This IE is absent only if measuring RSSI only (broadband measurement.) |  |
| >>Primary CPICH Tx power | OP |  | Primary CPICH Tx power 10.3.6.61 | Required if calculating pathloss. |  |
| >>TX Diversity Indicator | MP |  | Boolean | TRUE indicates that transmit diversity is used. |  |
| >TDD |  |  |  |  |  |
| >>Primary CCPCH info | MP |  | Primary CCPCH info 10.3.6.57 |  |  |
| >>Primary CCPCH TX power | OP |  | Primary CCPCH TX power 10.3.6.59 |  |  |
| >>Timeslot list | OP | 1 to <maxTS> |  | The UE shall report Timeslot ISCP values according the order of the listed Timeslot numbers. If the cell from which this IE has been received is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |  |
| >>>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>>Timeslot number | MP |  | Integer (0...14) | Timeslot numbers, for which the UE shall report Timeslot ISCP |  |
| >>>>>Burst Type | MD |  | Enumerated (Type1, Type2) | Use for Timeslot ISCP measurements only. Default value is "Type1" |  |
| >>>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>>Timeslot number | MP |  | Integer (0...14) | Timeslot numbers, for which the UE shall report Timeslot ISCP | REL-7 |
| >>>>>Burst Type | MD |  | Enumerated (Type1, Type2) | Use for Timeslot ISCP measurements only. Default value is "Type1" | REL-7 |
| >>>>1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>>>>Timeslot number | MP |  | Integer (0...6) | Timeslot numbers, for which the UE shall report Timeslot ISCP | REL-4 |
| Cell Selection and Re-selection Info | CV-BCHopt |  | Cell Selection and Re-selection for SIB11/12Info 10.3.2.4 | This IE is absent for the serving cell. If the cell from which this IE has been received is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |  |

| Condition | Explanation |
| --- | --- |
| BCHopt | This IE is Optional when sent in SYSTEM INFORMATION or for 1.28 Mcps TDD when sent in the MEASUREMENT CONTROL, Otherwise, the IE is not needed |

#### 10.3.7.3	Cell measured results

Includes non-frequency related measured results for a cell.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Cell Identity | OP |  | Cell Identity 10.3.2.2 |  |  |
| CSG PLMN List | CV-CSG | 1 to 6 |  |  | REL-12 |
| >CSG PLMN Identity | MP |  | PLMN identity 10.3.1.11 |  | REL-12 |
| CSG Identity | CV-CSG |  | CSG Identity 10.3.2.8 |  | REL-9 |
| CSG Member indication | CV-CSG |  | Enumerated(member) |  | REL-9 |
| Cell synchronisation information | OP |  | Cell synchronisation information 10.3.7.6 |  |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  |  |
| >>CPICH Ec/N0 | OP |  | Integer(0..49) | According to CPICH_Ec/No in [19] and [20]. Fourteen spare values are needed. |  |
| >>CPICH RSCP | OP |  | Integer(0..91) | According to CPICH_RSCP in [19] and [20]. Thirty-six spare values are needed. |  |
| >>DeltaCPICH RSCP | CV-RSCP |  | Integer(-5..-1) | If present, the actual value of CPICH RSCP = CPICH RSCP+ DeltaCPICH RSCP | REL-5 |
| >>Pathloss | OP |  | Integer(46..158) | In dB. Fifteen spare values are needed. |  |
| >TDD |  |  |  |  |  |
| >>Cell parameters Id | MP |  | Cell parameters Id 10.3.6.9 |  |  |
| >>Proposed TGSN | OP |  | Integer (0..14) | Proposal for the next TGSN |  |
| >>Primary CCPCH RSCP | OP |  | Primary CCPCH RSCP info 10.3.7.54 |  |  |
| >>Pathloss | OP |  | Integer(46..158) | In dB. Fifteen spare values are needed. |  |
| >>Timeslot list | OP | 1 to < maxTS> |  |  |  |
| >>>Timeslot ISCP | MP |  | Timeslot ISCP Info 10.3.7.65 | The UE shall report the Timeslot ISCP in the same order as indicated in the cell info |  |

| Condition | Explanation |
| --- | --- |
| RSCP | This IE is mandatory if the IE "Cell measured results" is included in the IE "Measured Results" (i.e. not included in the IE "Additional Measured Results") and if CPICH RSCP is present and if the value of the CPICH RSCP is below 0. It is not needed otherwise. |
| CSG | This IE is optionally present if the target cell’s Primary scrambling code (as determined from Primary CPICH info IE) is found in the CELL_INFO_CSG_LIST set of PSCs. Otherwise it is not needed. |

#### 10.3.7.4	Cell measurement event results

Includes non-frequency related cell reporting quantities.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Primary CPICH info | MP | 1 to <maxCellMeas> | Primary CPICH info 10.3.6.60 |  |  |
| >TDD |  |  |  |  |  |
| >>Primary CCPCH info | MP | 1 to <maxCellMeas> | Primary CCPCH info 10.3.6.57 |  |  |

#### 10.3.7.5	Cell reporting quantities

Includes non-frequency related cell reporting quantities.

For all boolean types TRUE means inclusion in the report is requested.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell synchronisation information reporting indicator | MP |  | Boolean |  |
| Cell Identity reporting indicator | MP |  | Boolean |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>CPICH Ec/N0 reporting indicator | MP |  | Boolean |  |  |
| >>CPICH RSCP reporting indicator | MP |  | Boolean |  |  |
| >>Pathloss reporting indicator | MP |  | Boolean |  |  |
| >TDD |  |  |  |  |  |
| >>Timeslot ISCP reporting indicator | MP |  | Boolean |  |  |
| >>Proposed TGSN Reporting required | MP |  | Boolean |  |  |
| >>Primary CCPCH RSCP reporting indicator | MP |  | Boolean |  |  |
| >>Pathloss reporting indicator | MP |  | Boolean |  |  |

#### 10.3.7.6	Cell synchronisation information

The IE "Cell synchronisation information" contains the OFF and Tm as defined in [7] and [8] and the four most significant bits of the difference between the 12 least significant bits of the RLC Transparent Mode COUNT-C in the UE and the SFN of the measured cell. It is notified to SRNC by Measurement Report message or Measurement Information Element in other RRC messages

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |
| >FDD |  |  |  |  |
| >>COUNT-C-SFN frame difference | OP |  |  |  |
| >>>COUNT-C-SFN high | MP |  | Integer(0..3840 by step of 256) | in frames |
| >>>OFF | MP |  | Integer(0..255) | in frames |
| >>Tm | MP |  | Integer(0..38399) | in chips |
| >TDD |  |  |  |  |
| >>COUNT-C-SFN frame difference | OP |  |  |  |
| >>>COUNT-C-SFN high | MP |  | Integer(0..3840 by step of 256) | in frames |
| >>>OFF | MP |  | Integer(0..255) | in frames |

#### 10.3.7.6a	E-UTRA event results

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Inter-RAT event identity | MP |  | Inter-RAT event identity 10.3.7.24 |  | REL-8 |
| E-UTRA events results list | MP | 1 to <maxReportedEUTRAFreqs> |  |  | REL-8 |
|  |  | 1 to <maxReportedEUTRAFreqs-ext> |  |  | REL-12 |
| >E-UTRA Carrier Frequency | MP |  | Integer (0..65535) | EARFCN of the downlink carrier frequency [64]. If the IE indicates a value of 65535, then the EARFCN for this instance should be read from the corresponding instance of IE "EARFCN extension" in the E-UTRA events results extension list. | REL-8 |
| >Reported cells | MP | 1 to <maxReportedEUTRACellPerFreq> |  |  | REL-8 |
| >>Physical Cell Identity | MP |  | Integer (0..503) |  | REL-8 |
| E-UTRA events results extension list | OP | 1 to <maxReportedEUTRAFreqs> |  |  | REL-11 |
|  |  | 1 to <maxReportedEUTRAFreqs-ext> |  |  | REL-12 |
| >EARFCN extension | OP |  | Integer (65536..262143) | EARFCN of the downlink carrier frequency [64]. | REL-11 |

#### 10.3.7.6b	E-UTRA frequency list

Contains the information for the list of measurement objects for E-UTRA measurements.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE E-UTRA frequency removal | MP |  |  |  | REL-8 |
| >Remove all frequencies |  |  |  | (no data) | REL-8 |
| >Remove some frequencies |  |  |  |  | REL-8 |
| >>Removed frequencies | MP | 1 to <maxNumEUTRAFreqs> |  |  | REL-8 |
| >>>E-UTRA frequencies | MP |  | Integer (0..65535) | EARFCN of the downlink carrier frequency [64]. If the IE indicates a value of 65535, then the EARFCN for this instance should be read from the corresponding instance of IE "EARFCN extension". | REL-8 |
| >>>EARFCN extension | OP |  | Integer (65536..262143) | EARFCN of the downlink carrier frequency [64]. | REL-11 |
| >Remove no frequencies |  |  |  | (no data) | REL-8 |
| New frequencies | OP | 1 to <maxNumEUTRAFreqs> |  |  | REL-8 |
| >E-UTRA carrier frequency | MP |  | Integer (0..65535) | EARFCN of the downlink carrier frequency [64]. If the IE indicates a value of 65535, then the EARFCN for this instance should be read from the corresponding instance of IE "EARFCN extension". It is always ensured by the UTRAN that more than one measurement object for the same physical frequency is not configured regardless of the EARFCN used to indicate this. | REL-8 |
| >EARFCN extension | OP |  | Integer (65536..262143) | EARFCN of the downlink carrier frequency [64]. It is always ensured by the UTRAN that more than one measurement object for the same physical frequency is not configured regardless of the EARFCN used to indicate this. | REL-11 |
| >Reduced measurement performance | OP |  | Enumerated (TRUE) | TRUE indicates that the frequency is configured for reduced measurement performance [19]. Absense indicates that the frequency is configured for normal measurement performance [19]. | REL-12 |
| >Measurement Bandwidth | MD |  | Enumerated (6, 15, 25, 50, 75, 100) | Measurement bandwidth information common for all neighbouring cells on the carrier frequency. It is defined by the parameter Transmission Bandwidth Configuration, NRB [36.104]. The values indicate the number of resource blocks over which the UE could measure. Default value is 6. | REL-8 |
| >Blacklisted cells list | OP | 1 to <maxEUTRACellPerFreq> |  | A list of blacklisted cells can be signalled per frequency | REL-8 |
| >>Physical Cell identity | MP |  | Integer (0..503) |  | REL-8 |
| >Wideband RSRQ measurements | CV-WB-RSRQ |  | Enumerated (TRUE) | If this field is present, the UE shall use a wider bandwidth when performing RSRQ measurements according to TS 36.133 [74]. | REL-11 |
| E-UTRA SI Acquisition | OP |  | 10.3.7.127 |  | REL-9 |
| RSRQ measurement on all symbols | OP |  | Boolean | TRUE means that RSRQ measurement on all OFDM symbols is configured.If wideband RSRQ measurement is enabled for the frequency, the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols with wider bandwidth for concerned frequency; FALSE or absence of this field means that the UE shall, when performing RSRQ measurements, perform RSRQ measurement only on CRS symbols; | REL-12 |

| Condition | Explanation |
| --- | --- |
| WB-RSRQ | The field is optionally present if the measurement bandwidth indicated by Measurement Bandwidth is 50 resource blocks or larger. Otherwise it is not needed. |

#### 10.3.7.6c	E-UTRA measured results

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-UTRA measured results list | MP | 1 to <maxReportedEUTRAFreqs> |  |  | REL-8 |
|  |  | 1 to <maxReportedEUTRAFreqs-ext> |  |  | REL-12 |
| >E-UTRA Carrier Frequency | MP |  | Integer (0..65535) | EARFCN of the downlink carrier frequency [64]. If the IE indicates a value of 65535, then the EARFCN for this instance should be read from the corresponding instance of IE "EARFCN extension" in the E-UTRA measured results extension list. | REL-8 |
| >Measured E-UTRA cells | MP | 1 to <maxReportedEUTRACellPerFreq> |  |  | REL-8 |
| >>Physical Cell Identity | MP |  | Integer (0..503) |  | REL-8 |
| >>RSRP | OP |  | Integer (0..97) | This shall be reported if the “Inter-RAT measurement quantity” IE is set to ‘RSRP’ or the “Inter-RAT reporting quantity” IE is set to ‘both’. RSRP is mapped to a value between 0 and 97 [36.133]. | REL-8 |
| >>RSRQ | OP |  | Integer (0..33) | This quantity shall be reported if the “Inter-RAT measurement quantity” IE is set to ‘RSRQ’ or the “Inter-RAT reporting quantity” IE is set to ‘both’. RSRQ_00 to RSRQ_33 in [36.133] are mapped to the value between 0 and 33. RSRQ_34 in [36.133] is mapped to the value 33. If the IE indicates a value of 0 and the RSRQ Extension IE is present, then the RSRQ for this instance should be read from the corresponding instance of IE "RSRQ extension", the range is from -30 to -1. If the IE indicates a value of 33 and the RSRQ Extension IE is present, then the RSRQ for this instance should be read from the corresponding instance of IE "RSRQ extension", the range is from 35 to 46. | REL-8 |
| >>RSRQ Extension | OP |  | Integer (-30..46) | RSRQ_-30 to RSRQ_-1 in [36.133] are mapped to the value between -30 and -1. RSRQ_35 to RSRQ_46 in [36.133] are mapped to the value between 35 and 46. | REL-12 |
| >>E-UTRA Results for SI Acquisition | OP |  | 10.3.7.128 |  | REL-9 |
| E-UTRA measured results extension list | OP | 1 to <maxReportedEUTRAFreqs> |  |  | REL-11 |
|  |  | 1 to <maxReportedEUTRAFreqs-ext> |  |  | REL-12 |
| >EARFCN extension | OP |  | Integer (65536..262143) | EARFCN of the downlink carrier frequency [64]. | REL-11 |

#### 10.3.7.7	Event results

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE event result | MP |  |  | One spare value is needed. |
| >Intra-frequency measurement event results |  |  | Intra-frequency measurement event results 10.3.7.37 |  |
| >Inter-frequency measurement event results |  |  | Inter-frequency measurement event results 10.3.7.17 |  |
| >Inter-RAT measurement event results |  |  | Inter-RAT measurement event results 10.3.7.28 | For IS-2000 results, include fields of the Pilot Strength Measurement Message from subclause 2.7.2.3.2.5 of TIA/EIA/IS-2000.5 |
| >Traffic volume measurement event results |  |  | Traffic volume measurement event results 10.3.7.69 |  |
| >Quality measurement event results |  |  | Quality measurement event results 10.3.7.57 |  |
| >UE internal measurement event results |  |  | UE internal measurement event results 10.3.7.78 |  |
| >UE positioning measurement event results |  |  | UE positioning measurement event results 10.3.7.101 |  |

| CHOICE event result | Condition under which the given event result is chosen |
| --- | --- |
| Intra-frequency measurement event results | If measurement type = intra-frequency measurement |
| Inter-frequency measurement event results | If measurement type = inter-frequency measurement |
| Inter-RAT measurement event results | If measurement type = inter-RAT measurement |
| Traffic volume measurement event results | If measurement type = traffic volume measurement |
| Quality measurement event results | If measurement type = Quality measurement |
| UE internal measurement event results | If measurement type = UE internal measurement |
| UE positioning measurement event results | If measurement type = UE positioning measurement |

#### 10.3.7.8	FACH measurement occasion info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| FACH Measurement occasion cycle length coefficient | OP |  | Integer(1..12) |  |  |
| Inter-frequency FDD measurement indicator | MP |  | Boolean | TRUE means that measurements are required |  |
| Inter-frequency TDD 3.84 Mcps measurement indicator | MP |  | Boolean | TRUE means that measurements are required | REL-4 |
| Inter-frequency TDD 7.68 Mcps measurement indicator | MP |  | Boolean | TRUE means that measurements are required | REL-7 |
| Inter-frequency TDD 1.28 Mcps measurement indicator | MP |  | Boolean | TRUE means that measurements are required | REL-4 |
| Inter-RAT measurement indicators | OP | 1 to <maxOtherRAT> |  |  |  |
| >RAT type | MP |  | Enumerated(GSM, IS2000) | NOTE1 |  |
| NOTE1: If "CELL_FACH Absolute Priority Measurement Indicator" is received in SIB19 then measurement occasions configured for GSM shall be used by the UE for GSM and/or EUTRAN measurements according to the rules in [4] |  |  |  |  |  |

#### 10.3.7.9	Filter coefficient

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Filter coefficient | MD |  | Integer(0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 13, 15, 17, 19) | Default value is 0 |

#### 10.3.7.9a	GSM cell group

This IE encodes a list of GSM cells specified by their ARFCN values [45].

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Starting ARFCN | MP |  | Integer (0..1023) | First ARFCN value in the set | REL-8 |
| Band Indicator | MP |  | Enumerated (dcs1800, pcs1900) | GSM BAND_INDICATOR [45] | REL-8 |
| CHOICE Following ARFCNs | MP |  |  |  | REL-8 |
| >Explicit list |  |  |  |  | REL-8 |
| >>List of ARFCNs | MP | 0 to 31 | Integer (0..1023) | Following ARFCN values | REL-8 |
| >Equally spaced |  |  |  |  | REL-8 |
| >>ARFCN spacing | MP |  | Integer (1..8) | Increment "d" ARFCN values | REL-8 |
| >>Number of following ARFCNs | MP |  | Integer (0..31) | Number "n" of following ARFCN values, NOTE 1 | REL-8 |
| >Variable bitmap format |  |  |  |  | REL-8 |
| >>Bitmap | MP |  | Octet string (1..16) | NOTE 2 | REL-8 |
| >Continuous range |  |  |  |  | REL-8 |
| >>Ending ARFCN | MP |  | Integer (0..1023) | Last ARFCN value in the set, NOTE 3 | REL-8 |
| NOTE 1: Let the IE "Starting ARFCN" = s. The complete set of (n+1) ARFCN values is defined as: {s, ((s + d) mod 1024), ((s + 2*d) mod 1024) ... ((s + n*d) mod 1024)}. |  |  |  |  |  |
| NOTE 2: Bitmap representing the following ARFCN values in the set. Let the IE "Starting ARFCN" = s. The first bit of the first octet in the bitmap corresponds to ARFCN = ((s + 1) mod 1024), the next bit to the ARFCN = ((s + 2) mod 1024), and so on. If the bitmap consist of N octets, the last bit of octet N corresponds to ARFCN = ((s + 8*N) mod 1024). The complete set of ARFCN values consists of ARFCN = s and the ARFCN values, where the corresponding bit in the bitmap is set to "1". |  |  |  |  |  |
| NOTE 3: Let the IEs "Starting ARFCN" = s and "Ending ARFCN" = t. The complete set of ARFCN values is defined as: {s, ((s + 1) mod 1024), ((s + 2) mod 1024) ... ((t − 1) mod 1024), t}. |  |  |  |  |  |

#### 10.3.7.10	HCS Cell re-selection information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Penalty_time | MD |  | Integer(0, 10, 20, 30, 40, 50, 60) | Default value is 0 which means = not used In seconds |
| Temporary_offsets | CV-Penalty used |  |  |  |
| >Temporary_offset1 | MP |  | Integer(3, 6, 9, 12, 15, 18, 21, inf) | [dB] |
| >Temporary_offset2 | CV-FDD-Quality-Measure |  | Integer(2, 3, 4, 6, 8, 10, 12, inf) | [dB] |

| Condition | Explanation |
| --- | --- |
| Penalty used | This IE is not needed if the IE "Penalty time" equals "not used", else it is mandatory present. |
| FDD-Quality-Measure | This IE is not needed if the IE "Cell selection and reselection quality measure" has the value CPICH RSCP, otherwise the IE is mandatory present. This conditional presence is implemented in ASN.1 by the use of a specific RSCP and EcN0 variant of 10.3.7.10. |

#### 10.3.7.11	HCS neighbouring cell information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| HCS_PRIO | MD |  | Integer (0..7) | Default value = 0 |
| Qhcs | MD |  | Qhcs 10.3.7.54a | Default value = 0 |
| HCS Cell Re-selection Information | MP |  | HCS Cell Re-selection Information 10.3.7.10 |  |

#### 10.3.7.12	HCS Serving cell information

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| HCS_PRIO | MD |  | Integer (0..7) | Default value = 0 |
| Qhcs | MD |  | Qhcs 10.3.7.54a | Default value = 0 |
| TCRmax | MD |  | Enumerated(not used, 30, 60, 120, 180, 240) | [s] Default value is not used |
| NCR | CV-UE speed detector |  | Integer(1..16) | Default value = 8 |
| TCrmaxHyst | CV-UE speed detector |  | Enumerated(not used, 10, 20, 30, 40, 50, 60, 70) | [s] |

| Condition | Explanation |
| --- | --- |
| UE Speed detector | This IE is not needed if TCrmax equals 'not used', else it is mandatory present. |

#### 10.3.7.12a	Idle Interval Information

NOTE:	Used for TDD only. If the multi-RAT UE needs idle interval for E-UTRAN measurements in CELL_DCH state, this IE may be configured.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| k | MD |  | Integer (2..3) | This IE is the coefficient parameter to calculate the idle interval period. Default value is 2. The actual idle interval period equal to 2^k radio frames. | REL-8 |
| Offset | MD |  | Integer (0..7) | The idle interval position in the period. Default value is 0. | REL-8 |

#### 10.3.7.13	Inter-frequency cell info list

Contains the information for the list of measurement objects for an inter-frequency measurement.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Inter-frequency cell removal | OP |  |  |  |  |
| >Remove all inter-frequency cells |  |  |  | No data |  |
| >Remove some inter-frequency cells |  |  |  |  |  |
| >>Removed inter-frequency cells | MP | 1 .. <maxCellMeas> |  |  |  |
|  |  | 1 .. <maxCellMeas-ext> |  | NOTE 2 | REL-12 |
| >>>Inter-frequency cell id | MP |  | Integer(0 .. <maxCellMeas>-1) |  |  |
|  |  |  | Integer(0 .. <maxCellMeas-ext>-1) | NOTE 2 | REL-12 |
| >No inter-frequency cells removed |  |  |  | No data |  |
| New inter-frequency cells | OP | 1 to <maxCellMeas> |  |  |  |
|  |  | 1 to <maxCellMeas-ext> |  | NOTE 2 | REL-12 |
| >Inter-frequency cell id | MD |  | Integer(0 .. <maxCellMeas>-1) |  |  |
|  |  |  | Integer(0 .. <maxCellMeas-ext>-1) | NOTE 2 | REL-12 |
| >Frequency info | MD |  | Frequency info 10.3.6.36 | Default value is the value of the previous "frequency info" in the list. It is always ensured by the UTRAN that more than one measurement object for the same physical frequency is not configured regardless of the UARFCN used to indicate this. NOTE: The first occurrence is then MP. |  |
| >Cell info | MP |  | Cell info 10.3.7.2 |  |  |
| >CHOICE mode specific info | MP |  |  |  | REL-7 |
| >>No information |  |  |  | No data for FDD, TDD 3.84 Mcps or TDD 7.68 Mcps | REL-7 |
| >>TDD 1.28 Mcps |  |  |  |  | REL-7 |
| >>>Intra-SecondaryFrequency Indicator | MP |  | Boolean | TRUE means the inter frequency neighbour cell has a secondary frequency which is the same as the current working frequency of UE. | REL-7 |
| >>>SNPL Monitor Set Indicator | OP |  | Bit string(5) | Each bit indicates whether the inter frequency neighbour cell has a secondary frequency which is the same as the frequency of a specific E-DCH carrier of UE. Bit0 indicate the E-DCH carrier with lowest UARFCN, and Bit1 indicate the E-DCH carrier with the second lowest UARFCN, ... ... Bit n indicate the E-DCH carrier with the n+1 lowest UARFCN. the Primary E-DCH carrier shall not be considered in the bitmap. | REL-10 |
| Reduced measurement performance frequency list | CV-Measure | 1 to <maxRMPfrequencies> |  |  | REL-12 |
| >Reduced measurement performance | MP |  | BOOLEAN | TRUE indicates that the frequency is configured for reduced measurement performance [19]. FALSE indicates that the frequency is configured for normal measurement performance [19]. The ith entry in the Reduced measurement performance frequency list corresponds to the ith frequency signalled in the "New inter-frequency cells" list. | REL-12 |
| Cells for measurement | CV-BCHopt | 1 to <maxCellMeas> |  |  |  |
|  |  | 1 to <maxCellMeas-ext> |  |  | REL-12 |
| >Inter-frequency cell id | MP |  | Integer(0 .. <maxCellMeas>-1) |  |  |
|  |  |  | Integer(0 .. <maxCellMeas-ext>-1) |  | REL-12 |
| CSG Inter-frequency cell info | CV-BCHopt |  | 10.3.7.120 |  | REL-9 |
| Inter-frequency SI Acquisition | CV-BCHopt |  | 10.3.7.124 |  | REL-9 |
| Multiple Frequency Info List FDD | CV- MCM_opt | 1 to <maxNumFDDFreqs > |  | Each entry in the list corresponds to a UARFCN listed in the "New inter-frequency cells" IE (common to all those cells sharing the same "Frequency info" value). See NOTE 1. | REL-10 |
| >Multiple Frequency bands Indicator List FDD | OP | 1 to< maxMultipleFrequencyBandsFDD > |  | Indicates, per neighbour UARFCN, a list of frequency bands which the cells belong to. Absence of this IE indicates that there are no Multiple Bands for the particular UARFCN. | REL-10 |
| >>CHOICE frequency bands indicator | MP |  |  |  | REL-10 |
| >>>Frequency band indicator |  |  | Frequency band indicator 10.3.6.35b |  | REL-10 |
| >>>Frequency band indicator 2 |  |  | Frequency band indicator 2 10.3.6.35c |  | REL-10 |
| >>>Frequency band indicator 3 |  |  | Frequency band indicator 3 10.3.6.35ca |  | REL-10 |

NOTE 1:	The UTRAN should signal the "Frequency info" IE only once in "New Inter-frequency cells" for each UARFCN i.e. for the first Inter-frequency neighbour cell using that UARFCN. For each subsequent Inter-frequency neighbour cell having the same UARFCN, the "Frequency info" should not be included, but rather the Mandatory Default (MD) value will be used. See Annex G.2.

NOTE 2:	Not applicable when sent in SYSTEM INFORMATION.

| Condition | Explanation |
| --- | --- |
| BCHopt | This IE is not needed when sent in SYSTEM INFORMATION. Otherwise, the IE is Optional |
| MCM_opt | This IE is OPTIONAL when sent in SYSTEM INFORMATION. Otherwise, the IE is not needed. |
| Measure | The IE is optional present in System Information Block type 11, 11bis, 11ter, 12 and MEASUREMENT CONTROL message. Otherwise the IE is not needed. |

#### 10.3.7.14	Inter-frequency event identity

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-frequency event identity | MP |  | Enumerated(2a, 2b, 2c, 2d, 2e, 2f, 2g) | One spare value is needed. |

#### 10.3.7.15	Inter-frequency measured results list

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-frequency measurement results | OP | 1 to <maxFreq> |  |  |
| >Frequency info | MD |  | Frequency info 10.3.6.36 | Default value is the value of the previous "frequency info" in the list. NOTE: The first occurrence is then MP. |
| >UTRA carrier RSSI | OP |  | Integer(0..76) | According to UTRA_carrier_RSSI_LEV in [19] and [20]. Fifty-one spare values are needed. |
| >Inter-frequency cell measurement results | OP | 1 to <maxCellMeas> |  | Only cells for which all reporting quantities are available should be included. |
| >>Cell measured results | MP |  | Cell measured results 10.3.7.3 |  |

#### 10.3.7.16	Inter-frequency measurement

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Inter-frequency measurement objects list | MP |  | Inter-frequency cell info list 10.3.7.13 |  |  |
| Inter-frequency measurement quantity | OP |  | Inter-frequency measurement quantity 10.3.7.18 |  |  |
| Inter-frequency reporting quantity | OP |  | Inter-frequency reporting quantity 10.3.7.21 |  |  |
| Reporting cell status | CV-reporting |  | Reporting cell status 10.3.7.61 |  |  |
| Measurement validity | OP |  | Measurement validity 10.3.7.51 |  |  |
| Inter-frequency set update | OP |  | Inter-frequency set update 10.3.7.22 |  |  |
| Adjacent frequency index | OP |  | Integer (0..<maxCellMeas-1>) | Index to an element in the IE “New inter-frequency cell” containing a cell on the adjacent frequency; index = 0 corresponds to the first element in the IE | REL-8 |
|  |  |  | Integer (0..<maxCellMeas-ext-1>) |  | REL-12 |
| Inter-band frequency index | OP |  | Integer (0..<maxCellMeas-1>) | Index to an element in the IE “New inter-frequency cell” containing a cell on an inter-band frequency; index = 0 corresponds to the first element in the IE | REL-9 |
|  |  |  | Integer (0..<maxCellMeas-ext-1>) |  | REL-12 |
| Frequency index list for enhanced measurement | OP | 1..<maxFreqMeasWithoutCM> |  |  | REL-10 |
| >Frequency index for enhanced measurement | MP |  | Integer (0..<maxCellMeas-1>) | Index to an element in the IE “New inter-frequency cell”; index = 0 corresponds to the first element in the IE | REL-10 |
|  |  |  | Integer (0..<maxCellMeas-ext-1>) |  | REL-12 |
| CHOICE report criteria | MP |  |  |  |  |
| >Intra-frequency measurement reporting criteria |  |  | Intra-frequency measurement reporting criteria 10.3.7.39 |  |  |
| >Inter-frequency measurement reporting criteria |  |  | Inter-frequency measurement reporting criteria 10.3.7.19 |  |  |
| >Periodical reporting criteria |  |  | Periodical reporting criteria 10.3.7.53 |  |  |
| >No reporting |  |  |  | (no data) Chosen when this measurement only is used as additional measurement to another measurement |  |

| Condition | Explanation |
| --- | --- |
| reporting | This IE is optional if the CHOICE "report criteria" is equal to "periodical reporting criteria" or "No reporting", otherwise the IE is not needed |

#### 10.3.7.17	Inter-frequency measurement event results

This IE contains the measurement event results that are reported to UTRAN for inter-frequency measurements.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Inter-frequency event identity | MP |  | Inter-frequency event identity 10.3.7.14 |  |  |
| Inter-frequency cells | OP | 1 to <maxFreq> |  |  |  |
| >Frequency info | MP |  | Frequency info 10.3.6.36 |  |  |
| >Non frequency related measurement event results | MP |  | Cell measurement event results 10.3.7.4 |  |  |
| Detected Set Trigger | OP |  | Enumerated(True) | Indicates whether or not inclusion of detected set cells triggered the event. | REL-10 |

#### 10.3.7.18	Inter-frequency measurement quantity

The quantity the UE shall measure in case of inter-frequency measurement. It also includes the filtering of the measurements.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE reporting criteria | MP |  |  |  |
| >Intra-frequency reporting criteria |  |  |  |  |
| >>Intra-frequency measurement quantity | MP |  | Intra-frequency measurement quantity 10.3.7.38 |  |
| >Inter-frequency reporting criteria |  |  |  |  |
| >>Filter coefficient | MP |  | Filter coefficient 10.3.7.9 |  |
| >>CHOICE mode | MP |  |  |  |
| >>>FDD |  |  |  |  |
| >>>>Measurement quantity for frequency quality estimate | MP |  | Enumerated( CPICH Ec/N0, CPICH RSCP) |  |
| >>>TDD |  |  |  |  |
| >>>>Measurement quantity for frequency quality estimate | MP |  | Enumerated( Primary CCPCH RSCP) |  |

#### 10.3.7.19	Inter-frequency measurement reporting criteria

The triggering of the event-triggered reporting for an inter-frequency measurements. All events concerning inter-frequency measurements are labelled 2x where x is a,b,c, ...

Event 2a: Change of best frequency.

Event 2b: The estimated quality of the currently used frequency is below a certain threshold and the estimated quality of a non-used frequency is above a certain threshold.

Event 2c: The estimated quality of a non-used frequency is above a certain threshold.

Event 2d: The estimated quality of the currently used frequency is below a certain threshold.

Event 2e: The estimated quality of a non-used frequency is below a certain threshold.

Event 2f: The estimated quality of the currently used frequency is above a certain threshold.

Event 2g: Change of best cell on a configured secondary downlink frequency.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Parameters required for each event | OP | 1 to <maxMeasEvent> |  |  |  |
| >Inter-frequency event identity | MP |  | Inter-frequency event identity 10.3.7.14 |  |  |
| >Threshold used frequency | CV–clause 0 |  | Integer(-115..0) | Ranges used depend on measurement quantity. CPICH Ec/No -24..0dB CPICH/Primary CCPCH RSCP -115..-25dBm |  |
|  |  |  | Integer (-120..0) | Ranges used depend on measurement quantity. CPICH Ec/No -24..0dB CPICH/Primary CCPCH RSCP -120..-25dBm | REL-6 |
| >DeltaThreshold used frequency | CV-clause 3 |  | Integer(-5..-1) | If present, the actual value of Threshold used frequency = Threshold used frequency + DeltaThreshold used frequency | REL-5 |
|  |  |  |  | Not used in Release 6 and later | REL-6 |
| >W used frequency | CV–clause 2 |  | Real(0, 0.1..2.0 by step of 0.1) |  |  |
| >Hysteresis | MP |  | Real(0, 0.5..14.5 by step of 0.5) | In event 2a, 2b, 2c, 2d, 2e, 2f, 2g |  |
| >Time to trigger | MP |  | Time to trigger 10.3.7.64 | Indicates the period of time during which the event condition has to be satisfied, before sending a Measurement Report. Time in ms. |  |
| >Reporting cell status | OP |  | Reporting cell status 10.3.7.61 |  |  |
| >Parameters required for each non-used frequency | OP | 1 to <maxFreq> |  | In this release, the first listed threshold, W and triggering condition parameter shall apply to all non-used frequencies. |  |
| >>Threshold non used frequency | CV–clause 1 |  | Integer(-115..0) | Ranges used depend on measurement quantity. CPICH Ec/No -24..0dB CPICH/Primary CCPCH RSCP -115..-25dBm. This IE is not needed if the IE "Inter-frequency event identity" is set to 2a. However, it is specified to be mandatory to align with the ASN.1. |  |
|  |  |  | Integer (-120..0) | Ranges used depend on measurement quantity. CPICH Ec/No -24..0dB CPICH/Primary CCPCH RSCP -120..-25dBm This IE is not needed if the IE "Inter-frequency event identity" is set to 2a. | REL-6 |
| >> DeltaThreshold non used frequency | CV-clause 4 |  | Integer(-5..-1) | If present, the actual value of Threshold non used frequency = Threshold non used frequency + DeltaThreshold non used frequency | REL-5 |
|  |  |  |  | Not used in Release 6 and later | REL-6 |
| >>W non-used frequency | CV-clause 1 |  | Real(0, 0.1..2.0 by step of 0.1) |  |  |
| >>Triggering Condition non-used frequency detected cells | CV-clause 4 |  | Enumerated(true) | If present the UE shall include detected set cells in the event evaluation | REL-10 |
| >Use CIO | CV-clause 6 |  | Enumerated (TRUE) | If present the UE shall use cell individual offset for event evaluation. | REL-12 |
| Blacklisted cells in non-used frequency | CV-clause 5 | 1 to <maxFreq> |  | If present the UE should exclude indicated cells in the event evaluation when non-used frequency detected cells is enabled. | REL-12 |
| >Frequency info | MP |  | Frequency info 10.3.6.36 |  | REL-12 |
| >Cells to be excluded in non-used frequency detected cells | MP | 1 to <maxExcludedDetectedSetCells> |  |  | REL-12 |
| >>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-12 |

| Condition | Explanation |
| --- | --- |
| Clause 0 | This IE is mandatory present if the IE "Inter frequency event identity" is set to 2b, 2d, or 2f, otherwise the IE is not needed. |
| Clause 1 | This IE is mandatory present if the IE "Inter frequency event identity" is set to 2a, 2b, 2c or 2e, otherwise the IE is not needed |
| Clause 2 | This IE is mandatory present if the IE "Inter-frequency event identity" is set to 2a, 2b, 2d or 2f, otherwise the IE is not needed. |
| Clause 3 | This IE is optional if the IE "Inter frequency event identity" is set to 2b, 2d, or 2f. Otherwise the IE is not needed. Note that in order to align with the ASN.1, this IE is always included when the IE “DeltaThreshold non used frequency“ is present, but the value shall be ignored if the "Inter-frequency event identity" is not set to 2b, 2d or 2f. |
| Clause 4 | This IE is optional if the IE "Inter frequency event identity" is set to 2a, 2b, 2c or 2e. Otherwise the IE is not needed. |
| Clause 5 | This IE is optional if the IE "Triggering Condition non-used frequency detected cells" is present. Otherwise the IE is not needed. |
| Clause 6 | This IE is optional if the IE "Inter frequency event identity" is set to 2g. Otherwise the IE is not needed. |

#### 10.3.7.20	Inter-frequency measurement system information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Inter-frequency cell info list | OP |  | Inter-frequency cell info list 10.3.7.13 |  |  |
| Inter-frequency RACH reporting information | OP |  | Inter-frequency RACH reporting information 10.3.7.20a |  | REL-6 |

#### 10.3.7.20a	Inter-frequency RACH reporting information

Contains the reporting configuration information for an inter-frequency measurement report, which is sent on the RACH.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  | REL-6 |
| >FDD |  |  |  |  | REL-6 |
| >> Inter-frequency RACH reporting quantity | MP |  | Enumerated(CPICH Ec/N0, CPICH RSCP) |  | REL-6 |
| >TDD |  |  |  |  | REL-6 |
| >>Reporting quantity list | MP | 1 to 2 |  |  | REL-6 |
| >>> Inter-frequency RACH reporting quantity | MP |  | Enumerated(Primary CCPCH RSCP) |  | REL-6 |
| Inter-frequency RACH reporting threshold | MP |  | Integer(-115..0) | Ranges used depend on measurement quantity. CPICH Ec/No -24..0dB CPICH/Primary CCPCH RSCP -115..-25dBm. | REL-6 |
| Maximum number of inter-frequency RACH reporting cells | MP |  | Integer(1..8) | Indicates the total number for all non- used frequencies | REL-6 |

#### 10.3.7.21	Inter-frequency reporting quantity

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UTRA Carrier RSSI | MP |  | Boolean | TRUE means report is requested. |
| Frequency quality estimate | MP |  | Boolean | TRUE means that report is requested. This parameter is not used in this release and should be set to FALSE. It shall be ignored by the UE. |
| Non frequency related cell reporting quantities | MP |  | Cell reporting quantities 10.3.7.5 |  |

#### 10.3.7.22	Inter-frequency SET UPDATE

NOTE 1:	Only for FDD.

This information makes it possible to use events defined for Intra-frequency measurement within the same non-used frequency for Inter-frequency measurement reporting criteria.

| Information Element/group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE autonomous update mode | MP |  | Enumerated (On with no reporting) | Required for backwards compatibility with a UE not supporting this revision of the standard |

#### 10.3.7.23	Inter-RAT cell info list

Contains the information for the list of measurement objects for an inter-RAT measurement.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Inter-RAT cell removal | MP |  |  |  |  |
| >Remove all inter-RAT cells |  |  |  | (no data) |  |
| >Remove some inter-RAT cells |  |  |  |  |  |
| >>Removed inter-RAT cells | MP | 1 to <maxCellMeas> |  |  |  |
| >>>Inter-RAT cell id | MP |  | Integer(0 .. <maxCellMeas> - 1) |  |  |
| >Remove no inter-RAT cells |  |  |  | (no data) |  |
| New inter-RAT cells | MP | 1 to <maxCellMeas> |  | Although this IE is not always required, need is MP to align with ASN.1 |  |
|  | OP |  |  |  | REL-4 |
| >Inter-RAT cell id | OP |  | Integer(0 .. <maxCellMeas> - 1) |  |  |
| >CHOICE Radio Access Technology | MP |  |  |  |  |
| >>GSM |  |  |  |  |  |
| >>>Cell individual offset | MP |  | Integer (-50..50 ) | In dB Used to offset measured quantity value |  |
| >>>Cell selection and re-selection info | OP |  | Cell selection and re-selection info for SIB11/12 10.3.2.4 | See subclause 8.6.7.3 |  |
| >>>BSIC | MP |  | BSIC 10.3.8.2 |  |  |
| >>>Band indicator | MP |  | Enumerated (DCS 1800 band used, PCS 1900 band used) | Indicates how to interpret the BCCH ARFCN |  |
| >>>BCCH ARFCN | MP |  | Integer (0..1023) | [45] |  |
| >>IS-2000 |  |  |  |  |  |
| >>>System specific measurement info | MP |  | enumerated (frequency, timeslot, colour code, output power, PN offset) | For IS-2000, use fields from TIA/EIA/IS-2000.5, subclause 3. 7.3.3.2.27, Candidate Frequency Neighbour List Message |  |
| >>None |  |  | (no data) | This value has been introduced to handle the case when IE "New inter-RAT cells" is not required |  |
| Cells for measurement | OP | 1 to <maxCellMeas> |  |  |  |
| >Inter-RAT cell id | MP |  | Integer(0 .. <maxCellMeas>-1) |  |  |
| Inter-RAT cell info indication | CV-Message |  | Integer (0..3) | NOTE 1 and 2 | REL-5 |

| Condition | Explanation |
| --- | --- |
| Message | The IE is optionally present in the MEASUREMENT CONTROL and in the SRNS RELOCATION INFO messages, otherwise the IE is not needed. |

NOTE 1:	The UTRAN may choose not to use the "Inter-RAT cell info indication" value "0" in the MEASUREMENT CONTROL message, to distinguish that case from those cases where the UE receives the IE "Inter-RAT cell info list" in SIB11 or SIB12.

NOTE 2:	In case of an SRNS relocation, if the UE has been sent the "Inter-RAT cell info indication" in the MEASUREMENT CONTROL message and the IE "Inter-RAT cell info list" is included in the SRNS RELOCATION INFO sent from the source RNC to the target RNC, the "Inter-RAT cell info indication" should be included in the IE "Inter-RAT cell info list".

NOTE 3:	If the UTRAN configures a UE with a "Inter-RAT cell info list" containing a set of GSM cells where one or more cell(s) belong to DCS 1800 band and one or more cell(s) belong to PCS 1900 band, the UE behaviour is unspecified.

#### 10.3.7.24	Inter-RAT event identity

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-RAT event identity | MP |  | Enumerated (3a, 3b, 3c, 3d) |  |

#### 10.3.7.25	Inter-RAT info

Inter-RAT info defines the target system for redirected cell selection.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Inter-RAT info | MP |  | Enumerated (GSM |  |  |
|  |  |  | , E-UTRA) |  | REL-8 |
| GSM target cell info | CV-GSM |  | GSM target cell info 10.3.8.4g |  | REL-6 |
| E-UTRA target info | CV-E-UTRA |  | E-UTRA target info 10.3.8.4L |  | REL-8 |

| Condition | Explanation |
| --- | --- |
| GSM | This IE is optionally present if the IE "Inter-RAT info" is set to ‘GSM’ and not needed otherwise. |
| E-UTRA | This IE is mandatory present if the IE "Inter-RAT info" is set to ‘E-UTRA’ and not needed otherwise. |

#### 10.3.7.26	Inter-RAT measured results list

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-RAT measurement results | OP | 1 to <maxOtherRAT-16> |  |  |
| >CHOICE system | MP |  |  | One spare value is needed. |
| >>GSM |  |  |  |  |
| >>>Measured GSM cells | MP | 1 to <maxReportedGSMCells> |  |  |
| >>>>GSM carrier RSSI | OP |  | bit string(6) | RXLEV is mapped to a value between 0 and 63, [46]. When mapping the RXLEV value to the RSSI bit string, the first/leftmost bit of the bit string contains the most significant bit. |
| >>>>CHOICE BSIC | MP |  |  |  |
| >>>>>Verified BSIC |  |  |  |  |
| >>>>>>inter-RAT cell id | MP |  | Integer(0..<maxCellMeas>-1) |  |
| >>>>>Non verified BSIC |  |  |  |  |
| >>>>>>BCCH ARFCN | MP |  | Integer (0..1023) | [45] |

#### 10.3.7.27	Inter-RAT measurement

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Inter-RAT measurement objects | OP |  |  |  | REL-8 |
| >Inter-RAT cell info list | MP |  | Inter-RAT cell info list 10.3.7.23 |  |  |
| >E-UTRA frequency list | MP |  | E-UTRA frequency list 10.3.7.6b |  | REL-8 |
| Inter-RAT measurement quantity | OP |  | Inter-RAT measurement quantity 10.3.7.29 |  |  |
| Inter-RAT reporting quantity | OP |  | Inter-RAT reporting quantity 10.3.7.32 |  |  |
| Reporting cell status | CV-reporting |  | Reporting cell status 10.3.7.61 |  |  |
| CHOICE report criteria | MP |  |  |  |  |
| >Inter-RAT measurement reporting criteria |  |  | Inter-RAT measurement reporting criteria 10.3.7.30 |  |  |
| >Periodical reporting criteria |  |  | Periodical reporting criteria 10.3.7.53 |  |  |
| >No reporting |  |  |  | (no data) Chosen when this measurement only is used as additional measurement to another measurement |  |
| Idle Interval Information | OP |  | Idle Interval Information 10.3.7.12a | This IE is used for TDD only. | REL-8 |

| Condition | Explanation |
| --- | --- |
| reporting | This IE is optional if the CHOICE "report criteria" is equal to "periodical reporting criteria" or "No reporting", otherwise the IE is not needed |

#### 10.3.7.28	Inter-RAT measurement event results

This IE contains the measurement event results that are reported to UTRAN for inter-RAT measurements.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-RAT event identity | MP |  | Inter-RAT event identity 10.3.7.24 |  |
| Cells to report | MP | 1 to <maxCellMeas> |  |  |
| >CHOICE BSIC | MP |  |  |  |
| >>Verified BSIC |  |  |  |  |
| >>>inter-RAT cell id | MP |  | Integer(0..<maxCellMeas>-1) |  |
| >>Non verified BSIC |  |  |  |  |
| >>>BCCH ARFCN | MP |  | Integer (0..1023) | [45] |

#### 10.3.7.29	Inter-RAT measurement quantity

The quantity the UE shall measure in case of inter-RAT measurement. It also includes the filtering of the measurements.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Measurement quantity for UTRAN quality estimate | OP |  | Intra-frequency measurement quantity 10.3.7.38 |  |  |
| CHOICE system | MP |  |  |  |  |
| >GSM |  |  |  |  |  |
| >>Measurement quantity | MP |  | Enumerated (GSM Carrier RSSI) |  |  |
| >>Filter coefficient | MP |  | Filter coefficient 10.3.7.9 |  |  |
| >>BSIC verification required | MP |  | Enumerated (required, not required) |  |  |
| >IS2000 |  |  |  |  |  |
| >>TADD Ec/I0 | MP |  | Integer (0..63) | Admission criteria for neighbours, see subclause 2.6.6.2.6 of TIA/EIA/IS-2000.5 |  |
| >>TCOMP Ec/I0 | MP |  | Integer (0..15) | Admission criteria for neighbours, see subclause 2.6.6.2.5.2 of TIA/EIA/IS-2000.5 |  |
| >>SOFT SLOPE | OP |  | Integer (0..63) | Admission criteria for neighbours, see subclause 2.6.6.2.3 and 2.6.6.2.5.2 of TIA/EIA/IS-2000.5 |  |
| >>ADD_INTERCEPT | OP |  | Integer (0..63) | Admission criteria for neighbours, see subclause 2.6.6.2.5.2 of TIA/EIA/IS-2000.5 |  |
| >E-UTRA |  |  |  |  | REL-8 |
| >>Measurement quantity | MP |  | Enumerated (RSRP, RSRQ) | This IE indicates the quantity that the UE uses to determine the event trigger. | REL-8 |
| >>Filter coefficient | MP |  | Filter coefficient 10.3.7.9 |  | REL-8 |

#### 10.3.7.30	Inter-RAT measurement reporting criteria

The triggering of the event-triggered reporting for an inter-RAT measurement. All events concerning inter-RAT measurements are labelled 3x where x is a,b,c, ...

Event 3a: The estimated quality of the currently used UTRAN frequency is below a certain threshold and the estimated quality of the other system is above a certain threshold.

Event 3b: The estimated quality of other system is below a certain threshold.

Event 3c: The estimated quality of other system is above a certain threshold.

Event 3d: Change of best cell in other system.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Parameters required for each event | OP | 1 to <maxMeasEvent> |  |  |  |
| >Inter-RAT event identity | MP |  | Inter-RAT event identity 10.3.7.24 |  |  |
| >Threshold own system | CV–clause 0 |  | Integer (-115..0) |  |  |
| >W | CV–clause 0 |  | Real(0, 0.1..2.0 by step of 0.1) | In event 3a |  |
| >Threshold other system | CV–clause 1 |  | Integer (-115..0) | In event 3a, 3b, 3c If the other system is EUTRA, the range used depends on: if measurement quantity is RSRQ, range should be (-39..-6), the actual value = Threshold other system/2 [dB], if measurement quantity is RSRP, range should be (-115..-19), the actual value = Threshold other system - 25 [dBm] |  |
|  |  |  | Integer (-115..6) | In event 3a, 3b, 3c If the other system is EUTRA, the range used depends on: if measurement quantity is RSRQ, range should be (-68..6), the actual value = Threshold other system/2 [dB], if measurement quantity is RSRP, range should be (-115..-19), the actual value = Threshold other system - 25 [dBm] | REL-12 |
| >Hysteresis | MP |  | Real(0..7.5 by step of 0.5) |  |  |
| >Time to trigger | MP |  | Time to trigger 10.3.7.64 | Indicates the period of time during which the event condition has to be satisfied, before sending a Measurement Report. |  |
| >Reporting cell status | OP |  | Reporting cell status 10.3.7.61 |  |  |

| Condition | Explanation |
| --- | --- |
| Clause 0 | The IE is mandatory present if the IE "Inter-RAT event identity" is set to "3a", otherwise the IE is not needed |
| Clause 1 | The IE is mandatory present if the IE "Inter-RAT event identity" is set to 3a, 3b or 3c, otherwise the IE is not needed |

#### 10.3.7.31	Inter-RAT measurement system information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-RAT cell info list | OP |  | Inter-RAT cell info list 10.3.7.23 |  |

#### 10.3.7.32	Inter-RAT reporting quantity

For all boolean types TRUE means inclusion in the report is requested.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UTRAN estimated quality | MP |  | Boolean | This parameter is not used in this release and should be set to FALSE. |  |
| CHOICE system | MP |  |  |  |  |
| >GSM |  |  |  |  |  |
| >>GSM Carrier RSSI Reporting indicator | MP |  | Boolean |  |  |
| >E-UTRA |  |  |  |  | REL-8 |
| >>Reporting quantity | MP |  | Enumerated (‘measurement quantity’, ‘both’) | This IE indicates the quantity that the UE reports; if this IE is set to ‘measurement quantity’ the UE reports either RSRP or RSRQ which is indicated in the “measurement quantity” (see subclause 10.3.7.29); if this IE is set to ‘both’, the UE reports both RSRP and RSRQ. | REL-8 |

#### 10.3.7.33	Intra-frequency cell info list

Contains the information for the list of measurement objects for an intra-frequency measurement.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |  | Version |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Intra-frequency cell removal | OP |  |  | Absence of this IE is equivalent to choice "Remove no intra-frequency cells". |  |  |
| >Remove all intra-frequency cells |  |  |  | (no data) |  |  |
| >Remove some intra-frequency cells |  |  |  |  |  |  |
| >>Removed intra-frequency cells | MP | 1 to <maxCellMeas> |  |  |  |  |
| >>>Intra-frequency cell id | MP |  | Integer(0 .. <maxCellMeas> - 1) |  |  |  |
| >Remove no intra-frequency cells |  |  |  | (no data) |  |  |
| New intra-frequency cells | OP | 1 to <maxCellMeas> |  | This information element must be present when "Intra-frequency cell info list" is included in the system information |  |  |
| >Intra-frequency cell id | OP |  | Integer(0 .. <maxCellMeas> - 1) |  |  |  |
| >Cell info | MP |  | Cell info 10.3.7.2 | This IE must be included for the serving cell when the IE "Intra frequency cell info list" is included in System Information Block type 11. |  |  |
| >CHOICE mode specific info | MP |  |  |  | REL-10 |  |
| >>No information |  |  |  | No data for FDD, TDD 3.84 Mcps or TDD 7.68 Mcps | REL-10 |  |
| >>TDD 1.28 Mcps |  |  |  |  | REL-10 |  |
| >>> SNPL Monitor Set Indicator | OP |  | Bit string(5) | Each bit indicates whether the intra frequency neighbour cell has a secondary frequency which is the same as the frequency of a specific E-DCH carrier of UE. Bit0 indicate the E-DCH carrier with lowest UARFCN, Bit1 indicate the E-DCH carrier with the second lowest ... Bit n indicate the E-DCH carrier with the n+1 lowest UARFCN, the E-DCH carrier on the Primary E-DCH carrier shall not be considered in the bitmap. | REL-10 |  |
| Cells for measurement | CV-BCHopt | 1 to <maxCellMeas> |  |  |  |  |
| >Intra-frequency cell id | MP |  | Integer(0 .. <maxCellMeas>-1) |  |  |  |
| CSG Intrafrequency cell info | CV-BCHopt |  | 10.3.7.121 |  | REL-9 |  |
| Intra-frequency SI Acquisition | CV-BCHopt |  | 10.3.7.125 |  | REL-9 |  |

| Condition | Explanation |
| --- | --- |
| BCHopt | This IE is not needed when sent in SYSTEM INFORMATION. Otherwise, the IE is Optional |

#### 10.3.7.34	Intra-frequency event identity

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Intra-frequency event identity | MP |  | Enumerated (1a,1b,1c,1d,1e,1f,1g,1h,1i, | Seven spare values are needed. |  |
|  |  |  | 1j) | Six spare values are needed. | REL-6 |

#### 10.3.7.35	Intra-frequency measured results list

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Intra-frequency measurement results | OP | 1 to < maxCellMeas> |  |  |  |
| >Cell measured results | MP |  | Cell measured results 10.3.7.3 | Only cells for which all reporting quantities are available should be included. |  |

#### 10.3.7.36	Intra-frequency measurement

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Intra-frequency measurement objects list | OP |  | Intra-frequency cell info list 10.3.7.33 |  |  |
| Intra-frequency measurement objects list on secondary UL frequency | OP |  | Intra-frequency cell info list on secondary UL frequency 10.3.7.116 |  | REL-9 |
| Intra-frequency measurement quantity | OP |  | Intra-frequency measurement quantity 10.3.7.38 |  |  |
| Intra-frequency reporting quantity | OP |  | Intra-frequency reporting quantity 10.3.7.41 |  |  |
| Reporting cell status | CV-reporting |  | Reporting cell status 10.3.7.61 |  |  |
| Measurement validity | OP |  | Measurement validity 10.3.7.51 |  |  |
| CHOICE report criteria | OP |  |  |  |  |
|  | MP |  |  | NOTE 1 | REL-6 |
|  | OP |  |  |  | REL-7 |
| >Intra-frequency measurement reporting criteria |  |  | Intra-frequency measurement reporting criteria 10.3.7.39 |  |  |
| >Periodical reporting criteria |  |  | Periodical reporting criteria 10.3.7.53 |  |  |
| >No reporting |  |  |  | (no data) Chosen when this measurement only is used as additional measurement to another measurement |  |
| Periodical reporting criteria for the secondary UL Frequency | OP |  | Periodical reporting criteria on secondary UL frequency 10.3.7.136 |  | REL-10 |

| Condition | Explanation |
| --- | --- |
| reporting | This IE is optional if the CHOICE "report criteria" is equal to "periodical reporting criteria" or "No reporting", otherwise the IE is not needed |

NOTE 1:	The IE "report criteria" is optional in all releases except Rel-6. In Rel-6 the IE "report criteria" is mandatory present in order to align the tabular format with the Rel-6 ASN.1.

#### 10.3.7.37	Intra-frequency measurement event results

This IE contains the measurement event results that are reported to UTRAN for intra-frequency measurements.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Intra-frequency event identity | MP |  | Intra-frequency event identity 10.3.7.34 |  |  |
| Cell measurement event results | MP |  | Cell measurement event results 10.3.7.4 |  |  |

#### 10.3.7.38	Intra-frequency measurement quantity

The quantity the UE shall measure in case of intra-frequency measurement. It also includes the filtering of the measurements.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Filter coefficient | MP |  | Filter coefficient 10.3.7.9 |  |
| CHOICE mode | MP |  |  |  |
| >FDD |  |  |  |  |
| >>Measurement quantity | MP |  | Enumerated(CPICH Ec/N0, CPICH RSCP, Pathloss) |  |
| >TDD |  |  |  |  |
| >>Measurement quantity list | MP | 1 to 4 |  |  |
| >>>Measurement quantity | MP |  | Enumerated(Primary CCPCH RSCP, Pathloss, Timeslot ISCP) |  |

#### 10.3.7.39	Intra-frequency measurement reporting criteria

The triggering of the event-triggered reporting for an intra-frequency measurement. All events concerning intra-frequency measurements are labelled 1x where x is a, b, c….

Event 1a: A Primary CPICH enters the Reporting Range (FDD only).

Event 1b: A Primary CPICH leaves the Reporting Range (FDD only).

Event 1c: A Non-active Primary CPICH becomes better than an active Primary CPICH (FDD only).

Event 1d: Change of best cell (FDD only).

Event 1e: A Primary CPICH becomes better than an absolute threshold (FDD only).

Event 1f: A Primary CPICH becomes worse than an absolute threshold (FDD only).

Event 1g: Change of best cell in TDD.

Event 1h: Timeslot ISCP below a certain threshold (TDD only).

Event 1i: Timeslot ISCP above a certain threshold (TDD only).

Event 1j: A Non-active E-DCH but active DCH Primary CPICH becomes better than an active E-DCH Primary CPICH (FDD only).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Parameters required for each event | OP | 1 to <maxMeasEvent> |  |  |  |
| >Intra-frequency event identity | MP |  | Intra-frequency event identity 10.3.7.34 |  |  |
| >Triggering condition 1 | CV–clause 0 |  | Enumerated(Active set cells, Monitored set cells, Active set cells and monitored set cells) | Indicates which cells can trigger the event |  |
| >Triggering condition 2 | CV–clause 6 |  | Enumerated(Active set cells, Monitored set cells, Active set cells and monitored set cells, Detected set cells, Detected set cells and monitored set cells) | Indicates which cells can trigger the event |  |
|  | CV-clause 10 |  |  |  | REL-5 |
| >Reporting Range Constant | CV–clause 2 |  | Real(0..14.5 by step of 0.5) | In dB. In event 1a,1b. |  |
| >Cells forbidden to affect Reporting range | CV–clause 1 | 1 to < maxCellMeas> |  | In event 1a,1b |  |
| >>CHOICE mode | MP |  |  |  |  |
| >>>FDD |  |  |  |  |  |
| >>>>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  |  |
| >>>TDD |  |  |  |  |  |
| >>>>Primary CCPCH info | MP |  | Primary CCPCH info 10.3.6.57 |  |  |
| >W | CV–clause 2 |  | Real(0.0..2.0 by step of 0.1) |  |  |
| >Hysteresis | MP |  | Real(0..7.5 by step of 0.5) | In dB. |  |
| >Threshold used frequency | CV-clause 3 |  | Integer (-115..165) | Range used depend on measurement quantity. CPICH RSCP -115..-25 dBm CPICH Ec/No -24..0 dB Pathloss 30..165dB ISCP -115..-25 dBm |  |
|  |  |  | Integer (-120..165) | Range used depend on measurement quantity. CPICH RSCP -120..-25 dBm CPICH Ec/No -24..0 dB Pathloss 30..165dB ISCP -120..-25 dBm | REL-6 |
| >DeltaThreshold used frequency | CV-clause 8 |  | Integer(-5..-1) | If present, the actual value of Threshold used frequency = Threshold used frequency + DeltaThreshold used frequency | REL-5 |
|  |  |  |  | Not used in Release 6 and later | REL-6 |
| >Reporting deactivation threshold | CV–clause 4 |  | Integer(0, 1, 2, 3, 4, 5, 6, 7) | In event 1a Indicates the maximum number of cells allowed in the active set in order for event 1a to occur. 0 means not applicable . |  |
| >Replacement activation threshold | CV-clause 5 |  | Integer(0, 1, 2, 3, 4, 5, 6, 7) | In event 1c Indicates the minimum number of cells allowed in the active set in order for event 1c to occur. 0 means not applicable |  |
|  |  |  |  | In event 1j Indicates the minimum number of cells allowed in the E-DCH active set in order for event 1j to occur. For event 1j values 5, 6 and 7 are not applicable. 0 means not applicable | REL-6 |
| >Time to trigger | MP |  | Time to trigger 10.3.7.64 | Indicates the period of time during which the event condition has to be satisfied, before sending a Measurement Report. Time in ms |  |
| >Amount of reporting | CV–clause 7 |  | Integer(1, 2, 4, 8, 16, 32, 64, Infinity) | In case the IE "Intra-frequency reporting criteria" is included in the IE "Inter-frequency measurement", this IE is not needed. |  |
| >Reporting interval | CV–clause 7 |  | Integer(0, 250, 500, 1000, 2000, 4000, 8000, 16000) | Indicates the interval of periodical reporting when such reporting is triggered by an event. Interval in milliseconds. 0 means no periodical reporting. In case the IE "Intra-frequency reporting criteria" is included in the IE "Inter-frequency measurement", this IE is not needed. |  |
| >Reporting cell status | OP |  | Reporting cell status 10.3.7.61 |  |  |
| >Periodical reporting information-1b | CV–clause 9 |  | Periodical reporting info-1b 10.3.7.53aa | In case the IE "Intra-frequency reporting criteria" is included in the IE "Inter-frequency measurement", this IE is not needed. | REL-5 |
| >Use CIO | CV-clause 10 |  | Boolean | TRUE indicates that the cell individual offset shall be used for event evaluation | REL-5 |
| Reporting criteria for secondary UL Frequency | CV-Clause 11 |  | Intra-frequency measurement reporting criteria on secondary UL frequency 10.3.7.119 |  | REL-9 |
| Cells to be excluded in detected set cells | CV-Clause 12 | 1 to < maxExcludedDetectedSetCells > |  | If present the UE should exclude indicated cells in the event evaluation when intra-frequency detected set is enabled. | REL-11 |
| >Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  |  |

| Condition | Explanation |
| --- | --- |
| Clause 0 | The IE is mandatory present if the IE "Intra-frequency event identity" is set to "1b" or "1f", otherwise the IE is not needed. |
| Clause 1 | The IE is optional if the IE "Intra-frequency event identity" is set to "1a" or "1b", otherwise the IE is not needed. |
| Clause 2 | The IE is mandatory present if the IE "Intra-frequency event identity" is set to "1a" or "1b", otherwise the IE is not needed. |
| Clause 3 | The IE is mandatory present if the IE "Intra-frequency event identity" is set to , "1e", "1f", "1h" or "1i", otherwise the IE is not needed. |
| Clause 4 | The IE is mandatory present if the IE "Intra-frequency event identity" is set to "1a", otherwise the IE is not needed. |
| Clause 5 | The IE is mandatory present if the IE "Intra-frequency event identity" is set to "1c" or 1j , otherwise the IE is not needed. |
| Clause 6 | The IE is mandatory present if the IE "Intra-frequency event identity" is set to "1a" or "1e", otherwise the IE is not needed. |
| Clause 7 | The IE is mandatory present if the IE "Intra-frequency event identity" is set to "1a", "1c" or 1j, otherwise the IE is not needed. |
| Clause 8 | The IE is optional if the IE "Intra-frequency event identity" is set to "1e", "1f", "1h" or "1i". Otherwise the IE is not needed. |
| Clause 9 | The IE is optional if the IE "Intra-frequency event identity" is set to "1b", otherwise the IE is not needed. |
| Clause 10 | The IE is optional if the IE "Intra-frequency event identity" is set to "1d", otherwise the IE is not needed. |
| Clause 11 | The IE is not needed when sent in SYSTEM INFORMATION or when IE "Intra-frequency measurement reporting criteria" is included in the IE "Inter-frequency measurement", otherwise the IE is optional. |
| Clause 12 | The IE is optional if the intra-frequency detected set is configured to trigger the measurement event, otherwise the IE is not needed. |

#### 10.3.7.40	Intra-frequency measurement system information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Intra-frequency measurement identity | MD |  | Measurement identity 10.3.7.48 | The intra-frequency measurement identity has default value 1. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |
| Intra-frequency cell info list | OP |  | Intra-frequency cell info list 10.3.7.33 |  |
| Intra-frequency measurement quantity | OP |  | Intra-frequency measurement quantity 10.3.7.38 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |
| Intra-frequency reporting quantity for RACH Reporting | OP |  | Intra-frequency reporting quantity for RACH Reporting 10.3.7.42 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |
| Maximum number of reported cells on RACH | OP |  | Maximum number of reported cells on RACH 10.3.7.43 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |
| Reporting information for state CELL_DCH | OP |  | Reporting information for state CELL_DCH 10.3.7.62 | Note 1 If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |

NOTE 1:	The reporting of intra-frequency measurements is activated when state CELL_DCH is entered.

#### 10.3.7.41	Intra-frequency reporting quantity

Contains the reporting quantity information for an intra-frequency measurement.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reporting quantities for active set cells | MP |  | Cell reporting quantities 10.3.7.5 |  |
| Reporting quantities for monitored set cells | MP |  | Cell reporting quantities 10.3.7.5 |  |  |
| Reporting quantities for detected set cells | OP |  | Cell reporting quantities 10.3.7.5 |  |  |

#### 10.3.7.42	Intra-frequency reporting quantity for RACH reporting

Contains the reporting quantity information for an intra-frequency measurement report, which is sent on the RACH.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SFN-SFN observed time difference reporting indicator | MP |  | Enumerated(No report, type 1, type 2) |  |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Reporting quantity | MP |  | Enumerated(CPICH Ec/N0, CPICH RSCP, Pathloss, No report) |  |  |
| >>Reporting quantity 2 | CV-Current_Cell |  | Enumerated(CPICH Ec/N0 and CPICH RSCP) | If this IE is present, the UE shall ignore the IE "Reporting quantity". One spare value. | REL-12 |
| >TDD |  |  |  |  |  |
| >>Reporting quantity list | MP | 1 to 2 |  |  |  |
| >>>Reporting quantity | MP |  | Enumerated(Timeslot ISCP, Primary CCPCH RSCP, No report) |  |  |

| Condition | Explanation |
| --- | --- |
| Current_Cell | This IE is optional for the current cell and is not needed otherwise. |

#### 10.3.7.42a	Logged ANR Configuration Info

Parameters used to configure logged UTRAN ANR measurements.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Logging Duration | MP |  | Enumerated (10 min, 30min, 1 hour，spare1, spare2) | Timer for logging duration. The timer is activated at the moment of configuration. When the timer expires the logging is stopped and the configuration is cleared. | REL-10 |
| Intra-UTRA ANR | OP |  |  | Included if intra-UTRA ANR shall be performed | REL-10 |
| >CHOICE Absolute Threshold | MP |  |  |  | REL-10 |
| >>RSCP for ANR |  |  |  |  |  |
| >>>RSCP | MD |  | Integer (-120..-25) | In dBm. Default value is -100. If present, it indicates CPICH RSCP for FDD, Primary CCPCH RSCP for TDD. | REL-10 |
| >>Ec/N0 for ANR |  |  |  |  |  |
| >>>Ec/N0 | MD |  | Integer (-24..0) | In dB. Default value is -10. For FDD only | REL-10 |
| >Logging Relative Threshold | OP |  | Integer (-4..16) | If present, it applies to RSCP by step of 2 and Ec/N0 by step of 1. | REL-10 |
| Inter-RAT ANR for E-UTRA Indicator | OP |  | Enumerated (TRUE) | True indicates the UE needs to perform inter-RAT ANR to E-UTRA. | REL-10 |
| Inter-RAT ANR for GSM Indicator | OP |  | Enumerated (TRUE) | True indicates the UE needs to perform inter-RAT ANR to GSM | REL-10 |

#### 10.3.7.42b	Logged ANR Report Info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Logged ANR Report Info List |  | 1…MaxNumANRLoggedItems |  |  | REL-10 |
| >Serving PLMN Identity | MP |  | PLMN identity 10.3.1.11 |  | REL-10 |
| >Serving Cell | MP |  | Cell ID 10.3.2.2 |  | REL-10 |
| >CHOICE logged cell info | MP |  |  |  | REL-10 |
| >>UTRAN |  |  |  |  |  |
| >>>PLMN Identity | MP |  | PLMN identity 10.3.1.11 |  | REL-10 |
| >>>Cell Identity | MP |  | Cell ID 10.3.2.2 |  | REL-10 |
| >>>UARFCN | MP |  | Integer(0..16383) |  | REL-10 |
| >>>Cell parameter ID | CV-Tdd |  | Cell parameter Id 10.3.6.9 |  | REL-10 |
| >>>Primary Scrambling Code | CV-Fdd |  | Primary CPICH info 10.3.6.60 |  | REL-10 |
| >>E-UTRA |  |  |  |  |  |
| >>>PLMN Identity | MP |  | PLMN identity 10.3.1.11 |  | REL-10 |
| >>>Tracking Area Code | MP |  | Bit string (16) | Setting specified in [67]. The first/leftmost bit of the bit string contains the most significant bit of the Tracking Area Code. | REL-10 |
| >>>Cell Identity | MP |  | Bit string (28) | Formatted and coded according to [67]. The first/leftmost bit of the bit string contains the most significant bit of the Cell Identity. | REL-10 |
| >>>EARFCN | MP |  | Integer (0..65535) | EARFCN of the downlink carrier frequency [64]. If the IE indicates a value of 65535, then the EARFCN for this instance should be read from the corresponding instance of IE "EARFCN extension" in the Logged ANR Report Info extension List. | REL-10 |
| >>>Physical Cell identity | MP |  | Integer (0..503) |  | REL-10 |
| >>GSM |  |  |  |  |  |
| >>>PLMN Identity | MP |  | PLMN identity 10.3.1.11 |  | REL-10 |
| >>>LAC | MP |  | Bit string(16) | The first/leftmost bit of the bit string contains the most significant bit of the LAC.. | REL-10 |
| >>>Cell Identity | MP |  | Bit string (16) | Formatted and coded according to [11]. The first/leftmost bit of the bit string contains the most significant bit of the Cell Identity. | REL-10 |
| >>>BCCH ARFCN | MP |  | Integer (0..1023) | [45] | REL-10 |
| >>>Band Indicator | MP |  | Enumerated (DCS 1800 band used, PCS 1900 band used) | Indicates how to interpret the BCCH ARFCN | REL-10 |
| >>>BSIC | MP |  | BSIC 10.3.8.2 |  | REL-10 |
| Logged ANR Report Info extension List | OP | 1 to <MaxNumANRLoggedItems> |  |  | REL-11 |
| >>>EARFCN extension | MP |  | Integer (65536..262143) | EARFCN of the downlink carrier frequency [64] | REL-11 |

| Condition | Explanation |
| --- | --- |
| Fdd | This IE is mandatory present for FDD, otherwise it is not needed. |
| Tdd | This IE is mandatory present for TDD, otherwise it is not needed. |

#### 10.3.7.43	Maximum number of reported cells on RACH

Contains the maximum number of intra-frequency cells to be reported on RACH.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum number of reported cells | MP |  | Enumerated (no report, current cell, current cell + best neighbour, current cell+2 best neighbours, …, current cell+6 best neighbours) |  |

#### 10.3.7.43a	MBMS preferred frequency information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS preferred frequency list | MP | 1 to < maxMBMS-Freq> |  |  | REL-6 |
| >MBMS preferred frequency | OP |  | Integer(1 .. <maxMBMS-Freq>) | Absence of this information element means the currently used frequency. Value n corresponds with the nth frequency included in the IE New inter-frequency cells that is specified within SIB 11 | REL-6 |
| >CHOICE Layer convergence information | MP |  |  |  | REL-6 |
| >>No HCS |  |  |  |  | REL-6 |
| >>>Qoffmbms | MP |  | Enumerated (4, 8, 12, 16, 20, 30, 40, infinity) | The offset in decibel [dB] added to cells on this MBMS preferred frequency | REL-6 |
| >>HCS |  |  |  |  | REL-6 |
| >>>HCS_OFFmbms | MP |  | Integer (0..7) | Offset added to the normal HCS priority level of cells on this MBMS preferred frequency | REL-6 |
| >MBMS PL Service Restriction Information | OP |  | Enumerated (TRUE) | Included if some service restrictions apply for this preferred frequency e.g. congestion | REL-6 |

#### 10.3.7.43b	MBSFN inter frequency neighbour list

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBSFN inter frequency neighbour list | MP | 1 to < maxMBSFNclusters> |  |  | REL-7 |
| >MBSFN frequency | MP |  | Frequency info 10.3.6.36 |  | REL-7 |
| >IMB indication | OP |  | Enumerated (TRUE) | If present: 3.84 Mcps TDD MBSFN IMB is applied on this frequency. | REL-8 |
| >CHOICE MBSFN services notification | MP |  |  |  | REL-7 |
| >>MBSFN services notified |  |  |  | (no data) | REL-7 |
| >>MBSFN services not notified |  |  |  |  | REL-7 |
| >>>All MBSFN services notified | OP |  | Enumerated (TRUE) |  | REL-7 |

#### 10.3.7.44	Measured results

Contains the measured results of the quantity indicated optionally by Reporting Quantity in Measurement Control. "Measured results" can be used for both event trigger mode and periodical reporting mode. For intra-frequency and inter-frequency measurements the list shall be in the order of the value of the measurement quantity (the first cell shall be the best cell). The "best" FDD cell has the largest value when the measurement quantity is "Ec/No" or "RSCP". On the other hand, the "best" cell has the smallest value when the measurement quantity is "Pathloss". The "best" TDD cell has the largest value when measurement quantity is "Primary CCPCH RSCP". For intra-frequency measurements, the ordering shall be applied to all cells included in the IE "Measured results". For inter-frequency measurements, the ordering shall be applied to all cells on the same frequency included in the IE "Measured results". For other measurements, the order of reported measurement objects is not specified.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Measurement | MP |  |  | One spare value is needed. |
| >Intra-frequency measured results list |  |  | Intra-frequency measured results list 10.3.7.35 |  |
| >Inter-frequency measured results list |  |  | Inter-frequency measured results list 10.3.7.15 |  |
| >Inter-RAT measured results list |  |  | Inter-RAT measured results list 10.3.7.26 |  |
| >Traffic volume measured results list |  |  | Traffic volume measured results list 10.3.7.67 |  |
| >Quality measured results list |  |  | Quality measured results list 10.3.7.55 |  |
| >UE Internal measured results |  |  | UE Internal measured results 10.3.7.76 |  |
| >UE positioning measured results |  |  | UE positioning measured results 10.3.7.99 |  |

#### 10.3.7.45	Measured results on RACH

For measurements on used frequency, this IE contains the measured results on RACH of the quantity indicated by Reporting quantity in the IE "Intra-frequency reporting quantity for RACH Reporting" in system information broadcast on BCH. The list, measurement results for monitored cells (not including the current cell) shall be in the order of the value of the measurement quantity as indicated by Reporting Quantity in the IE "Intra-frequency reporting quantity for RACH Reporting" (the first cell shall be the best cell).

For measurements on non-used frequencies, this IE includes the best cell on each non- used frequency, in order of decreasing quality and up to a maximum number as specified by the IE "Maximum number of inter-frequency RACH reporting cells". If the UE supports Increased UE carrier monitoring UTRA, then:

1>	if all the inter-frequency cells in the variable CELL_INFO_LIST belong to the legacy cell ID range (0.. maxCellMeas-1), then the best cell on each non-used frequency are included into the Rel-6 version of this IE;

1>	if all the inter-frequency cells in the variable CELL_INFO_LIST belong to both legacy cell ID range (0.. maxCellMeas-1) and extended cell ID range (maxCellMeas.. maxCellMeas-ext-1), then:

2>	the Rel-12 version of this IE includes the best cell on each non-used frequency constructed from the list of cells belonging to both legacy and extended cell ID range; and

2>	the Rel-6 version of this IE includes the best cell on each non-used frequency constructed from the list of cells belonging only to the legacy cell ID range.

The "best" FDD cell has the largest value when the measurement quantity is "Ec/No" or "RSCP". On the other hand, the "best" cell has the smallest value when the measurement quantity is "Pathloss". The "best" TDD cell has the largest value when measurement quantity is "Primary CCPCH RSCP".

For measurements on E-UTRA frequencies, this IE includes a bit string, of which the i-th bit should be set to 1 only if the quantity of the i-th E-UTRA frequency configured for measurement and reporting on RACH in the IE "E-UTRA frequency list indicator" received in System Information Block Type 19 exceeds the threshold specified by

1>	if the "E-UTRA frequency RACH reporting quantity" is set to RSRP:

2>	the threshold specified by the IE "E-UTRA frequency RACH reporting threshold".

1>	if the "E-UTRA frequency RACH reporting quantity" is set to RSRQ:

2>	the threshold specified by the IE "E-UTRA frequency RACH reporting threshold" minus the offset "EUTRA-RSRQ-offsetWB" and "EUTRA-RSRQ-OnAllSymbolsOffset", if stored in the variable PRIORITY_INFO_LIST for that frequency.

If the threshold is not exceeded, set the corresponding bit to 0.

| Information Element/group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Measurement result for current cell | MP |  |  |  |  |
| >CHOICE mode | MP |  |  |  |  |
| >>FDD |  |  |  |  |  |
| >>>CHOICE measurement quantity | MP |  |  | One spare value is needed. |  |
| >>>>CPICH Ec/N0 |  |  | Integer(0..49) | In dB. According to CPICH_Ec/No in [19]. Fourteen spare values are needed. |  |
| >>>>CPICH RSCP |  |  | Integer(0..91) | In dBm. According to CPICH_RSCP_LEV in [19]. Thirty-six spare values are needed. |  |
| >>>>Pathloss |  |  | Integer(46..158) | In dB. Fifteen spare values are needed. |  |
| >>>>CPICH Ec/N0 and CPICH RSCP |  |  |  |  | REL-12 |
| >>>>>CPICH Ec/N0 | MP |  | Integer(0..49) | In dB. According to CPICH_Ec/No in [19]. Fourteen spare values are needed. | REL-12 |
| >>>>>CPICH RSCP | MP |  | Integer(0..91) | In dBm. According to CPICH_RSCP_LEV in [19]. Thirty-six spare values are needed. | REL-12 |
| >>TDD |  |  |  |  |  |
| >>>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>>Timeslot List | OP | 1 to 14 |  |  |  |
| >>>>>>Timeslot ISCP | MP |  | Timeslot ISCP info 10.3.7.65 | The UE shall report the Timeslot ISCP in the same order as indicated in the cell info |  |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Timeslot List | OP | 1 to 14 |  |  | REL-7 |
| >>>>>Timeslot ISCP | MP |  | Timeslot ISCP info 10.3.7.65 | The UE shall report the Timeslot ISCP in the same order as indicated in the cell info | REL-7 |
| >>>>1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>>>>Timeslot List | OP | 1 to 6 |  |  | REL-4 |
| >>>>>>Timeslot ISCP | MP |  | Timeslot ISCP info 10.3.7.65 | The UE shall report the Timeslot ISCP in the same order as indicated in the cell info | REL-4 |
| >>>Primary CCPCH RSCP | OP |  | Primary CCPCH RSCP info 10.3.7.54 |  |  |
| Measurement results for monitored cells on used frequency | OP | 1 to 8 |  |  |  |
| >SFN-SFN observed time difference | OP |  | SFN-SFN observed time difference 10.3.7.63 |  |  |
| >CHOICE mode | MP |  |  |  |  |
| >>FDD |  |  |  |  |  |
| >>>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  |  |
| >>>CHOICE measurement quantity | OP |  |  | One spare value is needed. |  |
| >>>>CPICH Ec/N0 |  |  | Integer(0..49) | In dB. According to CPICH_Ec/No in [19]. Fourteen spare values are needed. |  |
| >>>>CPICH RSCP |  |  | Integer(0..91) | In dBm. According to CPICH_RSCP_LEV in [19]. Thirty-six spare values are needed. |  |
| >>>>Pathloss |  |  | Integer(46..158) | In dB. Fifteen spare values are needed. |  |
| >>TDD |  |  |  |  |  |
| >>>Cell parameters Id | MP |  | Cell parameters Id 10.3.6.9 |  |  |
| >>>Primary CCPCH RSCP | MP |  | Primary CCPCH RSCP info 10.3.7.54 |  |  |
| Measurement results for monitored cells on non-used frequencies | OP |  |  |  | REL-6 |
| >Inter-frequency cell indication- SIB11 | MP |  | Integer (0..1) |  | REL-6 |
| >Inter-frequency cell indication- SIB12 | MP |  | Integer (0..1) |  | REL-6 |
| >Inter-frequency cell list | MP | 1 to <maxFreq> |  |  | REL-6 |
| >>Inter-frequency cell id | MP |  | Integer(0 .. <maxCellMeas>-1) |  | REL-6 |
|  |  |  | Integer(0 .. <maxCellMeas-ext>-1) |  | REL-12 |
| Measurement results for E-UTRA frequencies | OP |  |  |  | REL-11 |
| > E-UTRA frequency indication- SIB19 | MP |  | Integer (0..1) |  | REL-11 |
| > E-UTRA frequency indicator | MP |  | Bit string(maxNumEUTRAFreqs_FACH) | Bit 0 is the first/leftmost bit of the bit string. Each bit indicates whether the radio quality of an E-UTRA frequency is above the reporting threshold or not, where a one value indicates above and a zero value not. Bit n corresponds to the n-th E-UTRA frequency configured for measurement and reporting on RACH in the IE " E-UTRA frequency list indicator" received in SIB19. | REL-11 |
|  |  |  | Bit string(maxNumEUTRAFreqs_FACH-ext) | Bit 0 is the first/leftmost bit of the bit string. Each bit indicates whether the radio quality of an E-UTRA frequency is above the reporting threshold or not, where a one value indicates above and a zero value not. Bit n corresponds to the n-th E-UTRA frequency configured for measurement and reporting on RACH in the IE "E-UTRA frequency list indicator" received in SIB19. | REL-12 |

NOTE:	Monitored cells consist of neighbouring cells.

#### 10.3.7.45a	Measured results on RACH FDD

NOTE 1:	Only for FDD.

For measurements on used frequency, this IE contains the measured results on RACH of the quantity indicated by Reporting quantity in the IE "Intra-frequency reporting quantity for RACH Reporting" in system information broadcast on BCH. The list, measurement results for monitored cells (not including the current cell) shall be in the order of the value of the measurement quantity as indicated by Reporting Quantity in the IE "Intra-frequency reporting quantity for RACH Reporting" (the first cell shall be the best cell).

For measurements on non-used frequencies, this IE includes the best cell on each non- used frequency, in order of decreasing quality and up to a maximum number as specified by the IE "Maximum number of inter-frequency RACH reporting cells".

The "best" FDD cell has the largest value when the measurement quantity is "Ec/No" or "RSCP". On the other hand, the "best" cell has the smallest value when the measurement quantity is "Pathloss".

For measurements on E-UTRA frequencies, this IE includes a bit string, of which the i-th bit should be set to 1 only if the quantity of the i-th E-UTRA frequency configured for measurement and reporting on RACH in the IE "E-UTRA frequency list indicator" received in System Information Block Type 19 exceeds the threshold specified by

1>	if the "E-UTRA frequency RACH reporting quantity" is set to RSRP

2>	the threshold specified by the IE "E-UTRA frequency RACH reporting threshold".

1>	if the "E-UTRA frequency RACH reporting quantity" is set to RSRQ

2>	the threshold specified by the IE "E-UTRA frequency RACH reporting threshold" minus the offset "EUTRA-RSRQ-offsetWB" and "EUTRA-RSRQ-OnAllSymbolsOffset", if stored in the variable PRIORITY_INFO_LIST for that frequency.

If the threshold is not exceeded, set the corresponding bit to 0.

| Information Element/group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Measurement result for current cell | MP |  |  |  |  |
| >CHOICE measurement quantity | MP |  |  | One spare value is needed. |  |
| >>CPICH Ec/N0 |  |  | Integer(0..49) | In dB. According to CPICH_Ec/No in [19]. Fourteen spare values are needed. |  |
| >>CPICH RSCP |  |  | Integer(0..91) | In dBm. According to CPICH_RSCP_LEV in [19]. Thirty-six spare values are needed. |  |
| >>Pathloss |  |  | Integer(46..158) | In dB. Fifteen spare values are needed. |  |
| >>CPICH Ec/N0 and CPICH RSCP |  |  |  |  | REL-12 |
| >>>CPICH Ec/N0 | MP |  | Integer(0..49) | In dB. According to CPICH_Ec/No in [19]. Fourteen spare values are needed. | REL-12 |
| >>>CPICH RSCP | MP |  | Integer(0..91) | In dBm. According to CPICH_RSCP_LEV in [19]. Thirty-six spare values are needed. | REL-12 |
| Measurement results for monitored cells on used frequency | OP | 1 to 8 |  |  |  |
| >SFN-SFN observed time difference | OP |  | SFN-SFN observed time difference 10.3.7.63 |  |  |
| >Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  |  |
| >CHOICE measurement quantity | OP |  |  | One spare value is needed. |  |
| >>CPICH Ec/N0 |  |  | Integer(0..49) | In dB. According to CPICH_Ec/No in [19]. Fourteen spare values are needed. |  |
| >>CPICH RSCP |  |  | Integer(0..91) | In dBm. According to CPICH_RSCP_LEV in [19]. Thirty-six spare values are needed. |  |
| >>Pathloss |  |  | Integer(46..158) | In dB. Fifteen spare values are needed. |  |
| Measurement results for monitored cells on non-used frequencies | OP |  |  |  |  |
| >Inter-frequency cell indication- SIB11 | MP |  | Integer (0..1) |  |  |
| >Inter-frequency cell indication- SIB12 | MP |  | Integer (0..1) |  |  |
| >Inter-frequency cell list | MP | 1 to <maxFreq> |  |  |  |
| >>Inter-frequency cell id | MP |  | Integer(0 .. <maxCellMeas>-1) |  |  |
| Measurement results for E-UTRA frequencies | OP |  |  |  |  |
| > E-UTRA frequency indication- SIB19 | MP |  | Integer (0..1) |  |  |
| > E-UTRA frequency indicator | MP |  | Bit string(maxNumEUTRAFreqs_FACH) | Bit 0 is the first/leftmost bit of the bit string. Each bit indicates whether the radio quality of an E-UTRA frequency is above the reporting threshold or not, where a one value indicates above and a zero value not. Bit n corresponds to the n-th E-UTRA frequency configured for measurement and reporting on RACH in the IE "E-UTRA frequency list indicator" received in SIB19. |  |

NOTE:	Monitored cells consist of neighbouring cells.

#### 10.3.7.46	Measurement Command

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Measurement command | MP |  | Enumerated(Setup, Modify, Release) |  |  |
|  |  |  | ReleaseAll, ReleaseSome) |  | REL-12 |

#### 10.3.7.46a	Measurement release enhancement

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Choice mode | MP |  |  |  | REL-12 |
| >Release all measurements |  |  | (no data) |  | REL-12 |
| >Release some measurements |  |  | Release some measurements 10.3.7.46b |  | REL-12 |

#### 10.3.7.46b	Release some measurements

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Released measurements | MP | 1 to < maxMeasIdentity> |  |  | REL-12 |
| >Measurement Identity | MP |  | Measurement identity 10.3.7.48 |  | REL-12 |

#### 10.3.7.47	Measurement control system information

| Information element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Use of HCS | MP |  | Enumerated (Not used, used) | Indicates if the serving cell belongs to a HCS structure If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received. |
| Cell selection and reselection quality measure | MP |  | Enumerated (CPICH Ec/N0, CPICH RSCP) | Choice of measurement (CPICH Ec/N0 or CPICH RSCP) to use as quality measure Q. NOTE: For TDD,each choice of this parameter represents PCCPCH RSCP. If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaves as if this IE would not have been received |
| Intra-frequency measurement system information | OP |  | Intra-frequency measurement system information 10.3.7.40 |  |
| Inter-frequency measurement system information | OP |  | Inter-frequency measurement system information 10.3.7.20 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |
| Inter-RAT measurement system information | OP |  | Inter-RAT measurement system information 10.3.7.31 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |
| Traffic volume measurement system information | OP |  | Traffic volume measurement system information 10.3.7.73 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3 the UE behaviour upon reception of this IE is unspecified. |

| Condition | Explanation |
| --- | --- |
| MBSFN | This IE is mandatory present in a cell operating in MBSFN mode. |

#### 10.3.7.47a	Measurement control system information extension

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Intra-frequency cell info list | OP |  |  |  | REL-6 |
| >New intra-frequency cells | MP | 1 to <maxCellMeas> |  |  | REL-6 |
| >>Intra-frequency cell id | OP |  | Integer(0 .. <maxCellMeas> - 1) |  | REL-6 |
| >>Cell info | MP |  | Cell info 10.3.7.2 |  | REL-6 |
| Inter-frequency cell info list | OP |  |  |  | REL-6 |
| >New inter-frequency cells | MP | 1 to <maxCellMeas> |  |  | REL-6 |
| >>Frequency Info | MD |  | Frequency info 10.3.6.36 | Default value is the value of the previous "frequency info" in the list. NOTE: The first occurrence is then MP. It is always ensured by the UTRAN that more than one entry for the same physical frequency is not configured regardless of the UARFCN used to indicate this. | REL-6 |
| >>Inter-frequency cell id | OP |  | Integer(0 .. <maxCellMeas> - 1) |  | REL-6 |
| >>Cell info | MP |  | Cell info 10.3.7.2 |  | REL-6 |
| Reduced measurement performance frequency list | OP | 1 to <maxRMPfrequencies> |  |  | REL-12 |
| >Reduced measurement performance | MP |  | BOOLEAN | TRUE indicates that the frequency is configured for reduced measurement performance [19]. FALSE indicates that the frequency is configured for normal measurement performance [19]. The ith entry in the Reduced measurement performance frequency list corresponds to the ith frequency signalled in the "New inter-frequency cells" list. | REL-12 |
| Inter-RAT cell info list | OP |  |  |  | REL-6 |
| >>New inter-RAT cells | MP | 1 to <maxCellMeas> |  |  | REL-6 |
| >>Inter-RAT cell id | OP |  | Integer(0 .. <maxCellMeas> - 1) |  | REL-6 |
| >>CHOICE Radio Access Technology | MP |  |  |  | REL-6 |
| >>>GSM |  |  |  |  | REL-6 |
| >>>>Cell individual offset | MP |  | Integer (-50..50 ) | In dB Used to offset measured quantity value | REL-6 |
| >>>>Cell selection and re-selection info | OP |  | Cell selection and re-selection info for SIB11/12 10.3.2.4 | See subclause 8.6.7.3 | REL-6 |
| >>>>BSIC | MP |  | BSIC 10.3.8.2 |  | REL-6 |
| >>>>Band indicator | MP |  | Enumerated (DCS 1800 band used, PCS 1900 band used) | Indicates how to interpret the BCCH ARFCN | REL-6 |
| >>>>BCCH ARFCN | MP |  | Integer (0..1023) | [45] | REL-6 |
| >>>IS-2000 |  |  |  |  | REL-6 |
| >>>>System specific measurement info | MP |  | enumerated (frequency, timeslot, colour code, output power, PN offset) | For IS-2000, use fields from TIA/EIA/IS-2000.5, subclause 3. 7.3.3.2.27, Candidate Frequency Neighbour List Message | REL-6 |
| >>>None |  |  | (no data) | This value has been introduced to handle the case when IE "New inter-RAT cells" is not required | REL-6 |
| Multiple Frequency Info List FDD | CV- MCM_opt | 1 to <maxNumFDDFreqs > |  | Each entry in the list corresponds to a UARFCN listed in the "New inter-frequency cells" IE (common to all those cells sharing the same "Frequency info" value). | REL-10 |
| >Multiple Frequency bands Indicator List FDD | OP | 1 to < maxMultipleFrequencyBandsFDD > |  | Indicates, per neighbour UARFCN, a list of frequency bands which the cells belong to. Absence of this IE indicates that there are no Multiple Bands for the particular UARFCN. | REL-10 |
| >>CHOICE frequency bands indicator | MP |  |  |  | REL-10 |
| >>>Frequency band indicator |  |  | Frequency band indicator 10.3.6.35b |  | REL-10 |
| >>>Frequency band indicator 2 |  |  | Frequency band indicator 2 10.3.6.35c |  | REL-10 |
| >>>Frequency band indicator 3 |  |  | Frequency band indicator 3 10.3.6.35ca |  | REL-10 |

| Condition | Explanation |
| --- | --- |
| MCM_opt | This IE is OPTIONAL when sent in SYSTEM INFORMATION. Otherwise, the IE is not needed. |

#### 10.3.7.47b	Inter-frequency cell info list extension

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Inter-frequency cell info list | OP |  |  |  | REL-12 |
| >New inter-frequency cells | MP | 1 to <maxCellMeas-ext2> |  |  | REL-12 |
| >>Frequency Info | MD |  | Frequency info 10.3.6.36 | Default value is the value of the previous "frequency info" in the list. NOTE: The first occurrence is then MP. It is always ensured by the UTRAN that more than one entry for the same physical frequency is not configured regardless of the UARFCN used to indicate this. | REL-12 |
| >>Inter-frequency cell id | OP |  | Integer(0 .. <maxCellMeas-ext> - 1) |  | REL-12 |
| >>Cell info | MP |  | Cell info 10.3.7.2 |  | REL-12 |
| Reduced measurement performance frequency list | OP | 1 to <maxRMPfrequencies> |  |  | REL-12 |
| >Reduced measurement performance | MP |  | BOOLEAN | TRUE indicates that the frequency is configured for reduced measurement performance [19]. FALSE indicates that the frequency is configured for normal measurement performance [19]. The ith entry in the Reduced measurement performance frequency list corresponds to the ith frequency signalled in the "New inter-frequency cells" list. | REL-12 |
| Multiple Frequency Info List FDD | CV- MCM_opt | 1 to <maxNumFDDFreqs > |  | Each entry in the list corresponds to a UARFCN listed in the "New inter-frequency cells" IE (common to all those cells sharing the same "Frequency info" value). | REL-12 |
| >Multiple Frequency bands Indicator List FDD | OP | 1 to< maxMultipleFrequencyBandsFDD > |  | Indicates, per neighbour UARFCN, a list of frequency bands which the cells belong to. Absence of this IE indicates that there are no Multiple Bands for the particular UARFCN. | REL-12 |
| >>CHOICE frequency bands indicator | MP |  |  |  | REL-12 |
| >>>Frequency band indicator |  |  | Frequency band indicator 10.3.6.35b |  | REL-12 |
| >>>Frequency band indicator 2 |  |  | Frequency band indicator 2 10.3.6.35c |  | REL-12 |
| >>>Frequency band indicator 3 |  |  | Frequency band indicator 3 10.3.6.35ca |  | REL-12 |

| Condition | Explanation |
| --- | --- |
| MCM_opt | This IE is OPTIONAL when sent in SYSTEM INFORMATION. Otherwise, the IE is not needed. |

#### 10.3.7.48	Measurement Identity

A reference number that is used by the UTRAN at modification and release of the measurement, and by the UE in the measurement report.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Measurement identity | MP |  | Integer(1..16 |  |  |
|  |  |  | 17..32) |  | REL-9 |

NOTE:	The specific handling of the Measurement Identity for UEs supporting the measurement identity extension is captured in ASN.1

#### 10.3.7.49	Measurement reporting mode

Contains the type of Measurement Report transfer mode and the indication of periodical/event trigger.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Measurement Report Transfer Mode | MP |  | enumerated (Acknowledged mode RLC, Unacknowledged mode RLC) |  |
| Periodical Reporting / Event Trigger Reporting Mode | MP |  | Enumerated (Periodical reporting, Event trigger) |  |

#### 10.3.7.50	Measurement Type

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Measurement Type | MP |  | Enumerated(Intra-frequency, Inter-frequency, Inter-RAT, Traffic volume, Quality, UE internal, UE positioning, CSG proximity) |  |

#### 10.3.7.51	Measurement validity

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE state | MP |  | Enumerated(CELL_DCH, all states except CELL_DCH, all states) |  |  |

#### 10.3.7.52	Void

#### 10.3.7.53	Periodical reporting criteria

Contains the periodical reporting criteria information. It is necessary only in the periodical reporting mode.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Amount of reporting | MD |  | Integer(1, 2, 4, 8, 16, 32, 64, Infinity) | The default value is infinity. |  |
| Reporting interval | MP |  | Integer(250, 500, 1000, 2000, 3000, 4000, 6000, 8000, 12000, 16000, 20000, 24000, 28000, 32000, 64000) | Indicates the interval of periodical report. Interval in milliseconds. If this IE is configured for SI acquisition, then it specifies the maximum time the UE can take for SI acquisition. |  |

#### 10.3.7.53aa	Periodical reporting info-1b

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Amount of reporting | MP |  | Integer(1, 2, 4, 8, 16, 32, 64, Infinity) |  | REL-5 |
| Reporting interval | MP |  | Integer(0, 250, 500, 1000, 2000, 4000, 8000, 16000) | Indicates the interval of periodical reporting when such reporting is triggered by an event. Interval in milliseconds. 0 means no periodical reporting. | REL-5 |

#### 10.3.7.53a	PLMN identities of neighbour cells

This IE contains the PLMN identities of neighbour cells.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PLMNs of intra-frequency cells list | OP | 1 to <maxCellMeas> |  | NOTE |  |
| >PLMN identity | OP |  | PLMN identity 10.3.1.11 | Action when not present is specified in subclause 8.1.1.6.18. |  |
| PLMNs of inter-frequency cells list | OP | 1 to <maxCellMeas> |  | NOTE |  |
| >PLMN identity | OP |  | PLMN identity 10.3.1.11 | Action when not present is specified in subclause 8.1.1.6.18. |  |
| PLMNs of inter-RAT cells list | OP | 1 to <maxCellMeas> |  |  |  |
| >PLMN identity | OP |  | PLMN identity 10.3.1.11 | Action when not present is specified in subclause 8.1.1.6.18. |  |
| Multiple PLMNs of intra-frequency cells list | OP | 1 to <maxCellMeas> |  |  | REL-8 |
| >Multiple PLMN list | OP | 1 to 6 |  | Action when not present is specified in subclause 8.1.1.6.18. | REL-8 |
| >> PLMN identity with Optional MCC | MP |  | PLMN identity with Optional MCC 10.3.1.11a |  | REL-8 |
| Multiple PLMNs of inter-frequency cells list | OP | 1 to <maxCellMeas> |  |  | REL-8 |
| >Multiple PLMN list | OP | 1 to 6 |  | Action when not present is specified in subclause 8.1.1.6.18. | REL-8 |
| >> PLMN identity with Optional MCC | MP |  | PLMN identity with Optional MCC 10.3.1.11a |  | REL-8 |

NOTE:	The IE "PLMNs of intra-frequency cells list" and/or the IE "PLMNs of inter-frequency cells list" can be used when each entry in those lists is either empty or includes a single PLMN identity. If multiple PLMNs need to be indicated in a non-empty entry, the network uses the IE "Multiple PLMNs of intra-frequency cells list" and/or the IE "Multiple PLMNs of inter-frequency cells list" instead.

#### 10.3.7.53b	PLMN identities of neighbour cells for SIB 11ter

This IE contains the PLMN identities of inter-frequency neighbour cells.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PLMNs of inter-frequency cells list | OP | 1 to <maxCellMeas-ext> |  | In this version of the specification, the maximum value is 48. NOTE | REL-12 |
| >PLMN identity | OP |  | PLMN identity 10.3.1.11 | Action when not present is specified in subclause 8.1.1.6.18. | REL-12 |
| Multiple PLMNs of inter-frequency cells list | OP | 1 to <maxCellMeas-ext> |  | In this version of the specification, the maximum value is 48. | REL-12 |
| >Multiple PLMN list | OP | 1 to 6 |  | Action when not present is specified in subclause 8.1.1.6.18. | REL-12 |
| >> PLMN identity with Optional MCC | MP |  | PLMN identity with Optional MCC 10.3.1.11a |  | REL-12 |

NOTE:	The IE "PLMNs of inter-frequency cells list" can be used when each entry in those lists is either empty or includes a single PLMN identity. If multiple PLMNs need to be indicated in a non-empty entry, the network uses the IE "Multiple PLMNs of inter-frequency cells list" instead.

#### 10.3.7.54	Primary CCPCH RSCP info

NOTE:	Only for TDD

| Information Element/Group name | Need | Multi | IE type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Primary CCPCH RSCP | MP |  | Integer(0..91) | According to P-CCPCH_RSCP_LEV in [19] and [20]. Thirty-six spare values are needed. |  |
| DeltaPrimary CCPCH RSCP | CV-Rel5 |  | Integer(-5..-1) | If present, the actual value of Primary CCPCH RSCP = Primary CCPCH RSCP+ DeltaPrimary CCPCH RSCP | REL-5 |

| Condition | Explanation |
| --- | --- |
| Rel5 | This IE is mandatory if the value of Primary CCPCH RSCP is below 0. Othewise the IE is not needed. |

#### 10.3.7.54a	Qhcs

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Qhcs | MP |  | Integer( 0..99) | Qhcs, mapped from CPICH Ec/No (FDD), see [4] [dB] 0: -24 1: -23.5 2: -23 3: -22.5 ... 45: -1.5 46: -1 47: -0.5 48: 0 49: (spare) ... 98: (spare) 99: (spare) |  |
|  |  |  |  | Qhcs, mapped from CPICH RSCP (FDD), see [4] [dBm] 0: -115 1: -114 2: -113 : 88: -27 89: -26 90: -(spare) 91: -(spare) : 98: -(spare) 99: -(spare) |  |
|  |  |  |  | Qhcs, mapped from PCCPCH RSCP (TDD), see [4] [dBm] 0: -115 1: -114 2: -113 : 88: -27 89: -26 90: -(spare) 91: -(spare) : 98: -(spare) 99: -(spare) |  |
|  |  |  |  | Qhcs level, mapped from Averaged received signal level RSSI (GSM), see [4] [dBm] 0: -110 1: -109 2: -108 : 61: -49 62: -48 63: -47 64: -46 65: -45 66: -44 67: -43 68: -42 69: -41 70: -40 71: -39 72: -38 73: -37 74: -(spare) : 98: -(spare) 99: -(spare) |  |
| DeltaQhcs-RSCP | CV-RSCP |  | Integer(-5..-1) | If present, the actual value of Qhcs = Qhcs + DeltaQhcs-RSCP | REL-5 |

| Condition | Explanation |
| --- | --- |
| RSCP | This IE is optional if Qhcs is mapped from CPICH RSCP or PCCPCH RSCP, and if the value of the RSCP is below 0 (-115dBm). It is not needed otherwise. |

#### 10.3.7.55	Quality measured results list

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BLER measurement results | OP | 1 to <maxTrCH> |  |  |
| >DL Transport channel identity | MP |  | Transport channel identity 10.3.5.18 | transport channel type = DCH |
| >DL Transport Channel BLER | OP |  | Integer (0..63) | According to BLER_LOG in [19] and [20] |
| CHOICE mode | MP |  |  |  |
| >FDD |  |  |  | No data |
| >TDD |  |  |  |  |
| >>SIR measurement results | OP | 1 to <MaxCCTrCH> |  | SIR measurements for DL CCTrCH |
| >>>TFCS ID | MP |  | Integer(1...8) |  |
| >>>Timeslot list | MP | 1 to <maxTS> |  | for all timeslot on which the CCTrCH is mapped on |
| >>>>SIR | MP |  | Integer(0..63) | According to UE_SIR in [20] |

#### 10.3.7.56	Quality measurement

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Quality reporting quantity | OP |  | Quality reporting quantity 10.3.7.59 |  |
| CHOICE report criteria | MP |  |  |  |
| >Quality measurement reporting criteria |  |  | Quality measurement reporting criteria 10.3.7.58 | Note Given this choice, the IE "DL Transport Channel BLER" shall be set to FALSE (see subclause 10.3.7.59) |
| >Periodical reporting criteria |  |  | Periodical reporting criteria 10.3.7.53 | Note |
| >No reporting |  |  |  | Note (no data) Chosen when this measurement only is used as additional measurement to another measurement |

NOTE:	In this version of the specification, BLER as additional measurement is not supported.

#### 10.3.7.57	Quality measurement event results

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport channels causing the event | OP | 1 to <maxTrCH> |  |  |
| >DL Transport channel identity | MP |  | Transport channel identity 10.3.5.18 | transport channel type = DCH |

#### 10.3.7.58	Quality measurement reporting criteria

Event 5a: Number of bad CRCs on a certain transport channel exceeds a threshold.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Parameters sent for each transport channel | MP | 1 to <maxTrCH> |  |  |
| >DL Transport channel identity | MP |  | Transport channel identity 10.3.5.18 | transport channel type = DCH |
| >Total CRC | MP |  | Integer(1..512) | Number of CRCs |
| >Bad CRC | MP |  | Integer(1..512) | Number of CRCs |
| >Pending after trigger | MP |  | Integer(1..512) | Number of CRCs |

#### 10.3.7.59	Quality reporting quantity

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL Transport Channel BLER | MP |  | Boolean | TRUE means report requested |
| Transport channels for BLER reporting | CV-BLER reporting | 1 to <maxTrCH> |  | The default, if no transport channel identities are present, is that the BLER is reported for all downlink transport channels |
| >DL Transport channel identity | MP |  | Transport channel identity 10.3.5.18 | transport channel type = DCH |
| CHOICE mode | MP |  |  |  |
| >FDD |  |  |  | No data |
| >TDD |  |  |  |  |
| >>SIR measurement list | OP | 1 to <maxCCTrCH> |  | SIR measurements shall be reported for all listed TFCS IDs |
| >>>TFCS ID | MP |  | Integer(1...8) |  |

| Condition | Explanation |
| --- | --- |
| BLER reporting | This IE is not needed if the IE "DL Transport Channel BLER" is FALSE and optional if the IE "DL Transport Channel BLER" is TRUE |

#### 10.3.7.60	Reference time difference to cell

In the System Information message, the reference time difference to cell indicates the timing difference between the primary CCPCH of the current cell and the primary CCPCH of a neighbouring cell..

In the Measurement Control message, the reference time difference to cell indicates the timing difference between UE uplink transmission timing and the primary CCPCH of a neighbouring cell.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE accuracy | MP |  |  |  |
| >40 chips |  |  |  |  |
| >>Reference time difference | MP |  | Integer(0..38400 by step of 40) | In chips |
| >256 chips |  |  |  |  |
| >>Reference time difference | MP |  | Integer(0.. 38400 by step of 256) | In chips |
| >2560 chips |  |  |  |  |
| >>Reference time difference | MP |  | Integer(0.. 38400 by step of 2560) | In chips |

#### 10.3.7.61	Reporting Cell Status

Indicates maximum allowed number of cells to report and whether active set cells and/or virtual active set cells and/or monitored set cells and/or detected set cells on used frequency and/or monitored set cells and/or detected set cells on non used frequency should/should not be included in the IE "Measured results".

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE reported cell | MP |  |  |  |  |
| >Report cells within active set |  |  |  | This choice is not valid for inter-RAT measurements. For inter-frequency measurements it is only valid for reporting events 2D and 2F. See NOTE 1. |  |
| >>Maximum number of reported cells | MP |  | Integer(1..6) |  |  |
| >Report cells within monitored set cells on used frequency |  |  |  | This choice is not valid for inter-RAT or inter-frequency measurements |  |
| >>Maximum number of reported cells | MP |  | Integer(1..6) |  |  |
| >Report cells within active set and/or monitored set cells on used frequency |  |  |  | This choice is not valid for inter-RAT or inter-frequency measurements |  |
| >>Maximum number of reported cells | MP |  | Integer(1..6) |  |  |
| >Report cells within detected set on used frequency |  |  |  | This choice is not valid for inter-RAT or inter-frequency measurements |  |
| >>Maximum number of reported cells | MP |  | Integer(1..6) |  |  |
| >Report cells within monitored set and/or detected set on used frequency |  |  |  | This choice is not valid for inter-RAT or inter-frequency measurements |  |
| >>Maximum number of reported cells | MP |  | Integer(1..6) |  |  |
| >Report all active set cells + cells within monitored set on used frequency |  |  |  | This choice is not valid for inter-RAT or inter-frequency measurements |  |
| >>Maximum number of reported cells | MP |  | Enumerated (virtual/active set cells+1, virtual/active set cells+2, …., virtual/active set cells+6) |  |  |
| >Report all active set cells + cells within detected set on used frequency |  |  |  | This choice is not valid for inter-RAT or inter-frequency measurements |  |
| >>Maximum number of reported cells | MP |  | Enumerated (virtual/active set cells+1, virtual/active set cells+2, …., virtual/active set cells+6) |  |  |
| >Report all active set cells + cells within monitored set and/or detected set on used frequency |  |  |  | This choice is not valid for inter-RAT or inter-frequency measurements |  |
| >>Maximum number of reported cells | MP |  | Enumerated (virtual/active set cells+1, virtual/active set cells+2, …., virtual/active set cells+6) |  |  |
| >Report cells within virtual active set |  |  |  | This choice is not valid for intra-frequency or inter-RAT measurements |  |
| >>Maximum number of reported cells per reported non-used frequency | MP |  | Integer(1..6) |  |  |
| >Report cells within monitored set on non-used frequency |  |  |  | This choice is not valid for intra-frequency or inter-RAT measurements |  |
| >>Maximum number of reported cells per reported non-used frequency | MP |  | Integer(1..6) |  |  |
| >Report cells within monitored and/or virtual active set on non-used frequency |  |  |  | This choice is not valid for intra-frequency or inter-RAT measurements |  |
| >>Maximum number of reported cells per reported non-used frequency | MP |  | Integer(1..6) |  |  |
| >Report all virtual active set cells + cells within monitored set on non-used frequency |  |  |  | This choice is not valid for intra-frequency or inter-RAT measurements |  |
| >>Maximum number of reported cells per reported non-used frequency | MP |  | Enumerated (virtual/active set cells+1, virtual/active set cells+2, …., virtual/active set cells+6) |  |  |
| >Report cells within active set or within virtual active set or of the other RAT |  |  |  | If this choice is selected for inter-RAT measurements, the UE shall report only cells of the other RAT. If this choice is selected for intra-frequency measurements, the UE shall report cells within the active set. If this choice is selected for inter-frequency measurements, the UE shall report cells within the virtual active set. |  |
| >>Maximum number of reported cells | MP |  | Integer (1..12) |  |  |
| >Report cells within active and/or monitored set on used frequency or within virtual active and/or monitored set on non-used frequency |  |  |  | This choice is not valid for inter-RAT measurements. If this choice is selected for intra-frequency measurements, the UE shall report cells within the active and/or monitored set. If this choice is selected for inter-frequency measurements, the UE shall report cells within the virtual active set and/or monitored set on non-used frequency. |  |
| >>Maximum number of reported cells | MP |  | Integer(1..12) |  |  |
| >Report all virtual active set cells + cells within monitored set and/or detected set on non-used frequency |  |  |  | This choice is not valid for inter-RAT or intra-frequency measurements | REL-10 |
| >>Maximum number of reported cells | MP |  | Enumerated (virtual/active set cells+1, virtual/active set cells+2, …., virtual/active set cells+6) |  | REL-10 |

NOTE 1:	For Inter-frequency reporting events 2D and 2F, only CHOICE "Report cells within active set" is valid.

#### 10.3.7.62	Reporting information for state CELL_DCH

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Intra-frequency reporting quantity | MP |  | Intra-frequency reporting quantity 10.3.7.41 |  |
| Measurement Reporting Mode | MP |  | Measurement Reporting Mode 10.3.7.49 |  |
| CHOICE report criteria | MP |  |  |  |
| >Intra-frequency measurement reporting criteria |  |  | Intra-frequency measurement reporting criteria 10.3.7.39 | This IE may only contain measurement events from 1A to 1I, but not measurement event 1J. |
| >Periodical reporting criteria |  |  | Periodical reporting criteria 10.3.7.53 |  |

#### 10.3.7.63	SFN-SFN observed time difference

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE type | MP |  |  |  |  |
| >Type 1 |  |  | Integer(0..9830399) | According to T1_SFN-SFN_TIME in [19] and [20]. For FDD and 3.84 Mcps TDD and 7.68 Mcps TDD: 6946816 spare values are needed. |  |
|  |  |  | Integer(0..3276799) | For 1.28 Mcps TDD: 13500416 spare values are needed. | Rel-4 |
| >Type 2 |  |  | Integer(0..40961) | According to T2_SFN-SFN_TIME in [19] and [20]. 24574 spare values are needed. |  |
|  |  |  | Integer(0..27649) | For 1.28 Mcps TDD: 37886 spare values are needed. | Rel-4 |

#### 10.3.7.64	Time to trigger

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Time to trigger | MP |  | Integer(0, 10, 20, 40, 60, 80, 100, 120, 160, 200, 240, 320, 640, 1280, 2560, 5000) | Time in ms |

#### 10.3.7.65	Timeslot ISCP info

NOTE:	Only for TDD

| Information Element/Group name | Need | Multi | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Timeslot ISCP | MP |  | Integer (0..91) | According to UE_TS_ISCP_LEV in [20]. Thirty-six spare values are needed. |

#### 10.3.7.66	Traffic volume event identity

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Traffic volume event identity | MP |  | Enumerated(4a, 4b) |  |

#### 10.3.7.67	Traffic volume measured results list

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Traffic volume measurement results | OP | 1 to <maxRB> |  |  |
| >RB Identity | MP |  | RB Identity 10.3.4.16 |  |
| >RLC Buffers Payload | OP |  | Enumerated(0, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2K, 4K, 8K, 16K, 32K, 64K, 128K, 256K, 512K, 1024K) | In bytes And N Kbytes = N*1024 bytes. Twelve spare values are needed. |
| >Average of RLC Buffer Payload | OP |  | Enumerated(0, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2K, 4K, 8K, 16K, 32K, 64K, 128K, 256K, 512K, 1024K) | In bytes And N Kbytes = N*1024 bytes. Twelve spare values are needed. |
| >Variance of RLC Buffer Payload | OP |  | Enumerated(0, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2K, 4K, 8K, 16K) | In bytes And N Kbytes = N*1024 bytes. Two spare values are needed. |

#### 10.3.7.68	Traffic volume measurement

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Traffic volume measurement Object | OP |  | Traffic volume measurement Object 10.3.7.70 |  |
| Traffic volume measurement quantity | OP |  | Traffic volume measurement quantity 10.3.7.71 |  |
| Traffic volume reporting quantity | OP |  | Traffic volume reporting quantity 10.3.7.74 |  |
| Measurement validity | OP |  | Measurement validity 10.3.7.51 |  |
| CHOICE report criteria | MP |  |  |  |
| >Traffic volume measurement reporting criteria |  |  | Traffic volume measurement reporting criteria 10.3.7.72 |  |
| >Periodical reporting criteria |  |  | Periodical reporting criteria 10.3.7.53 |  |
| >No reporting |  |  |  | (no data) Chosen when this measurement only is used as additional measurement to another measurement |

#### 10.3.7.69	Traffic volume measurement event results

Contains the event result for a traffic volume measurement.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Uplink transport channel type causing the event | MP |  | Enumerated(DCH,RACH,USCH) | USCH is TDD only. RACH is the currently configured default in the uplink. For 1.28 Mcps TDD, if E-DCH is configured to the UE, and the Uplink transport channel type is USCH and UL Transport Channel identity is 32, it means E-DCH TVM is configured. |
| UL Transport Channel identity | CV-UL-DCH/USCH |  | Transport channel identity 10.3.5.18 |  |
| Traffic volume event identity | MP |  | Traffic volume event identity 10.3.7.66 |  |

| Condition | Explanation |
| --- | --- |
| UL-DCH/USCH | If IE "Uplink transport channel type" is equal to "DCH" or "USCH" (TDD only) this IE is mandatory present. Otherwise the IE is not needed. |

#### 10.3.7.70	Traffic volume measurement object

Contains the measurement object information for a traffic volume measurement.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Traffic volume measurement objects | MP | 1 to <maxTrCH> |  |  |
| >Uplink transport channel type | MP |  | Enumerated(DCH,RACH,USCH) | USCH is TDD only. RACH is the currently configured default in the uplink. For 1.28 Mcps TDD, if E-DCH is configured to the UE, and the Uplink transport channel type is USCH and UL Transport Channel identity is 32, it means E-DCH TVM is configured. |
| >UL Target Transport Channel ID | CV-UL-DCH/USCH |  | Transport channel identity 10.3.5.18 |  |

| Condition | Explanation |
| --- | --- |
| UL-DCH/USCH | If IE "Uplink transport channel type" is equal to "DCH" or "USCH" (TDD only) this IE is mandatory present. Otherwise the IE is not needed. |

#### 10.3.7.71	Traffic volume measurement quantity

Contains the measurement quantity information for a traffic volume measurement.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Measurement quantity | MP |  | Enumerated(RLC buffer payload, Average RLC buffer payload, Variance of RLC buffer payload) | This parameter should be ignored. |
| Time Interval to take an average or a variance | CV-A/V |  | Integer(20, 40, ..260, by steps of 20) | In ms |

| Condition | Explanation |
| --- | --- |
| A/V | This IE is mandatory present when "Average RLC buffer" or "Variance of RLC buffer payload" is chosen and not needed otherwise. |

#### 10.3.7.72	Traffic volume measurement reporting criteria

Contains the measurement reporting criteria information for a traffic volume measurement.

Event 4a: Transport Channel Traffic Volume [15] exceeds an absolute threshold.

Event 4b: Transport Channel Traffic Volume [15] becomes smaller than an absolute threshold.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Parameters sent for each transport channel | OP | 1 to <maxTrCH> |  | This IE is always required, need is OP to align with ASN.1 |
| >Uplink transport channel type | OP |  | Enumerated(DCH,RACH,USCH) | USCH is TDD only. RACH is the currently configured default in the uplink. For 1.28 Mcps TDD, if E-DCH is configured to the UE, and the Uplink transport channel type is USCH and UL Transport Channel identity is 32, it means E-DCH TVM is configured. |
| >UL Transport Channel ID | CV-UL-DCH/USCH |  | Transport channel identity 10.3.5.18 |  |
| >Parameters required for each Event | OP | 1 to <maxMeasparEvent> |  |  |
| >>Traffic volume event identity | MP |  | Traffic volume event identity 10.3.7.66 |  |
| >>Reporting Threshold | MP |  | Enumerated(8,16,32,64,128,256,512,1024,2K,3K,4K,6K,8K,12K,16K,24K,32K,48K,64K,96K,128K,192K,256K,384K,512K,768K) | Threshold in bytes And N Kbytes = N*1024 bytes |
| >>Time to trigger | OP |  | Time to trigger 10.3.7.64 | Indicates the period of time during which the event condition has to be satisfied, before sending a Measurement Report. Time in ms |
| >>Pending time after trigger | OP |  | Integer(250, 500, 1000, 2000, 4000, 8000, 16000) | Indicates the period of time during which it is forbidden to send any new measurement reports with the same Traffic volume event identity even if the triggering condition is fulfilled. Time in milliseconds |
| >>Tx interruption after trigger | OP |  | Integer (250, 500, 1000, 2000, 4000, 8000, 16000) | Time in milliseconds. Indicates how long the UE shall block DTCH transmissions on the RACH after a measurement report is triggered. |

| Condition | Explanation |
| --- | --- |
| UL-DCH/USCH | If IE "Uplink transport channel type" is equal to "DCH" or "USCH" (TDD only) this IE is optional. Otherwise the IE is not needed. |

#### 10.3.7.73	Traffic volume measurement system information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Traffic volume measurement identity | MD |  | Measurement identity 10.3.7.48 | The traffic volume measurement identity has default value 4. |
| Traffic volume measurement object | OP |  | Traffic volume measurement object 10.3.7.70 |  |
| Traffic volume measurement quantity | OP |  | Traffic volume measurement quantity 10.3.7.71 |  |
| Traffic volume reporting quantity | OP |  | Traffic volume reporting quantity 10.3.7.74 |  |
| Measurement validity | OP |  | Measurement validity 10.3.7.51 |  |
| Measurement Reporting Mode | MP |  | Measurement Reporting Mode 10.3.7.49 |  |
| CHOICE reporting criteria | MP |  |  |  |
| >Traffic volume measurement reporting criteria |  |  | Traffic volume measurement reporting criteria 10.3.7.72 |  |
| >Periodical reporting criteria |  |  | Periodical reporting criteria 10.3.7.53 |  |

#### 10.3.7.74	Traffic volume reporting quantity

Contains the reporting quantity information for a traffic volume measurement.

For all boolean types TRUE means inclusion in the report is requested.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RLC Buffer Payload for each RB | MP |  | Boolean |  |
| Average of RLC Buffer Payload for each RB | MP |  | Boolean |  |
| Variance of RLC Buffer Payload for each RB | MP |  | Boolean |  |

#### 10.3.7.75	UE internal event identity

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE internal event identity | MP |  | Enumerated(6a,6b,6c,6d,6e, 6f, 6g) |  |

#### 10.3.7.76	UE internal measured results

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>UE Transmitted Power | OP |  | UE Transmitted Power info 10.3.7.85 |  |  |
| >>UE Rx-Tx report entries | OP | 1 to <maxRL> |  |  |  |
| >>>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 | Primary CPICH info for each cell included in the active set |  |
| >>>UE Rx-Tx time difference type 1 | MP |  | UE Rx-Tx time difference type 1 10.3.7.83 | UE Rx-Tx time difference in chip for each RL included in the active set |  |
| >TDD |  |  |  |  |  |
| >>UE Transmitted Power list | OP | 1 to <maxTS> |  | UE Transmitted Power for each used uplink timeslot in ascending timeslot number order |  |
| >>>UE Transmitted Power | MP |  | UE Transmitted Power info 10.3.7.85 |  |  |
| >>CHOICE TDD option | MP |  |  |  | REL-4 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>Applied TA | OP |  | Uplink Timing Advance 10.3.6.95 | Uplink timing advance applied by the UE |  |
| >>>>Extended Applied TA | OP |  | Extended Uplink Timing Advance 10.3.6.95a | Uplink timing advance applied by the UE | REL-7 |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Extended Applied TA | OP |  | Extended Uplink Timing Advance 10.3.6.95a | Uplink timing advance applied by the UE | REL-7 |
| >>>1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>>>TADV | OP |  | TADV info 10.3.7.112 |  | REL-4 |

#### 10.3.7.77	UE internal measurement

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE internal measurement quantity | OP |  | UE internal measurement quantity 10.3.7.79 |  |
| UE internal reporting quantity | OP |  | UE internal reporting quantity 10.3.7.82 |  |
| CHOICE report criteria | MP |  |  |  |
| >UE internal measurement reporting criteria |  |  | UE internal measurement reporting criteria 10.3.7.80 |  |
| >Periodical reporting criteria |  |  | Periodical reporting criteria 10.3.7.53 |  |
| >No reporting |  |  |  | (no data) Chosen when this measurement only is used as additional measurement to another measurement |

| CHOICE report criteria | Condition under which the given report criteria is chosen |
| --- | --- |
| UE internal measurement reporting criteria | Chosen when UE internal measurement event triggering is required |
| Periodical reporting criteria | Chosen when periodical reporting is required |
| No reporting | Chosen when this measurement only is used as additional measurement to another measurement |

#### 10.3.7.78	UE internal measurement event results

This IE contains the measurement event results that are reported to UTRAN for UE internal measurements.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE internal event identity | MP |  | UE internal event identity 10.3.7.75 |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >Primary CPICH info | CV-clause 1 |  | Primary CPICH info 10.3.6.60 |  |  |
| >TDD |  |  |  | (no data) |

| Condition | Explanation |
| --- | --- |
| Clause 1 | This IE is mandatory present if the IE "UE internal event identity" is set to "6f" or "6g", otherwise the IE is not needed. |

#### 10.3.7.79	UE internal measurement quantity

The quantity the UE shall measure in case of UE internal measurement.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>Measurement quantity | MP |  | Enumerated(UE Transmitted Power, UTRA Carrier RSSI, UE Rx-Tx time difference) |  |  |
| >TDD |  |  |  |  |  |
| >>Measurement quantity | MP |  | Enumerated(UE Transmitted Power, UTRA Carrier RSSI, |  |  |
|  |  |  | TADV) | Measurement on Timing Advance is for 1.28 Mcps TDD | REL-4 |
| Filter coefficient | OP |  | Filter coefficient 10.3.7.9 | If the IE "Measurement quantity" is set to "Rx-Tx time difference" and this IE is present, the UE behaviour is unspecified. |  |

#### 10.3.7.80	UE internal measurement reporting criteria

The triggering of the event-triggered reporting for a UE internal measurement. All events concerning UE internal measurements are labelled 6x where x is a, b, c…. In TDD, the events 6a - 6d are measured and reported on timeslot basis.

Event 6a: The UE Transmitted Power becomes larger than an absolute threshold

Event 6b: The UE Transmitted Power becomes less than an absolute threshold

Event 6c: The UE Transmitted Power reaches its minimum value

Event 6d: The UE Transmitted Power reaches its maximum value

Event 6e: The UE RSSI reaches the UEs dynamic receiver range

Event 6f (FDD): The UE Rx-Tx time difference for a RL included in the active set becomes larger than an absolute threshold

Event 6f (1.28 Mcps TDD): The time difference indicated by TADV becomes larger than an absolute threshold

Event 6g: The UE Rx-Tx time difference for a RL included in the active set becomes less than an absolute threshold

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Parameters sent for each UE internal measurement event | OP | 1 to <maxMeasEvent> |  |  |  |
| >UE internal event identity | MP |  | UE internal event identity 10.3.7.75 |  |  |
| >Time-to-trigger | MP |  | Integer(0, 10, 20, 40, 60, 80, 100, 120, 160, 200, 240, 320, 640, 1280, 2560, 5000) | Time in ms. Indicates the period of time between the timing of event detection and the timing of sending Measurement Report. |  |
| >UE Transmitted Power Tx power threshold | CV-clause 1 |  | Integer(-50..33) | Power in dBm. In event 6a, 6b. |  |
| >UE Rx-Tx time difference threshold | CV-clause 2 |  | Integer(768..1280) | Time difference in chip. In event 6f, 6g. |  |
| >TADV threshold | CV-clause 3 |  | Real (0..63 step 0.125) | Time difference in chip. In event 6f | REL-4 |

| Condition | Explanation |
| --- | --- |
| Clause 1 | The IE is mandatory present if the IE "UE internal event identity" is set to "6a" or "6b", otherwise the IE is not needed. |
| Clause 2 | In FDD, the IE is mandatory present if the IE "UE internal event identity" is set to "6f" or "6g", otherwise the IE is not needed. |
| Clause 3 | In 1.28 Mcps TDD the IE is mandatory present if the IE "UE internal event identity" is set to "6f", otherwise the IE is not needed. |

#### 10.3.7.81	Void

#### 10.3.7.82	UE Internal reporting quantity

For all boolean types TRUE means inclusion in the report is requested.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UE Transmitted Power | MP |  | Boolean |  |  |
| CHOICE mode | MP |  |  |  |  |
| >FDD |  |  |  |  |  |
| >>UE Rx-Tx time difference | MP |  | Boolean |  |  |
| >TDD |  |  |  |  |  |
| >>CHOICE TDD option |  |  |  |  | REL-4 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-4 |
| >>>>Applied TA | MP |  | Boolean |  |  |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Applied TA | MP |  | Boolean |  | REL-7 |
| >>>1.28 Mcps TDD |  |  |  |  | REL-4 |
| >>>>TADV info | MP |  | Boolean |  | REL-4 |

#### 10.3.7.83	UE Rx-Tx time difference type 1

The difference in time between the UE uplink DPCCH/DPDCH frame transmission and the first detected path (in time), of the downlink DPCH or F-DPCH frame from the measured radio link, as defined in [7]. This measurement is for FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Rx-Tx time difference type 1 | MP |  | Integer(768..1280) | In chips. 511 spare values are needed. |

#### 10.3.7.84	UE Rx-Tx time difference type 2

The difference in time between the UE uplink DPCCH/DPDCH frame transmission and the first detected path (in time), of the downlink DPCH or F-DPCH frame from the measured radio link, as defined in [7].

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Rx-Tx time difference type 2 | MP |  | Integer (0..8191) | According to [19]. |

#### 10.3.7.85	UE Transmitted Power info

| Information Element/Group name | Need | Multi | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Transmitted Power | MP |  | Integer (0..104) | According to UE_TX_POWER in [19] and [20] |

#### 10.3.7.86	UE positioning Ciphering info

This IE contains information for the ciphering of UE positioning assistance data broadcast in System Information.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Ciphering Key Flag | MP |  | Bit string(1) |  |
| Ciphering Serial Number | MP |  | Integer(0..65535) | The serial number used in the DES ciphering algorithm |

#### 10.3.7.87	UE positioning Error

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Error reason | MP |  | Enumerated(Not Enough OTDOA Cells, Not Enough GPS Satellites, Assistance Data Missing, Not Accomplished GPS Timing Of Cell Frames, Undefined Error, Request Denied By User, Not Processed And Timeout, Reference Cell Not Serving Cell, | Note 1 |  |
|  |  |  | Not Enough GANSS Satellites, Not Accomplished GANSS Timing Of Cell Frames) | Note 1 | REL-7 |
| GPS Additional Assistance Data Request | CV-GPSdataMissing |  | UE positioning GPS Additional Assistance Data Request 10.3.7.88a |  |  |
| GANSS Additional Assistance Data Request | CV-GANSSdataMissing |  | UE positioning GANSS Additional Assistance Data Request 10.3.7.88c |  | REL-7 |

NOTE 1:	The following table describes each value of the IE "Error reason".

| Value | Indication |
| --- | --- |
| Not Enough OTDOA Cells | There were not enough cells to be received. |
| Not Enough GPS Satellites | There were not enough GPS satellites to be received. |
| Assistance Data Missing | UE positioning GANSS or/and GPS assistance data missing. |
| Not Accomplished GPS Timing Of Cell Frames | UE was not able to accomplish the GPS timing of cell frames measurement. |
| Undefined Error | Undefined error. |
| Request Denied By User | UE positioning request denied by upper layers. |
| Not Processed And Timeout | UE positioning request not processed by upper layers and timeout. |
| Reference Cell Not Serving Cell | UE was not able to read the SFN of the reference cell. |
| Not Enough GANSS Satellites | There were not enough GANSS satellites to be received. |
| Not Accomplished GANSS Timing Of Cell Frames | UE was not able to accomplish the GANSS timing of cell frames measurement. |

| Condition | Explanation |
| --- | --- |
| GPSdataMissing | The IE is optional if the IE "Error reason" is "Assistance Data Missing" and not needed otherwise. |
| GANSSdataMissing | The IE is optional if the IE "Error reason" is "Assistance Data Missing" and not needed otherwise. |

#### 10.3.7.88	UE positioning GPS acquisition assistance

This IE contains parameters that enable fast acquisition of the GPS signals in UE-assisted GPS positioning.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GPS TOW msec | MP |  | Integer(0..6.048*108-1) | GPS Time of Week in milliseconds rounded down to the nearest millisecond unit. It is also the time when satellite information is valid. |  |
| UTRAN GPS reference time | OP |  |  |  |  |
| >UTRAN GPS timing of cell frames | MP |  | Integer(0 ... 2322431999999) | GPS timing of cell frames in steps of 1 chip. |  |
| >CHOICE mode | OP |  |  |  |  |
| >>FDD |  |  |  |  |  |
| >>>Primary CPICH Info | MP |  | Primary CPICH Info 10.3.6.60 | Identifies the reference cell for the GPS TOW-SFN relationship |  |
| >>TDD |  |  |  |  |  |
| >>>cell parameters id | MP |  | Cell parameters id 10.3.6.9 | Identifies the reference cell for the GPS TOW-SFN relationship |  |
| >SFN | MP |  | Integer(0..4095) | The SFN which the UTRAN GPS timing of cell frames time stamps. |  |
| UE Positioning GPS ReferenceTime Uncertainty | OP |  | UE positioning GPS reference time uncertainty 10.3.7.96a |  | REL-7 |
| Confidence | OP |  | Integer(0..100) | Confidence level (in percent) of the reference location area or volume used to calculate the Satellite information parameters (search windows). | REL-12 |
| Satellite information | MP | 1 to <maxSat> |  |  |  |
| >SatID | MP |  | Integer (0..63) | Identifies the satellite and is equal to (SV ID No - 1) where SV ID No is defined in [12]. |  |
| >Doppler (0th order term) | MP |  | Real(-5120..5117.5 by step of 2.5) | Hz |  |
| >Extra Doppler | OP |  |  |  |  |
| >>Doppler (1st order term) | MP |  | Real (-0.966..0.483 by step of 0.023) | Hz/s |  |
| >>Doppler Uncertainty | MP |  | Enumerated (12.5,25,50,100,200) | Hz. Three spare values are needed. The Doppler experienced by a stationary UE is in the range “Doppler – Doppler Uncertainty” to “Doppler + Doppler Uncertainty”. |  |
| >Extra Doppler Extension | OP |  |  | If this field is present, the "Extra Doppler" field should not be present. | REL-12 |
| >>Doppler (1st order term) | MP |  | Real (-0.966..0.483 by step of 0.023) | Hz/s | REL-12 |
| >>Doppler Uncertainty Extension | MP |  | Enumerated (300, 400, 500, 600, ‘No Information’) | Hz. Three spare values are needed. The Doppler experienced by a stationary UE is in the range "Doppler – Doppler Uncertainty Extension" to "Doppler + Doppler Uncertainty Extension". | REL-12 |
| >Code Phase | MP |  | Integer(0..1022) | GPS chips. Increasing binary values of the field signify increasing predicted pseudoranges. |  |
| >Integer Code Phase | MP |  | Integer(0..19) | Number of code periods that have elapsed since the latest GPS bit boundary, in units of C/A code period. |  |
| >GPS Bit number | MP |  | Integer(0..3) | Specifies GPS bit number modulo 4 |  |
| >Code Phase Search Window | MP |  | Integer(1023,1,2,3,4,6,8,12,16,24,32,48,64,96,128,192) | Expected code-phase is in the range “Code Phase – Code Phase Search Window” to “Code Phase + Code Phase Search Window”. |  |
| >Azimuth and Elevation | OP |  |  |  |  |
| >>Azimuth | MP |  | Real(0..348.75 by step of 11.25) | Degrees An angle of x degrees means the satellite azimuth a is in the range x ≤ a < x+11.25 degrees. |  |
| >>Elevation | MP |  | Real(0..78.75 by step of 11.25) | Degrees An angle of y degrees means the satellite elevation e is in the range y ≤ e < y+11.25 degrees except for y=78.75 where the range is extended to include 90 degrees. |  |
| >>Azimuth LSB | MP |  | Real(0..10.546875 by step of 0.703125) | Degrees The full satellite azimuth is constructed as “Azimuth” + “Azimuth LSB”. An azimuth angle of x degrees means the satellite azimuth a is in the range x ≤ a < x+0.703125 degrees. Range 0-359.296875 deg. | REL-10 |
| >>Elevation LSB | MP |  | Real(0..10.546875 by step of 0.703125) | Degrees The full satellite elevation is constructed as “Elevation” + “Elevation LSB”. An elevation angle of y degrees means the satellite elevation e is in the range y ≤ e < y+0.703125 degrees. Range 0-89.296875 deg. | REL-10 |

#### 10.3.7.88a	UE positioning GPS Additional Assistance Data Request

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Almanac | MP |  | Boolean | TRUE means requested |
| UTC Model | MP |  | Boolean | TRUE means requested |
| Ionospheric model | MP |  | Boolean | TRUE means requested |
| Navigation Model | MP |  | Boolean | TRUE means requested |
| DGPS Corrections | MP |  | Boolean | TRUE means requested |
| Reference Location | MP |  | Boolean | TRUE means requested |
| Reference Time | MP |  | Boolean | TRUE means requested |
| Acquisition Assistance | MP |  | Boolean | TRUE means requested |
| Real-Time Integrity | MP |  | Boolean | TRUE means requested |
| Navigation Model Additional data | CV-Navigation Model |  |  |  |
| >GPS Week | MP |  | Integer (0..1023) |  |
| >GPS_Toe | MP |  | Integer (0..167) | GPS time of ephemeris in hours of the latest ephemeris set contained by the UE. Eighty-eight spare values needed. |
| >T-Toe limit | MP |  | Integer (0..10) | ephemeris age tolerance of the UE to UTRAN in hours. Five spare values needed. |
| >Satellites list related data | MP | 0 to <maxSat> |  |  |
| >>SatID | MP |  | Integer (0..63) | Identifies the satellite and is equal to (SV ID No - 1) where SV ID No is defined in [12]. |
| >>IODE | MP |  | Integer (0..255) | Issue of Data Ephemeris for SatID |

| Condition | Explanation |
| --- | --- |
| Navigation Model | This IE is mandatory present if "Navigation Model" is set to TRUE otherwise it is absent. |

#### 10.3.7.88b		UE positioning GANSS reference measurement information

This IE provides reference code and Doppler measurement information of visible satellites of a GNSS constellation. The information enables fast acquisition of the GANSS signals in UE-assisted GANSS positioning. If the IE "UE positioning GPS acquisition assistance" is not included in IE "UE positioning measurement", the Satellite Information is valid at the time as given in the IE "UE positioning GANSS reference time" IE. I.e., if IE "UE positioning GANSS reference measurement information" is included in IE "UE positioning GANSS assistance data" and IE "UE positioning GPS acquisition assistance" is not included in IE "UE positioning measurement", the IE "UE positioning GANSS reference time" shall also be included. If the IE "UE positioning GPS acquisition assistance" is included in IE "UE positioning measurement", the IE "GPS TOW msec" and IE "UTRAN GPS reference time" included in IE "UE positioning GPS acquisition assistance" is also valid for the Satellite information in IE "UE positioning GANSS reference measurement information".

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GANSS Signal ID | OP |  | GANSS Signal Id 10.3.3.45a | Absence of this field means the default value as defined in 10.3.3.45a for the GANSS identified by GANSS_ID. If this IE is asked by SIB 15.6, the GANSS shall be solved by the scheduling information. | REL-7 |
| Confidence | OP |  | Integer(0..100) | Confidence level (in percent) of the reference location area or volume used to calculate the Satellite information parameters (search windows). | REL-12 |
| Satellite information | MP | 1 to <maxGANSSSat> |  |  | REL-7 |
| >SatID | MP |  | Integer (0..63) | Identifies the satellite within a GANSS constellation. For coding description, see Note 2. | REL-7 |
| >Doppler (0th order term) | MP |  | Real(-1024..1023.5 by step of 0.5) | m/s Conversion between m/s and Hz shall be made by using the nominal wavelength of the assisted signal. | REL-7 |
| >Extra Doppler | OP |  |  |  | REL-7 |
| >>Doppler (1st order term) | MP |  | Real (-0.2..0.1 by step of 1/210) | m/s2 | REL-7 |
| >>Doppler Uncertainty | MP |  | Enumerated (40,20,10,5,2.5) | m/s. The Doppler experienced by a stationary UE is in the range “Doppler – Doppler Uncertainty” to “Doppler + Doppler Uncertainty”. Three spare values are needed. | REL-7 |
| >Extra Doppler Extension | OP |  |  | If this field is present, the "Extra Doppler" field should not be present. | REL-12 |
| >>Doppler (1st order term) | MP |  | Real (-0.2..0.1 by step of 1/210) | m/s2 | REL-12 |
| >>Doppler Uncertainty Extension | MP |  | Enumerated (60, 80, 100, 120, ‘No Information’) | m/s. The Doppler experienced by a stationary UE is in the range "Doppler – Doppler Uncertainty Extension" to "Doppler + Doppler Uncertainty Extension". Three spare values are needed. | REL-12 |
| >Code Phase | MP |  | Integer(0..1023) | ms, scaling factor 2-10 Nominal chipping rate of the GNSS signal shall be used in conversion. Increasing binary values of the field signify increasing predicted code phases. | REL-7 |
| >Integer Code Phase | MP |  | Integer(0..127) | ms. Integer code phase (expressed modulo 128 ms) . The satellite integer milli-seconds code phase currently being transmitted at the Reference Time included in either IE "UE positioning GANSS reference time” or IE “UE positioning GPS acquisition assistance”, as seen by a receiver at the Reference Location is calculated as Reference Time (expressed in milli-seconds) minus (Integer_Code_Phase + (n×128 ms)), with n = …-2,-1,0,1,2….. The UE can calculate the expected code phase as follows: Reference Time – Integer Code Phase + Code Phase. | REL-7 |
| >Code Phase Search Window | MP |  | Bit string (5) | Coded expected code-phase is in the range “Code Phase – Code Phase Search Window” to “Code Phase + Code Phase Search Window”. For coding description, see Note 1 | REL-7 |
| >Azimuth and Elevation | OP |  |  |  | REL-7 |
| >>Azimuth | MP |  | Real(0..348.75 by step of 11.25) | Degrees An angle of x degrees means the satellite azimuth a is in the range x ≤ a < x+11.25 degrees. | REL-7 |
| >>Elevation | MP |  | Real(0..78.75 by step of 11.25) | Degrees An angle of y degrees means the satellite elevation e is in the range y ≤ e < y+11.25 degrees except for y=78.75 where the range is extended to include 90 degrees. | REL-7 |
| >>Azimuth LSB | MP |  | Real(0..10.546875 by step of 0.703125) | Degrees The full satellite azimuth is constructed as “Azimuth” + “Azimuth LSB”. An azimuth angle of x degrees means the satellite azimuth a is in the range x ≤ a < x+0.703125 degrees Range 0-359.296875 deg. | REL-10 |
| >>Elevation LSB | MP |  | Real(0..10.546875 by step of 0.703125) | Degrees The full satellite elevation is constructed as “Elevation” + “Elevation LSB”. An elevation angle of y degrees means the satellite elevation e is in the range y ≤ e < y+0.703125 degrees. Range 0-89.296875 deg. | REL-10 |

NOTE 1:	Code phase search window parameter format:

| CODE_PHASE_SEARCH_WINDOW | Code Phase Search Window [ms] |
| --- | --- |
| '00000' | No information |
| '00001' | 0,002 |
| '00010' | 0,004 |
| '00011' | 0,008 |
| '00100' | 0,012 |
| '00101' | 0,016 |
| '00110' | 0,024 |
| '00111' | 0,032 |
| '01000' | 0,048 |
| '01001' | 0,064 |
| '01010' | 0,096 |
| '01011' | 0,128 |
| '01100' | 0,164 |
| '01101' | 0,200 |
| '01110' | 0,250 |
| '01111' | 0,300 |
| '10000' | 0,360 |
| '10001' | 0,420 |
| '10010' | 0,480 |
| '10011' | 0,540 |
| '10100' | 0,600 |
| '10101' | 0,660 |
| '10110' | 0,720 |
| '10111' | 0,780 |
| '11000' | 0,850 |
| '11001' | 1,000 |
| '11010' | 1,150 |
| '11011' | 1,300 |
| '11100' | 1,450 |
| '11101' | 1,600 |
| '11110' | 1,800 |
| '11111' | 2,000 |

NOTE 2:	Interpretation of SatID:

| GANSS | Value of SatID | Interpretation of SatID |
| --- | --- | --- |
| Galileo | ‘0’ – ‘35’ ‘36’ – ‘63’ | Code No. 1 to 36 Reserved |
| Modernized GPS | ‘0’ – ‘62’ | Satellite PRN Signal No. 1 to 63 |
|  | ‘63’ | Reserved |
| SBAS | ‘0’ – ‘38’ | Satellite PRN Signal No. 120 to 158 |
|  | ‘39’ – ‘63’ | Reserved |
| QZSS | ‘0’ – ‘4’ | Satellite PRN Signal No. 193 to 197 |
|  | ‘5’ – ‘63’ | Reserved |
| GLONASS | ‘0’ – ‘23’ | Slot Number 1 to 24 |
|  | ‘24’ – ‘63’ | Reserved |
| BDS | ‘0’ – ‘36’ | Satellite ranging code number signal No.1 to 37 [86] |
|  | ‘37’ – ‘63’ | Reserved |

#### 10.3.7.88c	UE positioning GANSS additional assistance data request

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GANSS Reference Time | MP |  | Boolean | TRUE means requested | REL-7 |
| GANSS Reference Location | MP |  | Boolean | TRUE means requested | REL-7 |
| GANSS Ionospheric model | MP |  | Boolean | TRUE means requested | REL-7 |
| GANSS Additional Ionospheric Model | OP |  | Data ID Bit String(2) | If present, the UE requests the “UE positioning GANSS additional ionospheric model” for a “Data ID” as defined in 10.3.7.92b. | REL-8 |
| GANSS Earth Orientation Parameters | OP |  | Enumerated (TRUE) | If present, the UE requests the “UE positioning GANSS Earth orientation parameters” | REL-8 |
| GANSS Requested Generic Assistance Data | MP | 1 to <maxGANSS> |  |  | REL-7 |
| >GANSS ID | OP |  | Integer(0..7) | Absence of this IE means Galileo Values 0-7 reserved for future use. | REL-7 |
|  |  |  |  | Absence of this IE means Galileo. For coding description see NOTE 1 in 10.3.7.90b. | REL-8 |
| >SBAS ID | CV-GANSS-ID-SBAS |  | UE positioning GANSS SBAS ID 10.3.7.97e |  | REL-8 |
| >GANSS Real-Time Integrity | MP |  | Boolean | TRUE means requested | REL-7 |
| >DGANSS Signal | OP |  | DGANSS Signal Id 10.3.7.88d | If present, the UE requests the "UE positioning DGANSS corrections" | REL-7 |
| >DBDS Signal | CV-GANSS-ID-BDS |  | DGANSS Signal Id 10.3.7.88d | If present, the UE requests the "UE positioning DBDS corrections" | REL-12 |
| >BDS Ionospheric Grid Model | CV-GANSS-ID-BDS |  | Enumerated (TRUE) | If present, the UE requests the "UE positioning BDS Ionospheric Grid Model" | REL-12 |
| >GANSS Almanac | MP |  | Boolean | TRUE means requested | REL-7 |
|  |  |  |  | TRUE means requested. If “GANSS ID” indicates “Modernized GPS” or “QZSS” and IE “GANSS additional assistance data choices” is not included, the GANSS Almanac model requested is “Model-4” for Modernized GPS, and “Model-2” for QZSS, as defined in 10.3.7.89a. | REL-8 |
| >GANSS Navigation Model | MP |  | Boolean | TRUE means requested | REL-7 |
| >GANSS Additional Navigation Models | OP |  | Enumerated (TRUE) | If present, the UE requests the “UE positioning GANSS additional navigation models”. If “GANSS ID” indicates “QZSS” and IE “GANSS additional assistance data choices” is not included, the GANSS orbit and clock model requested is “Model-2” as defined in 10.3.7.91e and 10.3.7.91f, respectively. | REL-8 |
| >GANSS Time Model GNSS-GNSS | OP |  | BIT STRING (8) | The reference system for requested GANSS time model GNSS-GNSS is as indicated in IE "GANSS ID". The models are requested for each system with respective bit set to “1”. Bit 0 is set for GPS, Bit 1 is set for Galileo. Other bits are reserved | REL-7 |
|  |  |  |  | Bit 2 is set for QZSS Bit 3 is set for GLONASS Other bits are reserved | REL-8 |
|  |  |  |  | Bit 4 is set for BDS Other bits are reserved | REL-12 |
| >GANSS Reference Measurement Information | MP |  | Boolean | TRUE means requested | REL-7 |
| >GANSS data bits | OP |  |  |  | REL-7 |
| >>GANSS TOD | MP |  | INTEGER (0..86399) | This field contains the reference time modulo 86400 seconds of the first data bit of the requested data in integer seconds in GNSS specific system time of the GNSS indicated by IE "GANSS ID". | REL-7 |
| >>Data bit assistance | MP |  |  |  | REL-7 |
| >>>GANSS Signal ID | MP |  | DGANSS Signal Id 10.3.7.88d |  | REL-7 |
| >>>GANSS Data Bit Interval | MP |  | Integer (0..15) | This field represents the time length for which the Data Bit Assistance is requested. The Data Bit Assistance shall be relative to the time interval (GANSS TOD, GANSS TOD + Data Bit Interval). The Data Bit Interval r, expressed in seconds, is mapped to a binary number K with the following formula: r =0.1* 2 K Value K=15 means that the time interval is not specified. | REL-7 |
| >>>Satellite Information | OP | 1 to <maxGANSSSat> |  |  | REL-7 |
| >>>>Satellite ID | MP |  | Integer (0..63) | Identifies the satellite for which the Data Bit Assistance request is applicable and is as defined in Note 2 of 10.3.7.88b. | REL-7 |
| >GANSS UTC model | MP |  | Boolean | TRUE means Requested | REL-7 |
| >GANSS Additional UTC Models | OP |  | Enumerated (TRUE) | If present, the UE requests the “UE positioning GANSS additional UTC models” | REL-8 |
| >GANSS Auxiliary Information | OP |  | Enumerated (TRUE) | If present, the UE requests the “UE positioning GANSS auxiliary information” | REL-8 |
| >GANSS Navigation Model Additional data | CV-GANSSNavigation Model |  |  |  | REL-7 |
| >>GANSS Week/Day | MP |  | Integer (0..4095) | If “GANSS ID” does not indicate “GLONASS”, this IE defines the GANSS Week number of the assistance currently held by the MS. | REL-7 |
|  |  |  |  | If “GANSS ID” is set to “GLONASS”, this IE defines the calendar number of day within the four-year interval starting from 1st of January in a leap year, as defined by the parameter NT in [73] of the assistance currently held by the MS. | REL-8 |
| >>GANSS_Toe | MP |  | Integer (0..167) | If “GANSS ID” does not indicate “GLONASS”, this IE defines the GANSS time of ephemeris in hours of the latest ephemeris set contained by the UE. | REL-7 |
|  |  |  |  | If “GANSS ID” is set to “GLONASS”, this IE defines the time of ephemeris in units of 15 minutes of the latest ephemeris set contained by the UE (range 0 to 95 representing time values between 0 and 1425 minutes). In this case, values 96 to 167 shall not be used by the sender. | REL-8 |
| >>T-Toe limit | MP |  | Integer (0..10) | If “GANSS ID” does not indicate “GLONASS”, this IE defines the ephemeris age tolerance of the UE to UTRAN in units of hours. Five spare values needed. | REL-7 |
|  |  |  |  | If “GANSS ID” is set to “GLONASS”, this IE defines the ephemeris age tolerance of the UE to UTRAN in units of 30 minutes (range 0 to 10 representing time values of 0 to 300 minutes). | REL-8 |
| >>Satellites list related data | OP | 1 to <maxGANSSSat> |  |  | REL-7 |
| >>>SatID | MP |  | Integer (0..63) | Identifies the satellite and is as defined in Note 2 of 10.3.7.88b. | REL-7 |
| >>>IOD | MP |  | Integer (0..1023) | 10 LSBs of Issue of Data for SatID | REL-7 |
| >GANSS additional assistance data choices | CV-AssistanceChoices |  |  |  | REL-8 |
| >>Orbit Model ID | OP |  | Integer (0..7) | This IE may be included if IE "GANSS Navigation Model" is set to TRUE or if the IE “GANSS Additional Navigation Models” is included and defines the non-native or non-default orbit model number as defined in 10.3.7.91c/10.3.7.91e. | REL-8 |
| >>Clock Model ID | OP |  | Integer (0..7) | This IE may be included if IE "GANSS Navigation Model" is set to TRUE or if the IE “GANSS Additional Navigation Models” is included and defines the non-native or non-default clock model number as defined in 10.3.7.91d/10.3.7.91f. | REL-8 |
| >>UTC Model ID | OP |  | Integer (0..7) | This IE may be included if IE “GANSS Additional UTC Models” is included and defines the non-native or non-default UTC model number as defined in 10.3.7.97d | REL-8 |
| >>Almanac Model ID | OP |  | Integer (0..7) | This IE may be included if IE “GANSS Almanac” set to TRUE and defines the non-native or non-default model number as defined in 10.3.7.89a. | REL-8 |

| Condition | Explanation |
| --- | --- |
| GANSS-ID-SBAS | This IE is mandatory present if theIE “GANSS ID” is “SBAS” and not needed otherwise. |
| GANSSNavigation Model | The IE is mandatory present if the IE "GANSS Navigation Model" is set to TRUE or if the IE “GANSS Additional Navigation Models” is included and not needed otherwise. |
| AssistanceChoices | This IE is mandatory present if non-native or non-default assistance data choices are requested and not needed otherwise. |
| GANSS-ID-BDS | This IE is optionally present if the IE "GANSS ID" is "BDS" and not needed otherwise. |

#### 10.3.7.88d	DGANSS Signal Id

The DGANSS signal Id refers to the signal for which differential corrections or data bit assistance are required. It depends on the GANSS ID. Each bit refers to a particular signal. When the bit is set to 1, this means that the differential corrections or data bit assistance are requested for this particular signal. Differential corrections or data bit assistance can be requested for several signals using the bit string.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| DGANSS Signal Id | MP |  | BIT STRING (8) | See Note 1. | REL-7 |

NOTE 1:

| GANSS Id | Bit number | Explanation | Version |
| --- | --- | --- | --- |
| Default : Galileo | 0 | Galileo E1 | REL-7 |
|  | 1 | Galileo E5A |  |
|  | 2 | Galileo E5B |  |
|  | 3 | Galileo E6 |  |
|  | 4 | spare |  |
|  | 5 | Spare |  |
|  | 6 | Spare |  |
|  | 7 | Spare |  |
| Modernized GPS | 0 | L1C | REL-8 |
|  | 1 | L2C |  |
|  | 2 | L5 |  |
|  | 3 | Spare |  |
|  | 4 | Spare |  |
|  | 5 | Spare |  |
|  | 6 | Spare |  |
|  | 7 | Spare |  |
| SBAS | 0 | L1 |  |
|  | 1 | Spare |  |
|  | 2 | Spare |  |
|  | 3 | Spare |  |
|  | 4 | Spare |  |
|  | 5 | Spare |  |
|  | 6 | Spare |  |
|  | 7 | Spare |  |
| QZSS | 0 | QZS-L1 |  |
|  | 1 | QZS-L1C |  |
|  | 2 | QZS-L2C |  |
|  | 3 | QZS-L5 |  |
|  | 4 | Spare |  |
|  | 5 | Spare |  |
|  | 6 | Spare |  |
|  | 7 | Spare |  |
| GLONASS | 0 | G1 |  |
|  | 1 | G2 |  |
|  | 2 | G3 |  |
|  | 3 | Spare |  |
|  | 4 | Spare |  |
|  | 5 | Spare |  |
|  | 6 | Spare |  |
|  | 7 | spare |  |
| BDS | 0 | B1I | REL-12 |
|  | 1 | Spare |  |
|  | 2 | Spare |  |
|  | 3 | Spare |  |
|  | 4 | Spare |  |
|  | 5 | Spare |  |
|  | 6 | Spare |  |
|  | 7 | Spare |  |

#### 10.3.7.89	UE positioning GPS almanac

This IE contains a reduced-precision subset of the ephemeris and clock correction parameters.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| WNa | MP |  | Bit string(8) | Almanac Reference Week [12] |  |
| Complete Almanac Provided | OP |  | Boolean | This field indicates whether the RNC provided almanac for the full GPS constellation or not. TRUE means complete GPS almanac is provided. | REL-10 |
| Satellite information | MP | 1 to <maxSat> |  |  |  |
| >DataID | MP |  | Integer(0..3) | See [12] |  |
| >SatID | MP |  | Enumerated(0..63) | Identifies the satellite and is equal to (SV ID No - 1) where SV ID No is defined in [12]. |  |
| >e | MP |  | Bit string(16) | Eccentricity [12] |  |
| >toa | MP |  | Bit string(8) | Reference Time of Almanac [12] |  |
| >i | MP |  | Bit string(16) |  |  |
| >OMEGADOT | MP |  | Bit string(16) | Longitude of Ascending Node of Orbit Plane at Weekly Epoch (semi-circles/sec) [12] |  |
| >SV Health | MP |  | Bit string(8) |  |  |
| >A1/2 | MP |  | Bit string(24) | Semi-Major Axis (meters)1/2 [12] |  |
| >OMEGA0 | MP |  | Bit string(24) | Longitude of Ascending Node of Orbit Plane at Weekly Epoch (semi-circles) [12] |  |
| >M0 | MP |  | Bit string(24) | Mean Anomaly at Reference Time (semi-circles) [12] |  |
| > | MP |  | Bit string(24) | Argument of Perigee (semi-circles) [12] |  |
| >af0 | MP |  | Bit string(11) | apparent clock correction [12] |  |
| >af1 | MP |  | Bit string(11) | apparent clock correction [12] |  |
| SV Global Health | OP |  | Bit string(364) | This enables GPS time recovery and possibly extended GPS correlation intervals. It is specified in page 25 of subframes 4 and 5 [12] |  |

#### 10.3.7.89a		UE positioning GANSS almanac

This IE contains a reduced-precision subset of the ephemeris and clock correction parameters.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Week Number | MP |  | Integer(0..255) | Almanac reference week , number of weeks since the beginning of GANSS specific system time (mod 256). Note, in case of Galileo, the almanac reference week number WNa natively contains only the 2 LSB’s [61, section 5.1.10]. | REL-7 |
|  |  |  |  | If “Model 5” or “Model 6” is included, the UE shall ignore the Week Number. | REL-8 |
| Complete Almanac Provided | OP |  | Boolean | This field indicates whether the RNC provided almanac for the full GANSS constellation or not. TRUE means complete GANSS almanac is provided. | REL-10 |
| Keplerian parameters (“Model 1”) | OP |  |  | NOTE | REL-7 |
| >Toa | MP |  | Integer(0..613800 by step of 600) | Almanac Reference Time common to all satellites in GANSS Almanac using Keplerian Parameters given in GNSS specific system time [61]. | REL-12 |
| >IODa | MP |  | INTEGER(0..15) | Issue-Of –Data, common to all satellites [61] | REL-12 |
| >Satellite information KP | MP | 1 to <maxGANSSSat> |  | Almanacs are in the order of the SV IDs, the smallest ID first. | REL-7 |
| >>SV ID | MP |  | Integer (0..63) | Satellite ID, as defined in Note 2 of 10.3.7.88b. | REL-7 |
| >>e | MP |  | Bit string(11) | Eccentricity, dimensionless [61] | REL-7 |
| >>i | MP |  | Bit string(11) | Parameter i, inclination at reference time relative to i0=56°; semi-circles [61] | REL-7 |
| >>OMEGADOT | MP |  | Bit string(11) | Parameter , rate of change of right ascension (semi-circles/sec) [61]. | REL-7 |
| >>SV Status INAV | MP |  | Bit string(4) | This field contains the I/NAV signal health status [61, section 5.1.10], E5bHS and E1-BHS, where E5bHS occupies the 2 MSBs and E1-BHS the two LSBs. | REL-7 |
| >>SV Status FNAV | OP |  | Bit string(2) | This field contains the F/NAV signal health status [61, section 5.1.10], E5aHS. | REL-12 |
| >>delta A1/2 | MP |  | Bit string(13) | Parameter (a1/2), difference with respect to the square root of the nominal semi-major axis, (meters)1/2 [61]. | REL-12 |
| >>OMEGA0 | MP |  | Bit string(16) | Longitude of ascending node of orbital plane at weekly epoch (semi-circles) [61] | REL-7 |
| >>M0 | MP |  | Bit string(16) | Mean Anomaly at Reference Time (semi-circles) [61] | REL-7 |
| >> | MP |  | Bit string(16) | Argument of Perigee (semi-circles) [61] | REL-7 |
| >>af0 | MP |  | Bit string(16) | Parameter af0, satellite clock correction bias. Seconds [61] | REL-12 |
| >>af1 | MP |  | Bit string(13) | Parameter af1, satellite clock correction linear, sec/sec [61] | REL-12 |
| NAV Keplerian Parameters (“Model 2”) | OP |  |  | NOTE | REL-8 |
| >Toa | MP |  | Integer(0..602112 by step of 4096) | Almanac Reference Time | REL-8 |
| >Satellite information NAV-KP | MP | 1 to <maxGANSSSat> |  |  | REL-8 |
| >>SV ID | MP |  | Integer (0..63) | Satellite ID, as defined in Note 2 of 10.3.7.88b. | REL-8 |
| >>e | MP |  | Bit String(16) | Eccentricity, dimensionless [72] | REL-8 |
| >>i | MP |  | Bit String(16) | Correction to inclination, semi-circles [72] | REL-8 |
| >>OMEGADOT | MP |  | Bit String(16) | Rate of right ascension, semi-circles/sec [72] | REL-8 |
| >>SV Health | MP |  | Bit String(8) | Satellite health [72] | REL-8 |
| >>A1/2 | MP |  | Bit String(24) | Square root of the semi-major axis, meters1/2 [72] | REL-8 |
| >>OMEGA0 | MP |  | Bit String(24) | Longitude of ascending node of orbit plane at weekly epoch, semi-circles [72] | REL-8 |
| >> | MP |  | Bit String(24) | Argument of perigee semi-circles [72] | REL-8 |
| >>M0 | MP |  | Bit String(24) | Mean anomaly at reference time semi-circles [72] | REL-8 |
| >>af0 | MP |  | Bit String(11) | Apparent satellite clock correction seconds [72] | REL-8 |
| >>af1 | MP |  | Bit String(11) | Apparent satellite clock correction sec/sec [72] | REL-8 |
| Reduced Keplerian Parameters (“Model 3”) | OP |  |  | NOTE | REL-8 |
| >Toa | MP |  | Integer(0..602112 by step of 4096) | Almanac Reference Time | REL-8 |
| >Satellite information RED-KP | MP | 1 to <maxGANSSSat> |  |  | REL-8 |
| >>SV ID | MP |  | Integer (0..63) | Satellite ID, as defined in Note 2 of 10.3.7.88b. | REL-8 |
| >>A | MP |  | Bit String(8) | meters [68,69,70,72] | REL-8 |
| >>0 | MP |  | Bit String(7) | semi-circles [68,69,70,72] | REL-8 |
| >>0 | MP |  | Bit String(7) | semi-circles [68,69,70,72] | REL-8 |
| >>L1 Health | MP |  | Bit String(1) | dimensionless [68,69,70,72] | REL-8 |
| >>L2 Health | MP |  | Bit String(1) | dimensionless [68,69,70,72] | REL-8 |
| >>L5 Health | MP |  | Bit String(1) | dimensionless [68,69,70,72] | REL-8 |
| Midi Keplerian Parameters (“Model 4”) | OP |  |  | NOTE | REL-8 |
| >Toa | MP |  | Integer(0..602112 by step of 4096) | Almanac Reference Time | REL-8 |
| >Satellite information MIDI-KP | MP | 1 to <maxGANSSSat> |  |  | REL-8 |
| >>SV ID | MP |  | Integer (0..63) | Satellite ID, as defined in Note 2 of 10.3.7.88b. | REL-8 |
| >>e | MP |  | Bit String(11) | dimensionless [68,69,70,72] | REL-8 |
| >>i | MP |  | Bit String(11) | semi-circles [68,69,70,72] | REL-8 |
| >>_dot | MP |  | Bit String(11) | semi-circles/sec [68,69,70,72] | REL-8 |
| >>sqrtA | MP |  | Bit String(17) | meters1/2 [68,69,70,72] | REL-8 |
| >>0 | MP |  | Bit String(16) | semi-circles [68,69,70,72] | REL-8 |
| >> | MP |  | Bit String(16) | semi-circles [68,69,70,72] | REL-8 |
| >>M0 | MP |  | Bit String(16) | semi-circles [68,69,70,72] | REL-8 |
| >>afo | MP |  | Bit String(11) | seconds [68,69,70,72] | REL-8 |
| >>af1 | MP |  | Bit String(10) | sec/sec [68,69,70,72] | REL-8 |
| >>L1 Health | MP |  | Bit String(1) | Dimensionless [68,69,70,72] | REL-8 |
| >>L2 Health | MP |  | Bit String(1) | dimensionless [68,69,70,72] | REL-8 |
| >>L5 Health | MP |  | Bit String(1) | dimensionless [68,69,70,72] | REL-8 |
| GLONASS Keplerian Parameters (“Model 5”) | OP |  |  | NOTE | REL-8 |
| >Satellite information GLO-KP | MP | 1 to <maxGANSSSat> |  |  | REL-8 |
| >>NA | MP |  | Bit String(11) | days [73] | REL-8 |
| >>nA | MP |  | Bit String(5) | dimensionless [73] | REL-8 |
| >>HnA | MP |  | Bit String(5) | dimensionless [73] | REL-8 |
| >>nA | MP |  | Bit String(21) | semi-circles [73] | REL-8 |
| >>tnA | MP |  | Bit String(21) | seconds [73] | REL-8 |
| >>inA | MP |  | Bit String(18) | semi-circles [73] | REL-8 |
| >>TnA | MP |  | Bit String(22) | sec/orbit period [73] | REL-8 |
| >>T_DOTnA | MP |  | Bit String(7) | sec/orbit period2 [73] | REL-8 |
| >>nA | MP |  | Bit String(15) | dimensionless [73] | REL-8 |
| >>nA | MP |  | Bit String(16) | semi-circles [73] | REL-8 |
| >>nA | MP |  | Bit String(10) | seconds [73] | REL-8 |
| >>CnA | MP |  | Bit String(1) | dimensionless [73] | REL-8 |
| >>MnA | OP |  | Bit String(2) | dimensionless [73] | REL-8 |
| SBAS ECEF Parameters (“Model 6”) | OP |  |  | NOTE | REL-8 |
| >Satellite information SBAS-ECEF | MP | 1 to <maxGANSSSat> |  |  | REL-8 |
| >>Data ID | MP |  | Bit String(2) | Dimensionless [71] |  |
| >>SV ID | MP |  | Integer (0..63) | Satellite ID, as defined in Note 2 of 10.3.7.88b. | REL-8 |
| >>Health | MP |  | Bit String(8) | Dimensionless [71] | REL-8 |
| >>XG | MP |  | Bit String(15) | meters [71] | REL-8 |
| >>YG | MP |  | Bit String(15) | meters [71] | REL-8 |
| >>ZG | MP |  | Bit String(9) | meters [71] | REL-8 |
| >>XG Rate-of-Change | MP |  | Bit String(3) | meters/sec [71] | REL-8 |
| >>YG Rate-of-Change | MP |  | Bit String(3) | meters/sec [71] | REL-8 |
| >>ZG Rate-of-Change | MP |  | Bit String(4) | meters/sec [71] | REL-8 |
| >>t0 | MP |  | Bit String(11) | seconds [71] | REL-8 |
| BDS Keplerian Parameters ("Model 7") | OP |  |  | NOTE | REL-12 |
| >Satellite information BDS-KP | MP | 1 to <maxGANSSSat> |  |  | REL-12 |
| >>SV ID | MP |  | INTEGER (0..63) | Satellite ID, as defined in Note 2 of 10.3.7.88b. | REL-12 |
| >>toa | MP |  | BIT STRING(8) | Almanac reference time (seconds) [86] | REL-12 |
| >>A1/2 | MP |  | BIT STRING(24) | Square root of semi-major axis (meters1/2) [86] | REL-12 |
| >>e | MP |  | BIT STRING(17) | Eccentricity , dimensionless [86] | REL-12 |
| >> | MP |  | BIT STRING(24) | Argument of Perigee (semi-circles) [86] | REL-12 |
| >>M0 | MP |  | BIT STRING(24) | Mean anomaly at reference time (semi-circles) [86] | REL-12 |
| >>0 | MP |  | BIT STRING(24) | Longitude of ascending node of orbital plane computed according to reference time (semi-circles) [86] | REL-12 |
| >>_dot | MP |  | BIT STRING(17) | Rate of right ascension (semi-circles/sec) [86] | REL-12 |
| >>i | MP |  | BIT STRING(16) | Correction of orbit reference inclination at reference time (semi-circles) [86] | REL-12 |
| >>a0 | MP |  | BIT STRING(11) | Satellite clock bias (seconds) [86] | REL-12 |
| >>a1 | MP |  | BIT STRING(11) | Satellite clock rate (sec/sec) [86] | REL-12 |
| >>Hea | CV-SV-ID |  | BIT STRING(9) | Satellite Health Information dimensionless [86] | REL-12 |

NOTE:	This IE shall contain exactly one of the optional fiels indicated by "NOTE".

| Condition | Explanation |
| --- | --- |
| SV-ID | This IE is mandatory present if the IE "SV ID" is between 0 and 29 and not needed otherwise |

#### 10.3.7.90	UE positioning GPS assistance data

This IE contains GPS assistance data.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE positioning GPS reference time | OP |  | UE positioning GPS reference time 10.3.7.96 |  |
| UE positioning GPS reference UE position | OP |  | Ellipsoid point with altitude and uncertainty ellipsoid 10.3.8.4c | A priori knowledge of UE 3-D position. |
| UE positioning GPS DGPS corrections | OP |  | UE positioning GPS DGPS corrections 10.3.7.91 |  |
| UE positioning GPS navigation model | OP |  | UE positioning GPS navigation model 10.3.7.94 |  |
| UE positioning GPS ionospheric model | OP |  | UE positioning GPS ionospheric model 10.3.7.92 |  |
| UE positioning GPS UTC model | OP |  | UE positioning GPS UTC model 10.3.7.97 |  |
| UE positioning GPS almanac | OP |  | UE positioning GPS almanac 10.3.7.89 |  |
| UE positioning GPS acquisition assistance | OP |  | UE positioning GPS acquisition assistance 10.3.7.88 |  |
| UE positioning GPS real-time integrity | OP |  | UE positioning GPS real-time integrity 10.3.7.95 |  |

#### 10.3.7.90a	Void

#### 10.3.7.90b	UE positioning GANSS assistance data

This IE contains GANSS assistance data.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UE positioning GANSS reference time | OP |  | UE positioning GANSS reference time 10.3.7.96o |  | REL-7 |
| UE positioning GANSS reference UE position | OP |  | Ellipsoid point with altitude and uncertainty ellipsoid 10.3.8.4c | A priori knowledge of UE 3-D position.If IE “UE positioning GPS assistance data” is present, this IE should not be included to both IEs. The reference UE position is provided in WGS-84 reference system. | REL-7 |
| UE positioning GANSS ionospheric model | OP |  | UE positioning GANSS ionospheric model 10.3.7.92a |  | REL-7 |
| UE positioning GANSS additional ionospheric model | OP |  | UE positioning GANSS additional ionospheric model 10.3.7.92b |  | REL-8 |
| UE positioning GANSS Earth orientation parameters | OP |  | UE positioning GANSS Earth orientation parameters 10.3.7.92c |  | REL-8 |
| GANSS Generic Assistance Data | OP | 1 to <maxGANSS> |  |  | REL-7 |
| >GANSS ID | OP |  | Integer(0..7) | Absence of this IE means Galileo Values 0-7 reserved for future use. | REL-7 |
|  |  |  |  | Absence of this IE means Galileo. For coding description see NOTE 1. | REL-8 |
| >UE positioning GANSS SBAS ID | CV-GANSS-ID-SBAS |  | UE positioning GANSS SBAS ID 10.3.7.97e |  | REL-8 |
| >GANSS Time Models | OP | 1 to <maxGANSS-1> |  |  | REL-7 |
| >>GANSS Time Model | MP |  | UE positioning GANSS time model 10.3.7.97a |  | REL-7 |
| >UE positioning DGANSS corrections | OP |  | UE positioning DGANSS corrections 10.3.7.91b |  | REL-7 |
| >UE positioning DBDS corrections | CV-GANSS-ID-BDS |  | UE positioning DBDS corrections 10.3.7.92e |  | REL-12 |
| >UE positioning BDS Ionospheric Grid Model | CV-GANSS-ID-BDS |  | UE positioning BDS Ionospheric Grid Model 10.3.7.92d |  | REL-12 |
| >UE positioning GANSS navigation model | OP |  | UE positioning GANSS navigation model 10.3.7.94a | NOTE | REL-7 |
| >UE positioning GANSS additional navigation models | OP |  | UE positioning GANSS additional navigation models 10.3.7.94b | NOTE | REL-8 |
| >UE positioning GANSS real-time integrity | OP |  | UE positioning GANSS real-time integrity 10.3.7.95b |  | REL-7 |
| >UE positioning GANSS data bit assistance | OP |  | UE positioning GANSS data bit assistance 10.3.7.97b |  | REL-7 |
| >UE positioning GANSS reference measurement information | OP |  | UE positioning GANSS reference measurement information 10.3.7.88b |  | REL-7 |
| >UE positioning GANSS almanac | OP |  | UE positioning GANSS almanac 10.3.7.89a |  | REL-7 |
| >UE positioning GANSS UTC model | OP |  | UE positioning GANSS UTC model 10.3.7.97c |  | REL-7 |
| >UE positioning GANSS additional UTC models | OP |  | UE positioning GANSS additional UTC models 10.3.7.97d |  | REL-8 |
| >UE positioning GANSS auxiliary information | OP |  | UE positioning GANSS auxiliary information 10.3.7.97f |  | REL-8 |

NOTE:	If included, only one of the optional fiels indicated by "NOTE" shall be present.

| Condition | Explanation |
| --- | --- |
| GANSS-ID-SBAS | The IE is mandatory present if the IE "GANSS ID" is "SBAS" and not needed otherwise. |
| GANSS-ID-BDS | The IE is optionally present if the IE "GANSS ID" is "BDS" and not needed otherwise. |

NOTE 1:	Coding of GANSS ID

| Value of GANSS ID | GANSS | Version |
| --- | --- | --- |
| 0 | SBAS | REL-8 |
| 1 | Modernized GPS |  |
| 2 | QZSS |  |
| 3 | GLONASS |  |
| 4 | BDS | REL-12 |
| 5-7 | Reserved for future use |  |

#### 10.3.7.91	UE positioning GPS DGPS corrections

This IE contains DGPS corrections to be used by the UE.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GPS TOW sec | MP |  | Integer(0..604799) | seconds GPS time-of-week when the DGPS corrections were calculated |  |
| Status/Health | MP |  | Enumerated(UDRE scale 1.0, UDRE scale 0.75, UDRE scale 0.5, UDRE scale 0.3, UDRE scale 0.2, UDRE scale 0.1, no data, invalid data) |  |  |
| DGPS information | CV-Status/Health | 1 to <maxSat> |  | If the Cipher information is included these fields are ciphered. |  |
| >SatID | MP |  | Enumerated (0…63) | Identifies the satellite and is equal to (SV ID No - 1) where SV ID No is defined in [12]. |  |
| >IODE | MP |  | Integer(0..255) |  |  |
| >UDRE | MP |  | Enumerated(UDRE 1.0 m, 1.0m < UDRE 4.0m, 4.0m < UDRE 8.0m, 8.0m < UDRE) | The value in this field shall be multiplied by the UDRE Scale Factor in the IE Status/Health to determine the final UDRE estimate for the particular satellite. |  |
| >PRC | MP |  | Real(-655.04..655.04 by step of 0.32) | meters (different from [13]) |  |
| >RRC | MP |  | Real(-4.064..4.064 by step of 0.032) | meters/sec (different from [13]) |  |
| >Delta PRC2 | MP |  | Integer(-127..127) | In this version of the protocol this IE should be set to zero and the UE shall ignore it |  |
| >Delta RRC2 | MP |  | Real(-0.224..0.224 by step of 0.032) | In this version of the protocol this IE should be set to zero and the UE shall ignore it |  |
| >Delta PRC3 | OP |  | Integer(-127..127) | This IE should not be included in this version of the protocol and if received the UE shall ignore it |  |
| >Delta RRC3 | OP |  | Real(-0.224..0.224 by step of 0.032) | This IE should not be included in this version of the protocol and if received the UE shall ignore it |  |
| >UDRE Growth Rate | OP |  | Enumerated(UDRE growth 1.5, UDRE growth 2, UDRE growth 4, UDRE growth 6, UDRE growth 8, UDRE growth 10, UDRE growth 12, UDRE growth 16) | This field provides an estimate of the growth rate of uncertainty (1-) in the corrections. The UDRE at time value specified in the Time of Validity for UDRE Growth Rate field is the value of this field times the value of UDRE. | REL-9 |
| >Time of Validity for UDRE Growth Rate | OP |  | Enumerated(val20sec, val40sec, val80sec, val160sec, val320sec, val640sec, val1280sec, val2560sec) | This field specifies the time when the UDRE Growth Rate field applies. | REL-9 |

| Condition | Explanation |
| --- | --- |
| Status/Health | This IE is mandatory present if "status" is not equal to "no data" or "invalid data", otherwise the IE is not needed. |

#### 10.3.7.91a	UE positioning GPS Ephemeris and Clock Correction parameters

This IE contains information for GPS ephemeris and clock correction.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| C/A or P on L2 | MP |  | Bit string(2) | Code(s) on L2 Channel [12] |
| URA Index | MP |  | Bit string(4) | User Range Accuracy [12] |
| SV Health | MP |  | Bit string(6) | [12] |
| IODC | MP |  | Bit string(10) | Issue of Data, Clock [12] |
| L2 P Data Flag | MP |  | Bit string(1) | [12] |
| SF 1 Reserved | MP |  | Bit string(87) | [12] |
| TGD | MP |  | Bit string(8) | Estimated group delay differential [12] |
| toc | MP |  | Bit string(16) | apparent clock correction [12] |
| af2 | MP |  | Bit string(8) | apparent clock correction [12] |
| af1 | MP |  | Bit string(16) | apparent clock correction [12] |
| af0 | MP |  | Bit string(22) | apparent clock correction [12] |
| Crs | MP |  | Bit string(16) | Amplitude of the Sine Harmonic Correction Term to the Orbit Radius (meters) [12] |
| n | MP |  | Bit string(16) | Mean Motion Difference From Computed Value (semi-circles/sec) [12] |
| M0 | MP |  | Bit string(32) | Mean Anomaly at Reference Time (semi-circles) [12] |
| Cuc | MP |  | Bit string(16) | Amplitude of the Cosine Harmonic Correction Term To The Argument Of Latitude (radians) [12] |
| e | MP |  | Bit string(32) | c |
| Cus | MP |  | Bit string(16) | Amplitude of the Sine Harmonic Correction Term To The Argument Of Latitude (radians) [12] |
| (A)1/2 | MP |  | Bit string(32) | Semi-Major Axis (meters)1/2 [12] |
| toe | MP |  | Bit string(16) | Reference Time Ephemeris [12] |
| Fit Interval Flag | MP |  | Bit string(1) | [12] |
| AODO | MP |  | Bit string(5) | Age Of Data Offset [12] |
| Cic | MP |  | Bit string(16) | Amplitude of the Cosine Harmonic Correction Term To The Angle Of Inclination (radians) [12] |
| OMEGA0 | MP |  | Bit string(32) | Longitude of Ascending Node of Orbit Plane at Weekly Epoch (semi-circles) [12] |
| Cis | MP |  | Bit string(16) | Amplitude of the Sine Harmonic Correction Term To The Angle Of Inclination (radians) [12] |
| i0 | MP |  | Bit string(32) | Inclination Angle at Reference Time (semi-circles) [12] |
| Crc | MP |  | Bit string(16) | Amplitude of the Cosine Harmonic Correction Term to the Orbit Radius (meters) [12] |
|  | MP |  | Bit string(32) | Argument of Perigee (semi-circles) [12] |
| OMEGAdot | MP |  | Bit string(24) | Longitude of Ascending Node of Orbit Plane at Weekly Epoch (semi-circles/sec) [12] |
| Idot | MP |  | Bit string(14) | Rate of Inclination Angle (semi-circles/sec) [12] |

#### 10.3.7.91b	UE positioning DGANSS corrections

This IE contains DGANSS corrections to be used by the UE.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| DGANSS Reference TIme | MP |  | Integer(0..3570 by step of 30) | Seconds. Time in GNSS system time (modulo 3600 s) when the DGANSS corrections were calculated | REL-7 |
| DGANSS information | MP | 1 to <maxSgnType> |  |  | REL-7 |
| >GANSS Signal ID | OP |  | GANSS Signal Id 10.3.3.45a | Absence of this field means the default value as defined in 10.3.3.45a for the GANSS identified by GANSS_ID. | REL-7 |
| >Status/Health | MP |  | Enumerated(UDRE scale 1.0, UDRE scale 0.75, UDRE scale 0.5, UDRE scale 0.3, UDRE scale 0.2, UDRE scale 0.1, no data, invalid data) |  | REL-7 |
| >DGANSS signal information | CV-Status/Health | 1 to <maxGANSSSat> |  | If the Cipher information is included these fields are ciphered. | REL-7 |
| >>SatID | MP |  | Integer (0…63) | Identifies the satellite and is as defined in Note 2 of 10.3.7.88b. | REL-7 |
| >>IOD | MP |  | Bit string(10) | 10 LSBs of Issue of Data field, which contains the identity of the GANSS Navigation Model. | REL-7 |
| >>UDRE | MP |  | Enumerated(UDRE 1.0 m, 1.0m < UDRE 4.0m, 4.0m < UDRE 8.0m, 8.0m < UDRE) | The value in this field shall be multiplied by the UDRE Scale Factor in the IE Status/Health to determine the final UDRE estimate for the particular satellite. | REL-7 |
| >>PRC | MP |  | Real(-655.04..655.04 by step of 0.32) | meters Pseudo-range corrections are provided with respect to GNSS specific geodetic datum (e.g., PZ-90.02 if GANSS ID indicates GLONASS). | REL-7 |
| >>RRC | MP |  | Real(-4.064..4.064 by step of 0.032) | meters/sec Pseudo-range rate corrections are provided with respect to GNSS specific geodetic datum (e.g., PZ-90.02 if GANSS ID indicates GLONASS). | REL-7 |
| >>UDRE Growth Rate | OP |  | Enumerated(UDRE growth 1.5, UDRE growth 2, UDRE growth 4, UDRE growth 6, UDRE growth 8, UDRE growth 10, UDRE growth 12, UDRE growth 16) | This field provides an estimate of the growth rate of uncertainty (1-) in the corrections. The UDRE at time value specified in the Time of Validity for UDRE Growth Rate field is the value of this field times the value of UDRE. | REL-9 |
| >>Time of Validity for UDRE Growth Rate | OP |  | Enumerated(val20sec, val40sec, val80sec, val160sec, val320sec, val640sec, val1280sec, val2560sec) | This field specifies the time when the UDRE Growth Rate field applies. | REL-9 |

| Condition | Explanation |
| --- | --- |
| Status/Health | This IE is mandatory present if "status" is not equal to "no data" or "invalid data", otherwise the IE is not needed. |

#### 10.3.7.91c	UE positioning GANSS orbit model

This IE contains information for GANSS orbit model parameters.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Keplerian Parameters (“Model 1”) | OP |  |  | NOTE | REL-7 |
| >toe | MP |  | Bit String(14) | Time-of-Ephemeris in seconds, scale factor 60 [61] | REL-7 |
|  | MP |  | Bit string(32) | Argument of Perigee (semi-circles) [61] | REL-7 |
| n | MP |  | Bit string(16) | Mean Motion Difference From Computed Value (semi-circles/sec) [61] | REL-7 |
| >M0 | MP |  | Bit string(32) | Mean Anomaly at Reference Time (semi-circles) [61] | REL-7 |
| >OMEGAdot | MP |  | Bit string(24) | Rate of change of right ascension (semi-circles/sec) [61] | REL-7 |
| >e | MP |  | Bit string(32) | Eccentricity, scale factor 2-33 [61] | REL-7 |
| >Idot | MP |  | Bit string(14) | Rate of change of Inclination Angle (semi-circles/sec) [61] | REL-7 |
| >sqrtA | MP |  | Bit String(32) | Square root of Semi-Major Axis in (meters)1/2 , scale factor 2-19 [61] | REL-7 |
| >i0 | MP |  | Bit string(32) | Inclination Angle at Reference Time (semi-circles) [61] | REL-7 |
| >OMEGA0 | MP |  | Bit string(32) | Longitude of Ascending Node of Orbit Plane at Weekly Epoch (semi-circles) [61] | REL-7 |
| >Crs | MP |  | Bit string(16) | Amplitude of the Sine Harmonic Correction Term to the Orbit Radius (meters) [61] | REL-7 |
| >Cis | MP |  | Bit string(16) | Amplitude of the Sine Harmonic Correction Term To The Angle Of Inclination (radians) [61] | REL-7 |
| >Cus | MP |  | Bit string(16) | Amplitude of the Sine Harmonic Correction Term To The Argument Of Latitude (radians) [61] | REL-7 |
| >Crc | MP |  | Bit string(16) | Amplitude of the Cosine Harmonic Correction Term to the Orbit Radius (meters) [61] | REL-7 |
| >Cic | MP |  | Bit string(16) | Amplitude of the Cosine Harmonic Correction Term To The Angle Of Inclination (radians) [61] | REL-7 |
| >Cuc | MP |  | Bit string(16) | Amplitude of the Cosine Harmonic Correction Term To The Argument Of Latitude (radians) [61] | REL-7 |

NOTE:	This IE shall contain exactly one of the optional fiels indicated by "NOTE".

#### 10.3.7.91d		UE positioning GANSS clock model

The IE contains fields needed to model the GANSS clock parameters.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Satellite clock model (“Model 1”) | MP | 1 to <maxSatClockModels> |  | There may be more than one clock model included if defined in SIS ICD (i.e., two for Galileo) [61]. | REL-7 |
| >toc | MP |  | BIT STRING(14) | Clock correction data reference Time of Week in seconds. Scale factor 60 seconds [61]. | REL-7 |
| >af2 | MP |  | BIT STRING (6) | SV clock drift rate correction coefficient, sec/sec2, scale factor 2-59 [61]. | REL-12 |
| >af1 | MP |  | BIT STRING (21) | Parameter af1, SV clock drift correction coefficient, sec/sec, scale factor 2-46 [61]. | REL-12 |
| >af0 | MP |  | BIT STRING (31) | Parameter af0, SV clock bias correction coefficient, sec, scale factor 2-34 [61]. | REL-12 |
| >TGD | OP |  | BIT STRING (10) | Broadcast Group Delay (BGD), sec, scale factor 2-32 [61]. | REL-7 |
| >SISA | MP |  | BIT STRING (8) | Signal-In-Space Accuracy (SISA), defined in [61] section 5.1.11. | REL-12 |
| >Model ID | OP |  | Integer(0..1) | NOTE 1 | REL-7 |

NOTE 1:	Coding of Model ID:

| GNSS | Value | Explanation |
| --- | --- | --- |
| Galileo | 0 | I/Nav (E1,E5b) |
|  | 1 | F/Nav (E1,E5a) |

#### 10.3.7.91e	UE positioning GANSS additional orbit models

This IE contains information for GANSS orbit model parameters.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| NAV-Keplerian Parameters (“Model 2”) | OP |  |  | NOTE | REL-8 |
| >URA Index | MP |  | Bit String(4) | SV accuracy (dimensionless) [72] | REL-8 |
| >Fit Interval Flag | MP |  | Bit String(1) | Fit interval indication (dimensionless) [72] | REL-8 |
| >toe | MP |  | Bit String(16) | Time of ephemeris (seconds) [72] | REL-8 |
| > | MP |  | Bit String(32) | Argument of perigee (semi-circles) [72] | REL-8 |
| >n | MP |  | Bit String(16) | Mean motion difference from computed value (semi-circles/sec) [72] | REL-8 |
| >M0 | MP |  | Bit String(32) | Mean anomaly at reference time (semi-circles) [72] | REL-8 |
| >OMEGAdot | MP |  | Bit String(24) | Rate of right ascension (semi-circles/sec) [72] | REL-8 |
| >e | MP |  | Bit String(32) | Eccentricity (dimensionless) [72] | REL-8 |
| >Idot | MP |  | Bit String(14) | Rate of inclination angle (semi-circles/sec) [72] | REL-8 |
| >sqrtA | MP |  | Bit String(32) | Square root of semi-major axis (meters1/2) [72] | REL-8 |
| >i0 | MP |  | Bit String(32) | Inclination angle at reference time (semi-circles) [72] | REL-8 |
| >OMEGA0 | MP |  | Bit String(32) | Longitude of ascending node of orbit plane at weekly epoch (semi-circles) [72] | REL-8 |
| >Crs | MP |  | Bit String(16) | Amplitude of sine harmonic correction term to the orbit radius (meters) [72] | REL-8 |
| >Cis | MP |  | Bit String(16) | Amplitude of sine harmonic correction term to the angle of inclination (radians) [72] | REL-8 |
| >Cus | MP |  | Bit String(16) | Amplitude of sine harmonic correction term to the argument of latitude (radians) [72] | REL-8 |
| >Crc | MP |  | Bit String(16) | Amplitude of cosine harmonic correction term to the orbit radius (meters) [72] | REL-8 |
| >Cic | MP |  | Bit String(16) | Amplitude of cosine harmonic correction term to the angle of inclination (radians) [72] | REL-8 |
| >Cuc | MP |  | Bit String(16) | Amplitude of cosine harmonic correction term to the argument of latitude (radians) [72] | REL-8 |
| CNAV/CNAV-2 Keplerian Parameters (“Model 3”) | OP |  |  | NOTE | REL-8 |
| >top | MP |  | Bit String(11) | Data predict time of week (seconds) [68,69,70,72] | REL-8 |
| >URAoe Index | MP |  | Bit String(5) | SV accuracy (dimensionless) [68,69,70,72] | REL-8 |
| >A | MP |  | Bit String(26) | Semi-major axis difference at reference time (meters) [68,69,70,72] | REL-8 |
| >A_dot | MP |  | Bit String(25) | Chane rate in semi-major axis (meters/sec) [68,69,70,72] | REL-8 |
| >n0 | MP |  | Bit String(17) | Mean motion difference from computed value at reference time (semi-circles/sec) [68,69,70,72] | REL-8 |
| >n0_dot | MP |  | Bit String(23) | Rate of mean motion difference from computed value (semi-circles/sec2) [68,69,70,72] | REL-8 |
| >M0-n | MP |  | Bit String(33) | Mean anomaly at reference time (semi-circles) [68,69,70,72] | REL-8 |
| >en | MP |  | Bit String(33) | Eccentricity (dimensionless) [68,69,70,72] | REL-8 |
| >n | MP |  | Bit String(33) | Argument of perigee (semi-circles) [68,69,70,72] | REL-8 |
| >0-n | MP |  | Bit String(33) | Reference right ascension angle (semi-circles) [68,69,70,72] | REL-8 |
| >_dot | MP |  | Bit String(17) | Rate of right ascension difference (semi-circles/sec) [68,69,70,72] | REL-8 |
| >io-n | MP |  | Bit String(33) | Inclination angle at reference time (semi-circles) [68,69,70,72] | REL-8 |
| >I0-n_dot | MP |  | Bit String(15) | Rate of inclination angle (semi-circles/sec) [68,69,70,72] | REL-8 |
| >Cis-n | MP |  | Bit String(16) | Amplitude of sine harmonic correction term to the angle of inclination (radians) [68,69,70,72] | REL-8 |
| >Cic-n | MP |  | Bit String(16) | Amplitude of cosine harmonic correction term to the angle of inclination (radians) [68,69,70,72] | REL-8 |
| >Crs-n | MP |  | Bit String(24) | Amplitude of sine harmonic correction term to the orbit radius (meters) [68,69,70,72] | REL-8 |
| >Crc-n | MP |  | Bit String(24) | Amplitude of cosine harmonic correction term to the orbit radius (meters) [68,69,70,72] | REL-8 |
| >Cus-n | MP |  | Bit String(21) | Amplitude of sine harmonic correction term to the argument of latitude (radians) [68,69,70,72] | REL-8 |
| >Cuc-n | MP |  | Bit String(21) | Amplitude of cosine harmonic correction term to the argument of latitude (radians) [68,69,70,72] | REL-8 |
| GLONASS Earth-Centered, Earth-fixed Parameters (“Model 4”) | OP |  |  | NOTE | REL-8 |
| >En | MP |  | Bit String(5) | Age of data (days) [73] | REL-8 |
| >P1 | MP |  | Bit String(2) | Time interval between two adjacent values of tb (minutes) [73] | REL-8 |
| >P2 | MP |  | Bit String(1) | Change of tb flag (dimensionless) [73] | REL-8 |
| >M | OP |  | Bit String(2) | Type of satellite (dimensionless) [73] | REL-8 |
| > | MP |  | Bit String(27) | x-coordinate of satellite at time tb (kilometers) [73] | REL-8 |
| > | MP |  | Bit String(24) | x-coordinate of satellite velocity at time tb (kilometers/sec) [73] | REL-8 |
| > | MP |  | Bit String(5) | x-coordinate of satellite acceleration at time tb (kilometers/sec2) [73] | REL-8 |
| > | MP |  | Bit String(27) | y-coordinate of satellite at time tb (kilometers) [73] | REL-8 |
| > | MP |  | Bit String(24) | y-coordinate of satellite velocity at time tb (kilometers/sec) [73] | REL-8 |
| > | MP |  | Bit String(5) | y-coordinate of satellite acceleration at time tb (kilometers/sec2) [73] | REL-8 |
| > | MP |  | Bit String(27) | z-coordinate of satellite at time tb (kilometers) [73] | REL-8 |
| > | MP |  | Bit String(24) | z-coordinate of satellite velocity at time tb (kilometers/sec) [73] | REL-8 |
| > | MP |  | Bit String(5) | z-coordinate of satellite acceleration at time tb (kilometers/sec2) [73] | REL-8 |
| SBAS Earth-Centered, Earth-fixed Parameters (“Model 5”) | OP |  |  | NOTE | REL-8 |
| >t0 | CV-ClockModel |  | Bit String(13) | Time of applicability (seconds) [71] | REL-8 |
| >Accuracy | MP |  | Bit String(4) | (dimensionless) [71] | REL-8 |
| >XG | MP |  | Bit String(30) | (meters) [71] | REL-8 |
| >YG | MP |  | Bit String(30) | (meters) [71] | REL-8 |
| >ZG | MP |  | Bit String(25) | (meters) [71] | REL-8 |
| >XG Rate-of-Change | MP |  | Bit String(17) | (meters/sec) [71] | REL-8 |
| >YG Rate-of-Change | MP |  | Bit String(17) | (meters/sec) [71] | REL-8 |
| >ZG Rate-of-Change | MP |  | Bit String(18) | (meters/sec) [71] | REL-8 |
| >XG Acceleration | MP |  | Bit String(10) | (meters/sec2) [71] | REL-8 |
| >YG Acceleration | MP |  | Bit String(10) | meters/sec2) [71] | REL-8 |
| >ZG Acceleration | MP |  | Bit String(10) | meters/sec2) [71] | REL-8 |
| BDS Keplerian Parameters ("Model 6") | OP |  |  | NOTE | REL-12 |
| >AODE | MP |  | Bit String(5) | Age of data,ephemeris (dimensionless) [86] | REL-12 |
| >URA Index | MP |  | Bit String(4) | SV accuracy (dimensionless) [86] | REL-12 |
| >toe | MP |  | BIT STRING (17) | Ephemeris reference time (seconds) [86] | REL-12 |
| >A1/2 | MP |  | BIT STRING (32) | Square root of semi-major axis (meters1/2) [86] | REL-12 |
| >e | MP |  | BIT STRING (32) | Eccentricity (dimensionless) [86] | REL-12 |
| > | MP |  | BIT STRING (32) | Argument of perigee (semi-circles) [86] | REL-12 |
| >n | MP |  | BIT STRING (16) | Mean motion difference from computed value (semi-circles/sec) [86] | REL-12 |
| >M0 | MP |  | BIT STRING (32) | Mean anomaly at reference time (semi-circles) [86] | REL-12 |
| >0 | MP |  | BIT STRING (32) | Longitude of ascending node of orbital of plane computed according to reference time (semi-circles) [86] | REL-12 |
| >_dot | MP |  | BIT STRING (24) | Rate of right ascension (semi-circles/sec) [86] | REL-12 |
| >i0 | MP |  | BIT STRING (32) | Inclination angle at reference time (semi-circles) [86] | REL-12 |
| >Idot | MP |  | BIT STRING (14) | Rate of inclination angle (semi-circles/sec) [86] | REL-12 |
| >Cuc | MP |  | BIT STRING (18) | Amplitude of cosine harmonic correction term to the argument of latitude (radians) [86] | REL-12 |
| >Cus | MP |  | BIT STRING (18) | Amplitude of sine harmonic correction term to the argument of latitude (radians) [86] | REL-12 |
| >Crc | MP |  | BIT STRING (18) | Amplitude of cosine harmonic correction term to the orbit radius (meters) [86] | REL-12 |
| >Crs | MP |  | BIT STRING (18) | Amplitude of sine harmonic correction term to the orbit radius (meters) [86] | REL-12 |
| >Cic | MP |  | BIT STRING (18) | Amplitude of cosine harmonic correction term to the angle of inclination (radians) [86] | REL-12 |
| >Cis | MP |  | BIT STRING (18) | Amplitude of sine harmonic correction term to the angle of inclination (radians) [86] | REL-12 |

NOTE:	This IE shall contain exactly one of the optional fiels indicated by "NOTE".

| Condition | Explanation |
| --- | --- |
| ClockModel | This IE is mandatory present if UE positioning GANSS additional clock models “Model 5” is not included in UE positioning GANSS additional navigation models, otherwise the IE is not needed. |

#### 10.3.7.91f		UE positioning GANSS additional clock models

The IE contains fields needed to model the GANSS clock parameters.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| NAV-Clock Model (“Model 2”) | OP |  |  | NOTE | REL-8 |
| >toc | MP |  | Bit String(16) | Time of clock (seconds) [72] | REL-8 |
| >af2 | MP |  | Bit String(8) | Clock correction polynomial coefficient (sec/sec2) [72] | REL-8 |
| >af1 | MP |  | Bit String(16) | Clock correction polynomial coefficient (sec/sec) [72] | REL-8 |
| >af0 | MP |  | Bit String(22) | Clock correction polynomial coefficient (seconds) [72] | REL-8 |
| >TGD | MP |  | Bit String(8) | Group delay (seconds) [72] | REL-8 |
| CNAV/CNAV-2 Clock Model (“Model 3”) | OP |  |  | NOTE | REL-8 |
| >toc | MP |  | Bit String(11) | Clock data reference time of week (seconds) [68, 69, 70, 72] | REL-8 |
| >top | MP |  | Bit String(11) | Clock data predict time of week (seconds) [68, 69, 70, 72] | REL-8 |
| >URAoc Index | MP |  | Bti String(5) | SV clock accuracy index (dimensionless) [68, 69, 70, 72] | REL-8 |
| >URAoc1 Index | MP |  | Bit String(3) | SV clock accuracy change index (dimensionless) [68, 69, 70, 72] | REL-8 |
| >URAoc2 Index | MP |  | Bit String(3) | SV clock accuracy change rate index (dimensionless) [68, 69, 70, 72] | REL-8 |
| >af2-n | MP |  | Bit String(10) | SV clock drift rate correction coefficient (sec/sec2) [68, 69, 70, 72] | REL-8 |
| >af1-n | MP |  | Bit String(20) | SV clock drift correction coefficient (sec/sec) [68, 69, 70, 72] | REL-8 |
| >af0-n | MP |  | Bit String(26) | SV clock bias correction coefficient (seconds) [68, 69, 70, 72] | REL-8 |
| >TGD | MP |  | Bit String(13) | Group delay correction (seconds) [68, 69, 70, 72] | REL-8 |
| >ISCL1CP | OP |  | Bit String(13) | Inter signal group delay correction (seconds) [70, 72] | REL-8 |
| >ISCL1CD | OP |  | Bit String(13) | Inter signal group delay correction (seconds) [70, 72] | REL-8 |
| >ISCL1C/A | OP |  | Bit String(13) | Inter signal group delay correction (seconds) [68, 69, 72] | REL-8 |
| >ISCL2C | OP |  | Bit String(13) | Inter signal group delay correction (seconds) [68, 69, 72] | REL-8 |
| >ISCL5I5 | OP |  | Bit String(13) | Inter signal group delay correction (seconds) [69, 72] | REL-8 |
| >ISCL5Q5 | OP |  | Bit String(13) | Inter signal group delay correction (seconds) [69, 72] | REL-8 |
| GLONASS Satellite Clock Model (“Model 4”) | OP |  |  | NOTE | REL-8 |
| n(tb) | MP |  | Bit String(22) | Satellite clock offset (seconds) [73] | REL-8 |
| n(tb) | MP |  | Bit String(11) | Relative frequency offset from nominal value (dimensionless) [73] | REL-8 |
| n | OP |  | Bit String(5) | Time difference between transmission in G2 and G1 (seconds) [73] | REL-8 |
| SBAS Satellite Clock Model (“Model 5”) | OP |  |  | NOTE | REL-8 |
| >t0 | MP |  | Bit String(13) | (seconds) [71] | REL-8 |
| >aGfo | MP |  | Bit String(12) | (seconds) [71] | REL-8 |
| >aGf1 | MP |  | Bit String(8) | (sec/sec) [71] | REL-8 |
| BDS Satellite Clock Model ("Model 6") | OP |  |  | NOTE | REL-12 |
| >AODC | MP |  | BIT STRING (5) | Age of data,clock (dimensionless) [86] | REL-12 |
| >toc | MP |  | BIT STRING (17) | Time of clock (seconds) [86] | REL-12 |
| >a0 | MP |  | BIT STRING (24) | Clock correction polynomial coefficient (seconds) [86] | REL-12 |
| >a1 | MP |  | BIT STRING (22) | Clock correction polynomial coefficient (sec/sec) [86] | REL-12 |
| >a2 | MP |  | BIT STRING (11) | Clock correction polynomial coefficient (sec/sec2) [86] | REL-12 |
| >TGD1 | MP |  | BIT STRING (10) | Equipment Group Delay Differential (seconds) [86] | REL-12 |

NOTE:	This IE shall contain exactly one of the optional fiels indicated by "NOTE".

#### 10.3.7.92	UE positioning GPS ionospheric model

The IE contains fields needed to model the propagation delays of the GPS signals through the ionosphere.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| 0 | MP |  | Bit string(8) | Note 1 |
| 1 | MP |  | Bit string(8) | Note 1 |
| 2 | MP |  | Bit string(8) | Note 1 |
| 3 | MP |  | Bit string(8) | Note 1 |
| 0 | MP |  | Bit string(8) | Note 2 |
| 1 | MP |  | Bit string(8) | Note 2 |
| 2 | MP |  | Bit string(8) | Note 2 |
| 3 | MP |  | Bit string(8) | Note 2 |

NOTE 1:	The parameters n are the coefficients of a cubic equation representing the amplitude of the vertical delay [12].

NOTE 2:	The parameters n are the coefficients of a cubic equation representing the period of the ionospheric model [12].

#### 10.3.7.92a	UE positioning GANSS ionospheric model

The IE contains fields needed to model the propagation delays of the GANSS signals through the ionosphere.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| ai0 | MP |  | Bit string(11) | Effective Ionisation Level 1st order parameter. This parameter is used as defined in [61] | REL-12 |
| ai1 | MP |  | Bit string(11) | Effective Ionisation Level 2nd order parameter. This parameter is used as defined in [61] | REL-12 |
| ai2 | MP |  | Bit string(14) | Effective Ionisation Level 3rd order parameter. This parameter is used as defined in [61] | REL-12 |
| GANSS Ionosphere Regional Storm Flags | OP |  |  |  | REL-7 |
| >Storm Flag 1 | MP |  | Boolean | This parameter is used as defined in [61] | REL-7 |
| >Storm Flag 2 | MP |  | Boolean | This parameter is used as defined in [61] | REL-7 |
| >Storm Flag 3 | MP |  | Boolean | This parameter is used as defined in [61] | REL-7 |
| >Storm Flag 4 | MP |  | Boolean | This parameter is used as defined in [61] | REL-7 |
| >Storm Flag 5 | MP |  | Boolean | This parameter is used as defined in [61] | REL-7 |

#### 10.3.7.92b	UE positioning GANSS additional ionospheric model

This IE contains parameters to model the propagation delays of the GANSS signals through the ionosphere.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Data ID | MP |  | Bit String(2) | The value ‘11’ indicates that the parameters have been generated by QZSS, and the parameters have been specialized and are applicable within the area defined in [72]. The value ‘01’ indicates that the parameters have been generated by BDS, and UE shall use these parameters according to the description given in 5.2.4.7 in [86]. When Data ID has the value ‘00’ it indicates the parameters are applicable worldwide [72]. All other values for Data ID are reserved. | REL-8 |
| 0 | MP |  | Bit string(8) | seconds [72] | REL-8 |
| 1 | MP |  | Bit string(8) | sec/semi-circle [72] | REL-8 |
| 2 | MP |  | Bit string(8) | sec/(semi-circle)2 [72] | REL-8 |
| 3 | MP |  | Bit string(8) | sec/(semi-circle)3 [72] | REL-8 |
| 0 | MP |  | Bit string(8) | seconds [72] | REL-8 |
| 1 | MP |  | Bit string(8) | sec/semi-circle [72] | REL-8 |
| 2 | MP |  | Bit string(8) | sec/(semi-circle)2 [72] | REL-8 |
| 3 | MP |  | Bit string(8) | sec/(semi-circle)3 [72] | REL-8 |

#### 10.3.7.92c	UE positioning GANSS Earth orientation parameters

This IE  provides parameters to construct the ECEF and ECI coordinate transformation as defined in [68]. The Earth Orientation Parameters (EOP) indicate the relationship between the Earth’s rotational axis and WGS-84 reference system.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| tEOP | MP |  | Bit String(16) | EOP data reference time (seconds) [68] | REL-8 |
| PM_X | MP |  | Bit String(21) | X-axis polar motion value at reference time (arc-seconds) [68] | REL-8 |
| PM_X_dot | MP |  | Bit String(15) | X-axis polar motion drift at reference time (arc-seconds/day) [68] | REL-8 |
| PM_Y | MP |  | Bit String(21) | Y-axis polar motion value at reference time (arc-seconds) [68] | REL-8 |
| PM_Y_dot | MP |  | Bit String(15) | Y-axis polar motion drift at reference time (arc-seconds/day) [68] | REL-8 |
| UT1 | MP |  | Bit String(31) | UT1-UTC difference at reference time (seconds) [68] | REL-8 |
| UT1_dot | MP |  | Bit String(19) | Rate of UT1-UTC difference at reference time (seconds/day) [68] | REL-8 |

#### 10.3.7.92d	UE positioning BDS Ionospheric Grid Model

This IE contains BDS Ionospheric Grid information to calculate the propagation delays of the B1I signal through the ionosphere.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| BDS Reference Time | MP |  | Integer(0..3570 by step of 30) | Seconds. Time in BDS system time (modulo 3600 s) when the BDS Ionospheric Grid Information is valid. | REL-12 |
| BDS Ionospheric Grid Information | MP | 1 to <maxIGPInfo> |  | Ionospheric information for upto 16 grid points will be included in this version of the specification. | REL-12 |
| >IGP number | MP |  | INTEGER (1..320) | Ionospheric grid point number (dimensionless) [86]. | REL-12 |
| >Vertical Delay | MP |  | BIT STRING (9) | Vertical Delay at Ionospheric Grid Points ,(meters) [86] | REL-12 |
| >GIVEI | MP |  | BIT STRING (4) | Grid Ionospheric Vertical Error Index (dimensionless) [86]. | REL-12 |

#### 10.3.7.92e	UE positioning DBDS corrections

This IE contains the BDS differential corrections to be used by UE.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| DBDS Reference Time | MP |  | Integer(0..3570 by step of 30) | Seconds. Time in BDS system time (modulo 3600 s) when the DBDS corrections are valid. | REL-12 |
| DBDS information | MP | 1 to <maxSgnType> |  |  | REL-12 |
| >DBDS Signal ID | OP |  | GANSS Signal Id 10.3.3.45a | Absence of this field means the B1I | REL-12 |
| >DBDS signal information | MP | 1 to <maxGANSSSat> |  |  | REL-12 |
| >>SatID | MP |  | Integer (0…63) | Identifies the satellite and is as defined in Note 2 of 10.3.7.88b. |  |
| >>UDREI | MP |  | Integer (0…15) | User Differential Range Error Index (dimensionless) [86]. | REL-12 |
| >>RURAI | MP |  | Integer (0…15) | BDS Regional User Range Accuracy Index, (dimensionless) [86]. | REL-12 |
| >>Δt | MP |  | BIT STRING (13) | Equivalent Clock Correction (meters) [86] | REL-12 |

#### 10.3.7.93	UE positioning GPS measured results

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Reference Time | MP |  |  |  |  |
| >UTRAN reference time |  |  |  |  |  |
| >>UE GPS timing of cell frames | MP |  | Integer(0.. 37158911999999 | GPS Time of Week in units of 1/16th UMTS chips according to [19]. 33209832177664 spare values are needed. |  |
| >>CHOICE mode | MP |  |  |  |  |
| >>>FDD |  |  |  |  |  |
| >>>>Primary CPICH Info | MP |  | Primary CPICH Info 10.3.6.60 | Identifies the reference cell for the GPS TOW-SFN relationship. |  |
| >>>TDD |  |  |  |  |  |
| >>>>cell parameters id | MP |  | Cell parameters id 10.3.6.9 | Identifies the reference cell for the GPS TOW-SFN relationship. |  |
| >>Reference SFN | MP |  | Integer(0..4095) | The SFN for which the location is valid. This IE indicates the SFN at which the UE timing of cell frames is captured. |  |
| >GPS reference time only |  |  |  |  |  |
| >>GPS TOW msec | MP |  | Integer(0..6.048*108-1) | GPS Time of Week in milliseconds (rounded down to the nearest millisecond unit). |  |
| UE Positioning GPS ReferenceTime Uncertainty | OP |  | UE positioning GPS reference time uncertainty 10.3.7.96a |  | REL-7 |
| Measurement Parameters | MP | 1 to <maxSat> |  |  |  |
| >Satellite ID | MP |  | Enumerated(0..63) | Identifies the satellite and is equal to (SV ID No - 1) where SV ID No is defined in [12]. |  |
| >C/No | MP |  | Integer(0..63) | the estimate of the carrier-to-noise ratio of the received signal from the particular satellite used in the measurement. It is given in units of dB-Hz (typical levels will be in the range of 20 – 50 dB-Hz). |  |
| >Doppler | MP |  | Integer(-32768..32768) | Hz, scale factor 0.2. |  |
| >Whole GPS Chips | MP |  | Integer(0..1022) | Unit in GPS chips. Whole value of the UE GPS code-phase measurement, where increasing binary values of the field signify increasing measured pseudoranges.The UE GPS code-phase measurement is divided into the fields “Whole GPS Chips” and “Fractional GPS Chips”. |  |
| >Fractional GPS Chips | MP |  | Integer(0..(210-1)) | Scale factor 2-10 Fractional value of the UE GPS code-phase measurement. |  |
| >Multipath Indicator | MP |  | Enumerated(NM, low, medium, high) | Note 1. |  |
| >Pseudorange RMS Error | MP |  | Enumerated(range index 0..range index 63) | Note 2. |  |

NOTE 1:	The following table gives the mapping of the multipath indicator field.

| Value | Multipath Indication |
| --- | --- |
| NM | Not measured |
| Low | MP error < 5m |
| Medium | 5m < MP error < 43m |
| High | MP error > 43m |

NOTE 2:	The following table gives the bitmapping of the Pseudorange RMS Error field.

| Range Index | Mantissa | Exponent | Floating-Point value, xi | Pseudorange value, P |
| --- | --- | --- | --- | --- |
| 0 | 000 | 000 | 0.5 | P < 0.5 |
| 1 | 001 | 000 | 0.5625 | 0.5 <= P < 0.5625 |
| I | X | Y | 0.5 * (1 + x/8) * 2y | xi-1 <= P < xi |
| 62 | 110 | 111 | 112 | 104 <= P < 112 |
| 63 | 111 | 111 | -- | 112 <= P |

#### 10.3.7.93a		UE positioning GANSS measured results

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Reference Time | MP |  |  |  | REL-7 |
| >UTRAN reference time |  |  |  |  | REL-7 |
| >>UE GANSS timing of cell frames | MP |  | Integer(0.. 86399999999750 by step of 250) | GANSS Time of Day in ns | REL-7 |
| >>GANSS Time Id | OP |  | INTEGER (0..7) | Absence means Galileo. For coding description see NOTE 2. |  |
| >>GANSS TOD Uncertainty | OP |  | Integer(0..127) | Coding as in 10.3.7.96a | REL-7 |
| >>CHOICE mode | MP |  |  |  | REL-7 |
| >>>FDD |  |  |  |  | REL-7 |
| >>>>Primary CPICH Info | MP |  | Primary CPICH Info 10.3.6.60 | Identifies the reference cell for the GANSS TOD-SFN relationship. | REL-7 |
| >>>TDD |  |  |  |  | REL-7 |
| >>>>cell parameters id | MP |  | Cell parameters id 10.3.6.9 | Identifies the reference cell for the GANSS TOD-SFN relationship. | REL-7 |
| >>Reference SFN | MP |  | Integer(0..4095) | The SFN for which the location is valid. This IE indicates the SFN at which the UE timing of cell frames is captured. | REL-7 |
| >GANSS reference time only |  |  |  |  | REL-7 |
| >>GANSS Time Id | OP |  | INTEGER (0..7) | Absence means Galileo. For coding description see NOTE 2. |  |
| >>GANSS TOD msec | MP |  | Integer(0..3599999) | GANSS Time of Day (modulo 1 hour) in milliseconds (rounded down to the nearest millisecond unit). | REL-7 |
| >>GANSS TOD Uncertainty | OP |  | Integer(0..127) | Coding as in 10.3.7.96a | REL-7 |
| GANSS Generic Measurement Information | MP | 1 to <maxGANSS> |  |  | REL-7 |
| >GANSS ID | OP |  | Integer(0..7) | Absence of this IE means Galileo. For coding description, see Note 1 in 10.3.7.90b. | REL-7 |
| >GANSS Signal Measurement Information | MP | 1 to <maxSgnType> |  |  | REL-7 |
| >>GANSS Signal ID | OP |  | GANSS Signal Id 10.3.3.45a | Absence of this field means the default value as defined in 10.3.3.45a for the GANSS identified by GANSS_ID. | REL-7 |
| >>GANSS Code Phase Ambiguity | OP |  | Integer (0..31) | The "GANSS Code Phase Ambiguity" field gives the ambiguity of the code phase measurement. It is given in ms and is an integer between 0 and 31. The Total Code Phase for a satellite k (Satk) is given modulo this "GANSS Code Phase Ambiguity" and is reconstructed with: Code_Phase_Tot(Satk) = Code_Phase (Satk)+ Integer Code Phase(Satk) If there is no code phase ambiguity, the "GANSS Code Phase Ambiguity" shall be set to 0. The field is optional. If "GANSS Code Phase Ambiguity” and “GANSS Code Phase Ambiguity Extension” is absent, the default value is 1ms. | REL-7 |
| >>GANSS Code Phase Ambiguity Extension | OP |  | Integer(32..127) | If the total GANSS code phase ambiguity is greater than 31 ms, then this field shall be used. The definition is the same as for the "GANSS Code Phase Ambiguity" field except the value range is from 32 to 127 ms. | REL-8 |
| >>GANSS Measurement Parameters | MP | 1 to <maxGANSSSat> |  |  | REL-7 |
| >>>Satellite ID | MP |  | Integer (0..63) | Identifies the satellite and is as defined in Note 2 of 10.3.7.88b. | REL-7 |
| >>>C/No | MP |  | Integer(0..63) | The estimate of the carrier-to-noise ratio of the received signal from the particular satellite used in the measurement. It is given in units of dB-Hz (typical levels will be in the range of 20 – 50 dB-Hz). | REL-7 |
| >>>Multipath Indicator | MP |  | Enumerated(NM, low, medium, high) | Coding as in 10.3.7.93 | REL-7 |
| >>>Carrier Quality Indicaton | OP |  | Bit string(2) | Note 1 | REL-7 |
| >>>GANSS Code Phase | MP |  | Integer(0..221-1) | Scale factor 2-21 Code phase for the particular satellite signal at the time of measurement in the units of milliseconds. Increasing binary values of the field signify increasing measured pseudoranges. GNSS specific code phase measurements (e.g. chips) are converted into unit of ms by dividing the measurements by the nominal values of the measured signal chipping rate. | REL-7 |
| >>>GANSS Integer Code Phase | OP |  | Integer(0..63) | In ms Indicates the integer millisecond part of the code phase, that is expressed modulo the GANSS Code Phase Ambiguity. | REL-7 |
| >>>GANSS Integer Code Phase Extension | OP |  | Integer(64..127) | If the total GANSS integer code phase is greater than 63 ms, then this field shall be used. The definition is the same as for the GANSS Integer Code Phase field except the value range is from 64 to 127 ms. | REL-8 |
| >>>Code Phase RMS Error | MP |  | Enumerated(range index 0..range index 63) | Coding as in Note 2 of 10.3.7.93 | REL-7 |
| >>>Doppler | MP |  | Integer(-32768..32767) | m/s, scale factor 0.04. Doppler measured by the UE for the particular satellite signal | REL-7 |
| >>>ADR | OP |  | Integer(0..33554431) | Meters, scale factor 2-10 ADR measurement measured by the UE for the particular satellite signal. | REL-7 |

NOTE 1:	Coding of Carrier quality indication:

| MSB | LSB | Explanation |
| --- | --- | --- |
| 0 |  | Carrier phase not continuous |
| 1 |  | Carrier phase continuous |
|  | 0 | Data direct |
|  | 1 | Data Inverted |

NOTE 2:	Coding of GANSS Time Id:

| Value of GANSS Time id | Explanation | Version |
| --- | --- | --- |
| 0 | GPS system time | REL-8 |
| 1 | QZSS system time |  |
| 2 | GLONASS system time |  |
| 3 | BDS system time | REL-12 |
| 4-7 | Reserved |  |

#### 10.3.7.94	UE positioning GPS navigation model

This IE contain information required to manage the transfer of precise navigation data to the GPS-capable UE.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Satellite information | MP | 1 to <maxSat> |  |  |
| >SatID | MP |  | Enumerated(0..63) | Identifies the satellite and is equal to (SV ID No - 1) where SV ID No is defined in [12]. |
| >Satellite Status | MP |  | Enumerated(NS_NN, ES_SN, ES_NN, REVD) | NOTE |
| >GPS Ephemeris and Clock Correction parameters | CV-Satellite status |  | UE positioning GPS Ephemeris and Clock Correction parameters 10.3.7.91a |  |

NOTE:	The UE shall interpret enumerated symbols as follows.

| Value | Indication |
| --- | --- |
| NS_NN | New satellite, new Navigation Model |
| ES_SN | Existing satellite, same Navigation Model |
| ES_NN | Existing satellite, new Navigation Model |
| REVD | Reserved |

| Condition | Explanation |
| --- | --- |
| Satellite status | The IE is not needed if the IE "Satellite status" is ES_SN and mandatory present otherwise. |

#### 10.3.7.94a		UE positioning GANSS navigation model

This IE contains information required to manage the transfer of precise navigation data to the GANSS-capable UE.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Non-Broadcast Indication | OP |  | Enumerated (TRUE) | If this IE is present, GANSS navigation model is not derived from satellite broadcast | REL-7 |
| Satellite information | MP | 1 to <maxGANSSSat> |  |  | REL-7 |
| >SatID | MP |  | Integer (0..63) | Identifies the satellite and is as defined in Note 2 of 10.3.7.88b. | REL-7 |
| >SV Health | MP |  | Bit string(9) | NOTE | REL-12 |
| >IOD | MP |  | Bit string(10) |  | REL-7 |
| >GANSS Clock Model | MP |  | UE positioning GANSS clock model 10.3.7.91d |  | REL-7 |
| > GANSS Orbit Model | MP |  | UE positioning GANSS orbit model 10.3.7.91c |  | REL-7 |

NOTE:	The Health values are GNSS specific. For Galileo, the UE shall interpret the bit string as follows [61]:

| Parameter | Bit | Type | Explanation |
| --- | --- | --- | --- |
| E5a Data Validity Status | 0 | Boolean |  |
| E5b Data Validity Status | 1 | Boolean |  |
| E1-B Data Validity Status | 2 | Boolean |  |
| E5a Signal Health Status | 3-4 | Bit string |  |
| E5b Signal Health Status | 5-6 | Bit string |  |
| E1-B Signal Health Status | 7-8 | Bit string |  |

#### 10.3.7.94b		UE positioning GANSS additional navigation models

This IE contains information required to manage the transfer of precise navigation data to the GANSS-capable UE.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Non-Broadcast Indication | OP |  | Enumerated (TRUE) | If this IE is present, GANSS navigation model is not derived from satellite broadcast | REL-8 |
| Satellite information | MP | 1 to <maxGANSSSat> |  |  | REL-8 |
| >SatID | MP |  | Integer (0..63) | Identifies the satellite and is as defined in Note 2 of 10.3.7.88b. | REL-8 |
| >SV Health | MP |  | Bit string(6) | NOTE 1 | REL-8 |
| >IOD | MP |  | Bit string(11) | NOTE 2 | REL-8 |
| >GANSS additional clock models | MP |  | UE positioning GANSS additional clock models 10.3.7.91f |  | REL-8 |
| >GANSS additional orbit models | MP |  | UE positioning GANSS additional orbit models 10.3.7.91e |  | REL-8 |

NOTE 1:	The SV Health values are GNSS specific as identified by the GANSS ID :

| GANSS | SV Health Bit String(6) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 (LSB) |
| Modernized GPS | L1C Health [70] | L1 Health [68,69] | L2 Health [68,69] | L5 Health [68,69] | ‘0’ (reserved) | ‘0’ (reserved) |
| SBAS | Ranging On (0), Off(1) [71] | Corrections On (0), Off (1) [71] | Integrity On (0), Off(1) [71] | ‘0’ (reserved) | ‘0’ (reserved) | ‘0’ (reserved) |
| QZSS QZS-L1 | SV Health [72] |  |  |  |  |  |
| QZSS QZS-L1C/L2C/L5 | L1C Health [72] | L1 Health [72] | L2 Health [72] | L5 Health [72] | ‘0’ (reserved) | ‘0’ (reserved) |
| GLONASS | Bn (MSB) [73, page 23] | FT [73, Table 4.4] |  |  |  | ‘0’ (reserved) |
| BDS | B1I Health (SatH1) [86] | ‘0’ (reserved) | ‘0’ (reserved) | ‘0’ (reserved) | ‘0’ (reserved) | ‘0’ (reserved) |

NOTE 2:	The IOD values are GNSS specific as identified by the GANSS ID :

| GANSS | IOD Bit String(11) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 | Bit 9 | Bit 10 | Bit 11 (LSB) |
| Modernized GPS | toe (seconds, scale factor 300, range 0 – 604500) [68, 69, 70] |  |  |  |  |  |  |  |  |  |  |
| SBAS | ‘0’ | ‘0’ | ‘0’ | Issue of Data ([71], Message Type 9) |  |  |  |  |  |  |  |
| QZSS QZS-L1 | ‘0’ | Issue of Data, Clock [72] |  |  |  |  |  |  |  |  |  |
| QZSS QZS-L1C/L2C/L5 | toe (seconds, scale factor 300, range 0 – 604500) [72] |  |  |  |  |  |  |  |  |  |  |
| GLONASS | ‘0’ | ‘0’ | ‘0’ | ‘0’ | tb (minutes, scale factor 15, range 0 – 1425) [73] |  |  |  |  |  |  |
| BDS | 11 MSB bits of toe (seconds, scale factor 512, range 0 – 604672) [86] |  |  |  |  |  |  |  |  |  |  |

#### 10.3.7.95	UE positioning GPS real-time integrity

This IE contains parameters that describe the real-time status of the GPS constellation.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Satellite information | MP | 1 to <maxSat> |  |  |
| >BadSatID | MP |  | Enumerated(0..63) | Identifies the satellite and is equal to (SV ID No - 1) where SV ID No is defined in [12]. |

#### 10.3.7.95a	Void

#### 10.3.7.95b		UE positioning GANSS real-time integrity

This IE contains parameters that describe the real-time status of the GANSS constellation.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Satellite information | MP | 1 to <maxGANSSSat> |  |  | REL-7 |
| >Bad GANSS SatID | MP |  | Integer (0..63) | Identifies the satellite and is as defined in Note 2 of 10.3.7.88b. | REL-7 |
| >Bad GANSS Signal ID | OP |  | Bit string (8) | Absence of this IE means that all signals of the specific SV are bad. NOTE | REL-7 |

NOTE:	Coding of Bad GANSS Signal ID

| GNSS | Bit | Explanation | Version |
| --- | --- | --- | --- |
| Galileo | 1 | E1 | REL-7 |
|  | 2 | E5A |  |
|  | 3 | E5B |  |
|  | 4 | E6 |  |
|  | 5 | E5A + E5B |  |
|  | 6-8 | Spare |  |
| Modernized GPS | 1 | L1C | REL-8 |
|  | 2 | L2C |  |
|  | 3 | L5 |  |
|  | 4-8 | Spare |  |
| SBAS | 1 | L1 |  |
|  | 2-8 | Spare |  |
| QZSS | 1 | QZS-L1 |  |
|  | 2 | QZS-L1C |  |
|  | 3 | QZS-L2C |  |
|  | 4 | QZS-L5 |  |
|  | 5-8 | Spare |  |
| GLONASS | 1 | G1 |  |
|  | 2 | G2 |  |
|  | 3 | G3 |  |
|  | 4-8 | Spare |  |
| BDS | 1 | B1I | REL-12 |
|  | 2-8 | Spare |  |

#### 10.3.7.96	UE positioning GPS reference time

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GPS Week | MP |  | Integer(0..1023) |  |  |
| GPS Week Cycle Number | OP |  | Integer(0..7) | This field provides the number of 1024 GPS week cycles occurred since the GPS zero time-point (midnight of the night of January 5, 1980/morning of January 6, 1980). The first 1024 GPS weeks since the zero time-point is GPS Week Cycle Number 0. | REL-10 |
| GPS TOW msec | MP |  | Integer(0..6.048*108-1) | GPS Time of Week in milliseconds (rounded down to the nearest millisecond unit). |  |
| UTRAN GPS reference time | OP |  |  |  |  |
| >UTRAN GPS timing of cell frames | MP |  | Integer(0.. 2322431999999) | UTRAN GPS timing of cell frames in steps of 1 chip |  |
| >CHOICE mode | OP |  |  |  |  |
| >>FDD |  |  |  |  |  |
| >>>Primary CPICH Info | MP |  | Primary CPICH Info 10.3.6.60 | Identifies the reference cell for the GPS TOW-SFN relationship |  |
| >>TDD |  |  |  |  |  |
| >>>cell parameters id | MP |  | Cell parameters id 10.3.6.9 | Identifies the reference cell for the GPS TOW-SFN relationship |  |
| >SFN | MP |  | Integer(0..4095) | The SFN which the UTRAN GPS timing of cell frames time stamps. |  |
| UE Positioning GPS ReferenceTime Uncertainty | OP |  | UE positioning GPS reference time uncertainty 10.3.7.96a | NOTE 1 | REL-7 |
| SFN-TOW Uncertainty | OP |  | Enumerated (lessThan10, moreThan10) | This field indicates the uncertainty of the relation GPS TOW/SFN. lessThan10 means the relation is accurate to at least 10 ms. NOTE 1 |  |
| TUTRAN-GPS drift rate | OP |  | Integer (0, 1, 2, 5, 10, 15, 25, 50, -1, -2, -5, -10, -15, -25, -50) | in 1/256 chips per sec. |  |
| GPS TOW Assist | OP | 1 to <maxSat> |  |  |  |
| >SatID | MP |  | Enumerated(0..63) | Identifies the satellite and is equal to (SV ID No - 1) where SV ID No is defined in [12]. |  |
| >TLM Message | MP |  | Bit string(14) |  |  |
| >TLM Reserved | MP |  | Bit string(2) |  |  |
| >Alert | MP |  | Boolean |  |  |
| >Anti-Spoof | MP |  | Boolean |  |  |
| NOTE 1: If the IE "UE Positioning GPS ReferenceTime Uncertainty" is present, the IE "SFN-TOW Uncertainty" is not needed and shall be ignored. |  |  |  |  |  |

#### 10.3.7.96o		UE positioning GANSS reference time

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GANSS Day | OP |  | Integer(0..8191) | The sequential number of days (with day count starting at 0) from the origin of the GNSS system time indicated by GANSS Time ID modulo 8192 days (about 22 years). If this field is not present and the UE cannot obtain GNSS system time information by other means, the UE behaviour is unspecified. NOTE. | REL-7 |
| GANSS Day Cycle Number | OP |  | Integer(0..7) | This field provides the number of 8192 day cycles occurred since the GANSS zero time-point defined in NOTE below. The first 8192 GANSS days since the zero time-point is GANSS Day Cycle Number 0. When this field is included, the GANSS Day field shall be included as well. | REL-10 |
| GANSS TOD | MP |  | Integer(0..86399) | GANSS Time of Day in seconds | REL-7 |
| GANSS TOD Uncertainty | OP |  | Integer(0..127) | Coding as in subclause 10.3.7.96a | REL-7 |
| GANSS Time ID | OP |  | Integer (0..7) | Absense of this IE means Galileo, values 0-7 are reserved for future use | REL-7 |
|  |  |  |  | Absense of this IE means Galileo, For coding description see NOTE 2 in 10.3.7.93a. | REL-8 |
| UTRAN GANSS reference time | OP |  |  |  | REL-7 |
| >UTRAN GANSS timing of cell frames | MP |  | Integer(0.. 999999750 by step of 250) | UTRAN GANSS timing of cell frames in steps of 250 ns. Indicates sub-second part of GANSS TOD | REL-7 |
| >CHOICE mode | MP |  |  |  | REL-7 |
| >>FDD |  |  |  |  | REL-7 |
| >>>Primary CPICH Info | MP |  | Primary CPICH Info 10.3.6.60 | Identifies the reference cell for the GANSS TOD-SFN relationship | REL-7 |
| >>TDD |  |  |  |  | REL-7 |
| >>>cell parameters id | MP |  | Cell parameters id 10.3.6.9 | Identifies the reference cell for the GANSS TOD-SFN relationship | REL-7 |
| >SFN | MP |  | Integer(0..4095) | The SFN which the UTRAN GANSS timing of cell frames time stamps. | REL-7 |
| TUTRAN-GANSS drift rate | OP |  | ENUMERATED (0, 1, 2, 5, 10, 15, 25, 50, -1, -2, -5, -10, -15, -25, -50) | in 1/256 chips per sec. One spare value needed. | REL-7 |

NOTE: 	Definition of GNSS system time origin:

| GANSS | GNSS System Time Origin |
| --- | --- |
| Galileo System Time | 13 seconds before midnight between 21st August and 22nd August 1999; i.e., GST was equal to 13 seconds at August 22, 1999; 00:00:00 UTC |
| GPS System Time | January 6, 1980; 00:00:00 UTC(USNO) |
| QZSS System Time | January 6, 1980; 00:00:00 UTC(USNO) |
| GLONASS System Time | December 31, 1995; 21:00:00 UTC(SU), which is local UTC Moscow January 1, 1996; 00:00:00, defined as UTC(SU) + 3 hours in [73] |
| BDS System Time | January 1, 2006; 00:00:00 UTC(NTSC) |

#### 10.3.7.96a	UE positioning GPS reference time uncertainty

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UE Positioning GPS Reference Time Uncertainty | MP |  | Integer (0..127) | This element provides the accuracy of the provided GPS time, or alternatively the accuracy of the provided relation between GPS and UTRAN time. If "GPS TOW" is the provided GPS time, or alternatively the GPS time corresponding to the UTRAN time provided, then the true GPS time lies in the interval ["GPS TOW" "GPS Reference Time Uncertainty", "GPS TOW" + "GPS Reference Time Uncertainty"]. The uncertainty r, expressed in microseconds, is mapped to a number K with the following formula: r = C*(((1+x)K)-1) with C = 0.0022 and x = 0.18. To encode any higher value of the uncertainty than that corresponding to K=127 in the formula above, or to indicate an undefined value of the "GPS TOW", the same value, K=127, shall be used. | REL-7 |

NOTE:	This IE is not needed in the SRNS RELOCATION INFO message.

#### 10.3.7.97	UE positioning GPS UTC model

The UTC Model field contains a set of parameters needed to relate GPS time to Universal Time Coordinate (UTC).

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| A1 | MP |  | Bit string(24) | sec/sec [12] |
| A0 | MP |  | Bit string(32) | seconds [12] |
| tot | MP |  | Bit string(8) | seconds [12] |
| WNt | MP |  | Bit string(8) | weeks [12] |
| tLS | MP |  | Bit string(8) | seconds [12] |
| WNLSF | MP |  | Bit string(8) | weeks [12] |
| DN | MP |  | Bit string(8) | days [12] |
| tLSF | MP |  | Bit string(8) | seconds [12] |

#### 10.3.7.97a		UE positioning GANSS time model

The GANSS time model field contains a set of parameters needed to relate GANSS time for the GANSS indicated by "GANSS ID" to selected time reference indicated by "GNSS_TO_ID".

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GANSS Time Model Reference Time | MP |  | Integer(0..604784 by step of 16) | GANSS reference time of week for GANSS Time Model, given in GNSS specific system time, with a 16s scale factor. | REL-7 |
| TA0 | MP |  | Integer (-2147483648 .. 2147483647) | Seconds, scale factor 2-35 | REL-7 |
| TA1 | OP |  | Integer (-8388608 .. 8388607) | sec/sec, scale factor 2-51 | REL-7 |
| TA2 | OP |  | Integer (-64 .. 63) | sec/sec2 , scale factor 2-68 | REL-7 |
| GNSS_TO_ID | MP |  | Enumerated (GPS, | GNSS Time Offset ID Seven spare values needed. | REL-7 |
|  |  |  | Galileo, QZSS, GLONASS, | GNSS Time Offset ID Four spare values needed. | REL-8 |
|  |  |  | BDS) | GNSS Time Offset ID Three spare values needed. | REL-12 |
| Week Number | OP |  | Integer(0..8191) | Reference week of GANSS Time Model given in GNSS specific system time | REL-7 |
| Delta_T | OP |  | Integer(-128..127) | This field specifies the integer seconds of the GNSS-GNSS Time Offset. Scale factor 1 second. | REL-10 |

#### 10.3.7.97b		UE positioning GANSS data bit assistance

This element provides data bit assistance data for specific satellite signals for data wipe-off.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GANSS TOD | MP |  | Integer(0..59) | Refererence time (modulo 1 minute) of the first bit of the data in IE “Data Bits”, in seconds | REL-7 |
| Data Bit Assistance List | MP | 1 to <maxGANSSSat> |  | The following fields are present N_Sat times, with 1 ≤ N_Sat ≤ maxGANSSSat | REL-7 |
| >Satellite ID | MP |  | Integer (0..63) | Identifies the satellite and is as defined in Note 2 of 10.3.7.88b. | REL-7 |
| >Data bit assistance Sgn List | MP | 1 to <maxSgnType> |  | The following fields are present N_Sgn times, with 1 ≤ N_Sgn ≤ maxSgnType | REL-7 |
| >>GANSS Signal ID | MP |  | GANSS Signal ID 10.3.3.45a |  | REL-7 |
| >>Data Bits | MP |  | Bit string (1..1024) | Raw data bits as transmitted from a specific satellite at the time indicated by GANSS_TOD. In case of Galileo, it contains the FEC encoded and interleaved modulation symbols. The logical levels 1 and 0 correspond to signal levels -1 and +1, respectively. N_BIT is the actual number of elements in Data bits: 1 ≤ N_BIT ≤ 1024. | REL-7 |
|  |  |  |  | In case of Modernized GPS L1C, it contains the encoded and interleaved modulation symbols as defined in [70] section 3.2.3.1. In case of Modernized GPS L2C, it contains either the NAV data modulation bits, the FEC encoded NAV data modulation symbols, or the FEC encoded CNAV data modulation symbols, dependent on the current signal configuration of this satellite as defined in [68, Table 3-III]. In case of Modernized GPS L5, it contains the FEC encoded CNAV data modulation symbols as defined in [69]. In case of SBAS, it contains the FEC encoded data modulation symbols as defined in [71]. In case of QZSS QZS-L1, it contains the NAV data modulation bits as defined in [72] section 5.2. In case of QZSS QZS-L1C, it contains the encoded and interleaved modulation symbols as defined in [72] section 5.3. In case of QZSS QZS-L2C, it contains the encoded modulation symbols as defined in [72] section 5.5. In case of QZSS QZS-L5, it contains the encoded modulation symbols as defined in [72] section 5.6. In case of GLONASS, it contains the 100 sps differentially Manchester encoded modulation symbols as defined in [73] section 3.3.2.2. | REL-8 |
|  |  |  |  | In case of BDS, it contains the encoded and interleaved modulation symbols as defined in [86] section 5.1.3. | REL-12 |

#### 10.3.7.97c	UE positioning GANSS UTC model

The UTC Model field contains a set of parameters needed to relate GANSS time to Universal Time Coordinate (UTC).

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| A1 | MP |  | Bit string(24) |  |
| A0 | MP |  | Bit string(32) |  |
| tot | MP |  | Bit string(8) |  |
| tLS | MP |  | Bit string(8) |  |
| WNt | MP |  | Bit string(8) |  |
| WNLSF | MP |  | Bit string(8) |  |
| DN | MP |  | Bit string(8) |  |
| tLSF | MP |  | Bit string(8) |  |

For a GANSS Id referring to Galileo, the definition and the unit of the elements are provided in [61].

#### 10.3.7.97d	UE positioning GANSS additional UTC models

This element contains contains several sets of parameters needed to relate GNSS system time to Universal Time Coordinate (UTC), as defined in [68, 69, 70, 71, 72, 73]. Only one parameter set can be included in one UE positioning GANSS additional UTC models and which set of parameters to include depends on the GANSS-ID and UE capabilities.

The UTC time standard, UTC(k), is GNSS specific. E.g., if GANSS ID indicates QZSS, UE positioning GANSS additional UTC models contains a set of parameters needed to relate QZST to UTC(NICT). If GANSS ID indicates Modernized GPS, UE positioning GANSS additional UTC models contains a set of parameters needed to relate GPS system time to UTC(USNO). If GANSS ID indicates GLONASS, UE positioning GANSS additional UTC models contains a set of parameters needed to relate GLONASS system time to UTC(RU). If GANSS ID indicates SBAS, UE positioning GANSS additional UTC models contains a set of pameters needed to relate SBAS network time for the SBAS indicated by SBAS ID to the UTC standard defined by the UTC Standard ID. If GANSS ID indicates BDS, UE positioning GANSS additional UTC models contains a set of parameters needed to relate BDS system time to UTC (NTSC).

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Model-1 | OP |  |  | NOTE | REL-8 |
| >A0-n | MP |  | Bit String(16) | Bias coefficient of GNSS time scale relative to UTC time scale (seconds) [68,69,70,72] | REL-8 |
| >A1-n | MP |  | Bit String(13) | Drift coefficient of GNSS time scale relative to UTC time scale (sec/sec) [68,69,70,72] | REL-8 |
| >A2-n | MP |  | Bit String(7) | Drift rate correction coefficient of GNSS time scale relative to UTC time scale (sec/sec2) [68,69,70,72] | REL-8 |
| >tLS | MP |  | Bit String(8) | Current or past leap second count (seconds) [68,69,70,72] | REL-8 |
| >tot | MP |  | Bit String(16) | Time data reference time of week (seconds) [68,69,70,72] | REL-8 |
| >WNot | MP |  | Bit String(13) | Time data reference week number (weeks) [68,69,70,72] | REL-8 |
| >WNLSF | MP |  | Bit String(8) | Leap second reference week number (weeks) [68,69,70,72] | REL-8 |
| >DN | MP |  | Bit String(4) | Leap second reference day number (days) [68,69,70,72] | REL-8 |
| >tLSF | MP |  | Bit String(8) | Current or future leap second count (seconds) [68,69,70,72] | REL-8 |
| Model-2 | OP |  |  | NOTE | REL-8 |
| >NA | MP |  | Bit String(11) | Callendar day number within four-year period beginning since the leap year (days) [73] | REL-8 |
| c | MP |  | Bit String(32) | GLONASS time scale correction to UTC(SU) (seconds) [73] | REL-8 |
| >Delta UT1 | OP |  |  |  |  |
| >>B1 | MP |  | Bit String(11) | Coefficient to determine UT1 (seconds) [73] | REL-8 |
| >>B2 | MP |  | Bit String(10) | Coefficient to determine UT1 (seconds/msd) [73] | REL-8 |
| >KP | OP |  | Bit String(2) | Notification of expected leap second correction (dimensionless) [73] | REL-8 |
| Model-3 | OP |  |  | NOTE | REL-8 |
| >A1WNT | MP |  | Bit String(24) | sec/sec ([71], Message Type 12) | REL-8 |
| >A0WNT | MP |  | Bit String(32) | seconds ([71], Message Type 12) | REL-8 |
| >tot | MP |  | Bit String(8) | seconds ([71], Message Type 12) | REL-8 |
| >WNt | MP |  | Bit String(8) | weeks ([71], Message Type 12) | REL-8 |
| >tLS | MP |  | Bit String(8) | seconds ([71], Message Type 12) | REL-8 |
| >WNLSF | MP |  | Bit String(8) | weeks ([71], Message Type 12) | REL-8 |
| >DN | MP |  | Bit String(8) | days ([71], Message Type 12) | REL-8 |
| >tLSF | MP |  | Bit String(8) | seconds ([71], Message Type 12) | REL-8 |
| >UTC Standard ID | MP |  | Bit String(3) | dimensionless NOTE 1 | REL-8 |
| Model-4 | OP |  |  | NOTE | REL-12 |
| >A0UTC | MP |  | Bit String (32) | Seconds [86] | REL-12 |
| >A1UTC | MP |  | Bit String (24) | sec/sec [86] | REL-12 |
| >tLS | MP |  | Bit String (8) | Seconds [86] | REL-12 |
| >WNLSF | MP |  | Bit String (8) | Weeks [86] | REL-12 |
| >DN | MP |  | Bit String (8) | Days [86] | REL-12 |
| >tLSF | MP |  | Bit String (8) | Seconds [86] | REL-12 |

NOTE:	This IE shall contain exactly one of the optional fields indicated by "NOTE".

NOTE 1:	Coding of UTC Standard ID:

| Value of UTC Standard ID | UTC Standard |
| --- | --- |
| 0 | UTC as operated by the Communications Research Laboratory (CRL), Tokyo, Japan |
| 1 | UTC as operated by the National Institute of Standards and Technology (NIST) |
| 2 | UTC as operated by the U. S. Naval Observatory (USNO) |
| 3 | UTC as operated by the International Bureau of Weights and Measures (BIPM) |
| 4-7 | Reserved for future definition |

#### 10.3.7.97e	UE positioning GANSS SBAS ID

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SBAS ID | MP |  | Enumerated( WAAS, EGNOS, MSAS, GAGAN) | Four spare values are needed. | REL-8 |

#### 10.3.7.97f	UE positioning GANSS auxiliary information

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE GANSS-ID |  |  |  |  | REL-8 |
| >GANSS-ID-1 |  |  |  | This choice may only be present if GANSS ID indicated “Modernized GPS” | REL-8 |
| >>Aux Info List | MP | 1 to <maxGANSSSat> |  |  | REL-8 |
| >>>Sat ID | MP |  | Integer(0..63) | Identifies the satellite and is as defined in Note 2 of 10.3.7.88b. | REL-8 |
| >>>Signals Available | MP |  | Bit String(8) | This field indicates the ranging signals supported by the satellite indicated by Sat ID. This field is given as a bit string as shown in NOTE 1 for Modernized GPS. If a bit is set to ‘1’ it indicates that the satellite identified by Sat ID transmits ranging signals according to the signal correspondence in NOTE 1. If a bit is set to ‘0’ it indicates that the corresponding signal is not supported on the satellite identified by Sat ID | REL-8 |
| >GANSS-ID-3 |  |  |  | This choice may be present if GANSS ID indicated “GLONASS” | REL-8 |
| >>Aux Info List | MP | 1 to <maxGANSSSat> |  |  | REL-8 |
| >>>Sat ID | MP |  | Integer(0..63) | Identifies the satellite and is as defined in Note 2 of 10.3.7.88b. | REL-8 |
| >>>Signals Available | MP |  | Bit String(8) | This field indicates the ranging signals supported by the satellite indicated by Sat ID. This field is given as a bit string as shown in NOTE 1 for GLONASS. If a bit is set to ‘1’ it indicates that the satellite identified by Sat ID transmits ranging signals according to the signal correspondence in NOTE 1. If a bit is set to ‘0’ it indicates that the corresponding signal is not supported on the satellite identified by Sat ID. | REL-8 |
| >>>Channel Number | MP |  | Integer(-7..13) | This field indicates the GLONASS carrier frequency number of the satellite identified by Sat ID, as defined in [73]. | REL-8 |

NOTE 1:	Coding of Signals Available

| GANSS ID | Signals Avaialble Bit String(8) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 (LSB) |
| 1 Modernized GPS | L1C | L2C | L5 | ‘0’ Spare | ‘0’ Spare | ‘0’ Spare | ‘0’ Spare | ‘0’ Spare |
| 3 GLONASS | G1 | G2 | G3 | ‘0’ Spare | ‘0’ Spare | ‘0’ Spare | ‘0’ Spare | ‘0’ Spare |

#### 10.3.7.98	UE positioning IPDL parameters

This IE contains parameters for the IPDL mode. The use of this parameters is described in [29].

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode |  |  |  |  | REL-4 |
| >FDD |  |  |  |  | REL-4 |
| >>IP spacing | MP |  | Integer(5,7,10,15,20,30,40,50) | See [29] |  |
| >>IP length | MP |  | Integer(5,10) | See [29] |  |
| >>IP offset | MP |  | Integer(0..9) | See [29] |  |
| >>Seed | MP |  | Integer(0..63) | See [29] |  |
| >TDD |  |  |  |  | REL-4 |
| >>IP spacing | MP |  | Integer(30,40,50,70,100) | See [33] | REL-4 |
| >>IP_Start | MP |  | Integer(0..4095) | See [33] | REL-4 |
| >>IP_Slot | MP |  | Integer(0..14) | See [33] | REL-4 |
| >>IP_PCCPCH | CV-channel |  | Boolean | See [33] | REL-4 |
| Burst mode parameters | OP |  |  |  |  |
| >Burst Start | MP |  | Integer(0..15) | See [29] and [33] |  |
| >Burst Length | MP |  | Integer(10..25) | See [29] and [33] |  |
| >Burst freq | MP |  | Integer(1..16) | See [29] and [33] |  |

| Condition | Explanation |
| --- | --- |
| channel | This IE is present only if the idle slot carries the PCCPCH |

#### 10.3.7.99	UE positioning measured results

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UE positioning OTDOA measured results | OP |  | UE positioning OTDOA measured results 10.3.7.105 |  |  |
| UE positioning Position estimate info | OP |  | UE positioning Position estimate info 10.3.7.109 |  |  |
| UE positioning GPS measured results | OP |  | UE positioning GPS measured results 10.3.7.93 |  |  |
| UE positioning error | OP |  | UE positioning error 10.3.7.87 | Included if UE positioning error occurred |  |
| UE positioning GANSS measured results | OP |  | UE positioning GANSS measured results 10.3.7.93a |  | REL-7 |
| UE positioning AddPos measured results | OP |  | UE positioning AddPos measured results 10.3.7.141 |  | REL-13 |

#### 10.3.7.100	UE positioning measurement

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UE positioning reporting quantity | MP |  | UE positioning reporting quantity 10.3.7.111 |  |  |
| Measurement validity | OP |  | Measurement validity 10.3.7.51 |  |  |
| CHOICE reporting criteria | MP |  |  |  |  |
| >UE positioning reporting criteria |  |  | UE positioning reporting criteria 10.3.7.110 |  |  |
| >Periodical reporting criteria |  |  | Periodical reporting criteria 10.3.7.53 |  |  |
| >No reporting |  |  |  | (no data) |  |
| UE positioning OTDOA assistance data for UE-assisted | OP |  | UE positioning OTDOA assistance data for UE-assisted 10.3.7.103 |  |  |
| UE positioning OTDOA assistance data for UE-based | OP |  | UE positioning OTDOA assistance data for UE-based 10.3.7.103a |  |  |
| UE positioning GPS assistance data | OP |  | UE positioning GPS assistance data 10.3.7.90 |  |  |
| UE positioning GANSS assistance data | OP |  | UE positioning GANSS assistance data 10.3.7.90b |  | REL-7 |

#### 10.3.7.101	UE positioning measurement event results

This IE contains the measurement event results that are reported to UTRAN for UE positioning measurements.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Event ID | MP |  |  | One spare value is needed. |
| >7a |  |  |  |  |
| >>UE positioning Position estimate info | MP |  | UE positioning Position estimate info 10.3.7.109 |  |
| >7b |  |  |  |  |
| >>UE positioning OTDOA measured results | MP |  | UE positioning OTDOA measured results 10.3.7.105 |  |
| >7c |  |  |  |  |
| >>UE positioning GPS measurement | MP |  | UE positioning GPS measured results 10.3.7.93 |  |
| >7d |  |  |  |  |
| >>UE positioning GANSS measurement | MP |  | UE positioning GANSS measured results 10.3.7.93a |  |

#### 10.3.7.102	Void

#### 10.3.7.103	UE positioning OTDOA assistance data for UE-assisted

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE positioning OTDOA reference cell info for UE-assisted | OP |  | UE positioning OTDOA reference cell info 10.3.7.108 |  |
| UE positioning OTDOA neighbour cell list for UE-assisted | OP | 1 to <maxCellMeas> |  |  |
| >UE positioning OTDOA neighbour cell info for UE-assisted | MP |  | UE positioning OTDOA neighbour cell info 10.3.7.106 |  |

#### 10.3.7.103a	UE positioning OTDOA assistance data for UE-based

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE positioning OTDOA reference cell info for UE-based | OP |  | UE positioning OTDOA reference cell info for UE-based 10.3.7.108a |  |
| UE positioning OTDOA neighbour cell list for UE-based | OP | 1 to <maxCellMeas> |  |  |
| >UE positioning OTDOA neighbour cell info for UE-based | MP |  | UE positioning OTDOA neighbour cell info for UE-based 10.3.7.106a |  |

#### 10.3.7.104	Void

#### 10.3.7.105	UE positioning OTDOA measured results

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| SFN | MP |  | Integer(0..4095) | SFN during which the last measurement was performed |
| CHOICE mode |  |  |  |  |
| >FDD |  |  |  |  |
| >>Reference cell id | MP |  | Primary CPICH info 10.3.6.60 |  |
| >>UE Rx-Tx time difference type 2 info | MP |  |  |  |
| >>>UE Rx-Tx time difference type 2 | MP |  | UE Rx-Tx time difference type 2 10.3.7.84 |  |
| >>>UE positioning OTDOA quality | MP |  | UE positioning OTDOA quality 10.3.7.107 | Quality of the UE Rx-Tx time difference type 2 measurement from the reference cell. |
| >TDD |  |  |  | (no data) |
| >>Reference cell id | MP |  | Cell parameters ID 10.3.6.9 |  |
| Neighbours | OP | 1 to <maxCellMeas> |  |  |
| >CHOICE mode | MP |  |  |  |
| >>FDD |  |  |  |  |
| >>>Neighbour Identity | MD |  | Primary CPICH info 10.3.6.60 | Default value is the same as in the first set of multiple sets. |
| >>>Frequency info | MD |  | Frequency info 10.3.6.36 | Default value is the existing value of frequency information |
| >>>UE Rx-Tx time difference type 2 info | OP |  |  | Included for cell in the active set excluding the reference cell. |
| >>>>UE Rx-Tx time difference type 2 | MP |  | UE Rx-Tx time difference type 2 10.3.7.84 |  |
| >>>>UE positioning OTDOA quality | MP |  | UE positioning OTDOA quality 10.3.7.107 | Quality of the UE Rx-Tx time difference type 2 measurement from the neighbour cell. |
| >>TDD |  |  |  |  |
| >>>Cell and Channel ID | MD |  | Cell and Channel Identity info 10.3.6.8a | Default value is the same as in the first set of multiple sets. |
| >>>UARFCN | MP |  | Integer (0..16383) |  |
| >UE positioning OTDOA quality | MP |  | UE positioning OTDOA quality 10.3.7.107 | Quality of the SFN-SFN observed time difference type 2 measurement from the neighbour cell. |
| >SFN-SFN observed time difference type 2 | MP |  | SFN-SFN observed time difference 10.3.7.63 | Gives the timing relative to the reference cell. Only type 2 is allowed. |

#### 10.3.7.106	UE positioning OTDOA neighbour cell info

This IE gives approximate cell timing in order to decrease the search window.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  |
| >FDD |  |  |  |  |
| >>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  |
| >TDD |  |  |  |  |
| >>cell and channel ID | MP |  | Cell and Channel Identity info 10.3.6.8a | Identifies the channel to be measured on. |
| Frequency info | MD |  | Frequency info 10.3.6.36 | Default value is the existing value of frequency information |
| IPDL parameters | CV-IPDLs |  | UE positioning IPDL parameters 10.3.7.98 |  |
| SFN offset | MP |  | Integer (0 .. 4095) | Although this IE is not always required, need is MP to align with ASN.1. Define Tref as the time of beginning of system frame number SFNref of the reference cell. Define Tnc as the beginning of a frame from the neighbour cell occurring immediately after the time Tref. Let the corresponding system frame number be SFNnc. Then SFNnc = SFNref-SFN offset modulo 4096. |
| SFN offset validity | MD |  | Enumerated (FALSE) | Absence of this element means SFN offset is valid. FALSE means SFN offset is not valid. |
| SFN-SFN relative time difference | MP |  | Integer(0.. 38399) | Gives the relative timing compared to the reference cell. Equal to (Tnc-Tref)*(3.84*106) where () denotes rounding to the nearest lower integer. In chips, Tnc = the time of beginning of a system frame from the neighbour cell, Tref = the time of beginning of a system frame from the reference cell. |
| SFN-SFN drift | OP |  | Integer (0, -1, -2, -3, -4, -5, -8, -10, -15, -25, -35, -50, -65, -80, -100, 1, 2, 3, 4, 5, 8, 10, 15, 25, 35, 50, 65, 80, 100) | in 1/256 chips per second |
| Search Window Size | MP |  | Integer(20, 40, 80, 160, 320, 640, 1280, infinity) | In chips. If the value is X then the expected SFN-SFN observed time difference is in the range [RTD-X, RTD+X] where RTD is the value of the field SFN-SFN relative time difference. Infinity means that the uncertainty is larger than 1280 chips. |
| CHOICE PositioningMode | MP |  |  |  |
| >UE based |  |  |  | (no data) |
| >UE assisted |  |  |  | (no data) |

| Condition | Explanation |
| --- | --- |
| IPDLs | This IE is mandatory present if IPDLs are applied and not needed otherwise. |

#### 10.3.7.106a	UE positioning OTDOA neighbour cell info for UE-based

This IE gives approximate cell timing in order to decrease the search window, as well as the cell locations and fine cell timing for UE based OTDOA.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UE positioning OTDOA neighbour cell info | MP |  | UE positioning OTDOA neighbour cell info 10.3.7.106 |  |  |
| Cell Position | MD |  |  | Default is the same as previous cell |  |
| >Relative North | OP |  | Integer(-20000..20000) | Seconds of angle, scale factor 0.03. Relative position compared to reference cell. |  |
| >Relative East | OP |  | Integer(-20000..20000) | Seconds of angle, scale factor 0.03. Relative position compared to reference cell. |  |
| >Relative Altitude | OP |  | Integer(-4000..4000) | Relative altitude in meters compared to ref. cell. |  |
| Fine SFN-SFN | MP |  | Real(0..0.9375 in steps of 0.0625) | Gives finer resolution |  |
| UE positioning Relative Time Difference Quality | MP |  | UE positioning OTDOA quality 10.3.7.109a | Quality of the relative time difference between neighbour and reference cell. |  |
| Round Trip Time | OP |  | Real (876.00 .. 2923.875 in steps of 0.0625) | In chips. Included if cell is in active set. |  |
| Round Trip Time Extension | CV-FDD |  | Real (0 .. 4392.125 in steps of 0.0625) | In chips. Included if cell is in active set. Default = 0. NOTE. | REL-7 |
| NOTE: Total Round Trip Time = IE "Round Trip Time" + IE "Round Trip Time Extension". |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | The IE is optionally present in FDD, otherwise it is not needed. |

#### 10.3.7.107	UE positioning OTDOA quality

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Std Resolution | MP |  | Bit string(2) | Std Resolution field includes the resolution used in Std of OTDOA Measurements field. Encoding on two bits as follows: '00' 10 meters '01' 20 meters '10' 30 meters '11' Reserved |
| Number of OTDOA Measurements | MP |  | Bit string(3) | The 'Number of OTDOA measurements' field indicates how many OTDOA measurements have been used in the UE to determine the sample standard deviation of the measurements. Following 3 bit encoding is used: '001' 5-9 '010' 10-14 '011' 15-24 '100' 25-34 '101' 35-44 '110' 45-54 '111' 55 or more Special case: '000':In this case the field 'Std of OTDOA measurements' contains the std of the reported SFN-SFN otd value = E[(x-µ)2], where x is the reported value and µ = E[x] is the expectation value (i.e. the true value) of x. This std can be used irrespective of the number of measurements and reporting of the number of measurements is not needed. Also other measurements such as Ec/No or Rx levels can be utilised in this case to evaluate the 'Std of OTDOA measurements' reported in this IE. |
| Std of OTDOA Measurements | MP |  | Bit string(5) | Std of OTDOA Measurements field includes sample standard deviation of OTDOA measurements (when number of measurements is reported in 'Number of OTDOA measurements field') or standard deviation of the reported SFN-SFN otd value = E[(x-µ)2], where x is the reported value and µ = E[x] is the expectation value (i.e. the true value) of x (when '000' is given in 'Number of OTDOA measurements' field). Following linear 5 bit encoding is used: '00000' 0 - (R*1-1) meters '00001' R*1 – (R*2-1) meters '00010' R*2 – (R*3-1) meters … '11111' R*31 meters or more where R is the resolution defined by Std Resolution field. E.g. R=20 m corresponds to 0-19 m, 20-39 m,…,620+ m. |

#### 10.3.7.108	UE positioning OTDOA reference cell info

This IE defines the cell used for time references in all OTDOA measurements.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| SFN | OP |  | Integer (0..4095) | Time stamp (SFN of Reference Cell) of the SFN-SFN relative time differences and SFN-SFN drift rates. Included if any SFN-SFN drift value is included in IE UE positioning OTDOA neighbour cell info. |
| CHOICE mode | MP |  |  |  |
| >FDD |  |  |  |  |
| >>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  |
| >TDD |  |  |  |  |
| >>cell and channel ID | MP |  | Cell and Channel Identity info 10.3.6.8a | Identifies the channel to be measured on. |
| Frequency info | MD |  | Frequency info 10.3.6.36 | Default value is the existing value of frequency information. This IE shall always be set to default value |
| CHOICE PositioningMode | MP |  |  |  |
| >UE based |  |  |  |  |
| >UE assisted |  |  |  | (no data) |
| IPDL parameters | OP |  | UE positioning IPDL parameters 10.3.7.98 | If this element is not included there are no idle periods present |

#### 10.3.7.108a	UE positioning OTDOA reference cell info for UE-based

This IE defines the cell used for time references in all OTDOA measurements for UE-based methods.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UE positioning OTDOA reference cell info | MP |  | UE positioning OTDOA reference cell info 10.3.7.108 |  |  |
| CHOICE Cell Position | OP |  |  | The position of the antenna that defines the cell. Used for the UE based method. |  |
| >Ellipsoid |  |  |  |  |  |
| >>Ellipsoid point | MP |  | Ellipsoid point 10.3.8.4a |  |  |
| >Ellipsoid with altitude |  |  |  |  |  |
| >>Ellipsoid point with altitude | MP |  | Ellipsoid point with altitude 10.3.8.4b |  |  |
| Round Trip Time | OP |  | Real (876.00 .. 2923.875 in steps of 0.0625) | In chips. |  |
| Round Trip Time Extension | CV-FDD |  | Real (0 .. 4392.125 in steps of 0.0625) | In chips. Default = 0. NOTE. | REL-7 |
| NOTE: Total Round Trip Time = IE "Round Trip Time" + IE "Round Trip Time Extension". |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | The IE is optionally present in FDD, otherwise it is not needed. |

#### 10.3.7.109	UE positioning position estimate info

The purpose of this IE is to provide the position estimate from the UE to the network, if the UE is capable of determining its own position.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Reference Time | MP |  |  |  |  |
| >UTRAN GPS reference time |  |  |  |  |  |
| >>UE GPS timing of cell frames | MP |  | Integer(0.. 37158911999999) | GPS Time of Week in units of 1/16th UMTS chips according to [19]. 33209832177664 spare values are needed. |  |
| >>CHOICE mode | MP |  |  |  |  |
| >>>FDD |  |  |  |  |  |
| >>>>Primary CPICH Info | MP |  | Primary CPICH Info 10.3.6.60 | Identifies the reference cell for the GPS TOW-SFN relationship |  |
| >>>TDD |  |  |  |  |  |
| >>cell parameters id | MP |  | Cell parameters id 10.3.6.9 | Identifies the reference cell for the GPS TOW-SFN relationship. |  |
| >>Reference SFN | MP |  | Integer(0..4095) | The SFN for which the location is valid. This IE indicates the SFN at which the UE timing of cell frame is captured. |  |
| >GPS reference time only |  |  |  |  |  |
| >>GPS TOW msec | MP |  | Integer(0..6.048*108-1) | GPS Time of Week in milliseconds (rounded down to the nearest millisecond unit). |  |
| >Cell timing |  |  |  |  |  |
| >>SFN | MP |  | Integer(0..4095) | SFN during which the position was calculated. |  |
| >>CHOICE mode | MP |  |  |  |  |
| >>>FDD |  |  |  |  |  |
| >>>>Primary CPICH Info | MP |  | Primary CPICH Info 10.3.6.60 | Identifies the reference cell for SFN |  |
| >>>TDD |  |  |  |  |  |
| >>cell parameters id | MP |  | Cell parameters id 10.3.6.9 | Identifies reference cell for SFN |  |
| >UTRAN GANSS reference time |  |  |  |  | REL-7 |
| >>UE GANSS timing of cell frames | MP |  | Integer(0.. 86399999999750 by step of 250) | GANSS Time of Day in ns | REL-7 |
| >>GANSS Time ID | OP |  | Integer(0..7) | Absence of this field means Galileo. For coding description see NOTE 2 in 10.3.7.93a. | REL-7 |
| >>GANSS TOD Uncertainty | OP |  | Integer(0..127) | Coding as in 10.3.7.96a | REL-7 |
| >>CHOICE mode | MP |  |  |  | REL-7 |
| >>>FDD |  |  |  |  | REL-7 |
| >>>>Primary CPICH Info | MP |  | Primary CPICH Info 10.3.6.60 | Identifies the reference cell for the GANSS TOD-SFN relationship | REL-7 |
| >>>TDD |  |  |  |  | REL-7 |
| >>>>cell parameters id | MP |  | Cell parameters id 10.3.6.9 | Identifies the reference cell for the GANSS TOD-SFN relationship. | REL-7 |
| >>Reference SFN | MP |  | Integer(0..4095) | The SFN for which the location is valid. This IE indicates the SFN at which the UE timing of cell frame is captured. | REL-7 |
| >GANSS reference time only |  |  |  |  | REL-7 |
| >>GANSS TOD msec | MP |  | Integer(0..3599999) | GANSS Time of Day in milliseconds (rounded down to the nearest millisecond unit). | REL-7 |
| >>GANSS Time ID | OP |  | Integer(0..7) | Absence of this field means Galileo. For coding description see NOTE 2 in 10.3.7.93a. | REL-7 |
| Location Time Stamp | OP |  | ASN.1 UTCTime expressed as a String (SIZE(13)) with format YYMMDDhhmmssZ | UTC Time when the position estimate was valid. This IE is optionally used only for the Additional Positioning methods. | REL-13 |
| CHOICE Position estimate | MP |  |  | The position estimate is provided in WGS-84 reference system. |  |
| >Ellipsoid Point |  |  | Ellipsoid Point; 10.3.8.4a |  |  |
| >Ellipsoid point with uncertainty circle |  |  | Ellipsoid point with uncertainty circle 10.3.8.4d |  |  |
| >Ellipsoid point with uncertainty ellipse |  |  | Ellipsoid point with uncertainty ellipse 10.3.8.4e |  |  |
| >Ellipsoid point with altitude |  |  | Ellipsoid point with altitude 10.3.8.4b |  |  |
| >Ellipsoid point with altitude and uncertainty ellipsoid |  |  | Ellipsoid point with altitude and uncertainty ellipsoid 10.3.8.4c |  |  |
| Position Data | MP |  | Bit string(16) | For each bit, if set to ‘1’, indicates that respective GNSS or position system was used for position calculation. Bit 0: OTDOA bit 1: GPS bit 2: Galileo bit 3-15: reserved | REL-7 |
|  |  |  |  | bit 3: SBAS bit 4: Modernized GPS bit 5: QZSS bit 6: GLONASS bit 7-15: reserved | REL-8 |
|  |  |  |  | bit 7: BDS | REL-12 |
|  |  |  |  | bit 8-15: reserved |  |
|  |  |  |  | bit 8: Barometric Pressure bit 9: MBS bit 10: WLAN bit 11: Bluetooth bit 12:-15: reserved | REL-13 |
| CHOICE Velocity estimate | OP |  |  |  | REL-7 |
| >Horizontal Velocity |  |  | Horizontal Velocity 10.3.8.4h |  | REL-7 |
| >Horizontal with Vertical Velocity |  |  | Horizontal with Vertical Velocity 10.3.8.4i |  | REL-7 |
| >Horizontal Velocity with Uncertainty |  |  | Horizontal Velocity with Uncertainty 10.3.8.4j |  | REL-7 |
| >Horizontal with Vertical Velocity and Uncertainty |  |  | Horizontal with Vertical Velocity and Uncertainty 10.3.8.4k |  | REL-7 |
| UE Positioning GPS ReferenceTime Uncertainty | CV-Tutran-gps |  | UE positioning GPS reference time uncertainty 10.3.7.96a |  | REL-7 |

| Condition | Explanation |
| --- | --- |
| Tutran-gps | This IE is optionally present if "UTRAN GPS reference time" is included and not needed otherwise. |

#### 10.3.7.109a	UE positioning Relative Time Difference quality

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Relative Time Difference Std Resolution | MP |  | Bit string(2) | Std Resolution field includes the resolution used in Std of Relative Time Difference field. Encoding on two bits as follows: '00' 10 meters '01' 20 meters '10' 30 meters '11' Reserved |
| Std of Relative Time Difference | MP |  | Bit string(5) | Std of Relative Time difference field includes standard deviation of (SFN-SFN relative time difference + Fine SFN-SFN). Following linear 5 bit encoding is used: '00000' 0 - (R*1-1) meters '00001' R*1 – (R*2-1) meters '00010' R*2 – (R*3-1) meters … '11111' R*31 meters or more where R is the resolution defined by Std Resolution field. E.g. R=20 m corresponds to 0-19 m, 20-39 m,…,620+ m. |

#### 10.3.7.110	UE positioning reporting criteria

The triggering of the event-triggered reporting for a UE positioning measurement.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Parameters required for each event | MP | 1 to <maxMeasEvent> |  | Note: need corrected to MP in Rel-7 to align with ASN.1. |  |
| >Amount of reporting | MP |  | Integer(1, 2, 4, 8, 16, 32, 64,infinite) |  |  |
| >Report first fix | MP |  | Boolean | If TRUE the UE reports the position once the measurement control is received, and then each time an event is triggered. |  |
| >Measurement interval | MP |  | Integer(5,15,60,300,900,1800,3600,7200) | Indicates how often the UE should make the measurement In seconds |  |
| >CHOICE Event ID | MP |  |  |  |  |
| >>7a |  |  |  |  |  |
| >>>Threshold Position Change | MP |  | Integer(10,20,30,40,50,100,200,300,500,1000,2000,5000,10000,20000,50000,100000) | Meters. Indicated how much the position should change compared to last reported position fix in order to trigger the event. |  |
| >>7b |  |  |  |  |  |
| >>>Threshold SFN-SFN change | MP |  | Real(0.25,0.5,1,2,3,4,5,10,20,50,100,200,500,1000,2000,5000) | Chips. Indicates how much the SFN-SFN measurement of ANY measured cell is allowed to change before the event is triggered. |  |
| >>7c |  |  |  |  |  |
| >>>Threshold SFN-GPS TOW | MP |  | Integer(1,2,3,5,10,20,50,100) | Time in ms. When the GPS TOW and SFN timer has drifted apart more than the specified value the event is triggered. |  |
|  |  |  |  | Time in micro seconds (NOTE 1). | REL-7 |
| >>7d |  |  |  |  | REL-7 |
| >>>Threshold SFN-GANSS TOW | MP |  | Integer(1,2,3,5,10,20,50,100) | Time in micro seconds. When the GANSS TOW and SFN timer has drifted apart more than the specified value the event is triggered. | REL-7 |
| NOTE 1: In previous versions of the protocol, the IE "Threshold SFN-GPS TOW" was provided in units of milli seconds. If the information is provided in units of micro seconds, the UE shall ignore any information provided in units of milli seconds. |  |  |  |  |  |

#### 10.3.7.111	UE positioning reporting quantity

The purpose of the element is to express the allowed/required location method(s), and to provide information desired QoS.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Method Type | MP |  | Enumerated(UE assisted, UE based, UE based is preferred but UE assisted is allowed, UE assisted is preferred but UE based is allowed) |  |  |
| Positioning Methods | MP |  | Enumerated(OTDOA, GPS, OTDOA or GPS, Cell ID) |  |  |
| Response Time | MP |  | Integer(1,2,4, 8, 16, 32, 64, 128) | This IE shall be ignored. |  |
| Horizontal Accuracy | CV-MethodType |  | Bit string(7) | The uncertainty is derived from the "uncertainty code" k by r = 10*(1.1k-1) in meters. |  |
| Vertical Accuracy | CV-MethodType |  | Bit string(7) | The uncertainty is derived from the "uncertainty code" k by r = 45*(1.025k-1) in meters. |  |
| GPS timing of Cell wanted | MP |  | Boolean | If TRUE the SRNC wants the UE to report the SFN-GPS timing of the reference cell. This is however optional in the UE. |  |
| Multiple Sets | MP |  | Boolean | This IE shall be ignored. |  |
| Additional Assistance Data Request | MP |  | Boolean | TRUE indicates that the UE is requested to send the IE "GPS Additional Assistance Data Request" and/or the “IE "GANSS Additional Assistance Data Request" (if the IE “GANSS Positioning Methods” is included) when the IE "UE positioning Error" is present in the UE positioning measured results. FALSE indicates that the UE shall use the assistance data available. |  |
| Environment Characterisation | OP |  | Enumerated(possibly heavy multipath and NLOS conditions, no or light multipath and usually LOS conditions, not defined or mixed environment) | One spare value is needed. |  |
| Velocity Requested | OP |  | Enumerated (TRUE) | If this element is present the UE is requested to send a velocity estimate | REL-7 |
| GANSS Positioning Methods | OP |  | Bit string(16) | For each bit, if set to ‘1’, indicates that respective GNSS is allowed. bit 0: GPS bit 1: Galileo bit 2-15: reserved for future GNSSes Note 1 | REL-7 |
|  |  |  |  | bit 2: SBAS (WAAS, EGNOS, MSAS, GAGAN) bit 3: Modernized GPS (L1C, L2C, L5) bit 4: QZSS bit 5: GLONASS bit 6-15: reserved for future GNSSes Note 1 | REL-8 |
|  |  |  |  | bit 6: BDS bit 7-15: reserved for future GNSSes Note 1 | REL-12 |
| GANSS timing of Cell wanted | OP |  | Bit string(8) | Each bit refers to a GANSS. Bit 0 is for Galileo. Other bits are reserved. If one bit is set to one the SRNC wants the UE to report the SFN-GANSS timing of the reference cell. This is however optional in the UE. | REL-7 |
|  |  |  |  | Bit 1 is for Modernized GPS; Bit 2 is for QZSS; Bit 3 is for GLONASS; Other bits are reserved. If one bit is set to one the SRNC wants the UE to report the SFN-GANSS timing of the reference cell. This is however optional in the UE. If more than one bit is set to one, the UE will select the GANSS among the allowed GANSS for the SFN-GANSS timing of the reference cell in the measurement report. | Rel-8 |
|  |  |  |  | Bit 4 is for BDS; Other bits are reserved. If one bit is set to one the SRNC wants the UE to report the SFN-GANSS timing of the reference cell. This is however optional in the UE. If more than one bit is set to one, the UE will select the GANSS among the allowed GANSS for the SFN-GANSS timing of the reference cell in the measurement report. | REL-12 |
| GANSS Carrier-Phase Measurement Requested | OP |  | Bit string(8) | Each bit refers to a GANSS. Bit 0 is for Galileo. Other bits are reserved. This is however optional in the UE. | REL-7 |
|  |  |  |  | Bit 1 is for SBAS; Bit 2 is for Modernized GPS; Bit 3 is for QZSS; Bit 4 is for GLONASS; Other bits are reserved. This is however optional in the UE. | Rel-8 |
|  |  |  |  | Bit 5 is for BDS; Other bits are reserved. This is however optional in the UE. | REL-12 |
| GANSS Multi-frequency Measurement Requested | OP |  | Bit String(8) | Each bit refers to a GANSS. Bit 0 is for Galileo; Bit 1 is for SBAS; Bit 2 is for Modernized GPS; Bit 3 is for QZSS; Bit 4 is for GLONASS; Other bits are reserved. This is however optional in the UE. | REL-8 |
|  |  |  |  | Bit 5 is for BDS; Other bits are reserved. This is however optional in the UE. | REL-12 |
| Additional Positioning Methods | OP |  | Bit string (8) | For each bit, if set to ‘1’, indicates that respective Additional Positioning method is allowed. bit 0: Barometric Pressure bit 1: WLAN bit 2: Bluetooth bit 3: MBS bit 4-7: reserved for future methods | REL-13 |

| Condition | Explanation |
| --- | --- |
| Method Type | The IE is optional if the IE "Method Type" is "UE assisted"; otherwise it is mandatory present. |

NOTE 1:	Bit 0 of this bitmap shall not be the only one set to 1.

#### 10.3.7.112	TADV info

NOTE:	Only for 1.28 Mcps TDD.

TADV indicates the difference between the Rx timing and Tx timing of a UE.

| Information Element/group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| TADV | MP |  | Integer (0..2047, | As defined in [20]. | REL-4 |
|  |  |  | 2048..8191) |  | REL-7 |
| SFN | MP |  | Integer(0..4095) | SFN during which the TADV measurement was performed. | REL-4 |

#### 10.3.7.113	UTRA priority info list

Contains priority based reselection information for UTRA.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UTRA Serving Cell | MP |  |  |  | REL-8 |
| >priority | MP |  | Integer (0..<maxPrio–1>) | 0 is the lowest priority and maxPrio-1 is the highest | REL-8 |
| >Sprioritysearch1 | MP |  | Integer (0..62 by step of 2) | dB | REL-8 |
| >Sprioritysearch2 | MD |  | Integer (0..7 by step of 1) | dB, default value is 0 | REL-8 |
| >Threshserving,low | MP |  | Integer (0..62 by step of 2) | RSCP, dB | REL-8 |
| >Threshserving,low2 | MD |  | Integer (0..31) | Ec/N0, dB, default value is 0 | REL-9 |
| UTRAN FDD Frequencies | OP | 1 to < maxNumFDDFreqs> |  |  | REL-8 |
| >UARFCN | MP |  | Integer(0..16383) | It is always ensured by the UTRAN that more than one entry for the same physical frequency is not configured regardless of the UARFCN used to indicate this. | REL-8 |
| >priority | MP |  | Integer (0..<maxPrio–1>) | 0 is the lowest priority and maxPrio-1 is the highest. | REL-8 |
| >Threshx, high | MP |  | Integer (0..62 by step of 2) | RSCP, dB | REL-8 |
| >Threshx, low | MP |  | Integer (0..62 by step of 2) | RSCP, dB | REL-8 |
| >QqualminFDD | MD |  | Integer (-24..0) | Ec/N0, [dB] Default value is Qqualmin for the serving cell | REL-8 |
| >QrxlevminFDD | MD |  | Integer (-119..-25 by step of 2) | RSCP, [dBm] Default value is Qrxlevmin for the serving cell | REL-8 |
| UTRAN TDD Frequencies | OP | 1 to < maxNumTDDFreqs> |  |  | REL-8 |
| >UARFCN | MP |  | Integer(0..16383) |  | REL-8 |
| >priority | MP |  | Integer (0..<maxPrio–1>) | 0 is the lowest priority and maxPrio-1 is the highest. | REL-8 |
| >Threshx, high | MP |  | Integer (0..62 by step of 2) | RSCP, dB | REL-8 |
| >Threshx, low | MP |  | Integer (0..62 by step of 2) | RSCP, dB | REL-8 |
| >QrxlevminTDD | MD |  | Integer (-119..-25 by step of 2) | RSCP, [dBm] Default value is Qrxlevmin for the serving cell | REL-8 |

NOTE:	The network should ensure that priorities for different Radio Access Technologies are always different.

#### 10.3.7.114	GSM priority info list

Contains priority based reselection information for GSM.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GSM Priority Info | MP | 1 to <maxNumGSMCellGroup> |  |  | REL-8 |
| >GSM cell group List | MP |  | GSM cell group 10.3.7.9a |  | REL-8 |
| >priority | MP |  | Integer (0..<maxPrio–1>) | 0 is the lowest priority and maxPrio-1 is the highest. | REL-8 |
| >QrxlevminGSM | MP |  | Integer (-115..-25 by step of 2) | GSM RSSI, [dBm] | REL-8 |
| >Threshx, high | MP |  | Integer (0..62 by step of 2) | GSM RSSI, dB | REL-8 |
| >Threshx, low | MP |  | Integer (0..62 by step of 2) | GSM RSSI, dB | REL-8 |

NOTE:	The network should ensure that priorities for different Radio Access Technologies are always different.

#### 10.3.7.115	E-UTRA frequency and priority info list

Contains information about neighbour E-UTRA frequencies (with associated blacklists), together with priority based reselection information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-UTRA frequency and priority |  | 1 to <maxNumEUTRAFreqs> |  |  | REL-8 |
| >EARFCN | MP |  | Integer(0..65535) | EARFCN of the downlink carrier frequency [64]. For UTRAN FDD, it is always ensured by the UTRAN that more than one entry for the same physical frequency is not configured regardless of the EARFCN used to indicate this. | REL-8 |
| >Measurement Bandwidth | MD |  | Enumerated(6, 15, 25, 50, 75, 100) | Measurement bandwidth information common for all neighbouring cells on the carrier frequency. It is defined by the parameter Transmission Bandwidth Configuration, NRB [36.104]. The values indicate the number of resource blocks over which the UE could measure. Default value is 6. | REL-8 |
| >priority | MP |  | Integer (0..<maxPrio–1>) | 0 is the lowest priority and maxPrio-1 is the highest. | REL-8 |
| >subpriority | CV-E-UTRA-priority |  | Enumerated(oDot2, oDot4, oDot6, oDot8) | Fractional priority value. If present, this value is added to the value of IE "priority". | REL-13 |
| >QrxlevminEUTRA | MP |  | Integer (-140..-44 by step of 2) | RSRP, [dBm] | REL-8 |
| >Threshx, high | MP |  | Integer (0..62 by step of 2) | RSRP, dB | REL-8 |
| >Threshx, low | MP |  | Integer (0..62 by step of 2) | RSRP, dB | REL-8 |
| >QqualminEUTRA | MD |  | Integer (-34..-3) | RSRQ, dB default value is negative infinity | REL-9 |
| >Threshx, high2 | OP |  | Integer (0..31) | RSRQ, dB | REL-9 |
| >Threshx, low2 | OP |  | Integer (0..31) | RSRQ, dB | REL-9 |
| >QqualminEUTRA-WB | CV-WB-RSRQ |  | Integer (-34..-3) | RSRQ, dB, If this IE is present, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [74] | REL-11 |
| >QqualminRSRQ-OnAllSymbols | CV-Thresh2 |  | Integer (-34..-3) | RSRQ, dB, If this IE is present, the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols in accordance with TS 25.215 [7] and TS 25.225 [8] | REL-12 |
| >Blacklisted cells per freq list | OP | 1 to <maxEUTRACellPerFreq> |  |  | REL-8 |
| >>Physical Cell identity | MP |  | Integer (0..503) | A list of blacklisted cells can be signalled per frequency | REL-8 |
| >E-UTRA detection | MP |  | Boolean | ‘TRUE’ means that the UE may detect the presence of a E-UTRA cell and report to NAS. NOTE 1. | REL-8 |
| >Reduced measurement performance | OP |  | Enumerated (TRUE) | TRUE indicates that the frequency is configured for reduced measurement performance [19]. Absense indicates that the frequency is configured for normal measurement performance [19]. | REL-12 |
| Number of applicable EARFCN | CV-EARFCNBands |  | Integer (0..7) | The number N of EARFCN in "E-UTRA frequency and priority" which are applicable. If present this IE indicates the first N occurrences of EARFCN in "E-UTRA frequency and priority" which shall be read by UE. | REL-11 |
| E-UTRA frequency and priority extension | OP | 1 to <maxNumEUTRAFreqs> |  | NOTE 2 | REL-11 |
| >EARFCN extension | MP |  | Integer (65536..262143) | EARFCN of the downlink carrier frequency [64]. For UTRAN FDD, it is always ensured by the UTRAN that more than one entry for the same physical frequency is not configured regardless of the EARFCN used to indicate this | REL-11 |
| >Measurement Bandwidth | MD |  | Enumerated(6, 15, 25, 50, 75, 100) | Measurement bandwidth information common for all neighbouring cells on the carrier frequency. It is defined by the parameter Transmission Bandwidth Configuration, NRB [36.104]. The values indicate the number of resource blocks over which the UE could measure. Default value is 6. | REL-11 |
| >priority | MP |  | Integer (0..<maxPrio–1>) | 0 is the lowest priority and maxPrio-1 is the highest. | REL-11 |
| >subpriority | CV-E-UTRA-priority |  | Enumerated(oDot2, oDot4, oDot6, oDot8) | Fractional priority value. If present, this value is added to the value of IE "priority". | REL-13 |
| >QrxlevminEUTRA | MP |  | Integer (-140..-44 by step of 2) | RSRP, [dBm] | REL-11 |
| >Threshx, high | MP |  | Integer (0..62 by step of 2) | RSRP, dB | REL-11 |
| >Threshx, low | MP |  | Integer (0..62 by step of 2) | RSRP, dB | REL-11 |
| >QqualminEUTRA | MD |  | Integer (-34..-3) | RSRQ, dB default value is negative infinity | REL-11 |
| >Threshx, high2 | OP |  | Integer (0..31) | RSRQ, dB | REL-11 |
| >Threshx, low2 | OP |  | Integer (0..31) | RSRQ, dB | REL-11 |
| >QqualminEUTRA-WB | CV-WB-RSRQ |  | Integer (-34..-3) | RSRQ, dB, If this IE is present, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [74] | REL-11 |
| >QqualminRSRQ-OnAllSymbols | CV-Thresh2 |  | Integer (-34..-3) | RSRQ, dB, If this IE is present, the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols in accordance with TS 25.215 [7] and TS 25.225 [8] | REL-12 |
| >Blacklisted cells per freq list | OP | 1 to <maxEUTRACellPerFreq> |  |  | REL-11 |
| >>Physical Cell identity | MP |  | Integer (0..503) | A list of blacklisted cells can be signalled per frequency | REL-11 |
| >E-UTRA detection | MP |  | Boolean | ‘TRUE’ means that the UE may detect the presence of a E-UTRA cell and report to NAS. NOTE 1. | REL-11 |
| >Reduced measurement performance | OP |  | Enumerated (TRUE) | TRUE indicates that the frequency is configured for reduced measurement performance [19]. Absense indicates that the frequency is configured for normal measurement performance [19]. | REL-12 |
| Multiple E-UTRA frequency info list | OP | 1 to <maxNumEUTRAFreqs> |  | Each entry in the list corresponds to an entry in the "E-UTRA frequency and priority" IE, or an entry in the "E-UTRA frequency and priority extension" IE. NOTE 3. | REL-10 |
| >Multiple E-UTRA frequency band indicator list | OP | 1 to <maxMultipleFrequencyBandsEUTRA> |  | Indicates, per neighbour EARFCN, a list of frequency bands which the E-UTRA cells belong to. Absence of this IE indicates that there are no Multiple Bands for the particular EARFCN. | REL-10 |
| >>E-UTRA Frequency band | MP |  | Integer (1..64) | As defined in [64]. | REL-10 |
| Multiple E-UTRA frequency info extension list | OP | 1 to <maxNumEUTRAFreqs> |  | Each entry in the list corresponds to an entry in the "E-UTRA frequency and priority" IE or the "E-UTRA frequency and priority extension" IE. NOTE 3 | REL-11 |
| >Multiple E-UTRA frequency band indicator extension list | OP | 1 to <maxMultipleFrequencyBandsEUTRA> |  | Indicates, per neighbour EARFCN, a list of frequency bands which the E-UTRA cells belong to. Absence of this IE indicates that there are no Multiple Bands for the particular EARFCN. | REL-11 |
| >>E-UTRA Frequency band extension | MP |  | Integer (65..256) | As defined in [64]. | REL-11 |
| NOTE 1: If the value of "E-UTRA detection" is not identical across the frequency layers, UE behaviour is unspecified. |  |  |  |  |  |
| NOTE 2: The number of instances of IE "E-UTRA frequency and priority extension" shall be less than or equal to maxNumEUTRAFreqs – the value specified in IE "Number of applicable EARFCN" |  |  |  |  |  |
| NOTE 3: The number of bands signalled in "Multiple E-UTRA frequency band indicator list" and "Multiple E-UTRA frequency band indicator extension list" combined, for the same occurrence of EARFCN or EARFCN extension, should be less than or equal to maxMultipleFrequencyBandsEUTRA. |  |  |  |  |  |

NOTE:	The network should ensure that priorities for different Radio Access Technologies are always different.

NOTE:	The network should ensure that priorities for which both Threshx,high2 and Threshx,low2 are provided are always different from the priorities for which both Threshx,high2 and Threshx,low2 are not provided.

| Condition | Explanation |
| --- | --- |
| EARFCNBands | This IE is mandatory present when the IE "E-UTRA frequency and priority extension" is present, otherwise it is not needed. |
| WB-RSRQ | The field is optionally present if the measurement bandwidth indicated by Measurement Bandwidth is 50 resource blocks or larger. Otherwise it is not needed. |
| Thresh2 | This IE is optionally present if both the IEs "Thresh x, high2" and "Thresh x, low2" are present, otherwise it is not needed. If this IE is present for any E-UTRA frequency, it is mandatory present for all the E-UTRA frequencies. |
| E-UTRA-priority | This IE is optionally present if IE "priority" is present. Otherwise it is not needed. |

#### 10.3.7.116		Intra-frequency cell info list on secondary UL frequency

NOTE:	For FDD only.

Contains the information for the list of measurement objects for an intra-frequency measurement on secondary uplink frequency.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Intra-frequency cell removal | OP |  |  | Absence of this IE is equivalent to choice "Remove no intra-frequency cells". | REL-9 |
| >Remove all intra-frequency cells |  |  |  | (no data) | REL-9 |
| >Remove some intra-frequency cells |  |  |  |  | REL-9 |
| >>Removed intra-frequency cells | MP | 1 to <maxCellMeasOnSecULFreq> |  |  | REL-9 |
| >>>Intra-frequency cell id | MP |  | Integer(0 .. < maxCellMeasOnSecULFreq > - 1) |  | REL-9 |
| >Remove no intra-frequency cells |  |  |  | (no data) | REL-9 |
| New intra-frequency cells | OP | 1 to <maxCellMeasOnSecULFreq> |  |  | REL-9 |
| >Intra-frequency cell id | OP |  | Integer(0 .. < maxCellMeasOnSecULFreq > - 1) |  | REL-9 |
| >Cell info | MP |  | Cell info 10.3.7.2 |  | REL-9 |
| Cells for measurement | OP | 1 to <maxCellMeasOnSecULFreq> |  |  | REL-9 |
| >Intra-frequency cell id | MP |  | Integer(0 .. <maxCellMeasOnSecULFreq>-1) |  | REL-9 |

#### 10.3.7.117	Cell measurement event results on secondary UL frequency

NOTE:	For FDD only.

Includes non-frequency related cell reporting quantities.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Intra-frequency event identity | MP |  | Intra-frequency event identity 10.3.7.34 |  | REL-9 |
| CHOICE mode | MP |  |  |  |  |
| > FDD |  |  |  |  |  |
| >>Primary CPICH info | MP | 1 to < maxCellMeasOnSecUlFreq > | Primary CPICH info 10.3.6.60 |  | REL-9 |
| > Other |  |  |  | One spare value is needed | REL-9 |

#### 10.3.7.118		Measured results on secondary UL frequency

NOTE:	For FDD only.

Includes non-frequency related cell reporting quantities.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Cell measured results on secondary UL frequency | MP | 1 to < maxCellMeasOnSecULFreq > |  |  | REL-9 |
| >Cell measured results | MP |  | Cell measured results 10.3.7.3 | Only cells for which all reporting quantities are available should be included. | REL-9 |

#### 10.3.7.119	Intra-frequency measurement reporting criteria on secondary UL frequency

NOTE:	For FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Frequency Info | MP |  | Frequency info 10.3.6.36 | NOTE2 | REL-9 |
| Parameters required for each event | MP | 1 to <maxMeasEventOnSecULFreq> |  |  | REL-9 |
| >Intra-frequency event identity | MP |  | Intra-frequency event identity 10.3.7.34 | NOTE1 | REL-9 |
| >Triggering condition 1 | CV–clause 0 |  | Enumerated(Active set cells, Monitored set cells, Active set cells and monitored set cells) | Indicates which cells can trigger the event | REL-9 |
| >Triggering condition 2 | CV–clause 6 |  | Enumerated(Active set cells, Monitored set cells, Active set cells and monitored set cells, Detected set cells, Detected set cells and monitored set cells) | Indicates which cells can trigger the event | REL-9 |
| >Reporting Range Constant | CV–clause 2 |  | Real(0..14.5 by step of 0.5) | In dB. In event 1a,1b. | REL-9 |
| >Cells forbidden to affect Reporting range on secondary UL frequency | CV–clause 1 | 1 to < maxCellMeasOnSecULFreq > |  | In event 1a,1b | REL-9 |
| >>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-9 |
| >W | CV–clause 2 |  | Real(0.0..2.0 by step of 0.1) |  | REL-9 |
| >Hysteresis | MP |  | Real(0..7.5 by step of 0.5) | In dB. | REL-9 |
| >Threshold used frequency | CV-clause 3 |  | Integer (-120..165) | Range used depend on measurement quantity. CPICH RSCP -120..-25 dBm CPICH Ec/No -24..0 dB Pathloss 30..165dB ISCP -120..-25 dBm | REL-9 |
| >Reporting deactivation threshold | CV–clause 4 |  | Integer(0, 1, 2, 3, 4, 5, 6, 7) | In event 1a Indicates the maximum number of cells allowed in the active set in order for event 1a to occur. 0 means not applicable . | REL-9 |
| >Replacement activation threshold | CV-clause 5 |  | Integer(0, 1, 2, 3, 4, 5, 6, 7) | In event 1c Indicates the minimum number of cells allowed in the active set in order for event 1c to occur. 0 means not applicable | REL-9 |
| >Time to trigger | MP |  | Time to trigger 10.3.7.64 | Indicates the period of time during which the event condition has to be satisfied, before sending a Measurement Report. Time in ms | REL-9 |
| >Amount of reporting | CV–clause 7 |  | Integer(1, 2, 4, 8, 16, 32, 64, Infinity) |  | REL-9 |
| >Reporting interval | CV–clause 7 |  | Integer(0, 250, 500, 1000, 2000, 4000, 8000, 16000) | Indicates the interval of periodical reporting when such reporting is triggered by an event. Interval in milliseconds. 0 means no periodical reporting. | REL-9 |
| >Reporting cell status | OP |  | Reporting cell status 10.3.7.61 |  | REL-9 |
| >Periodical reporting information-1b | CV–clause 9 |  | Periodical reporting info-1b 10.3.7.53aa |  | REL-9 |

| Condition | Explanation |
| --- | --- |
| Clause 0 | The IE is mandatory present if the IE "Intra-frequency event identity" is set to "1b" or "1f", otherwise the IE is not needed. |
| Clause 1 | The IE is optional if the IE "Intra-frequency event identity" is set to "1a" or "1b", otherwise the IE is not needed. |
| Clause 2 | The IE is mandatory present if the IE "Intra-frequency event identity" is set to "1a" or "1b", otherwise the IE is not needed. |
| Clause 3 | The IE is mandatory present if the IE "Intra-frequency event identity" is set to "1e", "1f", otherwise the IE is not needed. |
| Clause 4 | The IE is mandatory present if the IE "Intra-frequency event identity" is set to "1a", otherwise the IE is not needed. |
| Clause 5 | The IE is mandatory present if the IE "Intra-frequency event identity" is set to "1c", otherwise the IE is not needed. |
| Clause 6 | The IE is mandatory present if the IE "Intra-frequency event identity" is set to "1a" or "1e", otherwise the IE is not needed. |
| Clause 7 | The IE is mandatory present if the IE "Intra-frequency event identity" is set to "1a", "1c", otherwise the IE is not needed. |
| Clause 9 | The IE is optional if the IE "Intra-frequency event identity" is set to "1b", otherwise the IE is not needed. |

NOTE1:	Event 1d and 1j shall not be applied for the secondary uplink frequency.

NOTE2:	If the IE "Frequency info" is different from the current downlink frequency associated with the secondary UL frequency, the UE behaviour is unspecified.

#### 10.3.7.120	CSG Inter-frequency cell info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CSG Frequency info | MP | 1 to < maxFreq > |  |  | REL-9 |
| >Frequency info | MP |  | Frequency info 10.3.6.36 |  | REL-9 |
| >CSG Inter-frequency cell info for the frequency | MP |  | 10.3.7.121a |  | REL-9 |

#### 10.3.7.121	CSG Intra-frequency cell info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CSG Intra-frequency cell info | MP |  | 10.3.7.121a |  | REL-9 |

#### 10.3.7.121a	CSG cell info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CSG cell info list | MP | 1 to <maxMeasCSGRange> |  |  | REL-9 |
| >CHOICE mode | MP |  |  |  | REL-9 |
| >>FDD |  |  |  |  | REL-9 |
| >>>Start PSC | MP |  | Primary CPICH info 10.3.6.60 | The value of this IE specifies the start PSC of the PSC range. | REL-9 |
| >>>Number of PSCs | MP |  | Integer (1..512) | This IE specifies the number of PSCs. | REL-9 |

#### 10.3.7.122	CSG Proximity Indication

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CSG Proximity Indication | MP |  | Enumerated (entering, leaving) |  | REL-9 |
| CHOICE Radio Access Technology |  |  |  |  | REL-9 |
| >UTRA |  |  |  |  | REL-9 |
| >>CSG Frequency info for UTRA | MP |  | Frequency info 10.3.6.36 | Indicates the UARFCN according to a band it previously considered suitable for accessing (one of) the CSG member cell(s), for which the proximity indication is sent. | REL-9 |
| >E-UTRA |  |  |  |  | REL-9 |
| >>CSG Frequency info for E-UTRA | MP |  | Integer (0..65535) | EARFCN of the downlink carrier frequency [64]. If the IE indicates a value of 65535, then the EARFCN for this instance should be read from the corresponding instance of IE "EARFCN extension". Indicates the EARFCN according to a band it previously considered suitable for accessing (one of) the CSG member cell(s), for which the proximity indication is sent. | REL-9 |
| >>EARFCN extension | OP |  | Integer (65536..262143) | EARFCN of the downlink carrier frequency [64]. Indicates the EARFCN according to a band it previously considered suitable for accessing (one of) the CSG member cell(s), for which the proximity indication is sent. | REL-11 |

#### 10.3.7.123	CSG Proximity detection

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UTRA CSG Proximity detection | MD |  | Enumerated (enable) | Absence of this IE implies that CSG proximity detection function for UTRA cells is disabled. | REL-9 |
| E-UTRA CSG Proximity detection | MD |  | Enumerated (enable) | Absence of this IE implies that CSG proximity detection function for E-UTRA cells is disabled. | REL-9 |

#### 10.3.7.124	Inter-frequency SI Acquisition

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Frequency info | MP |  | Frequency info 10.3.6.36 |  | REL-9 |
| CHOICE mode | MP |  |  |  | REL-9 |
| >FDD |  |  |  |  | REL-9 |
| >>Primary Scrambling Code | MP |  | Primary CPICH info 10.3.6.60 |  | REL-9 |
| >TDD |  |  | NULL |  | REL-9 |

#### 10.3.7.125	Intra-frequency SI Acquisition

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE mode | MP |  |  |  | REL-9 |
| >FDD |  |  |  |  | REL-9 |
| >>Intra-frequency SI acquisition Info | MD | 1 to <maxSIrequest> |  | Absence of this IE implies that UE should perform SI acquisition for all PSC ranges specified in the "CSG Intra-frequency cell info" IE. UE is at minimum required to acquire the SI of the strongest PSC in those configured PSC ranges. | REL-9 |
| >>>Primary Scrambling Code | MP |  | Primary CPICH info 10.3.6.60 |  | REL-9 |
| >TDD |  |  | NULL |  | REL-9 |

#### 10.3.7.126	CELL_DCH measurement occasion info LCR

NOTE:	Only for 1.28 Mcps TDD in CELL_DCH state.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CELL_DCH measurement occasion pattern sequence |  | 1 to <maxDCHMeasurementOccasionPatternSequence> |  |  |  |
| >Pattern sequence identifier | MP |  | Integer(0.. maxDCHMeasurementOccasionPatternSequence-1) |  | REL-9 |
| >Status Flag | MP |  | Enumerated(activate, deactivate) | This flag indicates whether the measurement occasion pattern sequence shall be activated or deactivated. | REL-9 |
| >Measurement purpose | OP |  | BIT STRING (5) | Measurement Purpose. Bit 0 is for Inter-frequency measurement. Bit 1 is for GSM carrier RSSI measurement. Bit 2 is for Initial BSIC identification. Bit 3 is for BSIC re-confirmation. Bit 4 is for E-UTRA measurement. The value 1 of a bit means that the measurement occasion pattern sequence is applicable for the corresponding type of measurement. Bit 0 is the first/leftmost bit of the bit string. | REL-9 |
| >Measurement occasion pattern sequence parameters | OP |  |  |  | REL-9 |
| >>k | MP |  | Integer(1..9) | CELL_DCH measurement occasion cycle length coefficient. The actual measurement occasion period equal to 2k radio frames. | REL-9 |
| >>Offset | MP |  | Integer(0..511) | In frames. The measurement occasion position in the measurement period. | REL-9 |
| >>M_Length | MP |  | Integer(1..512) | The measurement occasion length in frames starting from the Offset. | REL-9 |
| >>Timeslot Bitmap | MD |  | Bit string (7) | Bitmap indicating which of the timeslot(s) is/are allocated for measurement. Bit 0 is for timeslot 0. Bit 1 is for timeslot 1. Bit 2 is for timeslot 2. Bit 3 is for timeslot 3. Bit 4 is for timeslot 4. Bit 5 is for timeslot 5. Bit 6 is for timeslot 6. The value 0 of a bit means the corresponding timeslot is not used for measurement. The value 1 of a bit means the corresponding timeslot is used for measurement. Bit 0 is the first/leftmost bit of the bit string. Absence of this IE means all the timeslots can be used for measurement. | REL-9 |

#### 10.3.7.127	E-UTRA SI Acquisition

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-UTRA carrier frequency | MP |  | Integer (0..65535) | EARFCN of the downlink carrier frequency [64]. If the IE indicates a value of 65535, then the EARFCN for this instance should be read from the corresponding instance of IE "EARFCN extension" | REL-9 |
| EARFCN extension | OP |  | Integer (65536..262143) | EARFCN of the downlink carrier frequency [64]. | REL-11 |
| Physical Cell identity | MP |  | Integer (0..503) |  | REL-9 |

#### 10.3.7.128	E-UTRA Results for SI Acquisition

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CGI-Info | MP |  |  |  | REL-9 |
| >PLMN Identity | MP |  | PLMN identity 10.3.1.11 | This IE indicates the Primary PLMN | REL-9 |
| >Cell Identity | MP |  | Bit string (28) | Formatted and coded according to [67]. The first/leftmost bit of the bit string contains the most significant bit of the Cell Identity. | REL-9 |
| >Tracking Area Code | MP |  | Bit string (16) | Setting specified in [67]. The first/leftmost bit of the bit string contains the most significant bit of the Tracking Area Code. | REL-9 |
| CSG Identity | OP |  | CSG Identity 10.3.2.8 |  | REL-9 |
| CSG Member indication | OP |  | Enumerated(member) |  | REL-9 |
| CSG PLMN List | OP | 1 to 5 |  |  | REL-12 |
| >CSG PLMN Identity | MP |  | PLMN identity 10.3.1.11 |  | REL-12 |
| Primary PLMN Suitable | OP |  | Enumerated(true) |  | REL-12 |

#### 10.3.7.129	Logged Measurement Info-FDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Relative Time Stamp | MP |  | INTEGER (0..7200) | Indicates the time of logging measurement results, measured relative to the Absolute Time Info, in seconds. | REL-10 |
| Logged Measurements Serving Cell | MP |  |  |  | REL-10 |
| >PLMN Identity | MP |  | PLMN Identity 10.3.1.11 |  | REL-11 |
| >Cell ID | MP |  | Cell ID 10.3.2.2 |  | REL-10 |
| >CPICH RSCP | MP |  | Integer(0..91) | CPICH RSCP of the serving Cell, when available In dBm. According to CPICH_RSCP_LEV in [19]. Thirty-six spare values are needed. | REL-10 |
| >CPICH Ec/N0 | MP |  | Integer(0..49) | CPICH Ec/N0 of the serving Cell, when available In dB. According to CPICH_Ec/No in [19]. Fourteen spare values are needed. | REL-10 |
| Logged Measurements Intra Frequency Neighbouring Cells list | OP | 1.. MaxnumLoggedMeas |  | Measurements done on neighbouring cells | REL-10 |
| >Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-10 |
| >CPICH RSCP | MP |  | Integer(0..91) | In dBm. According to CPICH_RSCP_LEV in [19]. Thirty-six spare values are needed. | REL-10 |
| >CPICH Ec/N0 | MP |  | Integer(0..49) | CPICH Ec/N0 of the serving Cell, when available In dB. According to CPICH_Ec/No in [19]. Fourteen spare values are needed. | REL-10 |
| Logged Measurements Inter Frequency list | OP | 1.. MaxNumFDDFreqs |  | Measurements done on neighbouring cells | REL-10 |
| >Frequency Info | MP |  | Frequency Info 10.3.6.36 |  | REL-10 |
| > Logged Measurements Inter-frequency Neighbouring Cells list | MP | 1..MaxnumLoggedMeas |  |  | REL-10 |
| >>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-10 |
| >>CPICH RSCP | MP |  | Integer(0..91) | In dBm. According to CPICH_RSCP_LEV in [19]. Thirty-six spare values are needed. | REL-10 |
| >>CPICH Ec/N0 | MP |  | Integer(0..49) | CPICH Ec/N0 of the serving Cell, when available In dB. According to CPICH_Ec/No in [19]. Fourteen spare values are needed. | REL-10 |
| Logged Measurements Inter-RAT Neighbour cells list | OP |  |  |  | REL-10 |
| >Logged Measurements E-UTRA frequency list | OP | 1.. MaxNumEUTRAFreqs |  |  | REL-10 |
| >>E-UTRA Carrier Frequency | MP |  | Integer (0..65535) | EARFCN of the downlink carrier frequency [64]. If the IE indicates a value of 65535, then the EARFCN for this instance should be read from the corresponding instance of IE "EARFCN extension" in the Logged Measurements E-UTRA frequency extension list. | REL-10 |
| >> Logged Measurements E-UTRA Frequency Neighbours List | MP | 1.. MaxnumLoggedMeas |  |  | REL-10 |
| >>>Physical Cell Identity | MP |  | Integer (0..503) |  | REL-10 |
| >>>RSRP | MP |  | Integer (0..97) | RSRP is mapped to a value between 0 and 97 [36.133]. | REL-10 |
| >>>RSRQ | MP |  | Integer (0..33) | RSRQ_00 to RSRQ_33 in [36.133] are mapped to the value between 0 and 33. RSRQ_34 in [36.133] is mapped to the value 33. If the IE indicates a value of 0 and the RSRQ Extension IE is present, then the RSRQ for this instance should be read from the corresponding instance of IE "RSRQ extension", the range is from -30 to -1. If the IE indicates a value of 33 and the RSRQ Extension IE is present, then the RSRQ for this instance should be read from the corresponding instance of IE "RSRQ extension", the range is from 35 to 46. | REL-10 |
| >>>RSRQ Extension | OP |  | Integer (-30..46) | RSRQ_-30 to RSRQ_-1 in [36.133] are mapped to the value between -30 and -1. RSRQ_35 to RSRQ_46 in [36.133] are mapped to the value between 35 and 46. | REL-12 |
| >>RSRQ Type | OP |  |  |  | REL-12 |
| >>>All Symbols | MP |  | BOOLEAN | TRUE indicates use of all OFDM symbols when performing RSRQ measurements. | REL-12 |
| >>>Wide band | MP |  | BOOLEAN | TRUE indicates use of a wider bandwidth when performing RSRQ measurements. | REL-12 |
| >Logged Measurements E-UTRA frequency extension list | OP | 1.. MaxNumEUTRAFreqs |  |  | REL-11 |
| >>EARFCN extension | OP |  | Integer (65535..262143) | EARFCN of the downlink carrier frequency [64]. | REL-11 |
| >Logged Measurements GSM Neighbouring Cells list | OP | 1.. MaxnumLoggedMeas |  |  | REL-10 |
| >>BSIC | MP |  | BSIC 10.3.8.2 |  | REL-10 |
| >>Band indicator | MP |  | Enumerated (DCS 1800 band used, PCS 1900 band used) | Indicates how to interpret the BCCH ARFCN | REL-10 |
| >>BCCH ARFCN | MP |  | Integer (0..1023) | [45] | REL-10 |
| >>GSM carrier RSSI | MP | Bit string(6) | RXLEV is mapped to a value between 0 and 63, [46]. When mapping the RXLEV value to the RSSI bit string, the first/leftmost bit of the bit string contains the most significant bit. |  | REL-10 |
| CHOICE GNSS UE Position | OP |  |  | The position of the UE when the measurement is made | REL-10 |
| >Ellipsoid |  |  |  |  | REL-10 |
| >>Ellipsoid point | MP |  | Ellipsoid point 10.3.8.4a |  | REL-10 |
| >Ellipsoid with altitude |  |  |  |  | REL-10 |
| >>Ellipsoid point with altitude | MP |  | Ellipsoid point with altitude 10.3.8.4b |  | REL-10 |
| >Ellipsoid with uncertainty circle |  |  |  |  | REL-11 |
| >>Ellipsoid point with uncertainty circle | MP |  | Ellipsoid point with uncertainty circle 10.3.8.4d |  | REL-11 |
| >Ellipsoid with uncertainty ellipse |  |  |  |  | REL-11 |
| >> Ellipsoid point with uncertainty ellipse | MP |  | Ellipsoid point with uncertainty ellipse 10.3.8.4e |  | REL-11 |
| >Ellipsoid with altitude and uncertainty ellipsoid |  |  |  |  | REL-11 |
| >>Ellipsoid point with altitude and uncertainty ellipsoid | MP |  | Ellipsoid point with altitude and uncertainty ellipsoid 10.3.8.4c |  | REL-11 |

#### 10.3.7.129a	Logged Connection Establishment Failure Info-FDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Access Information | MP |  |  |  | REL-11 |
| >Number Of RRC Msg Transmitted | MP |  | Integer (0..8) | Number of times the RRC message is sent by the UE, e.g. after receiving ACK and AICH. | REL-11 |
| Logged Measurements Failed Cell | MP |  |  |  | REL-11 |
| >Cell ID | MP |  | Cell ID 10.3.2.2 |  | REL-11 |
| >CPICH RSCP | MP |  | Integer(0..91) | CPICH RSCP of the serving Cell, when available In dBm. According to CPICH_RSCP_LEV in [19]. Thirty-six spare values are needed. | REL-11 |
| >CPICH Ec/N0 | MP |  | Integer(0..49) | CPICH Ec/N0 of the serving Cell, when available In dB. According to CPICH_Ec/No in [19]. Fourteen spare values are needed. | REL-11 |
| Logged Measurements Intra Frequency Neighbouring Cells list | OP | 1.. MaxnumLoggedMeas |  | Measurements done on neighbouring cells | REL-11 |
| >Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-11 |
| >CPICH RSCP | MP |  | Integer(0..91) | In dBm. According to CPICH_RSCP_LEV in [19]. Thirty-six spare values are needed. | REL-11 |
| >CPICH Ec/N0 | MP |  | Integer(0..49) | CPICH Ec/N0 of the serving Cell, when available In dB. According to CPICH_Ec/No in [19]. Fourteen spare values are needed. | REL-11 |
| Logged Measurements Inter Frequency list | OP | 1.. MaxNumFDDFreqs |  | Measurements done on neighbouring cells | REL-11 |
| >Frequency Info | MP |  | Frequency Info 10.3.6.36 |  | REL-11 |
| >Logged Measurements Inter-frequency Neighbouring Cells list | MP | 1..MaxnumLoggedMeas |  |  | REL-11 |
| >>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-11 |
| >>CPICH RSCP | MP |  | Integer(0..91) | In dBm. According to CPICH_RSCP_LEV in [19]. Thirty-six spare values are needed. | REL-11 |
| >>CPICH Ec/N0 | MP |  | Integer(0..49) | CPICH Ec/N0 of the serving Cell, when available In dB. According to CPICH_Ec/No in [19]. Fourteen spare values are needed. | REL-11 |
| Logged Measurements Inter-RAT Neighbour cells list | OP |  |  |  | REL-11 |
| >Logged Measurements E-UTRA frequency list | OP | 1.. MaxNumEUTRAFreqs |  |  | REL-11 |
| >>E-UTRA Carrier Frequency | MP |  | Integer (0..65535) | EARFCN of the downlink carrier frequency [64]. If the IE indicates a value of 65535, then the EARFCN for this instance should be read from the corresponding instance of IE "EARFCN extension" in the Logged Measurements E-UTRA frequency extension list. | REL-11 |
| >>Logged Measurements E-UTRA Frequency Neighbours List | MP | 1.. MaxnumLoggedMeas |  |  | REL-11 |
| >>>Physical Cell Identity | MP |  | Integer (0..503) |  | REL-11 |
| >>>RSRP | MP |  | Integer (0..97) | RSRP is mapped to a value between 0 and 97 [36.133]. | REL-11 |
| >>>RSRQ | MP |  | Integer (0..33) | RSRQ_00 to RSRQ_33 in [36.133] are mapped to the value between 0 and 33. RSRQ_34 in [36.133] is mapped to the value 33. If the IE indicates a value of 0 and the RSRQ Extension IE is present, then the RSRQ for this instance should be read from the corresponding instance of IE "RSRQ extension", the range is from -30 to -1. If the IE indicates a value of 33 and the RSRQ Extension IE is present, then the RSRQ for this instance should be read from the corresponding instance of IE "RSRQ extension", the range is from 35 to 46. | REL-11 |
| >>>RSRQ Extension | OP |  | Integer (-30..46) | RSRQ_-30 to RSRQ_-1 in [36.133] are mapped to the value between -30 and -1. RSRQ_35 to RSRQ_46 in [36.133] are mapped to the value between 35 and 46. | REL-12 |
| >>RSRQ Type | OP |  |  |  | REL-12 |
| >>>All Symbols | MP |  | BOOLEAN | TRUE indicates use of all OFDM symbols when performing RSRQ measurements. | REL-12 |
| >>>Wide band | MP |  | BOOLEAN | TRUE indicates use of a wider bandwidth when performing RSRQ measurements. | REL-12 |
| >Logged Measurements E-UTRA frequency extension list | OP | 1.. MaxNumEUTRAFreqs |  |  | REL-11 |
| >>EARFCN extension | OP |  | Integer (65535..262143) | EARFCN of the downlink carrier frequency [64]. | REL-11 |
| >Logged Measurements GSM Neighbouring Cells list | OP | 1.. MaxnumLoggedMeas |  |  | REL-11 |
| >>BSIC | MP |  | BSIC 10.3.8.2 |  | REL-11 |
| >>Band indicator | MP |  | Enumerated (DCS 1800 band used, PCS 1900 band used) | Indicates how to interpret the BCCH ARFCN | REL-11 |
| >>BCCH ARFCN | MP |  | Integer (0..1023) | [45] | REL-11 |
| >>GSM carrier RSSI | MP | Bit string(6) | RXLEV is mapped to a value between 0 and 63, [46]. When mapping the RXLEV value to the RSSI bit string, the first/leftmost bit of the bit string contains the most significant bit. |  | REL-11 |
| Location Information | OP |  |  | The position of the UE when the measurement is made | REL-11 |
| >CHOICE Location | MP |  |  |  | REL-11 |
| >>Ellipsoid |  |  |  |  | REL-11 |
| >>>Ellipsoid point | MP |  | Ellipsoid point 10.3.8.4a |  | REL-11 |
| >>Ellipsoid with altitude |  |  |  |  | REL-11 |
| >>>Ellipsoid point with altitude | MP |  | Ellipsoid point with altitude 10.3.8.4b |  | REL-11 |
| >>Ellipsoid with uncertainty circle |  |  |  |  | REL-11 |
| >>>Ellipsoid point with uncertainty circle | MP |  | Ellipsoid point with uncertainty circle 10.3.8.4d |  | REL-11 |
| >>Ellipsoid with uncertainty ellipse |  |  |  |  | REL-11 |
| >>>Ellipsoid point with uncertainty ellipse | MP |  | Ellipsoid point with uncertainty ellipse 10.3.8.4e |  | REL-11 |
| >>Ellipsoid with altitude and uncertainty ellipsoid |  |  |  |  | REL-11 |
| >>>Ellipsoid point with altitude and uncertainty ellipsoid | MP |  | Ellipsoid point with altitude and uncertainty ellipsoid 10.3.8.4c |  | REL-11 |
| >Horizontal velocity | OP |  | Horizontal velocity 10.3.8.4h |  | REL-11 |

#### 10.3.7.130	Logged Measurement Info-TDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Relative Time Stamp | MP |  | INTEGER (0..7200) | Indicates the time of logging measurement results, measured relative to the Absolute Time Info, in seconds. | REL-10 |
| Logged Measurements Serving Cell | MP |  |  |  | REL-10 |
| > PLMN Identity | MP |  | PLMN Identity 10.3.1.11 |  | REL-11 |
| >Cell ID | MP |  | Cell ID 10.3.2.2 |  | REL-10 |
| >Primary CCPCH RSCP | MP |  | Integer(0..91) | In dBm. According to P-CCPCH_RSCP_LEV in [19] and [20]. Thirty-six spare values are needed. | REL-10 |
| Logged Measurements Intra Frequency list | OP | 1.. MaxnumLoggedMeas |  | Measurements done on neighbouring cells | REL-10 |
| >Cell parameters Id | MP |  | Cell parameters Id 10.3.6.9 |  | REL-10 |
| >Primary CCPCH RSCP | MP |  | Integer(0..91) | In dBm. According to P-CCPCH_RSCP_LEV in [19] and [20]. Thirty-six spare values are needed. | REL-10 |
| Logged Measurements Inter Frequency Neighbouring Cells list | OP | 1.. MaxNumTDDFreqs |  | Measurements done on neighbouring cells | REL-10 |
| >Frequency Info | MP |  | Frequency Info 10.3.6.36 |  | REL-10 |
| >Logged Measurements Inter-frequency Neighbouring Cells list | MP | 1.. MaxnumLoggedMeas |  |  | REL-10 |
| >>Cell parameters Id | MP |  | Cell parameters Id 10.3.6.9 |  | REL-10 |
| >>Primary CCPCH RSCP | MP |  | Integer(0..91) | In dBm. According to P-CCPCH_RSCP_LEV in [19] and [20]. Thirty-six spare values are needed. | REL-10 |
| Logged Measurements Inter-RAT Neighbour cells list | OP |  |  |  | REL-10 |
| >Logged Measurements E-UTRA frequency list | OP | 1.. MaxNumEUTRAFreqs |  |  | REL-10 |
| >>E-UTRA Carrier Frequency | MP |  | Integer (0..65535) | EARFCN of the downlink carrier frequency [64]. If the IE indicates a value of 65535, then the EARFCN for this instance should be read from the corresponding instance of IE "EARFCN extension" in the Logged Measurements E-UTRA frequency extension list. | REL-10 |
| >>Logged Measurements E-UTRA Frequency Neighbours List | MP | 1.. MaxnumLoggedMeas |  |  | REL-10 |
| >>>Physical Cell Identity | MP |  | Integer (0..503) |  | REL-10 |
| >>>RSRP | MP |  | Integer (0..97) | RSRP is mapped to a value between 0 and 97 [36.133]. | REL-10 |
| >>>RSRQ | MP |  | Integer (0..33) | RSRQ_00 to RSRQ_33 in [36.133] are mapped to the value between 0 and 33. RSRQ_34 in [36.133] is mapped to the value 33. If the IE indicates a value of 0 and the RSRQ Extension IE is present, then the RSRQ for this instance should be read from the corresponding instance of IE "RSRQ extension", the range is from -30 to -1. If the IE indicates a value of 33 and the RSRQ Extension IE is present, then the RSRQ for this instance should be read from the corresponding instance of IE "RSRQ extension", the range is from 35 to 46. | REL-10 |
| >>>RSRQ Extension | OP |  | Integer (-30..46) | RSRQ_-30 to RSRQ_46 in [36.133] are mapped to the value between -30 and 46. | REL-12 |
| >>RSRQ Type | OP |  |  |  | REL-12 |
| >>>All Symbols | MP |  | BOOLEAN | TRUE indicates use of all OFDM symbols when performing RSRQ measurements. | REL-12 |
| >>>Wide band | MP |  | BOOLEAN | TRUE indicates use of a wider bandwidth when performing RSRQ measurements. | REL-12 |
| >Logged Measurements E-UTRA frequency extension list | OP | 1.. MaxNumEUTRAFreqs |  |  | REL-11 |
| >>EARFCN extension | OP |  | Integer (65536..262143) | EARFCN of the downlink carrier frequency [64]. | REL-11 |
| >Logged Measurements GSM Neighbouring Cells list | OP | 1.. MaxnumLoggedMeas |  |  | REL-10 |
| >>BSIC | MP |  | BSIC 10.3.8.2 |  | REL-10 |
| >>Band indicator | MP |  | Enumerated (DCS 1800 band used, PCS 1900 band used) | Indicates how to interpret the BCCH ARFCN | REL-10 |
| >>BCCH ARFCN | MP |  | Integer (0..1023) | [45] | REL-10 |
| >>GSM carrier RSSI | MP | Bit string(6) | RXLEV is mapped to a value between 0 and 63, [46]. When mapping the RXLEV value to the RSSI bit string, the first/leftmost bit of the bit string contains the most significant bit. |  | REL-10 |
| CHOICE GNSS UE Position | OP |  |  | The position of the UE when the measurement is made | REL-10 |
| >Ellipsoid |  |  |  |  | REL-10 |
| >>Ellipsoid point | MP |  | Ellipsoid point 10.3.8.4a |  | REL-10 |
| >Ellipsoid with altitude |  |  |  |  | REL-10 |
| >>Ellipsoid point with altitude | MP |  | Ellipsoid point with altitude 10.3.8.4b |  | REL-10 |
| >Ellipsoid with uncertainty circle |  |  |  |  | REL-11 |
| >>Ellipsoid point with uncertainty circle | MP |  | Ellipsoid point with uncertainty circle 10.3.8.4d |  | REL-11 |
| >Ellipsoid with uncertainty ellipse |  |  |  |  | REL-11 |
| >> Ellipsoid point with uncertainty ellipse | MP |  | Ellipsoid point with uncertainty ellipse 10.3.8.4e |  | REL-11 |
| >Ellipsoid with altitude and uncertainty ellipsoid |  |  |  |  | REL-11 |
| >>Ellipsoid point with altitude and uncertainty ellipsoid | MP |  | Ellipsoid point with altitude and uncertainty ellipsoid 10.3.8.4c |  | REL-11 |

#### 10.3.7.130a	Logged Connection Establishment Failure Info-TDD

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Access Information | MP |  |  |  | REL-11 |
| >Number Of RRC Msg Transmitted | MP |  | Integer (0..8) | Number of times the RRC message is sent by the UE, e.g. after receiving FPACH. | REL-11 |
| >FPACH Received | MP |  | BOOLEAN | TRUE indicates that FPACH has been successfully received at least once. | REL-11 |
| >E-RUCCH Failure | OP |  | BOOLEAN | TRUE indicates the failure of the E-RUCCH transmission has been detected | REL-11 |
| Logged Measurements Failed Cell | MP |  |  |  | REL-11 |
| >Cell ID | MP |  | Cell ID 10.3.2.2 |  | REL-11 |
| >Primary CCPCH RSCP | MP |  | Integer(0..91) | In dBm. According to P-CCPCH_RSCP_LEV in [19] and [20]. Thirty-six spare values are needed. | REL-11 |
| Logged Measurements Intra Frequency Cells list | OP | 1.. MaxnumLoggedMeas |  | Measurements done on neighbouring cells | REL-11 |
| >Cell parameters Id | MP |  | Cell parameters Id 10.3.6.9 |  | REL-11 |
| >Primary CCPCH RSCP | MP |  | Integer(0..91) | In dBm. According to P-CCPCH_RSCP_LEV in [19] and [20]. Thirty-six spare values are needed. | REL-11 |
| Logged Measurements Inter Frequency Neighbouring Cells list | OP | 1.. MaxNumTDDFreqs |  | Measurements done on neighbouring cells | REL-11 |
| >Frequency Info | MP |  | Frequency Info 10.3.6.36 |  | REL-11 |
| >Logged Measurements Inter-frequency Neighbouring Cells list | MP | 1.. MaxnumLoggedMeas |  |  | REL-11 |
| >>Cell parameters Id | MP |  | Cell parameters Id 10.3.6.9 |  | REL-11 |
| >>Primary CCPCH RSCP | MP |  | Integer(0..91) | In dBm. According to P-CCPCH_RSCP_LEV in [19] and [20]. Thirty-six spare values are needed. | REL-11 |
| Logged Measurements Inter-RAT Neighbour cells list | OP |  |  |  | REL-11 |
| >Logged Measurements E-UTRA frequency list | OP | 1.. MaxNumEUTRAFreqs |  |  | REL-11 |
| >>E-UTRA Carrier Frequency | MP |  | Integer (0..65535) | EARFCN of the downlink carrier frequency [64]. If the IE indicates a value of 65535, then the EARFCN for this instance should be read from the corresponding instance of IE "EARFCN extension" in the Logged Measurements E-UTRA frequency extension list. | REL-11 |
| >>Logged Measurements E-UTRA Frequency Neighbours List | MP | 1.. MaxnumLoggedMeas |  |  | REL-11 |
| >>>Physical Cell Identity | MP |  | Integer (0..503) |  | REL-11 |
| >>>RSRP | MP |  | Integer (0..97) | RSRP is mapped to a value between 0 and 97 [36.133]. | REL-11 |
| >>>RSRQ | MP |  | Integer (0..33) | RSRQ_00 to RSRQ_33 in [36.133] are mapped to the value between 0 and 33. RSRQ_34 in [36.133] is mapped to the value 33. If the IE indicates a value of 0 and the RSRQ Extension IE is present, then the RSRQ for this instance should be read from the corresponding instance of IE "RSRQ extension", the range is from -30 to -1. If the IE indicates a value of 33 and the RSRQ Extension IE is present, then the RSRQ for this instance should be read from the corresponding instance of IE "RSRQ extension", the range is from 35 to 46. | REL-11 |
| >>>RSRQ Extension | OP |  | Integer (-30..46) | RSRQ_-30 to RSRQ_46 in [36.133] are mapped to the value between -30 and 46. | REL-12 |
| >>RSRQ Type | OP |  |  |  | REL-12 |
| >>>All Symbols | MP |  | BOOLEAN | TRUE indicates use of all OFDM symbols when performing RSRQ measurements. | REL-12 |
| >>>Wide band | MP |  | BOOLEAN | TRUE indicates use of a wider bandwidth when performing RSRQ measurements. | REL-12 |
| >Logged Measurements E-UTRA frequency extension list | OP | 1.. MaxNumEUTRAFreqs |  |  | REL-11 |
| >>EARFCN extension | OP |  | Integer (65536..262143) | EARFCN of the downlink carrier frequency [64]. | REL-11 |
| >Logged Measurements GSM Neighbouring Cells list | OP | 1.. MaxnumLoggedMeas |  |  | REL-11 |
| >>BSIC | MP |  | BSIC 10.3.8.2 |  | REL-11 |
| >>Band indicator | MP |  | Enumerated (DCS 1800 band used, PCS 1900 band used) | Indicates how to interpret the BCCH ARFCN | REL-11 |
| >>BCCH ARFCN | MP |  | Integer (0..1023) | [45] | REL-11 |
| >>GSM carrier RSSI | MP | Bit string(6) | RXLEV is mapped to a value between 0 and 63, [46]. When mapping the RXLEV value to the RSSI bit string, the first/leftmost bit of the bit string contains the most significant bit. |  | REL-11 |
| Location Information | OP |  |  | The position of the UE when the measurement is made | REL-11 |
| >CHOICE Location | MP |  |  |  | REL-11 |
| >>Ellipsoid |  |  |  |  | REL-11 |
| >>>Ellipsoid point | MP |  | Ellipsoid point 10.3.8.4a |  | REL-11 |
| >>Ellipsoid with altitude |  |  |  |  | REL-11 |
| >>>Ellipsoid point with altitude | MP |  | Ellipsoid point with altitude 10.3.8.4b |  | REL-11 |
| >Ellipsoid with uncertainty circle |  |  |  |  | REL-11 |
| >>Ellipsoid point with uncertainty circle | MP |  | Ellipsoid point with uncertainty circle 10.3.8.4d |  | REL-11 |
| >Ellipsoid with uncertainty ellipse |  |  |  |  | REL-11 |
| >>Ellipsoid point with uncertainty ellipse | MP |  | Ellipsoid point with uncertainty ellipse 10.3.8.4e |  | REL-11 |
| >Ellipsoid with altitude and uncertainty ellipsoid |  |  |  |  | REL-11 |
| >>Ellipsoid point with altitude and uncertainty ellipsoid | MP |  | Ellipsoid point with altitude and uncertainty ellipsoid 10.3.8.4c |  | REL-11 |
| >Horizontal velocity | OP |  | Horizontal velocity 10.3.8.4h |  | REL-11 |

#### 10.3.7.131	Logged Meas Report

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Absolute Time Info | MP |  | BIT STRING (SIZE(48)), | Indicates the reference to network absolute time absoluteTimeInfo provided at the point of measurement logging configuration. Format is YY-MM-DD HH:MM:SS using BCD encoding | REL-10 |
| Trace reference | MP |  | Trace Reference 10.3.7.133 | Parameter trace reference: See TS 32.422 [81] | REL-10 |
| Trace recording session | MP |  | Trace Recording Session 10.3.7.134 | Parameter trace recording session reference: See TS 32.422 [81] | REL-10 |
| TCE Id | MP |  | TCE Id 10.3.7.135 | Parameter TCE Id: See TS 32.422 [81] | REL-10 |
| CHOICE mode | OP |  |  |  | REL-10 |
| >FDD |  |  |  |  | REL-10 |
| >>List of measurements FDD |  | 1.. MaxLoggedMeasReport |  |  | REL-10 |
| >>>Logged Measurement Info-FDD | MP |  | Logged Measurement Info-FDD 10.3.7.129 |  | REL-10 |
| >TDD |  |  |  |  | REL-10 |
| >>List of measurements TDD |  | 1.. MaxLoggedMeasReport |  |  | REL-10 |
| >>>Logged Measurement Info-TDD | MP |  | Logged Measurement Info-TDD 10.3.7.130 |  | REL-10 |
| Logged Meas Available | OP |  | Enumerated (TRUE) | Indicates the UE has logged measurements to report to the network | REL-10 |

#### 10.3.7.132	Logged Measurements Configuration Info

Parameters used to configure logged measurements.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Absolute Time Info | MP |  | Bit Sting (48) | Indicates the reference to network absolute time absoluteTimeInfo provided at the point of measurement logging configuration. Format is YY-MM-DD HH:MM:SS using BCD encoding | REL-10 |
| Logging Duration | MP |  | Enumerated (10 min,20 min, 40 min, 1 hour, 1 hour 30 min, 2 hour) | Timer for logging duration. The timer is activated at the moment of configuration. When the timer expires the logging is stopped and the configuration is cleared. 2 spare values are needed. | REL-10 |
| Logging Interval | MP |  | Enumerated (1.28, 2.56, 5.12, 10.24, 20.48, 30.72, 40.96, 61.44) | Indicates periodicity for storing measurement results. UE behaviour is unspecified when the UE is configured with a DRX period larger than the logging interval.In seconds. | REL-10 |
| Trace reference | MP |  | Trace Reference 10.3.7.133 | Parameter trace reference: See TS 32.422 [81] | REL-10 |
| Trace recording session | MP |  | Trace Recording Session 10.3.7.134 | Parameter trace recording session reference: See TS 32.422 [81] | REL-10 |
| TCE Id | MP |  | TCE Id 10.3.7.135 | Parameter TCE Id: See TS 32.422 [81] | REL-10 |
| CHOICE Area Configuration | OP |  |  | Indicates area for which the UE is requested to perform logging. If absent, the configuration is valid in the RPLMN of the UE and the PLMNs provided in IE "PLMN Identity List", if present. | REL-10 |
| >Cell IDList |  | 1..32 |  |  | REL-10 |
| >>Cell ID | MP |  | Cell ID 10.3.2.2 |  | REL-10 |
| >>PLMN identity | OP |  | PLMN identity 10.3.1.11 | Indicates the PLMN for which the Cell ID is applicable. If not present, the Cell ID is applicable in the RPLMN. | REL-11 |
| >Location Area Code List |  | 1..8 |  |  | REL-10 |
| >>LAI | MP |  | Location area identification 10.3.1.7 |  | REL-10 |
| >Routing Area Code List |  | 1..8 |  |  | REL-10 |
| >>RAI | MP |  | Routing area identification 10.3.1.16 |  | REL-10 |
| PLMN Identity List | OP | 1 to < MaxnumMDTPLMN> |  |  | REL-11 |
| >PLMN identity | MP |  | PLMN identity 10.3.1.11 |  | REL-11 |

#### 10.3.7.132a	Connection Establishment Failure Report

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Time Since Failure | MP |  | INTEGER (0..172800) | Indicates the elapsed time in seconds between Logging and reporting of Connection establishment failure info. If the elapsed time is greater than 172800s then UE shall use the value 172800. | REL-11 |
| CHOICE mode | MP |  |  |  | REL-11 |
| >FDD |  |  |  |  | REL-11 |
| >>Logged Connection Establishment Failure Info-FDD | MP |  | Logged Connection Establishment Failure Info-FDD 10.3.7.129a |  | REL-11 |
| >TDD |  |  |  |  | REL-11 |
| >>Logged Connection Establishment Failure Info-TDD | MP |  | Logged Connection Establishment Failure Info-TDD 10.3.7.130a |  | REL-11 |

#### 10.3.7.133	Trace Reference

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| PLMN Identity | MP |  | PLMN identity 10.3.1.11 | PLMN Identity | REL-10 |
| Trace ID | MP |  | Octet string (3) |  | REL-10 |

#### 10.3.7.134	Trace Recording Session

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Trace Recording Session | MP |  | Octet string (2) |  | REL-10 |

#### 10.3.7.135	TCE Id

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| TCE Id | MP |  | Octet string (1) |  | REL-10 |

#### 10.3.7.136	Periodical reporting criteria on secondary UL frequency

Contains the periodical reporting criteria information. It is necessary only in the intra-frequency periodical reporting mode on the secondary uplink frequency.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Frequency info | MP |  | Frequency info 10.3.6.36 | NOTE1 | REL-10 |
| Periodical reporting criteria | MP |  | Periodical reporting criteria 10.3.7.53 |  | REL-10 |
| Reporting cell status | OP |  | Reporting cell status 10.3.7.61 |  | REL-10 |

NOTE1:	If the IE "Frequency info" is different from the current downlink frequency associated with the secondary UL frequency, the UE behaviour is unspecified.

#### 10.3.7.137	E-UTRA measurement for CELL_FACH

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-UTRA frequencies for measurement | MP | 1 to <maxNumEUTRAFreqs_FACH> |  |  | REL-11 |
|  |  | 1 to <maxNumEUTRAFreqs_FACH-ext> |  |  | REL-12 |
| >EARFCN | MP |  | Integer (0..65535) | EARFCN of the downlink carrier frequency [64]. If the IE indicates a value of 65535, then the EARFCN for this instance should be read from the corresponding instance of IE "EARFCN extension" | REL-11 |
| >EARFCN extension | OP |  | Integer (65536..262143) | EARFCN of the downlink carrier frequency [64]. | REL-11 |
| Report criteria | MP |  | Enumerated (Event-triggered, Periodical reporting) |  | REL-11 |
| Measurement quantity | MP |  | Enumerated (RSRP, RSRQ) | This IE indicates the quantity that the UE uses to determine the event trigger. | REL-11 |
| Reporting Threshold | MP |  | Integer (0..97) | In dB. Ranges used depend on measurement quantity: if measurement quantity is RSRQ, range should be (0..34), if measurement quantity is RSRP, range should be (0..97). | REL-11 |
|  |  |  | Integer (-30..-97) | In dB. Ranges used depend on measurement quantity: if measurement quantity is RSRQ, range should be (-30..46), if measurement quantity is RSRP, range should be (0..97). | REL-12 |

#### 10.3.7.138	E-UTRA results for CELL_FACH

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-UTRA frequency indicator | MP |  | Bit string(maxNumEUTRAFreqs_FACH) | Bit 0 is the first/leftmost bit of the bit string. Each bit indicates whether the radio quality of an E-UTRA frequency is above the reporting threshold or not, where a one value indicates above and a zero value not. Bit n corresponds to the n-th E-UTRA frequency in the list provided by the IE E-UTRA frequencies for measurement. | REL-11 |
|  |  |  | Bit string(maxNumEUTRAFreqs_FACH-ext) | Bit 0 is the first/leftmost bit of the bit string. Each bit indicates whether the radio quality of an E-UTRA frequency is above the reporting threshold or not, where a one value indicates above and a zero value not. Bit n corresponds to the n-th E-UTRA frequency in the list provided by the IE E-UTRA frequencies for measurement. | REL-12 |

#### 10.3.7.139	E-UTRA frequency RACH reporting information

NOTE:	Only for FDD.

Contains the reporting configuration information for an E-UTRA frequency measurement report, which is sent on the RACH.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-UTRA frequency list indicator | MP |  | Bit string(maxNumEUTRAFreqs) | Bit 0 is the first/leftmost bit of the bit string. Each bit indicates the E-UTRA frequency that the UE is required to measure and report on RACH. Bit n corresponds to the n-th E-UTRA frequency received in SIB19. Value '1' for a bit means the corresponding E-UTRA frequency has been configured to measure and report on RACH. Max. 4 EUTRA frequencies shall be configured in this version of the specification. | REL-11 |
| E-UTRA frequency list indicator2 | OP |  | Bit string(maxNumEUTRAFreqs) | Bit 0 is the first/leftmost bit of the bit string. Each bit indicates the E-UTRA frequency that the UE is required to measure and report on RACH. Bit n corresponds to the n-th E-UTRA frequency received in SIB19. Value '1' for a bit means the corresponding E-UTRA frequency has been configured to measure and report on RACH. | REL-12 |
| E-UTRA frequency RACH reporting quantity | MP |  | Enumerated (RSRP, RSRQ) |  | REL-11 |
| E-UTRA frequency RACH reporting threshold | MP |  | Integer (0..97) | In dB. Ranges used depend on measurement quantity: if measurement quantity is RSRQ, range should be (0..46), if measurement quantity is RSRP, range should be (0..97). In case measurement quantity is RSRQ, if the IE indicates a value of 0 and the "E-UTRA frequency RACH reporting threshold Extension" IE is present, then the RSRQ for this instance should be read from the corresponding instance of IE "E-UTRA frequency RACH reporting threshold Extension", the range is from -30 to -1. | REL-11 |
| E-UTRA frequency RACH reporting threshold Extension | OP |  | Integer (-30..-1) |  | REL-12 |
| RACH reporting priority | MD |  | Enumerated (IntraEUTRAInter, InterIntraEUTRA, InterEUTRAIntra, EUTRAIntraInter, EUTRAInterIntra) | The default value is that E-UTRA measurements shall be omitted first, followed by limiting the number of inter-frequency cells, before limiting the number of intra-frequency cells. Measurement result for the current cell shall be omitted last. Three spare values are needed. | REL-11 |

#### 10.3.7.140	Filtered UE power headroom reporting information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Time-to-trigger | MP |  | Integer(0, 10, 20, 40, 60, 80, 100, 120, 160, 200, 240, 320, 640, 1280, 2560, 5000) | Time in ms. Indicates the period of time between the detection of the triggering condition [15] and the sending of the Filtered UPH Report. | REL-12 |
| Hysteresis1 | OP |  | Real(0..7.5 by step of 0.5) |  | REL-12 |
| Hysteresis2 | OP |  | Real(0..7.5 by step of 0.5) |  | REL-12 |
| UE Power Headroom threshold1 | OP |  | Integer(-50..33) |  | REL-12 |
| UE Power Headroom threshold2 | OP |  | Integer(-50..33) |  | REL-12 |
| Filter coefficient | OP |  | Filter coefficient 10.3.7.9 |  | REL-12 |

#### 10.3.7.141	UE positioning AddPos measured results

Contains the UE positioning AddPos measurement results.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| AddPos Measurement Information | MP | 1 to <maxAddPos> |  |  | REL-13 |
| >Time Stamp | OP |  | ASN.1 UTCTime expressed as a Visible String (SIZE(13)) with format YYMMDDhhmmssZ | UTC Time when the measurement results were valid. | REL-13 |
| >CHOICE AddPos ID | MP |  |  | Four spare values are needed. | REL-13 |
| >>Barometric Pressure |  |  |  |  | REL-13 |
| >>>Uncompensated Barometric Pressure | MP |  | Integer (30000..115000) | The measured barometric pressure in units of Pa | REL-13 |
| >>WLAN |  |  |  | WLAN method [87]. | REL-13 |
| >>>WLAN Measured result | MP | 1 to <maxWLANs> |  |  | REL-13 |
| >>>>WLAN AP BSSID | MP |  | Octet string (6) | MAC address of the WLAN AP | REL-13 |
| >>>>WLAN AP SSID | OP |  | Octet string (1..32) | The identifier of the wireless network served by the AP. | REL-13 |
| >>>>WLAN RSSI | OP |  | Integer(-127..128) | AP signal strength (RSSI) of a beacon frame, probe response frame or measurement pilot frame measured at the target in dBm. | REL-13 |
| >>>>WLAN RTT value | OP |  | Integer(0..16777215) | The measured round trip time between the target and WLAN. | REL-13 |
| >>>>WLAN RTT units | OP |  | Enumerated( microseconds,hundredsofnanoseconds, tensofnanoseconds,nanoseconds,tenthsofnanosecond) | The units for the RTT value. Three spares are needed. | REL-13 |
| >>>>WLAN RTT accuracy | OP |  | Integer(0..255) | The estimated accuracy of the provided RTT value expressed as the standard deviation in units given by the RTT units. | REL-13 |
| >>>>WLAN AP Channel Frequency | OP |  | Integer(0..256) | The AP channel number identification of the reported WLAN AP. | REL-13 |
| >>>>Serving Flag | OP |  | Boolean | Indicates whether a set of WLAN AP measurements were obtained for a serving WLAN AP (TRUE) or a non-serving WLAN AP (FALSE). A target device with multiple radio support may indicate more than one type of serving access for the same time instant. | REL-13 |
| >>BT |  |  |  | Bluetooth method [90] | REL-13 |
| >>>BT Measured results | MP | 1 to <maxBTs> |  |  | REL-13 |
| >>>>BT address | MP |  | Octet string (6) |  | REL-13 |
| >>>>BT RSSI | OP |  | Integer(-127..128) | AP signal strength (RSSI) of a beacon frame, probe response frame or measurement pilot frame measured at the target in dBm. | REL-13 |
| >>MBS |  |  |  | MBS method [89] | REL-13 |
| >>>MBS Measurement Parameters | MP | 1 to <maxBeacons> |  |  | REL-13 |
| >>>>Transmitter ID | MP |  | Integer (0..32767) | Identifies the MBS beacon transmitter | REL-13 |
| >>>>Code Phase | MP |  | Integer(0..2097151) | Scale factor 2-21 milli-seconds, in the range from 0 to (1-2-21) milli-seconds. The code-phase measurement made by the target device for the particular MBS signal at the time of measurement in the units of ms. MBS specific code phase measurements (e.g. chips) are converted into unit of ms by dividing the measurements by the nominal values of the measured signal chipping rate. | REL-13 |
| >>>>Code Phase RMS Error | MP |  | Enumerated(range index 0..range index 63) | Coding as in Note 2 of 10.3.7.93 | REL-13 |

### 10.3.8	Other Information elements

#### 10.3.8.1	BCCH modification info

Indicates modification of the System Information on BCCH.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MIB Value tag | MP |  | MIB Value tag 10.3.8.9 |  |  |
| BCCH modification time | OP |  | Integer (0.. 4088 in step of 8) | All SFN values in which MIB may be mapped are allowed. |  |
| SB3 value tag | OP |  | Cell Value tag 2 10.3.8.4o |  | REL-12 |

#### 10.3.8.2	BSIC

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Base transceiver Station Identity Code (BSIC) | MP |  |  | [11] |
| >Network Colour Code (NCC) | MP |  | bit string(3) | The first/leftmost bit of the bit string contains the most significant bit of the NCC. |
| >Base Station Colour Code (BCC) | MP |  | bit string(3) | The first/leftmost bit of the bit string contains the most significant bit of the BCC. |

#### 10.3.8.3	CBS DRX Level 1 information

This information element contains the CBS discontinuous reception information to be broadcast for CBS DRX Level 1 calculations in the UE.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Period of CTCH allocation (N) | MP |  | Integer (1..256) | MTTI N 256, N multiple of MTTI |  |
| CBS frame offset (K) | MP |  | Integer (0..255) | 0 K N-1, K multiple of MTTI |  |
| Period of BMC scheduling messages (P) | CV-SIB5 |  | Integer (8, 16, 32, 64, 128, 256) |  | REL-6 |

| Condition | Explanation |
| --- | --- |
| SIB5 | The IE is optional when the IE "CBS DRX Level 1 information" in SIB5, otherwise the IE is not needed |

#### 10.3.8.4	Cell Value tag

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Value tag | MP |  | Integer (1..4) |  |

#### 10.3.8.4o	Cell Value tag 2

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Value tag 2 | MP |  | Integer (1..16) |  |

#### 10.3.8.4a	Ellipsoid point

This IE contains the description of an ellipsoid point as in [24].

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Latitude sign | MP |  | Enumerated (North, South) |  |
| Degrees Of Latitude | MP |  | Integer (0...223-1) | The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degree (0°.. 90°) |
| Degrees Of Longitude | MP |  | Integer (-223...223-1) | The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degree (-180°..+180°) |

#### 10.3.8.4b	Ellipsoid point with Altitude

This IE contains the description of an ellipsoid point with altitude as in [24].

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Latitude sign | MP |  | Enumerated (North, South) |  |
| Degrees Of Latitude | MP |  | Integer (0...223-1) | The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degree (0°.. 90°) |
| Degrees Of Longitude | MP |  | Integer (-223...223-1) | The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degree (-180°..+180°) |
| Altitude Direction | MP |  | Enumerated (Height, Depth) |  |
| Altitude | MP |  | Integer (0..215-1) | The IE value (N) is derived by this formula: Na  N+1 a being the altitude in metres |

#### 10.3.8.4c	Ellipsoid point with Altitude and uncertainty ellipsoid

This IE contains the description of an ellipsoid point with altitude and uncertainty ellipsoid as in [24].

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Latitude sign | MP |  | Enumerated (North, South) |  |
| Degrees Of Latitude | MP |  | Integer (0...223-1) | The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degree (0°.. 90°) |
| Degrees Of Longitude | MP |  | Integer (-223...223-1) | The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degree (-180°..+180°) |
| Altitude Direction | MP |  | Enumerated (Height, Depth) |  |
| Altitude | MP |  | Integer (0..215-1) | The IE value (N) is derived by this formula: Na  N+1 a being the altitude in metres |
| Uncertainty semi-major | MP |  | Integer (0...127) | The uncertainty r is derived from the "uncertainty code" k by r = 10x(1.1k-1) |
| Uncertainty semi-minor | MP |  | Integer (0...127) | The uncertainty r is derived from the "uncertainty code" k by r = 10x(1.1k-1) |
| Orientation of major axis | MP |  | Integer (0..89) | The IE value (N) is derived by this formula: 2N a  2(N+1) a being the orientation in degree (0°.. 179°) |
| Uncertainty Altitude | MP |  | Integer(0..127) | The uncertainty in altitude, h, expressed in metres is mapped from the IE value (K), with the following formula: with C = 45 and x = 0.025. |
| Confidence | MP |  | Integer (0..100) | in percentage |

#### 10.3.8.4d	Ellipsoid point with uncertainty Circle

This IE contains the description of an ellipsoid point with an uncertainty circle as in [24].

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Latitude sign | MP |  | Enumerated (North, South) |  |
| Degrees Of Latitude | MP |  | Integer (0...223-1) | The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degree (0°.. 90°) |
| Degrees Of Longitude | MP |  | Integer (-223...223-1) | The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degree (-180°..+180°) |
| Uncertainty Code | MP |  | Integer (0...127) | The uncertainty r is derived from the "uncertainty code" k by r = 10x(1.1k-1) |

#### 10.3.8.4e	Ellipsoid point with uncertainty Ellipse

This IE contains the description of an ellipsoid point with an uncertainty ellipse as in [24].

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Latitude sign | MP |  | Enumerated (North, South) |  |
| Degrees Of Latitude | MP |  | Integer (0...223-1) | The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degree (0°.. 90°) |
| Degrees Of Longitude | MP |  | Integer (-223...223-1) | The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degree (-180°..+180°) |
| Uncertainty semi-major | MP |  | Integer (0...127) | The uncertainty r is derived from the "uncertainty code" k by r = 10x(1.1k-1) |
| Uncertainty semi-minor | MP |  | Integer (0...127) | The uncertainty r is derived from the "uncertainty code" k by r = 10x(1.1k-1) |
| Orientation of major axis | MP |  | Integer (0..89) | The IE value (N) is derived by this formula: 2Na  2(N+1) a being the orientation in degree (0°.. 179°) |
| Confidence | MP |  | Integer (0..100) | in percentage |

#### 10.3.8.4ea	ETWS information

Indication of an ETWS primary notification. The ETWS warning type is used to notify upper layers.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Warning Type | MP |  | Octet string (1..2) | IE "Warning Type" as defined in [77] | REL-8 |
| Message Identifier | MP |  | Octet string (2) | IE "Message Identifier" as defined in [77] | REL-8 |
| Serial Number | MP |  | Octet string (2) | IE "Serial Number" as defined in [77] | REL-8 |

#### 10.3.8.4eb	Void

#### 10.3.8.4f	GERAN system information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| >>> GERAN System Info | MP | 1 to <maxGERANSI> |  |  | REL-5 |
| >>>> GERAN system info block | MP |  | Octet string(1..23) | The first octet contains octet 1 of the GERAN system information block, the second octet contains octet 2 of the GERAN system information block and so on. | REL-5 |

#### 10.3.8.4g	GSM Target Cell Info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GSM Target Cell Info List | MP | 1 to <maxGSMTargetCells> |  |  | REL-6 |
| >BCCH ARFCN | MP |  | Integer (0..1023) | [45] | REL-6 |
| >Band indicator | MP |  | Enumerated (DCS 1800 band used, PCS 1900 band used) | Indicates how to interpret the BCCH ARFCN | REL-6 |
| >BSIC | OP |  | BSIC 10.3.8.2 |  | REL-6 |

#### 10.3.8.4h	Horizontal Velocity

This IE contains the description of a horizontal velocity as in [24].

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bearing | MP |  | Integer (0...359) | The direction of movement is given in degrees where ‘0’ represents North, ‘90’ represents East, etc. |
| Horizontal Speed | MP |  | Integer (0...211-1) | The relationship between (N) and the horizontal speed (h) in kilometers per hour it describes is: N ≤ h < N + 0.5, for (N=0) N-0.5 ≤ h < N + 0.5, for (0<N<211-1) N – 0.5 h, for (N = 211-1) |

#### 10.3.8.4i	Horizontal with Vertical Velocity

This IE contains the description of horizontal with vertical velocity as in [24].

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Vertical Speed direction | MP |  | ENUMERATED (upward, downward) |  |
| Bearing | MP |  | Integer (0...359) | The direction of movement is given in degrees where ‘0’ represents North, ‘90’ represents East, etc. |
| Horizontal Speed | MP |  | Integer (0...211-1) | The relationship between (N) and the horizontal speed (h) in kilometers per hour it describes is: N ≤ h < N + 0.5, for (N=0) N-0.5 ≤ h < N + 0.5, for (0<N<211-1) N – 0.5 h, for (N = 211-1) |
| Vertical Speed | MP |  | Integer (0..255) | The relationship between (N) and the vertical speed (v) in kilometres per hour it describes is given by the formula: N ≤ v < N + 0.5, for (N=0) N-0.5 ≤ v < N + 0.5, for (0<N<28-1) N – 0.5 v, for (N = 28-1) |

#### 10.3.8.4j	Horizontal Velocity with Uncertainty

This IE contains the description of horizontal velocity with uncertainty as in [24].

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bearing | MP |  | Integer (0...359) | The direction of movement is given in degrees where ‘0’ represents North, ‘90’ represents East, etc. |
| Horizontal Speed | MP |  | Integer (0...211-1) | The relationship between (N) and the horizontal speed (h) in kilometers per hour it describes is: N ≤ h < N + 0.5, for (N=0) N-0.5 ≤ h < N + 0.5, for (0<N<211-1) N – 0.5 h, for (N = 211-1) |
| Horizontal Uncertainty Speed | MP |  | Integer (0..255) | The IE value is encoded in increments of 1 kilometer per hour. The value of N give the uncertainty speed except for N=255 which indicates that the uncertainty is not specified |

#### 10.3.8.4k	Horizontal with Vertical Velocity and Uncertainty

This IE contains the description of horizontal with vertical velocity and uncertainty as in [24].

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| Vertical Speed direction | MP |  | ENUMERATED (upward, downward) |  |
| Bearing | MP |  | Integer (0...359) | The direction of movement is given in degrees where ‘0’ represents North, ‘90’ represents East, etc. |
| Horizontal Speed | MP |  | Integer (0...211-1) | The relationship between (N) and the horizontal speed (h) in kilometers per hour it describes is: N ≤ h < N + 0.5, for (N=0) N-0.5 ≤ h < N + 0.5, for (0<N<211-1) N – 0.5 h, for (N = 211-1) |
| Vertical Speed | MP |  | Integer (0..255) | The relationship between (N) and the vertical speed (v) in kilometres per hour it describes is given by the formula: N ≤ v < N + 0.5, for (N=0) N-0.5 ≤ v < N + 0.5, for (0<N<28-1) N – 0.5 v, for (N = 28-1) |
| Horizontal Uncertainty Speed | MP |  | Integer (0..255) | The IE value is encoded in increments of 1 kilometer per hour. The value of N give the uncertainty speed except for N=255 which indicates that the uncertainty is not specified |
| Vertical Uncertainty Speed | MP |  | Integer (0..255) | The IE value is encoded in increments of 1 kilometer per hour. The value of N give the uncertainty speed except for N=255 which indicates that the uncertainty is not specified |

#### 10.3.8.4L	E-UTRA Target Info

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| E-UTRA Target Frequency Info List | MP | 1 to <maxEUTRATargetFreqs> |  |  | REL-8 |
| >DL Carrier frequency | MP |  | Integer (0..65535) | EARFCN of the downlink carrier frequency [64]. If the IE indicates a value of 65535, then the EARFCN for this instance should be read from the corresponding instance of IE "EARFCN extension" in the E-UTRA Target Frequency Info extension List. | REL-8 |
| >Blacklisted cells per freq list | OP | 1 to <maxEUTRACellPerFreq> |  |  | REL-8 |
| >>Physical Cell identity | MP |  | Integer (0..503) |  | REL-8 |
| E-UTRA Target Frequency Info extension List | OP | 1 to <maxEUTRATargetFreqs> |  |  | REL-11 |
| >EARFCN extension | OP |  | Integer (65536..262143) | EARFCN of the downlink carrier frequency [64]. | REL-11 |

#### 10.3.8.4m	HNB Name

This information element is used to carry the name of the home NodeB, coded in UTF-8 with variable number of bytes per character [75].

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| HNB Name | MP |  | Octet string (1 to < maxHNBNameSize>) | Carries the name of the Home NodeB with a maximum of maxHNBNameSize octets, using UTF-8 coding. | REL-8 |

#### 10.3.8.5	Inter-RAT change failure

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-RAT change failure cause | MP |  | Enumerated(Configuration unacceptable, physical channel failure, protocol error, unspecified) | Four spare values are needed. |
| Protocol error information | CV-ProtErr |  | Protocol error information 10.3.8.12 |  |

| Condition | Explanation |
| --- | --- |
| ProtErr | The IE is mandatory present if the IE "Inter-RAT change failure cause" has the value "Protocol error" and not needed otherwise. |

#### 10.3.8.6	Inter-RAT handover failure

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-RAT handover failure cause | MD |  | Enumerated(Configuration unacceptable, physical channel failure, protocol error, inter-RAT protocol error, unspecified) | Default value is "unspecified". Eleven spare values are needed. |
| Protocol error information | CV-ProtErr |  | Protocol error information 10.3.8.12 |  |

| Condition | Explanation |
| --- | --- |
| ProtErr | The IE is mandatory present if the IE "Inter-RAT handover failure cause" has the value "Protocol error" and not needed otherwise. |

#### 10.3.8.7	Inter-RAT UE radio access capability

This Information Element contains the inter-RAT UE radio access capability that is structured and coded according to the specification used for the corresponding system type.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE system | MP |  |  |  |  |
| >GSM |  |  |  |  |  |
| >>Mobile Station Classmark 2 | MP |  | Octet string (5) | This IE is formatted as 'TLV' and is coded in the same way as the Mobile Station Classmark 2 information element in [5]. The first octet is the Mobile station classmark 2 IEI and its value shall be set to 33H (except in the case of NOTE 2). The second octet is the Length of mobile station classmark 2 and its value shall be set to 3. The octet 3 contains the first octet of the value part of the Mobile Station Classmark 2 information element, the octet 4 contains the second octet of the value part of the Mobile Station Classmark 2 information element and so on. For each of these octets, the first/ leftmost/ most significant bit of the octet contains b8 of the corresponding octet of the Mobile Station Classmark 2. See NOTE 2. |  |
| >>Mobile Station Classmark 3 | MP |  | Octet string (1..32) | This IE is formatted as 'V' and is coded in the same way as the value part in the Mobile station classmark 3 information element in [5]. The first octet contains octet 1 of the value part of Mobile station classmark 3, the second octet contains octet 2 of the value part of Mobile station classmark 3 and so on. See NOTE 1, NOTE 2. |  |
| >>MS Radio Access Capability | OP |  | Octet string (1..64) | This IE is formatted as 'V' and is coded in the same way as the value part in the MS Radio Access Capability information element in [5]. | REL-6 |
| >GERAN Iu |  |  |  |  | REL-5 |
| >>MS GERAN Iu mode Radio Access Capability | MP |  | Bit string (1..170) | Formatted and coded according to [53]. The first/leftmost/most significant bit of the bit string contains bit 8 of the first octet of the IE. | REL-5 |
| >cdma2000 |  |  |  |  |  |
| >>cdma2000Message | MP | 1.to.<maxInterSysMessages> |  |  |  |
| >>>MSG_TYPE(s) | MP |  | Bit string (8) | Formatted and coded according to cdma2000 specifications. The first/leftmost/most significant bit of the bit string contains bit 7 of the MSG_TYPE. |  |
| >>>cdma2000Messagepayload(s) | MP |  | Bit string (1..512) | Formatted and coded according to cdma2000 specifications. The first/leftmost/most significant bit of the bit string contains bit 7 of the first octet of the cdma2000 message. |  |
| >E-UTRA |  |  |  |  | REL-8 |
| >>UE E-UTRA Capability | MP |  | Octet string | Formatted and coded according to the UE-EUTRA-Capability information element in [67]. The first/leftmost/most significant bit of the first octet of the octetstring contains bit 8 of octet 1 of the UE-EUTRA-Capability IE. | REL-8 |
| NOTE 1: The value part is specified by means of CSN.1, which encoding results in a bit string, to which final padding may be appended upto the next octet boundary [5]. The first/ leftmost bit of the CSN.1 bit string is placed in the first/ leftmost/ most significant bit of the first octet. This continues until the last bit of the CSN.1 bit string, which is placed in the last/ rightmost/ least significant bit of the last octet. |  |  |  |  |  |
| NOTE 2: The information in these IEs is optionally present in the INTER RAT HANDOVER INFO WITH INTER RAT CAPABILITIES message, otherwise it is mandatory present. If the information in the IEs "Mobile Station Classmark 2" and "Mobile Station Classmark 3" is not included, it is indicated by setting the first octet (IEI) of the IE "Mobile Station Classmark 2" to 00H. In this case, the receiver should ignore the value parts of the Mobile Station Classmark 2 and the Mobile Station Classmark 3. The contents of those are set to zero. |  |  |  |  |  |

#### 10.3.8.8	Void

#### 10.3.8.8a	Inter-RAT UE security capability

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE system | MP |  |  |  |  |
| >GSM |  |  |  |  |  |
| >>GSM security capability | MP |  |  | The value TRUE means that the indicated ciphering algorithm is supported. |  |
| >>>A5/7 supported | MP |  | Boolean |  |  |
| >>>A5/6 supported | MP |  | Boolean |  |  |
| >>>A5/5 supported | MP |  | Boolean |  |  |
| >>>A5/4 supported | MP |  | Boolean |  |  |
| >>>A5/3 supported | MP |  | Boolean |  |  |
| >>>A5/2 supported | MP |  | Boolean |  |  |
| >>>A5/1 supported | MP |  | Boolean |  |  |

#### 10.3.8.9	MIB Value tag

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MIB Value tag | MP |  | Integer (1..8) |  |  |
| MIB Value tag extension | CV-MBMS |  | Integer (0..1) | NOTE 1 | REL-12 |

NOTE 1:	The MIB Value tag value equals IE "MIB Value tag" plus (IE "MIB Value tag extension" times 8).

| Condition | Explanation |
| --- | --- |
| MBMS | This IE is not needed in MBMS Modified services Information, otherwise it is optional. |

#### 10.3.8.10	PLMN Value tag

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Value tag | MP |  | Integer (1..256) |  |

#### 10.3.8.10a	PNBSCH allocation

UTRAN may use this IE to provide silent periods in the cell that may be used for cell synchronisation purposes.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Number of repetitions per SFN period | MP |  | Integer(2, 3, 4, 5, 6, 7, 8, 9, 10, 12, 14, 16, 18, 20, 24, 28, 32, 36, 40, 48, 56, 64, 72, 80) |  | REL-4 |

#### 10.3.8.11	Predefined configuration identity and value tag

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predefined configuration identity | MP |  | Predefined configuration identity 10.3.4.5 |  |
| Predefined configuration value tag | MP |  | Predefined configuration value tag 10.3.4.6 |  |

#### 10.3.8.12	Protocol error information

This information element contains diagnostics information returned by the receiver of a message that was not completely understood.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE diagnostics type | MP |  |  | One spare value is needed. |
| >Protocol error cause |  |  | Protocol error cause 10.3.3.26 |  |

#### 10.3.8.13	References to other system information blocks

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| References to other system information blocks | MP | 1 to <maxSIB> |  | System information blocks for which multiple occurrences are used, may appear more than once in this list |  |
| >Scheduling information | MP |  | Scheduling information, 10.3.8.16 | NOTE 1 |  |
| >SIB type SIBs only | MP |  | SIB Type SIBs only, 10.3.8.22 |  |  |
| References to other extension system information blocks | OP | 1 to <maxSIB> |  | System information blocks for which multiple occurrences are used, may appear more than once in this list | REL-6 |
| >Scheduling information | MP |  | Scheduling information, 10.3.8.16 | NOTE 1 | REL-6 |
| >Extension SIB type | MP |  | SIB type extension 10.3.8.18b |  | REL-6 |
| References to other extension system information blocks2 | OP | 1 to <maxSIB> |  | System information blocks for which multiple occurrences are used, may appear more than once in this list | REL-8 |
| >Scheduling information | MP |  | Scheduling information, 10.3.8.16 | NOTE 1 | REL-8 |
| >Extension SIB type | MP |  | SIB type extension2 10.3.8.18c |  | REL-8 |
| References to GANSS system information blocks | OP | 1 to <maxGANSS> |  |  | REL-8 |
| >GANSS ID | OP |  | Integer (0..7) | Absence of this IE means Galileo. Coded as defined in NOTE 1 of 10.3.7.90b | REL-8 |
| >SBAS ID | CV-GANSS-ID-SBAS |  | UE positioning GANSS SBAS ID 10.3.7.97e |  | REL-8 |
| >GANSS system information blocks scheduling | MP | 1 to <maxSIB> |  | System information blocks for which multiple occurrences are used, may appear more than once in this list | REL-8 |
| >>Scheduling information | MP |  | Scheduling information, 10.3.8.16 | NOTE 1 | REL-8 |
| >>GANSS SIB type | MP |  | GANSS SIB type 10.3.8.18d |  | REL-8 |
| References to other extension system information blocks3 | OP | 1 to <maxSIB> |  | System information blocks for which multiple occurrences are used, may appear more than once in this list | REL-12 |
| >Scheduling information | MP |  | Scheduling information, 10.3.8.16 | NOTE 1 | REL-12 |
| >Extension SIB type | MP |  | SIB type extension 3 10.3.8.18ca |  | REL-12 |
| NOTE 1: If this IE is present in the variable SYSTEM_INFORMATION_CONTAINER, the IE shall be discarded. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| GANSS-ID-SBAS | This IE is mandatory present if the IE “GANSS ID” is “SBAS”, and not needed otherwise. |

#### 10.3.8.13a	References to other system information blocks 2

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| References to other system information blocks | MP | 1 to <maxSIB2> |  | System information blocks for which multiple occurrences are used, may appear more than once in this list | REL-12 |
| >Scheduling information | MP |  | Scheduling information, 10.3.8.16 | NOTE 1 | REL-12 |
| >SIB type SIBs only | MP |  | SIB type SIBs only 2 10.3.8.22ac |  | REL-12 |
| NOTE 1: If this IE is present in the variable SYSTEM_INFORMATION_CONTAINER, the IE shall be discarded. |  |  |  |  |  |

#### 10.3.8.14	References to other system information blocks and scheduling blocks

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| References to other system information blocks | MP | 1 to <maxSIB> |  | System information blocks for which multiple occurrences are used, may appear more than once in this list |  |
| >Scheduling information | MP |  | Scheduling information, 10.3.8.16 | NOTE 1 |  |
| >SIB and SB type | MP |  | SIB and SB Type, 10.3.8.18a |  |  |
| References to other extension system information blocks | OP | 1 to <maxSIB> |  | System information blocks for which multiple occurrences are used, may appear more than once in this list | REL-6 |
| >Scheduling information | MP |  | Scheduling information, 10.3.8.16 | NOTE 1 | REL-6 |
| >Extension SIB type | MP |  | SIB type extension 10.3.8.18b |  | REL-6 |
| References to other extension system information blocks2 | OP | 1 to <maxSIB> |  | System information blocks for which multiple occurrences are used, may appear more than once in this list | REL-8 |
| >Scheduling information | MP |  | Scheduling information, 10.3.8.16 | NOTE 1 | REL-8 |
| >Extension SIB type | MP |  | SIB type extension2 10.3.8.18c |  | REL-8 |
| References to GANSS system information blocks | OP | 1 to <maxGANSS> |  |  | REL-8 |
| >GANSS ID | OP |  | Integer (0..7) | Absence of this IE means Galileo. Coded as defined in NOTE 1 of 10.3.7.90b | REL-8 |
| >SBAS ID | CV-GANSS-ID-SBAS |  | UE positioning GANSS SBAS ID 10.3.7.97e |  | REL-8 |
| >GANSS system information blocks scheduling | MP | 1 to <maxSIB> |  | System information blocks for which multiple occurrences are used, may appear more than once in this list | REL-8 |
| >>Scheduling information | MP |  | Scheduling information, 10.3.8.16 | NOTE 1 | REL-8 |
| >>GANSS SIB type | MP |  | GANSS SIB type 10.3.8.18d |  | REL-8 |
| References to other extension system information blocks3 | OP | 1 to <maxSIB> |  | System information blocks for which multiple occurrences are used, may appear more than once in this list | REL-12 |
| >Scheduling information | MP |  | Scheduling information, 10.3.8.16 | NOTE 1 | REL-12 |
| >SIB and SB type | MP |  | SIB type extension 3 10.3.8.18ca |  | REL-12 |
| NOTE 1: If this IE is present in the variable SYSTEM_INFORMATION_CONTAINER, the IE shall be discarded. |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| GANSS-ID-SBAS | This IE is mandatory present if the IE “GANSS ID” is “SBAS”, and not needed otherwise. |

#### 10.3.8.14a	SB3 information

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SEG_COUNT | MD |  | SEG COUNT 10.3.8.17 | Default value is 1 | REL-12 |
| SIB_REP | MP |  | Integer (8, 16, 32, 64) |  | REL-12 |
| BCH channelization code | MP |  | Integer(2..33) | Channelization code of the second system information broadcast channel (BCH mapped on S-CCPCH). | REL-12 |

#### 10.3.8.15	Rplmn information

Contains information to provide faster RPLMN selection in the UE.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GSM BA Range | OP | 1 to maxNumGSMFreqRanges |  | GSM BA Range |  |
| >GSM Lower Range (UARFCN) | MP |  | Integer(0..16383) | Lower bound for range of GSM BA freqs |  |
| >GSM Upper Range (UARFCN) | MP |  | Integer(0..16383) | Upper bound for range of GSM BA freqs |  |
| FDD UMTS Frequency list | OP | 1 to maxNumFDDFreqs |  |  |  |
| >UARFCN (Nlow) | MP |  | Integer(0..16383) | [21] |  |
| >UARFCN (Nupper) | OP |  | Integer(0..16383) | [21] This IE is only needed when the FDD frequency list is specifying a range. |  |
| 3.84 Mcps TDD UMTS Frequency list | OP | 1 to maxNumTDDFreqs |  |  |  |
| >UARFCN | MP |  | Integer(0..16383) | [22] |  |
| 7.68 Mcps TDD UMTS Frequency list | OP | 1 to maxNumTDDFreqs |  |  | REL-7 |
| >UARFCN | MP |  | Integer(0..16383) | [22] | REL-7 |
| 1.28 Mcps TDD UMTS Frequency list | OP | 1 to maxNumTDDFreqs |  |  | REL-4 |
| >UARFCN | MP |  | Integer(0..16383) | [22] | REL-4 |
| CDMA2000 UMTS Frequency list | OP | 1 to maxNumCDMA200Freqs |  |  |  |
| >BAND_CLASS | MP |  | Bit string(5 bits) | TIA/EIA/IS-2000 When mapping the BAND_CLASS to the bit string, the first/leftmost bit of the bit string contains the most significant bit.. |  |
| >CDMA_FREQ | MP |  | Bit string (11 bits) | TIA/EIA/IS-2000 When mapping the CDMA_FREQ to the bit string, the first/leftmost bit of the bit string contains the most significant bit.. |  |

#### 10.3.8.16	Scheduling information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Value tag | OP |  |  |  |  |
| >PLMN Value tag |  |  | PLMN Value tag 10.3.8.10 | This IE is included if the following conditions are fulfilled: the SIB type equals system information block type 1 Note: IE ‘PLMN value tag’ is used for SIB1 but the area scope for SIB 1 is Cell. |  |
| >Predefined configuration identity and value tag |  |  | Predefined configuration identity and value tag 10.3.8.11 | This IE is included if the following conditions are fulfilled: the SIB type equals system information block type 16 |  |
| >Cell Value tag |  |  | Cell Value tag 10.3.8.4 | This IE is included if the following conditions are fulfilled: the SIB type does not equal system information block type 1; and the area scope for the system information block is set to "cell" in table 8.1.1; and a value tag is used to indicate changes in the system information block. |  |
| >Cell Value tag extension | OP |  | Integer(0..3) | This IE may be included if the following conditions are fulfilled: the SIB type equals system information block type 3, 5, 5bis, 21 or 22. NOTE 1. | REL-12 |
| >SIB occurrence identity and value tag |  |  | SIB occurrence identity and value tag 10.3.8.20b | This IE is included if the following conditions are fulfilled: the SIB type equals system information block types 15.2, 15.2bis, 15.2ter, 15.3 and 15.3bis |  |
| Scheduling | MP |  |  |  |  |
| >SEG_COUNT | MD |  | SEG COUNT 10.3.8.17 | Default value is 1 |  |
|  | MD |  | SEG COUNT 10.3.8.17 | Default value is size of SIB_POS offset info plus one when SIB_POS offset info is present, otherwise it is 1. NOTE 2. | REL-12 |
| >SIB_REP | MP |  | Integer (4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048, 4096) | Repetition period for the SIB in frames |  |
| >SIB_POS | MP |  | Integer (0 ..Rep-2 by step of 2) | Position of the first segment Rep is the value of the SIB_REP IE |  |
| >SIB_POS offset info | MD | 1..15 |  | see below for default value |  |
| >>SIB_OFF | MP |  | Integer(2..32 by step of 2) | Offset of subsequent segments |  |
|  | MD |  | Integer(2..32 by step of 2) | Offset of subsequent segments. Default value is 2. NOTE 3. | REL-12 |

| Field | Default value |
| --- | --- |
| SIB_POS offset info | The default value is that all segments are consecutive, i.e., that the SIB_OFF = 2 for all segments except when MIB segment/complete MIB is scheduled to be transmitted in between segments from same SIB. In that case, SIB_OFF=4 in between segments which are scheduled to be transmitted at SFNprime = 8 *n-2 and 8*n + 2, and SIB_OFF=2 for the rest of the segments. |

NOTE 1:	The Cell Value tag value equals IE "Cell Value tag" plus (IE "Cell Value tag extension" times 4).

NOTE 2:	The UE behaviour is unspecified when no value is transmitted for system information that is not of REL-12 or later, and the system information consists of more than one segment.

NOTE 3:	The UE behaviour is unspecified when no value is transmitted for system information that is not of REL-12 or later.

#### 10.3.8.17	SEG COUNT

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SEG_COUNT | MP |  | Integer (1..16) | Number of segments in the system information block |

#### 10.3.8.18	Segment index

Each system information segment has an individual segment index.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Segment index | MP |  | Integer (1..15) | Segments of a system information block are numbered starting with 0 for the first segment and 1 for the next segment, which can be the first subsequent segment or a last segment. |

#### 10.3.8.18a	SIB and SB type

The SIB type identifies a specific system information block.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SIB and SB type | MP |  | Enumerated, see below | Two spare values are needed |

| List of enumerated values | Version |
| --- | --- |
| System Information Type 1, System Information Type 2, System Information Type 3, System Information Type 4, System Information Type 5, System Information Type 6, System Information Type 7, System Information Type 11, System Information Type 12, System Information Type 13, System Information Type 13.1, System Information Type 13.2, System Information Type 13.3, System Information Type 13.4, System Information Type 14, System Information Type 15, System Information Type 15.1, System Information Type 15.2, System Information Type 15.3, System Information Type 15.4, System Information Type 15.5, System Information Type 16, System Information Type 17, System Information Type 18, Scheduling Block 1, Scheduling Block 2, |  |
| System Information Type 5bis | REL-6 |

#### 10.3.8.18b	SIB type extension

The SIB type identifies a specific extension system information block.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SIB type extension | MP |  | Enumerated, see below |  | REL-6 |

| List of enumerated values | Version |
| --- | --- |
| System Information Type 11bis | REL-6 |
| System Information Type 15bis, System Information Type 15.1bis, System Information Type 15.2bis, System Information Type 15.3bis, System Information Type 15.6, System Information Type 15.7 System Information Type 15.8 | REL-7 |

#### 10.3.8.18c	SIB type extension2

The SIB type identifies a specific extension system information block.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SIB type extension2 | MP |  | Enumerated, see below |  | REL-8 |

| List of enumerated values | Version |
| --- | --- |
| System Information Type 19 | REL-8 |
| System Information Type 15.2ter |  |
| System Information Type 20 |  |
| System Information Type 21 | REL-11 |
| System Information Type 22 | REL-11 |
| System Information Type 15.1ter | REL-12 |
| System Information Type 23 | REL-12 |
| System Information Type 24 | REL-12 |

#### 10.3.8.18ca	SIB type extension3

The SIB type identifies a specific extension system information block.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SIB type extension3 | MP |  | Enumerated, see below | Six spare values are needed | REL-12 |

| List of enumerated values | Version |
| --- | --- |
| System Information Type 11ter | REL-12 |
| System Information Type 25 | REL-13 |

#### 10.3.8.18d	GANSS SIB type

The GANSS SIB type identifies a GANSS specific system information block.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| GANSS SIB type | MP |  | Enumerated, see below | Nine spare value is needed | REL-8 |

| List of enumerated values | Version |
| --- | --- |
| System Information Type 15.1bis, System Information Type 15.2bis, System Information Type 15.2ter, System Information Type 15.3bis, System Information Type 15.6, System Information Type 15.7 System Information Type 15.8 | REL-8 |

#### 10.3.8.19	SIB data fixed

Contains the result of a master information block or a system information block after encoding and segmentation. The IE is used for segments with fixed length (segments filling an entire transport block).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SIB data fixed | MP |  | Bit string ( 222) | The first bit contains the first bit of the segment. |

#### 10.3.8.19a	SIB data fixed 2

Contains the result of a system information block after encoding and segmentation. The IE is used for segments with fixed length (segments filling an entire transport block). The IE is used on the second system information broadcast channel (BCH on SCCPCH).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SIB data fixed | MP |  | Bit string ( 232) | The first bit contains the first bit of the segment. |

#### 10.3.8.20	SIB data variable

Contains either a complete system information block or a segment of a system information block. Contains the result of a master information block or a system information block after encoding and segmentation. The IE is used for segments with variable length. The system information blocks are defined in clauses10.2.48.8.1 to10.2.48.8.18.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SIB data variable | MP |  | Bit string ( 1..214) | The first bit contains the first bit of the segment. |

#### 10.3.8.20o	SIB data variable 2

Contains either a complete system information block or a segment of a system information block. Contains the result of a system information block after encoding and segmentation. The IE is used for segments with variable length. The system information blocks are defined in clauses10.2.48.8.1 to10.2.48.8.18. The IE is used on the second system information broadcast channel (BCH on SCCPCH).

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SIB data variable | MP |  | Bit string ( 1..224) | The first bit contains the first bit of the segment. |

#### 10.3.8.20a	SIB occurrence identity

This information element identifies a SIB occurrence for System Information Block types 15.2, 15.2bis, 15.2ter, 15.3 and 15.3bis. For System Information Block type 15.2, 15.2bis and 15.2ter, this identity is assigned to the visible satellite only. Unused identities are claimed by newly rising satellites.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SIB occurrence identity | MP |  | Integer (0..15) |  |

#### 10.3.8.20b	SIB occurrence identity and value tag

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SIB occurrence identity | MP |  | SIB occurrence identity 10.3.8.20a |  |
| SIB occurrence value tag | MP |  | SIB occurrence value tag 10.3.8.20c |  |

#### 10.3.8.20c	SIB occurrence value tag

This information element is used to identify different versions of SIB occurrence for System Information Block types 15.2, 15.2bis, 15.2ter, 15.3 and 15.3bis.

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description |
| --- | --- | --- | --- | --- |
| SIB occurrence value tag | MP |  | Integer(0..15) |  |

#### 10.3.8.21	SIB type

The SIB type identifies a specific system information block.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SIB type | MP |  | Enumerated, see below | NOTE 1 |
| NOTE 1: If the value "Extension Type" is signalled, the UE shall use the scheduling information in the MIB and, if present, in the SB1 and SB2 to identify the specific type of system information block. |  |  |  |  |

| List of enumerated values | Version |
| --- | --- |
| Master information block, System Information Type 1, System Information Type 2, System Information Type 3, System Information Type 4, System Information Type 5, System Information Type 6, System Information Type 7, System Information Type 11, System Information Type 12, System Information Type 13, System Information Type 13.1, System Information Type 13.2, System Information Type 13.3, System Information Type 13.4, System Information Type 14, System Information Type 15, System Information Type 15.1, System Information Type 15.2, System Information Type 15.3, System Information Type 15.4, System Information Type 15.5, System Information Type 16, System Information Type 17, System Information Type 18, Scheduling Block 1, Scheduling Block 2, |  |
| System Information Type 5bis, Extension Type | REL-6 |

#### 10.3.8.22	SIB type SIBs only

The SIB type identifies a specific system information block.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SIB type SIBs only | MP |  | Enumerated, see below | Four spare values are needed |

| List of enumerated values | Version |
| --- | --- |
| System Information Type 1, System Information Type 2, System Information Type 3, System Information Type 4, System Information Type 5, System Information Type 6, System Information Type 7, System Information Type 11, System Information Type 12, System Information Type 13, System Information Type 13.1, System Information Type 13.2, System Information Type 13.3, System Information Type 13.4, System Information Type 14, System Information Type 15, System Information Type 15.1, System Information Type 15.2, System Information Type 15.3, System Information Type 15.4, System Information Type 15.5, System Information Type 16, System Information Type 17, System Information Type 18, |  |
| System Information Type 5bis | REL-6 |

#### 10.3.8.22ab	SIB and SB type 2

The SIB and SB type 2 identifies a specific system information block or scheduling block on the second system information broadcast channel.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SIB and SB type 2 | MP |  | Enumerated, see below | Twenty spare values are needed |

| List of enumerated values | Version |
| --- | --- |
| Scheduling Block 3, System Information Type 1, System Information Type 2, System Information Type 3, System Information Type 4, System Information Type 5, System Information Type 5bis, System Information Type 6, System Information Type 7, System Information Type 11, System Information Type 11bis, System Information Type 12, System Information Type 13, System Information Type 13.1, System Information Type 13.2, System Information Type 13.3, System Information Type 13.4, System Information Type 14, System Information Type 15, System Information Type 15bis, System Information Type 15.1, System Information Type 15.1bis, System Information Type 15.1ter, System Information Type 15.2, System Information Type 15.2bis, System Information Type 15.2ter, System Information Type 15.3, System Information Type 15.3bis, System Information Type 15.4, System Information Type 15.5, System Information Type 15.6, System Information Type 15.7 System Information Type 15.8, System Information Type 16, System Information Type 17, System Information Type 18, System Information Type 19, System Information Type 20 , System Information Type 21 , System Information Type 22, System Information Type 23, System Information Type 24, System Information Type 11ter, System Information Type 25 | REL-12 |

#### 10.3.8.22ac	SIB type SIBs only 2

The SIB type identifies a specific system information block on the second system information broadcast channel.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SIB type SIBs only | MP |  | Enumerated, see below | Twenty-one spare values are needed |

| List of enumerated values | Version |
| --- | --- |
| System Information Type 1, System Information Type 2, System Information Type 3, System Information Type 4, System Information Type 5, System Information Type 5bis, System Information Type 6, System Information Type 7, System Information Type 11, System Information Type 11bis System Information Type 12, System Information Type 13, System Information Type 13.1, System Information Type 13.2, System Information Type 13.3, System Information Type 13.4, System Information Type 14, System Information Type 15, System Information Type 15bis, System Information Type 15.1, System Information Type 15.1bis, System Information Type 15.1ter, System Information Type 15.2, System Information Type 15.2bis, System Information Type 15.2ter, System Information Type 15.3, System Information Type 15.3bis, System Information Type 15.4, System Information Type 15.5, System Information Type 15.6, System Information Type 15.7 System Information Type 15.8, System Information Type 16, System Information Type 17, System Information Type 18, System Information Type 19, System Information Type 20, System Information Type 21, System Information Type 22, System Information Type 23, System Information Type 24, System Information Type 11ter, System Information Type 25 |  |

#### 10.3.8.23	UE History Information

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UE inactivity period | MP |  | Integer (1..120) | Provides expired time since last u-plane activity. If the integer value x is between 1 and 59, it represents the expired time x in seconds. If the integer value x is between 60 and 119, it represents the expired time (x-59) in minutes. If integer value is set to 120, no u-plane activity has been for 60 minutes or more. | REL-8 |
| UE Mobility State indicator | OP |  | Enumerated (High-mobilityDetected) | Absence of this IE implies that, according to [4] either the High mobility state is not applicable or it has not been detected by the UE. | REL-8 |
| UL data volume history | OP |  | Data volume history 10.3.8.24 |  | REL-8 |
| DL data volume history | OP |  | Data volume history 10.3.8.24 |  | REL-8 |

#### 10.3.8.24	Data volume history

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Data volume monitoring window | MP |  | Integer (1..120) | If the integer value x is between 1 and 59, it represents seconds. If the integer value is between 60 and 120, it represents the the value minus 59 in minutes. | REL-8 |
| Data transmission frequency | OP |  | Integer (100, 250, 500, 1000, 2000, 5000, 10000, 60000) | In ms | REL-8 |
| >Data transmission frequency granularity | MP |  | Integer (100, 250, 500, 1000, 2000, 5000, 10000, 60000) | In ms | REL-8 |
| >Number of data transmission occasions | MP |  | Integer (0..610) | Measure of data transmission frequency. The data volume monitoring window is organised in time periods of “Data transmission frequency granularity”, and each transmission within such a period is counted. | REL-8 |
| Data volume per RB | OP | 1 to <maxRB> |  |  | REL-8 |
| >RB identity | MP |  | RB identity 10.3.4.16 |  | REL-8 |
| >Data volume | MP |  | Integer (0..4294967295) | Data volume monitored during the data volume monitoring window, in byte. | REL-8 |

#### 10.3.8.25	Retrievable configuration info

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Retrievable configuration to be invoked | OP |  | Integer (0.. maxRetrievConfig-1) |  | REL-13 |
| Retrievable configuration to be stored | OP |  | Integer (0.. maxRetrievConfig-1) |  | REL-13 |
| Retrievable configuration to be removed | OP | 1 to <maxRetrievConfig> |  |  | REL-13 |
| >Retrievable configuration identity | MP |  | Integer (0.. maxRetrievConfig-1) |  | REL-13 |
| Preconfigured retrievable configuration | OP | 1 to <maxRetrievConfig> |  | In this version of the specification, the maximum value is 4 in one RRC message. | REL-13 |
| >Retrievable configuration identity | MP |  | Integer (0.. maxRetrievConfig-1) |  | REL-13 |
| >Retrievable configuration | MP |  | Retrievable configuration 10.3.8.26 |  | REL-13 |

#### 10.3.8.26	Retrievable configuration

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| RB Information Elements |  |  |  |  | REL-13 |
| Signalling RB information to setup list | OP | 1 to <maxSRBsetup> |  |  | REL-13 |
| >Signalling RB information to setup | MP |  | Signalling RB information to setup 10.3.4.24 |  | REL-13 |
| >>RAB information to setup list | OP | 1 to < maxRABsetup > |  |  | REL-13 |
| >>>RAB information for setup | MP |  | RAB information for setup 10.3.4.10 |  | REL-13 |
| RB information to setup list | OP | 1 to <maxRB> |  |  | REL-13 |
| >RB information to setup | MP |  | RB information to setup 10.3.4.20 |  | REL-13 |
| TrCH Information Elements |  |  |  |  |  |
| Uplink transport channels |  |  |  |  |  |
| UL Transport channel information common for all transport channels | OP |  | UL Transport channel information common for all transport channels 10.3.5.24 |  | REL-13 |
| Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  | REL-13 |
| >Added or Reconfigured UL TrCH information | MP |  | Added or Reconfigured UL TrCH information 10.3.5.2 |  | REL-13 |
| Downlink transport channels |  |  |  |  |  |
| DL Transport channel information common for all transport channels | OP |  | DL Transport channel information common for all transport channels 10.3.5.6 |  | REL-13 |
| Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  | REL-13 |
| >Added or Reconfigured DL TrCH information | MP |  | Added or Reconfigured DL TrCH information 10.3.5.1 |  | REL-13 |
| PhyCH information elements |  |  |  |  |  |
| DTX-DRX Information | OP |  | DTX-DRX Information 10.3.6.34a |  | REL-13 |
| HS-SCCH less Information | OP |  | HS-SCCH less Information 10.3.6.36ab |  | REL-13 |
| MIMO parameters | OP |  | MIMO parameters 10.3.6.41a |  | REL-13 |
| MIMO mode with four transmit antennas parameters | OP |  | MIMO mode with four transmit antennas parameters 10.3.6.142 |  | REL-13 |
| DCH Enhancements info FDD | OP |  | DCH Enhancements info FDD 10.3.6.149 |  | REL-13 |
| Uplink radio resources |  |  |  |  |  |
| E-DCH Info | OP |  | E-DCH Info 10.3.6.97 |  | REL-13 |
| Downlink radio resources |  |  |  |  |  |
| Downlink HS-PDSCH Information | OP |  | Downlink HS-PDSCH Information 10.3.6.23a |  | REL-13 |
| Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a |  | REL-13 |
| Additional downlink secondary cell info list FDD | OP | 2 |  |  | REL-13 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a |  | REL-13 |
| Additional downlink secondary cell info list FDD 2 | OP | 4 |  |  | REL-13 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a |  | REL-13 |

#### 10.3.8.27	Other state configuration info

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Other state configuration to be stored | OP | 1 to <maxOtherStateConfig> |  |  | REL-13 |
| >Other state configuration identity | MP |  | Integer (0.. maxOtherStateConfig-1) |  | REL-13 |
| >Source RRC State Indicator | MP |  | RRC State Indicator 10.3.3.35a |  | REL-13 |
| >Target RRC State Indicator | MP |  | RRC State Indicator 10.3.3.35a |  | REL-13 |
| >URA identity | CV-URA_PCH |  | URA identity 10.3.2.6 |  | REL-13 |
| >CHOICE RNTIs | MP |  |  |  | REL-13 |
| >>Clear |  |  |  |  | REL-13 |
| >>Continue |  |  |  |  | REL-13 |
| >>New |  |  |  |  | REL-13 |
| >>>New C-RNTI | MP |  | C-RNTI 10.3.3.8 |  | REL-13 |
| >>>New H-RNTI | MP |  | H-RNTI 10.3.3.14a |  | REL-13 |
| >>>New Primary E-RNTI | MP |  | E-RNTI 10.3.3.10a |  | REL-13 |
| >CHOICE configuration | OP |  |  |  | REL-13 |
| >>Retrievable configuration id |  |  | Integer (0.. maxRetrievConfig-1) |  | REL-13 |
| >>Retrievable configuration |  |  | Retrievable configuration 10.3.8.26 |  | REL-13 |
| Other state configuration to be removed | OP | 1 to <maxOtherStateConfig> |  |  | REL-13 |
| >Other state configuration identity | MP |  | Integer (0.. maxOtherStateConfig-1) |  | REL-13 |

| Condition | Explanation |
| --- | --- |
| URA_PCH | This IE is optionally present when IE "Target state indicator" is set to URA_PCH. Otherwise it is not needed. |

### 10.3.9	ANSI-41 Information elements

#### 10.3.9.1	ANSI-41 Core Network Information

| Information element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| P_REV | MP |  | P_REV 10.3.9.10 |  |
| MIN_P_REV | MP |  | MIN_P_REV 10.3.9.8 |  |
| SID | MP |  | SID 10.3.9.11 |  |
| NID | MP |  | NID 10.3.9.9 |  |

#### 10.3.9.2	ANSI-41 Global Service Redirection information

This Information Element contains ANSI-41 Global Service Redirection information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ANSI-41 Global Service Redirection information | MP |  | ANSI-41 NAS parameter, 10.3.9.3 | Formatted and coded according to the 3GPP2 document "G3G CDMA DS on ANSI-41" |

#### 10.3.9.3	ANSI-41 NAS parameter

This Information Element contains ANSI-41 User Zone Identification information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ANSI-41 NAS parameter | MP |  | Bit string (size (1..2048)) | The first bit contains the first bit of the ANSI-41 information. |

#### 10.3.9.4	ANSI-41 NAS system information

This Information Element contains ANSI-41 system information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NAS (ANSI-41) system information | MP |  | ANSI-41 NAS parameter, 10.3.9.3 | Formatted and coded according to the 3GPP2 document "G3G CDMA DS on ANSI-41" |

#### 10.3.9.5	ANSI-41 Private Neighbour List information

This Information Element contains ANSI-41 Private Neighbour List information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ANSI-41 Private Neighbour List information | MP |  | ANSI-41 NAS parameter, 10.3.9.3 | Formatted and coded according to the 3GPP2 document "G3G CDMA DS on ANSI-41" |

#### 10.3.9.6	ANSI-41 RAND information

This Information Element contains ANSI-41 RAND information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ANSI-41 RAND information | MP |  | ANSI-41 NAS parameter, 10.3.9.3 | Formatted and coded according to the 3GPP2 document "G3G CDMA DS on ANSI-41" |

#### 10.3.9.7	ANSI-41 User Zone Identification information

This Information Element contains ANSI-41 User Zone Identification information.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ANSI-41 User Zone Identification information | MP |  | ANSI-41 NAS parameter, 10.3.9.3 | Formatted and coded according to the 3GPP2 document "G3G CDMA DS on ANSI-41" |

#### 10.3.9.8	MIN_P_REV

This Information Element contains minimum protocol revision level.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MIN_P_REV | MP |  | Bit string (8) | Minimum protocol revision level. The first/leftmost bit of the bit string contains the most significant bit of the MIN_P_REV. |

#### 10.3.9.9	NID

This Information Element contains Network identification.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NID | MP |  | Bit string (16) | Network identification. The first/leftmost bit of the bit string contains the most significant bit of the NID. |

#### 10.3.9.10	P_REV

This Information Element contains protocol revision level.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| P_REV | MP |  | Bit string (8) | Protocol revision level. The first/leftmost bit of the bit string contains the most significant bit of the P_REV. |

#### 10.3.9.11	SID

This Information Element contains System identification.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SID | MP |  | Bit string (15) | System identification. The first/leftmost bit of the bit string contains the most significant bit of the SID. |

### 10.3.9a	MBMS Information elements

#### 10.3.9a.1	MBMS Common CCTrCH identity

Identifies a Coded Composite Transport channel configuration included within the MBMS COMMON P-T-M RB INFORMATION message.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS Common CCTrCh identity | MP |  | Integer (1..32) |  | REL-6 |

#### 10.3.9a.2	MBMS Common PhyCh identity

Identifies a physical channel configuration included within the MBMS COMMON P-T-M RB INFORMATION message.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS Common PhyCh identity | MP |  | Integer (1..32) |  | REL-6 |

#### 10.3.9a.3	MBMS Common RB identity

Identifies a radio bearer channel configuration included within the MBMS COMMON P-T-M RB INFORMATION message.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS Common RB identity | MP |  | Integer (1..32) |  | REL-6 |

#### 10.3.9a.4	MBMS Common TrCh identity

Identifies a transport channel configuration included within the MBMS COMMON P-T-M RB INFORMATION message.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS Common TrCh identity | MP |  | Integer (1..32) |  | REL-6 |

#### 10.3.9a.5	MBMS Current cell S-CCPCH identity

Identifies one of the current cell's Secondary CCPCH's.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS Current cell S-CCPCH identity | MP |  | Integer (1..maxSCCPCH) |  | REL-6 |

#### 10.3.9a.6	Void

#### 10.3.9a.7	MBMS L1 combining schedule

Includes information about the MBMS L1 combining schedule.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS L1 combining cycle length | MP |  | Enumerated (32, 64, 128, 256, 512, 1024) | In number of radio frames | REL-6 |
| MBMS L1 combining cycle offset | MD |  | Integer (0.. MBMS L1 combining cycle length - 4) by step of 4 | Start of the L1 combining cycle (relative to the timing of the current cell) in number of radio frames. Default value is no offset | REL-6 |
| MTCH L1- combining period list | MP | 1 to < maxMBMS-L1CP> |  | One or more periods in which L1 combining is performed | REL-6 |
| >Start | MP |  | Integer (0.. MBMS L1 combining cycle length - 4) by step of 4 | Number of frames from the end of the previous combining period or the start of the cycle (for the first period) | REL-6 |
| >Duration | MP |  | Integer (4.. MBMS L1 combining cycle length) by step of 4 | Number of frames (see note) | REL-6 |

NOTE:	The MTCH L1- combining period should indicate one or more complete TTIs.

#### 10.3.9a.7o	MBMS p-t-m activation time

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS p-t-m activation time | MP |  | Integer (0..2047) | The 11 LSB of the SFN. Note 1 and 2. | REL-6 |
| Note 1: The “MBMS p-t-m activation time” indicates the start of the 10 ms frame corresponding to the indicated SFN value and of the primary CCPCH of the cell where this IE is transmitted. |  |  |  |  |  |
| Note 2: The range of the “MBMS p-t-m activation time” is from 10 ms after the beginning of the MCCH modification period wherein it is transmitted and to the end of next following MCCH modification period. The UE shall consider a value out of this range as expired. |  |  |  |  |  |

#### 10.3.9a.7a	MBMS p-t-m RB information

Includes information about an MBMS p-t-m radio bearers mapped on a TrCH.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| RB information | CV-Curr |  | MBMS Common RB identity 10.3.9a.3 |  | REL-6 |
| MBMS short transmission ID | MP |  | MBMS Short transmission identity 10.3.9a.10 |  | REL-6 |
| MBMS logical channel identity | MP |  | Integer (1..15) | This identifier is used to distinguish different MTCH mapped on to a TrCh (“MBMS-Id” within the MAC header[15]) | REL-6 |
| L1 combining status | CV-Neigh |  | Boolean | The IE is only applicable in case of partial L1 comining, in which case value TRUE means that L1 combining is used for this radio bearer | REL-6 |

| Condition | Explanation |
| --- | --- |
| Curr | The IE is mandatory present if the IE “MBMS p-t-m RB information list” is included in the MBMS CURRENT CELL P-T-M RB INFORMATION message and not needed otherwise |
| Neigh | The IE is mandatory present if the IE “MBMS p-t-m RB information list” is included in the MBMS NEIGHBOURING CELL P-T-M RB INFORMATION message and not needed otherwise |

#### 10.3.9a.7b	MBMS Selected Service Info

This IE indicates whether the UE has any MBMS Selected Services, and if it does it includes the list of MBMS Selected Services.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| CHOICE Status |  |  |  |  |  |
| >None |  |  |  | (no data) |  |
| >Some | MP |  | MBMS Selected Services Full 10.3.9a.7c |  | REL-6 |

#### 10.3.9a.7c	MBMS Selected Services Full

This IE provides the MBMS Selected Services by means of the full identity.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS Selected Services | MP | 1 to < maxMBMSservSelect> |  |  | REL-6 |
| >MBMS Selected Service ID | MP |  | MBMS service identity 10.3.9a.8 |  | REL-6 |

#### 10.3.9a.7d	MBMS Selected Services Short

This IE indicates the MBMS Selected Services by means of a short identity to identify the MBMS transmission by referencing the services included in the MBMS MODIFIED SERVICES INFORMATION and the MBMS UNMODIFIED SERVICES INFORMATION messages.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS Selected Services | MP | 1 to < maxMBMSservSelect> |  |  | REL-6 |
| >MBMS Selected Service ID | MP |  | MBMS Short transmission identity 10.3.9a.10 |  | REL-6 |
| Modification period identity | MP |  | Integer (0..1) | Indicates the modification period the MBMS short transmission identities refer to | REL-6 |

#### 10.3.9a.8	MBMS Service identity

Includes information about the identity of an MBMS service.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS Service ID | MP |  | MBMS Service ID 10.3.9a.8a |  | REL-6 |
| CHOICE PLMN identity | MP |  |  |  | REL-6 |
| >SameAs-MIB |  |  |  | (No data) The PLMN identity is the same as indicated by the IE “PLMN identity” in MIB | REL-6 |
| >SameAsMIB-MultiPLMN-Id |  |  |  |  | REL-6 |
| >>Multi PLMN id | MP |  | Integer (1..5) | The PLMN identity is one of the PLMN identities (1 to 5) in the IE "Multiple PLMN List" in MIB | REL-6 |
| >ExplicitPLMN-Id |  |  |  |  | REL-6 |
| >>PLMN identity | MP |  | PLMN identity 10.3.1.11 |  | REL-6 |

#### 10.3.9a.8a		MBMS Service ID

Uniquely identifies an MBMS bearer service within a PLMN.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS Service ID | MP |  | Octet string (3 ) | The content of the MBMS Service ID field is coded as octets 3 to 5 of the IE Temporary Mobile Group Identity [5] | REL-6 |

#### 10.3.9a.9	MBMS Session identity

Includes information about the the identity of a session of an MBMS service.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS Session ID | MP |  | Octet string (1) |  | REL-6 |

#### 10.3.9a.10	MBMS Short transmission identity

Includes a short identity of the MBMS transmission identity, which concerns a session of a specific service.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS short transmission identity | MP |  | Integer (1..maxMBMSservUnmodif) | Reference/ index to a transmission listed in the MBMS Modified Services Information or MBMS Unmodified Services Information | REL-6 |

#### 10.3.9a.10a	MBMS Soft Combining Timing Offset

Indicates the timing offset applied in the CFN calculation in sub-clause 8.5.15.5 for a secondary CCPCH carrying only MBMS logical channels. It is used for FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| >MBMS Soft Combining Timing Offset | MP |  | Integer (0, 10, 20, 40) | Timing offset applied in the CFN calculation in sub-clause 8.5.15.5, in ms. | REL-6 |

#### 10.3.9a.11	MBMS specific timers and counters

Includes MBMS specific timers and counters.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| T318 | MD |  | Integer(250... 2000 by step of 250, 3000, 4000, 6000, 8000, 10000, 12000, 16000) | Value in milliseconds. Default value is 1000. | REL-6 |

#### 10.3.9a.12	MBMS Transmission identity

Includes information about the MBMS transmission identity, which concerns a session of a specific service.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBMS Service ID | MP |  | MBMS Service identity 10.3.9a.8 |  | REL-6 |
| MBMS Session ID | OP |  | MBMS Session identity 10.3.9a.9 |  | REL-6 |

#### 10.3.9a.12a	MBSFN frequency list

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBSFN frequency list | MP | 0 to <maxMBSFNclusters> |  |  | REL-7 |
| >MBSFN frequency | MP |  | Frequency info 10.3.6.36 |  | REL-7 |
| >IMB indication | OP |  | Enumerated (TRUE) | If present: 3.84 Mcps TDD MBSFN IMB is applied on this frequency. | REL-8 |
| >Cell parameter ID | OP | <0..127> | Cell parameter Id 10.3.6.9 | Only for 1.28 Mcps TDD [32]. NOTE 1 | REL-7 |

NOTE 1:	For 1.28 Mcps TDD the IE "Cell Parameter ID" indicates the cell which is providing MBMS service in MBSFN only mode on the corresponding frequency; if it is not included, the IE "MBSFN frequency" indicates the secondary frequency providing MBMS service in non-MBSFN only mode of current multi-frequency cell.

#### 10.3.9a.12b	MBSFN TDM Information List

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MBSFN TDM Information List | MP | 1 to <maxMBMSservUnmodif> |  |  | REL-7 |
| >MBMS short transmission ID | MP |  | MBMS Short transmission identity 10.3.9a.10 |  | REL-7 |
| >TDM_Rep | MP |  | Integer (2..9) |  | REL-7 |
| >TDM_Offset | MP |  | Integer (0..8) |  | REL-7 |
| >TDM_Length | MP |  | Integer (1..8) |  | REL-7 |

#### 10.3.9a.13	MCCH configuration information

Includes information about the MCCH configuration.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Access Info Period coefficient | MP |  | Integer (0..3) | Represents a, the access information coefficient. The number of repetitions per modification period equals 2a while the actual access information period, in number of frames, equals MP DIV 2a | REL-6 |
| Repetition Period coefficient | MP |  | Integer (0..3) | Represents r, the repetition period coefficient. The number of repetitions per modification period equals 2r while the actual repetition period, in number of frames, equals MP DIV 2r | REL-6 |
| Modification period coefficient | MP |  | Integer (7..10) | Represents m, the modification period coefficient. The actual modification period (MP), in number of frames, equals 2m | REL-6 |
| RLC info | MP |  | RLC info MBMS 10.3.4.23a |  | REL-6 |
| TCTF presence | CV-MBMS |  | Enumerated (FALSE) | By default the TCTF is present even though the FACH only carries one logical channel (type). When this IE is included, the TCTF is absent | REL-6 |

| Condition | Explanation |
| --- | --- |
| MBMS | This IE is not needed if the IE is contained within the IE “Secondary CCPCH system information”, otherwise the IE is optional. |

#### 10.3.9a.14	MICH configuration information

Includes information about the MICH configuration.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MICH Power offset | MP |  | MICH Power offset 10.3.9a.15 |  | REL-6 |
| CHOICE mode | MP |  |  |  | REL-6 |
| >FDD |  |  |  |  | REL-6 |
| >>Channelisation code | MP |  | Integer (0..255) |  | REL-6 |
| >>Number of NI per frame | MP |  | Integer (18, 36, 72, 144) |  | REL-6 |
| >>STTD indicator | MP |  | STTD Indicator 10.3.6.78 | If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3, the UE behaviour upon reception of this IE is unspecified. | REL-6 |
| >TDD |  |  |  |  | REL-6 |
| >>Timeslot number | MP |  | Timeslot number 10.3.6.84 |  | REL-6 |
| >>Midamble shift and burst type | MP |  | Midamble shift and burst type 10.3.6.41 | For 1.28 Mcps TDD, if the cell is operating in MBSFN mode, the UE shall ignore the contents of this IE. | REL-6 |
| >>CHOICE TDD option | MP |  |  |  | REL-6 |
| >>>3.84 Mcps TDD |  |  |  |  | REL-6 |
| >>>>Channelisation code | MP |  | Enumerated ( (16/1)…(16/16)) |  | REL-6 |
| >>>7.68 Mcps TDD |  |  |  |  | REL-7 |
| >>>>Channelisation code | MP |  | Enumerated ( (32/1)…(32/32)) |  | REL-7 |
| >>>1.28 Mcps TDD |  |  |  |  | REL-6 |
| >>>>Codes list | MP | 1 to 2 |  |  | REL-6 |
| >>>>>Channelisation code | MP |  | Enumerated ( (16/1)…(16/16)) |  | REL-6 |
| >>>> MBSFN Special Time Slot | OP |  | Time Slot LCR Extension 10.3.6.83a | For 1.28 Mcps TDD MBSFN only mode, this IE indicates the SCCPCH is deployed on the MBSFN Special Time Slot [30]. The IE ‘Timeslot number’ shall be ignored if this IE appears. | REL-7 |
| >>Repetition period/length | MD |  | Enumerated((4/2),(8/2), (8/4),(16/2), (16/4), (32/2),(32/4),(64/2),(64/4)) | Default value is “(64/2)”. | REL-6 |
| >>Offset | MP |  | Integer (0…Repetition period -1) | SFN mod Repetitionperiod = Offset. | REL-6 |
| >>MBMS Notification indicator length | MD |  | Integer (4, 8, 16) | Indicates the length of one MBMS Notification indicator in bits. Default value is 4. | REL-6 |
| >3.84 Mcps TDD MBSFN IMB |  |  |  |  | REL-8 |
| >>Channelisation code | MP |  | Integer (0..255) | SF = 256; only the channelisation codes 2 to 15 are applicable | REL-8 |
| >>Number of NI per frame | MP |  | Integer (16, 32, 64, 128) |  | REL-8 |

#### 10.3.9a.15		MICH Power offset

This is the power transmitted on the MICH minus power of the Primary CPICH in FDD and Primary CCPCH Tx Power in TDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MICH Power offset | MP |  | Integer(-10 .. +5) | Offset in dB | REL-6 |

#### 10.3.9a.16	MSCH configuration information

Includes information about the MSCH configuration.

| Information element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| MSCH Scheduling information | MP |  |  | For FDD, scheduling information is provided starting at (SFN (SCTO div 10ms)) mod MSCH_REP = MSCH_OFF For TDD, scheduling information is provided starting at SFN mod MSCH_REP = MSCH_OFF | REL-6 |
| >Scheduling period | MD |  | Enumerated (32, 64, 128, 256, 512, 1024) | The period, in number of frames, between MBMS scheduling messages (MSCH_REP) Default value is the value included in the MBMS GENERAL INFORMATION message | REL-6 |
| >Scheduling offset | MD |  | Integer (0..(MSCH_REP-1)) | The position of MBMS scheduling messages relative to timing of the corresponding cell (MSCH_OFF) Default value is the value included in the MBMS GENERAL INFORMATION message | REL-6 |
| >RLC info | MD |  | RLC info MBMS 10.3.4.23a | Default value is the one included in the MBMS GENERAL INFORMATION message | REL-6 |
| TCTF presence | OP |  | Enumerated (FALSE) | By default the TCTF is present even though the FACH only carries one logical channel (type). When this IE is included, the TCTF is absent | REL-6 |

#### 10.3.9a.17	Network Standard Time Information

NOTE:	Only for 1.28Mcps TDD

This IE includes information about the Network Standard Time and a related SFN value. If Correlative SFN is absent, the default Correlative SFN is equal to zero, this means the "Network Standard Time" indicate the standard time value when SFN equal zero.

The "Network Standard Time" occupied 40 bits; the format refers to "ETSI EN 300 468 V1.5.1 (2003-05)".

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Network Standard Time | MP |  | BIT STRING (40) | Network Standard Time | REL-8 |
| Correlative SFN | OP |  | Integer (0..4095) | System frame number which is correlative with Network Standard Time Information. If it is absent, Correlative SFN is equal to zero. | REL-8 |

### 10.3.9b	WLAN Information elements

#### 10.3.9b.1	WLAN Offload Configuration

This IE indicates information for RAN-assisted WLAN interworking.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Threshold Serving RSCP | OP |  |  |  | REL-12 |
| >ThreshServingOffloadWLAN, Low | MP |  | Integer (-119..-25 by step of 2) | RSCP, dBm | REL-12 |
| >ThreshServingOffloadWLAN, High | MP |  | Integer (-119..-25 by step of 2) | RSCP, dBm | REL-12 |
| Threshold Serving Ec/N0 | OP |  |  |  | REL-12 |
| >ThreshServingOffloadWLAN, Low2 | MP |  | Integer (-24..0) | Ec/N0, dB | REL-12 |
| >ThreshServingOffloadWLAN, High2 | MP |  | Integer (-24..0) | Ec/N0, dB | REL-12 |
| Threshold Channel Utilization | OP |  |  |  | REL-12 |
| >ThreshChUtilWLAN, Low | MP |  | Integer (0..255) |  | REL-12 |
| >ThreshChUtilWLAN, High | MP |  | Integer (0..255) |  | REL-12 |
| Threshold Backhaul Bandwidth | OP |  |  |  | REL-12 |
| >ThreshBackhRateDLWLAN, Low | MP |  | WLAN Threshold Backhaul Rate 10.3.9b.4 | Value in kbps. | REL-12 |
| >ThreshBackhRateDLWLAN, High | MP |  | WLAN Threshold Backhaul Rate 10.3.9b.4 | Value in kbps. | REL-12 |
| >ThreshBackhRateULWLAN, Low | MP |  | WLAN Threshold Backhaul Rate 10.3.9b.4 | Value in kbps. | REL-12 |
| >ThreshBackhRateULWLAN, High | MP |  | WLAN Threshold Backhaul Rate 10.3.9b.4 | Value in kbps. | REL-12 |
| Threshold Beacon RSSI | OP |  |  |  | REL-12 |
| >ThreshBeaconRSSIWLAN, Low | MP |  | Integer (0..255) | Actual value = IE value – 128 in dBm. | REL-12 |
| >ThreshBeaconRSSIWLAN, High | MP |  | Integer (0..255) | Actual value = IE value – 128 in dBm. | REL-12 |
| Offload Preference Indicator | OP |  | Bitstring (16) | Parameter: OPI defined in [88]. Only applicable to RAN-assisted WLAN interworking based on ANDSF policies. | REL-12 |
| TsteeringWLAN | OP |  | Integer (0..31) | Value in seconds. Only applicable to RAN-assisted WLAN interworking based on access network selection and traffic steering rules. | REL-12 |

#### 10.3.9b.2	WLAN Identifier

This IE indicates the WLAN identifier for RAN-assisted WLAN interworking.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| SSID | OP |  | Octet string (1..32) | Service Set Identifier defined in IEEE 802.11-2012 [87]. | REL-12 |
| BSSID | OP |  | Octet string (6) | Basic Service Set Identifier defined in IEEE 802.11-2012 [87]. | REL-12 |
| HESSID | OP |  | Octet string (6) | Homogenous Extended Service Set Identifier defined in IEEE 802.11-2012 [87]. | REL-12 |

#### 10.3.9b.3	WLAN Offload Information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| WLAN Offload Configuration | CV-Dedicated |  | WLAN Offload Configuration 10.3.9b.1 |  | REL-12 |
| WLAN Identifier List | OP | 1 to <maxWLANID> |  |  | REL-12 |
| >WLAN Identifier | MP |  | WLAN Identifier 10.3.9b.2 |  | REL-12 |

| Condition | Explanation |
| --- | --- |
| Dedicated | The IE is mandatory present in the UTRAN MOBILITY INFORMATION or CELL UPDATE CONFIRM message. Otherwise the IE is optionally present. |

#### 10.3.9b.4	WLAN Threshold Backhaul Rate

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| WLAN Threshold Backhaul Backhaul Rate | MP |  | Enumerated {r0, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, r4096, r8192, r16384, r32768, r65536, r131072, r262144, r524288, r1048576, r2097152, r4194304, r8388608, r16777216, r33554432, r67108864, r134217728, r268435456, r536870912, r1073741824, r2147483648, r4294967296} | Value in kbps. | REL-12 |

#### 10.3.9b.5	Specific ACDC Barring Information

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| ACDC Barring | OP | 1 to < maxNumAcdcCategory > |  | The first entry represents the ACDC category 1 and so on. 1 is the highest ranked ACDC category value and 16 is the lowest ranked ACDC category value, refer to [75] | REL-13 |
| >CHOICE ACDC per Category | OP |  |  |  |  |
| >>ACDC-Barred |  |  | Enumerated (TRUE, FALSE) | Value of TRUE indicates access for this ACDC category is barred. Value of FALSE indicates access for this ACDC category is not barred. | REL-13 |
| >>ACDC-ACB-barringBitmap |  |  | Bit string(SIZE (10)) | Bit 0 is the first/leftmost bit of the bit string. Bit 0 corresponds to Access Class 0, bit 1 to Access Class 1 and so on up to Access Class 9. The value 1 of a bit indicates that the corresponding Access Class is blocked. The value 0 of a bit indicates that the corresponding Access Class is not blocked. | REL-13 |
| >Expiration Time Factor 2 | MD |  | Expiration Time Factor 2 10.3.3.12b | Default is 1. | REL-13 |
| Note: Empty entry indicates the corresponding ACDC category is not configured in the SIB. |  |  |  |  |  |

### 10.3.10	Multiplicity values and type constraint values

The following table includes constants that are either used as multi bounds (name starting with "max") or as high or low value in a type specification (name starting with "lo" or "hi"). Constants are specified only for values appearing more than once in the RRC specification. In case a constant is related to one or more other constants, an expression is included in the "value" column instead of the actual value.

| Constant | Explanation | Value | Version |
| --- | --- | --- | --- |
| CN information |  |  |  |
| maxCNdomains | Maximum number of CN domains | 4 |  |
| UTRAN mobility information |  |  |  |
| maxRAT | Maximum number or Radio Access Technologies | maxOtherRAT + 1 |  |
| maxOtherRAT | Maximum number or other Radio Access Technologies | 15 |  |
| maxURA | Maximum number of URAs in a cell | 8 |  |
| maxInterSysMessages | Maximum number of Inter System Messages | 4 |  |
| maxRABsetup | Maximum number of RABs to be established | 16 |  |
| maxDedicatedCSGFreq | Maximum number of dedicated CSG frequencies | 4 | REL-8 |
| UE information |  |  |  |
| maxtransactions | Maximum number of parallel RRC transactions in downlink | 25 |  |
| maxPDCPalgoType | Maximum number of PDCP algorithm types | 8 |  |
| maxMultipleFrequencyBandsFDD | Maximum number of additional frequency bands broadcasted in SIB5, SIB6, SIB11, SIB11bis, SIB11ter, SIB12 | 8 | REL-10 |
| maxMultipleFrequencyBandsEUTRA | Maximum number of additional frequency bands broadcasted in SIB19 | 8 | REL-10 |
| maxNumAccessGroups | Maximum number of Access Groups | 16 | REL-12 |
| maxNumAcdcCategory | Maximum number of ACDC Categories | 16 | REL-13 |
| maxFreqBandsFDD | Maximum number of frequency bands as defined in [21] and treated by RRC | 8 |  |
| maxFreqBandsFDD2 | Maximum number of frequency bands as defined in [21] and treated by RRC | 22 | REL-6 |
| maxFreqBandsFDD3 | Maximum number of frequency bands as defined in [21] and treated by RRC | 86 | REL-10 |
| maxFreqBandsFDD-ext | Used in asn.1 to signal additional bands maxFreqBandsFDD2 – (maxFreqBandsFDD-1) | 15 | REL-6 |
| maxFreqBandsFDD-ext2 | Used in asn.1 to signal additional bands maxFreqBandsFDD3 – maxFreqBandsFDD2 | 64 | REL-10 |
| maxFreqBandsFDD-ext3 | Used in asn.1 to signal additional bands maxFreqBandsFDD3 – maxFreqBandsFDD | 78 | REL-10 |
| maxFreqBandsTDD | Maximum number of frequency bands supported by the UE as defined in [22] | 4 |  |
| maxFreqBandsTDD-ext | Maximum number of frequency bands supported by the UE as defined in [22] | 16 | REL-8 |
| maxFreqBandsGSM | Maximum number of frequency bands supported by the UE as defined in [45] | 16 |  |
| maxFreqBandsEUTRA | Maximum number of frequency bands supported by the UE as defined in [36.101] | 16 | REL-8 |
| maxFreqBandsEUTRA-ext | Maximum number of frequency bands supported by the UE as defined in [36.101] | 48 | REL-11 |
| maxFreqBandsIndicatorSupport | Maximum number of frequency bands broadcasted in SIB5/5bis | 2 | REL-10 |
| maxPage1 | Number of UEs paged in the Paging Type 1 message | 8 |  |
| maxSystemCapability | Maximum number of system specific capabilities that can be requested in one message. | 16 |  |
| MaxURNTIgroup | Maximum number of U-RNTI groups in one message | 8 | REL-5 |
| maxCommonHRNTI | Maximum number of common H-RNTI per cell | 4 | REL-7 |
| maxERNTIgroup | Maximum number of common E-RNTI group per cell (1.28 Mcps TDD) | 32 | REL-8 |
| maxERNTIperGroup | Maximum number of common E-RNTI in a group (1.28 Mcps TDD) | 2 | REL-8 |
| RB information |  |  |  |
| maxPredefConfig | Maximum number of predefined configurations | 16 |  |
| maxOtherStateConfig | Maximum number of other state configurations | 4 | REL-13 |
| maxRB | Maximum number of RBs | 32 |  |
| maxSRBsetup | Maximum number of signalling RBs to be established | 8 |  |
| maxRBperRAB | Maximum number of RBs per RAB | 8 |  |
| maxRBallRABs | Maximum number of non signalling RBs | 27 |  |
| maxRBperTrCh | Maximum number of RB per TrCh | 16 | REL-6 |
| maxRBMuxOptions | Maximum number of RB multiplexing options | 8 |  |
| maxLoCHperRLC | Maximum number of logical channels per RLC entity | 2 |  |
| maxRLCPDUsizePerLogChan | Maximum number of RLC PDU sizes per logical channel mapped on E-DCH | 32 | REL-6 |
| MaxROHC-PacketSizes | Maximum number of packet sizes that are allowed to be produced by ROHC. | 16 |  |
| MaxROHC-Profiles | Maximum number of profiles supported by ROHC on a given RB. | 8 |  |
| maxRFC 3095-CID | Maximum number of available CID values per radio bearer | 16384 | REL-5 |
| maxRetrievConfig | Maximum number of retrievable configurations | 8 | REL-13 |
| TrCH information |  |  |  |
| maxCommonQueueID | Maximum number of common Mac-ehs queues | 2 | REL-7 |
| maxE-DCHMACdFlow | Maximum number of E-DCH MAC-d flows | 8 | REL-6 |
| MaxHProcesses | Maximum number of H-ARQ processes | 8 | REL-5 |
| MaxHSDSCH_TB_index | Maximum number of TB set size configurations for the HS-DSCH. | 64 (FDD and 1.28 MCPS TDD); 512 (3.84 Mcps TDD); 1024 (7.68 Mcps TDD) | REL-5 |
| maxMACdPDUSizes | Maximum number of MAC-d PDU sizes per queue permitted for MAC-hs | 8 | REL-5 |
| maxTrCH | Maximum number of transport channels used in one direction (UL or DL) | 32 |  |
| maxTrCHpreconf | Maximum number of preconfigured Transport channels, per direction | 16 |  |
| maxNrOfConcatTrCH | Maximum number of DL Transport Channels of type DCH that are subject to concatenation in the physical layer | 3 | REL-12 |
| maxCCTrCH | Maximum number of CCTrCHs | 8 |  |
| maxQueueID | Maximum number of Mac-hs queues | 8 | REL-5 |
| MaxTF | Maximum number of different transport formats that can be included in the Transport format set for one transport channel | 32 |  |
| maxTFC | Maximum number of Transport Format Combinations | 1024 |  |
| maxTFCsub | Maximum number of Transport Format Combinations Subset | 1024 |  |
| maxSIBperMsg | Maximum number of complete system information blocks per SYSTEM INFORMATION message | 16 |  |
| maxSIB | Maximum number of references to other system information blocks. | 32 |  |
| maxSIB2 | Maximum number of references to other system information blocks on second system information broadcast channel. | 64 | REL-12 |
| maxSIB-FACH | Maximum number of references to system information blocks on the FACH | 8 |  |
| PhyCH information |  |  |  |
| maxHSSCCHs | Maximum number of HSSCCH codes that can be assigned to a UE | 4 | REL-5 |
| maxHS-SCCHLessTrBlk | Maximum number of HS-DSCH Transport Block Sizes used for HS-SCCH-less operation | [4] | REL-7 |
| maxTDD128Carrier | The maximum number of carriers for 1.28 Mcps TDD | 6 | REL-7 |
| maxAC | Maximum number of access classes | 16 |  |
| maxASC | Maximum number of access service classes | 8 |  |
| maxASCmap | Maximum number of access class to access service classes mappings | 7 |  |
| maxASCpersist | Maximum number of access service classes for which persistence scaling factors are specified | 6 |  |
| maxPRACH | Maximum number of PRACHs in a cell | 16 |  |
| maxEDCHs | Maximum number of common E-DCH resources in a cell | 32 | REL-8 |
| maxPRACH_EUL | Maximum number of PRACH preamble control parameters (for Enhanced Uplink) in a cell | 4 | REL-11 |
| MaxPRACH_FPACH | Maximum number of PRACH / FPACH pairs in a cell (1.28 Mcps TDD) | 8 | REL-4 |
| maxFACHPCH | Maximum number of FACHs and PCHs mapped onto one secondary CCPCHs | 8 |  |
| maxRL | Maximum number of radio links | 8 |  |
| maxEDCHRL | Maximum number of E-DCH radio links | 4 | REL-6 |
| maxSCCPCH | Maximum number of secondary CCPCHs per cell | 16 |  |
| maxDPDCH-UL | Maximum number of DPDCHs per cell | 6 |  |
| maxDPCH-DLchan | Maximum number of channelisation codes used for DL DPCH | 8 |  |
| maxPUSCH | Maximum number of PUSCHs | (8) |  |
| maxPDSCH | Maximum number of PDSCHs | 8 |  |
| maxTS | Maximum number of timeslots used in one direction (UL or DL) | 14 (3.84 Mcps TDD and 7.68 Mcps TDD) |  |
|  |  | 6 (1.28 Mcps TDD) | REL-4 |
| hiPUSCHidentities | Maximum number of PUSCH Identities | 64 |  |
| hiPDSCHidentities | Maximum number of PDSCH Identities | 64 |  |
| maxNumE-AGCH | Maximum number of E-AGCHs (TDD) | 4 | REL-7 |
| maxNumE-HICH | Maximum number of E-HICHs (1.28 Mcps TDD) | 4 | REL-7 |
| maxERUCCH | Maximum number of E-RUCCH on primary frequency (1.28Mcps TDD) | 256 | REL-8 |
| Measurement information |  |  |  |
| maxTGPS | Maximum number of transmission gap pattern sequences | 6 |  |
| maxAdditionalMeas | Maximum number of additional measurements for a given measurement identity | 4 |  |
| maxExcludedDetectedSetCells | Maximum number of cells that can be excluded from a detected set measurement | 64 | REL-11 |
| maxNonContiguousMultiCellCombinations | Maximum number of non-contiguous multi-cell HSDPA combinations in one band | 3 | REL-10 |
| maxMeasEvent | Maximum number of events that can be listed in measurement reporting criteria | 8 |  |
| maxMeasEventOnSecULFreq | Maximum number of events that can be listed in measurement reporting criteria on the downlink frequency associated with secondary uplink frequency | 8 | REL-9 |
| maxMeasIdentity | Maximum number of measurement identities that can be removed from variable MEASUREMENT_IDENTITY | 32 | REL-12 |
| maxMeasParEvent | Maximum number of measurement parameters (e.g. thresholds) per event | 2 |  |
| maxMeasIntervals | Maximum number of intervals that define the mapping function between the measurements for the cell quality Q of a cell and the representing quality value | 1 |  |
| maxCellMeas | Maximum number of cells to measure | 32 |  |
| maxCellMeas-ext | Maximum number of cells to measure | 80 | REL-12 |
| maxCellMeas-ext2 | Maximum number of cells to measure in SIB11ter | 48 | REL-12 |
| maxMeasCSGRange | Maximum number of ranges of cells for CSG measurement | 4 | REL-9 |
| maxSIrequest | Maximum number of cell request for reading System Information | 4 | REL-9 |
| maxCellMeasOnSecULFreq | Maximum number of cells to measure on the downlink frequency associated with the secondary uplink frequency | 32 | REL-9 |
| maxReportedGSMCells | Maximum number of GSM cells to be reported | 8 |  |
| maxReportedEUTRAFreqs | Maximum number of E-UTRA frequencies to report | 4 | REL-8 |
| maxReportedEUTRAFreqs-ext | Maximum number of E-UTRA frequencies to report | 8 | REL-12 |
| maxReportedEUTRACellperFreq | Maximum number of E-UTRA cells to report per frequency | 4 | REL-8 |
| maxFreq | Maximum number of frequencies to measure | 8 |  |
| maxFreqMeasWithoutCM | Maximum number of inter frequencies to measure without CM | 2 | REL-10 |
| maxNumPrio | Maximum number of priorities to store | 11 | REL-8 |
|  |  | 16 | REL-13 |
| maxPrio | Maximum number of RAT or Frequency Priority levels | 8 | REL-8 |
| maxPrio-ext | Maximum number of RAT or Frequency Priority levels | 16 | REL-13 |
| maxRMPfrequencies | Maximum number of UTRA inter-frequencies that the network can request to be measured with Reduced Measurement Performance. | 8 | REL-12 |
| maxEUTRACellPerFreq | Maximum number of E-UTRA cells per frequency | 16 | REL-8 |
| maxSat | Maximum number of satellites to measure | 16 |  |
| maxGANSSSat | Maximum number of GANSS satellites to measure | 64 | REL-7 |
| maxGANSS | Maximum number of GANSS supported | 8 | REL-7 |
| maxAddPos | Maximum number of AddPos types supported | 8 | REL-13 |
| maxBeacons | Maximum number of MBS beacons reported | 64 | REL-13 |
| maxWLANs | Maximum number of WLANs reported | 64 | REL-13 |
| maxBTs | Maximum number of Bluetooth nodes reported | 32 | REL-13 |
| maxSgnType | Maximum number of additional signals in GANSS to measure | 8 | REL-7 |
| maxSatClockModels | Maximum number of clock models in a GANSS | 4 | REL-7 |
| maxSatAlmanacStorage | Maximum number of satellites for which to store GPS Almanac information | 32 |  |
| HiRM | Maximum number that could be set as rate matching attribute for a transport channel | 256 |  |
| maxDCHMeasurementOccasionPatternSequence | Maximum number of CELL_DCH measurement occasion pattern sequences | 5 | REL-9 |
| maxIGPInfo | Maximum number of ionospheric grid points for BDS as defined in [86]. | 320 | REL-12 |
| Frequency information |  |  |  |
| MaxFDDFreqList | Maximum number of FDD carrier frequencies to be stored in USIM | 4 |  |
| MaxTDDFreqList | Maximum number of TDD carrier frequencies to be stored in USIM | 4 |  |
| MaxFDDFreqCellList | Maximum number of neighbouring FDD cells to be stored in USIM | 32 |  |
| MaxTDDFreqCellList | Maximum number of neighbouring TDD cells to be stored in USIM | 32 |  |
| MaxGSMCellList | Maximum number of GSM cells to be stored in USIM | 32 |  |
| Other information |  |  |  |
| MaxGERANSI | Maximum number of GERAN SI blocks that can be provided as part of NACC information | 8 | REL-5 |
| maxNumGSMFreqRanges | Maximum number of GSM Frequency Ranges to store | 32 |  |
| MaxNumFDDFreqs | Maximum number of FDD centre frequencies to store | 8 |  |
| MaxNumTDDFreqs | Maximum number of TDD centre frequencies to store | 8 |  |
| maxNumCDMA200Freqs | Maximum number of CDMA2000 centre frequencies to store | 8 |  |
| maxGSMTargetCells | Maximum number of GSM target cells | 32 | REL-6 |
| maxNumGSMCellGroup | Maximum number of GSM frequency groups to store | 16 | REL-8 |
| maxNumEUTRAFreqs | Maximum number of EUTRA centre frequencies to store | 8 | REL-8 |
| maxNumEUTRAFreqs_FACH | Maximum number of EUTRA centre frequencies to store when in CELL_FACH | 4 | REL-11 |
| maxNumEUTRAFreqs_FACH-ext | Maximum number of EUTRA centre frequencies to store when in CELL_FACH | 8 | REL-12 |
| maxEUTRATargetFreqs | Maximum number of target E-UTRA frequencies | 8 | REL-8 |
| maxHNBNameSize | Maximum number of octets for the HNB Name | 48 | REL-8 |
| MBMS information |  |  |  |
| maxMBMS-CommonCCTrCh | Maximum number of CCTrCh configurations included in the MBMS COMMON P-T-M RB Information message | 32 | REL-6 |
| maxMBMS-CommonPhyCh | Maximum number of PhyCh configurations included in the MBMS COMMON P-T-M RB Information message | 32 | REL-6 |
| maxMBMS-CommonRB | Maximum number of RB configurations included in the MBMS COMMON P-T-M RB Information message | 32 | REL-6 |
| maxMBMS-CommonTrCh | Maximum number of TrCh configurations included in the MBMS COMMON P-T-M RB Information message | 32 | REL-6 |
| maxMBMS-Freq | Maximum number of MBMS preferred frequencies | 4 | REL-6 |
| maxMBMS-L1CP | Maximum number of periods in which layer 1 combining applies | 4 | REL-6 |
| maxMBMSservCount | Maximum number of MBMS services in a Access Info message | 8 | REL-6 |
| maxMBMSservModif | Maximum number of MBMS services in a MBMS Modified Services Information message | 32 | REL-6 |
| maxMBMSservSched | Maximum number of MBMS services in a MBMS Scheduling Information message | 16 | REL-6 |
| maxMBMSservSelect | Maximum number of MBMS Selected Services in a CELL UPDATE or RRC CONNECTION REQUEST message | 8 | REL-6 |
| maxMBMSservUnmodif | Maximum number of MBMS services in a MBMS Unmodified Services Information message | 64 | REL-6 |
| maxMBMSTransmis | Maximum number of transmissions for which scheduling information is provided within a scheduling period | 4 | REL-6 |
| maxMBMS-Services | Maximum number of MBMS services the UE stores in the variable MBMS_ACTIVATED_SERVICES | 64 | REL-6 |
| maxMBSFNclusters | Maximum number of MBSFN clusters indicated in system information | 16 | REL-7 |
| Logged Measurement information |  |  | REL-10 |
| MaxnumLoggedMeas | Maximum number of Cells or Frequencies that can be listed in Logged Measurement Report. | 8 | REL-10 |
| MaxLoggedMeas | Maximum number of logged measurements entries that can be stored by the UE | 4405 | REL-10 |
| MaxLoggedMeasReport | Maximum number of Logged Measurment Info sent in LoggedMeasReport | 128 | REL-10 |
| MaxnumMDTPLMN | Maximum number of PLMNs in PLMN list for MDT | 15 | REL-11 |
| ANR information |  |  | REL-10 |
| MaxNumANRLoggedItems | Maximum number of entries in Logged ANR Report info | 4 | REL-10 |
| WLAN information |  |  | REL-12 |
| maxWLANID | Maximum number of WLAN identifiers | 16 | REL-12 |

#### 10.3.11	Void

### 14.12.4	RRC messages exchanged between network nodes

#### 14.12.4.0	HANDOVER TO UTRAN COMMAND

This RRC message is sent between network nodes to transfer the actual handover command including the details of the radio configuration to be used upon handover to UTRAN as compiled by the target RNC.

Direction: target RNC source RAT

The message is exactly the same as the HANDOVER TO UTRAN COMMAND defined in subclause 10.2.16a.

#### 14.12.4.0a	INTER RAT HANDOVER INFO

This RRC message is sent between network nodes to transfer information relevant for the target RNC when preparing for handover to UTRAN.

Direction: source RAT nodetarget RAT node

The message is exactly the same as the INTER RAT HANDOVER INFO defined in subclause 10.2.16d.

#### 14.12.4.1	INTER RAT HANDOVER INFO WITH INTER RAT CAPABILITIES

This RRC message is sent between network nodes when preparing for an inter RAT handover to UTRAN.

The radio access capabilities for the source RAT and target RAT shall always be included; the radio access capabilities for other RATs may be included if available at the source RAT.

Direction: source RATtarget RNC

| Information Element/Group Name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| UE Information elements |  |  |  |  |  |
| Predefined configuration status information | OP |  | Predefined configuration status information 10.3.4.5a |  |  |
| Predefined configuration status information compressed | OP |  | Predefined configuration status information compressed 10.3.4.5b |  | REL-5 |
| UE security information | OP |  | UE security information 10.3.3.42b |  |  |
| UE security information2 | OP |  | UE security information2 10.3.3.42c |  | REL-6 |
| UE Specific Behaviour Information 1 interRAT | OP |  | UE Specific Behaviour Information 1 interRAT 10.3.3.52 | This IE shall not be included in this version of the protocol |  |
| UE capability container | OP |  |  |  |  |
| >UE radio access capability | MP |  | UE radio access capability 10.3.3.42 |  |  |
| >UE radio access capability extension | MP |  | UE radio access capability extension 10.3.3.42a | Although this IE is not always required, the need has been set to MP to align with the ASN.1 |  |
| UE radio access capability compressed | OP |  | UE radio access capability compressed 10.3.3.42o |  | REL-5 |
| UE radio access capability comp 2 | OP |  | UE radio access capability comp 2 10.3.3.42oa |  | REL-6 |
| Other Information elements |  |  |  |  |  |
| UE system specific capability | OP | 1 to <maxSystemCapability> |  |  |  |
| >Inter-RAT UE radio access capability | MP |  | Inter-RAT UE radio access capability 10.3.8.7 |  |  |
| Failure cause | OP |  | Failure cause 10.3.3.13 | Diagnostics information related to an earlier handover to UTRAN request |  |
| Protocol error information | CV-ProtErr |  | Protocol error information 10.3.8.12 |  |  |
| UE inactivity period | OP |  | Integer (1..120) | Provides expired time since last u-plane activity. If the integer value x is between 1 and 59, it represents the expired time x in seconds. If the integer value x is between 60 and 119, it represents the expired time (x-59) in minutes. If integer value is set to 120, no u-plane activity has been for 60 minutes or more. | REL-9 |

| Condition | Explanation |
| --- | --- |
| ProtErr | This IE is mandatory present if the IE "Protocol error indicator" is included and has the value TRUE. Otherwise it is not needed. |

NOTE1:	To facilitate that network nodes can transparently forward the RRC information received, the INTER RAT HANDOVER INFO WITH INTER RAT CAPABILITIES should include the entire INTER RAT HANDOVER INFO message. The network node, which may not be able to decode the information received, may only append some information to what was received.

NOTE2:	The above table does not need to reflect the order of the information elements in the actual encoded message. The order, that is reflected in the ASN.1, should be chosen in a manner that avoids that network nodes need to perform reordering of information elements.

#### 14.12.4.2	SRNS RELOCATION INFO

This RRC message is sent between network nodes when preparing for an SRNS relocation or a handover/cell reselection from GERAN Iu mode.

With the presence or absence of the IE "RB identity for Hard Handover message" the source RNC indicates to the target SRNC whether the source RNC expects to receive the choice "DL DCCH message" in the IE "RRC information, target RNC to source RNC" in case the SRNS relocation is of type "UE involved". Furthermore the target RNC uses this information for the calculation of the MAC-I.

Direction: source RNC/RATtarget RNC

| Information Element/Group Name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Non RRC IEs |  |  |  |  |  |
| >RB identity for Handover message | OP |  | RB identity 10.3.4.16 | Gives the id of the radio bearer on which the source RNC will transmit the RRC message in the case the relocation is of type "UE involved". In handover from GERAN Iu mode this IE is always set to 2. |  |
| >State of RRC | MP |  | RRC state indicator, 10.3.3.35a |  |  |
| >State of RRC procedure | MP |  | Enumerated (await no RRC message, await RB Release Complete, await RB Setup Complete, await RB Reconfiguration Complete, await Transport CH Reconfiguration Complete, await Physical CH Reconfiguration Complete, await Active Set Update Complete, await Handover Complete, send Cell Update Confirm, send URA Update Confirm, , others) |  |  |
| Ciphering related information |  |  |  |  |  |
| >Ciphering status for each CN domain | MP | <1 to maxCNDomains> |  |  |  |
| >>CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >>Ciphering status | MP |  | Enumerated(Not started, Started) |  |  |
| >>START | MP |  | START 10.3.3.38 | START value to be used in this CN domain. |  |
| >Latest configured CN domain | MP |  | CN domain identity 10.3.1.1 | Value contained in the variable of the same name. In case this variable is empty, the source RNC can set any CN domain identity. In that case, the Ciphering status and the Integrity protection status should be Not started and the target RNC should not initialise the variable Latest configured CN domain. |  |
| >Calculation time for ciphering related information | CV-Ciphering |  |  | Time when the ciphering information of the message were calculated, relative to a cell of the target RNC. In handover and cell reselection from GERAN Iu mode this field is not present. |  |
| >>Cell Identity | MP |  | Cell Identity 10.3.2.2 | Identity of one of the cells under the target RNC and included in the active set of the current call |  |
| >>SFN | MP |  | Integer(0..4095) |  |  |
| >COUNT-C list | OP | 1 to <maxCNdomains> |  | COUNT-C values for radio bearers using transparent mode RLC |  |
| >>CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >>COUNT-C | MP |  | Bit string(32) |  |  |
| >Ciphering info per radio bearer | OP | 1 to <maxRB> |  | For signalling radio bearers this IE is mandatory. |  |
| >>RB identity | MP |  | RB identity 10.3.4.16 |  |  |
| >>Downlink HFN | MP |  | Bit string(20..25) | This IE is either RLC AM HFN (20 bits) or RLC UM HFN (25 bits) |  |
| >>Downlink SN | CV-SRB1 |  | Bit String(7) | VT(US) of RLC UM |  |
| >>Uplink HFN | MP |  | Bit string(20..25) | This IE is either RLC AM HFN (20 bits) or RLC UM HFN (25 bits) |  |
| Integrity protection related information |  |  |  |  |  |
| >Integrity protection status | MP |  | Enumerated(Not started, Started) |  |  |
| >Signalling radio bearer specific integrity protection information | CV-IP | 4 to <maxSRBsetup> |  |  |  |
| >>Uplink RRC HFN | MP |  | Bit string (28) | For each SRB, in the case activation times for the next IP configuration to be applied on this SRB have already been reached this IE corresponds to the last value used. Else this value corresponds to the value the source would have initalized the HFN to at the activation time, not considering any increment of HFN due to RRC SN roll over. Increment of HFN due to RRC SN roll over is taken care of by target based on value sent by the source. |  |
| >>Downlink RRC HFN | MP |  | Bit string (28) | For each SRB, in the case activation times for the next IP configuration to be applied on this SRB have already been reached this IE corresponds to the last value used. Else this value corresponds to the value the source would have initalized the HFN to at the activation time, not considering any increment of HFN due to RRC SN roll over. Increment of HFN due to RRC SN roll over is taken care of by target based on value sent by the source. In particular, for SRB2, this IE should not take into account the RRC message that will trigger the relocation. NOTE: In order to have the possibility of sending downlink messages after the construction of the IE "SRNS RELOCATION INFO", the source may choose a value ahead of the last value used. |  |
| >>Uplink RRC Message sequence number | MP |  | Integer (0.. 15) | For each SRB, this IE corresponds to the last value received or in the case activation time was not reached for a configuration the value equals (activation time -1). |  |
| >>Downlink RRC Message sequence number | MP |  | Integer (0.. 15) | For each SRB, this IE corresponds to the last value used or in the case activation time was not reached for a configuration the value equals (activation time -1). In particular, for SRB2, this IE should not take into account the RRC message that will trigger the relocation. NOTE: In order to have the possibility of sending downlink messages after the construction of the IE "SRNS RELOCATION INFO", the source may choose a value ahead of the last value used for SRB3 and SRB4. |  |
| >Implementation specific parameters | OP |  | Bit string (1..512) |  |  |
| RRC IEs |  |  |  |  |  |
| UE Information elements |  |  |  |  |  |
| >U-RNTI | MP |  | U-RNTI 10.3.3.47 | G-RNTI is placed in this field when performing handover or cell reselection from GERAN Iu mode. |  |
| >C-RNTI | OP |  | C-RNTI 10.3.3.8 |  |  |
| >UE radio access Capability | MP |  | UE radio access capability 10.3.3.42 |  |  |
| >UE radio access capability extension | OP |  | UE radio access capability extension 10.3.3.42a |  |  |
| >Last known UE position | OP |  |  |  |  |
| >>SFN | MP |  | Integer (0..4095) | Time when position was estimated |  |
| >>Cell ID | MP |  | Cell identity; 10.3.2.2 | Indicates the cell, the SFN is valid for. |  |
| >>CHOICE Position estimate | MP |  |  |  |  |
| >>>Ellipsoid Point |  |  | Ellipsoid Point; 10.3.8.4a |  |  |
| >>>Ellipsoid point with uncertainty circle |  |  | Ellipsoid point with uncertainty circle 10.3.8.4d |  |  |
| >>>Ellipsoid point with uncertainty ellipse |  |  | Ellipsoid point with uncertainty ellipse 10.3.8.4e |  |  |
| >>>Ellipsoid point with altitude |  |  | Ellipsoid point with altitude 10.3.8.4b |  |  |
| >>>Ellipsoid point with altitude and uncertainty ellipsoid |  |  | Ellipsoid point with altitude and uncertainty ellipsoid 10.3.8.4c |  |  |
| >UE Specific Behaviour Information 1 idle | OP |  | UE Specific Behaviour Information idle 1 10.3.3.51 | This IE should be included if received via the "INTER RAT HANDOVER INFO", the "RRC CONNECTION REQUEST", the IE "SRNS RELOCATION INFO" or the "Inter RAT Handover Info with Inter RAT Capabilities" |  |
| >UE Specific Behaviour Information 1 interRAT | OP |  | UE Specific Behaviour Information 1 interRAT 10.3.3.52 | This IE should be included if received via the "INTER RAT HANDOVER INFO", the "RRC CONNECTION REQUEST", the IE "SRNS RELOCATION INFO" or the "Inter RAT Handover Info with Inter RAT Capabilities" |  |
| Other Information elements |  |  |  |  |  |
| >UE system specific capability | OP | 1 to <maxSystemCapability> |  |  |  |
| >>Inter-RAT UE radio access capability | MP |  | Inter-RAT UE radio access capability 10.3.8.7 |  |  |
| UTRAN Mobility Information elements |  |  |  |  |  |
| >URA Identifier | OP |  | URA identity 10.3.2.6 |  |  |
| >T305 | OP |  | Integer (5, 10, 30, 60, 120, 360, 720, infinity) | If UE is in CELL_FACH state, this information element shall be included. Value in minutes. Infinity means no update. | REL-7 |
| CN Information Elements |  |  |  |  |  |
| >CN common GSM-MAP NAS system information | MP |  | NAS system information (GSM-MAP) 10.3.1.9 |  |  |
| >CN domain related information | OP | 1 to <MaxCNdomains> |  | CN related information to be provided for each CN domain |  |
| >>CN domain identity | MP |  |  |  |  |
| >>CN domain specific GSM-MAP NAS system info | MP |  | NAS system information (GSM-MAP) 10.3.1.9 |  |  |
| >>CN domain specific DRX cycle length coefficient | MP |  | CN domain specific DRX cycle length coefficient, 10.3.3.6 |  |  |
| Measurement Related Information elements |  |  |  |  |  |
| >For each ongoing measurement reporting | OP | 1 to <MaxNoOfMeas> |  | The source RNC should include the measurement IEs for each measurement configured and ongoing in the UE. |  |
| >>Measurement Identity | MP |  | Measurement identity 10.3.7.48 |  |  |
| >>Measurement Command | MP |  | Measurement command 10.3.7.46 | This IE should have the value Setup. |  |
| >>Measurement Type | CV-Setup |  | Measurement type 10.3.7.50 |  |  |
| >>Measurement Reporting Mode | OP |  | Measurement reporting mode 10.3.7.49 |  |  |
| >>Additional Measurements list | OP |  | Additional measurements list 10.3.7.1 |  |  |
| >>CHOICE Measurement | CV-Setup |  |  |  |  |
| >>>Intra-frequency |  |  |  |  |  |
| >>>>Intra-frequency cell info | OP |  | Intra-frequency cell info list 10.3.7.33 |  |  |
| >>>>Intra-frequency cell info on secondary UL frequency | OP |  | Intra-frequency cell info list on secondary UL frequency 10.3.7.116 |  |  |
| >>>>Intra-frequency measurement quantity | OP |  | Intra-frequency measurement quantity 10.3.7.38 |  |  |
| >>>>Intra-frequency reporting quantity | OP |  | Intra-frequency reporting quantity 10.3.7.41 |  |  |
| >>>>Reporting cell status | OP |  | Reporting cell status 10.3.7.61 |  |  |
| >>>>Measurement validity | OP |  | Measurement validity 10.3.7.51 |  |  |
| >>>>CHOICE report criteria | OP |  |  |  |  |
| >>>>>Intra-frequency measurement reporting criteria |  |  | Intra-frequency measurement reporting criteria 10.3.7.39 |  |  |
| >>>>>Periodical reporting |  |  | Periodical reporting criteria 10.3.7.53 |  |  |
| >>>>>No reporting |  |  | NULL |  |  |
| >>>Inter-frequency |  |  |  |  |  |
| >>>>Inter-frequency cell info | OP |  | Inter-frequency cell info list 10.3.7.13 |  |  |
| >>>>Inter-frequency measurement quantity | OP |  | Inter-frequency measurement quantity 10.3.7.18 |  |  |
| >>>>Inter-frequency reporting quantity | OP |  | Inter-frequency reporting quantity 10.3.7.21 |  |  |
| >>>>Reporting cell status | OP |  | Reporting cell status 10.3.7.61 |  |  |
| >>>>Measurement validity | OP |  | Measurement validity 10.3.7.51 |  |  |
| >>>>Inter-frequency set update | OP |  | Inter-frequency set update 10.3.7.22 |  |  |
| >>>>CHOICE report criteria | MP |  |  |  |  |
| >>>>>Intra-frequency measurement reporting criteria |  |  | Intra-frequency measurement reporting criteria 10.3.7.39 |  |  |
| >>>>>Inter-frequency measurement reporting criteria |  |  | Inter-frequency measurement reporting criteria 10.3.7.19 |  |  |
| >>>>>Periodical reporting |  |  | Periodical reporting criteria 10.3.7.53 |  |  |
| >>>>>No reporting |  |  | NULL |  |  |
| >>>Inter-RAT |  |  |  |  |  |
| >>>>Inter-RAT cell info | OP |  | Inter-RAT cell info list 10.3.7.23 |  |  |
| >>>>Inter-RAT measurement quantity | OP |  | Inter-RAT measurement quantity 10.3.7.29 |  |  |
| >>>>Inter-RAT reporting quantity | OP |  | Inter-RAT reporting quantity 10.3.7.32 |  |  |
| >>>>Reporting cell status | OP |  | Reporting cell status 10.3.7.61 |  |  |
| >>>>Measurement validity | OP |  | Measurement validity 10.3.7.51 |  |  |
| >>>>E-UTRA frequency list | OP |  | E-UTRA frequency list 10.3.7.6b |  | REL-8 |
| >>>>CHOICE report criteria | MP |  |  |  |  |
| >>>>>Inter-RAT measurement reporting criteria |  |  | Inter-RAT measurement reporting criteria 10.3.7.30 |  |  |
| >>>>>Periodical reporting |  |  | Periodical reporting criteria 10.3.7.53 |  |  |
| >>>>>No reporting |  |  | NULL |  |  |
| >>>Traffic Volume |  |  |  |  |  |
| >>>>Traffic volume measurement Object | OP |  | Traffic volume measurement object 10.3.7.70 |  |  |
| >>>>Traffic volume measurement quantity | OP |  | Traffic volume measurement quantity 10.3.7.71 |  |  |
| >>>>Traffic volume reporting quantity | OP |  | Traffic volume reporting quantity 10.3.7.74 |  |  |
| >>>>Measurement validity | OP |  | Measurement validity 10.3.7.51 |  |  |
| >>>>CHOICE report criteria | MP |  |  |  |  |
| >>>>>Traffic volume measurement reporting criteria |  |  | Traffic volume measurement reporting criteria 10.3.7.72 |  |  |
| >>>>>Periodical reporting |  |  | Periodical reporting criteria 10.3.7.53 |  |  |
| >>>>>No reporting |  |  | NULL |  |  |
| >>>Quality |  |  |  |  |  |
| >>>>Quality measurement quantity | OP |  | Quality measurement quantity 10.3.7.59 |  |  |
| >>>>CHOICE report criteria | MP |  |  |  |  |
| >>>>>Quality measurement reporting criteria |  |  | Quality measurement reporting criteria 10.3.7.58 |  |  |
| >>>>>Periodical reporting |  |  | Periodical reporting criteria 10.3.7.53 |  |  |
| >>>>>No reporting |  |  | NULL |  |  |
| >>>UE internal |  |  |  |  |  |
| >>>>UE internal measurement quantity | OP |  | UE internal measurement quantity 10.3.7.79 |  |  |
| >>>>UE internal reporting quantity | OP |  | UE internal reporting quantity 10.3.7.82 |  |  |
| >>>>CHOICE report criteria | MP |  |  |  |  |
| >>>>>UE internal measurement reporting criteria |  |  | UE internal measurement reporting criteria 10.3.7.80 |  |  |
| >>>>>Periodical reporting |  |  | Periodical reporting criteria 10.3.7.53 |  |  |
| >>>>>No reporting |  |  | NULL |  |  |
| >>>UE positioning |  |  |  |  |  |
| >>>>LCS reporting quantity | OP |  | LCS reporting quantity 10.3.7.111 |  |  |
| >>>>CHOICE report criteria | MP |  |  |  |  |
| >>>>>LCS reporting criteria |  |  | LCS reporting criteria 10.3.7.110 |  |  |
| >>>>>Periodical reporting |  |  | Periodical reporting criteria 10.3.7.53 |  |  |
| >>>>>No reporting |  |  |  |  |  |
| Radio Bearer Information Elements |  |  |  |  |  |
| >Predefined configuration status information | OP |  | Predefined configuration status information 10.3.4.5a |  |  |
| >Signalling RB information list | MP | 1 to <maxSRBsetup> |  | For each signalling radio bearer |  |
| >>Signalling RB information | MP |  | Signalling RB information to setup 10.3.4.24 |  |  |
| >RAB information list | OP | 1 to <maxRABsetup> |  | Information for each RAB |  |
| >>RAB information | MP |  | RAB information to setup 10.3.4.10 |  |  |
| >PDCP ROHC target mode | OP |  | PDCP ROHC target mode 10.3.4.2a |  | REL-8 |
| Transport Channel Information Elements |  |  |  |  |  |
| Uplink transport channels |  |  |  |  |  |
| >UL Transport channel information common for all transport channels | OP |  | UL Transport channel information common for all transport channels 10.3.5.24 |  |  |
| >UL transport channel information list | OP | 1 to <MaxTrCH> |  |  |  |
| >>UL transport channel information | MP |  | Added or reconfigured UL TrCH information 10.3.5.2 |  |  |
| Downlink transport channels |  |  |  |  |  |
| >DL Transport channel information common for all transport channels | OP |  | DL Transport channel information common for all transport channels 10.3.5.6 |  |  |
| >DL transport channel information list | OP | 1 to <MaxTrCH> |  |  |  |
| >>DL transport channel information | MP |  | Added or reconfigured DL TrCH information 10.3.5.1 |  |  |
| PhyCH information elements |  |  |  |  |  |
| >TPC Combination Info list | OP | 1 to <maxRL> |  |  |  |
| >>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  |  |
| >>TPC combination index | MP |  | TPC combination index 10.3.6.85 |  |  |
| >TPC Combination Info list on secondary UL frequency | OP | 1 to < maxEDCHRL> |  |  | REL-9 |
| >>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-9 |
| >>TPC combination index | MP |  | TPC combination index 10.3.6.85 |  | REL-9 |
| >E-RGCH Combination Info list | OP | 1 to <maxEDCHRL> |  |  | REL-8 |
| >>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-8 |
| >>RG combination index | MP |  | Integer (0..5) |  | REL-8 |
| >E-RGCH Combination Info list on secondary UL frequency | OP | 1 to <maxEDCHRL> |  |  | REL-9 |
| >>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-9 |
| >>RG combination index | MP |  | Integer (0..5) |  | REL-9 |
| >Transmission gap pattern sequence | OP | 1 to <maxTGPS> |  |  | REL-5 |
| >>TGPSI | MP |  | TGPSI 10.3.6.82 |  | REL-5 |
| >> Current TGPS Status Flag | MP |  | Enumerated(active, inactive) | This flag indicates the current status of the Transmission Gap Pattern Sequence, whether it is active or inactive | REL-5 |
| >>TGCFN | CV-Active |  | Integer (0..255) | Connection Frame Number of the latest past frame of the first pattern within the Transmission Gap Pattern Sequence. | REL-5 |
| >>Transmission gap pattern sequence configuration parameters | OP |  |  |  | REL-5 |
| >>>TGMP | MP |  | Enumerated(TDD measurement, FDD measurement, GSM carrier RSSI measurement, GSM Initial BSIC identification, GSM BSIC re-confirmation, Multi-carrier measurement, | Transmission Gap pattern sequence Measurement Purpose. | REL-5 |
|  |  |  | E-UTRA measurements) |  | REL-8 |
| >>>TGPRC | MP |  | Integer (1..511, Infinity) | The number of remaining transmission gap patterns within the Transmission Gap Pattern Sequence. | REL-5 |
| >>>TGSN | MP |  | Integer (0..14) | Transmission Gap Starting Slot Number The slot number of the first transmission gap slot within the TGCFN. | REL-5 |
| >>>TGL1 | MP |  | Integer(1..14) | The length of the first Transmission Gap within the transmission gap pattern expressed in number of slots | REL-5 |
| >>>TGL2 | MD |  | Integer (1..14) | The length of the second Transmission Gap within the transmission gap pattern. If omitted, then TGL2=TGL1. The value of TGL2 shall be ignored if TGD is set to ”undefined” | REL-5 |
| >>>TGD | MP |  | Integer(15..269, undefined) | Transmission gap distance indicates the number of slots between starting slots of two consecutive transmission gaps within a transmission gap pattern. If there is only one transmission gap in the transmission gap pattern, this parameter shall be set to undefined. | REL-5 |
| >>>TGPL1 | MP |  | Integer (1..144) | The duration of transmission gap pattern 1. | REL-5 |
| >>>RPP | MP |  | Enumerated (mode 0, mode 1). | Recovery Period Power control mode during the frame after the transmission gap within the compressed frame. Indicates whether normal PC mode or compressed PC mode is applied | REL-5 |
| >>>ITP | MP |  | Enumerated (mode 0, mode 1). | Initial Transmit Power is the uplink power control method to be used to compute the initial transmit power after the compressed mode gap. | REL-5 |
| >>>CHOICE UL/DL mode | MP |  |  |  | REL-5 |
| >>>>DL only |  |  |  | Compressed mode used in DL only | REL-5 |
| >>>>>Downlink compressed mode method | MP |  | Enumerated (SF/2, higher layer scheduling) | Method for generating downlink compressed mode gap | REL-5 |
| >>>>UL only |  |  |  | Compressed mode used in UL only | REL-5 |
| >>>>>Uplink compressed mode method | MP |  | Enumerated (SF/2, higher layer scheduling) | Method for generating uplink compressed mode gap | REL-5 |
| >>>>UL and DL |  |  |  | Compressed mode used in UL and DL | REL-5 |
| >>>>>Downlink compressed mode method | MP |  | Enumerated (SF/2, higher layer scheduling) | Method for generating downlink compressed mode gap | REL-5 |
| >>>>>Uplink compressed mode method | MP |  | Enumerated (SF/2, higher layer scheduling) | Method for generating uplink compressed mode gap | REL-5 |
| >>>Downlink frame type | MP |  | Enumerated (A, B) |  | REL-5 |
| >>>DeltaSIR1 | MP |  | Real(0..3 by step of 0.1) | Delta in DL SIR target value to be set in the UE during the frame containing the start of the first transmission gap in the transmission gap pattern (without including the effect of the bit-rate increase) | REL-5 |
| >>>DeltaSIRafter1 | MP |  | Real(0..3 by step of 0.1) | Delta in DL SIR target value to be set in the UE one frame after the frame containing the start of the first transmission gap in the transmission gap pattern. | REL-5 |
| >>>DeltaSIR2 | OP |  | Real(0..3 by step of 0.1) | Delta in DL SIR target value to be set in the UE during the frame containing the start of the second transmission gap in the transmission gap pattern (without including the effect of the bit-rate increase) When omitted, DeltaSIR2 = DeltaSIR1. | REL-5 |
| >>>DeltaSIRafter2 | OP |  | Real(0..3 by step of 0.1) | Delta in DL SIR target value to be set in the UE one frame after the frame containing the start of the second transmission gap in the transmission gap pattern. When omitted, DeltaSIRafter2 = DeltaSIRafter1. | REL-5 |
| >>>N Identify abort | CV-Initial BSIC |  | Integer(1..128) | Indicates the maximum number of repeats of patterns that the UE shall use to attempt to decode the unknown BSIC of the GSM cell in the initial BSIC identification procedure | REL-5 |
| >>>T Reconfirm abort | CV-Re-confirm BSIC |  | Real(0.5..10.0 by step of 0.5) | Indicates the maximum time allowed for the re-confirmation of the BSIC of one GSM cell in the BSIC re-confirmation procedure. The time is given in steps of 0.5 seconds. | REL-5 |
| >Scrambling Code Change List | CH-SF/2 | 1 to <maxRL> |  |  | REL-5 |
| >>Primary CPICH info | MP |  | Primary CPICH info 10.3.6.60 |  | REL-5 |
| >>Scrambling code change | MP |  | Enumerated (code change, no code change) | Indicates whether the alternative scrambling code is used for compressed mode method 'SF/2'. | REL-5 |
| >Serving HS-DSCH cell information | OP |  | Serving HS-DSCH cell information 10.3.6.74a |  | REL-12 |
| >DTX-DRX Information | OP |  | DTX-DRX Information 10.3.6.34a |  | REL-12 |
| >Uplink secondary cell info FDD | OP |  | Uplink secondary cell info FDD 10.3.6.115 |  | REL-12 |
| >DCH Enhancements info FDD | OP |  | DCH Enhancements info FDD 10.3.6.149 |  | REL-12 |
| >Downlink information common for all radio links | OP |  | Downlink information common for all radio links 10.3.6.24 |  | REL-12 |
| >Downlink information per radio link list | OP | 1 to <maxRL> |  | Send downlink information for each radio link to be set-up | REL-12 |
| >>Downlink information for each radio link | MP |  | Downlink information for each radio link 10.3.6.27 |  | REL-12 |
| Other Information elements |  |  |  |  |  |
| >Measurement report | OP |  | MEASUREMENT REPORT 10.2.19 | The source RNC should include the Measurement report the UE sent that triggered the SRNS relocation. This information could e.g. be used by the target RNC to set initial power when establishing a DCH. |  |
| >Failure cause | OP |  | Failure cause 10.3.3.13 | Diagnostics information related to an earlier SRNC Relocation request (see NOTE 2 in 14.12.0a) |  |
| >Protocol error information | CV-ProtErr |  | Protocol error information 10.3.8.12 |  |  |
| UE history information | OP |  | UE history information 10.3.8.23 |  | REL-8 |
| MBMS information elements |  |  |  |  |  |
| MBMS joined information | OP |  |  | Included if the UE has joined one or more MBMS services | REL-6 |
| >P-TMSI | OP |  | P-TMSI (GSM-MAP) 10.3.1.13 | In case the UE is in PMM-Idle | REL-6 |
| MBMS Selected Service Info | OP |  | MBMS Selected Services Info 10.3.9a.7b |  | REL-6 |

| Multi Bound | Explanation |
| --- | --- |
| MaxNoOfMeas | Maximum number of active measurements, upper limit 16 |

| Condition | Explanation |
| --- | --- |
| Setup | The IE is mandatory present when the IE Measurement command has the value "Setup", otherwise the IE is not needed. |
| Ciphering | The IE is mandatory present when the IE Ciphering Status has the value "started" and the ciphering counters need not be reinitialised, otherwise the IE is not needed. |
| IP | The IE is mandatory present when the IE Integrity protection status has the value "started" and the integrity protection counters need not be reinitialised, otherwise the IE is not needed. |
| ProtErr | This IE is mandatory present if the IE "Protocol error indicator" is included and has the value TRUE. Otherwise it is not needed. |
| SRB1 | The IE is mandatory present for RB1. Otherwise it is not needed. |
| Active | This IE is mandatory present when the value of the IE "Current TGPS Status Flag" is "Active" and not needed otherwise. |
| Initial BSIC | This IE is mandatory present when the value of the IE "TGMP" is set to "GSM Initial BSIC identification" and not needed otherwise. |
| Re-confirm BSIC | This IE is mandatory present when the value of the IE "TGMP" is set to "GSM BSIC re-confirmation" and not needed otherwise. |
| SF/2 | The IE is mandatory present if the IE “Transmission Gap Pattern Sequence” is included and has the value “SF/2” as the compressed mode method, and already sent the UE the IE “Scrambling Code Change” for each RL in the active set. Otherwise the IE is not needed. |

#### 14.12.4.3	Void

#### 14.12.4.4	RFC 3095 CONTEXT INFO

This RRC message is sent between network nodes in SRNS relocation. It is used to transfer the compressor and decompressor context information of the ROHC protocol.

Direction: source RNC target RNC

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| RFC 3095 context | MP | 1 to <maxRBallRABs> |  |  | REL-5 |
| >RB identity | MP |  | RB identity 10.3.4.16 |  | REL-5 |
| >RFC 3095 context list | MP | 1 to <maxRFC3095-CID> |  |  | REL-5 |
| >>Downlink RFC 3095 context | OP |  |  |  | REL-5 |
| >>>Downlink RFC 3095 context identity | MP |  | Integer (0..16383) |  | REL-5 |
| >>>DL_MODE | MP |  | Enumerated (u, o, r) | ROHC mode in downlink before SRNS relocation. | REL-5 |
| >>>REF_IR | MP |  | Octet string (1..3000) | The RTP IR header (see section 5.7.7 of RFC3095 for detailed format) corresponding to the oldest header in the compressor sliding window. | REL-5 |
| >>>REF_TIME | OP |  | Integer (0..4294967295) | Arrival time (at the compressor) of REF_IR in milliseconds. See sections 4.5.4 and 6.5.1 of RFC3095. | REL-5 |
| >>>CURR_TIME | OP |  | Integer (0..4294967295) | Current time in milliseconds. See section 6.5.1 of RFC3095. | REL-5 |
| >>>SYN_OFFSET_ID | OP |  | Integer (0..65535) | Last synchronized offset of IP-ID. See section 4.5.5 and 6.5.1 of RFC3095 (termed "Offset_I"). It is related to the compression and decompression of IP-ID and is the synchronized offset between the IP-ID value and the SN value (in the same header) during the last SO state before the relocation procedure. | REL-5 |
| >>>SYN_SLOPE_TS | OP |  | Integer (0..4294967295) | Last synchronized slope of TS. See sections 5.5.1.2 and 5.7 of RFC3095. In SO state, TS(n) = TS(m) + (n-m) * SYN_SLOPE_TS, where n and m are, the RTP SN of the current and the reference packet, respectively. The unit of SYN_SLOPE_TS depends on whether TS is scaled before compression or not. | REL-5 |
| >>>DYN_CHANGED | MP |  | Boolean | Information whether dynamic fields other than RTP SN, RTP TS and IP-ID have changed in the headers that are stored in the sliding window. Set to TRUE if changed and FALSE if not changed. | REL-5 |
| >>Uplink RFC 3095 context | OP |  |  |  | REL-5 |
| >>>Uplink RFC 3095 context identity | MP |  | Integer (0..16383) |  | REL-5 |
| >>>UL_MODE | MP |  | Enumerated (u, o, r) | ROHC mode in uplink | REL-5 |
| >>>REF_IR | MP |  | Octet string (1..3000) | The RTP IR header (see section 5.7.7 of IETF RFC3095 for detailed format) corresponding to the last correctly decompressed header. | REL-5 |
| >>>REF_TIME | OP |  | Integer (0..4294967295) | Arrival time (at the decompressor) of REF_IR in milliseconds. See sectionss 4.5.4 and 6.5.1 of RFC3095. | REL-5 |
| >>>CURR_TIME | OP |  | Integer (0..4294967295) | Current time in milliseconds. See section 6.5.1 of RFC3095. | REL-5 |
| >>>SYN_OFFSET_ID | OP |  | Integer (0..65535) | Last synchronized offset of IP-ID. See sectionss 4.5.5 and 6.5.1 of RFC3095 (termed"Offset_I"). It is related to the compression and decompression of IP-ID and is the synchronized offset between the IP-ID value and the SN value (in the same header) during the last SO state before the relocation procedure. | REL-5 |
| >>>SYN_SLOPE_TS | OP |  | Integer (0..4294967295) | Last synchronized slope of TS. See sectionss 5.5.1.2 and 5.7 of RFC3095. In SO state, TS(n) = TS(m) + (n-m) * SYN_SLOPE_TS, where n and m are, the RTP SN of the current and the reference packet, respectively. The unit of SYN_SLOPE_TS depends on whether TS is scaled before compression or not. | REL-5 |
| >>>REF_SN_1 | OP |  | Integer (0..65535) | Corresponds to the RTP Sequence Number of the predecessor of the latest RTP packet. This could be used to perform local repair of context by decompressor in U or O mode (see “ref - 1” in section5.3.2.2.5 in IETF RFC3095 for further explanation). | REL-5 |