3GPP TSG-RAN WG2 NB-IOT	R2-161411

15 to 19 Feb. 2016, Malta

Agenda Item:			7.16.2.1

Souce:	Samsung

Title:	Specifying solution 2 in the AS specification

Document for:	Discussion

Introduction

This contribution discusses the details of solution 2 in AS point of view.

Discussion

Solution 2 is basically NAS centric mechanism consisted with three component procedures; CIoT Attach Procedure, MO small data transfer procedure and MT small data transfer procedure. In the following, each procedure is discussed in AS perspective.

CIoT Attach Procedure

It is described in section 6.2.1.2 of [1]. The red highlighted parts are inside of RAN2 scope.

In step 0, UE establishes RRC connection with CIoT-BS for attach. RRC connection establishment is a general procedure usually transparent to NAS procedure except EstablishmentCause, which is set according to the mapping table captured below [2].

Table D.1.1: Mapping of NAS procedure to establishment cause and call type

| NAS procedure | RRC establishment cause (according 3GPP TS 36.331 [22]) | Call type |
| --- | --- | --- |
| Attach | If an ATTACH REQUEST has EPS attach type not set to "EPS emergency attach", the RRC establishment cause shall be set to MO signalling except when the UE initiates attach procedure to establish emergency bearer services. (See Note 1) | "originating signalling" |
|  | If an ATTACH REQUEST contains the Device properties IE with low priority indicator set to "MS is configured for NAS signalling low priority", the RRC establishment cause shall be set to Delay tolerant. (See Note 1) | "originating signalling" |
|  | If an ATTACH REQUEST has EPS attach type set to "EPS emergency attach" or if the ATTACH REQUEST has EPS attach type not set to "EPS emergency attach" but the UE initiates the attach procedure on receiving request from upper layer to establish emergency bearer services, the RRC establishment cause shall be set to Emergency call. (See Note 1) | "emergency calls" |

There seems no reason to distinguish the normal attach procedure and CIoT attach procedure during RRC connection establishment phase. They share the same signalling purpose and pose the same signalling overhead. NAS-AS interaction and the mapping rule can be reused as well if the same establishment cause is used.

Proposal 1: Establishment cause for CIoT attach procedure is same as the normal attach procedure

RRCConnectionSetupComplete message conveys Attach Request message (Step1). No impact to AS layer is forseen in this step. In Step 8, RRC connection is released. No impact to AS layer is foreseen.

MO Small Data Transfer

It is described in section 6.2.1.3 of [1]. The red highlighted parts are inside of RAN2 scope.

In step 1, UE establishes RRC connection with CIoT-BS. The purpose is to transfer user data. In the legacy network, service request procedure is triggered for such case and the establishment cause is set to mo-Data. Reusing mo-Data would be a possible option.

Distinguishing normal MO data procedure and small MO data procedure may or may not be beneficial in ENB perspective. If beneficial, mo-Exception-Data instead of mo-Data can be applied. At least for the initial deployment and for the most IoT UEs, small data transfer procedure would be sufficient and the normal procedure may not be used widely. Then the motivation to distinguish two procedures seems not strong.

There is a clear drawback in introducing the new cause value that the current mapping rule should be updated, which may delay the completion of the NB-IOT standardization. Without strong motivation but with the risk of delaying standardization, it would be logical to not use new establishment cause value.

Proposal 2: establishment cause value is set to mo-data for MO small data transfer procedure.

How to carry (or in which message to carry) Ack/Rsp Ind is also an issue. The aim of the indication is to manage S1 connection in more battery friendly manner. Since the S1 connection can be managed by C-SGN (i.e. Solution 2 enabled MME), the indication does not need to be seen by ENB. Then the straightforward way would be to carry the indication inside of NAS message.

Proposal 3: Ack/Rsp Ind is carried by NAS message and transparent to AS layers.

In step 6, as having already been agreed last meeting, small data is transferred over SRB1 in DLInformationTransfer or ULInformationTransfer. No further impact is foreseen.

MT Small Data Transfer

It is described in section 6.2.1.4 of [1]. The red highlighted parts are inside of RAN2 scope.

As discussed in the MO data transfer section, no further impact is foreseen in NAS PDU transfer using DLInformationTransfer or ULInformationTransfer.

Conclusion

Solution 2 is NAS centric mechanism with limited impact to AS. With considerable agreements made last RAN2 meeting, main open issues seem establishment cause and Ack/Rsp Ind. Regarding those issues, following three are proposed.

Proposal 1: Establishment cause for CIoT attach procedure is same as the normal attach procedure

Proposal 2: establishment cause value is set to mo-data for MO small data transfer procedure.

Proposal 3: Ack/Rsp Ind is carried by NAS message and transparent to AS layers.

Reference

[1]	TR 23.270	Architecture enhancements for Cellular Internet of Things

[2]	TS 24.301	Non-Access-Stratum (NAS) protocol for Evolved Packet System

- Annex. Agreements/Proposals and corresponding impacts

| Agreements from the last meeting | Impacts |
| --- | --- |
| For solution 2, a data radio bearer (DRB) is not used in NB-IOT | DRB management is ENB responsibility; No impact foreseen |
| As a baseline agreement, At most one NAS signallig message or NAS message carrying small data can be piggybacked in RRCConnectionSetupComplete in the RRC connection establishment procedure for solution 2. | Already as such; No impact foreseen |
| A UL NAS signalling message or UL NAS message carrying small data can be transmitted by a UL RRC container message for solution 2. A DL NAS signaling or DL NAS small data can be transmitted by a DL RRC container message for solution 2. | No RAN2 issue (however no impact foreseen for RAN3 specification) |
| We assume that AS security is not needed for NB-IoT solution 2. | AS security is controlled by ENB; No impact foreseen |
| We assume that for solution 2, there is no need to differentiate between the different data types (i.e. IP, non-IP or SMS) in the AS level. | Already as such; No impact foreseen |
| There is an RRC establishment cause. | Already as such; No impact foreseen |
| We assume that the following values of RRC establishment cause may be applicable for NB-IOT: mt-Access, mo-Signalling, mo-Data,mo-Exception-Data; FFS if different cause values should be used for CP and UP solution. | TBD (if proposal 1 & 2 agreed, no impact foreseen) |
| The RRC Connection is established for small data transfer for solution 2. | Already as such; No impact foreseen |
| There will be a “wait time” in the RRC connection reject. | Already as such; No impact foreseen |
| We assume that RRC connection reconfiguration is not required for short RRC connections for the solution 2 when the connection is used for small data transfer. | RRC reconfiguration is controlled by ENB; No impact foreseen |
| DL information transfer and UL information transfer messages are supported to carry small data and carried over SRB1 in solution 2. | No impact foreseen |
| RRC connection establishment cause can be used for differentiated handling, e.g. of data and signalling, in AS. It is FFS if anything else is needed. | Already as such; No impact foreseen |
| Proposals in this contribution | Impacts |
| Establishment cause for CIoT attach procedure is same as the normal attach procedure | No impact foreseen |
| establishment cause value is set to mo-data for MO small data transfer procedure | No impact foreseen |
| Ack/Rsp Ind is carried by NAS message and transparent to AS layers | No impact foreseen |

Based on the analysis in the table, we observe followings.

Among the AS core specifications, only 36.331 is relevant to solution 2 (36.304, 36.321, 36.322 and 36.323 are irrelevant).

Regarding RRC specification impacts, RRC connection establishment in 5.3.3, DL information transfer in 5.6.1 and UL information transfer in 5.6.2 would be the relevant sections. For solution 2 to be specified, none of those relevant sections require further update and the current specification seem reusable.