3GPP TSG-RAN WG2 Meeting #93				 								Tdoc R2-161478

Malta, 15 – 19 February 2016

Agenda item:	7.19.2

Source:	Ericsson

Title:	MBMSInterestIndication signalling in Rel-13 (Issues #E.341, #H.052 and #N.028 in ASN.1 review)

WID/SID:	LTE_SC_PTM - Release 13

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we discuss issues related to ASN.1 review issues #E.341, #H.052 and #N.028 pertaining to the signalling when MBMS services of interest (identified by TMGI) are signalled in the MBMS Interest Indication message.

# 2	Background

RAN2 made the following agreements in RAN2#91bis:

Agreements

1	Any service continuity solution should consider both scenarios identified in the WID, i.e. mobility from a cell providing a MBMS service via SC-PTM to a cell also providing this MBMS service via SC-PTM and mobility from a cell providing a MBMS service via SC-PTM to a cell where the UE needs to go to unicast reception to receive this MBMS service.

2	Service continuity solutions shall not require new solution to achieve synchronization.

Following agreements should be used as baseline:

3	Reuse the existing SIB15 based MBMS service continuity concept for SC-PTM. RRC_CONNECTED UEs will send the MBMSInterestIndication message so that the eNB can assist the UE to stay or get on the right carrier frequency during handover. RRC_IDLE UEs will perform autonomous frequency prioritization (i.e. prioritize the SC-PTM frequency for reselection).

4	The UE in RRC_CONNECTED state can report the MBMS service(s) of interest (e.g. TMGI) in the MBMSInterestIndication message.

Agreements

1	The availability in neighbour cells of each MBMS service included in SC-MCCH in the source cell should be indicated. SI or MCCH can be FFS.

In RAN2#92 it was agreed:

Agreements: 1: RoHC is not supported for SC-PTM in Rel-13. 2: PDCP ciphering is not needed for SC-PTM. 3: Apply the specified configuration of MBMS to SC-PTM. 4: For MBMS services not provided via SC-PTM in the serving cell, the serving cell does not broadcast their SC-PTM service availability of neighbour cells. 5: SC-PTM service continuity information is provided in SC-MCCH. The information should not be used to idle mode mobility. 6: Agree on option 2 in R2-156301 as the RRC signalling design for the service continuity information. 7: SC-PTM reception is only supported on PCell for connected UEs. For idle UEs, the behaviour is the same as the current MBMS mechanism. 8: It is optional for a SC-PTM capable UE to support simultaneous reception of unicast transmission and SC-PTM transmission as well as multiple SC-PTM transmissions in one subframe on one carrier. 9: A SC-PTM UE will report the capability on simultaneous reception of unicast transmission and SC-PTM transmission in one subframe on one carrier. 10: It is up to the UE implementation to decide whether to monitor the PDCCH for SC-PTM reception in its measurement gaps. 11: Agree on the following terminologies for SC-PTM: - G-RNTI: RNTI for SC-MTCH scheduling. - SC-RNTI: RNTI for SC-MCCH scheduling. - SC-N-RNTI: RNTI for SC-MCCH change notification.

In summary: To support service continuity, a UE receiving MBMS service via SC-PTM, can also find the availability in neighbour cells of each MBMS service that is included in SC-MCCH in the source cell. The UE reports the MBMS service(s) of interest (e.g. TMGI) in the MBMSInterestIndication message in RRC_CONNECTED state.

In the discussion of current issues for the ASN.1 review, the following issues were indicated:

| N.028 | 5.8.5.3 Determine MBMS frequencies of interest | No clear condition of enetring the further procedures 2> if the UE is MBMS capable (…) 2> if the UE is SC-PTM capable, at most one frequency is included; and | 2 | Clarify what do the procedures mean Ericsson: Agree this is unclear. it can also be assumed that a SC-PTM capable UE supports MBMS. See also comment E.023 ZTE: The SC-PTM UE only supports PCell, namely “at most one frequency”. But the legacy MBMS UE supports more frequencies. Huawei: See H.052 Samsung: The statement ‘at most one frequency is included’ does not seem like a concidion i.e. better placed seperately (e.g. end of section |
| --- | --- | --- | --- | --- |

| E.341 | 5.8.5.3, 5.8.5.3a | 2> if the UE is SC-PTM capable, at most one frequency is included; and I think that the UE should be allowed to follow the legacy behaviour and thus may report more than one frequency. Especially for a UE supporting SC-PTM but interested in “legacy” MBMS shall be allowed to send more than one frequency in MII. Also change may be needed in 5.8.5.3a: 2> one or more MBMS SAIs in the USD for this service is included in SystemInformationBlockType15 acquired from the PCell for a frequency belonging to the set of MBMS frequencies of interest, determined according to 5.8.5.3. | 3 | Needs to be discussed and resolved. However, my understanding is that we agreed in general that we follow legacy behaviour. ASN.1 seems to support signalling of more than one frequency (?). E-mail discussion suggest further discussion in ASN.1 review or later. Huawei: This scenario wasn't discussed. In this scenario, indicating more than one frequency could result in the eNB configuring the PCell on a frequency which isn't the one providing the MBMS service that the UE prefers. We could discuss whether it is more important to ensure reception of at least the UE most preferred MBMS service or to allow reception of multiple MBMS services. For 5.8.5.3a, it is true that today there will only be one frequency, it could be changed to "the MBMS frequency of interest". Samsung: The UE may support both MBSFN and SCPTM, but will network uses both mechanisms also e.g. popular services by MFSFN? In such case I assume intention is that UE just includes one SCPTM frequency ZTE: The legacy UE is not allowed to report the number of frequencies exceeding the UE capability. As the SC-PTM UE only supports PCell, then we think the legacy rule can be applied to only one frequency (namely PCell). |
| --- | --- | --- | --- | --- |

This was summarised in the RAN2 ASN.1 AdHoc minutes:

R2-160009	[E.341] Reporting of MBMS frequencies of interest for SC-PTM	Nokia Networks	discussion

R2-160012	Issues #E.341 and #N.028 - MBMSInterestIndication message signalling in Rel-13	Ericsson	discussion

=>	Intention is to solve this issue to avoid disadvantaging UEs that support SC-PTM and MBMS when operating in a network that does not support SC-PTM.

=>	Issue will be revisited at RAN2#93.

# 3	Discussion

The following can be assumed, e.g.:

- A UE supporting SC-PTM is also likely supporting MBMS

- A UE supporting SC-PTM may be interested in a MBMS service provided on a MBSFN MRB, either with higher priority or in situations where interested services only are present via MBSFN MRB.

- Other similar scenarios (not listed)

In the current procedure text, it can be seen that the UE behaviour is unclear in what the UE should indicate with the MII. For example, in the draft CR section 5.8.5.3 it is not clear if the list of frequencies of interest is influenced from the capability only, or if the presence of services on SC-PTM or MBMS (subject to UE prioritization) influence the resulting list.

For instance, an UE that is MBMS and SC-PTM capable (e.g. supports MRB reception on SCell), should be able report more than one frequency.

In summary, the signaling procedure needs to support the following cases:

| Cases | UE |  | eNB |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  | Legacy MBMS supported | SC-PTM supported | Legacy MBMS in use | SC-PTM in use |  |
| Case 1 | Y | N | * | * | Case for legacy UEs. |
| Case 2 | Y | * | Y | N | Case for legacy eNBs. |
| Case 3 | Y | Y | Y | Y | Everything is supported and in use. |
| Case 4 | N | Y | * | * | Can be considered a corner case. |
| Case 5 | * | Y | N | Y | Only SC-PTM is used in the NW. |

We note the following:

-	For cases 1 and 2, the UE behavior should be the same as legacy MBMS, i.e. should not depend on UE support of SC-PTM

-	For case 3 the UE behavior depends on what services are provided where, for example a UE could prioritize a service over MBMS even if another is provided over SC-PTM (and vice versa). Also here the NW uses only legacy MBMS for services while supporting SC-PTM.

-	For case 4 the signaling may support this, but we consider this to be a corner case as the discussion has been to base the functionality on “legacy MBMS”.

-	For case 5 we need to take into consideration that the network may only use SC-PTM.

The target of the current optimizations in procedures seems too narrow to encompass what is needed; the question for 4 is what the UE actually supports (compared to MBMS) and what/if any choice can actually be made at the MCE.

With regards to solutions, one option is to allow an UE that is (MBMS and) SC-PTM capable to reduce the number of reported frequencies when to receive MBMS service via SC-PTM. For example in those cases service availability has been verified in the SC-MCCH. By basing the reported MBMS frequencies according to 5.8.5.3 and SC-PTM frequency according to 5.8.5.3a, the UE can adapt MBMS-FreqList according to prioritization.

Alternatively, since the UE also provides the TMGI in the MII along with the frequencies, the eNB will know that the UE is interested in receiving a service (e.g. via SC-PTM if applicable) and can hence configure that one as PCell, while the other “frequencies” are configured as SCells.

Note that the information exchange of services provided in neighbour cells is already in place, as the availability of each neighbour SC-PTM MBMS service is included on SC-MCCH in the source cell.

For any solution, the UE behaviour has to also maintain the legacy behaviour w.r.t MBMS.

In section 3, a possible solution of UE procedures w.r.t to MII is outlined, where the UE behaviour is aimed to result in a more predictable outcome. Note that the prioritization as such should still be left for UE implementation, similar to current specification of MII.

# 4	Text proposal

### 5.8.5	MBMS interest indication

#### 5.8.5.1	General

Figure 5.8.5.1-1: MBMS interest indication

The purpose of this procedure is to inform E-UTRAN that the UE is receiving or is interested to receive MBMS service(s) via an MRB or SC-MRB, and if so, to inform E-UTRAN about the priority of MBMS versus unicast reception.

#### 5.8.5.2	Initiation

An MBMS or SC-PTM capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment, upon entering or leaving the service area, upon session start or stop, upon change of interest, upon change of priority between MBMS reception and unicast reception or upon change to a PCell broadcasting SystemInformationBlockType15.

Upon initiating the procedure, the UE shall:

1>	if SystemInformationBlockType15 is broadcast by the PCell:

2>	ensure having a valid version of SystemInformationBlockType15 for the PCell;

2>	if the UE did not transmit an MBMSInterestIndication message since last entering RRC_CONNECTED state; or

2>	if since the last time the UE transmitted an MBMSInterestIndication message, the UE connected to a PCell not broadcasting SystemInformationBlockType15:

3> if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, is not empty:

4>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

2>	else:

3>	if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, has changed since the last transmission of the MBMSInterestIndication message; or

3>	if the prioritisation of reception of all indicated MBMS frequencies compared to reception of any of the established unicast bearers has changed since the last transmission of the MBMSInterestIndication message:

4>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

NOTE:	The UE may send an MBMSInterestIndication even when it is able to receive the MBMS services it is interested in i.e. to avoid that the network allocates a configuration inhibiting MBMS reception.

3>	else if SystemInformationBlockType20 is broadcast by the PCell:

4>	if since the last time the UE transmitted an MBMSInterestIndication message, the UE connected to a PCell not broadcasting SystemInformationBlockType20; or

4>	if the set of MBMS services of interest determined in accordance with 5.8.5.3a is different from mbms-Services included in the last transmission of the MBMSInterestIndication message;

5>	initiate the transmission of the MBMSInterestIndication message in accordance with 5.8.5.4.

#### 5.8.5.3	Determine MBMS frequencies of interest

The UE shall:

1>	consider a frequency to be part of the MBMS frequencies of interest if the following conditions are met:

2>	at least one MBMS session the UE is receiving or interested to receive via an MRB or SC-MRB is ongoing or about to start; and

NOTE 1:	The UE may determine whether the session is ongoing from the start and stop time indicated in the User Service Description (USD), see 3GPP TS 36.300 [9] or 3GPP TS 26.346 [57].

2>	for at least one of these MBMS sessions SystemInformationBlockType15 acquired from the PCell includes for the concerned frequency one or more MBMS SAIs as indicated in the USD for this session; and

NOTE 2:	The UE considers a frequency to be part of the MBMS frequencies of interest even though E-UTRAN may (temporarily) not employ an MRB or SC-MRB for the concerned session. I.e. the UE does not verify if the session is indicated on (SC-)MCCH

NOTE 3:	The UE considers the frequencies of interest independently of any synchronization state, e.g. [9, Annex J.1]

2>	if the UE is MBMS capable, the UE is capable of simultaneously receiving MRBs on the set of MBMS frequencies of interest, regardless of whether a serving cell is configured on each of these frequencies or not; and

2>	the supportedBandCombination the UE included in UE-EUTRA-Capability contains at least one band combination including the set of MBMS frequencies of interest;

NOTE 4:	Indicating a frequency implies that the UE supports SystemInformationBlockType13 acquisition for the concerned frequency i.e. the indication should be independent of whether a serving cell is configured on that frequency.

NOTE 5:	When evaluating which frequencies it can receive simultaneously, the UE does not take into account the serving frequencies that are currently configured i.e. it only considers MBMS frequencies it is interested to receive.

NOTE 6:	The set of MBMS frequencies of interest includes at most one frequency for a given physical frequency. The UE only considers a physical frequency to be part of the MBMS frequencies of interest if it supports at least one of the bands indicated for this physical frequency in SystemInformationBlockType1 (for serving frequency) or SystemInformationBlockType15 (for neighbouring frequencies). In this case, E-UTRAN may assume the UE supports MBMS reception on any of the bands supported by the UE (i.e. according to supportedBandCombination).

#### 5.8.5.3a	Determine MBMS services of interest

The UE shall:

1>	consider a MBMS service to be part of the MBMS services of interest if the following conditions are met:

2>	the UE is SC-PTM capable; and

2>	the UE is receiving or interested to receive this service via an SC-MRB; and

2>	one session of this service is ongoing or about to start; and

2>	one or more MBMS SAIs in the USD for this service is included in SystemInformationBlockType15 acquired from the PCell for a frequency belonging to the set of MBMS frequencies of interest, determined according to 5.8.5.3.

#### 5.8.5.4	Actions related to transmission of MBMSInterestIndication message

The UE shall set the contents of the MBMSInterestIndication message as follows:

1>	if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, is not empty:

2>	include mbms-FreqList and set it to include the MBMS frequencies of interest, using the EARFCN corresponding with freqBandIndicator included in SystemInformationBlockType1 (for serving frequency), if applicable, and the EARFCN(s) as included in SystemInformationBlockType15 (for neighbouring frequencies);

NOTE 1:	The EARFCN included in mbms-FreqList is merely used to indicate a physical frequency the UE is interested to receive i.e. the UE may not support the band corresponding to the included EARFCN (but it does support at least one of the bands indicated in system information for the concerned physical frequency).

2>	include mbms-Priority if the UE prioritises reception of all indicated MBMS frequencies above reception of any of the unicast bearers;

1>	if an MBMS service is considered to be part of the MBMS services of interest in accordance with 5.8.5.3a:

2>	include mbms-FreqList and set the mbms-FreqList to include the MBMS frequencies of interest, using the EARFCN corresponding with freqBandIndicator included in SystemInformationBlockType1 (for serving frequency), if applicable, and the EARFCN(s) as included in SystemInformationBlockType15 for neighbouring frequencies, if applicable;

NOTE 3:	If the UE is SC-PTM capable, the UE may reduce the number of frequencies of interest if the UE prioritises MBMS reception via a SC-MRB.

2>	if SystemInformationBlockType20 is broadcast by the PCell:

3>	include mbms-Services and set it to indicate the set of MBMS services of interest determined in accordance with 5.8.5.3a;

NOTE 2:	If the UE prioritises MBMS reception and unicast data cannot be supported because of congestion on the MBMS carrier(s), E-UTRAN may initiate release of unicast bearers. It is up to E-UTRAN implementation whether all bearers or only GBR bearers are released. E-UTRAN does not initiate re-establishment of the released unicast bearers upon alleviation of the congestion.

The UE shall submit the MBMSInterestIndication message to lower layers for transmission.

# 4	Conclusion

In this paper, we discuss issues #E.341, #H.52 and #N.028 pertaining to the signalling when MBMS services of interest (identified by TMGI) are signalled in the MBMS Interest Indication message. It is proposed to discuss and amend the procedures according to the suggestions in section 4, text proposal.