3GPP TSG RAN WG2 Meeting #93                        R2-161483

St Julian’s, Malta 15th -19th Feb 2016

Source: 		NEC

Title:			Serving WLAN reporting in LWA

Agenda Item:		7.6.2.2

Document for:	     Discussion and Decision

# Introduction

In this contribution we look into the need to report serving WLAN to the eNB during intra mobility set mobility.

# 2. Discussion

We consider a scenario where more than single WLAN is part of the mobility set being controlled by a WT in the coverage of an LTE cell and look into measurements reported over Uu and Xw interface.

UE measurement report includes:

MeasResultListWLAN-r13 ::=		SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultWLAN-r13

MeasResultWLAN-r13 ::=	SEQUENCE {

wlan-Identifiers-r13					WLAN-Identifiers-r12,

carrierInfo-r13						WLAN-CarrierInfo-r13			OPTIONAL,

rssiWLAN-r13							WLAN-RSSI-Range-r13			OPTIONAL,

avaiableAdmissionCapacity-r13		INTEGER (0..31250)	OPTIONAL,

backhaulDL-Bandwidth-r13				WLAN-backhaulRate-r12	OPTIONAL,

backhaulUL-Bandwidth-r13				WLAN-backhaulRate-r12	OPTIONAL,

channelUtilization-r13				INTEGER (0..255)			OPTIONAL,

stationCount-r13						INTEGER (0..65535),

nonCriticalExtension					SEQUENCE {}				OPTIONAL

}

WLAN-RSSI-Range-r13 ::=	INTEGER (-128..127)

UE is allowed to report multiple WLANs but does not report serving BSSID. Even though best WLAN is included first in the measurement results but this does not ensure for the cases where best cell is not the serving one. Also, new RRC message WLAN Status Report message does not allow reporting of serving BSSID:

WLAN-Status-IEs ::=	SEQUENCE {

wlan-Status-r13					WLAN-Status-r13,

wlan-Identifiers-13				WLAN-Identifiers-r12			OPTIONAL	,	-- Cond Fail

...

}

Even though UE reports WLAN identifiers but only in the case when association fails with a particular WLAN.

Similarly, there is no information over Xw interface informing the eNB about UE’s serving WLAN. We look into WT ASSOCIATION CONFIRM.

#### 9.1.26 WT ASSOCIATION CONFIRMATION This message is sent by the WT to the eNB to confirm that a certain UE successfully associated with the WLAN. Direction: WT eNB. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.1; YES; reject. eNB UE XwAP ID; M; 9.2.25; Assigned by the eNB; YES; ignore. WT UE XwAP ID; M; 9.2.25; Assigned by the WT; YES; ignore.

This message does not contain any WLAN identifier.

Further, measurements related to multiple WLANs is included in the WT STATUS REPORT

#### 9.1.10 WT STATUS REPORT This message is sent by the WT to the eNB to report the results of the requested measurements. Direction: WT eNB. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.1; YES; reject. eNB Measurement ID; M; INTEGER (1..4095,...); Allocated by eNB; YES; reject. WT Measurement ID; M; INTEGER (1..4095,...); Allocated by eNB2; YES; reject. BSS Measurement Result List; 1; YES; ignore. >BSS Measurement Result Item; 1 .. <maxnoofBSSsperWT>; EACH; ignore. >>BSSID; M; 9.2.8. >>BSS Load; O; 9.2.11. >>WAN Metrics; O; 9.2.12. Range bound; Explanation. maxnoofBSSsperWT; 4096.

But serving BSSID or WLAN is missing in this message as well.

Observation 1: There is no signalling to report UE’s serving WLAN over Uu and/or Xw interface if there is more than one WLAN in the UE mobility set

As shown in figure 1 below as an example, mobility between AP1 and AP2 involves UE, WLAN AP, and WT but not eNB. Though eNB receives WLAN measurements both from the UE and WT regarding WLAN AP1 and AP2 as mentioned above.

Figure 1: WLAN mobility with no LTE mobility

We believe that there are many scenarios whereby eNB must have the information about UE’s serving WLAN or AP as discuss below:

In the example of figure 1, if all LWA configured UEs are in AP1 and none in AP2 then eNB does not need any measurements from AP2 and can potentially reduce the signalling over Xw and Uu interface by removing the need of measurements being reported from AP1. It is worth mentioning that the measurement results itself from AP1 won’t indicate the absence of LWA UEs because these APs may be shared with non-LWA UEs. If multiple WLANs are involved then eNB will receive multiple reports causing excessive signalling over RRC and Xw.

Also, if the eNB has no knowledge of UE distribution in different WLANs, the whole purpose of sharing measurement results (BSS load and WLAN metric) gets defeated because it is not clear how would eNB use these measurements. The argument to report such measurements, in our understanding, is to facilitate the eNB with the information about resource utilisation on the WLAN side. But without knowing the serving WLAN information, how would eNB coordinate such information to the individual UE’s scheduling is not clear.

If eNB is aware of serving WLAN AP then it can e.g. project probable path and deactivate LWA, match required QoS and throughput as ensuring QoS for a bearer. The eNB can also optimise the configuration for measurements both in the UE as well as in the WT.

We therefore propose that:

Proposal 1: RAN2 to agree that UE shall report the serving WLAN node within a mobility set.

The stage-3 changes are minimal as shown below:

Option 1: Explicit reporting in MeasResultListWLAN-r13 by adding either a new IE or listing WLAN identifiers with serving WLAN as the first entity (no new IE needed)

Option 2: Remove the condition (Cond Fail) below and allow the UE to report for successful case as well in WLAN-Status-IEs

wlan-Identifiers-13	WLAN-Identifiers-r12	OPTIONAL	,	-- Cond Fail

Proposal 2: RAN2 to discuss between option 1 and option 2 for stage-3

# Conclusion

We propose RAN2 to discuss and agree following observations and proposals

Observation 1: There is no signalling to report UE’s serving WLAN over Uu and/or Xw interface if there is more than one WLAN in the UE mobility set

Proposal 1: RAN2 to agree that UE shall report the serving WLAN node within a mobility set.

Proposal 2: RAN2 to discuss between option 1 and option 2 for stage-3

# References

- RAN2#92 Chairman notes

- R2-157095: Introduction of LTE-WLAN Aggregation 36.331, Qualcomm

- Draft 36.463: Xw application protocol (XwAP) V0.4.0