3GPP TSG-RAN2 Meeting #93	R2-161509

St. Julian's, Malta, 15th – 19th February 2016

Agenda item:	7.6.2.1

Source: 	Intel Corporation

Title: 	Remaining LWA stage-2 issues

Document for:	Discussion and decision

# 1. Introduction

In this contribution we discuss a few remaining LWA Stage-2 issues, specifically LWA operation in UM (Unacknowledged) mode, and editorial changes to figures in 36.300 CR to clarify naming of bearers.

# 2. Discussion

## AM/UM

In LWA discussions RAN2 have always assumed, that similar to DC, LWA only works with RLC AM. However, this implicit assumption has not yet been captured in RAN2 specifications. It should be noted that, DC stage-2 description [2], captures this assumption explicitly, with  the following text: “Only the RLC AM bearer can be configured for the split bearer”.

To resolve any possible ambiguity, we suggest to capture, at least in stage-2, a similar text for LWA.

Proposal 1: to capture, at least in stage-2, a statement  reflecting  that only RLC AM bearer can be configured as LWA bearer.

The stage-2 text might be as follows: “Only the RLC AM bearer can be configured for the LWA bearer”.

## LWA bearers

Figures 22A.1.2-1 and 22A.1.2-2 in TS 36.300 [1], reproduced below, show a bearer, which is using the LTE link only.

Figures 22A.1.2-1 and 22A.1.2-2 from TS 36.300 [1]

We believe that in these figures the bearer which is using LTE only should not be referred to as LWA bearer, but rather LTE bearer.

Proposal 2: to modify “LWA bearer” name for bearers using LTE only in figures 22A.1.2-1 and 22A.1.2-2 in TS 36.300 [1].

# 3. Proposals

Proposal 1: to capture, at least in stage-2, a statement reflecting that only RLC AM bearer can be configured as LWA bearer.

Proposal 2: to modify “LWA bearer” name for bearers using the LTE link only in figures 22A.1.2-1 and 22A.1.2-2 in TS 36.300 [1].

A companion CR based on the above proposals is provided in [2].

# 4. References

[1] TS 36.300, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2

[2] R2-161510, TS 36.300 Stage-2 CR correction for LWA