3GPP TSG-RAN WG2 Meeting #93	R2-161613

# St. Julian's, Malta, 15th – 19th February 2016

# Agenda Item: 6.2.9.2 (TEI12)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 0841 | rev | - | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCH MAC PDU for unscheduled or suspended MTCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12 |  |  |  |  |  |  | Date: |  | 2016-02-15 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | It is not clear whether the MAC subheader and MAC SDU for the unscheduled MTCH is included in the MAC PDU or not. It is not clear whether the MAC subheader and MAC CE for the MSI or Extended MSI is included in the MAC PDU or not if all MTCHs are suspended. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a NOTE for the eNB behavior to ensure synchronous MBMS transmission. Impact analysis Impacted functionality: MCH MAC PDU construction in eNB Inter-operability: If the network is implemented according to the CR and the UE is not, there is no problem. If the UE is implemented according to the CR and the network is not, the UE may not perform soft-combining of MCH MAC PDU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Different eNBs may transmit MCH MAC PDU in different format, and the UE cannot perform soft-combining of MCH MAC PDU. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

5.12	MCH reception

MCH transmission may occur in subframes configured by upper layer for MCCH or MTCH transmission. For each such subframe, upper layer indicates if signallingMCS or dataMCS applies. The transmission of an MCH occurs in a set of subframes defined by PMCH-Config. An MCH Scheduling Information MAC control element is included in the first subframe allocated to the MCH within the MCH scheduling period to indicate the position of each MTCH and unused subframes on the MCH. If pmch-InfoListExt is configured for an MCH, an Extended MCH Scheduling Information MAC control element is included in the first subframe allocated to the corresponding MCH within the MCH scheduling period to indicate the position of each MTCH and unused subframes on the MCH, and to indicate whether MTCH transmission is to be suspended. The MAC entity shall assume that the first scheduled MTCH starts immediately after the MCCH or the MCH Scheduling Information MAC control element or the Extended MCH Scheduling Information MAC control element if the MCCH is not present, and the other scheduled MTCH(s) start immediately after the previous MTCH, at the earliest in the subframe where the previous MTCH stops. When the MAC entity needs to receive MCH, the MAC entity shall:

-	attempt to decode the TB on the MCH;

-	if a TB on the MCH has been successfully decoded:

-	demultiplex the MAC PDU and deliver the MAC SDU(s) to upper layers.

When the MAC entity receives the Extended MCH Scheduling Information MAC control element, the MAC entity shall indicate the MTCH(s) to be suspended to the upper layers.

NOTE1:	The MAC entity should continue receiving MCH until the MTCH is removed from the MCCH.

NOTE2:	For the unscheduled MTCH, the eNB should not include the MAC subheader and MAC SDU for the corresponding MTCH in the MAC PDU.

NOTE3:	If all MTCHs are suspended, the eNB should not include the MAC subheader and the MAC control element for the MCH Scheduling Information or Extended MCH Scheduling Information in the MAC PDU.