3GPP TSG-RAN WG2 Meeting #93	                                                                   				    R2-161747

St. Julian's, Malta, 15th – 19th February 2016

Agenda item:	7.18

Source:	CMCC

Title:	Discussion on uplink data compression

Document for:	Discussion

1	Introduction

UDC (Uplink data compression) is a method to increase uplink channel capacity by reducing redundancy of uplink data. In this contribution, we firstly introduce the basic principle of UDC and illustrate one of UDC algorithms as an example. And then, we present some field test results to evaluate UDC performance, based on which we share some observations on UDC.

2	Discussion on uplink data compression

2.1	Algorithm of uplink data compression

Uplink data compression is a method to increase uplink channel capacity by reducing uplink data redundancy, especially repetitive bytes. Many algorithms can be used for uplink data compression. In this section, we take one of them as an example.

As shown in figure 1, the UE and eNB maintain same memories, which are synchronously updated. In uplink transmission, the compressor in UE removes byte-strings which occur in memory, and adds a header to the compressed packet. By processing the header of compressed packet, the decompressor in eNB can determine where in memory to retrieve missing bytes and obtain the decompressed packet. The efficiency of this UDC algorithm depends on the size of repeated byte-strings in memory.

Figure 1 An example of UDP algorithm

2.2	UDC Evaluation

In this section, field test results are provided to evaluate the performance of UDC in our network.

Evaluation criteria

In order to evaluate the performance of UDC, compression gain is defined as follows. The value of compression gain indicates the efficiency of UDC. And the higher the compression gain, the more efficient the UDC algorithm is.

compression gain= bytes before compressionbytes after compression-1×100%

Field test results

In order to evaluate the performance of UDC for different applications in our network, we conduct field tests based on algorithm in section 2.1.

Figure 2 Compression gains of different applications

Figure 2 shows compression gain of different applications in field tests. UDC achieves the highest compression gain for web surfing, and a relatively high compression gain for text uploading and online video. For instant messaging, text shows higher gain than voice and video, since voice and video over instant messaging have already been compressed in application layer.

Observation 1: UDC can achieve over 50% compression gain for web surfing, text uploading, online video and text over instant message.

Application ratio in real network

Field tests on application ratio in Zhejiang China are shown in order to find out whether UDC is useful in real network.

Figure 3 Uplink data ratio of application in Zhejiang China

Figure 3 shows that Web surfing, which can achieve 150% compression gain, is the main contributor of uplink data transmission. According to Figure 2, web surfing, text over instant messaging and online video can all achieve over 50% compression gain, which means that almost 70% of uplink data transmission can benefit from UDC. Hence, we believe that UDC can achieve high compression gain and increase uplink capacity in real network.

it is reasonable to conclude that UDC can achieve uplink capacity gain in our network.

Observation 2: UDC can achieve high compression gain and increase uplink capacity in real network.

3	Conclusions

In this paper, we have discussed the effect of uplink data compression. We share some observations:

Observation 1: UDC can achieve over 50% compression gain for web surfing, text uploading, online video and text over instant message.

Observation 2: UDC can achieve high compression gain and increase uplink capacity in real network.