3GPP TSG-RAN WG2 Meeting#95bis                                                                  R2-166211

Kaohsiung, 10th-14th October 2016

Source:	CATT

Title:	Multicast on Non-anchor Carrier for NB-IoT

Agenda Item:	8.11.1

Document for:	Discussion and Decision

- Introduction

In RAN2#95, general aspects of multi-cast  for NB- IoT have been discussed in [1] and [2] and agreed that:

The Rel-13 SC-PTM architecture is assumed for multi-cast design for NB-IoT and MTC.

Reception of multi-cast in RRC_IDLE mode is required by both NB-IoT and MTC.

Reception of multi-cast in RRC_CONNECTED mode is not required for NB-IoT and FFS for MTC.

Service continuity of multi-cast should be supported as in Rel-13 for Idle mode for NB-IoT and MTC.

RAN2 assumes that the legacy SC-MTCH mechanism in which the SC-MTCH is scheduled by PDCCH is reused for multi-cast in NB-IoT and MTC to achieve flexible scheduling.

RAN2 assumes that repetition for SC-MTCH transmission will be introduced for multi-cast in NB-IoT and MTC.

To use SIB20 (or a somewhat modified variant) for SC-MCCH configuration (a new SIB20-NB for NB-IoT).

Both SC-MCCH and SC-MTCH can maybe be scheduled on anchor carrier and/or non-anchor carrier for NB-IoT.

SC-MCCH and SC-MTCH can maybe be scheduled on different carriers for NB-IoT and for MTC (narrowband for MTC). Need to consider e.g. MCCH modification and e.g. frequency hopping for MTC.

SC-MCCH and SC-MTCH scheduled on anchor carrier and/or non-anchor carrier for NB-IoT needs further discussion. The mechanism of multicast on anchor carrier and/or non-anchor carrier will be discussed in the contribution.

- Discussion

Multicast services transmission would be discussed in the aspects of SC-MCCH scheduling information, SC-MCCH, SC-MTCH and SC-MCCH change notification. It is agreed that SIB20 (or a somewhat modified variant) is used for SC-MCCH configuration (a new SIB20-NB for NB-IoT). It is unnecessary to transmit SIB20 on non-anchor carrier since the size of SIB20 is acceptable to be transmitted on anchor carrier.

Porposal1. Scheduling information for SC-MCCH is transmitted on anchor carrier.

In R13 SC-PTM, a notification mechanism is used to announce changes of SC-MCCH and SC-MCCH notification is sent in the same subframe with SC-MCCH, using a SC-N-RNTI. It is unnecessary to send different SC-MCCH change notification for different multicast services.

Porposal2. SC-MCCH change notification is transmitted on one carrier only and the carrier is notified in scheduling information for SC-MCCH.

In case of multicast on non-anchor carrier, three options can be used to transmit SC-MCCH.

Option1. SC-MCCH is transmitted on one carrier.

Option2. SC-MCCH for different multicast services is transmitted on different carriers.

Option3. SC-MCCH for the same multicast services is transmitted on different carriers.

It is obvious that option 3 introduces more signalling overhead than the other two options. The maximum number of SC-MTCHs in one cell is 1024 in R13 and each SC-MTCH information includes MBMS service TMGI and optional session ID, associated G-RNTI and scheduling information. Option2 is flexible and feasible for load balance between carriers and load balance need to be considered. In addition, if SC-MCCH change could be notified for each carrier which is used to transmit SC-MCCH, UE’s power consumption might be reduced since UE would not receive SC-MCCH on the carrier(s) on which SC-MCCH does not change.  Option1 is simple but less flexible.

Porposal3. RAN2 discuss the maximum number of SC-MTCH would be used for NB-IoT and which option should be used for SC-MCCH transmission.

Option1. SC-MCCH is transmitted on one carrier.

Option2. SC-MCCH for different multicast services is transmitted on different carriers.

It is obvious that SC-MTCH should be transmitted on anchor carrier and/or non-anchor carrier depending on the scheduling of SC-MCCH for load balancing.

Porposal4. SC-MTCH is transmitted on any carrier(s) notified on SC-MCCH.

- Conclusion

This contribution discusses the issues of the multicast enhancement on non-anchor carrier for NB-IoT. The following proposal is made:

Porposal1. Scheduling information for SC-MCCH would be transmitted on anchor carrier.

Porposal2. SC-MCCH change notification is transmitted on one carrier only and the carrier is notified in scheduling information for SC-MCCH.

Porposal3. RAN2 discuss the maximum number of SC-MTCH would be used for NB-IoT and which option should be used for SC-MCCH transmission.

Option1. SC-MCCH is transmitted on one carrier.

Option2. SC-MCCH for different multicast services is transmitted on different carriers.

Porposal4. SC-MTCH is transmitted on any carriers depending on scheduling on SC-MCCH.

- Reference

[1] R2-165375	General Aspects of Multi-Cast in NB-IoT	Huawei, HiSilicon, Neul Ltd.

[2] R2-164857	Consideration on supporting multicast in NB-IoT	ZTE Corporation