3GPP TSG-RAN WG2 Meeting #98                                                                 	R2-1704230

Hangzhou, China, 15th - 19th May 2017

Source:	CATT

Title:	RRC connection re-establishment and resume procedures in NR

Agenda Item:	10.4.1.2

Document for:	Discussion and Decision

# Introduction

In last RAN2 meeting, some guidelines are agreed for the RRC messages and procedures [1].

Agreement 1 Aim to limit the number of RRC messages i.e. avoid introducing several messages with similar content/ similar procedural handling (details can be discusses when more progress has been made on the individual procedures)

This contribution discusses the details of individual procedures for RRC connection re-establishment and resume.

# Discussion

## RRC connection re-establishment

In LTE, The purpose of this procedure is to re-establish the RRC connection, which involves the resumption of SRB1 operation, the re-activation of security and the configuration of only the PCell.

Figure 1 RRC connection re-establishment, successful

Figure 2 RRC connection re-establishment, failure

The function of re-establishing the RRC connection should naturally be supported in NR for UE in connected mode, including the successful and failure procedures.

Proposal 1: Support the successful and failure procedures for RRC connection re-establishment in NR.

The content of the RRC messages for RRC connection re-establishment are listed in the table 1.

Table 1 content of the RRC messages for RRC connection re-establishment

| RRC message | Content in LTE | Proposed Content in NR |
| --- | --- | --- |
| RRCConnectionReestablishmentRequest | - ue-Identity - reestablishmentCause | - ue-Identity - reestablishmentCause |
| RRCConnectionReestablishment | - rrc-TransactionIdentifier - radioResourceConfigDedicated (for SRB1) - NextHopChainingCount | - rrc-TransactionIdentifier - radioResourceConfigDedicated (for SRB1) - NextHopChainingCount |
| RRCConnectionReestablishmentComplete | - rrc-TransactionIdentifier - rlf-InfoAvailable-r9 - logMeasAvailable-r10 - connEstFailInfoAvailable-r11 - logMeasAvailableMBSFN-r12 | - rrc-TransactionIdentifier |
| RRCConnectionReestablishmentReject | - No IE | - No IE |

For RRCConnectionReestablishmentRequest, RRCConnectionReestablishment and RRCConnectionReestablishmentReject messages, all parameters for LTE are also needed in NR.

For RRCConnectionReestablishmentComplete message, the IEs rlf-InfoAvailable-r9, logMeasAvailable-r10, connEstFailInfoAvailable-r11 and logMeasAvailableMBSFN-r12 are related to MDT/SON which are not supported in NR phase I, so only IE rrc-TransactionIdentifier is needed in NR.

The RRC connection re-establishment procedure is used for SRB1 resumption and re-activation of security and the configuration of only the PCell, so the following parameters are proposed to support.

Proposal 2: Support the following parameters for the RRC messages in RRC connection re-establishment procedure, the details of each parameter can be discussed further:

- RRCConnectionReestablishmentRequest: UE ID, Re-establishment Cause

- RRCConnectionReestablishment: Transaction ID, dedicated radio resource configuration for SRB1, NCC

- RRCConnectionReestablishmentComplete: Transaction ID

## RRC connection resume

In LTE, The purpose of this procedure is to resume an RRC connection, which involves the resumption of all SRBs and DRBs, and the re-activation of security.

Figure 3 RRC connection resume, successful

Figure 4 RRC connection resume fallback to RRC connection establishment, successful

Figure 5 RRC connection resume, network reject or release

The function of resuming the RRC connection should also be supported in NR for UE in inactive mode, including the successful resume, fallback to RRC connection establishment, network reject and release procedures.

The content of the RRC messages for RRC connection resume are listed in the table 1.

Table 2 content of the RRC messages for RRC connection resume

| RRC message | Content in LTE | Proposed Content in NR |
| --- | --- | --- |
| RRCConnectionResumeRequest | - resumeIdentity-r13 - shortResumeMAC-I-r13 - resumeCause-r13 | - resumeIdentity-r13 - shortResumeMAC-I-r13 - resumeCause-r13 |
| RRCConnectionResume | - rrc-TransactionIdentifier - radioResourceConfigDedicated-r13(for all SRBs and DRBs) - nextHopChainingCount-r13 - measConfig-r13 - antennaInfoDedicatedPCell-r13 - drb-ContinueROHC-r13 | - rrc-TransactionIdentifier - radioResourceConfigDedicated-r13(for all SRBs and DRBs) - nextHopChainingCount-r13 - measConfig-r13 - antennaInfoDedicatedPCell-r13 - drb-ContinueROHC-r13 |
| RRCConnectionResumeComplete | - rrc-TransactionIdentifier - selectedPLMN-Identity-r13 - dedicatedInfoNAS-r13 - rlf-InfoAvailable-r13 - logMeasAvailable-r13 - connEstFailInfoAvailable-r13 - mobilityState-r13 - mobilityHistoryAvail-r13 - logMeasAvailableMBSFN-r13 | - rrc-TransactionIdentifier - selectedPLMN-Identity-r13 - dedicatedInfoNAS-r13 |
| RRCConnectionReject | - rrc-SuspendIndication-r13 | - rrc-SuspendIndication-r13 |

For RRCConnectionResumeRequest, RRCConnectionResume and RRCConnectionReject messages, all parameters for LTE are also needed in NR.

For RRCConnectionResumeComplete message, the IEs rlf-InfoAvailable-r13, logMeasAvailable-r13, connEstFailInfoAvailable-r13 and logMeasAvailableMBSFN-r13 are related to MDT/SON which are not supported in NR phase I. The IEs mobilityState-r13 and mobilityHistoryAvail-r13 are related to UE mobility history which is not supported in NR phase I. So IEs rrc-TransactionIdentifier, selectedPLMN-Identity-r13 and dedicatedInfoNAS-r13 are needed in NR.

Comparison can be made based on the analysis of section 2.1 and 2.2. The details are shown below:

- RRC re-establishment is used for connected mode UE, and RRC resume is used for inactive mode UE. In these two cases, the network both has UE’s AS context.

- They are both used to re-active the security.

- RRC re-establishment can resume SRB1, and RRC resume can resume all SRBs and DRBs.

- From the procedure point of view, the successful and reject procedures are similar. The RRC resume procedure has a fallback mechanism to RRC connection establishment, which is not supported by RRC re-establishment.

- From the content point of view:

  - both RRCConnectionReestablishmentRequest and RRCConnectionResumeRequest have UE ID and cause, although the details are not the same.

  - Both RRCConnectionReestablishment and RRCConnectionResume have Transaction ID, dedicated radio resource configuration and NCC, and the latter has more parameters for other purpose.

  - Both RRCConnectionReestablishmentComplete and RRCConnectionResumeComplete have Transaction ID, and the latter has more parameters for other purpose.

  - For reject case, only reject message for resume procedure has a Suspend Indication.

So there are many commonalities between RRC connection re-establishment procedure and RRC connection resume procedures. Thus it should be aim to combine the RRC messages used for RRC connection re-establishment procedure and RRC connection resume procedure.

Proposal 3: Aim to combine the following RRC messages for NR:

- RRCConnectionReestablishmentRequest and RRCConnectionResumeRequest

- RRCConnectionReestablishment and RRCConnectionResume

- RRCConnectionReestablishmentComplete and RRCConnectionResumeComplete

- RRCConnectionReestablishmentReject and RRCConnectionReject

# Conclusion

In this contribution, we discuss the individual procedures for RRC connection re-establishment and resume, and propose:

Proposal 1: Support the successful and failure procedures for RRC connection re-establishment in NR.

Proposal 2: Support the following parameters for the RRC messages in RRC connection re-establishment procedure, the details of each parameter can be discussed further:

- RRCConnectionReestablishmentRequest: UE ID, Re-establishment Cause

- RRCConnectionReestablishment: Transaction ID, dedicated radio resource configuration for SRB1, NCC

- RRCConnectionReestablishmentComplete: Transaction ID

Proposal 3: Aim to combine the following RRC messages for NR:

- RRCConnectionReestablishmentRequest and RRCConnectionResumeRequest

- RRCConnectionReestablishment and RRCConnectionResume

- RRCConnectionReestablishmentComplete and RRCConnectionResumeComplete

- RRCConnectionReestablishmentReject and RRCConnectionReject

# Reference

[1] RAN2#97bis, Chairman notes.