3GPP TSG-RAN WG2 Meeting #98		R2-1704576

Hangzhou, China, 15th - 19th May 2017

Source:	vivo

Title:	Enhanced BSR report and timer configuration in NR

Agenda Item:	10.3.1.5

Document for:	Discussion and Decision

- Introduction

At the RAN 97 bis meeting, BSR aspects in NR was discussed and made the following agreements [1]

-The existing LTE BSR framework is used as baseline for NR BSR framework. Further enhancements at least related to numerologies and granularity and can be further discussed

In this contribution, we further discuss the detailed enhancements for the NR BSR report and timer.

- Discussion

  - The granularity of BSR report enhancement

It is expected that NR has to support more variety of services than LTE. In LTE, there are two types of BSR formats that can be reported to eNB. In LTE, there are 4 Logical Channel Groups (LCGs) for long BSR. In NR, more number of LCGs can be defined to provide the finer granularity of the data priorities or per logical channel BSR can be defined to provide the finer granularity.

Proposal 1: In NR, to provide the finer granularity of the data priorities: (a) more number of BSR LCGs or (b) per logical channel BSR can be defined.

  - The granularity of BSR timer enhancement

In LTE, RRC controls the BSR reporting by configuring the two timers periodicBSR-Timer, retxBSR-Timer . Both of the timers are configured per UE, which cannot optimally support the diversity of services in NR. If the granularity of BSR timers can be configured per logical channel or LCG, the BSR trigger and reporting can also be performed per logical channel or LCG. It means that the resource allocation and updating of buffer states will get a better trade-off.

Proposal 2: In NR, BSR timers per LCGs or per logical channel can be defined to provide a finer granularity for the resource allocation and updating of buffer states.

- Conclusion

In this contribution, we discuss the detailed enhancement for the NR BSR e.g., the granularity of BSR report and the BSR trigger. And provide below proposals.

Proposal 1: In NR, to provide the finer granularity of the data priorities: (a) more number of BSR LCGs or (b) per logical channel BSR can be defined.

Proposal 2: In NR, BSR timers per LCGs or per logical channel can be defined to provide a finer granularity for the resource allocation and updating of buffer states.

- References

- R2-17xxxx,  UP Chairman's minutes RAN2_#97bis