3GPP TSG-RAN WG2 #AH	Tdoc R2-1707174

Qingdao, P.R. of China, 27-29 June 2017

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	Grant Free and Semi-Persistent Scheduling in NR

Document for:	Discussion, Decision

# Introduction

In RAN2#97, the following was agreed for semi-persistent scheduling (SPS):

- NR supports an SPS scheme similar to LTE - NR supports skipping UL grant scheme similar to LTE

In RAN2#97bis, the following agreements were reached:

Agreements on grant-free

=>	From RAN2 point of view it would be beneficial to be able to share “SPS/grant free” UL resources amongst different UE.  Mechanism to identify the UE for collision resolution purpose may be needed.   The details can be discussed in RAN1.

Agreements on SPS:

-	Like in legacy LTE, at least SPS period is configured by RRC.  FFS how frequency resources, MCS, etc., for SPS are provided to the UE depends on RAN1 discussion.

-	UL skipping for dynamic grant should be configurable.  FFS if UL skipping for SPS is configurable

-	Working assumption:  Like in LTE, DRX behaviour with SPS UL should be to restart inactivity timer when UL data is transmitted, and not to restart when SPS UL grant is not used.  This behaviour depends on outcome of DRX design.

In RAN2#98, the following agreements were reached:

Agreements

1.	In NR, when the UE is configured with SPS, the UE should always skip SPS grant if there is no data to transmit, i.e., Skipping SPS grant is mandated in NR regardless of SPS periodicity.

2.	LCP is performed the same regardless whether the grant is dynamic or SPS.  SPS is a “configured grant”.

3.	FFS is multiple SPS is supported for duplication or to support different numerologies

4.	Implicit release of UL SPS resources is not supported

Additionally, In RAN2#98, RAN2 received an LS from RAN1 informing on agreements w.r.t Grant Free [1]:

Agreements:

- If network configures, UL data transmission without UL grant can be performed after semi-static resource configuration in RRC without L1 signalling

- If network configures, L1 signaling for activation/deactivation and/or modification on parameters for UL data transmission without UL grant can be applied

- RAN1 is discussing whether the mechanism to distinguish UL SPS and UL data transmission without UL grant is necessary.

# Discussion

SPS in LTE and NR is mainly used to configure the UE semi-statically with a periodic resource allocation which ideally matches a periodic traffic pattern, where periods of approximately 20ms are typical, or for URLLC type of services where the resource request procedure(s) incur an unwanted latency in receiving an UL grant.

In LTE, and in NR, “UL skipping” is used as for uplink scheduling as well as SPS-based scheduling. Thereby, the mandate for the UE to always utilize an UL grant – even if no UL data is available by a padding transmission – is removed. This way, the UE skips the UL padding transmission. The motivation for this feature on SPS was that SPS intervals were reduced to very short intervals, and that too frequent redundant padding transmissions are thereby avoided.

It becomes evident that in NR, the SPS framework will be used for these two distinct use cases: low-latency uplink access on the one side with a minimum periodicity, and VoIP-like periodic traffic with relaxed periodicities of about 20ms.

As in LTE, some of the parameters that need to be configured for semi-persistent scheduling in LTE are periodicity, address/ID (e.g., SPS-C-RNTI), resource allocation and MCS to be used in the UL grant. For SPS, part of these parameters (periodicity, address) are configured by RRC (SPS Configuration), and the remaining configured by PDCCH (SPS Activation/Deactivation). This allows for flexibility of the resource allocation and link adaptation, since a new configuration message can be sent in any PDCCH. It can be assumed that in general a similar configuration is used for NR SPS.

Like in legacy LTE, SPS period and RNTI are configured by RRC.

Like in legacy LTE, frequency resources, MCS, etc., for SPS are semi-statically configured by L1 (PDCCH).

The RAN1 agreements [1] for “UL data transmission without UL grant” enatil that additionally, a configured grant, optionally “without L1 signalling”, can be given to the UE; for where a “semi-static resource configuration” can be given. This is suggested to be set by RRC signalling.

In addition to the above, is also agreed that “If network configures, L1 signaling for activation/deactivation and/or modification on parameters for UL data transmission without UL grant can be applied”.

From a layer 2 p.o.v the above fit directly with the concept of “configured grants” in MAC, and especially, as an extension of legacy SPS with the additional add-on of providing also the L1 related information on MCS, resource allocation etc, in the SPS RRC configuration.

“UL data transmission without a grant” can be defined as a natural extension of the current SPS framework.

In SPS, “UL skipping” comes with the need for a procedure to confirm the activation/deactivation. As concluded for LTE, the overhead of this is minimal, as it is only sent once.

Also for NR SPS including the extension of a “Grant Free” mode, the optional activation/deactivation and/or modification and to avoid error cases, a confirmation message acknowledging the UL resource L1 activation/deactivation or modification signalling should be sent to the gNB, following legacy LTE and agreed NR baseline. Note that, for a RRC configuration activated without L1 control, the need for a confirmation message on the SPS resource should be discussed. This could be made with a MAC CE similar to LTE or rely on the completion of the RRC procedure or a combination thereof.

“UL data transmission without UL grant” is modelled as a part of configurable options of the NR SPS framework.

Like in legacy LTE and NR baseline SPS, when NR UP SPS resources are activated (granted), deactivated or modified by L1 signalling, UE should send a confirmation to the gNB

With mandated “Skipping SPS grant” when buffer is empty, it may impact the behaviour of periodic messages. In legacy LTE-SPS with “UL skipping” configured, periodic messages such as CSI and PHR are transmitted when no data is available. These periodic transmissions can be employed for UL link adaptation. In legacy LTE-SPS with “UL skipping” configured and if a periodic BSR is triggered and the buffer is empty, then the usefulness of reporting an empty buffer is not clear. It can be assumed that in general a similar configuration is used for NR and exceptions like this should be discussed further.

With Skipping SPS grant mandated in NR, periodic transmissions of PHR, CSI and periodic BSR should follow similar operations in LTE with “UL skipping” configured.

# Conclusion

Based on the discussion in section 2 we make the following observations and propose the following:

- Like in legacy LTE, SPS period and RNTI are configured by RRC.

Like in legacy LTE, frequency resources, MCS, etc., for SPS are semi-statically configured by L1 (PDCCH).

With Skipping SPS grant mandated in NR, periodic transmissions of PHR, CSI and periodic BSR should follow similar operations in LTE with “UL skipping” configured.

Proposal 1	“UL data transmission without a grant” can be defined as a natural extension of the current SPS framework.

Proposal 2	“UL data transmission without UL grant” is modelled as a part of configurable options of the NR SPS framework.

Proposal 3	Like in legacy LTE and NR baseline SPS, when NR UP SPS resources are activated (granted), deactivated or modified by L1 signalling, UE should send a confirmation to the gNB

# References

R2-1706103	 LS on UL data transmission without UL grant for NR, RAN1.