3GPP TSG-RAN WG2 NR AH#3	Tdoc R2-1800341

Vancouver, Canada, 22nd – 26th January 2018

Agenda Item:	10.3.1.4.3

Source:	Ericsson

Title:	Preamble transmission power

Document for:	Discussion, Decision

# Introduction

In the Random Access procedure for NR in 38.321, the MAC layer determines the Preamble transmission power according to

- set PREAMBLE_RECEIVED_TARGET_POWER to ra-PreambleInitialReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) * powerRampingStep;

This contribution discusses the DELTA_PREAMBLE parameter.

# Discussion

In 38.331, values for ra-PreambleInitialReceivedTargetPower and powerRampingStep are defined. There is no need to signal DELTA_PREAMBLE in RRC as that value is deduced from the preamble format configured. In LTE, the DELTA_PREAMBLE is included in a table in 36.321. However, for NR, no values have yet been defined.

We therefore propose

Send an LS to RAN1 requesting DELTA_PREAMBLE values for the PRACH preamble formats specified for NR.

The corresponding LS can be found in [1].

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Send an LS to RAN1 requesting DELTA_PREAMBLE values for the PRACH preamble formats specified for NR.

# References

R2-1800342, “Draft LS on preamble transmission power”, Ericsson, RAN2 NR AH#3, Vancouver, Canada, 22nd – 26th January 2018