3GPP TSG-RAN WG2 #AH NR 1801	Tdoc R2-1800708

Vancouver, Canada, Jan 22nd – 26th, 2018

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	Confirmation of Configured UL Grant Type 2 Activation for Multiple Aggregated Cells

Document for:	Discussion, Decision

# Introduction

For Configured Scheduling, upon reception of a PDCCH to activate or deactivate a configured grant Type 2 by a UE, the UE shall confirm to the serving gNB using a confirmation MAC CE using a predefined format. Upon receiving the confirmation MAC CE, the gNB knows that the L1 signal on PDCCH is received by the UE and at what point in time the configured grant is activated or deactivated.

The support for several configured grants (one per serving cell) introduces an additional problem. If the gNB activates several configured grants but does not receive as many confirmation MAC CEs as activation commands, the gNB does not know which configured grants were activated. This problem is not solved in the specifications.

This paper discusses this problem and presents two solutions.

# Discussion

## Introduction

In LTE, there can be one UL SPS configuration per MAC entity. This means that the serving gNB only activates or deactivates a single UL SPS grant at a time. When a UE receives PDCCH to activate or deactivate a UL SPS grant, the UE responds with a confirmation MAC CE. Figure 1 shows the signalling transmission for UL SPS grant activation/deactivation in LTE. If the eNB does not receive the confirmation MAC CE, it determines that the activation or deactivation command was lost and retransmits it. As there can be only one UL SPS configuration, there is no ambiguity.

Figure 1 Confirmation MAC CE for UL SPS grant activation/deactivation in LTE

In LTE, there is no ambiguity as to whether the configured grant was activated/deactivated or not.

A major addition in NR is the support for one configured grant per serving cell. As a result, it is possible that a UE needs to confirm more than one activation or deactivation operation of a configured grant Type 2 in case of multiple aggregated cells using the confirmation MAC CE according to the following procedures.

In section 6.1.3.7 of 3GPP TS 38.321 v15.0, the confirmation MAC CE is defined to be with only subheader:

“The Configured Grant Confirmation MAC CE is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-2.

It has a fixed size of zero bits.”

In Section 5.4.1 of 3GPP 38.321 v15.0, it is specified that a UE should generate confirmation MAC CE upon the configured grant Type 2 activation:

“1>	if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity’s C-RNTI or Temporary C-RNTI; or

1>	if an uplink grant has been received in a Random Access Response:

…

1>	else if an uplink grant for this PDCCH occasion has been received for this serving cell on the PDCCH for the MAC entity’s CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

…

2>	else if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate configured grant Type 2 deactivation:

4>	trigger configured grant confirmation.

3>	else if PDCCH contents indicate configured grant Type 2 activation:

4>	trigger configured grant confirmation;

…”

Furthermore, in Section 5.8.2 of 3GPP TS 38.321 v15.0, it is specified that a UE should send a confirmation MAC CE upon deactivation of a configured grant Type 2:

“For a configured grant Type 2, the MAC entity shall clear the configured uplink grant immediately after first transmission of Configured Grant Confirmation MAC CE triggered by the configured grant deactivation.”

## Problem with existing specifications

From the available description in the specifications, the condition when the confirmation MAC CE is triggered is well defined, but on what resource to transmit the confirmation MAC CE is not explicitly described. For instance, if there are multiple carriers activated or deactivated, in which carrier or using which type of uplink grant for the generated confirmation MAC CEs is not specified. Although one could say, with the current specification the UE shall follow multiplexing procedures and include MAC CEs in the earliest grant, dynamic or configured, on any carrier that can accommodate the MAC CEs.

The confirmation MAC CE will be transmitted on any UL resource on any UL serving cell.

This creates an ambiguity as there is no way for the gBN to determine which received confirmation MAC CE is associated with which PDCCH activation/deactivation command. To illustrate the ambiguity, Figure 2 shows one example where a UE has 4 active serving cells which belong one MAC entity. There are 3 PDCCH commands, which are transmitted to activate or deactivate the configured grant Type 2 in 3 serving cells (i.e. aggregated component carrier (CC)):

The actions are as follows:

- The first PDCCH activates the configured UL grant Type 2 in CC 0;

- The second PDCCH deactivates the configured UL grant Type 2 in CC 1;

- The third PDCCH activates the configured UL grant Type 2 in CC 2;

Figure 2 configured UL grant Type 2 activation/deactivation in multiple serving cells

After receiving the 3 PDCCHs (to operate the 3 configured UL grants of Type 2 in the 3 CCs by the UE respectively), a PUSCH transmission is scheduled, in which the corresponding confirmation MAC CE(s) is carried. If all PDCCH commands are received, then three MAC CEs will be transmitted and the gNB can determine that two configured grants are active and one is deactivated.

However, if any out of 3 PDCCH commands are missed by the UE, i.e. the UE only sends two confirmation MAC CEs to the serving gNB using the PUSCH, then upon receiving the confirmation MAC CEs, the serving gNB does not know which PDCCH command was missed by the UE. Figure 4 exemplifies this problem.

Figure 4 Example of ambiguity according to Option 2

There can be ambiguity in the activation or deactivation confirmation of the configured grant Type 2 for multiple aggregated cells.

## Alternatives resolving the ambiguity.

In this section we present three options.

### Alternative 0

Without incurring changes to the standard:

-	The gNB may choose to retransmit all the PDCCH commands. This increases the load on PDCCH which negates the purpose of using configured grants.

-	The gNB can refrain from transmitting another PDCCH command until the initial command has been confirmed. This introduces additional delay should the gNB want to activate/deactivate several configured grants. E.g. timely activation may be important for URLLC with duplication. It also incurs an increased power consumption in the UE.

The cost of these solutions is high, so solutions which incur changes to the standard should be explored.

### Alternative 1

Perform a “self-cell contained” confirmation MAC CE transmission without MAC CE format change. This alternative requires the UE to send the confirmation MAC CE on the same carrier in which the corresponding configured grant Type 2 is activated/deactivated. This alternative could even be seen as the assumption when designing the CS functionality in RAN2 although not entirely captured as such. In here, upon reception of the confirmation MAC CE from PUSCH in a carrier, the gNB can determine PDCCH to have been successfully received by the UE. Figure 5 shows one example.

Figure 5 self-cell contained confirmation MAC CE transmission according to Alt.1

### Alternative 2

The second alternative is to define a new confirmation MAC CE format to indicate the confirmation correspondence to carrier or CS configuration, for instance, either cell index or CS configuration index can be included. Figure 6 shows one such example.

Figure 6 example of new confirmation MAC CE with fixed payload size

### Evaluation

Alternative 1 requires less specification changes than the second alternative. The second alternative is expected to simplify on what resource/grant in what carrier the confirmation is sent.

Either alternative 1 or 2 of the following is used to confirm configured grant Type 2 activation/deactivation for multiple aggregated cells:

- 1: self-cell contained confirmation MAC CE transmission without MAC CE format change;

- 2: new confirmation MAC CE format to indicate the correspondence to carrier.

For Alt.1, the relevant part in Section 5.8.2 of 38.321-15.0.0 can be updated as the following. The additions are marked with green color.

…

The MAC entity shall:

1>	if the configured grant confirmation for a Serving Cell has been triggered and not cancelled; and

1>	if the MAC entity has UL resources allocated for new transmission on that Serving Cell:

2>	instruct the Multiplexing and Assembly procedure to generate an Configured Grant Confirmation MAC CE as defined in subclause 6.1.3.7 to be transmitted on that Serving Cell;

2>	cancel the triggered configured grant confirmation.

For a configured grant Type 2, the MAC entity shall clear the configured uplink grant immediately after first transmission of Configured Grant Confirmation MAC CE triggered by the configured grant deactivation.

…

# Conclusion

In section 2 we made the following observations:

Observation 1	In LTE, there is no ambiguity as to whether the configured grant was activated/deactivated or not.

Observation 2	The confirmation MAC CE will be transmitted on any UL resource on any UL serving cell.

Observation 3	There can be ambiguity in the activation or deactivation confirmation of the configured grant Type 2 for multiple aggregated cells.

Based on the discussion in section 2 we propose the following:

- Either alternative 1 or 2 of the following is used to confirm configured grant Type 2 activation/deactivation for multiple aggregated cells:

- 1: self-cell contained confirmation MAC CE transmission without MAC CE format change;

- 2: new confirmation MAC CE format to indicate the correspondence to carrier.

# References

Tdoc Number, Title, Source, Meeting, Date

Spec number, Title, Source, Version, Date