3GPP TSG-WG2 Meeting #AH-1807 	R2-1809658

# Montreal, Canada, 2 – 6 July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | draft | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR for Cell reselection during on-demand system information acquisition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is possible that the UE reselects to a different cell after initiating on demand SI acquisition in a given cell. It should be noted that once SI request is initiated, MAC will initiate (potentially multiple) RACH (re)transmissions (depending on RACH being successfully received or not) and may also initiate Msg3 transmission (in case of Msg3 based SI request). If cell reselection happens any time after the SI request is initiated by RRC, then this SI request procedure should be cancelled and the UE shall adopt the procedures necessary in the new cell (for instance reinitiate SI acquisition in the cell to which it has reselected to – if needed). Specifically, the MAC entity in the UE shall be stopped from transmitting any further messages either in the old cell or in the new cell subsequent to reselection as this will either result in unnecessary interference in the old cell or transmission of irrelevant messages in the new cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Upon cell reselection during ongoing on demand SI procedure (i.e. after initiation of the SI request and before receiving the requested SI from the network), the UE (RRC) shall reset MAC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary interference in the old cell or transmission of irrelevant messages in the new cell will result if MAC continues to (re)transmit the Originally initiated SI request messages |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.2.3.3	Request for on demand system information

The UE shall:

1>	if SIB1 includes PRACH preamble and/or PRACH resource for the SI message(s) that the UE is required to acquire:

2>		trigger the lower layer to initiate the Random Access procedure in accordance with  [3] using the PRACH preamble and PRACH resource corresponding to the SI message(s) that the UE is required to acquire;

2>	if acknowledgement for SI request is received from lower layers;

3>	acquire the requested SI message(s) as defined in sub-clause 5.2.2.3.2;

2> else if cell reselection occurs

3> reset MAC

Editor’s Note: To be updated with details of the Msg1 request procedure.FFS_Standalone.

1>	else

2>	initiate transmission of the RRCSystemInfoRequest message in accordance with 5.2.2.3.X;

2>	if acknowledgement for RRCSystemInfoRequest message is received from lower layers;

3>	acquire the requested SI message(s) as defined in sub-clause 5.2.2.3.2;

2> else if cell reselection occurs

3> reset MAC