3GPP TSG-RAN WG2 Meeting NR Adhoc 1807	R2-1809911

Montreal, Canada, 2nd - 6th July 2018

Agenda item:	10.4.1.6.8

Source:	Nokia, Nokia Shanghai Bell

Title:	Required SIBs

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In this paper we discuss how to capture required SIBs in 38.331

# 2	Essential SIB1 missing

Currently in 38.331 it is written:

#### 5.2.2.1	General UE requirements

Figure 5.2.2.1-1: System information acquisition

The UE applies the SI acquisition procedure to acquire the AS- and NAS information. The procedure applies to UEs in RRC_IDLE, in RRC_INACTIVE and in RRC_CONNECTED.

The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 as well as SIB X through SIB Y (depending on support of the concerned RATs for UE controlled mobility).

The UE in RRC_CONNECTED shall ensure having a valid version of (at least) the MIB, SIB1 as well as SIB X (depending on support of mobility towards the concerned RATs).

Editor’s Note: [FFS if different versions of SIBs are provided].

We are in position now in RAN2 that we could write in more explicit manner which SIBs are required and which not in IDLE, INACTIVE and CONNECTED states.

Genarlly we consider that in IDLE/INACTIVE state UE needs to ensure having valid reselection SIBs but this is not required in CONNECTED state. Additionally we think UE should keep any PWS information up to date.

Proposal: In IDLE/INACTIVE state UE needs to keep reselection SIBs up to date but not in CONNECTED state. Additionally UE needs to ensure having valid PWS information if it supports ETWS/CMAS.

# 5	Conclusion

In this paper we proposed how to capture details general UE requirements which SIBs should be kept up to date. Genarlly saying in IDLE/INACTIVE reselection SIBs are needed but not in CONNECTED state. Additionally UE needs to ensure having valid PWS information if it supports ETWS/CMAS.

Proposal: In IDLE/INACTIVE state UE needs to keep reselection SIBs up to date but not in CONNECTED state. Additionally UE needs to ensure having valid PWS information if it supports ETWS/CMAS.

Proposal: Agree on the changes as provided in the Annex Text Proposal

# Annex TP

#### 5.2.2.1	General UE requirements

Figure 5.2.2.1-1: System information acquisition

The UE applies the SI acquisition procedure to acquire the AS- and NAS information. The procedure applies to UEs in RRC_IDLE, in RRC_INACTIVE and in RRC_CONNECTED.

The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 through SIB4 and SIB5 (if UE supports UE controlled mobility towards E-UTRA).

The UE in RRC_CONNECTED shall ensure having a valid version of (at least) the MIB and SIB1.

ETWS and/or CMAS capable UE shall ensure having a valid PWS information.

Editor’s Note: [FFS if different versions of SIBs are provided].

##### --------------------------------------------NEXT CHANGES--------------------------------------------