3GPP TSG-RAN WG2 Meeting NR Adhoc 1807		DRAFT R2-1810791

Montreal, Canada, 2nd - 6th July 2018

Title:	[DRAFT] Reply LS on MDBV for delay critical QoS flows

Response to:	R2-1806659/S2-184613

Release:	Release 15

Work Item:	NR_newRAT

Source:	Nokia [TSG RAN WG2]

To:	TSG SA WG2

Cc:

Contact Person:

#### Name:	Benoist Sébire

E-mail Address:	benoist.sebire@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank SA2 for their LS on MDBV for delay critical QoS flows. SA2 asked the following:

SA2 respectfully asks RAN2 to confirm that the PDB and PER requirements are satisfied in the UL and DL for all delay critical QoS flows that do not exceed the MDBV, even if other delay critical QoS Flows  (which may also be of higher priority) for the same UE or other UE exceeds the MDBV.

RAN2 has discussed the issue and would like to inform SA2 of the following:

- In the downlink, the gNB can enforce individual MDBV(s) through scheduling, regardless of how QoS flows are multiplexed together on the air interface.

- In the uplink, RAN2 did not achieve consensus on whether changes are needed in order to always enforce individual MDBV(s). When logical channels/radio bearers share the same resource and compete for the same grant, there was no consensus whether in some scenarios delay critical QoS flows may exceed the MDBV. RAN2 has agreed not to introduce any new mechanisms for this feature in Rel-15.

2. Actions:

To SA2 group.

ACTION: 	RAN2 respectfully asks SA2 to take the above into account.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP TSG RAN WG2 #103	20 – 24 Aug 2018		Gothenburg

3GPP TSG RAN WG2#103bis	8 - 12 Oct 2018   	 	Chengdu

3GPPRAN2#104 		12 - 16 Nov 2018    		US