3GPP TSG-RAN WG3 #101	Tdoc R3-185041

Gothenburg, Sweden, 20th– 24th August 2018

Agenda Item:	31.3.4.14

Source:	Ericsson

Title:	RLC Mode Indication over E1

Document for:	TP for BL CR for TS 38.463

# Introduction

This CR for follows the discussion in R3-185039. To align E1AP and RRC, we propose to add two code points to the RLC Mode IE for unidirectional bearers.

NOTE: since E1AP ASN.1 is no yet frozen, there are on problems with non-backward compatibility.

# TP for BL CR for TS 38.463

<<<<<< NEXT CHANGE >>>>>>

#### 9.3.1.38	PDCP Configuration

This IE carries the PDCP configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDCP SN Size | M |  | ENUMERATED (12, 18, …) | Indicates the PDCP SN size in bits. For more information see PDCP-Config IE in TS 38.331. |
| RLC mode | M |  | ENUMERATED (RLC-TM, RLC-AM, RLC-UM-Bidirectional, RLC-UM-Unidirectional-UL, RLC-UM-Unidirectional-DL, …) | Indicates the RLC mode for the DRB. For more information see PDCP-Config IE in TS 38.331. |
| ROHC Parameters | O |  | 9.3.1.40 |  |
| T-Reordering Timer | O |  | 9.3.1.41 |  |
| Discard Timer | O |  | 9.3.1.42 |  |
| UL Data Split Threshold | O |  | 9.3.1.43 |  |
| PDCP Duplication | O |  | ENUMERATED (True, …) | Indicates whether PDCP duplication is to be configured for the DRB. |

<<<<<< NEXT CHANGE >>>>>>

#### -- R

RAT-Type	::=	ENUMERATED	{

e-UTRA,

nR,

...

}

RLC-Mode	::=	ENUMERATED	{

,

rlc-tm,

rlc-am,

rlc-um-bidirectional,

rlc-um-unidirectional-ul,

rlc-um-unidirectional-dl,

...

}

ROHC-Parameters	::= SEQUENCE {

rOHC-Profiles		ROHC-Profiles,

iE-Extensions		ProtocolExtensionContainer { { ROHC-Parameters-ItemExtIEs} } OPTIONAL,

...

}

ROHC-Parameters-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

ROHC-Profiles	::=	INTEGER	(0..511, ...)