3GPP TSG-RAN WG3 Meeting #101	R3-185005

# Gothenburg, Sweden, 20th- 24th August  2018

Agenda Item:	31.3.4.3

Source:	Ericsson

Title: 	On signalling of dynamic 5QI values, E1AP

Document for:	Discussion and Approval

# 1	Information

Changes included in this text proposal:

- Update the Dynamic 5QI Descriptor and ASN.1. Also add comment to indicate the condition.

- Update the Non Dynamic 5QI Descriptor.

- Introduce in ASN.1 an new IE FiveQI

- Modify Non-Dynamic5QIDescriptor to refer to FiveQI.

# 2	Text Proposal on BL R3-184394 on TS 38.463

-------------------------------

Start the first change!

-------------------------------

#### 9.3.1.27	Non Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a standardized or pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5QI | M |  | INTEGER (0..255) | This IE contains the standardized or pre-configured 5QI value Details in TS 23.501 [11]. |
| Priority Level | O |  | 9.3.1.51 | For details see TS 23.501 [11]. When included overrides standardized or pre-configured value. |
| Averaging Window | O |  | 9.3.1.49 | This IE applies to GBR QoS Flows only. For details see TS 23.501 [11]. When included overrides standardized or pre-configured value. |
| Maximum Data Burst Volume | O |  | 9.3.1.50 | For details see TS 23.501 [11]. When included overrides standardized or pre-configured value. |

#### 9.3.1.28	Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a Non-standardised or not pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5QI | O |  | INTEGER (0.255) | This IE contains the dynamic 5QI value. Details in TS 23.501 [11} |
| Priority Level | M |  | 9.3.1.51 | For details see TS 23.501 [11]. |
| Packet Delay Budget | M |  | 9.3.1.47 | For details see TS 23.501 [11]. |
| Packet Error Rate | M |  | 9.3.1.48 | For details see TS 23.501 [11]. |
| Delay Critical | C-ifGBRflow |  | ENUMERATED (delay critical, non-delay critical) | For details see TS 23.501 [11]. |
| Averaging Window | C-ifGBRflow |  | 9.3.1.49 | For details see TS 23.501 [11]. |
| Maximum Data Burst Volume | O |  | 9.3.1.50 | For details see TS 23.501 [11]. |

| Condition | Explanation |
| --- | --- |
| ifGBRflow | This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE. |

------------------------

Skip to next change

------------------------

### 9.3.5	Information Element definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AreaOfInterest-Item,

id-dataforwardingInfoperPDUSession-Item,

id-DataForwardingResponseDRBItem,

id-DRBsSubjectToStatusTransfer-Item,

id-DRBToQoSFlowMapping-Item,

id-PDUSessionResourceAdmittedResponseTransferItem,

id-PDUSessionResourcesAdmitted-Item,

id-PDUSessionResourcesNotAdmitted-Item,

id-PDUSessionResourcesToBeSetup-Item,

------------------------

Skip to next change

------------------------

#### -- D

Data-Forwarding-Information-Request	::=	SEQUENCE {

data-Forwarding-Request				Data-Forwarding-Request	OPTIONAL,

uL-Data-Forwarding						UP-TNL-Information			OPTIONAL,

dL-Data-Forwarding						UP-TNL-Information			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-Request-ExtIEs } }	OPTIONAL,

...

}

------------------------

Skip to next change

------------------------

Dynamic5QIDescriptor	::= SEQUENCE {

fiveQI						FiveQI													OPTIONAL,

qoSPriorityLevel					QoSPriorityLevel,

packetDelayBudget					PacketDelayBudget,

packetErrorRate						PacketErrorRate,

delayCritical						ENUMERATED {delay-critical, non-delay-critical}		OPTIONAL,

-- This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

averagingWindow 					AveragingWindow										OPTIONAL,

-- This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

maxDataBurstVolume					MaxDataBurstVolume									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Dynamic5QIDescriptor-ExtIEs } } OPTIONAL

}

Dynamic5QIDescriptor-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

------------------------

Skip to next change

------------------------

#### -- F

FiveQI ::= INTEGER (0..255)

#### -- G

------------------------

Skip to next change

------------------------

#### -- N

New-UL-TNL-Information-Required::= 	ENUMERATED {

required,

...

}

NGRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {NGRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL

}

NGRANAllocationAndRetentionPriority-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

NG-RAN-QoS-Support-List ::= SEQUENCE (SIZE(1.. maxnoofNGRANQOSParameters)) OF NG-RAN-QoS-Support-Item

NG-RAN-QoS-Support-Item ::= SEQUENCE {

non-Dynamic5QIDescriptor	Non-Dynamic5QIDescriptor,

iE-Extensions				ProtocolExtensionContainer { { NG-RAN-QoS-Support-Item-ExtIEs } }	OPTIONAL

}

NG-RAN-QoS-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

Non-Dynamic5QIDescriptor	::= SEQUENCE {

fiveQI						FiveQI,

qoSPriorityLevel			QoSPriorityLevel			OPTIONAL,

averagingWindow 			AveragingWindow					OPTIONAL,

maxDataBurstVolume			MaxDataBurstVolume				OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { Non-Dynamic5QIDescriptor-ExtIEs } } OPTIONAL

}

------------------------

The End!

------------------------