3GPP TSG-RAN WG3 Meeting #103	R3-190133

Athens, Greece, Feb. 25th – Mar.1st, 2019

Agenda item:		23.2.

Source:	LG Electronics Inc.

Title:	Text Proposal on Service Authorization

Document for:		pCR

1. Introduction

In last meeting, a summary on the potential issues of NR V2X to be investigated in RAN3 was listed in [2]. Based on that, this paper targets to investigate further the open issue that RAN3 can make decisions on Service authorization. The corresponding text proposal for TR 38.885 is also provided.

2. Discussion

In last meeting, the following agreements were achieved on service authorization:

- V2X-capable UE shall transmit in the sidelink only if authorized by the serving gNB; this authorization is signaled over NGAP and XnAP

- NR V2X UE authorization should allow at least “vehicle” and “pedestrian” services authorization, not precluding a UE being authorized for both at the same time

There is one open issue left on whether the authorization is per PC5 RAT, i.e. per gNB/eNB, or not.

However, NG-RAN can be either gNB or ng-eNB, that is, from radio point of view, it is either NR or LTE. A UE may support NR or LTE or both. Sidelink radio resource management mechanisms are different for NR and LTE. It seems necessary to separate the information on V2X Services Authorized Indication from gNB and ng-eNB in order to adopt different resource modes for NR and LTE in radio part. Thus, it requests AMF to provide V2X Services Authorized Information to gNB and ng-eNB separately.

In addition, SA2 has discussed Service Authorization to NG-RAN for eV2X communications over PC5 reference point and the related solution captured in clause 6.6 “Solution #6: eV2X impacts to 5GC procedures” of TR 23.786 [3], which is also given as follows:

6.6.2.6 V2X capability indication and V2X related information per PC5 RAT A UE may support multiple radio access technologies (RATs) over PC5 reference point, including LTE and NR. For such UE, the V2X capability indication and V2X related information can be per PC5 RAT as below: - The V2X capability indication sent by the UE described in clause 6.6.2.1 is per PC5 RAT. The V2X capability per PC5 RAT indicates whether the UE is capable of supporting V2X communication over PC5 reference point, i.e. over LTE PC5, over NR PC5 or over both. - The "V2X services authorized" indication sent to NG-RAN described in clause 6.6.2.1 to clause 6.6.2.5 is per PC5 RAT. - The UE-PC5-AMBR sent to NG-RAN described in clause 6.6.2.1 to clause 6.6.2.5 is per PC5 RAT. - The cross-RAT PC5 control authorization sent to the NG-RAN described in clause 6.6.2.1 to clause 6.6.2.5 is per PC5 RAT.

Firstly, it can be seen that SA2 agreed that "V2X services authorized" indication sent to NG-RAN is per PC5 RAT.

Secondly, in the table above, SA2 also agreed an additional information on cross-RAT PC5 control authorization, this is based on the scenarios in TR 38.885, which are also given as follows here:

Figure 1: Scenario 1                                   Figure 2: Scenario 2

In scenario 1, a gNB provides control/configuration for a UE's V2X communication in both LTE SL and NR SL;

In scenario 2, an ng-eNB provides control/configuration for a UE's V2X communication in both LTE SL and NR SL;

It can be seen that in order to support the scenarios above the cross-RAT PC5 control authorization is needed for gNB/ng-eNB to decide on whether the resource allocation for the V-UE/P-UE on the other RAT is allowed or not. Of course, on how to realize the authorization in stage 3, i.e., whether implicit or explicit indication from AMF, can be decided in WI phase.

Based on the analysis above, the following proposals are suggested to RAN3:

Proposal 1): The “V2X Services Authorized” Information sent to NG-RAN is per PC5 RAT.

Proposal 2): The cross-RAT PC5 control authorization sent to the NG-RAN is needed and is per PC5 RAT.

Proposal 3): To adopt the text proposal to TR 38.885.

3. Conclusion

In this contribution, the issues on V2X service authorization were further investigated. The following proposals are suggested to RAN3:

Proposal 1): The “V2X Services Authorized” Information sent to NG-RAN is per PC5 RAT.

Proposal 2): The cross-RAT PC5 control authorization sent to the NG-RAN is needed and is per PC5 RAT.

Proposal 3): To adopt the text proposal to TR 38.885.

4. References

- RP-182111, “Study on NR V2X”, LG Electronics

- R3-187223, “Summary of offline discussion: WF for “Study on NR V2X” in RAN3”, LG Electronics

- 3GPP TR 23.786 V1.1.0: Study on architecture enhancements for EPS and 5G System to support advanced V2X services

Text Proposal for TR 38.885

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

10	Network aspects

10.1	V2X service authorization

Following similar principles as in LTE, the V2X service authorization information will be conveyed by 5GC to the NG-RAN node over NG interface and provided by Xn between NG-RAN nodes during mobility events. The V2X service authorization information is PC5 RAT specific. The cross-RAT PC5 control authorization information is also necessary for supporting scenario 1 and 2 in section 4.1.

10.2	UE SL aggregate maximum bit rate

The assignment of UE SL AMBR follows similar principles as in LTE and will be conveyed by 5GC to the NG-RAN node over the NG interface and provided between NG-RAN nodes by the Xn interface during mobility events. There are currently proposals to use either RAT specific or common UE SL AMBR.

10.3	Impacts on the F1 interface

The study of impacts on the F1 interface includes whether the resource pool for side link communication is configured in the gNB-CU or gNB-DU.

10.4	Slicing aspects

NR V2X shall support the use of slicing similarly to NG-RAN, i.e. the same general principles as captured in [7] shall apply.