3GPP TSG-RAN WG3 #105								R3-193588

Ljubljana, Slovenia, 26-30 August 2019

Agenda Item:	31.3.1

Source:	China Telecom

Title: 	Support of CSI-RS Configuration Transfer for SA and EN-DC

Document for:	Discussion

Introduction

The CSI-RS based inter-node mobility has not been fully supported in current specifications. In Rel-15, RAN2 had specified the CSI-RS measurement configuration procedure. However, the RAN3 related topic on how to obtain the neighbour cell’s CSI-RS configuration was postponed to Rel-16. Two approaches on CSI-RS measurement configuration over Xn have been proposed and discussed in the past several meetings. A LS had been sent to RAN1 and RAN2 to ask the following questions [1]:

- to RAN1: CSI-RS pattern and variability w.r.t. time(?)

- to RAN2: to define the appropriate container to be sent over Xn

In this contribution, we provide the further comparison of the two approaches and give our proposal on how to support CSI-RS based mobility in SA and EN-DC scenario.

Discussion

The Need of Support CSI-RS based Mobility in Rel-16

In RAN3#101 meeting, the offline discussion on CSI-RS configuration transfer was summarized in [2]. The benefits of CSI-RS based mobility against SSB based mobility had been confirmed. Compare to SSB, the 1-port CSI-RS used for mobility purpose can be configured with larger frequency bandwidth and narrower beam.  In order to reduce the RE overhead, SSB may not be transmitted in some cases, e.g., in CA scenario. Therefore, the measurement based on SSB may not always be able to reflect the channel quality of the narrow beam which carrying user data of the target cell. The CSI-RS based mobility, as an essential function in NR, can provide higher accurate measurement result to improve handover success rate. Moreover, the SSB based handover is not applicable for some scenarios, such as SFN scenario. The different TRPs share the same physical cell ID. UE cannot differentiate the SSBs which are transmitted from different TRPs. The Network may have problem in determining the TRP and resource for data transmission to UE in SSB based handover. However, if CSI-RS based handover is supported, network can immediately configure UE-specific TRP for data transmission based on CSI-RS and RRM measurement results that triggered the handover.

Due to CSI-RS based mobility is not supported in Rel-15, so far some chip vendors and network vendors do not have a clear roadmap to support this feature. This has made against the maturity of the industry chain for CSI-RS based mobility. Therefore, in order to construct the industry chain of CSI-RS based mobility, the feature of CSI-RS based mobility shall be supported and approved in RAN3 as soon as possible. In addition, considering that most operators have chosen EN-DC as their NR initial deployment, no X2/Xn interface between NR nodes is also a common case in the practical network deployment. The CSI-RS based mobility shall be considered in the EN-DC scenario. The offline discussion [1] also had pointed out the benefit of MR-DC to discovery more suitable SCG cells. Anyway, the support of CSI-RS based mobility for SA and EN-DC shall be supported in Rel-16.

Proposal 1: The CSI-RS based mobility for SA and EN-DC scenario shall be supported in Rel-16

Candidate Approaches Comparison

Based on the above motivation, two approaches on CSI-RS configuration transfer over Xn were proposed in past several meetings. We will discuss and compare the following two approaches:

- Approach 1: exchanged in Xn setup and updated in NG-RAN node configuration update. By this approach, the CSI-RS configuration of cells will be exchanged when the Xn interface is setup between two neighbouring nodes.

- Approach 2: New class 2 procedures with registration and periodic report, like resource status request/report in X2AP. By this approach, one gNB may register a CSI-RS resource report request to a neighbouring gNB with a proper periodicity. The neighbouring gNB should report its CSI-RS resource configuration as required by the initiating gNB.

The contribution [2] gives a comparison of two approaches from the standard impact. The drawbacks of approach 1 are as follows:

- Signalling overhead: Assuming the CSI-RS configuration may change frequently, massive update signalling of NG-RAN node configuration will be introduced. However, how frequent and dynamic will the CSI-RS configuration be changed needs to consult to RAN1.

- Energy saving：CSI-RS in a cell designed by RAN1 can be switched on and off. This approach needs to transmit the CSI reference signals in a cell frequently even in case there is no UE.

The assumption of the above analysis is the configuration of the CSI-RS measurement changing frequently. Although in theory the CSI-RS configuration can be configured with per UE level and changed frequently, from the perspective of practical network there seems no benefit to do so. As per RAN1 LS reply, whether to configure CSI-RS as per UE level or per cell level is up to NW implementation. In our understanding, CSI-RS configuration shared by groups of UEs or all UEs can effectively decrease CSI-RS overhead and improve UE’s downlink data rate.

It is RAN1’s understanding that a serving NG-RAN node will configure CSI-RS measurements of neighbor cells to its served UEs by UE-specific RRC message (i.e., the IE of CSI-RS-ResourceConfigMobility in TS38.331), and CSI-RS measurements of neighbour cells are allocated UE-specifically. Whether to configure CSI-RS measurements of neighbour cells dedicated to one UE, or shared by group of UEs, or shared by all UEs within one cell, is up to NW implementation and transparent to the UE.

Regarding the frequency of updating configuration, RAN1 gives the following reply:

The frequency of updating configuration of CSI-RS transmitted for the purpose of connected mode inter-cell mobility is up to NW implementation. RAN1 assumes that the configuration of CSI-RS transmitted for the purpose of connected mode inter-cell mobility in a cell is semi-static and not changed frequently. In some implementation, low latency, e.g. within 100 msec, of CSI-RS configuration information exchange between gNBs may be required.

Observation 1: The semi-static or static CSI-RS configuration for mobility purpose is sufficient for network deployment and optimization

Therefore, there is no obvious difference between the approach 1 and 2 from the perspective of energy saving and signalling overhead. The approach 1 has less impact on the RAN3 specification compare to approach 2, and only new optional IEs shall be defined and contained in serving cell information NR in Xn setup or gNB Configuration updated procedure for SA scenario.

Proposal 2: CSI-RS configuration exchanged in Xn setup and updated in NG-RAN node configuration update for SA scenario shall be approved in RAN3.

For EN-DC scenario, the following scenarios shown in Fig1 and Fig2 shall be considered:

Figure 1 SN change

Figure 2 inter-MN HO with SN change

In the above scenarios, the CSI-RS measurement configuration is generated by SN node. Since there is no direct interface between en-gNB, the SN node should send its updated configuration via X2 interface to the eNB. And the eNB should forward the updated information to its neighbour eNBs and en-gNBs. It is clear that the approach 1 is also a simple solution without additional X2 signalling occurs for the EN-DC scenario. Only new optional IE shall be contained in served cell information NR in following X2 procedures:

- EN-DC X2 Setup

- EN-DC Configuration Update

- X2 Setup

- eNB Configuration Update

Proposal 3: In EN-DC scenario, the CSI-RS configuration information for mobility shall be contained in X2 related signalling to support CSI-RS based mobility.

Additional information for Switching off CSI-RS based Mobility

Regarding the detail CSI-RS configuration information, RAN2 will discuss and define the appropriate container to be sent between nodes. From RAN3 side, we still have the need to discuss whether or not to introduce additional information to indicate CSI-RS based mobility switching off. The following figure illustrates the scenario:

Figure 3 inter-vendor scenarios

The gNBs in vendor A region may switch off the CSI-RS based mobility function according to the OMC’s policy. In order to avoid HO failure between the boundary area, the gNBs in vendor B region shall acquire this information in time. In principle the vendor B’s OMC can provide the switching off information to the gNBs in vendor B’s region . But in the practical network, we cannot assume that any vendor’s OMC can configure all the information or configuration you need to its gNB. Because OMC is not everything. The following alternatives can be used to indicate the CSI-RS based mobility switching off .

- Alt 1: No CSI-RS configuration information contained in X2/Xn message. As per current specifications, the target node may not update corresponding parameters or configuration when an optional configuration is not contained in Served Cell Information or NR Neighbour Information IE in (EN-DC) Configuration Update procedure.

- Alt 2: An explicit indication in X2/Xn message. This is a straightforward solution to solve this problem.

Proposal 4: An optional IE to indicate the CSI-RS based mobility function turning off shall be contained in EN-DC Configuration Update/eNB Configuration Update procedure.

Proposal

This paper mainly discusses the support of inter-node CSI-RS based mobility. Based on the above analysis, we have the following proposals:

Proposal 1: The CSI-RS based mobility for SA and EN-DC scenarios shall be supported in Rel-16

Proposal 2: CSI-RS configuration exchanged in Xn setup and updated in NG-RAN node configuration update for SA scenario shall be approved in RAN3

Proposal 3: In EN-DC scenario, the CSI-RS configuration information for mobility shall be contained in X2 related signalling to support CSI-RS based mobility.

Proposal 4: An optional IE to indicate the CSI-RS based mobility function turning off shall be contained in EN-DC Configuration Update/eNB Configuration Update procedure.

Reference

- R3-192100, RAN3, LS on CSI-RS configuration transfer over Xn

- R3-185188, Huawei, Summary of offline discussion for CSI-RS Configuration Transfer,3GPP TSG-RAN3 Meeting #101

- R1-1907810, RAN1, Reply LS on CSI-RS configuration transfer over Xn